Cyclades Cyclom-Y INT14 Driver

Function Reference Guide

------------------------

This is a reference guide to the Cyclom-Y INT14 Driver.  All of these

functions are available through the INT 14h system call.

Throughout this guide, references are made to the Line Status, Modem Status,

and Modem Control values.  The meanings of these values are:

Line Status Bits:

0 - Data Ready

1 - Overrun Error

2 - Parity Error

3 - Framing Error

4 - Break Received

5 - Transmitter Holding Register Empty

6 - Transmitter Empty

7 - Error in receiver FIFO

Note:  Bits 5 and 6 report if there is space in the

transmit buffer, and not the actual state of the

hardware.  Likewise, bit 0 is set to one if there

is data available in the receive buffer.

Modem Status Bits:

0 - Delta CTS

1 - Delta DSR

2 - Trailing Edge Ring Indicator

3 - Delta DCD

4 - CTS

5 - DSR

6 - RI

7 - DCD

Modem Control Bits:

0 - DTR

1 - RTS

2 - Out 1

3 - Out 2

4 - Local Loopback

5 - 0

6 - 0

7 - 0

Note:  Bits 2-4 are not implemented in these functions.

###########################################################################

###########################################################################

Initalize Port                                               Function 00h

This function initalizes a port.

See also, Function 04h -- Extended Port Initalization

DX = xxxx	Channel Number

AH = 00h	Function Number

AL = xx	Channel Parameters:

Bits 0-1:  Data Bits

00 = 5 bits

01 = 6 bits

10 = 7 bits

11 = 8 bits

Bit 2:  Stop bits

0 = 1 Stop Bit

1 = 2 Stop bits

Bits 3-4:  Parity

00 = None

01 = Odd

10 = None

11  Even

Bits 5-7:  Baud Rate

000 =  110 or  19200   Depending on weither EBIOS

001 =  150 or  19200   support was enabled upon driver

010 =  300 or  38400   installation.

011 =  600 or  57600

100 = 1200 or 115200

101 = 2400

110 = 4800

111 = 9600

Return Values:

AH = FFh	Error, or:

AH = xx	Line Status

AL = xx	Modem Status

###########################################################################

###########################################################################

Output A Single Character                                    Function 01h

Output a single character on the specified channel.

DX = xxxx	Channel Number

AH = 01h	Function Number

AL = xx	Character to transmit

Return Values:

AL = xx	Character transmitted

AH = xx	Line Status

###########################################################################

##########################################################################

Input A Single Character                                     Function 02h

Read a single character from the specified channel.

DX = xxxx	Channel number

AH = 02h	Function number

Return Values:

AL = xx	Character Received

AH = xx	Line Status

###########################################################################

###########################################################################

Get Status                                                   Function 03h

Get the line and modem status of the specified channel.

DX = xxxx	Channel Number

AH = 03h	Function Number

Return Values:

AH = FFh	Error, or:

AH = xx	Line Status

AL = xx	Modem Status

###########################################################################

###########################################################################

Extended Port Initalization                                  Function 04h

This function provides more initalization options than Function 00h.

If AL=1, a break is initiated and no further initalization is done.

The break condition is stopped when this function is called again with

AL=0, or a character is transmitted.

DX = xxxx	Channel Number

AH = 04h	Function Number

AL = xx	Break Setting:

0 = No break

1 = Break until reset by this function w/AL = 0.

BH = xx	Parity:

0 = None

1 = Odd

2 = Even

3 = Stick Parity Odd

4 = Stick Parity Even

BL = xx	Stop Bits:

0 = 1 Stop Bit

1 = 2 Stop Bits

CH = xx	Character Length:

0 = 5 bits

1 = 6 bits

2 = 7 bits

3 = 8 bits

CL = xx	Baud Rate

0 =   110 Baud	 9 =  38400 Baud

1 =   150		10 =  57600

2 =   300		11 =  76800

3 =   600		12 = 115200

4 =  1200		13 =     50

5 =  2400		14 =     75

6 =  4800		15 =    134

7 =  9600		16 =    200

8 = 19200		17 =   1800

Return Values:

AH = FFh	Error, or:

AH = xx	Line Status

AL = xx	Mdem Status

###########################################################################

###########################################################################

Extended Communications Port Control                         Function 05h

This function enables softwaret to directly monitor and control

the handshaking lines DTR and RTS.  If automatic hardware handshaking

is enabled, this function will not affect DTR and/or RTS.

DX = xxxx	Channel Number

AH = 05h	Function Number

AL = xx	00h = Read Modem Control Reg

01h = Write Modem Control Reg

BX = xx	If AL = 01h, and hardware handshaking is disabled

Bit 0 = DTR  (1 = Raise DTR)

Bit 1 = RTS

Bit 2 = OUT1 -- Modem Output Control Line 1

Bit 3 = OUT2 -- Modem Output Control Line 2

Return Values:

AH = xx	Line Status

AL = xx	Modem Status

BL = xx	Modem Control Register

###########################################################################

##########################################################################

Get the Port Name                                            Function 06h

Sub-fctn 00h

Get the port name of the channel.

DX = xxxx	Channel Number

AH = 06h	Function Number

AL = 00h	Sub-function Number

Return Values:

AH = FFh	Error, or:

AL = xx	Number of the highest INT14 Function Supported

ES:BX	Pointer to 8 byte ASCII String which contains the name of the

port.  The string is padded with spaces!

###########################################################################

###########################################################################

Get Driver Information                                       Function 06h

Sub-fctn 01h

Get information about the driver.

DX = xxxx	Channel Number

AH = 06h	Function Number

AL = 01h	Sub-function Number

Return Values:

AH = FFh	Error, or:

AX = xxxx	otal number of channels supported

BX = xxxx	Driver version number (V2.01 == BX=0201h)

CX = xxxx	Total number of boards supported.

DX = xxxx	Lowest channel number supported by this driver

###########################################################################

###########################################################################

Get Board Information                                        Function 06h

Sub-fctn 02h

Get information about the board.

DX = xxxx	Channel Number

AH = 06h	Function number

AL = 02h	Sub-function number

BX = xxxx	Board number

Return Values:

AH = FFh	Error, or:

AH = xx	IRQ Number

AL = xx	Board Type (10h = Cyclom-X, 11h = Cyclom-Y, 12h = Cyclom-Z)

BX = xxxx	Board's dual-ported memory segment

CX = xxxx	Number of channels on the board

DX = xxxx	Board's I/O port address.

SI = xxxx	Channel number of the frst channel on the board.

###########################################################################

###########################################################################

Get Driver Information                                       Function 06h

Sub-fctn FFh

Get driver name, version number, and number of channels

DX = xxxx	Channel number

AH = 06h	Function Number

AL = FFh	Sub-function Number

Return Values:

AX = xxxx	Driver Version Number

CX = xxxx	Number of channels supported

ES:BX	Pointer to 8 byte string of the drivers name -- padded with

spaces and not null-terminated.

###########################################################################

###########################################################################

Send break                                                   Function 07h

Force a BREAK for a specified interval (default 250 milliseconds).

Note:  The maximum break time is 255 milliseconds.  Any attempt to

generate a longer break will result in a break of 255 milliseconds.

Longer breaks can be done through a call to INT14 Function 04h.

DX = xxxx	Channel Number

AH = 07h	Function Number

AL = 00h	Default Break Time, or:

AL = 01h	Use the value in BX to determine the break time.

BX = xxxx	Break Time = 10ms * BX

Return Values:

AH = FFh	Error, or:

AH = 00h	No Errors.

###########################################################################

###########################################################################

Alternate Status Check                                       Function 08h

Perform a non-descructive read from the receive buffer.

DX = xxxx	Channel Number

AH = 08h	Function Number

Return Values:

ZF = 0	Next character in input buffer will appear in AL.

AL = xx	Next charactger to be read by a "Read Single Character"

or "Read String" function.

AH = xx	Line Status associated with the next character to be read.

ZF = 1	No character in buffer

AH = FFh	No character in buffer

###########################################################################

###########################################################################

Clear Buffers                                                Function 09h

This function clears/flushes both the transmit and receive buffers

for the designated channel.

DX = xxxx	Channel number

AH = 09h

Return Values:

AH = FFh	Error

AH = 00h	No Errors

###########################################################################

###########################################################################

Input Queue Check                                            Function 0Ah

This function returns the number of characters currently in the receive

buffer.

DX = xxxx	Channel Number

AH = 0Ah	Function Number

Return Values:

AX = xxxx	Number of characters in the input buffer

DH = FFh	Error.

ZF = 1	Error.

###########################################################################

###########################################################################

Drop Handshake Lines                                         Function 0Bh

This function causes DTR and RTS to drop.

If hardware handshaking is enabled, DTR and RTS are droped for an instant

and then reactivated.  If hardware handshaking is disabled, then DTR and

RTS will remain low until a call to function 13h or until hardware hand-

shaking is enabled though function 1Eh.

DX = xxxx	Channel Number

AH = 0Bh	Function Number

Return Values:

AH = FFh	Error

AH = 00h	No Error.

###########################################################################

###########################################################################

Get Channel Paramaters                                       Function 0Ch

This function returns the current settings of flow control, baud rate,

parity and framing parameters for the designated channel.

DX = xxxx	Channel Number

AX = 0Ch	Function Number

Return Values:

AH = FFh	Error, or:

AH = xx	Software Flow Control (XON/XOFF):

Bit 0 = Transmit Flow Control (0 = inactive, 1 = enabled)

Bit 1 = Receive Flow Control (0 = inactive, 1 = enabled)

AL = xx	Hardware Flow Control:

Bit 0 = DTR receive control (1=DTR used to control flow)

Bit 1 = RTS receive control (1=RTS used to control flow)

Bit 4 = CTS transmit control (1=CTS is used)

Bit 5 = DSR transmit control (1=DSR is used)

Bit 7 = DCD transmit control (1=DCD is used to control flow)

BH = xx	Parity:

0 = None

1 = Odd

2 = Even

BL = xx	Stop Bits

0 = 1 Stop Bit

1 = 2 Stop Bits

CH = xx	Character Length

0 = 5 Bits

1 = 6 Bits

2 = 7 Bits

3 = 8 Bits.

CL = xx	Baud Rate

0 =   110 Baud	 9 =  38400 Baud

1 =   150		10 =  57600

2 =   300		11 =  76800

3 =   600		12 = 115200

4 =  1200		13 =     50

5 =  2400		14 =     75

6 =  4800		15 =    134

7 =  9600		16 =    200

8 = 19200		17 =   1800

###########################################################################

###########################################################################

Get pointer to character ready flag                          Function 0Dh

This function returns the pointer to a byte in host memory which can

be used as a glag to indicate the presence of absence of input data

characters in the receive budder for the designated channel.  This

permits a quick test for received data without requiring an INT14 call.

DX = xxxx	Channel Number

AH = 0Dh	Function Number

Return Values:

ES;BX	Pointer to the flag.  If the byte pointed to by ES:BX is 00h,

the receive buffer is empty.  If the byte is FFh, the buffer

is not empty.

###########################################################################

###########################################################################

Write String                                                 Function 0Eh

Place a string into the transmit buffer.  Timeout after two seconds if

there is not enough space in the buffer for the string.

DX = xxxx	Channel Number

AH = 0Eh	Function Number

CX = xxxx	Number of bytes in the string

ES:BX	Pointer to the string

Return Values:

AX = xxxx	Number of characters transmitted

DH = FFh	Error indication

ZF = 1	Error indication

###########################################################################

###########################################################################

Read String                                                  Function 0Fh

This function takes a string from the receive buffer and puts it into the

buffer specified.

DX = xxxx	Channel Number

AH = 0Fh	Function Number

CX = xxxx	Number of characters to read.

ES:BX	Pointer to the buffer.

Return Values:

AX = xxxx	Number of characters read

DH = FFh	Error flag

ZF = 1	Error flag

###########################################################################

###########################################################################

Clear Receive Buffer                                         Function 10h

Clear/Flush the receive buffer.

DX = xxxx	Channel number

AH = 10h	Function Number

Return Values:

AH = FFh	Error Flag

AH = 00h	No Error

###########################################################################

###########################################################################

Clear Transmit Buffer                                        Function 11h

Clear/Flush the transmit buffer.

DX = xxxx	Channel number

AH = 11h	Function Number

Return Values:

AH = FFh	Error Flag

AH = 00h	No Error

###########################################################################

###########################################################################

Get transmit buffer free space                               Function 2h

This function returns the amout of free space in the transmit buffer.

DX = xxxx	Channel Number

AH = 12hFunction Number

Return Values:

AX = xxxx	Number of free bytes in the transmit buffer

###########################################################################

###########################################################################

Set Handshake Lines                                          Function 13h

This function raises the RTS and DTR control lines.  If hardware

handshaking is enabled, it is temporarily suspended, the control

lines are raised, then hardware handshaking is re-activated and the

control lines return to their previous state.

DX = xxxx	Channel Number

AH = 13h	Function Number

Return Values:

AH = FFh	Error, or:

AH = 00h	No error.

###########################################################################

###########################################################################

Non-Destructive Character Read                               Function 14h

This function is similar to function 08h, alternate status check,

except the ZF flag is not set.

DX = xxxx	Channel Number

AH = 14h 	Function Number

Return

AH = Line Status

AL = xx 	Next charater to be read by a single character or read string

command.

###########################################################################

###########################################################################

Input Byte Count                                             Function 15h

This function returns the number of bytes waiting in the receive buffer.

This is different than the number of characters, because under certain

conditions, there can be more bytes in the buffer than there are

characters.

DX = xxxx	Channel Number

AH = 15h	Function Number

Return Values:

AX = xxxx	Number of bytes in the receive buffer.

###########################################################################

##########################################################################

Pause/Resume Transmit                                        Function 17h

This function behaves as if an XON/XOFF character was received, and

will pause transmission accordingly.

This function overrides XON/XOFF output flow control.

Note:  This function will not override hardware flow control

DX = xxxx	Channel Number

AH = 17h 	Function Number

AL = xx	1 = Pause Transmission.  2 = Resume.

Return Values:

AH = FFh	Error, or:

AH = 00h	No Errors.

###########################################################################

###########################################################################

Pause/Resume Receive                                         Function 18h

This function is immediately activates or released receive flow control.

Note:  If the number of characters in the input buffer is less than the

receive low water mark, any attempts to pause reception will be ignored.

Similarly, if the number of characters in the input buffer is greater

than the receive high water mark, any attempts to resume reception

will be ignored.

DX = xxxx	Channel Number

AH = 18h	Function Number

AL = xx	0 = Pause Reception, 1 = Resume reception

Return Values:

AH = FFh	Error, or:

AH = 00h	No Error

###########################################################################

###########################################################################

Get Water Marks and Buffer Size                              Function 1Bh

This function returns the size of the selected buffer and the water marks

for that buffer.

DX = xxxx	Channel Number

AH = 1Bh	Function Number

AL = xx	0 = Get tramsmit low water mark

1 = Get Receive Low Water mark

2 = Get Reveive High Water Mark

Return Values:

AX = xxxx	The requested water mark.

BX = xxxx	Size of the buffer

DH = FFh	Error flag

ZF = 1	Error flag

##########################################################################

###########################################################################

Set Water Marks                                             Function 1Ch

This function provides control over the points at which flow control

actions take place during read operations, and at which an interrupt

may be requested during write operations.

Receive High Water Mark:

Then the number of bytes in the receive buffer reaches the Receive

High Water Mark, the buffer is considered to be nearly full, and

appropriate handshaking is invoked (XOFF is sent and/or any active

hardware handshake lines are dropped) totell the sender to pause

transmission.

Receive Low Water Mark:

Once handskaing has been invoked, to pause remote transmission, it

remains in effect until characters are read from the buffer until the

number of bytes in the buffer has been reduced to the Receive Low

Water Mark.  Then appropriate action is taken-- XON is sent and/or

the active hardware handshake lines re raised.

Transmit Low Water Mark:

Boards bay be configured to generate an IRQ when the number of

characters in the tramsmit buffer drops to the Transmit Low Water

Mark.

DX = xxxx	Channel Number

AH = 1Ch	Function Number

AL = xx	0 = Set transmit low water mark

1 = Set receive low water mark

2 = Set receive high ware mark

BX = xxxx	Water Mark Value

Return Values:

AH = FFh	Error, or:

AH = 00h	No Errors.

###########################################################################

###########################################################################

Set Flow Control                                             Function 1Eh

This function defines the method used for input and output flow control.

DX = xxxx	Channel Number

AH = 1Eh	Function Number

BH = xx	Software Flow Control (XON/XOFF)

Bit 0 = Transmit flow control (1 = active, 0 = off)

Bit 1 = Receive flow control (1 = active, 0 = off)

Bit 2 = Set new XON/XOFF Characters.  1 = Set new XON/XOFF

CH = XOFF, CL = XON

BL = xx	Hardware Flow Control

Bit 0 = DTR receive flow control (1 = active)

Bit 1 = RTS reveive flow control

Bit 4 = CTS transmit flow control

Bit 5 = DSR transmit flow control

Bit 7 = DCD transmit flow control

Return Values:

AH = FFh	Error, or:

AH = 00h	No errors.

###########################################################################

###########################################################################

Enable/Disable Read/Write Timeouts                           Function 20h

Change timeout settings.

DX = xxxx	Channel Number

AH = 20h	Function Number

AL = xx	0 = Disable Timeout, 1 = Enable Timeout

Return Values:

AH = FFh	Error, or:

AH = 00h	No Errors.

###########################################################################

###########################################################################

EBIOS - Presence Test                                        Function F4h

Test to determine whether the device driver has been configured for EBIOS

support.

DX = xxxx	Channel Number

AH = F4h	Function Number

Return Values:

AX = 0000h	EBIOS Functions present

AH = FFh	EBIOS Functions not present.

###########################################################################

###########################################################################

EBIOS - Specify Modem Leads                                  Function FBh

Set Modem Line Handshaking options.

Note:  If EBIOS Support was not turned on when the driver was installed,

this function will return with AH=FFh.

DX = xxxx	Channel Number

AH = FBh	Function Number

AL = xx	Modem Leads:

Bit 0:  Raise DTR if 1, lower if 0.

Bit 1:  Raise RTS

Bit 4:  Use CTS for output flow control.

Bit 5:  Use DSR for output flow control.

Bit 6:  Use RI for output flow control.

Bit 7:  Use DCD for output flow control.

Return Values:

None.

###########################################################################

###########################################################################

EBIOS - Receive with No Wait                                 Function FCh

Read a single character without waiting.  If the buffer is empty, this

function returns immidiately.  See also, function 02h.

Note:  If EBIOS Support was not turned on when the driver was installed,

this function will return with AH=FFh.

DX = xxxx	Channel Number

AH = FCh	Function Number

Return Values:

AL = xx	Character Read

AH = xx	Line Status.

###########################################################################

###########################################################################

EBIOS - Retrieve Buffer Count                                Function FDh

Return the number of characters currently in the buffer.

Note:  If EBIOS Support was not turned on when the driver was installed,

this function will return with AH=FFh.

DX = xxxx	Channel Number

AH = FDh	Function Number

AL = xx	Buffer to check (1=transmit buffer, 2 = receive buffer)

Return Values:

AX = FFh	Error, or:

AX = 00h	No error

CX = xxxx	Character Count

###########################################################################

###########################################################################

EBIOS - Set Pacing Mode                                      Function FEh

Enable or disable XON/XOFF handshaking for a specified channel.

Note:  If EBIOS Support was not turned on when the driver was installed,

this function will return with AH=FFh.

DX = xxxx	Channel Number

AH = FEh	Function Number

AL = xx	Pacing mode.

Bit 6 = XON/XOFF for receive

Bit 7 = XON/XOFF for transmit

Return Values:

AH = FFh	Error, or:

AH = 00h	No error

###########################################################################

###########################################################################

EBIOS - Set Buffered Mode                                    Function FFh

Set buffered mode.

Note:  If EBIOS Supprt was not turned on when the driver was installed,

this function will return with AH=FFh.

A non-standard "feature" of this function is that when the buffer ptr

is NULL, or the buffer size is set to zero the default buffer is

re-enabled.

DX = xxxx	Channel Number

AH = FEh	Function Number

AL = xx	Buffer to affect (1 = Transmit Buffer, 2 = Receive Buffer)

CX = xxxx	Buffer Size

ES:BX 	Pointer to buffer

Return Values:

AH = FFh	Error, or:

AH = 00h	No error