From: "R+S Gordon" <ritchcom@execulink.com>

Date: Thu, 13 Mar 1997 22:23:42 -0500

Subject: Foil cap

Would aluminum foil be just as effective as flashing in a stacked plate or

rolled capacitor?  Also I am considering chrome plating a copper tube

primary (for visual effect only) is this allright?

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>Would aluminum foil be just as effective as flashing in a stacked plate or

T>rolled capacitor?  Also I am considering chrome plating a copper tube

T>primary (for visual effect only) is this allright?

Foil should be OK, but the problems of eliminating sharp edges to

prevent sparking is something to consider.  I used to use foil when I

built glass plate caps.  I folded the edge of the foil back on itself,

which seemed to help somewhat.  Another problem is the difficulty of

connecting to the thin foil without tearing it.  Securing the

connections is a must, since the foil tears very easily if the cap is

jarred excessively.

Chrome has a higher RF resistance than copper, but the slight

reduction in output may not make that much difference for your

demonstration coil.  It would look pretty, that's for sure!  Of course,

you could have the coil gold plated, since gold is an excellent

conductor, and looks beautiful as well.

Ralph

---

Driving Lesson #1: Shiny side up; Rubber side down.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 13 Mar 1997 23:28:24 -0600

Subject: Re: Foil cap

At 03:23 AM 3/14/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Thu, 13 Mar 1997 22:23:42 -0500

>Subject: Foil cap

>

>Would aluminum foil be just as effective as flashing in a stacked plate or

>rolled capacitor?  Also I am considering chrome plating a copper tube

>primary (for visual effect only) is this allright?

>

>        Ritch Gordon

>

Yes aluminum foil works well in a flat plate type capacitor.

I used Reynolds heavy duty foil when I built mine.

Do use the heavy duty foil. The standard weight foil is

just too delicate.

I have made a couple small rolled caps using foil but its really

easy to crinkle it. If your technique is good go ahead and use the foil

but I would recommend using the flashing for a rolled cap.

I wouldn't chrome it. It will lower the efficiency of the primary

because chrome has a higher resistance than copper. And because

of skin effect the RF current will tend to flow on the outside of the

conductor. It will be flowing mostly in the chrome plating.

Now if you wanted to silver plate it that would be another story.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Fri, 14 Mar 1997 01:39:06 -0500

Subject: Flybacks

Is there any good high voltage experiments you can do with a flyback trans,

because i can't find a neon in my price range.

is it hard to repair neon or is it not worth all the money youll save?

will a flyback work for a jacobs ladder

where on the flyback do i put the low voltage in and where will the high

come out

??

thanx a lot

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Fri, 14 Mar 1997 01:10:37 -0600

Subject: Re: Flybacks

At 06:39 AM 3/14/97 GMT, you wrote:

>From: "Chad Westbrook" <emu@clarityconnect.com>

>Date: Fri, 14 Mar 1997 01:39:06 -0500

>Subject: Flybacks

>

>Is there any good high voltage experiments you can do with a flyback trans,

>because i can't find a neon in my price range.

>

>is it hard to repair neon or is it not worth all the money youll save?

>

>will a flyback work for a jacobs ladder

>

>where on the flyback do i put the low voltage inn and where will the high

>come out

>??

>

You need a pulse, a B+, and a heater voltage . There is many ways to go

about this but it all depends on what type of flyback you have. Try looking

at the plans on bill beatys page under plasma balls, they have some flyback

curcuit designs there. You can get there from my site

http://www.icorp.net/users/kev/tesla/zap.html then go to the links page.

The pulse voltage will come from one high speed horiz output transistor, I

forget the popular part numbers but I have heard some talk about

incorporating a 555 in place of the transistor but I do not think it will

stand up to the current and will smoke in seconds.

I hope I have helped in some way.

>thanx a lot

>

>

>

>* Send list Replies to: Tesla-2@emachine.com

>

>* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>  in the body of your message to: Listserv@Emachine.com

>

>-----

>Processed with Listserv v2.93 for Wildcat v4

>

>

end

Regards,

Kevin Nardelle

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Fri, 14 Mar 1997 11:14:06 -0500

Subject: Tuning of Coil

I'm getting close to having all my components connected and ready for

firing up my coil.  Wondered if anyone could go over the tuning process

and what order to tune what pieces.  Mike covered the safety gap pretty

well yesterday.  Any input on tuning of the primary and the gap?

Also, what about powering up using a Variac.  Is it ok to adjust the

voltage input while the coil is operating or is it better to switch off the coil

for this because it will have to be retuned?

Also, if I tune my coil for a 10Kv23ma funace transformer, will I then

have to retune when I use my 15Kv60ma transformer instead?

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 14 Mar 1997 11:05:53 -0600

Subject: Re: Flybacks

At 06:39 AM 3/14/97 GMT, you wrote:

>From: "Chad Westbrook" <emu@clarityconnect.com>

>Date: Fri, 14 Mar 1997 01:39:06 -0500

>Subject: Flybacks

>

>Is there any good high voltage experaments you can do with a flyback trans,

>because i can't find a neon in my price range.

They make good supplies for plasma globe experiments

but dont do very well powering a Tesla coil.

>

>is it hard to repair neon or is it not worth all the money youll save?

>

Yes repairing dead neons is hard work. Especially the first one.

But after you have done a few of them it gets easier.

It is definitely worthwhile. You end up with a more rugged and

reliable transformer at virtually no cost. All neon shops

end up with dead transformers. Some places don't even send back

in warranty units. They send in the plate and toss the transformer

in the dumpster.

Go talk to these places and get them to save you dead units.

Very often they can be revived just by removing the tar potting

compound. Sometimes they are just fine with no problems.

I have unpotted 8 neon transformers and was able to revive 7

of them. Number 8 was really toasted and unrepairable.

Also once repaired you can remove some of the magnetic shunts

and boost the current output.

You can boost 30ma neons to around 40ma and I have two 60ma

neons that I've boosted to 90ma. The 60s are a lot more rugged than

the 30s and will take much more abuse.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 14 Mar 1997 12:05:44 -0600

Subject: Re: Tuning of Coil

At 04:14 PM 3/14/97 GMT, you wrote:

>From: Kevin Batchelor <KBATCHELOR@arslimited.com>

>Date: Fri, 14 Mar 1997 11:14:06 -0500

>Subject: Tuning of Coil

>

>I'm getting close to having all my components connected and ready for

>firing up my coil.  Wondered if anyone could go over the tuning process

>and what order to tune what pieces.  Mike covered the safety gap pretty

>well yesterday.  Any input on tuning of the primary and the gap?

Start with a small number of gaps. (3 or 4) to keep the

primary energy low while the coil is out of tune.

Tap the primary at about 75 percent of total turns.

In other words if you have 14 primary turns tap at turn 10 or 11.

Use a small terminal or if using a toroid place a thumbtack

or nail on it with the point sticking up or out. This will

promote sparking at low power.

Turn on your coil and dial up the variac until the gap starts

firing. With only 3 or 4 gaps this will be at a relatively low

voltage.

If there is no visible spark from the terminal (or thumbtack)

bring a grounded wire close to the terminal and try to draw

a spark.

Also useful is a small flourescent light tube. This will light

up easily when brought near the output terminal.

Try moving the tap one turn at a time (POWER OFF) and checking

each time for longest spark. Once you find the tap position

that gives the longest spark (or brighest flourescent tube)

you are close to proper tune.

Now open up the gap a little by adding one more electrode.

This time you will be able to run a little more voltage input

from the variac before the gap breaks down and you will have

more output from the secondary.

Again try different primary tap positions for best spark.

Kevin this is where you will stop for a while.

You will want to run your new cap at this low power setting

for a little while to break it in. Each time you run it you will

see more air bubbles come out.

Once your cap has more or less stopped bubbling increase power

level by opening the gap one electrode at a time. Each time

recheck for best tune.

>

>Also, what about powering up using a Variac.  Is it ok to adjust the

>voltage input while the coil is operating or is it better to switch

>off the coil for this because it will have to be retuned?

The variac will not affect the tuning of your coil. It only

varies the input voltage to your neon. You can adjust it

at will while the coil is running.

>

>Also, if I tune my coil for a 10Kv23ma funace transformer, will I then

>have to retune when I use my 15Kv60ma transformer instead?

The tuning may change slightly but not significantly. What will

change is the gap requirement. You will need more series gaps

with the higher voltage transformer. Again start with

a small number of gaps and work your way up.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Fri, 14 Mar 1997 14:14:47 -0800

Subject: Re: 10KV RMS @ 300MA for less than $100

Tesla List wrote:

>

> Subject: (Fwd) Re: 10KV RMS @ 300MA for less than $100

> Date: Wed, 12 Mar 1997 00:16:38 -0500

> From: "Robert W. Stephens" <rwstephens@headwaters.com>

> To: tesla@pupman.com

>

> ------- Forwarded Message Follows -------

> From:          Self <Single-user mode>

> To:            "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Subject:       Re: 10KV RMS @ 300MA for less than $100

> Reply-to:      rwstephens@headwaters.com

> Date:          Wed, 12 Mar 1997 00:12:22 -0500

>

> > From:          "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > To:            <tesla@pupman.com>

> > Subject:       10KV RMS @ 300MA for less than $100

> > Date:          Mon, 10 Mar 1997 12:41:18 -0500

>

> > Fellow Coilers,

> > Yes, 10KV RMS @ 300ma for less than $100 delivered in the USA.

> >

> > I have run across a transformer that is just about perfect for Tesla Coil

> > work, and I want to let you all in on it.

> >

> > Herbach and Rademan have 100 of these transformers in stock. They are rated

> > at 5,000 volts RMS, 300 ma continuous duty. These are *open-frame*

> > transformers, absolutely NO tar or anything else to give you problems. Mine

> > came with power factor correction capacitors included.

> >

> > There are current shunts installed... which is actually a *plus*, because

> > it means you do not need any reactance added to the primary! If you insist

> > on removing the shunts, it is very easy to do so, since it is all open

> > frame construction!

> >

> > The secondary is entirely isolated from the core!

> >

> > I bought two of these and wired the  primaries in anti-parallel. This makes

> > the phase of the two secondaries *opposite*. That allows me to take the

> > inner wires of each secondary and wire them together so that the two

> > *outer* wires of the secondaries are oppositely phased. In other words, the

> > total series output is now 10,000 volts AC RMS (14KV Peak!) at 300 ma. If

> > desired, the inner wires of the secondaries can be connected to AC ground,

> > so that the electrical stresses are reduced. But if you want to, you can

> > also leave them floating. Both ways work.

> >

> > By two for $39.50 each, throw in about $15 for shipping, and you are under

> > $100 for an almost perfect 10KV RMS 300ma *open frame* NO TAR transformer

> > for your Tesla coil.

> >

> > The really daring among you can figure out other combinations of these

> > transformers, I am sure.

> >

> > Part Number TM92XFR2019  5KV 300ma Transformer  $39.50 each

> >

> > Herbach and Rademan

> > 18 Canal St.

> > P.O. Box 122

> > Bristol, PA  19007-0122

> >

> > Orders 1-800-848-8001

> > Fax 215-788-9577

> > Business hours Mon-Fri 8 am - 5pm EST

> >

> > Ask to be placed on their mailing list!

> >

> > Hope that this is something useful for all of you. This is not exactly a

> > pole pig, but two of these will deliver 3,000 VA at continuous duty. Not

> > bad!

Could you please tell me how you tested the transformers and exactly

how you wired them in series? I have just ordered two of them and I

don't want to take a chance on frying them the first time I apply power.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 14 Mar 1997 17:11:38 -0600

Subject: Re: 10KV RMS @ 300MA for less than $100

At 10:14 PM 3/14/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Fri, 14 Mar 1997 14:14:47 -0800

>Subject: Re: 10KV RMS @ 300MA for less than $100

>

>Tesla List wrote:

>

>  Could you please tell me how you tested the transformers and exactly

>how you wired them in series? I have just ordered two of them and I

>don't want to take a chance on frying them the first time I apply power.

>

>

>

>

>                               Frankensteins Helper

>                                      Max

>

>

Roderick,

Those transformers may not be suitable as TC power supplies. It

appears that they may be ferro resonant transformers and even more

fragile than neons.

I was tempted to get a couple myself but I'm waiting until

someone actually reports results.

The verdict is still out on these. Since you already ordered 2 of them

report back to us what results you get. (even if bad)

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 14 Mar 1997 20:00:26 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

>

>

>   Could you please tell me how you tested the transformers and exactly

> how you wired them in series? I have just ordered two of them and I

> don't want to take a chance on frying them the first time I apply power.

>

>

>

>

>                                Frankensteins Helper

>                                       Max

Max,

I have been busy trying to gather all the info I can both by experimenting

with the two units I bought, and by checking with people that I know have

experienced success with these units.

Since you expressed a desire for this information I am attaching some of my

experiments, one of my queries, and the response I got so far.. as you will

see I followed it up with a request for more information.

The good news is that these puppies are really quite useful, even though

some people have had some bad experiences with them. Barry's experience

shows both the problem aspect, but even more importantly, the final great

result that can be had with a little patience.

Hope you find this useful.

From: "tom_mcgahee"@sigmais.com@PMDF@PAXMB1

To: Benson Barry

Subject: H&R Transformers

Date: Tuesday, March 11, 1997 9:09PM

<<File Attachment: 00000000.TXT>>

Barry,

I included information you posted on February 11 to Malcolm Watts in the

following post that I have sent out to the Tesla list. I am sending this

copy direct to you, because I would appreciate it if you would send a post

concerning your use of this type of transformer to the Tesla list under the

subject heading "RE: H&R Transformers"

In particular, if you could share with the rest of us your impression of

this transformer, and any pertinent details about using it that you can

share with us. Like, did you remove the shunts? Do you use the AC capacitor

that it came with? Are you happy with the transformer? I know that you are

probably very busy (as we all are), but this is something that you could

share with us that would be most useful. I will be looking foreward to

hearing from you. I would appreciate it if you would CC me a copy of your

response when you send it to the Tesla list, as this will allow me to read

your response earlier.

Thank you for helping Malcolm, by the way. I hope that I can perhaps get

you to fill in a Project Coiler form some time in the near future. Project

Coiler is my ongoing attempt to gather information on operational coils and

make this information available to all interested persons on the Tesla

list. I have a standard form that helps to organize this data in a way that

will hopefully be useful to many coilers.

Fr. Tom McGahee

TRANSFORMER, PHOTOCOPIER, H & R

Hi Fr. Tom McGahee, All,

I started using these transformers after seeing

a system built with them in the T.C.B.A. news.  I have been

able to get 56" of spark with 120VAC before tripping the

breaker with a 6" X 27" X #18 gauge magnet wire wound

secondary terminated with a 4 X 20" toroid using a 0.06 uF

capacitor and 14 point X 10" diameter X 1725 RPM driven

rotary wheel using 8-32 stainless steel screws with 4

parallel sets of 1/8" stationary tungsten rods.

When I first bought a pair of these transformers 10 years

ago I cooked both of them.  The high voltage winding lead out

was put too close to the low voltage side of the winding.

When stressed with Tesla coil use without protective gaps

it arced over and made one the transformers useless.  I

rebuilt the other by scraping off the carbonized insulation

and repotting with RTV.  I now run with safety gaps and

a 4" X 3" X 1" cross section cored common mode choke

with 100 turns for each side.  This limits the current and

protects the transformers from RF spikes.  I ordered a

new transformer recently that came with silicon rubber

insulated wires for the high voltage lead.  This is a vast

improvement.

These are serious transformers.  They are like mini

pole pigs.  I suppose that if you put one in oil that it

would be indestructible.  I have been thinking of using

six of these transformers with primaries in series parallel

for running 10 KW off of 220 VAC.

Barry

----------

Barry,

Thank you ever so much for responding to my request, and for sending a copy

to the Tesla list. Forgive me, but I have a few questions to ask. They may

seem trivial to you, since you have been using these type transformers for

a while, but there are many of us who are only beginning to use them.

1) Do you use the resonating winding at all? The capacitor?

2) Did you make *any* modifications at all to the transformer itself, such

as removing the shunts? If so, could you explain what you did and how you

did it? What were the practical results? Any "gotchas"?

3) You mentioned repairing one of the transformers. What is the quality of

their original construction in your opinion? Do you remember approximately

what size wire was used for the secondary? Can they really handle the rated

300 ma continuously in your opinion?

4) Of the failure modes you have experienced, what were they, and how can

they be reduced?

5) Have you added any auxiliary stuff such as external current limiting? If

so, be as explicit as you can as to what you have added, and why.

6) The common mode choke you mentioned: is this homemade, or can we buy one

somewhere? Details?

There are a fair number of Tesla list members (myself included) that have

just acquired a few of these puppies, and many more looking on from the

sidelines to see what becomes of our venture. Your practical experience in

this matter can be of great benefit to all of us. I hope that my request

for more information does not put you out, and on behalf of all of us, I

thank you again for what you have already shared with us!

Fr. Tom McGahee

Fellow Coilers,

Had a few hours yesterday and today to do some experimenting with the two

H&R

transformers that I have. At the moment I have only one Tesla Coil set up

in my shop. All the others are in various states of disarray as I am

attempting some other experiments with various component parts of the

systems.

The working Tesla coil was being used with a 7,500 volt @ 60 ma

transformer. Not your standard transformer, but that's par for the course

around here, where most everything is scrounged rather than bought. Anyway,

I disconnected the regular transformer and connected up *two* of the H&R

transformers with their primaries in parallel and their secondaries in

series. I did *not* connect up the resonant capacitor that was supplied.

Spark gap used was a single variable static gap (because I wanted to test

the transformers at the extreme by opening the gap all the way out to where

the spark gap stopped firing. For all these tests I removed all  chokes, as

I wanted to see if the transformer was prone to failure).

The results are: the secondary coil produced a spark that was twice as long

and much

hotter than with the original transformer. I attribute this to the extra

current capability of the H&R transformers. Please note that I was *not*

using the transformer in its resonant mode. In its resonant mode it

produces 5KV per transformer, but without the resonant mode it acts as a

2,600 volt 300 ma straight transformer as far as I can tell. That means

that together the two transformers were producing about 5,200 volts at 300

ma.

While I am disappointed that the 5KV rating per transformer could not be

attained in straight transformer mode at 120 AC input, I am still happy

with my purchase. I ran the poor thing for 15 minutes continuously as a

sort of torture test. The transformers were just barely warm at the end of

the test. In fact, they were almost cold.

I am interested in seeing what these transformers will do when driving a

decent multiple static gap. By the way, quenching becomes even more

important as you increase the *current* supplied by the transformer. If the

arc is not blasted with air the spark gap spark can begin to flame. My

single static spark gap looks more like a Jacob's ladder or flamethrower

when I turn off the blower. If the gap is allowed to become too hot, then

it flames and secondary spark activity decreases accordingly. This is not

due to any defect in the transformers being tested... it is just what

happens naturally with higher input currents. Make sure you have a spark

gap setup that can handle the increased currents! Otherwise you will get

disappointing results.

I tried running *one* of these puppies with 220 volts on the

primary in straight transformer mode. The core must have been saturating,

because the quality of the spark at the spark gap changed drastically. The

primary warmed up considerably, so I terminated this part of the experiment

after only about 4 minutes of testing. I definitely do not recommend

running these things too much above the rated primary voltage. Looks like

you can push them to 150 VAC with little problem, however, so you can crank

up your variacs on these! At 220 VAC there was no permanent damage done to

the transformer, but it did heat up, and the  output became very flame-like

at the spark gap.

By the way, I have been informed that H&R is supplying several variations

on this transformer. One of them uses quick connect terminals and is very

clean looking, and the other model has insulated wires for all the inputs

and outputs. This second model is the same internally, but does not look as

"spiffy" as the first model. They work identically and have the same

ratings.

I am sorry that these transformers are not exactly as great as I at first

thought they were going to be. However, it turns out that they are still

quite useable. My tests indicate that it should be possible to run at least

three and possibly even four of these with the secondaries in series.

Remember that the insulation is designed for 5KV operation per transformer,

and that without the resonating

capacitor the voltage out is about half that. Exercise prudent caution with

these transformers. The raw arcs really flame up! If you are concerned

about the level of high voltage when putting these in series, you can opt

to connect the secondary center tap of the two-transformer pair to a Good

RF Ground.

There is no need to remove the resonating winding... just don't connect

anything up to it.

Several people asked about using these transformers for Jacob's ladders.

Two of them in series make a pretty good Jacob's ladder. You might want to

try playing around with using the resonant winding and its associated

capacitor in this application. There are several possible combinations that

can be tried. Be aware that the arc is quite HOT, and it will melt metal

electrodes quite readily if the arc is allowed to stay in one place for

more than a fraction of a second. Mine chewed up a chunk of #12 solid

copper wire and spit out little metal balls. So your metal electrodes need

to be something that can take the heat. I only ran a quick test for Jacob's

ladder operation. Enough to show me that the arc produced is very nice. I

did not run an extended test to see if the transformers got warm or

anything. I'll leave that to someone who has nothing better to do with

their transformers. I'm putting *my* two transformers into permanent

service on my present coil. And I hope to maybe add another transformer in

the near future to raise the voltage level up a bit higher.

Someone asked about putting these units in oil. It looks like they would

operate well in such an environment. If you do this, you might want to

carefully perforate the outer protective covering on the HV winding so that

air bubbles can be more readily removed. I will be running my two

transformers as-is for now, and if I experience any future difficulties I

will report them to the Tesla list.

It is evident that the increased current capability over that of a neon is

very desireable. I will continue to look for other high current

transformers that can be used in Tesla coil applications.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Fri, 14 Mar 1997 21:42:12 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

> Those transformers may not be suitable as TC power supplies. It

> appears that they may be ferro resonant transformers and even more

> fragile than neons.

> I was tempted to get a couple myself but I'm waiting until

> someone actually reports results.

> The verdict is still out on these. Since you already ordered 2 of them

> report back to us what results you get. (even if bad)

I ordered four of these transformers.  One had quick-disconnect

terminals and the other three were the "directly connected wire" type.

I hooked them up as stated on the instruction sheet, with the provided

capacitor to resonate the iron core.

I wired the secondaries in series and the primaries in parallel to get

10kv out with 120v in.  I found that I had to tighten up my spark gaps

considerably to get the coil to fire  (I had been using a 12kv neon).

The spark gaps had a much greener look from the vaporizing copper

because of the increased current drawn.  I suspect the gaps would not

last very long with 3 KW of power applied!

The spark output was down considerably from my 12kv 30ma neon presumably

because of the lower voltage and because the spark gaps were overloaded

badly.  A much bigger main capacitor might have helped soak up the extra

current available.

Sadly, the transformers only ran for a few seconds, then I saw smoke.

Turned out the transformer with the quick-disconnect terminals arced

over between one of the high voltage terminals and the secondary

windings.  I'm afraid this transformer is totalled, because the paper

insulation between the secondary layers charred away.

I'm not sure if I should send back the two transformers I have not

touched yet or keep them.  At a few seconds of operation per

transformer, they are pretty expensive.  If they can be made to hold

together they would be fantastic...

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>From: Roderick Maxwell <major@vicksburg.com>

T>Date: Sat, 15 Mar 1997 21:11:57 -0800

T>Subject: Re: 10KV RMS @ 300MA for less than $100

T>Tesla-2 wrote:

T>>

T>> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

T>> Date: Fri, 14 Mar 1997 20:00:26 -0500

T>> Subject: Re: 10KV RMS @ 300MA for less than $100

T>>

T>> >

T>> >

T>> >   Could you please tell me how you tested the transformers and exactly

T>> > how you wired them in series? I have just ordered two of them and I

T>> > don't want to take a chance on frying them the first time I apply power.

T>> >

T>> >

T>> >

T>> >

T>> >                                Frankensteins Helper

T>> >                                       Max

T>>

T>> Max,

T>> I have been busy trying to gather all the info I can both by experimenting

T>> with the two units I bought, and by checking with people that I know have

T>> experienced success with these units.

<SNIP!!>

T>  Thank you for your information I'll TRY not to use them until I put

T>them under oil, and build a larger capacitor.

Hey folks!  PLEASE let's not quote 300+ lines of message for a 4 line

reply!  Please help unstuff everyones mailboxes by cutting out

unnecessary parts of the messages we are replying to.  Thanks!!

Ralph Hartwell

Listowner, TESLA-2 and NEWMAN-L  Mailing Lists

---

Electrons: What goes around comes around.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Mon, 24 Feb 1997 23:50:00 -0600

Subject: Well I am coiling

Since I have some time on my hands, I have obtained THIS AEWSOME REAL

ALUMINUM TOROID!

This sucker came from a military auction and was part of some Sci.

equipment, it is 16 inches in dia. and i.5 inches thick and solid!

This is the find of all finds for me!! I am excited can't you tell.

Regards,

Kevin Nardelle

*********************

* T   E   S   L   A *

*********************

Mail list information for acdcx, the place

to buy-sell-trade tesla and HV electronic

parts and surplus, send me mail for info

on how to subscribe.

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 24 Feb 1997 15:54:49 -0600

Subject: Anybody out there?

Everyone has sure been quiet on this list the

past few days.

I know some of you are working on coils and

some others are planning coils.

Why don't some of you building coils post

some of your specs?

It will help others who are thinking about building

coils by seeing what the rest of you are up to and

we may find some problems with your design that can

be averted. No sense building a problem into your

coil that can be prevented. Thats what this list is for.

This list will only be as good as the subscribers make it.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>Everyone has sure been quiet on this list the

T>past few days.

Things get slow occasionally on all the lists I believe.  I think it

depends on the weather. Good weather drags folks away from their

computers. (Maybe they're in the workshop, coiling away!)

T>I know some of you are working on coils and

T>some others are planning coils.

T>Why don't some of you building coils post

T>some of your specs?

Perhaps those of us who have some of the various computer programs

for designing Tesla Coils can do some data runs for some of the others

who have some parts and don't know the best way to use them; caps,

xfmrs, spools of wire, etc.

T>It will help others who are thinking about building

T>coils by seeing what the rest of you are up to and

There's nothing to get you interested in building like

reading/watching someone's get ready to build!

T>we may find some problems with your design that can

T>be averted. No sense building a problem into your

T>coil that can be prevented. Thats what this list is for.

Sigh...  all I need to do is post a list of the things I have done

that I SHOULDN'T have done when coiling! <G>  Just don't do any of the

things on that list and you'll have a great coil! <VBG>

T>This list will only be as good as the subscribers make it.

You heard'm folks - let's have your input, questions, problems,

whatever!

Happy Coiling!

Ralph

---

C:\DAMSEL.EXE x-linked with DISTRESS.DAT.  Rescue? (Y/n)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 24 Feb 1997 15:54:51 -0600

Subject: Cheap HV putty

Some time ago Jerry Gore told of using soft

rope caulking as an insulator in rebuilding neons.

I have contacted Jerry privately to see how this worked

out for him. He reports good success in using this material

in his rebuilt neons.

So I went and picked up some of this stuff and proceeded to

test it. I put some of it between the electrodes in one

of my spark gaps and cranked the voltage to it. About

40 mils of this stuff held off 14000 volts. So its definitely

a good insulator.

I used it in my latest neon rebuild to insulate the secondaries

from the core and it works just fine. Use it like HV putty. Just

mold it onto and around the secondary and fill the gap between

the secondary and core with the stuff.

It stays soft and pliable and can be removed if you need to.

The material I used is made by W.J. Dennis Company and is

called Rope Caulk Weather Strip. The small box contains 8

3/16th inch ropes 30 feet long. Quite a lot of it for $1.75

at a local Menard's. Use the white color as the gray color

may not insulate as well due to the coloring material.

Those of you rebuilding neons should give this stuff a try.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>Some time ago Jerry Gore told of using soft

T>rope caulking as an insulator in rebuilding neons.

T>of my spark gaps and cranked the voltage to it. About

T>40 mils of this stuff held off 14000 volts. So its definitely

T>a good insulator.

Well, I learned something useful today.  Thanks!  I'll have to

remember this idea for my next coil.  I can also think of some other

places where it may be useful as well.  Do you have any idea of what

the RF characteristics of this stuff are?

Ralph

C:\GRAPHICS\GIF\NAUGHTY\FILTHY\DISGUSTING\WOW!

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 15 Mar 1997 15:42:25 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

Dear Fellow Coiler,

I am sending this compendium of posts to all persons that I am currently

aware of who have purchased the H&R transformers, or who expressed an

interest in buying them. I am also sending this set of posts to the Tesla

list and the Tesla2 list (Tesla2 is a new list dedicated to helping

beginner coilers.) The good news is that these babies are quite useful. The

bad news is that I am still awaiting full details on several aspects of

their successful use. Please read *everything* in these posts. The info is

somewhat scattered throughout the posts.

Bottom line: you *need* a safety gap for each transformer. Failure to do so

will almost surely result in early demise of these excellent transformers.

The ones with quick connect connectors are more prone to failure due to the

position of the inside winding quick connect. Immersion in oil should

eliminate all problems with insulation, but safety gaps are still a must.

Due to the large currents produced you need to use a large (such as .06

mfd) HV capacitor in tank circuit. Otherwise your spark gap will flame. The

spark gap has to be really heavy-duty, as the currents are high.

As I get more information in, I will make sure that it gets posted to the

Tesla list and the Tesla2 list. I am sending the current posting to

individuals as well as to the list since I want to alert users of this

transformer as soon as possible so that no one else unneccesarily damages

one of their transformers.

I know that the urge is to just slap these babies into an existing Tesla

system as-is. That could be disastrous. It would be a real shame if these

transformers got a bad reputation simply because we didn't take the time to

implement them properly.

You will note that my own experiments were run without using the resonant

winding and its associated capacitor. Thus I was only generating 1/2 of the

available voltage. That may explain why my abuse of the transformers did

not result in any damage. Personally, I am waiting until Barry Benson

answers my second set of questions before I attempt to run these suckers in

resonant mode. I plan to oil immerse the secondaries first!

Please share with the rest of us your experiences with these transformers.

I hope to take all the information we gather on these things and create a

fact sheet so that others can benefit from what we learn.

Please be careful! There is a *known* failure mode related to the

secondary. Thus we should all be able to work around this failure mode.

Some may try oil immersion. Others may try moving the secondary connections

after careful disassembly. Some may try additional insulation such as RTV

silicone, etc. Whatever you do, please let me know both about your

successes and any failures. Be as explicit as possible about what you did.

Your information is only useful if it is complete and exact. For example,

if using RTV silicone, let me know what brand, how much you used, and any

nitty gritty details that will help others duplicate your success.

I would appreciate it if all users of this transformer would send me copies

of any e-mail discussing these transformers so that I can gather all the

relevant information into a single document that we can post to the lists

and get into the archives.

Fr. Tom McGahee

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: 10KV RMS @ 300MA for less than $100

> Date: Friday, March 14, 1997 9:42 PM

>

> From: Steve Falco <sfalco@worldnet.att.net>

> Date: Fri, 14 Mar 1997 21:42:12 -0500

> Subject: Re: 10KV RMS @ 300MA for less than $100

>

> > Those transformers may not be suitable as TC power supplies. It

> > appears that they may be ferro resonant transformers and even more

> > fragile than neons.

> > I was tempted to get a couple myself but I'm waiting until

> > someone actually reports results.

> > The verdict is still out on these. Since you already ordered 2 of them

> > report back to us what results you get. (even if bad)

>

> I ordered four of these transformers.  One had quick-disconnect

> terminals and the other three were the "directly connected wire" type.

> I hooked them up as stated on the instruction sheet, with the provided

> capacitor to resonate the iron core.

>

> I wired the secondaries in series and the primaries in parallel to get

> 10kv out with 120v in.  I found that I had to tighten up my spark gaps

> considerably to get the coil to fire  (I had been using a 12kv neon).

> The spark gaps had a much greener look from the vaporizing copper

> because of the increased current drawn.  I suspect the gaps would not

> last very long with 3 KW of power applied!

>

This is an important point. The increased current will indeed chew up spark

gaps more quickly. This should come as no big surprise to any of us. The

tuning of the gaps is obviously also an issue. You can't just throw these

transformers in a system an not expect to have to do some tuning and

upgrading of the spark gap system. The extra current availability of the

transformer requires that the capacitor be bigger, otherwise the spark gap

will flame instead of spark. Again, this is not a defect in the

transformer, but something that must be contended with when increasing the

available current in any Tesla coil system.

> The spark output was down considerably from my 12kv 30ma neon presumably

> because of the lower voltage and because the spark gaps were overloaded

> badly.  A much bigger main capacitor might have helped soak up the extra

> current available.

>

Absolutely correct. Unless a larger capacitor is used, the spark gap will

flame and power will be *lost* instead of used.

> Sadly, the transformers only ran for a few seconds, then I saw smoke.

> Turned out the transformer with the quick-disconnect terminals arced

> over between one of the high voltage terminals and the secondary

> windings.  I'm afraid this transformer is totalled, because the paper

> insulation between the secondary layers charred away.

This is precisely the problem that Barry Benson had with his first attempts

at using these transformers. The ones with the high voltage wires tend to

stand up better. The manufacturer has both quick connect HV terminals on

the same side of the transformer, and fairly close to one another. But the

real problem here is that the wire from the center of the secondary coil

has been brought up to be on top of the secondary so that the quick connect

terminal can be soldered to it.

In normal operation this appears to work OK, but in Tesla service the kick

back voltage can initiate a spark between the inner secondary wire and the

outer portion of the secondary. Once a spark has begun, it ignites with a

vengeance because of the current available.

It seems to me that immersion in oil and removal of all air bubbles after

immersion would totally eliminate this failure mechanism. To facilitate the

removal of air bubbles it might be useful to carefully perforate the

protective layer that currently covers the secondary.

The transformer looks like it would be very easy to dismantle. If this was

done, then careful unwrapping of the outer insulatory material might allow

one to attach High Voltage insulated wire in place of the quick connects.

Barry Benson mentioned that he packed his transformers with RTV silicone

for increased HV safety. I assume that he meant that he did that to the

secondary windings after disassembly.

Note that if you use oil immersion you should be able to get by without

having to worry about re-wiring the quick connects, because the oil will

provide adequate insulation.

Another very important thing that we can do to prevent these transformers

from self-destructing is to use a good protective spark gap *at each

transformer*. Please do not skimp here! Even if oil immersed, this item

seems to me to be very important! Do not try to protect all the

transformers with just one protective spark gap! Use a separate gap for

each transformer. Begin with the gaps set too close, and then widen the

safety gaps until they don't fire anymore. This will go a long way towards

protecting these transformers. Do not omit the safety gaps!!!

>

> I'm not sure if I should send back the two transformers I have not

> touched yet or keep them.  At a few seconds of operation per

> transformer, they are pretty expensive.  If they can be made to hold

> together they would be fantastic...

>

Steve, these transformers require some special care, but I believe that in

the end the extra effort will be worth it. I am still waiting for word from

Barry Benson as to further details concerning what we have to do to get

these transformers working in a safe way. I recommend that all of us that

have these transformers *WAIT* until Barry Benson (and anyone else that has

sucessfully used these transformers) can get back to us with full details.

No sense in any more of us having to learn the hard way what the "gotchas"

are with these babies!

By the way, Steve, don't throw the ruined unit out. The core is still

useable, as is the primary. You should be able to use the remnants to build

something useful, such as a primary choke. By adding a heavy duty shorted

secondary (just a few turns of really *thick* wire or rod or bar stock),

you can have a mini equivalent of a shorted arc welder. for use in driving

a primary circuit.

Below I have appended some additional stuff that I hope is useful to all

who have bought these transformers.

>         Steve Falco

>         sfalco@worldnet.att.net

>

>

----------

> From: Mike Hammer <mhammer@misslink.net>

> To: Thomas McGahee <tom_mcgahee@sigmais.com>

> Subject: H&R transformers

> Date: Friday, March 14, 1997 6:28 PM

>

> Fr. Tom,

>

> I guess by now you know that you have created a minor

> upheavel in the coiling community when you told us about

> the transformers that H&R has.

> I received 5 emails today alone from people asking me

> if these transformers are for real.

> It seems almost too good to be true.

>

> Well some people have cast doubt on the suitability of these

> because of them being ferro resonant devices.

>

> I once used a ferro resonant transformer from a large IBM copier

> that sounds similar to these and it expired rather quickly.

>

> So my question is do you have a pair of these?

> Have you done any tests on them?

> Have you run them on a coil?

> Any info would be helpful.

>

> I would like to get a pair of them myself but

> I firmly believe that if something seems to good to be true

> it usually is.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Mike,

I have been busy trying to gather all the info I can both by experimenting

with the two units I bought, and by checking with people that I know have

experienced success with these units.

Since you expressed a desire for this information I am attaching some of my

experiments, one of my queries, and the response I got so far.. as you will

see I followed it up with a request for more information.

The good news is that these puppies are really quite useful, even though

some people have had some bad experiences with them. Barry's experience

shows both the problem aspect, but even more importantly, the final great

result that can be had with a little patience.

These transformers are a bit out of the ordinary, that I will admit! For me

the bottom line is useful results. And Barry Benson is one user who has

used them with success. You may want to wait until he answers the second

set of questions I sent him. I'll tell you this much: if I had the money I

would order another one or two myself and see how far I can push these

suckers! I would want to oil immerse them first, though, as they are *not*

indestructible as they come out of the box. But a little oil can easily

clear up *that* problem!

Please keep in mind that I am new to these transformers myself. I, too have

a lot of questions. But from what I have already seen of the two units I

bought, I would have to say that they were worth what I paid. Stay tuned!

Hope you find this useful.

From: "tom_mcgahee"@sigmais.com@PMDF@PAXMB1

To: Benson Barry

Subject: H&R Transformers

Date: Tuesday, March 11, 1997 9:09PM

<<File Attachment: 00000000.TXT>>

Barry,

I included information you posted on February 11 to Malcolm Watts in the

following post that I have sent out to the Tesla list. I am sending this

copy direct to you, because I would appreciate it if you would send a post

concerning your use of this type of transformer to the Tesla list under the

subject heading "RE: H&R Transformers"

In particular, if you could share with the rest of us your impression of

this transformer, and any pertinent details about using it that you can

share with us. Like, did you remove the shunts? Do you use the AC capacitor

that it came with? Are you happy with the transformer? I know that you are

probably very busy (as we all are), but this is something that you could

share with us that would be most useful. I will be looking foreward to

hearing from you. I would appreciate it if you would CC me a copy of your

response when you send it to the Tesla list, as this will allow me to read

your response earlier.

Thank you for helping Malcolm, by the way. I hope that I can perhaps get

you to fill in a Project Coiler form some time in the near future. Project

Coiler is my ongoing attempt to gather information on operational coils and

make this information available to all interested persons on the Tesla

list. I have a standard form that helps to organize this data in a way that

will hopefully be useful to many coilers.

Fr. Tom McGahee

Here is Barry Benson's response:

Benson_Barry%PAX5@mr.nawcad.navy.mil

TRANSFORMER, PHOTOCOPIER, H & R

Hi Fr. Tom McGahee, All,

I started using these transformers after seeing

a system built with them in the T.C.B.A. news.  I have been

able to get 56" of spark with 120VAC before tripping the

breaker with a 6" X 27" X #18 gauge magnet wire wound

secondary terminated with a 4 X 20" toroid using a 0.06 uF

capacitor and 14 point X 10" diameter X 1725 RPM driven

rotary wheel using 8-32 stainless steel screws with 4

parallel sets of 1/8" stationary tungsten rods.

When I first bought a pair of these transformers 10 years

ago I cooked both of them.  The high voltage winding lead out

was put too close to the low voltage side of the winding.

When stressed with Tesla coil use without protective gaps

it arced over and made one the transformers useless.  I

rebuilt the other by scraping off the carbonized insulation

and repotting with RTV.  I now run with safety gaps and

a 4" X 3" X 1" cross section cored common mode choke

with 100 turns for each side.  This limits the current and

protects the transformers from RF spikes.  I ordered a

new transformer recently that came with silicon rubber

insulated wires for the high voltage lead.  This is a vast

improvement.

These are serious transformers.  They are like mini

pole pigs.  I suppose that if you put one in oil that it

would be indestructible.  I have been thinking of using

six of these transformers with primaries in series parallel

for running 10 KW off of 220 VAC.

Barry

----------

Barry,

Thank you ever so much for responding to my request, and for sending a copy

to the Tesla list. Forgive me, but I have a few questions to ask. They may

seem trivial to you, since you have been using these type transformers for

a while, but there are many of us who are only beginning to use them.

1) Do you use the resonating winding at all? The capacitor?

2) Did you make *any* modifications at all to the transformer itself, such

as removing the shunts? If so, could you explain what you did and how you

did it? What were the practical results? Any "gotchas"?

3) You mentioned repairing one of the transformers. What is the quality of

their original construction in your opinion? Do you remember approximately

what size wire was used for the secondary? Can they really handle the rated

300 ma continuously in your opinion?

4) Of the failure modes you have experienced, what were they, and how can

they be reduced?

5) Have you added any auxiliary stuff such as external current limiting? If

so, be as explicit as you can as to what you have added, and why.

6) The common mode choke you mentioned: is this homemade, or can we buy one

somewhere? Details?

There are a fair number of Tesla list members (myself included) that have

just acquired a few of these puppies, and many more looking on from the

sidelines to see what becomes of our venture. Your practical experience in

this matter can be of great benefit to all of us. I hope that my request

for more information does not put you out, and on behalf of all of us, I

thank you again for what you have already shared with us!

Fr. Tom McGahee

*****

Fellow Coilers,

Had a few hours yesterday and today to do some experimenting with the two

H&R

transformers that I have. At the moment I have only one Tesla Coil set up

in my shop. All the others are in various states of disarray as I am

attempting some other experiments with various component parts of the

systems.

The working Tesla coil was being used with a 7,500 volt @ 60 ma

transformer. Not your standard transformer, but that's par for the course

around here, where most everything is scrounged rather than bought. Anyway,

I disconnected the regular transformer and connected up *two* of the H&R

transformers with their primaries in parallel and their secondaries in

series. I did *not* connect up the resonant capacitor that was supplied.

Spark gap used was a single variable static gap (because I wanted to test

the transformers at the extreme by opening the gap all the way out to where

the spark gap stopped firing. For all these tests I removed all  chokes, as

I wanted to see if the transformer was prone to failure).

The results are: the secondary coil produced a spark that was twice as long

and much

hotter than with the original transformer. I attribute this to the extra

current capability of the H&R transformers. Please note that I was *not*

using the transformer in its resonant mode. In its resonant mode it

produces 5KV per transformer, but without the resonant mode it acts as a

2,600 volt 300 ma straight transformer as far as I can tell. That means

that together the two transformers were producing about 5,200 volts at 300

ma.

While I am disappointed that the 5KV rating per transformer could not be

attained in straight transformer mode at 120 AC input, I am still happy

with my purchase. I ran the poor thing for 15 minutes continuously as a

sort of torture test. The transformers were just barely warm at the end of

the test. In fact, they were almost cold.

*** note: I am waiting for input from Barry Benson as to whether or not he

uses the resonant winding or not. Until then I am *not* using the resonant

winding, because this way I am using it as a straight transformer. The fact

that I have not burned up my transformers despite the abuse I have been

giving them may be due to the fact that I am *not* using the resonant

winding. I will wait until I hear from Barry Benson before attempting

*that* part of my experimentation!

***

I am interested in seeing what these transformers will do when driving a

decent multiple static gap. By the way, quenching becomes even more

important as you increase the *current* supplied by the transformer. If the

arc is not blasted with air the spark gap spark can begin to flame. My

single static spark gap looks more like a Jacob's ladder or flamethrower

when I turn off the blower. If the gap is allowed to become too hot, then

it flames and secondary spark activity decreases accordingly. This is not

due to any defect in the transformers being tested... it is just what

happens naturally with higher input currents. Make sure you have a spark

gap setup that can handle the increased currents! Otherwise you will get

disappointing results.

I tried running *one* of these puppies with 220 volts on the

primary in straight transformer mode. The core must have been saturating,

because the quality of the spark at the spark gap changed drastically. The

primary warmed up considerably, so I terminated this part of the experiment

after only about 4 minutes of testing. I definitely do not recommend

running these things too much above the rated primary voltage. Looks like

you can push them to 150 VAC with little problem, however, so you can crank

up your variacs on these! *** oil immerse first, please!***

At 220 VAC there was no permanent damage done to

the transformer, but it did heat up, and the  output became very flame-like

at the spark gap.

*** I was foolish to try this, but since it was my post that has persuaded

a number of others to stick their necks out and purchase some of these

transformers for themselves, I feel a certain moral obligation to do all

that I can to make sure that no one else has to unnecessarily destroy one

of these transformers.

***

By the way, I have been informed that H&R is supplying several variations

on this transformer. One of them uses quick connect terminals and is very

clean looking, and the other model has insulated wires for all the inputs

and outputs. This second model is the same internally, but does not look as

"spiffy" as the first model. They work identically and have the same

ratings.

*** It turns out that the one with quick connect terminals is less

desireable, as it has more of a tendency to flash over. Read the reply from

Barry Benson for more details.

***

I am sorry that these transformers are not exactly as great as I at first

thought they were going to be. However, it turns out that they are still

quite useable. My tests indicate that it should be possible to run at least

three and possibly even four of these with the secondaries in series. ***

in oil! ***

Remember that the insulation is designed for 5KV operation per transformer,

and that without the resonating

capacitor the voltage out is about half that. Exercise prudent caution with

these transformers. The raw arcs really flame up! If you are concerned

about the level of high voltage when putting these in series, you can opt

to connect the secondary center tap of the two-transformer pair to a Good

RF Ground.

There is no need to remove the resonating winding... just don't connect

anything up to it.

*** I am waiting for more information before I attempt to use the

resonating winding. When it is forthcoming I will communicate it to the

Tesla list.

***

Several people asked about using these transformers for Jacob's ladders.

Two of them in series make a pretty good Jacob's ladder. You might want to

try playing around with using the resonant winding and its associated

capacitor in this application. There are several possible combinations that

can be tried. Be aware that the arc is quite HOT, and it will melt metal

electrodes quite readily if the arc is allowed to stay in one place for

more than a fraction of a second. Mine chewed up a chunk of #12 solid

copper wire and spit out little metal balls. So your metal electrodes need

to be something that can take the heat. I only ran a quick test for Jacob's

ladder operation. Enough to show me that the arc produced is very nice. I

did not run an extended test to see if the transformers got warm or

anything. I'll leave that to someone who has nothing better to do with

their transformers. I'm putting *my* two transformers into permanent

service on my present coil. And I hope to maybe add another transformer in

the near future to raise the voltage level up a bit higher.

Someone asked about putting these units in oil. It looks like they would

operate well in such an environment. If you do this, you might want to

carefully perforate the outer protective covering on the HV winding so that

air bubbles can be more readily removed. I will be running my two

transformers as-is for now, and if I experience any future difficulties I

will report them to the Tesla list.

*** Note: It is extremely important that *each* transformer is equipped

with a safety gap. This one little item alone can save you much grief. I

did not use safety gaps during any of my experiments, but that was very

foolish. It appears that the major failure mode for these transformers is

related to internal flash-over arcing when used for Tesla service. This is

especially the case for the units that come with the quick-connect

terminals. Failure to observe safety precautions mentioned can result in

very quick destruction of the transformer. If at all possible, oil immerse

the transformer. I found a sort of rectangular plastic container at the

local plastics store that is just large enough to hold two of the

transformers. I am goind to bolt the two of them together using short metal

strips so that they will not shift relative to one another. I will use 30KV

HV silicone wire to connect the transformer secondaries to HV terminals

that I will install on the lid. I am going to bring out the HV center tap

as well as the two end HV wires, so that I can attach a separate safety gap

to each secondary. The primary wires will also be brought out to the lid.

Since the oil insulation is really only needed at the secondaries, I plan

on stuffing the container with non-conducting stuff that is heavier than

oil. This should greatly reduce the amount of oil I have to use to bring

the level up so that it covers the secondary with an inch to spare. It will

look awful, but this should save me a couple of bucks.

***

It is evident that the increased current capability over that of a neon is

very desireable. I will continue to look for other high current

transformers that can be used in Tesla coil applications.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Sat, 15 Mar 1997 21:11:57 -0800

Subject: Re: 10KV RMS @ 300MA for less than $100

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Fri, 14 Mar 1997 20:00:26 -0500

> Subject: Re: 10KV RMS @ 300MA for less than $100

>

> >

> >

> >   Could you please tell me how you tested the transformers and exactly

> > how you wired them in series? I have just ordered two of them and I

> > don't want to take a chance on frying them the first time I apply power.

> >

> >

> >

> >

> >                                Frankensteins Helper

> >                                       Max

>

> Max,

> I have been busy trying to gather all the info I can both by experimenting

> with the two units I bought, and by checking with people that I know have

> experienced success with these units.

>

> Since you expressed a desire for this information I am attaching some of my

> experiments, one of my queries, and the response I got so far.. as you will

> see I followed it up with a request for more information.

>

> The good news is that these puppies are really quite useful, even though

> some people have had some bad experiences with them. Barry's experience

> shows both the problem aspect, but even more importantly, the final great

> result that can be had with a little patience.

>

> Hope you find this useful.

>

> From: "tom_mcgahee"@sigmais.com@PMDF@PAXMB1

> To: Benson Barry

> Subject: H&R Transformers

> Date: Tuesday, March 11, 1997 9:09PM

>

> <<File Attachment: 00000000.TXT>>

> Barry,

> I included information you posted on February 11 to Malcolm Watts in the

> following post that I have sent out to the Tesla list. I am sending this

> copy direct to you, because I would appreciate it if you would send a post

> concerning your use of this type of transformer to the Tesla list under the

> subject heading "RE: H&R Transformers"

>

> In particular, if you could share with the rest of us your impression of

> this transformer, and any pertinent details about using it that you can

> share with us. Like, did you remove the shunts? Do you use the AC capacitor

> that it came with? Are you happy with the transformer? I know that you are

> probably very busy (as we all are), but this is something that you could

> share with us that would be most useful. I will be looking foreward to

> hearing from you. I would appreciate it if you would CC me a copy of your

> response when you send it to the Tesla list, as this will allow me to read

> your response earlier.

>

> Thank you for helping Malcolm, by the way. I hope that I can perhaps get

> you to fill in a Project Coiler form some time in the near future. Project

> Coiler is my ongoing attempt to gather information on operational coils and

> make this information available to all interested persons on the Tesla

> list. I have a standard form that helps to organize this data in a way that

> will hopefully be useful to many coilers.

>

> Fr. Tom McGahee

>

> TRANSFORMER, PHOTOCOPIER, H & R

> Hi Fr. Tom McGahee, All,

>      I started using these transformers after seeing

> a system built with them in the T.C.B.A. news.  I have been

> able to get 56" of spark with 120VAC before tripping the

> breaker with a 6" X 27" X #18 gauge magnet wire wound

> secondary terminated with a 4 X 20" toroid using a 0.06 uF

> capacitor and 14 point X 10" diameter X 1725 RPM driven

> rotary wheel using 8-32 stainless steel screws with 4

> parallel sets of 1/8" stationary tungsten rods.

>

> When I first bought a pair of these transformers 10 years

> ago I cooked both of them.  The high voltage winding lead out

> was put too close to the low voltage side of the winding.

> When stressed with Tesla coil use without protective gaps

> it arced over and made one the transformers useless.  I

> rebuilt the other by scraping off the carbonized insulation

> and repotting with RTV.  I now run with safety gaps and

> a 4" X 3" X 1" cross section cored common mode choke

> with 100 turns for each side.  This limits the current and

> protects the transformers from RF spikes.  I ordered a

> new transformer recently that came with silicon rubber

> insulated wires for the high voltage lead.  This is a vast

> improvement.

>

> These are serious transformers.  They are like mini

> pole pigs.  I suppose that if you put one in oil that it

> would be indestructible.  I have been thinking of using

> six of these transformers with primaries in series parallel

> for running 10 KW off of 220 VAC.

>

> Barry

>

>  ----------

>

> Barry,

> Thank you ever so much for responding to my request, and for sending a copy

> to the Tesla list. Forgive me, but I have a few questions to ask. They may

> seem trivial to you, since you have been using these type transformers for

> a while, but there are many of us who are only beginning to use them.

>

> 1) Do you use the resonating winding at all? The capacitor?

>

> 2) Did you make *any* modifications at all to the transformer itself, such

> as removing the shunts? If so, could you explain what you did and how you

> did it? What were the practical results? Any "gotchas"?

>

> 3) You mentioned repairing one of the transformers. What is the quality of

> their original construction in your opinion? Do you remember approximately

> what size wire was used for the secondary? Can they really handle the rated

> 300 ma continuously in your opinion?

>

> 4) Of the failure modes you have experienced, what were they, and how can

> they be reduced?

>

> 5) Have you added any auxiliary stuff such as external current limiting? If

> so, be as explicit as you can as to what you have added, and why.

>

> 6) The common mode choke you mentioned: is this homemade, or can we buy one

> somewhere? Details?

>

> There are a fair number of Tesla list members (myself included) that have

> just acquired a few of these puppies, and many more looking on from the

> sidelines to see what becomes of our venture. Your practical experience in

> this matter can be of great benefit to all of us. I hope that my request

> for more information does not put you out, and on behalf of all of us, I

> thank you again for what you have already shared with us!

>

> Fr. Tom McGahee

>

> Fellow Coilers,

> Had a few hours yesterday and today to do some experimenting with the two

> H&R

> transformers that I have. At the moment I have only one Tesla Coil set up

> in my shop. All the others are in various states of disarray as I am

> attempting some other experiments with various component parts of the

> systems.

>

> The working Tesla coil was being used with a 7,500 volt @ 60 ma

> transformer. Not your standard transformer, but that's par for the course

> around here, where most everything is scrounged rather than bought. Anyway,

> I disconnected the regular transformer and connected up *two* of the H&R

> transformers with their primaries in parallel and their secondaries in

> series. I did *not* connect up the resonant capacitor that was supplied.

> Spark gap used was a single variable static gap (because I wanted to test

> the transformers at the extreme by opening the gap all the way out to where

> the spark gap stopped firing. For all these tests I removed all  chokes, as

> I wanted to see if the transformer was prone to failure).

>

> The results are: the secondary coil produced a spark that was twice as long

> and much

> hotter than with the original transformer. I attribute this to the extra

> current capability of the H&R transformers. Please note that I was *not*

> using the transformer in its resonant mode. In its resonant mode it

> produces 5KV per transformer, but without the resonant mode it acts as a

> 2,600 volt 300 ma straight transformer as far as I can tell. That means

> that together the two transformers were producing about 5,200 volts at 300

> ma.

>

> While I am disappointed that the 5KV rating per transformer could not be

> attained in straight transformer mode at 120 AC input, I am still happy

> with my purchase. I ran the poor thing for 15 minutes continuously as a

> sort of torture test. The transformers were just barely warm at the end of

> the test. In fact, they were almost cold.

>

> I am interested in seeing what these transformers will do when driving a

> decent multiple static gap. By the way, quenching becomes even more

> important as you increase the *current* supplied by the transformer. If the

> arc is not blasted with air the spark gap spark can begin to flame. My

> single static spark gap looks more like a Jacob's ladder or flamethrower

> when I turn off the blower. If the gap is allowed to become too hot, then

> it flames and secondary spark activity decreases accordingly. This is not

> due to any defect in the transformers being tested... it is just what

> happens naturally with higher input currents. Make sure you have a spark

> gap setup that can handle the increased currents! Otherwise you will get

> disappointing results.

>

> I tried running *one* of these puppies with 220 volts on the

> primary in straight transformer mode. The core must have been saturating,

> because the quality of the spark at the spark gap changed drastically. The

> primary warmed up considerably, so I terminated this part of the experiment

> after only about 4 minutes of testing. I definitely do not recommend

> running these things too much above the rated primary voltage. Looks like

> you can push them to 150 VAC with little problem, however, so you can crank

> up your variacs on these! At 220 VAC there was no permanent damage done to

> the transformer, but it did heat up, and the  output became very flame-like

> at the spark gap.

>

> By the way, I have been informed that H&R is supplying several variations

> on this transformer. One of them uses quick connect terminals and is very

> clean looking, and the other model has insulated wires for all the inputs

> and outputs. This second model is the same internally, but does not look as

> "spiffy" as the first model. They work identically and have the same

> ratings.

>

> I am sorry that these transformers are not exactly as great as I at first

> thought they were going to be. However, it turns out that they are still

> quite useable. My tests indicate that it should be possible to run at least

> three and possibly even four of these with the secondaries in series.

> Remember that the insulation is designed for 5KV operation per transformer,

> and that without the resonating

> capacitor the voltage out is about half that. Exercise prudent caution with

> these transformers. The raw arcs really flame up! If you are concerned

> about the level of high voltage when putting these in series, you can opt

> to connect the secondary center tap of the two-transformer pair to a Good

> RF Ground.

>

> There is no need to remove the resonating winding... just don't connect

> anything up to it.

>

> Several people asked about using these transformers for Jacob's ladders.

> Two of them in series make a pretty good Jacob's ladder. You might want to

> try playing around with using the resonant winding and its associated

> capacitor in this application. There are several possible combinations that

> can be tried. Be aware that the arc is quite HOT, and it will melt metal

> electrodes quite readily if the arc is allowed to stay in one place for

> more than a fraction of a second. Mine chewed up a chunk of #12 solid

> copper wire and spit out little metal balls. So your metal electrodes need

> to be something that can take the heat. I only ran a quick test for Jacob's

> ladder operation. Enough to show me that the arc produced is very nice. I

> did not run an extended test to see if the transformers got warm or

> anything. I'll leave that to someone who has nothing better to do with

> their transformers. I'm putting *my* two transformers into permanent

> service on my present coil. And I hope to maybe add another transformer in

> the near future to raise the voltage level up a bit higher.

>

> Someone asked about putting these units in oil. It looks like they would

> operate well in such an environment. If you do this, you might want to

> carefully perforate the outer protective covering on the HV winding so that

> air bubbles can be more readily removed. I will be running my two

> transformers as-is for now, and if I experience any future difficulties I

> will report them to the Tesla list.

>

> It is evident that the increased current capability over that of a neon is

> very desireable. I will continue to look for other high current

> transformers that can be used in Tesla coil applications.

>

> Fr. Tom McGahee

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Thank you for your information I'll TRY not to use them until I put

them under oil, and build a larger capacitor.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Peter Dalinis <pdalinis@xnet.com>

Date: Tue, 25 Feb 1997 09:12:26 -0600

Subject: Re: Anybody out there?

Hey guys,

I am out here!

:)

I guess my first question for u guys -

I have a budget of about $10 a week to putinto a coil.

Where do I find the ABSOLUTE cheapest items of the

highest quality?

(I guess everyone wants to know that about everything).

In otherwords, I dont want to dish out $20 on a neon

xfr and have it burn up after a lousy 5 hours like some

people have.

I also do not have the desire to build any type of

caps from bottles as I feel it makes this project

too large, sloppy, hard to move, heavy, unprofesional

looking, and delecate.

I recieved a catalog from an electronic place that

sells high voltage mica caps.. will these work?

Thanks,

Pete

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Sat, 15 Mar 1997 21:13:40 -0800

Subject: Re: 10KV RMS @ 300MA for less than $100

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 14 Mar 1997 17:11:38 -0600

> Subject: Re: 10KV RMS @ 300MA for less than $100

>

> At 10:14 PM 3/14/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Fri, 14 Mar 1997 14:14:47 -0800

> >Subject: Re: 10KV RMS @ 300MA for less than $100

> >

> >Tesla List wrote:

>

> >

> >  Could you please tell me how you tested the transformers and exactly

> >how you wired them in series? I have just ordered two of them and I

> >don't want to take a chance on frying them the first time I apply power.

> >

> >

> >

> >

> >                               Frankensteins Helper

> >                                      Max

> >

> >

>

> Roderick,

> Those transformers may not be suitable as TC power supplies. It

> appears that they may be ferro resonant transformers and even more

> fragile than neons.

> I was tempted to get a couple myself but I'm waiting until

> someone actually reports results.

> The verdict is still out on these. Since you already ordered 2 of them

> report back to us what results you get. (even if bad)

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Will do!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 25 Feb 1997 11:01:08 -0600

Subject: Re: Anybody out there?

At 03:12 PM 2/25/97 GMT, you wrote:

>From: Peter Dalinis <pdalinis@xnet.com>

>Date: Tue, 25 Feb 1997 09:12:26 -0600

>Subject: Re: Anybody out there?

>

>Hey guys,

>

>I am out here!

>

>:)

>

>I guess my first question for u guys -

>

>I have a budget of about $10 a week to put into a coil.

>

>Where do I find the ABSOLUTE cheapest items of the

>highest quality?

Steal them. :)

Seriously though the best way is surplus. Dive into trash piles and dumpsters.

Visit salvage yards. Go to hamfests.

>In otherwords, I dont want to dish out $20 on a neon

>xfr and have it burn up after a lousy 5 hours like some

>people have.

The cheapest way is to rebuild dead neon transformers.

Most neon shops will give these away. Unpot them and repair them.

Its hard messy work but is the way to go for those of us with

more time than money.

The good news is you will end up with a better transformer

than it was when new. Once the tar is removed you have eliminated

the number one cause of neon failure, carbon tracking in the potting

compound.

>I also do not have the desire to build any type of

>caps from bottles as I feel it makes this project

>too large, sloppy, hard to move, heavy, unprofesional

>looking, and delecate.

Yes saltwater caps have all those disadvantages and more.

But you left out the one and only good point of SW caps.

They are CHEAP!

Plus they have the advantage of being variable.

You can add or subtract bottles to find just the right

amount of capacitance needed for you system.

>

>I received a catalog from an electronic place that

>sells high voltage mica caps.. will these work?

Well sort of. They are most likely DC rated caps. To use them in a pulse

AC environment you will need quite high ratings. A 20 KVDC rated cap

will probably only be good for 7500 volts AC.

If you can get these cheaply they can do but will not be the best.

If you are going to pay real money for them then don't.

Invest the money into building a polyethylene film cap.

Most of us end up building our own caps. Its laborious

work but you can build a really superb operating cap.

For me I spent more time and effort building my cap than I did

building the coil I used it on.

My suggestion is to make some SW caps first to get your

coil sparking. This keeps the investment low. Perhaps you will

lose interest and not make another coil. More likely you will

be hooked and will end up making a large number of coils.

Build a poly film cap for your second coil or use it to replace

the SW caps on your first coil.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 25 Feb 1997 11:50:31 -0600

Subject: Re: Well I am coiling

At 05:50 AM 2/25/97 GMT, you wrote:

>From: Kevin Nardelle <knardell@mailhost.accesscom.net>

>Date: Mon, 24 Feb 1997 23:50:00 -0600

>Subject: Well I am coiling

>

>Since I have some time on my hands, I have obtained THIS AEWSOME REAL

>ALUMINUM TOROID!

>This sucker came from a military auction and was part of some Sci.

>equipment, it is 16 inches in dia. and i.5 inches thick and solid!

>

>This is the find of all finds for me!! I am excited can't you tell.

>

>                                       Regards,

>                                            Kevin Nardelle

Some guys have all the luck.

If this thing is solid isn't it pretty heavy?

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 25 Feb 1997 11:50:32 -0600

Subject: Re: Cheap HV putty

At 07:00 AM 2/25/97 GMT, you wrote:

>T>Some time ago Jerry Gore told of using soft

>T>rope caulking as an insulator in rebuilding neons.

>

>T>of my spark gaps and cranked the voltage to it. About

>T>40 mils of this stuff held off 14000 volts. So its definitely

>T>a good insulator.

>

>   Well, I learned something useful today.  Thanks!  I'll have to

>remember this idea for my next coil.  I can also think of some other

>places where it may be useful as well.  Do you have any idea of what

>the RF characteristics of this stuff are?

>

>   Ralph

>

No I don't have any idea what its like RF wise.

The package has no indication of the ingredients.

But it stays soft and doesn't harden so I wouldn't

be suprised if it contained some type of silicone

oil.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Tue, 25 Feb 1997 16:17:46 -0800

Subject: magnet wire

Roderick Maxwell wrote:

I have found some magnet wire at good price from a company called

Electronic Goldmine. I ordered 8Lbs of 21 gauge for 24 dollars! Many

other sizes available.

http://www.goldmine-elec.com

Tel:1-800-445-0697

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Tue, 25 Feb 1997 19:09:18 -0600

Subject: Re: Well I am coiling

>Some guys have all the luck.

>If this thing is solid isn't it pretty heavy?

>

>Mike Hammer

>mhammer@misslink.net

>

Heavy yes but cool, YES! I am going to the shop with it and going to put it

on the mill.

I want to refine it some, will send a photo later so you can check it out.

>

>

>* Send list Replies to: Tesla-2@emachine.com

>

>* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>  in the body of your message to: Listserv@Emachine.com

>

>-----

>Processed with Listserv v2.93 for Wildcat v4

>

>

Regards,

Kevin Nardelle

/*

*********************                                               \

* T   E   S   L   A *                                               |

*********************                                  *************

Mail list information for acdcx, the place                *****| |*****

to buy-sell-trade tesla and HV electronic                      | |

parts and surplus!                                             | |

Send Mail to majordomo@icorp.net                               | |

in body of message type: subscribe acdcx                       | |

=================

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Wed, 19 Feb 1997 00:39:57 -0600

Subject: Re: Check my specs (first coil)

At 10:42 PM 2/17/97 GMT, you wrote:

>From: Al <jonese@erols.com>

>Date: Mon, 17 Feb 1997 17:42:55 -0500

>Subject: Check my specs (first coil)

>

>I am starting a new coil...this is what i am thinking of doing. Correct me

>if I am doing something wrong. :-]

>                                          ____

>                                         |____| Doornob

>                                           O

>                                           O

>                                           O

>                              Spark Gap    O

>        _______      __________|~|____   O O

>--/-/--|       ----      |               O O

>120 VAC| 12 kv|           <Caps> Primary>O O  <-Secondary

>-------|_30ma_|----  ____|____________   O O

>                                           O

>                                            Gnd

>

similar to my coil. VERY!

>

>For my caps, I will either buy caps rated at 40 kv 340 pf, or I will make

>SW caps in wine bottles. (you know, salt water and oil)

These will work fine for now.

The coil itself

>will be on pvc pipe, 18 inches high, 5 or 6 inches wide.

The winding should be 18" or better 20" with a inch of form on each end,

don't forget to seal the coil form with caps on both ends cut from lexan or

similar material.

I will wrap about

>650 turns of 22 awg wire for the secondary. I think i am gonna use enamel

>coated magnet wire. (btw, does ne1 know where to get some cheap wire?) ]

Get the wire from a motor winding shop, they will most likely give it to you.

you are going to need 1000 feet or better, I would try to get around 800 or

so turns but not more.

The

>primary will probable be flexible 3/8 inch copper tubing.

Try 1/4 inch, it is just as good and cheaper but put about 18 turns so you

can add a large toroid later.

>I will wrap about

>8 turns of that up the coil.

Not a good idea unless you want to destroy the secondary, (unless you wrap

it with 1/2" thick poly.) Make a flat primary, they work great.

>For the toroid, i will use a brass doorknob.

Doorknob is cool but a small toroid is the direction you might want to go,

take it from me( The hard head ) I will never use anything but a toroid now.

When I started coiling I used a brass doorknob and ti only kept me from

seeing the real sparks.

>The spark gap will be two bolts next to each other, and will be adjustable.

>

See the light NOW and don't go that way it is only going to slow you down,

try the MH gaps (MH explain this please:))) The bolts will heat up fast and

not run efficient at all, you will get unwanted RFI and the output will be

minimal. Build good gaps now before you get started and use them for future

coils as well.

>*NOTE* - I havnt included RFC chokes because i dont think u need them for a

>coil to function. If i do NEED them i will buy them for allied electronics.

>

YOU WILL need them due to RFI, make some. Another thing is RF feeding back

into the xfmr and AC line, this will eat your transformer in time.

>Does ne1 have an idea of how long a spark im gonna get from this setup?

A bunch of corona with a 2-3 inch spark.

>

>Lastly, I would like to say i am open for *any* suggestions or

>modifications. This is my first coil - speak your mind. :-] Later all.

>

I speak only truth, I learned the hard way myself and would like to see you

get a better start than I did not that it was not educational and fun but..

One more thing, most all of the above I stated is FACT but, my opinion on

primaries is a well made one will serve you much better and I also believe

that good spacing globally (in primary only) is a factor in getting good

spark output. That is my opinion on that. Note that I say nothing about caps

shunt across the neon, mine are still that way also.

BTW I can say now THAT my coil is finished, complete, done, exacto! Time to

start on a very new one.

Regards,

Kevin Nardelle

********************************************

* T   E   S   L   A      C   O   I   L   S *

********************************************

Kevin Nardelle   knardell@accesscom.net

My main web page IS UP all TESLA

related with lotsa pictures and links.

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

Non related sites I have

The Louisiana Swamps Online (Under construction)

http://www.icorp.net/users/kev

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Wed, 19 Feb 1997 00:53:52 -0600

Subject: Re: Check my specs (first coil)

>A safety gap is even more important.

>

>

>>

>>Does ne1 have an idea of how long a spark im gonna get from this setup?

>I have had 20 inch sparks from my 4 inch diameter coil running

>a single 12kv/30ma neon.

>

>

>Questions and/or comments welcome.

>

>Mike Hammer

>mhammer@misslink.net

>

>

Mike Hammer is the master,listen to him as he has taught me well. Ask him

about HIS gap design, they work great!!! In fact if you look at them you will

see that they are better than the RQ gaps because they cool better and can

also be used in a forced air or vacuum configuration. I made mine from 1

inch Cu tube 6 inches long with eight gaps. My sparks (once in a blue moon)

exceed the length of my coil. I do notice that weather has a tremendous

effect on coiling.

This is a picture of my gaps designed by Mike Hammer. Also a picture of my

primary, BTW look at the web site and you will see what bolts and a doorknob

will do with a 12KV 30MA neon, that is my first, the background will give

you an idea of what you will get.

--=====================_856356832==_

Content-Type: application/mac-binhex40; name="KEVTC4D.JPG"

Content-Disposition: attachment; filename="KEVTC4D.JPG"

(Note following file is encoded base64.  MH)

:#dY&9P4$0%3Z5P"(!%**6N&YC'pc!!!!!"-q!!!!!2ihrpMri!!35NC*4J!"!3%

",!%X!!$rf`"$!"!,$!i-#K!1$3i5%4!6'#JD'"B@'$%M*4dS1M-p2$Nc1$G!5&a

13%4A46Fi8'e49epLCfKR2NeaHA"NH&aPCf2rf`"$!4%5%KJ9'#mD'LpM3MK#Bf0

MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf0MBf2

r`!!4#!#@!-J$!5)!!K%"!a%"rm3!(`!!!38"!3%"!3%!!!!!!!!!!!%#!`3&"JF

)#3S,rm3!Y4!!!J%$!`)%!`8&"!3!!!&p!3)$!!34"4)K-8%'%e&K"b*a&$+"ND%

))d+a`495dI!N-f*bJJN+&KFB'4SP*LFS+5Sd06Bh1$Nk3d4&4NG)58T69&9@9eK

C@Q0NC@CRD'PUFh4eGRGiHAU$K)@'KiL*LT+6P*@@PjLCQU+MT+@QTkLTUV+cY,@

fYlLjZX,$a-A'amM*bY,6e0A@epMCfZ(Liq6PjZISkHVam[2dpIEhq2Rkrm3!(`%

!!`%"!3%"!3%"!3!!!!!!!!%#!`3&"JF)#3S,rm3!Y4%!!J%#"!3$"!F&"!3!!3*

h!!%#!a%%"5%a"K*"83GKF4-L-S%)&%+4SE("#5-c8[!9BR,4#KBN01%Pm4FB'4S

Q*bJT+M8f0cJj1N0%48C(5%P+8e499PGB@9TMC'9QCfKTDR0dGAChH(PkJS1%KBD

(L)Q+NT18PCDAQ*QDSU1NTDDRU+QUXV1dYEDhZ,Qk`X2%aFE(b-R+dY28eGEAf0R

DiZ2NjHERk1RUm[2dpIEhq2RkrpS!$!-"!!)4!a%!2`#['2P(dU3,6BamSqP5!!9

LcC%%JrHmG-8S&,)2h`p#+!"6!6(Y80f!YZ6MT9R(04A5EiYS"15"J8A(Bb[+CQi

aelQMb@8mir!jrPAA,iB"8%'C6lN85q'#2q@M[q02R,XFQ)fpDH)b&lIQ+kEr!)4

L6X@qQ44r`M8J1$[r!1qk6QLV,ZFc+Q$`!2a"Ur"V9c$!X8@!U$!VAI`dmJ)B1IV

*QS'm+cJN4YKI3Y5ZQ*SV`q*VY2[$2dUFDjBh(&cB4R28l3$qBjTIq%BQfmS#I81

+BhKQjASQ4r[LPD239KG[KqjE2Pb4%rh'r`!DG(T'R'32EAacf9aMpFeRckC,E(-

YY)!1r*(je%JYakVr!-#SelLj6FE5CFIZj)j2Se4'dQK1jicNG+T354Cq5qmSMqm

h&A8ZjN)fDPCbqhR+$8h(b-JC@*qBFd`aJeDE8)bj&c#MBkY'34qBT0PM2md-j3r

h5FLLiV&&P045RDK1+Z5@XN4*"hTeb1DU1[Q5Bl#V6*DX4aTY3$(024IQEda6L+@

-$Fh&&a$5J(3GU6DIHT!!LQrjkLJ#H-C8Cp+P!k8b)I+2T8S(Y5B%%SrHVrZRqP!

(5R6!#9H1arT4LQ!J&3AQ4E-35##-%9CadU'l6I"Y!b53!$pD%0#(a&HF$cC112[

QK2%0bA'qD8*h)BNrKc8,H(G9hYYY34RM,8b6`pUkTN@RrI*bIjd[C`,pV)Y(a(F

j'f9b#-mXDCr`NGlfPEr[Se@Jm1ka*NQefir[R&6$`eUhr2"2qqUIXi!UNQ2r!1%

L[-IIEr[SdhqhVSjqC[cT$iDeBIm!,"2`DJH'p9rjiS2aTFN!pT-3klG(Ua2e0-E

@VNJr08hr!!M1UNrkU2m!1Jq'Y8!j4+15!rD6lPBk[FRq,&35AVb!lJ2U+YR`lU1

IZV6KiGe,*q4IbTT44,FRZCCQE1346$-ap1YE(r#0DN"c'Rj8e[$HSJIkP6li0@R

%R8b$+`2bNmQQLD30N!$N'Y3k&U'5[N+51[Ar!"TTd(8#T2fC#"e`6rM4G#XafRh

pb91(b91-%eG&h()F64qA)Hi'+Uf&Xm*GCS`MUF%9DG&CF-Z4le$5Z'SjN!!4N!"

b288b-FYq&30&*#F`-52lT0,$H)c%10VGk,!6Y`#6`"c8IQTrI@Lk2qLbNIh$@&Z

EeS5%fG2'2P(dUB9(&pdFGUP&*P)LP'C"kl6r!%TZ1+I+2hJqP*3!Q1DK[A-GZA8

N%-##1e6J9"HUAYLUM*,!!$[63"r`NPdA*-Vr!%$R&54q*lT5FXcM'2[ialpk5(5

lU1fH163hNGZNRRBaq'+ScD2UA"MdfC2ahId&046,NhBd9m8A@##rIeT2q%PZ1cr

V@DQLDRJCX*6pH+Ir!'(U4(r(Jir'Pb)DQc4(LHjk%LMr!)5Hi"ia@H0$e(rR`Em

kErBHTFrk'D15)Fl0)q+,Me&4YiQZHcIV9$q`p5rjp52a0!d(8crbkrRQPb4$RNA

2q%MR)j`FHZD2q%NZJ53q+TR3G6(@f!r1Qrf*UAHh(ejSj)Kcb,cH*,TKJX+LEa$

G%IIr!&09[l#e(Vj)r@NrX28-i%32jdq5)ZH42*VFMSSfN!$iqCph"p1-FIR9GGE

ZPk([qP40SQTEp[N%mpUQA`pU*rjCB"k%Jdq@*22-X@Gbpd**(qpZjr+V"&3@&Ze

[!b5$$lMZUbIT8qJYAZ4N98Zi&D&jPqqK'FGa9de&+3YT16d`4qP0-4AXRmqhH0[

6(k8RpQVk8h5JF50fiV3h(r*TYf&U64Mj"RdU8IV8FAh!2DT48XT%8SrH!mp+5R5

rI8HarT5!G"a3(8-$(HQ6c2E+Nm4!H0``b-LT!+VhqIXKqSTSDGRF[rm!#@5Gp[j

90Cq*(ZCa'md8+rhh`"A+H6)$Mb*MMqkP-P,aB2NbU$rHA&(*jPZHQb1TIa6+MXR

")2hJ!3DB2&8hVrik+jU&CC&h#hPIh8Ir!&UP%-r(qJcr!1I`TF[Q05[d4[RaA2J

i2k$r!!TRr#9A(pi(rJ)r`V&0[FBapJRckjrqY6"BhC(r!"j6Ijr#Rb*p3FRfAh'

iI&0afBIpmLNri5Qjcb`rlj&Bidkp2!X*[cT6TGmHPM0RhSj%,RIEm$8Ia6G0dE(

d!TRr!!NecM'mrT@B0,[cdXC2cS1PArm!cj5#Pb4&cb0%q*EV(qX04rm!#4ABk50

caeUPrC'S(J@EjqY$D2U+MQdBIM6j)Kcb0'EA5&3a56l`"[hiaR[M"2&92q%L[pT

AH53f36k94q`AlbE2XdRiLTIl'e$r!*pc6j)S5R-dV@jDkMD4ZTBjU8e$BfXeVEK

*SpM%Ni*UFe1a2U4YeUMU%qf%a,eCXR&@ELi@)C2*p+V@X$A%`ZC#0S2!p69)6*l

'%`f`h$jQj0@H+8r@NbI@N!!64IG(Z+N&44if,p+Q&*M4&+IhLMf2p+1Y%h%LI3r

dSS!8BU[IF@K*(F92p+VhjcD%ClLQJf0b(a*D)LjM8I+"JHY12LI6h'*88,h,!X2

b!0F1#ZrDlE"h1-dMb)[hA,I8BTmVlPZ5hXG`RLDb4!&J#$(3GU$iUYqdDiVK4)T

j,(2TLP%LRZIbSjAh$QMf1i2LQ$2%DiTVH+S4Rpf2bVL[-AeEm"6JbRrRTrhc5j(

h$QABl(rK+Sar![j8Km9U$Mber+Z2E'l#L4KkK64Kb1)C6r`'PbHE$RAC(@Ra@-(

j&r+SciV*cmJ&FYXN2r,#AmU2+Pcr!+LEmU16c$RmNG,r!-*8r)`2bU+6a6-911$

fiV!%$((lQEmU6b*#Ip4,q92N3HdCUViNZYah0hiqRDPEa4GqB!VC8MNN9NICj#I

p6,arXdm@XRDfNTmN4+T)eMI5DL"-ciCIP"26r209jlXUQcV)H2PUYEac%Q"&+Fj

EGfUkPT%LP5#c(UakrK4Da,ECALY#jhcR*l,kI@VX)#ai!ibHP4)a9M()HHap48m

Ih2a2mk'a!HY*akLP0*NIj05"0%IP(dU89$&pdI5T430%FhqY6rG2p+"fT*[pG(c

e9[k8$mD"$Ze9Vmrk2Rh&@!HHY9Y3)qbrm#&0EM0Qe[G)LLMh@iCpRc%U$cqG55k

MSM,mpQMRdmYDi[E[Er@+$rY0LP+qAPK0%FGJqFdZ4hh,FPf1aA8p%93)V52Efq8

!IbTieM5e(&T'2`(q&F5K%VB-b)"cPUGmQ2m!AapIIr#Rb[Z*0GMXMV@QGV@-GqJ

r`Thp[kH"r`!HkiqJVLaXkqHP--LMT*Rm+A)qj9ef1eri5+bcaEVMm+3q)l3$jB9

(i9aAQ$qmIbShMeEmU2C[Z,RABl,r!)5@h"*mTIbT`m83K[m!9,MYLZ-cl[m!pmd

!RVmrrI0(Xr-1GGMYIq%VYmrF)r$r!1[50iVK"i3iqPF@FJIGNrljT!1ff6r[NdH

cm`jef1Zri5i%VqilRFHhYLScicAGYmR"cMTA+C*1d)hAd1D2+FRjBj!!RrGTqc4

+Rj(4bhhfq3h'-EZhk9'69E6P+fLjb$NRRkeB2eT@X*Zj()JGGVIRk'QS,P9qAbj

&"k-5TTj2ec6Sr`$9pHjrR6k#)fPGI[35CpKNIT6IY$rmqmhr!(a8a)pD6mIeTD!

64Rj4p+P"iU#-MD2T8SS!MR2lk2m!h6r5P"0-QEpp(ek(qP+$a6'2c9Dr)qcmqSU

I26LUemGd'"ecQQJ0E3B0&Ndb0VaBfR*1lIm!AMp-9TQcm2E-E,E"pkiY,Cj')MN

L!(pk98rQ46M")LEQQK+p5"-Tr3'Nh+qM,j8pE-lD#hd'hA%FGVJpb!hkQR[,Sa!

3Lh'HQ&!VKC,XcP&)M3p!9!AMVckdME!Q6-ZIpqMh`5LpVRG0,S``6j!!FF$j49H

5id9@aYM11i!VLQPAD3C4M1FEUJDIRL6LPD6kKlU1jDpdBR*M6r[QQ0Ik1Sq@*6q

&F2j`r[d'CFIkbMNPh(c4l(D0UZP+$LfM2i#S@eE5mj&T(A)HD[Gk6c&cpqMNIF(

0GMVMVHRJF@XAj8hqhl-G,5(r!,jVNLkpQc6GrS6q92NmaHdmMV6iJY&'IXF1Hf%

U16a0ET)#YY%112PVP6,MMRRfSqpJl@*lF8rCVU,fMl(3VGIDGdS99h%m!BT5DTk

F-@`lR*0@5D9L@lJ6eTdC)3(kd`RLPL2lX8#&Bqp*NqJSGJ!56J$[8AfQ(rRS[jd

J,8Cq8I5T!DTTFI)-*fpDHYbHdI(eTM5**Mqq6RXIk8!jU'5I-Lj'2P2p+"1ZH!D

B@*LDJZFE9h%JC'525R'F!F!jTMN5$"cq(&)D1TDbd'6$b#&cM'5HY4R6[$T3TjF

@@'!36NI5Z14*Qb"RejT3mpX4+NK4ad)1#+,6lP2Pl-kL$30!Yja+dlblFrZj'"(

iJ#Vr!2CQK-#"E3NH`VK[Y%XlXc6EQ2hLcmNrM8LjarVP*p20!rV5ERh"+2QGK,T

@J#)Mb8(I)j)USG1m2M14qH+j@9R!jN8pZ*!!(qY4HBl0MIcpD,6I82FA3kdfhKj

HLCr#QqAS!12+cpDj3Pr@M,HY(,,Z(0(XGB'd*1PZ'r'Q'kdF%PE8ILDj0R+p@(j

dRQFrH&2NIF1H2Bk`hZN!%IC&r&M86hqRBb,@-9c(Q(e&)C12[#MNIFA1Zad$AQR

1f@YBJ3FB'H+NHrX5SffdBr1ZC-Q6b46@IF1@&8iHBP2b0X5)dM'-"36N!G+F*!H

$`DTf43@q'jj25TYf1#53!$SHp)PNT2&"N@+$Fj`"eU)5MSHDKH*CT&D9fC&k*Yi

rR3&Q0c,IR**5!G-G@ThpR`qXRrI96qF!"K12DQqGrX'RIX+`XB!8(e&2"2i8a-K

4cfU3!(eU64$*-Ed2XIk8GZp,+ahS28'Njld`%)chr1PMqD491"Nic5%qTTNKfS@

cdT!!'h(iCXRGa*H)#S'F0krM3rKE6L0Se&!IpiIiecMAFT2,(0$AG`b+V5[X8j9

Fm!r5QP)'i(4*iAdY"mqSUa2qd2m!'R,iGdER0m$q2rekjX66-FEMc3mdLI,[2jd

@Ph'Z3kAr!)4l4$cpYr@JD&S3kh4*qYF[pSITNdRR[rH0&Tp`[$XG4rBZJMrPj2j

dRpMk%2m!PZaVQ21IqmD"+jrLTFXZiAKf1R1Pk!SbC'0!dr`mFjGZ[TA,Q4XF[5H

B`rLTfPh#mHadcfAKm(j5rAd&-0VS+r`b(mUj[c'r[8RQ((hU1@AFA0(XG!BG"$(

l`1-r0hpZ"ejU-*Sjj#0MkeK&ccc6#5H!e2PIFRQABfVXfUbUPQV+Qh*ccc8"*p6

9@f(FRN9Cb$hT@X1pa-jlN8j3-CcRmDC6PaX(V5!#31Y*ZpU#"5B&!&!DhJImHrm

!irm!r@TIlFrkGrm!arm!qY@44@[+M(RCUYV@jJIXr6rEr`$V8[m!EHIqAIm!mIm

!rV9N8Y(+JjfDTeV2r,[r!12r!2eUDGAceJrmIrm!V9Q88@3Fc,VkJA2qU!r'QIE

56pdrpp99STf&GPa,i+`h4PPp0f2k8MhSCL9M)(TZcr5UP&!AC2m!DRp"5VH5!jf

Tq99k"3&fA4UFS14(&Re`ImD$UNarjCaINIm!'U9*4C!!qCPPVf9ZS@QrDRp&r+S

++"ACEM[jBfh")bF%I-Z4b#1KqY-0h)HbrP9HLJ9bIl5qHLdj,f4$N!#SIU+V88$

ZA4U8S2#4Mm$r!)d[pTcIh)rb2q08U5Lb(c-[IfR0rG6mMrM306Q(m%Ij(r'U0,5

XJjQA[l8QrZ4rNIm!'NrY5Eqj(q4raUP44C!!Fc2rf9*8!!!!:

--=====================_856356832==_

Content-Type: text/plain; charset="us-ascii"

Regards,

Kevin Nardelle

********************************************

* T   E   S   L   A      C   O   I   L   S *

********************************************

Kevin Nardelle   knardell@accesscom.net

My main web page IS UP all TESLA

related with lotsa pictures and links.

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

Non related sites I have

The Louisiana Swamps Online (Under construction)

http://www.icorp.net/users/kev

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 26 Feb 1997 00:25:21 -0600

Subject: Re: Looking for Neon Transformers

At 10:17 PM 2/25/97 GMT, you wrote:

>From: Chris Crochet <ccrochet@premier.net>

>Date: Tue, 25 Feb 1997 16:17:14 -0600

>Subject: Looking for Neon Transformers

>

>

>Hello,

>

>I have been looking for over two months now for a used or new (but cheap)

>neon transformer suitable for my first Tesla Coil project, of at least

>12kv/60ma or 15kv/30ma.

>

>I have already checked many, many resources, with no luck.  None of the

>local neon shops have used transformers, they tell me "we send them off for

>credit".  I've tried junkyards, wreckers, antique stores, pawn shops, bars,

>and restaurants.  Nothing.

>

>There are three people who have offered to sell me neon transformers.  The

>first now tells me he has no more.  The second no longer responds to

>e-mails.  The third person, which was my best hope, has now selfishly

>decided that instead of selling them, will only accept trades.  Too bad for

>all the newbies trying to get their foot in the door, with nothing to trade...

>

>Can anyone help?

>

>Thanks in advance,

>Chris C.

>

I can empathize with your problem. Lack of suitable transformers

held me back for quite a while. I had to travel 40 miles to a larger

city to find some sign shops.

I find it hard to believe that none of the sign shops would sell you

a used transformer. Try a different approach. Call one of them on the

phone and try to buy a transformer for a sign. Tell them you have

a window sign and you need a 9kv/30ma transformer to replace one that

died. They will of course want to sell you a new one for big bucks

but try to negotiate for a used one. If that doesn't work then you

will have to look elsewhere.

One other source for usable transformers is heating contractors.

Oil burning furnaces use a hi voltage transformer for ignition.

A common rating for these is 10kv at 23 ma. This will make a good

transformer for a small coil. I had two of these transformers given

to me for the asking. Of course I had to do the dirty work and remove

them from a couple of old furnaces to be scrapped. These two in parallel

yielded 10kv at 46 ma and worked rather well. They are internally current

limited like neon transformers.

Good luck with the search.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Tim Maleske <maleske@worldchat.com>

Date: Wed, 26 Feb 1997 01:31:24 -0500

Subject: For Anybody (was:  Anybody out there?)

> I know some of you are working on coils and

>  some others are planning coils.

>  Why don't some of you building coils post

>  some of your specs?

>  It will help others who are thinking about building

>  coils by seeing what the rest of you are up to and

>  we may find some problems with your design that can

>  be averted. No sense building a problem into your

>  coil that can be prevented. That's what this list is for.

Ok Mike. Here are some of the things We are working on.

Just finished a new capacitor set-up. I used ceramic doorknob type (I

can hear the moans and groans from here) because they were the only high

voltage kind we could find. I built a holding container out of a

tupperware container. Two capacitors were screwed together. This gave us

a 30 KVDC=A0rating at 5nF. I soldered lugs and brass bolts onto copper

tubing and connected the lugs to the caps and stuck the bolts up through

the lid of the container. There are two sets of these series caps in the

container (four bolt connections on the top) so we can arrange them in a

number of series parallel combinations. The container was fitted with a

ball cock valve and all the holes were siliconed shut (including the

lid). I then filled the container with heated Insul-Oil and sealed it

up. I was left with a vacuum sealed container and small bubble of air

the size of a pencil eraser, but the lid buckles in the center and it

keeps well away from any terminations. And that is it. Hopefully it will

work better than the old Leyden jar trick, but they were just re-built

too :)

Another thing we are working on is a new primary. Something really nice

looking and flashy. We have ribs made up for a saucer type primary

rising at 30 degrees. Along the hypotenuse there are 1\4 inch holes

drilled at 5\8 centers. This will let us use 1\4 inch conductor and have

3\8 spacing between the turns. We also started each consecutive rib 3\16

lower than the previous one so that there is no "dog-leg" and it is a

true spiral. This will give us the ability to tune out up to 14 turns.

The holes will have the tops ground out enough to snap the tubing into

place. The tubing will be tight wound around a wooden form and then

snapped into place. The ribs are translucent black plexi and the base is

opaque red.

Has anyone ever used starters to isolate the capacitors from the circuit

when it is de-energized? I was thinking of using a size 2-3 starter for

this. We all know that we should be disconnecting (or discharging) our

capacitors before we work on the primary cct. We are also building a

power pack for tesla use. Auto transformer, meters, lights, you

know. Would this work? I think they would hold the voltage and I think

there is enough of a gap when open to isolate them effectively. What

about hook-up wire for primary cct's? I have heard people say use 10 ga.

solid (awful to work with). Has anyone ever tried the carbon type spark

plug wire? What about something flexible that works reasonably well.

What about something that isn't a pain to work with? Any ideas anyone?

Well that should keep all the Anybody's out there busy for a little

while:) Look forward to hearing your responses.

--=3D=3D Tim =3D=3D--

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Tue, 25 Feb 1997 21:00:12 -0800

Subject: fried chokes

Well folks I'm going to be out of business for some time both of my RF

chokes shorted out. Even though I had them submerged in mineral oil they

still shorted! They were both ferrite bobbin core chokes wound with #24

magnet wire. Since they have me shut down for awhile I've decided to

make some modifications! I'm moving the neons offboard to accomidate

another cap which I'm in the process of building, and putting them and

the resistors in a wooden box. Instead of the transfomer outputs both

being fed into two resistors I'll have a 3K resistor on each leg of each

transformer. I read in one of the other posts that this helps to balance

the load on the transformers. Why?

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Wed, 26 Feb 1997 07:00:58 -0500

Subject: Re: fried chokes

Tesla-2 wrote:

> Instead of the transfomer outputs both

> being fed into two resistors I'll have a 3K resistor on each leg of each

> transformer. I read in one of the other posts that this helps to balance

> the load on the transformers. Why?

When you tie two transformers directly together, they both have to agree

on the voltage.  This means one transformer (the slightly higher voltage

one) will waste some electricity raising the voltage in the lower

voltage one.  And even if the transformers are "identical", the voltage

will be a little different from each owing to manufacturing tolerences.

If the transformers have a fairly low internal resistance, then a

significant amount of power can be wasted.

When you put a resistor in series with each transformer, then the

transformers can each run at their prefered voltage.  Some energy will

be lost in the resistors, but at least the transformers won't be

fighting each other as much.

To take an extreme case, suppose you had a 12 volt and a 6 volt

transformer, and hooked them up without the resistors.  The transformers

would probably both burn up, trying to agree on that average 9 volts.

But with resistors in series, they would not burn up.

Steve Falco

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 26 Feb 1997 06:44:18 -0800

Subject: Re: fried chokes

Tesla-2 wrote:

>

> From: Roderick Maxwell <major@vicksburg.com>

> Date: Tue, 25 Feb 1997 21:00:12 -0800

> Subject: fried chokes

>

> Well folks I'm going to be out of business for some time both of my RF

> chokes shorted out. Even though I had them submerged in mineral oil they

> still shorted! They were both ferrite bobbin core chokes wound with #24

> magnet wire. Since they have me shut down for awhile I've decided to

> make some modifications! I'm moving the neons offboard to accomidate

> another cap which I'm in the process of building, and putting them and

> the resistors in a wooden box. Instead of the transfomer outputs both

> being fed into two resistors I'll have a 3K resistor on each leg of each

> transformer. I read in one of the other posts that this helps to balance

> the load on the transformers. Why?

>

>                                  Frankensteins Helper

>                                          Max

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Roderick,

Some ferrites are somewhat conductive. If the chokes were directly wound

onto the ferrite, this may have been the problem. Remember that the

chokes, if working properly, may be standing off voltage peaks that are

twice the voltage seen on each leg of the neon. Air-core chokes, wound

on some 2" or 3" PVC will be able to take the voltage. Use #24 - #28

gauge, and make the winding length at least 6"close-wound. You won't

need to submerge these in oil. Putting resistors in each leg will work

OK, but is not essential. I assume you're using neons with the same

voltage and current rating?

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 19 Feb 1997 12:15:25 -0600

Subject: Re: Check my specs (first coil)

At 06:53 AM 2/19/97 GMT, you wrote:

>

>Mike Hammer is the master,listen to him as he has taught me well. ASk him

>about HIS gap design, they work great!!! Infact if you look at them you will

>see that they are better than the RQ gaps because they cool better and can

>also be used in a forced air or vacuum configuration. I made mine from 1

>inch Cu tube 6 inches long with eight gaps. My sparks (once in a blue moon)

>exceed the legnth of my coil. I do notice that weather has a tremendous

>effect on coiling.

>This is a picture of my gaps designed by Mike Hammer. Also a picture of my

>primary, BTW look at the web site and you will see what bolts and a doorknob

>will do with a 12KV 30MA neon, that is my first, the background will give

>you an idea of what you will get.

>

>

>                                       Regards,

>                                            Kevin Nardelle

>

The gap unit Kevin is referring to was something I recommended

to him for his coil. Its not my original design.

The Richard Quick type gap has copper tubing electrodes

mounted inside a piece of PVC pipe. This encloses everything and makes

it easy to cool by blowing air through the PVC pipe.

The gap Kevin built is the same thing layed out flat.

Use 1 inch diameter hard copper tubing cut into 4 to 6 inch

lengths. The longer each electrode the more heat it can sink.

I used 4 inch long electrodes in my gap, Kevin used 6.

Cut 9 of these electrodes and mount them on a rectangular

piece of 1/4 inch thick plastic. The dimensions depend on the

size of your electrodes. I used Plexiglass and I believe Kevin

used Lexan. Mount each electrode with two small bolts. One in

each end. Drill the holes in the plastic base a bit oversize

to allow some movement. This is to allow for setting the gap

distance between each electrode.

Place a nut on the mounting bolts under each electrode to space

the electrode up off the plastic base. This will improve airflow

and keep the firing gap from carbonizing the plastic.

A fan should be used to blow air across the electrodes for cooling.

You can completely enclose the gap as Kevin has done and blow air

through the box with a fan or blower.

Space each electrode .030 inches apart. Use a feeler gauge

or spark plug guage. The 9 electrodes will have 8 gaps between

them. Each individual gap is .03 inch so the total gap distance

is .240 inch.

This will give good performance on up to 12kv neons.

If you are using 15kv neons better results might be had by

increasing the number of electrodes to 11 to give 10 gaps.

Space these at .025 inch for a total of .25 inch.

Not that we didn't increase the total gap distance. We have

a greater number of smaller gaps.

Connections are made to the back side of the plastic base.

You can select the number of gaps you want in series by making

the proper connections.

Start off with only 3 or 4 gaps. Then tune your coil for maximum

output. Then move one set of connections to the next electrode.

This places one more electrode in series.As you add more gaps

your output will increase. Iif at any time the coil starts

running sporadically instead of smoothly you have too much

gap.

Once in a while clean up the electrodes by running some fine sandpaper

between the electrodes.

Questions and comments welcome.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jeremy Bair <pwac@flinet.com>

Date: Fri, 07 Feb 1997 15:55:55 +0500

Subject: Re: mini coil project

Organization: PWAC Inc.

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Tue, 18 Feb 1997 13:09:24 -0600

> Subject: Re: mini coil project

>

> Once you start working with neons you wont want to mess with

> ignition coils again.

>

> Mike Hammer

> mhammer@misslink.net

Yeah, especially when you play with one for a good 5 hours, running with

ever test imaginable (Yes, even cookiung hot dogs ;).

Roderick, you will have lots of fun with neons, they seem to be very

sturdy, and you can purchase them REAL cheap, and before you make your

coil, you can play around with them for a while...

Jeremy Bair

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: xtal@airmail.net (David Christal)

Date: Wed, 19 Feb 1997 22:01:24 GMT

Subject: picofarads - is one enough? are a few too many?

Recipe:

3' x 6" clear acrylic tube

22 ga. magnet wire

10' x 6" (thin wall drainage type) pvc with 6 flat end caps for

capacitors=20

.093" ldpe sheets cut into 16" widths

5 gal Mobil Velocite #10 oil

1 roll 14" 10 mil aluminum roof flashing

9kv 30ma neon

1' x 6" sched 40 for spark gap

6" flex drain pipe and alum. tape for toroid

so, my immediate question at hand is:

according to teslac.exe, I should use .005 pf cap.

this appears to me to be a 40" x 14" rolled cap.  What do I do with

the rest of this capacitor stuff?

David Christal 3341 Cloverdale Lane, Dallas, TX  75234

214 349-6972 (214 FIX MY PC) (Pool Cues and Personal Computers)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Al <jonese@erols.com>

Date: Wed, 19 Feb 1997 17:22:49 -0500

Subject: Re: Check my specs (first coil)

>The gap unit Kevin is referring to was something I recommended

>to him for his coil. Its not my original design.

>

>The Richard Quick type gap has copper tubing electrodes

>mounted inside a piece of PVC pipe. This encloses everything and makes

>it easy to cool by blowing air through the PVC pipe.

>

>The gap Kevin built is the same thing layed out flat.

>

>Use 1 inch diameter hard copper tubing cut into 4 to 6 inch

>lengths. The longer each electrode the more heat it can sink.

>I used 4 inch long electrodes in my gap, Kevin used 6.

>

>Cut 9 of these electrodes and mount them on a rectangular

>piece of 1/4 inch thick plastic. The dimensions depend on the

>size of your electrodes. I used Plexiglass and I believe Kevin

>used Lexan. Mount each electrode with two small bolts. One in

>each end. Drill the holes in the plastic base a bit oversize

>to allow some movement. This is to allow for setting the gap

>distance between each electrode.

>Place a nut on the mounting bolts under each electrode to space

>the electrode up off the plastic base. This will improve airflow

>and keep the firing gap from carbonizing the plastic.

>A fan should be used to blow air across the electrodes for cooling.

>You can completely enclose the gap as Kevin has done and blow air

>through the box with a fan or blower.

>Space each electrode .030 inches apart. Use a feeler gauge

>or spark plug guage. The 9 electrodes will have 8 gaps between

>them. Each individual gap is .03 inch so the total gap distance

>is .240 inch.

>This will give good performance on up to 12kv neons.

>

>If you are using 15kv neons better results might be had by

>increasing the number of electrodes to 11 to give 10 gaps.

>Space these at .025 inch for a total of .25 inch.

>Not that we didn't increase the total gap distance. We have

>a greater number of smaller gaps.

>

>Connections are made to the back side of the plastic base.

>You can select the number of gaps you want in series by making

>the proper connections.

>Start off with only 3 or 4 gaps. Then tune your coil for maximum

>output. Then move one set of connections to the next electrode.

>This places one more electrode in series.As you add more gaps

>your output will increase. Iif at any time the coil starts

>running sporadically instead of smoothly you have too much

>gap.

>

>Once in a while clean up the electrodes by running some fine sandpaper

>between the electrodes.

>

>Questions and comments welcome.

>

>Mike Hammer

>mhammer@misslink.net

Thanks for the advice. I'll make that kind of spark gap. I just bought a

neon sign transformer today. I am going to go out to home depot on

saturaday for the rest of my supplies. I am well on my way to my first coil!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Wed, 19 Feb 1997 19:02:40 -0600

Subject: Re: Check my specs (first coil)

>>Mike Hammer

>>mhammer@misslink.net

>

>Thanks for the advice. I'll make that kind of spark gap. I just bought a

>neon sign transformer today. I am going to go out to home depot on

>saturaday for the rest of my supplies. I am well on my way to my first coil!

>

>

The best of luck on this coil, have safe fun!

Regards,

Kevin Nardelle

********************************************

* T   E   S   L   A      C   O   I   L   S *

********************************************

Kevin Nardelle   knardell@accesscom.net

My main web page IS UP all TESLA

related with lotsa pictures and links.

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

Non related sites I have

The Louisiana Swamps Online (Under construction)

http://www.icorp.net/users/kev

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Wed, 19 Feb 1997 19:06:39 -0600

Subject: MY NEW LIST!!!!!!!!!

Well I have started a maillist, automatic but private/closed so you need to

email me for access. It is ONLY for tesla parts and HV power

supplies/electronics for sale,trade,buy. I hope everyone will enjoy it.

I will send out the address in two days or so when I get it configured.

Regards,

Kevin Nardelle

********************************************

* T   E   S   L   A      C   O   I   L   S *

********************************************

Kevin Nardelle   knardell@accesscom.net

My main web page IS UP all TESLA

related with lotsa pictures and links.

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

Non related sites I have

The Louisiana Swamps Online (Under construction)

http://www.icorp.net/users/kev

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 19 Feb 1997 19:45:17 -0800

Subject: Re: mini coil project

Tesla-2 wrote:

>

> From: Jeremy Bair <pwac@flinet.com>

> Date: Fri, 07 Feb 1997 15:55:55 +0500

> Subject: Re: mini coil project

> Organization: PWAC Inc.

>

> Tesla-2 wrote:

> >

> > From: Mike Hammer <mhammer@misslink.net>

> > Date: Tue, 18 Feb 1997 13:09:24 -0600

> > Subject: Re: mini coil project

> >

> > Once you start working with neons you wont want to mess with

> > ignition coils again.

> >

> > Mike Hammer

> > mhammer@misslink.net

>

> Yeah, especially when you play with one for a good 5 hours, running with

> ever test imaginable (Yes, even cookiung hot dogs ;).

>

> Roderick, you will have lots of fun with neons, they seem to be very

> sturdy, and you can purchase them REAL cheap, and before you make your

> coil, you can play around with them for a while...

>

> Jeremy Bair

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

I'm already using neons in my first coil! I just have a couple of

ignition coils that I want to find a use for.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Wed, 19 Feb 1997 22:17:29 -0500

Subject: Re: picofarads - is one enough? are a few too many?

> according to teslac.exe, I should use .005 pf cap.

> this appears to me to be a 40" x 14" rolled cap.  What do I do with

> the rest of this capacitor stuff?

I think you mean uF rather than pF.  .005 pF would be a grain of sand.

I built a cap based on teslac's capacitor calculator - almost exactly

like what you are proposing - and it measured out at .005 uF when I got

done.

Don't worry about the left over materials.  You will probably build

another cap some day (when you decide to scale up your coil)!

Steve Falco

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: " Erik  Hennix" <mr_white@hotmail.com>

Date: Wed, 26 Feb 1997 05:43:36 -0800 (PST)

Subject: Doubling output voltage w. diodes

Hi everyone!

I've got a rather low voltage (3.75 kV) neon rated at 200 W.

Now that's a bit low a voltage for a TC, so I thought maybe I can

connect two diode bridges (full wave rectifiers) made from HV diodes

to the neon's AC outlet and then connect the rectified outlets from

the diode bridges in series, like you connect batteries.

(I tried to make some ascii illustration, but it didn't survive cut-paste.)

HV diodes are easily obtained by wiring cheap 1kV 1A diodes in series.

Would this work?

If so it would be a rather neat way of doubling the

primary voltage.

Cheers,

Erik.

---------------------------------------------------------

Get Your *Web-Based* Free Email at http://www.hotmail.com

---------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Chris Crochet <ccrochet@premier.net>

Date: Tue, 25 Feb 1997 16:17:14 -0600

Subject: Looking for Neon Transformers

Hello,

I have been looking for over two months now for a used or new (but cheap)

neon transformer suitable for my first Tesla Coil project, of at least

12kv/60ma or 15kv/30ma.

I have already checked many, many resources, with no luck.  None of the

local neon shops have used transformers, they tell me "we send them off for

credit".  I've tried junkyards, wreckers, antique stores, pawn shops, bars,

and restaurants.  Nothing.

There are three people who have offered to sell me neon transformers.  The

first now tells me he has no more.  The second no longer responds to

e-mails.  The third person, which was my best hope, has now selfishly

decided that instead of selling them, will only accept trades.  Too bad for

all the newbies trying to get their foot in the door, with nothing to trade...

Can anyone help?

Thanks in advance,

Chris C.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 26 Feb 1997 11:29:48 -0600

Subject: Re: For Anybody (was:  Anybody out there?)

At 06:31 AM 2/26/97 GMT, you wrote:

>From: Tim Maleske <maleske@worldchat.com>

>Date: Wed, 26 Feb 1997 01:31:24 -0500

>Subject: For Anybody (was:  Anybody out there?)

>

>> I know some of you are working on coils and

>>  some others are planning coils.

>>  Why don't some of you building coils post

>>  some of your specs?

>>  It will help others who are thinking about building

>>  coils by seeing what the rest of you are up to and

>>  we may find some problems with your design that can

>>  be averted. No sense building a problem into your

>>  coil that can be prevented. That's what this list is for.

>

>Ok Mike. Here are some of the things We are working on.

>

>Just finished a new capacitor set-up. I used ceramic doorknob type (I

>can hear the moans and groans from here) because they were the only high

>voltage kind we could find. I built a holding container out of a

>tupperware container. Two capacitors were screwed together. This gave us

>a 30 KVDC=A0rating at 5nF. I soldered lugs and brass bolts onto copper

>tubing and connected the lugs to the caps and stuck the bolts up through

>the lid of the container. There are two sets of these series caps in the

>container (four bolt connections on the top) so we can arrange them in a

>number of series parallel combinations. The container was fitted with a

>ball cock valve and all the holes were siliconed shut (including the

>lid). I then filled the container with heated Insul-Oil and sealed it

>up. I was left with a vacuum sealed container and small bubble of air

>the size of a pencil eraser, but the lid buckles in the center and it

>keeps well away from any terminations. And that is it. Hopefully it will

>work better than the old Leyden jar trick, but they were just re-built

>too :)

Ceramic caps will work but just remember that they are lossy and

will get hot. And this heat will eventually cause them to

fail. Keep your run times short. Putting them in oil

is probably unnnecessary. This won't keep the dielectric cool

only the outside. With the 30 KVDC rating these will be just

barely able to handle a 12kv neon. I would stick to 9 KV.

Thats not a serious handicap. I have run a 9 KV/60ma neon on my

coil and had 24 inch sparks.

>

>Another thing we are working on is a new primary. Something really nice

>looking and flashy. We have ribs made up for a saucer type primary

>rising at 30 degrees. Along the hypotenuse there are 1\4 inch holes

>drilled at 5\8 centers. This will let us use 1\4 inch conductor and have

>3\8 spacing between the turns. We also started each consecutive rib 3\16

>lower than the previous one so that there is no "dog-leg" and it is a

>true spiral. This will give us the ability to tune out up to 14 turns.

>The holes will have the tops ground out enough to snap the tubing into

>place. The tubing will be tight wound around a wooden form and then

>snapped into place. The ribs are translucent black plexi and the base is

>opaque red.

Sounds like a winner. When not being used for your coil you

can hang it on the wall and call it art! :)

>

>Has anyone ever used starters to isolate the capacitors from the circuit

>when it is de-energized? I was thinking of using a size 2-3 starter for

>this.

Starters? Not sure what you mean.

>We all know that we should be disconnecting (or discharging) our

>capacitors before we work on the primary cct.

In the normal coil circuits you will see there is a discharge path

for the main cap through the transformer secondary winding.

So the main cap won't hold a charge when the power is removed.

BUT if your transformer secondary winding should just happen

to open up on you a lethal charge could remain.

Best practice is to get in the habit of discharging your main cap.

You only get one mistake.

>We are also building a power pack for tesla use. Auto transformer,

>meters, lights, you know.

>Would this work?

Sure. I built a box to house my variac and AC line filters.

It contains my main power switch (keylock) and indicator lights

so I know whewn things are energized. I also have AC voltage

and current panel meters and a DC power supply to run my gap

fan motor.

>I think they would hold the voltage and I think

>there is enough of a gap when open to isolate them effectively.

Not sure what you mean here. Sorry if I'm missing the boat.

>What about hook-up wire for primary cct's? I have heard people say use 10 ga.

>solid (awful to work with). Has anyone ever tried the carbon type spark

>plug wire? What about something flexible that works reasonably well.

>What about something that isn't a pain to work with? Any ideas anyone?

I used stranded AWG 14 wire on my first 2 coils. I later replaced

this with AWG 8 stranded ground wire when I started running more

power. You really need the heavy conductors in your RF ground lead

and in the primary tank circuit.

The 60 hz high voltage leads are low current and large conductors

are not needed.

Please note that most of the PVC jacketed wire you buy has insulation

rated at only 600 volts. If you use wire of this type allow plenty

of room around it so it won't arc to something.

HV neon wire can be used in low current areas and is rated for 15kv.

For higher current areas you can use RG-213 coaxial cable. This is

quite useful if you have problems with your coil output striking

your primary circuit wiring. The outer brad of the coax can be grounded

while the inner conductor carries the juice.

Spark plug wire is not suitable as most have high resistance to

curb ignition noise.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 25 Feb 1997 20:41:25 -0600

Subject: Re: magnet wire

At 12:17 AM 2/26/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Tue, 25 Feb 1997 16:17:46 -0800

>Subject: magnet wire

>

>Roderick Maxwell wrote:

>

> I have found some magnet wire at good price from a company called

> Electronic Goldmine. I ordered 8Lbs of 21 gauge for 24 dollars! Many

> other sizes available.

>

>                  http://www.goldmine-elec.com

>

>                      Tel:1-800-445-0697

>

>                          Frankensteins Helper

>                                Max

>

>

Hmmm. $3 per lb for magnet wire is pretty good.

I've been paying $4.15 per lb at a local motor rebuild shop.

I guess it would depend on the shipping charges.

Let us know how this wire turns out.

I think I had a catalog from these guys but might

have tossed it out the last time I shoveled off my desk.

(a clean desk is a sign of a sick mind)

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: xtal@airmail.net (David Christal)

Date: Fri, 21 Feb 1997 14:51:24 GMT

Subject: Re: Two big Neons

On Fri, 21 Feb 1997 05:27:55 GMT, you wrote:

>From: Kevin Nardelle <knardell@mailhost.accesscom.net>

>Date: Thu, 20 Feb 1997 23:27:55 -0600

>Subject: Two big Neons

>

>What would happen if I was to take two 5KV 120MA neons and parallel =

them??

>I know what you are going to say, 5KV 240MA right?

>

>Would my spark be better with 240MA at 5KV than 12KV at 120MA?

>

>                                       Regards,

>                                            Kevin Nardelle

...

shorter fatter louder?

David Christal 3341 Cloverdale Lane, Dallas, TX  75234

214 349-6972 (214 FIX MY PC) (Pool Cues and Personal Computers)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Tue, 25 Feb 1997 23:01:48 -0600

Subject: Re: Looking for Neon Transformers

OK chris, you want a NEON send me some money.

I hesitate to sell via mail in feer of being ripped so you know where I am

if you want one, I have a 15KV 30MA (not so heavy) and a 15KV 60MA (quite

heavy).

Send me US CURRENCY in the amount of $55 to:

Plexus Systems

C.O. Kevin Nardelle

4904 Belle Dr. Metairie, Louisiana.

USA  70006

That is for the 30MA. For the 15KV 60MA since the postage is the bulk of the

cost send me  $70

I am telling you that will cover the postage and cost of the neon, and it

will NOT BE DOA.

120VAC input on both.

At 10:17 PM 2/25/97 GMT, you wrote:

>From: Chris Crochet <ccrochet@premier.net>

>Date: Tue, 25 Feb 1997 16:17:14 -0600

>Subject: Looking for Neon Transformers

>

>

>Hello,

>

>I have been looking for over two months now for a used or new (but cheap)

>neon transformer suitable for my first Tesla Coil project, of at least

>12kv/60ma or 15kv/30ma.

>

>I have already checked many, many resources, with no luck.  None of the

>local neon shops have used transformers, they tell me "we send them off for

>credit".  I've tried junkyards, wreckers, antique stores, pawn shops, bars,

>and restaurants.  Nothing.

>

>There are three people who have offered to sell me neon transformers.  The

>first now tells me he has no more.  The second no longer responds to

>e-mails.  The third person, which was my best hope, has now selfishly

>decided that instead of selling them, will only accept trades.  Too bad for

>all the newbies trying to get their foot in the door, with nothing to trade...

>

>Can anyone help?

>

>Thanks in advance,

>Chris C.

>

Regards,

Kevin Nardelle

/*

*********************                                               \

* T   E   S   L   A *                                               |

*********************                                  *************

Mail list information for acdcx, the place                *****| |*****

to buy-sell-trade tesla and HV electronic                      | |

parts and surplus!                                             | |

Send Mail to majordomo@icorp.net                               | |

in body of message type: subscribe acdcx                       | |

=================

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 21 Feb 1997 14:04:09 -0600

Subject: Re: Two big Neons

At 02:51 PM 2/21/97 GMT, you wrote:

>>From: Kevin Nardelle <knardell@mailhost.accesscom.net>

>>Date: Thu, 20 Feb 1997 23:27:55 -0600

>>Subject: Two big Neons

>>

>>What would happen if I was to take two 5KV 120MA neons and parallel =

>them??

>>I know what you are going to say, 5KV 240MA right?

>>

>>Would my spark be better with 240MA at 5KV than 12KV at 120MA?

>>

>>                                       Regards,

>>                                            Kevin Nardelle

The simple answer is no.

Spark gaps dont work as well at low voltage. To get reliable quenching

you need special gap designs. Very large flat parallel surfaces and very close

spacing.

Plus look at the power. 12kv/120ma is 1440 watts.

5kv/240ma is 1200 watts.

Just because you have more available current doesn't mean

your coil can utilize it.

In order for your primary circuit to process more available current

the gap has to strike quickly and after the cap discharges the gap must

quench quickly. By quench I mean for the arc to go out and the gap

to become an open circuit again. The cap will not begin charging again

until the gap quenches.

If you have an efficient gap you may get several charge/discharge

cycles per AC cycle.

To get this quenching you must force the gap to quench quickly.

Blowing hi speed air through a gap will help to blow out the arc.

Sucking air through the gap with a hi speed vacuum cleaner motor

is also effective.

Rotary gaps also will force a break but these are too hard on neons

so reserve them for pole pig transformers.

Using a larger tank cap will help to use some more of the

available current. This is most effective if you  balance that with a

larger top capacitance on the secondary. (bigger toroid)

This will lower the resonant frequency and allow you to keep the

primary coil tapped out near the last few turns.

Comments and questions welcome.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 26 Feb 1997 11:29:46 -0600

Subject: Re: fried chokes

At 05:00 AM 2/26/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Tue, 25 Feb 1997 21:00:12 -0800

>Subject: fried chokes

>

>Well folks I'm going to be out of business for some time both of my RF

>chokes shorted out. Even though I had them submerged in mineral oil they

>still shorted! They were both ferrite bobbin core chokes wound with #24

>magnet wire. Since they have me shut down for awhile I've decided to

>make some modifications! I'm moving the neons offboard to accomidate

>another cap which I'm in the process of building, and putting them and

>the resistors in a wooden box. Instead of the transfomer outputs both

>being fed into two resistors I'll have a 3K resistor on each leg of each

>transformer. I read in one of the other posts that this helps to balance

>the load on the transformers. Why?

>

>

>

>

>                                 Frankensteins Helper

>                                         Max

>

Roderick,

Did you put some insulation on these ferrite cores before

putting on the windings? Some ferrite materials are at

least semi conductive. A couple layers of good quality

vinyl tape should be adequate. I used 3 layers of tape

on my ferrite toroids and havn't had any problems.

The use of series resistors can help to dampen any

unwanted resonances that may occur in your neon

secondary.

If your neons have slightly different output voltages

the resistors will keep them from fighting one another

due to the voltage difference.

But I havn't found the use of resistors necessary

but they won't do any harm other than reduce your

output slightly.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Thu, 20 Feb 1997 23:27:55 -0600

Subject: Two big Neons

What would happen if I was to take two 5KV 120MA neons and parallel them??

I know what you are going to say, 5KV 240MA right?

Would my spark be better with 240MA at 5KV than 12KV at 120MA?

Regards,

Kevin Nardelle

*********************

* T   E   S   L   A *

*********************

Mail list information for acdcx, the place

to buy-sell-trade tesla and HV electronic

parts and surplus, send me mail for info

on how to subscribe.

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 26 Feb 1997 12:51:03 -0500

Subject: Re: Doubling output voltage w. diodes

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Doubling output voltage w. diodes

> Date: Wednesday, February 26, 1997 8:43 AM

>

> From: " Erik  Hennix" <mr_white@hotmail.com>

> Date: Wed, 26 Feb 1997 05:43:36 -0800 (PST)

> Subject: Doubling output voltage w. diodes

>

>

> Hi everyone!

>

> I've got a rather low voltage (3.75 kV) neon rated at 200 W.

> Now that's a bit low a voltage for a TC, so I thought maybe I can

> connect two diode bridges (full wave rectifiers) made from HV diodes

> to the neon's AC outlet and then connect the rectified outlets from

> the diode bridges in series, like you connect batteries.

> (I tried to make some ascii illustration, but it didn't survive

cut-paste.)

>

> HV diodes are easily obtained by wiring cheap 1kV 1A diodes in series.

>

> Would this work?

>

> If so it would be a rather neat way of doubling the

> primary voltage.

>

> Cheers,

>

> Erik.

>

>

> ---------------------------------------------------------

> Get Your *Web-Based* Free Email at http://www.hotmail.com

> ---------------------------------------------------------

>

Erik,

You cannot put the outputs of the two diode bridges in series if they share

the same transformer at their inputs. You can PARALLEL them for extra

current capacity, but you cannot connect them in series. If you draw the

circuit and trace out the cuurent paths, you will see that there are

"sneak" paths.

If you want to double the voltage, then you can use what is termed a

full-wave doubler as shown below:

s---*-->|-----*---------O (+)2E

---------p s   |         |

p s   |         |

p s   |        --- +

---------p s   |        --- -

s   |         |

s-------------*

|         |

|        --- +

|        --- -

|         |

|         |

--|<-----*---------O (-)

This is far more efficient than a Marx type doubler, but is limited to

multiplying by a factor of two. By the way, each capacitor will actually

see 1.414*RMS, and the diodes will each see 1.414*2*RMS. For safety reasons

these actual peak values should be multiplied by at least 1.5 to arrive at

a SAFE peak rating.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 26 Feb 1997 20:33:25 -0800

Subject: Re: fried chokes

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 26 Feb 1997 11:29:46 -0600

> Subject: Re: fried chokes

>

> At 05:00 AM 2/26/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Tue, 25 Feb 1997 21:00:12 -0800

> >Subject: fried chokes

> >

> >Well folks I'm going to be out of business for some time both of my RF

> >chokes shorted out. Even though I had them submerged in mineral oil they

> >still shorted! They were both ferrite bobbin core chokes wound with #24

> >magnet wire. Since they have me shut down for awhile I've decided to

> >make some modifications! I'm moving the neons offboard to accomidate

> >another cap which I'm in the process of building, and putting them and

> >the resistors in a wooden box. Instead of the transfomer outputs both

> >being fed into two resistors I'll have a 3K resistor on each leg of each

> >transformer. I read in one of the other posts that this helps to balance

> >the load on the transformers. Why?

> >

> >

> >

> >

> >                                 Frankensteins Helper

> >                                         Max

> >

>

> Roderick,

>

> Did you put some insulation on these ferrite cores before

> putting on the windings? Some ferrite materials are at

> least semi conductive. A couple layers of good quality

> vinyl tape should be adequate. I used 3 layers of tape

> on my ferrite toroids and havn't had any problems.

>

> The use of series resistors can help to dampen any

> unwanted resonances that may occur in your neon

> secondary.

> If your neons have slightly different output voltages

> the resistors will keep them from fighting one another

> due to the voltage difference.

> But I havn't found the use of resistors necessary

> but they won't do any harm other than reduce your

> output slightly.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

>

Yes I put tape on the cores before I wound them. On closer examination

I think the center post of the choke was arcing between the two end

discs because when I unwound them none of the magnet wire seemed burned.

I have already rewound the cores using high voltage tape and 12# house

wire. I was able to get approx. 25 turns on the cores. The cores were

originally low pass filter chokes for subwoofer speakers in car stereo

systems! I am also going to wind some air chokes to use in series with

them. They will be single layer solenoids 7.5" tall and 4" in diameter

wound with 24# magnet wire.

Frankenstiens Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 26 Feb 1997 20:38:16 -0800

Subject: Re: fried chokes

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Wed, 26 Feb 1997 06:44:18 -0800

> Subject: Re: fried chokes

>

> Tesla-2 wrote:

> >

> > From: Roderick Maxwell <major@vicksburg.com>

> > Date: Tue, 25 Feb 1997 21:00:12 -0800

> > Subject: fried chokes

> >

> > Well folks I'm going to be out of business for some time both of my RF

> > chokes shorted out. Even though I had them submerged in mineral oil they

> > still shorted! They were both ferrite bobbin core chokes wound with #24

> > magnet wire. Since they have me shut down for awhile I've decided to

> > make some modifications! I'm moving the neons offboard to accomidate

> > another cap which I'm in the process of building, and putting them and

> > the resistors in a wooden box. Instead of the transfomer outputs both

> > being fed into two resistors I'll have a 3K resistor on each leg of each

> > transformer. I read in one of the other posts that this helps to balance

> > the load on the transformers. Why?

> >

> >                                  Frankensteins Helper

> >                                          Max

> >

>

> Roderick,

>

> Some ferrites are somewhat conductive. If the chokes were directly wound

> onto the ferrite, this may have been the problem. Remember that the

> chokes, if working properly, may be standing off voltage peaks that are

> twice the voltage seen on each leg of the neon. Air-core chokes, wound

> on some 2" or 3" PVC will be able to take the voltage. Use #24 - #28

> gauge, and make the winding length at least 6"close-wound. You won't

> need to submerge these in oil. Putting resistors in each leg will work

> OK, but is not essential. I assume you're using neons with the same

> voltage and current rating?

>

> Safe coilin' to you!

>

> -- Bert --

>

Yes both of the transformers are 7500vac Franceformers @30ma. I'll also

take your advice and use two air chokes. These are single layer

solenoids are they not?

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 26 Feb 1997 20:39:34 -0800

Subject: Re: fried chokes

Tesla-2 wrote:

>

> From: Steve Falco <sfalco@worldnet.att.net>

> Date: Wed, 26 Feb 1997 07:00:58 -0500

> Subject: Re: fried chokes

>

> Tesla-2 wrote:

> > Instead of the transfomer outputs both

> > being fed into two resistors I'll have a 3K resistor on each leg of each

> > transformer. I read in one of the other posts that this helps to balance

> > the load on the transformers. Why?

>

> When you tie two transformers directly together, they both have to agree

> on the voltage.  This means one transformer (the slightly higher voltage

> one) will waste some electricity raising the voltage in the lower

> voltage one.  And even if the transformers are "identical", the voltage

> will be a little different from each owing to manufacturing tolerences.

> If the transformers have a fairly low internal resistance, then a

> significant amount of power can be wasted.

>

> When you put a resistor in series with each transformer, then the

> transformers can each run at their prefered voltage.  Some energy will

> be lost in the resistors, but at least the transformers won't be

> fighting each other as much.

>

> To take an extreme case, suppose you had a 12 volt and a 6 volt

> transformer, and hooked them up without the resistors.  The transformers

> would probably both burn up, trying to agree on that average 9 volts.

> But with resistors in series, they would not burn up.

>

>         Steve Falco

Thanks for clearing up some of the fog for me.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Krumins  Aaron <krumins@bvsd.k12.co.us>

Date: Wed, 26 Feb 1997 19:27:40 -0700 (MST)

Subject: transformers

I have had a good deal of trouble locating a used 9,000 30 neon

transformer and after hearing someone suggest using a car transformer was

wondering if one of these would handle AC current and if it would produce

a sufficient amount of energy to jump the spark gap. Any help concerning

this would be much appreciated.

_____________________________________________

|Aaron Krumins                              |

|Devoted learner of computers i.e internet  |

|Krumins@bvsd.k12.co.us                     |

|                                           |

|"Silicone Valley or bust"                  |

---------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 26 Feb 1997 23:16:00 -0600

Subject: Re: fried chokes

At 04:33 AM 2/27/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Wed, 26 Feb 1997 20:33:25 -0800

>Subject: Re: fried chokes

>

>Tesla-2 wrote:

>>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Wed, 26 Feb 1997 11:29:46 -0600

>> Subject: Re: fried chokes

>>

>> At 05:00 AM 2/26/97 GMT, you wrote:

>> >From: Roderick Maxwell <major@vicksburg.com>

>> >Date: Tue, 25 Feb 1997 21:00:12 -0800

>> >Subject: fried chokes

>> >

>> >Well folks I'm going to be out of business for some time both of my RF

>> >chokes shorted out. Even though I had them submerged in mineral oil they

>> >still shorted! They were both ferrite bobbin core chokes wound with #24

>> >magnet wire. Since they have me shut down for awhile I've decided to

>> >make some modifications! I'm moving the neons offboard to accomidate

>> >another cap which I'm in the process of building, and putting them and

>> >the resistors in a wooden box. Instead of the transfomer outputs both

>> >being fed into two resistors I'll have a 3K resistor on each leg of each

>> >transformer. I read in one of the other posts that this helps to balance

>> >the load on the transformers. Why?

>> >

>> >

>> >                                 Frankensteins Helper

>> >                                         Max

>> >

>>

>> Roderick,

>>

>> Did you put some insulation on these ferrite cores before

>> putting on the windings? Some ferrite materials are at

>> least semi conductive. A couple layers of good quality

>> vinyl tape should be adequate. I used 3 layers of tape

>> on my ferrite toroids and havn't had any problems.

>>

>> The use of series resistors can help to dampen any

>> unwanted resonances that may occur in your neon

>> secondary.

>> If your neons have slightly different output voltages

>> the resistors will keep them from fighting one another

>> due to the voltage difference.

>> But I havn't found the use of resistors necessary

>> but they won't do any harm other than reduce your

>> output slightly.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>

>  Yes I put tape on the cores before I wound them. On closer examination

>I think the center post of the ckoke was arcing between the two end

>discs because when I unwound them none of the magnet wire seemed burned.

>I have already rewound the cores using high voltage tape and 12# house

>wire. I was able to get approx. 25 turns on the cores. The cores were

>originally low pass filter chokes for subwoofer speakers in car stereo

>systems! I am also going to wind some air chokes to use in series with

>them. They will be single layer solenoids 7.5" tall and 4" in diameter

>wound with 24# magnet wire.

>

>

>

>                                  Frankenstiens Helper

>                                          Max

Your air core chokes would appear to be worth about 5 mh.

Thats probably more than you had with the other chokes.

If you are going to build these then I wouldn't bother

putting the ferrite ones in series. The air core ones will be fine.

I would recommend keeping your series resistors inline though.

These chokes can ring up like mini Tesla coils. The resistance

will help dampen that.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 26 Feb 1997 23:15:59 -0600

Subject: Re: transformers

At 02:27 AM 2/27/97 GMT, you wrote:

>From: Krumins  Aaron <krumins@bvsd.k12.co.us>

>Date: Wed, 26 Feb 1997 19:27:40 -0700 (MST)

>Subject: transformers

>

>I have had a good deal of trouble locating a used 9,000 30 neon

>transformer and after hearing someone suggest using a car transformer was

>wondering if one of these would handle AC current and if it would produce

>a sufficient amount of energy to jump the spark gap. Any help concerning

>this would be much appreciated.

>

>Aaron Krumins

Aaron,

You aren't the only one having trouble locating neons.

For some reason they seem to be in short supply in

some areas. I live in a small city of about 34k population

in agricultural central Illinois but I found 3 neon shops

within an hours drive with used transformers.

I can only advise you to keep looking.

By car transformer I assume you mean an automotive ignition coil.

While I have seen plans for very small coils powered this way

I have never seen one work well. I tried to build one

quite some time ago and was only able to coax 2 inch sparks from it.

Its simply a matter of power. Most common ignition coils

get mighty warm running more than 60 watts or so.

But a small 7500 30ma neon transformer runs 225 watts.

Also worth looking for are ignition transformers from old oil

burning furnaces. These are usually rated at 10kv/23 ma.

These aren't quite as hardy as neons but work fairly well.

Try heating contractors and furnace repair companies.

Keep hunting.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Wed, 26 Feb 1997 23:45:29 -0600

Subject: Re: transformers

At 02:27 AM 2/27/97 GMT, you wrote:

>From: Krumins  Aaron <krumins@bvsd.k12.co.us>

>Date: Wed, 26 Feb 1997 19:27:40 -0700 (MST)

>Subject: transformers

>

>I have had a good deal of trouble locating a used 9,000 30 neon

>transformer and after hearing someone suggest using a car transformer was

>wondering if one of these would handle AC current and if it would produce

>a sufficient amount of energy to jump the spark gap. Any help concerning

>this would be much appreciated.

>

No problem if you want to blow the house fuse and blow up the coil since it

can not handle that type of voltage or current.

Follow the road Mike Hammer paved for that other fellow, try a sign shop and

tell them you have a Bud Light sign you want a transformer for. They are

sure to sell you one at a great price but that would get you going anyhoo.

Regards,

Kevin Nardelle

/*

*********************                                               \

* T   E   S   L   A *                                               |

*********************                                  *************

Mail list information for acdcx, the place                *****| |*****

to buy-sell-trade tesla and HV electronic                      | |

parts and surplus!                                             | |

Send Mail to majordomo@icorp.net                               | |

in body of message type: subscribe acdcx                       | |

=================

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: CURTIS SANDOVAL <s002cgs@discover.wright.edu>

Date: Thu, 27 Feb 1997 10:47:30 -0500

Subject: transformers

Organization: Wright State University, Dayton, Ohio 45435

Okay, was down at MECI yesterday and they had a 1-1/4 amp variac for

about $30us and a 10 amp for $50us.  Are these good deals??  Also, they

had a BIG transformer (about the size of a computer monitor) that said

2.144KVA 50/60Hz on the side.  Will this thing be useful??  They have a

lot of unlabelled and used items there, i wish i knew what i was looking

for by sight...  They also have some big vacuum tubes there, some of you

tubers might want to check them out at Http://www.meci.com .

cgs

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 27 Feb 1997 11:43:08 -0600

Subject: Re: transformers

At 03:47 PM 2/27/97 GMT, you wrote:

>From: CURTIS SANDOVAL <s002cgs@discover.wright.edu>

>Date: Thu, 27 Feb 1997 10:47:30 -0500

>Subject: transformers

>Organization: Wright State University, Dayton, Ohio 45435

>

>Okay, was down at MECI yesterday and they had a 1-1/4 amp variac for

>about $30us and a 10 amp for $50us.  Are these good deals??  Also, they

>had a BIG transformer (about the size of a computer monitor) that said

>2.144KVA 50/60Hz on the side.  Will this thing be useful??  They have a

>lot of unlabelled and used items there, i wish i knew what i was looking

>for by sight...  They also have some big vacuum tubes there, some of you

>tubers might want to check them out at Http://www.meci.com .

>

>cgs

>

The 1-1/4 amp variac is too small to be of much use

but the 10 amp one is very useful. $50 is about right

but not a steal. If its a Powerstat then its a good deal.

Hard to tell about the transformer. Not enough info.

Many times large transformers like that are isolation

transformers. 120VAC in 120VAC out. Useful but not

terribly so for coiling.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jeremy Bair <pwac@flinet.com>

Date: Sat, 15 Feb 1997 17:36:18 +0500

Subject: Good question... More Caps = Larger streamers?

Organization: PWAC Inc.

I have a 12kv 30ma neon sign transformer.

From what I understand about caps are they store the voltage and

repeatedly store the amperage, increasing the capacitance at each output

of the xmfr.

So the more caps, the more amps, but longer charging time.

If you added about twice as much capacitance to your tank, and widen

your spark gap, CAN you achieve more output in the secondary

compromising firing speed? (Like one fire a second or faster, or however

long it takes to charge).

Of course when you make these modifications, proper tuning is necessary.

Jeremy Bair

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 28 Feb 1997 00:57:38 -0600

Subject: Re: Good question... More Caps = Larger streamers?

At 12:36 PM 2/15/97 GMT, you wrote:

>From: Jeremy Bair <pwac@flinet.com>

>Date: Sat, 15 Feb 1997 17:36:18 +0500

>Subject: Good question... More Caps = Larger streamers?

>Organization: PWAC Inc.

>

>I have a 12kv 30ma neon sign transformer.

>

>From what I understand about caps are they store the voltage and

>repeatedly store the amperage, increasing the capacitance at each output

>of the xmfr.

>

>So the more caps, the more amps, but longer charging time.

>

>If you added about twice as much capacitance to your tank, and widen

>your spark gap, CAN you achieve more output in the secondary

>compromising firing speed? (Like one fire a second or faster, or however

>long it takes to charge).

>

>Of course when you make these modifications, proper tuning is necessary.

>

>Jeremy Bair

>

True to a certain point adding more capacitance will result

in higher primary circuit energies.

Thats assuming your tranformer is capable of charging the bigger cap.

A 12kv/30ma neon can only charge a cap up to .0066 mfd. How do

we arrive at this value?

Maximum power transfer occurs when the source and load impedances

are equal.

Lets look at a couple formulas.

1. Determine your neon sign transformer (or other transformer's)

Impedence:

E

Z  =  ---

I

Z = Impedence

E = volts

I = current in Amps

Note: divide milliamps by 1000 to get Amps. 30 milliamps = .030 Amps.

Example: 12kv/.03ma=400,000 ohms

Now the Impedence of the primary capacitor should match the Impedence of the

transformer at 60 Hz (if outside the US then substitute your cycle rate).

2. To match Impedence and determine capacitor value:

1

C =   -------------------

2 x pi x Z x .00006

C = capacitance in microfarads needed for primary capacitor.

Z = Impedence from equation one (Transformer Impedence)

pi = 3.141592654

Note: The .00006 is the 60 Hz AC, if you live outside the US then

substitute your cycle rate.

Example: 2 X pi X 400,000 X .00006 = 150.79

1

--------- = .0066 mfd

150.79

This represents the largest value cap this particular transformer can

charge. This is not exact. Neons vary from manufacturer to manufacturer

as far as impedance and output current capability go. But this serves

as a good guide and gives us an idea of what size our cap should be.

Comments criticism and cash donations welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Tim Maleske <maleske@worldchat.com>

Date: Mon, 03 Mar 1997 20:37:30 -0500

Subject: Some "get the ball rolling" Questions

Greetings Coilers.

Just a quick question (or two).

What is the easiest/most convenient/most effective way of connecting the

torrid and RF ground to the coil. This coil is portable, so the

connections must facilitate the breaking down of the system.

What about tapping the primary? Hose clamps Burny connectors? Any other

ideas?

Ok. Just one more.

What are easy effective toroids. Can you use aluminum conduit? I was

thinking of bending some 45's and cutting and welding them together. Any

ideas what the capacitance of this torrid would be? Any other ideas?

Tim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Mon, 03 Mar 1997 23:17:31 -0800

Subject: new cap

Sunday I finished my second poly plate capacitor. I now have 6000

picofarads. What I would like to know is how long I should let it

degass before I put it to work?

The first cap was no problem since I built it months before I completed

the rest of the coil. I'm just itching to see how much more output I get

with all that extra stored energy!

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 03 Mar 1997 23:31:19 -0800

Subject: Re: new cap

Tesla-2 wrote:

>

> From: Roderick Maxwell <major@vicksburg.com>

> Date: Mon, 03 Mar 1997 23:17:31 -0800

> Subject: new cap

>

> Sunday I finished my second poly plate capacitor. I now have 6000

>  picofarads. What I would like to know is how long I should let it

> degass  before I put it to work?

> The first  cap was no problem since I built it  months before I

>  completed the rest of the coil. I'm just itching to see

>  how much more output I get with all that extra stored energy!

>

>                                     Frankensteins Helper

>                                              Max

>

Max,

You can start using it immediately. Just close your gap spacing down so

that you don't initially overstress the new cap. The best thing for

"breaking these in" (if you don't have a vacuum pump) is Tesla Coil use,

since the disruptive discharging helps work out any entrapped air.

Gradually increase the gap spacing over a period of time.

Safe cappin' to you!

-- Bert H --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 04 Mar 1997 00:52:32 -0600

Subject: Re: Some "get the ball rolling" Questions

At 01:37 AM 3/4/97 GMT, you wrote:

>From: Tim Maleske <maleske@worldchat.com>

>Date: Mon, 03 Mar 1997 20:37:30 -0500

>Subject: Some "get the ball rolling" Questions

>

>

>Greetings Coilers.

>

>Just a quick question (or two).

>

>What is the easiest/most convenient/most effective way of connecting the

>torrid and RF ground to the coil. This coil is portable, so the

>connections must facilitate the breaking down of the system.

I've always used the Richard Quick method of using epoxy to attach a

rectangular copper plate to the side of the form beneath the secondary winding.

The bottom end of the secondary is soldered to this plate and a heavy

ground lead can be attached also to the plate. I have held a ground

wire on with a couple turns of tape or a large rubber band. I have also

soldered a ground lead to the plate.

For the top I seal the end of the tube with 1/4 inch of plexiglass or Lexan.

Epoxy this in place. I then mount a 1/4 20 bolt in the end of a PVC end cap.

Slip the end cap over the top end of the secondary form with the bolt sticking

up. Attach toroid to this bolt and spiral up the free end of the secondary

winding up to it. I have attached the secondary wire to a ring terminal and

attach it to the bolt and I have also just run the terminal down on top of the

wire. Avoid any sharp bends which could promote corona.

>

>What about tapping the primary? Hose clamps Burny connectors? Ant other

>ideas?

I've used large alligator clips and small battery clamps.

Right now I'm using a large alligator clip with 2 small curved

pieces of copper soldered to the jaws to increase the surface area.

>

>Ok. Just one more.

>

>What are easy effective torrids. Can you use aluminum conduit? I was

>thinking of bending some 45's and cutting and welding them together. Any

>ideas what the capacitance of this torrid would be? Any other ideas?

By far the best and easiest method is to use flexible aluminum dryer duct.

Form it into a ring and mount a pair of pie pans bottom to bottom

inside the ring.

Heres a crude ascii diagram that I stole from somebody.

.                        .

.     .\    Pie Pans    /.     .

\--------------/     Dryer Duct

.     . /--------------\ .     .

.   /                \   .

This makes a quick easy toroid that works great. Looks pretty good too.

This ducting comes in 3 ,4, and 6 inch diameters. It is available

in larger diameters but most places only stock these 3 sizes.

The 3 inch duct makes a good toroid for a small coil. If you

are running a medium size coil near 1kva or more then use the 4 inch duct.

The 6 inch really needs 1500 va or more to break out well.

Good questions Tim.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 04 Mar 1997 00:52:29 -0600

Subject: Re: new cap

At 07:17 AM 3/4/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Mon, 03 Mar 1997 23:17:31 -0800

>Subject: new cap

>

>Sunday I finished my second poly plate capacitor. I now have 6000

> picofarads. What I would like to know is how long I should let it

>degass before I put it to work?

>The first  cap was no problem since I built it  months before I

> completed the rest of the coil. I'm just itching to see

> how much more output I get with all that extra stored energy!

>

>                                    Frankensteins Helper

>                                             Max

>

>

Roderick,

I assume by what you said above this is a flat plate cap.

When I made my flat plate cap I had intended to let it set a week

before using it. But I got anxious and started using it after only 3 days.

I ran it at very low power. Use very little gap and the smallest neon

you have. I ran with a 7500 volt neon with only .075 inch of gap.

This was enough to produce 3 or 4 inch sparks once I had it tuned right.

Operating your new cap in a Tesla coil at low power helps to break it in.

The pulsing action of the tank circuit actually vibrates the cap

plates and jars loose air bubbles.

Work it up slowly in power over a couple week period. Add more gap a

little at a time. If you have a low voltage neon use it here.

7500 and 9000 volt neons are great for this purpose.

If you only have a 15kv neon use just half of it for breaking in

your cap. It will take weeks for all the air to come out.

My plate cap is 10 months old and still has occasional bubbles.

gap.

I recommend letting it sit a week. Then another week of low power

break in. Then a third week of slowly increasing power level.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: " Brad Blackburn" <cave@ldd.net>

Date: Tue, 4 Mar 1997 19:11:20 -0600

Subject: Trying to open a Tesla file

I downloaded some files from ftp.nic.funet.fi/pub/sci/electrical/tesla.

Winzip asks me how I want to view these files. The files are  mtesla 1 &

tesla dp. The files are unzipped and I am running windows 95.Still learning

obviously.

Brad Blackburn

cave@ldd.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 04 Mar 1997 20:43:26 -0600

Subject: Re: Trying to open a Tesla file

At 01:11 AM 3/5/97 GMT, you wrote:

>From: " Brad Blackburn" <cave@ldd.net>

>Date: Tue, 4 Mar 1997 19:11:20 -0600

>Subject: Trying to open a Tesla file

>

>I downloaded some files from ftp.nic.funet.fi/pub/sci/electrical/tesla.

>Winzip asks me how I want to view these files. The files are  mtesla 1 &

>tesla dp. The files are unzipped and I am running windows 95.Still learning

>obviously.

>

>Brad Blackburn

>cave@ldd.net

>

>

Brad,

I not familiar with tesla dp but mtesla1 is an archive of

the magnifier mailing list from Australia. Much of the info

on the funet site is compressed with the UNIX

GZIP program. Winzip will decode these files.

Evidently Winzip can't extract the filename from the archive

so it prompts you for one.

I usually use whatever the archive name is with a .TXT extension.

That seems to work fine.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Wed, 5 Mar 1997 19:13:23 +0100 (CET)

Subject: Hi everybody!

<see Subject> !

I'm new on the list, and I'd like to introduce myself:

My name is Jochen Kronjaeger, I study physics in Marburg (Germany).

I'm kind of experienced in general high voltage topics, but not

that much in Tesla coils. I've build one primitive model (that more or

less worked), and I'm now thinking about building a really good coil (and

hope to collect some information here).

read you soon,

Jochen

+--------------------------------------------------------------------------+

| Jochen Kronjaeger                    Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                          |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments   |

+--------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 05 Mar 1997 22:38:49 -0800

Subject: Re: new cap

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Tue, 04 Mar 1997 00:52:29 -0600

> Subject: Re: new cap

>

> At 07:17 AM 3/4/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Mon, 03 Mar 1997 23:17:31 -0800

> >Subject: new cap

> >

> >Sunday I finished my second poly plate capacitor. I now have 6000

> > picofarads. What I would like to know is how long I should let it

> > degass before I put it to work? The first  cap was no problem since

> > I built it months before I completed the rest of the coil.

> > I'm just itching to see how much more output I get with all that extra

> > stored energy!

> >

> >                                    Frankensteins Helper

> >                                             Max

> >

> >

> Roderick,

> I assume by what you said above this is a flat plate cap.

> When I made my flat plate cap I had intended to let it set a week

> before using it. But I got anxious and started using it after only 3 days.

> I ran it at very low power. Use very little gap and the smallest neon

> you have. I ran with a 7500 volt neon with only .075 inch of gap.

> This was enough to produce 3 or 4 inch sparks once I had it tuned right.

>

> Operating your new cap in a Tesla coil at low power helps to break it in.

> The pulsing action of the tank circuit actually vibrates the cap

> plates and jars loose air bubbles.

>

> Work it up slowly in power over a couple week period. Add more gap a

> little at a time. If you have a low voltage neon use it here.

> 7500 and 9000 volt neons are great for this purpose.

> If you only have a 15kv neon use just half of it for breaking in

> your cap. It will take weeks for all the air to come out.

> My plate cap is 10 months old and still has occasional bubbles.

> gap.

>

> I recommend letting it sit a week. Then another week of low power

> break in. Then a third week of slowly increasing power level.

>

> Mike Hammer

> mhammer@misslink.net

>

Thank you for your information. It is a vertical plate type capacitor

made from 1/8" thick low density poly. My first cap is identical to the

first. I read them in parallel with my multimeter and they read 6000

picofarads. I only have at the moment two 7500vac neons wired in

parallel anyway so no danger there. Sometime this spring I plan to get

some higher current, higher voltage transfomers probably 15,000vac.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 05 Mar 1997 22:41:12 -0800

Subject: Re: new cap

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Mon, 03 Mar 1997 23:31:19 -0800

> Subject: Re: new cap

>

> Tesla-2 wrote:

> >

> > From: Roderick Maxwell <major@vicksburg.com>

> > Date: Mon, 03 Mar 1997 23:17:31 -0800

> > Subject: new cap

> >

> > Sunday I finished my second poly plate capacitor. I now have 6000

> >  picofarads. What I would like to know is how long I should let it

> > degass

> >  before I put it to work? The first  cap was no problem since I built it

> >  months before I completed the rest of the coil. I'm just itching to see

> >  how much more output I get with all that extra stored energy!

> >

> >                                     Frankensteins Helper

> >                                              Max

> >

> Max,

>

> You can start using it immediately. Just close your gap spacing down so

> that you don't initially overstress the new cap. The best thing for

> "breaking these in" (if you don't have a vacuum pump) is Tesla Coil use,

> since the disruptive discharging helps work out any entrapped air.

> Gradually increase the gap spacing over a period of time.

>

> Safe cappin' to you!

>

> -- Bert H --

>

I'm not brave enough to do that!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 06 Mar 1997 00:12:07 -0600

Subject: Re: Hi everybody!

At 06:13 PM 3/5/97 GMT, you wrote:

>From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

>Date: Wed, 5 Mar 1997 19:13:23 +0100 (CET)

>Subject: Hi everybody!

>

><see Subject> !

>

>I'm new on the list, and I'd like to introduce myself:

>

>My name is Jochen Kronjaeger, I study physics in Marburg (Germany).

>I'm kind of experienced in general high voltage topics, but not

>that much in Tesla coils. I've build one primitive model (that more or

>less worked), and I'm now thinking about building a really good coil (and

>hope to collect some information here).

>

>read you soon,

>

>   Jochen

>

Jochen,

Welcome aboard.

If you have some ideas about your new coil go ahead

and share them with us. Perhaps you will have some

problems in your design we can work out. Thats what the list

is for.

No sense in you making the same mistakes I did.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "steve pelet" <stevepel@axs2000.net>

Date: Thu, 6 Mar 1997 06:29:41 -0500

Subject: re:please reads soon  can someone e-mail back as soon as possible

where can I find these parts:

1. LM567 Tone Decoder

2. 7808 Voltage Regulator

3. 2 pounds of #26 magnet wire

4. AMIDON EA-77-250 ferrite core and bobbin

i need them for my tesla coil

please reply soon

steve

stevepel@axs2000.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Chris Crochet <ccrochet@premier.net>

Date: Thu, 06 Mar 1997 11:28:31 -0600

Subject: re:please reads soon  can someone e-mail back as soon as

At 11:29 AM 3/6/97 GMT, you wrote:

>where can I find these parts:

>1. LM567 Tone Decoder

>2. 7808 Voltage Regulator

>3. 2 pounds of #26 magnet wire

>4. AMIDON EA-77-250 ferrite core and bobbin

>

>stevepel@axs2000.net

Steve,

Electronics Goldmine (1-800-445-0697) carries the 7808 VReg for only 59

cents.  They also carry spools of high quality enameled magnet wire, in

gauges from 15 to 39.  They do not have 26 gauge, however, they do have 25

and 27 gauge.

Order the VReg by the part number, order the wire by one of these:

Gauge=25, Approx weight (lbs)=7, Stock #G8071, Price=$21.00

Gauge=27, Approx weight (lbs)=8, Stock #G8072, Price=$24.00

They only sell complete spools.  For smaller quantities, you may want to

look for a local motor rebuilding shop, they will often sell magnet wire by

the pound.

Digikey (1-800-344-4539) carries the LM567, as will many other places (it's

a pretty common part; unfortunately, Electronics Goldmine didn't have any).

Note that for such a small order, they will add a $5 handling surcharge in

addition to shipping.

I don't know of a source for Amidon cores.

Hope this helps,

Chris C.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: rjonese@erols.com

Date: Thu, 06 Mar 1997 16:34:21 -0500

Subject: re:please reads soon  can someone e-mail back as soon as

>where can I find these parts:

>1. LM567 Tone Decoder

>2. 7808 Voltage Regulator

>3. 2 pounds of #26 magnet wire

>4. AMIDON EA-77-250 ferrite core and bobbin

>

>i need them for my tesla coil

>please reply soon

>steve

>stevepel@axs2000.net

Try a magnet rewinding shop. If u can get the end of a roll, they wont give

u a cutting charge, so it will end up being about 7 bucks a pound. If they

DO have to spool the wire off for u, it wont cost more than 10 bucks.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Fri, 07 Mar 1997 10:45:09 -0800

Subject: new cap

I ran my coil last night with my new capacitor! I seem to be getting

the same amount of discharge at the top of the coil with the variac at

65% as I got with the variac at 100% with one cap and another gap! I am

now tapped in at turn number ten. Previously I was tapped in at turn

number 13. I'm going to run it at low power for about a week and then

gradually increase power levels.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Fri, 07 Mar 1997 10:49:21 -0800

Subject: magnetic gaps

Does anyone have any experience with magnetic gaps? Your input would be

greatly appreciated. I plan to get some higher voltage transformers and

wish to run it in series with my existing gaps.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 07 Mar 1997 13:16:07 -0600

Subject: Re: magnetic gaps

At 06:49 PM 3/7/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Fri, 07 Mar 1997 10:49:21 -0800

>Subject: magnetic gaps

>

>Does anyone have any experiance with magnetic gaps? Your input would be

>greatly appreciated. I plan to get some higher voltage transformers and

>wish to run it in series with my existing gaps.

>

Not direct experience. The theory is that very strong

magnets can be used to help quench the gap. And it takes

very very strong magnets. Like neodymium iron boron magnets.

Much simpler and easier to implement would be a vacuum gap.

This is nothing more than a conventional static gap with a strong

vacuum cleaner motor sucking air through the teeth of the gap.

Very efficient and works well with neons.

I am presently building one of these. I should have it ready

soon after my shop warms up in the spring. (been a long winter)

My RQ gap stops quenching well at about 1.5 KVA.

The vacuum gap should be much better.

But it won't do anything for you until you start running

more than 1.5 KVA or so. Below that level the standard RQ gap

is just fine.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 07 Mar 1997 13:16:06 -0600

Subject: Re: new cap

At 06:45 PM 3/7/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Fri, 07 Mar 1997 10:45:09 -0800

>Subject: new cap

>

>I ran my coil last night with my new capacitor! I seem to be getting

>the same amount of discharge at the top of the coil with the variac at

>65% as I got with the variac at 100% with one cap and another gap! I am

>now tapped in at turn number ten. Previously I was tapped in at turn

>number 13. I,m going to run it at low power for about a week and then

>gradually increase power levels.

>

>

>

>

>                                  Frankensteins Helper

>                                         Max

Very good!  Keep us posted on your results.

When you get your cap broken in you will want to add

a larger toroid to your system so you can regain those

lost turns on the primary.

This will lower your resonant frequency and the larger toroid

will promote longer sparks.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Fri, 07 Mar 1997 23:49:56 -0800

Subject: Re: magnetic gaps

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 07 Mar 1997 13:16:07 -0600

> Subject: Re: magnetic gaps

>

> At 06:49 PM 3/7/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Fri, 07 Mar 1997 10:49:21 -0800

> >Subject: magnetic gaps

> >

> >Does anyone have any experiance with magnetic gaps? Your input would be

> >greatly appreciated. I plan to get some higher voltage transformers and

> >wish to run it in series with my existing gaps.

> >

>

> Not direct experience. The theory is that very strong

> magnets can be used to help quench the gap. And it takes

> very very strong magnets. Like neodymium iron boron magnets.

>

> Much simpler and easier to implement would be a vacuum gap.

> This is nothing more than a conventional static gap with a strong

> vacuum cleaner motor sucking air through the teeth of the gap.

>

> Very efficient and works well with neons.

>

> I am presently building one of these. I should have it ready

> soon after my shop warms up in the spring. (been a long winter)

> My RQ gap stops quenching well at about 1.5 KVA.

> The vacuum gap should be much better.

> But it won't do anything for you until you start running

> more than 1.5 KVA or so. Below that level the standard RQ gap

> is just fine.

> Mike Hammer

> mhammer@misslink.net

>

>

I want to try it just for the novelty! By the way I just got that

magnet wire today. Looks like very good wire. The label on the side of

the spool says MWS WIRE INDUSTRIES and they have a lot of manufacturer

numbers and so forth but no other info. Looks like they threw in a

couple of pounds more because the spool says 10.87 LBS. The main reason

I think they are selling it so cheap is the spool the wire is wound on

is cracked. But the wire itself is undamaged. Looks like a good buy for

all who are interested. I myself plan on buying some 15# and 32# wire to

build a large induction coil.

Frankenstiens Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 08 Mar 1997 08:05:15 -0800

Subject: Re: magnetic gaps

Tesla-2 wrote:

>

> From: Roderick Maxwell <major@vicksburg.com>

> Date: Fri, 07 Mar 1997 10:49:21 -0800

> Subject: magnetic gaps

>

> Does anyone have any experiance with magnetic gaps? Your input would be

> greatly appreciated. I plan to get some higher voltage transformers and

> wish to run it in series with my existing gaps.

>

Roderick,

In the "Old" days, magnetic quenching was used extensively to improve

performance of high-power spark-gap radio transmitters. The main problem

is in providing a way to insulate the magnet's pole pieces from the arc,

both electrically and thermally. Sheet mica works very well - and

American Science and Surplus caries this material in a number of sizes

(http://www.sciplus.com/cgi-bin/basket/834245226.0/home.html). If you

use some powerful rare-earth magnets, you may be able to try seeing the

effects.

You certainly can see the effects on a simple "power arc" coming

straight off a neon. The strong magnetic field causes the arc to

"stretch", helping to cool it. I've not heard of anyone actually

demonstrating improved performance in Tesla Coil operation however...

Good Luck, and let us know how it worked!

Safe quenching to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Sat, 08 Mar 1997 11:08:45 -0800

Subject: Re: magnetic gaps

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Sat, 08 Mar 1997 08:05:15 -0800

> Subject: Re: magnetic gaps

>

> Tesla-2 wrote:

> >

> > From: Roderick Maxwell <major@vicksburg.com>

> > Date: Fri, 07 Mar 1997 10:49:21 -0800

> > Subject: magnetic gaps

> >

> > Does anyone have any experiance with magnetic gaps? Your input would be

> > greatly appreciated. I plan to get some higher voltage transformers and

> > wish to run it in series with my existing gaps.

> >

>

> Roderick,

>

> In the "Old" days, magnetic quenching was used extensively to improve

> performance of high-power spark-gap radio transmitters. The main problem

> is in providing a way to insulate the magnet's pole pieces from the arc,

> both electrically and thermally. Sheet mica works very well - and

> American Science and Surplus caries this material in a number of sizes

> (http://www.sciplus.com/cgi-bin/basket/834245226.0/home.html). If you

> use some powerful rare-earth magnets, you may be able to try seeing the

> effects.

>

> You certainly can see the effects on a simple "power arc" coming

> straight off a neon. The strong magnetic field causes the arc to

> "stretch", helping to cool it. I've not heard of anyone actually

> demonstrating improved performance in Tesla Coil operation however...

>

> Good Luck, and let us know how it worked!

>

> Safe quenching to you!

>

> -- Bert --

>

Thank you for your response. Really I just want to try it out for

novelties sake. I also plan to place them in such a way that the air flow

from the fan in the first spark gap flows over the magnetic gap.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Sun, 9 Mar 1997 15:26:15 -0500

Subject: secondary frequency

Hi, my name is Ritch Gordon and although I am not really new to Tesla coil

constuction I am a beginner in full system design.  I am currently working

on the details of my new system and I have one nagging question -how do I

choose the secondary operating frequency?  Working from the power supply

end I have a neon (15000V @60mA =250kohms) this set the cap at about

0.01uf. For the secondary I have a 4" form and a generous supply of #22

magnet wire. Any thoughts on choosing an operating frequency so I can wind

my secondary?

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "steve pelet" <stevepel@axs2000.net>

Date: Sun, 9 Mar 1997 18:30:41 -0500

Subject: re:please read everyone with knowlege or has written down instruction

for a traditonal spark coil

could someone send me instructions on how-to make the traditional spark gap

coil

like step-by-step

here are the parts I have and was told to get:

Steve,

One little pointer on Internet etiquette. No need to send the whole

message back to me. Your reply was 2 lines and you sent the whole thing

back to me. This is especially bad on a mailing list where your post

may be sent out dozens of times to different people.

OK. Well there is no Tesla coil shop where you can go buy parts.

You will have to fabricate your own parts using what you have laying

around or can find. Some things you will have to buy.

First thing is you need a neon sign transformer. Get out the yellow pages

and find all the sign shops in your area. Especially good are the ones

that advertise repairs and service. Go to these places and get a pair

of 9000 volt/30 ma neons. Get 2 so you have a spare and later on you

can parallel them for more power. Expect to pay around $15 -20 each.

Sometimes you will find places that will give away old transformers.

I had a place give me 4 12000 volt neons free. 2 were bad and 2 were just

fine.

While you have the yellow pages out look for electric motor rebuilding

shops. Virtually every community with any industry at all will have one

of these. Check one out and buy some AWG 24 or 26 magnet wire. Its

sold by the pound and you will need about 3 pounds. If you get

a good deal get 5 pounds. Better to have too much than not enough.

Expect to pay $4 - 7 per pound.

I buy all my wire at a local motor shop for $4.15/lb. Cheapest

price I've found.

You will need a form for your secondary coil. Since this is to be your

first coil I recommend a 3 inch diameter coil. It will be more efficient

than the commonly recommended 2 inch and wont cost much more.

Best material is white PVC pipe. Check out construction sites. PVC pipe

is laying around everywhere. If you have to but some its cheap. You

will need about 2 feet.

For your primary coil you will need some heavy solid wire

or copper tubing. AWG 8 or 10 solid house wire will make a fine

primary for a small coil. Look around construction sites.

If you have to buy get 1/4 inch copper tubing. You will need

around 40 feet or so. You can wait on this for a while.

Constructing the secondary will take a bit of time.

For a cap I recommend a saltwater leyden jar made in 12 oz beer bottles.

This is not the best cap but it works and is cheap. Gather up about

10 of these. You want long necks. I had best luck with brown glass

bottles. The clear ones from Miller are too thin and wont hold

up to the voltage.

You can also build a first class cap with aluminum flashing and

polyethylene sheeting. But its not cheap and a lot of work. You will want

to build one of these if you decide to build a second larger coil.

You will need some 3/4 inch hard copper tubing for the spark gap.

Household plumbing copper is OK. You need about 18 inches.

This will get you started. Find these parts first. Especially the neon

transformer. If you can't get one you wont need the rest of the stuff.

<><><><><><><><><><><><><><><><><><><><><><><><><><><><><><>

I have already asked mike hammer. I think is on the list. He helped me out

alot but he said he didn't have time to write me step-by-step instructions.

thanks for you help

steve

stevepel@axs2000.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Sun, 09 Mar 1997 17:41:51 -0600

Subject: Re: secondary frequency

At 08:26 PM 3/9/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Sun, 9 Mar 1997 15:26:15 -0500

>Subject: secondary frequency

>

>Hi, my name is Ritch Gordon and although I am not really new to Tesla coil

>constuction am am a beginner in full system design.  I am currently working

>on the details of my new system and I have one nagging question -how do I

>choose the secondary operating frequency?  Working from the power supply

>end I have a neon (15000V @60mA =250kohms) this set the cap at about

>0.01uf. For the secondary I have a 4" form and a generous supply of #22

>magnet wire. Any thoughts on choosing an operating frequency so I can wind

>my secondary?

>

>        Ritch Gordon

Ritch,

The secondary resonant frequency is actually irrelevant.

What is more important is the H/D (height/diameter) ratio

of your secondary.

The best coils have ratios from 3 to 1 to 6 to 1. Low ratios

for larger coils.

Looks like you will be building the exact some secondary I have.

The best raio for a 4 incher is about 5 to 1. PVC is 4 1/2 inches

OD so 5 to 1 gives you a 22 1/2 inch winding length. Now you

need to wind from 700 - 900 turns in the allotted space.

This makes AWG 24 the best choice. This would allow for nearly 900

turns in a 20 inch winding.

Using AWG 22 you will only get slightly over 800 in a 22 inch winding.

On my coil I got 756 turns in a 22 inch winding. (I used double formvar)

The exact number of turns you get depends on your skill and the insulation

build thickness.

Your resonant frequency with 816 turns of # 22 in a 22 inch

long winding will be approximately 475 Khz. This is the bare frequency

with no discharge terminal.

A discharge terminal will lower this frequency

considerably. A 6 inch sphere down to about 400 KHZ

and a 15 X 3 inch toroid to 320 KHZ. Larger toroids add even

more capacitance and result in lower frequencies.

So don't worry about the secondary frequency. Just make

your secondary and then accept whatever frequency it happens to be.

Then use a large enough primary to resonate your main cap

at the same frequency with at least 2 turns to spare.

Make that primary large enough so you can add a large toroid and

still reach resonance. 12 - 14 turns is not too large.

My primary currently has 17 turns. (I added turns twice)

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Tue, 11 Mar 1997 14:29:39 +0100 (CET)

Subject: Q: cm or in ?

Hi,

one more question before I actually start writing on topic:

Living in Europe, I'm familiar with the metric system (meter, cm, mm ...).

Is it o.k. to use it here, or should I convert to inches etc.?

Could anybody tell me how to convert the diameter of wire in mm

into "wire gauge" units?

Thanks,

Jochen

+--------------------------------------------------------------------------+

| Jochen Kronjaeger                    Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                          |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments   |

+--------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Tue, 11 Mar 1997 21:36:12 -0800

Subject: missing messages

If anyone sent me any messages Saturday or Sunday I had trouble on the

local server and did not receive them. And thus could not reply. I don't

want anyone to get the idea I'm snubbing them.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mac Art <ummacius@cc.UManitoba.CA>

Date: Tue, 11 Mar 1997 21:28:46 -0600 (CST)

Subject: bi-polar coil and secondary question

Hi my name is Artur and I'm new to this list, I have not completed

my first coil yet but I'm working on it.

I was wondering how does the change in the secondaries coil

diameter affect the coils perfomance?  It is obvious that reducing

the diameter changes the number of turns in the secondary, but would

you have to add turns to the secondary coil in order to keep the

same output performance as it was before you reduced the diameter?

I'm curious did anyone ever try and build a bi-polar tesla coil?

You can see the picture at

http://www.mip.berkeley.edu/images/physics/D+75+04.gif

Did anyone construct such a coil I would be interested in the

specifications on it.

There is another picture of that same design but I was unable to locate it.

Thanks to anyone who can help.

Artur

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 12 Mar 1997 00:18:28 -0600

Subject: Re: missing messages

At 05:36 AM 3/12/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Tue, 11 Mar 1997 21:36:12 -0800

>Subject: missing messages

>

>If anyone sent me any messages saturday or sunday I had trouble on the

>local server and did not recieve them. And thus could not reply. I don't

>want anyone to get the idea I'm snubbing them.

>

>

>

>                        Frankensteins Helper

>                               Max

Roderick,

No there hasn't been a lot going on the past few days.

Things have been a bit quiet.

I suppose everyone has been outside now that the weather

looks a bit like spring

With the weather improving I actually spent some time

in the garage working on my new vacuum gap. This was a project

I started last fall but it got shelved when it became to cold

to work in the garage.

I hope to been throwing some truly wicked sparks soon.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 12 Mar 1997 00:46:28 -0600

Subject: Re: bi-polar coil and secondary question

At 03:28 AM 3/12/97 GMT, you wrote:

>From: Mac Art <ummacius@cc.UManitoba.CA>

>Date: Tue, 11 Mar 1997 21:28:46 -0600 (CST)

>Subject: bi-polar coil and secondary question

>

>

>Hi my name is Artur and I'm new to this list, I have not completed

>my first coil yet but I'm working on it.

>

>I was wondering how does the change in the secondaries coil

>diameter affect the coils perfomance?  It is obvious that reducing

>the diameter changes the number of turns in the secondary, but would

>you have to add turns to the secondary coil in order to keep the

>same output performance as it was before you reduced the diameter?

Reducing the diameter would only change the numbers of turns

if you kept the same length of wire.

Larger diameter means more inductance. More inductance means

lower frequency.

Larger diameter secondaries tend to be more efficient.

Smaller diameter secondaries will generally perform poorer

than larger ones regardless of the number of turns.

You should have from 700-900 turns on your secondary regardless

of its diameter.

>

>

>I'm curious did anyone ever try and build a bi-polar tesla coil?

>You can see the picture at

>

>http://www.mip.berkeley.edu/images/physics/D+75+04.gif

>

>Did anyone construct such a coil I would be interested in the

>specifications on it.

>There is another picture of that same design but I was unable to locate it.

>

>

>Thanks to anyone who can help.

>

>Artur

I've never built a bipolar coil but have seen one operate. It was

very unimpressive. Quite poor performance compared to normal

coils I've built. It may have just been a poor implementation.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 11 Mar 1997 11:08:36 -0600

Subject: Re: Q: cm or in ?

At 01:29 PM 3/11/97 GMT, you wrote:

>From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

>Date: Tue, 11 Mar 1997 14:29:39 +0100 (CET)

>Subject: Q: cm or in ?

>

>Hi,

>

>one more question before I actually start writing on topic:

>

>Living in Europe, I'm familiar with the metric system (meter, cm, mm ...).

>Is it o.k. to use it here, or should I convert to inches etc.?

Use what your most comfortable with. Most of us technical types

have no problem with the metric system.

>

>Could anybody tell me how to convert the diameter of wire in mm

>into "wire gauge" units?

>

> Thanks,

>

>    Jochen

I don't have any kind of formula but I have a chart that shows

the equivalents. I"ll paste it in here.

AWG     dia in mils     dia in mm

2       257.6           6.544

4       204.3           5.189

6       162.0           4.115

8       128.5           3.264

10      101.9           2.588

12      80.8            2.053

14      64.1            1.628

16      50.8            1.291

18      40.3            1.024

20      32.0            0.812

22      25.3            0.644

24      20.1            0.511

26      15.9            0.405

28      12.6            0.321

30      10.0            0.255

32      8.0             0.202

34      6.3             0.160

36      5.0             0.127

38      4.0             0.101

40      3.1             0.080

Hope this is of some help.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Wed, 12 Mar 1997 20:32:46 -0500

Subject: more questions

Hi! Ritch Gordon here

I wound my secondary on the weekend on a 4.5" form with #22 mag wire.

I wound 26" at about 36 turns per inch for a total of 936 or so. (this thing

looks impresive)  this should yeild an unloaded secondary frequency of 429

kHz and with my 18"x 5" toroid having a capacitance of around 20 pf I expect

a frequency of 240 kHz. I realize this coils HD ratio is a little over 5:1

but is under 6:1.  I would appreciate any confirmation of these numbers.

Now for my question.  What are your views on primary design as to form ie.

flat, helical or conic? Also what about choice of material?  (on my first

coil I used 8 gauge alum. ground wire) Hollow or solid?  Please share any

views you have on this subject.

P.S. I am using a 15kv@60 mA neon and would like to parallel another

identical one.  Is this worthwhile? Also is the poly referred to in most

cap plans high density or low density?

Thanks

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 13 Mar 1997 00:31:29 -0600

Subject: Re: more questions

At 01:32 AM 3/13/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Wed, 12 Mar 1997 20:32:46 -0500

>Subject: more questions

>

>Hi! Ritch Gordon here

>

>        I wound my secondary on the weekend on a 4.5" form with #22 mag wire.

>I wound 26" at about 36 turns per inch for a total of 936 or so. (this thing

>looks impresive)  this should yield an unloaded secondary frequency of 429

>kHz and with my 18"x 5" toroid having a capacitance of around 20 pf I expect

>a frequency of 240 kHz. I realize this coils HD ratio is a little over 5:1

>but is under 6:1.  I would appreciate any confirmation of these numbers.

Sounds very good to me. Maybe a bit tall but should work well.

For first tests use a smaller terminal. You may have a hard time

tuning your coil with that large toroid because it won't want to break

out with spark at low power levels. And you wont get the high power levels

you need until you are in tune.

You could make a small toroid for testing or use a small sphere.

I always liked the copper toilet tank float as a test terminal.

It sparks easily and is cheap and easy to get.

Once you get your coil running well put that toroid on for big sparks.

Your numbers look right on to me. I came up with 423 KHZ, buts whats

a few KHZ between friends? :)

>Now for my question.  What are your views on primary design as to form ie.

>flat, helical or conic? Also what about choice of material?  (on my first

>coil I used 8 gauge alum. ground wire) Hollow or solid?  Please share any

>views you have on this subject.

The helical form of primary will work but is prone to overcoupling.

The conical form is very popular as is the flat spiral. Either of these

two should work well. I would use 1/4 copper tubing. That will handle

just about any amount of power you want. Make sure to have plenty of turns

in the primary. At least 12-14. Turn to turn spacing can be around 1/4 to

3/8 inch.

>

>        P.S. I am using a 15kv@60 mA neon and would like to parallel another

>identical one.  Is this worthwhile?

Yes. But you need efficient components to utilize all that power.

Especially in the spark gap department. Start small and work your way up.

A single 15kv/60 will give quite spectacular results with a good coil.

> Also is the poly referred to in most cap plans high density or low density?

Low density. LDPE is the material of choice. HDPE is also good but is

stiffer and more rigid. Not soft and supple like LDPE. Also good

is polypropylene.

Keep us posted on your progress.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Thu, 13 Mar 1997 07:23:52 -0500

Subject: Re: more questions

Tesla-2 wrote:

>

> From: "R+S Gordon" <ritchcom@execulink.com>

> Date: Wed, 12 Mar 1997 20:32:46 -0500

> Subject: more questions

>

> Hi! Ritch Gordon here

>

>         I wound my secondary on the weekend on a 4.5" form with #22 mag wire.

> identical one.  Is this worthwhile? Also is the poly referred to in most

> cap plans high density or low density?

I used LDPE because it is softer, and hence can be rolled.  If you are

building a plate cap it probably doesn't matter.

Steve

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Thu, 13 Mar 1997 08:24:08 -0600

Subject: Re: more questions

>> P.S. I am using a 15kv@60 mA neon and would like to parallel another

>>identical one.  Is this worthwhile?

>

>Yes. But you need efficient components to utilize all that power.

>Especially in the spark gap department. Start small and work your way up.

>A single 15kv/60 will give quite spectacular results with a good coil.

>

>> Also is the poly referred to in most cap plans high density or low density?

>

>Low density. LDPE is the material of choice. HDPE is also good but is

>stiffer and more rigid. Not soft and supple like LDPE. Also good

>is polypropylene.

>

>Keep us posted on your progress.

>

>Mike Hammer

>mhammer@misslink.net

>

>

Mike,

This guy sounds just like me a while back. Only one thing though, is his

head as hard as mine was?? Sounds just like the situation I was in, push

those fantastic gaps on him, they work great! I am building a second set for

my coil, I need more power!

More caps + More gaps = More and LONGER spark. I started stacking toroids

for fun and found that they won't break out until the tune is exact but the

spark is fantastic.

Well, I am off to a job interview. Gotta meet the boss this morning. That

other place I was working sucked eggs.

end

Regards,

Kevin Nardelle

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Thu, 13 Mar 1997 08:28:44 -0600

Subject: Re: more questions

>>Hi! Ritch Gordon here

>>

>> I wound my secondary on the weekend on a 4.5" form with #22 mag wire.

>

>>I wound 26" at about 36 turns per inch for a total of 936 or so. (this thing

>>looks impresive)  this should yield an unloaded secondary frequency of 429

>>kHz and with my 18"x 5" toroid having a capacitance of around 20 pf I expect

>>a frequency of 240 kHz. I realize this coils HD ratio is a little over 5:1

>>but is under 6:1.  I would appreciate any confirmation of these numbers.

>

>Sounds very good to me. Maybe a bit tall but should work well.

>

>For first tests use a smaller terminal. You may have a hard time

>tuning your coil with that large toroid because it won't want to break

>out with spark at low power levels. And you wont get the high power levels

>you need until you are in tune.

>You could make a small toroid for testing or use a small sphere.

>I always liked the copper toilet tank float as a test terminal.

>It sparks easily and is cheap and easy to get.

>Once you get your coil running well put that toroid on for big sparks.

Ritch,

Listen to Mike, he knows his stuff! BTW ask him about the MH gaps, sounds

like you could use some. You will find just as I did that if you do as he

says your sparks will keep growing and growing!

Shoot for a giant toroid but first get that tune right so that means start

off with something small. Another good coiler said if you try to tune with a

big terminal, place a thunb tack on it and it will allow a place to break out.

Regards,

Kevin Nardelle

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 13 Mar 1997 10:34:06 -0600

Subject: Re: more questions

At 02:24 PM 3/13/97 GMT, you wrote:

>Mike,

>

>This guy sounds just like me a while back. Only one thing though, is his

>head as hard as mine was?? Sounds just like the situation I was in, push

>those fantastic gaps on him, they work great! I am building a second set for

>my coil, I need more power!

>More caps + More gaps = More and LONGER spark. I started stacking toroids

>for fun and found that they won't break out untill the tune is exact but the

>spark is fantastic.

>                                       Regards,

>                                            Kevin Nardelle

>

Kevin, I've told you this many times but I thought to repeat

it here for the benefit of the group.

>More caps + More gaps = More and LONGER spark.

True. More and wider gaps will give longer spark because

it takes a greater voltage before the gap breaks down.

So with a wide gap your main cap will charge to a higher voltage

before the gap breaks down. Great! Right? Well... to a point.

To wide a gap will overvoltage your cap and neon transformer.

In fact you will probably get your longest sparks just before

something blows up.

How do we know how much gap we can run safely?

Thats where the safety gap comes in. The safety gap is a single static

gap placed directly across the transformer high voltage terminals.

You set the spacing on this gap to a point just above what the transformer

can break down on its own. Lets say you are using a 15kv neon.

You might set the safety so that it wont break down until it reaches 16kv.

Now as you run your coil with ever increasing gap you will start

to have flashes across the safety gap. At first with small main gap settings

these will be infrequent. But as you increase the main gap the activity

at the safety gap will increase. If you start having continous activity

at the safety gap then your main gap is too wide. If your safety

fires every few seconds then you are about right.

You don't need wide gaps to get big sparks. I have never run any more

than .175 inch of gap on my 4 inch dia coil. I have succeeded in

getting 34 inch sparks from this with only 1 KVA input.

You want big sparks? Build a bigger toroid.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----