Installation Instruction for NINDY X1.04

------------ ----------- --- ----- -----

(needed if you want to use gdb960)

How Supplied

--- --------

NINDY is shipped in two ways: on magnetic media and in EPROMs.

Most customers will only receive NINDY on magnetic media.  If you receive

NINDY EPROMs, please send back your old NINDY EPROMs to us as soon as

you have verified NINDY X1.04 is working for you.

Existing Board Types

-------- ----- -----

There are currently two QT960 board types, the QT960E20 board (or

E-Board) and the QT960F20 board (or F-Board).  The E-board has no EPROM,

and NINDY is in Flash memory.  The F-Board as both Flash and EPROM.  On

the F-Board, NINDY will be in the EPROM.  The EPROM will be located in

sockets U33, U34, U43 and U44.  Flash will be located in sockets U35, U36,

U37, and U38.

Installation for QT960E20 Boards

------------ --- -------- ------

This board has no EPROM and your old NINDY release is in Flash.

(1) Find 4 suitable EPROMs and rejumper for EPROM speed.

All jumper changes should be made with the board powered off.

The currently installed EPROMs on the F-Board are 27C202's,

however any fast 256k bit or larger, by 16 EPROM will work

(such as the 27C210-150).  170ns EPROMs or faster are suitable.

Depending on the speed of your EPROMs, you will need to jumper

the QT960 appropriately.  For the following: 'in' means connect

the two jumpers together and 'out' means do not connect the

jumpers together.

For 70ns or faster EPROMs

--- ---- -- ------ ------

These are High Speed EPROMs that use 1-0-0-0 waitstates on

a 20MHz QT960.  The following jumpers are located next to

the serial port on the QT960 board.

Jumpers:	J7-J8    in

J9-J10   out

J11-J12  in

For 120ns or faster EPROMs

--- ----- -- ------ ------

These EPROMs require  2-1-1-1 waitstates on a 20MHz QT960.

The following jumpers are located next to the serial port

on the QT960 board.

Jumpers:	J7-J8    out

J9-J10   in

J11-J12  out

For 170ns or faster EPROMs

--- ----- -- ------ ------

These EPROMs are the slowest that will run on the 20 MHz QT960.

They require 3-1-1-1 waitstates memory system.

The following jumpers are located next to the serial port

on the QT960 board.

Jumpers:	J7-J8    out

J9-J10   out

J11-J12  out

(2) Burn NINDY into the EPROMs.

NINDY is provided in Intel Hex format for burning in the

EPROMs.  The four files (one for each EPROM) are located in

your "rootdir/ROMS" directory.  Note that "rootdir" is the

path to which you loaded the media.  The files to be burned

into the EPROM's are:

File		Board Location

----		----- --------

out0.hex	U33

out1.hex	U34

out2.hex	U43

out3.hex	U44

You should label each EPROM as you are burning them with

the "NINDY X1.04" and the board location, eg. "U33" if you

are burning "out0.hex".  Note there are many vendors that

provide EPROM burning equipment.  You should follow the

vendors documented procedure for burning the EPROMs.

(3) Install new EPROMs.

With the POWER OFF, carefully insert the EPROMs paying

attention to board location and location of pin 1.

(4) Jumper board for booting from EPROM instead of Flash.

With the power off, the following jumpers may be configured

to tell the QT960 where to boot from.

J22

J23 o o o J24

o   o J25

J26

For EPROM booting		For Flash booting

--- ----- -------		--- ----- -------

+------+---+		    	   +---+------+

J23 | o  o | o |		       J23 | o | o  o |

+------+   |			   |   +------+

o    | o | J25		   | o |    o   J25

+---+			   +---+

(5) Test new EPROMs.

Turn on power.  A self-test will execute and if all goes

well, a prompt will appear.

Installation for QT960F20 Boards

------------ --- -------- ------

This board has EPROM and Flash.  You have two options: put

NINDY in EPROM or put it in Flash.

Option 1: NINDY in EPROM

------ -- ----- -- -----

(1) With the power off, carefully remove existing EPROMs.

(2) Erase EPROMs if same ones are to be used.

(3) Burn NINDY X1.04 into EPROMs and install.

(see step 2 and 3 above).

(4) Test new EPROMs.

Turn on power.  A self-test will execute and if all goes

well, a prompt will appear.

OR	Option 2: NINDY in Flash

------ -- ----- -- -----

Note this option does not allow your programs to be put in

Flash (since NINDY is in Flash) as Option 1 above does.

(1) Using the new "comm960" utility with the "-f" (for Flash)

option, download "testfl" (in the same "ROMS" directory as

the "out*.hex" files) into Flash.

comm960 -f testfl

(NOTE: The GNU/960 tool set must already be installed.)

(NOTE: see the comm960 manual page if you need further

information about using comm960.)

(2) Rejumper for booting from Flash.

See step 4 above.

(3) Test new NINDY.

Turn on power.  A self-test will execute and if all goes

well, a prompt will appear.