"sound"

"sound" activates the speaker.

Format:		(XCALL "sound" <select> <freq> <duration> <freq-1> <freq-2>)

Parameters:	<select> is a number in the range 0..4 which determines

which sound command to execute.

<freq> is the frequency in Hz.  Very low and very high

frequencies are mapped to 20 Hz.  This number is used for

timer control of the speaker.

<duration> is a number in the range 1..65535.

Zero is mapped to 65536.  The speaker is activated for

this many "ticks".

<freq-1>, <freq-2> are numbers in the range 1..65535.

Zero is mapped to 65536.  These numbers are used

for manual control of the speaker, and the sum

<freq-1> + <freq-2> determines the size of 1 "tick".

The length of a tick is processor-speed dependent.

All arguments are optional.  Once given, they remain in effect

until explicitly changed.  Also, the meaning of some arguments

may vary depending on <select>.

Explanation:	The PC has limited sound generation capabilities, lacking,

among other things, volume control, multiple voices, and

envelope control.  The only control is over the pitch and

duration of a sound.  However, the PC provides two different

ways for generating pitches, which can be combined for a

sort of timbre control, and even volume can be simulated.

The first approach uses the timer chip.  After being set

to a given frequency, the timer can run the speaker

independently of the processor.  Since processor speed

does not affect the timer, a given set of parameters run on

machines of different speeds will yield identical sounds.

The second approach is to control the speaker yourself.

This amounts to you pushing the speaker cone in and out

at the appropriate rate.  Since the rate at which you can

do this depends on the processor speed, a given set of

parameters run on machines of different speeds will give

different sounds.

The two approaches can be combined, where you drive the

speaker at one rate while the timer drives it at a

different rate.  This adds timbre to the sound, making it

more interesting.

<select> = 0 selects overall sound control.  <freq> = 0

disables sound commands and <> 0 enables them.  The speaker

is turned off.  Sound is initially enabled.

<select> = 1 selects timer control.  The <freq> is the

frequency in Hz of the sound to be generated.  The sound

lasts for <duration> ticks, except for <duration> = 0,

which leaves sound running while control returns to Scheme.

<select> = 2 selects manual control.  <freq> is ignored--any

number will do as a placeholder.  The sum <freq-1> + <freq-2>

determines the pitch through the use of delay loops.  The

speaker is off for a delay count of <freq-1> and on for

<freq-2>.  Different sums adding up to the same number

give various timbres to the basic pitch.  Larger sums decrease

the pitch--doubling the sum will drop the sound one octave,

for example.  The sound lasts for <duration> ticks before

returning to Scheme.

<select> = 3 combines 1 and 2.  The timer superimposes its

<freq> value on top of the manual control.  Otherwise,

operation is the same as selection 2.

<select> = 4 turns off the speaker.  If <freq> = 0, the

speaker is turned off and control returns immediately to

Scheme.  If <freq> <> 0, control returns only after

<duration> ticks.

Examples:	(XCALL "sound" 0 1)

;enable sound commands if they were disabled

(XCALL "sound" 1 440 2000 200 200)

;on any computer, gives the "A" (= 440 Hz)

;above middle C.  The last 3 parameters are duration

;values--increasing them give longer durations.

;The sum 200+200 determines the length of 1 tick

;and the sound lasts for 2000 ticks.

(XCALL "sound" 1 256)

;an example of defaulting--the frequency changes

;to middle C (= 256 Hz) and other parameters are

;unchanged

(XCALL "sound" 2 256 200 1075 1075)

;on a TI Business-Pro (turbo mode), gives roughly the

;same "A" above middle C.  The sum 1075+1075 determines

;the pitch and the sound lasts for 200 ticks.

;The "256" argument is ignored.

(XCALL "sound" 3 256 200 1075 1075)

;sounds similar to previous example except now a

;256 Hz sound (middle C) is superimposed

(XCALL "sound")

;repeats the previous sound

(XCALL "sound" 1 440 0)

;sound "A" and return to Scheme, leaving the sound on

(XCALL "sound" 4 0)

;turn off the sound