FM - 3812 Resident Driver

Substitute for "sound.com"

Syntax

fm

fmmin

fmmax

Options

None.

Operation

This program is a TSR that installs into interrupt 65h and handles

the function calls as specified in the 1987 Ad Lib Synthesizer Card

Programmer's Manual.

Run the program fm.com to load the driver.  The program will not load

more than once unless interrupt 65h is intercepted by another program

that does not have a valid signature.

Use the fmmin.com program to use the minimum amount of memory while

providing a 400-note buffer size, and without the background sound

option.  Use the fmmax.com program to obtain a 2200-note buffer size,

and fm.com for a 1200-note buffer size, both with the background sound

option.

Auxiliary Programs

A program can be written to load the queue and terminate before

turning the driver on.

The "mp.com" and "mm.com" programs switch the card into percussive and

melodic modes, respectively.

The programs "qo.com" and "qf.com" turn on and off the driver.

If you wish to remove the TSR from memory, first run "qf"

to turn the driver off in order to reset the clock timer to

its normal speed;  then, release the memory.

Disassembly of these programs will show you that they

are simply function calls to interrupt 65h.

The programs "bo.com" and "bf.com" turn on and off background sound.

The program "i65.com" simply reinitializes the driver.

Programming Information

Interrupt Service Functions

F#	Definition

0	Initialize; set volume to 100%, set pitch to 0,

set melodic mode, set timbre for all voices

to piano, empty queues, and set tempo to 90 bpm.

2	Set Relative Time Start;

3	Set State; either 1, active, or 0, stopped.

4	Get State; either 1, playing, or 0, complete or stopped.

5	Empty queues; silence all voices, stay active.

6	Set Mode; set melodic or percussive, set volume to 100%,

set timbre for all voices to piano, set pitch to 0.

7	Get Mode; return 0 for melodic, 1 for percussive.

8	Set Volume at Time.

9	Set Tempo at Time.

10	Set Transpose;  immediately affect all voices by specified

number of semitones.

11	Get Transpose.

12	Set Active Voice.

13	Get Active Voice.

14	Play Note with Delay; play note of pitch for length, set delay.

15	Play Note without Delay; play note of pitch for length.

16	Set Voice Timbre at Time.

17	Set Pitch at Time; affect all voices.

18	Set Ticks per Beat;  all notes should fall on multiple of

1/TickBeat, and obey: 18.2 <= (TickBeat * Tempo / 60).

19	Direct Note On;  play voice at pitch.

20	Direct Note Off;  stop voice.

21	Direct Set Timbre;  set timbre for voice.

Development Notes

02/23/92 - Queue entry size halved, reducing memory needs dramatically.

01/27/92 - All known defects corrected.

01/23/92 - Corrected delta pitch effect.

01/22/92 - Beta version 3.0 released.  Fully functional.  Queue size

too small.

01/09/92 - A defect with the combination of the timer and music interrupt

routines caused the computer to enter an endless loop, with

interrupts disabled: lock-up.  Version 2.1 corrects this.

01/07/92 - Additional functions 8000h, 8001h return pointers to internal

data structures and the count of voices;  8002h and 8003h start

and stop background fm sounds;  8004h returns the offset to the

note-queue;  8005h single-steps the note queue.

mov si, 8002h		; start background sound

int 65h

mov si, 8003h		; stop background sound

int 65h

12/30/91 - Only the Reset Chip and Direct Note On/Off and Set Timbre

functions are implemented.  All other functions simply return 0.

Otherwise, fully resident with proper signature placement and

checking.

Supplied test program fmfun.exe allows manipulation of voice

parameters and pitch;  the tab key causes the program to begin

reading memory at 0:0, determining a pitch and length for each

voice.

Notes

Use the fmfun.exe program to modify the characteristics of the voice

operators internal to the driver, when experimenting with the background

sound feature.

The keystrokes accepted by the program are grouped as so:

< >	Previous/Next Voice

[ ]	Lower/Higher Pitch, Play Note

{ }	Lower/Higher Pitch, no Play

M	Select Modulator Operator

C	Select Carrier Operator

a	Attack Rate

d	Decay Rate

l	Sustain Level

r	Release Rate

s	Sustain Sound On/Off

e	KSR Value

m	Multiply Frequency Value

v	Vibrato On/Off

b	Feedback Value

o	Output Level

k	KSL value

t	Tremolo

f	FM Toggle

+	Increment Selected Operator Variable

-	Decrement same

SPACE	Play Current Voice, Current Pitch

TAB	Begin Random Play

BS	Display Current Random Position and Time Factor

CR	Display Voice Data: Pitch, Octave, FNUM, Operator Data

ESC	Quit Program

Comparison with "sound.com"

Command Line		Max Notes 	Memory (K)

fm			1217	  	18

sound			 410	  	18

fmmin			 417	  	11

sound b1000		 410	  	18

fmmax			2225	  	26

sound b3000		1229	  	26

As can be seen from the above, the fm program queues almost three times

as many notes as does sound.com, but uses the same amount of memory.

When memory usage is of no concern, fmmax provides double the note

buffer space while using the same amount of memory as "sound b3000".

Essentially, by simply using "fm.com", you obtain "sound b3000" results

using only "sound b1000" memory.