From: tesla-2@emachine.com (Tesla-2)

Subject: ID for Primarys

Date: Thu,  1 Jan 1998 21:21:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Matt <ch038map@mode.lanl.k12.nm.us>

Date: Thu, 1 Jan 1998 14:21:49 -0700 (MST)

Subject: ID for Primarys

Greetings All-

After browsing the internet and reading a few archives I have seen

quite a bit of difference for recommended diameter for primarys. I

originally thought that you should use 2 to 3 times the diameter of the

secondary for the ID of the Primary, but I have seen people use a 6"

diameter for a 4" coil, and a 8" one for a 6" coil. I am trying to

construct a 3" coil with a inverse cone-type primary (at 30 degrees). Does

the type of primary make a difference on its diameter ? What would you

recommend using? Thanks for your help,

Matt

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ID for Primarys

Date: Fri,  2 Jan 1998 03:57:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Thu, 01 Jan 1998 21:57:25 -0600

Subject: Re: ID for Primarys

>       After browsing the internet and reading a few archives I have seen

>quite a bit of difference for recommended diameter for primarys. I

>originally thought that you should use 2 to 3 times the diameter of the

>secondary for the ID of the Primary, but I have seen people use a 6"

>diameter for a 4" coil, and a 8" one for a 6" coil. I am trying to

>construct a 3" coil with a inverse cone-type primary (at 30 degrees). Does

>the type of primary make a difference on its diameter ? What would you

>recommend using? Thanks for your help,

Actually a cone primary isn't really nessecary if your secondary is only

going to be 3" in diamter.  I would recommend a helical (vertical) primary

for a coil of that size.  Big coils use flat primaries, smaller coils use

vertical primaries, and mid-sized use inclined.  Thats just a general rule

of thumb I have heard... :)

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: tesla pics

Date: Fri,  2 Jan 1998 03:57:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Thu, 01 Jan 1998 21:57:31 -0600

Subject: tesla pics

For those of you interested, I put 10 pictures of my coil setup on my

high voltage page.  The URL for the page is :

http://www.harvestcomm.net/personal/corr/hv/index.htm

I also completed a step-by-step walkthrough "guide" to build your

first coil.  It documents what I did, and what I learned you should

not do in it as well.  It also includes pictures showing various parts

of the construction, and a program for calculating the values of your

Tesla coil.  You can download this at the above address.  Thanks.

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: ID for Primarys

Cc:

Bcc:

X-Attachments:

At 09:21 PM 1/1/98 GMT, you wrote:

>From: Matt <ch038map@mode.lanl.k12.nm.us>

>Date: Thu, 1 Jan 1998 14:21:49 -0700 (MST)

>Subject: ID for Primarys

>

>

>Greetings All-

>        After browsing the internet and reading a few archives I have seen

>quite a bit of difference for recommended diameter for primarys. I

>originally thought that you should use 2 to 3 times the diameter of the

>secondary for the ID of the Primary, but I have seen people use a 6"

>diameter for a 4" coil, and a 8" one for a 6" coil. I am trying to

>construct a 3" coil with a inverse cone-type primary (at 30 degrees). Does

>the type of primary make a difference on its diameter ? What would you

>recommend using? Thanks for your help,

>                        Matt

>

>

The general rule of twice the secondary diameter is mainly for

solenoid types of primary coils. In your case I would build an 8 inch

diameter solenoid primary for your 3 incher.

On saucer or conical primary coils the inner most turn must be

spaced far enough from the secondary to prevent arcing. The bottom

turn of the secondary is at ground potential and you could get

a 60 hz arc from the primary to ground if the inner turn is too

small.

If the inner turn is too large the coupling will be low and you

may not be able to get enough coupling for good performance.

Normally you would want 1 full inch of clearance around the secondary.

The OD of your 3 inch secondary is probably 3.5 inches so the inner

most turn should be about 5.5 inches diameter. The rest of the turns

spiral outward with about a 1/4-3/8 inch turn to turn spacing. The

final outside diameter depends on the number of turns.

If you are running 15kv a little extra clearance wouldnt hurt.

Maybe go to 6 inches diameter for the inner turn.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: pri/sec freqs

Date: Sat,  3 Jan 1998 14:33:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 3 Jan 1998 14:33:44 +0000

Subject: pri/sec freq.s

Now I'm really confused.

I did all the calculations myself and came up with the res. freq. of

my sec' as 245.27KHz, and my primary as 248.68KHz.  I checked them

against the numbers produced using Teslac and they come out right.

(using the length of the primary at 15", and the mf of the sec' of

.112<--the value of the 8" spherical topload).

These numbers look pretty darn close, to me.   But I think I'm doing

something wrong here.  The program (teslac) states that using the

11.2pf topload will drop the resonant freq. to 175.??KHz.  And if

that's the case, I'm guessing I shouldn't be using that 11.2pf value

in determining the resonant freq of the sec'. (duhh:)

So I guess my questions are. . .what value for capacitance should

I be using to determine the resonant freq of the sec', (or is it a

different formula altogether from the one used to determine the

tank circuit freq?)and how do I determine the resulting freq of the

sec' with a particular topload?  And am I shooting for a tank

circuit resonant frequency that matches the resonant freq of

the sec' with or without the topload?

Thanks All,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: new gap distance

Date: Sun,  4 Jan 1998 07:10:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sun, 4 Jan 1998 02:10:27 -0500

Subject: new gap distance

Hi coilers,

I am preparing to make a new gap .

after melting my first 2 I plan on a permanent one.

I have 13 copper tubes which are .965" diameter and 12" long,

I sorted thru a boxful of these for  the absolute straightest ones.

These will be going in a 6" sch 35 pvc pipe with a squirrel cage

blower pulling thru it.

My Question is what is the maximum gap distance I will ever need

or stated another way what distance should I space each tube?

I do not have a rotary and my present transformers are rated at 8kv .5a

I intend to be up to 12kv .5a sometime in the future.

also has anyone ever seen heat sinks that go inside a .875id tube?

Thanks in advance.

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: new gap distance

Cc:

Bcc:

X-Attachments:

At 07:10 AM 1/4/98 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Sun, 4 Jan 1998 02:10:27 -0500

>Subject: new gap distance

>

>Hi coilers,

>I am preparing to make a new gap .

>after melting my first 2 I plan on a permanent one.

>

>I have 13 copper tubes which are .965" diameter and 12" long,

>I sorted thru a boxful of these for the absolute straightest ones.

>

>These will be going in a 6" sch 35 pvc pipe with a squirrel cage

>blower pulling thru it.

>

>My Question is what is  the maximum gap distance I will ever need

>or stated another way what distance should I space each tube?

>

>I do not have a rotary and my present transformers are rated at 8kv .5a

>I intend to be up to 12kv .5a sometime in the future.

>

>Thanks in advance.

>Chris Watkins

>

Chris,

Sounds very good. 12 inches long is probably way more than

necessary. I would cut them in half for 6 inch long electrodes.

My gap is using 4 inch long 3/4 inch copper pipe and handles

everything Ive thrown at it very well.

For 12kv 8 series gaps (9 electrodes) works well. Space them

.03 inches apart for a total maximum gap of .240 inch. Just

use less than the maximum for 9kv, perhaps 6 of the gaps for

.180 inch.

>also has anyone ever seen heat sinks that go inside a .875id tube?

Probably unnecessary. The almost 1 inch diameter X 6 inch long

electrodes will sink plenty of heat. With adequate airflow it

will handle plenty of power.

Consider making this a vacuum gap with a strong vacuum cleaner motor

or shop vac sucking air through the gap. This gives the best

performance.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: pri/sec freqs

Cc:

Bcc:

X-Attachments:

At 02:33 PM 1/3/98 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sat, 3 Jan 1998 14:33:44 +0000

>Subject: pri/sec freq.s

>

>

>Now I'm really confused.

>

>I did all the calculations myself and came up with the res. freq. of

>my sec' as 245.27KHz, and my primary as 248.68KHz.  I checked them

>against the numbers produced using Teslac and they come out right.

>(using the length of the primary at 15", and the mf of the sec' of

>.112<--the value of the 8" spherical topload).

>

>These numbers look pretty darn close, to me.   But I think I'm doing

>something wrong here.  The program (teslac) states that using the

>11.2pf topload will drop the resonant freq. to 175.??KHz.  And if

>that's the case, I'm guessing I shouldn't be using that 11.2pf value

>in determining the resonant freq of the sec'. (duhh:)

>

>So I guess my questions are. . .what value for capacitance should

>I be using to determine the resonant freq of the sec', (or is it a

>different formula alltogether from the one used to determine the

>tank circuit freq?)and how do I determine the resulting freq of the

>sec' with a particular topload?  And am I shooting for a tank

>circuit resonant frequency that matches the resonant freq of

>the sec' with or without the topload?

>

>Thanks All,  Chris

>

>

Chris,

The value of capacitance you use for your secondary resonant

frequency is the self capacitance of the coil PLUS topload

capacitance.

You want your primary resonant frequency to match the secondary

frequency WITH topload. This is why you need extra primary turns.

As you add larger and larger toroids your secondary resonant frequency

drops. You need to add more primary turns to bring your primary

frequency to this new lower frequency.

Plus there isnt really a good way to estimate topload capacitance.

The actual real value will sometimes be considerably different

than the formulas and programs predict. So having a couple extra

primary turns insures that you have enough tuning range to reach

resonance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Some Coil Problems

Date: Mon,  5 Jan 1998 10:50:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Harvey" <jeff@techgallery.com>

Date: Mon, 05 Jan 1998 10:50:52 +0000

Subject: Some Coil Problems

Organization: Life & Energy Systems

Hello Fellow Coilers: I'm a first timer looking for some help. I have

put up a webpage describing the specs and problem, if anyone would take

a look and offer some help or suggestions...fantastic! The movie is

flattened but 4.5megs.

http://www.intellimg.com/tesla

I'll try to reduce the size of the movie today to make it faster to

download. TIA Jeff

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: High quality non messy poly plate caps

Date: Mon,  5 Jan 1998 12:49:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Vince Cothran" <vince@techie.com>

Date: Mon, 5 Jan 1998 07:49:26 -0500

Subject: Re: High quality non messy poly plate caps

>For those who are interested I will provide

>instructions for making inexpensive poly plate type

>caps. Using my method you can make a cap in about 2 1/2

>hours (.026 uf). These caps look very prefessional.

>They are extreamly efficant and almost bullet proof !

>I do not wish to beat the cap thing to death and will

>not post any more information unless requested by

>individuals.

Sounds like it would be a useful contribution to the list and info that

everyone would appreciate you sharing.  Tell us more, Paul!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: High quality non messy poly plate caps

Date: Sat,  3 Jan 1998 03:31:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Fireworkar <Fireworkar@aol.com>

Date: Fri, 2 Jan 1998 22:31:58 EST

Subject: Re: High quality non messy poly plate caps

Organization: AOL (http://www.aol.com)

I would love to see the instructions

Erick Allen

E-mail at Fireworkar@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Plate Cap Construction (yet another version)

Date: Mon,  5 Jan 1998 16:13:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 5 Jan 1998 08:13:03 -0800 (PST)

Subject: Plate Cap Construction (yet another version)

I just finished my second plate cap, and this time, it looks like

it's not going to leak, so I thought I'd post the construction

technique I used for the enclosure - as I think anybody can stack

poly and aluminum :)

Enclosure in my case was constructed of 3/8" polycarbonate - as that

is what I had handy [we have lots of this as scrap]. You can use Plexi

as well, and probably cut down on costs.

I used Weld-On #16 glue to glue the 4 sides together. This is a

thickened form of Methylene Chloride, and fills the small gaps that

occur when you don't have machined plastic to glue together.

After this had set, I cut a bottom plate, to the outside dimensions. My

thinking was that by lifting from the bottom, a full coverage bottom

will not stress the joints trying to support the weight of the oil

and core. This was also glued in place with #16.

After this has set, a small bead of #16 was run down the inside of each

seam. I was looking for addtional bonding if needed, as well as a

better seal.

Next came the top, and the source of my leaks in  my last cap. To solve

this problem, I changed from a cork gasket to an O-ring. I drilled and

tapped for 6/32 screws, one in each corner, and then on 1" centers

around the perimiter (far outside edge). I then used a ball cutter

on a dremel tool to cut a 1/16" deep semi-circular channel along the

inside edge of box. Into this channel went a custom o-ring.  The screws

are started in the corners, and the o-ring seated in the channel. Then

the remaining screws are placed, using a cross pattern to torque them

down until the ring is compressed. When I tested this, I had not

drilled the holes for the other connections yet, but had filled the

enclosure with water. Tightening things placed a slight positive

pressure on the case, and uncovered a small leak in one of the seams. I

fixed this by sealing all seams with RTV.

Holes were

drilled and tapped into the top for:

Attachment terminals: I used 1" 1/4-20 set screws to pass through,

winding them with teflon tape to seal the threads. This allows me to

use nuts on both sides, so the top can be detached from the core if

needed.

Vent Hole: I use a 1/4"-20 set screw to seal this opening when the cap

is not in use. This is the only place I get seepage right now, as I

don't seal the threads.

This gives me a 0.007uf Cap, Dimensions are :8"W x 7"D x 11"H.

The core is actually 3 0.021uf caps in series, using 6 mil sheets of

poly, with a total thickness (in series) of 90mil. It should be damn

near bullet proof at 15KV, and I am only using 12KV.

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Continous improvement

Date: Mon,  5 Jan 1998 16:14:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 05 Jan 1998 10:14:57 -0600

Subject: Continous improvement

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

It seems as if there are always some improvements to

be made once you have built a coil. This weekend I

changed the taps on the pig from 120v to 240v. This

is done by placing the two primary coils in series. I

Connected the primary to the 240v variac. I also took

all of my caps and placed them in better containers.

I beefed up the terminals internally and externally.

I plan to replace all of the stranded wire in the tank

circuit with 10 AWG solid wire. After doing this I'll

Ballast the system using 2 microwave transformers by

shorting the secondaries. Next on the agenda is to

replace the spark gap. Its just too small. I added a

capacitor to the tank circuit now I'm running a full

.026uf. The streamers are at least 48" long with

occasional reaches of 50" to 52". At this distance I

believe I'm Max'ed out.

I guess I'll have to go to that 6" coil next. Oh by

the way I'm using a 7" diameter dryer duct now.

Take good care & hope you had a happy holiday.

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New Tranny questions

Date: Mon,  5 Jan 1998 17:45:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Mon, 05 Jan 1998 11:45:46 -0600

Subject: Re: New Tranny questions

Tesla-2 wrote:

>

> From: rpittman@juno.com (Ronnie R Pittman)

> Date: Sun, 28 Dec 1997 12:48:18 -0500

> Subject: New Tranny questions

>

> Hey Mike and all,

> Well I am finally coming out of my tesla dormancy :).  For christmas this

> year,  santa claus (A.K.A. my wife)  bought me a used 15 kv 30 ma

> transformer.

Ronnie:

My question is this: Where can I get a wife like yours? If anyone can

help, let me know. My model didn't come with the transformer

awareness/interest option installed. When I say 'Tesla coil' I get a get

a funny look and a message about a lost or missing command interpreter.

Has anyone else had this problem? ;-)

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil forms

Date: Mon,  5 Jan 1998 18:37:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 05 Jan 1998 12:37:53 -0600

Subject: Coil forms

Organization: MailExcite  (http://www.mailexcite.com)

Fellow coilers,

I'm about to begin a new coil. I have two lenghts

of PVC tubing, one is thick wall and the other thin.

The thick walled stuff has a wall thickness of 1/2 "

the thin wall is about 3/16" which is best ? the thick

or the thin wall ? The outside diameters are about

6 5/8".

Please advise

Thanks,

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Some Coil Problems

Date: Mon,  5 Jan 1998 18:56:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 5 Jan 1998 10:56:53 -0800 (PST)

Subject: Re: Some Coil Problems

Jeff: I am wagging this, as I am still new, but I'll hazard a guess

that you are forcing your coil at a harmonic of the natural resonance.

I cannot tell what your cap is really in terms of uf. Doing some

guestimates (since you did not spec it) on your primary, of using

3/8" spacing between turns, I calc that you'd need a cap of 0.002uf

at 14 turns.  This is *tiny*.  I'd guess you have much more than that

in your caps right now. At 7 turns, the cap needs to be 0.010.

You may have tried this already but: Run at low power (if you can),

and set the tap at 7 turns. then move in or out 1 turn and see

which way increases spark output. Continue in that direction

by 1/2 turn increments.

Like I said, lots of guestimates, sorry.

On  5 Jan, Tesla-2 wrote:

>

> Hello Fellow Coilers: I'm a first timer looking for some help. I have

> put up a webpage describing the specs and problem, if anyone would take

> a look and offer some help or suggestions...fantastic! The movie is

> flattened but 4.5megs.

> http://www.intellimg.com/tesla

> I'll try to reduce the size of the movie today to make it faster to

> download. TIA Jeff

>

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil forms

Cc:

Bcc:

X-Attachments:

At 06:37 PM 1/5/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Mon, 05 Jan 1998 12:37:53 -0600

>Subject: Coil forms

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Fellow coilers,

>     I'm about to begin a new coil. I have two lengths

>of PVC tubing, one is thick wall and the other thin.

>The thick walled stuff has a wall thickness of 1/2 "

>the thin wall is about 3/16" which is best ? the thick

>or the thin wall ? The outside diameters are about

>6 5/8".

>

>Please advise

>

>Thanks,

>

>Paul Marshall

>---

Paul,

Go with the thinner wall pipe. Since PVC is so lossy

less lossy material is better.

Be sure to throughly dry and seal the PVC before winding.

Since a pipe that size isnt likely to fit in your oven

use a heat gun. Get it nice and toasty and keep it very warm

for 45 minutes to an hour. Dont get it too hot or it will

deform on you. After drying apply polyurethane varnish while

still warm. As the pipe cools it will draw the varnish into the

pores and seal it quite effectively. Apply one coat inside

the pipe and two coats on the outside.

Drying and sealing your PVC form this way will decrease

losses and give you a few extra inches of spark for free.

And your coil will be less affected by weather changes.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Some Coil Problems

Cc:

Bcc:

X-Attachments:

At 10:50 AM 1/5/98 GMT, you wrote:

>From: "Jeff Harvey" <jeff@techgallery.com>

>Date: Mon, 05 Jan 1998 10:50:52 +0000

>Subject: Some Coil Problems

>Organization: Life & Energy Systems

>

>Hello Fellow Coilers: I'm a first timer looking for some help. I have

>put up a webpage describing the specs and problem, if anyone would take

>a look and offer some help or suggestions...fantastic! The movie is

>flattened but 4.5megs.

>http://www.intellimg.com/tesla

>I'll try to reduce the size of the movie today to make it faster to

>download. TIA Jeff

>

>

Jeff,

I didnt download the movie due to its size and being in

quicktime format. But a few comments on the info you provided.

>Specs:

>

>12K/60 ma. Neon

>Secondary: 3"X20" #24

>Top load 4"X26" Torroid

>Primary: 6" ID 14 turns of 3/8 cu tubing Flat Spiral

>Caps: (1) flat glass plate 10"X 96"X1/4" and (1) 8 poly/plate 10"X 8"X1/8" in

>mineral oil

>Gap: modified RQ gap 6- 4"X1" set at .025 fan quenched

>Why is the discharge happening in the middle of the secondary?

Your secondary and primary look fine but that toroid is very

large for your secondary size. I would bet that your secondary resonant

frequency is too low for your primary circuit to tune.

You are probably tuning to a harmonic of your secondary frequency

and the 1/4 wave resonance is therefore occuring in the middle

of the secondary rather than at the top.

I couldnt figure what your primary capacitance is from your

specs but you want to be in the range of .01 - .0133 mfd

for a 12kv/60ma neon.

First thing I would do is remove the large toroid and replace it

with a small spherical terminal. Use something like a copper

toilet tank float or a 5 inch styrofoam ball covered with foil.

This will raise your secondary frequency considerably and may

put it within range on your primary circuit.

Also lose the glass plate cap. Build a poly cap appropriate

for your neon.

Your gap could use more electrodes. With 6 electrodes spaced

at .025 inch you have only 5 gaps with a total of .125 inch.

This is a little low for 12kv. Expand your gap to 9 electrodes (8 gaps)

spaced at .025 for a total of .200 inch. This will greatly increase

your primary energy level. But dont do this until your coil is in tune.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Some Coil Problems

Date: Mon,  5 Jan 1998 22:50:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Mon, 5 Jan 98 17:50:35 -0500

Subject: Re: Some Coil Problems

>Hello Fellow Coilers: I'm a first timer looking for some help. I have

>put up a webpage describing the specs and problem, if anyone would take

>a look and offer some help or suggestions...fantastic! The movie is

>flattened but 4.5megs.

>http://www.intellimg.com/tesla

>I'll try to reduce the size of the movie today to make it faster to

>download.

Your coil is not tuned at the fundamental frequency of the secondary.

Your large toroid is bringin' down the resonant freq of your secondary,

and your tank is most likely driving the coil at the 2nd harmonic (for

example: if your secondary's self resonance is at 150kHz, then you have

tuned the primary to 300kHz).

You can correct this problem by using as much tank capacitance as your

transformer can charge, and then adding primary inductance if you are

still not there yet.  Doubling both the cap and the inductance will half

the frequency, as will quadrupling either quantity and leaving the other

alone.  Of course, there are also infinite combinations between these

extremes.  Also, using less toroid will raise the secondary freq, if

adjustment is still necessary (last resort).

This brings up an important point:  notably missing from the specs on

your webpage is the total value of your tank cap.  You should have a

ballpark idea of what it is, right?  If not, could you post more detailed

specs for the caps? (Plate overlap area, number of plates, thickness of

the dielectric, etc...)  From that data, we can calculate a very good

estimate and give you much more specific advice.

You should also consider using all polyethylene capacitors, no glass.

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 8 inch coil

Date: Tue,  6 Jan 1998 00:33:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Mon, 05 Jan 1998 18:33:49 -0600

Subject: 8 inch coil

Organization: MailExcite  (http://www.mailexcite.com)

Well, I'm planning on building a coil with an 8 inch diameter. Before I start,

though, I have a few questions.

1.) what ratio of height to diameter should I use?

2.) what frequency would this be unloaded, and how many primary turns should I

begin with (while unloaded)?

3.) What size cap. should I use?

Thanks a bunch! I'll let you all know how it turns out!

--Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: sw caps

Date: Tue,  6 Jan 1998 02:14:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Mon, 5 Jan 1998 21:14:58 EST

Subject: sw caps

Organization: AOL (http://www.aol.com)

I have a couple of questions on salt water caps. Does the number of layers of

aluminum foil on a dry cap make any difference? I have read a posting by Fr.

Dan saying that putting the bottles in a salt water solution instead of using

aluminum foil eliminates the problem of air gaps between the foil & the glass

- would using a spray glue to adhere the foil solve that problem or cause more

(possible fire potential or loss of conduction)? Also what is the safest way

to fully discharge the caps after use? Does grounding them individually make

the most sense?

Sorry my questions to date are so very basic but I'm trying to re-learn from

the "ground" up.

Thanks for all the help so far, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Good Neon transformer info, esp for beginners.

Date: Tue,  6 Jan 1998 05:52:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Tue, 6 Jan 98 00:52:16 -0500

Subject: Good Neon transformer info, esp for beginners.

For those of you who don't know about it, France has a decent page with a

neon transformer FAQ:

http://www.franceformer.com/html/questions.html

Check it out!

-Adam

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 8 inch coil

Cc:

Bcc:

X-Attachments:

At 12:33 AM 1/6/98 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Mon, 05 Jan 1998 18:33:49 -0600

>Subject: 8 inch coil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Well, I'm planning on building a coil with an 8 inch diameter. Before I start,

>though, I have a few questions.

>

>1.) what ratio of height to diameter should I use?

For a 8 inch secondary the proper H/D ratio is 3 to 1.

If using 8 inch PVC with a 8 1/2 inch OD this means 25.5

inches. Make it 26 inches for a nice even number.

Wound with AWG 22 this comes to around 965 turns

and an unloaded resonant frequency of about 185 KHZ.

A fairly good size toroid can push this down to 130khz

or even less.

This is a very serious coil. Not something to be made

as a first coil. If you have made other coils successfully

then go for it.

>

>2.) what frequency would this be unloaded, and how many primary turns should I

>begin with (while unloaded)?

This size coil will do very well with a flat spiral or shallow saucer

shaped primary. You want plenty of turns in this since this is going to be

a low frequency coil. I would go with a saucer primary with a 15-20 degree

slope and 14 primary turns. This should give plenty of room to tune some

decent size toroids.

>

>3.) What size cap. should I use?

This depends a great deal on your power supply. You will need a fairly

large cap to get the primary resonant frequency low enough so you will

need a large transformer to charge it. I would think in terms of .025 mfd.

This will require around 12kv/120ma worth of neons minimum.

>

>Thanks a bunch! I'll let you all know how it turns out!

Good luck.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: 8 inch coil

Date: Tue,  6 Jan 1998 08:18:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Tue, 6 Jan 98 03:18:24 -0500

Subject: Re: 8 inch coil

>Well, I'm planning on building a coil with an 8 inch diameter. Before I

>start though, I have a few questions.

>

>1.) what ratio of height to diameter should I use?

3:1 is what I used.  Works great.

>2.) what frequency would this be unloaded, and how many primary turns

>should I begin with (while unloaded)?

I figured out that for my 9kV 120mA supply, I needed to make a secondary

that will work with my 0.03uF cap and 30mH flat spiral primary.  I

used 25.125" of #20 (750t) and this gave me a measured freq of 160kHz

with my 6" x 20" toroid.  Perfect match.  Your ideal secondary winding

length and wire gauge will vary depending on how much voltage and current

you plan to supply.  I never gave any regards to the unloaded freq,

'cause mine's never going to run without a huge toroid.  You risk

secondary coil burn damage without the toroid.

>3.) What size cap. should I use?

What size transformer do you have now, and how much might you use on this

coil in the future?

>Thanks a bunch! I'll let you all know how it turns out!

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

From: tesla-2@emachine.com (Tesla-2)

Subject: secondary height limits

Date: Tue,  6 Jan 1998 19:50:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 06 Jan 1998 13:50:50 -0600

Subject: secondary height limits

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Recently I've seen many posts from folk using

different diameter secondaries.  I noticed that the

ideal H/D ratio tends to decrease as D increases.

Perhaps what is really happening is that secondaries

longer than a certain length no longer pick up any

usable magnetic field from the primary.

That is, for a particular primary size and shape, the

magnetic field will tend to be zilch past a certain

height, etc...?

This length would be a function of the primary

dimensions, which are of course rellated to the

secondary diameter, which might explain why the best

secondary height is not an absolute limit.

Perhaps, for a fixed primary size and shape, the

optimal secondary height might be closer to a

constant and much strongly related to the sec.

diameter.

Thoughts?  Example - seems like both a 4 inch and a 6

inch secondary operate best at a length of about 26 inches.  Comments encouraged!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla Secondaries

Date: Wed,  7 Jan 1998 11:30:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Ringwood <ccntesla@yahoo.com>

Date: Wed, 7 Jan 1998 03:30:46 -0800 (PST)

Subject: Tesla Secondaries

To All,

Is it possible to wind a secondary coil from bottom to top, and then

when you get to the top, continue winding back down over the first

layer of windings like on a normal transformer?

If you know could you reply.

Chris Ringwood

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Strange arc gap tests

Date: Wed,  7 Jan 1998 16:02:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 07 Jan 1998 10:02:01 -0600

Subject: Strange arc gap tests

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Last night I constructed a nice variable safety gap

out of a couple of nuts, bolts, and some steel angle

brackets.

I noticed something very odd about the current consumed by the gap.  Here is my

setup:

current-limited microwave oven transformer, provides

about 2Kv.

Variable safety gap

Clip-on inductive ampmeter on the transformer primary

side (110v)

Note that I did not have any caps, coils, or other stuff

attached to this circuit - just the transformer and the

variable gap.

First - I set the gap to about 1/32 inch and started an

arc.  The primary current drawn was measured as

about 8 amps.

Then I turned on a small muffin fan and aimed the air

at the gap.

Here is the odd thing - at certain airflow rates, the

measured primary current *Increased*!  The peak

measured current was >15 amps!

At this point, supplying more air would blow out the

arc.  The arc would stay out until the airflow was reduced.

Why would the current increase as air was applied?

Why would the current ever exceed the current

consumed by an "un-bothered" arc, with no airflow?

Perhaps I'm seeing an artifact of my ammeter?

If anyone has any ideas I'd love to hear them!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Pacemaker / tesla coil

Date: Wed,  7 Jan 1998 16:45:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 07 Jan 1998 10:45:47 -0600

Subject: Pacemaker / tesla coil

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Can anyone tell me what effect if any a tesla coil

can have on a pacemaker ? If there are any can they be

protected by a faraday cage ?

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: one more time

Date: Wed,  7 Jan 1998 17:20:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Wed, 7 Jan 1998 12:20:03 -0500

Subject: one more time

O.K.  Mike you convinced me that the plate caps are "superior" in

pulse performance.   Now I am still a little unsure on how to calculate

the capacitance.

I found the formula    { [ (.224) *  K (which in poly would be around 2)

* A (now is that the area of one plate or is that the sum of the areas of

1/2 the plates or the sum of the area of all the plate) ]  /  D ( is 6

mil .06 inhes or .006) ( is this the distance between any 2 individual

plates or is this the total distance between all the plates? }  *  (N-1)

(Is this simply the total number of plates in the cap?)

Also I noticed that in the design you sent me you made the plates 6" X 6"

to use the 12" foil.  You also used 8 mils per layer with 24 layers and

24 plates to get .048 mfd.

I remember you saying that I needed .0053 mfd for each 15kv 30 ma.  So if

I make the above cap with and wire 9 in series I will get .0053 mfd per

unit.  Then I could build three units and just put another one in

parallel every time I add another transformer?  Am I somewhere close?

The one problem I see is that in a post you sent me you advised me to use

24 mils of poly (6X4)  for each layer of the dielectric.  In the above

design there are only 8 mils per layer

So If I made the plates the same size (6X6)  and made the poly 24 mils /

layer,  how many plates would I need to get the .0053 mfd?

I am really sorry to ask such time consuming questions, but being

a teacher it's a bad habit I have.  I figure once I really understand all

this stuff, I won't have to bug you guys as much.

And as always,  I really really appreciate all the help.

Thanks a whole bunch,

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Strange arc gap tests

Date: Wed,  7 Jan 1998 17:36:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 07 Jan 1998 11:36:57 -0600

Subject: Re: Strange arc gap tests

Organization: MailExcite  (http://www.mailexcite.com)

Bill,

It could just be that hot ionized air is less dense

than cold air, consequently more resistive. I have

heard that when power is supplied to a neon sign the

inital current flow is much greater than the power

required to keep the gas in an ionized state.

Paul The Non-engineer Engineer !

---

Einigkeit und Freiheit

>From: arcstarter@mailexcite.com

>Date: Wed, 07 Jan 1998 10:02:01 -0600

>Subject: Strange arc gap tests

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>Last night I constructed a nice variable safety gap

>out of a couple of nuts, bolts, and some steel angle

>brackets.

>

>I noticed something very odd about the current consumed by the gap.  Here is my

> setup:

>

>

>current-limited microwave oven transformer, provides

>about 2Kv.

>

>Variable safety gap

>

>Clip-on inductive ampmeter on the transformer primary

>side (110v)

>

>Note that I did not have any caps, coils, or other stuff

>attached to this circuit - just the transformer and the

>variable gap.

>

>First - I set the gap to about 1/32 inch and started an

>arc.  The primary current drawn was measured as

>about 8 amps.

>

>Then I turned on a small muffin fan and aimed the air

>at the gap.

>

>Here is the odd thing - at certain airflow rates, the

>measured primary current *Increased*!  The peak

>measured current was >15 amps!

>

>At this point, supplying more air would blow out the

>arc.  The arc would stay out until the airflow was

>reduced.

>

>Why would the current increase as air was applied?

>

>Why would the current ever exceed the current

>consumed by an "un-bothered" arc, with no airflow?

>

>Perhaps I'm seeing an artifact of my ammeter?

>

>If anyone has any ideas I'd love to hear them!

>

>-Bill the arcstarter

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Strange Phenomon /  Tesla gap

Date: Wed,  7 Jan 1998 18:32:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 07 Jan 1998 12:32:31 -0600

Subject: Strange Phenomon /  Tesla gap

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I just completed the rewiring of my tank circuit.

One of the wires connecting two caps in parallel

passed over the spark gap,(RQ style 9 gaps). The wire

ran perpendicular to the gap electrodes, and was

approximatly .5 - .7" from the gap terminals. When I

brought the power up and fired the coil (max. power)

a huge arc jumped from terminal 1 to 9 a

circumferential distance of about 2.5 ". It was very

bright and extreamly loud. This occurance did not

effect the performance and the safety gap did not fire.

I repeated this and the same exact thing occured. I

decided to move the tank circuit wire away from the

gap. The arcs seem to go away and there were no further

ocurrances. Does anyone have a theory as to what may

have occured ?

I do not think that I have ever seen an arc this bright

before. It was almost blinding ! And the sound

incredible.

Paul Marshall

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: one more time

Date: Wed,  7 Jan 1998 18:50:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Wed, 7 Jan 1998 10:50:40 -0800 (PST)

Subject: Re: one more time

On  7 Jan, Tesla-2 wrote:

> From: rpittman@juno.com

> Date: Wed, 7 Jan 1998 12:10:02 -0500

> Subject: one more time

>

>

> I found the formula    { [ (.224) *  K (which in poly would be around 2)

> * A (now is that the area of one plate or is that the sum of the areas of

> 1/2 the plates or the sum of the area of all the plate) ]  /  D ( is 6

> mil .06 inhes or .006) ( is this the distance between any 2 individual

> plates or is this the total distance between all the plates? }  *  (N-1)

> (Is this simply the total number of plates in the cap?)

The formula is [(0.222 * k * A)/D) *(N-1)]

where

K is the dielectric constant (and yes, for LDPE 2 works great.)

A is the area, in sq. inches, of plate overlap [best way to describe it.]

D is the distance between plates, again in inches.

N is the number of parallel plates in your cap. [Note that a rolled cap

is a special case, as you get 2 for one, so to speak]

>

>

> I remember you saying that I needed .0053 mfd for each 15kv 30 ma.  So if

> I make the above cap with and wire 9 in series I will get .0053 mfd per

> unit.  Then I could build three units and just put another one in

> parallel every time I add another transformer?  Am I somewhere close?

Right on the mark

> The one problem I see is that in a post you sent me you advised me to use

> 24 mils of poly (6X4)  for each layer of the dielectric.  In the above

> design there are only 8 mils per layer

The idea is that the total mils is what counts. Since you are putting

9 in series, you will get a total of 72 mils of dielectric. Each cap,

if close to identical will see V/n volts, where V is your input voltage

and n is the number of series caps. So each of your nine units will see

about 15/9 KV (rms) or about 1.7KV (rms), which 8mil should handle.

Personally, I am a bit more paranoid, and I aim for 90mil total to

handle 15KV (rms)

You notice I keep saying rms?  This is important (forgive me if you

already knew this) but the peak voltage from your neon will be much

more than 15KV, I forget exactly how much, I think on the order of 1.5?

Never mind the resonant rise with your transformer...

>So If I made the plates the same size (6X6)  and made the poly 24 mils /

> layer,  how many plates would I need to get the .0053 mfd?

Assume you keep all constant, 36sq inches plate, and use 24 mils, you get:

0.0155 uf for 24 plates, 3 in series would give you .0052

0.0168 uf for 26 plates, 3 in series would give you .0056

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Pros & Cons of HV power supplies

Date: Wed,  7 Jan 1998 21:23:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: crowekri <crowekri@scott.net>

Date: Wed, 07 Jan 1998 13:23:53 -0800

Subject: Pros & Cons of HV power supplies

Organization: home

Hi all,

I am still in the research stage of building my first Tesla coil

which I intend to make as small and inexpensive as possible learning all

I can in the process which can later be applied to the construction of

larger more powerful coils.  My question what is the lowest voltage

supply available that can still be used to construct a demonstrable

Tesla coil.  I have seen many different types of HV power supplies used

to power a Tesla coil such as neon transformers, T.V. flybacks, and car

coils. Anyone care to share the good and bad characteristics of these

other HV power supplies?

All information is appreciated,

Shane

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Strange arc gap tests

Date: Thu,  8 Jan 1998 00:12:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred C. Erpel" <aerpel@Op.Net>

Date: Wed, 7 Jan 1998 19:12:36 -0500

Subject: Re: Strange arc gap tests

>From: arcstarter@mailexcite.com

>Date: Wed, 07 Jan 1998 10:02:01 -0600

>Subject: Strange arc gap tests

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>Last night I constructed a nice variable safety gap

>out of a couple of nuts, bolts, and some steel angle brackets.

>

>I noticed something very odd about the current consumed by the gap.

> Here is my setup:

>

>

>current-limited microwave oven transformer, provides

>about 2Kv.

>

>Variable safety gap

>

>Clip-on inductive ampmeter on the transformer primary

>side (110v)

>

>Note that I did not have any caps, coils, or other stuff

>attached to this circuit - just the transformer and the

>variable gap.

>

>First - I set the gap to about 1/32 inch and started an

>arc.  The primary current drawn was measured as

>about 8 amps.

>

>Then I turned on a small muffin fan and aimed the air

>at the gap.

>

>Here is the odd thing - at certain airflow rates, the

>measured primary current *Increased*!  The peak

>measured current was >15 amps!

>

>At this point, supplying more air would blow out the

>arc.  The arc would stay out until the airflow was reduced.

>

>Why would the current increase as air was applied?

>

>Why would the current ever exceed the current

>consumed by an "un-bothered" arc, with no airflow?

>

>Perhaps I'm seeing an artifact of my ammeter?

>

>If anyone has any ideas I'd love to hear them!

>

>-Bill the arcstarter

Bill,

OK. Here is an idea. The secondary of your transformer has its own

resonant frequency, and you had the spark breaking at this frequency or a

harmonic thereof. Would this do it?

Regards,

Alfred Erpel

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Strange arc gap tests

Date: Thu,  8 Jan 1998 01:18:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>current-limited microwave oven transformer, provides

T>about 2Kv.

That's the key, right there; the current limit.

T>First - I set the gap to about 1/32 inch and started an

T>arc.  The primary current drawn was measured as

T>about 8 amps.

T>Then I turned on a small muffin fan and aimed the air

T>at the gap.

T>Here is the odd thing - at certain airflow rates, the

T>measured primary current *Increased*!  The peak

T>measured current was >15 amps!

T>Why would the current increase as air was

T>applied?

Since the transformer is current limited, the actual wattage passed

through the transformer will depend on the voltage the transformer is

able to supply to the load (arc).

When the arc length is short, the resistance of the arc is low.  The

transformer, being current limited, cannot supply very much voltage -

maybe 200 volts across the arc - at the secondary current limit of about

3 amps.  That equates to about 600 watts of power, which is reasonable

for an 8 amp primary current draw, taking the inductive reactance of the

transformer into account.

As the air blast lengthens the arc, the resistance of the arc

increases, and the transformer must supply more voltage to maintain the

3 amp secondary current.  I'd guess the voltage across the arc is about

480 or so just before the fan blows it out.  That voltage will give you

about 1,440 watts, hence, the transformer will pull quite a bit more

current from the AC line with a longer arc.

Ralph   W5JGV

---

Sure there was a "Big Bang"!  But WHO lit the fuse?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: nemesis

Date: Thu,  8 Jan 1998 01:23:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Wed, 07 Jan 1998 19:23:54 -0600

Subject: nemesis

It was a great coil that unfortunately got dismantled.

The parts went there various ways, Wysock has its

primary I heard... does anyone know what happened

to some of its other parts....?

...and what power levels was he running?

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: sw caps

Date: Thu,  8 Jan 1998 01:24:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Wed, 07 Jan 1998 19:24:04 -0600

Subject: Re: sw caps

>I have a couple of questions on salt water caps. Does the number of layers of

>aluminum foil on a dry cap make any difference? I have read a posting by Fr.

Number of layers has no difference.  All that affects the SW caps is how close

the alluminum is to the glass.

>Dan saying that putting the bottles in a salt water solution instead of using

>aluminum foil eliminates the problem of air gaps between the foil & the glass

The salt water bath DOES work MUCH better.  I got twice the output when

I switched from aluminum to SW.  Unfortuantly, it is MESSY and heavy.

>- would using a spray glue to adhere the foil solve that problem or cause more

>(possible fire potential or loss of conduction)? Also what is the safest way

Using a glue of some sort would have the same effect as having air under the

foil.  ANYTHING between the foil and glass lessens performance, air included.

That is why SW baths on the outside work so well, the water completely

touches all outside area of the glass.

>to fully discharge the caps after use? Does grounding them individually make

>the most sense?

SW caps from my experience do not even need to be discharged.  They are so

lossy that within one second after cutting the power, they discharge through

the neon transformer anyway.  I myself have NEVER shorted the caps to discharge

them.  After a 10 minute run, a cap wire of mine came off.  I flipped the

coil off, reattached the wire, and turned it on.  No shock.

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: tesla pics

Date: Thu,  8 Jan 1998 01:24:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Wed, 07 Jan 1998 19:24:40 -0600

Subject: tesla pics

For those of you interested, I put 10 pictures of my coil setup on my

high voltage page.  The URL for the page is :

http://www.harvestcomm.net/personal/corr/hv/index.htm

I also completed a step-by-step walkthrough "guide" to build your

first coil.  It documents what I did, and what I learned you should

not do in it as well.  It also includes pictures showing various parts

of the construction, and a program for calculating the values of your

Tesla coil.  You can download this at the above address.  Thanks.

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ID for Primarys

Date: Thu,  8 Jan 1998 01:24:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Wed, 07 Jan 1998 19:24:57 -0600

Subject: Re: ID for Primarys

>        After browsing the internet and reading a few archives I have seen

>quite a bit of difference for recommended diameter for primarys. I

>orignally thought that you should use 2 to 3 times the diameter of the

>secondary for the ID of the Primary, but I have seen people use a 6"

>diameter for a 4" coil, and a 8" one for a 6" coil. I am trying to

>construct a 3" coil with a inverse cone-type primary (at 30 degrees). Does

>the type of primary make a difference on its diameter ? What would you

>recommend using? Thanks for your help,

Actually a cone primary isn't really nessecary if your secondary is only

going to be 3" in diamter.  I would recommend a helical (vertical) primary

for a coil of that size.  Big coils use flat primaries, smaller coils use

vertical

primaries, and mid-sized use inclined.  Thats just a general rule of thumb

I have heard... :)

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: poly caps

Date: Thu,  8 Jan 1998 01:26:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Wed, 07 Jan 1998 19:26:03 -0600

Subject: poly caps

Well I completed my first successful poly cap!  Thanks for all who

have helped me with this and my MANY other questions!

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Strange Phenomon /  Tesla gap

Date: Thu,  8 Jan 1998 12:43:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 8 Jan 1998 07:43:03 EST

Subject: Re: Strange Phenomon /  Tesla gap

Organization: AOL (http://www.aol.com)

Paul,

It sounds like the wire connecting your caps has some corona loss.  This

probably created an ionized air pathway for the current to follow that had

less resistance than the spark gap itself.  Once you moved it away from the

gap, the path reverted back to the intended path across the gap.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Pacemaker / tesla coil

Date: Thu,  8 Jan 1998 12:54:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 8 Jan 1998 07:54:07 EST

Subject: Re: Pacemaker / tesla coil

Organization: AOL (http://www.aol.com)

Paul,

I have heard that the electormagnetic field generated by a Tesla coil is

very bad for a pacemaker.  While I have not confrimed this personally,  I

would not take a chance like that.  A properly built and grounded Faraday cage

should provide sufficient protection from this field.  Years ago, I fired a

small coil(100kV output) while carrying a digital stopwatch in my jacket

pocket.  It stopped dead in its tracks!  I removed the battery and repaced it

and the stopwatch resumed normal functioning, so I was lucky.   It would be

hard to do this with a pacemaker.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary height limits

Date: Thu,  8 Jan 1998 13:13:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 8 Jan 1998 08:13:56 EST

Subject: Re: secondary height limits

Organization: AOL (http://www.aol.com)

Bill,

I picked up a very old book titled "Electrical Engineer's Handbook -

Electric Communications and Electronics" by Pender and McIlwain.   In this

book it states that the maximum inductance for a single layer coil with a

given pitch and length of wire occurs when the diameter of the coil is

approximately 2.46 times the height of the coil(mathematically, the Wheeler

equation shows that ratio to be 2.22 - fairly good agreement).  This just

means that the Q of the coil is maximized with this geometry.  Unfortunately,

this would be a very short, fat coil that would have a high voltage stress

between adjacent windings.  A compromise must be made for Tesla coils to

handle this stress.  The taller coil also allows for a better separation

between the secondary terminal and the primary coil without the need for a

mast or pipe to conduct the secondary output up to the terminal.   If you look

at pictures of Tesla's coils,  you will see that most of his coils were made

very short and fat.  The height of his coils might go as much as twice the

diameter.  He dealt with the voltage stress between windings by spacing the

wires sufficiantly(which also reduces the self capacitance).

One concept that I have heard many times is that Tesla coils function

best when the secondary is limited to about six hundred turns or fewer.  One

source even suggested that the limit should be four hundred turns.  The

reasoning behind this is that the resonant rise in the secondary voltage, not

the ratio of the secondary to primary turns is what provides the high voltage

output.  The resonant rise in the secondary voltage is due to the addition of

the amplitudes of the initial and reflected waves(which only occurs when the

system is properly tuned).  I know that many coilers use secondaries with as

many as a thousand turns and get good results, but their results might be even

better using a larger diameter with fewer turns.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla Secondaries

Date: Thu,  8 Jan 1998 13:30:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 8 Jan 1998 08:30:43 EST

Subject: Re: Tesla Secondaries

Organization: AOL (http://www.aol.com)

Chris,

Layering your secondary as you suggest would not be advisable.  This would

cause a very large voltage difference at the base of the coil(and probably

other places along the coil). The Tesla coil operates by creating a standing

wave along the coil with a high voltage, low current node at the top and a

low(zero?) voltage, high current node at the base.  How would you prevent

arcing between the layers?  This would also increase the self capacitance of

the coil which would be detrimental to the operation of the coil.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tungsten electroplating?

Date: Thu,  8 Jan 1998 16:20:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 08 Jan 1998 10:20:16 -0600

Subject: Tungsten electroplating?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Anyone know if it is possible to electroplate tungsten

onto copper?  Perhaps this would increase the

runtime between spark gap cleanings?

I suppose it boils down to whether tungsten

compounds such as tungsten sulfate (etc) are stable,

water-soluable ionic compounds.

It'd be neat if I could just dissolve some TIG

electrodes in HCl or sulfuric to make these salts,

then perform a traditional electroplating operation.

I have a 1 lb chunk of tungsten out of an old

X-ray tube which I'd be willing to sacrifice for the

cause...

But - I'm not a chemist by any stretch of the

imagination.

Comments?  I suppose it'd be nice to know if

tungsten compounds are very toxic as well...

-Bill the arcstarter

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: more fuzzy cap ?'s

Date: Thu,  8 Jan 1998 17:07:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Thu, 8 Jan 1998 12:07:50 -0500

Subject: more fuzzy cap ?'s

O.K.  Mike you convinced me that the plate caps are "superior" in

pulse performance.   Now I am still a little unsure on how to calculate

the capacitance.

I found the formula    { [ (.224) *  K (which in poly would be around 2)

* A (now is that the area of one plate or is that the sum of the areas of

1/2 the plates or the sum of the area of all the plate) ]  /  D ( is 6

mil .06 inhes or .006) ( is this the distance between any 2 individual

plates or is this the total distance between all the plates? }  *  (N-1)

(Is this simply the total number of plates in the cap?)

Also I noticed that in the design you sent me you made the plates 6" X 6"

to use the 12" foil.  You also used 8 mils per layer with 24 layers and

24 plates to get .048 mfd.

I remember you saying that I needed .0053 mfd for each 15kv 30 ma.  So if

I make the above cap with and wire 9 in series I will get .0053 mfd per

unit.  Then I could build three units and just put another one in

parallel every time I add another transformer?  Am I somewhere close?

The one problem I see is that in a post you sent me you advised me to use

24 mils of poly (6X4)  for each layer of the dielectric.  In the above

design there are only 8 mils per layer

So If I made the plates the same size (6X6)  and made the poly 24 mils /

layer,  how many plates would I need to get the .0053 mfd?

I am really sorry to ask such time consuming questions, but being

a teacher it's a bad habit I have.  I figure once I really understand all

this stuff, I won't have to bug you guys as much.

And as always,  I really really appreciate all the help.

Thanks a whole bunch,

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:  Strange arc gap tests

Date: Thu,  8 Jan 1998 19:07:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 08 Jan 1998 13:07:36 -0600

Subject: Re:  Strange arc gap tests

Organization: MailExcite  (http://www.mailexcite.com)

Alfred wrote:

>From: "Alfred C. Erpel"

...

>    OK. Here is an idea. The secondary of your transformer has its own

>resonant frequency, and you had the spark breaking at this frequency or a

>harmonic thereof. Would this do it?

This gap is a simple non-rotating safety gap.

I can't see how that would change any resonant

frequency.  There are no capacitors, etc involved.

The gap should fire every 1/120 sec, etc.

-Bill

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Odd spark gap measurements

Date: Thu,  8 Jan 1998 19:37:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 08 Jan 1998 13:37:01 -0600

Subject: Odd spark gap measurements

Organization: MailExcite  (http://www.mailexcite.com)

Ralph responded:

>   As the air blast lengthens the arc, the resistance of the arc

>increases, and the transformer must supply more voltage to maintain the

>3 amp secondary current.  I'd guess the voltage across the arc is about

>480 or so just before the fan blows it out.  That voltage will give you

>about 1,440 watts, hence, the transformer will pull quite a bit more

>current from the AC line with a longer arc.

>

>   Ralph   W5JGV

Yes - this seems quite reasonable.  Sort of a peak

power optimization or impedence matching process.

Thanks for your help!

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: more fuzzy cap ?'s

Date: Thu,  8 Jan 1998 22:10:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Thu, 08 Jan 1998 22:10:15

Subject: Re: more fuzzy cap ?'s

Ronnie,

To get the value you need you need to make 6 .0318uF caps with 24

mils of poly. Here's the plans. .024 inches = 24 mils. 24 mils of poly

6" X 6" plates 42 plates. Series 6 of these and it will .0053uF. you

could also make caps with 12" X 12" plates. 12 layers for those with 24 mils

of poly and series 6 of them. I would recamend the 6" X 6" plates

though as it will spread the voltage over more plates. Happy coiling.

Shaun

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Huge Cap

Date: Thu,  8 Jan 1998 22:14:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Thu, 08 Jan 1998 22:14:25

Subject: Re: Huge Cap

Most likely this cap is to low of voltage for tesla use or it's a D.C.

cap. If it isn't and you can use it your transformer will never be able

to charge the thing up unless it can put a lot of current.

May the arcs be with you.

Shaun

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Huge Cap

Date: Thu,  8 Jan 1998 22:48:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: torghn@juno.com (Jared Neilsen P Neilsen)

Date: Thu, 8 Jan 1998 15:48:03 -0700

Subject: Huge Cap

Can you have a Capacitor that is too big for your tesla coil?  My physics

teacher has a 220000 UF  capacitor that I could probably use.  This

capacitor MUCH larger than the capacitors used in most coils and I was

wondering what effect it would have on a coil.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Huge Cap

Cc:

Bcc:

X-Attachments:

At 10:48 PM 1/8/98 GMT, you wrote:

>From: torghn@juno.com (Jared Neilsen P Neilsen)

>Date: Thu, 8 Jan 1998 15:48:03 -0700

>Subject: Huge Cap

>

>Can you have a Capacitor that is too big for your tesla coil?

Yep. You can only use a capacitor as large as your transformer

is able to charge up. This is generally in the range of .005-.03 mfd.

The size needed depends on your transformer.

>My physics teacher has a 220000 UF  capacitor that I could probably use.

>This capacitor MUCH larger than the capacitors used in most coils and I was

>wondering what effect it would have on a coil.

This is an electrolytic capacitor. This is a DC rated cap and

is not at all suitable for use in a Tesla coil.

You would need a cap rated for use with AC.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: cap ?

Date: Fri,  9 Jan 1998 01:00:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Thu, 8 Jan 1998 20:00:50 -0500

Subject: cap ?

I am sorry if anyone noticed that I accidentaly sent a duplicate

of a post I wrote asking a bunch of questions on capacitors.  What

happened was that the program that I was using to get my mail decided to

freeze up on me right after it was supposed to have finished sending and

getting my messages.

Since I was not able to recieve any messages i figured it did not

send any messages thus the second message.

Anyway if anyone else sent me replies, could you also repost

them.  I got the problem fixed with my software  but lost the last couple

days worth of messages.

Sorry for the malfunction and the inconvenience.

Thanks alot

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tungsten electroplating?

Date: Fri,  9 Jan 1998 01:20:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 8 Jan 1998 20:20:08 EST

Subject: Re: Tungsten electroplating?

Organization: AOL (http://www.aol.com)

Mr. Arcstarter,

I do not know if it is possible(or very easy) to plate Tungsten.  I can

easily look this up for you tomorrow at work.  I work in the Hydrometallurgy

department of a mining and metallurgy research company.  I will let you know

what I find.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla Secondaries

Date: Fri,  9 Jan 1998 01:34:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 08 Jan 1998 17:34:44 -0800

Subject: Re: Tesla Secondaries

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Hollmike <Hollmike@aol.com>

> Date: Thu, 8 Jan 1998 08:30:43 EST

> Subject: Re: Tesla Secondaries

> Organization: AOL (http://www.aol.com)

>

> Chris,

>     Layering your secondary as you suggest would not be advisable.  This woul

d

> cause a very large voltage difference at the base of the coil(and probably

> other places along the coil). The Tesla coil operates by creating a standing

> wave along the coil with a high voltage, low current node at the top and a

> low(zero?) voltage, high current node at the base.  How would you prevent

> arcing between the layers?  This would also increase the self capacitance of

> the coil which would be detrimental to the operation of the coil.

>

>     Mike Hollingsworth

>

Chris,

I concur with Mike. However, some coilers HAVE run with layered coils by

winding a matching layer over the first and connecting them in parallel.

While the approach works quite well for magnifier resonators, it's not

clear that the added effort results in any significant performance gains

for simple two-coil Tesla Coil systems over a single leyer winding of an

equivalent guage...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Huge Cap

Date: Fri,  9 Jan 1998 02:30:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 08 Jan 1998 18:30:17 -0800

Subject: Re: Huge Cap

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: torghn@juno.com (Jared Neilsen P Neilsen)

> Date: Thu, 8 Jan 1998 15:48:03 -0700

> Subject: Huge Cap

>

> Can you have a Capacitor that is too big for your tesla coil?  My physics

> teacher has a 220000 UF  capacitor that I could probably use.  This

> capacitor MUCH larger than the capacitors used in most coils and I was

> wondering what effect it would have on a coil.

>

Jared,

A Tesla coil requires that the secondary and primary resonate at the

same frequency. A 220000 uF primary cap is about 500,000 - 2 million

times larger than the capacitance you want. Also, Tesla Coils require

capacitors that can run at either polarity, since they will be part of

an oscillating tank circuit. The cap that your physics teacher has is

called an electrolytic capacitor. It uses a very thin insulating film of

aluminum or tantalum oxide, and cannot withstand the voltage reversals

that are required for a Tesla Coil tank cap. Hope this helps!

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Streamer / strikes

Date: Fri,  9 Jan 1998 14:43:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 09 Jan 1998 08:43:04 -0600

Subject: Streamer / strikes

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have read several articles and viewed photos of people setting on coils,

touching streamers or allowing the output to travel through themselves, often

discharging through aluminum foil wraped around their finger tips. I understand

that they are not grounded, however it does seem that they would recieve burns

from the streamer discharges.

I have gained a lot of respect for the power output, having seen 4'disharges from

my own coil. Can anyone explain this phenomonen ?

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: heat sinking your gaps

Date: Fri,  9 Jan 1998 17:00:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Fri, 09 Jan 1998 11:00:53 -0600

Subject: heat sinking your gaps

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Someone recently asked how to heat sink your

RQ style gaps more efficiently.  Seems to me if you

could keep the copper tubes sealed off, and filled

with (possibly circulated) distilled or deionized water,

that would do the trick.  The water would both

absorb a substantial amount of heat, as well as distribute

it to other areas of the tubing for transfer into the air.

If the pipe sections were oriented vertically, you

might be able to get away with sealing the bottom

ends of the pipes, and keeping the top end open.

You would have to add water to account for any

evaporation, etc.  The usual airflow (either blower

or vacuum styles) would have to be maintained to

disperse ionized gases from between the gaps,etc

The local hardware store has plumbing reduction

fittings which fit inside the i.d. of std sizes of pipe.

This sort of fitting seems appropriate as it wouldn't

interfere with the parallel copper faces between

adjacent tubes.  Using the usual copper endcaps

would of course cause severe arcing from the

endcap since it fits over the outside of the copper

pipe.

Thoughts? Perhaps by the time you get to this power

level, you'd be better off going with a rotary gap instead?

The local welding store has boxes of copper-plated

rectangular carbon bars, about 1/8 inch thick and

about 5/8 wide.  These are used for air-arc cutting of

heavy steel.  I've wondered if these electrodes,

having large parallel faces, might make a decent

quenched gap?

Of course, what you really need is a bank

of tungsten arc gaps out of an old 1940's induction

heating rig.  These units consisted of 30 tungsten

gaps at 0.004 inch each.  Typically water cooled.

Found in units consuming 10-50KW of input power.

That ought to drive most coils quite nicely!

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

------

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Vacuum pump.

Date: Fri,  9 Jan 1998 17:34:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Erik Hennix" <erik.hennix@swipnet.se>

Date: Fri, 9 Jan 1998 17:34:57 +0000

Subject: Vacuum pump.

I've read that it's possible to convert refrigerator compressors to

vacuum pumps, to pump capacitors with.

Can someone please tell me where I can find info/plans  about how

this is done?

Thanks,

Erik Hennix.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Vacuum pump.

Date: Fri,  9 Jan 1998 17:11:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Fri, 09 Jan 1998 11:11:58 -0600

Subject: Re: Vacuum pump.

Erik:

There is really no conversion necessary. Just remove the compressor from

the rest of the refrigeration equipment and you are ready to go. Most

small compressors are of the rotary vane type and do not create a very

impressive vacuum (my guess would be a few millimeters Hg). They would

be more than adequate for removing air from a rolled cap I would think.

Also, don't expect much in the way of capacity (CFM/LPM). You should

periodically add a FEW DROPS of a light oil to the intake since there is

no oil in the system. Lastly, don't run it for extended lengths of time

like over night, remember there is no oil in the system. Hope this

helps.

Eric

edavidson@icva.gov

Tesla-2 wrote:

>

> From: "Erik Hennix" <erik.hennix@swipnet.se>

> Date: Fri, 9 Jan 1998 17:34:57 +0000

> Subject: Vacuum pump.

>

> I've read that it's possible to convert refrigerator compressors to

> vacuum pumps, to pump capacitors with.

>

> Can someone please tell me where I can find info/plans  about how

> this is done?

>

> Thanks,

>

> Erik Hennix.

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Vacuum pump.

Cc:

Bcc:

X-Attachments:

At 05:34 PM 1/9/98 GMT, you wrote:

>From: "Erik Hennix" <erik.hennix@swipnet.se>

>Date: Fri, 9 Jan 1998 17:34:57 +0000

>Subject: Vacuum pump.

>

>I've read that it's possible to convert refrigerator compressors to

>vacuum pumps, to pump capacitors with.

>

>Can someone please tell me where I can find info/plans  about how

>this is done?

>

>

>Thanks,

>

>

>Erik Hennix.

Erik,

No plans needed. I used the compressor from an old dehumidifier.

All you need do is cut loose the old compressor making note

of the AC wiring so you can power it up again. Some larger compressors

have starting caps so make sure you get those too.

Cut the lines and remove the compressor. The larger diameter

line is generally the suction (vacuum) line. Power it up.

It will spew a little oil out the exhaust line for a while

but it should stop after a little while.

Solder the fitting of your choice to the suction line and

thats it. It will eventually run dry and lock up but should

work for quite a while.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Streamer / strikes

Cc:

Bcc:

X-Attachments:

At 02:43 PM 1/9/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 09 Jan 1998 08:43:04 -0600

>Subject: Streamer / strikes

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>  I have read several articles and viewed photos of people setting on coils,

>touching streamers or allowing the output to travel through themselves, often

>discharging through aluminum foil wrapped around their finger tips.

>I understand that they are not grounded, however it does seem that they would

>receive burns from the streamer discharges.

>I have gained a lot of respect for the power output, having seen 4' disharges from

>my own coil. Can anyone explain this phenomenon ?

>

>Paul S. Marshall

>---

Paul the output of Tesla coils is RF. Rf exhibits a characteristic know as

skin effect. This causes the RF currents to flow along the outside

of conductors. It doesnt matter if that conductor is copper tubing

or your arm. And it seems that the nerves in our bodies dont respond

well to RF energy.

Given that information it would seem to be safe to take streamer hits

to the body. Not so. You can get very nasty RF burns from the streamers.

I have a permanent scar on my right hand from a nasty unintentional

streamer hit. You can minimize the burn problem by taking the hit

to a metal object held in your hand. But you cant depend on the streamers

to only hit where you want them to.

And the streamers from our coils are not just RF energy. They contain

a considerable mix of frequencies along with 60hz. The 60 hz energy

can give you a nasty shock as well as having the potentional (NPI)

to kill.

Some people foolishly play Russian roulette with their coils. Once our

coils start producing long powerful streamers they become too

dangerous for such stunts.

Live long and prosper

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: more fuzzy cap ?'s

Cc:

Bcc:

X-Attachments:

At 05:07 PM 1/8/98 GMT, you wrote:

>From: rpittman@juno.com

>Date: Thu, 8 Jan 1998 12:07:50 -0500

>Subject: more fuzzy cap ?'s

>

> O.K.  Mike you convinced me that the plate caps are "superior" in

>pulse performance.

Good. You wont be sorry. Plate caps are a pain to build but they sure

perform great.

> Now I am still a little unsure on how to calculate the capacitance.

>

>I found the formula    { [ (.224) *  K (which in poly would be around 2)

>* A (now is that the area of one plate or is that the sum of the areas of

>1/2 the plates or the sum of the area of all the plate) ]  /  D ( is 6

>mil .06 inhes or .006) ( is this the distance between any 2 individual

>plates or is this the total distance between all the plates? }  *  (N-1)

>(Is this simply the total number of plates in the cap?)

The formula is:

Capacitance (in picofarads) = (0.224 KA / d) (N-1)

Where K = dielectric constant. use 2.0 for polyethylene.

A = overlapping area of each plate in sq inches.

6X6 plates = 36 sq inches.

d = distance between plates in mils. 1 mil = .001 inch.

N = total number of plates

So lets say we want a plate cap with 24 mils of poly and 10

plates that overlap 6 X 6 inches.

(.224*2*36/.024)*9 = 6048 pf = .006048 mfd

>

>Also I noticed that in the design you sent me you made the plates 6" X 6"

> to use the 12" foil.  You also used 8 mils per layer with 24 layers and

>24 plates to get .048 mfd.

Yes this was the design of my first plate cap. Its still working

just fine after two years of hard use.

>

>I remember you saying that I needed .0053 mfd for each 15kv 30 ma.  So if

>I make the above cap with and wire 9 in series I will get .0053 mfd per

>unit.  Then I could build three units and just put another one in

>parallel every time I add another transformer?  Am I somewhere close?

Yes but you dont need that many.

>

>The one problem I see is that in a post you sent me you advised me to use

>24 mils of poly (6X4)  for each layer of the dielectric.  In the above

>design there are only 8 mils per layer.

Like I said that plate cap was my first one. I have changed

my approach to building caps somewhat since then. I used 6 caps

in series to use the thin 4 mil poly I had available. I now

use thicker stacks of poly with fewer series caps. You need

a minimum of two caps in series for 12kv and 3 in series for

15kv. I like to build all my caps with 24 mils of poly dielectric.

This results in a cap that will handle 9kv. You just add enough of them

in series to handle the voltage you want.

>

>So If I made the plates the same size (6X6)  and made the poly 24 mils /

>layer,  how many plates would I need to get the .0053 mfd?

Since you are running 15kv you want 3 series caps. That means

each cap in the string needs to be .0159 mfd.

So using 6 X 6 plates (actual overlapping area) and 24 mils

(4 X 6 mils) of poly for each dielectric layer it will take

24 plates. 24 plates will yield .0154 mfd. Thats close enough.

Or going to 26 plates will yield .0168 mfd. Build 3 such units

and wire them in series.

You can reduce the required number of plates by increasing the plate size.

Using 7 X 7 plates reduces the requirement to 18 plates - .0155 mfd.

I chose 6 X 6 since it was a convenient size to cut from a 12 inch

wide roll of foil.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Streamer / strikes

Date: Fri,  9 Jan 1998 20:21:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Fri, 9 Jan 98 15:21:50 -0500

Subject: Re: Streamer / strikes

>>I have gained a lot of respect for the power output, having seen 4'

>>disharges from my own coil. Can anyone explain this phenomenon ?

>Paul the output of Tesla coils is RF. Rf exhibits a characteristic know as

>skin effect.

Skin effect can protect you from damage from high-frequency relatively

low power coils (like table top models) since the depth of the skin

effect decreases as frequency of the current goes up.  For larger lower

frequency coils, the secondary current is much higher, and the skin depth

is much deeper.  These coils are certainly capable of output that can

burn and even kill you, and should be treated as such.

If you are getting 4' discharges, I would not recommend using yourself as

a ground rod.  Too much risk.

-Adam

Adam Smith

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Top load capacitance

Date: Fri,  9 Jan 1998 22:09:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 09 Jan 1998 16:09:47 -0600

Subject: Top load capacitance

Organization: MailExcite  (http://www.mailexcite.com)

Could some give me the equation for caluclating the capacitance of a given top

load toroid.

Thanks,

Paul M.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Top load capacitance

Cc:

Bcc:

X-Attachments:

At 10:09 PM 1/9/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 09 Jan 1998 16:09:47 -0600

>Subject: Top load capacitance

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Could some give me the equation for caluclating the capacitance of a given top

>load toroid

>

>Thanks,

>

>Paul M.

>---

Paul,

The only formula I am aware of is the one derived

by Bert Poole. Its been my experience that such formulas

do not accurately predict the capacitance provided by a

toroid. But for the sake of arguement I will paste in

Berts formula.

What I have done is derive the true capacitance of my

terminals by measuring my secondary resonant frequency

with and without the toroid.

___________________

/    2

C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  2 pi  (d1-d2)(d2/2)

/   -------------------

\/      4 pi

C = capacitance in picofarads (+- 5% )

d1 = outside diameter of toroid in inches

d2 = diameter of cross section (cord) of toroid in inches

Equation courtesy of Bert Pool

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: heat sinking your gaps

Cc:

Bcc:

X-Attachments:

At 05:00 PM 1/9/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Fri, 09 Jan 1998 11:00:53 -0600

>Subject: heat sinking your gaps

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Someone recently asked how to heat sink your

>RQ style gaps more efficiently.  Seems to me if you

>could keep the copper tubes sealed off, and filled

>with (possibly circulated) distilled or deionized water,

>that would do the trick.  The water would both

>absorb a substantial amt of heat, as well as distribute

>it to other areas of the tubing for transfer into the air.

>

<SNIP>

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

>

Bill,

Cooling the electrodes is far less important than cooling the arc.

What is needed is to have the hot ions that are clogging up

the gap removed quickly so the gap can quench.

Your suggestion will help keep the electrodes cooler and

maybe cut down on oxidation but doesnt do much for quenching.

High speed air is the simplest and probably the best method.

I know of one coiler who is running 9.6 KVA to his 10 inch

diameter secondary using nothing more than a vacuum quenched

static gap patterned after mine. He is getting consistant

8 foot streamers and a longest of 116 inches.

This gap is just a modified RQ style cylinder gap with airflow

supplied by a high speed vacuum cleaner motor.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: first coil

Cc:

Bcc:

X-Attachments:

At 02:42 AM 3/29/97 GMT, you wrote:

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Fri, 28 Mar 1997 20:42:08 -0600

>Subject: first coil

>

> I finishied enough of my coil to run a test tonight. Pretty cool but I

>only got about 8" of spark at half power. It turns out that I have made

>an error in my calculations. my current coil specs are:

>secondary  4" pvc  22" of 24 guage wire

>4" * 12" toroid

>

>15000v 30 ma

>safety gap and 2  2.5 mh air core chokes

>2 .01uF 7500 v plate poly caps in series

>R.Q. gap with hair drier fan using 4" pvc and 10 6"long 5/8" copper pipe

>.25" copper primary 15deg 11 turns inside diameter 8"

>after complete I found out that I should have used 15 turns to get into tune

>or I could up my cap to .01uF at 15,000v. If I someday want to move up

>to 90ma @15kv which would be better? Do I have any other options that I

>have over looked? Thanks for any advice anyone could give me.

I always recommend a large primary. I wouldnt make one with less

than 14 turns. So increasing your primary inductance is a good idea.

If you have a second 15kv/30ma neon then you could parallel them for 15kv/60ma

and up your tank capacitance to .01 mfd.

I would do both. You will need that larger primary inductance to

handle the larger toroids you will want to use later. The power increase

and the larger cap will greatly increase your primary energy levels.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: voltage increase

Date: Sat, 10 Jan 1998 07:11:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Sat, 10 Jan 1998 01:11:36 -0600

Subject: voltage increase

Organization: MailExcite  (http://www.mailexcite.com)

I have a few questions about the relationship between secondary size and maximum

voltage output.

1.) what is the relationship between diameter of the coil and the maximum spark

length?

2.) I've heard that Tesla's original coils were very short and fat. On the other

hand, some claim that tall coils work better. Which is really better overall? I've

heard that a 3 to 1 ratio works well for an 8 inch coil, would a 2:1 work as well?

or even a 1:1?

3.) I've heard that the ratio of diameter to the secondary length increases as the

diameter increases. If that is so, then would a, say, 15 inch coil require a 1:1

ratio? Would a .5 inch coil have about a 20:1 ratio?

4.) what is an ideal spark length that can probably be obtained by an 8 inch coil

(3:1 ratio)?

Thanks! I got a really good deal on some magnet wire, and so I am winding an 8

inch coil. I'm just wondering if anyone understands this size ratio stuff!

Any comments would be much appreciated!

--Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Vacuum pump.

Date: Sat, 10 Jan 1998 12:04:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Erik Hennix" <erik.hennix@swipnet.se>

Date: Sat, 10 Jan 1998 12:04:59 +0000

Subject: Re: Vacuum pump.

> From:          tesla-2@emachine.com (Tesla-2)

> Subject:       Re: Vacuum pump.

> Date:          Fri,  9 Jan 1998 17:11:58 GMT

> Organization:  The Energy Machine Information System 504-733-3462

> To:            erik.hennix@swipnet.se

> From: Eric Davidson <edavidson@icva.gov>

> Date: Fri, 09 Jan 1998 11:11:58 -0600

> Subject: Re: Vacuum pump.

>

> Erik:

>

> There is really no conversion necessary. Just remove the compressor from

> the rest of the refrigeration equipment and you are ready to go. Most

> small compressors are of the rotary vane type and do not create a very

> impressive vacuum (my guess would be a few millimeters Hg). They would

> be more than adequate for removing air from a rolled cap I would think.

> Also, don't expect much in the way of capacity (CFM/LPM). You should

> periodically add a FEW DROPS of a light oil to the intake since there is

> no oil in the system. Lastly, don't run it for extended lengths of time

> like over night, remember there is no oil in the system. Hope this

> helps.

>

> Eric

> edavidson@icva.gov

>

That easy eh?

I'm off to find a compressor. Thanks a  lot Eric,

Erik Hennix.

Skal,

Erik Hennix

TCBOS

(Tesla Coil Builders Of Sweden)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: heat sinking your gaps

Date: Sat, 10 Jan 1998 17:00:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sat, 10 Jan 1998 11:00:43 -0600

Subject: Re: heat sinking your gaps

Organization: MailExcite  (http://www.mailexcite.com)

Mike wrote:

>Cooling the electrodes is far less important than cooling the arc.

>What is needed is to have the hot ions that are clogging up

>the gap removed quickly so the gap can quench.

>

>Your suggestion will help keep the electrodes cooler and

>maybe cut down on oxidation but doesnt do much for quenching.

<snip>

Mike,

Your point is well taken!  But there might be more to it that just

that, as follows:

It seems that there may be different requirements for

gap quenching for various different types of gap-driven

equipment.  Let me explain.

It appears that, for Tesla coil use, it is very

important for the gap to quench quickly.  This is,

I presume, to prevent the energy present in the

secondary from "coming back across" into the primary,

which would reduce your spark length, etc.

Some coilers refer to this as "quenching on the

first notch", etc.  (I've been reading the archives again...)

This is also why (I assume) rotary gaps work better

for the more highly powered coils than static gaps

do.  Although the information about the 9.6KVa coil

using a static gap was quite unique :)

Now - the very first spark gaps used were (to the best

of my knowledge) in old radio transmitters.  As Mike

Hammer informed me, these spark gaps were sometimes

constructed of multiple disks of copper, with thin mica

washers between the disks.  This provides an area (the center of the mica disk)

where the sparking would occurr.

In this sort of gap, there can be no airflow to

remove the hot ionized air.  I believe the intent of

this type of gap was to allow the LC circuit to continue ringing (by KEEPING the

conductive path present) until (ideally) all of the energy was dissipated.

This requirement is different than those of the Tesla coil gap.

Further evidence is seen in modern (1970's) TIG welder arc starters, which

typically have 2-4 tungsten disks, 3/4 inch diameter, 0.008 inch gap apart,

with passive heatsinking, but no forced air.  There is a smallish high current

capacitor present (don't recall exact values at the moment) and the thing is

powered by a small current-limited 6-10KV transformer running off the mains.

My final information is in the large spark gap arrays

found in to old induction heating rigs, which were

water cooled, but did not provide for removal of the

ionized gasses.

So let me just summarize that apparently there are

differing requirements between Tesla coiling and

induction heating, as far as gap design.  I'm not

trying to start a fight, just stating what I believe is

going on.  For Tesla coiling, you definitely need

the air cooling and gap extinguishing action which

the airflow provides.

If anyone has any comments about what I have written,

either positive or negative, please don't hesitate

to post them to the list or to me directly!  I want

to understand what is going on here!

-Bill the arcstarter

Starting arcs and doing induction heating in my garage

in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Top load capacitance

Date: Sat, 10 Jan 1998 20:39:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: CJ Hill <cjhill@hills.u-net.com>

Date: Sat, 10 Jan 1998 20:39:14 +0000

Subject: Re: Top load capacitance

Mike,

Can't Bert's formula be simplified to:  (?)

_____________________

C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  pi  (d1-d2)(d2/2)

/   -------------------

\/      2

Maybe I'm missing something! (Or the formula has been corrupted as it's

passed down the e-mails!)

I would guess that the biggest thing to consider when thinking about top

load capacitance is the presence of nearby objects. If something is nearby,

then I would guess that the capacitance would be much greater. An

interesting thought... perhaps we detune a coil simply by trying to draw an

arc from it!

Regards, Chris

>The only formula I am aware of is the one derived

>by Bert Poole.

>                                                    ___________________

>                                                   /    2

>         C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  2 pi  (d1-d2)(d2/2)

>                                               /   -------------------

>                                            \/      4 pi

>

-------------------------------------------------

Chris Hill, Cardiff, UK

cjhill@hills.u-net.com

http://www.hills2.u-net.com/tesla.htm

-------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Vacuum tube tesla coils

Date: Mon, 12 Jan 1998 01:44:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Mon, 12 Jan 1998 01:44:12

Subject: Vacuum tube tesla coils

I am looking for any info on vacuum tube tesla coils. If you have any

please e-mail me at. choad69@usa.net Thanks for your help.

Shaun O'Brien

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: more fuzzy cap ?'s

Date: Mon, 12 Jan 1998 12:35:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Mon, 12 Jan 1998 07:35:08 -0500

Subject: Re: more fuzzy cap ?'s

>

>So lets say we want a plate cap with 24 mils of poly and 10

>plates that overlap 6 X 6 inches.

>

>(.224*2*36/.024)*9 = 6048 pf = .006048 mfd

>

>

Thanks for the clarification,  The example makes it much easier.  I can

now plug in variables for the information I want.

>So using 6 X 6 plates (actual overlapping area) and 24 mils

>(4 X 6 mils) of poly for each dielectric layer it will take

>24 plates. 24 plates will yield .0154 mfd. Thats close enough.

>Or going to 26 plates will yield .0168 mfd. Buid 3 such units

>and wire them in series.

It's off to cap building I go.

>

>You can reduce the required number of plates by increasing the plate size.

>Using 7 X 7 plates reduces the requirement to 18 plates - .0155 mfd.

>

>I chose 6 X 6 since it was a convenient size to cut from a 12 inch

>wide roll of foil.

Yep I have the same rolls.

Well thanks alot, Michael also,  I do feel like I understand now how to

calc and build my cap.

One other idea I had,  has anyone tried cutting the aluminum foil on the

roll with say a razorblade maybe.  You could at lease cut the roll in

half giving 2  - 6" rolls thus only having to cut them with the scissors

or a paper cutter once at every 6 or 8 inches to get the 6X6 or maybe

6X8.  Any suggestions?

Thanks alot,

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Gap improvments

Date: Mon, 12 Jan 1998 16:14:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 12 Jan 1998 10:14:19 -0600

Subject: Gap improvments

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Recently I made some gap improvements. I am currently using an RQ style gap.

This week I drilled approximatly 5) 9/32" holes between the electrodes and

placed snap on end plugs to seal the gap. I cut a window in one end making a

snug fit with my exhaust fan. In doing this I supplied air directed thru the gaps.

My streamer length increased and with the top load 7"x 18" dryer duct, I managed to

trip the 20 amp breaker. What's going on ? This did not happen until I supplied

the direct air.

Paul Marshall

--

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Solid tungsten carbide gaps!

Date: Mon, 12 Jan 1998 20:15:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 12 Jan 1998 14:15:10 -0600

Subject: Solid tungsten carbide gaps!

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Over the weekend I purchased (for $15 from MECI in

Dayton) 10 solid tungsten carbide endmill inserts.

These are small blocks, about 3/4 x 3/4 x 3/16

inches.  I'd hoped that these would serve as some

decent low-erosion spark gaps.

I brazed copper wires to two of the blocks (solder did

not want to stick), and placed four blocks edge to

edge on the glazed surface of a floor tile.  Each block

was separated by roughly 0.028 inch (one hacksaw

blade).

I energized this circuit with a 7500v 60ma neon, in

parallel with a 0.026 uf capacitor. I did not use a fan

or any form of cooling. I ran this for about 10 minutes.

I did have some chokes/safety gaps/bypass caps

on the neon.  The capacitive arc discharges were

extremely loud!

After shutdown, I inspected the surfaces of the

carbides.  It appears that there was very little spark

erosion on the surfaces.  Light sanding elininated all

of the deposits, revealing a largely unblemished

carbide face underneath.

Perhaps this would make a longer lasting spark gap

than the traditional copper pipe in RQ gaps?  If

properly aligned, the area between each arc gap

would be about 3/4 by 3/4 inch square of carbide!

I may try to align the large faces of the carbide and

glue them into a heap or stack, to evaluate them

further.  I envision a rectangular "monolith" of

stacked carbide blocks with 0.028 inch gaps

between each block.  Does anyone have any ideas

how I could accomplish this?  The blocks have no

holes and drilling any would be about bloody

impossible.  Cleaning, if required, could be

accomplished by shooting the sandblaster between

the blocks.

An aside - the sandblaster has nozzle made of a solid

cylinder of carbide with a hole down the center -

overall about 1.25 inches long and about 5/8

diameter.  These would be perfect for use in a

RQ-type arrangement if the nozzles weren't $20 a

pop!

By the way - the heat of the arcs burned some

substantial holes into the front surface of the ceramic

tile!  Thats hot stuff.  I'll try it out on some 3000

degree F-rated refractory blocks later tonight.

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Gap improvments

Date: Mon, 12 Jan 1998 21:42:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 12 Jan 1998 16:42:52 -0500

Subject: Re: Gap improvments

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Gap improvments

> Date: Monday, January 12, 1998 11:14 AM

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 12 Jan 1998 10:14:19 -0600

> Subject: Gap improvments

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>    Recently I made some gap improvements. I am currently using an RQ style gap.

>  This week I drilled approximatly 5) 9/32" holes between the electrodes and

> placed snap on end pluges to seal the gap. I cut a window in one end making a

>snug fit with my exhaust fan. In doing this I supplied air directed thru the gaps.

>My streamer length increased and with the top load 7"x 18" dryer duct, I managed

>to trip the 20 amp breaker. What's going on ? This did not happen until I supplied

>the  direct air.

>

> Paul Marshall

> --

> Einigkeit und Freiheit

>

Paul,

When the spark is lengthened by the air flowing through the gap,

the voltage across the gap increases. This causes the CURRENT

drawn from the primary to increase (since the voltage at the primary

is a function of your variac setting).

This is an aspect of the spark gap that is often overlooked. It is

particularly noticeable with current limited transformers. While

the output current from the transformer is limited, the POWER is

a function of voltage times current. So if the output voltage is

allowed to rise (same thing as opening your gap), then the output

power increases. This causes a like increase in the input power

the transformer has to provide.

Opening the gap too wide can cause the secondary voltage to rise

too high and kill neons. Pole pigs can take the abuse, but most

neons roll over and play dead eventually.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: first coil up and running.

Cc:

Bcc:

X-Attachments:

At 01:07 AM 4/1/97 GMT, you wrote:

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Mon, 31 Mar 1997 19:07:02 -0600

>Subject: first coil up and running.

>

>

>

>Just got my coil up to full power and finally got some good sparks from

>it, bit over 2 foot.

Congratualtions! 2 foot is pretty good for a first coil.

> Just wanted to ask how much more could I expect if

>I doubled the power.

Power input vs spark output is a very nonlinear function.

You will not double your output by doubling the power.

In fact doubling your power will probably only add a few inches

of additional spark.

To make best use of a power increase you need to also increase

your primary capacitor size to match and balance that increase

with a larger toroid so you can still tap the same number

of primary turns. Also you need to make sure your gap can

handle the power increase. Often simple rather crude gaps

will work adequately at low power but fail to process any

additional current as the power is increased.

> I also found two 3.5uF 440v AC caps. If I put these

>in the 120V supply would they help to cut down some of the power

>flucuations I see when I run my coil? I also wanted to thank everyone

>who answered my questions or gave me advice.

These caps will be of minimal use. They are too small to

use as power factor correction caps. They may help smooth

out some transients on your 60 HZ AC line preventing them

from getting into your house wiring. Other than that they

arent of much value and wont affect your coil output in any way.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Solid tungsten carbide gaps!

Cc:

Bcc:

X-Attachments:

At 08:15 PM 1/12/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Mon, 12 Jan 1998 14:15:10 -0600

>Subject: Solid tungsten carbide gaps!

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Over the weekend I purchased (for $15 from MECI in

>Dayton) 10 solid tungsten carbide endmill inserts.

>These are small blocks, about 3/4 x 3/4 x 3/16

>inches.  I'd hoped that these would serve as some

>decent low-erosion spark gaps.

>

<BIG SNIP>

>

>-Bill

Bill,

A friend of mine and fellow coiler Gary Weaver tried

tungsten carbide in one of his gaps. He had a pile of

worn out or broken carbide tools so decided to give them

a try. He had moderate success but has given them up

in favor of copper pipe.

Gary is also the one getting 116 inches at 9.6kva

on his ten inch coil using only a static gap. (copper pipe)

He has some pics of his coil running. The URL is:

http://home.earthlink.net/~gweaver/tc-page.htm

Scroll down to coil #7.

Some of the possible problems I see with carbide are its

conductivity as compared to copper and the characteristics

of the oxidized electrodes.

The carbide electrodes may have enough resistance to

be significant at higher power levels. At least when

compared to copper. And the oxidized surface of many

materials may actually be an insulator. Copper oxide

is a good conductor so even a heavily oxidized copper

electrode is still effective. I dont actually know the

characteristics of oxidized carbide electrodes.

I'm just suggesting a possiblity.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Top load capacitance

Cc:

Bcc:

X-Attachments:

At 08:39 PM 1/10/98 GMT, you wrote:

>From: CJ Hill <cjhill@hills.u-net.com>

>Date: Sat, 10 Jan 1998 20:39:14 +0000

>Subject: Re: Top load capacitance

>

>Mike,

>

>Can't Bert's formula be simplified to:  (?)

>

>                                              _____________________

>     C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  pi  (d1-d2)(d2/2)

>                                            /   -------------------

>                                          \/      2

>

>Maybe I'm missing something! (Or the formula has been corrupted as it's

>passed down the e-mails!)

>

Yes that seems right to me. But Im not that great with math

so dont take my word for it.

>I would guess that the biggest thing to consider when thinking about top

>load capacitance is the presence of nearby objects.

Exactly. And this is quite unpredictable.

Ive read where some derate the calculated toroid capacitance

by 20 percent to get a more accurate working figure. This

seems to be pretty close with my toroids.

> If something is nearby,

>then I would guess that the capacitance would be much greater. An

>interesting thought... perhaps we detune a coil simply by trying to draw an

>arc from it!

Yes spark loading will reduce your operating frequency

slightly. As will the cloud of ions that form around your toroid

during operation. This can amount to as much as a 1/2 turn of primary

inductance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Multiple Questions...

Date: Tue, 13 Jan 1998 20:46:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 13 Jan 1998 12:46:59 -0800 (PST)

Subject: Multiple Questions...

I am slowly working toward bringing my 6.25x25 secondary online, mostly

by testing some of the new components on my 3.5x10" coil first. And I'd

like some comments from the more experienced on some of my results that

have me just a little confused.

1) New cap: Works just fine, total capacitance is now around 0.013uf,

I still need to snarf the meter from work to double-check. BTW:

New cap only needed about 1.5 quarts oil.. I'm learning. [ Side

note: I 'burped' this cap using nothing more than a simple

home vacuum cleaner, as I did not trust my glue to support 740lbs

of pressure (56 sq inch at 14PSi). Little or no bubbles during

runs now]

First gen cap: 0.0061uf, 35lbs, about a gallon of oil

Second Gen cap: 0.0072 (dry), 20lbs, 1.5quarts oil. [and no leaks!],

I'll be getting wet readings soon.

For starters, I just hooked this in with my 12KV/30 (yes I know

its too big!) but was surprised to find the performance about

the same, despite the retapping required (tapped at about 4.5

turns). Is this unusual?, by this, I mean having too much cap

and yet still getting good output.

I then added the second 12KV/30, and had output go up from 14" to

16" , but output would fall off rapidly. Based on comments from the

list, I guess my RQ style gap was not meeting the power

requirements. Mostly because it is in a linear arrangement, where

a fan cannot help much. This lead to:

2) New RQ gap. Currently 3" long 1"dia copper pipe. Gapping is

approximatly 0.030, and I am currently using 5 gaps. This is in

a 6" drainpipe.

This was the wierd part.. I can only assume my gapping is not

correct, as my previous RQ gap used 6 gaps at 0.030, but if I try

this with the new gap, my saftey gap goes insane. Note that I have

a high volume muffin fan assisting in the cooling of the gap.

Any ideas of what is happening here??

With this new gap, the coil is producing 18" arcs, and some that

have appeared to go beyond, but since there was not a strikepoint, I

can't prove it. I can also run for about 10 minutes, at which point

the 14awg cable I use for the tap starts to warm up. The pipes of the

gap are barely warm.

One other thing: with the new cap and gap, my coil has become

hypersensitive to tuning it seems. Changing my tap less than

1/4 turn has substantial effect on output, and even someone

walking around in the garage can affect the output (5ft or more

away from the coil) What up with dat?

3) Primary: For my 6" I am making a 'temporary' primary from

12awg wire, but spacing at if I were using 1/4" with 1/4" gap.

This is to allow me to get a better idea of how much copper pipe

to buy to build the final primary.

Am I off the wall in doing this?  [the wire was scrap I had on a

shelf from a rewire project 5 years ago]

4) Question: how would 6AWG wire work for a primary, or is solid wire

really a no-no?

5) I am having major problems with the arc wanting to dive down to my

strike rail on the little coil. Any tricks for solving this one?

6) RF-Ground: I am moving to a new home in June, as soon as construction

finishes. I tried to get a connection to the rebar of the garage

floor, but the construction gods ripped out what I had :/

As a result, I am planning to 'build a ground' and would like

comments on my proposal:  I need to build 2 of them actually, one

for indoor work, and one for outdoor [ our lot is very narrow ]

For the indoor: I was planning 2 6ft rods, 6ft apart, connected

(via trench) by 4 or 6 AWG bare wire. Then taking that into the

garage via flat strap, or similar.

For the outdoor: 5, or more 6' rods, Connected in a star pattern

using the 4 or 6 AWG wire to interconnect. Access to the

connection point will be via an 'irrigation valve' box, embedded in

the lawn.

The reason behind the 6' rods is that I may not even be able to drive

those in, this house is built at the top of a low hill, on live

granite.

Any better ideas?

Thanks for your patience :)

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Multiple Questions...

Cc:

Bcc:

X-Attachments:

At 08:46 PM 1/13/98 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Tue, 13 Jan 1998 12:46:59 -0800 (PST)

>Subject: Multiple Questions...

>

>I am slowly working toward bringing my 6.25x25 secondary online, mostly

>by testing some of the new components on my 3.5x10" coil first. And I'd

>like some comments from the more experienced on some of my results that

>have me just a little confused.

>

>1) New cap: Works just fine, total capacitance is now around 0.013uf,

>   I still need to snarf the meter from work to double-check. BTW:

>   New cap only needed about 1.5 quarts oil.. I'm learning. [ Side

>   note: I 'burped' this cap using nothing more than a simple

>   home vacuum cleaner, as I did not trust my glue to support 740lbs

>   of pressure (56 sq inch at 14PSi). Little or no bubbles during

>   runs now]

Nothing wrong with that. On my first rolled cap I used

lung power to vacuum the cap. Not a recommended procedure

but does show that a high vacuum is not needed.

I've also heard of someone using a vacuum line from their

automobile to pump their caps.

>

>   First gen cap: 0.0061uf, 35lbs, about a gallon of oil

>   Second Gen cap: 0.0072 (dry), 20lbs, 1.5quarts oil. [and no leaks!],

>                   I'll be getting wet readings soon.

>

>    For starters, I just hooked this in with my 12KV/30 (yes I know

>    its too big!) but was surprised to find the performance about

>    the same, despite the retapping required (tapped at about 4.5

>    turns). Is this unusual?, by this, I mean having too much cap

>    and yet still getting good output.

The actual value isnt very critical. Often neons can successfully

charge larger caps than the math indicates. I've used .008 mfd with

12kv/30ma neons quite successfully.

>

>    I then added the second 12KV/30, and had output go up from 14" to

>    16" , but output would fall off rapidly. Based on comments from the

>    list, I guess my RQ style gap was not meeting the power

>    requirements. Mostly because it is in a linear arrangement, where

>    a fan cannot help much. This lead to:

>

>2) New RQ gap. Currently 3" long 1"dia copper pipe. Gapping is

>   approximatly 0.030, and I am currently using 5 gaps. This is in

>   a 6" drainpipe.

>   This was the wierd part.. I can only assume my gapping is not

>   correct, as my previous RQ gap used 6 gaps at 0.030, but if I try

>   this with the new gap, my saftey gap goes insane. Note that I have

>   a high volume muffin fan assisting in the cooling of the gap.

>   Any ideas of what is happening here??

Sure. The high speed air is stretching the arc between the electrodes.

This effectively widens the gap and allows the cap to charge to a higher

voltage. High speed air can also inhibit initial breakdown for a bit also

allowing for higher primary voltages.

Back down one gap on your main gap and things will probably settle down.

You could probably open your safety slightly if its present setting is

rather conservative.

>

>   With this new gap, the coil is producing 18" arcs, and some that

>   have appeared to go beyond, but since there was not a strikepoint, I

>   can't prove it. I can also run for about 10 minutes, at which point

>   the 14awg cable I use for the tap starts to warm up. The pipes of the

>   gap are barely warm.

This is a common weak point. Beef that cable and tap up a bit

until you eliminate the heating.

>

>   One other thing: with the new cap and gap, my coil has become

>   hypersensitive to tuning it seems. Changing my tap less than

>   1/4 turn has substantial effect on output, and even someone

>   walking around in the garage can affect the output (5ft or more

>   away from the coil) What up with dat?

Sensitive tuning is an indication of high primary circuit Q.

On my coil I see much the same thing when using my flat plate

caps. The tuning broadens out considerably when using my rolled caps.

>

>3) Primary: For my 6" I am making a 'temporary' primary from

>   12awg wire, but spacing at if I were using 1/4" with 1/4" gap.

>   This is to allow me to get a better idea of how much copper pipe

>   to buy to build the final primary.

>   Am I off the wall in doing this?  [the wire was scrap I had on a

>   shelf from a rewire project 5 years ago]

This is a good idea. Your first primary will almost always

be insufficient for very long. Making a temp primary gives you

a much better idea of what you will need.

Unfortunately my temp primary became permanent.

>

>4) Question: how would 6AWG wire work for a primary, or is solid wire

>   really a no-no?

My primary consists of 17 turns of AWG 6 solid copper.

Works great. If you have it use it. If you are buying then copper

tubing will probably be cheaper.

>

>5) I am having major problems with the arc wanting to dive down to my

>   strike rail on the little coil. Any tricks for solving this one?

Try a foil bump on the outer perimeter of your toroid. This will

provide a place for streamers to break out from laterally.

You can even place them a little higher to discourage downward

strikes.

Also a larger diameter toroid gets to streamers farther away

from the coil. Primary strikes are a fact of life and have to

be dealt with. Thats why strike rails are necessary.

>

>6) RF-Ground: I am moving to a new home in June, as soon as construction

>   finishes. I tried to get a connection to the rebar of the garage

>   floor, but the construction gods ripped out what I had :/

>   As a result, I am planning to 'build a ground' and would like

>   comments on my proposal:  I need to build 2 of them actually, one

>   for indoor work, and one for outdoor [ our lot is very narrow ]

>

>   For the indoor: I was planning 2 6ft rods, 6ft apart, connected

>   (via trench) by 4 or 6 AWG bare wire. Then taking that into the

>   garage

>   via flat strap, or similar.

>

>   For the outdoor: 5, or more 6' rods, Connected in a star pattern

>   using the 4 or 6 AWG wire to interconnect. Access to the

>   connection point will be via an 'irrigation valve' box, embedded in

>   the lawn.

>

>   The reason behind the 6' rods is that I may not even be able to drive

>   those in, this house is built at the top of a low hill, on live

>   granite.

>

>   Any better ideas?

Sounds just fine to me. As far as RF grounds go more is always better.

>--

>Michael Baumann

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Cardboard coil forms and large caps?

Date: Wed, 14 Jan 1998 14:29:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 14 Jan 1998 08:29:16 -0600

Subject: Cardboard coil forms and large caps?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Last night I was at the local building supply store,

where I noticed several sizes of cardboard tubes,

used for making concrete footers, etc.

Would these be any good for a tesla coil secondary?

The cardboard is uncoated and would appear to

absorb polyurethane varnish quite well.

Secondly - if I wanted to try some exploding wire

experiments - how would I make a large enough

capacitor which would be smaller than my entire

garage?  Is there a trick for making high-valued

DC-pulse capacitors??

Lastly - If I were to build two secondaries of identical

overall size, but wound one using magnet wire and

the other with PVC-insulated bell-wire,  what could be

said about their resonant frequencies?  Of course, the

bell-wire one would contain fewer turns since the

wire is thicker, etc.

(Our present coil is wound with 24g plastic coated

hook-up wire)

Thanks!  More later (especially about carbide gaps!)

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: more fuzzy cap ?'s

Cc:

Bcc:

X-Attachments:

At 12:35 PM 1/12/98 GMT, you wrote:

>From: rpittman@juno.com

>Date: Mon, 12 Jan 1998 07:35:08 -0500

>Subject: Re: more fuzzy cap ?'s

>

>

<SNIP>

>>I chose 6 X 6 since it was a convenient size to cut from a 12 inch

>>wide roll of foil.

>

>

>Yep I have the same rolls.

If you want to go to large plates buy the 18 inch wide rolls of

foil. This is what I did when I built my rolled caps.

I cut the roll down into two 8 inch wide rolls.

>

>Well thanks alot, michael also,  I do feel like I understand now how to

>calc and build my cap.

No problem.

>

>One other Idea I had,  has anyone tried cutting the aluminum foil on the

>roll with say a razorblade maybe.  You could at lease cut the roll in

>half giving 2  - 6" rolls thus only having to cut them with the scissors

>or a paper cutter once at every 6 or 8 inches to get the 6X6 or maybe

>6X8.  Any suggestions?

Yes I cut the rolls in half first. What I did was wrap a strip

of masking tape around the roll and then mark a line on the tape.

I then cut the roll in half using a razor saw. A razor saw is a

very fine toothed hand saw meant for modeling. Buy at any hobby

shop. Go slowly so you dont tear the foil.

When cutting out the foil plates the best bet is a fresh

razor blade and a straightedge. Dont forget to round the corners

with a sharp pair of scissors.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cardboard coil forms and large caps?

Cc:

Bcc:

X-Attachments:

At 02:29 PM 1/14/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Wed, 14 Jan 1998 08:29:16 -0600

>Subject: Cardboard coil forms and large caps?

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Last night I was at the local building supply store,

>where I noticed several sizes of cardboard tubes,

>used for making concrete footers, etc.

>

> Would these be any good for a tesla coil secondary?

>The cardboard is uncoated and would appear to

>absorb polyurethane varnish quite well.

>

Yes you can use cardboard forms but they generally result

in lower performance coils than dried and sealed

PVC. You need to soak them well with polyurethane to improve

the loss factor as well as stiffen them up a little.

They also dont have the voltage breakdown rating that a PVC

pipe would.

> Secondly - if I wanted to try some exploding wire

>experiments - how would I make a large enough

>capacitor which would be smaller than my entire

>garage?  Is there a trick for making high-valued

>DC-pulse capacitors??

You really cant build this type of cap at home. You need

many microfarads of capacitance. This calls for materials

with high dielectric constants that just arent readily available.

Most of the experiments of this type are done with banks of

electrolytic capacitors. Careful though. Electrolytics

often do not take kindly to pulse discharging.

>

> Lastly - If I were to build two secondaries of identical

>overall size, but wound one using magnet wire and

>the other with PVC-insulated bell-wire,  what could be

>said about their resonant frequencies?  Of course, the

>bell-wire one would contain fewer turns since the

>wire is thicker, etc.

>(Our present coil is wound with 24g plastic coated hook-up wire)

Thats the main thing. Fewer turns equals lower inductance

so means a higher resonant frequency.

Use of magnet wire is preferred as its gives you the highest

inductance per unit area.

>-Bill the arcstarter

>Starting arcs in Cinci, OH

>

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Vacuum tube tesla coils

Cc:

Bcc:

X-Attachments:

At 01:44 AM 1/12/98 GMT, you wrote:

>From: <choad69@usa.net>

>Date: Mon, 12 Jan 1998 01:44:12

>Subject: Vacuum tube tesla coils

>

>I am looking for any info on vacuum tube tesla coils. If you have any

>please e-mail me at. choad69@usa.net Thanks for your help.

>

>Shaun O'Brien

>

The only site Im aware of with vacuum tube Tesla coil

info are Tero Ranto's page at: http://koti.icenet.fi/~dixie/

Brent Turner's book also has some info on building tube coils.

http://www.apc.net/bturner/hv-page.htm

There is some vacuum tube data available on funet.fi

ftp://nic.funet.fi/pub/sci/electrical/tesla/tubes/

John Freau has a series of video tapes available

several of which are on vacuum tube coils.

Contact John at: FutureT@aol.com for details.

Thats about all I could turn up after a short search.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Solid tungsten carbide gaps

Date: Thu, 15 Jan 1998 02:20:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 14 Jan 1998 20:20:29 -0600

Subject: Re: Solid tungsten carbide gaps

Organization: MailExcite  (http://www.mailexcite.com)

Mike Hammer wrote:

>Some of the possible problems I see with carbide are its conductivity as

>compared to copper and the characteristics of the oxidized electrodes.

Yes - this might end the project. I'm gambling

that the large cross sectional area of the

carbide block will more than offset the greater bulk

resistance of the material.

2ndly- the skin effect will work against me by

preventing the entire cross sectional area from

carrying current!

3rdly - I can't compute the skin depth w/o knowing the

material properties of carbide!  Catch-22! (Does anyone

know these values for tungsten carbide? Bulk

resistivity and magnetic permeability?)

I will answer all these questions empirically - by

building one!  McMaster-Carr has kindly agreed to send

me 50 ft of 1/8 inch copper tubing for the pittance of

$12.  Can't beat that!  Water-cooled gap here I come..

I will send Mr. Weaver an Email and see what

advice he has to offer.  Thanks for mentioning the

name.

>The carbide electrodes may have enough resistance to

>be significant at higher power levels. At least when

>compared to copper. And the oxidized surface of many

>materials may actually be an insulator. Copper oxide

>is a good conductor so even a heavily oxidized copper

>electrode is still effective. I dont actually know the

>characteristics of oxidized carbide electrodes.

>I'm just suggesting a possiblity.

Yep - Won't know until I build it, but the initial

experiment indicated that it might work rather well.

I'm told the stuff won't oxidize but that sounds too

good to be true. I did heat a chunk up red hot with

the torch, and it didn't care a bit, still nice and

shiny!

Mike - thanks you for your advise on cardboard forms

and stuff!

-Bill "I need a web page" arcstarter

Wasting time AND money in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cardboard coil forms and large caps?

Date: Thu, 15 Jan 1998 04:46:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 14 Jan 1998 20:46:27 -0800

Subject: Re: Cardboard coil forms and large caps?

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: arcstarter@mailexcite.com

> Date: Wed, 14 Jan 1998 08:29:16 -0600

> Subject: Cardboard coil forms and large caps?

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>  Last night I was at the local building supply store,

> where I noticed several sizes of cardboard tubes,

> used for making concrete footers, etc.

>

>  Would these be any good for a tesla coil secondary?

> The cardboard is uncoated and would appear to

> absorb polyurethane varnish quite well.

After treatment with polyurethane, they should work satisfactorily. One

of the more notable coilers, Ed Wingate, uses 24" diameter cardboard

coilforms for his big twin coil with great results. He also uses

PVC-insulated machine wire (#16 gauge) for his twin coils.

>

>  Secondly - if I wanted to try some exploding wire

> experiments - how would I make a large enough

> capacitor which would be smaller than my entire

> garage?  Is there a trick for making high-valued

> DC-pulse capacitors??

>

The best trick is to find commercial caps on the surplus market or at

hamfests. For simple exploding wire experiments, mylar caps can also be

used, as well as power utility company power factor correction caps. One

of the guys on Chip's list uses a big electrolytic capacitor bank to

explode wires. You also may want to check periodically at your local

junk yard for non-PCB caps. Making large valued plastic dielectric caps

is a very labor intensive undertaking, as the caps will be MUCH larger

than commercial equivalents. Make sure you are VERY careful - at the

energy levels required to make exploding wires, a discharge through any

part of your body can cause bone marrow to explode, effectively doing an

instant amputation as well as probably killing you. Hearing protection is

also mandatory!

>  Lastly - If I were to build two secondaries of identical

> overall size, but wound one using magnet wire and

> the other with PVC-insulated bell-wire,  what could be

> said about their resonant frequencies?  Of course, the

> bell-wire one would contain fewer turns since the

> wire is thicker, etc.

The coil with fewer turns will have less inductance, and will therefore

also have a higher self-resonant frequency.

>

> (Our present coil is wound with 24g plastic coated

> hook-up wire)

>

> Thanks!  More later (especially about carbide gaps!)

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: salt water caps

Date: Fri, 16 Jan 1998 07:42:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gretchen Kulpa <gwkulpa@earthlink.net>

Date: Thu, 15 Jan 1998 23:42:57 -0800

Subject: salt water caps

Being new to this I have many questions. One of them is: when building

beer bottle capacitors, do they need to be shaken up after letting them

sit for a day or so to keep the salt in solution? If so, how often do

they need to be shook up?

walt Kulpa

gwkulpa@earthlink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: salt water caps

Date: Fri, 16 Jan 1998 11:30:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Fri, 16 Jan 1998 11:30:09

Subject: Re: salt water caps

Walt,

I dodn't really think you have to shake the bottles up. Try not

shaking them for a few days then check the performance then shake them

really well and check the performance again. I haven't noticed any

differance in shaking them and not shaking them, but you never know until you

try it yourself. I have learned in building tesla coils that it's not an

exact science. If your building your first coil I wouldnt wory about the

details until you have it working then "beef up" what you have for max

performance.

Happy coiling!

Shaun

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: salt water caps

Date: Fri, 16 Jan 1998 12:47:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Fri, 16 Jan 1998 07:47:02 EST

Subject: Re: salt water caps

Organization: AOL (http://www.aol.com)

Walt,

If salt has settled out of your electrolyte solution,  It is not helping

the ionic strength of the electrolyte.  I would allow this salt to settle out

and decant the solution from the solid before filling the bottles.  You might

even add a little extra water to the solution before filling the bottles to

prevent the salt from crystallizing out inside the bottles.  I do not know

exactly what effect the salt crystals might have, but the solid has dielectric

properties(the dielectric constant at 10kHz is about 6).  If crystals

precicitate from the solution,  it could change the capacitance of your  bank

over time.  This might require you to adjust the tuning your coil after a

while.  Merely shaking the bottles will not put the salt back into solution if

the solution is saturated.   The solublilty of sodium chloride is 357g/ liter

of distilled water(or just under 3 lb per gallon) at 0 deg. C.  The solubility

goes up just a little at room temperature.  This might be useful info in

making up your solution.

I hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: salt water caps

Date: Fri, 16 Jan 1998 18:14:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Fri, 16 Jan 1998 12:14:59 -0600

Subject: Re: salt water caps

>being new to this I have many questions. One of them is: when building

>beer bottle capacitors, do they need to be shook up after letting them

>sit for a day or so to keep the salt in solution? If so, how often do

>they need to be shook up?

I used to use SW caps, and am now building rolled caps.  Those SW caps

outta work alright until you want more output.  In any case, I never found

it nessecary to "shake" my caps.  I did not let them sit for a day after making

them.  I did not supersaturate them, I put in 3 tablespoons of salt for every 12oz.

With 9 12oz beer bottles, the value was .0113uf.  Six months later the value is .0110uf.  Very small difference, not even worth worrying about.

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Faraday cage

Date: Fri, 16 Jan 1998 20:46:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Fri, 16 Jan 1998 20:46:18 +0000

Subject: Faraday cage

Will a Faraday cage block e-m fields? (such as those emitted by a

pulsing power supply causing ripples in the display of a monitor)

And, if so, what are the critical elements in constructing one?

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary materials

Date: Sat, 17 Jan 1998 17:28:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 17 Jan 1998 17:28:59 +0000

Subject: primary materials

What sort of materials should I avoid using when building a flat

primary?  I was considering a plywood platform, and vinyl tubing for

spacing.  I'm guessing the vinyl tubing would be OK, but what about

the plywood?

And, further, what sort of characteristics should I avoid in

materials that will be in, or cutting across my primary field?

Closed, grounded loops are a no-no, but what about non-grounded

stuff?  Anything to watch out for there?

Thanks

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: transformer current

Date: Sat, 17 Jan 1998 21:00:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sat, 17 Jan 1998 16:00:08 -0500

Subject: transformer current

can someone tell me the best way to find out the current rating

of a transformer. I have 4 identical ones without labels.

(small microwave units)

I get 121.65 Ohms across the secondary and the inputs are 120volt 60Hz.

I am looking for a formula to figure current so I can use these for a coil.

I am replacing 4 larger ones on a coil in hopes of needing less caps.

thanks in advance

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary Design

Date: Sun, 18 Jan 1998 04:44:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 18 Jan 1998 04:44:56 +0000

Subject: Primary Design

Would someone do me the favor of checking my numbers to see if I've

figured everything correctly for a flat primary?

The resonant freq of my secondary is ~ 175Khz loaded (8" sphere).

Here are the spec.s for my proposed primary:

ID                      7"

T2T spacing     .25"

# turns         10.7

OD                      17.2"

Ave. radius     6.05"

Which gives me 15.5 mh.  Along with a cap value of .007mf, results in

a resonant freq of 153Khz.  I think. :)

And, if these numbers are correct, are they close enough to resonate

at the freqs listed?  (I was shooting for a slightly lower primary

freq to accommodate a larger toroid topload).

TIA. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: primary materials

Cc:

Bcc:

X-Attachments:

At 05:28 PM 1/17/98 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sat, 17 Jan 1998 17:28:59 +0000

>Subject: primary materials

>

>

>What sort of materials should I avoid using when building a flat

>primary?  I was considering a plywood platform, and vinyl tubing for

>spacing.  I'm guessing the vinyl tubing would be OK, but what about

>the plywood?

>

Chris,

Plywood is no problem. I used wood as the form for

my primary with no ill effects. Many others have used

wood for primary supports with no problems.

Vinyl tubing is really unnecessary on your primary.

Just leave the turns bare. You would have to slice away

the vinyl to provide tap points.

>And, further, what sort of characteristics should I avoid in

>materials that will be in, or cutting across my primary field?

>Closed, grounded loops are a no-no, but what about non-grounded

>stuff?  Anything to watch out for there?

Avoid metal. Anything close to the primary should

be plastic or wood.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: For Sale:  Neon Transformers & HV Wire

Date: Sun, 18 Jan 1998 20:08:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tedd Payne <tpayne@netnitco.net>

Date: Sun, 18 Jan 1998 14:08:21 -0600

Subject: For Sale:  Neon Transformers & HV Wire

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your zip code (to

determine shipping cost).

Thanks,

Tedd

"CONVENTIONAL" neon transformers in clean (not old, not rusty), good

working condition.  They are not labeled, the ratings are from my

testing/measuring.

$6  used  6kv30ma     in = 2 wires     out =2 sockets

$5  used  6kv60ma     in = porc ins     output=porc insul  1 missing insul

$6  used  6kv60ma  in = porc ins  output=porc insul no lid, 1 chipped insul

$7  used  6kv60ma     in = porc ins     output=porc insul no lid

$9  used  6kv60ma     in = porc ins     output=porc insul

$9  used  7.5kv38ma    in = porc ins     output=porc insul no lid

$5  NEW   7.5kv30ma   input=3 wires    out =2 sockets (soc damaged)

$14 used  12kv25ma   in=porc ins     out=porc insul  no lid (pair avail)

$16 used  12kv30ma   in=porc ins     out=porc insul  no lid (pair avail)

$17 used  12kv30ma   in = porc ins     output=porc insul

$12 used  15kv30ma  in=porc ins  out=porc ins, no lid, missing 1 insul

$17 used  15kv30ma    in = 2 wires     output= 2 wires, w/switch

$14 used  15kv30ma  in=porc ins  out=porc ins, no lid, 1 chipped ins & bent HV terminal

$18 used  15kv30ma    in = porc ins     output=porc insul

NEW Neon transformer HV wire, flexible gray, 14 gauge, multistrand

copper, rated 15KV, $1.00 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $1.00

"ELECTRONIC" neon transformer in good working condition:

$19  used, 9kv30ma, about 20kHz(?),  SMALL SIZE 4x5.5x2 inches, LIGHT

WEIGHT about 2 pounds,  AC cord, on-off switch, 10" HV leads, good for

Jacob's ladder, neon, experimenting, but probably NOT Tesla coil.

-

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary materials

Date: Sun, 18 Jan 1998 21:25:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Sun, 18 Jan 98 16:25:13 -0500

Subject: Re: primary materials

>What sort of materials should I avoid using when building a flat

>primary?  I was considering a plywood platform, and vinyl tubing for

>spacing.  I'm guessing the vinyl tubing would be OK, but what about

>the plywood?

Plywood is fine.  Make sure that it is dry and sealed with polyurethane

before use; you don't want it absorbing any moisture.  The low cost easy

to cut nature of plywood also makes it an excellent choice for trying out

different primary shapes.  Plastics like plexiglas and polycarbonate are

also common in finished high-quality coils.  I used 1/2" Plexiglas for my

finished primary base and supports.  It is about 4 times the price of

plywood, but I find it more aesthetic- it's a matter of personal

preference, not really a performance issue.

>And, further, what sort of characteristics should I avoid in

>materials that will be in, or cutting across my primary field?

>Closed, grounded loops are a no-no, but what about non-grounded

>stuff?  Anything to watch out for there?

There are a few things to watch out for:

Don't use wet/moist wood.

Second, don't paint the primary form with anything conductive (e.g. black

paint that's colored with carbon black, or metallic paints that contain

aluminum).  Clear polyurethane is safest.  I used black polyurethane on

my wooden coil parts, but I found some that contained no conductive

pigments (no carbon).  If you are using anything other than a clear

sealant, paint a test stick and try it with a HV tranny first, to see if

it will conduct.

Third, avoid all metal- do not screw, nail or staple your form together,

use the appropriate glue for the material.  Nylon screws may be used in

the case of a plastic form.

-Adam

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: first contact

Date: Mon, 19 Jan 1998 00:25:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: bob <yubba@clara.net>

Date: Mon, 19 Jan 98 00:25:43 GMT

Subject: first contact

Organization: Your organization

Hi,

as I am new to the list I shall introduce myself. I am a mature student

studying electronic instrumentation. I have been interested in the work of tesla

for many years. This interest was rekindeled after seeing Richard Hulls group on

UK television recently in a program about lightning.  I am just about to start

on my first coil after locating a neon xmf via yellow pages under signs. Thanks

for the tip Mike! The transformer  I am buying is rated at 9 Kv 50 Ma. I am

thinking of a 4" by 24" secondary using AWG 22 wire. Does this sound about

right? My main problem might be R F I as I live in a flat in a densley populated

area, and dont want to upset the neighbours. I have access to a field miles from

anywhere but i would have to use a genarator which is where I can forsee

problems. What would be the best way to earth this sort of setup? Would the R F

current affect the RCCD on the genarator or damage it in any way. As the rating

of the transformer is about 450 watts at 240 volts what sort of effciency can I

expect to work out what size of genny to use. ? should the genny be earthed

separarly  from the R F ground or   should they be grounded together.  I know

this is a bit premature as i havent started to build the coil yet but I like to

sort these things out in advance. I have two variacs A 8 amp and a twenty amp

one that iI can use between the genny and the coil Any comments or advice very

wecome. Hope this is not to long  any feedback welcome,

bob golding

yubba@clara.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Design

Date: Mon, 19 Jan 1998 07:48:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 18 Jan 1998 23:48:01 -0800

Subject: Re: Primary Design

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Sun, 18 Jan 1998 04:44:56 +0000

> Subject: Primary Design

>

> Would someone do me the favor of checking my numbers to see if I've

> figured everything correctly for a flat primary?

>

> The resonant freq of my secondary is ~ 175Khz loaded (8" sphere).

>

> Here are the spec.s for my proposed primary:

>

> ID                      7"

> T2T spacing     .25"

> # turns         10.7

> OD                      17.2"

> Ave. radius     6.05"

>

> Which gives me 15.5 mh.  Along with a cap value of .007mf, results in

> a resonant freq of 153Khz.  I think. :)

>

> And, if these numbers are correct, are they close enough to resonate

> at the freq.s listed?  (I was shooting for a slightly lower primary

> freq to accommodate a larger toroid topload).

>

> TIA. . .Chris

>

>

Chris,

First, based upon your assumed outer diameter, (17.2") inner diameter

(7") and number of turns (10.7), when you say you'll have a turn-to-turn

spacing of 0.25", I assume you're talking about the spacing between

turns and not the center-line spacing. Usually the centerline T2T

spacing is used (in your case it looks like 0.5") when specifying the

primary.

Taking your parameters, I get about 40 uH at turn 10.7. When combined

with a tank cap of 0.007 uF, the resonant frequency would be about 300

kHz. In order to get down to 153 kHz, you'll either need to increase the

number of primary turns to about 20, or increase your primary cap to

about 0.03 uF, or some mix of the above. However, the current delivery

capability of your neon may prevent going to a 0.03 uF tank cap. Looks

like you've got an error somewhere in your calculations...

Safe coilin to you, Chris!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: first contact

Date: Mon, 19 Jan 1998 12:33:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 19 Jan 1998 07:33:43 EST

Subject: Re: first contact

Organization: AOL (http://www.aol.com)

Hi Bob,

Welcome to Tesla coil fun.  If you need a good RF ground, you could use

several grounding stakes tied together.   I cannot help you with the

generator.

You may want to consider one thing instead of the generator.  A Faraday

cage along with a good line filter (to prevent RF current from feeding back to

the AC line ) might  prove to be cheaper than buying a generator.  This would

also be more convenient than hauling all your equipment out to a field each

time you wish to operate your coil.  You would probably be wise to use a line

filter for protection of a generator as well.  The one thing that might bother

your neighbours is the noise produced by a Tesla coil.  The  sound is louder

than a arc welder.

There are many geometries used for secondary coils.  What you are

proposing will work, but I prefer to keep the height-to-diameter ratio at 4:1

or less.  This was suggested in a design manual I purchased at the

International Tesla Society museum(in Colorado Springs, CO).

I hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Where to see pics of our killer coil!

Date: Mon, 19 Jan 1998 17:00:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 19 Jan 1998 11:00:21 -0600

Subject: Where to see pics of our killer coil!

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Pics of the dreaded fleischwulf/arcstarter coil are

now on the web.  Check out:

http://www.geocities.com/CapeCanaveral/Hangar/7697/tesla.html

Now that I'm up to 3.000 hours of webpage design, I

have achieved the rank of:

"HTML Imbecile"

Please be kind to me!  (Or I'll turn the coil back on...)

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Where to see pics of our killer coil!

Cc:

Bcc:

X-Attachments:

At 05:00 PM 1/19/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Mon, 19 Jan 1998 11:00:21 -0600

>Subject: Where to see pics of our killer coil!

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Pics of the dreaded fleischwulf/arcstarter coil are

>now on the web.  Check out:

>

>http://www.geocities.com/CapeCanaveral/Hangar/7697/tesla.html

Bill,

Great pics.

One suggestion on your spark pictures is to use a little

more room light and a shorter exposure. With a shorter exposure

the individual streamers stand out better. I find that exposures

around 1/4 sec work out well with ASA400 film. Set F stop according

to room light and let sparks take care of themselves.

>

>Now that I'm up to 3.000 hours of webpage design, I

>have achieved the rank of:

>

>                      "HTML Imbecile"

You need only 3 more hours of webpage design to come

up to my level of:

"HTML Moron"

>Please be kind to me!  (Or I'll turn the coil back on...)

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: first contact

Cc:

Bcc:

X-Attachments:

At 12:25 AM 1/19/98 GMT, you wrote:

>From: bob <yubba@clara.net>

>Date: Mon, 19 Jan 98 00:25:43 GMT

>Subject: first contact

>Organization: Your organization

>

>Hi,

> as I am new to the list I shall introduce myself. I am a mature student

>studying electronic instrumentation. I have been interested in the work of tesla

>for many years. This interest was rekindeled after seeing Richard Hulls group on

>UK television recently in a program about lightning.  I am just about to start

>on my first coil after locating a neon xmfr via yellow pages under signs. Thanks

>for the tip Mike! The transformer I am buying is rated at 9 Kv 50 Ma. I am

>thinking of a 4" by 24" secondary using AWG 22 wire. Does this sound about

>right?

Best wire guage for a 4 inch is AWG 24 with a winding length of 20-22 inches.

Mine has a 21 inch winding length.

AWG 22 can be used. My first 4 inch secondary was wound with

21 inches of AWG 22. It performed well but I got better performance

from using the AWG 24.

>My main problem might be R F I as I live in a flat in a densely populated

>area, and dont want to upset the neighbours.

This is usually not a problem. If you use some RF chokes in the HV lines

and an AC line filter before your neon transformer you should be able

to keep the RF out of the AC mains. Direct RF radiation from your coil

will only travel a short distance. Usually not more than 100 feet or so.

> I have access to a field miles from anywhere but i would have to use a genarator

>which is where I can forsee problems. What would be the best way to earth this sort of setup?

Ground rods driven into moist soil.

> Would the RF current affect the RCCD on the generator or damage it in any way?

> As the rating of the transformer is about 450 watts at 240 volts what sort of

> effciency can I expect to work out what size of genny to use?

> should the genny be earthed separately from the R F ground or should they be grounded

> together.  I know this is a bit premature as i havent started to build the coil yet but

> I like to sort these things out in advance.

I dont think it will be necessary to use a generator at a remote site for

your coil. I operate my coil in the basement of my home and have no problems

with RFI except with my cordless telephone. And I always turn off and disconnect

the computer before running the coil.

>I have two variacs A 8 amp and a twenty amp

>one that iI can use between the genny and the coil Any comments or advice very

>wecome. Hope this is not to long  any feedback welcome,

>      bob golding

> yubba@clara.net

>

Welcome aboard Bob.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: caps

Date: Tue, 20 Jan 1998 04:17:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: torghn@juno.com (Jared Neilsen P Neilsen)

Date: Mon, 19 Jan 1998 21:17:54 -0700

Subject: caps

I have seen many post about different types of capacitors, and would like

to know the differences between all the many types that one can build.

Also I have only seen info on how to build the beer bottle SW caps, so if

any one has some good information on the different types of capacitors

and how to build them I would greatly appreciate it if they would let me

know where I can find it.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My coil pictures are up.

Date: Tue, 20 Jan 1998 05:59:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Tue, 20 Jan 98 00:59:55 -0500

Subject: My coil pictures are up.

Hi all, I have finally slapped together a crude webpage showing my 1kVA

coil setup and the results.  I have included a full set of shots of the

actual component layout on the unit's base.  I've been to a lot of

coilers' pages, and I always find myself wishing there were shots of the

"works" as well as the arcs, so I am serving some up.

The page is here:

http://www.tiac.net/users/absmith/tesla/tcpicts.html

Enjoy, and let me know what you think!

-Adam

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: caps

Date: Tue, 20 Jan 1998 13:24:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Tue, 20 Jan 1998 07:24:51 -0600 (CST)

Subject: Re: caps

On Tue, 20 Jan 1998, Tesla-2 wrote:

> From: torghn@juno.com (Jared Neilsen P Neilsen)

> Date: Mon, 19 Jan 1998 21:17:54 -0700

> Subject: caps

>

> I have seen many post about different types of capacitors, and would like

> to know the differences between all the many types that one can build.

>

Hey, Jared.  I'm also new at capacitor construction.  One place I've found

that describes several different designs is on Bill Beaty's home page at

the following address...

http://www.eskimo.com/~billb/tesla/tesla.html#tcaps

Hope this helps.

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: caps

Cc:

Bcc:

X-Attachments:

At 04:17 AM 1/20/98 GMT, you wrote:

>From: torghn@juno.com (Jared Neilsen P Neilsen)

>Date: Mon, 19 Jan 1998 21:17:54 -0700

>Subject: caps

>

>I have seen many post about different types of capacitors, and would like

>to know the differences between all the many types that one can build.

>Also I have only seen info on how to build the beer bottle SW caps, so if

>any one has some good information on the different types of capacitors

>and how to build them I would greatly appreciate it if they would let me

>know where I can find it.

>

Some of the various types of homebuilt caps are saltwater filled bottle

caps, glass plate, rolled polyethylene, flat plate polyethylene.

The saltwater cap is patterned after the famous Leyden jar.

A glass bottle serves as the dielectric and the plates are composed

of saltwater solution. These caps are cheap and easy to build but

are heavy, bulky and deliver poor performance.

Glass plate caps use alternating layers of aluminum foil and sheets

of glass. This does away with the need for saltwater but the performance

is still poor due to the poor RF dissipation factor of glass.

Substituting polyethylene for the sheets of glass results in a high

performance low cost cap. The flat plate cap has alternating layers of

aluminum foil or flashing for the plates and polyethylene plastic for

the dielectric. This type of cap needs to be submerged in mineral oil

or transformer oil to exclude air.

The rolled cap uses long strips of polyethylene and foil or flashing

rolled up tightly. This is usually placed in a tank made from PVC pipe.

This type of cap also needs to be in oil. The rolled type cap offers

somewhat more capacitance in a smaller package. Its performance

is good but not quite as good as the flat plate cap.

This is just a simple overview of homebuilt cap types.

More detailed information can be found in the list archives

as well as in various text files with construction information.

The archives as well as various cap building files can be downloaded

from Dave Bronakowski's archive site. The URL is:

http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: transformer current

Date: Wed, 21 Jan 1998 05:26:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gretchen Kulpa <gwkulpa@earthlink.net>

Date: Tue, 20 Jan 1998 21:26:12 -0800

Subject: Re: transformer current

Tesla-2 wrote:

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Sat, 17 Jan 1998 16:00:08 -0500

> Subject: transformer current

>

> can someone tell me the best way to find out the current rating

> of a transformer. I have 4 identical ones without labels.

> (small microwave units)

> I get 121.65 Ohms across the secondary and the inputs are 120volt 60Hz.

> I am looking for a formula to figure current so I can use these for a coil.

> I am replacing 4 larger ones on a coil in hopes of needing less caps.

> thanks in advance

> Chris Watkins

> Chris, My name is Walt. I am gathering materials to build my first coil.

I am starting out with a neon transformer design but am also very

interested in building a coil with a microwave or t.v. flyback

transformer. can you or anyone out there direct me to a good location

with complete instructions on building a small unit with this type of

transformer? Thanks, Walt

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Newbie

Date: Wed, 21 Jan 1998 18:55:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Wed, 21 Jan 1998 12:55:43 -0600 (CST)

Subject: Newbie

Hello, all.  My name is Tim Osmulski.  I've always been interested in

electric and electronic devices.  I became interested in Tesla coils after

seeing a picture of one on the web.

I collected the archives to this list, and from there began construction

on my first coil.  I have pictures at the following site...

http://hpcl.cs.ttu.edu/~tim/tesla

I hope to have it tuned within a week.  Comments and criticisms are

welcome.

Working on the budget of a poor, starving college kid isn't easy.  I

intend to keep making minor improvements to the coil, as time and money

allow.

Tim

hpcl.cs.ttu.edu/~tim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Newbie

Cc:

Bcc:

X-Attachments:

At 06:55 PM 1/21/98 GMT, you wrote:

>From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

>Date: Wed, 21 Jan 1998 12:55:43 -0600 (CST)

>Subject: Newbie

>

>Hello, all.  My name is Tim Osmulski.  I've always been interested in

>electric and electronic devices.  I became interested in Tesla coils after

>seeing a picture of one on the web.

>

>I collected the archives to this list, and from there began construction

>on my first coil.  I have pictures at the following site...

>http://hpcl.cs.ttu.edu/~tim/tesla

>I hope to have it tuned within a week.  Comments and criticisms are welcome.

>

>Working on the budget of a poor, starving college kid isn't easy.  I

>intend to keep making minor improvements to the coil, as time and money

>allow.

>

>Tim

>hpcl.cs.ttu.edu/~tim

Welcome aboard Tim.

Nice looking coil. I would say you have done a fine job.

Once you get up and running take some spark pictures.

Only criticism I can make is your RF ground.

You need some improvements in this area.

I've found that two 4 foot ground rods are quite sufficient

if the ground lead to the base of the secondary is short.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: another newbie

Date: Thu, 22 Jan 1998 05:51:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 21 Jan 1998 21:51:20 -0800

Subject: another newbie

Hi everyone! I am another newbie who until now has has not exhibited

proper manners and made a proper introduction. My name is Walt Kulpa and

I live in Bellingham Washington. I have always had an intrest in

electricity - went to tech school in the mid 70's to become an

electrician and have been working in the field ever since. I did some

reading on Tesla about 10 years ago and have been facinated by him ever

since. It has only been in the past few weeks that I went to research

what was available on the net in regard to tesla coils and am very

pleasently impressed with what's going on here. I'm rabidly looking for

information and materials to construct my first coil. unfortunately, I'm

not nearly net savy enough to put pictures up on the net at this time,

but I guess that, that will also happen in time.

looking forward to following the the discussions and information from you

folks as it comes my way. best regards, walt

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: A really really big capacitor...

Date: Thu, 22 Jan 1998 14:31:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 22 Jan 1998 08:31:49 -0600

Subject: A really really big capacitor...

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Check this site out:

http://www.wlmn.eglin.af.mil/public/facility/hyper.html

These guys have an 11,000 volt 4.8 million joule capacitor!

That works out to 0.079 F, or 79000 uF!

What kind of transformer would you need for resonant charging of that one??? :)

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A really really big capacitor...

Cc:

Bcc:

X-Attachments:

At 02:31 PM 1/22/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Thu, 22 Jan 1998 08:31:49 -0600

>Subject: A really really big capacitor...

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Check this site out:

>

>http://www.wlmn.eglin.af.mil/public/facility/hyper.html

>

>These guys have an 11,000 volt 4.8 million joule capacitor!

>

>That works out to 0.079 F, or 79000 uF!

>

>What kind of transformer would you need for resonant charging of that one??? :)

>

>-Bill

>

A really really big transformer. A couple quick guesstimates

figure 12kv at around 360,000 AMPS should do it.

Hmm. Probably need your own nuclear power plant too.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: A way to build an oil-filled cap

Date: Thu, 22 Jan 1998 21:31:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 22 Jan 1998 15:31:20 -0600

Subject: A way to build an oil-filled cap

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I've heard about people who have tried to build caps

using just metal plates and a dielectric oil.

One of the big problems is how to keep the plates

separated and properly aligned.

Does anyone know if you can buy polyethylene

window screen or other poly screen/mesh structures?

How about poly monofilament??  Any sources for this stuff?

Then you could build this type of plate cap using a

few sheets of poly mesh between the plates.  This

way - perhaps 90% of the space is open - and could

be filled with your castor oil or whatever dielectric oil

you choose.

Has anyone tried this??

Does anyone know what regular window screen (the

plastic type) is made of?  I would think it is probably

something high-loss.  But it'd be worth a shot...

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: A way to build an oil-filled cap

Date: Thu, 22 Jan 1998 22:38:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Thu, 22 Jan 1998 14:38:25 -0800 (PST)

Subject: Re: A way to build an oil-filled cap

> Coilers,

>  I've heard about people who have tried to build caps

> using just metal plates and a dielectric oil.

> Does anyone know what regular window screen (the

> plastic type) is made of?  I would think it is probably

> something high-loss.  But it'd be worth a shot...

Usually, window screen is made of one two things.. one lossy,

the other coductive (though rare now)

Either: Some form of plastic coated fiberglass

Or    : Aluminum < not your best choice :) >

But: something to consider: Weedblock material?

Personally, If I was going to do this, I'd use rigid materal

and 'slotting' to mount the plates. Plate size would have to

be such that the bowing caused by field attraction/repulsion

would not result in shorting. This would be a function

of the thickness of the material.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: A way to build an oil-filled cap

Date: Fri, 23 Jan 1998 01:08:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 22 Jan 1998 20:08:25 EST

Subject: Re: A way to build an oil-filled cap

Organization: AOL (http://www.aol.com)

Bill,

For the type of capacitor you are talking about, I would think about using

"scotch brite" pads to separate the plates.  You could then compress the whole

setup to vary the capacitance to suit your needs.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A way to build an oil-filled cap

Cc:

Bcc:

X-Attachments:

At 09:31 PM 1/22/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Thu, 22 Jan 1998 15:31:20 -0600

>Subject: A way to build an oil-filled cap

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I've heard about people who have tried to build caps

>using just metal plates and a dielectric oil.

>

> One of the big problems is how to keep the plates

>separated and properly aligned.

Been there. Done that.

While it is possible to build such a cap in practice

it doesnt work out too well. Such a cap would have no advantage

over a poly cap and several disadvantages.

First you need very rigid plates. A Tesla cap under goes

some pretty severe pulsing action that vigorously vibrates

the plates. I used some plates of galvanized steel and nylon

bolts and washers to assemble the unit.

You will need much wider plate spacing than with a poly cap.

You can count on poly having a break down voltage of around

350 - 375 volts AC per mil. Typical mineral based oils have

breakdowns closer to 200 VAC per mil and some transformer

oils are as low as 75 VAC per mil.

This wider plate spacing means lower capacitance so requires

larger and/or more plates.

Many believe that an oil cap would be self healing in case of

a breakdown. Thats only partially true. A heavy arc will

often damage a plate leaving a sharp point or burr. This

will promote further breakdowns.

Also an arc through oil will leave a plume of carbon particles

suspended in the oil.  This will seriously degrade the breakdown

rating. It will only take a few such breakdowns before the entire

batch of oil is contaminated and needs to be replaced.

>

>Does anyone know if you can buy polyethylene

>window screen or other poly screen/mesh structures?

Window screen is usually fiberglass or vinyl or aluminum.

None of which sound too appealing.

>

> How about poly monofilament??  Any sources for this

>stuff?

Dont know of a source for this.

>-Bill

>

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: A way to build an oil-filled cap

Date: Sat, 24 Jan 1998 00:39:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Fri, 23 Jan 1998 18:39:08 -0600

Subject: Re: A way to build an oil-filled cap

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I received many interesting replies regarding the

construction of oil plate caps w/o using poly, etc.

All of the responses indicated novel material porous

materials to place between the plate to act as a

separator.  In no particular order, here they are:

-weedmat cloth, available at the hardware store.

-Scotchbright pads - makes for a compressable

(read - adjustable) capacitance!

-fishing line !

-fiberglass cloth - from an auto body or boat shop

-mica sheets - available from McMaster-Carr!

But one of the best ideas was to use a heap of

plain old double-sided PC boards, available at a

surplus place or mail-order!

This seems quite obvious!  Someone must have tried this.

Tell us - Did it work????

The only way I could see it might fail is if the board

material itself wasn't thick enough to put up with

the voltage.  The problem of the arcs wrapping over

the edges of the pcbs could be solved by etching off

the perimeters of the board!!

Just kicking it around...

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: First Tesla coil run

Date: Sat, 24 Jan 1998 03:01:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Fri, 23 Jan 1998 21:01:37 -0600 (CST)

Subject: First Tesla coil run

Recently I fired up my first coil and tuned it out.  The specs of the coil

are available at hpcl.cs.ttu.edu/~tim/tesla.  Basically, it's a 4" coil

driven by a 15K/30 France using bottle caps.  I tried tuning it with a

4x14 toroid, but I was unable to make it break out without a nail in the

side.  So, I tuned it with a 4" yarn ball wrapped in aluminum foil.  The

best tune I could obtain was with 6 bottles, primary tapped at turn 8 (of

14), giving me several 9" sparks from the crumpled aluminum surface.  It's

obvious that I'm not achieving great results.  Even after retuning for

the toroid I was unable to make it break out without the nail.  I have

several theories as to why.  I'd like to know how much each of the

following is contributing to my lack of performance:

1) My 15K/30 only puts out 450VA.  Is this the best I can expect until I

get more power?  The 15K/30 Jefferson I was attempting to repot is

history.  The thing was so smoked that the secondary crumbled when I got

in to it.  I've got a line on several other transformers, though.  So more

power is soon to come.

2) I have glass caps.  Is glass really THAT lossy so as to steal the

energy before can be transfered to my seconday?  The bottles don't feel

warm after use.  My first rebuild job will be the caps, so if this is a

major source of power loss, I'll get started on it.

3) My ground stake is pitiful.  It's only 18" of copper pipe hammered into

the ground.  This probably isn't enough to sink all that current.  I'm

going to get two 4' grounding stakes at my first opportunity.

4) The spark gap is probably less than 0.25".  Also, there's no air

blowing through it.  However, the pipes are oriented vertically to allow

heat to rise out of the tubes.  After rebuilding the caps, this is my

second priority.  I'm going to make an RQ style with a strong fan.

5) Changing the coupling seems to have little effect.  I raise and lower

the secondary by a total of 3", with little change in performance.  Is

this very important to coil performance or is it just a matter of

preventing arc-overs?

I'm definitely getting resonance of some sort.  The effect is strong

enough that charge developes on the exterior of the secondary.  More than

enough to get your attention if you're not expecting it.

Speaking of getting zapped, I got brave and held a small florescent tube

close to the discharge terminal.  I can't say it wasn't painful.  Not

painful enough to rule out trying again, of course, but I think I may have

some serious DC or AC stuff in there.

Any comments or criticisms are welcome.  I'll try to have pictures of my

working coil up before the end of Monday.  I'll post when I get them up.

Thanks for you opinions,

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: First Tesla coil run

Date: Sat, 24 Jan 1998 05:47:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Fri, 23 Jan 1998 23:47:23 -0600

Subject: Re: First Tesla coil run

>14), giving me several 9" sparks from the crumpled aluminum surface.  It's

>obvious that I'm not achieving great results.  Even after retuning for

Ah don't worry.  I still get that 6 months after my first coil, heheheh...

>1) My 15K/30 only puts out 450VA.  Is this the best I can expect until I

Correction - your transformer only puts out 30ma of current.  It PULLS 450VA

from the wall.  450VA is irrelevant.

>2) I have glass caps.  Is glass really THAT lossy so as to steal the

>energy before can be transfered to my seconday?  The bottles don't feel

>warm after use.  My first rebuild job will be the caps, so if this is a

>major source of power loss, I'll get started on it.

They are lossy, but I have gotten 41" inches out of my bottle caps.  You

should test the value of your caps, they should be around .0053uf.

>3) My ground stake is pitiful.  It's only 18" of copper pipe hammered into

This is a problem.  Make sure the wire connecting to the ground rod from

your coil is a thick wire (I use awg #6 for mine)

>4) The spark gap is probably less than 0.25".  Also, there's no air

>blowing through it.  However, the pipes are oriented vertically to allow

Also a MAJOR problem.  I ran my coil, got 30 inches on average and the

fan wire came lose.  My sparks ceased to less than 6 inches.  Air helps

quench the gap, even if it is not yet hot.

>5) Changing the coupling seems to have little effect.  I raise and lower

>the secondary by a total of 3", with little change in performance.  Is

>this very important to coil performance or is it just a matter of

>preventing arc-overs?

Mainly preventing arc-overs.  Of course it will affect the coupling, but at

small power levels, you won't notice the change.

>Speaking of getting zapped, I got brave and held a small florescent tube

>close to the discharge terminal.  I can't say it wasn't painful.  Not

>painful enough to rule out trying again, of course, but I think I may have

>some serious DC or AC stuff in there.

Of course you have AC in there.  You mean 60hz.  Its unlikely.  All coils

shock the hell out of you if you touch them.  You've no doubt seen pictures

of people touching the sparks.  They are doing this is certain ways as to

"lessen" the pain.  Here's a weird one for you to try.  Turn it on and quickly

place your hand flat on the toroid.  You'll have all that energy going through

you but you won't feel it at ALL.  The sparks will shock you, but once you

put your hand on the terminal, you can no longer feel it.

Jeff Corr

corr@mail.enid.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Tesla coil run

Cc:

Bcc:

X-Attachments:

At 03:01 AM 1/24/98 GMT, you wrote:

>From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

>Date: Fri, 23 Jan 1998 21:01:37 -0600 (CST)

>Subject: First Tesla coil run

>

>Recently I fired up my first coil and tuned it out.  The specs of the coil

>are available at hpcl.cs.ttu.edu/~tim/tesla.  Basically, it's a 4" coil

>driven by a 15K/30 France using bottle caps.  I tried tuning it with a

>4x14 toroid, but I was unable to make it break out without a nail in the

>side.  So, I tuned it with a 4" yarn ball wrapped in aluminum foil.  The

>best tune I could obtain was with 6 bottles, primary tapped at turn 8 (of

>14), giving me several 9" sparks from the crumpled aluminum surface.  It's

>obvious that I'm not achieving great results.  Even after retuning for

>the toroid I was unable to make it break out without the nail.  I have

>several theories as to why.  I'd like to know how much each of the

>following is contributing to my lack of performance:

Actually 9 inches isnt bad for a start. I usually recommend that

people start with a small spherical terminal rather than a large

toroid to start. A sphere breaks out at low power easily. Once

you have some spark it becomes much easier to find the tune.

The failure of your toroid to break out is due to the low amount

of power in your secondary. Improving efficiency will get that power

level up.

Try a larger spherical terminal. Try a 6 inch diameter styrofoam ball

covered with foil. This will lower your resonant frequency a little

and allow you to tap in a couple more primary turns. This should improve

your efficiency.

>

>1) My 15K/30 only puts out 450VA.  Is this the best I can expect until I

>get more power?

No. This is plenty of power to start. I can get in excess of 20 inches

of spark from a 12kv/30ma neon on my 4 inch coil. Your 450VA should

be plenty for 24 inches. It takes more than input power to get long sparks.

At this point increasing your input power will have very little effect.

>

>2) I have glass caps.  Is glass really THAT lossy so as to steal the

>energy before can be transfered to my secondary?  The bottles don't feel

>warm after use.  My first rebuild job will be the caps, so if this is a

>major source of power loss, I'll get started on it.

Yes. Glass IS that lossy. Bottle caps are a great way to start simple

and cheap but should be replaced with poly caps as a first priority.

>

>3) My ground stake is pitiful.  It's only 18" of copper pipe hammered into

>the ground.  This probably isn't enough to sink all that current.  I'm

>going to get two 4' grounding stakes at my first opportunity.

Very important. Although at your present level you probably will

not see a big improvement. Your gap loss and the dielectric loss

in your caps are your biggest source of loss right now.

>

>4) The spark gap is probably less than 0.25".  Also, there's no air

>blowing through it.  However, the pipes are oriented vertically to allow

>heat to rise out of the tubes.  After rebuilding the caps, this is my

>second priority.  I'm going to make an RQ style with a strong fan.

.25 inch of gap is plenty. Dont get into the habit of wide gaps.

Sure you may get a little more output with a wide gap but you will

only succeed in damaging your neon. Wide gaps are not necessary

for long sparks. I have never run any more than .225 inch of gap

at 12kv. For 15kv you should never exceed .300 inch.

Dont forget the safety gap. Set it just a little wider than your main gap.

Airflow through the gap is important not only for heat removal but

to aid the quenching of the gap. Gap quenching is one of the most

important aspects of coil operation. The airflow removes hot ions

from between the gap faces that are clogging up the arc channel.

>

>5) Changing the coupling seems to have little effect.  I raise and lower

>the secondary by a total of 3", with little change in performance.  Is

>this very important to coil performance or is it just a matter of

>preventing arc-overs?

At your low power level you probably arent not seeing the signs of

overcoupling - corona and arcing. The coupling will make dramatic

changes in output. For now leave the secondary down pretty close

to the primary for best coupling. As your efficiency and power output

improve you may need to raise the secondary to prevent arcing.

>

>

>I'm definitely getting resonance of some sort.  The effect is strong

>enough that charge developes on the exterior of the secondary.  More than

>enough to get your attention if you're not expecting it.

The charge on your secondary is static electricity. Nothing to do

with resonance.

Go through all your connections and wiring and improve things with an eye

toward handling more current. Use heavy solid conductors in the primary tank

circuit. After a coil run feel all the wiring and connections with the power off.

Notice any heating. Any conections that show signs of heating need to be

beefed up. The primary coil tap is often a source of loss.

Correct any signs of corona. Corona represents power loss.

Add airflow to your gap. This is very important.

Keep up the good work and take plenty of notes for future reference.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A way to build an oil-filled cap

Cc:

Bcc:

X-Attachments:

At 12:39 AM 1/24/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Fri, 23 Jan 1998 18:39:08 -0600

>Subject: Re: A way to build an oil-filled cap

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I received many interesting replies regarding the

>construction of oil plate caps w/o using poly, etc.

>All of the responses indicated novel material porous

>materials to place between the plate to act as a

>separator.  In no particular order, here they are:

>

<SNIP>

>But one of the best ideas was to use a heap of

>plain old double-sided PC boards, available at a

>surplus place or mail-order!

>

>This seems quite obvious!  Someone must have tried this.

>Tell us - Did it work????

>

Been there. Done that. Doesnt work too well in practice.

The breakdown voltage is low. The board material is

fiberGLASS. So the RF dissipation is high. The amount

of capacitance obtained isnt great.

>Just kicking it around...

>

>-Bill

Keep kicking.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: First Tesla coil run

Date: Sat, 24 Jan 1998 23:25:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 24 Jan 1998 15:25:43 -0800 (PST)

Subject: Re: First Tesla coil run

On Sat, 24 Jan 1998, Tesla-2 wrote:

> From: Jeff Corr <corr@enid.com>

> >5) Changing the coupling seems to have little effect.  I raise and lower

> >the secondary by a total of 3", with little change in performance.  Is

> >this very important to coil performance or is it just a matter of

> >preventing arc-overs?

>

> Mainly preventing arc-overs.  Of course it will affect the coupling, but at

> small power levels, you won't notice the change.

Hmmm, I'm not so sure I agree with this - I am currently running

at 720VA - yet a difference of 2" makes a major difference in output.

[On my small coil, [360VA input] 2" of coupling difference added

almost 4" to the output.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Any hope here?

Date: Sat, 24 Jan 1998 23:46:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 24 Jan 1998 15:46:23 -0800 (PST)

Subject: Any hope here?

Well, I am feeling a little defeated.. I think I got a bit ambitious.

Fired up my new coil today after rebuildng the primary, and building

a new topload, and could not get breakout to save my life. With

a bump on the outside edge I did mangage to get regular 30" arcs.

I'd love to have this beast firing without the bump though.

So I am open for suggestions!!!

Parameters:

Primary: 13 1/2 turns, tapped at 12 3/8 [ yes, the Q of this

system is sensitive to 1/8 turn tapping] 8.25" inner

0.25" tubing, 0.25" spacing, 30degree rise.

Secondary: 6.25" x 25" 22AWG

Toroid: 15" center-to-center, 6" Aluminum duct [covers top

2 inches of secondary at the moment]

Gap: RQ style [1" pipe, in 2" segments, my 3" based one is

being rebuilt]

Cap: Total of 0.0135 uf of homebrew poly plate.

Power: 2 12KV/30ma NST in parallel.

I am wondering if my topload is just too big, and if I should make a

4" based one? or if there is some trick of tuning I could use to get

breakout without the bump?

[Note that this same coil, with a 12"x3" torid produces several

18" arcs at the same time.]

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Any hope here?

Cc:

Bcc:

X-Attachments:

At 11:46 PM 1/24/98 GMT, you wrote:

>From: Michael Baumann <baumann@proton.llumc.edu>

>Date: Sat, 24 Jan 1998 15:46:23 -0800 (PST)

>Subject: Any hope here?

>

>Well, I am feeling a little defeated.. I think I got a bit ambitious.

>Fired up my new coil today after rebuildng the primary, and building

>a new topload, and could not get breakout to save my life. With

>a bump on the outside edge I did mangage to get regular 30" arcs.

>I'd love to have this beast firing without the bump tho-

>So I am open for suggestions!!!

<snip specs>

>I am wondering if my topload is just too big, and if I should make a

>4" based one? or if there is some trick of tuning I could use to get

>breakout without the bump?

I would say that the 6 inch cross section toroid might be

a bit large for your power level. Before I went to a 6 inch thick

toroid I was running 12kv/90ma and getting 38 inch streamers

with a 16 X 4 toroid.

I got to the point that I was getting multiple short streamers

rather than 1 long one. This clearly indicated a need for a larger

toroid.

I use a bump on my 19 X 6 toroid to provide directional

control over the streamers due to the cramped space I run

the coil in.

You didnt mention in your specs what your gap spacing was.

If I remember correctly your gap was a little on the conservative

side. A small increase in gap spacing will boost your primary

energy level a bit and that might be enough for breakout.

I use .225 inch of gap at 12kv divided into 9 individual gaps.

If you are running less than this then you should add another electrode

to your gap. If you are at or above this amount of gap then you

probably have enough.

Despite your problem 30 inches with 12kv/60 is pretty good.

Best I got with 12kv/60 was 34 inches.

>--

>Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

>San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Any hope here?

Date: Sun, 25 Jan 1998 03:58:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 24 Jan 1998 19:58:21 -0800

Subject: Re: Any hope here?

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Sat, 24 Jan 1998 15:46:23 -0800 (PST)

> Subject: Any hope here?

>

> Well, I am feeling a little defeated.. I think I got a bit ambitious.

> Fired up my new coil today after rebuildng the primary, and building

> a new topload, and could not get breakout to save my life. With

> a bump on the outside edge I did mangage to get regular 30" arcs.

> I'd love to have this beast firing without the bump though.

> So I am open for suggestions!!!

>

> Parameters:

> Primary: 13 1/2 turns, tapped at 12 3/8 [ yes, the Q of this

>          system is sensitive to 1/8 turn tapping] 8.25" inner

>          0.25" tubing, 0.25" spacing, 30degree rise.

> Secondary: 6.25" x 25" 22AWG

> Toroid: 15" center-to-center, 6" Aluminum duct [covers top

>         2 inches of secondary at the moment]

> Gap: RQ style [1" pipe, in 2" segments, my 3" based one is

>       being rebuilt]

> Cap: Total of 0.0135 uf of homebrew poly plate.

> Power: 2 12KV/30ma NST in parallel.

>

> I am wondering if my topload is just too big, and if I should make a

> 4" based one? or if there is some trick of tuning I could use to get

> breakout without the bump?

> [Note that this same coil, with a 12"x3" toroid produces several

>  18" arcs at the same time.]

>

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

You may need to go with a bit smaller diameter drier duct - your peak

voltage may not be quite enough to break out with 6" ducting. Try 4"

ducting instead with the same larger diameter and it should sing! Save

the 6" toroid for when you bump the tank cap or input power level up a

bit higher..

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Any hope here?

Date: Sun, 25 Jan 1998 05:08:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 24 Jan 1998 21:08:50 -0800 (PST)

Subject: Re: Any hope here?

On Sun, 25 Jan 1998, Tesla-2 wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 24 Jan 1998 20:37:42 -0600

> Subject: Re: Any hope here?

>

<insert some major snippage>

>

> You didnt mentioned in your specs what your gap spacing was.

> If I remember correctly your gap was a little on the conservative

> side. A small increase in gap spacing will boost your primary

> energy level a bit and that might be enough for breakout.

I hadn't mentioned it as I was not exactly sure what it was.

Lacking feeler gauges, I just did a cheater's method of using

stacked paper and calipers <I know.. crazy, I have calipers

but not a feeler. *shrug*> and found I was using

a gap of around 0.13 [I'll take my gap setup to work and

get better measurements there] I guess my safety gap is way

too conservative as well (currently set at approx 0.095 on

each side to ground)

One problem I have been having is that the coil will run

*great* cold, but after the gap warms up a little, the SG starts

firing a *lot*, so I have gapped down. Any idea what is causing this

rather interesting phenomenon?

>

> I use .225 inch of gap at 12kv divided into 9 individual gaps.  If you

> are running less than this then you should add another electrode to your

> gap. If you are at or above this amount of gap then you probably have

> enough.

>

Not even close.. so perhaps there is hope after all! The gap system

I built has a total of .216 available, in 8 gaps [Assuming my rough

measurments are even close]. But I would have to open my SG substantially,

and you know how much I really want to do that.

[For those that did not know, I blew my first NST inside of 5 minutes

of operation on my small coil. And I had paid full price for it before

I learned of a) Safety gaps, and b) scrounging 'blown' NSTs from shops]

>

> Despite your problem 30 inches with 12kv/60 is pretty good.  Best I got

> with 12kv/60 was 34 inches.

Thanks for the complement. Oh.. I tried the CD-rom trick mentioned

on beatty's homepage - definitely fun thing to do with AOL disks!

Blows the aluminum and ink right off the back, leaving an almost clear

disk. I'm using them for coasters - nice conversation starter.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: spark gap question

Date: Sun, 25 Jan 1998 17:36:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Sun, 25 Jan 1998 12:36:29 EST

Subject: spark gap question

Organization: AOL (http://www.aol.com)

My static spark gap consists of eight 1/4" brass bolts housed in a box with a

muffin fan at one end. I just finished a salt water cap and when I first

hooked it up I was getting very strong, loud arcs across all four gaps. Now I

am getting very weak arcs across one or two gaps at best. Any suggestions as

to my problem?

Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Any hope here?

Cc:

Bcc:

X-Attachments:

At 05:08 AM 1/25/98 GMT, you wrote:

>From: Michael Baumann <baumann@proton.llumc.edu>

>Date: Sat, 24 Jan 1998 21:08:50 -0800 (PST)

>Subject: Re: Any hope here?

>

>On Sun, 25 Jan 1998, Tesla-2 wrote:

>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Sat, 24 Jan 1998 20:37:42 -0600

>> Subject: Re: Any hope here?

>>

><insert some major snippage>

>>

>> You didnt mentioned in your specs what your gap spacing was.

>> If I remember correctly your gap was a little on the conservative

>> side. A small increase in gap spacing will boost your primary

>> energy level a bit and that might be enough for breakout.

>I hadn't mentioned it as I was not exactly sure what it was.

>Lacking feeler gauges, I just did a cheater's method of using

>stacked paper and calipers <I know.. crazy, I have calipers

>but not a feeler. *shrug*> and found I was using

>a gap of around 0.13 [I'll take my gap setup to work and

>get better measurements there] I guess my safety gap is way

>too conservative as well (currently set at approx 0.095 on

>each side to ground)

>

Michael. .13 inch is pretty low for 12kv. Ive run more gap

than that at 9kv with no problem. You will certainly benefit

from a wider gap. You will need to widen your safety also

to accomodate the increased primary voltage. Set your safety

just a bit wider than your main gap.

>One problem I have been having is that the coil will run

>*great* cold, but after the gap warms up a little, the SG starts

>firing a *lot*, so I have gapped down. Any idea what is causing this

>rather interesting phenomenon?

Poor quenching due to heat build up. Not necessarily the electrodes

themselves getting hot but the arc channel between the gap faces

clogging up with hot ions. More airflow is needed. Also increasing

the NUMBER of series gaps will help. If you are using a cylinder

gap try blocking up the center of the PVC pipe to force the airflow

around the electrodes. If the fan you are using to provide airflow

is weak get a stronger one.

>

>>

>> I use .225 inch of gap at 12kv divided into 9 individual gaps.  If you

>> are running less than this then you should add another electrode to your

>> gap. If you are at or above this amount of gap then you probably have

>> enough.

>>

>Not even close.. so perhaps there is hope after all! The gap system

>I built has a total of .216 available, in 8 gaps [Assuming my rough

>measurments are even close]. But I would have to open my SG substantially,

>and you know how much I really want to do that.

>

I would think you can open up your gap considerably without any

problem. You are probably limiting your primary voltage to

considerably less than 12kv with your narrow gap spacing.

With each increase in main gap make a commensurate increase

in your safety gap. .200 of main gap is pretty safe for 12kv.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gap question

Cc:

Bcc:

X-Attachments:

At 05:36 PM 1/25/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sun, 25 Jan 1998 12:36:29 EST

>Subject: spark gap question

>Organization: AOL (http://www.aol.com)

>

>My static spark gap consists of eight 1/4" brass bolts housed in a box with a

>muffin fan at one end. I just finished a salt water cap and when I first

>hooked it up I was getting very strong, loud arcs across all four gaps. Now I

>am getting very weak arcs across one or two gaps at best. Any suggestions as

>to my problem?

>Thanks, Jim

>

Jim,

Three thoughts come to mind.

First might be that the gap faces have oxidized heavily.

Brass is much worse in this regard than copper. Clean

up the electrode faces with some fine sandpaper and see

if that helps.

Second is that one or more of your caps may be defective.

Try each one individually to see if you can find the culprit.

Third is that you may have damaged your transformer.

If its a neon try drawing an arc from each HV terminal

to the neon case. Do this with the neon alone. Nothing else

connected to it. Each side should have a nice healthy fat

spark. Each side should be the same. If one side shows

a weak thin spark or no spark at all then thats the problem.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap question

Date: Mon, 26 Jan 1998 15:31:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 26 Jan 1998 10:31:48 -0500

Subject: Re: spark gap question

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: spark gap question

> Date: Sunday, January 25, 1998 12:36 PM

>

> From: Dsurfr <Dsurfr@aol.com>

> Date: Sun, 25 Jan 1998 12:36:29 EST

> Subject: spark gap question

> Organization: AOL (http://www.aol.com)

>

> My static spark gap consists of eight 1/4" brass bolts housed in a box with a

> muffin fan at one end. I just finished a salt water cap and when I first

> hooked it up I was getting very strong, loud arcs across all four gaps. Now I

> am getting very weak arcs across one or two gaps at best. Any suggestions as

> to my problem?

> Thanks, Jim

>

>

Jim,

Sounds like one or more of your saltwater caps may have gone belly up.

Locate and replace.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Any hope here?

Date: Mon, 26 Jan 1998 17:39:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 26 Jan 1998 12:39:43 -0500

Subject: Re: Any hope here?

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Any hope here?

> Date: Saturday, January 24, 1998 6:46 PM

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Sat, 24 Jan 1998 15:46:23 -0800 (PST)

> Subject: Any hope here?

>

> Well, I am feeling a little defeated.. I think I got a bit ambitious.

> Fired up my new coil today after rebuildng the primary, and building

> a new topload, and could not get breakout to save my life. With

> a bump on the outside edge I did mangage to get regular 30" arcs.

> I'd love to have this beast firing without the bump though.

> So I am open for suggestions!!!

>

> Parameters:

> Primary: 13 1/2 turns, tapped at 12 3/8 [ yes, the Q of this

>          system is sensitive to 1/8 turn tapping] 8.25" inner

>          0.25" tubing, 0.25" spacing, 30degree rise.

> Secondary: 6.25" x 25" 22AWG

> Toroid: 15" center-to-center, 6" Aluminum duct [covers top

>         2 inches of secondary at the moment]

> Gap: RQ style [1" pipe, in 2" segments, my 3" based one is

>       being rebuilt]

> Cap: Total of 0.0135 uf of homebrew poly plate.

> Power: 2 12KV/30ma NST in parallel.

>

> I am wondering if my topload is just too big, and if I should make a

> 4" based one? or if there is some trick of tuning I could use to get

> breakout without the bump?

> [Note that this same coil, with a 12"x3" toroid produces several

>  18" arcs at the same time.]

>

Michael,

Looks to me like the 15x6 torroid is a bit too large for the power

you are pumping in. If you want to continue using the same toroid

there are several things you might want to try.

1) Add another NST in parallel. You can try operation with the

existing cap, although obviously an additional .007 mfd would

be best. This will help increase the available power.

2) Make sure your RQ gap has sufficient air flow THROUGH the gaps.

Many muffin-fan RQ gaps are adequately COOLED but not adequately

QUENCHED. For quenching you want the air going right through the

gap space. I have my gaps mounted on a rectangular box to which

I have attached a vacuum cleaner motor. There should be some way

to adjust the vacuum, such as using a variac or having a plate

that slides over a hole to allow air to get in by bypassing the

actual gaps. If you OVER quench, then output will also be degraded.

3) Raise your toroid a bit higher. The lowest you need it is at a

point where the bottom of the toroid is even with the top winding.

You can raise the toroid several inches higher and still get

shading of the top of the coil. Raising it will usually increase the

maximum sparking length, but beyond a certain point you will

lose the desired electrostatic shielding of the secondary top.

Be aware that raising and lowering the toroid will change your

tune point slightly, as the capacitnce of the toroid is not JUST

a function of its size and shape. It also depends on its proximity

to other objects, such as the secondary. Raising and lowering it

affects the shape of the electrostatic field around the torroid,

and thus affects the ion 'cloud' shape and size (which influences

effective capacitance).

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: re: spark gap problem

Date: Tue, 27 Jan 1998 02:06:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Mon, 26 Jan 1998 21:06:14 EST

Subject: re: spark gap problem

Organization: AOL (http://www.aol.com)

Mike & Fr. Dan,

Thanks for the advice but it seems the problem was one of my own creation. I

had just re-assembled my gap & had filed down the ends so I knew they were

clean. The caps were newly built so I assume they are OK for now. I had the

gaps set @ 1/8" and for the heck of it opened them a couple of turns (about

3/16"), turned the muffin fan on to max (I have it on a rheostat) and all four

gaps began barking like before. I didn't realize you could gap too closely but

that must have been the problem. I have another question to ask. Is a muffin

fan strong enough to force enough air across the gaps? (I have it pulling air

through the box & not blowing directly on them) It seems like things slow down

a bit after it runs for a bit.

I'm also glad to see Fr. Dan is back to answering posts. I've missed his

insightful comments.

Thanks again for the help, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Thanks for the help!

Date: Tue, 27 Jan 1998 04:38:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Mon, 26 Jan 1998 20:38:15 -0800 (PST)

Subject: Thanks for the help!

Based on the suggestions of the lists, I did *almost* get my

6"x15" toroid to break out.

Actually.. it does break out - once in a while. I think I may

still be a tad conservitive on my SG, as after a few seconds

at full power, it start firing.

On the other hand, with all the suggested tweaks, I hit a

personal best of 34" with a breakout bump.

The following changes were made:

Safety gap widened from 0.095/side to 0.125 [from .190 to .250 total]

Spark Gap widened from 0.13 to .213

A channel built to funnel the air across the gaps with more velocity

Toroid raised about 3", so that the bottom is about 1" above the top turns

All of these changes combined managed to eke out 4 more inches. Mind you,

this was to a well grounded wire, but still makes me pretty happy :)

[oh, and is straight line measurement from bump to strike point]

Thanks again!

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc..edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap problem

Date: Tue, 27 Jan 1998 13:14:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 27 Jan 1998 08:14:23 -0500

Subject: Re: spark gap problem

> From: Dsurfr <Dsurfr@aol.com>

> Date: Mon, 26 Jan 1998 21:06:14 EST

> Subject: re: spark gap problem

> Organization: AOL (http://www.aol.com)

>

> Mike & Fr. Dan,

> Thanks for the advice but it seems the problem was one of my own creation. I

> had just re-assembled my gap & had filed down the ends so I knew they were

> clean. The caps were newly built so I assume they are OK for now. I had the

> gaps set @ 1/8" and for the heck of it opened them a couple of turns (about

> 3/16"), turned the muffin fan on to max (I have it on a rheostat) and all four

> gaps began barking like before. I didn't realize you could gap too closely but

> that must have been the problem. I have another question to ask. Is a muffin

> fan strong enough to force enough air across the gaps? (I have it pulling air

> through the box & not blowing directly on them) It seems like things slow down

> a bit after it runs for a bit.

> I'm also glad to see Fr. Dan is back to answering posts. I've missed his

> insightful comments.

> Thanks again for the help, Jim

>

>

Dsurfr,

If the quality of your output sparks drops off after several seconds of

run time, it is usually a sign that your spark gap is not being

cooled and quenched optimally.

Muffin fans are not very good at sucking air.

Sometimes a really good muffin fan can be used to BLOW air directly

through the gaps, but then some form of containment is required,

otherwise the majority of the air is not actually passed through the gap.

My own personal experience has been that SUCKING the air through the

gaps is more efficient than trying to BLOW it through. And a vacuum

cleaner motor is about the best thing you can use to do this.

Muffin fans are not designed for very high air pressures. They are good

at moving moderate volumes of air at moderate pressures. A muffin

fan is better than no fan at all, but when we speak of a vacuum gap

we mean one supplied with a motor that can really move a lot of air at

fairly high pressures. I personally prefer to work with a genuine

vacuum cleaner motor. I bolt the sucker directly to a box and usually

use a small variac to adjust the speed. An alternate way of adjusting

the vacuum is to have a large hole in the box, and use a moveable

hole cover to adjust how much air sneaks in via the large hole.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Thanks for the help!

Cc:

Bcc:

X-Attachments:

At 04:38 AM 1/27/98 GMT, you wrote:

>From: Michael Baumann <baumann@proton.llumc.edu>

>Date: Mon, 26 Jan 1998 20:38:15 -0800 (PST)

>Subject: Thanks for the help!

>

>Based on the suggestions of the lists, I did *almost* get my

>6"x15" toroid to break out.

>Actually.. it does break out - once in a while. I think I may

>still be a tad conservitive on my SG, as after a few seconds

>at full power, it start firing.

>On the other hand, with all the suggested tweaks, I hit a

>personal best of 34" with a breakout bump.

>The following changes were made:

>Safety gap widened from 0.095/side to 0.125 [from .190 to .250 total]

>Spark Gap widened from 0.13 to .213

>A channel built to funnel the air across the gaps with more velocity

>Toroid raised about 3", so that the bottom is about 1" above the top turns

>

>All of these changes combined managed to eke out 4 more inches. Mind you,

>this was to a well grounded wire, but still makes me pretty happy :)

>[oh, and is straight line measurement from bump to strike point]

>

Sounds pretty good Michael. Now that your efficiency

is up check again for any heating in your primary

circuit. Your primary tap is often a source of loss.

Try somewhat tighter coupling now that your gap

is most likely quenching better. I would lower

your secondary until you are clearly overcoupled

then reduce coupling in small increments looking for

the sweet spot. The best coupling is highly dependant

on the performance of your gap. The poorer your gap performs

the looser your coupling needs to be for best performance.

If you have some spare primary turns left I would increase

your toroid diameter to increase its capacitance and lower

your resonant frequency. This will allow you to tap in more

primary inductance which will further improve gap operation.

This wont affect the breakout of the toroid any. I would

thing about going to 19 X 6 or 20 X 6.

Performance gains at this point are going to come more slowly.

Sometimes just an inch at a time.

34 inches from 12kv/60 is quite good. But there are always ways

to squeeze out a bit more.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: re: spark gap problem

Cc:

Bcc:

X-Attachments:

At 02:06 AM 1/27/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Mon, 26 Jan 1998 21:06:14 EST

>Subject: re: spark gap problem

>Organization: AOL (http://www.aol.com)

>

>Mike & Fr. Dan,

>Thanks for the advice but it seems the problem was one of my own creation. I

>had just re-assembled my gap & had filed down the ends so I knew they were

>clean. The caps were newly built so I assume they are OK for now. I had the

>gaps set @ 1/8" and for the heck of it opened them a couple of turns (about

>3/16"), turned the muffin fan on to max (I have it on a rheostat) and all four

>gaps began barking like before. I didn't realize you could gap too closely but

>that must have been the problem. I have another question to ask. Is a muffin

>fan strong enough to force enough air across the gaps? (I have it pulling air

>through the box & not blowing directly on them) It seems like things slow down

>a bit after it runs for a bit.

A muffin type fan works best by blowing air through the gap.

You want a fairly strong fan that moves plenty of air. If your

fan is relatively weak then look for a stronger unit.

Using a high speed vacuum cleaner motor to suck air through

the gap is even better.

The problem with the gap not working as well after running

a short time is due to heat build up. Breaking up the gap

with more series electrodes will help as will increasing the air

flow to help remove heat faster.

>I'm also glad to see Fr. Dan is back to answering posts. I've missed his

>insightful comments.

It happens to be Fr. Tom rather than Dan but I hear he will

respond to a wide range of names. :)

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: power up makes problem

Cc:

Bcc:

X-Attachments:

At 07:58 PM 4/15/97 GMT, you wrote:

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Tue, 15 Apr 1997 14:58:06 -0500

>Subject: power up makes problem

>

>        I recently upgraded my poly plate cap to .01uf and added a 2nd 15000v

>30ma transformer. I was getting sparks in excess of 24" before now I

>can`t even get anything to break out. my new coil specs are:

>

>4" pvc

>22" of 24 gauge wire

>4" * 12" toroid

>primary

>15000v 60 ma

>safety gap and 2  2.5 mh air core chokes

>4 .01uF 7500 v plate poly caps two in series then in paraell R. Q. gap

>with hair drier fan using 4" pvc and 10 6"long 5/8" copper pipe

>.25" copper primary 15deg 16 turns inside diameter 7"

>My new ground is still in the works could the extra power have made my

>old one worthless. I retuned and still I get nothing from the coil,

>except some primary secondary strikes. I raised the secondary and still

>nothing. I am sure of the phase on the two transformers is correct and

>that they are both working I am at a loss for an explanation and any

>sugestions would be helpful.

>

>Mike White

>

Mike,

The most likely explanation is that your coil is no longer

in resonance. Doubling the size of your primary cap will

dramatically lower your primary resonant frequency.

You may be able to regain tune by tapping in fewer primary turns.

If you system initially tuned up with only a few primary turns

tapped in then you may be outside the tuning range of your

primary circuit.

I would consider going to a larger toroid. Since you are using

a 12 X 4 now a good choice might be 16 X 4 or even 18 X 4.

You could also make good use of a toroid with a 6 inch cross

section. A 19 X 6 would probably work well with your power level.

The larger toroid will lower your secondary resonant frequency

and make it a better match for your new lower frequency primary circuit.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: power up makes problem

Date: Thu, 29 Jan 1998 13:08:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 29 Jan 1998 07:08:18 -0600

Subject: Re: power up makes problem

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

Their are two things to look for here:

I was reciently embarresed when I brought some friends over to see my 48" streamers.

They were barely 24" in length. Come two find out the gaps had closed up due to heavy

oxidation of the electrodes. The other thing to look at is your primary tap. As you

raise capacitance the inductance changes. You should have to tap in on your primary.

Fleischwulf

---

Einigkeit und Freiheit

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Tue, 15 Apr 1997 14:58:06 -0500

>Subject: power up makes problem

>

>   I recently upgraded my poly plate cap to .01uf and added a 2nd 15000v

>30ma transformer. I was getting sparks in excess of 24" before now I

>can`t even get anything to break out. my new coil specs are:

>

>4" pvc

>22" of 24 gage wire

>4" * 12" toroid

>primary

>15000v 60 ma

>safety gap and 2 2.5 mh air core chokes

>4 .01uF 7500 v plate poly caps two in series then in paraell R. Q. gap=

>with hair drier fan using 4" pvc and 10 6"long 5/8" copper pipe

>.25" copper primary 15 deg 16 turns inside diameter 7"

>My new ground is still in the works could the extra power have made my

>old one worthless. I retuned and still I get nothing from the coil,

>except some primary secondary strikes. I raised the secondary and still

>nothing. I am sure of the phase on the two transformers is correct and

>that they are both working I am at a loss for an explanation and any

>sugestions would be helpful.

>

>Mike White

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Current Limiting

Date: Thu, 29 Jan 1998 16:15:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Thu, 29 Jan 1998 08:15:54 -0800 (PST)

Subject: Current Limiting

I was thinking this morning about current limiting.. [I still dream of

the day I can get that Pig... and want to be ready] I thought of

something that is a) very simple, and b) probably does not work.

So someone tell me why please :)

Since I am mostly a software kinda guy, with a smattering of digital

design, dealing with this high-power AC stuff always confuses me.

If I understand it correctly - the idea behind reactive or resistive

loads is to place an IR drop in series with the pig, such that when

the current reaches a certain point, the IR loss prevents it from

rising further [I know, the wording sucks, but the idea is

clear, I hope]

So.. based on this idea.. and the fact that lightbulbs don't exactly

short things out, why not use lightbulbs as resistive ballast?

I am not exactly sure what the VA rating for a bulb is.. but for

grins sake, assume a 100W bulb is 100VA, paralleling 2 bulbs gives

you total power flow of 200VA, 3.. etc. [ for 220, use 2 110V bulbs in

series].

Power control is two fold: Voltage via variac, and current by switching

in more or less bulbs in series.

Like So  (god I hate ascii art)

HOT--------------------------------------------

|         |          |         |       |

B         B          B         B       B       B=lightbulb

|         |          |         |       |       S= Switch

VARIAC  S         S          S         S       S

|         |          |         |       |

----------------------------------------------

Pig

NEU---------------------------------------------------

So. it's stupidly simple. Despite the engineering approach of KISS,

there must be something wrong - else it would be used. So someone please

exlain where my logic went off the deep end.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Current Limiting

Date: Thu, 29 Jan 1998 17:40:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 29 Jan 1998 12:40:44 -0500

Subject: Re: Current Limiting

Michael Baumann,

What you wrote about is do-able, and in fact some coilers have even

actually done it that way. Light bulbs make for very inexpensive

ballast, but they do have one rather nasty negative: They are a

non-linear resistance. They change resistance drastically, in fact.

Most coilers who use resistance prefer a more linear resistance

such as a heater element. Or many heater elements :)

Stovetop heating elements work quite well... but obviously

you have to take care to ensure you don't have a fire hazard.

But the major problem with all purely resistive methods of

limiting the current is that they convert electrical energy

into heat. You are literally throwing energy away because of

the IR losses.

Inductive methods, on the other hand, are much less lossy. The

favorite method is to use an arc welder. You short out the

welder's secondary by clamping the cables together and wire

the primary in series with your pole pig's 'primary'. Arc

welders have a wheel or dial or lever you adjust to adjust

the max current flow. Very convenient, and it does not hurt

the welder in any way, and requires absolutely NO modifications

to the actual welder. You *do* have to buy a socket for the

welder to plug into, but that's just a couple of bucks.

Inductive loads by themselves can sometimes operate in a sort

of 'bumpy' fashion, and many coilers find that some resistive

ballast either in series or parallel with the welder helps to

smooth things out.

BTW, a few coilers have experimented with using microwave

transformer cores, old NST cores, and even the cores from

destroyed variacs as inductive ballasts. You have to 'gap'

these, and only the gapped variacs are inherently adjustable.

John Freau has wound heavy duty coils and then made the

cores from welding rods and iron rods. Kludgy but it works.

Big Bombarder transformers for neon sign work used to have

humongous VARIABLE iron core coils that would make good ballasts.

Several small coils can be wired so as to handle more current.

A visit to your local industrial waste recycle center (dump)

might put you in possession of some items that you could use

to build your own ballast. Or just buy a dumpy old used

arc welder.

Hope this helps,

Fr. Tom McGahee

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Current Limiting

> Date: Thursday, January 29, 1998 11:15 AM

>

> From: baumann@proton.llumc.edu

> Date: Thu, 29 Jan 1998 08:15:54 -0800 (PST)

> Subject: Current Limiting

>

> I was thinking this morning about current limiting.. [I still dream of

> the day I can get that Pig... and want to be ready] I thought of

> something that is a) very simple, and b) probably does not work.

> So someone tell me why please :)

> Since I am mostly a software kinda guy, with a smattering of digital

> design, dealing with this high-power AC stuff always confuses me.

>

> If I understand it correctly - the idea behind reactive or resistive

> loads is to place an IR drop in series with the pig, such that when

> the current reaches a certain point, the IR loss prevents it from

> rising further [I know, the wording sucks, but the idea is

> clear, I hope]

> So.. based on this idea.. and the fact that lightbulbs don't exactly

> short things out, why not use lightbulbs as resistive ballast?

> I am not exactly sure what the VA rating for a bulb is.. but for

> grins sake, assume a 100W bulb is 100VA, paralleling 2 bulbs gives

> you total power flow of 200VA, 3.. etc. [ for 220, use 2 110V bulbs in

> series].

>

> Power control is two fold: Voltage via variac, and current by switching

> in more or less bulbs in series.

>

> Like So  (god I hate ascii art)

>

> HOT--------------------------------------------

>         |         |          |         |       |

>         B         B          B         B       B       B=lightbulb

>         |         |          |         |       |       S= Switch

> VARIAC  S         S          S         S       S

>         |         |          |         |       |

>         ----------------------------------------------

>                                                       Pig

> NEU---------------------------------------------------

>

> So. it's stupidly simple. Despite the engineering approach of KISS,

> there must be someting wrong - else it would be used. So someone please

> exlain where my logic went off the deep end.

>

> --

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Current Limiting

Date: Thu, 29 Jan 1998 20:48:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 29 Jan 1998 14:48:41 -0600

Subject: Re: Current Limiting

Organization: MailExcite  (http://www.mailexcite.com)

Michael wrote:

>

>HOT--------------------------------------------

>        |         |          |         |       |

>        B         B          B         B       B       B=lightbulb

>        |         |          |         |       |       S= Switch

>VARIAC  S         S          S         S       S

>        |         |          |         |       |

>        ----------------------------------------------

>                                                      Pig

>NEU---------------------------------------------------

>

>So. it's stupidly simple. Despite the engineering approach of KISS,

>there must be someting wrong - else it would be used. So someone please

>exlain where my logic went off the deep end.

>

Michael,

When we first started pigging, we wired an

outlet strip in series with the pig.  We controlled power

by how many lamps we plugged into the strip.  This is

basically the same as what you described.

Our particular pig has about 100 watts of core loss,

even with no load on the output.  So bulbs <=100

watts would stay lit but provide a severely reduced

(unuseful) output voltage.

Fr. Tom  is right about the nonlinearity of the bulbs.

For awhile we were using a 500-w halogen spotlight

for a limiter.  When the coil would fire, the bulb would

heat up, the filament resistance would increase, the

bulb would light, and the coil would stop firing due to

the reduced power!  When the bulb cooled down it

would fire some more.  This would repeat about once

a second or so.  Interesting, but not terribly useful!!

We are presently using a microwave oven xfmr with

the secondary shorted out as a limiter.  (in conjunction

with a variac)

You can get 1000 watt 220v stove elements from MECI for $2.00 or some low price

like that...  We aren't using any at the moment.  Or you could go to goodwill and

buy a bunch of old toaster ovens/ electric skillets/ hotplates for resistance loading...

View our tesla (and pig) pics at:

http://www.geocities.com/capecanaveral/hanger/6160

(the arcstarter/fleischwulf coil)

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Current Limiting

Date: Thu, 29 Jan 1998 21:07:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 29 Jan 1998 15:07:40 -0600

Subject: Re: Current Limiting

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

A very simple method to limit current is to buy some old microwave transformers,

put the primaries in series with the pig and short the secondary of the mw transformer

to the core. This will give you about 1 kw. Add 2 more and you get 3 kw. The mw

transformer cost about $5.00

Hope this was helpful

Paul

---

Einigkeit und Freiheit

>From: baumann@proton.llumc.edu

>Date: Thu, 29 Jan 1998 08:15:54 -0800 (PST)

>Subject: Current Limiting

>

>I was thinking this morning about current limiting.. [I still dream of

>the day I can get that Pig... and want to be ready] I thought of

>something that is a) very simple, and b) probably does not work.

>So someone tell me why please :)

>Since I am mostly a software kinda guy, with a smattering of digital

>design, dealing with this high-power AC stuff always confuses me.

>

>If I understand it correctly - the idea behind reactive or resistive

>loads is to place an IR drop in series with the pig, such that when

>the current reaches a certain point, the IR loss prevents it from

>rising further [I know, the wording sucks, but the idea is

>clear, I hope]

>So.. based on this idea.. and the fact that lightbulbs don't exactly

>short things out, why not use lightbulbs as resistive ballast?

>I am not exactly sure what the VA rating for a bulb is.. but for

>grins sake, assume a 100W bulb is 100VA, paralleling 2 bulbs gives

>you total power flow of 200VA, 3.. etc. [ for 220, use 2 110V bulbs in

>series].

>

>Power control is two fold: Voltage via variac, and current by switching

>in more or less bulbs in series.

>

>Like So  (god I hate ascii art)

>

>HOT--------------------------------------------

>        |         |          |         |       |

>        B         B          B         B       B       B=lightbulb

>        |         |          |         |       |       S= Switch

>VARIAC  S         S          S         S       S

>        |         |          |         |       |

>        ----------------------------------------------

>                                                      Pig

>NEU---------------------------------------------------

>

>So. it's stupidly simple. Despite the engineering approach of KISS,

>there must be someting wrong - else it would be used. So someone please

>exlain where my logic went off the deep end.

>

>--

>--

>Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

>San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Current Limiting

Date: Thu, 29 Jan 1998 22:16:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 29 Jan 1998 17:16:36 -0500

Subject: Re: Current Limiting

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: cwatkins@seidata.com

> Subject: Current Limiting

> Date: Thursday, January 29, 1998 11:15 AM

>

> From: baumann@proton.llumc.edu

> Date: Thu, 29 Jan 1998 08:15:54 -0800 (PST)

> Subject: Current Limiting

>

> I was thinking this morning about current limiting..

(snip)

> So.. based on this idea.. and the fact that lightbulbs don't exactly

> short things out, why not use lightbulbs as resistive ballast?

Michael,

I use light bulbs regularly to limit current so my 15a variac

can handle the load. And it seems to work well for me.

although it may not be a true linear resistance each bulb will

ultimately pass only its rated wattage.

it does create wasted heat but in winter who cares?

I have 6 - 300watt bulbs wired 3 in each leg of my 220v input.

if you use 220 you must put equal wattage on each leg.

also I have found on the 300w bulbs the Sylvania ones

will hold up tremendiously longer than the GE brand.

good luck

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 24 gauge wire source

Date: Fri, 30 Jan 1998 20:34:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Fri, 30 Jan 1998 15:34:05 EST

Subject: 24 gauge wire source

I just received 5 lbs of 24 gauge enameled wire I bought from Gary Weaver at a

mere $3.00 a pound! I think he still has more for sale and would recommend it

to anyone having trouble getting wire locally. His email address is:

gweaver@earthlink.net.

Jim Sheen

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: winding a secondary coil

Date: Fri, 30 Jan 1998 20:46:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Fri, 30 Jan 1998 15:46:00 EST

Subject: winding a secondary coil

Will using a very thin double faced tape applied in strips on my secondary

coil form to hold the windings in place cause any problems from the slight air

gap between the wire and the form? If so any other suggestions? (I'm using a 4

1/2" acrylic tube)

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: winding a secondary coil

Cc:

Bcc:

X-Attachments:

At 08:46 PM 1/30/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Fri, 30 Jan 1998 15:46:00 EST

>Subject: winding a secondary coil

>

>Will using a very thin double faced tape applied in strips on my secondary

>coil form to hold the windings in place cause any problems from the slight air

>gap between the wire and the form? If so any other suggestions? (I'm using a 4

>1/2" acrylic tube)

>Jim

>

>

Jim,

That wont really cause any problems but is really unnecessary.

Just anchor one end of the wire to the form with a drop

of epoxy or hot glue and wind away. A dab of hot glue

or even a strip of masking tape will hold the winding in place

in case you need to take a break.

Once winding is complete lay on several fairly heavy coats

of polyurethane varnish.

Be careful though. The varnish will debond the hot glue

and if its the only thing holding the winding on you'll

get a big suprise.

Thats why I like to use epoxy to secure the start and finish

and a strip of masking tape to hold down the turns temporarily

when I take a break.

Your acrylic tube wont require any of the drying and sealing

thats done for PVC forms. Should make an excellent secondary.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Large Tesla Coil

Date: Fri, 30 Jan 1998 22:36:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Ppowellme@aol.com

Date: Fri, 30 Jan 1998 17:36:01 EST

Subject: Large Tesla Coil

Can anyone tell me how to build a 2-3 1/2 foot tesla coil with ajustable

amperage (from one amp or lower to 3 amps or higher) and voltage (15 volts or

lower to 15,000 volts or higher) and would not take over $50 to buy supplies?

Thanks.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large Tesla Coil

Date: Sat, 31 Jan 1998 00:08:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Fri, 30 Jan 1998 18:08:57 -0600

Subject: Re: Large Tesla Coil

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>      Can anyone tell me how to build a 2-3 1/2 foot tesla coil with ajustable

>amprage (from one amp or lower to 3 amps or higher) and voltige (15 volts or

>lower to 15,000 volts or higher) and would not take over $50 to buy supplies?

>Thanks.

I suspect the answer will be no.  Those specs would be

very difficult the achieve. IMHO

I once fed my coil DC output from a rectified ignition

coil, about 50 watts max, using a 12 volt supply, but I

was only able to get 1-2 inch sparks off of it.

The rep rate of the capacitive discharges was about 1

per second, a function of the cap size, gap distance,

and available power to recharge the cap.  This was

simply a case of not enough power.  This same coil

produces 50 inches of spark when fed 2 KW!

-Bill the arcstarter

http://www.geocities.com/capecanaveral/hangar/6160.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: secondary coil winding question

Date: Sat, 31 Jan 1998 00:44:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Fri, 30 Jan 1998 19:44:58 EST

Subject: secondary coil winding question

Thanks for the quick reply to my question on the double face tape. The wire I

am going to use is enameled . Do I still have to coat it with polyurethane

varnish after it is wound?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: secondary coil winding question

Cc:

Bcc:

X-Attachments:

At 12:44 AM 1/31/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Fri, 30 Jan 1998 19:44:58 EST

>Subject: secondary coil winding question

>

>Thanks for the quick reply to my question on the double face tape. The wire I

>am going to use is enameled . Do I still have to coat it with polyurethane

>varnish after it is wound?

>

>

No. But its really a good idea if you do. It anchors down

the winding solidly so it doesnt move, protects the winding

from minor bumps and dings and improves the insulation of

the secondary winding. This increased insulation allows for somewhat

tighter coupling between primary and secondary before breakdown occurs.

So, yes I would coat your secondary after you wind it. Put on

at least 4 or 5 medium coats. Put on enough coats so you can no

longer feel the individual wires. Should feel perfectly smooth.

By the way. Use real polyurethane varnish. The ecologically correct

latex based varnishes are not what you want for this application.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coil winding question

Date: Sat, 31 Jan 1998 01:36:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Fri, 30 Jan 1998 20:36:16 EST

Subject: Re: secondary coil winding question

Jim,

I would still coat the secondary wire after winding it.   If nothing else,

it will reduce any corona losses that may be present.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Large Tesla Coil

Cc:

Bcc:

X-Attachments:

At 10:36 PM 1/30/98 GMT, you wrote:

>From: Ppowellme@aol.com

>Date: Fri, 30 Jan 1998 17:36:01 EST

>Subject: Large Tesla Coil

>

>      Can anyone tell me how to build a 2-3 1/2 foot tesla coil with ajustable

>amprage (from one amp or lower to 3 amps or higher) and voltige (15 volts or

>lower to 15,000 volts or higher) and would not take over $50 to buy supplies?

>Thanks.

>

>

Jim,

By 2 - 3 1/2 foot do you mean a coil 2 foot in diameter? I hope not.

That is an exceedingly large coil and beyond the means of most coilers.

If you mean 2 - 3 1/2 foot tall thats a different story.

Secondary coils are normally measured by their diameter with their

H/D (height/diameter) ratios being between 3 to 1 and 6 to 1.

The tall skinny coils of the past have been replaced by shorter

fatter more efficient coils.

A secondary coil wound on 4 inch PVC pipe would work well with a

5 to 1 H/D ratio. Since 4 inch PVC is 4.5 inches OD that comes

to 22.5 inches. A 20 - 22 inch winding of AWG 24 would make a great

performing secondary.

For a power supply most use neon sign transformers. They are commonly

available in voltages from 6-15KV. The 12kv ones are the most practical

for coil use. They can be made adjustable by varying their input voltage

through the use of a variac.

Cost requirements of building a coil depends on the size of the coil

and the power level. Everything depends on the size of your supply

transformer. So first order of business is to hunt up some neon sign

transformers. Costs can be quite low if you are a good scrounger.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coil winding question

Date: Sat, 31 Jan 1998 02:27:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Fri, 30 Jan 1998 18:27:04 -0800 (PST)

Subject: Re: secondary coil winding question

On Sat, 31 Jan 1998, Tesla-2 wrote:

> From: Dsurfr@aol.com

> Date: Fri, 30 Jan 1998 19:44:58 EST

> Subject: secondary coil winding question

>

> Thanks for the quick reply to my question on the double face tape. The wire I

> am going to use is enameled . Do I still have to coat it with polyurethane

> varnish after it is wound?

Yes. The varnish serves a number of purposes.. .

1) Reduces corona from the upper turns

2) Helps shield lower turns [not much, but a little :) ]

3) Seals the windings from dirt/water/muck/mung

4) Protects the windings from minor nicks and dings

5) Looks damn nice in the pictures :)

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large Tesla Coil

Date: Sat, 31 Jan 1998 02:44:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Fri, 30 Jan 1998 20:44:06 -0600

Subject: Re: Large Tesla Coil

>>      Can anyone tell me how to build a 2-3 1/2 foot tesla coil with

ajustable

>>amprage (from one amp or lower to 3 amps or higher) and voltige (15 volts or

>>lower to 15,000 volts or higher) and would not take over $50 to buy

supplies?

>>Thanks.

>I suspect the answer will be no.  Those specs would be

>very difficult the achieve. IMHO

What if you used a 12v car battery and a custom built DC to AC converter?

A car battery is capable of supplying 800+ amps at 12 volts, you could convert

that to a makeshift AC sin waveform to then power a neon transformer.

Your transistors would be large of course.  I have seen DC to AC converters

in radioshack for $50 that plug into your cig lighter and output 120vac at

8amps.

This would be enough for a good 900 volt-amps...

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coil winding question

Date: Sat, 31 Jan 1998 02:45:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Fri, 30 Jan 1998 20:45:18 -0600

Subject: Re: secondary coil winding question

>Thanks for the quick reply to my question on the double face tape. The wire I

>am going to use is enameled . Do I still have to coat it with polyurethane

>varnish after it is wound?

Have to?  No.

Recommended?  YES!

Everyone's wire is enameled, or it wouldn't be a coil.  It would just short

straight through...  and people STILL coat the tube.  Its recommended you

coat before AND after also...

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: secondary coil winding question

Date: Sun,  1 Feb 1998 16:07:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 01 Feb 1998 10:07:04 -0600

Subject: Re: Re: secondary coil winding question

Organization: MailExcite  (http://www.mailexcite.com)

Jeff wrote:

>Everyone's wire is enameled, or it wouldn't be a coil.  It would just short

>straight through...  and people STILL coat the tube.  Its recommended you

>coat before AND after also...

>

>Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

>http://www.harvestcomm.net/personal/corr/index.htm

Actually - we broke tradition and used regular

old PVC/teflon-insulated solid copper wire for our

coil secondary.

24 gauge - outer diameter measured as 0.028, copper

diameter is 0.020, for a theoretical 35 turns per inch.

For our next coil (on a 6.875 diameter core) I've acquired a roll of 22 gauge wire

- outer diameter = 0.054, copper diameter = 0.033 for a theoretical 18 turns per inch.

We haven't had any problems with arcing between turns.

Plus - the few times we have had strikes from the top

terminal to the secondary didn't seem to hurn anything.

I'm not sure but I would think that an enameled wire

secondary would be more susceptable to insulation

breakdown, especially if any accidental strikes

should occur.

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large Tesla Coil

Date: Sun,  1 Feb 1998 16:11:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 01 Feb 1998 10:11:20 -0600

Subject: Re: Large Tesla Coil

Organization: MailExcite  (http://www.mailexcite.com)

Jeff wrote:

>From: Jeff Corr <corr@enid.com>

>What if you used a 12v car battery and a custom built DC to AC converter?

>A car battery is capable of supplying 800+ amps at 12 volts, you could convert

>that to a makeshift AC sin waveform to then power a neon transformer.

>Your transistors would be large of course.  I have seen DC to AC converters

>in radioshack for $50 that plug into your cig lighter and output 120vac at

>8amps.

>This would be enough for a good 900 volt-amps...

Thats a good idea! But I'm not sure the cheaper

inverters produce a sin-wave output.  Perhaps you could

use a computer UPS?  I'm not sure these devices would

appreciate driving a highly inductive load like a NST.

Perhaps you could get away with it given the right

filtering between the supply and the neon.

Has anyone tried this??

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Large Tesla Coil

Cc:

Bcc:

X-Attachments:

At 04:11 PM 2/1/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Sun, 01 Feb 1998 10:11:20 -0600

>Subject: Re: Large Tesla Coil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Jeff wrote:

>>From: Jeff Corr <corr@enid.com>

>>What if you used a 12v car battery and a custom built DC to AC converter?

>>A car battery is capable of supplying 800+ amps at 12 volts, you could convert

>>that to a makeshift AC sin waveform to then power a neon transformer.

>>Your transistors would be large of course.  I have seen DC to AC converters

>>in radioshack for $50 that plug into your cig lighter and output 120vac at

>>8amps.

>>This would be enough for a good 900 volt-amps...

>

>Thats a good idea! But I'm not sure the cheaper

>inverters produce a sin-wave output.  Perhaps you could

>use a computer UPS?  I'm not sure these devices would

>appreciate driving a highly inductive load like a NST.

>Perhaps you could get away with it given the right

>filtering between the supply and the neon.

>

>Has anyone tried this??

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

Typical inverters will not successfully drive highly

inductive loads. Trying to do so will usually result

in failed components.

In my service truck I have a 2500 watt Tripp-Lite

inverter for powering service equipment in the field.

Works great for lights and soldering equipment and

even my test equipment but anything else is asking for trouble.

I know. Ive blown the thing up more times than I care

to remember. And I have to fix it if I blow it up.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Re: secondary coil winding question

Cc:

Bcc:

X-Attachments:

At 04:07 PM 2/1/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Sun, 01 Feb 1998 10:07:04 -0600

>Subject: Re: Re: secondary coil winding question

>Organization: MailExcite  (http://www.mailexcite.com)

>

>

>

>Actually - we broke tradition and used regular

>old PVC/teflon-insulated solid copper wire for our

>coil secondary.

>

>24 gauge - outer diameter measured as 0.028, copper

>diameter is 0.020, for a theoretical 35 turns per inch.

>

>For our next coil (on a 6.875 diameter core) I've acquired a roll of

>22 gauge wire

>outer diameter = 0.054,

>copper diameter = 0.033 for a theoretical 18 turns per inch.

The reason for using magnet wire instead of insulated

wire is that magnet wire provides the highest inductance

per unit area.

If you are only going to get 18 turns per inch from your

22 guage wire you will really be short changing yourself

inductance wise.

>

>We haven't had any problems with arcing between turns.

>Plus - the few times we have had strikes from the top

>terminal to the secondary didn't seem to hurn anything.

The actual voltage between adjacent turns is quite low.

You can push enough power into a coil to have arcs between

turns several inches apart though. A heavy coating of polyurethane

varnish helps here.

>

>I'm not sure but I would think that an enameled wire

>secondary would be more susceptable to insulation

>breakdown, especially if any accidental strikes

>should occur.

Wont make any difference. Most PVC insulated wire has

a 600 V rating. Thats no insulation at all compared

the the voltage in the secondary streamers.

>

>-Bill the arcstarter

>

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large Tesla Coil

Date: Sun,  1 Feb 1998 18:23:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Sun, 01 Feb 1998 12:23:37 -0600

Subject: Re: Large Tesla Coil

Actually I have used my UPS with a 9kv neon.  It ran for

7 minutes before the UPS ran down.  As for cheap inverters,

OR computer UPS's, no, they don't produce a sine wave.  Its

more of a funky square wave.  My UPS was this way, we looked

at it on an O-scope.  Didn't seem to downgrade performance

in the coil any, but then again my coil sucks anyway :-)

>Thats a good idea! But I'm not sure the cheaper

>inverters produce a sin-wave output.  Perhaps you could

>use a computer UPS?  I'm not sure these devices would

>appreciate driving a highly inductive load like a NST.

>Perhaps you could get away with it given the right

>filtering between the supply and the neon.

>Has anyone tried this??

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

ICQ UIN # 1303360 | Nick TacoBell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: h/d aspect of secondary

Date: Sun,  1 Feb 1998 21:52:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sun, 1 Feb 1998 16:52:52 EST

Subject: h/d aspect of secondary

Can anyone give me some advice on the height /diameter ratio for winding a

secondary coil? I have a 4 1/2" D. acrylic form and was going to try for a 6/1

ratio w/ a resonant freq. of 303.73 but that would mean 1,279 turns or 27"

total height using 24 ga. wire. I have read that the total number of turns

should not exceed 1000. I'm using a 15k 30 MA  neon transformer w/ .0053 Cap.

I also have a lot of # 10 solid copper wire I would like to use to form the

primary. "WinTesla" says I should have 9 turns 1/4" apart (I'm making a

helical primary) advice on this would also be much appreciated.

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: h/d aspect of secondary

Cc:

Bcc:

X-Attachments:

At 09:52 PM 2/1/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sun, 1 Feb 1998 16:52:52 EST

>Subject: h/d aspect of secondary

>

>Can anyone give me some advice on the height /diameter ratio for winding a

>secondary coil? I have a 4 1/2" D. acrylic form and was going to try for a 6/1

>ratio w/ a resonant freq. of 303.73 but that would mean 1,279 turns or 27"

>total height using 24 ga. wire. I have read that the total number of turns

>should not exceed 1000.

True. More than 1000 turns is usually just wasting wire.

That doesnt mean that a secondary with 1001 turns wont work.

Its that best operation occurs with 700-1000 turns.

Here is a little table I made up for H/D ratio using standard sizes

of PVC pipe.

Pipe      ratio    winding length

2 inch    8 to 1   19 inches

3 inch    6 to 1   21 inches

4 inch    5 to 1   22.5 inches

6 inch    4 to 1   26 inches

8 inch    3 to 1   25.59 inches

Once you have an idea of the required winding length

then choose the largest guage of wire that will give you

700-1000 turns in the allotted space.

For your 4.5 inch secondary you want a winding around 22 inches.

It doesnt have to be exact. You can vary it considerable without

affecting performance.

My 4 inch secondary has a 21 inch winding of AWG 24. This is around

975 turns and has achieved in excess of 50 inches of spark at around

1200 watts input.

>

>I'm using a 15k 30 MA  neon transformer w/ .0053 Cap.

>I also have a lot of # 10 solid copper wire I would like to use to form the

>primary. "WinTesla" says I should have 9 turns 1/4" apart (I'm making a

>helical primary) advice on this would also be much appreciated.

>Jim

>

Consider that 9 turns to be a minimum. You want many more

than that. I would recommend at least 14 turns. You will

appreciate having these extra turns when you start adding

larger toroids.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil Improvements

Date: Mon,  2 Feb 1998 02:30:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Sun, 1 Feb 1998 20:30:25 -0600 (CST)

Subject: Coil Improvements

Hey, all.  Taking the advice I received on the list, I've recently

improved my spark gap and ground connection.

The new spark gap is an RQ-style in PVC pipe.  It's 10 pieces of 7/8"

copper pipe, 4" long.  Total gap is 0.3".  It's quenched with a blower I

kiped from a microwave exhaust.

The new ground is 2 pieces of 1/2" copper rod, each 4' long, hammered all

the way into the ground.  The ground is connected to the coil with about

5' of pretty thick wire.  This replaces my 18" copper pipe.  The rods are

about 4' apart.

Results aren't bad.  I can finally make the toroid break out on its own.

However, I'm getting about 4 8" sparks instead of one long one.  Could my

toroid be too bumpy?  It's a 4x14" aluminum dryer duct wrapped with

individual strips of aluminum tape to cover the surface.  The surface of

it is a little crinckley.

Any suggestions?  My next inprovement will be a plate cap to replace my

bottle caps.

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Improvements

Cc:

Bcc:

X-Attachments:

At 02:30 AM 2/2/98 GMT, you wrote:

>From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

>Date: Sun, 1 Feb 1998 20:30:25 -0600 (CST)

>Subject: Coil Improvements

>

>Hey, all.  Taking the advice I received on the list, I've recently

>improved my spark gap and ground connection.

>

>The new spark gap is an RQ-style in PVC pipe.  It's 10 pieces of 7/8"

>copper pipe, 4" long.  Total gap is 0.3".  It's quenched with a blower I

>kiped from a microwave exhaust.

Sounds good to me. Make sure this fan moves plenty of air.

Some oven fans Ive seen have been relatively weak.

>

>The new ground is 2 pieces of 1/2" copper rod, each 4' long, hammered all

>the way into the ground.  The ground is connected to the coil with about

>5' of pretty thick wire.  This replaces my 18" copper pipe.  The rods are

>about 4' apart.

Sounds like a great improvement here. Your ground lead should

be good heavy wire not braided cable. Stranded wire is fine as long

as the strands arent twisted. I use AWG 8 ground wire. This is stranded

wire without any twist.

>

>Results aren't bad.  I can finally make the toroid break out on its own.

>However, I'm getting about 4 8" sparks instead of one long one.  Could my

>toroid be too bumpy?  It's a 4x14" aluminum dryer duct wrapped with

>individual strips of aluminum tape to cover the surface.  The surface of

>it is a little crinckley.

I never cover my aluminum duct toroids. I only use a strip of aluminum

tape to cover and smooth over the joint where the ends of the duct are

joined together. The rest of the duct does not need covered. The accordian

like folds in the duct are not a problem.

If after getting things smoothed up you are still getting several

short streamers instead of one long one then try a larger toroid.

>

>Any suggestions?  My next inprovement will be a plate cap to replace my

>bottle caps.

>

>Tim Osmulski

>tim.o@ttu.edu

>

Keep up the good work.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla-2 list archive

Cc:

Bcc:

X-Attachments:

Notice to all,

Despite several acute attacks of laziness I managed

to get the list archive for November and December

edited and uploaded to the archive sites.

The file name is tes2arc9.zip and can be downloaded

from Dave Bronakowski's archive site. The URL is:

http://www.velocity.net/~djb/files.shtml

The archive is also available from funet.fi

thanks to Kristian Ukkonen. That URL is:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil Improvements

Date: Mon,  2 Feb 1998 05:31:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 01 Feb 1998 21:31:29 -0800

Subject: Re: Coil Improvements

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

> Date: Sun, 1 Feb 1998 20:30:25 -0600 (CST)

> Subject: Coil Improvements

>

> Hey, all.  Taking the advice I received on the list, I've recently

> improved my spark gap and ground connection.

>

> The new spark gap is an RQ-style in PVC pipe.  It's 10 pieces of 7/8"

> copper pipe, 4" long.  Total gap is 0.3".  It's quenched with a blower I

> kiped from a microwave exhaust.

>

> The new ground is 2 pieces of 1/2" copper rod, each 4' long, hammered all

> the way into the ground.  The ground is connected to the coil with about

> 5' of pretty thick wire.  This replaces my 18" copper pipe.  The rods are

> about 4' apart.

>

> Results aren't bad.  I can finally make the toroid break out on its own.

> However, I'm getting about 4 8" sparks instead of one long one.  Could my

> toroid be too bumpy?  It's a 4x14" aluminum dryer duct wrapped with

> individual strips of aluminum tape to cover the surface.  The surface of

> it is a little crinckley.

>

> Any suggestions?  My next inprovement will be a plate cap to replace my

> bottle caps.

>

> Tim Osmulski

> tim.o@ttu.edu

Tim,

Congrats - You're getting there! You don't need to put tape on top of

the aluminum dryer duct - the smooth profile works great and the

corrugations left as they are do not cause any problems. I'd suggest

removing the tape except for where the ends come together.

Alternatively, you can carefully smooth out foil to reduce premature

corona breakout. Getting a better cap will also improve performance

dramatically...

Safe coilin' to you, Tim!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Rotary Gap

Date: Mon,  2 Feb 1998 16:37:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 02 Feb 1998 10:37:39 -0600

Subject: Rotary Gap

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have replaced my static gap with a rotary gap, which I borrowed from a friend.

The performance was incredable. Continuous 48"-50" streamers. The sound was similar

to that of a gas turbine engine. Tonight I plan on doubling the power output.

I'll be going from 3 KVA to about 6 KVA. This is only an experiment. I don't plan on

anything except a  short run.

Until tomarrow,

Fleischwulf

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Beware of conductive black tubing!

Date: Mon,  2 Feb 1998 17:50:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 02 Feb 1998 11:50:01 -0600

Subject: Beware of conductive black tubing!

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Be advised - some black rubber tubing is actually

quite conductive.  Using anything of this nature for an

insulator on your coil is probably a bad idea...

I spent about an hour hooking up about 20 pieces of

black rubber automotive vacuum line to a

water-cooled sparkgap, only to discover that the

tubing wasn't quite the insulator I had assumed...

(More about the gap itself in a later post or on my site)

Later in the day I switched to silicone tubing (fuel line

for gas-powered RC cars), which worked much

better.

In my case, a three inch length of 1/8 i.d. black

rubber hose measures infinite resistance when the

test probes are applied to the outside of the tubing.

But - if the tubing is installed over the test probes (or,

in my case, over 1/8 copper tubing) then the same

three inch length measures 500 ohms!  Yes - ohms,

not even K-ohms!

I had expected maybe 100K or 1Meg but would

have never thought anything as low as ~200 ohms

per inch!!

So look out - the stuff can be misleading...  It'd be a

bad idea to use this stuff anywhere near the

secondary of a coil - as I have no doubt a 1/4 million

volt streamer would have no difficulty running

down a length of tubing right into your hand, etc...

That'd ruin your day at the least.

-Bill the arcstarter

Starting water-cooled arcs in Cinci, OH

Future member of the 50KW club.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Quenching is the problem. what is the solution?

Date: Mon,  2 Feb 1998 17:58:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 2 Feb 1998 09:58:07 -0800 (PST)

Subject: Quenching is the problem. what is the solution?

If you will recall, I'm the guy trying to get a 6.25x25 inch coil

to fire with only 720VA input, and a 6"x15" toriod. I was needing a

bump to make it fire.

I did a little experiment this weekend that I believe points to the fact

that quenching is indeed my problem.

I set my variac to just below the point where the gap would fire

constantly. Net result was the coil went into a sort of single-shot

mode, where I was getting about 1 arc/sec in the gap. With this setting

each time the gap fired, I would get an approximate 12" breakout from

the toroid in random places. In other words,

the coil is indeed able to break out - but I think gap quench is

limiting my performance.

I am looking for a motor to modify to sync operation - and will

series this with my RQ gap. Does it sound like I am barking up the

wrong tree, or on the road to better performace?

[ one other point - since I am looking for sync gap, just how much drift

in phase is there for a non-salient pole sync motor? I would have

thought it would grab at 90 degree intervals.. which would not matter.

however, if it is not locked that way, I can see problems in the future]

[and I almost have the wife talked into the Pig :)!]

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Need Help

Date: Mon,  2 Feb 1998 18:16:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: NebStrebor@aol.com

Date: Mon, 2 Feb 1998 13:16:33 EST

Subject: Need Help

Hi everyone my name Is Ben Roberts from Ohio, I guess I could be called an

ambitious teenager that wants to build a tesla coil. I've been reading the

archives but I'm so new that I really need some of it explained in English :)

I need to know some of the specs on what kind of transformer I'll need and a

simple way to build a capacitor ( and I might have a cheap source for a good

capacitor so what are the specs on a economy capacitor?) Basically I need a

detailed parts list for a small coil.

Neb

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Beware of conductive black tubing!

Date: Mon,  2 Feb 1998 20:57:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 2 Feb 1998 15:57:31 -0500

Subject: Re: Beware of conductive black tubing!

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Beware of conductive black tubing!

> Date: Monday, February 02, 1998 12:50 PM

>

> From: arcstarter@mailexcite.com

> Date: Mon, 02 Feb 1998 11:50:01 -0600

> Subject: Beware of conductive black tubing!

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>  Be advised - some black rubber tubing is actually

> quite conductive.  Using anything of this nature for an

> insulator on your coil is probably a bad idea...

>

>  I spent about an hour hooking up about 20 pieces of

> black rubber automotive vacuum line to a

> water-cooled sparkgap, only to discover that the

> tubing wasn't quite the insulator I had assumed...

>

BIG SNIP...

Bill,

You may also find it something of a surprise to realize that

the WATER conducts pretty darn well, too. So even if you

use wonderfully insulative rubber tubing, be prepared to

have a lot more than just *water* flowing through your

tubing.

Oil cooling is possible, but then use a light transformer

oil or you won't be able to pump the stuff too easily!!!

Pouring a little oil on troubled waters,

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Rotary Gap

Date: Mon,  2 Feb 1998 21:13:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 2 Feb 1998 16:13:37 -0500

Subject: Re: Rotary Gap

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Rotary Gap

> Date: Monday, February 02, 1998 11:37 AM

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 02 Feb 1998 10:37:39 -0600

> Subject: Rotary Gap

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>   I have replaced my static gap with a rotary gap, which I browered from a friend.

> The performance was incredable. Continious 48"-50" streamers. The sound was similar

> to that of a gas turbine engine. Tonight I plan on doubling the power output.

> I'll be going from 3 KVA to about 6 KVA. This is only an experiment. I don't plan on

> anything except a  short run.

>

> Until tomarrow,

>

> Fleischwulf

> ---

> Einigkeit und Freiheit

>

Fleischwulf,

Welcome to the wonderful world of rotaries!

The usual static gap is only good out to about 3 KVA. Beyond that you are

generally better off with a rotary gap, especially one with multiple

active gaps. One reason is quenching. A rotary is better at quenching.

We assume, of course, that the rotary in question is a fairly robust

rotary. Some of the smaller rotaries can easily suffer from over-

heating. A well designed rotary actually runs pretty cool.

Hope this helps.

Fr, Tom Mcgahe

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Beware of conductive black tubing!

Date: Mon,  2 Feb 1998 22:56:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 2 Feb 1998 14:56:01 -0800 (PST)

Subject: Re: Beware of conductive black tubing!

On  2 Feb, Tesla-2 wrote:

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Mon, 2 Feb 1998 15:57:31 -0500

> Subject: Re: Beware of conductive black tubing!

> Bill,

> You may also find it something of a surprise to realize that

> the WATER conducts pretty darn well, too. So even if you

> use wonderfully insulative rubber tubing, be prepared to

> have a lot more than just *water* flowing through your

> tubing.

>

> Oil cooling is possible, but then use a light transformer

> oil or you won't be able to pump the stuff too easily!!!

>

> Pouring a little oil on troubled waters,

> Fr. Tom McGahee

Almost right. Water is, by itself, a wonderful insulator. Really.

It's all those damnnable dissolved salts that are the problem.

We use de-ionized water to cool the magnets on our accelerator.

Granted, we are talking only 50-200V, so a few megaohms is sufficient

to prevent shorting.

The catch is that it costs us a small fortune to keep that water

de-ionized, and the filters are considered hazmat when we are done.

*and* our copper buss pipes are slowly dissolving.

Emulsifying that oil slightly...

Michael Baumann

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary to secondary diameter

Date: Tue,  3 Feb 1998 00:24:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Mon, 2 Feb 1998 19:24:59 EST

Subject: primary to secondary diameter

Is there a desired ratio between the diameter of your secondary and the

diameter of your primary? (by the way, thanks for the advice on the secondary

length - I would have over wound trying to get a 6 to 1 ratio h/d) Also is the

spacing of the turns on the secodary critical? I'm going with your advice on a

4 1/2"D. / 24 gauge coil length of 22" and plan on using #10 copper wire to

wind the primary. I was going to try a 10"D. (now 14 turns at 1/4"spacing,

again thanks to your advice) but I was wondering if a different spacing or

O.D. would be better. I'm using 15000V/30ma neon tranny w/.0053 Cap. Thanks

for all the help so far. I'll get it finished yet!

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rotary Gap

Cc:

Bcc:

X-Attachments:

At 04:37 PM 2/2/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Mon, 02 Feb 1998 10:37:39 -0600

>Subject: Rotary Gap

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>   I have replaced my static gap with a rotary gap, which I borrowed from

>a friend. The performance was incredible. Continuous 48"-50" streamers.

>The sound was similar to that of a gas turbine engine. Tonight I plan on doubling

>the power output.

>I'll be going from 3 KVA to about 6 KVA. This is only an experiment. I don't plan on

>anything except a short run.

>

>Until tomorrow,

>

>Fleischwulf

>---

>Einigkeit und Freiheit

Paul,

Sounds good. Make sure that you use some of your

static gap in series with the rotary. Dont try to

make the rotary do all the work. Especially as you ramp

up the power. Treat the rotary as two static gaps. (depends on design)

So if you were using 8 series gaps on your static

gap cut it down to 6 with the rotary in series.

A couple less to start might be wise.

This breaks up the arc and most of the heat will be

dissipated in your static gap rather than the rotary.

The static gaps do the work and the rotary is merely

a commutator.

If your rotary is asynchronous (variable speed)

resist the temptation to run high speeds. Very high

pulse rates will literally beat your capacitors to death.

One note to others. Dont view a rotary gap as a means

toward great performance from your coil. If your coil

is a poor performer on a static gap it will still be

poor with a rotary. Also dont use a rotary with neon

sign transformers. They cant take the abuse.

A properly designed SYNCHRONOUS rotary can be used

successfully with neons but is really unnecessary.

Neons work just as well with a good static gap. And if

your sync rotary gap gets out of sync...poof...

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: NST Problems...

Date: Tue,  3 Feb 1998 02:12:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Matt P." <ch038map@mode.lanl.k12.nm.us>

Date: Mon, 2 Feb 1998 19:12:48 -0700 (MST)

Subject: NST Problems...

Hi All-

The other day I was experimenting with my NST and something bad

happened. I don't recall what I did, but my transformer suddenly wouldnt

hardly spark. I can still draw a small arc, but its not as big as it

should be. What could have I done? How can I fix it, am I going to have to

unpot it or something? thanks for any help.

Newbie in distress,

Matt P.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Variac for Sale

Date: Tue,  3 Feb 1998 04:14:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tedd Payne <tpayne@netnitco.net>

Date: Mon, 02 Feb 1998 22:14:00 -0600

Subject: Variac for Sale

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me your zip code (to determine shipping cost).

Thanks,

Tedd

General Radio Company "Variac", model V10, 10 amp.  Input 120VAC, output 0-135VAC.

Includes knob and dial plate.  Good condition.  Could probably be used

for a little higher current for short periods of time.  $45 plus

shipping.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: NST Problems...

Date: Tue,  3 Feb 1998 04:14:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Mon, 02 Feb 1998 22:14:40 -0600

Subject: Re: NST Problems...

Consider your neon arced through at one HV terminal.  A carbon track.

You can fix this, but it involves taking the neon out of the case, and

removing the tar.  It will work, but is messy and time consuming for a "newbie".

Don't hit the windings either :)

>Hi All-

>        The other day I was experimenting with my NST and something bad

>happened. I don't recall what I did, but my transformer suddenly wouldnt

>hardly spark. I can still draw a small arc, but its not as big as it

>should be. What could have I done? How can I fix it, am I going to have to

>unpot it or something? thanks for any help.

>                Newbie in distress,

>                    Matt P.

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

ICQ UIN # 1303360 | Nick TacoBell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: The Six Kilowatt Flop !

Date: Tue,  3 Feb 1998 13:23:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 03 Feb 1998 07:23:31 -0600

Subject: The Six Kilowatt Flop !

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Apparently a 4" coil is not capable of processing 6 Kw, or perhaps I do not

have enough capacitance. The inital power on looked promising. The first arc hit the

52" mark but then the coil began to sputter, it was kind of strange. I then reduced

the power to 4.5 Kw. The streamers were very bright, almost white. Can anyone tell

if my hypothesis is correct for this power level ?

Paul M.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Beware of conductive black tubing!

Date: Tue,  3 Feb 1998 15:11:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 03 Feb 1998 09:11:09 -0600

Subject: Re: Beware of conductive black tubing!

Organization: MailExcite  (http://www.mailexcite.com)

Fr. Tom wrote:

>BIG SNIP...

>

>Bill,

>You may also find it something of a surprise to realize that

>the WATER conducts pretty darn well, too. So even if you...

Yes - I was aware of this.  I plan on using distilled

or deionized water to cut down on the conductivity,

and plan on having to flush/replace this fluid often.

I expect it to pick up copper (etc) ions rather quicky,

since it'll have 15000 volts across it...

And if it doesn't work, then I'll have to use oil as

per your suggestion.

Thanks! (Haven't actually tried running water through it yet)

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: The Six Kilowatt Flop !

Date: Tue,  3 Feb 1998 17:42:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 3 Feb 1998 09:42:51 -0800 (PST)

Subject: Re: The Six Kilowatt Flop !

On  3 Feb, Tesla-2 wrote:

> From: fleischwulf@mailexcite.com

> Date: Tue, 03 Feb 1998 07:23:31 -0600

> Subject: The Six Kilowatt Flop !

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>  Apparently a 4" coil is not capable of processing 6 Kw, or perhaps I do not

>have enough capacitance. The inital power on looked promising. The first arc

>hit the  52" mark but then the coil began to sputter, it was kind of strange.

>I then reduced the  power to 4.5 Kw. The streamers were very bright, almost white.

> Can anyone tell if  my hypothesis is correct for this power level ?

>

> Paul M.

Paul: This is a shot in the dark on my part, as I am still learning,

but based on what I have gleaned from my research, your 4" coil

should not have a problem handling the power.. *provided* you have

the ability to deal with the current. This means 2 things--

1)You must have the mother of all grounds.  But this you already knew.

2)your secondary must be wound with some pretty substantial wire, as

secondaries go that is. 22 or 20 AWG minimum would be my guess.

Consider trying a 2-3 minute run and then checking the lower 1/3 of your

secondary for warmth. If its getting warm, that shows excessive

currents.

[ This might explain the 52" arcs that devolved to sputter. the coil

warmed, so the resistance rose.]

Then again, I could be completely wrong. Comments from the more

experienced coilers???

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Need Help

Cc:

Bcc:

X-Attachments:

At 06:16 PM 2/2/98 GMT, you wrote:

>From: NebStrebor@aol.com

>Date: Mon, 2 Feb 1998 13:16:33 EST

>Subject: Need Help

>

>

>

>   Hi everyone my name Is Ben Roberts from Ohio, I guess I could be called an

>ambitious teenager that wants to build a tesla coil. I've been reading the

>archives but I'm so new that I really need some of it explained in English :)

>I need to know some of the specs on what kind of transformer I'll need and a

>simple way to build a capacitor ( and I might have a cheap source for a good

>capacitor so what are the specs on a economy capacitor?) Basically I need a

>detailed parts list for a small coil.

>            Neb

Ben,

The best transformers for coils are neon sign

transformers with voltage ratings of 9kv or 12kv.

Avoid 15kv neons for now. The extra voltage doesnt

give you that much more performance and is very much

more difficult to build capacitors for.

So look up all the sign shops in your area and look

for used 9kv/30ma or 12kv/30ma neons. 60ma units are

more robust and withstand more abuse than the 30ma units

but deliver more power than you need for a first coil.

You can parallel 30ma units together to increase your

power level later.

Most of the coil will be built around whatever transformer

you have so the first step is to acquire a couple transformers.

This is especially true of your capacitor. Its capacitance and voltage

rating are determined by your supply transformer.

For capacitor building info check out Dave Bronakowski's

archive site and you will find several files on cap

construction there. The URL is:

http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Quenching is the problem. what is the solution?

Cc:

Bcc:

X-Attachments:

At 05:58 PM 2/2/98 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Mon, 2 Feb 1998 09:58:07 -0800 (PST)

>Subject: Quenching is the problem. what is the solution?

>

>If you will recall, I'm the guy trying to get a 6.25x25 inch coil

>to fire with only 720VA input, and a 6"x15" toriod. I was needing a

>bump to make it fire.

>

>I did a little experiment this weekend that I believe points to the fact

>that quenching is indeed my problem.

>

>I set my variac to just below the point where the gap would fire

>constantly. Net result was the coil went into a sort of single-shot

>mode, where I was getting about 1 arc/sec in the gap. With this setting

>each time the gap fired, I would get an approximate 12" breakout from

>the toroid in random places. In other words,

>the coil is indeed able to break out - but I think gap quench is

>limiting my performance.

>

>I am looking for a motor to modify to sync operation - and will

>series this with my RQ gap. Does it sound like I am barking up the

>wrong tree, or on the road to better performace?

>[ one other point - since I am looking for sync gap, just how much drift

>in phase is there for a non-salient pole sync motor? I would have

>thought it would grab at 90 degree intervals.. which would not matter.

>however, if it is not locked that way, I can see problems in the future]

>

>

>[and I almost have the wife talked into the Pig :)!]

>

>Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

>San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

I believe you would probably benefit from improving your gap

but going to a rotary is not necessary. Especially considering

your power level. A rotary is never really necessary below 3 KVA.

You may want to consider building a more robust static gap.

Larger diameter and longer electrodes will improve operation

as well as improving airflow.

Consider changing your gap over to a vacuum quenched gap.

Suck air through the gap with a high speed vacuum cleaner

or shop vac rather than blowing air through it.

My vacuum gap is a modified version of the RQ style cylinder

gap. Both ends of the PVC pipe housing are sealed with disks

of plexiglass. One of the disks has an outlet to which I attach

the vacuum cleaner hose.

The airflow is admitted into the gap through 1/8 inch wide slots

cut into the PVC pipe housing. The airflow goes in through

the slots and directly between the electrodes.

This gap design has worked very well and quenches as well

as any rotary gap. In fact Gary Weaver used my gap design

on his 10 inch coil at over 9KVA to get 116 inches of spark.

No rotary gap needed.

You may also want to increase your power level some.

34 inches from 720 VA is not bad at all. I would consider

adding a 12kv/30ma neon in parallel with your 60ma to

get up to 1080 VA. This should give you enough extra that

breakout wont be a problem.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: primary to secondary diameter

Cc:

Bcc:

X-Attachments:

At 12:24 AM 2/3/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Mon, 2 Feb 1998 19:24:59 EST

>Subject: primary to secondary diameter

>

>Is there a desired ratio between the diameter of your secondary and the

>diameter of your primary? (by the way, thanks for the advice on the secondary

>length - I would have over wound trying to get a 6 to 1 ratio h/d) Also is the

>spacing of the turns on the secodary critical? I'm going with your advice on a

>4 1/2"D. / 24 gauge coil length of 22" and plan on using #10 copper wire to

>wind the primary. I was going to try a 10"D. (now 14 turns at 1/4"spacing,

>again thanks to your advice) but I was wondering if a different spacing or

>O.D. would be better. I'm using 15000V/30ma neon tranny w/.0053 Cap. Thanks

>for all the help so far. I'll get it finished yet!

>Jim

>

>

Jim,

Your primary diameter depend on the type of primary you

are building. If you are building a solenoid type primary

then it should be more than twice the diameter of your secondary.

So 10 inches diameter would be good for your 4.5 inch diameter

secondary.

If you are building a saucer shaped primary with a 15-30 degree

slope then you want the innermost turn spaced far enough away

from the secondary to prevent arcing but no more. At 15kv

you will want at least 1 inch spacing or a little more.

This means a 6.5 inch diameter for the inner turn. Going

to 7 inches would be fine. Space the turns 1/4 inch.

Either primary will work fine. The solenoid might be easier

to build and would be just fine for a first coil. Just make

provision for varying the distance between the primary and

secondary coils so you can adjust the coupling.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST Problems...

Cc:

Bcc:

X-Attachments:

At 02:12 AM 2/3/98 GMT, you wrote:

>From: "Matt P." <ch038map@mode.lanl.k12.nm.us>

>Date: Mon, 2 Feb 1998 19:12:48 -0700 (MST)

>Subject: NST Problems...

>

>

>Hi All-

>        The other day I was experimenting with my NST and something bad

>happened. I don't recall what I did, but my transformer suddenly wouldnt

>hardly spark. I can still draw a small arc, but its not as big as it

>should be. What could have I done? How can I fix it, am I going to have to

>unpot it or something? thanks for any help.

>                Newbie in distress,

>                    Matt P.

Matt you've fallen victim to a problem lots of us face.

Your neon was probably a used unit. As such it may well

have been walking wounded. It probably failed in service

and was tossed in the junk pile until you came along.

Sitting around for a few years allows some of these neons

to partially heal themselves. So when you first test the unit

it works fine. After a little use the old wound opens back up

and the transformer fails.

I've had neons test fine and work great on a Jacobs Ladder

only to fail in seconds on a Tesla coil. This is the chance

we take when using used transformers.

To repair your unit you have two options.

First you can heat it up in an oven until the tar melts

and flows. This will repair the most common failure in

neon transformers. This is the carbon track short.

An area of the tar potting compound becomes carbonized

and conductive after a few arcs go through it. Melting the

tar  breaks up the carbon track. But this is only about

a 50 percent chance of success.

The best way is to unpot the transformer completely

removing the potting compound. This is messy work

but you can end up with a better transformer in the long

run. Ive successfully unpotted and rebuilt 7 neon transformers.

For some info on neon rebuilding check the file neons.txt

on dave Bronakowski's archive site.

http://www.velocity.net/~djb/files.shtml

For some pictures of my latest rebuilding effort

check my web site at: http://www.misslink.net/mhammer/rebuild.htm

good luck.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: The Six Kilowatt Flop !

Cc:

Bcc:

X-Attachments:

At 01:23 PM 2/3/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Tue, 03 Feb 1998 07:23:31 -0600

>Subject: The Six Kilowatt Flop !

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>   Apparently a 4" coil is not capable of processing 6 Kw, or perhaps I do not have

>enough capacitance. The inital power on looked promising. The first arc hit the 52"

>mark but then the coil began to sputter, it was kind of strange. I then reduced the

>power to 4.5 Kw. The streamers were very bright, almost white. Can anyone tell if

>my hypothesis is correct for this power level ?

>

>Paul M.

>

>---

>Einigkeit und Freiheit

Paul,

What exactly do you mean by sputter?

Do you mean the gap was sputtering and not firing properly?

Or do you mean something else entirely?

If its a gap problem you probably over whelmed your gap system.

It no longer quenched. In fact it may not have quenched at all

staying lit all the time.

Did you use some static gaps in series with the rotary?

This is very important. Dont expect the rotary to do the

job all by itself.

If the gap wasnt firing at all you may have lost a

resonant charging effect that was boosting your primary

voltage. If you had a series static gap try closing down the

gap a bit.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: The Six Kilowatt Flop !

Date: Wed,  4 Feb 1998 04:22:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 03 Feb 1998 22:22:10 -0600

Subject: Re: The Six Kilowatt Flop !

Organization: MailExcite  (http://www.mailexcite.com)

It was written: (by fleishwulff)

>Coilers,

>   Apparently a 4" coil is not capable of processing 6 Kw, or perhaps I do not

.....

>power to 4.5 Kw. The streamers were very bright, almost white. Can anyone tell

...

Paul,

When are you going to burn your house down?  I would

think all the ionized smoke particles would greatly

increase the streamer lengths...

If you do this, don't pour any conductive water on

the ensuing mess... :)

Coilers - we will soon have streaming video of our coil

assaulting various inanimate objects via judicious use of 30 gauge wire, strung

from the toroid to ...?? Coming soon!

-Bill the arcstarter

Future member of the 50KW club.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Humidity?

Date: Thu,  5 Feb 1998 01:54:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Wed, 4 Feb 1998 19:54:09 -0600 (CST)

Subject: Humidity?

Recently I was able to get 18" sparks from my 4" coil.  I felt that I was

nearing the end of my tweaking, having to build a better cap before

continuing.  However, I recently fired up my coil, and couldn't get more

than 6".  Re-tunning and sanding the gap clean didn't help.  It's raining

in Texas right now.  I know humidity can affect performance, but in what

way?  Is my poor performance tonight due to the rain?

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Humidity?

Date: Thu,  5 Feb 1998 08:18:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Thu, 05 Feb 1998 02:18:09 -0600

Subject: Re: Humidity?

Tesla-2 wrote:

>

> From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

> Date: Wed, 4 Feb 1998 19:54:09 -0600 (CST)

> Subject: Humidity?

>

> Recently I was able to get 18" sparks from my 4" coil.  I felt that I was

> nearing the end of my tweaking, having to build a better cap before

> continuing.  However, I recently fired up my coil, and couldn't get more

> than 6".  Re-tunning and sanding the gap clean didn't help.  It's raining

> in Texas right now.  I know humidity can affect performance, but in what

> way?  Is my poor performance tonight due to the rain?

>

> Tim Osmulski

> tim.o@ttu.edu

>

It is very possible that it is the weather. I have always had poor

results when the humidity is real high or when it is cold and moist

outside. I wish I knew why but I guess that is just one of those things.

By the way where in Texas are you, I live in Texas too. I also have a 4"

coil and two 6" coils. My for inch coil did ok considering it was my

first. I got a max of 19" sparks, but instead of enhancing it. I just

built a new coil. The coil I have now, 6", Should prove to be pretty

good. I am only testing it with a 7kv@60ma neon. I have a couple of salt

water caps running it. The caps value only total .001uf, this tranny can

power up to .02uf. So as you can see I am only running 5% of the maximum

capacitance. My best spark so far is 10" with a 5 series gap and .001uf

capcitance. I am hoping it will hit the 2' marl at least. If not I have

a 12kv@80ma neon I am going to replace the 7kv with.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Mail Order Coils

Date: Fri,  6 Feb 1998 21:05:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE drew <gordrew@home.com>

Date: Fri, 06 Feb 1998 13:05:59 -0800

Subject: Mail Order Coils

Any suggestions on a good company to order a tesla coil kit from? I am

looking for a smaller coil that might come in pieces (trans, cap, etc.)

Thanks,

MIKE

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Humidity?

Cc:

Bcc:

X-Attachments:

At 01:54 AM 2/5/98 GMT, you wrote:

>From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

>Date: Wed, 4 Feb 1998 19:54:09 -0600 (CST)

>Subject: Humidity?

>

>Recently I was able to get 18" sparks from my 4" coil.  I felt that I was

>nearing the end of my tweaking, having to build a better cap before

>continuing.  However, I recently fired up my coil, and couldn't get more

>than 6".  Re-tunning and sanding the gap clean didn't help.  It's raining

>in Texas right now.  I know humidity can affect performance, but in what

>way?  Is my poor performance tonight due to the rain?

>

>Tim Osmulski

>tim.o@ttu.edu

>

Tim,

Did you find out what your problem was yet? I would be

suprised if it was due to humidity. Not that big of a change.

My first thought was oxide build up on the gap electrodes.

But you said that you cleaned those up. How does that gap

fire? Smoothly and steadily?

Perhaps its a capacitor failure. If using bottle SW caps

its fairly common for one or more bottles to go bad or

change value.

Also your neon transformer could be having problems.

Unfortunately its all too common to have a neon go bad

on one side. Remove your neon from the coil and try to draw

an arc from each HV terminal to the case. The resulting arc

should be about the same on each side. If one side shows a

nice fat arc and the other side a weak spindly arc (or no arc at all)

then thats your problem.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Mail Order Coils

Cc:

Bcc:

X-Attachments:

At 09:05 PM 2/6/98 GMT, you wrote:

>From: MIKE drew <gordrew@home.com>

>Date: Fri, 06 Feb 1998 13:05:59 -0800

>Subject: Mail Order Coils

>

>

>

>Any suggestions on a good company to order a tesla coil kit from? I am

>looking for a smaller coil that might come in peices (trans, cap, etc.)

>Thanks,

>MIKE

>

Mike,

I can tell you where NOT to buy a kit. Do not buy

the kit offered by Information Unlimited.

It is a poor design, cannot be brought to resonance with

the supplied components and is grossly overpriced.

I recommend you build your own "kit". You can find all the

information you need on the Internet to build a much better

coil for 1/4 the price of that kit.

Check out Dave Bronakowski's archive site and download

and read some of the material there. Dave has collected

some of the best Tesla material and put it on his well

organized site. Also download and read through the Tesla-2

list archives available there if you havnt already done so.

The URL for Dave's site is:

http://www.velocity.net/~djb/files.shtml

One of the first things you need to do is locate a neon

sign transformer. Get out the yellow pages and find all

the sign companies in your area. You want 9kv/30ma or 12kv/30ma

neon sign transformers to start. Other voltages are usable

and you often have to take what you can get.

Also usable are oil burner igntion transformers. (OBIT)

These are used to ignite the burners on oil burning furnaces.

The have no resale value so you can often get them for nothing

from furnace contactors and repair companies. They are usually

rated 10kv/23ma and work well for a small to medium size coil.

The rest of the coil is designed around your transformer

so you need to get the transformer first.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: R F Chokes

Date: Sat,  7 Feb 1998 01:18:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Ryan <jryan@exis.net>

Date: Fri, 06 Feb 1998 20:18:20 -0500

Subject: R F Chokes

From: Jim

I see on the net and understand the caps, spark gaps, primary and secondary

coils and transformers. I have yet to see what or how the RF Choke is used in the

circuit.

Can anyone explain ?

Thanks

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: R F Chokes

Cc:

Bcc:

X-Attachments:

At 01:18 AM 2/7/98 GMT, you wrote:

>From: Jim Ryan <jryan@exis.net>

>Date: Fri, 06 Feb 1998 20:18:20 -0500

>Subject: R F Chokes

>

>From: Jim

>

>I see on the net and understand the caps, spark gaps, primary and secondary

>coils and transformers.

> I have yet to see what or how the RF Choke is used in the circuit.

>

>Can anyone explain ?

>

>Thanks

>

>Jim

>

Jim,

RF chokes are used as safety devices to offer a measure of

protection to your neon sign transformer. Neons are relatively

fragile and excess RF in their secondary windings will cause

premature failure.

The chokes are placed in series with each HV lead in between

the safety gap and main gap. They should have a value of

between 2-10 mh.

They help to isolate the neon secondary windings from the

primary tank circuit by providing a high impedance to the

RF energy.

RF chokes cant provide all the needed protection by themsleves.

They only serve to knock down the RF level somewhat.

Also needed and more important is the safety gap. This gap

is placed directly across the neon output. It is set just slightly

wider than the main gap. Its function is to break down and clamp

the voltage to a safe level if it begins to rise too high.

You can run your coil without RF chokes but you CANNOT run without

a safety gap.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Tap lead

Date: Sat,  7 Feb 1998 18:57:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 7 Feb 1998 18:57:50 +0000

Subject: Tap lead

I just finished my new primary, and have to make a new, longer,

primary tap lead.  My question is, should I keep the length of this

wire to a minimum, or does it really matter (as long as it is

suitable for the current/voltage being carried)?

The reason I ask is that I have all my tap points set on the "front"

half of the primary.  I can make the lead only long enough to reach

these points, but if later I find I need to tap somewhere on the

"back" side of the coil, I'll have to have a longer lead.

Which leads me to a second question:  I have taps points at 13-3/4,

13-7/8, 14-1/8, 14-1/4, 14-3/4, 14-7/8. . .and so on, up to 16-1/4

turns.  How likely is it that I will not be able to find a tap point

that tunes reasonably well only using these points (neglecting the

3/8, 1/2 and 5/8 points)?

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Variac

Date: Sat,  7 Feb 1998 19:00:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 7 Feb 1998 19:00:57 +0000

Subject: Variac

If anyone is looking for a good price on a new 10amp variac, MCM

Electronics has one on sale for $79.99.  ($5 more than I paid for

mine, used, without the original case or connections, and without an

ammeter built in. . .boy-howdy am I kicking myself!)

If you need the contact info post me a note.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RF Coils

Date: Sat,  7 Feb 1998 22:46:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Ryan <jryan@exis.net>

Date: Sat, 07 Feb 1998 17:46:14 -0500

Subject: RF Coils

I guess I asked the wrong Question.  Does a coiler make RF Chokes

or buy them?

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 00:28:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sat, 7 Feb 1998 19:28:58 EST

Subject: Specific functions of Tesla coil parts

I am a "newbie" coil builder (although quite old in actual years) and would

appreciate an explanation of the "role" or function of the spark gap and

capacitor in the Tesla Coil circuit. (please excuse my seeming ignorance but I

would like to confirm my own opinions on their functions). Also I would like

to understand why a poly cap works so much better than salt water caps with

the same rating. Sorry if this is such a basic question but, for my own sake,

I need to fully understand this. Thanks to all who care to reply.

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Top load design

Date: Sun,  8 Feb 1998 01:37:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 8 Feb 1998 01:37:21 +0000

Subject: Top load design

I fired-up up my new coil and actually achieved spontaneous streamer

break-outs (12" or so, nothing spectacular).  Unfortunately several

of which came off of the top winding of the sec'.

So, by advice of various list members, it seems that my next

step is to build a better top load, a toroid.  Any suggestions

regarding the diameter of a 4" toroid, to be placed atop a 6.25x19"

sec'?  How much does the diameter effect the pf value of the toroid?

TIA,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My tesla coil's avis

Date: Sun,  8 Feb 1998 01:56:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Sat, 07 Feb 1998 19:56:46 -0600

Subject: My tesla coil's avis

I recently captured some video of my new poly caps in action!

I am getting 36"-41" streamers after my SW caps were replaced.

I recorded some video of the coil running with a camcorder and

placed those AVI's on the internet at my webpage.  Check em

out if ya feel inclined! :)

http://www.harvestcomm.net/personal/corr/hv/index.htm

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

ICQ UIN # 1303360 | Nick TacoBell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF Coils

Date: Sun,  8 Feb 1998 02:01:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Sat, 07 Feb 1998 20:01:19 -0600

Subject: Re: RF Coils

>I guess I asked the wrong Question.  Does a coiler make RF Chokes

>buy them?

Either.  Chanced are good that if you get wire and wind a secondary,

you'll have enough wire to wind RF chokes too.  I was told to make

my chokes the following way :

150-300 turns of #24 wire wound around 2" PVC pipe.

(kinda like a small secondary I guess).  Wind it 8" long.

Make two of these.  One for each high voltage terminal on the neon.

They don't do any good though unless you have a safety gap between

them and the neon....

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

ICQ UIN # 1303360 | Nick TacoBell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 02:20:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jeff Corr <corr@enid.com>

Date: Sat, 07 Feb 1998 20:20:32 -0600

Subject: Re: Specific functions of Tesla coil parts

>I am a "newbie" coil builder (although quite old in actual years) and would

>appreciate an explanation of the "role" or function of the spark gap and

>capacitor in the Tesla Coil circuit. (please excuse my seeming ignorance

but I

>would like to confirm my own opinions on their functions). Also I would like

The "spark gap" acts as a high speed on/off switch.  It and the capacitors

set up oscillations which cause the coil to resonate.  Without this resonation,

the coil would simply be a big air-core step-up transformer, which a much lower

voltage.

>to understand why a poly cap works so much better than salt water caps with

>the same rating.

For one, glass is MUCH more lossy.  When in operation, the glass will have

a blue corona on the inside of it, representing losses.

Another thing, poly works better than glass at RF frequencies, which are

generated by the coil.

I just got rid of my SW caps and the output tripled from around 12 inches to

36-41 inches continously.  Comparitively speaking, the poly caps are just

as easy to make as the glass caps.  Timewise, they take the same amount of

time to construct.  The only downside is they must be vaccuumed to get all the

air bubbles out, which isn't always easy.

Jeff Corr   |   corr@mail.enid.com   |   580-234-6749

http://www.harvestcomm.net/personal/corr/index.htm

ICQ UIN # 1303360 | Nick TacoBell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 03:07:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Sat, 07 Feb 1998 21:07:02 -0600

Subject: Re: Specific functions of Tesla coil parts

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Sat, 7 Feb 1998 19:28:58 EST

> Subject: Specific functions of Tesla coil parts

>

> I am a "newbie" coil builder (although quite old in actual years) and would

> appreciate an explanation of the "role" or function of the spark gap and

> capacitor in the Tesla Coil circuit. (please excuse my seeming ignorance but I

> would like to confirm my own opinions on their functions). Also I would like

> to understand why a poly cap works so much better than salt water caps with

> the same rating. Sorry if this is such a basic question but, for my own sake,

> I need to fully understand this. Thanks to all who care to reply.

> Jim

>

The purpose of a spark gap and capacitor in a tank circuit is

complicated if you sit there and analyse it. But in simple terms with

out them there is no tesla coil. I am not real experienced at coils.

I have only been tinkering with them for almost a year now. I have built

only two coils and was successful at both.

Let me try to make this fairly simple. First you have your

transformer/s. The capacitor is in direct association with the

transformer, meaning the value of your capacitor is a result from how

much capacitance the transformer can handle. The spark gap acts as a

switch for the tank circuit. All in all, this means that the transformer

charges the capacitor to a point that the spark gap fires, releasing the

charge into the primary resulting in the voltage rise in the secondary.

With out the capacitor in the circuit the coil would not operate due to

lack of power from the cap plus it is highly unlikely the circuit will

resonate with out it. With out the spark gap, the circuit will not work

at all due to fact that the cap is being shorted out, not allowing it to

charge. Personally speaking I think that most emphasis should be placed

on the cap and spark gap, becuse they are the heart of the circuit.

With out them the circuit will not "come alive".

As for the poly caps being better than glass caps. I can't really tell

you a whole lot about that. I have yet to build any poly caps. I have

stuck with saltwater caps for their cheapness basically. For example my

first coil I used saltwater caps and after a few trail and errors I

finally got a fairly good constant spark of 19". It was a 4"x20" wound

with 20awg wire w/a 1/4" 15 turn primary banked at 30 degrees. I ran

this coil with a 15kv@30ma neon until I toasted it because I did not use

a safety gap. Back to the poly caps, they are better because they have

one of the lowest loss factors at high frequencies. Glass is a very

lossy material, why I don't know. My guess is that it has something to

with it density.

Hopefully I helped out some what. If not e-mail me and I will try and

help a little more when time permitts.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 03:38:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 7 Feb 1998 19:38:31 -0800 (PST)

Subject: Re: Specific functions of Tesla coil parts

On Sun, 8 Feb 1998, Tesla-2 wrote:

> I am a "newbie" coil builder (although quite old in actual years) and would

Welcome to the crowd.. (Sometimes I think I am one of the older

"newbies" as well..

> appreciate an explanation of the "role" or function of the spark gap and

> capacitor in the Tesla Coil circuit. (please excuse my seeming ignorance but I

> would like to confirm my own opinions on their functions). Also I would like

> to understand why a poly cap works so much better than salt water caps with

> the same rating. Sorry if this is such a basic question but, for my own sake,

> I need to fully understand this. Thanks to all who care to reply.

> Jim

Ok.. here comes a newbie approach at explaining it:

When you look at the tesla 'tank' there are actually 3 components

involved, The primary coil, the capacitor, and the sparkgap.

The cap serves 2 purposes: One: It stores a tremendous amount of energy

from your power source, and two: forms 1/2 of a LC resonant circut.

The sparkgap serves as a switch, closing the LC circut that forms the

primary tank. All of the stored energy dumps out of the cap, into the

coil, and back again... at the resonant frequency of the tank. If the

tank is tuned to the resonant frequence of the secondary, we get the big

sparks.

As for why Poly works better than glass: Every diaelectric resists

changing charge. When the capacitor is resonating with the primary

coil, it is changing charge pretty fast, very fast actually. In a

lossy diaelectric, this resistance to change appears as heat. It takes

energy out of the system that could be making sparks. The loss factors

are relative to air, which is normalized to 1. Glass I think is in the

50 range, where poly is something like .001. So we are talking 5 orders

of magnitude here. Fairly significant.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 04:10:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sat, 7 Feb 1998 23:10:58 EST

Subject: Re: Specific functions of Tesla coil parts

Hi Jim,

Your power supply charges your primary capacitor every 1/120 second.  The

spark gap acts as a switch, allowing the energy to be built up until the

voltage in the cap becomes high enough to ionize the air between the gap,

making the air conductive.  Once this occurs, the energy is released into the

primary coil.  This surge occurs very quickly by comparison to the charging

time and then oscillates between the primary coil and the capacitor(called the

tank circuit).  The gap shuts off once the voltage drops below the ionization

potential of the air and allows the capacitor to be charged for the next

cycle.   It is also said that very quick quenching of that spark traps the

resonant energy in the secondary coil where it is transmitted (or dissipated)

into the air.  If rapid quenching is not achieved,  some of that energy is

transferred between the primary and secondary coils several more times and is

dissipated in that transfer (due to the transfer being less than 100%

efficient).

The reason that plates caps are superior to salt water caps is the

internal resistance.  A capacitor builds up a charge on the surfaces of the

dielectric.  This energy is transferred into and out of the cap by conduction

through the plates or through salt water.  Being that metal is far superior in

conducting electric current than salt water,  less energy is wasted through

resistance in the plate cap.   This resistance loss appears simply as heat

within the capacitor and reduces the efficiency of the system.

I hope this helps.

Mike Hollingsworth

+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Humidity?

Date: Sun,  8 Feb 1998 18:40:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Sun, 8 Feb 1998 12:40:18 -0600 (CST)

Subject: Re: Humidity?

On Fri, 6 Feb 1998, Tesla-2 wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 06 Feb 1998 15:24:41 -0600

> Subject: Re: Humidity?

>

<snip>

> Did you find out what your problem was yet? I would be

> suprised if it was due to humidity. Not that big of a change.

I'm not sure what was wrong, but when the sun came back, so did my sparks.

Maybe there's something weird about my coil that makes it overly

susceptible to changes in the weather.  I did notice something

interesting, however.  When it was rainy, when I first plugged in the

coil, it would occasionally jump out to 14" for an instant before

shrinking back down to 6-8".  It would only do this after I unplugged and

then restarted it.  Since it only did this for a brief instant after

starting, I'm questioning whether it was heat-related.  Any thoughts?

> But you said that you cleaned those up. How does that gap

> fire? Smoothly and steadily?

The gap is firing smooth and steady, but I noticed that the sparks

themselves consist of 2 or 3 hot points of light between each electrode.

I wasn't going to sit and stare at it to see if they moved around, but I

believe they were very stationary.  Is this normal?

> Perhaps its a capacitor failure. If using bottle SW caps

> its fairly common for one or more bottles to go bad or

> change value.

I've just started designing my poly caps.  I'll post the specifics when I

get them figured out.

Thanks,

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tap lead

Cc:

Bcc:

X-Attachments:

At 06:57 PM 2/7/98 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sat, 7 Feb 1998 18:57:50 +0000

>Subject: Tap lead

>

>

>I just finished my new primary, and have to make a new, longer,

>primary tap lead.  My question is, should I keep the length of this

>wire to a minimum, or does it really matter (as long as it is

>suitable for the current/voltage being carried)?

Always best to keep this length to a minimum. The heavier

the conductor used the more length you can tolerate.

The point of most loss will be the tap itself. Try to maximize

surface area and contact area here.

>

>The reason I ask is that I have all my tap points set on the "front"

>half of the primary.  I can make the lead only long enough to reach

>these points, but if later I find I need to tap somewhere on the

>"back" side of the coil, I'll have to have a longer lead.

>

>Which leads me to a second question:  I have taps points at 13-3/4,

>13-7/8, 14-1/8, 14-1/4, 14-3/4, 14-7/8. . .and so on, up to 16-1/4

>turns.  How likely is it that I will not be able to find a tap point

>that tunes reasonably well only using these points (neglecting the

>3/8, 1/2 and 5/8 points)?

>

You probably dont need to tap closer than 1/2 turn. In many cases

just tapping at 1 turn intervals will be sufficient.

On my coil I can just barely note a difference of 1/2 turn.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF Coils

Cc:

Bcc:

X-Attachments:

At 10:46 PM 2/7/98 GMT, you wrote:

>From: Jim Ryan <jryan@exis.net>

>Date: Sat, 07 Feb 1998 17:46:14 -0500

>Subject: RF Coils

>

>I guess I asked the wrong Question.  Does a coiler make RF Chokes

>or buy them?

>

>Jim

>

Jim,

You will need to make them yourself.

The most satisfactory RF chokes are wound on large ferrite

toroids. Look for toroids 2.5 inches in diameter. Smaller

ones may be used but you may need to put two in series

to get enough inductance and prevent saturation.

Cover these toroids with 3 layers of good quality vinyl

tape. The core material is slightly conductive.

Wind 100 or more turns of small guage wire. AWG 26 or 28

will be fine. You do not need heavy guage wire as you are

passing a couple 100 ma at most. Leave at least a 3/4 inch

gap between the start and finish to prevent arcing.

You need to get 2-10 mh of inductance to be effective.

The actual inductance value is not critical.

Ferrite rods will work just as well as toroids but are not

as compact. Ferrite rods and toroids show up often at the

surplus electronics dealers and I always find a few at hamfests.

If you cannot find any suitable ferrite cores you can wind

air core chokes on PVC pipe. Wind a 6 inch long winding

of AWG 26 on a piece of 2 inch PVC pipe. This will be about

350 turns and 2.5 mh of inductance.

Be careful with air core chokes. Sometimes they can ring up

quite high voltages. They begin to act like mini Tesla coils.

They can actually do more harm than good in this case.

Here is a little test you can easily make. Set up and fire your

coil with no chokes but WITH a safety gap. Note the amount

and intensity of the safety gap discharges. Now fire your coil

again but with your chokes in place. The intensity of the safety gap

discharges should be much less if your chokes are doing their job.

On my coil the safety gap sounds like a rifle shot without the RF chokes.

But with the chokes in place this is down to a soft pop.

If you see more activity with the chokes in place then your chokes

are ringing and doing more harm than good. The cure for this is to

place some resistance in series the the HV leads and RF chokes

to dampen any oscillations. Use power resistors from 300-1500 ohms

and 10 or more watts. The more power you are running the more

power your resistors need to dissipate. 10 watts should be sufficient

for most coils under 1000 watts. These resistors are not normally

needed for ferrite toroidal chokes but often are called for with air

core chokes.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: rf choke question

Date: Sun,  8 Feb 1998 19:19:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sun, 8 Feb 1998 14:19:14 EST

Subject: rf choke question

Most of the postings I've seen on proper coil set up include rf chokes between

the safety gap and the spark gap. Can someone explain just what it is they do

(how they function)  From the descriptions I've read they seem like small

secondary coils. Also what should I build to protect a 15K 30ma transformer.

I've read that 150 to 300 turns of 24 gauge wire on a 2" D. pvc pipe would be

adequate. Could anyone be a little more specific and are they wound and sealed

the same as a secondary? Thanks again all for all the help and advice. Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Humidity?

Cc:

Bcc:

X-Attachments:

At 06:40 PM 2/8/98 GMT, you wrote:

>From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

>Date: Sun, 8 Feb 1998 12:40:18 -0600 (CST)

>Subject: Re: Humidity?

>

>On Fri, 6 Feb 1998, Tesla-2 wrote:

>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Fri, 06 Feb 1998 15:24:41 -0600

>> Subject: Re: Humidity?

>>

><snip>

>> Did you find out what your problem was yet? I would be

>> suprised if it was due to humidity. Not that big of a change.

>

>I'm not sure what was wrong, but when the sun came back, so did my sparks.

>Maybe there's something weird about my coil that makes it overly

>susceptible to changes in the weather.  I did notice something

>interesting, however.  When it was rainy, when I first plugged in the

>coil, it would occasionally jump out to 14" for an instant before

>shrinking back down to 6-8".  It would only do this after I unplugged and

>then restarted it.  Since it only did this for a brief instant after

>starting, I'm questioning whether it was heat-related.  Any thoughts?

Hmm. This sounds suspiciously like poor gap quenching.

Before your gap fires that first time its clean. No heat. No

hot ions clogging up the arc channel. Once the gap fires

the area between the electrodes is clogged up with hot conductive

ions. This keeps the gap "lit" longer than it should and decreases

performance. The cure is improved airflow. Try to use a strong fan

to move plenty of air or even better use a vacuum cleaner to suck air

through the gap. Direct airflow between the electrodes so it can cool

the arc directly and best remove the hot ions.

Use series connected gaps. You should be using around 8 series

gaps for 12kv and 10 series gaps for 15kv. Having series gaps

distributes the heat across several gap surfaces instead of all

of it at one spot.

>> But you said that you cleaned those up. How does that gap

>> fire? Smoothly and steadily?

>

>The gap is firing smooth and steady, but I noticed that the sparks

>themselves consist of 2 or 3 hot points of light between each electrode.

>I wasn't going to sit and stare at it to see if they moved around, but I

>believe they were very stationary.  Is this normal?

Check to see that your electrode faces are smooth and as exactly

parallel as you can make them.

>I've just started designing my poly caps.  I'll post the specifics when I

>get them figured out.

>

Good. Keep us posted.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specific functions of Tesla coil parts

Date: Sun,  8 Feb 1998 19:53:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:46 +0500

Subject: Re: Specific functions of Tesla coil parts

On Sat, 7 Feb 1998 23:10:58 EST Mike Hollingsworth

<Hollmike@aol.com> wrote;

>     The reason that plates caps are superior to salt water caps is the

> internal resistance.  A capacitor builds up a charge on the surfaces of the

> dielectric.  This energy is transferred into and out of the cap by conduction

> through the plates or through salt water.  Being that metal is far superior in

> conducting electric current than salt water,  less energy is wasted through

> resistance in the plate cap.   This resistance loss appears simply as heat

> within the capacitor and reduces the efficiency of the system.

Mike, although you are correct in saying that salt water is a poorer

conductor than aluminum or copper, that is not the reason plate capacitors

are superior to salt water capacitors. The more significant reason is

coronal leakage! In the vast majarity of plate capacitors the entire

capacitor is immersed in insulating oil and this eliminates coronal

leakage. Most salt water capacitors have significant coronal leakage! If

you look at an operating Tesla coil in the dark that has salt water caps

you can see the strong blue glow of coronal leakage on the surface of the

water. Even pouring a layer of oil on top of the salt water does not

substantialy reduce this coronal leakage. Another big difference between

plate and salt water capacitors is the dielectric. In salt water capacitors

the dielectric is invariably glass, while most plate capacitors use

polyethylene film as the dielectric. Polyethylene has a much smaller power

factor than glass! The power factor is a measure of the amount of R.F.

energy being wasted in a capacitor heating it up, thus the lower the power

factor of a dielectric the less R.F. losses.

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: h/d aspect of secondary

Date: Sun,  8 Feb 1998 19:53:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:48 +0500

Subject: Re: h/d aspect of secondary

On Sun, 1 Feb 1998 16:52:52 EST Jim

<Dsurfr@aol.com> wrote;

> Subject: h/d aspect of secondary

>

> Can anyone give me some advice on the height /diameter ratio for winding a

> secondary coil? I have a 4 1/2" D. acrylic form and was going to try for a 6/

1

> ratio w/ a resonant freq. of 303.73 but that would mean 1,279 turns or 27"

> total height using 24 ga. wire. I have read that the total number of turns

> should not exceed 1000. I'm using a 15k 30 MA  neon transformer w/ .0053 Cap.

> I also have a lot of # 10 solid copper wire I would like to use to form the

> primary. "WinTesla" says I should have 9 turns 1/4" apart (I'm making a

> helical primary) advice on this would also be much appreciated.

Jim, experience has shown that the best height/diameter ratio on small

Tesla coils is between 3:1 and 4:1, also a secondary should not have more

than 700 turns. On one hand you are trying to get as much inductance as

possible in a Tesla coil secondary BUT you also what to keep the resistance

as low as possible to minimize losses. having a long thin secondary will

give you high inductance BUT it also gives very high resistance losses!

On the issue of primary turns, Tesla rarely used more than one or two turns

on all of his coils, today many coilers use from 5 to 15 turns in their

primaries. The general idea is to have as high an inductance as possiple in

the secondary and as low an inductance as possible in the primary, yet have

enough inductance in the primary to send significant amounts of energy to

the secondary. BTW you also should strive to have as low a distributed

capacitance as possible in the secondary while having as high a capacitance

in the primary circuit as possible.

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Faraday cage

Date: Sun,  8 Feb 1998 19:53:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:49 +0500

Subject: Re: Faraday cage

On Fri, 16 Jan 1998 20:46:18 +0000 Chris Federico

<cfed@dynasty.net> wrote;

> Will a Faraday cage block e-m fields? (such as those emitted by a

> pulsing power supply causing ripples in the display of a monitor)

>

> And, if so, what are the critical elements in constructing one?

Chris, a Faraday cage will stop R.F. or "radio waves" BUT it will do

nothing to stop magnetic fields! The general rule with Farady cages is that

the width of the holes in the cage must be less than 1/4 wavelength to

prevent a given frequency from entering the cage, and the cage must be

grounded. I prefer to be conservative and use screening such that the width

of the holes is 1/10 of a wave or less. Solid cages of course will block

all frequencies. BTW to stop magnetic fields you simply have to give them a

better path around whateveryou want to shield so if you built a Faraday

cage out of feromagnetic material (iron, nickle, cobalt, ect.) it will give

the magnetic field a good path around whatever is inside the cage.

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Vacuum pump.

Date: Sun,  8 Feb 1998 19:53:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:51 +0500

Subject: Re: Vacuum pump.

On Fri, 9 Jan 1998 17:34:57 +0000 Erik Hennix

<erik.hennix@swipnet.se> wrote;

> I've read that it's possible to convert refrigerator compressors to

> vacuum pumps, to pump capacitors with.

>

> Can someone please tell me where I can find info/plans  about how

> this is done?

Hi Erik, not all refrigerator compressors can be converted into good

vacuum pumps! The piston type pumps can never be made into anything

respectable in the way of a vacuum pump. The rotary type compressors

however are a different story! The classic was the Rolator compressor, by

adding a lead plug to block a flow back passage thay could be made into

remarkably good vacuum pumps. Rolator compressors can still occasionaly be

found in junk yards, trash dumps, the curb on trash day and in some older

refrigerator compressor repair shops, BUT they are rare. If you can find

back issues of "Science Experimenter" from the 60's they had Rolator

conversion instructions. There are a few automobile air conditioner

compressors that are rotary but which ones and how to convert each model I

don't know. There is a periodical devoted to amature vacuum research called

"The Bell Jar" it is run by Steve Hansen, he has a WEB site;

http://www.tiac.net/users/shansen/belljar/articles.htm

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Pacemaker / tesla coil

Date: Sun,  8 Feb 1998 19:53:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:53 +0500

Subject: Re: Pacemaker / tesla coil

On Wed, 07 Jan 1998 10:45:47 -0600 Paul S. Marshall

<fleischwulf@mailexcite.com> wrote;

> Coilers,

>    Can anyone tell me what effect if any a tesla coil

> can have on a pacemaker ? If there are any can they be

> protected by a faraday cage ?

Paul, thus far there have been no studies of the effects of Tesla coils on

pacemakers. Most modern pacemakers have very effective Faraday shields

built into them, but as to whether it is adaquite to prevent a near by

Tesla coil from interfearing with it's operation, one can only speculate. A

good Faraday cage, where the holes in the screen are spaced 1/10 wavelength

or less and with the cage properly grounded should stop any R.F. from a

Tesla coil reaching someone inside the cage. Be sure not to touch the cage

when inside or you will provide a possible conducting path for the R.F. to

your body.

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: transformer

Date: Sun,  8 Feb 1998 19:53:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 9 Feb 1998 00:53:54 +0500

Subject: Re: transformer

On Tue, 30 Dec 1997 12:10:23 -0700 Jared Neilsen P Neilsen

<torghn@juno.com> wrote;

> Thanks for the info on the plans for a testacy coil, now I'm left with

> finding a transformer.  Do any of you know of a good place to get them?

> I don't even know where to begin.

A good transformer to start with is a neon sign transformer, they yield

typicaly from 7,000 volts up to about 15,000 volts. To find them, look in

your local phone book for neon sign builders or repair shops and go to a

few and tell them you want some of their "burned out" transformers to build

a Tesla coil with. I said burned out, because typicaly these shops will

remove transformers from service when thay are still working fine but are

old enough where they expect them to fail soon. If the transformes are free

GREAT! But if they are going to charge you for them, take a plug with about

6 feet of wire on it and put two insulated aligator clips on one end so you

can clip onto the neon sign transformers input terminals and plug it in to

test it. When testing use a foot or so of good neon sign wire and draw an

arc from each terminal to the case, be sure to unplug the transformer

before changing the connections. Another somewhat less desirable

transformer is the OBIT or Oil Burner Ignition Transformer, they yield

10,000 volts at only 23 ma. but their better than nothing! look for these

on oil burner units thrown out on trash day ( it's a 4 to 6 inch metal

cube mounted on the side of the motor, it will have an identification

plate on it) or ask at a local heating system installer or repair shop.

Some heating oil suppliers will have OBITs too. DO NOT TRY PLAYING WITH A

POLE PIG (power line distribution transformer) untill you have some solid

experience building several coils with OBITs and or neon sign transformers!

Pole pigs can kill you on the slightest mistake! The same is true of neon

sign bombarder transformers. Both of these transformers come in 5

killowatts and up.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: holes in secondary form

Date: Sun,  8 Feb 1998 21:26:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sun, 8 Feb 1998 16:26:23 EST

Subject: holes in secondary form

I have just finished reading a posting found on the Tesla Coil Archives (a

must for all newbies!!) from Richard Quick which says no holes should be

drilled in the secondary form and it should be sealed top & bottom. I was

going along with the old way I built a coil (admittedly some 30 years ago)

where I drilled a small hole 1 1/2" at the top & bottom to run the wire

through and didn't seal either end. I assume Richard is correct (I have read

so many references to his postings and know about the RQ Gap) but I just

wanted to double check. I'm using a 4 1/2 D. acrylic form for a 22" L. winding

of 24 G. wire w/ a 15K 30ma Neon Tranny. Thanks again for the advice. Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rf choke question

Cc:

Bcc:

X-Attachments:

At 07:19 PM 2/8/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sun, 8 Feb 1998 14:19:14 EST

>Subject: rf choke question

>

>Most of the postings I've seen on proper coil set up include rf chokes between

>the safety gap and the spark gap. Can someone explain just what it is they do

>(how they function)  From the descriptions I've read they seem like small

>secondary coils. Also what should I build to protect a 15K 30ma transformer.

>I've read that 150 to 300 turns of 24 gauge wire on a 2" D. pvc pipe would be

>adequate. Could anyone be a little more specific and are they wound and sealed

>the same as a secondary? Thanks again all for all the help and advice. Jim

>

>

Jim,

To understand the purpose of the RF chokes first you need to understand

just what a Tesla coil does. A Tesla coil is a radio frequency (RF)

oscillator. It generates an RF signal that is then coupled

to the secondary coil where it rises to very high voltages.

The Tesla coil primary circuit composed of the primary coil and

main capacitor form a resonant circuit. This resonant circuit

is fed pulses of energy from the action of the spark gap.

These pulses of energy keep the flywheel action of the primary

resonant circuit going.

Now we want to keep that RF energy where it belongs. Thats in the

primary tank circuit and then coupled to the secondary. We DONT

want any of that RF energy to find its way back into our neon

sign transformer. And if we let it thats exactly what will happen.

RF energy inside the secondary windings of a neon sign transformer

will eat away and degrade the insulation until a failure occurs.

You can destroy a neon in seconds this way.

So we place RF chokes in each HV lead to block the RF energy.

An inductor shows high impedance to high frequencies and

a low impedance at low frequencies. So it shows little or

no impedance to the 60hz energy we are sending to the spark

gap and shows a high impedance to any RF energy try to sneak

into our neons.

Simple RF chokes will not completely block the RF. To do that

would require more extensive filter circuits. But more extensive

filtering is not needed. All we need to do is knock down the level

of the RF some and provide a safety gap across the transformer to

keep the voltage at a safe level.

Hope this clears it up a little for you.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: holes in secondary form

Cc:

Bcc:

X-Attachments:

At 09:26 PM 2/8/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sun, 8 Feb 1998 16:26:23 EST

>Subject: holes in secondary form

>

>I have just finished reading a posting found on the Tesla Coil Archives (a

>must for all newbies!!) from Richard Quick which says no holes should be

>drilled in the secondary form and it should be sealed top & bottom. I was

>going along with the old way I built a coil (admittedly some 30 years ago)

>where I drilled a small hole 1 1/2" at the top & bottom to run the wire

>through and didn't seal either end. I assume Richard is correct (I have read

>so many references to his postings and know about the RQ Gap) but I just

>wanted to double check. I'm using a 4 1/2 D. acrylic form for a 22" L. winding

>of 24 G. wire w/ a 15K 30ma Neon Tranny. Thanks again for the advice. Jim

>

>

Richard gave good advice. No holes should ever be drilled

in your secondary form. To see why this is important

consider the fact that you can get streamers more than 2 times

the length of your secondary winding. My own coil has had streamers

up to 54 inches in length from a winding just 21 inches long.

If you provide a path for a discharge down the center of the form

along this much shorter path thats exactly what will happen.

Drilling a hole in the form weakens the electrical strength of the form.

And pulling any part of the secondary wiring inside the form is

a BIG no-no for the same reason.

So if you have already drilled holes and pulled the secondary

inside the form you need to fix it. Pull the wire back outside

and seal the holes with epoxy. Dont worry if you have to lose

a couple of turns. Wont hurt at all.

You do need to dry and seal PVC pipe before you wind on the secondary.

Bake it at low temp in an oven or use a heat gun. Get it nice and toasty

but dont overheat. Keep it warm for 30-45 minutes and then put on a

coat of polyurethan varnish while still warm. You want to put 2 coats

on the outside and one coat inside the form. As the form cools it will

pull the vanish into the pores of the PVC and seal it very well

against moisture.

You do not need to seal the PVC form with a plexiglass disk at the bottom.

I leave the bottom open on all my coils. However you DO need the plexiglass

or Lexan disk in the top of the form. This is to provide electrical

isolation to prevent discharges down the inside of the secondary.

Use at least 1/8 inch of plexiglass and 1/4 would be better.

Attach it with good epoxy DOWN inside the form just above the secondary

winding. Do not just glue it across the end.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Top load design

Cc:

Bcc:

X-Attachments:

At 01:37 AM 2/8/98 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sun, 8 Feb 1998 01:37:21 +0000

>Subject: Top load design

>

>I fired-up up my new coil and actually achieved spontaneous streamer

>break-outs (12" or so, nothing spectacular).  Unfortunately several

>of which came off of the top winding of the sec'.

>

>So, by advice of various list members, it seems that my next

>step is to build a better top load, a toroid.  Any suggestions

>regarding the diameter of a 4" toroid, to be placed atop a 6.25x19"

>sec'?  How much does the diameter affect the pf value of the toroid?

>

>

>TIA,  Chris

>

I used a 16 X 4 inch toroid on my 4.5 X 22 inch secondary

with good success. I started to get multiple streamers rather

than one long streamer so changed to a 19 X 6 toroid. The larger

minor diameter (6 inches) allowed for a higher voltage buildup

before breakout.

The major diameter of a toroid has the most effect on

the capacitance.

As long as you have sufficient power to breakout I would

recommend at least 16 inches diameter for your toroid.

good luck

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: very small coil

Date: Mon,  9 Feb 1998 04:40:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Sun, 08 Feb 1998 22:40:56 -0600

Subject: very small coil

Organization: MailExcite  (http://www.mailexcite.com)

Fellow coilers,

I've heard a few reports of people building very small coils successfully.

By small I mean a coil with a diameter of 1 inch or less. Has anyone had any

experience with these?

I have a small inverter that will deliver 900 volts from a 9 volt battery.

If I run it through a tripler, I get 2700 volts along with a small amount

of capacitance.

Could I use this setup as my power supply and capacitor?

Also, would anyone know the specs of a coil with a 1cm diameter? Any thoughts are

appreciated!

-Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: ADMIN - Delayed Mail

Date: Mon,  9 Feb 1998 06:16:00 GMT

Organization: The Energy Machine Information System 504-733-3462

Sorry, folks, but some of the messages addressed to the server got

sidetracked, and I just located them this evening.  As a result, you may

see a few messages from last month arrive in your mailbox.  I dunno'

what happened.

Ralph

---

You can't do that. It's been digitally cursed.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: very small coil

Date: Mon,  9 Feb 1998 17:23:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: NebStrebor@aol.com

Date: Mon, 9 Feb 1998 12:23:18 EST

Subject: Re: very small coil

Jay,

Ive never built a coil before but I think it would be good for me to start

with a small one. where in the world did you get such a small transformer? I

think Id like to start with a small coil if it would work. input anyone?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Raising the top terminal

Date: Mon,  9 Feb 1998 20:18:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 09 Feb 1998 14:18:21 -0600

Subject: Raising the top terminal

Organization: MailExcite  (http://www.mailexcite.com)

I believe someone posted an article about raising the top terminal to increase

streamer length. How does this work ? I will probably give it a try, but I would

like to know a little bit more about it before I do.

Paul M

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary coil construction

Date: Tue, 10 Feb 1998 01:56:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Mon, 9 Feb 1998 20:56:19 EST

Subject: primary coil construction

I have just finished reading a bunch of files downloaded from the FTP site on

primary construction. I have a lot of #10 solid copper wire I was planning to

use in the "old way" by winding a solenoid shaped coil of 14 turns. One

posting in particular by Richard Quick said to never! use solid wire. He went

on to explain why 1/4" copper tubing in a 30 degree saucer shape functioned so

much better and all the other postings I read agreed on the 1/4" tube for a

coil of my size (4 1/2"D. x 22"L Secondary of 24 gauge, 15K 30ma Neon, static

multiple gap w/ muffin fan & salt water caps of .0053uf). My question is

should I scrap my current plans for my primary and move on or will I be able

to obtain some "somewhat" satisfactory results with what I have made so far.

(the plexi form is made and I only have to wind the wire)? Any advice will be

of much help. Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: very small coil

Date: Tue, 10 Feb 1998 03:40:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 09 Feb 1998 21:40:22 -0600

Subject: Re: very small coil

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>From: NebStrebor@aol.com

>   Jay,

>

>Ive never built a coil before but I think it would be good for me to start

>with a small one. where in the world did you get such a small transformer? I

>think Id like to start with a small coil if it would work. input anyone?

Me too.  It'd be fun to have a very small one,

desktop sized, perhaps with a secondary wound on a

papertowel / toiletpaper tube or something similar.

You might be able to power it with a (possibly

rectified) car ignition coil and a 555 (yawn), etc.

This'd make it portable.  I'd still insist on a

capacitive discharge complete with an RQ gap, etc.

You'd get major bonus points for building a small rotary!!

Better yet :) - use a dual rotary - one to

pulse current through the ignition coil, and another

(on the same shaft) for full-synchronous secondary

operation at any frequency!!

I have wondered if it is possible to build something

like this into (!) a baseball hat, and wear it to the

local hamvention!!  Hey - don't laugh!  Nobody has done

this yet as far as I know!  Use you balding head/body

as the groundplane, make a small toroid out of the

aluminum flex-hose you can get at the car part places

for air intakes, power supply in the backpack, etc...

Six-inch arcs off the top of your noggin would get

everyone's attention...  Might even get on Letterman

with this rig...

I bet those hams don't like spark-gap transmitters too much...

Even better - this'd be your own personal security system!

-Bill the arcstarter - Belfast, Ireland.

Home of the three kilowatt Bratt.

Served nestled within a magazine of our

world-famous hand-rolled semtex brisance Croissants.

http://www.geocities.com/capecanaveral/hangar/6160

(The Dayton, OH Hamvention is mid-May this year...)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Neon Transformers for Sale

Date: Tue, 10 Feb 1998 04:09:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tedd Payne <tpayne@netnitco.net>

Date: Mon, 09 Feb 1998 22:09:47 -0600

Subject: Neon Transformers for Sale

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your zip code (to

determine shipping cost).

Thanks,

Tedd

"CONVENTIONAL" neon transformers in clean (not old, not rusty), good

working condition.  They are not labeled, the ratings are from my

testing/measuring.

$9  used  6kv60ma  in = porc ins   output=porc insul

$15 used  9kv60ma  in = porc ins   output=porc insul

$11 used  12kv22ma  in=porc ins   out=porc insul  no lid (pair avail)

$17 used  12kv30ma  in = porc ins   output=porc insul (pair avail)

$15 used  12kv45ma  in=porc ins  out=porc ins, no lid, 1 broken ins

$15 used  15kv30ma  in=3 wires  out=2 wires

$15 used  15kv30ma  in = 2 wires  out=2 wires, w/switch

NEW Neon transformer HV wire, flexible gray, 14 gauge, multistrand

copper, rated 15KV, $1.00 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $1.00

"ELECTRONIC" neon transformer in good working condition:

$19  used, 9kv30ma, about 20kHz(?),  SMALL SIZE 4x5.5x2 inches, LIGHT

WEIGHT about 2 pounds,  AC cord, on-off switch, 10" HV leads, good for

Jacob's ladder, neon, experimenting, but probably NOT Tesla coil.

-

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: coil stuff

Date: Tue, 10 Feb 1998 04:13:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 09 Feb 1998 20:13:58 -0800

Subject: coil stuff

Hi everyone, I'm about to begin the construction phase of my first coil

and I think that I have a descent plan although I am lacking a few

details.

- the secondary, I will be using std. 4' pvc & #24 wire. My plan is to

use 5 minute epoxy to attach the wire before I wind the wire on the

lathe I will use epoxy to hold the other end in place also. my pvc is 24"

long and I plan to wind 21-22 inches. I will place a plexi plate on the

inside of the pvc (just above the windings) to provent corona leakage.

-  attaching the pvc to a board, can I Use a router to make a cylindrical

groove and tack it in with silicone goop? I have also thought about using

a apx. 3.75" or (whatever it works out to) wooden disk with goop- how

does that sound?

- about the toroid attachment - I'm thinking about using 1-2"dia. pvc

pipe caps glued together on a 1-2" piece of pipe as I have read at

various sites. This little capsule would have a nylon screw poking out of

it so that I can attach the torroid with a 1/4" hole on the aluminum

plate/disc . What thoughts are out there on this attachment? other

options are what? What are the rules regarding the hieght of the torroid

in relationship with the scondary? does the little attachment capsule

pose a problem?

- the toroid will be a 4" plastic "ADS" drain pipe and drywall

plaster/primer, with aluminum tape. When I bring the pipe ends together,

what's a good method for attaching the two ends?

- on the primary- I plan to use 1/4" copper tubing 15 degree

rise/30 degree slope. 1/4" between the winds - I'm assuming that this is

1/2" center to center on the od of the copper pipe?. 1" clearance between

the first wind of the primary and the secondary tube, does this seem

correct?

- for the electrical attachment made to the primary, I have seen some

pictures using hippo clamps or jumper cable clamps. seems like minimal

surface area - is this the way this connection is made? what are the

options?

Thanks, Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Is a 6kv 30ma NST any good

Date: Tue, 10 Feb 1998 10:42:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: D Mills <ap7012@qmw.ac.uk>

Date: Tue, 10 Feb 1998 10:42:07 +0000

Subject: Is a 6kv 30ma NST any good

Organization: University of London (QMW)

Hi,

I want to build a small tesla coil that will give about 6" arc's.  I

have a 6kv 30ma NST and a salt water cap of 0.052uf.  Are these going

to be useful.

Also any Ideas for a sutible design for the coil would be gratefuly

accepted.

cheers

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Is a 6kv 30ma NST any good

Date: Tue, 10 Feb 1998 11:48:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Tue, 10 Feb 1998 06:48:43 EST

Subject: Re: Is a 6kv 30ma NST any good

Hi Dave,

Your nst will work for a coil with approx. 5 - 6" output.  Your cap should

be more like 0.013uF rather than 0.052uF, however to match the impedance of

the nst.   A three inch PVC pipe should work for this size coil.  I have one

this size that has 552 turns of 24g wire( this makes it 12" tall).  This

design worked fine for me, although you can probably go with more turns.  I

think you would want to limit your design to about 700 turns(15" tall with 24g

wire).

Hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary coil construction

Date: Tue, 10 Feb 1998 14:02:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 10 Feb 1998 08:02:12 -0600

Subject: Re: primary coil construction

Organization: MailExcite  (http://www.mailexcite.com)

Jim wrote:

>From: Dsurfr@aol.com

>primary construction. I have a lot of #10 solid copper wire I was planning to

>use in the "old way" by winding a solenoid shaped coil of 14 turns. One

>posting in particular by Richard Quick said to never! use solid wire. He went

>on to explain why 1/4" copper tubing in a 30 degree saucer shape functioned so

...

> My question is

>should I scrap my current plans for my primary and move on or will I be able

>to obtain some "somewhat" satisfactory results with what I have made so far.

I would think you will still get a decent output if you

use the #10 wire in lieu of tubing.  Have you started

building the primary yet or are you still in the planning

stages?  Have any other coilers had experiences

contrary to this?

My read on this is that if you have to go BUY either

solid wire or tubing (of an equal outer diameter), then

you'd be better off using the tubing since it'd be

cheaper.  The skin effect constrains the current flow

to the outermost layer of the conductor, so having a

hollow inside doesn't affect anything since no HF

current flows there anyway.

Also - using a solenoid shape on the primary will likely

lead to overcoupling between the coils.  Plus - you will have

problems preventing flashovers between the

two.  We used a flat spiral primary for our coil.

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary coil construction

Date: Tue, 10 Feb 1998 14:20:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 10 Feb 1998 06:20:14 -0800

Subject: Re: primary coil construction

Organization: Stoneridge Engineering

Jim,

Not to worry - Tesla Coils can be made to function with a wide variety

of materials. Although #10 AWG is not quite as good as 1/4" coppper

tubing, it will function quite adequately at low power levels. If you're

using a helical primary, make sure you have adequate clearance between

your primary and secondary to prevent flashover between the two. Go with

at least a 10-12" diameter primary and don't make it very high (say 3-4"

max). You may also need to add a grounded strike-rail above the top turn

to prevent streamers from the top from hitting the primary. Also, is the

wire bare or insulated (like house wire)? You'll need to have adequate

spacing between primary turns (at least 1/4") if using bare wire. Flat

or saucer primaries will allow you run at higher power (with longer

spark lengths) before getting too many streamer hits to the primary

strike rail - if you're planning on increasing the power level soon you

may want to consider using this type construction instead of the helical

primary.

Safe coilin' to you, Jim!

-- Bert --

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Mon, 9 Feb 1998 20:56:19 EST

> Subject: primary coil construction

>

> I have just finished reading a bunch of files downloaded from the FTP site on

> primary construction. I have a lot of #10 solid copper wire I was planning to

> use in the "old way" by winding a solenoid shaped coil of 14 turns. One

> posting in particular by Richard Quick said to never! use solid wire. He went

> on to explain why 1/4" copper tubing in a 30 degree saucer shape functioned s

o

> much better and all the other postings I read agreed on the 1/4" tube for a

> coil of my size (4 1/2"D. x 22"L Secondary of 24 gauge, 15K 30ma Neon, static

> multiple gap w/ muffin fan & salt water caps of .0053uf). My question is

> should I scrap my current plans for my primary and move on or will I be able

> to obtain some "somewhat" satisfactory results with what I have made so far.

> (the plexi form is made and I only have to wind the wire)? Any advice will be

> of much help. Thanks, Jim

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Caps,secondary terminal,chokes

Date: Tue, 10 Feb 1998 16:26:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Tue, 10 Feb 1998 08:26:22 -0800 (PST)

Subject: Caps,secondary terminal,chokes

Greetings from Urth:

1. My partner and I are gathering mat'ls to build a flat plate cap, and as

I perused a list of mat'l I downloaded somewhere (math.txt) I saw that

mylar was listed with a dielectric value of 3, and a voltage protection

rating of 7,500 VOLTS PER MIL!!! This means 2 layers of 3 mil mylar will

give 45,000 volt protection, a 200% safety factor for our 15Kv neon. Have

I discovered the ultimate cap dielectric, or am I all wet? (Local plastic

supplier has 3 mil clear mylar for $2.49/ft, 4 feet wide. $5.00 will get

us all we need for .01 cap)

2. Can anyone settle an argument? My partner, the engineer, wants to terminate

our secondary by gluing a flat copper plate on top of the end cap, with the

tail of the secondary wire soldered to it. Then he says we can just set

whatever terminus we want on top of the coil, with a conducting rod, if

necessary, for height adjustment, to contact the plate. All the pics I've

seen show the top of the secondary with a loose wire, continuing the spiral,

up to the toroid, or ball, etc. Will his design work, or will there be more

"resonant efficiency" continuing the spiral with the secondary wire?

3. I'm ready to wind my RF chokes on 2" PVC. Is there any point in putting a

coat or two of polyurethane on the PVC before winding, as in secondary prep?

Can't wait to fire this thing up!   <danmurphy@1stnetusa.com>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary coil construction

Date: Tue, 10 Feb 1998 18:58:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Tue, 10 Feb 1998 12:58:32 -0600

Subject: Re: primary coil construction

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Mon, 9 Feb 1998 20:56:19 EST

> Subject: primary coil construction

>

> I have just finished reading a bunch of files downloaded from the FTP site on

> primary construction. I have a lot of #10 solid copper wire I was planning to

> use in the "old way" by winding a solenoid shaped coil of 14 turns. One

> posting in particular by Richard Quick said to never! use solid wire. He went

> on to explain why 1/4" copper tubing in a 30 degree saucer shape functioned s

o

> much better and all the other postings I read agreed on the 1/4" tube for a

> coil of my size (4 1/2"D. x 22"L Secondary of 24 gauge, 15K 30ma Neon, static

> multiple gap w/ muffin fan & salt water caps of .0053uf). My question is

> should I scrap my current plans for my primary and move on or will I be able

> to obtain some "somewhat" satisfactory results with what I have made so far.

> (the plexi form is made and I only have to wind the wire)? Any advice will be

> of much help. Thanks, Jim

>

If you already have the #10 wire use it. I recently made a coil using a

7.5kv@60ma w/#8awg solid wire and got steady 15" strikes to ground.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Builders Assoc in SoCal?

Date: Tue, 10 Feb 1998 19:37:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 10 Feb 1998 11:37:50 -0800 (PST)

Subject: Builders Assoc in SoCal?

I am curious: Is there a Tesla Builders group in the SoCal area?

Preferably not in the L.A. area: no Offense, but that drive sucks!

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Caps,secondary terminal,chokes

Cc:

Bcc:

X-Attachments:

At 04:26 PM 2/10/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Tue, 10 Feb 1998 08:26:22 -0800 (PST)

>Subject: Caps,secondary terminal,chokes

>

>Greetings from Urth:

>

>1. My partner and I are gathering mat'ls to build a flat plate cap, and as

>   I perused a list of mat'l I downloaded somewhere (math.txt) I saw that

>   mylar was listed with a dielectric value of 3, and a voltage protection

>   rating of 7,500 VOLTS PER MIL!!! This means 2 layers of 3 mil mylar will

>   give 45,000 volt protection, a 200% safety factor for our 15Kv neon. Have

>   I discovered the ultimate cap dielectric, or am I all wet? (Local plastic

>   supplier has 3 mil clear mylar for $2.49/ft, 4 feet wide. $5.00 will get

>   us all we need for .01 cap)

Sorry but you're "all wet".

7500 volts per mil is highly unlikely. Take ratings from various charts

with a grain of salt. Often times these ratings are DC values.

In Tesla coil service a dielectric will only withstand around 40 percent

of its DC rating.

Mylar does indeed have fairly good ratings for dielectric constant

and strength but there is one  even more important parameter that you

neglected. RF dissipation factor.

LDPE (low density polyethylene) has an RF dissipation

factor of .08 at 1 mhz. PET (mylar) has a dissipation

factor of 20 at 1 mhz. In other words mylar has 250 times

the RF dissipation as poly. This much RF dissipation will result

in a poor performing lossy capacitor.

Mylar makes great DC caps but its really terrible for RF.

>2. Can anyone settle an argument? My partner, the engineer, wants to terminate

>   our secondary by gluing a flat copper plate on top of the end cap, with the

>   tail of the secondary wire soldered to it. Then he says we can just set

>   whatever terminus we want on top of the coil, with a conducting rod, if

>   necessary, for height adjustment, to contact the plate. All the pics I've

>   seen show the top of the secondary with a loose wire, continuing the spiral

>   up to the toroid, or ball, etc. Will his design work, or will there be

>   more "resonant efficiency" continuing the spiral with the secondary wire?

>

The only problem I see with this is corona. The edge of this copper plate will

be sharp and promote corona. In fact you will probably have all your streamers

come from this area. I would say this alone makes it a bad idea.

>3. I'm ready to wind my RF chokes on 2" PVC. Is there any point in putting a

>   coat or two of polyurethane on the PVC before winding, as in secondary prep?

Its not necessary but wont hurt. No need to dry and seal

the PVC for RF chokes. A little loss is just fine. In fact a

nice low loss HI Q choke is NOT what you want at all. Low Q

and high loss is just what you want.

>   Can't wait to fire this thing up!   <danmurphy@1stnetusa.com>

Good luck.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Is a 6kv 30ma NST any good

Cc:

Bcc:

X-Attachments:

At 10:42 AM 2/10/98 GMT, you wrote:

>From: D Mills <ap7012@qmw.ac.uk>

>Date: Tue, 10 Feb 1998 10:42:07 +0000

>Subject: Is a 6kv 30ma NST any good

>Organization: University of London (QMW)

>

>Hi,

>

>I want to build a small tesla coil that will give about 6" arc's.  I

>have a 6kv 30ma NST and a salt water cap of 0.052uf.  Are these going

>to be useful.

Yes that would be a nice size for a small coil. Anything less

than 6 kv becomes difficult to get good gap operation.

I managed 14 inches of output from my 3.5 X 16 inch

secondary with a 7500V/30ma neon.

Your .052 mfd cap is way too large however.

The largest size cap your neon wil be able to handle

is around .0133 mfd assuming 60 hz mains.

With a 50 hz mains frequency you can use up to .0159 mfd.

Dont think you need these EXACT values. Anything in this area

will work fine. The important thing is that your secondary + toroid

resonates at the same frequency as your primary coil/primary cap.

>

>Also any Ideas for a suitable design for the coil would be gratefuly

>accepted.

>

For a 3 incher I would go with a 16 inch winding

of AWG 26. This is about 925 turns and an unloaded resonant

frequency of 525 KHZ.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: very small coil

Date: Tue, 10 Feb 1998 21:37:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Tue, 10 Feb 1998 15:37:08 -0600

Subject: Re: very small coil

Organization: MailExcite  (http://www.mailexcite.com)

>From: arcstarter@mailexcite.com

>Date: Mon, 09 Feb 1998 21:40:22 -0600

>Subject: Re: very small coil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>It was written:

>>From: NebStrebor@aol.com

>>   Jay,

>>

>>Ive never built a coil before but I think it would be good for me to start

>>with a small one. where in the world did you get such a small transformer? I

>>think Id like to start with a small coil if it would work. input anyone?

>

>Me too.  It'd be fun to have a very small one,

>desktop sized, perhaps with a secondary wound on a

>papertowel / toiletpaper tube or something similar.

>

> You might be able to power it with a (possibly

>rectified) car ignition coil and a 555 (yawn), etc.

>This'd make it portable.  I'd still insist on a

>capacitive discharge complete with an RQ gap, etc.

>You'd get major bonus points for building a small rotary!!

>Better yet :) - use a dual rotary - one to

>pulse current through the ignition coil, and another

>(on the same shaft) for full-synchronous secondary

>operation at any frequency!!

>

>I have wondered if it is possible to build something

>like this into (!) a baseball hat, and wear it to the

>local hamvention!!  Hey - don't laugh!  Nobody has done

>this yet as far as I know!  Use you balding head/body

>as the groundplane, make a small toroid out of the

>aluminum flex-hose you can get at the car part places

>for air intakes, power supply in the backpack, etc...

>

>Six-inch arcs off the top of your noggin would get

>everyone's attention...  Might even get on Letterman

>with this rig...

>

>I bet those hams don't like spark-gap transmitters too much...

>Even better - this'd be your own personal security system!

>

>-Bill the arcstarter - Belfast, Ireland.

That's a great idea Bill! It's the type I've been shooting for,

but your application is much better than mine. : ) Actually, I have

built a small coil that ran off of a 9 volt battery. It used a 100:1

inverter which I fed into a high voltage tripler, which then fed into

a small capacitor. The time intervals between sparks were limited

to about 1 per sec., but it did make some pretty nice sparks (they were'nt much

more than 2 inches because I haven't been able to tune it).

I'm still wondering if anyone would have any specs for a coil with about

a 2cm. diameter secondary. I haven't been able to find any! What's the smallest

coil ever built, anyway?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Caps,secondary terminal,chokes

Date: Wed, 11 Feb 1998 03:16:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 10 Feb 1998 19:16:07 -0800

Subject: Re: Caps,secondary terminal,chokes

Organization: Stoneridge Engineering

Dan,

My answers are interspersed below...

-- Bert --

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Tue, 10 Feb 1998 08:26:22 -0800 (PST)

> Subject: Caps,secondary terminal,chokes

>

> Greetings from Urth:

>

> 1. My partner and I are gathering mat'ls to build a flat plate cap, and as

>    I perused a list of mat'l I downloaded somewhere (math.txt) I saw that

>    mylar was listed with a dielectric value of 3, and a voltage protection

>    rating of 7,500 VOLTS PER MIL!!! This means 2 layers of 3 mil mylar will

>    give 45,000 volt protection, a 200% safety factor for our 15Kv neon. Have

>    I discovered the ultimate cap dielectric, or am I all wet? (Local plastic

>    supplier has 3 mil clear mylar for $2.49/ft, 4 feet wide. $5.00 will get

>    us all we need for .01 cap)

No such Luck! That's for pure mylar under DC conditions. Tesla coil

usage is considerably harder on dielectric materials due to dielectric

heating effects - the lossier the material, the more the heating effect.

Mylar is fairly lossy at RF frequencies, so the dielectric material

heats up fairly quickly. Unfortunately, mylar also gets lossier as its

temperature rises, so you can easily get a thermal runaway condition

which rapidly destroys the dielectric. A much better choice is LDPE or

polyproylene.

>

> 2. Can anyone settle an argument? My partner, the engineer, wants to terminate

>    our secondary by gluing a flat copper plate on top of the end cap, with the

>    tail of the secondary wire soldered to it. Then he says we can just set

>    whatever terminus we want on top of the coil, with a conducting rod, if

>    necessary, for height adjustment, to contact the plate. All the pics I've

>    seen show the top of the secondary with a loose wire, continuing the spiral

>    up to the toroid, or ball, etc. Will his design work, or will there be

>    more "resonant efficiency" continuing the spiral with the secondary wire?

>

It'll work, but it's really not necessary. It will have minimal impact

on your coil's performance. You won't have to worry about eddy current

effects if the plate is at the top of the secondary.

> 3. I'm ready to wind my RF chokes on 2" PVC. Is there any point in putting a

>    coat or two of polyurethane on the PVC before winding, as in secondary prep?

>

It wouldn't hurt, but really's not necessary.

>    Can't wait to fire this thing up!   <danmurphy@1stnetusa.com>

>

Safe coilin' to you, Dan!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coil stuff

Cc:

Bcc:

X-Attachments:

At 04:13 AM 2/10/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Mon, 09 Feb 1998 20:13:58 -0800

>Subject: coil stuff

>

>Hi everyone, I'm about to begin the construction phase of my first coil

>and I think that I have a descent plan although I am lacking a few

>details.

>- the secondary, I will be using std. 4' pvc & #24 wire. My plan is to

>use 5 minute epoxy to attach the wire before I wind the wire on the

>lathe I will use epoxy to hold the other end in place also. my pvc is 24"

>long and I plan to wind 21-22 inches. I will place a plexi plate on the

>inside of the pvc (just above the windings) to provent corona leakage.

Sounds good to me.

>-  attaching the pvc to a board, can I Use a router to make a cylindrical

>groove and tack it in with silicone goop? I have also thought about using

>a apx. 3.75" or (whatever it works out to) wooden disk with goop- how

>does that sound?

I made a wooden foot for my secondary. I cut a disk of 3/4 inch

wood so it was a snug fit inside the bottom of the form.

I then mounted this disk in the center of a larger disk. The smaller

disk fits inside the form and the larger becomes the foot.

Ive seen coils where the builder used a length of pipe that would

telescope inside the secondary form. By making it a snug fit the

secondary can easily be moved up and down to adjust the coupling.

Whatever method you use make sure you will be able to vary

the distance between the primary and secondary to adjust the coupling.

>

>- about the toroid attachment - I'm thinking about using 1-2"dia. pvc

>pipe caps glued together on a 1-2" piece of pipe as I have read at

>various sites. This little capsule would have a nylon screw poking out of

>it so that I can attach the torroid with a 1/4" hole on the aluminum

>plate/disc . What thoughts are out there on this attachment? other

>options are what? What are the rules regarding the hieght of the torroid

>in relationship with the scondary? does the little attachment capsule

>pose a problem?

This sounds fine.

I place a 1/4 20 carriage bolt through a disk of wood that fits

into the top of the secondary coil. The bolt head is countersunk

and covered with RTV and a disk of plexiglass. This is in addition

to the plexiglass disk you have already placed into the secondary form.

I then run the secondary wire up the OUTSIDE of the form then over

to the central bolt. I attach the secondary wired permanently

to the central bolt with a ring terminal. I bed this entire

connection down with epoxy. No loose wires.

Discharge terminals are threaded onto the central bolt making height

changes and toroid changes a snap.

>

>- the toroid will be a 4" plastic "ADS" drain pipe and drywall

>plaster/primer, with aluminum tape. When I bring the pipe ends together,

>what's a good method for attaching the two ends?

Ive always used the flexible aluminum dryer duct. I join the ends

with vinyl tape and then cover the joint with aluminum tape.

The rest of the toroid does not need any covering.

With your plastic pipe you could tape the ends or use a cement

suitable for ABS. RTV could probably also be used.

>

>- on the primary- I plan to use 1/4" copper tubing 15 degree

>rise/30degree slope. 1/4" between the winds - I'm assuming that this is

>1/2" center to center on the od of the copper pipe?. 1" clearance between

>the first wind of the primary and the secondary tube, does this seem

>correct?

1 inch is as close as you would want. It will probably be fine

for 12kv or less. If you are using 15kv neons a bit more space

might be necessary. Dont make it too spacious or coupling will suffer.

1 1/4 to maybe a max of 1 1/2 would be fine.

>

>- for the electrical attachment made to the primary, I have seen some

>pictures using hippo clamps or jumper cable clamps. seems like minimal

>surface area - is this the way this connection is made? what are the

>options?

I use a large alligator clip with extra copper plates soldered

on the to increase contact area. Once your coil is running check

this area for signs of heating. (Power off please!)

If its getting warm you need to beef it up some.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Is a 6kv 30ma NST any good

Date: Wed, 11 Feb 1998 06:13:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Wed, 11 Feb 1998 00:13:43 -0600

Subject: Re: Is a 6kv 30ma NST any good

Tesla-2 wrote:

>

> From: D Mills <ap7012@qmw.ac.uk>

> Date: Tue, 10 Feb 1998 10:42:07 +0000

> Subject: Is a 6kv 30ma NST any good

> Organization: University of London (QMW)

>

> Hi,

>

> I want to build a small tesla coil that will give about 6" arc's.  I

> have a 6kv 30ma NST and a salt water cap of 0.052uf.  Are these going

> to be useful.

>

> Also any Ideas for a sutible design for the coil would be gratefuly

> accepted.

>

> cheers

>

> Dave

>

The capacitor has way to much capacitance, but the transformer yes.

I built a coil utilizing a 7.5kv@60ma and got 15" streamers the first

time I fired it up.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil builders in S.F. Valley?

Date: Wed, 11 Feb 1998 09:20:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 11 Feb 1998 01:20:08 -0800 (PST)

Subject: Re: Coil builders in S.F. Valley?

I would be interested in contacting any coil builders here in the San Fernando

valley in SoCal. Reply directly to: danmurphy@1stnetusa.com

Coilin away!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Flammability test on cap oil

Date: Wed, 11 Feb 1998 19:59:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 11 Feb 1998 13:59:52 -0600

Subject: Flammability test on cap oil

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I performed a non-scientific flammability test on some

Diala-EX transformer oil.  Here are the results.

I poured about a tablespoon of oil onto the center

of a paper towel.  This made an oil stain on the towel

about 4 inches across.

Next I put the towel on the floor, over a piece of steel

wire which I had bent into a pyramid-shape.  The

purpose of this wire was to keep the center of

the towel elevated off the floor so it could get air as it

burned.

Finally I lit one edge of the towel and watched it go!

Strangely enough - the flame did NOT light the oil at

all!  The non-oiled areas completely burned away,

leaving a small patch of oily towel behind.  I was

never able to get the oil to light into a self-sustaining

fire, even by using a propane plumber's torch!

Seems like this stuff is more of a retardant than an

accelerant.  Even WD-40 or kerosene burns much

more readily.

So apparently, if your diala-ex immersed cap fails, it won't

go up in a huge ball of flame unless you really work

at it.  This is good to know since our caps contain

some 5+ gallons of oil all together.

I don't know what mineral oil would do.  Has anyone

tried this?

(Now if we bubbled some O2 through it first...)

-Bill the arcstarter

Starting arcs in Cinci, OH (Not Ireland)

Making "Bomb-proof" capacitors in my garage.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flammability test on cap oil

Cc:

Bcc:

X-Attachments:

At 07:59 PM 2/11/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Wed, 11 Feb 1998 13:59:52 -0600

>Subject: Flammability test on cap oil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I performed a non-scientific flammability test on some

>Diala-EX transformer oil.  Here are the results.

>

> I poured about a tablespoon of oil onto the center

>of a paper towel.  This made an oil stain on the towel

>about 4 inches across.

>

>Next I put the towel on the floor, over a piece of steel

>wire which I had bent into a pyramid-shape.  The

>purpose of this wire was to keep the center of

>the towel elevated off the floor so it could get air as it

>burned.

>

> Finally I lit one edge of the towel and watched it go!

>

>Strangely enough - the flame did NOT light the oil at

>all!  The non-oiled areas completely burned away,

>leaving a small patch of oily towel behind.  I was

>never able to get the oil to light into a self-sustaining

>fire, even by using a propane plumber's torch!

>

>Seems like this stuff is more of a retardant than an

>accelerant.  Even WD-40 or kerosene burns much

>more readily.

>

> So apparently, if your diala-ex immersed cap fails, it won't

>go up in a huge ball of flame unless you really work

>at it.  This is good to know since our caps contain

>some 5+ gallons of oil all together.

>

>I don't know what mineral oil would do.  Has anyone

>tried this?

>

> (Now if we bubbled some O2 through it first...)

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH (Not Ireland)

>Making "Bomb-proof" capacitors in my garage.

Bill,

Very interesting. I had no idea transformer oil

was that flame retardant.

I never tried to actually burn any mineral oil

but I HAVE blown up plenty of mineral oil caps.

About all that happens is a gurgling sound and some bright

flashes, then usually followed by silence. :(

There are plumes of carbon after an arc passes through it

and the smell of burnt poly and mineral oil is quite nasty.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary Coil Construction

Date: Thu, 12 Feb 1998 02:01:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Wed, 11 Feb 1998 21:01:45 EST

Subject: Primary Coil Construction

Thanks for all the advice on primaries so far. As a result I'm scrapping my

original plan of a helical formed w/ #10 copper wire and going for a saucer

shaped coil w/ 1/4" copper tubing. Now, of course, I have more questions.

Should the coil forms to hold the tubing be located on the inside or outside

of the coil? I'm going to cut notched plexi pieces w/ half circles at 1/4"

spacing. Does the tubing try to expand outward when winding (hence forms on

the outside) or will it stay in place when bent to shape? Is there a specific

type of copper tubing (refrigeration water line?) that is best to use and does

anyone have an idea of what I should be paying for it. Should it be treated in

any way? (sanded, sealed, glued in place etc.) Lastly does any one know of web

sites I can get any of this information from without bothering you all for

replies and that would have GIF photos I could view of their construction.

Thanks once again, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Coil Construction

Date: Thu, 12 Feb 1998 04:10:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Wed, 11 Feb 1998 22:10:56 -0600

Subject: Re: Primary Coil Construction

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Wed, 11 Feb 1998 21:01:45 EST

> Subject: Primary Coil Construction

>

> Thanks for all the advice on primaries so far. As a result I'm scrapping my

> original plan of a helical formed w/ #10 copper wire and  going for a saucer

> shaped coil w/ 1/4" copper tubing. Now, of course, I have more questions.

> Should the coil forms to hold the tubing be located on the inside or outside

> of the coil? I'm going to cut notched plexi pieces w/ half circles at 1/4"

> spacing. Does the tubing try to expand outward when winding (hence forms on

> the outside) or will it stay in place when bent to shape? Is there a specific

> type of copper tubing (refrigeration water line?) that is best to use and does

> anyone have an idea of what I should be paying for it. Should it be treated in

> any way? (sanded, sealed, glued in place etc.) Lastly does any one know of web

> sites I can get any of this information from without bothering you all for

> replies and that would have GIF photos I could view of their construction.

> Thanks once again, Jim

>

Once the coil is shaped it should stay fairly well in a workable state.

The kind od tubing you should use comes in a wound up coil of about 60',

the cost of the last roll I bought was like 24.99+tax. I got it at a

hardware store. Also if I were you I would get with some of the people

on the list and find out the easiest way to wind your coil. Experience

on my first rolled copper tubing primary taught me that it is not that

easy. Because of the softness of this tubing and the way it is wound up

makes it kind of hard to wind it into a perfect coil. Most of the tubing

comes in a roll that is wound on both ends until it gets to the middle,

instead of being wound on one end to the other. So it has to be

straightened out and then wound carefully.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: Is a 6kv 30ma NST any good

Date: Thu, 12 Feb 1998 12:36:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 12 Feb 1998 06:36:49 -0600

Subject: Re: Re: Is a 6kv 30ma NST any good

Organization: MailExcite  (http://www.mailexcite.com)

Dave,

It should work fine. I actually built one coil, wound on a paper towel tube.

I did this as a teenager. I got 6" discharges and never scorched the tube.

Paul

---

Einigkeit und Freiheit

>From: Zuma <mwise@ns.sosis.com>

>Date: Wed, 11 Feb 1998 00:13:43 -0600

>Subject: Re: Is a 6kv 30ma NST any good

>

>Tesla-2 wrote:

>>

>> From: D Mills <ap7012@qmw.ac.uk>

>> Date: Tue, 10 Feb 1998 10:42:07 +0000

>> Subject: Is a 6kv 30ma NST any good

>> Organization: University of London (QMW)

>>

>> Hi,

>>

>> I want to build a small tesla coil that will give about 6" arc's.  I

>> have a 6kv 30ma NST and a salt water cap of 0.052uf.  Are these going

>> to be useful.

>>

>> Also any Ideas for a sutible design for the coil would be gratefuly

>> accepted.

>>

>> cheers

>>

>> Dave

>>

>The capacitor has way to much capacitance, but the transformer yes.

>I built a coil utilizing a 7.5kv@60ma and got 15"streamers the first

>time I fired it up.

>

>Chris

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: ADMIN - Encoded Files - A

Date: Thu, 12 Feb 1998 15:25:00 GMT

Organization: The Energy Machine Information System 504-733-3462

This message was posted in my other mailing list, the NEWMAN-L mailing

list.  However, the same problem exists on the TESLA-2 list, so please

consider this as simply a heads-up message.

Ralph

----------------------------------------

OK, folks, now Listen up!

>>> DO NOT SEND ENCODED FILES THROUGH THE NEWMAN-L MAILING LIST. <<<

IF YOU DO SO IN THE FUTURE, YOU WILL BE BANNED FROM THE LIST,

WITHOUT FURTHER WARNING.

========================

Last night, someone (and you know who you are) sent an encoded GIF

file through the list.  The result was a mess.  That ONE encoded file

produced the following:

* an attempt by the listserver to send over 3,000 messages, because

the file was split into segments for transmission to the list,

* an outbound mail packet in excess of 13 Mb !!

* a crashed mail server which choked on the large amount of messages.

* required me spending over 4 hours cleaning up and rebuilding the

database.

* the loss of about 10 inbound messages (everything from about 6PM)

addressed to both the NEWMAN-L and the TESLA-2 mailing lists.

* an aggravated sysop, since I did not get to bed until 0300 because

I had to rebuild the database.

I am sorry that I have to take this stance on encoded files, but they

are simply NOT compatible with the listserver, not to mention that they

can seriously clog up peoples mail boxes.  Remember that not everyone is

interested in your latest and greatest picture.

If you have a file which you think may be of interest to the list

members, post an announcement about it on the list, then place the file

on a web site for downloading, or email it directly to those who are

interested in it.

Free web sites and free email may be obtained from geocities.com.

Thanks!

Ralph hartwell

Listowner, Newman-l & Tesla-2

---

... Moderator Insanity Meter:  90%

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Is a 6kv 30ma NST any good

Date: Thu, 12 Feb 1998 17:01:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Thu, 12 Feb 1998 12:01:03 -0500

Subject: Re: Is a 6kv 30ma NST any good

>Hi Dave,

>    Your nst will work for a coil with approx. 5 - 6" output.  Your

>cap should

I managed to squueze about 8.5 - 9 " to a grounded rod using a 6kv 30ma

transformer and saltwater caps.  I do not believe that it was the most

efficient coil in the world either.

Good luck.

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Coil Construction

Date: Thu, 12 Feb 1998 21:02:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 12 Feb 1998 16:02:04 -0500

Subject: Re: Primary Coil Construction

>

> From: Zuma <mwise@ns.sosis.com>

> Date: Wed, 11 Feb 1998 22:10:56 -0600

> Subject: Re: Primary Coil Construction

>

> Tesla-2 wrote:

> >

> > From: Dsurfr@aol.com

> > Date: Wed, 11 Feb 1998 21:01:45 EST

> > Subject: Primary Coil Construction

> >

> > Thanks for all the advice on primaries so far. As a result I'm scrapping my

> > original plan of a helical formed w/ #10 copper wire and  going for a saucer

> > shaped coil w/ 1/4" copper tubing. Now, of course, I have more questions.

> > Should the coil forms to hold the tubing be located on the inside or outside

> > of the coil? I'm going to cut notched plexi pieces w/ half circles at 1/4"

> > spacing. Does the tubing try to expand outward when winding (hence forms on

> > the outside) or will it stay in place when bent to shape? Is there a specific

> > type of copper tubing (refrigeration water line?) that is best to use and does

> > anyone have an idea of what I should be paying for it. Should it be treated in

> > any way? (sanded, sealed, glued in place etc.) Lastly does any one know of web

> > sites I can get any of this information from without bothering you all for

> > replies and that would have GIF photos I could view of their construction.

> > Thanks once again, Jim

> >

> Once the coil is shaped it should stay fairly well in a workable state.

> The kind od tubing you should use comes in a wound up coil of about 60',

> the cost of the last roll I bought was like 24.99+tax. I got it at a

> hardware store. Also if I were you I would get with some of the people

> on the list and find out the easiest way to wind your coil. Experience

> on my first rolled copper tubing primary taught me that it is not that

> easy. Because of the softness of this tubing and the way it is wound up

> makes it kind of hard to wind it into a perfect coil. Most of the tubing

> comes in a roll that is wound on both ends until it gets to the middle,

> instead of being wound on one end to the other. So it has to be

> straightened out and then wound carefully.

>

> Chris

>

Chris,

The key is not to bend the tubing any more than is absolutely

necessary. Do NOT straighten the tubing out, but rather gently

change the radius of curvature to the new desired curvature.

I *highly* recommend buying a tubing bender accessory that looks

like a big spring. They sell these for a couple of bucks, and

in various sizes. You slip it over the tubing and then bend the

spring with the tubing inside. It really eliminates the kinks

you would otherwise get, and helps you generate very smooth curves.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Coil Construction

Date: Thu, 12 Feb 1998 23:22:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Thu, 12 Feb 1998 15:22:23 -0800 (PST)

Subject: Re: Primary Coil Construction

On 12 Feb, Tesla-2 wrote:

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Thu, 12 Feb 1998 16:02:04 -0500

> Subject: Re: Primary Coil Construction

>

>

> Chris,

> The key is not to bend the tubing any more than is absolutely

> necessary. Do NOT straighten the tubing out, but rather gently

> change the radius of curvature to the new desired curvature.

> I *highly* recommend buying a tubing bender accessory that looks

> like a big spring. They sell these for a couple of bucks, and

> in various sizes. You slip it over the tubing and then bend the

> spring with the tubing inside. It really eliminates the kinks

> you would otherwise get, and helps you generate very smooth curves.

>

> Hope this helps.

> Fr. Tom McGahee

>

>

A minor point: If you have managed to work-harden

the pipe, and want to regain some of the give, heat it with a torch.

Another idea: for bending pipe, if you don't have the nifty spring

jobbie, you can fill the pipe with sand. This will assist in preventing

the kinks as well. Of course, it might be a real pain to get the

sand back out in this application.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Coil Construction

Date: Thu, 12 Feb 1998 23:42:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Thu, 12 Feb 1998 23:42:53 +0000

Subject: Re: Primary Coil Construction

I bought an equal length of polyprop 1/4" o.d. tubing and wound it

with the copper.  It gave me a relatively consistant .25 spacing and

let me form the copper to the appropriate shape without much trouble.

Then I made 8 brackets that fit over the the coil from i.d to o.d. to

hold the whole thing secure.   But I have to admit, while the

finished product looks pretty good, it still wasn't as easy as I

thought it would be.

GL,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitor Wars...

Date: Fri, 13 Feb 1998 14:15:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Fri, 13 Feb 1998 06:15:46 -0800 (PST)

Subject: Capacitor Wars...

Well,

The debate rages on... I want to use poly for the dielectric in our

cap (3 layers of 20 mil, times 14 plates, times 3 modules, for .01 mfd, with

voltage rating of 36kv) for our 15kv, 30 ma neon. My partner, the old coot

(retired engineer) keeps looking for something "easier". Now he wants to use

epoxy circuit board mat'l, a single layer, approx. .06" thick. Will this give

use the voltage protection we need? or do we need multiple layers, as with

poly? We can probably find the mat'l at our local surplus/salvage dealer at a

per pound, reasonable price.

danmurphy@1stnetusa.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Fri, 13 Feb 1998 15:37:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 13 Feb 1998 07:37:32 -0800 (PST)

Subject: Re: Capacitor Wars...

On 13 Feb, Tesla-2 wrote:

>

> Well,

>         The debate rages on... I want to use poly for the dielectric in our

> cap (3 layers of 20 mil, times 14 plates, times 3 modules, for .01 mfd, with

> voltage rating of 36kv) for our 15kv, 30 ma neon. My partner, the old coot

> (retired engineer) keeps looking for something "easier". Now he wants to use

> epoxy circuit board mat'l, a single layer, approx. .06" thick. Will this give

> use the voltage protection we need? or do we need multiple layers, as with

> poly? We can probably find the mat'l at our local surplus/salvage dealer at a

> per pound, reasonable price.

well.. most "epoxy" circut boards are made of G-10, or its relatives.

This is a fiberglass impregnated resin. Key word is glass. Might

as well go for a saltwater cap, much less work, similar loss factor.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitor Wars...

Cc:

Bcc:

X-Attachments:

At 02:15 PM 2/13/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Fri, 13 Feb 1998 06:15:46 -0800 (PST)

>Subject: Capacitor Wars...

>

>Well,

>        The debate rages on... I want to use poly for the dielectric in our

>cap (3 layers of 20 mil, times 14 plates, times 3 modules, for .01 mfd, with

>voltage rating of 36kv) for our 15kv, 30 ma neon. My partner, the old coot

>(retired engineer) keeps looking for something "easier". Now he wants to use

>epoxy circuit board mat'l, a single layer, approx. .06" thick. Will this give

>use the voltage protection we need? or do we need multiple layers, as with

>poly? We can probably find the mat'l at our local surplus/salvage dealer at a

>per pound, reasonable price.

>                                        danmurphy@1stnetusa.com

>

>

>

Dan,

A capacitor for RF use is only as good as its dielectric.

That means the use of polyethylene or polypropylene for the

dielectric. Printed circuit board material is most commonly

made from fiberGLASS. This gives it a very high RF

dissipation factor thus poor performance. If you are going

to go to the effort to build a cap build the best performing

one you can.

The voltage rating from circuit board material is also quite low

requiring a number of series connected units.

Stick with the poly cap in mineral or preferably transformer

oil and you wont be disappointed.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Sat, 14 Feb 1998 05:28:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>>(retired engineer) keeps looking for something "easier". Now he wants to use

T>>epoxy circuit board mat'l, a single layer, approx. .06" thick. Will this give

T>A capacitor for RF use is only as good as its dielectric.

T>That means the use of polyethylene or polypropylene for the

T>dielectric. Printed circuit board material is most commonly

T>made from fiberGLASS. This gives it a very high RF

T>dissipation factor thus poor performance. If you are going

Reminds me of the time I once designed and built a five section low

pass filter for my KW ham tx.  I used double sided circuit board for the

filter caps.  Thought it should work, as the RF voltage was not

excessive.  Tuned up just great; nice bandpass.  Fired up the rig, and

the filter lasted about 5 minutes before the caps blew. 8()  Went back

to good old mica caps.

Ralph  W5JGV

---

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Sat, 14 Feb 1998 12:02:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sat, 14 Feb 1998 07:02:21 EST

Subject: Re: Capacitor Wars...

Hi Dan,

I would side with you on the poly. While circuit boards seem like a good

idea, I don't believe that they are as good as poly. I don't think the boards

have the dielectric strength needed to make a single cap with the voltage

rating needed.

MIke Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitor design

Date: Sat, 14 Feb 1998 14:13:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

Date: Sat, 14 Feb 1998 08:13:53 -0600 (CST)

Subject: Capacitor design

Hey, all.  I've designed my poly plate caps and am about to begin

building.  Here's the plan, for the perusal of the list.

I will make 3 caps, each 0.0038uf at 15KV.  Since I will have 3 15/30

neons, I can swap in caps with transformers for power changes.  The caps

will be connected in parallel for higher capacitance.

Each cap will have 8 sections.  Each section is 0.0304uf at 2KV.  The

sections are connected in series to achieve the higher voltage rating.

Each section is 14 6x6 aluminum foil plates separated by two layers of

4mil LDPE.  Poly extends past aluminum by 1/2" on all sides.  Since the

highest voltage that can exist between two adjacent sections is twice the

rated voltage for a single section, I was thinking about separating the

sections with 4 layers of 4mil poly.

The sections will be compressed between a couple of plexi plates and put

in mineral oil, in a Rubbermaid container.  Since I'm stacking vertically,

I'm going to have to pump the air out of the container.  I'll be using an

air compressor I removed from an O2 concentrator.  It pulls a pretty good

vacuum on the intake side.

Any comments are welcome.

Tim Osmulski

tim.o@ttu.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Sat, 14 Feb 1998 15:20:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 14 Feb 1998 10:20:44 -0500

Subject: Re: Capacitor Wars...

> From: Hollmike@aol.com

> Date: Sat, 14 Feb 1998 07:02:21 EST

> Subject: Re: Capacitor Wars...

>

> Hi Dan,

>    I would side with you on the poly. While circuit boards seem like a good

> idea, I don't believe that they are as good as poly. I don't think the boards

> have the dielectric strength needed to make a single cap with the voltage

> rating needed.

>

> MIke Hollingsworth

>

>

Mike & Dan,

The dissipation factor of circuit board material is not good. I have used

HV caps made for DC service from circuit board material (G10) with much

success. But the RF characteristics are pathetic! After a short running time

the material gets excessively hot and self destructs.

Another issue is the clearance between the foil and the edge of the board.

If you do not etch away at least 1" of foil you will get arcing around the

edge.

BTW, on my DC HV caps built with PC board stock all I had to do to get

the things interconnected was to first clean the metal surfaces, thoroughly

dry the boards, and then place the boards up against one another. The

foil surfaces touching made a decent interconnection. To get compression I

mounted the plates in a plexiglass box. I connected to the ends of the

capacitor using flat strips of copper. The plates were inserted one at a time

until I couldn't insert any more. Then the unit was sealed, transformer

oil was introduced while a vacuum was being pulled, and then the inlets

were sealed by placing a short length of poly tubing on the inlet pipe.

The short piece of poly tubing was previously sealed on one end by first

heating it with a hot air gun until the end became soft, and then squishing

that end with a pair of flat nose pliers.

It was a great DC HV cap, and I used it for many years, but it fried itself

when a student of mine paralleled it across the main cap of a Tesla coil

in an effort to bring a system into tune. So I know whereof I speak :(

Polyethelene and polypropylene are the materials to use for Tesla coil caps.

Hope this helps.

Fr. Tom Mcgahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor design

Date: Sat, 14 Feb 1998 18:04:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 14 Feb 1998 10:04:48 -0800

Subject: Re: Capacitor design

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Tim Osmulski <tim@hpcl.cs.ttu.edu>

> Date: Sat, 14 Feb 1998 08:13:53 -0600 (CST)

> Subject: Capacitor design

>

> Hey, all.  I've designed my poly plate caps and am about to begin

> building.  Here's the plan, for the perusal of the list.

>

> I will make 3 caps, each 0.0038uf at 15KV.  Since I will have 3 15/30

> neons, I can swap in caps with transformers for power changes.  The caps

> will be connected in parallel for higher capacitance.

>

> Each cap will have 8 sections.  Each section is 0.0304uf at 2KV.  The

> sections are connected in series to achieve the higher voltage rating.

>

> Each section is 14 6x6 aluminum foil plates separated by two layers of

> 4mil LDPE.  Poly extends past aluminum by 1/2" on all sides.  Since the

> highest voltage that can exist between to adjacent sections is twice the

> rated voltage for a single section, I was thinking about separating the

> sections with 4 layers of 4mil poly.

>

> The sections will be compressed between a couple of plexi plates and put

> in mineral oil, in a Rubbermaid container.  Since I'm stacking vertically,

> I'm going to have to pump the air out of the container.  I'll be using an

> air compressor I removed from an O2 concentrator.  It pulls a pretty good

> vacuum on the intake side.

>

> Any comments are welcome.

> Tim Osmulski

> tim.o@ttu.edu

>

Tim,

Overall, it sounds like a pretty good design approach! You may want to

assume an "effective" relative dielectric constant of about 2.0 instead

of 2.2 or 2.3 for the poly and mineral oil (this is what most of us

actually see). This takes into consideration that you don't have

extremely tight packing between the foil and the poly plates. Using k of

2.0, I get about 2023 pF for each 6"x6"x 8 mil element. Fourteen

elements in parallel is about .0283 uF per section, and eight sections

in series total about 0.0035 uF. You may need to add one or two more

elements/section if you're aiming for 0.0038 - 0.004 uF.

64 mils of dielectric is a little bit on the low side when running off a

15 kV source, but by splitting the cap into 8 sections and using 2 thin

sheets/element, you're doing the right things for sharing the voltage

stress. As long as you carefully inspect the poly sheets and reject

those with any obvious inclusions or defects, AND keep your main gap

spacing to no more than about 3/8" you'll probably be OK. The design

sounds like it will be sturdy and compact. You may want to carefully

fold the foil edges to give them less of a sharp profile. Most cap

failures occur along these foil edges because of the very high field

stresses seen there.

How do you plan on making electrical connections to the foil sheets and

bringing out the end terminal connections?

Safe cappin' to you, Tim!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary to secondary spacing

Date: Sat, 14 Feb 1998 18:25:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sat, 14 Feb 1998 13:25:13 EST

Subject: primary to secondary spacing

I plan on building a 30 degree primary w/ 1/4" tubing but want to learn more

about the problems of winding it first. In the meantime (the rest of my set up

is finished & I want to try and get some sparks!) I have a bunch of #10 solid

copper wire I want to wind a helical primary with. A previous reply suggested

that a copper tube saucer shape should have about a 1 1/4" spacing between the

first turn of the primary and the secondary. Does anyone know what distance I

should keep on a helical? I have a 22" L. x 4 1/2"D. primary of 24 gauge wire

and am using a 15K 30MA tranny. Some old plans I have suggest a primary

diameter of twice the secondary but I wanted to check. Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: primary to secondary spacing

Cc:

Bcc:

X-Attachments:

At 06:25 PM 2/14/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sat, 14 Feb 1998 13:25:13 EST

>Subject: primary to secondary spacing

>

>I plan on building a 30 degree primary w/ 1/4" tubing but want to learn more

>about the problems of winding it first. In the meantime (the rest of my set up

>is finished & I want to try and get some sparks!) I have a bunch of #10 solid

>copper wire I want to wind a helical primary with. A pevious reply suggested

>that a copper tube saucer shape should have about a 1 1/4" spacing between the

>first turn of the primary and the secondary. Does anyone know what distance I

>should keep on a helical? I have a 22"L. x 4 1/2"D. primary of 24 gauge wire

>and am using a 15K 30MA tranny Some old plans I have suggest a primary

>diameter of twice the secondary but I wanted to check. Thanks, Jim

>

>

Jim,

For a helical (solenoid) primary you want it to be MORE than TWICE

the diameter of your secondary. Twice your 4.5 inch diameter secondary

is 9 inches so a 10 inch diameter primary would work out well.

Even going to 12 inches in diameter would not hurt.

Dont sell a solenoid primary short. They do work very well once

you deal with the possibilty of overcoupling. The primary on my

coil is a tapered solenoid. 8 inches diameter at the bottom and

12 inches diameter at the top with 17 turns. This primary has allowed

me to get a maximum of 54 inches of spark with a little over 1200 watts

on my 4.5 X 21 inch secondary.

One advantage with a solenoid primary is its close proximity

to the secondary reducing the likelihood of the output streamers

striking the primary. In fact I have never had that problem and never

needed to add a grounded strike rail.

A saucer or flat spiral primary will be out in the strike zone

and become a frequent target so a strike rail is necessary with these

primary shapes.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: first real sparks

Date: Sat, 14 Feb 1998 22:46:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sat, 14 Feb 1998 17:46:58 -0500

Subject: first real sparks

Hi coilers

I got my first real output the other day

better than 45" as best I could measure. This was in my garage and

I will have to go outside to isolate it down to a single strike point.

Considering the size of my coil this is not all that amazing.

but I only have half of the calculated capacitance necessary.

I learned the hard way how modern electronics & coiling dont mix

after coiling for a while I came in to record the results on my PC.

Although I did turn the pc off the external modem was on, as were

the speakers.  Now both are totally fried. I had to install the old 14.4k

modem to reconnect.

after investigating- my phone line runs directly above the coil in the

attic and I noticed some streamers hitting the ceiling.

Luckily my pc seems fine.

I will be working on more caps and waiting for a new modem to arrive.

I also am working on chokes.

for those interested my spec's are

secondary 10.75"  800 turns 18ga wire

toroid 40"major 5"cross section

primary 20 turns 1/4" refrig tubing (3/8 od) 30 deg inv cone, tapped at turn 5

transformer's   4 microwave units in series est 8000 volts .5a

present cap's   1 unit , single .030 layer rolled poly in 6"pvc =.093uF

spark gap  RQ type 13- .937 od copper tubes  12" long

divided into 2 parallel paths for a total gap capability of .180" each

with a fan in one end pulling thru it.

at present I only use .120" each until I get more caps in series.

note my planned capacitance is .18uF when finished

a lot of rolling !!

comments,questions welcome

safe coiling everyone

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: TC's & computers

Date: Sun, 15 Feb 1998 00:34:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sat, 14 Feb 1998 19:34:55 EST

Subject: TC's & computers

I just read a posting on accidentally frying a modem while running a coil. My

computer is less than 12' from my work area for my coil and phone lines, water

pipes and heating ducts (its in the basement) are all fairly near. Short of

relocating the computer or coil, what safety measures should I be taking?

Another safety question: should my safety gap (two brass bolts w/ acorn nuts

gapped just wider than it can arc ) have a mid point terminal grounded to my

coil's dedicated ground located between them? Thanks again, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC's & computers

Cc:

Bcc:

X-Attachments:

At 12:34 AM 2/15/98 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sat, 14 Feb 1998 19:34:55 EST

>Subject: TC's & computers

>

>I just read a posting on accidentally frying a modem while running a coil. My

>computer is less than 12' from my work area for my coil and phone lines, water

>pipes and heating ducts (its in the basement) are all fairly near. Short of

>relocating the computer or coil, what safety measures should I be taking?

Jim I recommend that you invest in the worlds best and cheapest

surge protector - Pull the plug! I unplug the computer including

the phone line whenever I run my coil. Never had a problem.

My computer is located in an adjacent room about 25 feet from

the coil.

>Another safety question: should my safety gap (two brass bolts w/ acorn nuts

>gapped just wider than it can arc ) have a mid point terminal grounded to my

>coil's dedicated ground located between them? Thanks again, Jim

>

>

A safety gap with a grounded center post is a good idea even

though I dont presently have one. My safety is simply across

the HV lines without a grounded center post. The grounded

center post does however provide better protection in case of

a RF kickback.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: very small coil

Date: Sun, 15 Feb 1998 09:39:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sun, 15 Feb 1998 14:39:08 +0500

Subject: Re: very small coil

On Sun, 08 Feb 1998 22:40:56 -0600 Jay

<tesla.coil@mailexcite.com> wrote;

> Fellow coilers,

>

> I've heard a few reports of people building very small coils successfully. By

> small I mean a coil with a diameter of 1 inch or less. Has anyone had

> any experience with these?

Hi Jay, I've built a couple of very tiny Tesla coils copying off of a

dissected "Violet-Ray" coil. The smallest had a secondary 1 cm. in diameter

and 5 cm. long, it was wound with #28 enamelled wire then paraffin coated

and then I wound a 5 turn primary out of #14 enamelled wire on top of the

secondary. It was powered in "kicker coil" fashion by the 120 volt mains,

the coil yielded strong blue sparks typically 2 inches in length or about

50 KV!

> I have a small inverter that will deliver 900 volts from a 9 volt battery. If I

> run it through a tripler, I get 2700 volts along with a small amount of capacitance

> Could I use this setup as my power supply and capacitor?

I wouldn't put much hope on that supply, it has extremely low current

output and in powering Tesla coils you need power to drive the sparks!

I have seen many small Tesla coils powered by a T.V. fly-back transformer

driven by a 555 timer oscillator and some power transistors but they usually

were at least 25 watts and up.

> Also, would anyone know the specs of a coil with a 1cm diameter? Any thoughts

> are appreciated!

Jay, check out a copy of Thomas Stanley Curtis's "High Frequency

Apparatus" 1916 (reprinted by Lindsay Publications 1988). It has plans

for several small Tesla coils in the size you describe and it also has a

chapter devoted to the "Kicker coil" driven Tesla coils I described.

Considering that neon sign transformers are being phased out in the sign

industry, the "Kicker coil" may be the best replacement.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Is a 6kv 30ma NST any good

Date: Sun, 15 Feb 1998 09:39:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sun, 15 Feb 1998 14:39:10 +0500

Subject: Re: Is a 6kv 30ma NST any good

On Tue, 10 Feb 1998 15:10:08 -0600 Mike Hammer

<mhammer@misslink.net> wrote;

> Yes that would be a nice size for a small coil. Anything less

> than 6 kv becomes difficult to get good gap operation.

Not really so. I have seen old hand held Tesla coils running off of 90

volts! Typically these coils are what I refer to as "kicker coil" powered

Tesla coils, where the coils is driven by an inductor's counter E.M.F.

which IF it's magnetic field is collapsed fast enough can develop to many

times the applied voltage. Below is an ASCII diagram of a typical "kicker

coil" driven Tesla coil.

___

(___)

)

)

)

)

) S

)

__________________)

VG ____  _|_         )

| | ___C        )P

#( |__|__________)

C #(    |

O #(    |

I #(    |

L #(    |

|    |

115V

AC or DC

You have an iron core coil connected at one side of it's windings to the

line. The other side of the coil connects to a vibrating armature almost

identical to those found on the old induction coils. The stationary contact

of the interrupter is connected to the Tesla coil primary P, the other side

of the primary is connected to the Tesla coil secondary S. the primary also

connects to the moving contact of the interrupter and the capacitor C is

across the vibrating spark gap VG. There should be an adjusting screw that

pushes the moving armature of the vibrating spark gap closer to or further

away from the stationary contact. Instructions for building several

different size Tesla coils driven by a "kicking coil" are described in

Thomas Stanley Curtis's "High Frequency Apparatus" 1916 (reprinted by

Lindsay Publications 1988)in chapter VIII "Kicking Coil Apparatus". BTW

this book also describes some of the smallest Tesla coils I have ever seen!

One uses a 15 x 2 centimeter glass test tube for the secondary form!

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RF ground connection

Date: Sun, 15 Feb 1998 14:00:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 15 Feb 1998 14:00:55 +0000

Subject: RF ground connection

When I power up to 100% on my 15kv nst I get a arc at my RF ground

that doesn't appear to strike the primary.

Could this be an arc from the sec' to the ground, if the connection

isn't all that great?

If it was arcing to the primary wouldn't the safety gap also fire?

And, is there a "no-heat" way to improve my ground connection to the

sec'?  (right now it's a solder lug screwed into the copper plate

that's epoxied to the base of the coil.  It feels tight, doesn't

budge with light-moderate finger pressure.)

Also, if my safety gap isn't firing at all, or at least I'm not

hearing it like I used to with my solenoid primary, and it's set at

3/8" each side of the center post, do I have a problem elsewhere that

I need to be looking for?

Thanks, Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Another Primary Question

Date: Sun, 15 Feb 1998 14:00:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 15 Feb 1998 14:00:55 +0000

Subject: Re: Another Primary Question

> For an optimum coefficient of coupling between the primary

> and secondary, the plane of a flat primary coil may need to

> be elevated a bit above the bottom end of the secondary

> winding.

Re-he-heallly! (in my best Jim Carrey, Pet Detective typing)

So, what I hear Bert saying is that I went from terrible

over-coupling, and poor frequency match with my solenoid primary, to

a good freq match, but potential under-coupling with my flat spiral.

So here's my question:  If the resonant freq.s of my two windings

match-up pretty well on paper, but I still don't get quite the

expected results, then I could be too loose in my coupling?  Any

other "tell-tales" to determine if this is a problem?

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary question

Date: Sun, 15 Feb 1998 17:14:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Sun, 15 Feb 1998 09:14:36 -0800 (PST)

Subject: Re: primary question

O.K -

I have to ask a dumb question - What is a "solenoid" primary? I'm familiar

with the flat "archimedes" spiral, the inverted saucer, and the cylindrical helix.

Could someone please enlighen me?

Dantheman

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Another Primary Question

Date: Sun, 15 Feb 1998 17:22:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Sun, 15 Feb 1998 09:22:17 -0800 (PST)

Subject: Another Primary Question

O.K. -

Waldo has qot another brilliant idea. He wants to turn the primary

mounting platform (plexiglass?) upside down, so the primary coil will be under

the plastic.

He thinks this will prevent arc overs from striking the primary. Will this work?

(He lays awake nights thinking this stuff up - Keep in mind he's never built a

coil... He's relying on his experience as an engineer.)

El Nino Dan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Model A Car spark coil

Date: Sun, 15 Feb 1998 20:01:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Ryan <jryan@exis.net>

Date: Sun, 15 Feb 1998 15:01:50 -0500

Subject: Model A Car spark coil

I just obtained an operational Model A car spark generator coil (cool wooden box).

Does anyone know if it has any use in a Tesla coil.

Thanks

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Another Primary Question

Date: Sun, 15 Feb 1998 21:45:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sun, 15 Feb 1998 13:45:33 -0800 (PST)

Subject: Re: Another Primary Question

On Sun, 15 Feb 1998, Tesla-2 wrote:

> O.K. -

>         Waldo has qot another brilliant idea. He wants to turn the primary

> mounting platform (plexiglass?) upside down, so the primary coil will be under

> the plastic.

> He thinks this will prevent arc overs from striking the primary. Will this work?

> (He lays awake nights thinking this stuff up - Keep in mind he's never built a

> coil... He's relying on his experience as an engineer.)

>                                                         El Nino Dan

It will work, in that your coil will still operate. As for 'Preventing

strikes to the primary.' Perhaps from the bottom to the inner turn.

From the topload to the coil? Nahh... 1/4" is about the same as

air for coils of any size :)

[The 130KVA coil Greg L is building has the primary *inside* the

secondary]

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: primary question

Cc:

Bcc:

X-Attachments:

At 05:14 PM 2/15/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Sun, 15 Feb 1998 09:14:36 -0800 (PST)

>Subject: Re: primary question

>

>O.K -

> I have to ask a dumb question - What is a "solenoid" primary? I'm familiar

>with the flat "archimedes" spiral, the inverted saucer, and the cylindrical

>helix.

>Could someone please enlighten me?

>                                        Dantheman

>

A solenoid is a coil wound with a constant diameter just

like your secondary. Also known as a helix by some.

Consider yourself "enlightened". :)

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Another Primary Question

Cc:

Bcc:

X-Attachments:

At 05:22 PM 2/15/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Sun, 15 Feb 1998 09:22:17 -0800 (PST)

>Subject: Another Primary Question

>

>O.K. -

> Waldo has qot another brilliant idea. He wants to turn the primary

>mounting platform (plexiglass?) upside down, so the primary coil will

>be under the plastic.

>He thinks this will prevent arc overs from striking the primary. Will this work?

>(He lays awake nights thinking this stuff up - Keep in mind he's never built a

>coil... He's relying on his experience as an engineer.)

>                                                        El Nino Dan

>

Sure that will work. But also realize that if your coil is overcoupled

that layer of plexiglass will not prevent arcing. The arcs will pop right

on through it. Or find a way around it. The plexiglass will help somewhat

but the only cure for overcoupling is to reduce the coupling until arcing stops.

Plexiglass also wont make a whole lot of difference when it comes to

secondary streamers striking the primary. When dealing with RF of several

hundred KV normal insulators dont help much. A grounded strike rail to

provide a more favored target is the cure.

Keep Waldo thinking.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Another Primary Question

Date: Sun, 15 Feb 1998 22:46:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 15 Feb 1998 14:46:42 -0800

Subject: Re: Another Primary Question

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Sun, 15 Feb 1998 09:22:17 -0800 (PST)

> Subject: Another Primary Question

>

<SNIP from earlier message>

O.K -

I have to ask a dumb question - What is a "solenoid" primary? I'm

familiar with the flat "archimedes" spiral, the inverted saucer, and the

cylindrical helix.

Could someone please enlighen me?

Dantheman

A solenoid primary is the same thing as  a cylindrical helix.

> O.K. -

>        Waldo has qot another brilliant idea. He wants to turn the primary

> mounting platform (plexiglass?) upside down, so the primary coil will be under

> the plastic.

> He thinks this will prevent arc overs from striking the primary. Will this work?

> (He lays awake nights thinking this stuff up - Keep in mind he's never built a

> coil... He's relying on his experience as an engineer.)

>                                                         El Nino Dan

>

This is not a bad idea, but is usually not necessary if:

1. You've got adequate clearance between the inner-most turn and the

secondary (1.5"+)

2. You use a flat or inverted saucer primary protected with a strike

rail. A strike rail is a single incomplete loop of 1/4" or 3/8" copper

tubing. Make it slightly less that a full turn (to avoid getting a

shorted-turn) and connec it to the RF ground (i.e., the base of the

secondary). The strike rail is positioned about 2" or more above the

outermost turn on the primary, and its purpose is to protect the primary

from direct strikes from the toroid. When you use a toroid as a top

terminal, it tends to throw the streamers outward. With an efficient

coil, these streamers can be long enough to come back and hit the outer

portion of the primary, where they could otherwise wipe out your neon or

tank cap. Instead, they'll hit the strikerail, protecting your primary

circuit.

There are a couple of other considerations as well. For an optimum

coefficient of coupling between the primary and secondary, the plane of

a flat primary coil may need to be elevated a bit above the bottom end

of the secondary winding. You may need to make provision for the

secondary to go through a hole in the Plexiglas cover so as to get

proper coupling. Also, you'll need to have adequate clearance betwen the

primary and the cover so that you can properly tap the primary to bring

the system into tune.

The Bottom Line: a Plexi cover is really not necessary but it also won't

hurt as long as you take the above factors into consideration.

Safe coilin' to you, Dan!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Model A Car spark coil

Date: Mon, 16 Feb 1998 01:37:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sun, 15 Feb 1998 20:37:02 EST

Subject: Re: Model A Car spark coil

Jim,

That spark coil will work fine for a small coil.  The coil may not shoot

sparks more than an inch or two, but it can be a fun demonstration coil for

kids.   Since it has the vibrator points built in, all you have to do is

charge a cap with it and use it in a normal spark gapl circuit.  The nice part

is that you can operate in using a six volt battery.  You can get replacement

points for it through J.C. Whitney catalog.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Is a 6kv 30ma NST any good

Date: Mon, 16 Feb 1998 01:52:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sun, 15 Feb 1998 20:52:14 EST

Subject: Re: Is a 6kv 30ma NST any good

Alfred,

Thanks for posting the schematic and explaining how these work.  I have a

commercially made one like you are describing, but did not know exactly how it

works.  These are commonly used by glass blowers to check for leaky glass

welds in  tubing connections.  I also saw one similar to mine in a museum with

interchangalbe glow discharge teriminals meant for medical therapy.

I also bought a tube driven coil from a garage sale that operated on 700v.

The secondary was about 2000 turns of what appears to be about 36g magnet wire

on a two inch acrylic tube.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Another Primary Question

Date: Mon, 16 Feb 1998 14:37:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 16 Feb 1998 06:37:52 -0800

Subject: Re: Another Primary Question

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Sun, 15 Feb 1998 14:00:55 +0000

> Subject: Re: Another Primary Question

>

> > For an optimum coefficient of coupling between the primary

> > and secondary, the plane of a flat primary coil may need to

> > be elevated a bit above the bottom end of the secondary

> > winding.

>

> Re-he-heallly! (in my best Jim Carrey, Pet Detective typiing)

> So, what I hear Bert saying is that I went from terrible

> over-coupling, and poor frequency match with my solenoid primary, to

> a good freq match, but potential under-coupling with my flat spiral.

>

> So here's my question:  If the resonant freq.s of my two windings

> match-up pretty well on paper, but I still don't get quite the

> expected results, then I could be too loose in my coupling?  Any

> other "tell-tales" to determine if this is a problem?

>

> Thanks. . .Chris

>

Chris,

Could be. If you're close to being in tune, try adjusting the tap a bit

one way or another at low power and carefully measure the output to see

if there's any performance improvement. Once you're sure that it's in

tune, you can then begin adjusting the coupling.

The best coupling for your coil will be a function of how good a main

gap you've got. A vacuum gap, for example, quenches better and will

allow you to run at higher coupling coefficients than a RQ-style

cylindrical gap. If you're overcoupling you'll see performance drop, and

you may also see other signs such as sparks jumping from the top of the

winding to other portions of the winding. Another symptom - if you run

the coil in a dark room, you may also see a "Christmas tree" of corona

filling the space between the top of the winding to the primary.

Safe coilin' to you, Chris!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Capacitor Wars...

Date: Mon, 16 Feb 1998 15:26:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Mon, 16 Feb 1998 09:26:02 -0600

Subject: RE: Capacitor Wars...

Hello,

I saw some PVC sheeting at the local home supply store would this be good for T

-coil caps?

Roy Minnich

<SNIP>

Polyethelene and polypropylene are the materials to use for Tesla coil caps.

Hope this helps.

Fr. Tom Mcgahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: TC's & computers

Date: Mon, 16 Feb 1998 15:53:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chip Atkinson <chip@pupman.com>

Date: Mon, 16 Feb 1998 08:53:26 -0700 (MST)

Subject: Re: TC's & computers

Greetings,

I, too, pull the plug.  My machine is too important to many more people

other than myself to risk on the quality of surge protection circuits.

Besides, if a spark can jump over 36" to ground, I suspect that surge

protection circuitry won't be totally effective.

Chip

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Chip Atkinson

http://www.pupman.com

--- If I can't fix it, I can fix it so it can't be fixed --

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

On Sun, 15 Feb 1998, Tesla-2 wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 14 Feb 1998 20:46:53 -0600

> Subject: Re: TC's & computers

>

> At 12:34 AM 2/15/98 GMT, you wrote:

> >From: Dsurfr@aol.com

> >Date: Sat, 14 Feb 1998 19:34:55 EST

> >Subject: TC's & computers

> >

> >I just read a posting on accidentally frying a modem while running a coil.

> >My computer is less than 12' from my work area for my coil and phone lines, water

> >pipes and heating ducts (its in the basement) are all fairly near. Short of

> >relocating the computer or coil, what safety measures should I be taking?

>

> Jim I recommend that you invest in the worlds best and cheapest

> surge protector - Pull the plug! I unplug the computer including

> the phone line whenever I run my coil. Never had a problem.

> My computer is located in an adjacent room about 25 feet from

> the coil.

>

>

> >Another safety question: should my safety gap (two brass bolts w/ acorn nuts

> >gapped just wider than it can arc ) have a mid point terminal grounded to my

> >coil's dedicated ground located between them? Thanks again, Jim

> >

> >

>

> A safety gap with a grounded center post is a good idea even

> though I dont presently have one. My safety is simply across

> the HV lines without a grounded center post. The grounded

> center post does however provide better protection in case of

> a RF kickback.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Mon, 16 Feb 1998 17:24:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 16 Feb 1998 09:24:38 -0800

Subject: Re: Capacitor Wars...

Tesla-2 wrote:

>

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Date: Mon, 16 Feb 1998 09:26:02 -0600

> Subject: RE: Capacitor Wars...

> Is there a big dielectric difference between high and low density

>polyethelene and polypropylene?

walt Kulpa

> Hello,

> I saw some PVC sheeting at the local home supply store would this be good for T

> -coil caps?

>

> Roy Minnich

>

> <SNIP>

> Polyethelene and polypropylene are the materials to use for Tesla coil caps.

>

> Hope this helps.

> Fr. Tom Mcgahee

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: TC's & computers

Date: Mon, 16 Feb 1998 17:59:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Mon, 16 Feb 1998 11:59:30 -0600

Subject: Re: TC's & computers

I suppose I too, should be doing this, but I have never unplugged my

computer when running my coil.  I'm obviously asking for it, but the

computer is 35 feet away, and on another circuit...  I always assumed

this would be fine... tell me I'm right, please? :-)

>I, too, pull the plug.  My machine is too important to many more people

>other than myself to risk on the quality of surge protection circuits.

>Besides, if a spark can jump over 36" to ground, I suspect that surge

>protection circuitry won't be totally effective.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Capacitor Wars...

Date: Mon, 16 Feb 1998 18:18:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 16 Feb 1998 12:18:20 -0600

Subject: RE: Capacitor Wars...

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>I saw some PVC sheeting at the local home supply store would this be good for

T

>-coil caps?

Sometimes the construction grade poly contains lots

of impurities.  I have a roll of the 0.006 inch sheet

which contains little lumps of "stuff", looks like sand

or something.  Possibly this is made from recycled

shredded plastic, so it might not even be 100% poly!

The use of this type of poly led me the the invention

of the LEP - or Light Emitting Capacitor (I suspect that

other coilers have gone down similar research

paths...)  Corona, anyone (and I don't mean a

beer)   :)  It ended up being a combination capacitor

and spark gap, all in one!

Also - the grey or black poly is to be avoided for obvious reasons...

Cadillac plastics sells 0.060 LDPE sheets, 4ft by 8ft

for $17.  For an extra buck or two the local distributor

will shear it into 8x12 inch plates for our plate

caps.  We have constructed six of these units and

have been running them for many months w/o failure.

Cadillac has distributors in most large cities.

-Bill the arcstarter

Starting arcs in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitor Wars...

Cc:

Bcc:

X-Attachments:

At 05:24 PM 2/16/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Mon, 16 Feb 1998 09:24:38 -0800

>Subject: Re: Capacitor Wars...

>

>Tesla-2 wrote:

>>

>> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>> Date: Mon, 16 Feb 1998 09:26:02 -0600

>> Subject: RE: Capacitor Wars...

>>

>> Is there a big dielectric difference between high and low density

>polyethelene and polypropylene?

>

>walt Kulpa

Walt,

High density polyethylene (HDPE) and low density polyethylene (LDPE)

have very nearly the same electrical characteristics. So both are

quite suitable as capacitor dielectrics.

However HDPE is quite stiff and would not roll up very well

so is not suitable for rolled caps. LDPE is soft and supple

and rolls up very well. HDPE would be excellent for flat

plate caps.

>

>> Hello,

>>I saw some PVC sheeting at the local home supply store would this be good for

>>T-coil caps?

>>

>> Roy Minnich

>>

Roy,

PVC sheeting is NOT suitable for caps. It has a very high RF dissipation

factor. Also watch out for some types of plastic films from hardware

stores. Sometimes these are mylar films or vinyl. If the box doesnt

say polyethylene dont buy it.

Also not suitable are Plexiglass (PMMA) or Lexan (polycarbonate).

You really need polyethylene or polypropylene for caps.

Polystyrene is also pretty good for plate caps but wont

roll up for rolled caps.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF ground connection

Cc:

Bcc:

X-Attachments:

At 02:00 PM 2/15/98 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sun, 15 Feb 1998 14:00:55 +0000

>Subject: RF ground connection

>

>When I power up to 100% on my 15kv nst I get a arc at my RF ground

>that doesn't appear to strike the primary.

>

>Could this be an arc from the sec' to the ground, if the connection

>isn't all that great?

>

Chris this sounds like to me that you are getting an arc from

your innermost primary turn to the ground at the bottom of your

secondary. Perhaps your spacing isnt quite enough here.

You can try raising up your secondary a little to provide additional

clearance or try to add some additional insulation between

primary and secondary in this area. If the problems persist

you may need to remove the innermost turn. This wont affect

your primary inductance much since most of the inductance

is in the outer turns.

>If it was arcing to the primary wouldn't the safety gap also fire?

Chances are its not arcing TO the primary but rather FROM the

primary to ground.

>

>And, is there a "no-heat" way to improve my ground connection to the

>sec'?  (right now it's a solder lug screwed into the copper plate

>that's epoxied to the base of the coil.  It feels tight, doesn't

>budge with light-moderate finger pressure.)

I solder a 3 foot long piece of AWG 8 stranded cable directly

to the ground plate. The 3 foot ground lead is terminated in a

heavy ring lug. I attach this to the ground system by bolting

the ring lug to a matching lug on my ground system.

My career in electronics has taught me to only trust

well soldered or bolted connections.

>

>Also, if my safety gap isn't firing at all, or at least I'm not

>hearing it like I used to with my solenoid primary, and it's set at

>3/8" each side of the center post, do I have a problem elsewhere that

>I need to be looking for?

>

I would bet that your coupling is a whole lot less than with

your solenoid primary so everything is running smoother. If

you are getting nearly the same output then thats great. If

your output is less you may be undercoupling.

If things are running fine otherwise you may need to narrow

your safety gap a bit. Ive had my safety gap down to where

it was nearly the same as the main gap without firing.

It just means the coil is running smoothly with no kickbacks.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: logistics

Date: Mon, 16 Feb 1998 21:03:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 16 Feb 1998 13:03:53 -0800

Subject: logistics

I'm just starting on my secondary and planning ahead. I have a small

wooden cabinet (24"x24"x 34" high - on wheels) on which i will be setting

my coil. I've been thinking about storing the transformer, caps, all the

other components on the shelving located within the cabinet box. I could

then fire the system up at the end of a long extension cord - my concern

about this is won't be able to see the spark gaps and my plug in wire

seems like it could be in proximity of a strike.

Next thought. I can stuff transformer and other components in a box

and run a piece of wire 15' or so over to the coil, this seems the most

reasonable.

My question is: what type of proximity is generally used between

components? What components are generally boxed together? is each

component transformer, spark gap and capacitor a separate module in a

separate box? with a 12/30 neon and rolled poly cap, what kind of

distance will my components (and myself) need to be away to operate

safely?

One other question is that I haven't seen anything on a disconnect means

(on/off switch) and over-current protection (fusing),  between the

transformer and the wall socket. I am assuming that it exists as a switch

and a fuse block or as a one unit circuit breaker box mounted somewhere.

I have considered using a 24v control circuit to run the neon while the

circuit is closed and then to short my capacitor to ground when the

circuit is de-energized - is this more work than necessary?. what good

ideas are out there in regard to control circuits? do they pose a

problem?

Thanks for the help.

Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Tue, 17 Feb 1998 03:36:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 16 Feb 1998 19:36:09 -0800

Subject: Re: Capacitor Wars...

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Date: Mon, 16 Feb 1998 09:26:02 -0600

> Subject: RE: Capacitor Wars...

>

> Hello,

> I saw some PVC sheeting at the local home supply store would this be good for T

> -coil caps?

>

> Roy Minnich

>

> <SNIP>

> Polyethelene and polypropylene are the materials to use for Tesla coil caps.

>

> Hope this helps.

> Fr. Tom Mcgahee

>

Roy,

Only if you want lossy caps that will heat up and self-destruct. :^)

Seriously, if you're going to go through the trouble and cost of

building rolled or flat-plate caps, use LDPE or PP as your dielectric

material. Period. Not PVC, PC, Mylar, Kapton, G-10, Phenolic, PMMA, CAB,

... well you get the picture!

Quite honestly, if for some reason you can't find/use LDPE or PP, then

you probably ought to consider staying with SW caps - you'll get similar

performance less expensively and with a whole lot less work.

Safe cappin' to you, Roy!

--- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: secondary hieght

Date: Tue, 17 Feb 1998 06:51:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 16 Feb 1998 22:51:05 -0800

Subject: secondary hieght

greetings everyone,

upon completion of my secondary, what's a good starting place in regard

to the hieght of the secondary? ie. is the first turn of the secondary

equal to the first wind of the primary? looking for a ballpark place to

start out.

Thanks,

Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Tue, 17 Feb 1998 07:25:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Mon, 16 Feb 1998 23:25:44 -0800 (PST)

Subject: Re: Capacitor Wars...

On Tue, 17 Feb 1998, Tesla-2 wrote:

> Roy,

>

> Only if you want lossy caps that will heat up and self-destruct. :^)

>

> Seriously, if you're going to go through the trouble and cost of

> building rolled or flat-plate caps, use LDPE or PP as your dielectric

> material. Period. Not PVC, PC, Mylar, Kapton, G-10, Phenolic, PMMA, CAB,

> ... well you get the picture!

>

> Quite honestly, if for some reason you can't find/use LDPE or PP, then

> you probably ought to consider staying with SW caps - you'll get similar

> performance less expensively and with a whole lot less work.

>

> Safe cappin' to you, Roy!

>

> --- Bert --

*ahem*

Are we indulging in just a bit of hyperbole here? While

poly does have the lovely loss factor of 0.001, some of the

materials you listed above have loss factors of < 10.

While I would not search out, or even seek to buy these in

place of poly, I most certainly would not throw them away if

I got them free. Consider kapton: Yes, its loss factor is

on the order of 3. but the dielectric constant is 3, and with

the superior strength - you can make a pretty compact cap, even

if you cannot use it for long runs.

Saltwater caps (at least in glass) have a loss of something like

80 - *really* disgusting.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: old performance record shattered !

Date: Tue, 17 Feb 1998 13:52:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 17 Feb 1998 07:52:29 -0600

Subject: old performance record shattered !

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have often wondered just what the limits of a four inch coil might be (streamer

length). last night I added more top load, about a 40% increase. My total capacitance

is .026 uf. I tipically produce 48"- 52" streamers last night I hit a good solid

60". I had to move my original target due to continous arcing. My capacitance will

allow even more top load ! There still is no crona at or arround the windings of

the secondary . I will continue to experiment. I never expected to produce streamers

this long.

Paul M

Co consperitor Bill the arcstarter

--

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary hieght

Date: Tue, 17 Feb 1998 15:05:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 17 Feb 1998 07:05:29 -0800

Subject: Re: secondary hieght

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Mon, 16 Feb 1998 22:51:05 -0800

> Subject: secondary hieght

>

> greetings everyone,

>

> upon completion of my secondary, what's a good starting place in regard

> to the hieght of the secondary? ie. is the first turn of the secondary

> equal to the first wind of the primary? looking for a ballpark place to

> start out.

>

> Thanks,

> Walt Kulpa

>

Walt,

Try first running with the bottom of the secondary winding a couple of

inches above the plane of the primary (if a flat spiral) or 3-6" higher

than the bottom-most turn if your primary is an inverted cone. Make sure

that you get your coil in-tune by running at lower power levels. Once

your coil's in tune, you can then begin increasing the power level in a

darkenned room, carefully looking for any signs of excessive corona or

arcing.

If you can run at full power without seeing any arcing (particulalry

from the top of the secondary winding to a lower part of your

secondary), you can then begin lowering the secondary to increase

coupling. If you get sparks on the secondary winding, you've gone too

far, and will need to raise the secondary a bit...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Tue, 17 Feb 1998 15:16:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 17 Feb 1998 07:16:00 -0800

Subject: Re: Capacitor Wars...

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Mon, 16 Feb 1998 23:25:44 -0800 (PST)

> Subject: Re: Capacitor Wars...

>

> On Tue, 17 Feb 1998, Tesla-2 wrote:

>

> > Roy,

> >

> > Only if you want lossy caps that will heat up and self-destruct. :^)

> >

> > Seriously, if you're going to go through the trouble and cost of

> > building rolled or flat-plate caps, use LDPE or PP as your dielectric

> > material. Period. Not PVC, PC, Mylar, Kapton, G-10, Phenolic, PMMA, CAB,

> > ... well you get the picture!

> >

> > Quite honestly, if for some reason you can't find/use LDPE or PP, then

> > you probably ought to consider staying with SW caps - you'll get similar

> > performance less expensively and with a whole lot less work.

> >

> > Safe cappin' to you, Roy!

> >

> > --- Bert --

> *ahem*

> Are we indulging in just a bit of hyperbole here? While

> poly does have the lovely loss factor of 0.001, some of the

> materials you listed above have loss factors of < 10.

> While I would not search out, or even seek to buy these in

> place of poly, I most certainly would not throw them away if

> I got them free. Consider kapton: Yes, its loss factor is

> on the order of 3. but the diaelectric constant is 3, and with

> the superior strength - you can make a pretty compact cap, even

> if you cannot use it for long runs.

>

> Saltwater caps (at least in glass) have a loss of something like

> 80 - *really* disgusting.

>

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

Only a slight bit of hyperbole - :^) Coilers have lucked into freebies

of Teflon, Kapton, etc. Teflon doesn't work very well since it's porous.

Finding Kapton in usable widths at reasonable cost may be a considerable

challenge.

BTW, if you've got some free Kapton film in 12" width rolls, I'm willing

to take it off your hands. Even free Mylar film...  :^)

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Tue, 17 Feb 1998 17:10:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chip Atkinson <chip@pupman.com>

Date: Tue, 17 Feb 1998 10:10:53 -0700 (MST)

Subject: Re: Capacitor Wars...

FYI, PP is quite stiff too.  I use a lathe with a low gear to roll the PP

caps and it's all I can do to get them less than 6" so they fit in the

case.

Chip

(I bought the PP because I had no choice in materials at that time)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Chip Atkinson

http://www.pupman.com

--- If I can't fix it, I can fix it so it can't be fixed --

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

On Mon, 16 Feb 1998, Tesla-2 wrote:

>

> However HDPE is quite stiff and would not roll up very well

> so is not suitable for rolled caps. LDPE is soft and supple

> and rolls up very well. HDPE would be excellent for flat

> plate caps.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: logistics

Cc:

Bcc:

X-Attachments:

At 09:03 PM 2/16/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Mon, 16 Feb 1998 13:03:53 -0800

>Subject: logistics

>

>I'm just starting on my secondary and planning ahead. i have a small

>wooden cabinet (24"x24"x 34" high - on wheels) on which i will be setting

>my coil. I've been thinking about storing the transformer, caps, all the

>other components on the shelving located within the cabinet box. I could

>then fire the system up at the end of a long extension cord - my concern

>about this is won't be able to see the spark gaps and my plug in wire

>seems like it could be in proximity of a strike.

>

>Next thought. I can stuff transformer and other components in a box

>and run a piece of wire 15' or so over to the coil, this seems the most

>reasonable.

>

>My question is: what type of proximity is generally used between

>components? What components are generally boxed together? is each

>component transformer, spark gap and capacitor a separate module in a

>separate box? with a 12/30 neon and rolled poly cap, what kind of

>distance will my components (and myself) need to be away to operate

>safely?

Walt,

The tank circuit components (primary, main cap and spark gap)

must be close together and interconnected with heavy conductors.

However your transformers, safety gap and RF chokes can be

located some distance away as these are only handling 60hz.

And the wiring connecting these to the tank circuit need

not be heavy as you are passing a couple hundred ma at most.

Good insulation would be more important.

>

>One other question is that I haven't seen anything on a disconnect means

>(on/off switch) and over-current protection (fusing),  between the

>transformer and the wall socket. I am assuming that it exists as a switch

>and a fuse block or as a one unit circuit breaker box mounted somewhere.

>I have considered using a 24v control circuit to run the neon while the

>circuit is closed and then to short my capacitor to ground when the

>circuit is de-energized - is this more work than necessary?. what good

>ideas are out there in regard to control circuits? do they pose a

>problem?

>

>

A low voltage control circuit is not really necessary for neons

but would be a good idea for larger transformers. Shorting your

cap to ground is not necessary. In most TC arrangements the cap

is discharged through the transformer winding as soon as power

is removed. Also consider that anything you attach to the capacitor

terminals becomes part of the tank circuit.

I dont recommend the use of circuit breakers. I prefer fuses.

You may want a slow blow fuse to the input of your variac

to keep it from popping on power up. Variacs often have large

initial current draws until the core energizes.

For a on/off switch use a heavy duty switch rated for the current.

I built a control box that houses my variac, fusing, AC line filters,

metering, and a DC power supply. The input to the box is fused with

a 20 amp slow blow fuse. The variac output is fused based on the

expected current draw of the neons Im using. The DC power supply

was added to power the cooling fan on my old static gap and has

its own separate fuse.

The control box contains a key switch so that the power can never be

turned on accidently and contains power on indicators so I KNOW when

the power is on.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: old performance record shattered !

Cc:

Bcc:

X-Attachments:

At 01:52 PM 2/17/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Tue, 17 Feb 1998 07:52:29 -0600

>Subject: old performance record shattered !

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>

>I have often wondered just what the limits of a four inch coil might be

>(streamer length). Last night I added more top load, about a 40% increase.

>My total capacitance is .026 uf. I typically produce 48"- 52" streamers.

>Last night I hit a good solid 60". I had to move my original target due to

>continous arcing. My capacitance will allow even more top load! There still is no >corona at or around the windings of the secondary . I will continue to experiment.

>I never expected to produce streamers this long.

>

>Paul M

>

>Co conspirator Bill the arcstarter

>

Paul,

You and Bill have done very well. You have surpassed my record of 54 inches

on a 4 inch secondary. Well done. Every time I think I have things maxed

out I find a way to squeeze out a couple more inches. It took a long time

for me to reach 48 inches but once I passed that mark and started to get

streamers in excess of 50 inches I started to think that 5 foot streamers

might be possible. Now Im sure of it.

As you are finding out bigger top loads result in longer streamers.

Not only does the larger toroid result in a more efficient lower

frequency but it allows more primary inductance to be tapped in.

This improves gap operation. Also the higher capacitance serves

to increase streamer current. This helps to "incinerate the air",

creating a stronger longer arc channel.

What size of toroid are you using now?

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Capacitor Wars...

Date: Tue, 17 Feb 1998 21:24:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Tue, 17 Feb 1998 15:24:52 -0600

Subject: RE: Capacitor Wars...

Hello,

I've seen LDPE film in clear, green tinted and black.  Are the tinted films bet

ter or worse for Tesla caps?

Thanks,

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Capacitor Wars...

Date: Tue, 17 Feb 1998 23:32:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 17 Feb 1998 15:32:04 -0800 (PST)

Subject: RE: Capacitor Wars...

On 17 Feb, Tesla-2 wrote:

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Hello,

>

> I've seen LDPE film in clear, green tinted and black.  Are the tinted films

> better or worse for Tesla caps?

While not set in stone, you can consider it a good idea to avoid

black, as it is often colored by using carbon. [but then,

my 6" coil uses a black HDPE form, with no apparent problems]

Uncertain about the green. I have been looking at it as well.

The local Home Depot has 'Pergo underlay' poly in 6mil thickness.

This is tinted green

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Wars...

Date: Wed, 18 Feb 1998 01:08:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Tue, 17 Feb 1998 20:08:00 EST

Subject: Re: Capacitor Wars...

Roy,

When in doubt, go with clear.  The dyes may adversely affect the dielectric

properties.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Capacitor Wars...

Cc:

Bcc:

X-Attachments:

At 09:24 PM 2/17/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Tue, 17 Feb 1998 15:24:52 -0600

>Subject: RE: Capacitor Wars...

>

>Hello,

>

>I've seen LDPE film in clear, green tinted and black.  Are the tinted films

>better or worse for Tesla caps?

>

>Thanks,

>Roy Minnich

>

Stick with the clear (actually kind of translucent) LDPE.

Colored films may have undesirable properties. For instance

carbon is a common coloring agent in black plastics.

Some colored films may be just fine but you have no

way to know that in advance. Safer to stick with the clear.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: First Light...and problems

Date: Wed, 18 Feb 1998 02:24:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Matt P." <ch038map@mode.lanl.k12.nm.us>

Date: Tue, 17 Feb 1998 19:24:54 -0700 (MST)

Subject: First Light...and problems

A few days ago I finally got all of my stuff in place and for the

first time fired up my coil. I got 6 in. sparks from it and was extremely

excited that it actually worked. Especially since I am using about the

worst materials that I can. But, there are a few problems that are

bothering me, so after much frustration I decided to take it to the

experts- you guys.

1. My primary keeps arcing to my secondary (I posted the specs

below) I coated the secondary with quite a few coats of polyurethane, but

the problem persists. Also, I added a few more turns than I needed when I

was making my primary, because I like to make sure I have enough of

whatever I'm doing...its a  habit :-), can this be contributing to my

arcing problem?

2. I'm using saltwater caps, and I noticed a corona on the inside

of the bottles ( I assumed this is normal), but on some of them there is a

REALLY heavy corona, have these been punctured or defective? should I

throw them out?

3. Is there some way I can figure out how wide my gap should be?

can it be too wide? does it really matter how wide it is? Will I ever quit

asking these stupid questions?

Here are the specs for my coil as now:

Transformer- NST 7Kv 30mA

Secondary- 3.25" by 15" wound with #24 magnent wire.

Primary- Helical 8" Diameter, 15 turns of #12 wire tapped at about 10

turns

Spark gap- very basic: two bolts with thier heads facing each

other....safety gap is the same thing...only wider.

Caps- 12 saltwater/beer bottle type- I don't have any idea how much

capacitance

Chokes- none yet, I'm going to wind some 1.5" pvc with some extra #24

magnet wire, would this be good enough?

Toroid- strofoam wreath wrapped with aluminum tape- od= 12" id=8.5" by

1.75"

I hope I made sense, I'm typing this in a hurry, thanks for any help.

Matt P.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary design

Date: Wed, 18 Feb 1998 05:19:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Tue, 17 Feb 1998 21:19:57 -0800 (PST)

Subject: Secondary design

Excitement mounting...

Getting closer every day... got 15 Kv xfrmr, safety gap, air core

chokes wound, static spark gap w/11 electrodes, 50 feet of 5/16" copper tube

for primary, and a 4" X 24" acrylic coil form. My question for today:

Is there an optimum winding length for a 4 inch diameter secondary, using

#24 AWG wire? Should I go for a higher or lower resonating frequency? (More or

fewer windings?) I have it narrowed down to 17" - 21".  Also - Should I get

some #22 wire, or will the #24 perform adequately?  I want the system to

have optimum performance, and getting the wire is not a problem. I know a

surplus dealer who has tons of wire, by the pound...

Thanks for the help.

danmurphy@1stnetusa.com

President and CEO, Murphy's Home for Sober, Celibate and Surly Bachelors.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: First Light...and problems

Date: Wed, 18 Feb 1998 05:42:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Tue, 17 Feb 1998 23:42:45 -0600

Subject: Re: First Light...and problems

Tesla-2 wrote:

>

> From: "Matt P." <ch038map@mode.lanl.k12.nm.us>

> Date: Tue, 17 Feb 1998 19:24:54 -0700 (MST)

> Subject: First Light...and problems

>

>         A few days ago I finally got all of my stuff in place and for the

> first time fired up my coil. I got 6 in. sparks from it and was extremely

> excited that it actually worked. Especially since I am using about the

> worst materials that I can. But, there are a few problems that are

> bothering me, so after much frustration I decided to take it to the

> experts- you guys.

>

>         1. My primary keeps arcing to my secondary (I posted the specs

> below) I coated the secondary with quite a few coats of polyurethane, but

> the problem persists. Also, I added a few more turns than I needed when I

> was making my primary, because I like to make sure I have enough of

> whatever I'm doing...its a  habit :-), can this be contributing to my

> arcing problem?

>         2. I'm using saltwater caps, and I noticed a corona on the inside

> of the bottles ( I assumed this is normal), but on some of them there is a

> REALLY heavy corona, have these been punctured or defective? should I

> throw them out?

The saltwater caps are going to have a lot of corona, because they are

lossy. As for trashing the ones that are giving off large amount of

corona you may want to replace them but it is not necessary because they

are still working.

>         3. Is there some way I can figure out how wide my gap should be?

> can it be too wide? does it really matter how wide it is? Will I ever quit

> asking these stupid questions?

Which gap, the safety gap or the main gap. If it is the safety gap just

set it right out of firing range when the tranny is not hooked to the

caps. Your main gap will vary depending on the voltage you are using.

But it was recommended to me that I use a series gap with no more than a

.25" total gap distance.

>

> Here are the specs for my coil as now:

>

> Transformer- NST 7Kv 30mA

This will work fine but in the future I would recommend at step up to

either more milliamps or a larger output voltage. I tested a coil, it

was its first run and got about 2 or 3 15" streamers at the same time.

The coil is powered by a 7.5kv@60ma.

>

> Secondary- 3.25" by 15" wound with #24 magnent wire.

>

> Primary- Helical 8" Diameter, 15 turns of #12 wire tapped at about 10

> turns

Go with a saucer shaped primary with a 15-30 degree angle at 15 turns,

allow a minimum of 1 inch between primary and secondary. I would

recoomend around .25" spacing between turns.

>

> Spark gap- very basic: two bolts with thier heads facing each

> other....safety gap is the same thing...only wider.

Try an adjustible series gap such as richard quicks cylindrical gap

using copper pipe or tube. It should boost performance somewhat.

>

> Caps- 12 saltwater/beer bottle type- I don't have any idea how much

> capacitance

These work great for their price but they are not anywhere as good as

poly caps. This area will give you probably the most dramatic results if

use swap to a poly cap.

>

> Chokes- none yet, I'm going to wind some 1.5" pvc with some extra #24

> magnet wire, would this be good enough?

I personally don't use chokes on the low voltage trannies such as 7kv.

I just make sure that I have a safety gap.

>

> Toroid- strofoam wreath wrapped with aluminum tape- od= 12" id=8.5" by

> 1.75"

This sounds fine to me.

>

> I hope I made sense, I'm typing this in a hurry, thanks for any help.

>

> Matt P.

You are on the right track, because the coil does work. All that is left

is fine tuning and upgrading a few areas.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Secondary design

Date: Wed, 18 Feb 1998 06:06:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Wed, 18 Feb 1998 00:06:24 -0600

Subject: Re: Secondary design

>Is there an optimum winding length for a 4 inch diameter secondary, using

>#24 AWG wire? Should I go for a higher or lower resonating frequency? (More

or

Anywhere from 20"-26" would be ok.  I chose 23" myself and get good

performance.  Lower frequency will give you better sparks, so its up to you.

>fewer windings?) I have it narrowed down to 17" - 21".  Also - Should I get

>some #22 wire, or will the #24 perform adequately?  I want the system to

>have optimum performance, and getting the wire is not a problem. I know a

Actually, #24 is BETTER for 4" coils, #22 being used for 6" coils.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Yet another cap question...

Date: Wed, 18 Feb 1998 06:19:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Tue, 17 Feb 1998 22:19:20 -0800 (PST)

Subject: Yet another cap question...

You guys must have a remarkable amount of patience, answering all these

questions from us "newbie" coil-heads...

And speaking of which, here's another: Can I put all three (or 4) of my

capacitors in the same tub of xfrmr oil? Any precautions I need to take,

what's the best way to separate them, etc?

Waldo says if I wire all 3 caps together in series, I might as well build one

big cap 3 times the size - He says its the same thing, either way. I politely

disagreed, but don't know enough about electronics to know why I did. Just a

feeling I had... So, who's right?

dantheman

<danmurphy@1stnetusa.com>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Yet another cap question...

Date: Wed, 18 Feb 1998 12:46:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Wed, 18 Feb 1998 04:46:59 -0800

Subject: Re: Yet another cap question...

Organization: EES

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Tue, 17 Feb 1998 22:19:20 -0800 (PST)

> Subject: Yet another cap question...

> Waldo says if I wire all 3 caps together in series, I might as well build one

> big cap 3 times the size - He says its the same thing, either way. I politely

> disagreed, but don't know enough about electronics to know why I did. Just a

> feeling I had... So, who's right?

>                                                 dantheman

>                                                 <danmurphy@1stnetusa.com>

>

>

Dan,

I'm new to the list, so I'm not quite sure what your goal is.  However,

I can tell you that putting three caps in SERIES (assuming that they are

identical physically and electrically) will triple the breakdown voltage

of the unit as a whole. The other effect is to cut the overall

capacitance to one third the value of the individual caps.

Regarding multiple vs. single caps: It can sometimes be impractical (for

reasons of dollars or size) to build a single unit to achieve the

capacitance and breakdown voltage your seeking. I see no reason why

(other than interconnection resistance and possible arcing sources)

multiple caps are a problem, as long as you calculate the TOTAL

capacitance and breakdown voltage correctly. Oops!...You may need to

consider the characteristics of the diaelectric material when making

your choice. For that, you should ask someone on the list who has

experimented with different materials for their advise.

Good luck,

Scotty

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: old performance record shattered !

Date: Wed, 18 Feb 1998 13:13:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 18 Feb 1998 07:13:46 -0600

Subject: Re: Re: old performance record shattered !

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

I have a toroid at the bottom of the stack is made from 4" diameter dryer duct,

it is approximatly 18" in diameter. Stacked on top of that is a toroid made from

7" diameter dryer duct covered with aluminumized tape. This toroid is 28" in diameter.

Just for the heck on it I placed a third toroid made from 6" diameter duct on top

of that one. The third is rectangular it measures 18" in width and is 28" in length.

This is only temperary. I hope to attach the 4" to the 7" by means of aluminumized

tape. What I would really like to do is find a single piece toroid or material to

build one 12" diameter, aluminum tube about 35" in diameter. Any Ideas ?

The aluminum on the 7" toroid is actually starting to oxidize. I believe this is

due to the heat generated by the streamers and the ozone present at the surface.

Hey maybe I could go to 6 kw now.

Catch you later,

Awaiting your reply

Paul & the arcstarter Bill

---

Einigkeit und Freiheit

>From: Mike Hammer <mhammer@misslink.net>

>Date: Tue, 17 Feb 1998 12:52:38 -0600

>Subject: Re: old performance record shattered !

>

>At 01:52 PM 2/17/98 GMT, you wrote:

>>From: fleischwulf@mailexcite.com

>>Date: Tue, 17 Feb 1998 07:52:29 -0600

>>Subject: old performance record shattered !

>>Organization: MailExcite  (http://www.mailexcite.com)

>>

>>Coilers,

>>

>>I have often wondered just what the limits of a four inch coil might be

>>(streamer length). Last night I added more top load, about a 40% increase.

>>My total capacitance is .026 uf. I typically produce 48"- 52" streamers.

>>Last night I hit a good solid 60". I had to move my original target due to

>>continous arcing. My capacitance will allow even more top load! There still

>is no >corona at or around the windings of the secondary . I will continue

>to experiment.

>>I never expected to produce streamers this long.

>>

>>Paul M

>>

>>Co conspirator Bill the arcstarter

>>

>

>Paul,

>

>You and Bill have done very well. You have surpassed my record of 54 inches

>on a 4 inch secondary. Well done. Every time I think I have things maxed

>out I find a way to squeeze out a couple more inches. I took a long time

>for me to reach 48 inches but once I passed that mark and started to get

>streamers in excess of 50 inches I started to think that 5 foot streamers

>might be possible. Now Im sure of it.

>

>As you are finding out bigger top loads result in longer streamers.

>Not only does the larger toroid result in a more efficient lower

>frequency but it allows more primary inductance to be tapped in.

>This improves gap operation. Also the higher capacitance serves

>to increase streamer current. This helps to "incinerate the air",

>creating a stronger longer arc channel.

>

>What size of toroid are you using now?

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Hi Everybody

Date: Wed, 18 Feb 1998 13:55:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 18 Feb 1998 05:55:45 -0800 (PST)

Subject: Re: Hi Everybody

To Anderson:

At 08:28 PM 2/17/98 -0800, you wrote:

I'm a big fan of Tesla but a newbee to coils. What is the best thing for me

to do to learn how to make a coil?

Answer: Lots of research! I had no idea, when I typed "Tesla Coil Design" into

the search engine, what I was getting into. There is TONS of information out

there for us novices, plus the guys on this list server are more than helpful.

(Thanks, guys... you know who you are...)

A great place to finds lots of info is: www.nic.funet.fi/pub  -- click on sci,

then electric, then tesla, then misc. download the file "coilbld.zip"  It has

instructions and graphic illustrations. Also, in the same site, after clicking

on tesla, click on mailist, then on tesla-2.  Download the 10 or so archive

files you find there. There is around a megabyte, compressed. These files con-

tain all the messages from this list server for the past year or so.  After

pk-unzipping them and reading them all, yu'l be a expurt telsa coylr lik i

am. (2 weaks ago i cudn't evun spel tulsa!) As Bert says, "Safe coiling!"

madmandan

<danmurphy@1stnetusa.com>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hello its me again

Date: Wed, 18 Feb 1998 13:58:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 18 Feb 1998 05:58:30 -0800 (PST)

Subject: Re: hello its me again

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: Re: hello its me again

On Sat, 14 Feb 1998, Ross Ogden wrote:

> i've now found a shop with a 10kv 37 ma neon is it too powerful? is it worth

> 100?. I am not very old in fact to young to even be coiling so i cant

> afford anything expensive. What should i do ?

>

> DOES ANY ONE HAVE SOME OIL BURNER IGTIN XFORMERS LYING AROUND ?

> A NEON WOULD BE NICE TOO

An obvious ploy for sympathy and a free xformr - good luck, Ross.  We

shopped around and found (at a surplus electronics yard - like a car junkyard,

only with dead oscilloscopes) a used 15 Kv, 30 ma neon for $30. Probably paid

too much. I've seen similar advertised on this list for $15. (Will try to

find the return address of that list and send it to you.)  Also - have gotten

good results by going to a neon sign shop and telling them "It's for my son's

science project..." In your case, you could say, "It's for MY science project"

(which isn't lying, now, is it?)

madmandan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Yet another cap question...

Date: Wed, 18 Feb 1998 15:09:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 18 Feb 1998 07:09:41 -0800

Subject: Re: Yet another cap question...

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Tue, 17 Feb 1998 22:19:20 -0800 (PST)

> Subject: Yet another cap question...

>

> You guys must have a remarkable amount of patience, answering all these

> questions from us "newbie" coil-heads...

>

> And speaking of which, here's another: Can I put all three (or 4) of my

> capacitors in the same tub of xfrmr oil? Any precautions I need to take,

> what's the best way to separate them, etc?

>

> Waldo says if I wire all 3 caps together in series, I might as well build one

> big cap 3 times the size - He says its the same thing, either way. I politely

> disagreed, but don't know enough about electronics to know why I did. Just a

> feeling I had... So, who's right?

>                                                 dantheman

>                                                 <danmurphy@1stnetusa.com>

Dan,

Tesla coil capacitors operate in a very demanding environment. They are

run at high voltages, subjected to virtually full voltage reversal,

undergo high RF current flow (typically hundreds or even thousands of

amps)... at rates of hundreds of times each second! It takes very robust

construction, a low-loss dielectric, and prevention of corona formation

to give them any reasonable chance of long-term survival.

You could build them all in the same container, and as long as each

section of the capacitor was identically constructed, you can also

connect them in series to increase standoff voltage. Just separate them

so that there's no chance of flashover between modules, using additional

sheets of LDPE. For example, if you built each section to be 0.1 uF,

using 8 mils of LDPE (at least 2 layers of dielectric between pairs of

plates), and you then connected 10 of these sections in series, you'd

have the equivalent of a 0.01 uF cap with a total of 80 mils of LDPE

that would be substantially more reliable than simply using 80 mils of

dielectric in a single cap. Although a "lumped parameter" model says

they are the same, a more detailed view of the internal stresses shows a

big difference...

Why??

Because the total electric field stress is evenly distributed amongstthe

individual sections, and the the stresses seen at the edges of

the capacitor plates would be about 1/10 that seen in a single cap

operated at the same total voltage. The chances are quite high that a

single cap will eventually fail due to corona formation, and resulting

LDPE and oil breakdown damage, leading to dielectric breakdown. Corona

is a cap killer - if you have it your cap is eventually doomed! That's

why it's essential to remove as much air as possible and substitute oil.

However, even oil will break down at the very high field levels seen

along the plate edges, liberating gas and permitting corona formation,

leading to irreversible damage of the oil and LDPE in the immediate

vicinity. However, a cap made from 10 identical sections will have 1/10

the electric field stress at the plate edges. If you can keep the

voltage stress per module to a maximum of 2-3 kV, you can prevent corona

formation and thus significantly extend the life of your cap. This is

one (of many) hard-won ecrets that those who are in the business of

making HV capacitors use to better insure their survival. You can choose

to ignore this secret and potentially (NPI) encounter early failures...

or use it for longer cap life.

Safe cappin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Light...and problems

Cc:

Bcc:

X-Attachments:

At 02:24 AM 2/18/98 GMT, you wrote:

>From: "Matt P." <ch038map@mode.lanl.k12.nm.us>

>Date: Tue, 17 Feb 1998 19:24:54 -0700 (MST)

>Subject: First Light...and problems

>

>

>        A few days ago I finally got all of my stuff in place and for the

>first time fired up my coil. I got 6 in. sparks from it and was extremely

>excited that it actually worked. Especially since I am using about the

>worst materials that I can. But, there are a few problems that are

>bothering me, so after much frustration I decided to take it to the

>experts- you guys.

>

>        1. My primary keeps arcing to my secondary (I posted the specs

>below) I coated the secondary with quite a few coats of polyurethane, but

>the problem persists. Also, I added a few more turns than I needed when I

>was making my primary, because I like to make sure I have enough of

>whatever I'm doing...its a  habit :-), can this be contributing to my

>arcing problem?

Primary secondary arcing is almost always a case of overcoupling.

Just raise up your secondary away from the primary an inch

at a time until the arcing stops.

Overcoupling is common in helical (solenoid) type primary coils.

Unused primary turns can bump up the voltage due to autotransformer

action. But the cure is the same. Reduce the coupling by moving

the secondary away from the primary.

>        2. I'm using saltwater caps, and I noticed a corona on the inside

>of the bottles ( I assumed this is normal), but on some of them there is a

>REALLY heavy corona, have these been punctured or defective? should I

>throw them out?

SW bottle caps will show lots of corona. Its just the nature

of the beast. Note that this corona represents power loss.

Another reason to change over to poly caps in oil ASAP.

>        3. Is there some way I can figure out how wide my gap should be?

>can it be too wide? does it really matter how wide it is? Will I ever quit

>asking these stupid questions?

The main gap distance depends mainly on the supply voltage.

At 7kv you will need a smaller total gap than someone running

12kv or 15kv. Ive run .150 inch of total gap with a 7500V/30ma

neon before with good results. I wouldnt think you would want

much more than this.

Running too wide of a main gap will lead to cap failure and insulation

breakdown in your neon.

I would consider replacing your single bolt spark gap with a series

static gap like the popular cylinder gap. Its commonly referred

to rather erroneously as a RQ gap.

With a very good gap you can take advantage of the extra coupling

your helical primary offers. The better your gap the tighter you

can run your coupling. At least up to the insulation limit of your

secondary.

Yes you can wind RF chokes on 2 inch PVC pipe with AWG 24.

I recommend using AWG 26 or 28 but if all you have is 24 then use it.

You will most likely need to insert some resistors in series

with these chokes to kill their Q and prevent them from ringing

up like mini Tesla coils. Air core chokes alone without any series

resistance will often do more harm than good.

Resisitor value is not critical. Anything from 300-1500 ohms and

10 or more watts dissipation.

<SNIP specs>

>

>I hope I made sense, I'm typing this in a hurry, thanks for any help.

>

Makes sense to me. :)

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Yet another cap question...

Cc:

Bcc:

X-Attachments:

At 06:19 AM 2/18/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Tue, 17 Feb 1998 22:19:20 -0800 (PST)

>Subject: Yet another cap question...

>

>You guys must have a remarkable amount of patience, answering all these

>questions from us "newbie" coil-heads...

Patience? Who me? Its never been one of my virtues.

>

>And speaking of which, here's another: Can I put all three (or 4) of my

>capacitors in the same tub of xfrmr oil? Any precautions I need to take,

>what's the best way to separate them, etc?

Yep you sure can. My plate cap has 6 series connected caps in the same tank

of oil. You can even wind series connected rolled caps ON THE SAME ROLL.

Check out Fr. Tom McGahees file "multirol.txt" on Dave Bronakowski's

archive site. The URL is: http://www.velocity.net/~djb/files.shtml

Only precautions are adequate spacing to prevent arcing.

>

>Waldo says if I wire all 3 caps together in series, I might as well build one

>big cap 3 times the size - He says its the same thing, either way. I politely

>disagreed, but don't know enough about electronics to know why I did. Just a

>feeling I had... So, who's right?

>

Well Waldo is wrong this time. By spreading the load over several series

connected cap you get a real boost in reliability. Its much easier

to build three 5kv caps than it is one 15kv cap.

The problem starts with corona. Any one cap seeing more than 12kv

will likely have some corona form on the plate edges. Even under oil.

This is almost guaranteed to happen at 15kv. If you have corona form

it wont matter how thick your poly dielectric is. It WILL eventually

fail. But if you apply that same 15kv to a series string of 3 IDENTICAL

caps then each cap sees only 5kv and corona will not form.

So if you want a cap with 60 mils of poly for 15kv use then string

three 20 mil caps in series. If I was making caps for 15kv ( I use 12kv)

I would build 3 series caps each with 4 X 6mil (24 mils) of poly.

The 3 series caps would have the equivalent of 72 mils. That should

be very reliable at 15kv.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Re: old performance record shattered !

Cc:

Bcc:

X-Attachments:

At 01:13 PM 2/18/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Wed, 18 Feb 1998 07:13:46 -0600

>Subject: Re: Re: old performance record shattered !

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Mike,

>  I have a toroid at the bottom of the stack is made from 4" diameter dryer duct,

>it is approximatly 18" in diameter. Stacked on top of that is a toroid made from

>7" diameter dryer duct covered with aluminumized tape. This toroid is 28" in diameter.

>Just for the heck on it I placed a third toroid made from 6" diameter duct on top

>of that one. The third is rectangular it measures 18" in width and is 28" in length.

>This is only temperary. I hope to attach the 4" to the 7" by means of aluminumized

>tape. What I would really like to do is find a single piece toroid or material

>to build one 12" diameter, aluminum tube about 35" in diameter.

>Any Ideas ?

>

>Paul & the arcstarter Bill

>---

Paul,

Try Mcmaster Carr. They sell almost everything. They prefer to sell

only to businesses and dont really like dealing with individuals

but they have never turned down any of my orders. The trouble is finding

a catalog. I have a catalog from my previous employer. They almost

always refuse to send catalogs to individuals.

They have outlets in nearly every major city. I order out of Chicago

and normally get next day service.

Im thinking about getting some 8 inch duct myself for a 24 X 8 toroid.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Want a neon?  Re: hello its me again

Date: Wed, 18 Feb 1998 23:00:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Wed, 18 Feb 1998 17:00:53 -0600

Subject: Want a neon?  Re: hello its me again

Well I have a surplus of 9kv 30ma neons.  Send me $11 for shipping and

its yours.

>> found a shop with a 10kv 37 ma neon is it too powerful? is it worth

>> 100?. I am not very old infact to young to even be coiling so i cant

>> afford anything expensive plus. What should i do ?

>>

>> DOES ANY ONE HAVE SOME OIL BURNER IGNITION XFORMERS LYING AROUND ?

>> A NEON WOULD BE NICE TOO

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Want a neon?  Re: hello its me again

Date: Thu, 19 Feb 1998 01:17:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Wed, 18 Feb 1998 19:17:44 -0600

Subject: RE: Want a neon?  Re: hello its me again

Hey Jeff,

If the kid doesn't take it I will.  Let me know what happens.

Thanks,

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Wednesday, February 18, 1998 7:15 PM

To:     Minnich, Roy

Subject:        Want a neon?  Re: hello its me again

From: "Jeff Corr" <corr@enid.com>

Date: Wed, 18 Feb 1998 17:00:53 -0600

Subject: Want a neon?  Re: hello its me again

Well I have a surplus of 9kv 30ma neons.  Send me $11 for shipping and

its yours.

>> found a shop with a 10kv 37 ma neon is it too powerful? is it worth

>> 100>. I am not very old infact to young to even be coiling so i cant

>> afford anything expensive. What should i do ?

>>=20

>> DOES ANY ONE HAVE SOME OIL BURNER IGNITION XFORMERS LYING AROUND ?

>> A NEON WOULD BE NICE TOO

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Yet another cap question...

Date: Thu, 19 Feb 1998 04:38:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 18 Feb 1998 20:38:57 -0800

Subject: Re: Yet another cap question...

<snip>

> > You guys must have a remarkable amount of patience, answering all these

> > questions from us "newbie" coil-heads...

> >

I would like to echo a sentiment of thanks to the wizards who so readily

answer MY naive qestions.

Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: variacs?

Date: Thu, 19 Feb 1998 04:49:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 18 Feb 1998 20:49:55 -0800

Subject: variacs?

I've seen infomrmation on using various welders for a variacs. I would

also imagine that an a-c motor drive would be suitable (although I'm

relatively sure that I won't find one that's settled to the bottom of my

junk box). I'm wondering what"down home remidies" there are for curbing

the flow of voltage? I'm trying to build my first coil on a shoestring

and, I'm a good scrounger.

I'm using a 12kv/30ma to start but would like to build in room for

growth. I can visit the local welding shop for junked out welder parts,

but this seems prety big and cumbersome. what other devices use such

variacs?

Thanks,

Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: archimedes heel

Date: Thu, 19 Feb 1998 06:30:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 18 Feb 1998 22:30:35 -0800

Subject: archimedes heel

greetings everyone,

getting ready to wind my primary and have been trying to work out a good

template for a form. I'm going to use a 1/4" copper pipe primary and a

4.5" pvc secondary. , spacing between the two would be 1" at the first

turn of the primary. rising helix 15/30 degree.

I'm using .25 lexan for the primary form,my spacing is 1/4" between

turns. Now, in forming the inverted helix in copper, I can place all of

the slots used for capturing the copper pipe in the same place on the

plexi and the make each individual form/fin taller and longer than the

previous or I can make each of the plexi form/fins a work of art and

individually cut each notch a bit further back toward the widest part of

the form fin (i think that this messes up the rise).

I'm sure that you guys have already figured this out. May I be so brazen

as to just ask for the answer without trigging the whole thing out? is

there a simple way of doing this?

And thanks for all fo the assistance I've gotten along the way here too!

Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: archimedes heel

Date: Thu, 19 Feb 1998 17:13:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Rick L. Kirby" <rkirby@earthlink.net>

Date: Thu, 19 Feb 1998 09:13:17 -0800

Subject: Re: archimedes heel

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Wed, 18 Feb 1998 22:30:35 -0800

> Subject: archimedes heel

>

> greetings everyone,

>

> getting ready to wind my primary and have been trying to work out a good

> template for a form. I'm going to use a 1/4" copper pipe primary and a

> 4.5" pvc secondary. , spacing between the two would be 1" at the first

> turn of the primary. rising helix 15/30 degree.

> I'm using .25 lexan for the primary form,my spacing is 1/4" between

> turns. Now, in forming the inverted helix in copper, I can place all of

> the slots used for capturing the copper pipe in the same place on the

> plexi and the make each individual form/fin taller and longer than the

> previous or I can make each of the plexi form/fins a work of art and

> individually cut each notch a bit further back toward the widest part of

> the form fin (i think that this messes up the rise).

> I'm sure that you guys have already figured this out. May I be so brazen

> as to just ask for the answer without trigging the whole thing out? is

> there a simple way of doing this?

> And thanks for all fo the assistance I've gotten along the way here too!

> Walt Kulpa

>

Hi Walt,

I to made a pancake pri. .25 copper tubing spaced .25" between turns. I

made 8 identical fins then I drew a 6.5" dia. circle on my base and marked

off 8 equally spaced points around this circle to place the fins, radiating

out from the circle. In order for the spiral of the tubing to be a smooth

even transition it must move out 1/8 of the distance at each spoke/fin.

fin #1 is set even with the drawn circle or 0" out, fin #2 is set 1/8 of the

spacing out from the circle or (.25" divided by 8 = .0312"). The next fin

#3 is set out .062", fin #4 is out .0937", fin #5 is out .125", fin #6 is

out .156", fin #7 is out .187" from the circle. This allows the tubing to

cross the first fin at the .25" point lining up just right with your 2nd

notch. I hope this is clear.

TTYL :)

Rick L. Kirby

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Cross winding arcs, why?

Date: Sat, 21 Feb 1998 06:07:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Fri, 20 Feb 1998 22:07:22 -0800 (PST)

Subject: Cross winding arcs, why?

Hi gang: quick question: I just finished winding a small coil that

I intend to give to a physics teacher friend as a gift. I decided

to test in my (admittedly) large primary. While it did fire off

some nice small sparks, I was getting some *very* nasty bright

white arcs turn-to-turn, anywhere from 1/8 to 1/4 the size of the coil.

What is triggering this? I would doubt overcoupling, but I could be wrong.

Details:

2.25" x 12" x 28awg secondary, I haven't sealed the windings yet.

8.5"id flat primary tapped at about 4

using 0.013 uf cap

Thanks!

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cross winding arcs, why?

Date: Sat, 21 Feb 1998 13:23:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sat, 21 Feb 1998 08:23:48 EST

Subject: Re: Cross winding arcs, why?

Michael,

I calculated your pimary inductance to be 7.07 uH.  Using

F=1/(2*pi*sqrt(LC)), I calculated your primary frequency to be 527.6 kHz.

Now, assuming your secondary winding has a .001 in enamel coating, your

secondary would have 822 turns.  This would be a quarter wave of 484 ft giving

a frequency of 507 kHz.  If the distributed capacitance lowers this frequency

by about 8%, the resulting frequency would be approximately 466 kHz.  If you

have a top load on the coil, the frequency would be lower still.

Using 28g wire gives enough DC resistance to give a low Q value for your

secondary.  This effect causes the tuning to not be very sharp.  In other

words, you can get power transfer to the secondary without being in perfect

tune.  You won't be getting the maximum transfer of power, but enough to get

an output from the secondary.  The problem would seem to be that you are

impressing two frequencies on the secondary(Its resonant frequency and the

primary frequency).  This would result in an interference pattern, much like

the crisscrossing pattern when dropping two objects near each other into

water.  If this is the case, you will be getting points along your secondary

where the voltage is additive and points  where they tend to cancel.  This

would stress the windings and cause breakouts at places other than the top of

the coil where you want them.  The arcs would be from the maximum points to

the points of minimum voltage.

Tesla warns in his patent of the magnifying transmitter(patent no.

1,119,732) that if points of maximum pressure(voltage) are shifted below the

teminal, then a ball of fire(plasma) could break out.  This may be similar to

what is going on with your coil.  If you think this is a valid possiblity, you

can fix the problem by tapping your primary with more turns.  This will lower

the primary frequency and should bring your coil into tune. If your coil has

fewer turns than I estimated, the problem may be something else.

Hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cross winding arcs, why?

Date: Sat, 21 Feb 1998 14:51:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 21 Feb 1998 06:51:42 -0800

Subject: Re: Cross winding arcs, why?

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Fri, 20 Feb 1998 22:07:22 -0800 (PST)

> Subject: Cross winding arcs, why?

>

> Hi gang: quick question: I just finished winding a small coil that

> I intend to give to a physics teacher friend as a gift. I decided

> to test in my (admittedly) large primary. While it did fire off

> some nice small sparks, I was getting some *very* nasty bright

> white arcs turn-to-turn, anywhere from 1/8 to 1/4 the size of the coil.

> What is triggering this? I would doubt overcoupling, but I could be wrong.

> Details:

> 2.25" x 12" x 28awg secondary, I haven't sealed the windings yet.

> 8.5"id flat primary tapped at about 4

> using 0.013 uf cap

>

> Thanks!

>

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

Either over-coupling or over-volting. A small coil can be over-volted if

you attempt to put to much energy into it - the voltage stress exceeds

the insulation capability of the coil, especially if you've got a short

coil. There's also a fairly good chance that you are overcoupled as

well. Basically, the smaller your secondary, the smaller it's self-C,

and the higher its maximum output voltage (all other parameters being

the same).

You should be able to prevent the flashovers by decreasing your main gap

distance. Elevating the secondary may also help.

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cross winding arcs, why?

Cc:

Bcc:

X-Attachments:

At 06:07 AM 2/21/98 GMT, you wrote:

>From: Michael Baumann <baumann@proton.llumc.edu>

>Date: Fri, 20 Feb 1998 22:07:22 -0800 (PST)

>Subject: Cross winding arcs, why?

>

>Hi gang: quick question: I just finished winding a small coil that

>I intend to give to a physics teacher friend as a gift. I decided

>to test in my (admittedly) large primary. While it did fire off

>some nice small sparks, I was getting some *very* nasty bright

>white arcs turn-to-turn, anywhere from 1/8 to 1/4 the size of the coil.

>What is triggering this? I would doubt overcoupling, but I could be wrong.

>Details:

>2.25" x 12" x 28awg secondary, I haven't sealed the windings yet.

>8.5"id flat primary tapped at about 4

>using 0.013 uf cap

>

Michael,

What you observed is called splitting and is most often

a symptom of overcoupling. It could also be from overvolting

the secondary which is possible considering its small size.

A good heavy coat of polyurethane will help here.

If your coil is out of tune somewhat this may also cause

similiar symptoms.

I would recommend reducing the coupling until the splitting

stops and then carefully adjust the tuning. You probably need

some additional primary inductance tapped in. Depends on how

much top load you have. Once the tune is right then increase

the coupling again.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: cap question

Date: Sat, 21 Feb 1998 19:24:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Sat, 21 Feb 1998 11:24:06 -0800

Subject: cap question

greetings everyone,

While I wait for my plastic to come in for rolled caps, I'm working on

some SW capacitors using information gleened from one of the archives

(can't remember which one). It says to wrap beer bottles in tin foil,

etc. etc. and then "connect all of the bolts sticking out together with

wire". I'm assuming that "connect all of the bottles together" means that

I will have something that looks like this when you look down at the six

bottles and that my incoming line lead can be connected to any one of the

bottles?

O----O----O            O = bottle cap

|    |    |            - & | = wire

O    O    O

I have the bottles contained in a wooden box with a piece of aluminum

sheet at the bottom.

Also, I am planning on using "ring type" solderless connectors, and

soldering them once I crimp them down. is this typically how it's done

or is there a better way?

Thanks, Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Caps and mixed drinks?

Date: Sun, 22 Feb 1998 02:49:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sat, 21 Feb 1998 20:49:55 -0600

Subject: Caps and mixed drinks?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

A simple statement:

"Ethyl alchohol has a dielectric constant of 81."

Any takers?  Bragging rights galore to the creater of the first operational "Everclear

Capacitor"!

Perhaps "InCapacitator" would be more appropriate?

-Bill the arcstarter

Working on constructing the world's first "Cognac Commutator"...

Vodka Voltmeter?

Kahlua Capacitor? Coiler? Quencher!

Tequila Tripler? Three-phase?

Bacardi Bypass?

Bourbon Ballast?

Rum Resonator?

Rotary RonRico?

Vermouth Variac?

(help me out here, guys)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: crazy carpet capacitors

Date: Sun, 22 Feb 1998 03:19:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sat, 21 Feb 1998 19:19:51 -0800

Subject: Re: crazy carpet capacitors

Anyone out there try using crazy carpet as a dielectric material. I'm

working one right now, if anyone has tryed this I would be curious of your

results. If it works it will be a very inexpensive capacitor

Tesla-2 wrote:

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Sat, 21 Feb 1998 11:24:06 -0800

> Subject: cap question

>

> greetings everyone,

>

> While I wait for my plastic to come in for rolled caps, I'm working on

> some SW capacitors using information gleened from one of the archives

> (can't remember which one). It says to wrap beer bottles in tin foil,

> etc. etc. and then "connect all of the bolts sticking out together with

> wire". I'm assuming that "connect all of the bottles together" means that

> I will have something that looks like this when you look down at the six

> bottles and that my incoming line lead can be connected to any one of the

> bottles?

>

>     O----O----O            O = bottle cap

>     |    |    |            - & | = wire

>     O    O    O

>

> I have the bottles contained in a wooden box with a piece of aluminum

> sheet at the bottom.

>

> Also, I am planning on using "ring type" solderless connectors, and

> soldering them once I crimp them down. is this typically how it's done

> or is there a better way?

>

> Thanks, Walt Kulpa

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Caps and mixed drinks?

Date: Sun, 22 Feb 1998 05:57:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobbyist@geeky1.ebtech.net>

Date: Sun, 22 Feb 1998 00:57:34 -0500 (EST)

Subject: Re: Caps and mixed drinks?

On Sun, 22 Feb 1998, Tesla-2 wrote:

> Vermouth Variac?

> (help me out here, guys)

>

Isopropyl Inductor?:)  (Yes, I am fully aware that isopropyl alcohol is

toxic.)  TTYL!

---

Paul Anderson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Caps and mixed drinks?

Date: Sun, 22 Feb 1998 12:15:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike@aol.com

Date: Sun, 22 Feb 1998 07:15:12 EST

Subject: Re: Caps and mixed drinks?

Hi all,

The Cuervo Capacitor, hmmmm.

The sloe gin generator.

The martini motor.

The daquari diode.

Electrostatic ethanol.

M. Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: It finally works!

Date: Sun, 22 Feb 1998 13:03:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Sun, 22 Feb 1998 14:03:41 +0100

Subject: It finally works!

Hi all!

I've now got my coil running after almost a month, i can get about 10"

streamer at first time i fired it up..) But, how can i improve my spark

length? maybe add som more primary turns? the specs:

And, i think i need a safety gap too, dont got one yet..I've done one with

2 bolts with round heads ( we call them "vagnsbultar" here in sweden, dont

know what they are in english), but about what distance between them should

i have? and would you recommend any rf chokes?

Primary: 11 turns with 3/16" copper tubing, the longest streamers is at

about the 8 turn. It wound with a inverted cone shape at 20 degrees.

secondary: 12" long winding with 28 AWG magnet wire on a 2" dryed and

sealed PVC tube.

Transfromer: A 15kV 20mA Oil burner tran.

capacitor: 10 layers baking type aluminum foil with 3 layers of 0.2mm poly

between the aluminum plates.

when i turned it on and saw the sparks I know that this was going to be my

future hobby..) But CAN i get longer streamers with this little one? And

just one question more, i havent really understood how this works, how can

the sparks just jump out in the air?. Theres no ground there..are they

*forced* to do that by the disharges of the cap and gap? And, i just want

to thank everyone in this list for all the help, especially Mike Hammer.

Thanks,

//Christopher Eriksson

//Sweden

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: crazy carpet capacitors

Date: Sun, 22 Feb 1998 16:53:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "William R. Langston" <blangsto@iwvisp.com>

Date: Sun, 22 Feb 1998 08:53:34 -0800

Subject: Re: crazy carpet capacitors

What is "crazy carpet"? If it's inexpensive, and I can find it, I'll try it.

Bill Langston

Tesla-2 wrote:

> From: mike nicholas <705728@ican.net>

> Date: Sat, 21 Feb 1998 19:19:51 -0800

> Subject: Re: crazy carpet capacitors

>

>   Anyone out there try using crazy carpet as a dielectric material. I'm

> working one right now, if anyone has tryed this I would be curious of your

> results. If it works it will be a very inexpensive capacitor

>

> Tesla-2 wrote:

>

> > From: Walt Kulpa <gwkulpa@earthlink.net>

> > Date: Sat, 21 Feb 1998 11:24:06 -0800

> > Subject: cap question

> >

> > greetings everyone,

> >

> > While I wait for my plastic to come in for rolled caps, I'm working on

> > some SW capacitors using information gleened from one of the archives

> > (can't remember which one). It says to wrap beer bottles in tin foil,

> > etc. etc. and then "connect all of the bolts sticking out together with

> > wire". I'm assuming that "connect all of the bottles together" means that

> > I will have something that looks like this when you look down at the six

> > bottles and that my incoming line lead can be connected to any one of the

> > bottles?

> >

> >     O----O----O            O = bottle cap

> >     |    |    |            - & | = wire

> >     O    O    O

> >

> > I have the bottles contained in a wooden box with a piece of aluminum

> > sheet at the bottom.

> >

> > Also, I am planning on using "ring type" solderless connectors, and

> > soldering them once I crimp them down. is this typically how it's done

> > or is there a better way?

> >

> > Thanks, Walt Kulpa

> >

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: It finally works!

Date: Sun, 22 Feb 1998 17:19:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 22 Feb 1998 12:19:59 -0500

Subject: Re: It finally works!

> From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

> Date: Sun, 22 Feb 1998 14:03:41 +0100

> Subject: It finally works!

>

> Hi all!

>

> I've now got my coil running after almost a month, i can get about 10"

> streamer at first time i fired it up..=3D) But, how can i improve my spark

> length? maybe add som more primary turns? the specs:

> And, i think i need a safety gap too, dont got one yet..I've done one with

> 2 bolts with round heads ( we call them "vagnsbultar" here in sweden, dont

> know what they are in english), but about what distance between them should

> i have?

vagnsbultar = wagon bolts = carriage bolts in english

Yeah, carriage bolts 'work', but not all that well. Have you noticed how

discolored and pitted they become? A Richard Quick style multiple static

gap is the way to go. This alone will give you a noticeable increase in

output. If you plan on moving up to bigger coils, then make it a vacuum

type Richard Quick static gap. The major difference is that the vacuum

gap forces air directly through the GAP, and not only cools, but also

affects the quenching. Total gap size would be around .25"  and I suggest

about 7 to ten gaps altogether.

> and would you recommend any rf chokes?

Yep. One on each HV bushing. Air core types or toroidal types are

quite popular. Air core ones: wind six inches of #28 to #24 wire onto

PVC pipe that is from 1.5 to 2" in diameter. They are not critical.

> Primary: 11 turns with 3/16" copper tubing, the longest streamers is at

> about the 8 turn. It wound with a inverted cone shape at 20 degrees.

Looks good.

> secondary: 12" long winding with 28 AWG magnet wire on a 2" dryed and

> sealed PVC tube.

You will do better with a larger diameter coil form. 4" and 6" are

popular sizes, and available in thinwall PVC.

> Transfromer: A 15kV 20mA Oil burner tran.

Two of these with the secondaries in parallel would make for a very

good power source. Three would be almost perfect!!!

> capacitor: 10 layers baking type aluminum foil with 3 layers of 0.2mm poly

> between the aluminum plates.

>

Hmmm. You didn't give enough info for us to determine the probable

capacitance value. Info required is ACTIVE plate area (area of plate

that overlaps facing plate), total poly thickness, number of plates,

and type of construction (rolled or flat).

> when i turned it on and saw the sparks I know that this was going to be my

> future hobby..=3D) But CAN i get longer streamers with this little one?

Yep. 10" is just the beginning. You should be able to double that easily.

> And just one question more, i havent really understood how this works, how can

> the sparks just jump out in the air?. Theres no ground there..are they

> *forced* to do that by the disharges of the cap and gap?

The air *IS* connected to the ground in a thousand subtle ways. The arc is

trying to complete the circuit in the laziest way it can, via the lowest

TOTAL resistance path that it can find from moment to moment.

> And, i just want to thank everyone in this list for all the help, especially

> Mike Hammer.

Yep, Mike does a magnificent job of answering questions on a regular

basis. And I am glad that so many other coilers are also pitching in

and answering questions, especially guys that were 'newbies' just a little

while ago.

> Thanks,

> //Christopher Eriksson

> //Sweden

>

Well, I hope that this helps in some way.

I am sure that you will receive answers from others as well.

Fr. Tom Mcgahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: It finally works!

Date: Sun, 22 Feb 1998 19:20:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 22 Feb 1998 11:20:56 -0800

Subject: Re: It finally works!

Organization: Stoneridge Engineering

Christopher,

Congratulations on first light!! It's always exciting to fire up a brand

new coil for the first time and actually see it throwing sparks! My

comments are interspersed below.

Tesla-2 wrote:

>

> From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

> Date: Sun, 22 Feb 1998 14:03:41 +0100

> Subject: It finally works!

>

> Hi all!

>

> I've now got my coil running after almost a month, i can get about 10"

> streamer at first time i fired it up..=3D) But, how can i improve my spark

> length? maybe add som more primary turns? the specs:

If your coil is in tune, simply adding more primary turns will not help.

There are a number of other things that may help, however. Have you got

a small toroid at the top of the coil? This will "hold off" breakout,

making for more powerful streamers. Increasing the main gap distance,

main cap size, and other factors will also increase streamer length.

However, you may need to make some cap changes, as well as adding chokes

and safety gaps. See comments below.

> And, i think i need a safety gap too, dont got one yet..I've done one with

> 2 bolts with round heads ( we call them "vagnsbultar" here in sweden, don t

> know what they are in english), but about what distance between them should

> i have?

These sound like the equivalent of what we call carriage bolts in the

USA. The safety gap should be adjusted by connecting it directly across

the output of your transformer with nothing else hooked up. Set it so

that it's just beyond the point of arcing over when you power up the

transformer. Once you've set the safety gap you can then hook up the

rest of your system.

> And would you recommend any rf chokes?

RF chokes can help extend the life of your transformer by blocking some

of the high frequency RF created when the gap fires from getting back

into your transformer where it can break down and destroy the insulation

in your HV windings. They're easy and inexpensive to make, and in

combination with a safety gap, provide significant protection. However,

eventually, your transformer will probably fail - be on the lookout for

spares now.

> Primary: 11 turns with 3/16" copper tubing, the longest streamers is at

> about the 8 turn. It wound with a inverted cone shape at 20 degrees.

> secondary: 12" long winding with 28 AWG magnet wire on a 2" dryed and

> sealed PVC tube.

> Transfromer: A 15kV 20mA Oil burner tran.

> capacitor: 10 layers baking type aluminum foil with 3 layers of 0.2mm poly

> between the aluminum plates.

3 layers of 0.2mm poly (about 0.024") is a little bit on the thin side

for running off a 15 kV source. One way to increase output is to begin

opening up the main gap a bit. However, you should add a safety gap

before doing this, and you will probably need to increase the overall

thickness of the dielectric (adding more plates to keep the capacitance

the same) so that you can safely increase the voltage stress on the

caps. What are the overall dimensions of your cap's plates?

Also, adding air-flow and using multiple gaps to aid in quenching may

also help improve performance if you're not already using these

techniques.

>

> when i turned it on and saw the sparks I know that this was going to be m y

> future hobby..) But CAN i get longer streamers with this little one? And

> just one question more, i havent really understood how this works, how can

> the sparks just jump out in the air?. Theres no ground there..are they

> *forced* to do that by the disharges of the cap and gap? And, i just want

> to thank everyone in this list for all the help, especially Mike Hammer.

> Thanks,

> //Christopher Eriksson

> //Sweden

>

At the top of the coil, the very high RF voltage tends to make the air

conductive (by ionizing it), starting at the point where the electric

fields are the strongest - the top terminal of the coil. As the air

begins to break down and conduct, a conductive channel, called a

streamer, is formed and as long as it can get enough energy from your

secondary coil, will continue to grow in an attempt to get to a lower

potential (ideally ground). The overall length of the streamer is a

function of the voltage and frequency of your coil, combined with the

amount of power you are supplying to the secondary.

Things that make for longer sparks include:

1. lower operating frequency (lower coil losses, and better streamer

propagation characteristics)

2. more breaks/second (more average power going into the secondary and

streamers). Usually requires a larger supply transformer for same size

tank cap to provide enough current to recharge the tank cap quickly

between gap firings.

3. more energy per bang (i.e., more tank capacitance and higher gap

voltage)

4. better gap quenching so that energy transferred to the secondary is

"trapped" there rather than being coupled back into the primary circuit

5. Efficient coil design, so that most of the energy ultimately makes it

to the streamers. A good RF ground, good RF layout, heavy primary

circuit wiring, high-Q secondary, and low-loss tank capacitor are all

parts of this. An efficient coil may deliver over 80% of its average

power to "incinerate the air".

Generally, bigger coils operate at lower frequencies, have larger tank

capacitiance, use bigger supply transformers to deliver more average

power to the streamers - all making for longer sparks.

Because the high voltage peaks only occur for brief periods of time,

followed by relatively long times between gap firings, individual

streamers may not have enough time or energy to make it all the way to

ground. However, if you make the gap fire rapidly enough, each streamer

will tend to follow a similar path (at least near the top terminal) as

the one that preceeded it. This appears to your eye as a continuous

spark. A rapidly firing spark gap will tend to give you longer streamers

since, all other things being the same, you are putting more power into

the sparks, and the air has less chance of cooling down (deionizing)

between gap firings.

Each streamer is sort of like a miniature lightning bolt, striving to

make its way to ground. Once a streamer begins forming, it begins to

take a substantial amount of energy from the secondary coil and toroid.

Once this energy in the secondary system has been used up in ionizing

and reheating the air in the streamer path, the streamer stops growing.

If you can add more energy per "bang" (when the gap fires), and the

streamer will be longer. By the way, there's still a lot of mystery and

conjecture about just how Tesla Coil streamers form and propagate, and

how to get the best/longest sparks. Maybe your future experiments will

help us answer some of these questions.

In any event, most of us never tire of watching these streamers play

about the top terminal of a well-running coil. Welcome aboard as an

active coiler!

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: It finally works!

Date: Sun, 22 Feb 1998 21:24:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 22 Feb 1998 13:24:40 -0800

Subject: Re: It finally works!

well there are many ways to improve preformance

add a ball discharge at the top

add another capacitor

retune your primary

Tesla-2 wrote:

> From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

> Date: Sun, 22 Feb 1998 14:03:41 +0100

> Subject: It finally works!

>

> Hi all!

>

> I've now got my coil running after almost a month, i can get about 10"

> streamer at first time i fired it up..) But, how can i improve my spark

> length? maybe add som more primary turns? the specs:

> And, i think i need a safety gap too, dont got one yet..I've done one with

> 2 bolts with round heads ( we call them "vagnsbultar" here in sweden, dont

> know what they are in english), but about what distance between them should

> i have? and would you recommend any rf chokes?

> Primary: 11 turns with 3/16" copper tubing, the longest streamers is at

> about the 8 turn. It wound with a inverted cone shape at 20 degrees.

> secondary: 12" long winding with 28 AWG magnet wire on a 2" dryed and

> sealed PVC tube.

> Transfromer: A 15kV 20mA Oil burner tran.

> capacitor: 10 layers baking type aluminum foil with 3 layers of 0.2mm poly

> between the aluminum plates.

>

> when i turned it on and saw the sparks I know that this was going to be my

> future hobby..=3D) But CAN i get longer streamers with this little one? And

> just one question more, i havent really understood how this works, how can

> the sparks just jump out in the air?. Theres no ground there..are they

> *forced* to do that by the disharges of the cap and gap? And, i just want

> to thank everyone in this list for all the help, especially Mike Hammer.

> Thanks,

> //Christopher Eriksson

> //Sweden

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Fri, 27 Feb 1998 17:38:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 27 Feb 1998 09:38:19 -0800 (PST)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

On 27 Feb, Tesla-2 wrote:

> From: mike nicholas <705728@ican.net>

> Date: Fri, 27 Feb 1998 10:48:27 -0800

> Subject: roll your own, aka(Krazy carpet capacitors)

>

>     It works......

>

>    I just made one capacitor, it costed around $7 Can.  it measures

> around 18" long by 13 1/4" in diameter.  The capacity is 0.0096 MFD it

> can with stand 7,500 vac in the air(I havent tested it for more yet) and

> fits nicely in a piece of 4" abs.

Ok.. I gotta ask,

WTF is Krazy Carpet?

Also, how can something 13.25" in diameter fit inside a pipe that is

4" in diameter???? [could that be 3.25"??? hmmm]

Color me confused!

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Fri, 27 Feb 1998 18:19:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: JAndrews13 <JAndrews13@aol.com>

Date: Fri, 27 Feb 1998 13:19:33 EST

Subject: Re: roll your own, aka(Krazy carpet capacitors)

so how about instructions!!!!!  Sounds like a good idea.

Have you ever used copper instead of aluminum. I hear it about doubles the

capacitance for the same size.

Chet

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: roll your own, aka(Krazy carpet capacitors)

Date: Fri, 27 Feb 1998 18:48:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Feb 1998 10:48:27 -0800

Subject: roll your own, aka(Krazy carpet capacitors)

It works......

I just made one capacitor, it costed around $7 Can.  it measures

around 18" long by 13 1/4" in diameter.  The capacity is 0.0096 MFD it

can with stand 7,500 vac in the air(I havent tested it for more yet) and

fits nicely in a piece of 4" abs.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: roll your own, aka(Krazy carpet capacitors)

Date: Fri, 27 Feb 1998 20:30:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Fri, 27 Feb 1998 14:30:20 -0600

Subject: RE: roll your own, aka(Krazy carpet capacitors)

I think Mike meant that the dielectric and foil was 18 X 13.25 in.

before rolling and fitting into the 4 in. ABS pipe.  Kind of short

though.   Right Mike?

And yeah, what the heck is Krazy Carpet?

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Friday, February 27, 1998 2:23 PM

To:     Minnich, Roy

Subject:        Re: roll your own, aka(Krazy carpet capacitors)

From: baumann@proton.llumc.edu

Date: Fri, 27 Feb 1998 09:38:19 -0800 (PST)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

On 27 Feb, Tesla-2 wrote:

> From: mike nicholas <705728@ican.net>

> Date: Fri, 27 Feb 1998 10:48:27 -0800

> Subject: roll your own, aka(Krazy carpet capacitors)

>

>     It works......

>

>    I just made one capacitor, it costed around $7 Can.  it measures

> around 18" long by 13 1/4" in diameter.  The capacity is 0.0096 MFD it

> can with stand 7,500 vac in the air(I havent tested it for more yet) and

> fits nicely in a piece of 4" abs.

Ok.. I gotta ask,

WTF is Krazy Carpet?

Also, how can something 13.25" in diameter fit inside a pipe that is

4" in diameter???? [could that be 3.25"??? hmmm]

Color me confused!

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Fri, 27 Feb 1998 22:05:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Fri, 27 Feb 1998 23:05:45 +0100 (MEZ)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

> Have you ever used copper instead of aluminum. I hear it about doubles the

> capacitance for the same size.

This is definitely not the case. Capacity only depends on geometry and the

dielectric used, not on the nature of the conducting material.

Jochen

---------------------------------------------------------------------------

Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de

The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/

info, plans, photos of home-built HV-sources, sparks, HV-experiments

---------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Sat, 28 Feb 1998 01:40:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Feb 1998 17:40:22 -0800

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Sorry it was a typo 3.25",  I guess you folks in warmer climates wouldnt know

what that is, it's a cheap sled for kids, its for sliding down snow covered hills

Tesla-2 wrote:

> From: baumann@proton.llumc.edu

> Date: Fri, 27 Feb 1998 09:38:19 -0800 (PST)

> Subject: Re: roll your own, aka(Krazy carpet capacitors)

>

> On 27 Feb, Tesla-2 wrote:

> > From: mike nicholas <705728@ican.net>

> > Date: Fri, 27 Feb 1998 10:48:27 -0800

> > Subject: roll your own, aka(Krazy carpet capacitors)

> >

> >     It works......

> >

> >    I just made one capacitor, it costed around $7 Can.  it measures

> > around 18" long by 13 1/4" in diameter.  The capacity is 0.0096 MFD it

> > can with stand 7,500 vac in the air(I havent tested it for more yet) and

> > fits nicely in a piece of 4" abs.

>

> Ok.. I gotta ask,

> WTF is Krazy Carpet?

>

> Also, how can something 13.25" in diameter fit inside a pipe that is

> 4" in diameter???? [could that be 3.25"??? hmmm]

>

> Color me confused!

>

> --

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Sat, 28 Feb 1998 01:43:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Feb 1998 17:43:39 -0800

Subject: Re: roll your own, aka(Krazy carpet capacitors)

I have never thought about copper, how would that double the capacaty? I

though the only two factors were dielectric thickness and the K value. Anyway

I thought tin foil would be cheaper.

Tesla-2 wrote:

> From: JAndrews13 <JAndrews13@aol.com>

> Date: Fri, 27 Feb 1998 13:19:33 EST

> Subject: Re: roll your own, aka(Krazy carpet capacitors)

>

> so how about instructions!!!!!  Sounds like a good idea.

> Have you ever used copper instead of aluminum. I hear it about doubles the

> capacitance for the same size.

>

> Chet

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Sat, 28 Feb 1998 01:47:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Feb 1998 17:47:34 -0800

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Actually that was a typo, unrolled its 18" X 50"  rolled its 18" X 3.25"

Krazy Carpet is a kind of cheap sled for kids, for places who get snow

anyway.

Tesla-2 wrote:

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Date: Fri, 27 Feb 1998 14:30:20 -0600

> Subject: RE: roll your own, aka(Krazy carpet capacitors)

>

> I think Mike meant that the dielectric and foil was 18 X 13.25 in. =

> before rolling and fitting into the 4 in. ABS pipe.  Kind of short =

> though.   Right Mike?

>

> And yeah, what the heck is Krazy Carpet?

>

> Roy Minnich

>

> ----------

> From:   Tesla-2[SMTP:tesla-2@emachine.com]

> Sent:   Friday, February 27, 1998 2:23 PM

> To:     Minnich, Roy

> Subject:        Re: roll your own, aka(Krazy carpet capacitors)

>

> From: baumann@proton.llumc.edu

> Date: Fri, 27 Feb 1998 09:38:19 -0800 (PST)

> Subject: Re: roll your own, aka(Krazy carpet capacitors)

>

> On 27 Feb, Tesla-2 wrote:

> > From: mike nicholas <705728@ican.net>

> > Date: Fri, 27 Feb 1998 10:48:27 -0800

> > Subject: roll your own, aka(Krazy carpet capacitors)

> >

> >     It works......

> >

> >    I just made one capacitor, it costed around $7 Can.  it measures

> > around 18" long by 13 1/4" in diameter.  The capacity is 0.0096 MFD it

> > can with stand 7,500 vac in the air(I havent tested it for more yet)

> and

> > fits nicely in a piece of 4" abs.

>

> Ok.. I gotta ask,

> WTF is Krazy Carpet?

>

> Also, how can something 13.25" in diameter fit inside a pipe that is

> 4" in diameter???? [could that be 3.25"??? hmmm]

>

> Color me confused!

>

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitance Questions

Date: Sat, 28 Feb 1998 03:04:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Fri, 27 Feb 1998 22:04:09 EST

Subject: Capacitance Questions

I would like some advice on sizing my toroid. I have a 4 1/2"D. x 22"L.

secondary of 24 gauge wire powered by a 15K 30ma neon xformr with a res freq

calculated at 357.04 hz. WinTesla says my ideal xformr to cap match is

.0053uf. What size toroid should I be making for maximum spark. Also can I

increase my tank capacitance (which I think will increase my energy transfer &

thus spark output ?) without risking damage to my xformr? To get my system in

tune I think I will  have to increase the tap on the number of turns on my

primary once I add the toroid?

Any advice will be much appreciated.      Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Date: Sat, 28 Feb 1998 03:25:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 27 Feb 1998 19:25:56 -0800

Subject: Re: roll your own, aka(Krazy carpet capacitors)

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: JAndrews13 <JAndrews13@aol.com>

> Date: Fri, 27 Feb 1998 13:19:33 EST

> Subject: Re: roll your own, aka(Krazy carpet capacitors)

>

> so how about instructions!!!!!  Sounds like a good idea.

> Have you ever used copper instead of aluminum. I hear it about doubles the

> capacitance for the same size.

>

> Chet

>

Chet,

Unfortunately, whoever told you that is wrong. There's no difference

capacitance-wise (or performance wise) between aluminum and copper

plates for Tesla Coil caps. And aluminum is one heck of a lot more

available and less expensive.

Safe cappin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance Questions

Date: Sat, 28 Feb 1998 05:35:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma <mwise@ns.sosis.com>

Date: Fri, 27 Feb 1998 23:35:22 -0600

Subject: Re: Capacitance Questions

Tesla-2 wrote:

>

> From: Dsurfr <Dsurfr@aol.com>

> Date: Fri, 27 Feb 1998 22:04:09 EST

> Subject: Capacitance Questions

>

> I would like some advice on sizing my toroid. I have a 4 1/2"D. x 22"L.

> I used a 2.5" x 13.25" toroid on my 4" coil and got 19" sparks from it.

> But I woul recommend a little larger than this, such 4" x 12".

> secondary of 24 gauge wire powered by a 15K 30ma neon xformr with a res freq

> calculated at 357.04 hz. WinTesla says my ideal xformr to cap match is

> .0053uf. What size toroid should I be making for maximum spark. Also can I

> increase my tank capacitance (which I think will increase my energy transfer &

> thus spark output ?) without risking damage to my xformr?

> I would not add any extra capacitance to you transformer because it will

> decrease performance and it can aid in the failure of your neon.

> To get my system in

> tune I think I will  have to increase the tap on the number of turns on my

> primary once I add the toroid?

> Any advice will be much appreciated.      Jim

You will definitly need to change your tap point. Also I would like to

point out something. Are you using the newest version of Wintesla, if so

it will calculate capacitance of a sphere or toroid. Also once you have

all of your info added in, goto the primary part and click on the part

where it asks you haow many turns your primary has. Now put in how many

turns you have on your primary in the box. Then slowly decrease your

turns and watch the little box in the bottom left corner where it says

required C. If you keep reducing turns until it says required C is

around .005 this should give you a fairly close place to where your

primary should be tapped.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance Questions

Date: Sat, 28 Feb 1998 16:14:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sat, 28 Feb 1998 11:14:29 EST

Subject: Re: Capacitance Questions

Jim,

I would think that a 4x16 toroid would be right for your system.    If

you go to a 6x20 size you will probably get good high voltage on the terminal,

but small streamers due to the charge it can hold, unless you put a pointed

"bleeder" on it to induce the streamers.

As for you capacitor,  it is safe to use the impedance matching, but you

can design your system for a higher rating.   The charge a cap can store is

governed by the equation E=0.5CV^2.  The voltage to use in this equation

should be the peak voltage of 15000/0.7071  or 21.2kV.  This would result in

your cap storing 1.19 joules. At a charging rate of 120 Hz, you are only

drawing 143 VA.  The maximum capacitance you could charge would be 0.016 uF at

the given voltage.  I am not suggesting that you go this high, but are just

giving you food for thought.  If you are using a homemade cap, I would suggest

that you not use more than 0.01 uF.  The impedance matching is meant for

transformers used in oscillator circuits in radios, TV's, etc.

Please don't take my word on this.  I have done much research on this and

believe it to be correct, but you should do some research yourself and confirm

or dispute what I have have stated.  My coil was not designed to be impedance

matched, but it turns out that it is pretty close.  I am running two xfrmrs

like yours in parallel and presently use a 0.0115 uF, 30 kV cap.  It seems to

run very well, but I am building a plate cap to be about 0.014 uF, 40kV cap to

see what happens.

I have a book that I obtained from the Tesla museum in Colorado Springs

written by D.C. Cox.  He has a company that he claims is the only full-time

production factory for TC's in the world.  This book does not deal with

construction details, but deals with the theory of Tesla coils very

extensively.  One of the points he makes is that the normal impedance matching

does not apply to Tesla coil charging systems.  It would apply if the the

charging cycle were of high frequency(perhaps 1kHz or higher), but not for 60

Hz.  He says this actually causes experimenters to design their caps with a

much larger capacitive reactance than their transformers can match.  He says

that using a cap that is .02 - .04uF greater than the normal impedance match

dictates usually works best.

I  invite comments on this and suggest you not base your design on this

information without researching this and drawing your own conclusions.

There is one other bit of information from this book that seems to be

valuable:

Mr. Cox presented an equation for calculating the output voltage for Tesla

coils.  This equation states that the secondary voltage is equal to the

primary voltage times the square root of the ratio of the secondary to primary

inductances.  In other words,  a higher output can be obtained by using larger

capacitance and lower inductance in the primary tank circuit (still keeping

the system in tune, of course).  This notion seems to be valid as far as I can

tell, and it seems to go hand in hand with the above notion of using larger

capacitance than the normal impedance match.  A design manual I have(also

obtained from the museum) warns not to use fewer than two turns in your

primary as this would make tuning quite difficult.

I hope this letter gives those who read it something to think about and

discuss.  I would like to get some feedback from coilers on this as I am only

repeating what I have read and would like to read any arguments to Mr. Cox's

theory.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance Questions

Date: Sat, 28 Feb 1998 17:29:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: John Ervin Jr <jwejr@aye.net>

Date: Sat, 28 Feb 1998 12:29:22 -0500

Subject: Re: Capacitance Questions

At 03:04 AM 2/28/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Fri, 27 Feb 1998 22:04:09 EST

>Subject: Capacitance Questions

>

>I would like some advice on sizing my toroid. I have a 4 1/2"D. x 22"L.

>secondary of 24 gauge wire powered by a 15K 30ma neon xformr with a res freq

>calculated at 357.04 hz. WinTesla says my ideal xformr to cap match is

.0053uf.

>

>Jim

I noticed that you made mention to something called "WinTesla".  I haven't

been here very long and am curious as to what this is.  If it is a program

like I am suspecting then where would I get it at.

John Ervin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: WinTesla

Cc:

Bcc:

X-Attachments:

At 05:29 PM 2/28/98 GMT, you wrote:

>From: John Ervin Jr <jwejr@aye.net>

>Date: Sat, 28 Feb 1998 12:29:22 -0500

>Subject: Re: Capacitance Questions

>

>

>I noticed that you made mention to something called "WinTesla".  I haven't

>been here very long and am curious as to what this is.  If it is a program

>like I am suspecting then where would I get it at.

>

>John Ervin

John,

WinTesla is an excellent Windows based Tesla coil calculation

program witten by Robert S. Coppersmith. You can download it from

his home page at: http://members.aol.com/rscopper/index.htm

It is alvailable on Dave Bronakowski's archive site along with other

good Tesla info. Daves site is at: http://www.velocity.net/~djb/files.shtml

One word of advice concerning the use of various formulas and

calculator programs. Use these for guidance only. They are great

for early planning stages on a coil but dont put absolute faith

in any of the predictions they make. The ONLY way to maximize

the performance of a coil is through experimentation. No program

will do that for you.

TTYL!