From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance Questions

Date: Sun,  1 Mar 1998 03:42:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 28 Feb 1998 19:42:45 -0800

Subject: Re: Capacitance Questions

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: John Ervin Jr <jwejr@aye.net>

> Date: Sat, 28 Feb 1998 12:29:22 -0500

> Subject: Re: Capacitance Questions

>

> At 03:04 AM 2/28/98 GMT, you wrote:

> >From: Dsurfr <Dsurfr@aol.com>

> >Date: Fri, 27 Feb 1998 22:04:09 EST

> >Subject: Capacitance Questions

> >

> >I would like some advice on sizing my toroid. I have a 4 1/2"D. x 22"L.

> >secondary of 24 gauge wire powered by a 15K 30ma neon xformr with a res freq

> >calculated at 357.04 hz. WinTesla says my ideal xformr to cap match is .0053uf.

> >

> >Jim

>

> I noticed that you made mention to something called "WinTesla".  I haven't

> been here very long and am curious as to what this is.  If it is a program

> like I am suspecting then where would I get it at.

>

WinTesla is one of a number of good design-aids available on the web.

You can find this particular one at:

http://members.aol.com/rscopper/index.htm.

> John Ervin

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance Questions

Date: Sun,  1 Mar 1998 16:38:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: JAndrews13 <JAndrews13@aol.com>

Date: Sun, 1 Mar 1998 11:38:52 EST

Subject: Re: Capacitance Questions

You referenced wintesla.  Tell me more .  Is this a program to help design a

tesla coil?

Help, Help, Help

Chet

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: WinTesla

Cc:

Bcc:

X-Attachments:

At 05:29 PM 2/28/98 GMT, you wrote:

>From: John Ervin Jr <jwejr@aye.net>

>Date: Sat, 28 Feb 1998 12:29:22 -0500

>Subject: Re: Capacitance Questions

>

>

>I noticed that you made mention to something called "WinTesla".  I haven't

>been here very long and am curious as to what this is.  If it is a program

>like I am suspecting then where would I get it at.

>

>John Ervin

John,

WinTesla is an excellent Windows based Tesla coil calculation

program witten by Robert S. Coppersmith. You can download it from

his home page at: http://members.aol.com/rscopper/index.htm

It is alvailable on Dave Bronakowski's archive site along with other

good Tesla info. Daves site is at: http://www.velocity.net/~djb/files.shtml

One word of advice concerning the use of various formulas and

calculator programs. Use these for guidance only. They are great

for early planning stages on a coil but dont put absolute faith

in any of the predictions they make. The ONLY way to maximize

the performance of a coil is through experimentation. No program

will do that for you.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Wire

Date: Sun,  1 Mar 1998 21:04:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Moleking <moleking@concentric.net>

Date: Sun, 01 Mar 1998 16:04:21 -0500

Subject: Wire

I've got a somewhat basic question about wiring.  I'm working on my first

coil, a 4 incher w/ a 15kv 30ma trannie.  I've got simple lamp cord running

into the trannie, but I'm not sure what type of wire I should use to

connect the rest of the unit together.  I saw a mention of coax somewhere.

I need something to run through the caps, to the primary, etc.  Where can I

find a good cheap wire with sufficient insulation?

Also, for my RF ground, I'm putting a 5'  copper pipe into the ground in

the backyard.  I'm thinking about making a terminal strip with a small

piece of copper sheet and some screws attached to a block of wood.  Does

this setup sound acceptable?  What should I run from the terminal to the

pipe?  Thanks

Josh Bedell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wire

Cc:

Bcc:

X-Attachments:

At 09:04 PM 3/1/98 GMT, you wrote:

>From: Moleking <moleking@concentric.net>

>Date: Sun, 01 Mar 1998 16:04:21 -0500

>Subject: Wire

>

>I've got a somewhat basic question about wiring.  I'm working on my first

>coil, a 4 incher w/ a 15kv 30ma trannie.  I've got simple lamp cord running

>into the trannie, but I'm not sure what type of wire I should use to

>connect the rest of the unit together.  I saw a mention of coax somewhere.

>I need something to run through the caps, to the primary, etc.  Where can I

>find a good cheap wire with sufficient insulation?

The requirements for the 60 hz wiring from your transformer to

your safety gap, RF chokes and main gap are pretty light.

The current is quite low so a heavy conductor is not needed.

You DO need wire with sufficient insulation.

Neon sign wire is rated for 15kv use and is quite good.

You can often get surplus lengths of it free from sign

companies.

Coax can be used but is rather expensive and I prefer

to save my coax for my radio equipment.

I normally use ordinary AWG 14 PVC insulated wire for this

purpose only because I have a 1000 ft roll of it. Since it is

only rated for 600 VAC plenty of space is required around it

to prevent arcing. And it should not be used anywhere you might

come into contact with it.

Some types of plain copper automotive ignition wire is usable.

But make sure you know what you are getting. Most ignition wire

is carbon cored to reduce ignition noise to radio systems.

This is not suitable.

>

>Also,  for my RF ground, I'm putting a 5'  copper pipe into the ground in

>the backyard.  I'm thinking about making a terminal strip with a small

>piece of copper sheet and some screws attached to a block of wood.  Does

>this setup sound acceptable?

Yes. Sounds fine. Two copper pipes a couple feet apart and bonded

together at the top would be even better.

> What should I run from the terminal to the pipe?

As heavy a wire as you can get. Make this run as short and as straight

as possible. I use AWG 8 about 6 foot long for my ground lead.

Do not use braid or braided cables. Solid conductors or non twisted

stranded cable is best.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Wire

Date: Mon,  2 Mar 1998 01:48:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 01 Mar 1998 17:48:41 -0800

Subject: Re: Wire

Well for the primary of your tranny you should use 14 guage wire with a ground,

also I recommend at least one surge absorber on your primary.

Tesla-2 wrote:

> From: Moleking <moleking@concentric.net>

> Date: Sun, 01 Mar 1998 16:04:21 -0500

> Subject: Wire

>

> I've got a somewhat basic question about wiring.  I'm working on my first

> coil, a 4 incher w/ a 15kv 30ma trannie.  I've got simple lamp cord running

> into the trannie, but I'm not sure what type of wire I should use to

> connect the rest of the unit together.  I saw a mention of coax somewhere.

> I need something to run through the caps, to the primary, etc.  Where can I

> find a good cheap wire with sufficient insulation?

>

For most of my HV connections I use solid spark plug wire, I bought a 25' spool

for around $15 Can. It has a 40KV insulation, solid copper wire will also work

if you have standoffs.

> Also,  for my RF ground, I'm putting a 5'  copper pipe into the ground in

> the backyard.  I'm thinking about making a terminal strip with a small

> piece of copper sheet and some screws attached to a block of wood.

You may have a hard time pounding it in the ground, I think a good used well

point would give you an easier time.

> Does this setup sound acceptable?  What should I run from the terminal to the

> pipe?  Thanks

>

Heavy copper wire would be best, but if you are on a budget just use the same

HV wire you used for every thing else.

> Josh Bedell

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: hamfests

Cc:

Bcc:

X-Attachments:

Hello all,

Im often asked where to find parts for building Tesla coils

and other projects. My favorite source is from hamfests.

A hamfest is a convention put on by Amateur radio operators (hams)

that usually includes a electronic/radio flea market. Here you

will find almost anything electronic at bargain basement prices.

If you want to find out where a hamfest is in your area check

out the hamfest calender maintained by the American Radio Relay League.

(ARRL). Their web site is: http://www.arrl.org/

The calender for hamfests is at: http://www.arrl.org/hamfests.html

What can you find at a hamfest flea market? Just about anything.

Switches, wire, transformers (even neons), variacs and test equipment

of all sorts.

I normally attend all the hamfests in central and northern Illinois

so if anyone plans to go to any of these let me know and maybe we

can meet at one of them.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: chokes

Date: Sat,  7 Mar 1998 04:49:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Fri, 6 Mar 1998 23:49:28 -0500

Subject: chokes

From Chris Watkins  cwatkins@seidata.com

Hi coilers

I am preparing to make 2 chokes for my coil.

I intend to purchase 2 iron powder toroids of

a #33 maganese-zinc ferrite compound.

the dim's are  5.2"od  3.08id height .8"

fyi these are $15.00 each.

you can find them at http://www.bytemark.com/amidon/index1.htm

does anyone know a formula to calculate the

inductance reqd to yield an effective choke?

My unloaded frequency is 171khz the loaded

frequency may get as low as 90khz.

I intend to pass 12kw across these (12kv-1a)

& havent decided on a wire size yet.

can someone please tell me how I should go

about making these into good chokes?

anyone planning to attend the hamfest at

the indianapolis fairgrounds this sunday?

I plan to check it out.

find out more details at

http://www.eb1.com/MCRA/

safe coiling to all;

Chris W

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Model A Car spark coil

Date: Sat,  7 Mar 1998 15:21:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 7 Mar 1998 20:21:47 +0500

Subject: Re: Model A Car spark coil

On Sun, 15 Feb 1998 15:01:50 -0500 Jim Ryan

<jryan@exis.net> wrote;

> I just obtained an operational Model A car spark generator coil (cool

> wooden box). Does anyone know if it has any use in a Tesla coil.

Jim, many a first Tesla coil have been built using old automobile spark

coils. Just don't expect any spectacular results, these are relatively low

power devices and you need power to drive sparks.

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/6251

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hamfests

Date: Sat,  7 Mar 1998 15:21:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 7 Mar 1998 20:21:49 +0500

Subject: Re: hamfests

On Fri, 06 Mar 1998 14:21:36 -0600 Mike Hammer

<mhammer@misslink.net> wrote;

> Hello all,

>

> Im often asked where to find parts for building

> Tesla coils and other projects. My favorite source

> is from hamfests.

Hi Mike, another good source for Tesla coil parts is electronics surplus

stores, these also are typicaly run by old HAMs and can be found mostly by

word of mouth at local HAM fests and sometimes in the local yellow pages

under surplus. There used to be a lot of them in the Philadelphia area,

like Capt. Goes surplus, Banton Electronics, The computer graveyard, ect.

most have gone but there is still one near me in North East Philadelphia

called Fertechs that typically sells variacs at like $5.00 to $10.00 each,

copper wire in all sizes for like $2.00 per pound, Tektronix scopes go

for $10.00 to $50.00 depending on the model, ect. You have to keep your

eyes and ears open.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: chokes

Cc:

Bcc:

X-Attachments:

At 04:49 AM 3/7/98 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Fri, 6 Mar 1998 23:49:28 -0500

>Subject: chokes

>

>From Chris Watkins  cwatkins@seidata.com

>

>Hi coilers

>I am preparing to make 2 chokes for my coil.

>

>I intend to purchase 2 iron powder toroids of

>a #33 maganese-zink ferrite compound.

>the dim's are  5.2"od  3.08id height .8"

> fyi these are $15.00 each.

>you can find them at http://www.bytemark.com/amidon/index1.htm

Chris,

I was just browsing the Amidon site and it doesnt appear

that the #33 ferrite material is available in toroid form, rods only.

Rods would be OK just not as compact or convenient as toroids.

>does anyone know a formula to calculate the

>inductance reqd to yield an effective choke?

You need a minimum of 2 mh inductance. The value

is not at all critical. Anything from 2-10 mh works

out very well.

Use the formulas Amidon provides on their site

for figuring the required number of turns.

>can someone please tell me how I should go

>about making these into good chokes?

>I intend to pass 12kw across these (12kv-1a)

>& havent decided on a wire size yet.

For your power requirements use AWG 22. This will

handle 1 amp easily. But I seriously doubt you

will ever be able to utilize this much current.

Cover the core with several layers of good quality vinyl

tape. Wind as many turns as you can fit on the core leaving

at least a 3/4 inc space between the start and finish to

prevent arcing problems. Coating the finished choke in

polyurethan varnish or even potting it in paraffin would

be a good idea.

One core that looks like it might be good is the FT240-77.

This is a 2.4 inch OD toroid that will give about 3.13 mh of

inductance with 100 turns. The 77 material is suitable from

1 khz to over 1 mhz. The 2.4 inch size is the largest size

and are $9.

>

>anyone planning to attend the hamfest at

>the indianapolis fairgrounds this sunday?

>I plan to check it out.

>

Ive been to the Indy hamfest several times but wont

be able to make the trip over there this year.

I would look for some toroidal cores at the hamfest.

I often see vendors with large ferrite toroids for

a couple bucks. Since the requirements for RF chokes

are pretty light surplus cores like these usually work well.

Im using surplus ferrite toroids I bought at the Peoria,Il

hamfest for 50 cents each. They work just fine.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hamfests

Date: Sun,  8 Mar 1998 07:39:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sat, 07 Mar 1998 23:39:13 -0800

Subject: Re: hamfests

Tesla-2 wrote:

> From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

> Date: Sat, 7 Mar 1998 20:21:49 +0500

> Subject: Re: hamfests

>

> On Fri, 06 Mar 1998 14:21:36 -0600 Mike Hammer

> <mhammer@misslink.net> wrote;

>

> > Hello all,

> >

> > Im often asked where to find parts for building

> > Tesla coils and other projects. My favorite source

> > is from hamfests.

>

> Hi Mike, another good source for Tesla coil parts is electronics surplus

> stores, these also are typicaly run by old HAMs and can be found mostly by

> word of mouth at local HAM fests and sometimes in the local yellow pages

> under surplus. There used to be a lot of them in the Philadelphia area,

> like Capt. Goes surplus, Banton Electronics, The computer graveyard, ect.

> most have gone but there is still one near me in North East Philadelphia

> called Fertechs that typically sells variacs at like $5.00 to $10.00 each,

> copper wire in all sizes for like $2.00 per pound, Tektronix scopes go

> for $10.00 to $50.00 depending on the model, ect. You have to keep your

> eyes and ears open.

>

> Got any addresses?. I'm in Canada

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 07:48:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sat, 07 Mar 1998 23:48:02 -0800

Subject: Re: human tricks

Tesla-2 wrote:

Is it safe to sit on top of a running tesla coil?/Anyone know about

good tricks to do while sitting on it.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 07:52:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 8 Mar 1998 01:52:21 -0600

Subject: Re: human tricks

I often wonder where the urge to sit on top of a million volt

generator comes from.  Oh well.  It could kill you, it probably

won't, but just drawing a spark to your finger hurts like hell,

so I would not want to try it.  Besides, wouldn't your secondary

collapse under the weight?

>     Is it safe to sit on top of a running tesla coil?/Anyone know about

>good tricks to do while sitting on it.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 13:10:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 8 Mar 1998 08:10:50 EST

Subject: Re: human tricks

Mike,

There is a picture of Bill Wysock in an issue of National Geographic

sitting on a platform that is connected to the secondary of a coil.  This is

probably a better idea that actually sitting on the coil itself.  In one shot,

he was holding a rod with a lightbulb attached about halfway up which was

shining fairly bright.  There were also good size streamers emanating from the

end of the rod.  I think that his coil was generating at least a million volt

output, but I don't remember if the article stated this or not.  The main

topic of this issue was lightning.  I  cannot tell you the exact date of the

issue.  You can probably find it in the library if you wish to.

One thing I did in a picture that everyone seemed to like was to simply

hold a fluorescent light bulb and draw arcs from my coil to it.  Like a metal

rod, this doesn't hurt  when the arc is drawn to it.  The bulb shines fairly

brightly when arcs are drawn in this manner.  It will light up just in the

vicinity of the coil without drawing which is safer than drawing an arc if you

wish to try this first.

Be safe when trying anything like this. Even though the output is high

frequency, there exists the possibility that you can get a 60 Hz shock which

is superimposed on the output.  If you are grounded and get a 60 Hz shock, it

could kill you.   I thnk it only requires about 10 milliamps to do this at 60

Hz.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hamfests

Date: Sun,  8 Mar 1998 15:39:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 08 Mar 1998 09:39:53 -0600

Subject: Re: hamfests

Organization: MailExcite  (http://www.mailexcite.com)

Mike Hammer wrote:

>I normally attend all the hamfests in central and northern Illinois

>so if anyone plans to go to any of these let me know and maybe we

>can meet at one of them.

Likewise, I'll be attending the Dayton, OH Hamvention,

which runs from May 15-17.  With luck, I'll get a few

of my local mad-scientist friends to go as well!

If anyone else from the list is going, perhaps we can

arrange a get-together of some sort...?  Please advise.

Also - coilers! Be sure to re-visit my page at:

http://www.geocities.com/CapeCanaveral/Hangar/6160

I've uploaded a scad of scanned pics relating to all

sorts of high voltage goodies, including (but not

limited to):

* X-ray tubes and paraphenalia

* Geisler tubes (just one)

* More tesla stuff (of course!)

* Thyratron data sheets

* An unwise experiment involving a propane tank and a

cutting torch...

* Things to do with a machine gun.

We also just bought a large 32uF 40KV pulse cap - this

should generate loads of amusing photos in the days to

come!  We already tried (and achieved) the old

"exploding wire" trick using only 500 of a possible

25000 Joules! :)

-Bill the arcstarter

Starting arcs (while not becoming one himself) in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 15:55:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 08 Mar 1998 09:55:54 -0600

Subject: Re: human tricks

Organization: MailExcite  (http://www.mailexcite.com)

mike nicholas wrote:

>

>     Is it safe to sit on top of a running tesla coil?/Anyone know about

>good tricks to do while sitting on it.

Ah, well, let me just say that the arcs from our coil

will *ignite* a 2x4 in about 3 seconds...  Granted -

there is a path to ground (via the wood) so this might

be different than just arcing to the air, but this

still demonstrates that substantial power is flowing!

You won't catch ME doing that.  Ever.  I suggest you do

likewise!

I think Mike Hollingsworth stated a very valid point

about the 60-Hz energy.  Any direct contact between

the primary and secondary (caused, perhaps, by a brief

arc-over) will couple enough 60-Hz energy into the secondary

to probably kill someone...

Be aware that this coupled energy will be supplied by

both the transformer and the capacitor bank.  Thus it

will be capable of significant currents!  Does "Death on contact"

sound about right?

Please - play safe!  Feel free to use your coil to burn

up inanimate objects (we do!), but not yourself.

Especially not in full view of your guests.  That'd be

rude.

-Bill the arcstarter

Co-inventor of the light-emitting capacitor

In Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: human tricks

Cc:

Bcc:

X-Attachments:

At 07:48 AM 3/8/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Sat, 07 Mar 1998 23:48:02 -0800

>Subject: Re: human tricks

>

>

>

>Tesla-2 wrote:

>

>     Is it safe to sit on top of a running tesla coil?/Anyone know about

>good tricks to do while sitting on it.

>

This is an exceedingly dangerous and quite stupid thing

to do. While some people have done it under controlled

conditions its like playing Russian roulette with a loaded

revolver. Sooner or later you are going to have a problem.

Popular stage performer Henry L. Transtrom was killed in

this manner during one of his electrical shows. A streamer

exited his body and connected to some overhead piping. This

placed him in the direct streamer path to ground.

You can take hits from a small coil with relative safety.

But as the power input grows larger and the streamers grow

longer it becomes increasingly unsafe to do.

Tesla coils may have RF output which is relatively safe

due to skin effect. But spark driven coils have a considerable

60 hz component superimposed on the RF. This 60 hz current

can kill. No second chances.

So play it smart and keep body parts out of range of your coils.

Live long and prosper.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 19:51:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 08 Mar 1998 11:51:08 -0800

Subject: Re: human tricks

Tesla-2 wrote:

> From: Hollmike <Hollmike@aol.com>

> Date: Sun, 8 Mar 1998 08:10:50 EST

> Subject: Re: human tricks

>

> Mike,

>    There is a picture of Bill Wysock in an issue of National Geographic

> sitting on a platform that is connected to the secondary of a coil.  This is

> probably a better idea that actually sitting on the coil itself.  In one shot,

> he was holding a rod with a lightbulb attached about halfway up which was

> shining fairly bright.  There were also good size streamers emanating from the

> end of the rod.  I think that his coil was generating at least a million volt

> output, but I don't remember if the article stated this or not.  The main

> topic of this issue was lightning.  I  cannot tell you the exact date of the

> issue.  You can probably find it in the library if you wish to.

>     One thing I did in a picture that everyone seemed to like was to simply

> hold a fluorescent light bulb and draw arcs from my coil to it.  Like a metal

> rod, this doesn't hurt  when the arc is drawn to it.  The bulb shines fairly

> brightly when arcs are drawn in this manner.  It will light up just in the

> vicinity of the coil without drawing which is safer than drawing an arc if you

> wish to try this first.

>     Be safe when trying anything like this. Even though the output is high

> frequency, there exists the possibility that you can get a 60 Hz shock which

> is superimposed on the output.  If you are grounded and get a 60 Hz shock, it

> could kill you.   I thnk it only requires about 10 milliamps to do this at 60 Hz.

>     Mike Hollingsworth

>

I guess thats why GFI outlets have a threshold of 5mA, I would use an

insulated platform if I were to attempt it, What speed film should I use?

I'm not sure of the voltage output just yet, but I will be feeding it 2,700 watts of

power, and its 10" diam.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Safe Coils

Date: Sun,  8 Mar 1998 19:57:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Robert C. McGrath" <rmcgra1@airmail.net>

Date: Sun, 08 Mar 1998 13:57:38 -0600

Subject: Safe Coils

Hi, I was wondering what voltage would be safe so that you could

actually touch the secondary..In my electronics class, we had a small

one, but I don't remember the specs...thanx,

-Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: hamfests

Cc:

Bcc:

X-Attachments:

At 03:39 PM 3/8/98 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Sun, 08 Mar 1998 09:39:53 -0600

>Subject: Re: hamfests

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Mike Hammer wrote:

>

>>I normally attend all the hamfests in central and northern Illinois

>>so if anyone plans to go to any of these let me know and maybe we

>>can meet at one of them.

>

>Likewise, I'll be attending the Dayton, OH Hamvention,

>which runs from May 15-17.  With luck, I'll get a few

>of my local mad-scientist friends to go as well!

>

Ah. Dayton is the Mecca of all hamfests. Ive only attended

it twice and my memories of it are rotten weather, lousy food,

impossible traffic, and great bargains!

>If anyone else from the list is going, perhaps we can

>arrange a get-together of some sort...?  Please advise.

I do wish I could go to it this year but circumstances dictate

that I remain close to home.

>

>Also - coilers! Be sure to re-visit my page at:

>http://www.geocities.com/CapeCanaveral/Hangar/6160

I took a look and all I can say is that Im glad the state

of Indiana lies between you and me as a buffer zone. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Sun,  8 Mar 1998 23:47:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 08 Mar 1998 15:47:20 -0800

Subject: Re: human tricks

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sun, 08 Mar 1998 10:44:02 -0600

> Subject: Re: human tricks

>

> At 07:48 AM 3/8/98 GMT, you wrote:

> >From: mike nicholas <705728@ican.net>

> >Date: Sat, 07 Mar 1998 23:48:02 -0800

> >Subject: Re: human tricks

> >

> >

> >

> >Tesla-2 wrote:

> >

> >     Is it safe to sit on top of a running tesla coil?/Anyone know about

> >good tricks to do while sitting on it.

> >

>

> This is an exceedingly dangerous and quite stupid thing

> to do. While some people have done it under controlled

> conditions its like playing Russian roulette with a loaded

> revolver. Sooner or later you are going to have a problem.

>

> Popular stage performer Henry L. Transtrom was killed in

> this manner during one of his electrical shows. A streamer

> exited his body and connected to some overhead piping. This

> placed him in the direct streamer path to ground.

>

> You can take hits from a small coil with relative safety.

> But as the power input grows larger and the streamers grow

> longer it becomes increasingly unsafe to do.

>

> Tesla coils may have RF output which is relatively safe

> due to skin effect. But spark driven coils have a considerable

> 60 hz component superimposed on the RF. This 60 hz current

> can kill. No second chances.

>

> So play it smart and keep body parts out of range of your coils.

>

> Live long and prosper.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

To add to Mike's excellent post...

In addition to the risks with 60 Hz (particularly if you happen to get a

primary:secondary flashover!), it turns out that there's also a

significant DC component that's inherent in Tesla Coils, particularly

those with significant toroidal or spherical discharge terminals. The

discharge terminal's self-capacitance adds to the self-capacitance of

the secondary coil. The combined capacitance, when combined with the

inductance of the secondary coil, forms the secondary's tuned LC

circuit. A coil that's "in tune" has the primary LC circuit tuned to

match the same resonant frequency as the secondary LC circuit.

In operation, the secondary's energy is alternately stored in the

capacitance, as a high voltage, or 1/4 cycle later, in the inductance as

a peak current. BTW, it's this high current that actually flows out the

base of the secondary to the RF ground as a normal part of Tesla Coil

operation. At the point when all the energy of the secondary resides in

the charged toroid and coil self-capacitance, the output will behave

much like a charged HV capacitor. The higher the voltage, the greater

the energy. It's this effect that causes the "jolts" from a small coil

when you just reach the point of arc over to a hand-held object. If you

rapidly move closer to the top terminal, the maximum energy stored in

the toroid:coil capacitance is much less, and you'll feel less, if any,

of a shock.

Let's look at a medium sized coil that outputs 400,000 volt peaks, with

50 pF of effective capacitance. If you happen to be in a direct path to

ground at maximum voltage, you'll get zapped with about 4 Joules of

energy - it will act, and feel, just like you got hit with a sledge

hammer! The worst part is when the other end of the path comes blasting

out your shoes to get to ground, or if some other part of your anatomy

becomes the quickest path to a ground (like a knee, elbow, or whatever!)

While you'll _probably_ survive, you'll also not try that trick again.

I've measured peak currents of over 600 Amps from the top of the toroid

to a grounded wire for 5 foot discharges off my 10" coil. You REALLY

don't want to be in the path of these!   :^(

Those that are doing these tricks are very experienced, know the risks

involved, and use a number of proven techniques to keep themselves from

becoming toast. Bill Wysock, for example, has been building coils since

the early '80's. Brent Turner shows a similar technique on his website.

However, these are NOT the techniques we normally discuss in the Tesla-2

beginners forum.

BTW, the particular article is in the July, 1993 National Geographic

("Lightning - Nature's High Voltage Spectacle"). This article also

includes pictures of another excellent coiler, Richard Hull, the Theater

of Electricity in Boston, plus triggered lighting research (rocket and

trailing wire) by some meteorologists in Florida. Check your local

library (or even garage sales!) for a copy - it's worth reading!

Play safe, and have fun... safely!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 00:32:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 8 Mar 1998 18:32:00 -0600

Subject: Re: human tricks

>He who coils safe today lives to coil another day!   I think I get the point,

>it would be considered "stupid human tricks"  What safe(but spectacular)

>tricks could I preform?

Go for the basic hold a copper pipe and draw a spark to it.  The more area

of your hand on the pipe, the less it will shock you.  This is because the

electricity is more spread out, rather than having a single entry point.

You can put a clear lightbulb in your mouth and when you walk near the coil, your

body picks up enough energy to light the lightbulb.  You can also have

flourescent bulbs that light up much farther away.

Rub deodorant on the toroid and the sparks change to a green color.

Other materials change it to other colors as well.  Put a bunch of different

materials on the toroid at once in random places and you'll basically have

a ball of sparks of multiple colors.  You could take a metal pan, fill it will

alcohol and ignite the alcohol and then power your coil up with it.   The sparks

will seem to appear out of the flames, neat to watch.

If your really feeling like a daredevil, place your hand on the toriod, hold it

there, and have someone else turn the coil on.  Your will feel a slight tingling,

but it will not hurt because your body's capacitance changes the frequency of your

coil.  If you remove your hand though, it will hurt like hell.  But then

again, this isn't much better than sitting on it :-)  Try putting a lightbulb on top of

the coil, or a 10 gallon fish tank full of water.

Whatever you do, be creative.  I find that people love to watch me put old

household appliances (tvs, vcrs...) on top of the coil.  The capacitors and ICs

burn up and smoke and it looks really cool.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hamfests

Date: Mon,  9 Mar 1998 01:53:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 08 Mar 1998 19:53:47 -0600

Subject: Re: hamfests

Organization: MailExcite  (http://www.mailexcite.com)

Mike Hammer wrote:

>Ah. Dayton is the Mecca of all hamfests. Ive only attended

>it twice and my memories of it are rotten weather, lousy food,

>impossible traffic, and great bargains!

I actually rented two consecutive spots at the Dayton

fest about four years ago.  Drove in my old pickup,

and sold 19 cases of soda off the tailgate for 50 cents

per can (which was 100 % markup), in four hours, on a

rainy and cold saturday!  Made enough to pay for the

spots and tickets for myself and two buddies! Then

we bought $4000 worth of computer stuff, so it more

than evened out in the end...

Of course this was technically illegal but we did it

anyway...  Who wants to pay $1.50 for a watered-down

ARA soda?? :)

You are right about the traditionally terrible weather

up there.  Once we saw folk paying a few $$ to get

their cars towed out of the (mud-filled) parking lot by

some guy with a tractor!!  This year they moved it fwd

one month into May, in an attempt to break the trend...

>>Also - coilers! Be sure to re-visit my page at:

>>http://www.geocities.com/CapeCanaveral/Hangar/6160

>

>I took a look and all I can say is that Im glad the state

>of Indiana lies between you and me as a buffer zone. :)

If anyone is in the vicinity feel free to drop me a

line and visit the place.  Just look for the smoke...

I might even buy you a beer.

-Bill the arcstarter

Crushing old soda cans electromagnetically at 900 J.

in Cinci, OH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 01:55:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 08 Mar 1998 17:55:43 -0800

Subject: Re: human tricks

Tesla-2 wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sun, 08 Mar 1998 10:44:02 -0600

> Subject: Re: human tricks

>

> At 07:48 AM 3/8/98 GMT, you wrote:

> >From: mike nicholas <705728@ican.net>

> >Date: Sat, 07 Mar 1998 23:48:02 -0800

> >Subject: Re: human tricks

> >

> >

> >

> >Tesla-2 wrote:

> >

> >     Is it safe to sit on top of a running tesla coil?/Anyone know about

> >good tricks to do while sitting on it.

> >

>

> This is an exceedingly dangerous and quite stupid thing

> to do. While some people have done it under controlled

> conditions its like playing Russian roulette with a loaded

> revolver. Sooner or later you are going to have a problem.

>

> Popular stage performer Henry L. Transtrom was killed in

> this manner during one of his electrical shows. A streamer

> exited his body and connected to some overhead piping. This

> placed him in the direct streamer path to ground.

>

> You can take hits from a small coil with relative safety.

> But as the power input grows larger and the streamers grow

> longer it becomes increasingly unsafe to do.

>

> Tesla coils may have RF output which is relatively safe

> due to skin effect. But spark driven coils have a considerable

> 60 hz component superimposed on the RF. This 60 hz current

> can kill. No second chances.

>

> So play it smart and keep body parts out of range of your coils.

>

> Live long and prosper.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

He who coils safe today lives to coil another day!   I think I get the point,

it would be considered "stupid human tricks"  What safe(but spectacular)

tricks could I preform?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 04:32:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 8 Mar 1998 22:32:07 -0600

Subject: Re: human tricks

Well I have done the copper rod at 900watts and it only tingles.

>Most of these trick sound great, I'm not sure about drawing a spark with a

>copper rod though, I've tried it with a very low powered coil(tv flyback ss),

>but reading the other messages I have some concerns about my safety with this

>larger one I built. Anyone know what speed film I should use?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 05:07:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 08 Mar 1998 21:07:49 -0800

Subject: Re: human tricks

Tesla-2 wrote:

> From: "Jeff Corr" <corr@enid.com>

> Date: Sun, 8 Mar 1998 18:32:00 -0600

> Subject: Re: human tricks

>

> >He who coils safe today lives to coil another day!   I think I get the point,

> >it would be considered "stupid human tricks"  What safe(but spectacular)

> >tricks could I perform?

>

> Go for the basic hold a copper pipe and draw a spark to it.  The more area

> of your hand on the pipe, the less it will shock you.  This is because the

> electricity is more spread out, rather than having a single entry point.

> You can put a clear lightbulb in your mouth and when you walk near the coil, your

> body picks up enough energy to light the lightbulb.  You can also have flourescent

> bulbs that light up much farther away.   Rub deoderent on the toriod and the

> sparks change to a green color.  Other materials change it to other colors as

> well.  Put a bunch of different materials on the toriod at once in random

> places and you'll basically have a ball of sparks of multiple colors.  You could

> take a metal pan, fill it will alcohol and ignite the alcohol and then power your

> coil up with it.   The sparks will seem to appear out of the flames, neat to watch.

> If your really feeling like a daredevil, place your hand on the toriod, hold it

> there, and have someone else turn the coil on.  Your will feel a slight tingling,

> but it will not hurt because your body's capacitance changes the frequency of your

> coil.  If you remove your hand though, it will hurt like hell.  But then

> again, this isn't much better than sitting on it :-)  Try putting a lightbulb on top of

> the coil, or a 10 gallon fish tank full of water.

>

> Whatever you do, be creative.  I find that people love to watch me put old

> household appliances (tvs, vcrs...) on top of the coil.  The capacitors and ICs

> burn up and smoke and it looks really cool.

>

Most of these trick sound great, I'm not sure about drawing a spark with a

copper rod though, I've tried it with a very low powered coil(tv flyback ss),

but reading the other messages I have some concerns about my safety with this

larger one I built. Anyone know what speed film I should use?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: human tricks

Cc:

Bcc:

X-Attachments:

At 05:07 AM 3/9/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Sun, 08 Mar 1998 21:07:49 -0800

>Subject: Re: human tricks

>

>

>

>Tesla-2 wrote:

>

>> From: "Jeff Corr" <corr@enid.com>

>> Date: Sun, 8 Mar 1998 18:32:00 -0600

>> Subject: Re: human tricks

>>

>>

>> Go for the basic hold a copper pipe and draw a spark to it.  The more area

>> of your hand on the pipe, the less it will shock you.  This is because the

>> electricity is more spread out, rather than having a single entry point.

This is not a very smart thing to do. A very good friend of mine and an

experienced coiler injured himself doing just this.

He was standing on a plastic milk crate drawing arcs to a piece of

pipe held in his hand. All was OK until a streamer broke out from his

elbow and connected to the furnace duct. This placed his arm directly

in the arc channel to ground. He received a very severe shock and fell

off the milk crate onto the concrete floor breaking his wrist.

His other arm that took the streamer hit remained numb and almost

immobile for a couple days.

Needless to say he no longer takes streamers to the body and

his story convinced me of the foolishness of doing so.

>

>Most of these trick sound great, I'm not sure about drawing a spark with a

>copper rod though, I've tried it with a very low powered coil(tv flyback ss),

>but reading the other messages I have some concerns about my safety with this

>larger one I built. Anyone know what speed film I should use?

The mistake most beginners make in taking pictures of their coils running

is to take the picture in total darkness. This results in a too long

exposure that shows nothing but a mass of streamers.

Light up the wall behind your coil dimly with a lamp and reflector.

If your camera has a light meter set the exposure to record the

wall well. This will take about 1/4 - 1/2 sec at F 5.6 using ASA 400

film. A tripod and remote cable release for the camera is a must.

This will record the background fairly well and the streamers

will show up quite well as bright against the dimmer background.

Take a look at the pics on my web page to see how this looks.

http://www.misslink.net/mhammer/

One other note. Dont take your super fancy microelectronic

do everything camera anywhere near your operating coil.

Instead invest in a totally mechanical camera like my trusty

old Olympus OM-1 or maybe a Pentax K-1000. Both of these

are available rather cheaply used.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 12:27:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Mon, 09 Mar 1998 12:27:21 GMT

Subject: Re: human tricks

Organization: White Wing Logic

>

>He who coils safe today lives to coil another day!   I think I get the point,

>it would be considered "stupid human tricks"  What safe(but spectacular)

>tricks could I perform?

You could probably acheive a similarly spectacular effect a lot safer

using a double exposure - for example, set up a rod for the discharge

(maybe paint it black), take a pic of you standing with your finger

obscuring the end of the rod, stand well back, fire up, and re-expose

on the same frame to make it look like the spark is emanating from

your finger.

OK, it may be cheating, but better fraud than fried!

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Simplest First Tesla Coil Design

Date: Mon,  9 Mar 1998 15:13:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 9 Mar 1998 10:13:07 -0500

Subject: Simplest First Tesla Coil Design

Hi everyone,

I'm new to this list and am planning to build my (sort of first) tesla

coil. I've made a few other attempts at solid state tesla coils (using

a flyback transformer) with very poor results. Now I'm ready to make a

real tesla coil.  I've downloaded most of the archives and all of the

information I could find regarding chokes, capactors, etc. and have a

couple of questions:

For my first coil, should I use:

Salt Water, PVC (oil), or "6 pack" (glass jars with oil) capacitors?

I'm planning to go with a Neon sign transformer, sound good?

Should I buy or build chokes? I'd like to go cheap :) (married)

My first attempts were made using a book called "Phasers, Lasers, and

other High Voltage Electronic Devices" (or something like that) in

which there were two different tesla coil experiments. Right now I'm

looking at the 250KV design which doesn't mention building your own

high voltage caps. Also, it says you should not run the coil for more

than 30 seconds at a time. Is this because of the poor commercial HV

capacitors they use in the circuit which are probably getting hot?

Thanks!

Tom Stock

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Mon,  9 Mar 1998 17:19:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Mon, 9 Mar 1998 11:19:24 -0600

Subject: Re: Simplest First Tesla Coil Design

>     Salt Water, PVC (oil), or "6 pack" (glass jars with oil) capacitors?

>     I'm planning to go with a Neon sign transformer, sound good?

>     Should I buy or build chokes? I'd to go cheap :) (married)

Poly - its just as easy as SW caps and much better.  Build your own chokes.

>     looking at the 250KV design which doesn't mention building your own

>     high voltage caps. Also, it says you should not run the coil for more

>     than 30 seconds at a time. Is this because of the poor commercial HV

>     capacitors they use in the circuit which are probably getting hot?

All capacitors will get hot, poly, glass, or commercial.  This general rule

of thumb carried over to all kinds of caps used in Tesla coil circuits.

Salt water capacitors tend to be able to take the heat much better though.

I ran my coil for 15 minutes nonstop, and a friend of mine ran his coil for

an hour nonstop using salt water capacitors.  I would dare not run my poly

cap longer than 2 minutes at the very most.

So, do you want better perforamce, or longer runtimes?  Thats what will

decide your capacitor design.  Have you ever noticed how life has trade-offs

like this?  Damn I hate that. :-)

(Personally I would take the poly caps for short run times, good performance.)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: human tricks

Date: Mon,  9 Mar 1998 17:53:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 09 Mar 1998 09:53:54 -0800

Subject: Re: human tricks

Tesla-2 wrote:

> >>

> >> Go for the basic hold a copper pipe and draw a spark to it.  The more area

> >> of your hand on the pipe, the less it will shock you.  This is because the

> >> electricity is more spread out, rather than having a single entry point.

>

> This is not a very smart thing to do. A very good friend of mine and an

> experienced coiler injured himself doing just this.

I think I will try a broomhandle with a set of keys on the end

>

>

> He was standing on a plastic milk crate drawing arcs to a piece of

> pipe held in his hand. All was OK until a streamer broke out from his

> elbow and connected to the furnace duct. This placed his arm directly

> in the arc channel to ground. He received a very severe shock and fell

> off the milk crate onto the concrete floor breaking his wrist.

> His other arm that took the streamer hit remained numb and almost

> immobile for a couple days.

>

> Needless to say he no longer takes streamers to the body and

> his story convinced me of the foolishness of doing so.

>

> >

> >Most of these trick sound great, I'm not sure about drawing a spark with a

> >copper rod though, I've tried it with a very low powered coil(tv flyback ss)

,

> >but reading the other messages I have some concerns about my safety with this

> >larger one I built. Anyone know what speed film I should use?

>

> The mistake most beginners make in taking pictures of their coils running

> is to take the picture in total darkness. This results in a too long

> exposure that shows nothing but a mass of streamers.

>

> Light up the wall behind your coil dimly with a lamp and reflector.

> If your camera has a light meter set the exposure to record the

> wall well. This will take about 1/4 - 1/2 sec at F 5.6 using ASA 400

> film. A tripod and remote cable release for the camera is a must.

>

> This will record the background fairly well and the streamers

> will show up quite well as bright against the dimmer background.

>

> Take a look at the pics on my web page to see how this looks.

> http://www.misslink.net/mhammer/

>

> One other note. Dont take your super fancy microelectronic

> do everything camera anywhere near your operating coil.

> Instead invest in a totally mechanical camera like my trusty

> old Olympus OM-1 or maybe a Pentax K-1000. Both of these

> are available rather cheaply used.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Mon,  9 Mar 1998 18:34:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 9 Mar 1998 13:34:05 -0500

Subject: Re: Simplest First Tesla Coil Design

>So, do you want better perforamce, or longer runtimes?  Thats what will

>decide your capacitor design.  Have you ever noticed how life has

>trade-offs like this?  Damn I hate that. :-)

>(Personally I would take the poly caps for short run times, good

>performance.)

Jeff, how much performance increase does one normally get when

switching to poly caps from SW? A simple comparison would be ok (my

sparks went from 12" to 16"!)...

Oh yeah, and I read somewhere that poly caps usually cost about $100 a

piece...in this case I may have to start with the Saltwater caps.

Thanks!

Tom Stock

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Introduction and Circuit Questions

Date: Mon,  9 Mar 1998 19:02:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Mon, 9 Mar 1998 12:02:19 -0700 (MST)

Subject: Introduction and Circuit Questions

Hey all!

I'm totally new to this list and am in the process of building my first

Tesla coil. I have built a few high voltage circuits using flyback

transformers with moderate to no success, but that is about all my

experience with HV.

I would like to build a rather small coil first off to get the basic idea

and my technique down. I've already wound my secondary which is 1000' of 28

awg wire on a 3.5 inch diameter, 18 inch long pvc pipe. I'm planning on

using saltwater caps to start out and later (hopefully) upgrading to poly

ones. I still haven't got a hold of a neon sign transformer (the small town

I live in doesn't have neon shops), but that is what I am planning on using

for the HV source. I have downloaded all the information I can get, and

still have a few questions.

1) Okay, there seems to be much difference between the actual Tesla coil

schematics that I have found. Some place the main capacitor(s) in parallel

with the HV source and the spark gap in series after the caps. Others

switch the spark gap and cap: the caps are in parallel and the spark gap is

in series after. What gives? Which design is best?

2) With a small coil like mine, are all the safety spark gaps, inductors,

etc needed? Can I go with just one or the other?

3) Exactly how long and what diameter should the inductors be? (the pipe

and the wire).

4) For my size of coil, with kind of primary design is best? The flat

pancake style, or the saucer one that is at on angle of 15-or-so degrees?

Thanks in advance for any of the questions you can answer!

Daniel Lockhart

p.s. Does anybody know how many people subscribe to this list?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Mon,  9 Mar 1998 19:03:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 9 Mar 1998 11:03:22 -0800 (PST)

Subject: Re: Simplest First Tesla Coil Design

On  9 Mar, Tesla-2 wrote:

> Poly - its just as easy as SW caps and much better.  Build your own chokes.

But not as cheap. And sorry, but not as easy, if done right. But

DEFINITELY worth doing.

> All capacitors will get hot, poly, glass, or commercial.  This general rule

> of thumb carried over to all kinds of caps used in Tesla coil circuits.

> Salt water capacitors tend to be able to take the heat much better though.

I disagree bigtime here. If the cap is built correctly, the amount

of heating can be very low.

> I ran my coil for 15 minutes nonstop, and a friend of mine ran his coil for

> an hour nonstop using salt water capacitors.  I would dare not run my poly

> cap longer than 2 minutes at the very most.

I have run my poly caps for upwards of 20 minutes doing various things

and the oil did not even get warm. But then I have pretty substantial

bussing in place. ( The wife cut that short, as the noise was "DRIVING

ME NUTS"  *shrug*)

On the other hand, I blew a saltwater cap after 5 minutes of operation

and they always get noticably warm.

Fr. Tom ( a member of this list, and a very experienced person)

is known to run his coils for multiple hours. Some people have all the luck.

>

> So, do you want better perforamce, or longer runtimes?  Thats what will

> decide your capacitor design.  Have you ever noticed how life has trade-offs

> like this?  Damn I hate that. :-)

> (Personally I would take the poly caps for short run times, good performance.)

Poly is one of those materials that gives you both. Better performance,

and longer run times.

This is because the loss factor is the source of *most* heating.

so at 0.001 vs 80, poly wins over glass every time.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introduction and Circuit Questions

Cc:

Bcc:

X-Attachments:

At 07:02 PM 3/9/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Mon, 9 Mar 1998 12:02:19 -0700 (MST)

>Subject: Introduction and Circuit Questions

>

>Hey all!

>

>I'm totally new to this list and am in the process of building my first

>Tesla coil. I have built a few high voltage circuits using flyback

>transformers with moderate to no success, but that is about all my

>experience with HV.

Welcome aboard Daniel.

>

>I would like to build a rather small coil first off to get the basic idea

>and my technique down. I've already wound my secondary which is 1000' of 28

>awg wire on a 3.5 inch diameter, 18 inch long pvc pipe. I'm planning on

>using saltwater caps to start out and later (hopefully) upgrading to poly

>ones. I still haven't got a hold of a neon sign transformer (the small town

>I live in doesn't have neon shops), but that is what I am planning on using

>for the HV source. I have downloaded all the information I can get, and

>still have a few questions.

Your secondary sounds fine.

A 9kv/30ma neon would be great for a small coil. I managed 14 inches

of output using a 9kv/30ma on my old 3 inch diamter coil using SW caps.

>

>1) Okay, there seems to be much difference between the actual Tesla coil

>schematics that I have found. Some place the main capacitor(s) in parallel

>with the HV source and the spark gap in series after the caps. Others

>switch the spark gap and cap: the caps are in parallel and the spark gap is

>in series after. What gives? Which design is best?

Performance wise there is no difference. Either configuration

will give identical results. Placing the spark gap directly

across the transformer offers a level of protection against

RF energy entering the transformer secondary winding.

After the gap fires and the tank circuit is oscillating

the transformer secondary is protected by the energized gap.

The energized gap is essentially a short circuit.

This doesnt offer foolproof protection but every little bit helps.

Placing the cap across the transformer puts the transformer

secondary directly in parallel with the high RF voltages

in the cap. A recipe for short transformer life. Especially with neons.

>

>2) With a small coil like mine, are all the safety spark gaps, inductors,

>etc needed? Can I go with just one or the other?

You can sometimes get by without RF chokes but I wouldnt recommend

it. Nothing is more demoralizing than destroying your transformer.

Neons need all the protection we can give them.

>

>3) Exactly how long and what diameter should the inductors be? (the pipe

>and the wire).

I dont recommend air core chokes wound on PVC pipe unless you

have no other choice. The best bet is a toroidal choke wound

on a large ferrite toroid. You need a minimum of 2 mh of inductance

but this value is not critical. Anything from 2-10 mh is fine.

My chokes are about 120 turns of AWG 26 wound on surplus 2.5 inch

diameter ferrite toroids. Ferrite rods will also work fine.

For air core chokes wind about 350 turns of AWG 26 on 2 inch PVC

pipe. This will result in a 6 inch winding and about 2.5 mh of

inductance. Like I said earlier the value is not critical. Juggle

dimensions to suit.

Air core chokes will often create additional problems as they have

a tendancy to ring up like mini Tesla coils. This will create the very

problem you are trying to correct. The cure is to add some series

resistance to the air core chokes to dampen any oscillations.

The resistor value is not critical. Anything from 300-1500 ohms

and with a sufficient power rating will do.

>

>4) For my size of coil, with kind of primary design is best? The flat

>pancake style, or the saucer one that is at on angle of 15-or-so degrees?

The type of primary is not really that important. The various primary

shapes differ only in the amount of coupling they provide. A flat spiral

offers the lowest amount of coupling while a solenoidal type of primary

offers the most. The saucer shape is a compromise between the two.

Once you make allowance for the difference in coupling any of the shapes

will provide good performance.

I would concern myself mainly with what shape would be easiest for

you to build and to provide sufficient inductance. I would recommend

a minimum of 12 turns and 14 or 15 would not be too much.

>

>Thanks in advance for any of the questions you can answer!

>

>Daniel Lockhart

>

>p.s. Does anybody know how many people subscribe to this list?

>

At last count (Monday morning) there were 155 subscribers with 4 new ones pending.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Mon,  9 Mar 1998 21:09:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 09 Mar 1998 13:09:19 -0800

Subject: Re: Simplest First Tesla Coil Design

>      Hi everyone,

>

>      I'm new to this list and am planning to build my (sort of first) tesla

>      coil. I've made a few other attempts at solid state tesla coils (using

>      a flyback transformer) with very poor results. Now I'm ready to make a

>      real tesla coil.  I've downloaded most of the archives and all of the

>      information I could find regarding chokes, capactors, etc. and have a

>      couple of questions:

>

>      For my first coil, should I use:

>

>      Salt Water, PVC (oil), or "6 pack" (glass jars with oil) capacitors?

>      I'm planning to go with a Neon sign transformer, sound good?

>      Should I buy or build chokes? I'd to go cheap :) (married)

>

for your first coil make glass plate caps, the're dirt cheap, as for chokes I

use two 10 uH chokes with two surge absorbing line caps on the 120V side, it does

a good job of eliminating line interference

>      My first attempts were made using a book called "Phasers, Lasers, and

>      other High Voltage Electronic Devices" (or something like that) in

>      which there were two different tesla coil experiments. Right now I'm

>      looking at the 250KV design which doesn't mention building your own

>      high voltage caps. Also, it says you should not run the coil for more

>      than 30 seconds at a time. Is this because of the poor commercial HV

>      capacitors they use in the circuit which are probably getting hot?

>

>      Thanks!

>      Tom Stock

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Simplest First Tesla Coil Design

Cc:

Bcc:

X-Attachments:

At 05:19 PM 3/9/98 GMT, you wrote:

>From: "Jeff Corr" <corr@enid.com>

>Date: Mon, 9 Mar 1998 11:19:24 -0600

>Subject: Re: Simplest First Tesla Coil Design

>

>

>

>All capacitors will get hot, poly, glass, or commercial.  This general rule

>of thumb carried over to all kinds of caps used in Tesla coil circuits.

>Salt water capacitors tend to be able to take the heat much better though.

>I ran my coil for 15 minutes nonstop, and a friend of mine ran his coil for

>an hour nonstop using salt water capacitors.  I would dare not run my poly

>cap longer than 2 minutes at the very most.

>

This is not true. Properly designed and built polyethylene or

polypropylene caps will run stone cold at even quite high power levels.

The reason is the nearly zero RF dissipation factor of poly.

If your poly caps are showing signs of heating that is an indication

of a problem. Probably corona due to trapped air. Corona will lead

to early failure. Thats the primary reason that single caps should

not be used over 9kv. For voltages above 9kv use series caps.

2 in series for 12kv and 3 in series for 15kv. That way no single

cap sees more than 9kv at any time and corona will not form.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Component sources

Date: Mon,  9 Mar 1998 21:49:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 9 Mar 1998 16:49:45 -0500

Subject: Component sources

Ok I'm ready to order some supplies. Does anyone have any favorite

electronics supply stores?

A good cheap source of magnet wire would be great.

Thanks

Tom Stock

tom.stock@gsbsc.gensig.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Simplest First Tesla Coil Design

Cc:

Bcc:

X-Attachments:

At 06:34 PM 3/9/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Mon, 9 Mar 1998 13:34:05 -0500

>Subject: Re: Simplest First Tesla Coil Design

>

>      how much performance increase does one normally get when

>     switching to poly caps from SW? A simple comparison would be ok (my

>     sparks went from 12" to 16"!)...

>

Getting increased performance from your coil is a process of eliminating

bottlenecks. All the various system losses add up. What you need to do

is address the greatest sources of loss first. Working to eliminate

minor losses while letting the big ones remain will do little good.

People often put extreme effort into building a super secondary

coil only to team it up with a poor spark gap and lossy caps.

The best lowest loss secondary will perform poorly in a coil

with a poor primary circuit. A mediocre secondary will perform

well when used with an efficient low loss primary system.

The primary system is where the effort must go. The single

greatest source of loss is the gap system. Anything you do here

to improve gap efficiency will pay big dividends. That means series

gaps will large rounded electrodes and plenty of forced air cooling.

Next up is RF loss in your capacitor dielectric if you are using a lossy

cap like SW caps or glass plate caps.

If you have other large losses in your system then changing from

glass caps to poly may not have a great effect in spark length.

If most of your system loss is in your caps then you will get

a larger benefit.

What you will notice however is that the streamers will be brighter,

bluer, and noisier.

Other important losses to consider are I2R and corona losses

in the primary circuit conductors. Minimizing this requires large

rounded conductors like copper tubing for the primary coil and

heavy conductors for all wiring in the tank circuit. This includes

wiring from spark gap to capacitor to primary coil.

>     Oh yeah, and I read somewhere that poly caps usually cost about $100 a

>     piece...in this case I may have to start with the Saltwater caps.

>

>

My first poly cap was a .008 mfd flate plate made with aluminum

foil, 4 mil polyethylene and mineral oil. Total cost was

quite low. The roll of poly cost $8 and I used less than a

third of it. The foil was a couple bucks from the grocery store.

I bought 10 pints of mineral oil from the drugstore for $1.45 per pint.

I used some scrap plywood and plexiglass to make the tank.

I sealed the tank with RTV silicone rubber.

This cap worked very well despite some rather rough usage.

It survives and continues to work well today nearly 3 years

later. Check out the file  mikecap.txt on Dave Bronakowski's

archive site for some additonal information on how I built this cap.

The URL for Dave's site is: http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Want to buy a 450W transformer

Date: Tue, 10 Mar 1998 00:16:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 09 Mar 1998 16:16:11 -0800

Subject: Re: Want to buy a 450W transformer

I have a 15Kv 30ma neon sign transformer for sale, highest bid plus shipping gets it.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Tue, 10 Mar 1998 00:49:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 09 Mar 1998 16:49:36 -0800

Subject: Re: Simplest First Tesla Coil Design

Tesla-2 wrote:

> From: <Tom.Stock@gsbsc.gensig.com>

> Date: Mon, 9 Mar 1998 13:34:05 -0500

> Subject: Re: Simplest First Tesla Coil Design

>

>

> >So, do you want better perforamce, or longer runtimes?  Thats what will

> >decide your capacitor design.  Have you ever noticed how life has

> >trade-offs like this?  Damn I hate that. :-)

> >(Personally I would take the poly caps for short run times, good

> >performance.)

>

>      Jeff, how much performance increase does one normally get when

>      switching to poly caps from SW? A simple comparison would be ok (my

>      sparks went from 12" to 16"!)...

>

How about a comparison from glass to poly?

>      Oh yeah, and I read somewhere that poly caps usually cost about $100 a

>      piece...in this case I may have to start with the Saltwater caps.

>

>      Thanks!

>      Tom Stock

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Tue, 10 Mar 1998 03:51:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Mon, 9 Mar 1998 21:51:33 -0600

Subject: Re: Simplest First Tesla Coil Design

My caps are not showing signs of heating, as far as I can tell, so

what run times would you say would be safe on them?

>This is not true. Properly designed and built polyethylene or

>polypropylene cap will run stone cold at even quite high power levels.

>The reason is the nearly zero RF dissipation factor of poly.

>If your poly caps are showing signs of heating that is an indication

>of a problem. Probably corona due to trapped air. Corona will lead

>to early failure. Thats the primary reason that single caps should

>not be used over 9kv. For voltages above 9kv use series caps.

>2 in series for 12kv and 3 in series for 15kv. That way no single

>cap sees more than 9kv at any time and corona will not form.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Want to buy a 450W transformer

Date: Tue, 10 Mar 1998 05:49:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 09 Mar 1998 23:49:12 -0600

Subject: Re: Want to buy a 450W transformer

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Mon, 09 Mar 1998 16:16:11 -0800

> Subject: Re: Want to buy a 450W transformer

>

>   I have a 15Kv 30ma neon sign transformer for sale, highest bid plus shipping

> gets it.

>

I can get my neons for less than $10 locally but I will offer you $20 + shipping.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Simplest First Tesla Coil Design

Cc:

Bcc:

X-Attachments:

At 03:51 AM 3/10/98 GMT, you wrote:

>From: "Jeff Corr" <corr@enid.com>

>Date: Mon, 9 Mar 1998 21:51:33 -0600

>Subject: Re: Simplest First Tesla Coil Design

>

>My caps are not showing signs of heating, as far as I can tell, so

>what run times would you say would be safe on them?

>

Run times will most likely not be limited by a poly cap.

The limiting factor is usually insufficent cooling of the

gap. If your coil runs great for 10 or 15 seconds then the output

begins to drop off thats a sure sign of the gap overheating.

A hot gap will not want to quench decreasing performance.

The other factor limiting run times is coronal heating

inside your transformer secondary. This leads to hot spots

and transformer failure. Neons are not suited for long run

times in Tesla coil service.

Run times of 2 to 3 minutes followed by a period for cooling

off should not be any problem.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Component sources

Cc:

Bcc:

X-Attachments:

At 09:49 PM 3/9/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Mon, 9 Mar 1998 16:49:45 -0500

>Subject: Component sources

>

>

>

>     Ok I'm ready to order some supplies. Does anyone have any favorite

>     electronics supply stores?

Tom,

As I just recently posted here on the list my favorite source

for parts is hamfests. The hamfest season is just getting started.

If you dont know of any in your area check out the ARRL web site.

The URL for the hamfest calender is: http://www.arrl.org/hamfests.html

One of my favorite mail order surplus places is MECI. They have

a web page at: http://www.meci.com/

I have links to some other suplus electronics dealers on my web page.

http://www.misslink.net/mhammer/links.htm

>

>     A good cheap source of magnet wire would be great.

The best source for magnet wire will be local electric motor

rebuilding shop. Check your yellow pages under electric motors.

I get most of my wire from a local shop at prices hard to beat.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Simplest First Tesla Coil Design

Date: Tue, 10 Mar 1998 07:57:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Tue, 10 Mar 1998 01:57:16 -0600

Subject: Re: Simplest First Tesla Coil Design

>Run times of 2 to 3 minutes followed by a period for cooling

>off should not be any problem.

So assuming the poly cap is constructed correctly, I should not have

to worry about it overheating?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safe Coils

Date: Tue, 10 Mar 1998 11:15:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 10 Mar 1998 16:15:54 +0500

Subject: Re: Safe Coils

On Sun, 08 Mar 1998 13:57:38 -0600 In Tesla-2

Robert C. McGrath <rmcgra1@airmail.net> wrote;

> Hi, I was wondering what voltage would be safe so that you could

> actually touch the secondary..In my electronics class, we had a small

> one, but I don't remember the specs...thanx,

Robert, it is not the voltage that is of concern but the current and the

frequency! Dr. Tesla gave many demonstrations in which he allowed the

discharge from a 1 million + volt coil to pass through his body but the

current was less than a few milliamps (ma.)and the frequency was over 100

kHz. The higher the frequency the greater the tendancy for the current to

travel over the body rather than through it, this is called the skin

effect.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Introduction and Circuit Questions

Date: Tue, 10 Mar 1998 11:15:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 10 Mar 1998 16:15:55 +0500

Subject: Re: Introduction and Circuit Questions

On Mon, 9 Mar 1998 12:02:19 -0700 (MST) In Tesla-2

Daniel Lockhart <molly@cmn.net> wrote;

> Hey all!

>

> I'm totally new to this list and am in the process of building my first

> Tesla coil. I have built a few high voltage circuits using flyback

> transformers with moderate to no success, but that is about all my

> experience with HV.

>

> I would like to build a rather small coil first off to get the basic idea

> and my technique down. I've already wound my secondary which is 1000' of 28

> awg wire on a 3.5 inch diameter, 18 inch long pvc pipe. I'm planning on

> using saltwater caps to start out and later (hopefully) upgrading to poly

> ones. I still haven't got a hold of a neon sign transformer (the small town

> I live in doesn't have neon shops), ...

Welcome to the list Daniel! While your looking for a neon sign transformer

you may consider an OBIT (Oil Burner Ignition Transformer) they are

typically 7,500 volts at 23 ma. (you can put several in parallel for

more current but DO NOT put them in series or you will burn out their

secondaries for sure) You can either find them in the trash connected to

defunct oil burners or ask at a heater repair or supply house, even some

oil distributers carry them.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Component sources

Date: Tue, 10 Mar 1998 21:26:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Moleking <moleking@concentric.net>

Date: Tue, 10 Mar 1998 16:26:29 -0500

Subject: Re: Component sources

At 04:49 PM 3/9/98 , you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Mon, 9 Mar 1998 16:49:45 -0500

>Subject: Component sources

>

>

>     Ok I'm ready to order some supplies. Does anyone have any favorite

>     electronics supply stores?

>

>     A good cheap source of magnet wire would be great.

I recently bought magnet wire from Gary Weaver.  The prices are pretty

good.  Email him at gweaver@earthlink.net.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Want to buy a 450W transformer

Date: Wed, 11 Mar 1998 01:55:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Ryan <jryan@exis.net>

Date: Tue, 10 Mar 1998 20:55:48 -0500

Subject: Re: Want to buy a 450W transformer

At 05:49 AM 3/10/98 GMT, you wrote:

>From: Zuma /or CJ <mwise@ns.sosis.com>

>Date: Mon, 09 Mar 1998 23:49:12 -0600

>Subject: Re: Want to buy a 450W transformer

>

>Tesla-2 wrote:

>>

>> From: mike nicholas <705728@ican.net>

>> Date: Mon, 09 Mar 1998 16:16:11 -0800

>> Subject: Re: Want to buy a 450W transformer

>>

>>   I have a 15Kv 30ma neon sign transformer for sale, highest bid plus shipping

>> gets it.

>>

>

>I can get my neons for less than $10 locally but I will offer you $20 +

>shipping.

>

>Chris

>

>

Humm, do it work ?

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Want to buy a 450W transformer

Date: Wed, 11 Mar 1998 06:34:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 10 Mar 1998 22:34:41 -0800

Subject: Re: Want to buy a 450W transformer

Tesla-2 wrote:

> From: Jim Ryan <jryan@exis.net>

> Date: Tue, 10 Mar 1998 20:55:48 -0500

> Subject: Re: Want to buy a 450W transformer

>

> At 05:49 AM 3/10/98 GMT, you wrote:

> >From: Zuma /or CJ <mwise@ns.sosis.com>

> >Date: Mon, 09 Mar 1998 23:49:12 -0600

> >Subject: Re: Want to buy a 450W transformer

> >

> >Tesla-2 wrote:

> >>

> >> From: mike nicholas <705728@ican.net>

> >> Date: Mon, 09 Mar 1998 16:16:11 -0800

> >> Subject: Re: Want to buy a 450W transformer

> >>

> >>   I have a 15Kv 30ma neon sign transformer for sale, highest bid plus shipping

> >> gets it.

> >>

> >

> >I can get my neons for less than $10 locally but I will offer you $20 +

> >shipping.

> >

> >Chris

> If you can buy it for less than $10, why would you pay $20?

> >

> >

> >Humm, do it work ?

>

> Jim

>

It works, made by Allanson, 15KV 30ma

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: ADMIN-replying to posts

Cc:

Bcc:

X-Attachments:

Hello all,

It appears that its time once again to remind everyone

to not overquote when replying to posts on the list.

The list server appends the following info to each post:

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

Please delete this from your quoted text.

If you leave it in there will just be another copy

of it appended to your reply.

A good illustration of this was the two posts with

the subject "Want to buy a 450W transformer"

posted by Jim Ryan and Mike Nicholas.

In Jim's reply those lines appeared 3 times and then

appears 4 times in Mike's reply.

Also only quote those parts that are relevant to your response.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Neon transformers & caps passing AC

Date: Wed, 11 Mar 1998 12:56:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Ringwood <ccntesla@yahoo.com>

Date: Wed, 11 Mar 1998 04:56:30 -0800 (PST)

Subject: Neon transformers & caps passing AC

To All

Does anyone have any neon transformers with a 240v primary they want

to sell, because I live in the UK. Also, why is it that when you use a

capacitor in parallel with your neon supply that the capacitor doesn't

just act as a resistor & pass the AC? I ask this as capacitors have an

AC resistance (reactance)

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Neon Transformers?

Date: Wed, 11 Mar 1998 14:04:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 11 Mar 1998 09:04:39 -0500

Subject: Neon Transformers?

Are Neon sign transformers the same as Neon light transformers? I'm

guessing not, but are neon light transformers any good for anything?

I have one that says "120V 60Hz 40W 460ma max light tube current"

It lights two 40W 4Ft neon lights, but doesn't say anywhere on it what

the output voltage and current is. It's black, and about 7" long by 3"

wide.

Tom Stock

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Neon Transformers?

Date: Wed, 11 Mar 1998 16:02:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Wed, 11 Mar 1998 10:02:11 -0600

Subject: Re: Neon Transformers?

Tom:

It sounds like you have a fluorescent lamp ballast, NOT a neon

sign/light transformer.  First of all 460ma is way too high for neon

lights (even cold cathode lamps don't run that high).  Plus, the 40W 4

ft sounds like a fluorescent lamp to me. Fluorescent lamps are not the

same as neon lights, and require much different auxiliary equipment to

run them.  All fluorescent lamps, except the single pin 'slimline' lamps

have electrically heated cathodes, powered by tapped windings on the

ballast. The slimline ballasts are about as close to a NST as you get in

a fluorescent lamp ballast. They generally put out around 700-800 volts

and 460 ma for a 8 ft lamp. IMHO not good for much Tesla coil-wise. Hope

this helps!

Eric

edavidson@icva.gov

>      Are Neon sign transformers the same as Neon light transformers? I'm

>      guessing not, but are neon light transformers any good for anything?

>

>      I have one that says "120V 60Hz 40W 460ma max light tube current"

>

>      It lights two 40W 4Ft neon lights, but doesn't say anywhere on it what

>      the output voltage and current is. It's black, and about 7" long by 3"

>      wide.

>

>      Tom Stock

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Neon Transformers?

Date: Wed, 11 Mar 1998 16:09:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Wed, 11 Mar 1998 08:09:37 -0800 (PST)

Subject: Re: Neon Transformers?

On 11 Mar, Tesla-2 wrote:

>

>      Are Neon sign transformers the same as Neon light transformers? I'm

>      guessing not, but are neon light transformers any good for anything?

>

>      I have one that says "120V 60Hz 40W 460ma max light tube current"

>

>      It lights two 40W 4Ft neon lights, but doesn't say anywhere on it what

>      the output voltage and current is. It's black, and about 7" long by 3" wide.

Sounds like you are talking about floresent tubes, not neon. The current

is an order of magnitude too high, and my guess the voltage too low.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Neon Transformers?

Date: Wed, 11 Mar 1998 16:58:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 11 Mar 1998 08:58:30 -0800 (PST)

Subject: Re: Neon Transformers?

At 02:04 PM 3/11/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Subject: Neon Transformers?

>     Are Neon sign transformers the same as Neon light transformers?

>     ...are neon light transformers any good for anything?

>     I have one that says "120V 60Hz 40W 460ma max light tube current"

>     It lights two 40W 4Ft neon lights...

Tom -

Those aren't neon lights - those are FLOURESCENT lights. Big difference.

And that black box is commonly known as a "ballast." And yes, it is good for

something - exciting the gas in 4ft 40W florescent light tubes...

Dan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Neon Transformers?

Date: Wed, 11 Mar 1998 17:14:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 11 Mar 1998 11:14:22 -0600

Subject: Re: Neon Transformers?

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>     Are Neon sign transformers the same as Neon light transformers? I'm

>     I have one that says "120V 60Hz 40W 460ma max light tube current"

>     It lights two 40W 4Ft neon lights, but doesn't say anywhere on it what

This sounds like a ballast transformer used to power

two fluorescent light bulbs.

If you take the 40W rating and divide it by 0.460

amps, you get an output voltage of about 85 volts.

Not too useful for tesla coils!

But I could be mistaken.

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Neon transformers & caps passing AC

Cc:

Bcc:

X-Attachments:

At 12:56 PM 3/11/98 GMT, you wrote:

>From: Chris Ringwood <ccntesla@yahoo.com>

>Date: Wed, 11 Mar 1998 04:56:30 -0800 (PST)

>Subject: Neon transformers & caps passing AC

>

>To All

>Does anyone have any neon transformers with a 240v primary they want

>to sell, because I live in the UK.

Neons with 240V primaries are not too common here in the USA.

Sometimes ones with 270 V primaries will show up.

The easiest thing for you to do is to use 120 V neons

in pairs. Wire the primaries in series and feed them 240 volts.

Wire up the secondaries in parallel.

You could also invest in a 240 to 120 volt isolation transformer.

These are pretty common as surplus.

> Also, why is it that when you use a capacitor in parallel with your neon

> supply that the capacitor doesn't just act as a resistor & pass the AC?

> I ask this as capacitors have an AC resistance (reactance)

If you had just a cap hanging on your neon that would be the case.

The reactance would then depend on the frequency (50 or 60 hz)

and the capacitance.

However in a Tesla coil you dont have just a capacitor.

You have a spark gap driven resonant circuit formed from the

main cap and primary coil.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Neon Transformers?

Date: Wed, 11 Mar 1998 17:36:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 11 Mar 1998 12:36:22 -0500

Subject: Re: Neon Transformers?

>They generally put out around 700-800 volts and 460 ma for a 8 ft lamp.

>IMHO not good for much Tesla coil-wise. Hope this helps!

>Eric

Thanks Eric! Time to start calling Neon sign places...

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Member List

Date: Wed, 11 Mar 1998 19:48:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Choad" <choad69@geocities.com>

Date: Wed, 11 Mar 1998 13:48:24 -0600

Subject: Member List

Everyone,

I will be compiling a list of everyone on the Tesla-2 list. Please Send

me your Name, E-Mail, State, and City if you wish for me to add you to the

list. I believe this will help bring coilers in the same area together so

that they might help one another and make bigger and better sparks. Thank

you and have a nice Day.

Shaun O'Brien

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Good to be in the CIRCUIT!

Date: Wed, 11 Mar 1998 20:40:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 11 Mar 1998 12:40:11 -0800

Subject: Good to be in the CIRCUIT!

Organization: Self

All:

Looks like I've been wired in!  Looking forward to more of your informed out put!

I located at least 2 ganged-3-element 240v (in) VARIACs.  They want $100 ea.

I understand they will work fine on 110v in.  They could be

mechanically and/or electrically separated, I'm sure. Comments, interest?

Also a big cap: Comm'l, I don't recall exact specs but about 24 kv @

.oo2 mfd or somwhere close; stand off's about 3" tall, grey can, about

12x5x18". More detail if needed.

_Another Igor_

Gene Waldo   gwaldo@1stnetusa.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Wed, 11 Mar 1998 22:40:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Wed, 11 Mar 1998 14:40:45 -0800

Subject: Re: Microwave transformers

I was wondering if microwave Xformers could be used in tesla design?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Thu, 12 Mar 1998 00:47:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Wed, 11 Mar 1998 19:47:45 EST

Subject: Re: Microwave transformers

Mike,

As long as the output voltage is greater than about 2000V, It should work.

This is about the minimum that a Tesla coil can work on(although I have an old

tube driven TC that runs on about 750V).  The 2000V is for spark gap excited

coils.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Microwave transformers

Cc:

Bcc:

X-Attachments:

At 10:40 PM 3/11/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Wed, 11 Mar 1998 14:40:45 -0800

>Subject: Re: Microwave transformers

>

>> I was wondering if microwave Xformers could be used in tesla design?

>

Microwave oven transformers are not really suitable for powering

tesla coils. They have several major shortcomings.

First the voltage output is way too low for reliable spark gap

operation. The low voltage of approximately 2200 volts would require

very close spacing. The close spacing would also result in a very poor

quenching gap.

Second MW transformers supply too much current. Typically 300-500ma.

This high current will literally cook a spark gap and further degrades

the quenching of the gap.

Third the insulation on these transformers is quite poor.

Some coilers have had some success wiring several MW transformers

in series to boost the output voltage. This usually results in

failures among the transformers on the outside of the series string

due to insulation breakdown. Some of this can be suppressed by

immersing the transformers in oil.

However this does nothing about the extremely high current.

It would be best to treat MW transformers as not having any current

limiting at all and supplying external ballasting to control the current.

So yes than can be used but the difficulty in making reliable

use of them makes it an unwise choice.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Thu, 12 Mar 1998 01:21:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Wed, 11 Mar 1998 20:21:33 EST

Subject: Re: Microwave transformers

As Mike stated, there are difficulties in using microwave transformers.

One way to overcome this is to use a voltage multiplier circuit(made from HV

diodes and caps in one of several arrangements).  This will increase the

voltage and reduce the current.  The one thing to be careful of if using this

type of circuit is that the output is DC and must be discharged after

use(before touching anything) to prevent a potentially lethal shock.  I have a

power supply of this type that steps 2000V up to 10kV at 2000W.

If you are interested in this type of circuit, contact me at

Hollmike@aol.com and I will send you info on how to construct one.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: poly cap "test to the death!"

Date: Thu, 12 Mar 1998 01:51:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Wed, 11 Mar 1998 19:51:48 -0600

Subject: poly cap "test to the death!"

Today after working on my Tesla Coil, I decided test a cap, to its

death, just to see how long it would last.  The value was 3000pF,

and it was rated at 11kv (using 350 volts per mil).  It was connected

to a 9kv, 30ma neon sign transformer across the high voltage

terminals, and in parallel with the cap I placed a spark gap.  The cap

was in 2" PVC and was soaked in mineral oil.   (Time is in minutes

past.)

Start time - started the neon

5:15 - Spark gap volume lowered slightly, no visual damage to cap yet.

13:13 - Slight burnt smell, cap warm to touch

15:15 - RTV Silicon fails, dripping a small amount of oil.

16:40 - Oil leaking from both endcaps

30:40 - PVC *VERY* hot

34:35 - gap goes silent

34:40 - one endcap burst off throwing burnt debris all over my garage

Jeff Corr

2114 Monitor

Enid, OK 73703

http://www.harvestcomm.net/personal/corr/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Motor oil vs mineral oil

Date: Thu, 12 Mar 1998 06:19:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Wed, 11 Mar 1998 22:19:36 -0800

Subject: Re: Motor oil vs mineral oil

can I substitute mineral oil for motor oil in my poly caps?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Motor oil vs mineral oil

Date: Thu, 12 Mar 1998 05:25:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Wed, 11 Mar 1998 23:25:15 -0600

Subject: Re: Motor oil vs mineral oil

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Wed, 11 Mar 1998 22:19:36 -0800

> Subject: Re: Motor oil vs mineral oil

>

>  can I substitute mineral oil for motor oil in my poly caps?

I would not recommend using motor oil. It does not have the same

insulating effects that mineral oil has. Plus motor oil is not pure

depending on the brand there is no telling what is in it. I know mineral

oil is higher but it does a better job at suppressing corona.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Motor oil vs mineral oil

Cc:

Bcc:

X-Attachments:

At 06:19 AM 3/12/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Wed, 11 Mar 1998 22:19:36 -0800

>Subject: Re: Motor oil vs mineral oil

>

>    can I substitute mineral oil for motor oil in my poly caps?

>

I assume you meant your question the other way around.

You probably meant " Can I substitute motor oil for mineral oil?"

The answer to that question is no. Motor oils contain

many additives. Some of them are semi-metallic. There

are additives for antiwear, antifoam, and detergents.

These additives will greatly degrade the insulating properties

of the oil.

You need to use mineral oil or transformer oil.

Many of the silicone based oils like refrigeration oil

are also usable.

Check out Dave Bronakowski's archive page for the file

"oiltypes.txt". This file is a compilation of posts

from the list about various oils used in building caps.

The archive URL is: http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Chokes

Date: Thu, 12 Mar 1998 07:11:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 11 Mar 1998 23:11:29 -0800

Subject: Chokes

greetings,

I'm getting ready to start on my rf chokes for a 12kv/30ma nst coil. Ive been

looking at instructions in a book that I recently purchased that shows the wire

going FROM the nst THROUGH the middle of the pvc pipe and over to

a spark gap, like this:

--mmmmmmmmmm!

0---------------!-----------!----------->to tank

I N I     wwwwwwwwww            !

I E I                           0

I O I     chokes              gap

I N I                           0

I   I    mmmmmmmmmm!            !

o--------------!------------!------------>to tank

--wwwwwwwwww

the drawing that I have shows one end of the choke connecting to the wire passing

through it and the other hanging there in space.

I kind of thought that the choke was a component with one end connected to the

wire coming from the nst and the other end of it connected to the wire that goes

over to the gap. do you run the wire THROUGH THE PVC AND CONNECT AT ONLY ONE END?

I'm planning on using 2" x 8" long pvc and #26 wire.

Getting closer to completion every day now, thanks, Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: spark gap

Date: Thu, 12 Mar 1998 07:19:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 11 Mar 1998 23:19:27 -0800

Subject: spark gap

Hi, I'm looking for ideas for main spark gaps. I've studied the Richard

Quick design using 6"pvc pipe and 2" copper pipe, but it seems like it

would be a little difficult to adjust the gaps between the times

when you are tuning and running full out. I'm wondering what other handy

dandy designs and/or materials people have used that are easiy

adjustable and effective?

Thanks, Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap

Date: Thu, 12 Mar 1998 11:11:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Thu, 12 Mar 1998 11:11:28 GMT

Subject: Re: spark gap

Organization: White Wing Logic

On Thu, 12 Mar 1998 07:19:27 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Wed, 11 Mar 1998 23:19:27 -0800

>Subject: spark gap

>

>Hi, I'm looking for ideas for main spark gaps. I've studied the Richard

>Quick design using 6"pvc pipe and 2" copper pipe, but it seems like it

>would be a little difficult to adjust the gaps between the times

>when you are tuning and running full out.

I just use a short thick lead with a couple of crocodile clips,

clipped over the ends of the pipes to bypass all but one of the gaps

for low-power running.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: poly cap "test to the death!"

Date: Thu, 12 Mar 1998 13:00:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 12 Mar 1998 07:00:18 -0600

Subject: Re: poly cap "test to the death!"

Organization: MailExcite  (http://www.mailexcite.com)

I suppose a failure is a failure, however this sounds like overtemping, not

dielectric failure. My caps are rated to 15kv they are in series however.

During a typical runthey often see 23.5kv.  They have been in service since november

of last year and they have never gotten warm. Total capacitance .026uf. I suppose

I could severly overvolt them to failure. I would like to perform such a test to see

just what it would take to make them fail.

Paul M.

---

Einigkeit und Freiheit

>From: "Jeff Corr" <corr@enid.com>

>Date: Wed, 11 Mar 1998 19:51:48 -0600

>Subject: poly cap "test to the death!"

>

>Today after working on my Tesla Coil, I decided test a cap, to its

>death, just to see how long it would last.  The value was 3000pF,

>and it was rated at 11kv (using 350 volts per mil).  It was connected

>to a 9kv, 30ma neon sign transformer across the high voltage

>terminals, and in parrellel with the cap I placed a spark gap.  The cap

>was in 2" PVC and was soaked in mineral oil.   (Time is in minutes

>past.)

>

>Start time - started the neon

>5:15 - Spark gap volume lowered slightly, no visual damage to cap yet.

>13:13 - Slight burnt smell, cap warm to touch

>15:15 - RTV Silicon fails, dripping a small amount of oil.

>16:40 - Oil leaking from both endcaps

>30:40 - PVC *VERY* hot

>34:35 - gap goes silent

>34:40 - one endcap burst off throwing burnt debris all over my garage

>

>

>Jeff Corr

>2114 Monitor

>Enid, OK 73703

>http://www.harvestcomm.net/personal/corr/

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Chokes

Date: Thu, 12 Mar 1998 14:50:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 12 Mar 1998 06:50:50 -0800

Subject: Re: Chokes

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Wed, 11 Mar 1998 23:11:29 -0800

> Subject: Chokes

>

> greetings,

> I'm getting ready to start on my rf chokes for a 12kv/30ma nst coil. Ive been

>looking at instructions in a book that I recently purchased that shows the wire

>going FROM the nst THROUGH the middle of the pvc pipe and over to

>a spark gap, like this:

>         --mmmmmmmmmm!

>     0---------------!-----------!----------->to tank

> I N I     wwwwwwwwww            !

> I E I                           0

> I O I     chokes              gap

> I N I                           0

> I   I    mmmmmmmmmm!            !

>     o--------------!------------!------------>to tank

>        --wwwwwwwwww

>

> the drawing that I have shows one end of the choke connecting to the wire passing

> through it and the other hanging there in space.

>

> I kind of thought that the choke was a component with one end connected to the

> wire coming from the nst and the other end of it connected to the wire that goes

> over to the gap. do you run the wire THROUGH THE PVC AND CONNECT AT ONLY ONE END?

> I'm planning on using 2" x 8" long pvc and #26 wire.

>

> Getting closer to completion every day now, thanks, Walt Kulpa

>

Walt,

Whatever their function, they will not be performing as chokes. Chokes

require current flowing through the coiled wire in order to generate the

appropriate inductive effects. Quack medical equipment is certainly

interesting and fun to play with though... please keep us posted on your

progress, and play safe!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap

Date: Thu, 12 Mar 1998 18:25:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 12 Mar 1998 10:25:06 -0800

Subject: Re: spark gap

Tesla-2 wrote:

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Wed, 11 Mar 1998 23:19:27 -0800

> Subject: spark gap

>

> Hi, I'm looking for ideas for main spark gaps. I've studied the Richard

> Quick design using 6"pvc pipe and 2" copper pipe, but it seems like it

> would be a little difficult to adjust the gaps between the times

> when you are tuning and running full out. I'm wondering what other handy

> dandy designs and/or materials people have used that are easiy

> adjustable and effective?

> Thanks, Walt Kulpa

>

I like using an air-blast spark gap, you can use an adjustable static gap

and just have about 15 psi blowing through it, its very noisy but works

great!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gap

Cc:

Bcc:

X-Attachments:

At 07:19 AM 3/12/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Wed, 11 Mar 1998 23:19:27 -0800

>Subject: spark gap

>

>Hi, I'm looking for ideas for main spark gaps. I've studied the Richard

>Quick design using 6"pvc pipe and 2" copper pipe, but it seems like it

>would be a little difficult to adjust the gaps between the times

>when you are tuning and running full out. I'm wondering what other handy

>dandy designs and/or materials people have used that are easiy

>adjustable and effective?

>Thanks, Walt Kulpa

>

Walt,

For use with neons the cylinder type of gap popularized (not invented)

by Richard Quick is a great performer. When adequate airflow is used

it will handle quite high power levels and offer good quenching.

It doesnt have to be made in 6 inch PVC pipe. My current design has

10 electrodes of 3/4 inch copper pipe 3 inches long. These fit inside

a piece of 4 inch PVC. Air flow is provided by a surplus vacuum

cleaner motor. Slots are cut into the PVC housing between each electrode

to admit the airflow.

The cylinder gap is adjustable simply by using a smaller number

of the available gaps. You dont have to use all of them all the time.

My gap has 10 electrodes that provide 9 series gaps. The gap spacing

is .025 inch. That provides a maximum gap of .225 inch when all gaps

are used. I normally use this gap for with 12kv neons.

For low power operation or tuning I normally only use 6 of the gaps

for a total of .150 inch. I also use 9kv neons sometimes and use

a maximum of 7 the series gaps for a total of .175 inch.

You dont have to house the electrodes inside PVC pipe either.

You can mount them flat side by side on a piece of plexiglass

or lexan. However directing airflow for efficient cooling is easier

in the cylinder design.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Chokes

Cc:

Bcc:

X-Attachments:

At 07:11 AM 3/12/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Wed, 11 Mar 1998 23:11:29 -0800

>Subject: Chokes

>

>greetings,

>I'm getting ready to start on my rf chokes for a 12kv/30ma nst coil.

>Ive been looking at instructions in a book that I recently purchased

>that shows the wire going FROM the nst THROUGH the middle of the pvc pipe

>and over to a spark gap, like this:

>        --mmmmmmmmmm!

>    0---------------!-----------!----------->to tank

>I N I     wwwwwwwwww            !

>I E I                           0

>I O I     chokes              gap

>I N I                           0

>I   I    mmmmmmmmmm!            !

>    o--------------!------------!------------>to tank

>       --wwwwwwwwww

>

>the drawing that I have shows one end of the choke connecting to the wire passing

>through it and the other hanging there in space.

I dont know what the purpose of this might be but it certainly isnt

an RF choke. I dont believe what you describe would do anything at all.

Perhaps the actual purpose is to sell books?  :)

>

>I kind of thought that the choke was a component with one end connected to

>the wire coming from the nst and the other end of it connected to the wire

>that goes over to the gap.

You thought correctly.

>do you run the wire THROUGH THE PVC AND CONNECT AT ONLY ONE END?

No.

>I'm planning on using 2" x 8" long pvc and #26 wire.

Thats fine. Dont forget the safety gap. Its even more important

than RF chokes. The safety gap should be right at your transformer.

Next comes your RF chokes then the main gap. The RF chokes serve

to isolate the safety gap and main gap from each other.

Also suggested if you are going to use air core RF chokes

is some series resistance. Put one resistor in series with

each RF choke to dampen any oscillations that might occur.

Without these damping resistors air core chokes can ring up

quite high voltages like mini Tesla coils.

The resistor value is not critical. Anything from 300-1500

ohms will be fine. Wattage rating should be appropriate

for the current capability of your neons. I picked up a sack full

of 310 ohm 50 watt resistors at a recent hamfest for 10 cents each.

They work perfect for this.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Neon Transformers, It's so easy!

Date: Thu, 12 Mar 1998 19:46:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Thu, 12 Mar 1998 14:46:57 -0500

Subject: Neon Transformers, It's so easy!

It's so easy it's almost fun!

I called a Neon sign place today, and told the woman that answered the

phone about my project and what I was looking for ("Some dead neon

transformers") and she started getting excited and asked me details

about the tesla coil. She apparently has wanted to build a plasma ball

for a long time (she was one of the custom neon artists) and wants me

to bring her some info on doing it!

She's holding a couple of working 12kv transformers for me right now!

I'm going to pick them up tomorrow!

Happy Happy Joy Joy

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Variable Primary Capacitor, feasability study

Date: Thu, 12 Mar 1998 19:58:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 12 Mar 1998 11:58:09 -0800

Subject: Variable Primary Capacitor, feasability study

Organization: Self

All:

Quoting Tom McGahee, <Oiltypes.txt>  "But never be afraid to

experiment..."  I left out, "...on a small scale...", as that may be too

restrictive!

UP-side FEATURES:

INFINITELY variable capacitance.

Gross capacitance easily adjustable, (in)finitely  expandable.

Modular units easily replaceble, repairable,

mass-produceable.

James Whale would have liked it!

Bank of knife swithches to configure to any Tesla size or power.

Simple, accessable module connections.

Lends itself to ser & par'l setup connections.

Single 'Knob' tuning control.

DOWN-side FEATURES:

Large, fairly stationary device.

DISCUSSION:

I have not researced this, or seen the concept in any literature, so

this may still be 1) a tried & true approach;  2) a silly and unfeasable

approach;  3) original.

So, I will be prepared to snail copies of sketches of the idea to to 2

or 3 of you capacitor experts who may wish to assist in an analysis of

the concept.

If any of the features seem worthy of pursuit to any of you, send me

your snail address, and I will send copies of my sketches of the concept

and will welcome all critique and suggestions.

Mostly, If I have to wear a dunce cap, over it, the fewer of you who can

laugh and point, the better!  :-(

One lonely Igor, exercising knob over hunch!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Motor oil vs mineral oil

Date: Fri, 13 Mar 1998 02:31:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 12 Mar 1998 18:31:03 -0800

Subject: Re: Motor oil vs mineral oil

> >

> >    can I substitute mineral oil for motor oil in my poly caps?

> >

>

> I assume you meant your question the other way around.

> You probably meant " Can I substitute motor oil for mineral oil?"

>

> The answer to that question is no. Motor oils contain

> many additives. Some of them are semi-metallic. There

> are additives for antiwear, antifoam, and detergents.

>

> These additives will greatly degrade the insulating properties

> of the oil.

>

> You need to use mineral oil or transformer oil.

> Many of the silicone based oils like refrigeration oil

> are also usable.

It's a good thing I'm a refrigeration tech.   I wonder if vacum pump

oil will be suitable? Its very dry and very thin.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: New to Tesla

Date: Fri, 13 Mar 1998 02:53:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Michael Koerner" <mkoerner@fhjc.org>

Date: Thu, 12 Mar 1998 21:53:24 -0500

Subject: New to Tesla

This is my first post to Tesla-2, so I'll make it a good one.  Four

years ago, I saw one of the most amazing things my young eyes have seen.

One of my upper classmates made what looked like a gun for a science

project.  That gun when pointed at the schools scoreboard 50 feet way, made

it light up.  This year, as I was choosing my science project, that image

popped into my head.  I wanted to reenact that moment.

Not until recently did I realize how popular Tesla Coils were.  While

browsing on the Internet for any information I could use, I was taken aback

when I saw how many results I got for the search.  I was looking for plans

for this gun-type Tesla coil, and some documentation as to why it works.

What I would like to ask of you, is if you send me all you could about the

science behind the Tesla, and plans to make a coil.

Thank you very much for your time.

Michael J. Koerner

jMk Computer Services

mkoerner@fhjc.org

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Bottle cap design?

Date: Fri, 13 Mar 1998 02:53:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 12 Mar 1998 18:53:25 -0800

Subject: Bottle cap design?

Organization: Self

All:

Is there any need for, or is it a detriment to have either venting or a

dead air space above the oil in the bottles?  Is failure generally

caused by overpressure due to heating or by voltage puncture of the

glass?   I'm hoping to RTV hermetically seal my lid/electrode assy to*

the bottles.

Are sharp corners and or wrinkles in the foil inducements to puncture?

*...and then, maybe not- the lid/electrode assy from a broken bottle

could be used again...

Our bottles mike out at .086" min.(.0999" max.)   If we assume K=7.5

and puncture voltage at is 225v/mil, then 225 X 86 =19,350 v. Are most

of the T-2 Dielectric table data for Ac or DC?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla*REMOVE*

Date: Fri, 13 Mar 1998 06:29:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Michael Koerner" <mkoerner@fhjc.org>

Date: Fri, 13 Mar 1998 01:29:20 -0500

Subject: Re: New to Tesla*REMOVE*

If you follow the responses back to the original, I was asking a

harmless question.  We just got slightly side tracked.  It's done with.

Though, I still want to know what type of Tesla coil illuminates fluorescent

bulbs.  At http://www.amazing1.com/tesla.htm I found plans for such a Tesla

coil, but I don't believe it since you are all saying that Tesla coils can't

do that.

Michael J. Koerner

jMk Computer Services

mkoerner@fhjc.org

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Plate caps & other stuff

Date: Fri, 13 Mar 1998 06:40:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: David Whalen <dwhalen@linknet.kitsap.lib.wa.us>

Date: Thu, 12 Mar 1998 22:40:24 -0800 (PST)

Subject: Plate caps & other stuff

Greetings,

I'm building a plate capacitor because they appear to be smaller and

less messy than bottle caps.  I really don't know how to do much of it and

I have some questions:

1)  How many layers do I need for 9kv 30ma?

2)  What sort of oil should I use and where do I get it?

3)  How can I keep it from burning/melting/exploding/dying/etc.?

4)  Where I live nst's are VERY difficult to acquire.  I just bought a

used, fairly nice one (9kv 30ma) and I don't want to hurt it.  Do I need

chokes and does this have anything to do with the capacitor?

5>  Not about caps, but I just sprayed my secondary with acryllic and gaps

formed between the wires!  I think this is bad.  Is it?  How fix?

6)  My primary is 15 turns of 1/4 inch copper tubing.  It spirals up at

a thirty degree angle.  How many turns should the secondary be if I'm

using 22 gauge?

7)  Is it illegal to disturb radio communications?  Do I need a Faraday's

cage?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle cap design?

Date: Fri, 13 Mar 1998 06:42:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: David Whalen <dwhalen@linknet.kitsap.lib.wa.us>

Date: Thu, 12 Mar 1998 22:42:53 -0800 (PST)

Subject: Re: Bottle cap design?

On Fri, 13 Mar 1998, Tesla-2 wrote:

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Thu, 12 Mar 1998 18:53:25 -0800

> Subject: Bottle cap design?

> Organization: Self

>

>

Someone wrote:

All:

>

> Is there any need for, or is it a detriment to have either venting or a

> dead air space above the oil in the bottles?  Is failure generally

> caused by overpressure due to heating or by voltage puncture of the

> glass?   I'm hoping to RTV hermetically seal my lid/electrode assy to*

> the bottles.

>

> Are sharp corners and or wrinkles in the foil inducements to puncture?

>

> *...and then, maybe not- the lid/electrode assy from a broken bottle

> could be used again...

>

> Our bottles mike out at .086" min.(.0999" max.)   If we assume K=7.5

> and puncture voltage at is 225v/mil, then 225 X 86 =19,350 v. Are most

> of the T-2 Dielectric table data for Ac or DC?

>

>

I recomend plate caps.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle cap design?

Date: Fri, 13 Mar 1998 07:18:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Fri, 13 Mar 1998 01:18:27 -0600

Subject: Re: Bottle cap design?

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Thu, 12 Mar 1998 18:53:25 -0800

> Subject: Bottle cap design?

> Organization: Self

>

> All:

>

> Is there any need for, or is it a detriment to have either venting or a

> dead air space above the oil in the bottles?  Is failure generally

> caused by overpressure due to heating or by voltage puncture of the

> glass?   I'm hoping to RTV hermetically seal my lid/electrode assy to*

> the bottles.

Bottle caps usually breakdown by means of overvoltage or excessive

corona. A vent hole is not extremely necessary but you can have one.

Besides nine chances out of ten if the cap built up to much heat it will

more than likly "pop the top" so to speak. I personally have had only

one bottle cap fail and it was because of corona. I have ran these type

of caps w/ a 15kv neon and had no breakdowns since that one.

>

> Are sharp corners and or wrinkles in the foil inducements to puncture?

Yes, the points or wrinkles will subject the glass to quite a bit of

corona. This is why it is better to place bottles caps in a salt water

solution to make up the outside plate. Doing so will cut the corona down

as low as possible.

>

> *...and then, maybe not- the lid/electrode assy from a broken bottle

> could be used again...

Yes, as long as you use the same type of bottle.

>

> Our bottles mike out at .086" min.(.0999" max.)   If we assume K=7.5

> and puncture voltage at is 225v/mil, then 225 X 86 =19,350 v. Are most

> of the T-2 Dielectric table data for Ac or DC?

What type of bottles are these? If they are plain glass your voltage

rating should be around 250-300v/mil ac. I don't recall how thick my

bottles were, but I know that they were just plain wine cooler bottles.

I never lost one of the wine cooler bottles on my 15kv neon.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & other stuff

Date: Fri, 13 Mar 1998 08:35:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Fri, 13 Mar 1998 02:35:49 -0600

Subject: Re: Plate caps & other stuff

Tesla-2 wrote:

>

> From: David Whalen <dwhalen@linknet.kitsap.lib.wa.us>

> Date: Thu, 12 Mar 1998 22:40:24 -0800 (PST)

> Subject: Plate caps & other stuff

>

> Greetings,

>

>      I'm building a plate capacitor because they appear to be smaller and

> less messy than bottle caps.  I really don't know how to do much of it and

> I have some questions:

>

> 1)  How many layers do I need for 9kv 30ma?

I would say to go ahead and go with 60mil polyethylene, you can add more

if it makes you feel more comfortable.

>

> 2)  What sort of oil should I use and where do I get it?

Mineral oil, at the drug store.

>

> 3)  How can I keep it from burning/melting/exploding/dying/etc.?

The basic and just about only way to keep these caps funtioning, would

be to design them well. You should make sure that you leave at least an

extra inch of poly all of the way around all plates and build it with a

fairly high breakdown voltage.

>

> 4)  Where I live nst's are VERY difficult to acquire.  I just bought a

> used, fairly nice one (9kv 30ma) and I don't want to hurt it.  Do I need

> chokes and does this have anything to do with the capacitor?

It is recommended that you do use chokes to prevent early neon death.

You also need a safety gap! For the chokes to be effective and compact

use ferrite toroids. Yes the caps play a large part in neon failure.

The caps have a tendency to "backlash" at the transformer with very high

voltages. Neons are not equipped to handle those types of "backlashes",

that is the sole reason for the safety gap and the chokes.

>

> 5>  Not about caps, but I just sprayed my secondary with acryllic and gaps

> formed between the wires!  I think this is bad.  Is it?  How fix?

How large of gaps are we talking here? Inches, centimeters or what?

>

> 6)  My primary is 15 turns of 1/4 inch copper tubing.  It spirals up at

> a thirty degree angle.  How many turns should the secondary be if I'm

> using 22 gauge?

Regardless of wire size, average turns should be between 500-1000.

How large is your secondary? I haven't dealt with a coil smaller than 4"

in diameter. So based on my three coils it should be similar to this:

4" x 20"(5:1 ratio),6" x 24"(4:1 ratio), and 8" x 24"(3:1 ratio)

>

> 7)  Is it illegal to disturb radio communications?  Do I need a Faraday's

> cage?

No you don't need a faraday cage. Altough it would be wise to keep

electronics well away from the coil when in operation. The scale of

a small Tesla Coil will not cause wide spread RFI. It will however give

off RFI in the general area where you are set up (given your coil size

probably no more than 10 feet maybe 15.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Misc. stuff for sale.

Date: Fri, 13 Mar 1998 08:45:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Fri, 13 Mar 1998 02:45:02 -0600

Subject: Misc. stuff for sale.

I have a couple of things I want to get rid of as they are taking up

space. I have three microwave transformers, all work. I have a 12v

ignition coil, brand new. I also have two motors. One is a weed eater

motor( pretty powerful)and is threaded on the end, so it will accompany

a fan blade or a small rotor for use in a spark gap. The second is a

General Electric motor, it runs at 3,450 rpm. It has a momentary start

winding in it. Anyone interested contact me.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 16.6 kv experiment

Date: Fri, 13 Mar 1998 13:13:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 13 Mar 1998 07:13:30 -0600

Subject: 16.6 kv experiment

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Using my trusty volt meter and my 240v variac I was able finally to determine

just how much voltage I could get out of my pig. There is a rotary switch inside

the pig which has 5 positions. When set at pos 5 the secondary output is 14.4 kv.

My variac will feed the pig 277 v, thus 16.6 kv can be acheived. My caps are in

series with 60 mil dielectrics between the plates. For the last 6 months I have

actually been running about 13.8 kv. With this voltage I get typical 54 " arcs

with some as long as 60". What do you think the possibilities are ?

I would like to hear from anyone who has tried a simular experiment, or anyone with

ideas along these lines.

THANKS.

Paul M.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: oil for the coil..

Date: Fri, 13 Mar 1998 15:41:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ms291 <ms291@mindspring.com>

Date: Fri, 13 Mar 1998 10:41:25 -0500

Subject: oil for the coil..

Greetings All,

After having little or no luck procuring anything smaller than a 55 gallon

of Shell Diala AX from the local oil jobber.. and becoming somewhat bored

with looking at all this 4" & 6" PVC pipe, wound 4" coils of various

lengths and wire gauges, a nice 15 kV 60ma NST, as well as rolls of poly

and aluminum... I am considering more drastic measures..

I do some flying at the local small airport.. and it seems to me that one

of the lubricants used for small plane engines (usually for break-in) is

mineral oil.. sold in convenient quart (much easier to transport [nevermind

store] than that 55 gallon drum..) sizes.. The only hitch is that I seem to

recall the term 'ashless dispersion'.. which might imply some breakdown

voltage reducing additives..

If anyone has any comment on this idea, or can give me some hints as to

where to get.. say.. a few gallons of transformer or pure mineral oil.. I

would appreciate it!

..sitting here with almost enough stuff to finish the coil...:)

Jim Horan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plate caps & other stuff

Cc:

Bcc:

X-Attachments:

At 06:40 AM 3/13/98 GMT, you wrote:

>From: David Whalen <dwhalen@linknet.kitsap.lib.wa.us>

>Date: Thu, 12 Mar 1998 22:40:24 -0800 (PST)

>Subject: Plate caps & other stuff

>

>Greetings,

Wow. A whole batch of questions David.

Let see what we have here.

>

>     I'm building a plate capacitor because they appear to be smaller and

>less messy than bottle caps.

Not to mention that a poly plate cap is several orders of magnitude

more efficient than SW bottle caps.

>I really don't know how to do much of it and I have some questions:

>

>1)  How many layers do I need for 9kv 30ma?

I have run caps with 24 mils of poly (4 X 6 mil) quite succesfully

at 9kv. However this is really marginal and doesnt allow much

room for error. I would recommend 30 mils for 9kv.

I dont however recommend going out and buying 30 mil poly.

Rather build up this thickness with layers of thinner poly.

6 mil poly is available cheaply at most hardware stores.

5 layers of this will give you a greater voltage rating

than a single 30 mil sheet.

You can run a single cap at 9kv. However you should not run

single caps at voltages greater than 9-10KV. You need to run a

series string of caps at higher voltages to distribute the strain.

2 caps in series for 12kv and 3 in series for 15kv.

So why not over build just a bit and plan a cap that will

be capable of moving up to 12kv neons in the future.

Build two 30 mil caps at twice your required capacitance and

wire them in series. You need around .0088 mfd for a 9kv/30ma

neon so you build two caps each with .0176 mfd. When used in series

they will yield .0088 mfd and handle 12kv easily.

Note you do not need exactly .0088 mfd. Anywhere in that area

is just fine. The exact value is not critical.

>

>2)  What sort of oil should I use and where do I get it?

Mineral oil is easy to find. You can get it at any drugstore.

I purchased my first batch of mineral oil at a local WalMart.

Less expensive mineral oil can be had from your local vet. Its

used as an animal laxative. Around here its available in 2 and 5

gallon pails.

Real transformer oil can be purchased from most oil distributers.

Or inquire at your local power company. Sometimes you can get

a couple free gallons this way. Real transformer oil is preferred

due to its lower viscosity. This allows air bubbles to escape

more easily.

Most silicone based oils can be used successfully. But you need

to make sure they are free from any additives.

Check out Dave Bronakowski's archive site and download the file

"oiltypes.txt" for further info on various oils.

The URL is :http://www.velocity.net/~djb/files.shtml

>

>3)  How can I keep it from burning/melting/exploding/dying/etc.?

Adequate margins around the plates (1 inch minimum) and sufficient

dielectric thickness for the voltage will help. Air bubble removal

is very important. This requires a slow break in period for plate

caps and vacuum pumping for rolled types.

Running caps in series helps to distribute the load and results

in a more reliable cap.

Running excessive gap width will result in cap and/or transformer

failure. MAXIMUM gap distances of .200 inch for 9kv, .250 inch for 12kv,

and .300 inch for 15kv are recommended. I have never run more than

.225 inch for my 12kv neons.

>

>4)  Where I live nst's are VERY difficult to acquire.  I just bought a

>used, fairly nice one (9kv 30ma) and I don't want to hurt it.  Do I need

>chokes and does this have anything to do with the capacitor?

You need chokes. Chokes wound on large ferrite toroids are best.

If you use air core chokes wound on PVC pipe you should use some

series resistors to dampen any possible oscillations.

Do not forget the safety gap. This is the single most important

protective measure to take.

Do not place your main cap in parallel across the transformer as

is shown in many older schematics. Instead the spark gap is placed

in the shunt position across the transformer. This helps to isolate

the transformer from RF kickbacks.

>

>5>  Not about caps, but I just sprayed my secondary with acryllic and gaps

>formed between the wires!  I think this is bad.  Is it?  How fix?

Depends on how much gap and how many. If its just a few gaps

here and there then dont worry about it. If its lots of gaps

then unwind it and start over.

Regular brush on polyurethane varnish works well.

>

>6)  My primary is 15 turns of 1/4 inch copper tubing.  It spirals up at

>a thirty degree angle.  How many turns should the secondary be if I'm

>using 22 gauge?

The best guage to use for a 4 inch diameter secondaryis AWG 24.

Wind from 20-22 inches of AWG 24. If you already have AWG 22

then by all means use it. My first 4 incher was wound with 22 inches

of AWG 22. My second 4 incher was 21 inches of AWG 24. I got

better performance from the second coil.

If you are building a 6 incher then wind 26 inches of AWG 22.

>

>7)  Is it illegal to disturb radio communications?  Do I need a Faraday's

>cage?

Yes it is illegal to interfere with other radio services.

But its unlikely you will do so with a Tesla coil.

When set up for spark a Tesla coil is a very poor RF radiator.

You may encounter some local RFI but it will generally not

extend any more than 100 feet or so.

The only problem I have with my coil is with a cordless telephone.

A Faraday cage is unnecessary.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: My Lucky Day

Date: Fri, 13 Mar 1998 17:54:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 13 Mar 1998 12:54:21 -0500

Subject: My Lucky Day

I just found a place that let me run free in their garbage pile. I

came back with everything they had except a 7500v transformer. here's

what I got:

5 15kv 30ma

2 12kv 30ma

1  9kv 30ma

Jackpot!

I'll probably need to cold cook all of them.

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: oil for the coil..

Date: Fri, 13 Mar 1998 19:04:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 13 Mar 1998 11:04:48 -0800 (PST)

Subject: Re: oil for the coil..

On 13 Mar, Tesla-2 wrote:

Jim:

You might check with your local electric company.. you might be surprised.

Also, consider:

1) Drug stores. Some people actually use mineral oil as a laxative.

available in quart bottles.

2) Large animal vet offices. Often available in gallon jugs.

If you are willing to pay shipping, you might try contacting Shell

direct.

Also: check refrigeration shops. THe silicon oil they use is fine for

dunking caps in.

"Waiting for that free 5 gallons of silicon oil...."

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My Lucky Day

Cc:

Bcc:

X-Attachments:

At 05:54 PM 3/13/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Fri, 13 Mar 1998 12:54:21 -0500

>Subject: My Lucky Day

>

>

>     I just found a place that let me run free in their garbage pile. I

>     came back with everything they had except a 7500v transformer. here's

>     what I got:

>

>

>     5 15kv 30ma

>     2 12kv 30ma

>     1  9kv 30ma

>

>

>     Jackpot!

You hit paydirt! Good haul.

>     I'll probably need to cold cook all of them.

Test them first. Seperate into two piles. Good ones

for immediate use and bad ones for unpotting/rebuilding.

Do not unpot functional transformers. At least not until

you have some experience in unpotting them. Its too easy

to damage a winding and ruin a perfectly good transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: oil for the coil..

Cc:

Bcc:

X-Attachments:

At 03:41 PM 3/13/98 GMT, you wrote:

>From: ms291 <ms291@mindspring.com>

>Date: Fri, 13 Mar 1998 10:41:25 -0500

>Subject: oil for the coil..

>

>

>Greetings All,

>

>After having little or no luck procuring anything smaller than a 55 gallon

>of Shell Diala AX from the local oil jobber.. and becoming somewhat bored

>with looking at all this 4" & 6" PVC pipe, wound 4" coils of various

>lengths and wire gauges, a nice 15 kV 60ma NST, as well as rolls of poly

>and aluminum... I am considering more drastic measures..

>

>I do some flying at the local small airport.. and it seems to me that one

>of the lubricants used for small plane engines (usually for break-in) is

>mineral oil.. sold in convenient quart (much easier to transport [nevermind

>store] than that 55 gallon drum..) sizes.. The only hitch is that I seem to

>recall the term 'ashless dispersion'.. which might imply some breakdown

>voltage reducing additives..

Are you sure this isnt castor oil? Ive seen castor used in lots of

small engines including model plane engines and racing engines.

It is supposed to be one of the best lubricants available.

Its also one of the best dielectric oils available. It has good

insulating properties as well as a higher dielectric constant

than mineral oils. This results in a higher capacitance.

The only problem is quite high cost.

>

>If anyone has any comment on this idea, or can give me some hints as to

>where to get.. say.. a few gallons of transformer or pure mineral oil.. I

>would appreciate it!

The local power company is always worth a try. I know some

people that got a few gallons for free this way. Never hurts to try.

Veterinary grade mineral oil is another choice.

Some people have even used generic baby oil. Baby oil is mineral

with a little fragrance and sometimes vitamin E added. Makes

for nice smelling caps.

One other possiblity is to order some mineral oil from Tulco Oil.

Tulco Oils

627 112th Street

Arlington, TX  76011

(800) 880-0045

LUBSNAP 100.  A 5 gallon pail of this cost $19.75 several months ago.

Shipping is via UPS. They just slap a shipping label on the pail.

I know several people who have used this oil with good success.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 16.6 kv experiment

Cc:

Bcc:

X-Attachments:

At 01:13 PM 3/13/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 13 Mar 1998 07:13:30 -0600

>Subject: 16.6 kv experiment

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Using my trusty volt meter and my 240v variac I was able finally to determine

>just how much voltage I could get out of my pig. There is a rotary switch inside

>the pig which has 5 positions. When set at pos 5 the secondary output is 14.4 kv.

>My variac will feed the pig 277 v, thus 16.6 kv can be acheived. My caps are in

>series with 60 mil dielectrics between the plates. For the last 6 months I have

>actually been running about 13.8 kv. With this voltage I get typical 54 " arcs

>with some as long as 60".

>What do you think the possibilities are ? I would like to hear from

>anyone who has tried a similar experiment, or anyone with I deas along these lines.

>

>

Paul I havnt exactly done this but my take is that the modest increase

input power probably wont make that much difference.

Since you have some robust caps (3 in series I believe) you shouldnt

have cap problems so it wont hurt to try. You may also want to open

your main gap slightly to accomodate the higher primary voltage.

Let us know how it goes.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RF Grounding

Date: Fri, 13 Mar 1998 21:58:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 13 Mar 1998 16:58:27 -0500

Subject: RF Grounding

What's a simple and safe way to construct an RF ground?

Do I really have to somehow hammer 8 FEET of copper pipe into the

ground? How many of these do I need?

I'm not quite sure how I would hammer an 8 foot length of copper pipe

the ground.

Also, what type of wire should I use to connect to the ground and how

long can it safely be?

Thanks!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF Grounding

Cc:

Bcc:

X-Attachments:

At 09:58 PM 3/13/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Fri, 13 Mar 1998 16:58:27 -0500

>Subject: RF Grounding

>

>

>     What's a simple and safe way to construct an RF ground?

>

>     Do I really have to somehow hammer 8 FEET of copper pipe into the

>     ground? How many of these do I need?

>

>     I'm not quite sure how I would hammer an 8 foot length of copper pipe

>     the ground.

You cant ever have too much ground for a Tesla coil.

The better and lower impedance your ground is the better.

Two four foot rods connected in parallel are better than one

eight footer.

Use copper clad steel ground rods. They are available at home

improvement centers like Lowes or Menards. These are easy to drive

into even very hard soil.

>

>     Also, what type of wire should I use to connect to the ground and how

>     long can it safely be?

>

Think heavy wire for lowest impedance. Make the ground lead

from the secondary to your ground rods as short and as straight

as possible. Use a heavy conductor. Do not use braid or braided

cables. Those are poor for RF.

I use stranded AWG 8 ground wire about 5 feet long.

The longer your ground lead the heavier it will need to be

to provide low impedance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap

Date: Sat, 14 Mar 1998 01:42:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "smash" <smash@eisa.net.au>

Date: Sat, 14 Mar 1998 12:42:45 +1100

Subject: Re: spark gap

To Anyone,

>I like using an air-blast spark gap, you can use an adjustable static gap

>and just have about 15 psi blowing through it, its very noisy but works

>great!

Where does one get a fan or compressor that would put out that much air?  :)

And a question totally off the topic!  How does one phase neons so that they

can be connected in parallel.  I have three 12000V/30mA neons at my house

and I would like to connect them all in parallel.  (Please explain simply,

I'm new to this)?

From Scott

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Member List

Date: Sat, 14 Mar 1998 05:37:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>    I will be compiling a list of everyone on the Tesla-2 list. Please Send

T>me your Name, E-Mail, State, and City if you wish for me to add you to the

T>list. I belive this will help bring coilers in the same area together so

T>that they might help one another and make bigger and better sparks. Thank

T>you and have a nice Day.

Shaun,

I like the idea, but don't put it on a linked web page!  If you do,

those blasted email address-gathering robots will scoop up everyones

address, and the flood of spam will start. :(

Make up the file and then post it to an unlinked page on your site.

Then, you can post the address of the page in the list here, and anyone

who wants the list can retrieve it directly.  That way, the spammers

can't get to the names.

Ralph

---

Anytime things go better, you have overlooked something.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Member List

Date: Sat, 14 Mar 1998 08:05:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Choad <choad69@geocities.com>

Date: Sat, 14 Mar 1998 02:05:22 -0600

Subject: Re: Member List

At 11:37 PM 3/13/98 , you wrote:

>T>    I will be compiling a list of everyone on the Tesla-2 list. Please Send

>T>me your Name, E-Mail, State, and City if you wish for me to add you to the

>T>list. I belive this will help bring coilers in the same area together so

>T>that they might help one another and make bigger and better sparks. Thank

>T>you and have a nice Day.

>

>

>Shaun,

>

>   I like the idea, but don't put it on a linked web page!  If you do,

>those blasted email address-gathering robots will scoop up everyones

>address, and the flood of spam will start. :(

>

>   Make up the file and then post it to an unlinked page on your site.

>Then, you can post the address of the page in the list here, and anyone

>who wants the list can retrieve it directly.  That way, the spammers

>can't get to the names.

>

>   Ralph

>

Ralph,

I didn't think of that. I think I'll have a list on a page w/o E-Mails

and have a downloadable list with E-Mail address.

Shaun O'Brien

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: oil for the coil..

Date: Sat, 14 Mar 1998 10:54:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 14 Mar 1998 02:54:44 -0800

Subject: Re: oil for the coil..

Organization: EES

Tesla-2 wrote:

>

> From: ms291 <ms291@mindspring.com>

> Date: Fri, 13 Mar 1998 10:41:25 -0500

> Subject: oil for the coil..

>

> Greetings All,

>

> After having little or no luck procuring anything smaller than a 55 gallon

> of Shell Diala AX from the local oil jobber.. and becoming somewhat bored

> with looking at all this 4" & 6" PVC pipe, wound 4" coils of various

> lengths and wire gauges, a nice 15 kV 60ma NST, as well as rolls of poly

> and aluminum... I am considering more drastic measures..

Jim,

What is the price quoted for the 55gal??

--

Regards,

Scott MacGregor, EES

************************************************************

Enterprise Engineering Services

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF Grounding

Date: Sat, 14 Mar 1998 11:43:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 14 Mar 1998 03:43:35 -0800

Subject: Re: RF Grounding

Organization: EES

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 13 Mar 1998 19:31:23 -0600

> Subject: Re: RF Grounding

>

> At 09:58 PM 3/13/98 GMT, you wrote:

> >From: <Tom.Stock@gsbsc.gensig.com>

> >Date: Fri, 13 Mar 1998 16:58:27 -0500

> >Subject: RF Grounding

> >

> >

> >     What's a simple and safe way to construct an RF ground?

> >

> >     Do I really have to somehow hammer 8 FEET of copper pipe into the

> >     ground? How many of these do I need?

> >

> >     I'm not quite sure how I would hammer an 8 foot length of copper pipe

> >     the ground.

>

> You cant ever have to much ground for a Tesla coil.

> The better and lower impedance your ground is the better.

>

> Two four foot rods connected in parallel are better than one

> eight footer.

Often, soil conditions play a part in deciding the depth and extent of

grounding. Most TV, radio, and cable transmit/receive sites require

ground resistance of <1 ohm. I have seen sandy conditions where 40 foot

(5 coupled 8 footers) ground rods were necessary! No practical number of

shorter rods would meet the 1 ohm specification without constant

addition of soil conditioning chemicals. Another option, in very hard or

rocky soil, is a grid or radial array. In these cases, 4" copper strap

or #6 solid copper wire is buried in a closely spaced pattern 4 to 6

inches deep. If your lucky, you may find a local cable or power company

that will let you check the resistance of your ground with their

"megger", a specialized device used to measure ground resistance.

--

Regards,

Scott MacGregor, EES

************************************************************

Enterprise Engineering Services

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF Grounding

Date: Sat, 14 Mar 1998 16:47:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Rick L. Kirby" <rkirby@earthlink.net>

Date: Sat, 14 Mar 1998 08:47:51 -0800

Subject: Re: RF Grounding

Tesla-2 wrote:

>

> From: <Tom.Stock@gsbsc.gensig.com>

> Date: Fri, 13 Mar 1998 16:58:27 -0500

> Subject: RF Grounding

>

>      What's a simple and safe way to construct an RF ground?

>

>      Do I really have to somehow hammer 8 FEET of copper pipe into the

>      ground? How many of these do I need?

>

>      I'm not quite sure how I would hammer an 8 foot length of copper pipe

>      the ground.

>

>      Also, what type of wire should I use to connect to the ground and how

>      long can it safely be?

>

>      Thanks!

>      -tom

>

Hi Tom,

Hammering is hard at times I use an electric hammer drill without

the cement bit, it slides over the end of my 8ft x 1/2"od rods just fine.

It will vibrate the grd. rods into the ground very fast and easy. I drove

three rods in at 45 deg angles in about 10 mins. I connected all 3 with

soft 3/4" copper refrig. tubing. The tubing runs to my back door then I

clamp 2" wide alum.flashing from this point to my coil in the living

room. 12 ft. of tubing and 10 ft. 2" flashing.

Rick

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: oil for the coil..

Date: Sat, 14 Mar 1998 20:13:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ms291@mindspring.com

Date: Sat, 14 Mar 1998 15:13:39 -0500

Subject: Re: oil for the coil..

>From: Scott MacGregor <smacgregor@aasp.net>

>Date: Sat, 14 Mar 1998 02:54:44 -0800

>Subject: Re: oil for the coil..

>Organization: EES

>> After having little or no luck procuring anything smaller than a 55 gallon

>> of Shell Diala AX from the local oil jobber.. and becoming somewhat

bored...

>Jim,

>

>What is the price quoted for the 55gal??

It wasn't as expensive as I thought it would be. As I recall, they quoted

me around $225 for the drum.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wire guage

Date: Sat, 14 Mar 1998 20:21:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sat, 14 Mar 1998 12:21:30 -0800

Subject: Re: wire guage

I've been using stranded metallic hv wire for a while now, I just found out

that it is twisted. I want to replace all this with #14 solid copper wire, I

have plenty of space to avoid arcs where I dont want them.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: oil for the coil..

Date: Sun, 15 Mar 1998 04:20:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 14 Mar 1998 20:20:26 -0800

Subject: Re: oil for the coil..

Organization: EES

Tesla-2 wrote:

>

> From: ms291@mindspring.com

> Date: Sat, 14 Mar 1998 15:13:39 -0500

> Subject: Re: oil for the coil..

>

> >From: Scott MacGregor <smacgregor@aasp.net>

> >Date: Sat, 14 Mar 1998 02:54:44 -0800

> >Subject: Re: oil for the coil..

> >Organization: EES

>

> >> After having little or no luck procuring anything smaller than a 55 gallon

> >> of Shell Diala AX from the local oil jobber.. and becoming somewhat

> bored...

>

> >Jim,

> >

> >What is the price quoted for the 55gal??

>

> It wasn't as expensive as I thought it would be. As I recall, they quoted

> me around $225 for the drum.

Sounds like a business opportunity!

Regards,

Scott MacGregor, EES

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Plate caps & connections

Date: Sun, 15 Mar 1998 06:16:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: David Whalen <dwhalen@linknet.kitsap.lib.wa.us>

Date: Sat, 14 Mar 1998 22:16:57 -0800 (PST)

Subject: Plate caps & connections

Anyone who can help:

I recently asked how many layers I needed in my plate caps.  I got

answers in mils.  That's not exactly what I meant.  Is there just one

dielectric and two plates, or lots of alternating ones and if so how many?

How big should the Tupperware I'm using be?  The capacitor is the only

element I need!  Then I'll be done!  I am doing an experiment with toroids

to find out what shape and size work the best with my system.  Also, are

alligator clips recomended for connections or is there something wrong

with them?

Thanks, Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Fun with Tesla coil & CD's and Argon etc. - Pictures (correct URL

Date: Sun, 15 Mar 1998 09:40:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Sun, 15 Mar 1998 09:40:00 GMT

Subject: Re: Fun with Tesla coil & CD's and Argon etc. - Pictures (correct URL

this time!)

Organization: White Wing Logic

Oops, sorry, finger trouble! the correct URL is :

www.netcomuk.co.uk/~wwl/electric.html

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Sun, 15 Mar 1998 13:44:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 15 Mar 1998 08:44:54 EST

Subject: Re: Plate caps & connections

Dave,

What people were trying to express was the overall thickness of the

dielectric needed(probably LDPE).   You should not use a single thickness

sheet between the metal plates as a defect in the material could easily go all

the way through, shorting the plates.  Two or three sheet are better since it

is unlikely that defects in all the layers would be close enough to allow this

to occur. The overall thickness must be at least twice the strength that you

would get from a table since these are DC values and that in Tesla coil

service,  the DC rating is not enough.

If instead, you were asking how many plates are needed to get the

capacitance you desire, that is an entirely different issue.  This can be

estimated by a relatively simple equation.  I would caution you to measure the

cap once built as you may not have the plates perfectly compressed and this

would lower the overal capacitance.

The equation for calculating the value in microfarads is:

C = 2.24E-7 * K * L * W * (N - 1) / T

K is the dielectric constant of you insulating material.

L is the length of the plate in inches

W is the width in inches

N is the  number of plates

T is the dielectric thickness in inches( this is mils divided by 1000)

As you should be able to see, The number of plate and the area of the plates

can be any values you need to get the design you want.  If you want smaller

plate area,  you will just need more plates.   You can design the plate area

to fit in just about any container size.

Hope this helps

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Motor oil vs mineral oil

Date: Sun, 15 Mar 1998 13:53:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sun, 15 Mar 1998 18:53:05 +0500

Subject: Re: Motor oil vs mineral oil

On Thu, 12 Mar 1998 18:31:03 -0800 In Tesla-2

Mike Nicholas <705728@ican.net> wrote;

>    It's a good thing I'm a refrigeration tech.   I wonder if vacum pump

> oil will be suitable? Its very dry and very thin

Actually some vacuum pump oils and regrigerator compressor oils are

superior to any transformer oils! Actually plain mineral oil is superior to

all transformer oils, but there are a few refrigerator compressor oils that

have double the dielectric constant of U.S.P. mineral oil and even a bit

better dielectric strength.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Sun, 15 Mar 1998 15:00:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sun, 15 Mar 1998 20:00:16 +0500

Subject: Re: Plate caps & connections

On Sun, 15 Mar 1998 13:31:15 -0800 Scott MacGregor

<smacgregor@aasp.net> wrote;

> > From: Hollmike <Hollmike@aol.com>

> > Date: Sun, 15 Mar 1998 08:44:54 EST

> > Subject: Re: Plate caps & connections

> >

> >   The equation for calculating the value in microfarads is:

> >

> > C = 2.24E-7 * K * L * W * (N - 1) / T

> >

> > K is the dielectric constant of you insulating material.

> > L is the length of the plate in inches

> > W is the width in inches

> > N is the  number of plates

> > T is the dielectric thickness in inches( this is mils divided by 1000)

>

> Mike,

>

> Do you know of an internet source of dialectric constants?

The dielectric constant of polyethylene is 2.3

> What is the "E" in the above equation?

The E would represent Exponentiation or power of 10 . Frankly I don't know

where he got that equation from, the correct equation is;

Capacitance (in microfarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Castor oil

Date: Sun, 15 Mar 1998 15:00:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sun, 15 Mar 1998 20:00:19 +0500

Subject: Castor oil

I don't remember if it was on this list or Chips list that someone

suggested using castor oil in TC capacitors, the problem with castor oil

is it has an extremely high power factor (7 according to the 27th edition

of the ARRL Radio Amateur's Handbook) which means it will heat up and

cook due to the large amounts of R.F. being absorbed.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Sun, 15 Mar 1998 21:31:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sun, 15 Mar 1998 13:31:15 -0800

Subject: Re: Plate caps & connections

Organization: EES

Tesla-2 wrote:

>

> From: Hollmike <Hollmike@aol.com>

> Date: Sun, 15 Mar 1998 08:44:54 EST

> Subject: Re: Plate caps & connections

>

>   The equation for calculating the value in microfarads is:

>

> C = 2.24E-7 * K * L * W * (N - 1) / T

>

> K is the dielectric constant of you insulating material.

> L is the length of the plate in inches

> W is the width in inches

> N is the  number of plates

> T is the dielectric thickness in inches( this is mils divided by 1000)

Mike,

Do you know of an internet source of dielectric constants?

What is the "E" in the above equation?

Regards,

Scott MacGregor, EES

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Mon, 16 Mar 1998 00:06:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 15 Mar 1998 19:06:32 EST

Subject: Re: Plate caps & connections

Scott,

First, the "E" is computer/calculator notation for 10 to some

power(exponent). In this case, 10 to the negative 7.  That results in that

number being 0.000000224

Next, I don't know right off hand the web address for dielectrics.  The

best choices for dielectrics for Tesla coil use are:

K              Dielectric Strength(approx)

polyethylene       2.3             1000V/mil   low density

polypropylene      2.3             1000V/mil

mica               5 - 7           4000V/mil*

teflon             2.1             1500V/mil

McMaster Carr (in Chicago) sells sheets of mica that they claim 20kV/mil

dielectric strength.  I just finished putting one together (assuming

20kV/sheet of .004mil thickness and using two sheets between plates).  I just

had to try it to see if it as good as they say.  I will let you know how they

work out if you wish.

Hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Ontario Coilers

Date: Mon, 16 Mar 1998 04:14:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 15 Mar 1998 20:14:12 -0800

Subject: Re: Ontario Coilers

Are there any coilers from Ontario here?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Motor oil vs mineral oil

Date: Mon, 16 Mar 1998 04:33:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 15 Mar 1998 20:33:20 -0800

Subject: Re: Motor oil vs mineral oil

>

>

> >    It's a good thing I'm a refrigeration tech.   I wonder if vacum pump

> > oil will be suitable? Its very dry and very thin

>

> Actually some vacuum pump oils and regrigerator compressor oils are

> superior to any transformer oils! Actually plain mineral oil is superior to

> all transformer oils, but there are a few refrigerator compressor oils that

> have double the dielectric constant of U.S.P. mineral oil and even a bit

> better dielectric strength.

I'm just about ready to finish off my Krazy Carpet Capacitors with some

vacum pump oil, wish me luck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Mon, 16 Mar 1998 04:36:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 15 Mar 1998 20:36:24 -0800

Subject: Re: Plate caps & connections

>

>

>   McMaster Carr (in Chicago) sells sheets of mica that they claim 20kV/mil

> dielectric strength.  I just finished putting one together (assuming

> 20kV/sheet of .004mil thickness and using two sheets between plates).  I just

> had to try it to see if it as good as they say.  I will let you know how they

> work out if you wish.

> Hope this helps.

What is the cost of this mica?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Dallas/Ft. Worth

Date: Mon, 16 Mar 1998 07:34:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Robert C. McGrath" <rmcgra1@airmail.net>

Date: Mon, 16 Mar 1998 01:34:58 -0600

Subject: Dallas/Ft. Worth

Hi.....Are there any coilers from the Dallas/Ft. Worth area that would

like to talk or get together sometime???If so, please respond to this

message.....thanx,

-Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Mon, 16 Mar 1998 12:12:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 16 Mar 1998 07:12:55 EST

Subject: Re: Plate caps & connections

Alfred,

Your equation is the same as the one I posted with a factor of a million

difference.  I just calculated my new cap with your equation and 20 plates  of

23.14sq. in separated by .008 in. of mica resulted in 66481 uF(I don't think

so).   Your equation puts the results in pF.

MH

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Mon, 16 Mar 1998 12:18:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 16 Mar 1998 07:18:12 EST

Subject: Re: Plate caps & connections

Mike,

The cost of the mica from McMaster Carr was around $10 for a 24"x36" sheet.

I don't remember the exact price, but it was not outrageous.  I will look it

up again if you wish.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Castor oil

Date: Mon, 16 Mar 1998 12:25:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 16 Mar 1998 06:25:46 -0600

Subject: Re: Castor oil

Organization: MailExcite  (http://www.mailexcite.com)

I am confused. I recently purchased a 40kv 32.2uf cap for some other experiments.

According to the the spec's listed on the cap, its filled with castor oil. Does

this RF absorb only in TC applications ? I am just a novice when it comes to TC's

Thanks

Paul M

---

Einigkeit und Freiheit

>From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

>Date: Sun, 15 Mar 1998 20:00:19 +0500

>Subject: Castor oil

>

>I don't remember if it was on this list or Chips list that someone

>suggested using castor oil in TC capacitors, the problem with castor oil

>is it has an extremely high power factor (7 according to the 27th edition

>of the ARRL Radio Amateur's Handbook) which means it will heat up and

>cook due to the large amounts of R.F. being absorbed.

>

>                                       Sincerely

>                                   Alfred A. Skrocki

>                               Alfred.Skrocki.Sr@JUNO.com

>                           alfred.skrocki@cybernetworking.com

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Castor oil

Cc:

Bcc:

X-Attachments:

At 03:00 PM 3/15/98 GMT, you wrote:

>From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

>Date: Sun, 15 Mar 1998 20:00:19 +0500

>Subject: Castor oil

>

>I don't remember if it was on this list or Chips list that someone

>suggested using castor oil in TC capacitors, the problem with castor oil

>is it has an extremely high power factor (7 according to the 27th edition

>of the ARRL Radio Amateur's Handbook) which means it will heat up and

>cook due to the large amounts of R.F. being absorbed.

>

Actually castor oil is used in some of the best pulse

caps available. It has low dissipation at normal Tesla coil

frequencies. Comparable to mineral or transformer oils.

It does however have high dissipation at HF frequencies which

is probably what the Handbook reference was alluding to.

The major problem is cost. Easily several times what good transformer

oils cost.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Castor oil

Cc:

Bcc:

X-Attachments:

At 12:25 PM 3/16/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Mon, 16 Mar 1998 06:25:46 -0600

>Subject: Re: Castor oil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>I am confused. I reciently purchased a 40kv 32.2uf cap for some other experiments.

>According to the the spec's listed on the cap, its filled with castor oil. Does

> this RF absorb only in TC applications ? I am just a novice when it comes to TC's

>

>Thanks

>

>Paul M

>---

Paul,

Castor oil is just fine for pulse caps. It has an RF dissipation

factor comparable to mineral and transformer oils at TC frequencies.

The high RF dissipation Alfred was talking about is mainly at HF frequencies.

Below about 500 Khz castor oil is one of the best dielectric oils

and is used in premium grade caps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plate caps & connections

Cc:

Bcc:

X-Attachments:

At 03:00 PM 3/15/98 GMT, you wrote:

>From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

>Date: Sun, 15 Mar 1998 20:00:16 +0500

>Subject: Re: Plate caps & connections

>

>> Mike,

>>

>> Do you know of an internet source of dialectric constants?

>

>The dielectric constant of polyethylene is 2.3

Actually a nominal value of 2.0 results in calculations

closer to actual capacitance in homebuilt caps. You never quite

get the amount of capacitance possible due to the plates

not be quite as close as they could be.

>

> Frankly I don't know where he got that equation from,

>the correct equation is;

Both equations are esentially the same. Only arranged differently.

>

>    Capacitance (in microfarads) = (0.224 KA / d) (N-1)

>

>        0.224 x Dielectric Constant x Area of plate

>    C = -----------------------------------------  x (Number of plates - 1)

>          distance between plates in inches

>

>

This equation is actually in error. The result is in picofarads not

microfarads.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Advise on Secondary coil

Date: Mon, 16 Mar 1998 17:07:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 16 Mar 1998 12:07:15 -0500

Subject: Advise on Secondary coil

I'm starting to plan my secondary coil, but I'm new to this and not

quite sure what parameters I should shoot for.

Right now I'm going for 4" Diameter x 22" long using #22 magnet wire.

This gives me a frequency of 522.90 KHz.

Is this ok for a 15KV 30ma Neon or should I go down to #24 or #26?

Thanks

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap

Date: Mon, 16 Mar 1998 18:27:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 16 Mar 1998 10:27:39 -0800

Subject: Re: spark gap

>

>

> To Anyone,

> >I like using an air-blast spark gap, you can use an adjustable static gap

> >and just have about 15 psi blowing through it, its very noisy but works

> >great!

>

> Where does one get a fan or compressor that would put out that much air?  :)

>

I use an air pig with a regulator.

> And a question totally off the topic!  How does one phase neons so that they

> can be connected in parallel.  I have three 12000V/30mA neons at my house

> and I would like to connect them all in parallel.  (Please explain simply,

> I'm new to this)?

>

> >From Scott

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: sec' ground arc

Date: Mon, 16 Mar 1998 18:42:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Mon, 16 Mar 1998 18:42:09 +0000

Subject: sec' ground arc

I am still having a problem with arcing between the primary and the

rf ground connection on the sec', even after placing approx. 3/16"

polyprop between the two.  Any suggestions on how to better insulate

the two?

Thanks, Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Streamers and tuning

Date: Mon, 16 Mar 1998 18:42:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Mon, 16 Mar 1998 18:42:09 +0000

Subject: Streamers and tuning

I have a question about streamer output.  I can tune my coil to two

different sorts of outputs, 1.  many small streamers breaking out

pretty much continuously, and 2.  fewer, larger streamers breaking

out occasionally.

The weather has not been ideal for running a TC lately, so I'm not

expecting optimal results, but which way should I be going for

the best tune?

Thanks,

CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Mon, 16 Mar 1998 18:51:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 16 Mar 1998 10:51:03 -0800

Subject: Re: Plate caps & connections

>

>

> Mike,

>    The cost of the mica from McMaster Carr was around $10 for a 24"x36" sheet

.

> I don't remember the exact price, but it was not outrageous.  I will look it

> up again if you wish.

I wish, thank you.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: My Lucky Day

Date: Mon, 16 Mar 1998 19:42:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Mon, 16 Mar 1998 13:42:02 -0600

Subject: RE: My Lucky Day

Hi,

What's cold cooking??

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Monday, March 16, 1998 12:40 PM

To:     Minnich, Roy

Subject:        My Lucky Day

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 13 Mar 1998 12:54:21 -0500

Subject: My Lucky Day

I just found a place that let me run free in their garbage pile. I

came back with everything they had except a 7500v transformer. here's

what I got:

5 15kv 30ma

2 12kv 30ma

1  9kv 30ma

Jackpot!

I'll probably need to cold cook all of them.

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: My Lucky Day

Cc:

Bcc:

X-Attachments:

At 07:42 PM 3/16/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Mon, 16 Mar 1998 13:42:02 -0600

>Subject: RE: My Lucky Day

>

>Hi,

>

>What's cold cooking??

>

>Roy Minnich

>

Roy,

I believe he was referring to a method used by Richard Quick

for unpotting neon transformers.

The idea is to remove the tar block from the case and discard the case.

Freeze the tar block outside during the winter or in the freezer

if you can get by with it.

Once its nice and frozen hook up the transformer to power.

Apply 120 V to the primary leads and short circuit the secondaries.

Let it cook this way for 20 minutes or so.

The freezing makes the tar hard and brittle. Running the transformer

for a bit warms up the tar right next to the transformer allowing

it to release easier. Sometimes the entire tar block will come off

in a couple big hunks this way.

I tried this method a couple times and it does help but these

days I just chisel away the tar without freezing or cooking.

I can clean up a tranny pretty good in a couple hours.

From there I disassemble the transformer and dump the parts

in some kerosene to dissolve off the remaining tar.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: spark gap & neon phase

Date: Tue, 17 Mar 1998 02:26:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Rick L. Kirby" <rkirby@earthlink.net>

Date: Mon, 16 Mar 1998 18:26:49 -0800

Subject: Re: spark gap & neon phase

Tesla-2 wrote:

>

> From: "smash" <smash@eisa.net.au>

> Date: Sat, 14 Mar 1998 12:42:45 +1100

> Subject: Re: spark gap

>

> To Anyone,

> >I like using an air-blast spark gap, you can use an adjustable static gap

> >and just have about 15 psi blowing through it, its very noisy but works

> >great!

>

> Where does one get a fan or compressor that would put out that much air?  :)

>

> And a question totally off the topic!  How does one phase neons so that they

> can be connected in parallel.  I have three 12000V/30mA neons at my house

> and I would like to connect them all in parallel.  (Please explain simply,

> I'm new to this)?

>

Hi Scott,

Gary Weaver has a web page on how to phase your neons at:

http://home.earthlink.net/~gweaver/trans-m.htm

Hope this helps. :>)

Rick Kirby

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: sec' ground arc

Date: Tue, 17 Mar 1998 03:51:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 16 Mar 1998 21:51:14 -0600

Subject: Re: sec' ground arc

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Mon, 16 Mar 1998 18:42:09 +0000

> Subject: sec' ground arc

>

> I am still having a problem with arcing between the primary and the

> rf ground connection on the sec', even after placing approx. 3/16"

> polyprop between the two.  Any suggestions on how to better insulate

> the two?

>

> Thanks, Chris

>

Chris,

Increasing the separation will work best. Because High Voltage RF can

jump further than the same voltage at 60 Hz, the easiest preventative is

simply to move the arcing elements further apart. Sometimes inserting a

dielectric material into the space between actually ends up worsenning

the arcing problem!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Advise on Secondary coil

Date: Tue, 17 Mar 1998 04:13:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 22:13:53 -0600

Subject: Re: Advise on Secondary coil

Tesla-2 wrote:

>

> From: <Tom.Stock@gsbsc.gensig.com>

> Date: Mon, 16 Mar 1998 12:07:15 -0500

> Subject: Advise on Secondary coil

>

>      I'm starting to plan my secondary coil, but I'm new to this and not

>      quite sure what parameters I should shoot for.

>

>      Right now I'm going for 4" Diameter x 22" long using #22 magnet wire.

>

>      This gives me a frequency of 522.90 KHz.

>

>      Is this ok for a 15KV 30ma Neon or should I go down to #24 or #26?

>

>      Thanks

>      -tom

>

Tom-

22 awg wire is fine to use. As a matter of fact I have a 4" coil wound

with 20 awg and a 6" coil wound with 22 awg. If you want a lower

frequency and more inductance go with 24 awg. But keep in mind that

either will work.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: sec' ground arc

Date: Tue, 17 Mar 1998 04:21:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 22:21:15 -0600

Subject: Re: sec' ground arc

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Mon, 16 Mar 1998 18:42:09 +0000

> Subject: sec' ground arc

>

> I am still having a problem with arcing between the primary and the

> rf ground connection on the sec', even after placing approx. 3/16"

> polyprop between the two.  Any suggestions on how to better insulate

> the two?

>

> Thanks, Chris

Chris

What type of primary are you using? Regardless, you probably don't have

enough space between the two. I would say that you should have at least

an inch between the two, I usually go more towards 1 1/2-2" to give me a

little bit more clearance. I hate to say it but it sounds like your

primary is way to close and will have to be widened around the

secondary. That is unless someone can provide another alternative to

recoiling it.

Chris J

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Streamers and tuning

Date: Tue, 17 Mar 1998 04:22:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 22:22:40 -0600

Subject: Re: Streamers and tuning

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Mon, 16 Mar 1998 18:42:09 +0000

> Subject: Streamers and tuning

>

> I have a question about streamer output.  I can tune my coil to two

> different sorts of outputs, 1.  many small streamers breaking out

> pretty much continuously, and 2.  fewer, larger streamers breaking

> out occasionally.

>

> The weather has not been ideal for running a TC lately, so I'm not

> expecting optimal results, but which way should I be going for

> the best tune?

>

> Thanks,

>

> CF

>

#2 the larger streamers!

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My Lucky Day

Date: Tue, 17 Mar 1998 04:39:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 22:39:38 -0600

Subject: Re: My Lucky Day

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 16 Mar 1998 20:04:42 -0600

> Subject: RE: My Lucky Day

>

> At 07:42 PM 3/16/98 GMT, you wrote:

> >From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> >Date: Mon, 16 Mar 1998 13:42:02 -0600

> >Subject: RE: My Lucky Day

> >

> >Hi,

> >

> >What's cold cooking??

> >

> >Roy Minnich

> >

>

> Roy,

>

> I believe he was referring to a method used by Richard Quick

> for unpotting neon transformers.

>

> The idea is to remove the tar block from the case and discard the case.

> Freeze the tar block outside during the winter or in the freezer

> if you can get by with it.

>

> Once its nice and frozen hook up the transformer to power.

> Apply 120 V to the primary leads and short circuit the secondaries.

> Let it cook this way for 20 minutes or so.

>

> The freezing makes the tar hard and brittle. Running the transformer

> for a bit warms up the tar right next to the transformer allowing

> it to release easier. Sometimes the entire tar block will come off

> in a couple big hunks this way.

>

> I tried this method a couple times and it does help but these

> days I just chisel away the tar without freezing or cooking.

> I can clean up a tranny pretty good in a couple hours.

>

> >From there I disassemble the transformer and dump the parts

> in some kerosene to dissolve off the remaining tar.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Something I wanted to point out, in my opinion. Is that I would not

recommend the cold cook method for salvaging reasons. What I mean is

that, it can be a real pain to get that large block of tar out of the

case to begin with. And personally I prefer not to destroy the case in

the process of tearing a neon down. So I would recommend the plain old

cooking method. The one where you cook it in the oven for a few hours.

It softens or melts the tar enough that you can clip the wires to the

insulators and the ground and it will then practically slide out of the

case without destroying the case. And the tar being as soft as it is

will pretty much "peel off" of the transformer easily. The only problem

is that the core gets pretty dang hot to the touch. If you are concerned

about the smell of the tar stinking everything up, then I can help.

First take a baking pan or something and line the pan with foil. Then

set your neon in the pan and get some wide aluminum foil and cover the

hole pan and kind of crumple the foil around all of the edges of the

pan. Tah dah..... this will help drastically on eliminating the tar

smell as you cook it.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: SERIOUS//Neon problem!!

Date: Tue, 17 Mar 1998 04:58:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 22:58:03 -0600

Subject: SERIOUS//Neon problem!!

I just got through responding to a post about cooking your neons in the

oven. It occured to me that maybe cooking neons in the oven can be a bad

thing. I had a problem one time where I unpotted a neon and cooked it in

the stove. Everything went fine, so I then disassembled it and cleaned

it in gasoline. I put everything back together exactly the way it came

apart. But here is the mystery when I would operate it the primary was

over heating badly. It was getting hot enough that the remaining tar

that was in between the primaries windings was oozing out and it was to

hot to even touch. Now this is what I am wondering. Could it be that the

heating in the oven could possibly breakdown the insulation on the

primary by melting it? Or could it be that I used gasoline to clean the

neon up with? I suspect the gasoline, because I see that most people

don't use gas to clean there neons up with. Most use kerosene, which if

I am not mistaken gasoline is a stronger solvent than kerosene is. I

think that when I am using gasoline I am destroying the insulation on

the primary wires and in the primary as a whole.

Someone please help me out on this one because I only have a few neons

around and I don't want them all to suffer the same problems with the

primary.  Thanks...

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tank circuit layout?

Date: Tue, 17 Mar 1998 05:21:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Mon, 16 Mar 1998 23:21:52 -0600

Subject: Tank circuit layout?

I currently use the cap across the transformer layout on my tank

circuit. But in recent posts on tesla and tesla-2 lists it is not

recommended to use this setup. I am wondering if someone can send me or

direct me to a diagram on how the other setup is? The main gap across

the transformer is what I am looking for. I have tried this setup before

with no luck. More than likely I was hooking something up wrong, but

hopefully someone can send some bits of knowledge on this my way.

thanks

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: My Lucky Day

Date: Tue, 17 Mar 1998 13:25:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Tue, 17 Mar 1998 08:25:14 -0500

Subject: RE: My Lucky Day

>Hi,

>What's cold cooking??

>Roy Minnich

oh, that's Mike Hammer's method of removing the tar from a supply so

you can rewind (actually unwind part of) the transformer.

You freeze the power supply, then connect the output terminals

together and run it for about 15-20 minutes. You then split the tar

block down the center to remove the transformer from the tar.

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Connecting Neons in Parallel

Date: Tue, 17 Mar 1998 15:38:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Tue, 17 Mar 1998 10:38:03 -0500

Subject: Connecting Neons in Parallel

I've read about connecting Neons in parallel to increase the max

current.  Can I do this with two Neons without wiring the primaries in

series and connecting to 240V?

In other words, can I connect both the primaries and secondaries in

parallel, with the primaries connected to 120V.

Thanks!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF Grounding

Date: Tue, 17 Mar 1998 16:56:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 17 Mar 1998 08:56:46 -0800

Subject: Re: RF Grounding

Organization: Self

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 13 Mar 1998 19:31:23 -0600

> Subject: Re: RF Grounding

>

> At 09:58 PM 3/13/98 GMT, you wrote:

> >From: <Tom.Stock@gsbsc.gensig.com>

> >Date: Fri, 13 Mar 1998 16:58:27 -0500

> >Subject: RF Grounding

> >     What's a simple and safe way to construct an RF ground?

> >

> >     Do I really have to somehow hammer 8 FEET of copper pipe into the

> >     ground? How many of these do I need?

Like to get opinion: My approach may be to bury 8 or 10' of .5" Cu tube

to 6" depth in yard, with 90 deg. bend(~10"), up against the wall, to

connect RFGnd lead to ; .062" perfs in lower surface of tubewall @ 1'

intervals; to drain NaCl solu'n, poured in at 10" up-leg.  'Green' use

of old bottle caps ? =:)-

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Connecting Neons in Parallel

Cc:

Bcc:

X-Attachments:

At 03:38 PM 3/17/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Tue, 17 Mar 1998 10:38:03 -0500

>Subject: Connecting Neons in Parallel

>

>

>     I've read about connecting Neons in parallel to increase the max

>     current.  Can I do this with two Neons without wiring the primaries in

>     series and connecting to 240V?

>

>     In other words, can I connect both the primaries and secondaries in

>     parallel, with the primaries connected to 120V.

>

>     Thanks!

>     -tom

You can just parallel the primaries and secondaries and

connect to 120 volts.

Or you can take pairs of neons and wire the secondaries

in parallel and primaries in series and feed with 240 volts.

You can parallel neons with different current ratings but

you cannot parallel neons with different voltage ratings.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tank circuit layout?

Cc:

Bcc:

X-Attachments:

At 05:21 AM 3/17/98 GMT, you wrote:

>From: Zuma /or CJ <mwise@ns.sosis.com>

>Date: Mon, 16 Mar 1998 23:21:52 -0600

>Subject: Tank circuit layout?

>

>I currently use the cap across the transformer layout on my tank

>circuit. But in recent posts on tesla and tesla-2 lists it is not

>recommended to use this setup. I am wondering if someone can send me or

>direct me to a diagram on how the other setup is? The main gap across

>the transformer is what I am looking for. I have tried this setup before

>with no luck. More than likely I was hooking something up wrong, but

>hopefully someone can send some bits of knowledge on this my way.

>

>thanks

>Chris

>

The two circuits differ only in the placement of the main

cap and main spark gap. Just swap the position of these two

components.

Performance wise there is no difference between the two configurations.

The shunt gap circuit is much easier on your neon transformers.

Here is a schematic for your reference.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: SERIOUS//Neon problem!!

Cc:

Bcc:

X-Attachments:

At 04:58 AM 3/17/98 GMT, you wrote:

>From: Zuma /or CJ <mwise@ns.sosis.com>

>Date: Mon, 16 Mar 1998 22:58:03 -0600

>Subject: SERIOUS//Neon problem!!

>

>I just got through responding to a post about cooking your neons in the

>oven. It occured to me that maybe cooking neons in the oven can be a bad

>thing. I had a problem one time where I unpotted a neon and cooked it in

>the stove. Everything went fine, so I then disassembled it and cleaned

>it in gasoline. I put everything back together exactly the way it came

>apart. But here is the mystery when I would operate it the primary was

>over heating badly. It was getting hot enough that the remaining tar

>that was in between the primaries windings was oozing out and it was to

>hot to even touch. Now this is what I am wondering. Could it be that the

>heating in the oven could possibly breakdown the insulation on the

>primary by melting it? Or could it be that I used gasoline to clen the

>neon up with? I suspect the gasoline, because I see that most people

>don't use gas to clean there neons up with. Most use kerosene, which if

>I am not mistaken gasoline is a stronger solvent than kerosene is. I

>think that when I am using gasoline I am destroying the insulation on

>the primary wires and in the primary as a whole.

>

>Someone please help me out on this one because I only have a few neons

>around and I don't want them all to suffer the same problems with the

>primary.  Thanks...

>

>Chris

Chris,

I believe it was probably the cooking that created your problem.

You probably got it a little too hot and removed some

of the insulation. Gasoline should not be a problem. Ive used

gas to clean some of my rebuilds with no apparent problems.

I normally chip away most of the tar. I try to get enough

removed so the core can be disassembled. Once its apart I

dump everything into kerosene to dissolve off the remaining

tar. After a couple days soaking in kero I dump everything into

gas for a day or two for final cleanup.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My Lucky Day

Cc:

Bcc:

X-Attachments:

At 04:39 AM 3/17/98 GMT, you wrote:

>From: Zuma /or CJ <mwise@ns.sosis.com>

>Date: Mon, 16 Mar 1998 22:39:38 -0600

>Subject: Re: My Lucky Day

>

>Something I wanted to point out, in my opinion. Is that I would not

>recommend the cold cook method for salvaging reasons. What I mean is

>that, it can be a real pain to get that large block of tar out of the

>case to begin with. And personally I prefer not to destroy the case in

>the process of tearing a neon down.

Actually I can remove the block rather quickly. I excavate out some of the

tar to get down to the HV bushings. I remove the bushings for later

reuse. Then I split the steel case open down two corners with a cold

chisel and a hammer. Pry down one of the sides of the case and pry out

the tar block with a large screwdriver. Toss the old case away.

I prefer to build a new lighter case from plastic or in the

case of my latest rebuild a sealed plywood tank to house the

transformer in oil.

> So I would recommend the plain old

>cooking method. The one where you cook it in the oven for a few hours.

>It softens or melts the tar enough that you can clip the wires to the

>insulators and the ground and it will then practically slide out of the

>case without destroying the case. And the tar being as soft as it is

>will pretty much "peel off" of the transformer easily. The only problem

>is that the core gets pretty dang hot to the touch. If you are concerned

>about the smell of the tar stinking everything up, then I can help.

>First take a baking pan or something and line the pan with foil. Then

>set your neon in the pan and get some wide aluminum foil and cover the

>hole pan and kind of crumple the foil around all of the edges of the

>pan. Tah dah..... this will help drastically on eliminating the tar

>smell as you cook it.

Im sure that many of us wont have the option of cooking a neon

in the kitchen oven. And I just plain prefer not to do it.

Heat is the enemy of transformers and I just cant see purposely

heating something up like that. Some neons have the primary coil

strapped together with plastic bands and some have plastic wedges

between the windings and core. These would all melt.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Advise on Secondary coil

Cc:

Bcc:

X-Attachments:

At 05:07 PM 3/16/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Mon, 16 Mar 1998 12:07:15 -0500

>Subject: Advise on Secondary coil

>

>

>     I'm starting to plan my secondary coil, but I'm new to this and not

>     quite sure what parameters I should shoot for.

>

>     Right now I'm going for 4" Diameter x 22" long using #22 magnet wire.

>

>     This gives me a frequency of 522.90 KHz.

>

>     Is this ok for a 15KV 30ma Neon or should I go down to #24 or #26?

>

>     Thanks

>     -tom

Tom,

A 4 incher wound with 22 inches of AWG 22 is exactly what

my older secondary coil was. This secondary was pretty well maxed

out at 45 inch streamers with around 12kv/95ma neon.

I replaced it with another 4 incher wound with 21 inches of AWG 24.

This resulted in higher inductance and a lower resonant frequency.

This allowed me to use a larger primary cap without sacrificing

primary coil turns.

I have yet to max this secondary out and have had 54 inch streamers

with 12kv/100ma.

If you have a choice go with AWG 24. It is the optimum size for

a 4 incher. If you already have the AWG 22 then use it.

Or save the AWG 22 for a future 6 inch diameter secondary.

AWG 22 is the optimum size for a 6 incher.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:spark gap and neon phasing

Cc:

Bcc:

X-Attachments:

At 06:27 PM 3/16/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Mon, 16 Mar 1998 10:27:39 -0800

>Subject: Re: spark gap

>

>>

>>

>> To Anyone,

>> >I like using an air-blast spark gap, you can use an adjustable static gap

>> >and just have about 15 psi blowing through it, its very noisy but works

>> >great!

>>

>> Where does one get a fan or compressor that would put out that much air?  :)

>>

>

The best method is to have a series arrangement of several gaps

and to suck air through the teeth of the gap with a powerful vacuum

cleaner motor. Or use your shop vac. This provides the best operation

of any of the static gap designs. A well made vacuum gap like this

can even be used with a pole pig transformer at several KVA.

>

>> And a question totally off the topic!  How does one phase neons so that they

>> can be connected in parallel.  I have three 12000V/30mA neons at my house

>> and I would like to connect them all in parallel.  (Please explain simply,

>> I'm new to this)?

>>

First of all you should start your coiling off with just

a single neon. Get things working well before worrying

about more power. There is a lot more to getting long sparks

than just power.

Once you need the additional power then set two of your

neons side by side. Connect the HV bushings together

on one side and connect a lead to the other side but

only connect it to one of the bushings.

Parallel up the 120 volt inputs and power them up.

Bring the loose lead that is attached to one HV bushing

to its counterpart on the other transformer completing

the parallel circuit. If you get a big spark then the phasing

of one transformer needs to be changed. Swap the 120 V leads

around on one transformer. If you get little or no spark

when completing the circuit you are phased correctly.

What I do is just wire then up and measure the current flow.

If its not whats expected I swap one set of 120v leads.

You can usually tell if they are not phased correctly.

If the transformers are fighting each other they will

usually hum rather loudly but make little noise when

phased properly.

Hope that helps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Specs

Date: Tue, 17 Mar 1998 21:10:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Moleking <moleking@concentric.net>

Date: Tue, 17 Mar 1998 16:10:30 -0500

Subject: Specs

I finished my first coil over the weekend and gave it the preliminary runs.

Here are the specs:

Power:  15kv 30ma NST

Main Gap:  4 L-bracket gaps in series w/ app. 2mm spacing each.  Fan for

quenching. Mounted on plexiglass

Caps:  2 .01uf rolled poly caps in series

Primary:  13 turns insulated #12 solid wire, cylindrical.  Tapped at 12

turns where i've gotten most output.

Secondary:  22" #24 wire on 4.5"  OD PVC form.

Protection:  Safety gap at neon, 2 chokes before main gap each 4" #28 wire

on 2" PVC.

Ground:  Solid #12 wire from bottom of secondary to large drainage pipe in

basement.

I'm wondering how I can get better performance.  So far, I've gotten sparks

of 8" to break out in 4 places at once.  I haven't added a toroid yet,

since I get no output at all when I add one made of 4" flexible aluminum

dryer duct.  I'm anxious to add one, since I'm losing lots of power now to

corona on the upper half of the secondary.  Can anyone suggest how to get

better output, and better performance from a toroid?  Thanks for the help.

Josh Bedell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: SERIOUS//Neon problem!!

Date: Wed, 18 Mar 1998 00:06:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 17 Mar 1998 16:06:29 -0800

Subject: Re: SERIOUS//Neon problem!!

Why would you want to take the neon apart and repot it anyway?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Advise on Secondary coil

Date: Tue, 17 Mar 1998 21:27:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Tue, 17 Mar 1998 16:27:02 -0500

Subject: Re: Advise on Secondary coil

>I replaced it with another 4 incher wound with 21 inches of AWG 24.

>This resulted in higher inductance and a lower resonant frequency.

>This allowed my to use a larger primary cap without sacrificing

>primary coil turns.

>I have yet to max this secondary out and have had 54 inch streamers

>with 12kv/100ma.

Thanks Mike,

I decided to go with #24 shortly after posting that message, although

the reason was mainly because I found a good deal on AWG 24. Now, it

looks like I lucked out. I'm planning on a 4" x 21" secondary. I'll be

running 3 of my 15KV transformers in parallel (providing I can get 3

to match up ok), so this should be exciting.

If I get 54 inch streamers I'll be extremely satisfied!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: SERIOUS//Neon problem!!

Cc:

Bcc:

X-Attachments:

At 12:06 AM 3/18/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Tue, 17 Mar 1998 16:06:29 -0800

>Subject: Re: SERIOUS//Neon problem!!

>

>

>

>Why would you want to take the neon apart and repot it anyway?

>

Mike,

Generally speaking you dont unpot good working neons.

But you can often get non working "dead" neons for free.

You can unpot and rebuild these dead neons and restore

them to operation. Not only that but the end result is

a transformer thats better than new.

It will have better insulation and better heat dissipation

by having the tar removed and you can boost the output

by removing some of the magnetic shunts.

My latest rebuild project took a dead France 15kv/60 ma

neon I got for free and reworked it into a working

12kv/100 ma neon. Some of one of the secondary windings was

damaged so I had to remove quite a few turns to get down

to good wire. There was enough left over for 12kv so I reworked

the other secondary as well.

I put this transformer into a tank of mineral oil and its

just about bullet proof.

You can see some pics of this transformer on my web page at:

http://www.misslink.net/mhammer/rebuild.htm

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Specs

Cc:

Bcc:

X-Attachments:

At 09:10 PM 3/17/98 GMT, you wrote:

>From: Moleking <moleking@concentric.net>

>Date: Tue, 17 Mar 1998 16:10:30 -0500

>Subject: Specs

>

>I finished my first coil over the weekend and gave it the preliminary runs.

> Here are the specs:

>

>Power:  15kv 30ma NST

>

>Main Gap:  4 L-bracket gaps in series w/ app. 2mm spacing each.  Fan for

>quenching. Mounted on plexiglass

Ditch this gap in favor of a RQ style cylinder gap made with hard copper

pipe. The cylinder gap is just about the best gap there is for neons.

For 15kv use you will want a total of 10 series gaps (11 electrodes)

spaced no more than .03 inches apart. This gives a total maximum

gap of .300 inch. Thats plenty for 15kv.

>

>Caps:  2 .01uf rolled poly caps in series

This is good.

>

>Primary:  13 turns insulated #12 solid wire, cylindrical.  Tapped at 12

>turns where i've gotten most output.

Sounds like you need more primary turns to me.

>

>Secondary:  22" #24 wire on 4.5"  OD PVC form.

Good.

>

>Protection:  Safety gap at neon, 2 chokes before main gap each 4" #28 wire

>on 2" PVC.

Good. You should probably use some resistors in series with your

air core chokes. It will help protect your neon from spikes

generated in your chokes. Anything from 300-1500 ohms will work

and should be at least 10 watts for a 30ma neon.

>

>Ground:  Solid #12 wire from bottom of secondary to large drainage pipe in

>basement.

Probably OK.

>

>I'm wondering how I can get better performance.  So far, I've gotten sparks

>of 8" to break out in 4 places at once.  I haven't added a toroid yet,

>since I get no output at all when I add one made of 4" flexible aluminum

>dryer duct.  I'm anxious to add one, since I'm losing lots of power now to

>corona on the upper half of the secondary.  Can anyone suggest how to get

>better output, and better performance from a toroid?  Thanks for the help.

>

>Josh Bedell

Josh,

You are getting no output when you add the toroid because the toroid

is lowering the resonant frequency of your secondary coil. You then

need to retune the primary circuit to tune it to this new lower

frequency. But you dont appear to have enough primary turns to tune

the frequency low enough.

First thing I suggest is using a smaller terminal than your big toroid.

Try a 5 or 6 inch diameter styrofoam ball covered with foil. You can even

use one of those copper toilet tank floats. These terminals wont lower the

frequency as much as the toroid.

Add several more primary turns to your primary coil. You will need

those extra turns to tune whatever terminal you want to use.

Also the additional primary inductance will help to improve

the operation of your spark gap. This will improve efficiency.

Keep at it.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Ferrite Choke Questions

Date: Tue, 17 Mar 1998 22:19:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Tue, 17 Mar 1998 15:19:57 -0700 (MST)

Subject: Ferrite Choke Questions

Once upon a time I wrote:

>>3) Exactly how long and what diameter should the inductors be? (the pipe

>>and the wire).

and Mike Hammer replied:

>I dont recommend air core chokes wound on PVC pipe unless you

>have no other choice. The best bet is a toroidal choke wound

>on a large ferrite toroid. You need a minimum of 2 mh of inductance

>but this value is not critical. Anything from 2-10 mh is fine.

I'm sorry -- I hardly know anything about chokes. Could you go into a

little more detail on how to make these toroidal-ferrite types? Any big

do's or don'ts?

>My chokes are about 120 turns of AWG 26 wound on surplus 2.5 inch

>diameter ferrite toroids. Ferrite rods will also work fine.

Where do you get this ferrite material? Ham catalogs? Radio Shack?

Where could I get some equations that would let me calculate inductance?

Thanks!

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specs

Date: Tue, 17 Mar 1998 22:41:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 17 Mar 1998 14:41:08 -0800 (PST)

Subject: Re: Specs

On 17 Mar, Tesla-2 wrote:

> Caps:  2 .01uf rolled poly caps in series

>

> Primary:  13 turns insulated #12 solid wire, cylindrical.  Tapped at 12

> turns where i've gotten most output.

This is going to be a problem, see later comments.

> I'm wondering how I can get better performance.  So far, I've gotten sparks

> of 8" to break out in 4 places at once.  I haven't added a toroid yet,

> since I get no output at all when I add one made of 4" flexible aluminum

> dryer duct.  I'm anxious to add one, since I'm losing lots of power now to

> corona on the upper half of the secondary.  Can anyone suggest how to get

> better output, and better performance from a toroid?  Thanks for the help.

Here's your problem: With a toroid, you cannot get tune. I don't know

what the diameter of your toroid is, but I can tell you that on my

small coil, I am in tune at about 8.5 turns, and if I pull off

the toroid, I have to tap at about 3. You need to add at least 4-5 more

turns to your primary to give you the room you need to tune in

the topload, or increase your capacitance.

You may want to consider switching to a helix, or inverted cone primary.

As an FYI: I have managed to get 14" arcs from 12KV/30ma on a 3"x10"

coil with a 3"x12" toriod.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: My Lucky Day

Date: Wed, 18 Mar 1998 00:01:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Tue, 17 Mar 1998 19:01:47 -0500 (EST)

Subject: RE: My Lucky Day

On Mon, 16 Mar 1998, Tesla-2 wrote:

>

> What's cold cooking??

>

Stuffing it in the freezer overnight, and whacking the tar off(more

precise than that)  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

"BETTER LIVING THROUGH RECKLESS EXPERIMENTATION" - Motto of The Mad Scientist

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Specs

Date: Wed, 18 Mar 1998 00:46:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Tue, 17 Mar 1998 19:46:17 EST

Subject: Re: Specs

I'm working on a very similar coil to Josh's & I thought the capacitor rating

should be about .0053 uf. Is it safe to increase the capacitance this much on

a 15K 30 ma neon xfrmr? If so, will it produce better output without reducing

it's life expectancy? I've been advised previously not to increase my cap

rating too high, but would also like better spark lengths.

Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ferrite Choke Questions

Cc:

Bcc:

X-Attachments:

At 10:19 PM 3/17/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Tue, 17 Mar 1998 15:19:57 -0700 (MST)

>Subject: Ferrite Choke Questions

>

>Once upon a time I wrote:

>

>>>3) Exactly how long and what diameter should the inductors be? (the pipe

>>>and the wire).

>

>and Mike Hammer replied:

>

>>I dont recommend air core chokes wound on PVC pipe unless you

>>have no other choice. The best bet is a toroidal choke wound

>>on a large ferrite toroid. You need a minimum of 2 mh of inductance

>>but this value is not critical. Anything from 2-10 mh is fine.

>

>I'm sorry -- I hardly know anything about chokes. Could you go into a

>little more detail on how to make these toroidal-ferrite types? Any big

>do's or don'ts?

They are rather easy to wind. Since ferrite is slightly conductive

you need to insulate the core. Use 3 layers of good quality vinyl

tape. Then wind on as many turns as you can. Leave at least

3/4 inch between the start and finish to prevent arcing.

I dipped mine in polyurethan varnish for a bit of extra

insulation and to secure the windings.

>

>>My chokes are about 120 turns of AWG 26 wound on surplus 2.5 inch

>>diameter ferrite toroids. Ferrite rods will also work fine.

>

>Where do you get this ferrite material? Ham catalogs? Radio Shack?

Well the cores I have were purchased from Hosfelt Electronics

as surplus. They were $1.50 each so I got 8 of them. Unfortunately

they have sold out of them and its unlikely they will have more.

I have seen similar cores at hamfests and last year I bought

some 3/4 inch ferrite rods at a hamfest.

New ferrite as well as powdered iron cores can be purchased

from Amidon Associates. Their web page is:

http://bytemark.com/amidon/index1.htm

>

>Where could I get some equations that would let me calculate inductance?

There are some formulas on Amidon's web page to determine inductance

values of inductors wound on their cores.

For air core RF chokes you can use Wheeler's formula:

L(uH) = (r^2) * (N^2) / (9*r + 10*h)

where:

r = coil RADIUS in inches

N = number of turns

h = coil height in inches

Note that this formula can also be used to calculate the inductance

of your secondary coil.

Air core RF chokes can be used effectively if you cant locate

or afford ferrite toroids. You should use a little series resistance

with air core chokes for added protection. Anything from 300-1500

ohms and enough wattage to handle the expected power will be fine.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Specs

Cc:

Bcc:

X-Attachments:

At 12:46 AM 3/18/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Tue, 17 Mar 1998 19:46:17 EST

>Subject: Re: Specs

>

>I'm working on a very similar coil to Josh's & I thought the capacitor rating

>should be about .0053 uf. Is it safe to increase the capacitance this much on

>a 15K 30 ma neon xfrmr? If so, will it produce better output without reducing

>it's life expectancy? I've been advised previously not to increase my cap

>rating too high, but would also like better spark lengths.

>Thanks, Jim

>

Jim,

Josh is using TWO .01 mfd caps in series to yield .005 mfd.

This will provide a good match for his neon as well as reducing

the stress on the caps.

You are correct that .0053 mfd is that matching size for a 15kv/30ma

neon. But the value is not critical.

You may be able to increase your capacitance slightly as most

neons put out a little more than their name plate suggests.

The matching cap value for neons just gives us an idea

how big a cap any particular transformer can effectively charge.

You dont have to be a slave to any particular value. Its a guideline only.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: SERIOUS//Neon problem!!

Date: Wed, 18 Mar 1998 02:35:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Tue, 17 Mar 1998 20:35:53 -0600

Subject: Re: SERIOUS//Neon problem!!

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Tue, 17 Mar 1998 16:06:29 -0800

> Subject: Re: SERIOUS//Neon problem!!

>

> Why would you want to take the neon apart and repot it anyway?

>

One of the secondaries was shorted to the case, so I pretty much had to

unpot it.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My Lucky Day

Date: Wed, 18 Mar 1998 03:24:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Tue, 17 Mar 1998 22:24:40 EST

Subject: Re: My Lucky Day

I was just reading about the different opinions on "cold cooking". Has anyone

ever tried freezing the potting material and just chipping it away.  I realize

that one would have to be careful not to damage the transformer, but tar

becomes very brittle when cold and this could be a way to remove much of it

without any gooey mess.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My Lucky Day

Cc:

Bcc:

X-Attachments:

At 03:24 AM 3/18/98 GMT, you wrote:

>From: Hollmike <Hollmike@aol.com>

>Date: Tue, 17 Mar 1998 22:24:40 EST

>Subject: Re: My Lucky Day

>

>I was just reading about the different opinions on "cold cooking". Has anyone

>ever tried freezing the potting material and just chipping it away.  I realize

>that one would have to be careful not to damage the transformer, but tar

>becomes very brittle when cold and this could be a way to remove much of it

>without any gooey mess.

>Mike Hollingsworth

>

>

Yes this is the way I do it now. During the

winter I place the tar blocks outside to freeze.

If Im doing one during warmer weather I forego

the freezing part and just chip the tar away.

Its often brittle enough to break off in rather large chunks.

The only danger is when you get down close to the secondaries.

Its very easy to damage one of the windings.

For that reason I use kerosene to remove the last bit.

Lots safer that way.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My Lucky Day too

Date: Wed, 18 Mar 1998 07:26:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 17 Mar 1998 23:26:41 -0800

Subject: Re: My Lucky Day too

Just found a whole ship load of 1/4" plexiglass at the dump, any good uses for

it

in tesla?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My Lucky Day too

Cc:

Bcc:

X-Attachments:

At 07:26 AM 3/18/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Tue, 17 Mar 1998 23:26:41 -0800

>Subject: Re: My Lucky Day too

>

>

>

>Just found a whole ship load of 1/4" plexiglass at the dump, any good uses

>for it in tesla?

>

Yes! Geez some guys have all the luck. It would be great for building

capacitor tanks and tanks for rebuilt neons.

Mounts for primary coil. Spark gap housings. All kinds of things.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Unpotting Neons

Date: Wed, 18 Mar 1998 13:40:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 18 Mar 1998 08:40:43 -0500

Subject: Unpotting Neons

Well, I cracked open my Neon last night and it wasn't easy. There is

no hope of the wife letting me put that thing in the freezer (no

matter how well I disguise it) and it's never cold enough in Florida

to make tar brittle, so the best i could do was set it in a bucket of

ice water.

I removed the tar and came to some odd discoveries. I'm awaitng

further guidance:

Both secondary coils were slightly damaged during unpotting. I should

be able to remove a few winds and get them back in shape. The weird

thing I noticed is that they were wound with parallel strands of fine

wire. In other words, there were two hair like wires soldered to each

lead wire, not one. This makes unwinding them a little more difficult,

but not much.

I didn't see any shunts! I DID find some cardboard (type stuff, now

pretty much destroyed) between the primary and secondary windings. But

only one piece between each. 2 pieces of cardboard stuff total. Was

this a cheapo shunt or just a support for the transformer before the

tar was poured in?? What do the shunts look like?

My transformer is soaking in a bucket of gasoline.

By the way I can't remember the name of this thing but it's not a

franceformer....it's something like "elektratek" I *think*.

(Grey box with insulators on each end)

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My Lucky Day too

Date: Wed, 18 Mar 1998 15:48:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 18 Mar 1998 07:48:44 -0800

Subject: Re: My Lucky Day too

Organization: Self

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Tue, 17 Mar 1998 23:26:41 -0800

> Subject: Re: My Lucky Day too

>

> Just found a whole ship load of 1/4" plexiglass at the dump, any good >uses f

or it in tesla?

Mike, If I had a shipload of .25" plex, there would be many uses! No

experimenter is of any value w/o a decent boneyard! 3'x3' piece w/ about

a 6" dia hole in it to wind my primary UNDER, would be my 1st option...

Don' know your sizes, but gaps, caps and general structures could consume

much of this stuff.

can you go into any sort of business..  ?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RFGnd ?

Date: Wed, 18 Mar 1998 16:27:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 18 Mar 1998 08:27:52 -0800

Subject: RFGnd ?

Organization: Self

All,

while bone-yarding, I came across remnant of reel (~15') of heavy

braided cable: tinned Cu.  Pulled taut, it is about .37" OD, and could

be pulled into an = length of .5"OD  Tygon tubing.... Now I know that

braid is NG for RF, but 'HOW' NG???  Even the tinning wouldn't enhance

RF conductivity enough to 'do' in the interim?  Mainly I like the clean

flexibility this would present, over solid heavy wire...  Haven't worked

out the minimum length we should need to access the GND installation,

yet (re: my note on horizontal burial of perforated .5" OD Cu tube

w/NaCl internal bath?).

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting Neons

Cc:

Bcc:

X-Attachments:

At 01:40 PM 3/18/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Wed, 18 Mar 1998 08:40:43 -0500

>Subject: Unpotting Neons

>

>

>     Well, I cracked open my Neon last night and it wasn't easy. There is

>     no hope of the wife letting me put that thing in the freezer (no

>     matter how well I disguise it) and it's never cold enough in florida

>     to make tar brittle, so the best i could do was set it in a bucket of

>     ice water.

I dont approve of water. Just the thought of water and electronic

devices gives me a headache.

>

>     I removed the tar and came to some odd discoveries. I'm awaiting

>     further guidance:

>

>     Both secondary coils were slightly damaged during unpotting. I should

>     be able to remove a few winds and get them back in shape. The weird

>     thing I noticed is that they were wound with parallel strands of fine

>     wire. In other words, there were two hair like wires soldered to each

>     lead wire, not one. This makes unwinding them a little more difficult,

>     but not much.

Thats interesting. Ive never seen that in any of the neons Ive unpotted.

Is this a 30ma or 60ma neon? Verify the continuity of the secondary

windings with an ohmeter.

The loss of some turns is no big deal. Ive repaired neons

and lost a couple hundred turns. A couple hundred turns out

of thousands isnt a great loss.

>

>     I didn't see any shunts! I DID find some cardboard (type stuff, now

>     pretty much destroyed) bewteen the primary and secondary windings. But

>     only one piece between each. 2 pieces of cardboard stuff total. Was

>     this a cheapo shunt or just a support for the transformer before the

>     tar was poured in?? What do the shunts look like?

The cardboard is el cheapo insulation. Sometimes it has mica imbedded

in it. Remove it and discard.

Replace the lost insulation with a strip of plastic. I used a strip

of HDPE cut from a plastic bottle. Or plan on immersing the rebuilt

transformer in oil.

If your transformer is a Jefferson Electric (Magnetek) then the

shunts are part of the core itself. Look for core extensions

between the primary and secondary windings. They are not seperate

pieces. You need to delaminate the core and cut the extensions off

of a few laminations.

Unfortunately this isnt a reversible procedure  so do only a couple

and then test. You can safely remove 3 shunts from 30 ma neons

and 4 or maybe 5 from a 60 ma unit.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Plate Capacitor construction instructions.

Date: Wed, 18 Mar 1998 18:27:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: John Brudvik <john.brudvik@wcom.com>

Date: Wed, 18 Mar 1998 12:27:34 -0600

Subject: Plate Capacitor construction instructions.

In Dec. someone offered to provide plate capacitor construction plans to

anyone interested in them.  I would like to get a copy of these plans.  You

can send them directly to me at john.brudvik@wcom.com.

Thanks.

John

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My lucky day three

Date: Wed, 18 Mar 1998 19:13:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 18 Mar 1998 14:13:47 -0500

Subject: My lucky day three

Ok so my dad used to work for the power company, and he says he can

get me a few gallons of transformer oil for free! :) Possibly as much

as I want!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Wed, 18 Mar 1998 22:16:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Wed, 18 Mar 1998 14:16:29 -0800

Subject: Re: RFGnd ?

Organization: EES

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Wed, 18 Mar 1998 08:27:52 -0800

> Subject: RFGnd ?

> Organization: Self

>

> All,

> while bone-yarding, I came across remnant of reel (~15') of heavy

> braided cable: tinned Cu.  Pulled taut, it is about .37" OD, and could

> be pulled into an = length of .5"OD  Tygon tubing.... Now I know that

> braid is NG for RF, but 'HOW' NG???  Even the tinning wouldn't enhance

> RF conductivity enough to 'do' in the interim?  Mainly I like the clean

> flexibility this would present, over solid heavy wire...  Haven't worked

> out the minimum length we should need to access the GND installation,

> yet (re: my note on horizontal burial of perforated .5" OD Cu tube

> w/NaCl internal bath?).

>

> Gene Waldo

Gene,

I've noticed a number of remarks from list members regarding the RF

properties of braided conductor, specifically, that it is unacceptable

for grounding purposes at RF. I can only tell you that untinned copper

braid is commonly REQUIRED for interconnect bonding and grounding of

equipment cabinets in radio stations, cable TV headends, and other RF

environments. My understanding is that the skin effect is enhanced by

the increased surface area of this type of conductor.

If there is something I'm missing here, I hope someone on this list will

correct me.

--

Regards,

Scott MacGregor, EES

************************************************************

Enterprise Engineering Services

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: BIG Coil!

Date: Wed, 18 Mar 1998 22:33:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 18 Mar 1998 14:33:19 -0800

Subject: BIG Coil!

Organization: Self

All,

Has anyone worked the numbers on a drive system for .... like a 24" x

12' secondary?  I immagine that only an institutuion could tackle

something like this? ....    Do the equations scale up in a regular

fashion?

What is the largest system "we" account for? ..could account for? Are

there limits on an 'air core' (or wooden) 2ndary?

Gene Waldo  ~~=:)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plate Capacitor construction instructions.

Cc:

Bcc:

X-Attachments:

At 06:27 PM 3/18/98 GMT, you wrote:

>From: John Brudvik <john.brudvik@wcom.com>

>Date: Wed, 18 Mar 1998 12:27:34 -0600

>Subject: Plate Capacitor construction instructions.

>

>In Dec. someone offered to provide plate capacitor construction plans to

>anyone interested in them.  I would like to get a copy of these plans.  You

>can send them directly to me at john.brudvik@wcom.com.

>

>Thanks.

>

>John

>

John,

I dont think it was me but I do have a text file

detailing how I constructed my first plate cap.

You can download the file "mikecap.txt" from

Dave Bronakowski's archive site. The URL is:

http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: BIG Coil!

Cc:

Bcc:

X-Attachments:

At 10:33 PM 3/18/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Wed, 18 Mar 1998 14:33:19 -0800

>Subject: BIG Coil!

>Organization: Self

>

>All,

>Has anyone worked the numbers on a drive system for .... like a 24" x

>12' secondary?  I immagine that only an institutuion could tackle

>something like this? ....    Do the equations scale up in a regular

>fashion?

The proper H/D (height/diameter) ratio is 3 to 1. So

a 24 incher would be around 72 inches tall.

>Do the equations scale up in a regular fashion?

Yes for the most part as do the costs and problems encountered.

More likely the problems and cost go up by the cube of the diameter. :)

>What is the largest system "we" account for? ..could account for? Are

>there limits on an 'air core' (or wooden) 2ndary?

>

The largest coils on record have been Robert Golkas magnifier

which was modeled after Tesla's Colorado Springs machine

and Bill Wysocks large magnifier.

Both of these machines may soon be dwarfed by Greg Leyh's monster

coil. It a conventional 2 coil Tesla coil with a 37 X 154 inch

secondary and 84 inch diameter spherical terminal. Anticipated

power input is to be 130 KW.

Some photos and information on this coil are available at:

http://www.lod.org/

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Water and High Voltage: A cure for TENSION.

Date: Thu, 19 Mar 1998 03:22:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 18 Mar 1998 21:22:18 -0600

Subject: Water and High Voltage: A cure for TENSION.

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>     Well, I cracked open my Neon last night and it wasn't easy. There is

>     no hope of the wife letting me put that thing in the freezer (no

>     matter how well I disguise it) and it's never cold enough in florida

>     to make tar brittle, so the best i could do was set it in a bucket of

>     ice water.

Then Mike Hammer replied:

>I dont approve of water. Just the thought of water and

>electronic devices gives me a headache.

Hmm. Last night I drove my open-bed pickemup truck

(filled with two X-ray tubes, a collimator, one image

intensifier, 100 feet of 70KV-rated coax cable, and a

plumbicon video camera) through a 10-mile

thunderstorm...

(I was at the right place - at the right time...)

Tonight I'll go back for more!  Want to tag along,

Mike? Anyone? :)

Keep coilin' !

-Bill the arcstarter/junkman

http://www.geocities.com/CapeCanaveral/Hangar/6160/electric.html for pics of a

recent haul...

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Thu, 19 Mar 1998 03:36:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 18 Mar 1998 21:36:27 -0600

Subject: Re: BIG Coil == big disaster?

Organization: MailExcite  (http://www.mailexcite.com)

It was written:

>Has anyone worked the numbers on a drive system for .... like a 24" x

>12' secondary?  I immagine that only an institutuion could tackle

>something like this? ....    Do the equations scale up in a regular

>fashion?

Hmm.  Do you mean a secondary which is 24 feet long and

1 foot in diameter?  This would be a less than optimal

h/d ratio... :)

I bet the $$$ and time will increase with the 3rd power of the diameter.  Any takers?

I know of two local fellows who are attempting LARGE

coils w/o first spending significant time on smaller

units.  This is probably a recipe for failure...

One guy wants to run his (pending) 50KW coil using a

humongous salt-water cap!  (Just like Tesla did).

But I bet Tesla would've used poly if he'd had it!

I ordered a couple of "TIG contactor points" from the

local welding shop.  These are supposedly the tungsten

spark gaps used in commercial TIG welders for the HF

arc stabilization circuit.  I'll see if they'd be good

for a rotary gap or not...  I'll let you know what

I learn.

My present coil secondary is 4.25 by 26 inches,

basically identical to Mike Hammer's design.  And I

could NOT have done it without help from tesla-2 -

especially Mr. Hammer's help.  Thanks Mike!!

Are there any coiler who still frequent tesla-2 who are

operating larger secondaries??

-Bill the arcstarter

Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: big caps

Date: Thu, 19 Mar 1998 04:22:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 18 Mar 1998 20:22:05 -0800

Subject: big caps

Organization: Self

All,

Need info: I've located 2 ea. 42.0mfd, 7.7kvdc jobs. $?; oper unkn @

this time, but they will test before we drag them out!

Along with several $50 ea. 2.0mfd, 12.5kvdc MICAMOLD.

Are these apt to be very durable at 7.5kvac? What kind of Tank ckt

would  it take to handle this capacitance?  Is anyone interested in

them?

Also If anyone is in need of special wire or cable, I have access to

almost anything that was ever made! At near scrap prices! Found several

hundred feet of looks like .75"od RF wire: ~.125"  solid Cu center,

white (TFE?) insul, in black neop jacket; Unshielded if I recall

correctly....

They have rolls and rolls of everything! From fine enam.magnet wire thru

Belden signal cable to big power cables. They don' ship, but it's there!

Any use for a lab type 0-50 RF Watt Meter?

Could we use inductive (hole thru center) lab type  5:1 current xfmrs

any where in our circuit? bunches of them...

Gene Waldo   ~~=:-)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: coupling adjusting mechanism

Date: Thu, 19 Mar 1998 07:08:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Wed, 18 Mar 1998 23:08:50 -0800

Subject: coupling adjusting mechanism

Hi, everyone. I'm wondering about a means to adjust my primary. I suppose

that when the time comes, I can simply prop it up with 2x4's, but I am

looking for a cleaner method. I have thought about using something like

(4) - 12" long x 3/4" plastic bolts. I could run the threaded part

through the base of the secondary first then add a nut and finally go

through the base plate of the primary. In this way, I could adjust the

hieght of the nuts up and down (thus moving my primary up and down) to do

my tuning. I think though that plasitic bolts such as these are rather

spendy and I'm sure someone out there has a good answer, What is it?

Thanks, Walt Kulpa, Bellingham, Wa.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: coupling adjusting mechanism

Date: Thu, 19 Mar 1998 06:40:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Thu, 19 Mar 1998 06:40:23

Subject: Re: coupling adjusting mechanism

At 07:08 AM 3/19/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Wed, 18 Mar 1998 23:08:50 -0800

>Subject: coupling adjusting mechanism

>

>Hi, everyone. I'm wondering about a means to adjust my primary. I suppose

> that when the time comes, I can simply prop it up with 2x4's, but I am

>looking for a cleaner method. I have thought about using something like

>(4) - 12" long x 3/4" plastic bolts. I could run the threaded part

>through the base of the secondary first then add a nut and finally go

>through the base plate of the primary. In this way, I could adjust the

>hieght of the nuts up and down (thus moving my primary up and down) to do

>my tuning. I think though that plasitic bolts such as these are rather

>spendy and I'm sure someone out there has a good answer, What is it?

>Thanks, Walt Kulpa, Bellingham, Wa.

>

Walt,

In your 'subject' matter, you have 'coupling' and in the body of

your message, you have 'tuning'. I am going to assume you meant coupling

instead of tuning. I thought about this same thing before and came up with

a method that I still use. Coupling is a placement of one coil with the

other coil's infuence. Instead of have my primary move, I decided to have

the secondary move. I find, or make, a tube whose outside diameter is the

secondary's inside diameter and make it a base for my secondary. Then I

have a 'telescopic' setup for my secondary and can adjust it's height to

the coupling I wish to achieve. By having the primary about an inch or 2

above the bas platform, I can move the secondary down to get over-coupling

if I want to, which I have puposely done on occasion to photograph the

field produced. Hope this helps,

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Thu, 19 Mar 1998 06:42:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 18 Mar 1998 22:42:51 -0800 (PST)

Subject: Re: BIG Coil == big disaster?

>It was written:

>

>>Has anyone worked the numbers on a drive system for .... like a 24" x

>>12' secondary?

>

>Hmm.  Do you mean a secondary which is 24 feet long and 1 foot in diameter?

>This would be a less than optimal h/d ratio... :)

>

No, I think he means 2 feet in diameter, by 12 feet tall. At least, that's

what he has written there.  But you're right, it's still less than optimal.

With a 2 foot diameter and 3:1 ratio, it would only need to be 6 feet tall.

(ONLY - right!)  ~<:-)

danmurphy@1stnetusa.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: coupling adjusting mechanism

Date: Thu, 19 Mar 1998 09:45:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 19 Mar 1998 01:45:04 -0800

Subject: Re: coupling adjusting mechanism

Organization: Self

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Wed, 18 Mar 1998 23:08:50 -0800

> Subject: coupling adjusting mechanism

>

> Hi, everyone. I'm wondering about a means to adjust my primary. I suppose

>  that when the time comes, I can simply prop it up with 2x4's, but I am

> looking for a cleaner method.  What is it?

> Thanks, Walt Kulpa, Bellingham, Wa.

Walt, I'm certain that for each winder there are several tried

approaches; and one which has become settled upon.  If I can convince

"ol' Quick and Dirty Dan", I think our approach should be to:

1)Mount 4" dia pvc end cap to center of Tesla Base, to recieve and

secure 2ndry Base w/3 nylon 1/4-20 screws @ 120 deg. At center of cap is

Cu RFGnd contact, which mates to similar at base of 2ndary.

2)Mount 4 vertical .5" dia wood dowels at corners of Tesla Base: ht=

about 8", to recieve and secure 4 .55" holes on corners of Pri Base,

made of 1/4" hard composit 'peg board' (2 or 3' sq.(w/1' hole grid?)),

underwhich, and secured w/small cable ties, is the (.312" Cu tube)flat

Pri Winding. This will also provide suitable work surface for coffee,

tools, etc., which can be shimmed up or down w/4 sets of wood shims,

until optimum coupling is achieved. There should be about a 5" dia perf

at center of Pri Base to accomodate the fixed 2ndary coil form thru.

3) I also would have several similarly symetrical end caps available at

top of 2ndary, for the fitting and application of various Terminals.

4) I am asumming (lacking good data to contrary) that the outer (tuning)

Pri adjustment can be a simple battery or aligator clip?

5) Assemble and test!  ~~:(

As I have indicated in earlier posts, this will be T#2 for me.  #1 was

many years ago and consisted of only the vaguest application of EE

principles! It worked fine! Sans any Gnd! Of course we had no standards

by which to judge, but we were happy! Two coils and a 12kvac Nxfmr!

The old 2ndary was, as I visualize it, 3" dia x 30" wind of perhaps

.015-.020" enam wire, wound on cardboard on Dad's screw-cutting lathe;

it was beautifull!  Pri was about 10" dia sol: 10 or 14 T of some solid

house wiring, dead across xfmr!  I was 16 or 17, in those days, I think.

So! I'm still guessing, too! And would appreciate critique, also. But I

think the above should serve to get things rolling fairly simply...

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Thu, 19 Mar 1998 09:54:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 19 Mar 1998 01:54:37 -0800

Subject: Re: BIG Coil == big disaster?

Organization: Self

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Wed, 18 Mar 1998 22:42:51 -0800 (PST)

> Subject: Re: BIG Coil == big disaster?

>

> >It was written:

> >

> >>Has anyone worked the numbers on a drive system for .... like a 24" x

> >>12' secondary?

> >

> >Hmm.  Do you mean a secondary which is 24 feet long and 1 foot in diameter?

> >This would be a less than optimal h/d ratio... :)

So! Where was I when it went from 4-4.5:1 to 3:1???  ~~<:-(

Or.... do I miss-remember?   Again?    I forget...  besides..N.T. did

it!  WITHOUT Tesla-2!!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Thu, 19 Mar 1998 13:21:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Thu, 19 Mar 1998 08:21:44 -0500

Subject: Re: New to Tesla

>Not until recently did I realize how popular Tesla Coils were.  While

>browsing on the internet for any information I could use, I was taken

>aback when I saw how many results I got for the search.  I was looking for

>plans for this gun-type Tesla coil, and some documentation as to why it

>works. What I would like to ask of you, is if you send me all you could

>about the science behind the Tesla, and plans to make a coil.

This doesn't sound like a tesla coil. I doubt anyone would be holding

one in their hand. I think what you saw was an "Ion Gun". This sends

out a stream of charged particles which can light flourescent lights

and even cause shocks.

I'll scan the plans and send them to you if you want.

-tom

tom.stock@gsbsc.gensig.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: coupling adjusting mechanism

Date: Thu, 19 Mar 1998 15:18:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 19 Mar 1998 07:18:27 -0800

Subject: Re: coupling adjusting mechanism

Organization: Self

Tesla-2 wrote:

>

> From: Bob Schumann <tesla@america.com>

> Date: Thu, 19 Mar 1998 06:40:23

> Subject: Re: coupling adjusting mechanism

>

> At 07:08 AM 3/19/98 GMT, you wrote:

> >From: Walt Kulpa <gwkulpa@earthlink.net>

> >Date: Wed, 18 Mar 1998 23:08:50 -0800

> >Subject: coupling adjusting mechanism

> >

Bob, you sent:

> Then I have a 'telescopic' setup for my secondary and can adjust it's

> height to the coupling I wish to achieve.  Hope this helps,

>

> Bob Schumann

What does this do for our admontiton: 'absitively NO holes in 2ndary

form!'?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Thu, 19 Mar 1998 15:44:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Thu, 19 Mar 1998 07:44:04 -0800 (PST)

Subject: Re: BIG Coil == big disaster?

On 19 Mar, Tesla-2 wrote:

> My present coil secondary is 4.25 by 26 inches,

> basically identical to Mike Hammer's design.  And I

> could NOT have done it without help from tesla-2 -

> especially Mr. Hammer's help.  Thanks Mike!!

>

> Are there any coiler who still frequent tesla-2 who are

> operating larger secondaries??

Yep, Me.

Although the coil has been packed away for the next few months

(we just bought a new home, and are preparing to move - the wife

does not approve of sparks in place of packing.. imagine that?)

It is 6.25x25 wound with 22AWG wire. Produces 3' arcs from 12KV/60ma

And the pig will be ordered as soon as we close escrow :)

Before packing away, I was having problems with quenching my gap.

I may pick up a shop vac (need one for the workshop anyway) and

try my hand at a vacuum quenched system once I get set up in the

new place.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Thu, 19 Mar 1998 15:46:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Pedro Bedard <pedro@pangea.ca>

Date: Thu, 19 Mar 98 09:46:54 -0600

Subject: Re: New to Tesla

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Thu, 19 Mar 1998 08:21:44 -0500

>Subject: Re: New to Tesla

>    >Not until recently did I realize how popular Tesla Coils were.  While

>>browsing on the internet for any information I could use, I was taken

>>aback when I saw how many results I got for the search.  I was looking for

>>plans for this gun-type Tesla coil, and some documentation as to why it

>>works.

>

>     This doesn't sound like a tesla coil. I doubt anyone would be holding

>     one in their hand. I think what you saw was an "Ion Gun". This sends

>     out a stream of charged particles which can light flourescent lights

>     and even cause shocks.

Actually it reminds me more of a HERF gun. It stands for High Energy

Radio Frequency. The U.S. military is apparently testing a prototype. It

is (if it works) a terrifying weapon. You simply point it at someone

within the weapon's range, pull the trigger and they die. Simple. Not

sure how much info on it is fact and how much is fantasy, time will tell.

Pedro.

>     I'll scan the plans and send them to you if you want.

I would very much like plans as well, if it's not too much trouble. :-)

P.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coupling adjusting mechanism

Cc:

Bcc:

X-Attachments:

At 03:18 PM 3/19/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Thu, 19 Mar 1998 07:18:27 -0800

>Subject: Re: coupling adjusting mechanism

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: Bob Schumann <tesla@america.com>

>> Date: Thu, 19 Mar 1998 06:40:23

>> Subject: Re: coupling adjusting mechanism

>>

>> At 07:08 AM 3/19/98 GMT, you wrote:

>> >From: Walt Kulpa <gwkulpa@earthlink.net>

>> >Date: Wed, 18 Mar 1998 23:08:50 -0800

>> >Subject: coupling adjusting mechanism

>> >

>

>Bob, you sent:

>> Then I have a 'telescopic' setup for my secondary and can adjust it's

>> height to the coupling I wish to achieve.  Hope this helps,

>>

>> Bob Schumann

>

>What does this do for our admontiton: 'absitively NO holes in 2ndary

>form!'?

>

>Gene Waldo

>

This doesnt produce any holes. What Bob is describing

is having one piece of pipe telescope inside another

with a friction fit. I think this is a simple and elegant

solution.

You can also mark off a scale on the inner pipe so you

can adjust to any desired coupling factor.

Since Bob first gave us this idea I have always planned

to rework my coil with this arrangement. Just never got

around to it.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Gaps Fixed or Adjustable?

Date: Thu, 19 Mar 1998 17:36:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Thu, 19 Mar 1998 12:36:24 -0500

Subject: Gaps Fixed or Adjustable?

I keep seeing conflicting info on how spark gaps should be

constructed. Some people say to use L brackets and bolts so that the

gap can be adjusted, but then other mention using copper pipe attached

to the inside of a PVC pipe with a fixed gap.

I thought the gap had to be tuned as well? Is it the quenching that

makes the spacing adjustment not necessary?

Thanks

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:H/D ratios

Cc:

Bcc:

X-Attachments:

At 09:54 AM 3/19/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Thu, 19 Mar 1998 01:54:37 -0800

>Subject: Re: BIG Coil == big disaster?

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: Dan Murphy <danmurphy@1stnetusa.com>

>> Date: Wed, 18 Mar 1998 22:42:51 -0800 (PST)

>> Subject: Re: BIG Coil == big disaster?

>>

>> >It was written:

>> >

>> >>Has anyone worked the numbers on a drive system for .... like a 24" x

>> >>12' secondary?

>> >

>> >Hmm.  Do you mean a secondary which is 24 feet long and 1 foot in diameter?

>> >This would be a less than optimal h/d ratio... :)

>

>So! Where was I when it went from 4-4.5:1 to 3:1???  ~~<:-(

>Or.... do I miss-remember?   Again?    I forget...  besides..N.T. did

>it!  WITHOUT Tesla-2!!

>Gene Waldo

Gene,

The optimal H/D ratio for secondary coils depends on the diameter.

For small coils higher ratios are needed to get adequate inductance.

For larger diameter coils a smaller h/d ratio provides better performane.

Some typical H/D ratios for standard PVC pipe might be:

PVC pipe   Actual OD   H/D ratio   winding length

2 inch     2.375       8 to 1      19 inches

3 inch     3.5         6 to 1      21 inches

4 inch     4.5         5 to 1      22.5 inches

6 inch     6.5         4 to 1      26 inches

10 inch    10.5        3 to 1      30 inches

In each case choose the largest wire guage that will

allow for 700-1000 turns.

These values are not cast in stone and considerable leeway

is allowable without affecting performance.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: big caps

Cc:

Bcc:

X-Attachments:

At 04:22 AM 3/19/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Wed, 18 Mar 1998 20:22:05 -0800

>Subject: big caps

>Organization: Self

>

>All,

>

>Need info: I've located 2 ea. 42.0mfd, 7.7kvdc jobs. $?; oper unkn @

>this time, but they will test before we drag them out!

Way too much capacitance to be useful for much of anything.

These are probably power supply filter caps and have a mylar

dielectric. Pretty useless for RF use.

>

>Along with several $50 ea. 2.0mfd, 12.5kvdc MICAMOLD.

>Are these apt to be very durable at 7.5kvac? What kind of Tank ckt

>would  it take to handle this capacitance?  Is anyone interested in

>them?

I would pass these up too. While mica caps do make good Tesla

coil caps the voltage rating of 12.5 kvDC is only good

for 6.25 KV AC and probably somewhat less in pulse duty.

Also the value of 2 mfd is way too much capcitance. It

would take a great number of them in series to bring the

capacitance down to useful levels. Thats not too practical

at $50 each.

>

>They have rolls and rolls of everything! From fine enam.magnet wire thru

>Belden signal cable to big power cables. They don' ship, but it's there!

Magnet wire is a staple item among coilers and we all are always

looking for a bargain here. I would be interested in some AWG 21 or 22

myself.

>Could we use inductive (hole thru center) lab type  5:1 current xfmrs

>any where in our curcuit? bunches of them...

The only real use for these is to measure current. These are

pretty common at hamfests really cheap so there isnt much value

in them.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFGnd ?

Cc:

Bcc:

X-Attachments:

At 10:16 PM 3/18/98 GMT, you wrote:

>From: Scott MacGregor <smacgregor@aasp.net>

>Date: Wed, 18 Mar 1998 14:16:29 -0800

>Subject: Re: RFGnd ?

>Organization: EES

>

>Tesla-2 wrote:

>>

>> From: Gene Waldo <gwaldo@1stnetusa.com>

>> Date: Wed, 18 Mar 1998 08:27:52 -0800

>> Subject: RFGnd ?

>> Organization: Self

>>

>> All,

>> while bone-yarding, I came across remnant of reel (~15') of heavy

>> braided cable: tinned Cu.  Pulled taut, it is about .37" OD, and could

>> be pulled into an = length of .5"OD  Tygon tubing.... Now I know that

>> braid is NG for RF, but 'HOW' NG???  Even the tinning wouldn't enhance

>> RF conductivity enough to 'do' in the interim?  Mainly I like the clean

>> flexibility this would present, over solid heavy wire...  Haven't worked

>> out the minimum length we should need to access the GND installation,

>> yet (re: my note on horizontal burial of perforated .5" OD Cu tube

>> w/NaCl internal bath?).

>>

>> Gene Waldo

>

>

>Gene,

>

>I've noticed a number of remarks from list members regarding the RF

>properties of braided conductor, specifically, that it is unacceptable

>for grounding purposes at RF. I can only tell you that untinned copper

>braid is commonly REQUIRED for interconnect bonding and grounding of

>equipment cabinets in radio stations, cable TV headends, and other RF

>environments. My understanding is that the skin effect is enhanced by

>the increased surface area of this type of conductor.

>

>If there is something I'm missing here, I hope someone on this list will

>correct me.

>--

>

>Regards,

>Scott MacGregor, EES

Gene and Scott,

Its precisely because of skin effect that braided conductors

are not optimum for RF. Due to skin effect RF currents want to flow

along the outside of conductors. If that conductor is braided the current

has to follow the ins and outs of the braided wires. Tinning the

braid does not help as this forces the current flow through the solder

which has a much higher resistance than copper.

Wide flat conductors like strap offer the lowest impedance.

Malcolm Watts once ran a series of test using various wires

and cables for Tesla grounding and found a definite drop in

system Q when braided cables were used.

In my work installing, maintaining, and repairing two-way

commercial radio equipment we do not use braid for RF or lightning

grounding. We normally solid AWG 6 copper for bonding equipment

cabinets and stranded AWG 4 for lightning grounds.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New to Tesla

Cc:

Bcc:

X-Attachments:

At 02:53 AM 3/13/98 GMT, you wrote:

>From: "Michael Koerner" <mkoerner@fhjc.org>

>Date: Thu, 12 Mar 1998 21:53:24 -0500

>Subject: New to Tesla

>

>    Not until recently did I realize how popular Tesla Coils were.  While

>browsing on the internet for any information I could use, I was taken aback

>when I saw how many results I got for the search.  I was looking for plans

>for this gun-type Tesla coil, and some documentation as to why it works.

>What I would like to ask of you, is if you send me all you could about the

>science behind the Tesla, and plans to make a coil.

>                        Thank you very much for your time.

>

>Michael J. Koerner

Michael,

The first thing you should do is download and read through the Tesla-2 list

archives. Many of your questions have already been asked and answered

there. Check out the archives and other files of interest on Dave

Bronakowski's archive site. The URL is:

http://www.velocity.net/~djb/files.shtml

Check out also the funet.fi ftp site. You will find many of the

same files there plus quite a large number of pictures of various

coils. The URL is: ftp://nic.funet.fi/pub/sci/electrical/tesla/

Armed with this background info check out some of the web pages

by other coilers. On these pages you will see pictures of various

coils and pictures of them in operation.

One thing you will notice is that no two coils are alike.

Each coil is unique. Thats why you wont find any "plans"

for Tesla coils. A coil is built around what components

you have or can acquire.

As for the science behind Tesla coils we are still learning

100 years later. Best way to learn the theory is by building

and operating a coil.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: direction of primary coil

Date: Thu, 19 Mar 1998 18:17:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fxphoto@centuryinter.net (fxphoto)

Date: Thu, 19 Mar 1998 13:17:20 -0500 (EST)

Subject: direction of primary coil

Does it make any difference which way the primary coil is wound?

That is .. looking from the toroid to the base of the secondary and starting

at the base, if the wire is coiled in a clockwise direction, should the

primary be coiled in a clockwise direction also? or should it be coiled in a

counterclockwise direction?

Bill Turbett

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Thu, 19 Mar 1998 18:38:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 19 Mar 1998 10:38:03 -0800

Subject: Re: New to Tesla

>

>

>      This doesn't sound like a tesla coil. I doubt anyone would be holding

>      one in their hand. I think what you saw was an "Ion Gun". This sends

>      out a stream of charged particles which can light flourescent lights

>      and even cause shocks.

>

>      I'll scan the plans and send them to you if you want.

I would enjoy the plans also.

Mike

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Thu, 19 Mar 1998 18:55:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Wikkerink <bert@mitra.com>

Date: Thu, 19 Mar 1998 13:55:57 -0500

Subject: Re: New to Tesla

Organization: Mitra Imaging Inc.

Uh, if you don't mind, those plans sound pretty interesting as well. Maybe you

can put them on someones web page and just post the URL.

Thanks,

Bert W.

Tesla-2 wrote:

> >From: Pedro Bedard <pedro@pangea.ca>

> Date: Thu, 19 Mar 98 09:46:54 -0600

> Subject: Re: New to Tesla

>

> >From: <Tom.Stock@gsbsc.gensig.com>

> >Date: Thu, 19 Mar 1998 08:21:44 -0500

> >Subject: Re: New to Tesla

>

> >    >Not until recently did I realize how popular Tesla Coils were.  While

> >>browsing on the internet for any information I could use, I was taken

> >>aback when I saw how many results I got for the search.  I was looking for

> >>plans for this gun-type Tesla coil, and some documentation as to why it

> >>works.

> >

> >     This doesn't sound like a tesla coil. I doubt anyone would be holding

> >     one in their hand. I think what you saw was an "Ion Gun". This sends

> >     out a stream of charged particles which can light flourescent lights

> >     and even cause shocks.

> Actually it reminds me more of a HERF gun. It stands for High Energy

> Radio Frequency. The U.S. military is apparently testing a prototype. It

> is (if it works) a terrifying weapon. You simply point it at someone

> within the weapon's range, pull the trigger and they die. Simple. Not

> sure how much info on it is fact and how much is fantasy, time will tell.

> Pedro.

> >     I'll scan the plans and send them to you if you want.

>

> I would very much like plans as well, if it's not too much trouble. :-)

> P.

> >     -tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Gaps Fixed or Adjustable?

Cc:

Bcc:

X-Attachments:

At 05:36 PM 3/19/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Thu, 19 Mar 1998 12:36:24 -0500

>Subject: Gaps Fixed or Adjustable?

>

>

>

>     I keep seeing conflicting info on how spark gaps should be

>     constructed. Some people say to use L brackets and bolts so that the

>     gap can be adjusted, but then other mention using copper pipe attached

>     to the inside of a PVC pipe with a fixed gap.

Forget L brackets and bolts. This makes for a poor performing

gap due to the too small electrode surface and and poor heat sinking.

The cylinder gap with copper pipe electrodes provides large

surface area with the ability to sink lots of heat.

>

>     I thought the gap had to be tuned as well?

The gap is almost like a power level adjustment. The wider

the gap the higher the voltage the main cap will charge to.

Narrow gaps break down at low voltage so limit the power input.

So it stands to reason that for maximum performance a wide

gap is called for. Within reason this is correct. But every increase

in gap spacing increases the strain on your caps and transformers.

Overwide gaps probably account for most of the neon transformer

failures. So you need enough spacing to make good use of the

available power but not so much as to overstress components.

Ive listed recommended maximum gap distances several times but will

repeat it here. For 9kv a max of .200 inch. For 12kv .250 inch

and for 15kv .300 inch. These are maximum values. Ive never run

more than .225 inch for 12kv.

Breaking this distance up into several smaller series gaps improves

efficiency by dividing up the heat loss. 6 series gaps for 9kv, 8

for 12kv and 10 for 15kv are recommended.

>Is it the quenching that makes the spacing adjustment not necessary?

The spacing on a cylinder gap IS adjustable. Its adjustable in increments.

You just use the total number of the gaps you need. My vacuum gap has

9 series gaps. I frequently use only 6 of them when running with

9kv neons and all 9 when going for maximum output at 12kv.

Quenching is the most important aspect of a good gap system.

Any two pieces of wire will function as a crude gap and will

break down when the voltage is high enough. But you need to have

that gap extinguish itself rapidly once the primary power is

transferred to the secondary. Any delay will allow some of the secondary

energy to transfer back to the primary where it can cause kickbacks

and lower efficiency.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Thu, 19 Mar 1998 21:10:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Rick L. Kirby" <rkirby@earthlink.net>

Date: Thu, 19 Mar 1998 13:10:24 -0800

Subject: Re: BIG Coil == big disaster?

Tesla-2 wrote:

>

> From: arcstarter@mailexcite.com

> Date: Wed, 18 Mar 1998 21:36:27 -0600

> Subject: Re: BIG Coil == big disaster?

> Organization: MailExcite  (http://www.mailexcite.com)

>

> My present coil secondary is 4.25 by 26 inches,

> basically identical to Mike Hammer's design.  And I

> could NOT have done it without help from tesla-2 -

> especially Mr. Hammer's help.  Thanks Mike!!

>

> Are there any coiler who still frequent tesla-2 who are

> operating larger secondaries??

>

> -Bill the arcstarter

Bill,

Yes I am running my 6" coil now. It is powered by 4 nst's

15kv/30ma each producing 50-60" sparks.

Rick Kirby

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: direction of primary coil

Cc:

Bcc:

X-Attachments:

At 06:17 PM 3/19/98 GMT, you wrote:

>From: fxphoto@centuryinter.net (fxphoto)

>Date: Thu, 19 Mar 1998 13:17:20 -0500 (EST)

>Subject: direction of primary coil

>

>Does it make any difference which way the primary coil is wound?

>That is .. looking from the toroid to the base of the secondary and starting

>at the base, if the wire is coiled in a clockwise direction, should the

>primary be coiled in a clockwise direction also? or should it be coiled in a

>counterclockwise direction?

>                Bill Turbett

>

Does not matter at all. In fact on my coil I just happen

to have the primary and secondary wound in opposite directions.

I didnt intend it that way. Thats just the way they turned out.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: ion gun/herf gun

Date: Fri, 20 Mar 1998 00:04:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 19 Mar 1998 19:04:28 -0500

Subject: ion gun/herf gun

Hi all

I would like to see any plans for  ion / herf  guns.

doubt that I would use them but its nice to know how.

please contact me at    cwatkins@seidata.com

or anyone know of a web page with such info?

thanks

Chris;

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Fri, 20 Mar 1998 00:12:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kent Rankin" <kentrankin@theatreorgans.com>

Date: Thu, 19 Mar 1998 19:12:42 -0500

Subject: Re: New to Tesla

I'd like them too, please.

Thanks,

Kent Rankin

-----Original Message-----

From: Tesla-2 <tesla-2@emachine.com>

To: kentrankin@theatreorgans.com <kentrankin@theatreorgans.com>

Date: Thursday, March 19, 1998 9:09 AM

Subject: Re: New to Tesla

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Thu, 19 Mar 1998 08:21:44 -0500

>Subject: Re: New to Tesla

>

>

>    >Not until recently did I realize how popular Tesla Coils were.  While

>>browsing on the internet for any information I could use, I was taken

>>aback when I saw how many results I got for the search.  I was looking for

>>plans for this gun-type Tesla coil, and some documentation as to why it

>>works. What I would like to ask of you, is if you send me all you could

>>about the science behind the Tesla, and plans to make a coil.

>

>

>     This doesn't sound like a tesla coil. I doubt anyone would be holding

>     one in their hand. I think what you saw was an "Ion Gun". This sends

>     out a stream of charged particles which can light flourescent lights

>     and even cause shocks.

>

>     I'll scan the plans and send them to you if you want.

>

>     -tom

>     tom.stock@gsbsc.gensig.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: ground connections

Date: Fri, 20 Mar 1998 00:34:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 19 Mar 1998 19:34:07 -0500

Subject: ground connections

Hi Coilers

moving my coil outside......

while driving 10ft ground rods in today (by hand) I had a crazy thought.

actually it was hard copper water pipe & worked fairly well.

what would happen if a capacitance was added in the ground line of the

secondary?

has anyone ever tried this?

would this have a similar effect as a larger toroid - lower frequency?

If so a variable cap here should give the ability to change freq.?

re tuning would of course be reqd.

I can think of several advantages if this did happen to work.

If anyone has tried this please inform me of the result.

I will not have my coil ready to run for a few days.

just some hairbrain ideas

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla

Date: Fri, 20 Mar 1998 01:37:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 19 Mar 1998 20:37:01 EST

Subject: Re: New to Tesla

Tom,

I would like to check out your scan of the ion gun plans.  You can post it

or send it to me directly if you prefer(hollmike@aol.com).  It sounds like a

fun and politically incorrect device, so I would like to make one.

Thanks,

Mike

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New to Tesla

Cc:

Bcc:

X-Attachments:

At 01:37 AM 3/20/98 GMT, you wrote:

>From: Hollmike <Hollmike@aol.com>

>Date: Thu, 19 Mar 1998 20:37:01 EST

>Subject: Re: New to Tesla

>

>Tom,

>  I would like to check out your scan of the ion gun plans.  You can post it

>or send it to me directly if you prefer(hollmike@aol.com).  It sounds like a

>fun and politically incorrect device, so I would like to make one.

>Thanks,

>Mike

>

>

Discussion of this device should be taken off list.

It is an off topic discussion.

Consider this thread terminated.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla*REMOVE*

Date: Fri, 20 Mar 1998 03:57:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Thu, 19 Mar 1998 21:57:04 -0600

Subject: Re: New to Tesla*REMOVE*

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 19 Mar 1998 20:34:00 -0600

> Subject: Re: New to Tesla

>

> At 01:37 AM 3/20/98 GMT, you wrote:

> >From: Hollmike <Hollmike@aol.com>

> >Date: Thu, 19 Mar 1998 20:37:01 EST

> >Subject: Re: New to Tesla

> >

> >Tom,

> >  I would like to check out your scan of the ion gun plans.  You can post it

> >or send it to me directly if you prefer(hollmike@aol.com).  It sounds like a

> >fun and politically incorrect device, so I would like to make one.

> >Thanks,

> >Mike

> >

> >

>

> Discussion of this device should be taken off list.

> It is an off topic discussion.

> Consider this thread terminated.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

I agree Mike! Not only should it be an off list message, but I

personally don't see what in the world it has to do with a TESLA COIL!!

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: BIG Coil == big disaster?

Date: Fri, 20 Mar 1998 04:14:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 19 Mar 1998 20:14:16 -0800

Subject: Re: BIG Coil == big disaster?

Organization: Stoneridge Engineering

Tesla-2 wrote:

On 19 Mar, Tesla-2 wrote:

> My present coil secondary is 4.25 by 26 inches,

> basically identical to Mike Hammer's design.  And I

> could NOT have done it without help from tesla-2 -

> especially Mr. Hammer's help.  Thanks Mike!!

>

> Are there any coiler who still frequent tesla-2 who are

> operating larger secondaries??

I've got a 10" x 31" coil that outputs 63" streamers - it can be seen

at:

http://www.nic.funet.fi/pub/sci/electrical/tesla/pictures/hickman

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Fri, 20 Mar 1998 04:53:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 19 Mar 1998 23:53:58 EST

Subject: Re: Plate caps & connections

This is for Mike Nicholas(and anyone else interested):

I am sorry that I did not get back to you sooner on this, but here is the

information on the mica sheets.  First the number for McMaster-Carr is

(630)-834-9600.  The mica is called flexible muscovite(they have another

variety, but I don't think it is quite as good according to some literature I

have read).   The part number in the catalog is 85825K61.    The sheets are

39.37"X23.62".  The next larger sheet had the part number 85825K71 which is

47.24"X39.37".  The smaller sheet cost $9.19 each.  The larger sheet cost

about twice that much.  These sheets are .004" thick and they claim a

dielectric strength of 20kV/mil.  I don't believe that and assume 20kV/ sheet

as the max  This would be more consistant with literature values I have seen,

but who knows?  I haven't tested my cap as of yet, but intend to this week.

I'll let you know how it works out.

Mike Hollingsworth

.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor dielectrics

Date: Fri, 20 Mar 1998 11:09:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: D Mills <ap7012@qmw.ac.uk>

Date: Fri, 20 Mar 1998 11:09:00 +0000

Subject: capacitor dielectrics

Organization: University of London (QMW)

Hello all

I have found a plastics firm, that will sell me rolls of plastic

sheeting very cheaply.  I want to know what is the best I can use to

make a good cap. (ie one that will handle 15KV).

I have got fedup with breathing ozone from my saltwater caps.

I intend to make a plate cap, as I have no access to a vac pump.

Are there any major problems with these type as compared with the

swiss role sort?

Thanks

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor dielectrics

Date: Fri, 20 Mar 1998 13:02:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 20 Mar 1998 07:02:40 -0600

Subject: Re: capacitor dielectrics

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: D Mills <ap7012@qmw.ac.uk>

> Date: Fri, 20 Mar 1998 11:09:00 +0000

> Subject: capacitor dielectrics

> Organization: University of London (QMW)

>

> Hello all

>

> I have found a plastics firm, that will sell me rolls of plastic

> sheeting very cheaply.  I want to know what is the best I can use to

> make a good cap. (ie one that will handle 15KV).

>

> I have got fedup with breathing ozone from my saltwater caps.

>

>  I intend to make a plate cap, as I have no access to a vac pump.

> Are there any major problems with these type as compared with the

> swiss role sort?

>

> Thanks

>

> Dave

>

Dave,

Clear (translucent?) polypropylene or polyethylene will work best. The

dielectric should be constructed of AT LEAST 2 layers of material

between foil plates. For reliable operation off a 15 kV source, you're

best course of action is to make a number of identical capacitors

connected in series to "share" the voltage stress. Total dielectric

thickness of the group should not be less than 80 mils. Make the

indicidual caps N times as large as the desired value (where N = number

of caps in series, typically 2-4). The biggest problem is determining

what to use for a container - solve THIS problem before you begin

cutting plate material. Many coilers make custom containers from glued

plexiglas, or use trashcans.

Safe cappin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor dielectrics

Date: Fri, 20 Mar 1998 15:54:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 07:54:11 -0800

Subject: Re: capacitor dielectrics

Organization: Self

Tesla-2 wrote:

>

> From: D Mills <ap7012@qmw.ac.uk>

> Date: Fri, 20 Mar 1998 11:09:00 +0000

> Subject: capacitor dielectrics

> Organization: University of London (QMW)

>

> Hello all

>

> I have found a plastics firm, that will sell me rolls of plastic

> sheeting very cheaply.  I want to know what is the best I can use to

> make a good cap. (ie one that will handle 15KV).

>

> I have got fedup with breathing ozone from my saltwater caps.

>

Dave,

I LOVE the smell of ozone in the morning! Lacking vacuum, some low

Hz-DCvoltage (on-off/disch-on)cycling operation, to cycle the

"clench-relax" effect, would allow gravity to pump air out of your

plates. Vacuum also presupposes a suitable, non-collapsing contianment

vessel. You might try to find a waterline type aspirator (w/water trap

in vac line) to expand air bubbles in your capacitor vessel. These

aspirators are inexpensive and usually available thru a lab supply shop.

Also heating will help, about 66 C(150 F).

Be sure your plastic is LDPE about 6 mil and of good physical quality.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitor dielectrics

Cc:

Bcc:

X-Attachments:

At 11:09 AM 3/20/98 GMT, you wrote:

>From: D Mills <ap7012@qmw.ac.uk>

>Date: Fri, 20 Mar 1998 11:09:00 +0000

>Subject: capacitor dielectrics

>Organization: University of London (QMW)

>

>Hello all

>

>I have found a plastics firm, that will sell me rolls of plastic

>sheeting very cheaply.  I want to know what is the best I can use to

>make a good cap. (ie one that will handle 15KV).

Dave,

You want low density polyethylene (LDPE) or polypropylene (PP)

film for your caps.

For a reliable 15kv cap you want to use three series connected

caps each with a minimum of 20 mils of poly. In series this

gives the equivalent of 60 mils.

If I was building a cap for 15kv use I would build 3 caps

each with 24 mils and run them in series for a total of 72 mils.

Should make a solid robust cap.

Get a thinner plastic film from your supplier.

A dielectric layer made of of several thin sheets

has a much better rating than a single thicker sheet.

Get either 6 or 8 mil poly. Three 8 mil sheets or four 6 mil

sheets will give you the required 24 mils.

Remember if you are going to series 3 caps each individual

cap needs to have 3 times the desired capacitance.

If .01 mfd is your goal to use with a 15kv/60ma neon then

each cap must be .03 mfd. In series those 3 caps would then

yield .01 mfd.

>

>I have got fed up with breathing ozone from my saltwater caps.

You will still have plenty of ozone from the spark gap as well

as from your secondary output. Just provide some ventilation.

>

> I intend to make a plate cap, as I have no access to a vac pump.

>Are there any major problems with these type as compared with the

>swiss role sort?

No major problems. They tend to require more oil and since

they are not vacuum pumped require a slow break in period

at low power. Keep your gap quite narrow and let the pulsing

action of the Tesla tank circuit vibrate air bubbles loose.

Increase your power level slowly. Usually after every increase

in power level a new batch of air bubbles will be seen rising

from the caps. Wait until this bubbling stops before increasing

the power level any further.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Variac

Date: Fri, 20 Mar 1998 18:23:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 10:23:01 -0800

Subject: Variac

Organization: Self

To: CHRIS,

Can't locate your E-addr...

Were 3 of them, yesterday, 1 left.  GenRad mod W-10H  (3 ganged), 240vac

in, 4A, $100.

Could do 8A on 110v in?

Gene waldo   gwaldo@1stnetusa.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Bottle Cap outer electrode??

Date: Fri, 20 Mar 1998 20:00:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 12:00:50 -0800

Subject: Bottle Cap outer electrode??

Organization: Self

All:

While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

oz. Bud, and the problems of wrapping Al foil on all of them, I asked

myself, "What about 2 or 3 coats of aluminum spray paint?"

Does anyone have data on the conductivity of such a film? I can envision

the 'paint' matrix 'insulating' all the Al particles, but then again....

maybe not?

I can easilly rig a turntable and spray them, turning slowly in the

breeze, necks down, in a mass production process.

I don't presently have an operating can of paint (it seems), or I would

run the test, my self.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Variac

Date: Fri, 20 Mar 1998 20:31:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Fri, 20 Mar 1998 14:31:11 -0600

Subject: RE: Variac

Hello all,

Just got the new All Electronics catalog and they have two nice variacs

listed on page 61.  There is a 5 Amp for $95.00 (CAT# SC-5M) and a 10

Amp for $135.00 (CAT# SC-10M).  Both are for 110VAC input, fully

enclosed, have an output meter, grounded receptacle, on/off switch, fuse

and 6 foot power cord.

Their order # is 1-800-826-5432

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Bottle Cap outer electrode??

Cc:

Bcc:

X-Attachments:

At 08:00 PM 3/20/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 20 Mar 1998 12:00:50 -0800

>Subject: Bottle Cap outer electrode??

>Organization: Self

>

>All:

>

>While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

>oz. Bud, and the problems of wrapping Al foil on all of them, I asked

>myself, "What about 2 or 3 coats of aluminum spray paint?"

>

>Does anyone have data on the conductivity of such a film? I can envision

>the 'paint' matrix 'insulating' all the Al particles, but then again....

>maybe not?

>

>I can easilly rig a turntable and spray them, turning slowly in the

>breeze, necks down, in a mass production process.

>

>I don't presently have an operating can of paint (it seems), or I would

>run the test, my self.

>

>Gene Waldo

Gene,

I just dont think its worth the expense or effort on a SW

bottle cap. And I dont think it will be possible to get an effective

conductive coating. When I made SW caps I disposed of the outer foil

wrap entirely by putting the bottles in a larger container

of saltwater. Thisroduced better results than trying to wrap

the bottles with foil.

What I did was lay a strip of foil in the bottom of the container

and up one side. The bottle were then set on the foil

and then the container was filled with saltwater to a level

equal with the SW level inside the bottles. A thin layer

of oil over the surface of the SW retarded evaporation.

Consider SW caps to be a very temporary solution to be used

only until you build some decent poly caps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Bottle Cap outer electrode??

Date: Fri, 20 Mar 1998 23:05:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Fri, 20 Mar 1998 17:05:45 -0600

Subject: RE: Bottle Cap outer electrode??

I've experimented with aluminum and copper colored paint and at normal voltages

it doesn't conduct at all.

Why not use the aluminum foil tape that HVAC guys use to seal up duct work.  Th

at stuff is easy to work with and conducts very well.

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Friday, March 20, 1998 3:22 PM

To:     Minnich, Roy

Subject:        Bottle Cap outer electrode??

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 12:00:50 -0800

Subject: Bottle Cap outer electrode??

Organization: Self

All:

While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

oz. Bud, and the problems of wrapping Al foil on all of them, I asked

myself, "What about 2 or 3 coats of aluminum spray paint?"

Does anyone have data on the conductivity of such a film? I can envision

the 'paint' matrix 'insulating' all the Al particles, but then again....

maybe not?

I can easilly rig a turntable and spray them, turning slowly in the

breeze, necks down, in a mass production process.

I don't presently have an operating can of paint (it seems), or I would

run the test, my self.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: LOD Group, too

Date: Fri, 20 Mar 1998 23:08:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 15:08:25 -0800

Subject: LOD Group, too

Organization: Self

All newbies:

Get thee hence to <htp://www.lod.org/electrum> post haste! See what the

potentials are! Pun intended!

Personal to any Los Angeles based coilers:  Can we think about a

sit-down over coffee, et al? Some neutral location, to compare notes and

chat, spit 'n argue? Month or so from now?  If interested, send me your

thoughts on such, and your ZIP.  I'll try to coordinate the thing.

Also I can turn any of you on to our local treasure house of electrical

& mechanical PARTS & EQUIPMENT! Figure 2 hours min to troop the place

and just see whats there!  It's ALL there.

Gene Waldo    gwaldo@1stnetusa.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 00:56:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Zuma /or CJ <mwise@ns.sosis.com>

Date: Fri, 20 Mar 1998 18:56:56 -0600

Subject: Re: Bottle Cap outer electrode??

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Fri, 20 Mar 1998 12:00:50 -0800

> Subject: Bottle Cap outer electrode??

> Organization: Self

>

> All:

>

> While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

> oz. Bud, and the problems of wrapping Al foil on all of them, I asked

> myself, "What about 2 or 3 coats of aluminum spray paint?"

>

> Does anyone have data on the conductivity of such a film? I can envision

> the 'paint' matrix 'insulating' all the Al particles, but then again....

> maybe not?

>

> I can easilly rig a turntable and spray them, turning slowly in the

> breeze, necks down, in a mass production process.

>

> I don't presently have an operating can of paint (it seems), or I would

> run the test, my self.

>

> Gene Waldo

>

I have tried this before, thinking on the same line of thought.

It did not work. The two different types of aluminum paint I used were

not conductive at all. I think that the companies only use the pigment

and only a small amount of aluminum is actually in the paint itself.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 01:02:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 17:02:53 -0800

Subject: Re: Bottle Cap outer electrode??

Organization: Self

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 20 Mar 1998 16:38:13 -0600

> Subject: Re: Bottle Cap outer electrode??

>

> At 08:00 PM 3/20/98 GMT, you wrote:

> >From: Gene Waldo <gwaldo@1stnetusa.com>

> >All:

> >

> >While cogitating on wiring up our Cap assy: 12 or 16 bottle units of

> >40oz. Bud, and the problems of wrapping Al foil on all of them, I asked

> >myself, "What about 2 or 3 coats of aluminum spray paint?"

> >Gene Waldo

>

> Gene,

> I just dont think its worth the expense or effort on a SW

> bottle cap. When I made SW caps I disposed of the outer foil

> wrap entirely by putting the bottles in a larger container

> of saltwater.

>

> Consider SW caps to be a very temporary solution...

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike,

EXACTAMUNDO!  We ARE cutting sheet metal and arguing over wheather or

not it would be worth the expense to use poly BAGS, instead of messing

w/the BS of hacking our way thruogh a roll of the stuff. I can get lg 6

mil bags to fit.

My next query is about TUNABLE SW cap?  VARY the level of SW! Simple

'siphon' tube arrangement from bottom of Cap tub to carboy on pulley...

??

I'll have the beer consumed (40 oz. BUD) long before we complete the

plate caps. 18 bottles w/~90^2" ea. of foil/immersion should give enough

capacitance to do something.

I've not heard of tuning the cap w/this method, and have no seat of the

pants feel for its effectiveness. Comments?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New to Tesla*REMOVE*

Cc:

Bcc:

X-Attachments:

At 06:29 AM 3/13/98 GMT, you wrote:

>From: "Michael Koerner" <mkoerner@fhjc.org>

>Date: Fri, 13 Mar 1998 01:29:20 -0500

>Subject: Re: New to Tesla*REMOVE*

>

>    If you follow the responses back to the original, I was asking a

>harmless question.  We just got slightly side tracked.  It's done with.

Yes. Thats why I let the thread go as long as I did

until it got totally beyond the scope of the list.

I very rarely excercise my authority as moderator.

>Though, I still want to know what type of Tesla coil illuminates fluorescent

>bulbs.  At http://www.amazing1.com/tesla.htm I found plans for such a Tesla

>coil, but I don't believe it since you are all saying that Tesla coils can't

>do that.

Any Tesla coil will illuminate flourescent lights. Its the RF energy

working on the rarified gases within the bulb. I can even light

flourescent lights with my amateur radio transmitter.

The plans and materials at the URL you listed are not

nearly as "amazing" as the advertising would have you believe.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 01:24:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 20 Mar 1998 17:24:34 -0800

Subject: Re: Bottle Cap outer electrode??

Organization: Self

Tesla-2 wrote:

>

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Date: Fri, 20 Mar 1998 17:05:45 -0600

> Subject: RE: Bottle Cap outer electrode??

>

> I've experimented with aluminum and copper colored paint and at normal

> voltages it doesn't conduct at all.

>

Roy,

As I suspected!  My current frame of mind is that while Plate caps are

under const, and since we have committed to the stop-gap (start-gap?)

use of SW caps in the interim, why not go the extra step and find out

about SW immersion; with the added feature of TUNABILITY, using a means

of varying the bath level in the bottle tubs? No one has opined on this

aspect, yet....

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New to Tesla*REMOVE*

Date: Sat, 21 Mar 1998 02:11:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Moleking <moleking@concentric.net>

Date: Fri, 20 Mar 1998 21:11:17 -0500

Subject: Re: New to Tesla*REMOVE*

>    If you follow the responses back to the original, I was asking a

>harmless question.  We just got slightly side tracked.  It's done with.

>Though, I still want to know what type of Tesla coil illuminates fluorescent

>bulbs.  At http://www.amazing1.com/tesla.htm I found plans for such a Tesla

>coil, but I don't believe it since you are all saying that Tesla coils can't

>do that.

>

>Michael J. Koerner

>jMk Computer Services

>mkoerner@fhjc.org

Mike

Tesla Coils are capable of lighting fluorescent bulbs, but not in the

manner which you described in your first email.  A coil capable of lighting

a scoreboard (probably not fluorescent anyway) from across a gym would have

to be pretty big.  WAY too big to hold in your hand, for sure.  The plans

that you found are for a relatively small coil, which I assume would put

out 12-20" sparks, and could probably light a flourescent tube from maybe 6

feet away.  Some of the HUGE coils which maybe you've seen pictures of

could possibly do it from across the gym.  If i recall correctly, Nikola

Tesla's Colorado Springs coil lit up a bulb 26 miles away!  Imagine that

for a science project..hehe.  But seriously, ANY coil would make a killer

Science fair project.  Forget lighting up a scoreboard -  Show people 2

foot arcs!

Good luck

Josh Bedell

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Sat, 21 Mar 1998 02:55:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Fri, 20 Mar 1998 18:55:39 -0800

Subject: Re: RFGnd ?

Organization: EES,Co.

Tesla-2 wrote:

> Gene and Scott,

>

> Its precisely because of skin effect that braided conductors

> are not optimum for RF. Due to skin effect RF currents want to flow

> along the outside of conductors. If that conductor is braided the current

> has to follow the ins and outs of the braided wires. Tinning the

> braid does not help as this forces the current flow through the solder

> which has a much higher resistance than copper.

>

> Wide flat conductors like strap offer the lowest impedance.

>

> Malcolm Watts once ran a series of test using various wires

> and cables for Tesla grounding and found a definite drop in

> system Q when braided cables were used.

>

> In my work installing, maintaining, and repairing two-way

> commercial radio equipment we do not use braid for RF or lightning

> grounding. We normally solid AWG 6 copper for bonding equipment

> cabinets and stranded AWG 4 for lighning grounds.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

You will agree, I'm sure, that there is a distinction between equipment

grounds, (for 60 Hz safety) and RF grounds. If I am right that most

Tesla coils operate "around" 500 kHz, then the length or type of copper

material used for an effective RF ground would depend primarily on its

DC resistance. This would appear to be so, since, the impedance of the

ground lead would insignificant in comparison to its DC resistance at

500 kHz. In fact, to become a significant factor, say a 1/4 wave match,

the ground lead would have to equal 150 meters, or 1968 feet.

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 03:19:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Fri, 20 Mar 1998 19:19:11 -0800

Subject: Re: Bottle Cap outer electrode??

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Fri, 20 Mar 1998 12:00:50 -0800

> Subject: Bottle Cap outer electrode??

> Organization: Self

>

> All:

>

> While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

> oz. Bud, and the problems of wrapping Al foil on all of them, I asked

> myself, "What about 2 or 3 coats of aluminum spray paint?"

>

> Does anyone have data on the conductivity of such a film? I can envision

> the 'paint' matrix 'insulating' all the Al particles, but then again....

> maybe not?

>

> I can easilly rig a turntable and spray them, turning slowly in the

> breeze, necks down, in a mass production process.

>

> I don't presently have an operating can of paint (it seems), or I would

> run the test, my self.

>

> Gene Waldo

Gene,

Too bad they don't list ALL the ingrediants on the beer, or spray cans,

but you could "proof" the results of you tests!

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 11:10:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sat, 21 Mar 1998 06:10:30 EST

Subject: Re: Bottle Cap outer electrode??

Roy,

Just a thought on your idea of tunable SW caps:  If you are using a neon,

then both ends of your cap are hot and you will need to come up with a way to

raise and lower the level without coming in contact with the salt water.

Otherwise, you could get a nasty shock as the tubing used for siphoning is now

a conducter.   I cannot think of any reason this idea shouldn't work.  Just be

careful.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Variable Salt Water Caps

Date: Sat, 21 Mar 1998 15:17:55 GMT

Organization: The Energy Machine Information System 504-733-3462

misslink.net

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 21 Mar 1998 10:17:55 -0500

Subject: Variable Salt Water Caps

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Fri, 20 Mar 1998 17:02:53 -0800

> Subject: Re: Bottle Cap outer electrode??

> Organization: Self

>

Snippidity Do Dah!

>

> My next query is about TUNABLE SW cap?  VARY the level of SW! Simple

> 'siphon' tube arrangement from bottom of Cap tub to carboy on pulley...

> ??

>

> I'll have the beer consumed (40 oz. BUD) long before we complete the

> plate caps. 18 bottles w/~90^2" ea. of foil/immersion should give enough

> capacitance to do something.

>

> I've not heard of tuning the cap w/this method, and have no seat of the

> pants feel for its effectiveness. Comments?

>

> Gene Waldo

>

Gene,

Immersing the saltwater caps as Mike Hammer mentioned is the way to go.

Immersion cuts down on corona loss and resuces heating, and absolutely

guarantees that you will get the maximum capacitance possible out

of your SW caps. Be aware of the fact that aluminum foil on the outside

of a SW cap ALWAYS has wrinkles and therefore air gaps. That is where

the corona builds up and causes pinpoint heating that leads to glass

stress and eventual failure. Immersion gets past all that. The downside

is the need for an insulated container, but that is a small price to pay.

You can connect SW caps in parallel by just immersing them in the same

tub and connecting the tops together.

If you want them in series, then you need separate tubs, ***OR***

you can get exactly TWO sets in series per tub by connecting the tops of

one half of the bottles together and calling that electrode (A), and then

doing the same with the others and calling that electrode (B). The salt

water in the tub will interconnect the two sets for you (no electrode

needed here...), and you use (A) and (B) as your capacitor terminals.

Simple and effective.

You asked about raising and lowering the salt water level. While

siphoning will work, it has some problems. Once air gets in the line

the siphoning action stops. A better method is to attach a hose near

the base of the tub and the base of another container, such as a 5 gallon

polyethelene bucket. There are plastic fittings you can buy at a hardware

store that will give you a nipple to attach a hose between the two

containers. Raising or lowering the bucket will cause the salt water level

in the tub to go up or down as desired, adjusting the capacitance over a

wide range.  Be aware that the caps will undergo more strain when the

water level is lower.

The saltwater, of course, conducts electricity, so be aware of the fact

that the bucket will have several thousand volts "at your finger tips".

Be careful, and make sure that whatever you do you have safety in mind!

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla at Colorado Springs

Cc:

Bcc:

X-Attachments:

At 02:11 AM 3/21/98 GMT, you wrote:

>

>Mike

>

>Tesla Coils are capable of lighting fluorescent bulbs, but not in the

>manner which you described in your first email.  A coil capable of lighting

>a scoreboard (probably not fluorescent anyway) from across a gym would have

>to be pretty big.  WAY too big to hold in your hand, for sure.  The plans

>that you found are for a relatively small coil, which I assume would put

>out 12-20" sparks, and could probably light a flourescent tube from maybe 6

>feet away.  Some of the HUGE coils which maybe you've seen pictures of

>could possibly do it from across the gym.  If i recall correctly, Nikola

>Tesla's Colorado Springs coil lit up a bulb 26 miles away!  Imagine that

>for a science project..hehe.

>

This is not true. Tesla did not light a bank of light

bulbs 26 miles away at Colorado Springs. This is one of

the myths about Tesla that keep getting repeated over and over.

Tesla did light some bulbs outside the Colorado Springs

lab at a distance of 60 FEET by induction. He then went on

to hypothesize that a bank of 30 lamps could be lit at full power.

No where in his lab notes or in any of his depositions about

his work in Colorado Springs does he ever mention lighting

light bulbs at multiple mile ranges.

References to check out if you are interested in Tesla's

Colorado Springs work include:

THE COLORADO SPRINGS NOTES, 1899-1900

By Nikola Tesla

NIKOLA TESLA ON HIS WORK WITH ALTERNATING CURRENTS

AND THEIR APPLICATION TO WIRELESS TELEGRAPHY, TELEPHONY,

AND TRANSMISSION OF POWER

by Leland I. Anderson

THE TESLA COIL BUILDERS'S GUIDE TO THE COLORADO SPRINGS

NOTES OF NIKOLA TESLA

by Richard L. Hull

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 16:06:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 08:06:27 -0800

Subject: Re: Bottle Cap outer electrode??

Organization: Self

Tesla-2 wrote:

>

> From: Hollmike <Hollmike@aol.com>

> Date: Sat, 21 Mar 1998 06:10:30 EST

> Subject: Re: Bottle Cap outer electrode??

>

> Gene, (actually..)

>    Just a thought on your idea of tunable SW caps:  If you are using a

> both ends of your cap are hot ... Just be careful.

>   Mike Hollingsworth

Mike,

Yes! That condition had occured to me.  Lg. poly carboys and a nylon

line thru pulley should be safe.

My real questions tho, are:  a system, built to calculated parameters,

usually needs fine adjusting....   it seems to me that having a (HOT!)+-

5 or 10% range on the capacitance should enable the real 'tweaking'

req'd to 'tune' (match) resonant Hz to an RCH!  Shouldn't  this 'hot

tweak' capability be an advantage in compensating for slight out of tune

conditions in both coupling and primary length?

My expeience in 'tuning' systems, both randomly  and sinusoidally

excited, has taught me that tight, hi-Q systems 'peak' at quite specific

frequencies.

I can envision a 'calibrated fluid-level' readout on the brine tank, a

simply controlled flow rate in the siphon, and a 'stand back, and wait

for it to go bang (PEAK)' means of tuning... When your spark maxes out,

note the fluid depth, and you're home! Then get an empirical readout on

capacitance.  It should then be possible to 'cold' tweak either or all:

terminal gap, coupling and pri length, and make another run....

Which brings me to my next (last?) question(s): 'Is capacitance always

capacitance?' Some report that SW caps giveup 'wimpy' sparks. Not being

an EE, my gut-feel is that this may be the mass-differentials of the

electrodes (SW vs plate caps), which lead to 'charge-rate' (or Q(?))

differences of the two capacitor types, which may account for the

reported wimpiness....

Can static (vs. dynamic)empirical C readings equate the two types? Or

does the SW cap simply not meet the dynamic requirements of our systems,

for some of the above reasons?

Next to(LAST!) question: Is there, among our enlightened literatEE, an

opinion that this approach has, or may have:  A) potential value as an

analytical method; or B) no value- drop it like a hot potato!

LAST!! question: WHY?

Mike I'm sorry to subject you to this, but that's what you get for being

the first to respond to my questions!   ~~<=:)

Gene Waldo aka An Iggerant Igor

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 16:23:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 21 Mar 1998 08:23:23 -0800 (PST)

Subject: Re: Bottle Cap outer electrode??

On Fri, 20 Mar 1998, Tesla-2 wrote:

>

> While cogitating on wiring up our Cap assy: 12 or 16 bottle units of 40

> oz. Bud, and the problems of wrapping Al foil on all of them, I asked

> myself, "What about 2 or 3 coats of aluminum spray paint?"

Even easier, and works better to-boot. Put the bottles in saltwater.

place electrode for the outer plate in the saltwater bath.

You get better capacitance, and you don't have to wrap them

in foil

[Remember to float some  motor oil on top to kill corona.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: LOD Group, too

Date: Sat, 21 Mar 1998 16:26:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sat, 21 Mar 1998 08:26:48 -0800 (PST)

Subject: Re: LOD Group, too

On Fri, 20 Mar 1998, Tesla-2 wrote:

> potentials are! Pun intended!

>

> Personal to any Los Angeles based coilers:  Can we think about a

> sit-down over coffee, et al? Some neutral location, to compare notes and

> chat, spit 'n argue? Month or so from now?  If interested, send me your

> thoughts on such, and your ZIP.  I'll try to coordinate the thing.

> Also I can turn any of you on to our local treasure house of electrical

> & mechanical PARTS & EQUIPMENT! Figure 2 hours min to troop the place

> and just see whats there!  It's ALL there.

LA? No.. Inland Empire, you betcha :)

[What the hell, living in SoCal, I have gotten used to commuting.

92501 is my zip, and I'd love to get together...

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Variable Salt Water Caps

Date: Sat, 21 Mar 1998 16:33:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 08:33:12 -0800

Subject: Re: Variable Salt Water Caps

Organization: Self

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Sat, 21 Mar 1998 10:17:55 -0500

> Subject: Variable Salt Water Caps

>

> > From: Gene Waldo <gwaldo@1stnetusa.com>

> > Date: Fri, 20 Mar 1998 17:02:53 -0800

> > Subject: Re: Bottle Cap outer electrode??

> > Organization: Self

> >

> Snippidity Do Dah!

> >

> > My next query is about TUNABLE SW cap?  .....Hope this helps.

> Fr. Tom McGahee

>

Tom,

You confirm much of my thinking, but please see further questions to

Mike Hollingsworth, this date.

Thanks,

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFGnd ?

Cc:

Bcc:

X-Attachments:

At 02:55 AM 3/21/98 GMT, you wrote:

>From: Scott MacGregor <smacgregor@aasp.net>

>Date: Fri, 20 Mar 1998 18:55:39 -0800

>Subject: Re: RFGnd ?

>Organization: EES,Co.

>

>Tesla-2 wrote:

>

>> Gene and Scott,

>>

>> Its precisely because of skin effect that braided conductors

>> are not optimum for RF. Due to skin effect RF currents want to flow

>> along the outside of conductors. If that conductor is braided the current

>> has to follow the ins and outs of the braided wires. Tinning the

>> braid does not help as this forces the current flow through the solder

>> which has a much higher resistance than copper.

>>

>> Wide flat conductors like strap offer the lowest impedance.

>>

>> Malcolm Watts once ran a series of test using various wires

>> and cables for Tesla grounding and found a definite drop in

>> system Q when braided cables were used.

>>

>> In my work installing, maintaining, and repairing two-way

>> commercial radio equipment we do not use braid for RF or lightning

>> grounding. We normally solid AWG 6 copper for bonding equipment

>> cabinets and stranded AWG 4 for lighning grounds.

>>

>> TTYL!

>> Mike Hammer

>> mhammer@misslink.net

>

>You will agree, I'm sure, that there is a distinction between equipment

>grounds, (for 60 Hz safety) and RF grounds.

Of course.

>If I am right that most Tesla coils operate "around" 500 kHz,

>then the length or type of copper material used for an effective

>RF ground would depend primarily on its DC resistance.

Primarily yes. Completely no. The RF resistance of any conductor

depends on many factors. Diameter and compositon of the conductor are only

two of the factors. Also the frequency and inductance of the conductor

needs to be taken into consideration.

The RF currents flowing in even a medium size Tesla coil

RF ground can be several tens of amperes. Hundreds of amperes in

large coils. It doesnt take much extra RF resistance to represent

significant power loss.

At RF frequencies a wire is no longer just a wire.

Its an inductor. The higher the frequency the more

inductance that wire has. For this reason heavy flat

conductors are recommended to reduce this inductance.

Not only in our RF grounds but also in the construction

of capacitors and tank circuit wiring.

Any inductance in the ground lead represents off axis

inductance that robs the secondary of energy.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: LOD Group, too

Date: Sat, 21 Mar 1998 18:35:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 10:35:17 -0800

Subject: Re: LOD Group, too

Organization: Self

Tesla-2 wrote:

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Sat, 21 Mar 1998 08:26:48 -0800 (PST)

> Subject: Re: LOD Group, too

>

> On Fri, 20 Mar 1998, Tesla-2 wrote:

>

> > potentials are! Pun intended!

> >

> > Personal to any Los Angeles based coilers:  Can we think about a

> > sit-down over coffee, et al?

> LA? No.. Inland Empire, you betcha :)

> [What the hell, living in SoCal, I have gotten used to commuting.

>

> 92501 is my zip, and I'd love to get together...

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical

> Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

Both Dan Murphy & I are 91341, (San Fernando)...  This puts us central

to the 210 fwy, along with Bill Wysock (91016, Monrovia), I think.  San

Berdo is off my Thomas Bro, so I couldn't actually find 92501, re: 210.

Looks like Bill may be our initial locus... if he's interested... I

certainly HOPE he will partake!  How about it, Bill?

Anyone else out there?

Gene Waldo  <gwaldo@1stnetusa.com>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: coupling adjusting mechanism

Date: Sat, 21 Mar 1998 19:19:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sat, 21 Mar 1998 14:19:04 -0500 (EST)

Subject: Re: coupling adjusting mechanism

On Thu, 19 Mar 1998, Tesla-2 wrote:

>

> This doesnt produce any holes. What Bob is describing

> is having one piece of pipe telescope inside another

> with a friction fit. I think this is a simple and elegant

> solution.

>

I was just thinking...  Why not hook it up to a compressed air source, and

use it as sort of a pneumatic cylinder.  Change the pressure and you

change the height.  Wouldn't have to worry about arcing, as at the low

pressures necessary plastic fittings would do fine.  You could even change

coupling on the fly:)  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

"BETTER LIVING THROUGH RECKLESS EXPERIMENTATION" - Motto of The Mad Scientist

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ion gun/herf gun

Date: Sat, 21 Mar 1998 19:20:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sat, 21 Mar 1998 14:20:15 -0500 (EST)

Subject: Re: ion gun/herf gun

On Fri, 20 Mar 1998, Tesla-2 wrote:

>

> or anyone know of a web page with such info?

>

I'm interested in that kind of stuff too, though maybe this query would be

better suited for the hvlist?  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

"BETTER LIVING THROUGH RECKLESS EXPERIMENTATION" - Motto of The Mad Scientist

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: ofRe: Bottle Cap outer electrode??

Date: Sat, 21 Mar 1998 19:36:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 21 Mar 1998 13:36:32 -0600

Subject: ofRe: Bottle Cap outer electrode??

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Sat, 21 Mar 1998 08:06:27 -0800

> Subject: Re: Bottle Cap outer electrode??

> Organization: Self

>

> Tesla-2 wrote:

<SNIP>

> My real questions tho, are:  a system, built to calculated parameters,

> usually needs fine adjusting....   it seems to me that having a (HOT!)+-

> 5 or 10% range on the capacitance should enable the real 'tweaking'

> req'd to 'tune' (match) resonant Hz to an RCH!  Shouldn't  this 'hot

> tweak' capability be an advantage in compensating for slight out of tune

> conditions in both coupling and primary length?

>

> My expeience in 'tuning' systems, both randomly  and sinusoidally

> excited, has taught me that tight, hi-Q systems 'peak' at quite specific

> frequencies.

This is correct. A well-constructed secondary coil for example can have

a Q in excess of 200. The primary circuit typically has a relatively low

Q (usually in the 10-20 range) due to the lower inductance and the

presence of the spark gap in series with the LC circuit.

>

> I can envision a 'calibrated fluid-level' readout on the brine tank, a

> simply controlled flow rate in the siphon, and a 'stand back, and wait

> for it to go bang (PEAK)' means of tuning... When your spark maxes out,

> note the fluid depth, and you're home! Then get an empirical readout on

> capacitance.  It should then be possible to 'cold' tweak either or all:

> terminal gap, coupling and pri length, and make another run....

>

This would work. However a less lossy way is to take a transmitting type

variable capacitor and immerse it under oil to increase its breakdown

voltage. These typically have plate-plate spacings of 1/4" - 1/2".

Adjustment can be done via an insulated shaft. Doing tuning "cold" won't

take into account some of the additional capacitance that comes from the

conductive streamers that are (hopefully) spewing from your toroid.

> Which brings me to my next (last?) question(s): 'Is capacitance always

> capacitance?' Some report that SW caps giveup 'wimpy' sparks. Not being

> an EE, my gut-feel is that this may be the mass-differentials of the

> electrodes (SW vs plate caps), which lead to 'charge-rate' (or Q(?))

> differences of the two capacitor types, which may account for the

> reported wimpiness....

> Can static (vs. dynamic)empirical C readings equate the two types? Or

> does the SW cap simply not meet the dynamic requirements of our systems,

> for some of the above reasons?

Capacitance is always capacitance. However, certain dielectric materials

are more efficient at storing/releasing energy when used as a capacitor

dielectric. Important characteristics of dielectric materials include

leakage current, dielectric loss, voltage breakdown, and dielectric

constant. Assuming that the last two characteristics provide the voltage

rating and capacitance needed, leakage current and dielectric loss are

also very important in for an efficient Tesla Coil tank capacitor. When

the main gap fires, the degree that the energy that's initially stored

in the tank cap can be efficiently transfererred and used is very

critically dependent upon the dielectric's loss and leakage.

"Lossy" dielectrics, such as Mylar or glass may no be very conductive,

but they'll still tend to lose a significant amount of energy under the

high frequency and pulse discharge operation seen in a Tesla Coil.

Considerable power may be lost simply heating up the dielectric material

instead of going into making hot streamers. In addition, this lossiness

also reduces the Q of the primary LC system, substantially decreasing

the tuning sharpness, and resulting in "wimpy" sparks. Using higher

quality materials such as polypropylene (PP) or polyethylene (LDPE/HDPE)

makes a significant improvement in coil efficiency and the appearance of

the streamers.

>

> Next to(LAST!) question: Is there, among our enlightened literatEE, an

> opinion that this approach has, or may have:  A) potential value as an

> analytical method; or B) no value- drop it like a hot potato!

>

> LAST!! question: WHY?

>

While it might prove to be a bit "Rube Goldbergish", the approach you've

proposed WILL work. The tuning will not be very sharp due to the poor

dielectric loss characteristics of the beer-bottle caps.  A more

efficient approach would be to use more efficient PP or LDPE

capacitor(s) in parallel with a "tuning" cap made from a transmitting

variable cap immersed in oil.

<SNIP>

Good luck, and safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Sat, 21 Mar 1998 20:55:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 12:55:43 -0800

Subject: Re: RFGnd ?

Organization: Self

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 21 Mar 1998 11:26:20 -0600

> Subject: Re: RFGnd ?

>

> At 02:55 AM 3/21/98 GMT, you wrote:

> >From: Scott MacGregor <smacgregor@aasp.net>

> >Date: Fri, 20 Mar 1998 18:55:39 -0800

> >Subject: Re: RFGnd ?

> >Organization: EES,Co.

> >

> >Tesla-2 wrote:

> >

> >> Gene and Scott,

> >>

> >> Its precisely because of skin effect that braided conductors

> >> are not optimum for RF.

> Mike Hammer

> >> mhammer@misslink.net

> >

> >You will agree, I'm sure, that there is a distinction between

> >equipment grounds, (for 60 Hz safety) and RF grounds.

>

> Of course.

>

> >If I am right that most Tesla coils operate "around" 500 kHz,

> >then the length or type of copper material used for an effective

> >RF ground would depend primarily on its DC resistance.

>

> Primarily yes. Completely, no.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Mike,

Thank you for your informed interest!

My sticking spot is, I guess, the actual practical trade-off between

mounting my 2ndary base to a Cu post in the backyard, and having some

kind of installation flexibility, which means a lead of some practical

length to the post.  Solid flat Cu strap is ideal, but cumbersome.

Solid Cu wire is next best but still some hard to manage in a portable

system. Stranded Cu cable is next best, as it becomes a bit more

manageable.

My question is: What 'physical' description would best describe the

RFGnd lead for our system, which will need to be largely portable: We

are building a 24ga. 4" x ~19" 2ndary powered by ('presently' largest)

12.5kvacN @30a. Most small cable is made up of 7 or 18 strands of Cu

wire of some ga.  I have some cable (? size), which is made up of 18 st

of 10 or 11 ga. (.1")wires.  That bundle mikes @ .37". Overkill?  Not

suitable? My sense is that it is too big, in any case, and we should

look for something smaller: stranded, solid, strap? Does stranded wire

'gauge' the same way as solid?

I should be able to turn a roll of Cu strip, but would be in the dark as

to the dimensions to get. I gather we need more surface than sectional

area? So 'foil' as opposed to barstock?  I have seen Gnd straps made up

of several layers of Cu strips terminated in lugs.

I would like to make do with an insulated cable of some (say 18st of 21

ga.) moderate size, for the lead (20' min) to my GND installation

I guess what I need is a firm, drop dead, compromise!

NOW! for the GND itself: I grasp: AVOID plumbing and any domestic gnd.

.5" cu tube is easy for me! What say you to: 10 to 12' of it perforated

.o6"dia about every foot, buried to about 8" in the yard, w/ 1' bent up,

at the wall to braze a lug to, for the RFGnd lead connection; and we can

periodicaly pour NaCl sol'n in to generate our plume, along it's length?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: checking capacitance

Date: Sat, 21 Mar 1998 22:10:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Sat, 21 Mar 1998 17:10:24 EST

Subject: checking capacitance

I need some advice on what I should buy to determine the actual capacitance of

my home made caps. By this I mean the best buy (I'm on a limited budget) and

also instructions on how to actually go about the testing.

Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Thanks for plate capacitor plans.

Date: Sat, 21 Mar 1998 22:13:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: John Brudvik <john.brudvik@wcom.com>

Date: Sat, 21 Mar 1998 16:13:44 -0600

Subject: Thanks for plate capacitor plans.

Just wanted to say thanks to all of you that sent me plate capacitor plans

they have been a big help.

John

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plate caps & connections

Date: Sat, 21 Mar 1998 22:45:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: John Brudvik <john.brudvik@wcom.com>

Date: Sat, 21 Mar 1998 16:45:20 -0600

Subject: Re: Plate caps & connections

>>> Tesla-2 <tesla-2@emachine.com> 03/19/98 10:53pm >>>

From: Hollmike <Hollmike@aol.com>

Date: Thu, 19 Mar 1998 23:53:58 EST

Subject: Re: Plate caps & connections

<snip>

>about twice that much.  These sheets are .004" thick and they claim a

>dielectric strength of 20kV/mil.  I don't believe that and assume 20kV/

>sheet as the max  This would be more consistant with literature values

>I have seen

<snip>,

>Mike Hollingsworth

If I am not missing anything here if you use the formula given in a earlier

post  you can make a .01mf plate cap with n = 4 and A = 64 sq/in

assuming k =2  using two layers (d=.008) of flexible muscovite with a

voltage ratting of 40kv.

If k =5 then the same capacitor would have n = 2.

Is this correct?

If I made a plate capacitor with flexible muscovite would I need to

immerse it in oil?

I have heard some comments that mica will heat up in a Tesla tank circuit

would flexible muscovite have this problem since it sounds like a

compound product?

Thanks for the help.

John

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ofRe: Bottle Cap outer electrode??

Date: Sun, 22 Mar 1998 00:25:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 16:25:01 -0800

Subject: Re: ofRe: Bottle Cap outer electrode??

Organization: Self

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Sat, 21 Mar 1998 13:36:32 -0600

> Subject: ofRe: Bottle Cap outer electrode??

> Organization: Stoneridge Engineering

>

> Tesla-2 wrote:

> >

> > From: Gene Waldo <gwaldo@1stnetusa.com>

> > Date: Sat, 21 Mar 1998 08:06:27 -0800

> > Subject: Re: Bottle Cap outer electrode??

> > Organization: Self

> >

> > Tesla-2 wrote:

> <SNIP>

> Good luck, and safe coilin' to you!

>

> -- Bert --

>

Bert,

Thanks the helpfull replies!  (Variable tuning Capacitor in ckt w/

bottle caps) One more piece of equipment!!!  Even in ckt with plate

jobs???  sin dudo? I guees they're not to critical, since few mention

them.

Years ago when I built my 1st TC, my uncle told me all you need (I had

been given a 12,000v N xfmr) is a big coil and a little one! He was

right!  It sparked up a storm! 6-8" sparks! No cap, no gnd!  What did I

know? what did he know?   We thought we were building a transformer! No

notion of a tuned resonator! Must have been close to 28" of maybe 28ga

on a 3" form, and about 16 t of #14 on a 10" form. Set the tool post

feed on Dad's lathe for about .0125 pitch and let'er rip! Prety coil!

Anyway, back on topic, Waldo! I'm lookig at my fresh color hardcopies of

<.../hickman/primary.jpg> and see your blue #1 AWG Gnd lead from 2ndary

base: solid? stranded? So ~.3" dia (cable) is acceptable for RFGnd? You

can get away with that close proximity to the primary? I was going to

bring the whole bottom of the 2nd form thru the pri base center, to make

that connection.

Also, I'm getting static from some about spending so much time on a

'useless' device!  They fail to see the utility of loud noise and ozone!

Can these things be configured to cook hot dogs? Or a egg?  Or do

something "usefull"?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Please Help

Date: Sun, 22 Mar 1998 00:52:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 22 Mar 1998 11:52:19 +1100

Subject: Please Help

After my first coil flopped a built another one using a lot of help from

Malcom Watts (Thanks!).  But I tried it this weekend and it didnt work :(

These are the specs:

Primary coil-7turns of 0.25" copper pipe spaced aprrox 0.25" inner dia. 6.2

outer dia. unknown

Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

bottom and a lot of space at the top (8" maybe) has 4 coats of polyurethane

varnish over the windings.

Top Electrode-A makeshift toroid that isn't smooth by any means so I

wouldn't count it's capacitance.

Capacitors-6 saltwater types in a box giving about .0045 uF (I have another

6 if needed)

Spark gap-two brass brackets with tapped holes carrying 1" wide x 1.5" long

pieces of brass.

Safety Gap- The same sort of thing as the spark gap except there is a groung

plate in the middle.

Transformer- One 12000V/30mA Neon sign transformer (I have 2 more if needed)

Chokes-none

RF Ground-2' aluminium pipe

Primary coil is resting on glass.  Secondary is capped at one end, sitting

on the glass at the other. The wire to the primary coil is attached by

alligator clips

From Scott

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: equipment protection

Cc:

Bcc:

X-Attachments:

At 12:34 AM 3/22/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Sat, 21 Mar 1998 16:34:09 -0800

>Subject: Re: equipment protection

>

>

>

>

>>

>>

>> Interesting point!  Short of an actual strike, how much danger is my CD,

>> TV, and other chip based gear in, while I'm operating my household Tesla

>> Coil? I gather most of you operate with impunity... I know,  RFGnd!!!

>> RFGnd!!  Any horror stories?

>>

>

> Well I was running my coil in the next room, but I had forgotten to turn off

> and unplug my computer, when I came back my windows were different sizes and

> I had programs running that were not running before, but no big failures

>

I always unplug my computer before running the coil. This should include

the phone line to your modem.

Various types of modern mircoelectronics wont take kindly to

being in the vicinity of an operating coil.

Some things Ive heard of that were damaged include watches, cameras

and garage door openers.

A good RF ground as well as RF chokes will help to minimize this

problem. This is also why you dont bring your 60hz house ground

any where near your coil. Neon cases should be grounded to RF ground

not the 60 hz ground.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFGnd ?

Cc:

Bcc:

X-Attachments:

At 08:55 PM 3/21/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Sat, 21 Mar 1998 12:55:43 -0800

>Subject: Re: RFGnd ?

>Organization: Self

>

>Mike,

>

>Thankyou for your informed interest!

>My sticking spot is, I guess, the actual practical trade-off between

>mounting my 2ndary base to a Cu post in the backyard, and having some

>kind of installation flexibility, which means a lead of some practical

>length to the post.  Solid flat Cu strap is ideal, but cumbersome.

>Solid Cu wire is next best but still some hard to manage in a portable

>system. Stranded Cu cable is next best, as it becomes a bit more

>manageable.

>My question is: What 'physical' description would best describe the

>RFGnd lead for our system, which will need to be largely portable: We

>are building a 24ga. 4" x ~19" 2ndary powered by ('presently' largest)

>12.5kvacN @30a. Most small cable is made up of 7 or 18 strands of Cu

>wire of some ga.  I have some cable (? size), which is made up of 18 st

>of 10 or 11 ga. (.1")wires.  That bundle mikes @ .37". Overkill?  Not

>suitable? My sense is that it is too big, in any case, and we should

>look for something smaller: stranded, solid, strap? Does stranded wire

>'gauge' the same way as solid?

The ground requirements go up as the power level increases.

Small coils can operate well with a marginal ground.

However as you try increasing your power level the poor

ground will become a bottleneck to increased performance.

Stranded cable is OK as long as the strands are not twisted.

My present ground lead is stranded AWG 8. I get good performance

from this as my ground lead is less than 6 feet long.

You can use longer ground leads but the longer you make it the heavier

the wire should be for lowest impedance.

If your ground isnt located too far away from the coil any

reasonable length for the ground lead is acceptable.

>

>I should be able to turn a roll of Cu strip, but would be in the dark as

>to the dimensions to get. I gather we need more surface than sectional

>area? So 'foil' as opposed to barstock?  I have seen Gnd straps made up

>of several layers of Cu strips terminated in lugs.

Seems like overkill to me. Maybe if you were running 8-10 KVA

this would be necessary.

>I would like to make do with an insulated cable of some (say 18st of 21

>ga.) moderate size, for the lead (20' min) to my GND installation

>I guess what I need is a firm, drop dead, compromise!

Use whatever cable you can lay your hands as long as its not braid

or twisted. Make it only as long as it needs to be and run it in

as straight a line as possible. Use at least AWG 8.

>

>NOW! for the GND itself: I grasp: AVOID plumbing and any domestic gnd.

>

>.5" cu tube is easy for me! What say you to: 10 to 12' of it perforated

>.o6"dia about every foot, buried to about 8" in the yard, w/ 1' bent up,

>at the wall to braze a lug to, for the RFGnd lead connection; and we can

>periodicaly pour NaCl sol'n in to generate our plume, along it's length?

>

I dont think its necessary to add salt to the ground to raise

conductivity unless your soil is very dry and sandy.

Instead of burying a single 12 foot length cut it in half and

bury the two 6 foot pieces about 2 feet apart. Bond them together

at the top.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coupling adjusting mechanism

Cc:

Bcc:

X-Attachments:

At 07:19 PM 3/21/98 GMT, you wrote:

>From: Paul Anderson <madhobby@geeky1.ebtech.net>

>Date: Sat, 21 Mar 1998 14:19:04 -0500 (EST)

>Subject: Re: coupling adjusting mechanism

>

>On Thu, 19 Mar 1998, Tesla-2 wrote:

>

>>

>> This doesnt produce any holes. What Bob is describing

>> is having one piece of pipe telescope inside another

>> with a friction fit. I think this is a simple and elegant

>> solution.

>>

>I was just thinking...  Why not hook it up to a compressed air source, and

>use it as sort of a pneumatic cylinder.  Change the pressure and you

>change the height.  Wouldn't have to worry about arcing, as at the low

>pressures necessary plastic fittings would do fine.  You could even change

>coupling on the fly:)  TTYL!

>

>

>Paul Anderson

While this could be done and might be interesting to try

I dont think the benefit is all that great. Coupling is not

something you really need to adjust that often. Mostly you

find the best coupling and hardly ever change it. I dont

think Ive changed the coupling on my coil in months.

Im also a big fan of keeping things simple. Id much rather

spend my time building a plate cap or new spark gap than

building an "on the fly" coupling adjuster.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: bombarder

Date: Sun, 22 Mar 1998 01:51:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sat, 21 Mar 1998 20:51:06 -0500

Subject: bombarder

Hi all

anyone know where I can get one of those bombarders that Mike described..

or something similar.

I would have paid  the 100$ for one without a second thought.

I would like to find the guy who had that one.

anyone with any info please contact me off list at  cwatkins@seidata.com

thanks

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Ground measurements

Date: Sun, 22 Mar 1998 06:51:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 21 Mar 1998 22:51:14 -0800

Subject: Ground measurements

Organization: EES,Co.

I came across an interesting way to calculate your grounding resistance.

Put it on the following page. Check out http://www.expage.com/page/tesla

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Book: WIZARD:

Date: Sun, 22 Mar 1998 07:20:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 21 Mar 1998 23:20:19 -0800

Subject: Book: WIZARD:

Organization: Self

I am thinking about ordering the book, 'WIZARD: The life & Times of

Nikola Tesla', by M. Siefer; $16.

reader comments?

I have read  'Man out of Time'

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Bottle Cap outer electrode??

Date: Sun, 22 Mar 1998 12:43:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 22 Mar 1998 07:43:35 EST

Subject: Re: Bottle Cap outer electrode??

Gene,

I think the real reason for wimpy outputs is SW caps arise from two

reasons.  The first is the obvious losses due to glass as the dielectric.  The

second is that you are trying to get a lot of current through an electrolyte

solution.  While SW caps are suitable for smaller power systems, there is much

more resistance in electrolytes than in metallic conducters. This would reduce

the Q of the cap significantly I would think.

I pulled a good size variable cap from an old tube radio.  The plate

separation is about 1/8"(that would be about 40 - 50 kV in mineral oil).  I

was thinking  immersing this in mineral oil and using it for fine tuning my

tank circuit.  I would not give a tremendous tuning range, but if the primary

was tapped close enough to perfect tuning, then this might be a way to fine

tune it.  I would just have to construct a means by which to remotely rotate

the plates while the coil is running.

As for your questions, I haven't the answers as I am a ChemE and not an EE.

I think you should experiment with your idea and tell us how it worked out.

Mike Hollingsworth.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Sun, 22 Mar 1998 12:52:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 22 Mar 1998 07:52:46 EST

Subject: Re: RFGnd ?

Hi All,

I had one question about the braided wire issue:   I have an old (1930's)

EE handbook that describes the use of Litz wire in air core inductors.  It

goes further to say that this braided type of wire is great for compensating

for the skin effect due to the large surface area of the conductor.  It says

that the braiding allows each wire to have equal exposure to the outside of

the wire and therfore provides better conduction than twisted wire.  I don't

remember if it stated numbers relative to magnet wire and will have to look

again later.  My question is, has anyone used Litz wire and, if so, can you

tell me if it has any value in your opinion.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: checking capacitance

Date: Sun, 22 Mar 1998 12:57:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Sun, 22 Mar 1998 07:57:19 -0500

Subject: Re: checking capacitance

At 10:10 PM 3/21/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sat, 21 Mar 1998 17:10:24 EST

>Subject: checking capacitance

>

>I need some advice on what I should buy to determine the actual

capacitance of

>my home made caps. By this I mean the best buy (I'm on a limited budget) and

>also instructions on how to actually go about the testing.

>Thanks, Jim

>

Jim,

I know about the limited budget syndrome as well :)

I did eventually by an LCR meter which was a bit expensive but was worth every

penny. Not only does it read inductance, capacitance and resistance, but also Q

an D factors of components. Before I got this, I used a 555 timer chip. I have had

an oscilliscope for a long time and I used to set up the 555 timer chip with 'known'

resistances, and then plug in the cap to the chip and read the frequency on the scope.

Then from the formula for the 555, I could solve for C. If you do a search on hobby

and electronics, you will find some cheap capcitance meter-type circuits out there as

well. Hope this helps,

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: coupling adjusting mechanism

Date: Sun, 22 Mar 1998 13:08:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Sun, 22 Mar 1998 08:08:26 -0500

Subject: Re: coupling adjusting mechanism

At 07:19 PM 3/21/98 GMT, you wrote:

>From: Paul Anderson <madhobby@geeky1.ebtech.net>

>Date: Sat, 21 Mar 1998 14:19:04 -0500 (EST)

>Subject: Re: coupling adjusting mechanism

>

>On Thu, 19 Mar 1998, Tesla-2 wrote:

>

>>

>> This doesnt produce any holes. What Bob is describing

>> is having one piece of pipe telescope inside another

>> with a friction fit. I think this is a simple and elegant

>> solution.

>>

>I was just thinking...  Why not hook it up to a compressed air source, and

>use it as sort of a pneumatic cylinder.  Change the pressure and you

>change the height.  Wouldn't have to worry about arcing, as at the low

>pressures necessary plastic fittings would do fine.  You could even change

>coupling on the fly:)  TTYL!

>

>

>---

>Paul Anderson

>madhobby@geeky1.ebtech.net

Paul,

That is the exact idea I had talked with another coiler one time and was

kinda the basis for my telescopic setup. I did not continue on with it to the air

pressure stage. Watching coupling change during operation would certaining be a

learning experience that would want to watch. Another is not tuning the primary

circuit by the coil but instead by the capacitor being variable and adjustable during

operation.

It is hard for me to remember the sounds and sights from one firing to

another while I adjust coupling and/or tuning. Wouldn't it be great to control

both of these in operation as the senses would be able to detect optimal performance quickly.

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ofRe: Bottle Cap outer electrode??

Date: Sun, 22 Mar 1998 13:16:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 22 Mar 1998 08:16:33 EST

Subject: Re: ofRe: Bottle Cap outer electrode??

Gene,

One last thing about SW caps is that the wiring necessary to connect them

together and into the circuit can create some unwanted inductance.  This is

somewhat parasitic as it does not help transfer the energy to the secondary.

My cap is connected with 1 in. wide copper strips less than six inches long to

the gap.  The length of conducter required to connect a bank of SW caps may

reduce the overall performance a bit.

Just a thought,

M Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Ferrite Choke Questions

Date: Sun, 22 Mar 1998 13:28:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Sun, 22 Mar 1998 08:28:54 -0500

Subject: Re: Ferrite Choke Questions

At 01:21 AM 3/18/98 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Tue, 17 Mar 1998 19:21:34 -0600

>Subject: Re: Ferrite Choke Questions

>

<SNIPPAGE>

>>Where could I get some equations that would let me calculate inductance?

>

>There are some formulas on Amidon's web page to determine inductance

>values of inductors wound on their cores.

>

>For air core RF chokes you can use Wheeler's formula:

>

>

>   L(uH) = (r^2) * (N^2) / (9*r + 10*h)

>

>where:

>   r = coil RADIUS in inches

>   N = number of turns

>   h = coil height in inches

>

Mike,

I don't believe that this formula's 'r' variable is correct. I think it

needs to be :

r = mean radius

I determine this variable as (coil diameter + wire diameter) / 2

I may be the one incorrect here but I have used this single layer coil

winding formula

for years and have always used the 'mean radius'.

Here is the one I have used:

http://www.america.com/~tesla/formulas.html

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Thoughts on making choke cores.

Date: Sun, 22 Mar 1998 13:54:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Sun, 22 Mar 1998 08:54:30 -0500

Subject: Thoughts on making choke cores.

Howdy coilers,

As many other coilers have here, I had bought some of the iron-powdered

core toroids from Hosfelt to make RF chokes from. I followed the directions from

some previous builders and wound the core with layers of electrical tape,etc..

and have had no problems at all , in fact I liked then but then Hosfelt ran out and so

did I.

I have been trying to come up with a better, simpler and cheaper way to

have some good beefy chokes and this is my newest thought:

The toroid part would be one of those plastic 'doughnuts' that you see in

the toy store for small infants. We have all seen them, they are about 4 to 5

inches in diameter and some have bigger inside 'doughnut holes' than others.

Some sort of opening would have to be made in the toroid for it to be filled.

What to fill it with? This is something I am thinking on as well. You can

go to any automotive shop that 'turns' brake rotors and find a goldmine of fine iron

shavings that have come off the brake rotors. Using a sieve, you can

collect the smallest particles. I am still trying to decide what makes a good compound

to mix with these iron shavings and the best I have come up with is plaster of

paris. Mixing the plaster with the shavings  will yeild a compound to fill the plastic

doughnut with.

Experimenting with iron shaving - plaster ratios, one should be able to

come up with a good ratio that would be just fine. I don't know how much iron shavings

is too much and may cause core saturation but that would be in the experimentation

process. I had also thought of using an extremely thick oil such as automotive

gear oil as the compund mix but I envision that the iron shavings might

'settle' which would not be acceptable. Any ideas are welcome. Neccessity is the mother

of invention.

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Book: WIZARD:

Cc:

Bcc:

X-Attachments:

At 07:20 AM 3/22/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Sat, 21 Mar 1998 23:20:19 -0800

>Subject: Book: WIZARD:

>Organization: Self

>

>I am thinking about ordering the book, 'WIZARD: The life & Times of

>Nikola Tesla', by M. Siefer; $16.

>

>reader comments?

Gene,

I bought the book when it first came out and thoroughly

enjoyed it. Its an excellent read. Unlike some other earlier

Tesla biographies this one isn't clouded by the opinions and

feelings of the author. Its a factual account of Tesla's life.

I recommend it.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: checking capacitance

Cc:

Bcc:

X-Attachments:

At 10:10 PM 3/21/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sat, 21 Mar 1998 17:10:24 EST

>Subject: checking capacitance

>

>I need some advice on what I should buy to determine the actual capacitance of

>my home made caps. By this I mean the best buy (I'm on a limited budget) and

>also instructions on how to actually go about the testing.

>Thanks, Jim

>

Jim,

I have had very good luck with a Radio Shack digital meter

Cat # 22-174. It measures capacitance quite well and the

accuracy compares very well with my much more expensive

Fluke LCR meter. Price is about $75 although I believe it was

recently on sale.

As far as to how to measure capacitance with a meter like this

all you due is connect the leads to the cap out of circuit.

Simple as that.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ofRe: Bottle Cap outer electrode??

Date: Sun, 22 Mar 1998 15:51:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sun, 22 Mar 1998 10:51:33 -0500 (EST)

Subject: Re: ofRe: Bottle Cap outer electrode??

On Sun, 22 Mar 1998, Tesla-2 wrote:

>

> Also, I'm getting static from some about spending so much time on a

> 'useless' device!  They fail to see the utility of loud noise and ozone!

> Can these things be configured to cook hot dogs? Or a egg?  Or do

> something "usefull"?

>

Just tell them you're single-handedly trying to repair the ozone layer:)

I can remember I was once thinking about using a massive potato gun, a

screen, and a tesla coil to produce high-speed french fries, but we where

just joking around...  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

"BETTER LIVING THROUGH RECKLESS EXPERIMENTATION" - Motto of The Mad Scientist

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Ground

Date: Sun, 22 Mar 1998 16:48:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sun, 22 Mar 1998 08:48:34 -0800

Subject: Ground

Organization: EES,Co.

To the list,

I found a simple way to measure the effectiveness of a ground system. I

put it on the following site: http://www.expage.com/page/tesla

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Thoughts on making choke cores.

Date: Mon, 23 Mar 1998 02:05:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sun, 22 Mar 1998 18:05:26 -0800

Subject: Re: Thoughts on making choke cores.

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Bob Schumann <tesla@america.com>

> Date: Sun, 22 Mar 1998 08:54:30 -0500

> Subject: Thoughts on making choke cores.

>

> Howdy coilers,

>   As many other coilers have here, I had bought some of the iron-powdered core

>toroids from Hosfelt to make RF chokes from. I followed the directions from

>some previous builders and wound the core with layers of electrical tape,etc..

>and have had no problems at all , in fact I liked then but then Hosfelt ran out

>and so did I.

>I have been trying to come up with a better, simpler and cheaper way to

>have some good beefy chokes and this is my newest thought:

>The toroid part would be one of those plastic 'doughnuts' that you see in the

>toy store for small infants. We have all seen them, they are about 4 to 5 inches

>in diameter and some have bigger inside 'doughnut holes' than others.

>Some sort of opening would have to be made in the toroid for it to be filled.

> What to fill it with? This is something I am thinking on as well. You can go to

>any automotive shop that 'turns' brake rotors and find a goldmine of fine iron

>shavings that have come off the brake rotors. Using a sieve, you can collect

>the smallest particles. I am still trying to decide what makes a good compound

>to mix with these iron shavings and the best I have come up with is plaster of

>paris. Mixing the plaster with the shavings  will yeild a compound to fill the

>plastic doughnut with.

> Experimenting with iron shaving - plaster ratios, one should be able to

>come up with a good ratio that would be just fine. I don't know how much iron

>shavings is too much and may cause core saturation but that would be in the

>experimentation process. I had also thought of using an extremely thick oil such

>as automotive gear oil as the compund mix but I envision that the iron shavings

>might 'settle' which would not be acceptable. Any ideas are welcome. Neccessity

>is the mother of invention.

>

> Bob Schumann

Bob,

A machine shop would be another great source of iron filings. Plaster of

Paris is cheap, but you could also use epoxy. Has anyone tried using old

transformer E laminations? Seems you could unwrap an old x-former and

clip off the center of the E. That would leave you with a square of

laminated iron. (something like a torroid)

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Thoughts on making choke cores.

Date: Mon, 23 Mar 1998 02:19:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sun, 22 Mar 1998 18:19:13 -0800

Subject: Re: Thoughts on making choke cores.

Organization: Self

Tesla-2 wrote:

>

> From: Bob Schumann <tesla@america.com>

> Date: Sun, 22 Mar 1998 08:54:30 -0500

> Subject: Thoughts on making choke cores.

>

> Howdy coilers,

>    As many other coilers have here, I had bought some of the iron-powdered

>  cores

[Choppedy-clipclip]

> Neccessity is the mother of invention.

>

> Bob Schumann

Bob,

What are preferred dim's of a good toroid?

How many do you want?

Observation: Iron, in the presence of a water based matrix would tend to

corrode a lot muy rapido! This would swell and fracture the POP matrix.

Ferrite rod may be easier to get....  Some EE types should explain toroid

over rod, fer us dummies ouchere.  I'm sure it must have something to do

with induced fields and their collapse... back EMF and such like.

Keep windin'

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Go THERE!

Date: Mon, 23 Mar 1998 02:56:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sun, 22 Mar 1998 18:56:59 -0800

Subject: Go THERE!

Organization: Self

All,

I'm new, so excuse my meddling; but if other newbies out there haven't

been to <http://www.ttr.com>, GO THERE NOW.

See what all this is about!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Thoughts on making choke cores.

Date: Mon, 23 Mar 1998 03:06:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 22 Mar 1998 19:06:52 -0800

Subject: Re: Thoughts on making choke cores.

Tesla-2 wrote:

> From: Bob Schumann <tesla@america.com>

> Date: Sun, 22 Mar 1998 08:54:30 -0500

> Subject: Thoughts on making choke cores.

>

> Howdy coilers,

>   As many other coilers have here, I had bought some of the iron-powdered

>  core toroids from Hosfelt to make RF chokes from.

<SNIP>

Epoxy glue might work well

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: coiler location list - last call

Date: Mon, 23 Mar 1998 03:49:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 22 Mar 1998 21:49:07 -0600

Subject: coiler location list - last call

Once again, last call for the Tesla Coiler location list.

Send me :

name, location (address, or town, or whatever), phone (if you want),

summary of your coil (25 words or less), a homepage if you have

one, and your e-mail address.

I will *NOT* be posting it in HTML format on the page.  It will be zipped.

So far there are 70 people on it.  If you haven't sent your info, send it

NOW to corr@enid.com

If you *HAVE* sent info and you would like to make corrections, such

as leaving coil info or phone number out, you may e-mail it to me also.

Jeff Corr

2114 Monitor

Enid, OK 73703

http://www.harvestcomm.net/personal/corr/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Making iron dust chokes.

Date: Mon, 23 Mar 1998 04:02:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sun, 22 Mar 1998 22:02:15 -0600

Subject: Making iron dust chokes.

Organization: MailExcite  (http://www.mailexcite.com:80)

Coilers,

Perhaps the fellow mixing iron dust with plaster should

use automotive-type fiberglass mix in lieu of plaster.

I'm talking about the stuff you buy by the quart, mix

with a bit of hardener, and paste onto a piece of

fiberglass.

Don't use the fiberglass cloth, just the epoxy portion.

This stuff can be used to create large castings if you reduce

the amount of hardener a bit.

If you attempt to pour a large thick object you must

reduce the amount of hardener or the inside of the

casting will get too hot, and it will (if you are

unlucky) catch fire.  I speak from experience!

I used about 1/2 to 2/3 of recommended amt of hardener.

I once mounted a coffee can to a motor, started it running, and

filled the inside of the coffee can with this epoxy.

The centrifugal force pulls the liquid out into a

parabola, where it stays until it hardens.

(I was making molds used for creating parabolic reflectors...)

Oh, if you go to your local AG (tractor/farm) supply store, you

can buy a 5-lb box of type 7014 welding rods for about

$9.00.  This type of rod is composed of an iron powder

which might be good for making homemade inductors.

Using powdered iron will reduce the eddy current losses

you would otherwise suffer if you were just using a

solid steel core, etc.  Pound the flux coating off with

a hammer to increase the packing density.

Or you could buy a assortment of ferrite rods from

Amidon, like I just did.  Results pending...

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitance

Date: Mon, 23 Mar 1998 04:37:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Sun, 22 Mar 1998 23:37:31 EST

Subject: Capacitance

An easy way too measure capacitance is too connect the capacitor to the wall

socket and use an ammeter to measure the current it draws.  Its also a good

idea to check the wall sockets voltage and not just assume its 115.

In the USA the frequency should be 60 Hz.  All units in SI.

C = i / (2 * Pi * V * f) or

Capacitance = current / (2 * 3.14159 * voltage * frequency)

Erik Schulz

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Inductance

Date: Mon, 23 Mar 1998 04:38:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Sun, 22 Mar 1998 23:38:42 EST

Subject: Inductance

Bob,

I found through testing that when using a ferrite or iron core for an

inductor that the inductance is about quadrupled at 60 Hz.

Erik Schulz

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Go THERE!

Date: Mon, 23 Mar 1998 07:26:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 22 Mar 1998 23:26:56 -0800

Subject: Re: Go THERE!

>

>

> All,

> I'm new, so excuse my meddling; but if other newbies out there haven't

> been to <http://www.ttr.com>, GO THERE NOW.

>

> See what all this is about!

>

Nice selection of odd parts, but as for getting a coil prebuilt, building

the coil is half the fun.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Go THERE!

Date: Mon, 23 Mar 1998 16:59:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 23 Mar 1998 08:59:36 -0800

Subject: Re: Go THERE!

Organization: Self

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Sun, 22 Mar 1998 23:26:56 -0800

> Subject: Re: Go THERE!

>

>> All,

> > I'm new, so excuse my meddling; but if other newbies out there haven't

> > been to <http://www.ttr.com>, GO THERE NOW.

> >

> > See what all this is about!

>>

> Nice sellection of odd parts, but as for getting a coil prebuilt, building

> the coil is half the fun.

>

Mike,

I agree! But it is nice to see the big stuff, nicely done. I come from a

prototype shop background, and anodized Al (or plastic) fixtures with

C'bored capscrew assy, simply beats plywood, nails, duct tape and stuff

proped up on plastic buckets, esthetically...

Any EEs:

Why should reversing polarity on VOM resistance measurements yield

differing indicated values?

I refer to the recent post on measuring the resistance of RFGnd array

installations, and his 'page' describing the procedure.

I can only immagine some kind of "diode" effect in earth...?  But that

stretches even my credulity.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Go THERE!

Date: Mon, 23 Mar 1998 17:16:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 23 Mar 1998 09:16:00 -0800

Subject: Re: Go THERE!

Organization: Self

Gene Waldo wrote:

>

> Tesla-2 wrote:

> >

> > From: mike nicholas <705728@ican.net>

> > Date: Sun, 22 Mar 1998 23:26:56 -0800

> > Subject: Re: Go THERE!

> >

> >> All,

> > > I'm new, so excuse my meddling; but if other newbies out there haven't

> > > been to <http://www.ttr.com>, GO THERE NOW.

> > >

> > > See what all this is about!

> >>

> > Nice sellection of odd parts, but as for getting a coil prebuilt, building

> > the coil is half the fun.

> >

>

> Mike,

> I agree! But it is nice to see the big stuff, nicely done. I come from a

> prototype shop background, and anodized Al (or plastic) fixtures with

> C'bored capscrew assy, simply beats plywood, nails, duct tape and stuff

> proped up on plastic buckets, esthetically...

>

> Any EEs:

> Why should reversing polarity on VOM resistance measurements yield

> differing indicated values?

> I refer to the recent post on measuring the resistance of RFGnd array

> installations, and his 'page' describing the procedure.

> I can only immagine some kind of "diode" effect in earth...?  But that

> stretches even my credulity.

>

> Gene Waldo

PS,  I refer to post by Scott MacGregor,  Subj: GROUND,  Date: 22 Mar

'98, Sun 08:48.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFGnd ?

Cc:

Bcc:

X-Attachments:

At 12:52 PM 3/22/98 GMT, you wrote:

>From: Hollmike <Hollmike@aol.com>

>Date: Sun, 22 Mar 1998 07:52:46 EST

>Subject: Re: RFGnd ?

>

>Hi All,

>   I had one question about the braided wire issue:   I have an old (1930's)

>EE handbook that describes the use of Litz wire in air core inductors.  It

>goes further to say that this braided type of wire is great for compensating

>for the skin effect due to the large surface area of the conductor.  It says

>that the braiding allows each wire to have equal exposure to the outside of

>the wire and therfore provides better conduction than twisted wire.  I don't

>remember if it stated numbers relative to magnet wire and will have to look

>again later.  My question is, has anyone used Litz wire and, if so, can you

>tell me if it has any value in your opinion.

>Mike Hollingsworth

>

Mike,

Litz wire is a thing entirely different altogether from braid, or multi

stranded cable.  Each individual parallel conductor in Litz is

separately insulated from all its neighbours.  Each strand is an

insulated fine solid copper magnet wire. The individual strands

are essentially in parallel. Litz is most effective at medium RF

frequencies. (100khz-1mhz)

It is rather hard to come by these days and I have only

a short 100 foot piece of it. It should be quite effective

at carrying RF currents.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ferrite Choke Questions

Cc:

Bcc:

X-Attachments:

At 01:28 PM 3/22/98 GMT, you wrote:

>From: Bob Schumann <tesla@america.com>

>Date: Sun, 22 Mar 1998 08:28:54 -0500

>Subject: Re: Ferrite Choke Questions

>

>At 01:21 AM 3/18/98 GMT, you wrote:

>>From: Mike Hammer <mhammer@misslink.net>

>>Date: Tue, 17 Mar 1998 19:21:34 -0600

>>Subject: Re: Ferrite Choke Questions

>>

><SNIPPAGE>

>

>>>Where could I get some equations that would let me calculate inductance?

>>

>>There are some formulas on Amidon's web page to determine inductance

>>values of inductors wound on their cores.

>>

>>For air core RF chokes you can use Wheeler's formula:

>>

>>

>>   L(uH) = (r^2) * (N^2) / (9*r + 10*h)

>>

>>where:

>>   r = coil RADIUS in inches

>>   N = number of turns

>>   h = coil height in inches

>>

>Mike,

>  I don't believe that this formula's 'r' variable is correct. I think it

>needs to be :

>r = mean radius

>I determine this variable as (coil diameter + wire diameter) / 2

>I may be the one incorrect here but I have used this single layer coil

>winding formula for years and have always used the 'mean radius'.

>

>

>Bob Schumann

>

>

>

>

Bob,

Yes you are essentially correct. The formula says "coil radius"

not the radius of the coil form so that would imply including 1/2

the wire diameter.

I have to admit to taking the easy way and just using

the form diameter but with the small guage wire we

are using I doubt it makes too much difference. After all

its only being used for ballpark estimates.

The diameter of AWG 24 is about 20 mils so 1/2 that is 10 mils

or .01 inch. That doesnt really account for much with a coil

form 4.5 inches in diameter. Im not going to take the time to

do the math but I cant imagine that using a value for r of 2.26

instead of 2.25 would really make any significant difference.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Mon, 23 Mar 1998 20:00:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ms291 <ms291@mindspring.com>

Date: Mon, 23 Mar 1998 15:00:38 -0500

Subject: Re: Capacitance

I think perhaps this deserves a caveat. It might not be a good idea to put

polarized caps such as electrolytics and tantalums in the wall socket

unless you enjoy projectile motion and some pyrotechnics..  Coilers being

coilers, this could be considered a feature.. :)

Jim Horan

>From: ESchulz531 <ESchulz531@aol.com>

>Date: Sun, 22 Mar 1998 23:37:31 EST

>Subject: Capacitance

>

>An easy way too measure capacitance is too connect the capacitor to the

>wall socket and use an ammeter to measure the current it draws.  Its als=

>o a good=0Aidea to check the wall sockets voltage and not just assume its=

> 115.  In the US=0Athe frequency should be 60 Hz.  All units in SI.

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Unpotting Neons (success)

Date: Mon, 23 Mar 1998 21:42:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 23 Mar 1998 16:42:13 -0500

Subject: Unpotting Neons (success)

I finished unpotting my first neon (15kV/30ma) last night, and it

works! I chose not to remove any shunts this time, but will probably

try it on my next one.

Overall, the whole thing isn't too difficult. Knocking the tar off

took me about an hour, cleaning off the rest took an hour, and

repairing the damage I did to the windings during unpotting took about

an hour. I tested the transformer last night and it works great.

Don't fool yourself though, it's not easy either. You will definitely

get covered in tar and make a huge mess. Wear some type of plastic or

latex gloves if you can, otherwise be ready for dirt under your

fingernails for days. Don't even THINK about doing ANY this in the

house.

As soon as I get time I'll mount it in a case (full of oil) and make

it look pretty.

4 more to go!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitance

Date: Tue, 24 Mar 1998 02:04:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Mon, 23 Mar 1998 21:04:53 EST

Subject: Capacitance

Yes sorry, please don't connect polarized capacitors to the wall socket

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: choke cores revisited

Date: Tue, 24 Mar 1998 03:16:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 23 Mar 1998 19:16:05 -0800

Subject: choke cores revisited

Organization: Self

All,

Ref the post on ferrite cores: substitutes.

I've seen 'wound iron' cores mentioned in the literature.  You can get

rolls of soft iron wire at the hardware store, used for tieing re-bar in

form settings... and the like.

Could we wind toroid cores of this material?  Are the ferrite mtls

usually magnetized?

Another question: As sinusoidal ac pwr passes thru our main gaps, what

becomes of the wave shape?  Get fairly white, or is it apt to be a of

another character?  If I was still working, I could tape local 'static'

and do a spectrum analysis: V^2/Hz or Vrms/Hz...  but alass...

I used to be able to lug my (their) 30 IPS recorder home.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil measurements

Date: Tue, 24 Mar 1998 03:44:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 23 Mar 1998 21:44:31 -0600

Subject: Coil measurements

Organization: MailExcite  (http://www.mailexcite.com:80)

Coilers,

I performed a measurement of our coil's operating

frequency this weekend.  Perhaps someone can critique

my technique??

Available equipment - a shagged-out Wavetek frequency

generator, a decent digital frequency counter, and a

cheap DVM.

First I constructed an envelope detector using a 1N914

diode in series with a small commercial polypropylene

capacitor.  This allows me to measure the amplitude of

the HF AC waveform using my cheapo-non-RMS DC voltmeter.

I wanted to measure the resonant frequency of

our primary circuit.  I tapped the primary at the usual

point, and shorted the gap.  I then attached my signal

generator (50-ohm output impedence) across the tank

capacitor.  Next the envelope detector was also

connected across the capacitor.

As I varied the frequency, I could see the voltage on

the voltmeter reach a peak value, and then drop off.

Our primary LC reached its peak at about 250 KHz.

For the second measurement - I attempted to measure the

self-resonant frequency of our secondary and topload.

I attached the envelope detector between the base of

the secondary and the top toroid.  The frequency

generator was connected across the two outermost turns

of the primary coil.  The primary coil was disconnected

from the remainder of the tesla setup (no caps, gaps,

etc).

I then adjusted the frerquency generator until the

indicated voltage had peaked.  This frequency worked out to 255 KHz.

Strangely - I did not see other peaks at multiples of

255 KHZ.  The voltage was quite low once the input

frequency was much off of 255 KHz. (This is a

consequence of the secondary's Q value).  Does this

make sense?  I had expected to see other peaks!

We observed the self-resonant frequency of the

secondary change as different toploads were applied.

It was also surprisingly sensitive to the positions of

various people in close proximity to the coil!

Is this how it is done?  The results seemed to make

a fair amount of sense.

Question - while performing this sort of measurement on

the secondary, I presume I want a very low coupling

between the exciting source and the secondary coil.  Is

this correct?

Also - ideally I'd build an HPF into the envelope

detector - to supress wayward 60 Hz noise, but I was

too lazy to do that!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Tue, 24 Mar 1998 04:53:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Mon, 23 Mar 1998 22:53:28 -0600

Subject: Re: Capacitance

>Yes sorry, please don't connect polarized capacitors to the wall socket

Granted that is cool, but how does that relate to Tesla Coils?  Oh yeah,

I am getting 69" of spark! :)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil measurements

Date: Tue, 24 Mar 1998 06:09:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>I performed a measurement of our coil's operating

T>frequency this weekend.  Perhaps someone can critique

T>my technique??

Sure!  Always glad to butt in. <VBG>

T>Available equipment - a shagged-out Wavetek frequency

T>generator, a decent digital frequency counter, and a

T>cheap DVM.

Perfectly good for the task at hand.

T>First I constructed an envelope detector using a 1N914

T>diode in series with a small commercial polypropylene

T>capacitor.  This allows me to measure the amplitude of

T>the HF AC waveform using my cheapo-non-RMS DC voltmeter.

That should work fine.

T>I wanted to measure the resonant frequency of

T>our primary circuit.  I tapped the primary at the usual

T>point, and shorted the gap.  I then attached my signal

T>generator (50-ohm output impedence) across the tank

T>capacitor.  Next the envelope detector was also

T>connected across the capacitor.

The generator should be somewhat isolated from the tank circuit, or it

will change the resonant frequency.  I generally drive the Device Under

Test (DUT) through a 100 ohm resistor.

If available, for a detector, I like to use an oscilloscope because

of its sensitivity.  You should connect your detector across a small

portion of the coil, for instance, between the end of the coil connected

to the low side of the signal generator and a turn or so further up the

coil.  This reduces the capacitive loading on the DUT.

T>As I varied the frequency, I could see the voltage on

T>the voltmeter reach a peak value, and then drop off.

T>Our primary LC reached its peak at about 250 KHz.

With a smaller section of the coil connected to the detector, the

voltage peak will be sharper, but lower in amplitude.

T>For the second measurement - I attempted to measure the

T>self-resonant frequency of our secondary and topload.

T>I attached the envelope detector between the base of

T>the secondary and the top toroid.  The frequency

T>generator was connected across the two outermost turns

T>of the primary coil.  The primary coil was disconnected

T>from the remainder of the tesla setup (no caps, gaps,

T>etc).

Here you may have a problem.  Remember that the capacity of the top

load is measured in PicoFarads, and the capacity of the connecting wire

between the top of the coil and your detector may easily have more

capacity than your top load!  I suggest either winding about 10 turns

temporarily around the bottom end of the secondary and using that to

drive the detector.  That pretty much will eliminate the loading effect

of the test equipment.

In fact, that's one of the biggest problems folks have when measuring

anything.  The measuring process may drastically change what they are

trying to measure!

T>Strangely - I did not see other peaks at multiples of

T> 255 KHZ.  The voltage was quite low once the input

T>frequency was much off of 255 KHz. (This is a

T>consequence of the secondary's Q value).  Does this

T>make sense?  I had expected to see other peaks!

Yep, that's what I'd expect.  The first resonance will be generally

dominant.  Secondary resonances usually only appear if your test

equipment loads the system so lightly that it is effectively disconnected

from the DUT.

T>We observed the self-resonant frequency of the

T>secondary change as different toploads were applied.

T>It was also surprisingly sensitive to the positions of

T>various people in close proximity to the coil!

Not surprising, considering the low capacity of the top load and the

wide range of the field from the top electrode to earth.

T>Question - while performing this sort of measurement on

T>the secondary, I presume I want a very low coupling

T>between the exciting source and the secondary coil.  Is

T>this correct?

Yes, to determine their separate resonant frequencies.

Ralph

---

Break up a relationship - buy a computer!!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:magnifier

Date: Tue, 24 Mar 1998 06:25:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 23 Mar 1998 22:25:32 -0800

Subject: Re:magnafier

I need more information on how tesla magnifier circuits work,

I just cant see it?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tuning/Theory

Date: Tue, 24 Mar 1998 07:37:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 23 Mar 1998 23:37:02 -0800

Subject: Tuning/Theory

Hi gang, I am working on my coil and I am thinking about tuning and

coupling. I know that adjusting the coupling is a means of inducing more

or less of the electro magnetic field into the secondary via more or less

engagement of the primary, right so far? Now, please tell me about

tuning. By incorperating more or less windings of the primary, I am doing

what? does this effect oscillations due to harmonic interference? What is

the theory behind tuning?  Thanks, Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning/Theory

Date: Tue, 24 Mar 1998 06:43:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Tue, 24 Mar 1998 06:43:35

Subject: Re: Tuning/Theory

At 07:37 AM 3/24/98 GMT, you wrote:

>From: Walt Kulpa <gwkulpa@earthlink.net>

>Date: Mon, 23 Mar 1998 23:37:02 -0800

>Subject: Tuning/Theory

>

>Hi gang, I am working on my coil and I am thinking about tuning and

>coupling. I know that adjusting the coupling is a means of inducing more

>or less of the electro magnetic field into the secondary via more or less

>engagement of the primary, right so far? Now, please tell me about

>tuning. By incorperating more or less windings of the primary, I am doing

>what? does this effect oscillations due to harmonic interference? What is

>the theory behind tuning?  Thanks, Walt Kulpa

>

>

Walt,

The formula for resonant frequency is:

Rf = 1 / 2 * pi * SQR(L * C)

Where:

Rf = Resonant Frequency in Hertz.

L = Inductance in Henrys.

C = Capacitance in Farads.

(The SQR(L * C) means the 'square root' of L times C)

The Tesla coil is in tune when the resonant frequency of the

primary circuit is the same as the secondary circuit. From the

above equation you can see that it is possible to increase L

and lower C (or vice versa) and keep the same resonant frequency (Rf).

This is exactly what happens in the Tesla coil. The primary circuit

has large C (primary tank capacitor) and small L (primary coil), but

the secondary has large L (secondary coil) and small C (top terminal).

Since the secondary circuit is somewhat fixed, the tuning matching

happens at the primary coil. When you tune using less turns of the

primary, you are making the primary a lower inductance (lesser L),

when you tune using more turns of the primary, you are making the

primary high in inductance (more L). Somewhere on the primary coil

you should be able to match the resonant frequency of the secondary.

If you can't then it is time to go look a the primary capacitor (C)

to make sure that you can tune 'in range' of the secondary (or change

the secondary circuit). Hope this helps,

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Reference list?

Date: Tue, 24 Mar 1998 10:17:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: D Mills <ap7012@qmw.ac.uk>

Date: Tue, 24 Mar 1998 10:17:57 +0000

Subject: Reference list?

Organization: University of London (QMW)

Hi All,

As I have access to a large physics library, I was wonering if anyone

has a list of references to tesla related material.

Thanks

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning/Theory

Date: Tue, 24 Mar 1998 13:01:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Tue, 24 Mar 1998 08:01:02 EST

Subject: Re: Tuning/Theory

Walt,

By changing the number of turns on the primary, you change the inductance.

As the inductance increases(more turns), the primary frequency decreases as

the function:

1

---------------  = F

(2*pi*sqrt(L*C))

L = inductance in Henries

C = capacitance in Farads

F = frequency in Hz

Since the capacitance is normally a fixed value, the tuning is made by

adjusting the primary inductance until the primary resonant frequency matches

the natural resonant frequency of the seconday.

Hope this helps,

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil measurements: more comments

Date: Tue, 24 Mar 1998 13:35:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 24 Mar 1998 08:35:49 -0500

Subject: Re: Coil measurements: more comments

I am adding my comments to Ralph's comments.

Hope this helps.

Fr. Tom McGahee

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Coil measurements

> Date: Tuesday, March 24, 1998 1:09 AM

>

> T>I performed a measurement of our coil's operating

> T>frequency this weekend.  Perhaps someone can critique

> T>my technique??

>

>   Sure!  Always glad to butt in. <VBG>

It's what we do best!

>

> T>Available equipment - a shagged-out Wavetek frequency

> T>generator, a decent digital frequency counter, and a

> T>cheap DVM.

>

>   Perfectly good for the task at hand.

An oscilloscope is best at the detector end, as you can

*see* things that don't show up on a DVM.

>

> T>First I constructed an envelope detector using a 1N914

> T>diode in series with a small commercial polypropylene

> T>capacitor.  This allows me to measure the amplitude of

> T>the HF AC waveform using my cheapo-non-RMS DC voltmeter.

>

>   That should work fine.

Yes and no. It will work fine provided the voltage is much

more than .7 volts. Using a Germanium such as a 1N34 would

reduce that to about .4 volts, but any diode is going to

introduce a voltage drop.

The capacitor should be kept as SMALL as possible. This for

several reasons. Formost is the fact that the detector circuit

is not only going to load the circuit under test, but it will

also contribute CAPACITANCE to the circuit under test. This

in turn will shift the resonant frequency. All the more reason

to use an oscilloscope and do away with the detector circuit.

Of course, even a scope will add some capacitance. You can

minimize this by putting a small capacitor such as 10 pf in

series with the scope probe.

>

> T>I wanted to measure the resonant frequency of

> T>our primary circuit.  I tapped the primary at the usual

> T>point, and shorted the gap.  I then attached my signal

> T>generator (50-ohm output impedence) across the tank

> T>capacitor.  Next the envelope detector was also

> T>connected across the capacitor.

>

>   The generator should be somewhat isolated from the tank circuit, or it

> will change the resonant frequency.  I generally drive the Device Under

> Test (DUT) through a 100 ohm resistor.

>

>    If available, for a detector, I like to use an oscilloscope because

> of its sensitivity.  You should connect your detector across a small

> portion of the coil, for instance, between the end of the coil connected

> to the low side of the signal generator and a turn or so further up the

> coil.  This reduces the capacitive loading on the DUT.

>

You can also wind a separate coil of about 40 turns temporarily

around the secondary coil and attach your scope to this third coil.

This will reduce the tendency of the scope to load down the circuit

and also reduces the capacitive problem. You get a lower output,

but a more accurate picture as regards the resonant frequency.

Be aware that the actual operating frequency will always be off,

because when the TC is working you have ion clouds and the like

that add to the capacitance, thus lowering the operating frequency.

> T>As I varied the frequency, I could see the voltage on

> T>the voltmeter reach a peak value, and then drop off.

> T>Our primary LC reached its peak at about 250 KHz.

>

>   With a smaller section of the coil connected to the detector, the

> voltage peak will be sharper, but lower in amplitude.

>

> T>For the second measurement - I attempted to measure the

> T>self-resonant frequency of our secondary and topload.

> T>I attached the envelope detector between the base of

> T>the secondary and the top toroid.  The frequency

> T>generator was connected across the two outermost turns

> T>of the primary coil.  The primary coil was disconnected

> T>from the remainder of the tesla setup (no caps, gaps,

> T>etc).

>

>    Here you may have a problem.  Remember that the capacity of the top

> load is measured in PicoFarads, and the capacity of the connecting wire

> between the top of the coil and your detector may easily have more

> capacity than your top load!  I suggest either winding about 10 turns

> temporarily around the bottom end of the secondary and using that to

> drive the detector.  That pretty much will eliminate the loading effect

> of the test equipment.

>

>   In fact, that's one of the biggest problems folks have when measuring

> anything.  The measuring process may drastically change what they are

> trying to measure!

>

> T>Strangely - I did not see other peaks at multiples of

> T> 255 KHZ.  The voltage was quite low once the input

> T>frequency was much off of 255 KHz. (This is a

> T>consequence of the secondary's Q value).  Does this

> T>make sense?  I had expected to see other peaks!

>

>   Yep, that's what I'd expect.  The first resonance will be generally

> dominant.  Secondary resonances usually only appear if your test

> equipment loads the system so lightly that it is effectively disconnected

> from the DUT.

Remember that your detector involves a diode drop. At harmonics

the signal may be down so far that the diode does not conduct

at all.

>

> T>We observed the self-resonant frequency of the

> T>secondary change as different toploads were applied.

> T>It was also surprisingly sensitive to the positions of

> T>various people in close proximity to the coil!

>

>    Not surprising, considering the low capacity of the top load and the

> wide range of the field from the top electrode to earth.

>

> T>Question - while performing this sort of measurement on

> T>the secondary, I presume I want a very low coupling

> T>between the exciting source and the secondary coil.  Is

> T>this correct?

>

>   Yes, to determine their separate resonant frequencies.

>

>     Ralph

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning/Theory

Date: Tue, 24 Mar 1998 14:57:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 24 Mar 1998 06:57:31 -0800

Subject: Re: Tuning/Theory

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Mon, 23 Mar 1998 23:37:02 -0800

> Subject: Tuning/Theory

>

> Hi gang, I am working on my coil and I am thinking about tuning and

> coupling. I know that adjusting the coupling is a means of inducing more

> or less of the electro magnetic field into the secondary via more or less

> engagement of the primary, right so far? Now, please tell me about

> tuning. By incorperating more or less windings of the primary, I am doing

> what? does this effect oscillations due to harmonic interference? What is

> the theory behind tuning?  Thanks, Walt Kulpa

>

Walt,

A 2-coil Tesla coil consists of two tuned circuits, each consisting of

an inductor (L) and a capacitor (C), commonly referred to as an LC

circuit. Under the right conditions, an LC circuit will "ring" at a

specific frequency (called its resonant frequency), which will be a

function of the amount of L and C. The larger the value of either, the

lower the LC circuit's resonant frequency. When an LC circuit is

electrically excited at its resonant frequency, very high voltages and

currents can be generated.

While the primary LC circuit is obvious, the secondary one isn't. The

secondary coil and its top terminal have a small but very important

amount of capacitance with the other "plate" being the earth itself. The

combination of this capacitance, plus the inductance of the secondary

coil, forms the secondary's LC circuit. Since the other plate is earth,

it's very important to have a very good connection between the base of

the secondary coil and a solid RF ground such as multiple ground rods to

minimize losses.

Tuning simply means that you adjust things so that the resonant requency

of the primary is the SAME as that of the secondary. Most coilers

roughly design the system to be in the right ballpark, and do the final

tuning by adjusting the primary inductance via an adjustable tap. During

operation, energy that's initially present in the charged primary

capacitor will then cause the primary LC circuit to "ring" (oscillate)

when the main spark gap fires. The rapidly oscillating magnetic field in

the primary coil electromagnetically couples to the secondary's tuned LC

circuit. Since the primary and secondary LC circuits are "tuned" to the

same frequency, the secondary circuit readily absorbs this energy and

also begins oscillating, building (or "ringing up") to maximum energy in

only a few cycles. The greater the coupling, the less time required to

transfer all available energy from the primary to the secondary LC

circuit. As the energy in the secondary builds, the resulting high

voltage at the top of the secondary causes the air to become conductive,

forming the streamers we desire.

A system that's not tuned properly doesn't couple energy as efficiently

between the primary and secondary, and therefore doesn't perform as

well. In fact, a secondary that's very badly out of tune may result in

the highest voltage on the secondary being developed at a point

significantly below the top of the winding. This typically causes

sparking across the winding itself, sometimes destroying the coil.

Always make sure you tune your system at lower power before running at

full power. Hope this helped!

Safe coilin' to you, Walt!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Unpotting Neons (success)

Date: Tue, 24 Mar 1998 15:56:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 24 Mar 1998 07:56:09 -0800

Subject: Re: Unpotting Neons (success)

Organization: Self

Tesla-2 wrote:

>

> From: <Tom.Stock@gsbsc.gensig.com>

> Date: Mon, 23 Mar 1998 16:42:13 -0500

> Subject: Unpotting Neons (success)

>

>      I finished unpotting my first neon (15kV/30ma) last night, and it works!

>      4 more to go!

>      -tom

Anyone,

What is the usual temp to melt out tar?

After that, what does "chipping tar out" mean and is that usually

necessary?

Best disposal of old solvent? usually gasoline?

Please list common fail modes and usual means of detection; I assume

either the pri or one of the 2nds is open?

Or shorted internally ? or to case, which I would guess is rarest mode?

What do shunts look like and how would one make the decision to remove

any of them?  Do they function as I limiting?

Common forms of coil repair, methods?

Thanks!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning/Theory

Date: Tue, 24 Mar 1998 16:39:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Tue, 24 Mar 1998 11:39:48 -0500 (EST)

Subject: Re: Tuning/Theory

On Tue, 24 Mar 1998, Tesla-2 wrote:

> What is  the theory behind tuning?  Thanks, Walt Kulpa

>

You're changing the resonant frequency of the primary, which is basically

an LC oscillator.  The spark gap is used to produce a pulse of electricity

into the inductor(the primary coil) when the capacitors charge to a

certain voltage, you have to just the inductance of the primary, or the

capacitance of the capacitor to make it oscillate the secondary's resonant

frequency.  When you do that, you get a significant improvement in the

sparks.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Tue, 24 Mar 1998 17:00:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 24 Mar 1998 09:00:16 -0800

Subject: Re: Capacitance

Organization: Self

Tesla-2 wrote:

>

> From: "Jeff Corr" <corr@enid.com>

> Date: Mon, 23 Mar 1998 22:53:28 -0600

> Subject: Re: Capacitance

>

> how does that relate to Tesla Coils?  Oh yeah, I am getting 69" of spark! :)

>

Jeff,

One of my favorite dimensions!!

Brifly outline your setup; helps us newbies develop 'seat-of-the-pants'

on system perfomance...

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting Neons (success)

Cc:

Bcc:

X-Attachments:

At 09:42 PM 3/23/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Mon, 23 Mar 1998 16:42:13 -0500

>Subject: Unpotting Neons (success)

>

>

>     I finished unpotting my first neon (15kV/30ma) last night, and it

>     works! I chose not to remove any shunts this time, but will probably

>     try it on my next one.

Good job. You will find your rebuild will work better

and last longer than other neons.

For a 30 ma neon only two or three plates should be

removed from each shunt. This will give you around 40-45 ma.

Its difficult to push 30ma neons very much harder without

risking damage.

>

>     Overall, the whole thing isn't too difficult. Knocking the tar off

>     took me about an hour, cleaning off the rest took an hour, and

>     repairing the damage I did to the windings during unpotting took about

>     an hour. I tested the transformer last night and it works great.

The next one will be even easier.

>

>     Don't fool yourself though, it's not easy either. You will definately

>     get covered in tar and make a huge mess. Wear some type of plastic or

>     latex gloves if you can, otherwise be ready for dirt under your

>     fingernails for days. Don't even THINK about doing ANY this in the

>     house.

>

>     As soon as I get time I'll mount it in a case (full of oil) and make

>     it look pretty.

>     4 more to go!

>

>     -tom

For some ideas about mounting your neon in oil take a look

at the rebuild I did on my web page. The URL is:

http://www.misslink.net/mhammer/rebuild.htm

Also check out Adam Smith's page. He has mounted a rebuilt

neon in a paint can. Kind of looks like a mini pole pig. (piglet?)

The URL is: http://www.tiac.com/users/absmith/minipig.html

One other thing I want to mention is that you should

try to get the transformer as clean as possible before

putting it in oil. Otherwise the oil will dissolve any

remaining tar and turn the oil dark. Probably doesnt

hurt the oil any but it doesnt look as nice.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFGnd/litz wire

Cc:

Bcc:

X-Attachments:

I sent this previously but dont think it went out.

Sorry if this is a duplicate.

At 12:52 PM 3/22/98 GMT, you wrote:

>From: Hollmike <Hollmike@aol.com>

>Date: Sun, 22 Mar 1998 07:52:46 EST

>Subject: Re: RFGnd ?

>

>Hi All,

>   I had one question about the braided wire issue:   I have an old (1930's)

>EE handbook that describes the use of Litz wire in air core inductors.  It

>goes further to say that this braided type of wire is great for compensating

>for the skin effect due to the large surface area of the conductor.  It says

>that the braiding allows each wire to have equal exposure to the outside of

>the wire and therfore provides better conduction than twisted wire.  I don't

>remember if it stated numbers relative to magnet wire and will have to look

>again later.  My question is, has anyone used Litz wire and, if so, can you

>tell me if it has any value in your opinion.

>Mike Hollingsworth

>

Mike,

Litz wire is a thing entirely different altogether from braid, or multi

stranded cable.  Each individual parallel conductor in Litz is

separately insulated from all its neighbours.  Each strand is an

insulated fine solid copper magnet wire. The individual strands

are essentially in parallel. Litz is most effective at medium RF

frequencies. (100khz-1mhz)

It is rather hard to come by these days and I have only

a short 100 foot piece of it. It should be quite effective

at carrying RF currents.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil measurements: more comments

Date: Tue, 24 Mar 1998 18:14:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Tue, 24 Mar 1998 10:14:25 PST

Subject: Re: Coil measurements: more comments

Coilers,

>I am adding my comments to Ralph's comments.

>Hope this helps.

>Fr. Tom McGahee

Thanks to both Ralph and Fr. Tom for your feedback!  Here are my

comments:

>An oscilloscope is best at the detector end, as you can

>*see* things that don't show up on a DVM.

Yes - unfortunately I chose not to drag my old Tektronics Type 543B (B

is for BEAST) over the the coil site.  I didn't know if it would survive

the journey...

>> T>First I constructed an envelope detector using a 1N914

>> T>diode in series with a small commercial polypropylene

>Yes and no. It will work fine provided the voltage is much

>more than .7 volts. Using a Germanium such as a 1N34 would

>reduce that to about .4 volts, but any diode is going to

>introduce a voltage drop.

Yes - but I was unable to locate my germaniums.  They are around here

somewhere... :)

I took care and chose my drive levels such that the peak voltage read on

the meter was from 8 to 25 volts, for both the primary and secondary

measurements.  Thus I was well above the diode drop.

>The capacitor should be kept as SMALL as possible. This for

>several reasons. Formost is the fact that the detector circuit

>is not only going to load the circuit under test, but it will

>also contribute CAPACITANCE to the circuit under test. This

>in turn will shift the resonant frequency. All the more reason

>to use an oscilloscope and do away with the detector circuit.

>Of course, even a scope will add some capacitance. You can

>minimize this by putting a small capacitor such as 10 pf in

>series with the scope probe.

Hmm.  I also added a 400K across the cap - I found that the high

(>20Meg) input of the meter was too large - the cap would charge up but

would take quite sometime to discharge.

My cap was a 1.0 uf - which works out to 1e-6 * 400e3 = 0.4 second time

constant, which is of course much smaller than the frequency being

measured!

About the loading - yes it will load it, until the cap charges up.  The

diode will prevent the cap from actually resonating along with the DUT.

Once the envelope detector is charged up, and the current through the

diode is basically zero, then is should have a negligable effect on the

DUT.  Or am I deluding myself?

>> T>I wanted to measure the resonant frequency of

>> T>our primary circuit.  I tapped the primary at the usual

>> T>point, and shorted the gap.  I then attached my signal

>> T>generator (50-ohm output impedence) across the tank

>> T>capacitor.  Next the envelope detector was also

>> T>connected across the capacitor.

>>

>>   The generator should be somewhat isolated from the tank circuit, or it

>> will change the resonant frequency.  I generally drive the Device Under

>> Test (DUT) through a 100 ohm resistor.

OK - I hadn't considered this.

>>    If available, for a detector, I like to use an oscilloscope because

>> of its sensitivity.  You should connect your detector across a small

>> portion of the coil, for instance, between the end of the coil connected

>> to the low side of the signal generator and a turn or so further up the

>> coil.  This reduces the capacitive loading on the DUT.

>>

>You can also wind a separate coil of about 40 turns temporarily

>around the secondary coil and attach your scope to this third coil.

>This will reduce the tendency of the scope to load down the circuit

>and also reduces the capacitive problem. You get a lower output,

>but a more accurate picture as regards the resonant frequency.

But won't this "sense coil" (being closely coupled to the secondary)

load it down even worse than what I did?  Remember - this part of the

measurement was dealing with the primary LC - which should be less

sensitive to pf-type disturbances than the secondary would be...

>Be aware that the actual operating frequency will always be off,

>because when the TC is working you have ion clouds and the like

>that add to the capacitance, thus lowering the operating frequency.

This might explain why when we retapped our primary to match the

measured secondary frequency, we really didn't see an improvement in

arc-length.

Once again - thanks for your suggestions.  I might repeat this

experiment again in the near future.

As I had mentioned, I borrowed this equipment from work.  However, I

think I'd like to purchase a freq counter and generator.  Do you have

any suggestions as to what I should get?  I see that C&H Sales has a

digital freq counter for $95 or something, which seems like a reasonable

price.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Check out our huge pulse capacitor can crusher at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/bigcap.html

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: First coil questions

Date: Tue, 24 Mar 1998 21:31:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Tue, 24 Mar 1998 16:31:13 -0500

Subject: First coil questions

Ok, I've finished my secondary coil, which came out beautiful. It's

4.5" x 21.75" of #24 wire. I overshot a little, but decided to leave

it since it didn't make much difference in the end.

Question:

I read somewhere that you should NOT connect the bottom of your

secondary and the Neon power supply case together because of the

danger of introducing 60hz currents into your secondary coil. It was

apparently drawn this way on some older schematics.

My question is this: Does this mean that the Neon is grounded to my

house wiring ground, and the seconary to the RF ground? This doesn't

seem right to me, since there is the possibility of introducing 15kv

into my house ground!

Do they each need their own RF (copper pipes hammered into the ground)

ground?

Thanks!

-tom

P.S. I should be sparking by next week (I hope!)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting Neons (success)

Cc:

Bcc:

X-Attachments:

At 03:56 PM 3/24/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 24 Mar 1998 07:56:09 -0800

>Subject: Re: Unpotting Neons (success)

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: <Tom.Stock@gsbsc.gensig.com>

>> Date: Mon, 23 Mar 1998 16:42:13 -0500

>> Subject: Unpotting Neons (success)

>>

>>      I finished unpotting my first neon (15kV/30ma) last night, and it works!

>>      4 more to go!

>>      -tom

>

>Anyone,

>

>What is the usual temp to melt out tar?

Depends on the manufacturer. I found that Jefferson Electric (Magnetek)

neons contain a very hard brittle compound that melts at a rather

high temperature. Franceformer neons seem to have a softer lower

melting point compound.

You would need 250 degrees F or so to liquify the tar.

On stubborn ones up to 300 F may be needed. Only use as much

heat as necessary.

I do not recommend melting out the tar.

>After that, what does "chipping tar out" mean and is that usually

>necessary?

Chipping the tar away from the core is the alternative method

to melting out the tar and the method I recommend. The tar is

rather brittle and will often come loose in large sections.

Freezing the tar block before hand makes the potting compound

even more brittle.

>

>Best disposal of old solvent? usually gasoline?

I usually use kerosene. Its cheaper and much less dangerous

to have sitting around in an open container.

I chip away all the tar I can then dump the core into kerosene

for a couple days to remove the remaining tar. Then I disassemble

the core and dump any parts still needing cleaning into fresh

kerosene.

The old tar laden kerosene I dump on the road in front of my house.

The road is an oil and chip road which is tar and small stones.

A little extra tar does no harm here.

>

>Please list common fail modes and usual means of detection; I assume

>either the pri or one of the 2nds is open?

>Or shorted internally ? or to case, which I would guess is rarest mode?

The primaries virtually never fail. The number one failure mode

in neons is carbon tracking through the potting compound.

Internal arcing carbonizes a spot in the tar and encourages

further arcing. Soon the secondary shorts out. Often the secondary

resistance will check normal only to break down under load.

Unpotting the neon will restore it to full operation.

Usually only one of the secondary windings will be bad.

I've also often seen the HV leads or common connection

to the core simply broken off.

>

>What do shunts look like and how would one make the decision to remove

>any of them?  Do they function as I limiting?

The shunts vary from manufacturer to manufacturer. Some like Franceformer

have the shunts as metal blocks situated between the primary and secondary

windings. These blocks are made up of a number of thin plates.

On Jefferson Electric neons the shunts are actually part of the core.

The core has extensions between the primary and secondary windings.

You can safely remove 2 or 3 plates from each shunt in 30ma neons.

This will get you 40-45 ma.

60 ma neons are a lot more rugged and you can usually remove 4

or 5 shunts and get 90ma from them. Dont remove too many

or you risk damaging one of the secondary windings.

The shunts operate by short circuiting the magnetic flux

in the core away from the secondary windings. Only enough

flux reaches the secondaries to allow the rated current to flow.

You can short circuit a neon forever and only the rated current

will flow.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Pole Pig

Date: Wed, 25 Mar 1998 00:22:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Tue, 24 Mar 1998 19:22:26 EST

Subject: Pole Pig

All,

Hi, I am looking for a pole pig, can somebody help me find one.

Erik Schulz

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Unpotting Neons (success)

Date: Wed, 25 Mar 1998 01:00:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 24 Mar 1998 17:00:41 -0800

Subject: Re: Unpotting Neons (success)

When you repot neon, how do you get more power out of them?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unpotting Neons (success)

Cc:

Bcc:

X-Attachments:

At 01:00 AM 3/25/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Tue, 24 Mar 1998 17:00:41 -0800

>Subject: Re: Unpotting Neons (success)

>

>When you repot neon, how do you get more power out of them?

>

Unpotting the neon does not increase the power available

however once the potting compound is removed this gives

you a chance to reduce the number of magnetic shunts.

Neon sign transformers (NST) are internally current

limited via magnetic shunts. The shunts divert the magnetic

flux away from the secondaries. Only enough flux reaches the

secondaries for the rated current to flow. (30 ma or 60 ma).

By removing a couple of the shunt plates an increased

amount of magnetic flux will reach the secondaries

allowing for increased current flow.

Since the secondary windings in NSTs is very small wire

(AWG 32-34 in 30 ma neons) to much current will easily

burn a winding up. You can get a reliable 40-45 ma from

a 30 ma neon and up to 90 ma from a 60ma neon.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: First coil questions

Date: Wed, 25 Mar 1998 07:42:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Wed, 25 Mar 1998 07:42:41

Subject: Re: First coil questions

At 09:31 PM 3/24/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Tue, 24 Mar 1998 16:31:13 -0500

>Subject: First coil questions

>

>

>     Ok, I've finished my secondary coil, which came out beautiful. It's

>     4.5" x 21.75" of #24 wire. I overshot a little, but decided to leave

>     it since it didn't make much difference in the end.

>

>     Question:

>

>     I read somewhere that you should NOT connect the bottom of your

>     secondary and the Neon power supply case together because of the

>     danger of introducing 60hz currents into your secondary coil. It was

>     apparently drawn this way on some older schematics.

>

>     My question is this: Does this mean that the Neon is grounded to my

>     house wiring ground, and the seconary to the RF ground? This doesn't

>     seem right to me, since there is the possibility of introducing 15kv

>     into my house ground!

>

>     Do they each need their own RF (copper pipes hammered into the ground)

>     ground?

>

>

Tom,

Most of today's neon sign transformers are 'mid -point grounded' which

means the neon sign casing is ground and the center tap of the neon

secondary is ground as well. If you are using a 15kv tranny then from

neon's case to each HV leg, you will have a potential of 7,500 volts

but from HV leg to Hv leg you will have the 15,000 volt potential.

The neon sign transformer casing (ground) I attach to the RF ground

at the bottom of my secondary which goes to a ground rod outside.

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Wed, 25 Mar 1998 08:14:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 25 Mar 1998 00:14:23 -0800

Subject: Re: RFGnd ?

Organization: Self

Tesla-2 wrote:

>

> From: Scott MacGregor <smacgregor@aasp.net>

> Date: Sun, 22 Mar 1998 16:43:36 -0800

> Subject: Re: RFGnd ?

> Organization: EES,Co.

>

> Tesla-2 wrote:

> >

> > From: Hollmike <Hollmike@aol.com>

> > Date: Sun, 22 Mar 1998 07:52:46 EST

> > Subject: Re: RFGnd ?

> >

> > Hi All,

> >    I had one question about the braided wire issue:   I have an old (1930's)

> > EE handbook that describes the use of Litz wire..  My question is, has anyone

> > used Litz wire and, if so, can you tell me if it has any value in your opinion.

> > Mike Hollingsworth

>

> I can tell you all I know about Litz wire in a few sentences. It is

> designed to be very flexable (for applications such as earphones) and it

> is very expensive. From what I remember, it is constructed as follows: A

> small number of thin flat copper conductors are wound in a spiral

> fashion around a cotton core. Soldering is difficult. I'm not familiar

> with its electrical properties.

>

> Regards,

> Scott MacGregor EES,Co.

> ************************************************************

> Enterprise Engineering Services Co.

> Member: Cable Television Advisory Committee, Plainville, MA

> http:/www.expage.com/page/cableguy

> ************************************************************

I have come across the wire described above, in OLD headsets and in

MaBell gear pre-40's, test leads and hand set stuff.. if I'm not

mistaken... very frail in re-wireing situations... but, flexible in

extremis.

So, what is LITZ, in actuality?  We now have about 2-3 various

descriptions!

Parallel-- twisted-- mag wire-- flat strips twisted--???

Does anyone really care? It doesn't sound very usefull for our work!

The parallel mag wire form may be usefull in RFGnd appl.

What is the effect of twisting stands vs straight or non-twisted

conductors? I recall that 'twisted' conductors were good at thwarting

induced noise and interference...

BRAID is used as shielding in in most RGU-type co-ax HF wire... Most

Belden-type LF signal wire is shielded with an aluminized mylar or some

other polymer.

What's really happening here?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RFGnd ?

Date: Wed, 25 Mar 1998 17:19:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Wed, 25 Mar 1998 09:19:10 -0800

Subject: Re: RFGnd ?

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Wed, 25 Mar 1998 00:14:23 -0800

> Subject: Re: RFGnd ?

> Organization: Self

>

> Tesla-2 wrote:

> >

> > From: Scott MacGregor <smacgregor@aasp.net>

> > Date: Sun, 22 Mar 1998 16:43:36 -0800

> > Subject: Re: RFGnd ?

> > Organization: EES,Co.

> >

> > Tesla-2 wrote:

> > >

> > > From: Hollmike <Hollmike@aol.com>

> > > Date: Sun, 22 Mar 1998 07:52:46 EST

> > > Subject: Re: RFGnd ?

> > >

> > > Hi All,

> > >    I had one question about the braided wire issue:   I have an old (1930's)

> > > EE handbook that describes the use of Litz wire..  My question is, has anyone

> > > used Litz wire and, if so, can you tell me if it has any value in your opinion.

> > > Mike Hollingsworth

> >

> > I can tell you all I know about Litz wire in a few sentences. It is

> > designed to be very flexable (for applications such as earphones) and it

> > is very expensive. From what I remember, it is constructed as follows: A

> > small number of thin flat copper conductors are wound in a spiral

> > fashion around a cotton core. Soldering is difficult. I'm not familiar

> > with its electrical properties.

> >

> > Regards,

> > Scott MacGregor EES,Co.

> > ************************************************************

> > Enterprise Engineering Services Co.

> > Member: Cable Television Advisory Committee, Plainville, MA

> > http:/www.expage.com/page/cableguy

> > ************************************************************

> I have come across the wire described above, in OLD headsets and in

> MaBell gear pre-40's, test leads and hand set stuff.. if I'm not

> mistaken... very frail in re-wireing situations... but, flexible in

> extremis.

> So, what is LITZ, in actuality?  We now have about 2-3 various

> descriptions!

> Parallel-- twisted-- mag wire-- flat strips twisted--???

Gene, I have a bunch of old (20's-30's)electronics mags....I think I'll

see if Litz wire is mentioned anywhere. As I said, though, it's a pain

to solder and strip.

> Does anyone really care? It doesn't sound very usefull for our work!

I dont't think it would be available in heavy enough gages to be useful

in T-coil applications.

> The parallel mag wire form may be usefull in RFGnd appl.

>

> What is the effect of twisting stands vs straight or non-twisted

> conductors? I recall that 'twisted' conductors were good at thwarting

> induced noise and interference...

M.H. says to use solid ground wire. I have seen many applications where

braid is used. It's frequency dependent.

> BRAID is used as shielding in in most RGU-type co-ax HF wire... Most

> Belden-type LF signal wire is shielded with an aluminized mylar or some

> other polymer.

I think the bottom line here is this: At HF, a good equipment ground

(for safety reasons) may not be a good RF ground, especially if its

length is some even fraction of a wavelength.(1/4,1/2, etc.) It is best

to keep the RF ground as short as possible. Of course, at f=400kHz, your

ground would have to be hundreds of feet long to run into this problem.

The other thing I agree with M.H. on, is that flat copper strips are

probably the best overall material. Say, 4" wide by .030 thick.

>

> What's really happening here?

>

> Gene Waldo

>

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Thoughts on making choke cores.

Date: Wed, 25 Mar 1998 18:01:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Wed, 25 Mar 1998 11:01:20 -0700 (MST)

Subject: Re: Thoughts on making choke cores.

Bob Schumann wrote:

>>I have been trying to come up with a better, simpler and cheaper way to

>>have some good beefy chokes and this is my newest thought:

>>The toroid part would be one of those plastic 'doughnuts' that you see

>>in the toy store for small infants. We have all seen them, they are about

>>4 to 5 inches in diameter and some have bigger inside 'doughnut holes' than others.

>>Some sort of opening would have to be made in the toroid for it to be filled.

>>What to fill it with? This is something I am thinking on as well. You can

>>go to any automotive shop that 'turns' brake rotors and find a goldmine of

>>fine iron shavings that have come off the brake rotors. Using a sieve, you can

>>collect the smallest particles. I am still trying to decide what makes a good

>>compound to mix with these iron shavings and the best I have come up with is

>>plaster of paris. Mixing the plaster with the shavings will yeild a compound to fill

>>the plastic doughnut with.

>>Neccessity is the mother of invention.

Aside from beginning to make Tesla Coils, I have much interest in amateur

rocketry. Although I have never done it, I have lots of information on how

to make solid composite rocket motors. Basically the idea is for the

"propellants" (the fuel and oxidizer) to me mixed in with binder that holds

everything together. I don't want to digress, so I'll get to the point. I

wonder if powdered iron or something could replace the rocket propellants?

Then the composition could be pumped into the "donut hole" toys that Bob

Schumann suggests. The "binder" is actually of two parts: a plastic

material (PVC, PBAN, HTPB, etc.) mixed with small amount of a curing agent.

The result is a substance similar to epoxy. In rockets, the composition is

approximately 20% binder mixture, 25% magnesium and 55% oxidizer. If one

was lucky maybe you could get 80% of the composition to be metallic by

replacing the fuel and oxidizer with powdered iron. Any comments?

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary wire

Date: Wed, 25 Mar 1998 20:03:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Wed, 25 Mar 1998 14:03:12 -0600

Subject: Secondary wire

Hello,

I'm collecting parts for my first small Tesla coil and I was wondering

if the magnet wire used to wind the secondary can be spliced?  I have

some #24 wire but it's only in 100 and 200 foot lengths.  Will splicing

degrade performance in some way????

Thanks,

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Chokes

Date: Wed, 25 Mar 1998 20:37:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 25 Mar 1998 15:37:27 -0500

Subject: Chokes

Can I make my own choke doughnuts by winding some sort of steel wire

into a doughnut shape?

How about a large (2") nut?

I saw someone else post this question and didn't see a reply.

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Good Price for Powerstat?

Date: Wed, 25 Mar 1998 20:53:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: BOARDMAN@ravel.unm.edu

Date: Wed, 25 Mar 1998 13:53:48 -0700

Subject: Good Price for Powerstat?

A local surplus place has some old Powerstat variable AC transformers,

both 29 amp and 45 amp versions.  I'd like some idea what a reasonable

offering price might be for one of these?  They have a 29 amp model that's

mounted on a 19" rack plate, with volt & amp meters, marked $100 - is this

a good deal?  I know this is a lot more amps than I'm going to need for

my coil, but who knows what the future might hold :-).

-Bob

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Welder as Variac?

Date: Wed, 25 Mar 1998 22:09:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Wed, 25 Mar 1998 16:09:56 -0600

Subject: Welder as Variac?

I have one of those $100.00 AC welders you might get from Sears or Wards

on sale.  Would it be useable as a source of adjustable AC voltage for a

future medium/large Tesla coil I might build??

Thanks,

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary wire

Cc:

Bcc:

X-Attachments:

At 08:03 PM 3/25/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Wed, 25 Mar 1998 14:03:12 -0600

>Subject: Secondary wire

>

>Hello,

>

>I'm collecting parts for my first small Tesla coil and I was wondering =

>if the magnet wire used to wind the secondary can be spliced?  I have =

>some #24 wire but it's only in 100 and 200 foot lengths.  Will splicing =

>degrade performance in some way????

>

>Thanks,

>

>Roy Minnich

>

Roy,

Its preferable not to do so but it can be done.

In fact my older 3 inch diameter secondary has a splice

and it worked well.

Try to make the splice as smooth and as flat as possible.

Covering the splice with a dab of epoxy once its wound on

the form would be a good idea. The idea is to prevent having

a bump or point where corona or sparks could form.

If you have a choice try to get the splice nearest the bottom

end of the secondary where the voltage is lower.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Chokes

Cc:

Bcc:

X-Attachments:

At 08:37 PM 3/25/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Wed, 25 Mar 1998 15:37:27 -0500

>Subject: Chokes

>

>

>     Can I make my own choke doughnuts by winding some sort of steel wire

>     into a doughnut shape?

>

>     How about a large (2") nut?

>

>     I saw someone else post this question and didn't see a reply.

>

>     -tom

Most common forms of steel would not make a suitable

core. It would saturate easily and have high losses

due to eddy currents.

I would simply make some air core chokes on 2 inch

PVC pipe. Air core chokes will not saturate and have

low losses. In fact the losses are too low so some

series resistance is recommended.

A 6 inch long winding of AWG 28 would have about

430 turns and 3.7 mh of inductance. Add a 300-1500 ohm

power resistor in series with each choke to dampen

any oscillations that may occur.

The choke inductance is not critical. Anything from

2-10 mh.

A toroidal inductor offers no particular advantage

other than compactness and a degree of self shielding.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Good Price for Powerstat?

Cc:

Bcc:

X-Attachments:

At 08:53 PM 3/25/98 GMT, you wrote:

>From: BOARDMAN@ravel.unm.edu

>Date: Wed, 25 Mar 1998 13:53:48 -0700

>Subject: Good Price for Powerstat?

>

>

>    A local surplus place has some old Powerstat variable AC transformers,

>  both 29 amp and 45 amp versions.  I'd like some idea what a reasonable

>  offering price might be for one of these?  They have a 29 amp model that's

>  mounted on a 19" rack plate, with volt & amp meters, marked $100 - is this

>  a good deal?  I know this is a lot more amps than I'm going to need for

>  my coil, but who knows what the future might hold :-).

>

>  -Bob

>

A 29 amp Powerstat in a 19 inch rack mount WITH voltage and current

panel meters for $100? That a pretty good deal. A ready made control

cabinet with plenty of room for the future.

A good thing you didnt say exactly where this place was.

I would have already been there and bought it by the time

you read this. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Secondary wire

Date: Thu, 26 Mar 1998 02:42:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Wed, 25 Mar 1998 18:42:24 -0800

Subject: Re: Secondary wire

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

> Date: Wed, 25 Mar 1998 14:03:12 -0600

> Subject: Secondary wire

>

> Hello,

>

> I'm collecting parts for my first small Tesla coil and I was wondering

> if the magnet wire used to wind the secondary can be spliced?  I have

> some #24 wire but it's only in 100 and 200 foot lengths.  Will splicing

> degrade performance in some way????

>

> Thanks,

>

> Roy Minnich

I'm no expert on T-coils, but I can tell you from experience with high

voltage transformers that any sharp points, knicks, or insulation damage

is likely to produce a discharge point that will fail sooner or later.

Considering the relatively low price of magnet wire, and the amount of

time your going to spend wrapping your coil, I'd opt for nice new wire!

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: measuring tools

Date: Thu, 26 Mar 1998 02:42:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE D <gordrew@home.com>

Date: Wed, 25 Mar 1998 18:42:51 -0800

Subject: measuring tools

I am building and plan on using my Tesla Coil for the basis of my science

project.

What kind of tools are available that would measure the output power of the

coil? or are there any ways to determine this mathamatically?

--MIKE

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Welder as Variac?

Cc:

Bcc:

X-Attachments:

At 10:09 PM 3/25/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Wed, 25 Mar 1998 16:09:56 -0600

>Subject: Welder as Variac?

>

>I have one of those $100.00 AC welders you might get from Sears or Wards

>on sale.  Would it be useable as a source of adjustable AC voltage for a

>future medium/large Tesla coil I might build??

>

>Thanks,

>

>Roy Minnich

Roy,

A welder is used as a current limiting device not to provide

variable voltage. Neons require no current limiting as they

contain internal shunts.

The current limiting is needed when using power company distribution

transformers (pole pig) or other non limited transformers.

In this case you not only need the welder to inductively limit

current but a large variac to vary the input voltage.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Chokes

Date: Thu, 26 Mar 1998 06:52:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bob Schumann <tesla@america.com>

Date: Thu, 26 Mar 1998 06:52:27

Subject: Re: Chokes

At 08:37 PM 3/25/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Wed, 25 Mar 1998 15:37:27 -0500

>Subject: Chokes

>

>

>     Can I make my own choke doughnuts by winding some sort of steel wire

>     into a doughnut shape?

>

>     How about a large (2") nut?

>

>     I saw someone else post this question and didn't see a reply.

>

>     -tom

>

>

Tom,

I believe that the reason for having an iron core to a choke

is partly due to the fact that the inductance increases far

over that of air core. I think that if you used just plain

steel or iron though that the choke might go to saturation

during usage. I don't think this would happen with powdered

iron core. I guess it may be possible to use a large nut but

maybe cut a slice through it would act as a shunt somewhat

and keep the choke from saturation. Don't know for sure....

When I can say I am an expert, it is time for me to stop

this hobby :)

Bob Schumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Chokes

Date: Thu, 26 Mar 1998 13:34:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Patrick Moser <lrichard@netsync.net>

Date: Thu, 26 Mar 1998 13:34:43 +0000

Subject: Chokes

If your looking for choke cores  checkout

Electronic Goldmine www.goldmine-elec.com

they have 2 1/8" x 7/16" x 5/8" ferrite toriods for $1.50

plus a ton of other cool stuff go to the site and request a catalog

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Welder as Variac?

Date: Thu, 26 Mar 1998 15:28:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Thu, 26 Mar 1998 07:28:08 PST

Subject: Re: Welder as Variac?

It was written:

>I have one of those $100.00 AC welders you might get from Sears or Wards

>on sale.  Would it be useable as a source of adjustable AC voltage for a

>future medium/large Tesla coil I might build??

Probably not directly.  These welders typically produce about 40-70

volts of output, at a selectable current.

Most coilers use these "tombstone" welders as current limiting for

running polepigs and the like.  You basically wire the 240 input of the

welder in series with your pig.  Shorting the welding cables and

selecting a current will limit the primary current flowing into the pig.

It *might* be possible to add your own high voltage windings to the

welding transformer.  If you could do this, you wouldn't need a pig.

The difficulty will be in maintaining sufficient insulation, etc.  But

passing a spool of wire through the core some 15000 times isn't my ideal

of a good time, either.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Wire Size & Cap Oil

Date: Thu, 26 Mar 1998 15:42:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "William R. Langston" <blangsto@iwvisp.com>

Date: Thu, 26 Mar 1998 07:42:48 -0800

Subject: Re: Wire Size & Cap Oil

Hello All,

I tried to send these questions once before but must have messed the send

up somehow. Well here they are again (I would rather hear from someone who

already knows than learn by wasting hours and dollars in self education).

My son and I are building our first serious attempt at a TC. We were able

to lay hands on 6" PVC  and lots of 24 gauge wire (from gweaver) for the

secondary, but in reading materials here we see that 22 gauge is "the

preferred wire size for a 6 coil"; why? Will a 24 gauge, 24" winding on 6" PVC

work greatly different than a 22 gauge coil?

Also, I was given about 12 gals of high-grade turbine oil (no impurities),

is that ok for caps?

Thanks for all the information on this list.

Bill & Peter

Langston, "blangsto@iwvisp.com"

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil measurements: even MORE comments

Date: Thu, 26 Mar 1998 16:34:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 26 Mar 1998 11:34:02 -0500

Subject: Re: Coil measurements: even MORE comments

Comments to the comments to the comments:)

All kinds of snips....

>

> >> T>First I constructed an envelope detector using a 1N914

> >> T>diode in series with a small commercial polypropylene

>

> >Yes and no. It will work fine provided the voltage is much

> >more than .7 volts. Using a Germanium such as a 1N34 would

> >reduce that to about .4 volts, but any diode is going to

> >introduce a voltage drop.

>

> Yes - but I was unable to locate my germaniums.  They are around here

> somewhere... :)

>

> I took care and chose my drive levels such that the peak voltage read on

> the meter was from 8 to 25 volts, for both the primary and secondary

> measurements.  Thus I was well above the diode drop.

Yep, that should be fine. The problem is that some people try doing

this with a signal generator that only puts out a fraction of a

volts. The older signal generators tend to have larger output

voltages.

>

> >The capacitor should be kept as SMALL as possible. This for

> >several reasons. Formost is the fact that the detector circuit

> >is not only going to load the circuit under test, but it will

> >also contribute CAPACITANCE to the circuit under test. This

> >in turn will shift the resonant frequency. All the more reason

> >to use an oscilloscope and do away with the detector circuit.

> >Of course, even a scope will add some capacitance. You can

> >minimize this by putting a small capacitor such as 10 pf in

> >series with the scope probe.

>

> Hmm.  I also added a 400K across the cap - I found that the high

> (>20Meg) input of the meter was too large - the cap would charge up but

> would take quite sometime to discharge.

>

> My cap was a 1.0 uf - which works out to 1e-6 * 400e3 = 0.4 second time

> constant, which is of course much smaller than the frequency being

> measured!

Hah! With 1 mfd it is no wonder you needed 400K across the cap. Try something

like .001 mfd in the future. Unless you have all of your .001 caps

hiding out with your Germanium diodes :)

>

> About the loading - yes it will load it, until the cap charges up.  The

> diode will prevent the cap from actually resonating along with the DUT.

> Once the envelope detector is charged up, and the current through the

> diode is basically zero, then is should have a negligable effect on the

> DUT.  Or am I deluding myself?

What you say is correct, but there is ALWAYS a net capacitive increase

whenever you hook anything up to the secondary. This is less of a problem

with the primary, but the *secondary* capacitance can be easily swamped by

the added capacitance of even just a scope probe.

>

> >>

> >You can also wind a separate coil of about 40 turns temporarily

> >around the secondary coil and attach your scope to this third coil.

> >This will reduce the tendency of the scope to load down the circuit

> >and also reduces the capacitive problem. You get a lower output,

> >but a more accurate picture as regards the resonant frequency.

>

> But won't this "sense coil" (being closely coupled to the secondary)

> load it down even worse than what I did?  Remember - this part of the

> measurement was dealing with the primary LC - which should be less

> sensitive to pf-type disturbances than the secondary would be...

>

Sorry.... my mind was fixed on secondary loading problems. Sensing

direct off the *primary* is no problem. Here the main cap will

swamp any additional capacitance, so there is no problem.

> >Be aware that the actual operating frequency will always be off,

> >because when the TC is working you have ion clouds and the like

> >that add to the capacitance, thus lowering the operating frequency.

>

> This might explain why when we retapped our primary to match the

> measured secondary frequency, we really didn't see an improvement in

> arc-length.

>

Once I have a coil reasonably in tune by tapping to the nearest

whole turn, I then slide my tap around to determine the EXACT

tune point. With a high Q system you would be suprised at how

much extra arc you can get by fine tuning.

> Once again - thanks for your suggestions.  I might repeat this

> experiment again in the near future.

>

> As I had mentioned, I borrowed this equipment from work.  However, I

> think I'd like to purchase a freq counter and generator.  Do you have

> any suggestions as to what I should get?  I see that C&H Sales has a

> digital freq counter for $95 or something, which seems like a reasonable

> price.

>

Hamfests are good places to get these sort of items at rock bottom

prices. My own "must have" test equipment includes:

Digital Volt/Ohm Meter

Capacitance/Inductance Meter

Digital Frequency counter

Oscilloscope

You can get by without some of these items, but it makes things

much easier when you have each of the above. If I was reduced to

having just one item I would make it the oscilloscope. Next up

would be the DVM. Then the Frequency meter. Then the Cap/Inductance

meter.

Hope this helps.

Fr. Tom

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wire Size & Cap Oil

Cc:

Bcc:

X-Attachments:

At 03:42 PM 3/26/98 GMT, you wrote:

>From: "William R. Langston" <blangsto@iwvisp.com>

>Date: Thu, 26 Mar 1998 07:42:48 -0800

>Subject: Re: Wire Size & Cap Oil

>

>Hello All,

>

>    I tried to send these questions once before but must have messed the send

>up somehow. Well here they are again (I would rather hear from someone who

>already knows than learn by wasting hours and dollars in self education).

>

>    My son and I are building our first serious attempt at a TC. We were able

>to lay hands on 6" PVC  and lots of 24 gauge wire (from gweaver) for the

>secondary, but in reading materials here we see that 22 gauge is "the

>preferred wire size for a 6 coil"; why? Will a 24 gauge, 24" winding on 6" PVC

>work greatly different than a 22 gauge coil?

Either will work. The idea is to keep the H/D ratio of the secondary

winding around 4 to 1 for a 6 inch coil and maybe up to 5 to 1

for a 4 incher. This gives you the winding length. Then select

the largest guage of wire that will result in from about 700-1000

turns. Use the largest guage to get the highest Q and lowest loss.

Remember that the OD of 6 inch PVC is 6.5 inches not 6. So

at 4 to 1 thats a 26 inch winding. With AWG 24 that results

in quite a few turns and higher resistive losses. The same

winding length with AWG 22 results in lower losses and higher Q.

Also remember that you must match resonant frequencies.

A winding with many more turns results in a lower frequency.

No problem in that but your primary components need to be

able to tune to this low frequency.

Also the larger the coil the higher the cost and the larger

the problems always seem to be.

The rules are only general guidelines and you can vary quite

a bit from the ideal and still have a workable coil.

Since you have lots of AWG 24 I recommend building a 4 incher first.

This is a good size coil that will work efficiently at low - medium

power levels. My own 4 inch coil gives 54 inch streamers at around

1200 watts.

Make the winding 20-22 inches long. Mine was 21 inches.

Prep the PVC pipe before hand by drying in a low temp (130 F)

oven for 30-45 minutes. If you cant use the oven heat the

pipe up with a heat gun or hair dryer. Keep it nice and

toasty for 30-45 minutes to drive out the moisture.

Seal the PVC with polyurethane varnish. Inside and out.

I recommend one coat inside and two outside. Apply the

varnish while the PVC is still warm. As it cools it will

draw the varnish into the pores and effectively seal it

against moisture. Drying and sealing the PVC this way

will get you a few extra inches of spark for almost free.

>

> Also, I was given about 12 gals of high-grade turbine oil (no impurities),

>is that ok for caps?

Possibly. Some types of turbine oil are virtually identical

to transformer oil. It really depends on its composition

and additives. Since you have it I would give it a try.

Make a small test cap first to test this oil before you go to the

effort to build a larger cap.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary and copper

Date: Thu, 26 Mar 1998 22:30:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Thu, 26 Mar 1998 17:30:04 EST

Subject: Primary and copper

Too all,

The following is a list of the 75% current depth in copper at different

frequency.

kHz, mils         mil is 0.001 inches

1000, 2.6

900, 2.7

800, 2.9

700, 3.1

600, 3.4

500, 3.7

400, 4.1

300, 4.7

200, 5.8

150, 6.7

100, 8.2

75,  9.5

50,  11.6

25,  16.4

"Share what you know, otherwise you wouldn't know it."

Erik Schulz

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: non ferrous chokes

Date: Fri, 27 Mar 1998 04:05:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 26 Mar 1998 20:05:00 -0800

Subject: Re: non ferrous chokes

Would a choke be effective if the core was made from a non magnetic metal such

as copper or aluminum?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: krazy carpet capacitors

Date: Fri, 27 Mar 1998 04:09:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 26 Mar 1998 20:09:03 -0800

Subject: Re: krazy carpet capacitors

Let me know if you want plans, I will have plans ready in around a week

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: Wire Size & Cap Oil

Date: Fri, 27 Mar 1998 12:21:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 27 Mar 1998 06:21:52 -0600

Subject: Re: Re: Wire Size & Cap Oil

Organization: MailExcite  (http://www.mailexcite.com:80)

Coilers,

I hope your not talking about Gas Turbine oil. I worked as a aerospace engineer

for 10 years. My responsibilites included bearings seals and drives. The lube oil

in jet aircraft engines is very caustic. It will not attack metal! It will however

attack anything organic. It will even attack the black top. These oils must withstand

temperatures as 380 - 400 F. The additives in the oil are anti-coking agents.

Be careful !

---

Paul M.

Einigkeit und Freiheit

>From: Mike Hammer <mhammer@misslink.net>

>Date: Thu, 26 Mar 1998 11:32:16 -0600

>Subject: Re: Wire Size & Cap Oil

>

>At 03:42 PM 3/26/98 GMT, you wrote:

>>From: "William R. Langston" <blangsto@iwvisp.com>

>>Date: Thu, 26 Mar 1998 07:42:48 -0800

>>Subject: Re: Wire Size & Cap Oil

>>

>>Hello All,

>>

>>    I tried to send these questions once before but must have messed the send

>>up somehow. Well here they are again (I would rather hear from someone who

>>already knows than learn by wasting hours and dollars in self education).

>>

>>    My son and I are building our first serious attempt at a TC. We were able

>>to lay hands on 6" PVC  and lots of 24 gauge wire (from gweaver) for the

>>secondary, but in reading materials here we see that 22 gauge is "the

>>preferred wire size for a 6 coil"; why? Will a 24 gauge, 24" winding on 6" PV

C

>>work greatly different than a 22 gauge coil?

>

>Either will work. The idea is to keep the H/D ratio of the secondary

>winding around 4 to 1 for a 6 inch coil and maybe up to 5 to 1

>for a 4 incher. This gives you the winding length. Then select

>the largest guage of wire that will result in from about 700-1000

>turns. Use the largest guage to get the highest Q and lowest loss.

>Remember that the OD of 6 inch PVC is 6.5 inches not 6. So

>at 4 to 1 thats a 26 inch winding. With AWG 24 that results

>in quite a few turns and higher resistive losses. The same

>winding length with AWG 22 results in lower losses and higher Q.

>

>Also remember that you must match resonant frequencies.

>A winding with many more turns results in a lower frequency.

>No problem in that but your primary components need to be

>able to tune to this low frequency.

>

>Also the larger the coil the higher the cost and the larger

>the problems always seem to be.

>

>The rules are only general guidelines and you can vary quite

>a bit from the ideal and still have a workable coil.

>

>Since you have lots of AWG 24 I recommend building a 4 incher first.

>This is a good size coil that will work efficiently at low - medium

>power levels. My own 4 inch coil gives 54 inch streamers at around

>1200 watts.

>

>Make the winding 20-22 inches long. Mine was 21 inches.

>Prep the PVC pipe before hand by drying in a low temp (130 F)

>oven for 30-45 minutes. If you cant use the oven heat the

>pipe up with a heat gun or hair dryer. Keep it nice and

>toasty for 30-45 minutes to drive out the moisture.

>

>Seal the PVC with polyurethane varnish. Inside and out.

>I recommend one coat inside and two outside. Apply the

>varnish while the PVC is still warm. As it cools it will

>draw the varnish into the pores and effectively seal it

>against moisture. Drying and sealing the PVC this way

>will get you a few extra inches of spark for almost free.

>

>

>

>>

>> Also, I was given about 12 gals of high-grade turbine oil (no impurities),

>>is that ok for caps?

>

>Possibly. Some types of turbine oil are virtually identical

>to transformer oil. It really depends on its composition

>and additives. Since you have it I would give it a try.

>

>Make a small test cap first to test this oil before you go to the

>effort to build a larger cap.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Low and high power coil operations

Date: Fri, 27 Mar 1998 15:10:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 27 Mar 1998 09:10:16 -0600

Subject: Low and high power coil operations

Organization: MailExcite  (http://www.mailexcite.com:80)

Last week I powered my 4" coil with almost 8 kw of power from my distribution

transformer. It was a success ! At least for a while. The spark gap didnt last

too long.  Aside from that it was pretty cool. The streamers were very bright but

not a lot longer.

I recently purchased a 15 kv 30 ma neon. I'm thinking about running some low p

ower experiments. Does anyone have an idea as to what I should expect. The four inch

coil usually produces streamers in the 50 - 54" range at 2.5 - 3 kw and occasionally

hits the 60" mark. I know the capacitance will have to be reduced. I currently run a

bout .026 uf. I guess I should run about .0053 uf for the 30 ma. Is this correct ?

Paul M

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Low and high power coil operations

Cc:

Bcc:

X-Attachments:

At 03:10 PM 3/27/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 27 Mar 1998 09:10:16 -0600

>Subject: Low and high power coil operations

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>Last week I powered my 4" coil with almost 8 kw of power from my

>distribution transformer.

>It was a success ! At least for a while. The spark gap didnt last too long.

Want to elaborate it on that? Was it just the gap overheating

and performance dropping off or was the gap actually damaged?

>Aside from that it was pretty cool. The streamers were very bright but not a

>lot longer.

You will find that modest increases on power dont yield too much

increase in spark length. In fact doubling your power level

may only add a few inches.

I would think its about time to build that 6 incher dont you think?

Your 4 seems pretty well maxed out.

>I recently purchased a 15 kv 30 ma neon. I'm thinking about running some

>low power experiments.

Good idea. You will learn far more from trying to maximize your output

with a fixed input.

>Does anyone have an idea as to what I should expect. The four inch

>coil usually produces streamers in the 50 - 54" range at 2.5 - 3 kw

>and occasionally hits the 60" mark.

You should get 18 inches easily and with careful attention to efficiency

24 inches should not be too difficult. I managed 22 inches on my 4 incher

with a single 12kv/30.

>I know the capacitance will have to be reduced. I currently run about

>.026 uf. I guess I should run about .0053 uf for the 30 ma. Is this correct ?

Yes. For 60 hz resonance you want .0053 for use with a 15kv/30ma.

Good luck and keep us posted.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Low and high power coil operations

Date: Fri, 27 Mar 1998 16:58:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Fri, 27 Mar 1998 08:58:09 PST

Subject: Re: Low and high power coil operations

Paul

You wrote:

>I receintly purchased a 15 kv 30 ma neon. I'm thinking about running

>some low power experiments. Does anyone have an idea as to what I should

>expect. The four inch .026 uf. I guess I should run about .0053 uf for

>the 30 ma. Is this correct ?

I confirm your calculation of 0.0053 uf for theoretical resonant

charging.

This is computed via C = I/(2pi*f*v) where V = volts, I = short circuit

current, F = frequency in Hz, and C = capacitance in Farads.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Small TC?

Date: Fri, 27 Mar 1998 17:02:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 27 Mar 1998 09:02:52 -0800

Subject: Small TC?

Organization: Self

All,

Has anyone worked out the general specs for a small Tesla Coil?

Something with about a 1' lg. 2ndry winding?  It might be fun to put

something like this together, while we are scrounging and building the

big one!

What kinds of performance could be expected?  Might it even employ shelf

caps?

Also, I have seen a reference to the inclusion of an oil-immersed large

mechanical tuning capacitor in the primary cap ckt.  But no one has

reported on the actualities of such....   More

'fine-tune-on-the-fly'....

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Rebuilding our static gap

Date: Fri, 27 Mar 1998 18:07:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Fri, 27 Mar 1998 10:07:38 PST

Subject: Rebuilding our static gap

Paul wrote:

>>Last week I powered my 4" coil with almost 8 kw of power from my

>>distribution transformer.

>>It was a success ! At least for a while. The spark gap didnt last too long.

Then Mike wrote:

>Want to elaborate it on that? Was it just the gap overheating

>and performance dropping off or was the gap actually damaged?

Performance was OK, but we saw sparks of molten metal blowing off of the

gap itself.  We are running an RQ gap in "blow-mode".  Holes cut in the

PVC parallel to the arc let the hot air out, etc.

The gap consisted of 10 copper pipes, I think 3/4 inch and 2 inches

long.  Initial spacing was 0.028 inch (one hacksaw blade), constructed

inside a 6 inch heavywall PVC pipe.

Due to numerous cleanings with emery paper, the actual gap was

substantially larger, as witnessed by the fact we had to tap-down and

use fewer gaps, etc.

The problem was that the connection between the bolts (which run through

the copper) and the copper itself had come loose, probably due the the

thermal expansion and contraction.  This was aggravated by Mr.

Fleischwulf not tightening the wire lugs down before running!  Isn't

that right, Paul?  :)

Anyway, I've completely disassembled the thing, and brazed new brass

bolts to the copper pipe pieces.  This will solve one of the problems

with the heat buildup caused by arcing between the bolt and the copper.

The PVC is severely yellowed, due to (I suppose) all the UV light

produced by the arc.  Areas of the PVC which were protected by epoxy are

unharmed.  Several areas of PVC under the copper were burned due to the

heat.

Tonight I'll re-epoxy the pipes in, and with luck will be running this

weekend!  I hope the pipes aren't so worn as to require much work in

re-establishing the proper spacing...

I would suggest that people who build these try to solder or braze (if

you have the means) the connection bolts directly to the copper pipes.

Another approach is to attach another piece of copper wire to the copper

pipes, and not use the bolts for both the mechanical tension and

electrical connection at once!!

>I would think its about time to build that 6 incher dont you think?

>Your 4 seems pretty well maxed out.

Yes - another coil will come together soon.  Exact plans pending, but it

will be either an 8 incher or an 18 incher!! Yow! THAT would be

ambitious!

Anyone out there every fooled with anything close to an 18 inch

secondary??  We have the time and resources for such nonsense!  Paul

will have to buy a new house to hold the 18 incher... :)

Either that or we put it on wheels and run it at my place (300x100 lot).

>Good idea. You will learn far more from trying to maximize your output

>with a fixed input.

Absolutely - we look forward to doing precisely that!

Or we could try the magnifier scene...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Wire Size & Cap Oil

Date: Fri, 27 Mar 1998 21:00:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 28 Mar 1998 02:00:36 +0500

Subject: Re: Wire Size & Cap Oil

On Thu, 26 Mar 1998 07:42:48 -0800 In Tesla-2

William R. Langston <blangsto@iwvisp.com> wrote;

>     My son and I are building our first serious attempt at a TC. We were able

> to lay hands on 6" PVC  and lots of 24 gauge wire (from gweaver) for the

> secondary, but in reading materials here we see that 22 gauge is "the

> preferred wire size for a 6 coil"; why? Will a 24 gauge, 24" winding on 6" PV

C

> work greatly different than a 22 gauge coil?

That 6 inch PVC tubing is actually 6.5 inches O.D. so making the length

over diameter ratio 4 would give you 26 inches of windings or roughly 1200

turns, which is a bit high (should be around 700) the resistance of #24

wire is about 26 ohms per 1000 feet. The resistance per 1000 feet of #22

wire is about 16 ohms or a bit over half that of the #24 wire, this would

lead to lower Q and greater losses using the #24 wire. BUT you have two

ways to improve the Q, one would be to wind two #24 wires on at the same

time, side - by - side thus when the two leads on each end are connected

together will yield about half the resistance or about 13 ohms and you will

have 600 turns which is much better. Your other alternative would be to

wind the coil with the #24 wire then coat it with polyurethane and after

that dries wind another layer of #24 on top of the first, this will leave

the turns at 1200 but will half the resistance again to about 13 ohms. The

second plan has the advantage of giving you a higher inductance secondary

and thus you should get bigger sparks.

>     Also, I was given about 12 gals of high-grade turbine oil (no impurities)

,

> is that ok for caps?

With any oils outside of either a commercial transformer oil or U.S.P.

mineral oil, no one knows what the electrical characteristics are so all

you can do is make a test capacitor and see how it performs.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: non ferrous chokes

Date: Fri, 27 Mar 1998 21:00:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 28 Mar 1998 02:00:38 +0500

Subject: Re: non ferrous chokes

On Thu, 26 Mar 1998 20:05:00 -0800 In Tesla-2

Mike Nicholas <705728@ican.net> wrote;

> Would a choke be effective if the core was made from a non magnetic metal such

> as copper or aluminum?

Firstly, unless you cut a slit length wise down the copper or aluminum tube

it will act like a shorted turn and do nothing but generate alot of heat!

Secondly, the use of any non-ferro magnetic material will result in less

inductance instead of more. Back in the late 1950's a popular toy amongst

electronics enthusiasts was a repulsion coil, which demonstrated a lot of

basic A.C. principles. The repulsion coils were typically constructed out

of the field coils from speakers (back then most speakers had electro

magnets rather than permanent magnets) with a bundle of iron wires about a

foot long filling the core. If a copper or aluminum ring was dropped over

the core and the coil connected to the A.C. line the ring would leap in the

air due to magnetic repulsion. If one held a sheet of copper in an

insulated pair of pliers on top of the core it would get hot enough to

literally fry an egg. And if one made a coil of a few dozen turns of

insulated wire and connected the leads to a low voltage bulb, lowering the

coil over the energized core would create a step down transformer and light

the lamp. BTW it was usual to add a parallel capacitor to make the coil

resonate at the 60 Hz mains frequency to maximize the magnetic field

effects.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Chokes

Date: Fri, 27 Mar 1998 21:00:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 28 Mar 1998 02:00:39 +0500

Subject: Re: Chokes

On Wed, 25 Mar 1998 15:37:27 -0500 In Tesla-2

Tom <Tom.Stock@gsbsc.gensig.com> wrote;

>      Can I make my own choke doughnuts by winding some sort of steel wire

>      into a doughnut shape?

>

>      How about a large (2") nut?

>

>      I saw someone else post this question and didn't see a reply.

There are several factor working against your idea, Tom. Firstly the steel

used to make nuts is not soft enough to have good magnetic properties, you

could improve it's magnetic characteristics by annealing it by heating it

to cherry red in a charcoal fire and allowing it to cool SLOWLY, then you

will still have the other problem of the nut acting as a shorted turn. This

can be eliminated by cutting a slit through one side of the nut. You are

much better off using an air core inductor. I differ with most others on

what I think constitutes a good enough inductance to prevent R.F. from

getting back to your transformer. I've always used about a pound or two of

#20 - #24 wire wound between two plexiglass sheets using a one inch length

of 1/2 inch plexiglass rod (or even wood) to act as the core.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Secondary wire

Date: Fri, 27 Mar 1998 21:00:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Sat, 28 Mar 1998 02:00:41 +0500

Subject: Re: Secondary wire

On Wed, 25 Mar 1998 14:03:12 -0600 In Tesla-2

Roy Minnich <rminnich@rolling-meadows.lib.il.us> wrote;

> Hello,

>

> I'm collecting parts for my first small Tesla coil and I was wondering =

> if the magnet wire used to wind the secondary can be spliced?  I have =

> some #24 wire but it's only in 100 and 200 foot lengths.  Will splicing =

> degrade performance in some way????

So long as you solder the joints well and insulate the joints with

polyurethane or the like, you will end up with a lumpy looking Secondary

but otherwise it should be fine. You may want to consider a often

overlooked source of wire, namely the degaussing coils in all color T.V.s

these are made of enamelled aluminum wire of gauge #20 to #24 and typically

are over 1000 feet in length! When you open the back of a disposed color

T.V. look around the back edges of the picture tube for what looks like a

1/2 inch thick cable that goes all the way around the circumference, that

is the degaussing coil and once carefully removed you VERY CAREFULLY cut

open the insulation and inside there are several hundred turns of enamelled

aluminum wire. The difference in conductivity between aluminum and copper

is not great enough to make any really significant degradation in the coils

output, besides you could always wind two layers in parallel.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Krazy carpet capacitor

Date: Fri, 27 Mar 1998 22:16:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Mar 1998 14:16:54 -0800

Subject: Re: Krazy carpet capacitor

Plans now available for my Krazy carpet capacitors.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Krazy carpet capacitor

Date: Fri, 27 Mar 1998 22:49:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 27 Mar 1998 14:49:44 -0800

Subject: Re: Krazy carpet capacitor

Organization: Self

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Fri, 27 Mar 1998 14:16:54 -0800

> Subject: Re: Krazy carpet capacitor

>

>   Plans now available for my Krazy carpet capacitors.

>

Mike Nicholas,

Please, a tad more elaboration on 'Krazy Carpet'????

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: junk yard rummageing

Date: Fri, 27 Mar 1998 23:36:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 27 Mar 1998 15:36:47 -0800

Subject: junk yard rummageing

Organization: Self

Mike Hammer & et al..

Mike, per your req: Enam mag wire, ga.:17, 19, 20, 22 most available,

new spools, 6"dia x 4" hi, $1.74/lb.

Also: 2/3 of 55 gal bbl full of CU.: tight rolls of sheet mtl: 4.75"w X

~.03thk x ~8lb. ea.roll; any use in RFGnd?

Also found cast Al spheres: (6"dia.) atop base-"cap" (fem.) attachment

for (to) 6.125" od tube. Cast wall thk: ~.187".  Realizing that anodize

is a dielectric, would that be a detriment in use as 2ndary  terminal?

Also found a screaming squirel cage ($10) for our "Clinton Unit":

primary gaps, which will need to be 'Well blown'!  Is there "Too much

air"?

Back to our RFGnd quandry: We WILL bury adequate Cu in the yard---

Interconnect from there to our TC unit is the question: Deploying the Cu

strips will be a logistical/hookp-up nightmare-  how much actual damage

to operation will be caused if we use #4 (fine) stranded, twisted cable?

$.65/ft.  We have lg. braid, but seem to have been talked out of using

that...  Don't know how to avoid twist, in any cable I've been able to

find.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Success -- So Far...

Date: Fri, 27 Mar 1998 23:39:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Fri, 27 Mar 1998 16:39:54 -0700

Subject: Success -- So Far...

Hurrah! I got my first neon-sign transformer today! I was wondering how I

was going to get one since there were no neon-sign shops in town, but

never-the-less I did. On the outside it looks brand new. It's 9 kV at the

secondary at 30 ma -- rather small but the best I could do (for now). Of

course, it's burned out, so after removing the tar I'm hopefully going to

remove two to three shunts. From what I have heard on this list so far, it

sounds like it is easier to remove shunts from transformers made by France

(such as mine) than by other companies. Is this correct?

It is in the freezer now since it is (unfortunately) way to warm outside --

even in Northern Colorado. I'll freeze it for a couple of days and then

short out the secondary leads and let it warm up while I attempt to remove

the tar. Do I have the idea right? I really don't want to botch-up this one

because I've had so much trouble getting it and I don't know where I could

get another. About how much tar to you leave on next to the secondary

windings for the gasoline or kerosine to get? 1/8"? How breakable are the

secondary windings?

If all goes smoothly, I'm going to mount it in a sealed  plastic box or a

paint can filled with mineral oil. Sound okay? What methods do you use to

insulate the transformer from the protective box?

Thanks!

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Krazy carpet capacitor

Date: Fri, 27 Mar 1998 23:41:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Fri, 27 Mar 1998 16:41:19 -0700

Subject: Re: Krazy carpet capacitor

>From: mike nicholas <705728@ican.net>

>Date: Fri, 27 Mar 1998 14:16:54 -0800

>Subject: Re: Krazy carpet capacitor

>

>

>

>  Plans now available for my Krazy carpet capacitors.

I'm new to this list. What are "Krazy carpet capacitors?" Could you

describe them?

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small TC?

Cc:

Bcc:

X-Attachments:

At 05:02 PM 3/27/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 27 Mar 1998 09:02:52 -0800

>Subject: Small TC?

>Organization: Self

>

>All,

>Has anyone worked out the general specs for a small Tesla Coil?

>Something with about a 1' lg. 2ndry winding?  It might be fun to put

>something like this together, while we are scrounging and building the

>big one!

>

>What kinds of performance could be expected?  Might it even employ shelf

>caps?

>

I built a mini coil last winter out of whatever I could find laying

around. Ive posted the specs on it a couple times already on the

list but some of it might bear repeating.

> Secondary. 660 turns # 32 on 2 inch PVC. OD 2 3/8 in.

>            Winding length 6 inches. Measured resonant frequency

>            920 KHZ

>

> Primary    9 1/2 turns # 6 solid copper in saucer shape

later increased to 12 turns

>

> Cap        initially used .006 mfd SW bottle caps then

used .0077 mfd plate cap.

>

> Gap        6  3/4 inch copper pipes mounted side by side on

>            plexiglass plate. Spacing .03 inch. Total gap distance .150 inch

>

> Power      single 9kv/30ma neon

>

> Terminal   copper toilet tank float. Resonant frequency with

>            terminal in place 650 KHZ

>            Resonant frequency with 5 inch diameter foil covered

>            styrofoam ball 630 KHZ

Best results were 13 inches using 5 inch diameter sphere.

This doesnt sound like a lot but it was over twice the

length of the secondary. Efforts at using a 12kv/30ma

failed. The secondary had a tendency to breakdown.

Most like due to the very small AWG 32 wire.

I would not recommend that small a wire for a secondary.

>Also, I have seen a reference to the inclusion of an oil-immersed large

>mechanical tuning capacitor in the primary cap ckt.  But no one has

>reported on the actualities of such....   More

>'fine-tune-on-the-fly'....

For the most part such things arent really necessary. Its sometimes

hard to tell the best tune moving the primary tap one full

turn at a time.

Unless your variable was a good percentage of the total capacitance

you may not see much variation at all.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small TC?

Date: Sat, 28 Mar 1998 01:42:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Fri, 27 Mar 1998 20:42:46 -0500 (EST)

Subject: Re: Small TC?

On Fri, 27 Mar 1998, Tesla-2 wrote:

>

> What kinds of performance could be expected?  Might it even employ shelf

> caps?

>

I've heard of people making coils that where about 2" long, gave about

inch long sparks, IIRC.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

"BETTER LIVING THROUGH RECKLESS EXPERIMENTATION" - Motto of The Mad Scientist

"Less talk, more synthahol." - Worf

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Success -- So Far...

Cc:

Bcc:

X-Attachments:

At 11:39 PM 3/27/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Fri, 27 Mar 1998 16:39:54 -0700

>Subject: Success -- So Far...

>

>Hurrah! I got my first neon-sign transformer today! I was wondering how I

>was going to get one since there were no neon-sign shops in town, but

>never-the-less I did. On the outside it looks brand new. It's 9 kV at the

>secondary at 30 ma -- rather small but the best I could do (for now). Of

>course, it's burned out, so after removing the tar I'm hopefully going to

>remove two to three shunts. From what I have heard on this list so far, it

>sounds like it is easier to remove shunts from transformers made by France

>(such as mine) than by other companies. Is this correct?

Franceformers seem to have the most straightforward core/shunt

design and I believe are the easiest to rebuild.

The only problem I have rebuilding them is the tar potting

compound they use is kind of soft and sticky. Its not extremely

brittle like the compound in Jeffersons. Freezing should help here.

>

>It is in the freezer now since it is (unfortunately) way to warm outside --

>even in Northern Colorado. I'll freeze it for a couple of days and then

>short out the secondary leads and let it warm up while I attempt to remove

>the tar. Do I have the idea right? I really don't want to botch-up this one

>because I've had so much trouble getting it and I don't know where I could

>get another. About how much tar to you leave on next to the secondary

>windings for the gasoline or kerosine to get? 1/8"? How breakable are the

>secondary windings?

First thing to do is to dig down into the tar near each HV bushing.

The bushings in France units are held in place with a clip. Remove

enough tar so the clip and then the bushing can be removed.

Also try to salvage the AC input bushing.

Once that is done then split the case open with a chisel and hammer

in two corners along the long dimension. Once the case is split

fold down the long side and work a large screwdriver under the tar

block and pry the block out of the case. Brute force and ugliness

is called for here.  Once the tar block is free of the case start

chiseling away the tar. I use a screwdriver and small hammer.

Work slowly. Be careful around the secondary windings.

One the majority of the tar is removed try to seperate the core.

Remove the 4 bolts (one in each corner) as well as the brackets

holding the core together. The core splits right in the middle

with the primary straddling the split. Work the halves of the

core apart and remove the primary and then the shunts.

At this point you can remove the secondaries if they will come

off but most likely they wont. Dump everything in kerosene

for a few days to dissolve the remaining tar. The secondaries

should come off with a little effort now.

The secondary windings are pretty fragile being wound with AWG 32

in 30 ma units. The worst thing you could do is to drive a screwdriver

into the windings while chipping away the tar.

>

>If all goes smoothly, I'm going to mount it in a sealed  plastic box or a

>paint can filled with mineral oil. Sound okay? What methods do you use to

>insulate the transformer from the protective box?

If its in oil it will be insulated by the oil. If you use a paint

can connect the core to the can so it can be grounded like a regular

neon. My rebuild in a plywood box runs with the core floating.

Good luck.

Just remember that rebuilding neons is fun.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Krazy carpet capacitor

Date: Sat, 28 Mar 1998 05:04:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Mar 1998 21:04:26 -0800

Subject: Re: Krazy carpet capacitor

> Mike Nicholas,

> Please, a tad more elaboration on 'Krazy Carpet'????

>

> "Krazy carpet" is a type of cheap sled for kids for sliding on snow in

> the winter, its flexible, rollable and best of all cheap, it has many

> other names, but this is the only one I know it by, if you live in

> warmer climates you're probably out of luck.

If anyone wants plans let me know.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Krazy carpet capacitor

Date: Sat, 28 Mar 1998 05:43:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Fri, 27 Mar 1998 21:43:52 -0800

Subject: Re: Krazy carpet capacitor

> >  Plans now available for my Krazy carpet capacitors.

>

> I'm new to this list. What are "Krazy carpet capacitors?" Could you

> describe them?

>

>

>   Its something like rolled poly caps, but cheaper, its made from a

> cheap sled for sliding on snow.

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Easy and reliable caps

Date: Sat, 28 Mar 1998 08:06:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sat, 28 Mar 1998 19:06:27 +1100

Subject: Easy and reliable caps

In Australia our milk comes in  high density polyethylene bottles.  The

plastic on these is about .5 millimetres thick.  I don't know how high the

voltage rating would be, but my idea was to have numerous bottles and

immerse them in saltwater (inside and out).  This seems to be a very quick

and easy way of making poly caps without rolling, buying materials, etc.  If

anyone else has any other comments on this please sent them to the list.  If

anyone can help me to get my finished but not yet working tesla coil to

work, I would be most appreciative.

From Scott Hayward

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small TC?

Date: Sat, 28 Mar 1998 08:13:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kent Rankin" <kentrankin@theatreorgans.com>

Date: Sat, 28 Mar 1998 03:13:18 -0500

Subject: Re: Small TC?

I have never built a Tesla Coil before, but this sounds like it is more to

my scale. Being only sixteen, I cannot afford to purchase into a lot of

materials for a coil, but I do have a supply of nearly one hundred

Franceformer Transformers at my disposal(I have worked on and off for my

father's sign company for a few years). What type of capacitor and primary

coil should one use in this case (very small coil, couple of inches high)?

Thanks in advance,

Kent Rankin

-----Original Message-----

From: Tesla-2 <tesla-2@emachine.com>

To: kentrankin@theatreorgans.com <kentrankin@theatreorgans.com>

Date: Friday, March 27, 1998 11:05 PM

Subject: Re: Small TC?

>From: Paul Anderson <madhobby@geeky1.ebtech.net>

>Date: Fri, 27 Mar 1998 20:42:46 -0500 (EST)

>Subject: Re: Small TC?

>

>On Fri, 27 Mar 1998, Tesla-2 wrote:

>

>>

>> What kinds of performance could be expected?  Might it even employ shelf

>> caps?

>>

>I've heard of people making coils that where about 2" long, gave about

>inch long sparks, IIRC.  TTYL!

>

>

>---

>Paul Anderson

>madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small TC?

Date: Sat, 28 Mar 1998 13:43:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sat, 28 Mar 1998 08:43:50 -0500 (EST)

Subject: Re: Small TC?

On Sat, 28 Mar 1998, Tesla-2 wrote:

> What type of capacitor and primary

> coil should one use in this case (very small coil, couple of inches high)?

>

Nah, nah, this was like a shirt pocket TC...  You really don't want to

build something like this for your first project.  You'll quickly want a

much bigger one.  A 3" diam. by 16" tall TC is good for getting

started.(BTW, I seriously doubt that shirt-pocket TC could handle the

output of a neon)  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummaging

Date: Sat, 28 Mar 1998 15:21:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Buck <jimbuck1@bellsouth.net>

Date: Sat, 28 Mar 1998 10:21:27 -0500

Subject: Re: junk yard rummaging

-snip-

> Also found a screaming squirel cage ($10) for our "Clinton Unit":

> primary gaps, which will need to be 'Well blown'!  Is there "Too much

> air"?

>

I have a squirrel cage fan series gap wired in series with a rotary on

my large (10") coil.  Yes you can have too much air.  I solved the

problem by making an adjustable bypass for some of the air.  With too

much air the sparks move farther out on the copper pipe electrodes

effectivly increasing the gap spacing.  I've even had a transformer blow

when running at low power with to much airflow through the gaps.

-snip-

Jim Buck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Web Space

Date: Sat, 28 Mar 1998 17:03:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tw33k3d <Tw33k3d@aol.com>

Date: Sat, 28 Mar 1998 12:03:15 EST

Subject: Web Space

If there is anyone out there in need of some web space for their TC projects i

have 20 meg + to be used E-mail me if interested

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Easy and reliable caps

Date: Sat, 28 Mar 1998 18:36:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sat, 28 Mar 1998 13:36:36 -0500

Subject: Re: Easy and reliable caps

>In Australia our milk comes in  high density polyethylene bottles.  The

>plastic on these is about .5 millimetres thick.  I don't know how high the

>voltage rating would be, but my idea was to have numerous bottles and

>immerse them in saltwater (inside and out).  This seems to be a very quick

>and easy way of making poly caps without rolling, buying materials, etc.

>anyone else has any other comments on this please sent them to the list.

>If anyone can help me to get my finished but not yet working tesla coil to

>work, I would be most appreciative.

>

>>From Scott Hayward

Scott

the poly bottles will work but you will have to wire several of them in series.

I have used Naya water botles in this way but it takes a lot of space for

the required # of bottles to get enough capacitance.

you will have to fill the top of the bottles with oil and the tank too,

to suppress corona or they will surely fail.

I have now started rolling caps with poly sheeting and it woks much

better and in much less space.

good luck;

Chris Watkins

cwatkins@seidata.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: secondary coating

Date: Sat, 28 Mar 1998 18:39:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Sat, 28 Mar 1998 13:39:20 EST

Subject: secondary coating

What's the best thing to use for sealing a secondary coil? Polyurethane,

Epoxy, Varnish or what? Winding a 21" x 4 1/2"D coil took a lot longer than I

thought, but in a way it's rather "Zen" like. I'd like to protect my finished

product as well as possible without diminishing performance.

Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary ground

Date: Sat, 28 Mar 1998 18:56:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Sat, 28 Mar 1998 13:56:41 EST

Subject: Secondary ground

I wound my secondary on a 1/4" thick acrylic tube & was thinking of setting a

piece of threaded brass rod about 1/8" into it & holding it in w/ epoxy. This

would be long enough to allow me to put a couple of nuts on to secure the

ground wire. Will the projection cause a problem w/ the primary? Is there a

better way to attach a ground? Thanks again, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummaging

Date: Sat, 28 Mar 1998 19:15:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 28 Mar 1998 11:15:16 -0800

Subject: Re: junk yard rummaging

Organization: Self

Tesla-2 wrote:

>

> From: Jim Buck <jimbuck1@bellsouth.net>

> Date: Sat, 28 Mar 1998 10:21:27 -0500

> Subject: Re: junk yard rummaging

>

> -snip-

>

> > Also found a screaming squirel cage ($10) for our "Clinton Unit":

> > primary gaps, which will need to be 'Well blown'!  Is there "Too much

> > air"?

> >

> > I have a squirrel cage fan series gap wired in series with a rotary on my large (10") coil.  > >Yes you can have to much air.

> -snip-

>

> Jim Buck

>

-More snipping-

Jim,

Rotary gap in series w/ stationary gaps??

I have heard that most avoid rotaries since they force the system into

an artificially induced  Rfreq., due to the contact rate, as opposed to

the frequency of the C in lc....  I need clarification, I think...

On my last rummage, I turned up what looks like a seriously well

engineered rotary gap: motor: lg, w/cap start can on side, maybe .25-.3

hp?; bearings seem in fine shape; didn't test for oper; stator: 2

electrodes @ 90 deg to each other??; rotor: ~1-1.2" thick, anodized Al

pl, ~10" dia., w/2 electrodes at 180 deg.; tripple carbon brush feed to

rotor- brass contact ring;  the whole thing is perhaps 40-50 lbs, on

good structure. electrodes might be tungsten? This from memory, as I

took no data, but can get particulars if any interest.

Couldn't understand the gapping theory that I saw, however... looks

fairly complete, otherwise, unless 2 stat.electrodes are simply missing,

which could be the case....  Certainly excellent starting point if

someone wants to make a R gap! Impressive piece of gear.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: secondary coating

Cc:

Bcc:

X-Attachments:

At 06:39 PM 3/28/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sat, 28 Mar 1998 13:39:20 EST

>Subject: secondary coating

>

>What's the best thing to use for sealing a secondary coil? Polyurethane,

>Epoxy, Varnish or what? Winding a 21" x 4 1/2"D coil took a lot longer than I

>thought, but in a way it's rather "Zen" like. I'd like to protect my finished

>product as well as possible without diminishing performance.

>Thanks, Jim

>

The best coating is epoxy. It not only provides a hard coating

but has excellent electrical characteristics. By it from the paint

store in the form of Behr Build 50. Unfortunately its quite expensive

and probably over kill for most coils. I dont use it on any of my coils.

You almost have to rig up something to rotate the form until the

epoxy cures.

Polyurethane varnish (real polyurethane not any of the latex varieties)

also works quite well and is relatively inexpensive. One pint

should be enough to seal and coat a 4 inch diameter secondary.

Get the glossy type rather than satin.

Apply enough coats so you can no longer feel the individual

wires. The coating should be smooth. This will take about 6

medium coats.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: I can't get it working!!!

Date: Sun, 29 Mar 1998 00:02:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 29 Mar 1998 10:02:27 +1000

Subject: I can't get it working!!!

After my first coil flopped a built another one using a lot of help from

Malcom Watts (Thanks!).  But I tried it this weekend and it didnt work :(

These are the specs:

Primary coil-7turns of 0.25" copper pipe spaced aprrox 0.25" inner dia. 6.2

outer dia. unknown

Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

bottom and a lot of space at the top (8" maybe) has 4 coats of polyurethane

varnish over the windings.

Top Electrode-A makeshift toroid that isn't smooth by any means so I

wouldn't count it's capacitance.

Capacitors-6 saltwater types in a box giving about .0045 uF (I have another

6 if needed)

Spark gap-two brass brackets with tapped holes carrying 1" wide x 1.5" long

pieces of brass.

Safety Gap- The same sort of thing as the spark gap except there is a groung

plate in the middle.

Transformer- One 12000V/30mA Neon sign transformer (I have 2 more if needed)

Chokes-none

RF Ground-2' aluminium pipe

Primary coil is resting on glass.  Secondary is capped at one end, sitting

on the glass at the other. The wire to the primary coil is attached by

alligator clips

From Scott

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummaging

Date: Sun, 29 Mar 1998 00:25:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Buck <jimbuck1@bellsouth.net>

Date: Sat, 28 Mar 1998 19:25:01 -0500

Subject: Re: junk yard rummaging

-snip-

> Jim,

> Rotary gap in series w/ stationary gaps??

Yes I'm using almost 10kw at times on the big coil.  The static gaps

just will not quench without the rotary at those power levels.

> I have heard that most avoid rotaries since they force the system into

> an artificially induced  Rfreq., due to the contact rate, as opposed to

> the frequency of the C in lc....  I need clarification, I think...

I'm using a asynchronous rotary gap.  You don't want to use an

asynchronous gap with neon transformers.  This type of rotary can break

at any point on the ac waveform which can cause problems.  A

synchronous  rotary gap breaks on the peak of the waveform which is

easier on your system and you can use this type with neons.

I hope this helps.. Maybe someone else can jump in and explain it more

clearly.

-snip-

Jim Buck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary ground

Cc:

Bcc:

X-Attachments:

At 06:56 PM 3/28/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sat, 28 Mar 1998 13:56:41 EST

>Subject: Secondary ground

>

>I wound my secondary on a 1/4" thick acrylic tube & was thinking of setting a

>piece of threaded brass rod about 1/8" into it & holding it in w/ epoxy. This

>would be long enough to allow me to put a couple of nuts on to secure the

>ground wire. Will the projection cause a problem w/ the primary? Is there a

>better way to attach a ground? Thanks again, Jim

>

This should be OK as long as no portion of that ground connection

goes inside the form. You want absolutely no part of your secondary

wiring inside the form. Doing so will seriously degrade the voltage

handling capability of your secondary.

My preferred method is to epoxy a rectangular plate to the side

of the the secondary form just below the winding. This can be a

piece of sheet copper or brass.

Once epoxied in place solder the bottom end of the secondary

to this plate. I then solder a 2 foot long ground lead of stranded

AWG 8 to the plate. The free end of this lead terminates in a ring terminal.

This ring terminal is then bolted to my ground lead when needed.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 04:53:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sat, 28 Mar 1998 22:53:23 -0600

Subject: Re: I can't get it working!!!

I am no expert, and don't claim to be, but here are some things

I have noticed that don't sound right :

>Primary coil-7turns of 0.25" copper pipe spaced aprrox 0.25" inner dia. 6.2

You may need more than that.  I use 16 turns for best tap on my 6"

secondary,and used 14 on my 4".

>Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

270 turns is by FAR not enough.  #18 is just too thick.  700-1000 would work best.

>Capacitors-6 saltwater types in a box giving about .0045 uF (I have another

For 30ma, throw three more bottles on there.  you won't have enough

capacitance for 60ma.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coating

Date: Sun, 29 Mar 1998 06:19:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 29 Mar 1998 01:19:02 EST

Subject: Re: secondary coating

Jim,

I don't know what the "best" protection would be, but I have  been happy

with the stuff I have been using.  It is made by Krylon and is called

insulating varnish.  It comes is spray cans and has a 2000V/mil dielectric

strength.  I obtained it through Grainger catalog.  There may be better

products available, but I thought I'd mention this to you.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 07:42:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 29 Mar 1998 17:42:16 +1000

Subject: Re: I can't get it working!!!

>You shold have more primary turns for tuning, what configuration?

Flat Pancake

You mean I have to make a new primary??

>> Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

>> bottom and a lot of space at the top (8" maybe)

>

>Too few secondary turns and too thick of wire, use at least #22 or #24 with 800

>- 1000 turns, you only need a few inches of space at both ends.

And a new secondary?  Is there any way of using the one I have?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 07:54:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sat, 28 Mar 1998 23:54:14 -0800

Subject: Re: I can't get it working!!!

Tesla-2 wrote:

> From: "Scott Hayward" <smash@eisa.net.au>

> Date: Sun, 29 Mar 1998 10:02:27 +1000

> Subject: I can't get it working!!!

>

> After my first coil flopped a built another one using a lot of help from

> Malcom Watts (Thanks!).  But I tried it this weekend and it didnt work :(

> These are the specs:

> Primary coil-7turns of 0.25" copper pipe spaced aprrox 0.25" inner dia. 6.2

> outer dia. unknown

You shold have more primary turns for tuning, what configuration?

> Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

> bottom and a lot of space at the top (8" maybe)

Too few secondary turns and too thick of wire, use at least #22 or #24 with 800

- 1000 turns, you only need a few inches of space at both ends.

> has 4 coats of polyurethane

> varnish over the windings.

Make sure its not a water based varnish

> Top Electrode-A makeshift toroid that isn't smooth by any means so I

> wouldn't count it's capacitance.

> Capacitors-6 saltwater types in a box giving about .0045 uF (I have another

> 6 if needed)

> Spark gap-two brass brackets with tapped holes carrying 1" wide x 1.5" long

> pieces of brass.

this spark gap will be ok for your first coil but you may want to try a series

gap later on.

> Safety Gap- The same sort of thing as the spark gap except there is a groung

> plate in the middle.

> Transformer- One 12000V/30mA Neon sign transformer (I have 2 more if needed)

> Chokes-none

> RF Ground-2' aluminium pipe

a better ground would be in order, a well point may be ok

> Primary coil is resting on glass.  Secondary is capped at one end, sitting

> on the glass at the other. The wire to the primary coil is attached by

> alligator clips

> >From Scott

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 13:25:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 29 Mar 1998 08:25:06 EST

Subject: Re: I can't get it working!!!

Scott,

The other responses I have seen about your troubles are good, but I

wouldn't toss out your secondary yet.  What you have done by using the 18g

wire is to create a very high Q secondary which is very sensitive to tuning.

It may not produce as high a voltage output as a 1000 turn secondary, but

should still work.  In order to get a higher output,  you will need to get as

high a ratio of secondary/primary inductances as possible.  Rather than trying

to put more turns onto your primary, why not try using a second neon in

parallel and doubling the cap size.  This should allow you to tune it with the

existing number of turns on the primary.  My coil setup is quite sensitive to

tuning (almost no output one turn off).  Just a few inches from the right spot

makes a big difference in the ouput.  This is a result of my trying to make

the highest Q  secondary with the materials I had available.  I used just

under 600 turns of 19g wire on my 5" pvc secondary because I found a roll

fairly cheap.  My tuning using two 15kV, 30mA neons and a 0.01uF cap is at

about 6 3/4 turns.  I haven't finished optimizing the coupling yet so this

might wind up being fewer turns once I have found the best coupling (I think

it is still less than critical coupling).

You may decide to go with the 800+ turns on a 4" pvc secondary, but you

should still be able to get good sparks out of your system if you adjust it

properly.

Hope you get it working well,

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummaging

Date: Sun, 29 Mar 1998 15:23:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 29 Mar 1998 10:23:07 EST

Subject: Re: junk yard rummaging

Gene,

I am writing in regards to the spools of wire you mentioned.  Is this

available?  I would love to get about 1200 - 1500 feet of 17g wire.  If this

is available and you would be willing to hook me up with some, let me know and

I'll send you enough $$ to take care of it.

Thanks,

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 17:18:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 29 Mar 1998 11:18:26 -0600

Subject: Re: I can't get it working!!!

>You mean I have to make a new primary??

Just add more turns.

>And a new secondary?  Is there any way of using the one I have?

I would say "heck no" at the power levels you are running.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Im a NEWBIE,,,help!!!

Date: Sun, 29 Mar 1998 17:38:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "PecosBill" <metalhed@netrom.com>

Date: Sun, 29 Mar 1998 12:38:12 -0500

Subject: Im a NEWBIE,,,help!!!

Does it matter if the wires are turned counter clockwise or clockwise

help!!!

thanx

Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummaging

Date: Sun, 29 Mar 1998 17:38:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sun, 29 Mar 1998 09:38:55 -0800

Subject: Re: junk yard rummaging

Organization: Self

Tesla-2 wrote:

>

> From: Hollmike <Hollmike@aol.com>

> Date: Sun, 29 Mar 1998 10:23:07 EST

> Subject: Re: junk yard rummaging

>

> Gene,

>    I am writing in regards to the spools of wire you mentioned.

Mike,

I will checkout wire if they are open Sun.  Get back to'ya.

Gene

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: I can't get it working!!!

Cc:

Bcc:

X-Attachments:

At 12:02 AM 3/29/98 GMT, you wrote:

>From: "Scott Hayward" <smash@eisa.net.au>

>Date: Sun, 29 Mar 1998 10:02:27 +1000

>Subject: I can't get it working!!!

>

>After my first coil flopped a built another one using a lot of help from

>Malcom Watts (Thanks!).  But I tried it this weekend and it didnt work :(

Scott,

Its not unusual for a coil to take a little tweaking to get

it working. Lets see what you have.

>These are the specs:

>Primary coil-7turns of 0.25" copper pipe spaced aprrox 0.25" inner dia. 6.2

>outer dia. unknown

This is really too few primary turns to give much tuning range.

Its always nice to have extra turns available to be able to

accomodate larger toroids.

>Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

>bottom and a lot of space at the top (8" maybe) has 4 coats of polyurethane

>varnish over the windings.

A rather low number number of turns. You didnt say what the winding length

was and if the turns are spaced or close wound. This low turns count

will result in a rather high resonant frequency.

I would assume you have the turns space wound as close winding would

result in a very short secondary winding.

A more normal 6 inch secondary would have a 26 inch winding of AWG 22.

>Top Electrode-A makeshift toroid that isn't smooth by any means so I

>wouldn't count it's capacitance.

What is the size of this toroid? The toroid size affects the secondary resonant

frequency. Most beginners start with far too large a toroid. A large toroid

does help create longer sparks but makes the coil more difficult to tune.

>Capacitors-6 saltwater types in a box giving about .0045 uF (I have another

>6 if needed)

>Spark gap-two brass brackets with tapped holes carrying 1" wide x 1.5" long

>pieces of brass.

This will serve to get you started but replace this with a multiple

electrode gap like the RQ cylinder gap.

>Safety Gap- The same sort of thing as the spark gap except there is a groung

>plate in the middle.

Fine.

>Transformer- One 12000V/30mA Neon sign transformer (I have 2 more if needed)

Start with the single neon. More power is not what you need at this point.

>Chokes-none

Add chokes as soon as your coil starts working. Important to keep

your neons healthy. If you use air core chokes be sure to add some

series resistors to dampen any oscillations.

>RF Ground-2' aluminium pipe

aluminum pipe where? Is this driven into the ground?

How is it connected to the coil?

>Primary coil is resting on glass.  Secondary is capped at one end, sitting

>on the glass at the other. The wire to the primary coil is attached by

>alligator clips

The basic problem here is resonance. You must realize exactly what

a Tesla coil is. It is 2 resonant circuits coupled magnetically.

One of the resonant circuits is the secondary coil. Its inductance

and self capacitance plus the capacitance of the toroid form

a resonant LC circuit. It will resonate at a specific frequency.

The other resonant circuit is the main capacitor and primary coil.

This circuit must be tuned to the same frequency that the secondary

coil resonates at. If the two circuits cannot be tuned to the same

frequency no output occurs.

My guess is that your secondary coil has too few turns

and is space wound causing its resonant frequency to be

very high. Your primary circuit is tuned to a much lower frequency

so there is little coupling between the circuits. What little output

you have is being hidden by a too large toroid.

Give me a few more details about your setup and I can make

some more informed guesses.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Im a NEWBIE,,,help!!!

Date: Sun, 29 Mar 1998 20:26:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 29 Mar 1998 14:26:28 -0600

Subject: Re: Im a NEWBIE,,,help!!!

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "PecosBill" <metalhed@netrom.com>

> Date: Sun, 29 Mar 1998 12:38:12 -0500

> Subject: Im a NEWBIE,,,help!!!

>

> Does it matter if the wires are turned counter clockwise or clockwise

> help!!!

> thanx

> Bill

>

<SNIP>

Bill,

Nope!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 20:41:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Sun, 29 Mar 1998 12:41:44 -0800

Subject: Re: I can't get it working!!!

>

>

> >You shold have more primary turns for tuning, what configuration?

> Flat Pancake

> You mean I have to make a new primary??

>

You can add on to it if you have the room by putting on more copper tubing

> >> Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

> >> bottom and a lot of space at the top (8" maybe)

> >

> >Too few secondary turns and too thick of wire, use at least #22 or #24 with 800

> >- 1000 turns, you only need a few inches of space at both ends.

> And a new secondary?

I would go with a new secondary, if you go 6" diam. the winding length should be

18" with the correct wire guage to get you 800 - 1000 turns

>  Is there any way of using the one I have?

> Its not likely you will see any spectacular sparks from this coil.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 20:51:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 29 Mar 1998 14:51:31 -0600

Subject: Re: I can't get it working!!!

>I would go with a new secondary, if you go 6" diam. the winding length should be

>18" with the correct wire guage to get you 800 - 1000 turns

I must disagree on this.  The winding length of a 6" secondary should

be 26" if using #24 or 22"

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: I can't get it working!!!

Date: Sun, 29 Mar 1998 21:01:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sun, 29 Mar 1998 16:01:01 -0500 (EST)

Subject: Re: I can't get it working!!!

On Sun, 29 Mar 1998, Tesla-2 wrote:

> Secondary coil-270 turns of #18(I think) on 6" PVC pipe with 1" space at the

> bottom and a lot of space at the top (8" maybe) has 4 coats of polyurethane

> varnish over the windings.

>

First off you have WAAY too few turns, and your wire gauge is way to big.

You should go with at least 22 gauge, with about 800 turns.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Im a NEWBIE,,,help!!!

Cc:

Bcc:

X-Attachments:

At 05:38 PM 3/29/98 GMT, you wrote:

>From: "PecosBill" <metalhed@netrom.com>

>Date: Sun, 29 Mar 1998 12:38:12 -0500

>Subject: Im a NEWBIE,,,help!!!

>

>Does it matter if the wires are turned counter clockwise or clockwise

>help!!!

>thanx

>Bill

>

Bill,

Makes no difference. In fact you can have the primary

going one way and the secondary going the other way.

Thats the way my coil turned out. Didnt plan it that

way but thats the way it ended up.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RF Choke question

Date: Sun, 29 Mar 1998 22:09:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Sun, 29 Mar 1998 17:09:58 EST

Subject: RF Choke question

I'm just about done with the construction of my first coil. Thanks to all for

the help & advice. Now I'm worried about protecting my neon xformer. I've read

a lot of postings about chokes. Just what do they do to add protection? In

other words How do they work?? I'm going with air chokes first (cost &

available materials at hand). I remember some postings saying to add resistors

to them (one of your's I think, Mike) Do I just add them on to the end of the

mini coils? If so what rating? Also is it correct to place them between the

safety gap & the spark gap? Why don't they "fry out" since all the other wire

I am using at this point is much heavier? Do I just tie them into the circuit

by connecting them in line? Sorry for all the questions but I'm close to

sparking up & don't want to blow it now. Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tuning Question

Date: Sun, 29 Mar 1998 22:59:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "PecosBill" <metalhed@netrom.com>

Date: Sun, 29 Mar 1998 17:59:41 -0500

Subject: Tuning Question

In Mike Hammer's txt file about Tuning a tesla coil he states:

"If you find the best tune point is with very few primary

turns then your primary cap is too large. Adding a larger toroid

to your secondary will lower its resonant frequency and allow

you to tap in more primary turns also."

Tap in more primary turns also?

The reason i'm asking what this means is because the coil we have built does

not discharge at all...but lights a lightbulb right up. We're using a

12000V/30mA neon transformer, wired to two wine bottle salt water caps,

wired to the main spark gap and to the primary. The primary and the

secondary then both ground on the other end of the spark gap. Primary is 10

turns of #18 AWG wire (solid), spaced an inch and 1 1/2 away from each turn.

Secondary is 2 inch diameter, 11 inches long, with #26 wire. Discharge

terminal is a brass doornob with a tumbtack on top to act as a discharge

point. Any modifications or suggestions on how we can improve this design?

(besides the obvious saftey gap and rf chokes)

We can light a bulb right up bright using the configuration outlined above,

but we can *not* get any form of discharge from the terminal! Does the

terminal need something to discharge to? Any suggestions welcome.

Bill Barry

Newbie Tesla'er

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF Choke question

Cc:

Bcc:

X-Attachments:

At 10:09 PM 3/29/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Sun, 29 Mar 1998 17:09:58 EST

>Subject: RF Choke question

>

>I'm just about done with the construction of my first coil. Thanks to all for

>the help & advice. Now I'm worried about protecting my neon xformer. I've read

>a lot of postings about chokes. Just what do they do to add protection? In

>other words How do they work?? I'm going with air chokes first (cost &

>available materials at hand). I remember some postings saying to add resistors

>to them (one of your's I think, Mike) Do I just add them on to the end of the

>mini coils? If so what rating? Also is it correct to place them between the

>safety gap & the spark gap? Why don't they "fry out" since all the other wire

>I am using at this point is much heavier? Do I just tie them into the circuit

>by connecting them in line? Sorry for all the questions but I'm close to

>sparking up & don't want to blow it now. Thanks, Jim

>

>

Jim,

Your RF chokes go in each HV line in between the safety gap

and main gap. The resistors are just connected into the same line.

One resistor and one RF choke in each HV line.

A RF choke works by offering a high impedance to RF energy

while offering little impedance at 60 hz. So the 60 hz

energy flows through the chokes with little loss and

the RF energy from the primary tank circuit sees a higher impedance.

This serves to keep most of the RF energy out of your neons.

Its not a perfect filter but you dont need perfection.

Just need to knock down the level of the RF to a manageable level.

For series resistors use anything you can find from 300-1500

ohms. The value is not critical. Ive had good success with

some 310 ohm 50 watt resistors I bought at a hamfest for

10 cents each.

The power rating needed depends on your power input.

10 watt resistors should be more than enough for 30ma neons.

>Why don't they "fry out" since all the other wire

>I am using at this point is much heavier?

The current level at this point is quite low.

Only 30ma at 60hz if you are using a 30ma neon.

You do not need heavy wiring here. You dont need

heavy wire until you get to the main spark gap.

The connections from the main gap to the main capacitor

and primary coil need to be heavy.

Good luck and keep us posted on your results.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning Question

Date: Mon, 30 Mar 1998 01:45:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Corr" <corr@enid.com>

Date: Sun, 29 Mar 1998 19:45:45 -0600

Subject: Re: Tuning Question

>12000V/30mA neon transformer, wired to two wine bottle salt water caps,

Two is not *EVEN* close to enough.  I used 9 and still needed more...

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning Question

Cc:

Bcc:

X-Attachments:

At 10:59 PM 3/29/98 GMT, you wrote:

>From: "PecosBill" <metalhed@netrom.com>

>Date: Sun, 29 Mar 1998 17:59:41 -0500

>Subject: Tuning Question

>

>In Mike Hammer's txt file about Tuning a tesla coil he states:

>

>"If you find the best tune point is with very few primary

>turns then your primary cap is too large. Adding a larger toroid

>to your secondary will lower its resonant frequency and allow

>you to tap in more primary turns also."

>

>Tap in more primary turns also?

By that I meant that adding more toroid to your secondary

will lower its resonant frequency. So in order to tune

your primary circuit to this new lower frequency you will

need to use more primary turns. More primary turns are better.

Remember your primary circuit and secondary coil must be

tuned to the same frequency.

>

>The reason i'm asking what this means is because the coil we have built does

>not discharge at all...but lights a lightbulb right up. We're using a

>12000V/30mA neon transformer, wired to two wine bottle salt water caps,

>wired to the main spark gap and to the primary. The primary and the

>secondary then both ground on the other end of the spark gap. Primary is 10

>turns of #18 AWG wire (solid), spaced an inch and 1 1/2 away from each turn.

>Secondary is 2 inch diameter, 11 inches long, with #26 wire. Discharge

>terminal is a brass doornob with a tumbtack on top to act as a discharge

>point. Any modifications or suggestions on how we can improve this design?

>(besides the obvious saftey gap and rf chokes)

First thing is to unhook your secondary from your primary coil.

There should be no direct connection between the primary and

secondary. The bottom end of the secondary should go instead to a

good RF ground. This is a safety as well as a performance issue.

>

>We can light a bulb right up bright using the configuration outlined above,

>but we can *not* get any form of discharge from the terminal! Does the

>terminal need something to discharge to? Any suggestions welcome.

Since you can light a bulb your coil is generating some RF.

But the voltage is not high enough to form streamers.

The most probable reason is that your primary and secondary are

not tuned to the same frequency. My guess is you do not have

enough primary capacitance. Add two more bottles to your cap

and try tapping your primary at various numbers of turns.

Bring a grounded wire near the secondary output and see if

you can draw a spark. Once you have a visible spark you can

use it as a tuning aid.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning Question

Date: Mon, 30 Mar 1998 05:03:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Sun, 29 Mar 1998 21:03:29 PST

Subject: Re: Tuning Question

Mike, PecosBill, HollMike, others:

Some comments and questions along this thread:

>>In Mike Hammer's txt file about Tuning a tesla coil he states:

>>

>>"If you find the best tune point is with very few primary

>>turns then your primary cap is too large. Adding a larger toroid

>>to your secondary will lower its resonant frequency and allow

>>you to tap in more primary turns also."

>>

>>Tap in more primary turns also?

>

>By that I meant that adding more toroid to your secondary

>will lower its resonant frequency. So in order to tune

>your primary circuit to this new lower frequency you will

>need to use more primary turns. More primary turns are better.

I must now unleash an equation or two:

Once the spark gap fires, the energy in the primary cap discharges

through the primary coil.  As this voltage oscillates (at the primary's

resonant frequency), the energy is periodically transferred to and from

the cap to the primary inductance.  If we write the state equations, and

invoke conservation of energy (and ignore all losses!) we find that the

sum of the cap energy and the inductor energy is a constant at each and

every point in time.

Thus -> 0.5 CV^2 = 0.5 LI^2

Solve for I: I=V * sqrt(C/L) where V=max cap voltage, I = max inductor

current, etc.  V is generally fixed by the available transformer and the

gap setting, etc.

What I'm trying to point out is that if you are forced to tap your

primary at a very small number of turns, then the inductance will be

very small (inductance is proportional to the square of the number of

turns), your capacitance will necessarily be (relatively) huge, and your

circulating currents in the primary will be very large!!  This places

immense stresses on your caps and wiring, etc.

Next - recall that in general, resistive losses are proportional to I^2.

Thus - large currents = (very lossy)*(very lossy), etc. :)

These are my own thoughts as to why you should strive towards designing

your coil such that a large number of primary turns may be tapped!

Plus - as Mike Hammer pointed out to me once before - tapping the coil

at only a few turns greatly increases it's sensitivity to the exact tap

point.  Tuning becomes rather difficult as a slight adjustment might

move you from poor results to NO results, right on past that illusive

point of GREAT results!

Also note that the orientation of your TAP WIRE will be significant, as

it also produces a magnetic field, just like the entire primary coil!  I

had a situation where I was changing the tap position and could see no

visible change in coil output.  Apparently each new tap position was in

line with the wire, and the current in the tap wire was "taking a

U-turn" at the tap, and flowing back under the wire.  Thus I was

effectively maintaining the same number of turns since the wire itself

was creating a field which cancelled out field of the newly tapped

primary!  (This is very hard to explain this late at night...)

(That problem can be minimized by keeping your primary feedwires

perpendicular to the plane of your primary coil as much as possible.)

>>The reason i'm asking what this means is because the coil we have built does

>>not discharge at all...but lights a lightbulb right up. We're using a

>>12000V/30mA neon transformer, wired to two wine bottle salt water caps,

>>wired to the main spark gap and to the primary. The primary and the

>>secondary then both ground on the other end of the spark gap. Primary is 10

>>turns of #18 AWG wire (solid), spaced an inch and 1 1/2 away from each turn.

#18 sounds rather thin, but it should get you off the ground.  Is it

bare wire, ie, can you tap it at convenient points?

>>Secondary is 2 inch diameter, 11 inches long, with #26 wire. Discharge

>>terminal is a brass doornob with a tumbtack on top to act as a discharge

<snip>

>>We can light a bulb right up bright using the configuration outlined above,

>>but we can *not* get any form of discharge from the terminal! Does the

>>terminal need something to discharge to? Any suggestions welcome.

>

>

>Since you can light a bulb your coil is generating some RF.

>But the voltage is not high enough to form streamers.

Question - is the phenomenon of the lighting bulb caused by the high

voltage/high frequency of the secondary?  Or would a bulb light in the

high frequency high intensity magnetic field produced by the primary

alone?  Has anyone tried this?

You might be able to use the lightbulb to see if your tuning attempts

are making things better or worse.  If you change your tap and the bulb

(at some fixed distance from the coil) lights up brighter, then you are

getting closer to the ideal tap position. (Only if the bulb is sensitive

to the E-field of the secondary more so than the H-field of the

primary).

>

>The most probable reason is that your primary and secondary are

>not tuned to the same frequency. My guess is you do not have

>enough primary capacitance. Add two more bottles to your cap

>and try tapping your primary at various numbers of turns.

Mike's advise sounds good.  Let us know when you are ready to build a

poly cap!  (I've never used bottlecaps so I can't give you any advice)

>Bring a grounded wire near the secondary output and see if

>you can draw a spark. Once you have a visible spark you can

>use it as a tuning aid.

When I first powered our coil, we were getting about 1 inch of arc from

the top of the primary to a grounded wire.  After tuning for about a

hour we increased that up to about 12-18 inches to the same grounded

wire.  The only thing which changed was the tap point!

(4.25 by 26 inch sec, 14 turns 0.25 copper refridge line, 7500 volt 120

ma transformer, 0.025 uf poly cap.)

Now - I think Mike Hollingsworth (HollMike) had mentioned something

about the ratio of sec to primary inductance for maximal results.  I

can't locate his post on that matter (must have pitched it too soon!) So

Mike Hollingsworth - if possible, could you re-explain your thoughts

about this?  I'm curious if it supports the "Use more turns and less

cap" concept!

Thanks!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF Choke question

Date: Mon, 30 Mar 1998 05:25:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>A RF choke works by offering a high impedance to RF energy

T>while offering little impedance at 60 hz. So the 60 hz

T>energy flows through the chokes with little loss and

T>the RF energy from the primary tank circuit sees a higher impedance.

T>This serves to keep most of the RF energy out of your neons.

T>Its not a perfect filter but you dont need perfection.

T>Just need to knock down the level of the RF to a manageable level.

Remember that every watt of RF that escapes back to the transformer is

one less watt available for the spark!

Ralph

---

Capital punishment drastically reduces second offenses.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning Question

Date: Mon, 30 Mar 1998 06:29:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sun, 29 Mar 1998 22:29:42 -0800

Subject: Re: Tuning Question

Organization: Self

Tesla-2 wrote:

>

> From: "PecosBill" <metalhed@netrom.com>

> Date: Sun, 29 Mar 1998 17:59:41 -0500

> Subject: Tuning Question

>

> In Mike Hammer's txt file about Tuning a tesla coil he states:

>

> "If you find the best tune point is with very few primary

> turns then your primary cap is too large. Adding a larger toroid

> to your secondary will lower its resonant frequency and allow

> you to tap in more primary turns also."

>

> Tap in more primary turns also?

>

> The reason i'm asking what this means is because the coil we have built does

> not discharge at all...but lights a lightbulb right up. We're using a

> 12000V/30mA neon transformer, wired to two wine bottle salt water caps,

> wired to the main spark gap and to the primary. Any suggestions welcome.

>

> Bill Barry

> Newbie Tesla'er

Bill,

I'm no expert, but my calcs lead me to the inevitable reclusion that you

should start drinking more WINE!

Gene Waldo

PS, If I'm wrong, what the hell!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning Question

Date: Mon, 30 Mar 1998 13:31:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 30 Mar 1998 08:31:16 EST

Subject: Re: Tuning Question

Bill, others,

My intention for suggesting that a larger cap and fewer turns was to give

Scott Hayward a possible means by which to get some output from his coil,

rather than starting over.   He posted earlier that he had built a secondary

with 270 turns of 18g wire.  This is out of the ordinary for TC,s, but should

still work(at least to some degree).  He also stated that his primary cap was

0.005uF and being powered by a single 15kV , 20 ma neon.  I was suggesting

that before he went back to the drawing board, he try paralleling two neons

and doubling the cap.  This would allow tuning at fewer turns and perhaps get

some decent output from his coil.  I did mention that the tuning would be very

sensitive, but should still be workable.

The outuput voltage being a function of the inductance ratio was quoted

from a book titled Modern Resonance Transformer Design by D.C. Cox.   The

equation presented is as follows:

Es = Ep * sqrt( Ls/Lp)

This is the maximum theoretical secondary voltage.  This was derived by

physicists at the Chalk River Atomic Energy Installation in Ontario, Canada

(according to the author)

I was not suggesting that Scott use just one or two turns on the primary

(have to have a reasonable balance) , just hoping that this might help him get

something good out of his coil with his existing coil design.  He could try to

use more primary turns, but with 270 turns on his secondary, the higher cap

rating seemed to me to be a better adjustment.  It would also allow him to use

twice the power input to his coil, which it seems quite capable of handling

(he was only using a single 15kV, 30ma neon and has others available).  The

biggest problem with this is the possibility of arcs forming between the top

and bottom of the secondary, but that can be dealt with by using a large

toroid and/or a variac to control the output.

I realize the power loss of the I^2R, and use 1/2" tubing on my primary

and 1" wide copper strips for all my primary connections.  I have not noticed

any heating problems using this.  I also use series spark gaps made from

silver relay contacts rated for more than 150kW .  I have had no pitting

problems with these, even after 30 minute run times.  I will probably go for

better quenching as soon as I get all other parameters optimized, but these

work well for now.

Finally,  I designed my secondary to be just under 600 turns of 19g wire on

a 5.5" O.D. pvc pipe.  Using two 15kV, 30 ma neon and a 0.0115uF cap(measured

value),  I get tuning at about 6 3/4 turns.  While it is sensitive, it does

seems to work reasonably well and the square root of the ratio of inductances

is probably similar to those using 1000 turn secondaries on 4" pvc and

primaries with 15 or so turns.

Note:  Yesterday I replaced the 4X16 toroid with a 6X20 and retuned my

coil.  I got at least 12" longer sparks than before with just this simple

change.  It only added about one turn to my primary.  I plan to increase my

primary cap by about 20% and see what results(tuning probably at about 5

turns).   I will post the results of this when I get a chance.

Happy coiling to all,

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Im a NEWBIE,,,help!!!

Date: Mon, 30 Mar 1998 13:44:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 30 Mar 1998 18:44:18 +0500

Subject: Re: Im a NEWBIE,,,help!!!

On Sun, 29 Mar 1998 12:38:12 -0500 In Tesla-2

PecosBill <metalhed@netrom.com> wrote;

> Does it matter if the wires are turned counter clockwise or clockwise

> help!!!

First Bill, welcome to the world of Tesla coils! Now for the issue of

winding direction, when the magnetic field of the primary is either

building  or collapsing that magnetic field will cut through the turns on

the secondary and induce a voltage in the secondary coils. If the secondary

turns were wound in the same direction as the primary the induced voltage

in the secondary would be of opposite polarity to that in the primary, and

if the secondary were wound in the opposite direction of the primary it

would induce a voltage in the secondary of the same polarity as the

primary. So unless you connect your secondary to the primary ( not wise) it

won't make any difference! Now why is it not wise to connect the secondary

to the primary, well for starters it would give the 60 Hz line a direct

path to your out put terminal and if you touched it you could get fried by

the 60 Hz current! Also if you were to connect the secondary to the primary

(this is called an Oudin coil configuration) THEN the direction of the

turns in the primary and secondary MUST be in the same direction or else

the currents in the two coils would buck each other and you would get

deminished output. You would get Vsecondary - Vprimary instead of

Vsecondary + Vprimary. Be wise though and avoid the Oudin coil

configuration! Ground your secondary, the life you save may be your own!

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 2 Neons available

Date: Mon, 30 Mar 1998 16:02:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 30 Mar 1998 11:02:07 -0500

Subject: 2 Neons available

I have 2 12kv/30ma Jefferson Electric neons if anyone is interested.

Make me an offer.

cya,

-tom

Wife:"Honey, what have you been building in there all day?"

Me:  "Um, It's a house cleaning robot"

Wife:"Uh...Oh. I guess that's ok then."

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Resonate Freq's

Date: Mon, 30 Mar 1998 16:55:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 30 Mar 1998 10:55:27 -0600

Subject: Resonate Freq's

Organization: MailExcite  (http://www.mailexcite.com:80)

A few weeks ago the arcstarter and I were using a frequency gen and counter to

determine the resonate frequency of our coil. In the process of this determination

we fed the primary circuit .55 volts and observed a voltage on the secondary.

The secondary voltage registered 22.1 volts. This represents a 40x the primary voltage.

Is it correct to assume, if the tank circuit is feed 16.620 kv. the secondary will be 40x

that or 664.8 kv.

Thanks

Bill & Paul MSOO (Mad Scientist of Ohio)Branch office in Northern Kentucky

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Large vs small primary L

Cc:

Bcc:

X-Attachments:

Arcstarter Bill and HollMike and others.

You both made valid points concerning primary inductance.

Allow me to add a few more comments along those lines.

Bill's analysis is correct. Low primary inductance

yields low impedance and high currents. These high

currents account for higher losses. High primary inductance

results in lower losses. But there are other factors to also

consider.

All the tank circuit wiring accounts for some stray

inductance. Hopefully not very much. But if your

primary coil has only a few turns and rather low inductance

then the off axis stray inductance can account for a significant

portion of the overall tank circuit inductance. This off axis

inductance contributes nothing to coil performance. In fact

it detracts from the performance.

A higher inductance primary will be less affected by stray inductance.

Still it is a worthwhile effort to minimize inductance in the tank circuit.

This means low inductance wiring and a minimum of connections.

Those using many parallel bottles in SW caps have this problem.

All those connections add stray inductance.

Flat plate caps exhibit the lowest inductance.

A high inductance primary also aids gap operation by increasing

the surge impedance. Since gap losses account for most of the system

losses anything that can be done to lower losses here will pay big

dividends. If your gap isnt quite what it should be then a high

inductance primary will help.

As Mike Hollingsworth pointed out the most important criteria

that the primary coil possess is that it resonates with the primary

cap at the secondary resonant frequency. All other considerations

are of less importance. A high inductance primary does no good

if it doesnt resonate with the main cap at the proper frequency.

Ive long been a fan of the high inductance low capacitance

primary circuit. It allows the use of smaller less expensive

capacitors that require smaller less expensive transformers

to charge them.

Comments, criticism, sarcasm and cash donations welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: waldos bottle caps

Cc:

Bcc:

X-Attachments:

At 04:48 PM 3/30/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Mon, 30 Mar 1998 08:48:34 -0800

>Subject: Re: Tuning Question

>Organization: Self

>

>Bill,

>Maybe you did... TWO wine bottles...?

>I am working on a tub of 14 40oz Bud bottles; I calc I need 1 more tub

>to get even close to the req'd calc'd C!

>But, like I said, "If I'm wrong, what the hell?"

>

>Gene

Gene how much capacitance you looking for? Im sure you probably

said in an earlier post but my feeble brain cant store very

much information these days.

Back when I was using SW bottle caps using 12oz beer

bottles (Leinenkugels) I was getting about 725 pf per bottle.

I would guess you are getting around 1500 pf per 40 oz bottle.

14 X 1500 pf is .021 mfd. Thats quite a bit of capacitance.

So again how much capacitance do you think you need and

what is your power supply? (voltage and current)

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large vs small primary L

Date: Mon, 30 Mar 1998 21:52:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 31 Mar 1998 02:52:02 +0500

Subject: Re: Large vs small primary L

On Mon, 30 Mar 1998 20:13:41 -0600 In Tesla-2

Mike Hammer <mhammer@misslink.net> wrote;

>  Arcstarter Bill and HollMike and others.

>

> You both made valid points concerning primary inductance.

> Allow me to add a few more comments along those lines.

>

> Bill's analysis is correct. Low primary inductance

> yields low impedance and high currents. These high

> currents account for higher losses. High primary inductance

> results in lower losses. But there are other factors to also

> consider...

I agree with every point you made but also point out that since;

Vsecondary = Vprimary * sqrt(Lsecondary/Lprimary)

or since at resonance XC = XL

Vsecondary = Vprimary * sqrt(Cprimary/Csecondary)

Which of course means that for a given secondary you will get a higher

voltage output with a smaller inductance primary. The larger capacitor

in the primary required by the smaller inductance again means higher

voltage output. So taking things to their ultimate limit would mean that to

get the maximum output from a Tesla coil would require that;

1. the secondary have as low a distributed capacitance as possible.

2. the secondary have as high an inductance as possible.

3. the primary have as large a capacitance as possible.

4. the primary have as low an inductance as possible.

Of course it is a given that both primary and secondary should have as low

a resistance as possible. Granted that the higher Q system will have very

sharp tuning, but the reward for this delicate tuning is higher output.

I would say that as long as the increased power is yielding a greater

increase in output than the increase in inherent losses, that is the way to

go. BTW if I remember correctly the majority of Dr. Tesla's coils had only

one or two turns on the primary.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary wire

Date: Mon, 30 Mar 1998 22:03:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Mon, 30 Mar 1998 16:03:49 -0600

Subject: Secondary wire

Hello all,

I saw a page someplace where they were selling magnet wire. Odd spools,

left over amounts etc.  I can't find it now.  Does this ring a bell with

anybody???

Thanks,

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary wire

Cc:

Bcc:

X-Attachments:

At 10:03 PM 3/30/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Mon, 30 Mar 1998 16:03:49 -0600

>Subject: Secondary wire

>

>Hello all,

>

>I saw a page someplace where they were selling magnet wire. Odd spools, =

>left over amounts etc.  I can't find it now.  Does this ring a bell with =

>anybody???

>

>Thanks,

>

>Roy Minnich

>

Roy,

That might possibly be Gary Weaver. Gary has been selling

respooled AWG 24 for a little while and several people have

purchased wire from him including me.

His page URL is: http://home.earthlink.net/~gweaver/

Gary's email address is: gweaver@earthlink.net

Ive known Gary for quite a while so if you mention

my name he will probably raise the price. :)

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 00:04:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Mon, 30 Mar 1998 19:04:19 EST

Subject: Re: waldos bottle caps

Mike, I agree with your question. I'm using 40 oz. bottles myself and 5 in

series gives about .0045uf  & 6 yields about .0058 (about .00098 per bottle)

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My tesla coil page

Date: Tue, 31 Mar 1998 00:21:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Mon, 30 Mar 1998 19:21:10 EST

Subject: My tesla coil page

Its not much but it has a few cool PICS, if you have time to spare drop by.

I made it yesterday, so given time it will eventually be good.  :)

members.aol.com/tesla00

Erik Schulz

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: junk yard rummageing

Date: Tue, 31 Mar 1998 02:24:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 30 Mar 1998 21:24:27 EST

Subject: Re: junk yard rummaging

Gene,

Thanks a bunch!

Mike

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 03:27:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 30 Mar 1998 19:27:55 -0800

Subject: Re: waldos bottle caps

Organization: Self

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 30 Mar 1998 15:38:19 -0600

> Subject: waldos bottle caps

>

> At 04:48 PM 3/30/98 GMT, you wrote:

> >From: Gene Waldo <gwaldo@1stnetusa.com>

> >Date: Mon, 30 Mar 1998 08:48:34 -0800

> >Subject: Re: Tuning Question

> >Organization: Self

> >

> Gene, how much capacitance you looking for?

>

> Back when I was using SW bottle caps using 12oz beer

> bottles (Leinenkugels) I was getting about 725 pf per bottle.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike, Well, now that you have asked, I've been hoping for someone to rub

my numbers against!

The 40 oz. bottle has: T=.0999", D=3.581"   &

C=11.25" x H=7.25" = 81.456"^2 + (r^2=3.204 x pi =

10.06)=91.626"^2       of 'foil/immersion' area per bottle.  T(hickness)

= .0999", avg.

T x 1 meg = 218,440*  I figure @ 300v/mil x 90 mills =

27kv          punch-thru.

Using:

.0884 X  91.626

uf = ----------------- = .000037 x 14 bottles = .005,

218,440*

which is close to our calculated req'd Capacitance, for a ~23" x 4" of

24 ga. 2ndary.  15kv, .03A Nxmfr, w/all the chokes and gaps.  Flat Pri

of ~15 turns of .312" Cu tube.

Where did I go wrong? Maybe it's having to empty all those damn BOTTLES!

But, like I said, "What the hell?"  We can easily disconnect bunches of

'em!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 03:34:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 30 Mar 1998 19:34:18 -0800

Subject: Re: waldos bottle caps

Organization: Self

Tesla-2 wrote:

>

> From: Dsurfr <Dsurfr@aol.com>

> Date: Mon, 30 Mar 1998 19:04:19 EST

> Subject: Re: waldos bottle caps

>

> Mike, I agree with your question. I'm using 40 oz. bottles myself and 5 in

> series gives about .0045uf  & 6 yields about .0058 (about .00098 per bottle)

> Jim

Jim,

"series"?  My notes say parallel.....  or did I miss that one, too?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tuning Question

Date: Tue, 31 Mar 1998 03:37:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 30 Mar 1998 22:37:06 EST

Subject: Re: Tuning Question

Bill,

Have you measured your caps?  I have done no measurements for wine bottles,

but it seems like you don't have enough of them.  You should have about

0.0066uF and I don't know, but two wine bottles doesn't seem like enough to

get this.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Resonate Freq's

Date: Tue, 31 Mar 1998 04:20:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 30 Mar 1998 20:20:48 -0800

Subject: Re: Resonate Freq's

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 30 Mar 1998 10:55:27 -0600

> Subject: Resonate Freq's

> Organization: MailExcite  (http://www.mailexcite.com:80)

>

> A few weeks ago the arcstarter and I were using a frequency gen and counter to

> determine the resonate frequency of our coil. In the process of this determination

> we fed the primary circuit .55 volts and observed a voltage on the secondary.

> The secondary voltage registered 22.1 volts. This represents a 40x the primary voltage.

> Is it correct to assume, if the tank circuit is feed 16.620 kv. the secondary will be

> 40x that or 664.8 kv.

>

> Thanks

>

> Bill & Paul MSOO (Mad Scientist of Ohio)Branch office in Northern Kentucky

> ---

> Einigkeit und Freiheit

>

Bill, Paul and all,

Basically yes! If you could excite the base with 16.62 kV (from a

relatively low impedance voltage source), you would probably get more

than that until something broke down (the air at the top, between turns,

etc).

The Q of a typical well constructed secondary coil is typically between

150 - 250. However, this "looks" like a series RLC resonant circuit. At

resonance, the effective impedance seen by the driving source will

collapse to the AC resistance of the secondary. Since this may only be a

couple of hundred ohms, your 16 kV source will need to be able to

provide about 80 amps of current.

I suspect the reason you saw only about a 40:1 rise was due to the

relatively high impedance of your signal generator "spoiling" the Q of

the overall system.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coating

Date: Tue, 31 Mar 1998 06:03:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Walt Kulpa <gwkulpa@earthlink.net>

Date: Mon, 30 Mar 1998 22:03:10 -0800

Subject: Re: secondary coating

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 28 Mar 1998 16:28:48 -0600

> Subject: Re: secondary coating

>

> At 06:39 PM 3/28/98 GMT, you wrote:

> >From: Dsurfr <Dsurfr@aol.com>

> >Date: Sat, 28 Mar 1998 13:39:20 EST

> >Subject: secondary coating

> >

> >What's the best thing to use for sealing a secondary coil?

I noticed that McMaster-Carr carries a spray product which is supposed to

be for re-coating motor windings. per the description in the book, it

seems to be the same type of coating used on magnet wire. I have wondered

if this material would be particularly suitable? It is very reasonable, 3

or 4 bucks per can.  Walt Kulpa

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 12:50:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 31 Mar 1998 06:50:31 -0600

Subject: Re: waldos bottle caps

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Mon, 30 Mar 1998 19:27:55 -0800

> Subject: Re: waldos bottle caps

> Organization: Self

>

> Tesla-2 wrote:

> >

> > From: Mike Hammer <mhammer@misslink.net>

> > Date: Mon, 30 Mar 1998 15:38:19 -0600

> > Subject: waldos bottle caps

> >

> > At 04:48 PM 3/30/98 GMT, you wrote:

> > >From: Gene Waldo <gwaldo@1stnetusa.com>

> > >Date: Mon, 30 Mar 1998 08:48:34 -0800

> > >Subject: Re: Tuning Question

> > >Organization: Self

> > >

> > Gene, how much capacitance you looking for?

> >

> > Back when I was using SW bottle caps using 12oz beer

> > bottles (Leinenkugels) I was getting about 725 pf per bottle.

> >

> > TTYL!

> > Mike Hammer

> > mhammer@misslink.net

> >

> Mike, Well, now that you have asked, I've been hoping for someone to rub

> my numbers against!

>

> The 40 oz. bottle has: T=.0999", D=3.581"   &

>     C=11.25" x H=7.25" = 81.456"^2 + (r^2=3.204 x pi =

> 10.06)=91.626"^2       of 'foil/immersion' area per bottle.  T(hickness)

> = .0999", avg.

>          T x 1 meg = 218,440*  I figure @ 300v/mil x 90 mills =

> 27kv          punch-thru.

>

> Using:

>                     .0884 X  91.626

>               uf = ----------------- = .000037 x 14 bottles = .005,

>                        218,440*

>

> which is close to our calculated req'd Capacitance, for a ~23" x 4" of

> 24 ga. 2ndary.  15kv, .03A Nxmfr, w/all the chokes and gaps.  Flat Pri

> of ~15 turns of .312" Cu tube.

>

> Where did I go wrong? Maybe it's having to empty all those damn BOTTLES!

> But, like I said, "What the hell?"  We can easily disconnect bunches of

> 'em!

>

> Gene Waldo

>

Gene,

There's a problem in the formula you're using. The denominator in the

capacitance equation should simply be the average dielectric thickness.

The dielectric strength of the material, while important, doesn't figure

into the capacitance calculation. I get about 1450 pf. The real

"wildcard" is the dielectric constant for glass which can easily span 5

- 8 (or more) for commonly used glass formulations:

Dielectric Constant (k):        7 (Approx.)

Jar Diameter (D)                3.6   Inches

Jar Height Used  (H)            7.25  Inches

Thickness of Jar (T)            0.1   Inches

Number of caps in parallel (N)  14

Area (A) = Pi*D*H + Pi*(D/2)^2 = 92.2 Square Inches

Plugging the above numbers into the capacitance equation:

C = 0.2248*k*A/T  (per bottle) = 1450 pF

Ctot = 14*C = 0.023 uF

Hope this helps!

--Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coating

Date: Tue, 31 Mar 1998 12:54:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 31 Mar 1998 06:54:00 -0600

Subject: Re: secondary coating

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Walt Kulpa <gwkulpa@earthlink.net>

> Date: Mon, 30 Mar 1998 22:03:10 -0800

> Subject: Re: secondary coating

>

> Tesla-2 wrote:

> >

> > From: Mike Hammer <mhammer@misslink.net>

> > Date: Sat, 28 Mar 1998 16:28:48 -0600

> > Subject: Re: secondary coating

> >

> > At 06:39 PM 3/28/98 GMT, you wrote:

> > >From: Dsurfr <Dsurfr@aol.com>

> > >Date: Sat, 28 Mar 1998 13:39:20 EST

> > >Subject: secondary coating

> > >

> > >What's the best thing to use for sealing a secondary coil?

>

> I noticed that McMaster-Carr carries a spray product which is supposed to

> be for re-coating motor windings. per the description in the book, it

> seems to be the same type of coating used on magnet wire. I have wondered

> if this material would be particularly suitable? It is very reasonable, 3

> or 4 bucks per can.  Walt Kulpa

>

Walt,

Please try it and let us know! Sounds like it'd work just fine.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 13:13:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Tue, 31 Mar 1998 08:13:59 EST

Subject: Re: waldos bottle caps

Sorry Gene, "parallel" is correct.  Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coating

Date: Tue, 31 Mar 1998 13:27:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Tue, 31 Mar 1998 08:27:53 EST

Subject: Re: secondary coating

Walt,

I have been using Krylon insulating varnish that I got through Grainger

catalog.  It has a 2000v/ mil dielectric strength.  This might be the same as

what is in the M-C catalog, but I haven't looked for it there. This varnish

seems to work well.  I use about ten or fifteen coats to cover my coils.

Hope this is useful,

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: secondary coating

Date: Tue, 31 Mar 1998 16:58:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 31 Mar 1998 21:58:32 +0500

Subject: Re: secondary coating

On Mon, 30 Mar 1998 22:03:10 -0800 In Tesla-2

Walt Kulpa <gwkulpa@earthlink.net> wrote;

> I noticed that McMaster-Carr carries a spray product which is supposed to

> be for re-coating motor windings. per the description in the book, it

> seems to be the same type of coating used on magnet wire. I have wondered

> if this material would be particularly suitable? It is very reasonable, 3

> or 4 bucks per can.

Hi Walt, I'm looking at my 1998 McMASTER-CARR catalog on page 1574 under

electrical maintenance areosols;

"RED INSULATING VARNISH - Use this varnish to restore dielectric

strength to armatures and frayed insulation. Adheres to metal and

insulating components. Varnish is resistant to acids, oils, alkalies,

moisture, and heat. Insulation rating is 1200 volt/mil cured wet,

2100 volt/mil cured dry. Varnish is supplied in 12 - oz. can.

No. 7437K16.......................NET EACH 1-11 $4.19  12-up $3.60"

If this is what you are talking about, I think it is very similar to the

varnish Mike Hollingsworth said he was getting from Grainger's, I think

he said it was a Krylon product. As for myself my first few coils were

coated with shellac 5 lb. cut. The rest of my coils all recieved several

coats of Polyurethane made by ZAR.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Large vs small primary L

Cc:

Bcc:

X-Attachments:

At 09:52 PM 3/30/98 GMT, you wrote:

>From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

>Date: Tue, 31 Mar 1998 02:52:02 +0500

>Subject: Re: Large vs small primary L

>

>On Mon, 30 Mar 1998 20:13:41 -0600 In Tesla-2

>Mike Hammer <mhammer@misslink.net> wrote;

>

>>  Arcstarter Bill and HollMike and others.

>>

>> You both made valid points concerning primary inductance.

>> Allow me to add a few more comments along those lines.

>>

>> Bill's analysis is correct. Low primary inductance

>> yields low impedance and high currents. These high

>> currents account for higher losses. High primary inductance

>> results in lower losses. But there are other factors to also

>> consider...

>

>I agree with every point you made but also point out that since;

>

>     Vsecondary = Vprimary * sqrt(Lsecondary/Lprimary)

>

>              or since at resonance XC = XL

>

>     Vsecondary = Vprimary * sqrt(Cprimary/Csecondary)

>

>Which of course means that for a given secondary you will get a higher

>voltage output with a smaller inductance primary. The larger capacitor

>in the primary required by the smaller inductance again means higher

>voltage output. So taking things to their ultimate limit would mean that to

>get the maximum output from a Tesla coil would require that;

>

>    1. the secondary have as low a distributed capacitance as possible.

>    2. the secondary have as high an inductance as possible.

>    3. the primary have as large a capacitance as possible.

>    4. the primary have as low an inductance as possible.

>

I also agree with the points you make here and thats exactly

how the math works out. But, as Im sure you'll agree, Tesla

coils often defy mathematical attempts to predict their behaviour.

While in theory what you outlined above is true in practice

I dont think its valid. In my opinion and in my experience

high L low C coils perform better than high C low L coils.

I believe the increase in I2R losses and gap losses in low

L coils more than make up for the theoretical increase.

Perhaps with a very low loss system with a truly exceptional

gap system this might apply but how many of us can say our coils

fall into that category?

A good example is John Freau's coil. He has had 42 inches

of output from his coil with a single 12kv/30ma neon.

There is really nothing remarkable about John's coil

EXCEPT for a 34 turn primary coil.

My own coil has delivered more output for less input

than most others with similar coils. I get 54 inches

of output with just over 1200 VA. I have a 17 turn

primary and the more turns I have tapped in the better

it works.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: waldos bottle caps

Date: Tue, 31 Mar 1998 19:17:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 31 Mar 1998 11:17:34 -0800

Subject: Re: waldos bottle caps

Organization: Self

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Tue, 31 Mar 1998 06:50:31 -0600

> Subject: Re: waldos bottle caps

> Organization: Stoneridge Engineering

>

> Gene,

> There's a problem in the formula you're using....

>

>      Dielectric Constant (k):        7 (Approx.)

>      Jar Diameter (D)                3.6   Inches

>      Jar Height Used  (H)            7.25  Inches

>      Thickness of Jar (T)            0.1   Inches

>      Number of caps in parallel (N)  14

>      Area (A) = Pi*D*H + Pi*(D/2)^2 = 92.2 Square Inches

>

>    Plugging the above numbers into the capacitance equation:

>      C = 0.2248*k*A/T  (per bottle) = 1450 pF

>      Ctot = 14*C = 0.023 uF

>

> Hope this helps!

>

> --Bert --

Bert,

That helps a lot!  I KNOW, however that someone published the eq w/the

denominator as:  T*1,000,000! I couldn't have made that up!  Could I?

Any way, I'm now relieved of the need to continue consuming BUD!  Really

a shitty beer! Vive la SAM ADAMS, SIERRA NEVADA, ANCHOR, PETE'S, and

some of the other serious brewers!

As to the rotary gap: still working on it! My old Polaroid is so old

that Polaroid only makes EXPIRED film for it, I think... couldn't find

any, any how.

So I finally bit the bullet and should be emailing you a *photo* along

with the other particulars! Soon, I hope, as soon as it quits raining

here!

>From the look of the machine, and my gut feel on their pricing policy

(Irv goes out and looks at the gear, and makes a snap pricing), as a

guess, I'm betting on about $50-80...  Aside from the 2 missing

electrodes, it looks all there..  I'll be able to roughly 'Blue Print'

the parts needed, for you, too.

Gene

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Variac substitute

Date: Wed,  1 Apr 1998 02:37:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Tue, 31 Mar 1998 21:37:52 EST

Subject: Variac substitute

My first coil (in quite a long time) is just about finished. I'm using a 15KV

30 ma neon xformer. Most of the postings & advice I've received often mention

a variac (I'm assuming this is a variable power transformer) should be placed

between the neon xformer & the 115 V input to it. Is this absolutely essential

or can I safely run without one (as I did many years ago)? Also on the same

topic, I'm an architect & we often have "cove lighting" of neon tubes that are

"dimmed" by a rheostat control. Could I substitute one of these instead of

spending an extra $100 or so on a variac? I don't want to fry my xformer &

want the best output possible from it so I'm willing to wait a bit longer &

buy one if need be. Thanks again for all the advice to my "Newbie" questions.

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Help !

Date: Wed,  1 Apr 1998 12:58:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 01 Apr 1998 06:58:24 -0600

Subject: Help !

Organization: MailExcite  (http://www.mailexcite.com:80)

Please bare with me a moment. I have a non-tesla project which I am working. It is

however a high voltage experment which requires a little help. I posted it to Tesla-2

because I realize there is a vast repository of electrical knowledge out there.

I need a high voltage power supply capable of delivering 30-40 kv at a minimum of 500w.

Any Ideas ? It may not be something you would want to post back to tesla-2, you can

e-mail me direct.

Fleischwulf@mailexcite.com

PS: If anyone has something they could sell me that would do the job please let me

know. There is no upper limit on wattage only a minimum requirement.

Thanks for all the help

Paul M

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Variac substitute

Cc:

Bcc:

X-Attachments:

At 02:37 AM 4/1/98 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Tue, 31 Mar 1998 21:37:52 EST

>Subject: Variac substitute

>

>My first coil (in quite a long time) is just about finished. I'm using a 15KV

>30 ma neon xformer. Most of the postings & advice I've received often mention

>a variac (I'm assuming this is a variable power transformer) should be placed

>between the neon xformer & the 115 V input to it. Is this absolutely essential

>or can I safely run without one (as I did many years ago)? Also on the same

>topic, I'm an architect & we often have "cove lighting" of neon tubes that are

>"dimmed" by a rheostat control. Could I substitute one of these instead of

>spending an extra $100 or so on a variac? I don't want to fry my xformer &

>want the best output possible from it so I'm willing to wait a bit longer &

>buy one if need be. Thanks again for all the advice to my "Newbie" questions.

>Jim

>

Jim,

A variac is not absolutely necessary. You can just throw

the switch on your neon and have at it.

However as you parallel additional neons and increase your

power level a variac will lower the stress on your coil components

by allowing you to increase the voltage level gradually.

An additional benefit is that most variacs can be wired to

give a 0-140 VAC output (or 0-280 for a 240V variac) with

120 VAC input. Running your neons with 140 VAC input will

give you a little more voltage and current output. This does

not seem to hurt the neons in any way. I do it all the time.

A rheostat is not the same thing and isnt really what you

want to use. A rheostat is a variable resistor and as such

will dissipate a considerable amount of heat and introduce loss.

The most important thing you can do to protect your neon

from damage is to put a safety gap on your neon right at the

HV terminals. Mount the safety right on top of the neon and make

it a permanent accessory.

Keep your eyes open. Variacs show up every now and then

on this list. Check out a hamfest in your area for the best prices.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: How do i make a terminal.???

Date: Wed,  1 Apr 1998 21:40:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "PecosBill" <metalhed@netrom.com>

Date: Wed, 1 Apr 1998 16:40:51 -0500

Subject: How do i make a terminal.???

Help!!!!

How do i make a terminal and what type should i use????

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: How do i make a terminal.???

Cc:

Bcc:

X-Attachments:

At 09:40 PM 4/1/98 GMT, you wrote:

>From: "PecosBill" <metalhed@netrom.com>

>Date: Wed, 1 Apr 1998 16:40:51 -0500

>Subject: How do i make a terminal.???

>

>Help!!!!

>How do i make a terminal and what type should i use????

>

The type and size of your discharge terminal depends on

the size and power level of your coil. Many beginners

make the mistake of using a too large toroid right off the bat.

The large toroid makes tuning more difficult especially

when you arent completely sure how to find the best tune.

So I recommend a small spherical terminal for your first effort.

The idea isnt to create long sparks but to provide a terminal

that breaks out easily at low powers or when your coil is mistuned.

I had great luck using one of those copper toilet tank floats.

They are only a couple bucks from the hardware store and

have a convenient 1/4 20 thread for easy attachment.

Try also a 5 or 6 inch diameter styrofoam ball from the craft

store covered with aluminum foil.

Once your coil is working well you will want to try a toroid

to get longer sparks. Make sure that you have enough extra turns

on your primary coil to tune to the lower frequency a larger

toroid will provide.

The quickest easiest way to build a toroid is with flexible

aluminum dryer duct. This is commonly available in 3,4 and 6 inch

diameters. Get also a pair of aluminum pie pans.

Form the duct into a ring by joining the free ends and

securing with a little tape. Cover the joint with aluminum tape

to make the joint conductive and smooth it out the best you can.

Bolt together the two pie pans bottom to bottom inside the

ring formed by the duct. As you tighten the bolt holding the pie

pans together and draw them together they will grip the duct

ring and make a solid unit.

The 3 inch duct will make about a 15 X 3 inch toroid. This is

a good starter toroid and the best for smaller coils.

The 4 inch duct forms a 16 X 4 inch toroid that works well

on a 4 inch secondary.

The 6 inch duct forms about a 19 or 20 X 6 toroid thats good

for medium to high power on a 4 inch coil or 6 inch coil.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Help !

Cc:

Bcc:

X-Attachments:

At 12:58 PM 4/1/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Wed, 01 Apr 1998 06:58:24 -0600

>Subject: Help !

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>Please bear with me a moment. I have a non-tesla project which I am working.

>It is however a high voltage experment which requires a little help.

>I posted it to Tesla-2 because I realize there is a vast repository of electrical

>knowledge out there.

Seems to be at least partly relevant so Ill reply here.

> I need a high voltage power supply capable of delivering 30-40 kv at a minimum of

>500w. Any Ideas ?

Paul,

How about a 15kv neon (or other suitable transformer) feeding a diode

voltage doubler circuit? That could theoritically deliver

2.828 X 15kv = 42420 VDC but it would end up being somewhat less under load.

It would take some hefty strings of diodes and some good filter caps.

Perhaps some of the goodies Arcstarter Bill scavenged from that X-ray machine.

Just a thought off the top of my head.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Increased capacitance

Date: Thu,  2 Apr 1998 12:42:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 02 Apr 1998 06:42:38 -0600

Subject: Increased capacitance

Organization: MailExcite  (http://www.mailexcite.com:80)

I have been thinking seriously about increasing the capacitance of my primary tank

circuit. I am currently using a bank of caps which gives me a total of .026uf.

I've been thinking of raising this to .036 uf. I have the power to drive it. Any

ideas about coil preformance ? The other thing I have been toying with is a redesign

of my primary. I would like to design a low inductance primary. Are there any thoughts

on this ?

Thanks for everything

Paul M.

--

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small Tesla Coil With Negative Ion Generator Transformer

Cc:

Bcc:

X-Attachments:

At 11:25 PM 2/19/98 GMT, you wrote:

>From: chrisw@kellnet.com (Chris Wendorff)

>Date: Thu, 19 Feb 98 18:25:57 EST

>Subject: Small Tesla Coil With Negative Ion Generator Transformer

>

>Hello Everyone. I am a newbie coiler and I was wondering if anyone had any

>experience with using a negative ion generator transformer for a Tesla Coil.

>I have seen that the output on these are generally around 10 - 15 KV DC at 5

>microamps. I have several high voltage capacitors to use for this. Any ideas

>on how large the secondary and primary coils should be and what type of wire

>to use for a small coil.

>

>Thanks,

>

>

>Chris Wendorff

>

Chris,

In my opinion thats just not a workable solution.

A Tesla coil is not just a voltage operated device.

The output streamers depend on the total amount of

power delivered to the secondary. With the exceedingly

low output current of your ion generator the total

power is next to nothing. .075 watts. And thats what

you could expect for output. Next to nothing.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: How do i make a terminal.???

Date: Thu,  2 Apr 1998 19:03:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Thu, 02 Apr 1998 11:03:43 PST

Subject: Re: How do i make a terminal.???

You wrote:

>Help!!!!

>How do i make a terminal and what type should i use????

This is probably one of the big problems for coilers.  I can tell you

what we did...

First - we ran w/o a terminal.  Through tuning - we were able to get

about 2-3 inches of spark to a grounded wire, but no free discharge to

air.

Next - we put on a large steel potlid, about 10 inches in diameter.

After a bit of retuning we were getting a "gas-flame" effect - where

three inch streamers were erupting (to free air) from  the outer

perimeter of the pot lid.  Arc to a grounded wire was about 6-7 inches.

Ozone production was immense!

Next we used a basketball covered in aluminum foil.  This produced 24

inches to a grounded object.

Finally we ended up making various terminals out of pie pans and dryer

duct.  I don't know the final dimensions by memory.

Sometimes we put on two or three toroids all at once.  After a bit of

retuning we get generally more arc when using more top terminal.

We tried a large (18 inch) aluminum sphere once, but it arced from the

bottom of the sphere down to the secondary and primary.  The toroidal

terminals never dod this, so we put the ball away.  (This was with more

power than the other experiments)

All experiments (except for the aluminum sphere) were powered by a 7500

volt 120 ma neon.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Progress report

Date: Thu,  2 Apr 1998 20:52:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Thu, 2 Apr 1998 15:52:24 -0500

Subject: Progress report

I was just reading the archives and happened to notice how many people

show up with intentions of building a large coil (for a first one) and

then disappear. I guess I'd better get to work on my primary before I

end up like this!

My first coil progress report:

4.5" x 21.75" Secondary wrapped with #24           done

6 Saltwater bottle caps (for simplicity)           done

2 15KV/30ma transformers (first one, then both)    ok

Primary 14 turns of .25" Cu tubing (flat spiral)   not done

Spark Gap                                          not yet!

Chokes                                             not yet!

Hopefully soon!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: rf ground

Date: Thu,  2 Apr 1998 21:18:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Thu, 02 Apr 1998 13:18:00 -0800

Subject: Re: rf ground

Would a well point make a good rf ground? Its 1-1/2" diam. and 8 feet down.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: How do i make a terminal.???

Cc:

Bcc:

X-Attachments:

At 09:40 PM 4/1/98 GMT, you wrote:

>From: "PecosBill" <metalhed@netrom.com>

>Date: Wed, 1 Apr 1998 16:40:51 -0500

>Subject: How do i make a terminal.???

>

>Help!!!!

>How do i make a terminal and what type should i use????

>

The type and size of your discharge terminal depends on

the size and power level of your coil. Many beginners

make the mistake of using a too large toroid right off the bat.

The large toroid makes tuning more difficult especially

when you arent completely sure how to find the best tune.

So I recommend a small spherical terminal for your first effort.

The idea isnt to create long sparks but to provide a terminal

that breaks out easily at low powers or when your coil is mistuned.

I had great luck using one of those copper toilet tank floats.

They are only a couple bucks from the hardware store and

have a convenient 1/4 20 thread for easy attachment.

Try also a 5 or 6 inch diameter styrofoam ball from the craft

store covered with aluminum foil.

Once your coil is working well you will want to try a toroid

to get longer sparks. Make sure that you have enough extra turns

on your primary coil to tune to the lower frequency a larger

toroid will provide.

The quickest easiest way to build a toroid is with flexible

aluminum dryer duct. This is commonly available in 3,4 and 6 inch

diameters. Get also a pair of aluminum pie pans.

Form the duct into a ring by joining the free ends and

securing with a little tape. Cover the joint with aluminum tape

to make the joint conductive and smooth it out the best you can.

Bolt together the two pie pans bottom to bottom inside the

ring formed by the duct. As you tighten the bolt holding the pie

pans together and draw them together they will grip the duct

ring and make a solid unit.

The 3 inch duct will make about a 15 X 3 inch toroid. This is

a good starter toroid and the best for smaller coils.

The 4 inch duct forms a 16 X 4 inch toroid that works well

on a 4 inch secondary.

The 6 inch duct forms about a 19 or 20 X 6 toroid thats good

for medium to high power on a 4 inch coil or 6 inch coil.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Increased capacitance

Cc:

Bcc:

X-Attachments:

At 12:42 PM 4/2/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Thu, 02 Apr 1998 06:42:38 -0600

>Subject: Increased capacitance

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>I have been thinking seriously about increasing the capacitance of my primary tank

>circuit. I am currently using a bank of caps which gives me a total of .026uf.

>I've been thinking of raising this to .036 uf. I have the power to drive it.

>Any ideas about coil performance ?

>The other thing I have been toying with is a redesign of my primary.

>I would like to design a low inductance primary. Are there any thoughts on this ?

Yea Im against the low inductance primary. A high inductance primary offers

so many advantages. The biggest advantage is improvement in gap operation

due to a higher surge impedance. Also a small primary tends to concentrate

energy in the lower 1/3 of the secondary. This is important if you are

pushing your coil for maximum output. You can easily overvolt your secondary.

If you want to use more primary capacitance and take advantage of the

larger bang size it would provide the way to do it is to lower your

operating frequency. Add a larger toroid to your secondary or make

a new lower frequency secondary. The lower frequency will allow

you to increase main cap size while still keeping lots of inductance

in the primary.

See my recent post and follow up titled "Large vs small primary L".

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rf ground

Cc:

Bcc:

X-Attachments:

At 09:18 PM 4/2/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Thu, 02 Apr 1998 13:18:00 -0800

>Subject: Re: rf ground

>

>

>

>Would a well point make a good rf ground? Its 1-1/2" diam. and 8 feet down.

>

Yes I would think so. Especially if the well still has water in it

and that water is as hard as the well water we have around here.

Our water has so much dissolved minerals in it its almost a good conductor.

Make sure your ground lead from the well to your coil is a

heavy conductor and as short and as straight as possible.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: rf ground

Date: Fri,  3 Apr 1998 03:12:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Thu, 02 Apr 1998 19:12:49 -0800

Subject: Re: rf ground

Organization: EES,Co.

Tesla-2 wrote:

>

> From: mike nicholas <705728@ican.net>

> Date: Thu, 02 Apr 1998 13:18:00 -0800

> Subject: Re: rf ground

>

> Would a well point make a good rf ground? Its 1-1/2" diam. and 8 feet down.

Probably. Especially if it's still in contact with water laden soil. Try

measuring it using the page I put up at:http://www.expage.com/page/tesla

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Progress report

Date: Fri,  3 Apr 1998 03:59:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 2 Apr 1998 22:59:50 EST

Subject: Re: Progress report

Tom,

You might want to drink another sixpack.  I think you will need a few more

bottles for two neons.   You will want 0.01uF(and maybe a wee bit more).

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Does it Work?

Date: Fri,  3 Apr 1998 04:40:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Thu, 2 Apr 1998 21:40:55 -0700

Subject: Does it Work?

Hey all!

I just got my first-ever neon transformer rebuilt a few days ago. I ran

into problems (I posted them to this list, but I don't know if they ever

made it). There were no screws or clips holding the core together -- as far

as I could tell it was brazed. The two secondaries were right next to each

other, with the primary on the extreme side of it -- not between the two

secondaries as I thought they would be (I'm definitely no expert though).

Anyway, I couldn't take the core apart or do much of anything else, so I

decided I didn't have anything to lose and gave it power.

Now, electricity came out of the secondaries -- believe me, but not like

what I was expecting. When working with television flyback transformers,

the high-voltage arc creates a loud hissing sound and jumps about one-half

to three-quarters of an inch. When I brought the two secondary leads of my

neon transformer together, I had to get them about one-eighth of an inch

from each other to create an arc. However after the arc was started I could

pull them away about an inch and a half before the arc died. There was no

definite "spark," just a very obvious purple corona-like haze. It was very

quiet -- no noise. It was also quite hot. In fact, it vapourized the

insulation on the temporary awg 20 solid copper wire (only 300 volts) and

quickly melted the copper into a red-hot goo -- easily even while they were

about an inch and a half apart.

What am I to expect? Does this neon transformer work? The specs before I

tore it apart were: 6000 volt secondaries, 30 ma, 120 AC primary. I suspect

that it is much more than 30ma now, because I took out two (from what

looked like the only two) shunts out and the primary is about 1/4-inch

closer to the secondaries than it should be (I'm going to fix that.). What

about the voltage? Can anybody make a guesstimation on how high (or low) it

is? Will this work for my first, small Tesla coil?

Thanks in advance!

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Doorknob Caps

Date: Fri,  3 Apr 1998 04:52:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Thu, 2 Apr 1998 21:52:40 -0700

Subject: Doorknob Caps

Hey All!

For my first coil I was planing on using saltwater capacitors to start out

with. Then I got to thinking. What is wrong with "doorknob" caps out of

television sets? Couldn't you connect however-many-you-wanted together and

then submerge them in mineral oil to keep them from arcing -- or something

similar? It seems like nobody is using them. Is there any reason? Are even

"crummy" saltwater caps better? To me they would be more efficient than

saltwater caps (or dare I even mention rolled poly ones...), smaller,

lighter, and possibly cheaper. I noticed Bert Pool suggests them in one of

his text files. Does anybody else?

Curious,

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Doorknob Caps

Date: Fri,  3 Apr 1998 12:20:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 03 Apr 1998 06:20:07 -0600

Subject: Re: Doorknob Caps

Organization: MailExcite  (http://www.mailexcite.com:80)

Dan,

I can tell you from personel experience, you can not beat the poly cap's for tesla

coil use. Plate or rolled. There is always a lot of discussion on tesla-2 regarding

caps and I suppose that there always be. I have tried other dielectrics but the poly

beats them all. Poly caps require work, and I put it off until I had exhausted all

other possibilities. I wish I would have just built the poly caps first.

Paul M

---

Einigkeit und Freiheit

>From: Daniel Lockhart <molly@cmn.net>

>Date: Thu, 2 Apr 1998 21:52:40 -0700

>Subject: Doorknob Caps

>

>Hey All!

>

>For my first coil I was planing on using saltwater capacitors to start out

>with. Then I got to thinking. What is wrong with "doorknob" caps out of

>television sets? Couldn't you connect however-many-you-wanted together and

>then submerge them in mineral oil to keep them from arcing -- or something

>similar? It seems like nobody is using them. Is there any reason? Are even

>"crummy" saltwater caps better? To me they would be more efficient than

>saltwater caps (or dare I even mention rolled poly ones...), smaller,

>lighter, and possibly cheaper. I noticed Bert Pool suggests them in one of

>his text files. Does anybody else?

>

>Curious,

>

>Daniel Lockhart

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Does it Work?

Date: Fri,  3 Apr 1998 12:24:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 03 Apr 1998 06:24:04 -0600

Subject: Re: Does it Work?

Organization: MailExcite  (http://www.mailexcite.com:80)

Dan,

I've seen a lot of neons and this one sounds like 6kv to me at 30 ma.

Paul M.

---

Einigkeit und Freiheit

>From: Daniel Lockhart <molly@cmn.net>

>Date: Thu, 2 Apr 1998 21:40:55 -0700

>Subject: Does it Work?

>

>Hey all!

>

>I just got my first-ever neon transformer rebuilt a few days ago. I ran

>into problems (I posted them to this list, but I don't know if they ever

>made it). There were no screws or clips holding the core together -- as far

>as I could tell it was brazed. The two secondaries were right next to each

>other, with the primary on the extreme side of it -- not between the two

>secondaries as I thought they would be (I'm definitely no expert though).

>Anyway, I couldn't take the core apart or do much of anything else, so I

>decided I didn't have anything to lose and gave it power.

>

>Now, electricity came out of the secondaries -- believe me, but not like

>what I was expecting. When working with television flyback transformers,

>the high-voltage arc creates a loud hissing sound and jumps about one-half

>to three-quarters of an inch. When I brought the two secondary leads of my

>neon transformer together, I had to get them about one-eighth of an inch

>from each other to create an arc. However after the arc was started I could

>pull them away about an inch and a half before the arc died. There was no

>definite "spark," just a very obvious purple corona-like haze. It was very

>quiet -- no noise. It was also quite hot. In fact, it vapourized the

>insulation on the temporary awg 20 solid copper wire (only 300 volts) and

>quickly melted the copper into a red-hot goo -- easily even while they were

>about an inch and a half apart.

>

>What am I to expect? Does this neon transformer work? The specs before I

>tore it apart were: 6000 volt secondaries, 30 ma, 120 AC primary. I suspect

>that it is much more than 30ma now, because I took out two (from what

>looked like the only two) shunts out and the primary is about 1/4-inch

>closer to the secondaries than it should be (I'm going to fix that.). What

>about the voltage? Can anybody make a guesstimation on how high (or low) it

>is? Will this work for my first, small Tesla coil?

>

>Thanks in advance!

>

>Daniel Lockhart

>

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Doorknob Caps

Date: Fri,  3 Apr 1998 13:09:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Fri, 3 Apr 1998 08:09:31 EST

Subject: Re: Doorknob Caps

Daniel,

I am using a bunch of ceramic caps similar to the doorknob ones(they may be

doorknobs by definition, but not sure).  I connected 25 of them to two 1/16

inch thick copper plates for a capacitance of 0.0115uF.  I found that I had to

submerge them to prevent arcing between the plates.  They have a 30kVDC

rating.  I have had decent results using them, so I would say try it.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: surge inductance? Gap models?

Date: Fri,  3 Apr 1998 16:21:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Fri, 03 Apr 1998 08:21:07 PST

Subject: surge inductance? Gap models?

Mike Hammer-

You wrote:

>Yea Im against the low inductance primary. A high inductance primary offers

>so many advantages. The biggest advantage is improvement in gap operation

>due to a higher surge impedance. Also a small primary tends to concentrate

What do you mean - "surge impedance"? Are you describing the ability of

the inductor to "force" current through the gap even when the cap

voltage is zero??

Or - the larger the inductance, the smaller the peak currents in the

primary - which might help the gap quench faster (just guessing).

Does anyone have any rough models of an RQ spark gap?  I'd guess that a

constant voltage drop would be close - but can we take into effect the

negative incremental resistance as the current increases??  I'm trying

to get a feel how losses in the gap interact with energy stored in the

cap/inductance, etc.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Doorknob Caps

Cc:

Bcc:

X-Attachments:

At 04:52 AM 4/3/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Thu, 2 Apr 1998 21:52:40 -0700

>Subject: Doorknob Caps

>

>Hey All!

>

>For my first coil I was planing on using saltwater capacitors to start out

>with. Then I got to thinking. What is wrong with "doorknob" caps out of

>television sets? Couldn't you connect however-many-you-wanted together and

>then submerge them in mineral oil to keep them from arcing -- or something

>similar? It seems like nobody is using them. Is there any reason? Are even

>"crummy" saltwater caps better? To me they would be more efficient than

>saltwater caps (or dare I even mention rolled poly ones...), smaller,

>lighter, and possibly cheaper. I noticed Bert Pool suggests them in one of

>his text files. Does anybody else?

>

>Curious,

>

>Daniel Lockhart

>

Daniel,

Very few use door knob caps because they usually provide

rather poor performance. As do "crummy" salt water caps.

The problem with door knobs is the ceramic dielectric

has rather high RF dissipation factor. That means lots

of your precious primary energy is turned to heat. Since

door knobs are encapsulated the heat cant escape. A door

knob that feels warm on the outside may be near melt down

on the inside.

Home built caps using polyethylene or polypropylene

while much bulkier have practically zero RF dissipation.

Poly caps can be run for extended periods while remaining

stone cold.

Many people start out with SW caps or doorknobs but wish

they had started right out with a poly cap. They ARE that

much better.

Poly caps can be rolled up or built in a stacked flat

plate configuration. I prefer the flat plate as it offers

the highest performance but rolled ones work well too.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Does it Work?

Cc:

Bcc:

X-Attachments:

At 04:40 AM 4/3/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Thu, 2 Apr 1998 21:40:55 -0700

>Subject: Does it Work?

>

>Hey all!

>

>I just got my first-ever neon transformer rebuilt a few days ago. I ran

>into problems (I posted them to this list, but I don't know if they ever

>made it). There were no screws or clips holding the core together -- as far

>as I could tell it was brazed. The two secondaries were right next to each

>other, with the primary on the extreme side of it -- not between the two

>secondaries as I thought they would be (I'm definitely no expert though).

>Anyway, I couldn't take the core apart or do much of anything else, so I

>decided I didn't have anything to lose and gave it power.

Sounds like it might be a Transco neon. If not tell me what the

manufacturer is. Im always interested in the internals of neons Ive

never unpotted. Smaller neon cores are often welded while the larger

ones are riveted or bolted.

>

>Now, electricity came out of the secondaries -- believe me, but not like

>what I was expecting. When working with television flyback transformers,

>the high-voltage arc creates a loud hissing sound and jumps about one-half

>to three-quarters of an inch. When I brought the two secondary leads of my

>neon transformer together, I had to get them about one-eighth of an inch

>from each other to create an arc. However after the arc was started I could

>pull them away about an inch and a half before the arc died. There was no

>definite "spark," just a very obvious purple corona-like haze. It was very

>quiet -- no noise. It was also quite hot. In fact, it vapourized the

>insulation on the temporary awg 20 solid copper wire (only 300 volts) and

>quickly melted the copper into a red-hot goo -- easily even while they were

>about an inch and a half apart.

A flyback yields a 15khz RF output. A neon is 60hz. Thats the

major difference. A flyback is also a low current device.

Not much more than a couple ma.

>

>What am I to expect? Does this neon transformer work? The specs before I

>tore it apart were: 6000 volt secondaries, 30 ma, 120 AC primary. I suspect

>that it is much more than 30ma now, because I took out two (from what

>looked like the only two) shunts out and the primary is about 1/4-inch

>closer to the secondaries than it should be (I'm going to fix that.). What

>about the voltage? Can anybody make a guesstimation on how high (or low) it

>is? Will this work for my first, small Tesla coil?

Do not completely remove the shunts. Doing so will ultimately

burn up a secondary winding. The shunts are probably built up

with a number of thin plates. You can remove some of the plates

while leaving the remainder in place. A 30ma neon can be boosted

to about 40-45 ma. No more.

The voltage output of the neon wont change. Thats dependent

on the turns ratio betweem primary and secondary.

This neon should do well for a small coil. I suggest a 3 inch

diameter secondary about 18 inches long wound with AWG 24.

Thats about 830 turns. Should yield 8-10 inches of spark.

You will need a rather narrow gap for 6 kv. I suggest

4 series gaps spaced .03 inches apart for a total of .120 inch

as a starting point.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Inductance of a multilayer coil

Date: Fri,  3 Apr 1998 23:41:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Fri, 3 Apr 1998 18:41:05 EST

Subject: Inductance of a multilayer coil

All,

Anybody know the equation for inductance of a multilayer coil?

Erik

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Inductance of a multilayer coil

Date: Sat,  4 Apr 1998 13:06:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 04 Apr 1998 07:06:27 -0600

Subject: Re: Inductance of a multilayer coil

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: ESchulz531 <ESchulz531@aol.com>

> Date: Fri, 3 Apr 1998 18:41:05 EST

> Subject: Inductance of a multilayer coil

>

> All,

>

> Anybody know the equation for inductance of a multilayer coil?

>

> Erik

>

Erik,

Unfortunately, there's no simple closed-form equation for the inductance

of a multilayer coil. The inductance is usually approximated by using

the coil length, mean radius ("middle radius between the outer and

innermost layers), and the number of turns, treating it as though it

were a "fat" single-layer coil. The computed inductance is then

multiplied by adjustment factor (sometimes called the Nagaoka constant)

which is a function of the coil's geometric parameters (length, mean

radius, and winding thickness) to arrive at an approximation. A related

method involves using a series of charts (called nomagrams) which

provide the appropriate compensation factors to the computed

single-layer inductance.

Unfortunately, some of the best references for exactly how this is done

are out of print ("The Theory and Design of Inductance Coils" by V.G.

Welsby and the "Radio Engineers Handbook" by Frederick Terman). An

example using Nagoaka's constant where the coil's cross section looks

like the following is shown below for those whose eyes are not already

glazed over...

|                   |

|<------  W  ------>|

-------------------

|                   |

|      N turns      |-------

|                   |   |

-------------------    R

|

- - - - - - - - - - - - - -  Coil Centerline

-------------------   -----

|                   |   |

|                   |   T

|                   |   |

-------------------   -----

Let:

W = coil length (cm)

R = mean radius (cm)

T = coil winding thickness

Then the unadjusted inductance is:

L(unadj) = 4*Pi*(R^2)*(N^2)/(1000*X)   uH

The actual ("adjusted") inductance will need to multiplied by the

Nagoaka constant K to compensate for the coil's geometry:

K = 1/(1+0.9*(R/X)+0.32(T/R)+0.84*T/X))

L(adj) = L(unadj) * K

Let's try an example. How about a 200 turn coil that's 5 cm long, has an

average radius of 2 cm, is 1 cm thick, and has 200 turns. First let's

find L(unadj):

X = 5 cm

R = 2 cm

T = 1 cm

L(unadj) = 4*(3.1519)*(2^2)*(200^2)/(1000*5)

L(unadj) = 402 uH

Solving for K:

K = 1/(1+0.9*(5/2)+0.32(1/2)+0.84*(1/5))

= 0.2795

L(adj) = 402*0.2795 = 112 uH

Hope this helps! If nothing else, it'll make good bedtime reading... :^)

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: surge inductance? Gap models?

Cc:

Bcc:

X-Attachments:

At 04:21 PM 4/3/98 GMT, you wrote:

>From: "Bill the arcstarter" <arcstarter@hotmail.com>

>Date: Fri, 03 Apr 1998 08:21:07 PST

>Subject: surge inductance? Gap models?

>

>Mike Hammer-

> You wrote:

>

>>Yea Im against the low inductance primary. A high inductance

>>primary offers so many advantages. The biggest advantage is

>> improvement in gap operation due to a higher surge impedance.

>

>What do you mean - "surge impedance"? Are you describing the ability of

>the inductor to "force" current through the gap even when the cap

>voltage is zero??

No. Zsurge is around SQRT(Lp/Cp).

>

> Or - the larger the inductance, the smaller the peak currents in the

>primary - which might help the gap quench faster (just guessing).

Yes. Higher inductance means lower peak primary currents.

As losses are I2R the higher current equates higher loss.

Not only in the primary itself from ohmic losses but in the

spark gap and main capacitor.

The surge impedance (Zsurge) is around SQRT(Lp/Cp).

I saved a little table and discussion from Bert Hickman that might be

illustrative at this point. So Bert be careful what you say.

Somebody might save it. :)

<begin include>

"Low" inductance is a relative parameter versus tank cap size. Primaries

with relatively few turns can have deceptively low inductance, since the

inductance scales as the square of the number of turns. Less primary

inductance means more peak primary current flow "through" the cap each

time the gap fires. For example, for a typical pancake primary, with a 9

inch inner diameter, 0.625" turn-turn spacing, driven from a 0.025 uF

cap and a gap breakdown voltage of 15 KV generates the following peak

primary currents. The factor of increase in the degree of ohmic heating

versus the 12 turn primary is shown in the far right column.  You can

eaisly see that a 4:1 decrease in turns results in a 16:1 increase in

ohmic heating alone.

(Turns)     (uH)    (Amp)  (Amp^2)   Ohmic

N         Lp      Ip     Ip^2    Heating

===       ====    ====    ====    =======

3        4.1     1475  2175625   15.9X

6       15.9      755   570025    4.2X

9       36.0      501   251001    1.8X

12       66.0      370   136900    1.0X

<end include>

That says it all better than I could of.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Ferrite Choke Question

Date: Sun,  5 Apr 1998 01:36:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>> Here is the one I have used:

T>> http://www.america.com/~tesla/formulas.html

T>>

T>> Bob Schumann

T>>

T>Bob and others with answers....

T>The '....formulas.html' ref above is neat and I want a copy....

T>however, the critical text is WHITE, which prints in white....  all I

T>get is the nice pages of boxed formatting, and NO TEXT COPY!

T>I dont know if it is your page or whose... but could this 'text color'

T>be changed so that I can print it?

T>I note this problem on other sites, as well...  should the rule be,

T>"Never use white text!", or am I still missing a lot?

There's a setting in your browser which allows you to make it print

ALL text as though it were black only.  In Netscape, you can get to it

by selecting [F]ile, then [P]age Setup, then [O]ptions, then check the

box marked Black Text.

Ralph

---

CONNECT 110?!? Help! I'm in s l o w  m o t i o n !!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Career Opportunities

Date: Sun,  5 Apr 1998 03:16:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 5 Apr 1998 13:16:34 +1000

Subject: Career Opportunities

To everyone on the list,

I'm doing my second last year at secondary school and I have to start

thinking seriously about what I am going to do when I leave school.  I would

like to continue the work of Nikola Tesla but I don't know what sort of a

paying full time job I would have to get and where.  I know that I'd

probably have to go to America for it but what the heck.  I need to know

what qualification I'd need (Electronics Engineering Degree?) and where the

best place would be to study it.  If anyone can fill me in on some details

(off the list would probably be better) I would be most grateful.

From Scott Hayward at smash@eisa.net.au

Should ozone sniffing be banned?  :)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Nagaoka rides again!

Date: Sun,  5 Apr 1998 03:52:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Sat, 04 Apr 1998 19:52:07 PST

Subject: Nagaoka rides again!

It was written:

>

>Unfortunately, there's no simple closed-form equation for the inductance

>of a multilayer coil. The inductance is usually approximated by using

>the coil length, mean radius ("middle radius between the outer and

>innermost layers), and the number of turns, treating it as though it

>were a "fat" single-layer coil. The computed inductance is then

>multiplied by adjustment factor (sometimes called the Nagaoka constant)

Oh dang - "Nagaoka" - I was hoping I wouldn't hear that name again...

I an unsure if this correction has a theoretical/physical basis for

existing, or if it was invented based upon real world measurements.  Any

thoughts?

>which is a function of the coil's geometric parameters (length, mean

>radius, and winding thickness) to arrive at an approximation. A related

>method involves using a series of charts (called nomagrams) which

>provide the appropriate compensation factors to the computed

>single-layer inductance.

In other words - someone wound a boatload of coils, measured them out,

and tabulated the results??

>Unfortunately, some of the best references for exactly how this is done

>are out of print ("The Theory and Design of Inductance Coils" by V.G. Welsby)

Hey - I have that book!  It's a bit of a rough read though.  (I keep it

next to my 1956 text on magnetic amplifiers/saturable reactors)  I've

been digging through it - looking for tidbits which might be useful for

coilers.  Can't say I've had any great revelations along those lines.

<snip>

>Let's try an example. How about a 200 turn coil that's 5 cm long, has an

>average radius of 2 cm, is 1 cm thick, and has 200 turns. First let's

>find L(unadj):

>        X = 5 cm

>        R = 2 cm

>        T = 1 cm

> L(unadj) = 4*(3.1519)*(2^2)*(200^2)/(1000*5)

> L(unadj) = 402 uH

>

>Solving for K:

>

> K = 1/(1+0.9*(5/2)+0.32(1/2)+0.84*(1/5))

>   = 0.2795

>

> L(adj) = 402*0.2795 = 112 uH

Wow - I have never worked the math for that before.  I had assumed that

the Nagaoka correction would be small - ie - negligible.  Apparently

this is NOT the case (at least for your example) if the correction

factor is 0.279!

Bert - is there a reasonably accurate formula for the inductance of the

types of primaries we tend to use - for example - the flat spiral type?

How might the above treatment hold for 'non-rectangular' coils such as

conical primaries, etc?  Seems like it is non-applicable.

I suppose the most accurate way is to build it, and go measure it, just

like they did in 1940 or whenever that book was published... :(

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Ferrite Choke Question

Date: Sun,  5 Apr 1998 04:39:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jim Ryan <jryan@exis.net>

Date: Sat, 04 Apr 1998 23:39:52 -0500

Subject: Re: Ferrite Choke Questio

Hi all, I have not started on my coil yet. I plan to build a 6" diameter

primary with poly caps. I keep seeing the need for inductors to protect

the neon transformer from RF feedback.

I have a supply of ferrite inductors that have 8 turns 2 layers of .040

copper wire in opposite directions. The ferrite core is .180 in diameter

.400 long with end caps.

The ferrite core looks likes a spool you wind wire on. Any estimate of the mH.

Can anyone give an estimate of the number of these I would need to protect the

transformer.  And let's see, inductors in series increase inductance, as I

remember from my 1994 Apprentice school training.

Thanks

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ferrite Choke Question

Cc:

Bcc:

X-Attachments:

At 04:39 AM 4/5/98 GMT, you wrote:

>From: Jim Ryan <jryan@exis.net>

>Date: Sat, 04 Apr 1998 23:39:52 -0500

>Subject: Re: Ferrite Choke Questio

>

>Hi all, I have not started on my coil yet. I plan to build a 6" diameter

>primary with poly caps.

If this is a first coil I recommend you build at least one smaller

coil first. If you have already built a successful coil then go for it.

>I keep seeing the need for inductors to protect the neon transformer

>from RF feedback.

>

>I have a supply of ferrite inductors that have 8 turns 2 layers of .040

>copper wire in opposite directions. The ferrite core is .180 in diameter

>.400 long with end caps.

> The ferrite core looks likes a spool you wind wire on. Any estimate of the mH.

This are mighty small cores and would not provide any significant

amount of inductance. Hard to say what the resulting inductance

would be without some information about the particular ferrite mix

in your cores.

My ferrite chokes are wound on toroids 2.5 inches in diameter.

Rather large cores. 120 turns of AWG 26 on these only results

in 2.5 mh.

>

>Can anyone give an estimate of the number of these I would need to protect the

>transformer.  And let's see, inductors in series increase inductance, as I

>remember from my 1994 Apprentice school training.

>

Correct. Inductors in series add up. However I dont think

your small cores are suitable. IT would take a great deal of them

to yield any significant amount of inductance. Any inductance

values less than 2 mh are of very little benefit.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Progress Report

Date: Sun,  5 Apr 1998 19:41:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Sun, 5 Apr 1998 15:41:23 -0400

Subject: Re: Progress Report

>Tom,

>You might want to drink another sixpack.  I think you will need a few

>more bottles for two neons.   You will want 0.01uF(and maybe a wee

>bit more). Mike Hollingsworth

Mike, drank 6 more and feeling great.

I finished my primary coil over the weekend. If anyone out there is

building their first coil and hasn't gotten to the primary yet, don't

underestimate the challenge. This was definately more difficult than

the secondary!

I ended up cutting ribs from 1/4" polyethylene plastic (white cutting

board stuff). I found that if I made slots which angled in towards the

center of the coil as they went deeper, I could easily snap the tubing

into the slots without having to wrestle with the tubing.

Cutting the slots was a pain (especially to get them accurate) but it

was well worth it when it came time to wind the coil.

Having someone to put a foot on the coil while you're working helps

alot too.

Quick drawing:

---  ---  ---  ---  ---  ---|      |---  ---  ---  ---  ---  ---

----  ---  ---  ---  ---  --|      |--  ---  ---  ---  ---  ----

----------------------------|      |----------------------------

----------------------------|      |----------------------------

First spark should be this week!

I still have to build the gaps, and wire me caps (hehe), and wind two

chokes,  and I'm done.

What type of wire can I get away with for starters? I'll be running

one 15KV/30ma Neon first, then two in parallel afterwards. Do I have

to use 15KV neon wire, or can I start with solid copper wire?

Thanks!

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Castor oil

Date: Sun,  5 Apr 1998 19:55:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Mon, 6 Apr 1998 00:55:52 +0500

Subject: Re: Castor oil

On Mon, 16 Mar 1998 08:26:05 -0600 In Tesla-2

Mike Hammer <mhammer@misslink.net> wrote;

> >From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

> >Date: Sun, 15 Mar 1998 20:00:19 +0500

> >Subject: Castor oil

> >

> >I don't remember if it was on this list or Chips list that someone

> >suggested using castor oil in TC capacitors, the problem with castor oil

> >is it has an extremely high power factor (7 according to the 27th edition

> >of the ARRL Radio Amateur's Handbook) which means it will heat up and

> >cook due to the large amounts of R.F. being absorbed.

> Actually castor oil is used in some of the best pulse

> caps available. It has low dissipation at normal Tesla coil

> frequencies. Comparable to mineral or transformer oils.

>

> It does however have high dissipation at HF frequencies which

> is probably what the Handbook reference was alluding to.

Very good point, Mike! In re-examining the old ARRL handbook, the power

factor given was for 1 Mhz. which would be one mighty small Tesla coil!

This is one of the things that makes searching the electronics literature

for dielectrics difficult at best. Most of the references list dielectric

constant and dielectric strength for materials but leave out the power

factor or more often than not when they do list the power factor they don't

stipulate at what frequency, and the few references that list the power

factor for several frequencies like the older CRC handbooks dont cover a

lot of substances that may indeed be very good. Eventualy, perhaps some of

us will get together and start measuring the dielectric characteristics of

various materials under conditions that would be typical of Tesla coil

usage so then we could have a good valid reference to use.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 12Kv 30ma coil specs

Date: Sun,  5 Apr 1998 20:26:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE D <gordrew@home.com>

Date: Sun, 05 Apr 1998 13:26:49 -0700

Subject: 12Kv 30ma coil specs

Here are the calculations Ive come up with for my primary and secondary

coils, any comments would be very apreciated.

Primary:

inside dia- 11in

spacing- 2 in

total dia- 18.34

wire dia- 0.23 in

Secondary:

dia- 8.2 in

length- 24 in

gauge- 22

turns/inch- 37.48

circumference- 25.76 in

--MIke Drew

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Progress Report/wire

Cc:

Bcc:

X-Attachments:

At 07:41 PM 4/5/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Sun, 5 Apr 1998 15:41:23 -0400

>Subject: Re: Progress Report

<major snip>

>

>     What type of wire can I get away with for starters? I'll be running

>     one 15KV/30ma Neon first, then two in parallel afterwards. Do I have

>     to use 15KV neon wire, or can I start with solid copper wire?

Tom,

Neon wire is a good choice. Remember that most common wire

has only about a 600 volt insulation rating.

If you space the conductors away from things far enough to

prevent arcing then almost any type of wire will do for

the 60 hz wiring from your neons to safety gap, RF chokes and

main gap. Heavier solid wire is best for the tank circuit

connections from the main gap to primary cap and primary coil.

If insulation is important then use neon wire or COPPER

core automotive ignition wire. Beware of common ignition

wire than has a resistive conductor to suppress RFI. This

wont be at all suitable. Copper igntion wire is often

sold at performance shops.

RG213 coaxial cable can also be used. The center conductor

is AWG 13 and the insulation is polyethylene. Be aware that

the common RG8 foam cables have much poorer insulation ratings.

The foam insulation is white while the poly is translucent.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Resonate Freq's

Date: Mon,  6 Apr 1998 12:29:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Mon, 06 Apr 1998 12:29:21 GMT

Subject: Re: Resonate Freq's

Organization: White Wing Logic

>I recently found out an alternate way to test for the secondary resonant

>frequency.  A friend hooked the grounds of the scope and signal

>generator together, and fed the base of the secondary through a 1K

>resistor (from the output connection of the sig gen).  Then the scope

>probe was connected to the other end of the 1K resistor (where the

>resistor was attached to the base of the secondary).  We then adjusted

>the sig-gen such that the scope signal was minimum.  There were no other

>connections made.

>

>Question - Does this make sense?  Why does this work?  I see no ground

>path back from the secondary to the signal generator so I'm confused by

>this - but I saw it work and the result was right on track!

>

Due to the 1K Resistor, you are measuring the current flow from the

sig-gen to the secondary (which I think is effectively acting as an

antenna) - when the sec is resonating, it absorbs the most power,

therefore the scope signal is at a minimum.

Another similar method I use is to connect the sig-gen to the base,

and put a few inches of wire on the scope probe and hang it a foot or

two away from the toroid, then tune for maximum signal.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Progress Report:  Murphy-Waldo enterprise

Date: Mon,  6 Apr 1998 14:47:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Mon, 06 Apr 1998 07:47:32 -0700

Subject: Progress Report:  Murphy-Waldo enterprise

Organization: Self

All,

In hopes of enhancing and stimulasting the discussion of relevent issues

and what not, here is our situation:

[]Variac- gathering funds

[]Nxfmr- 15kv, 30ma on hand

[]Safety gap- completed

[]pri series gaps-  completed; blower acquired, plenum not designed*

[]chokes- completed

[]cap 1 - 14 or up to 28 ea.40 oz Bud bottles, (92 sq" ea.) under construction*

[]cap 2- immersed poly plates, in design*  **

[]Primary coil- ~17 turns, in design, .31" od Cu tube on hand.

[]2ndary coil- 4"od plex tube on hand, 24 ga mag wire on hand; coil termiminals in design*

[]termnal toroid- completed

[]RFGnds (2) one here and probaly one at Dan's, since we may be lugging

this lashup to and fro, a lot!

** I want to use 6 mil poly 'cleanroom' bags: 1 on ea plate and

several      stacked between; Dan wants to cut corners by hacking his

way thru a      large drop cloth!*

In our small, but poorly equipped facilities, progress is intense,

albeit slow. Dan persists in his seemingly neverending obsession with

his 'job', while the 'project' languishes! I, on the other hand, have

devoted my self to cap 1!  Only about 14 more bottles to empty!

While both Dan and I seem to have photographic avocations in our

backrounds, I have recently acquired a digital still camera, partly to

justify my new $9.50 per month added 'net' access expense, and to show

and tell more stuff.

Which brings me to my question of the day:

Given the size of 'appended photos/e-mail', is it desireable to begin

this practice? Will there be problems with either the servers or

recipients of such materials?

* we argue a lot.

Feedback solicited.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Monday, April 06, 1998 8:48 AM

To:     Minnich, Roy

Subject:        Neon Transformers & HV Wire For Sale

From: Tedd Payne <tpayne@netnitco.net>

Date: Sat, 04 Apr 1998 21:39:27 -0600

Subject: Neon Transformers & HV Wire For Sale

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your zip code (to

determine shipping cost).

Thanks,

Tedd

"CONVENTIONAL" neon transformers in clean (not old, not rusty), good

working condition.  Some have a label, some do not.  For those not

labeled, the ratings are from my testing/measuring.

Good for Tesla coil, Jacob's ladder, neon, crackle tubes, HV

experiments, etc.

$17 used  12kv30ma  in porc ins  out porc insul

$17 used  12kv30ma  in porc ins  out porc insul (label)

$14 used  12kv30ma  in porc ins  out porc insul (miss 1 HV insul)(label)

$15 used  9kv60ma   in   porc ins  output porc insul (no lid)

$11 used  7.5kv60ma  in porc ins  out porc ins (miss 1 HV ins, other chipped)

$11 used  6kv60ma   in porc ins   output porc insul

$11 used  9kv30ma   in porc ins  output porc insul (label)

$11 used  9kv25ma   in AC cord  out 2 wires, w/switch (label)

$10 used  7.5kv35ma  in AC cord  out 2 wires, w/switch

$10 used  7.5kv30ma  in AC cord  out 2 wires, w/switch (label)

$10 used  7.5kv25ma  in AC cord  out 2 wires, w/switch

$10 used  7.5kv25ma  in AC cord  out 2 wires, w/switch

$8  used  3-4kv60ma  in porc ins   output porc insul

NEW Neon transformer HV wire:

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

AC power cord for neon transformer, 3-wire, grounded plug, used, $1.00

-

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Neon Transformers & HV Wire For Sale

Date: Mon,  6 Apr 1998 14:58:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Mon, 6 Apr 1998 09:58:32 -0500

Subject: RE: Neon Transformers & HV Wire For Sale

Hi Ted,

Whoah!  Where did you come from??  Do you get these in regularly? I'd

like to keep you as my source.

The 12KV 30 mA with good insulators and a label (second from top) on

your list for $17 and the 9KV 30 mA with good insulators and a label

(seventh from top) for $11 look interesting.

My Zip is 60453

Roy Minnich

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Progress Report:  Murphy-Waldo enterprise

Cc:

Bcc:

X-Attachments:

At 02:47 PM 4/6/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Mon, 06 Apr 1998 07:47:32 -0700

>Subject: Progress Report:  Murphy-Waldo enterprise

>Organization: Self

>

>All,

>In hopes of enhancing and stimulasting the discussion of relevent issues

>and what not, here is our situation:

>

>[]Variac- gathering funds

>[]Nxfmr- 15kv, 30ma on hand

>[]Safety gap- completed

>[]pri series gaps-  completed; blower acquired, plenum not designed*

>[]chokes- completed

>[]cap 1 - 14 or up to 28 ea.40 oz Bud bottles, (92 sq" ea.)

>under                     construction*

Remember Gene you only can charge up about .0053 mfd

worth of capacitance with your 15kv/30ma neon.

Since your 40 oz SW caps are going to be around 1400-1500 pf

each thats only 3 or 4 bottles.

>[]cap 2- immersed poly plates, in design*  **

>[]Primary coil- ~17 turns, in design, .31" od Cu tube on hand.

>[]2ndary coil- 4"od plex tube on hand, 24 ga mag wire on hand;

>coil              termiminals in design*

>[]termnal toroid- completed

>[]RFGnds (2) one here and probaly one at Dan's, since we may be

>lugging           ths lashup to and fro, a lot!

All sounds good to me.

>  ** I want to use 6 mil poly 'cleanroom' bags: 1 on ea plate and

>several      stacked between; Dan wants to cut corners by hacking his

>way thru a  large drop cloth!*

Either way is fine. The method I use for cutting lots of poly squares

is to cut 2 pieces of plywood the exact dimensions of your poly squares.

Sandwich 20 or more sheets of poly between the plywood squares and

secure with a few C clamps. Cut off the overhang with a fresh single

edge razor blade. You can make a couple hundred squares pretty quickly

this way.

If making a poly cap for 15kv use do your self a favor and

use 3 series connected caps. Its much easier to make 3 reliable

5kv caps and put them in series than it is to make a single

reliable 15kv cap.

If using 6 mil poly I suggest 4 layers of poly for each layer

for a total of 24 mils. 3 of these 24 mil caps gives the equivalent

of 72 mils with a much greater reliability.

Also remember that each cap in the series string must be 3 times

the required final value. If .0053 mfd is your desired value

then each cap must be .0159 mfd when the 3 are wired in series.

>

>While both Dan and I seem to have photographic avocations in our

>backrounds, I have recently acquired a digital still camera, partly to

>justify my new $9.50 per month added 'net' access expense, and to show

>and tell more stuff.

>

>Which brings me to my question of the day:

>

>Given the size of 'appended photos/e-mail', is it desireable to begin

>this practice? Will there be problems with either the servers or

>recipients of such materials?

Unfortunately the list server does not tolerate binary files well.

So do not post binaries to the list. If you want to place

some of your pics on the Internet for all to see I would be glad

to host them on my web site for you.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Fwd: Applications of the Coil?

Cc:

Bcc:

X-Attachments:

At 05:22 PM 4/6/98 -0700, you wrote:

>

>From: Trevor Riehl <triehl@compusmart.ab.ca>

>Date: Sun, 05 Apr 1998 12:25:09 -0600

>Subject: Applications of the Coil?

>

>Originally, what was the coil created for?

Tesla first started to use his coils to generate

high frequency high voltage AC. He had started to build dynamos

with many poles to generate high frequencies but mechanical

limitation kept him from exploring even higher frequencies.

Since this was before the invention of the vacuum tube triode

and its use as an RF oscillator the Tesla coil was put into that

use.

It eventually became the basis of all early radio transmitters.

Marconi's famous trans-Atlantic radio equipment was nothing

but a copy of an earlier Tesla design.

At its most basic level a Tesla coil is just a RF oscillator.

>

>Are there any modern day applications?  Future applications?

Not many. Some uses exist for the detection of leaks in

neon sign tubes and generating high voltages for insulation testing.

>

>To what degree is the HAARP project related to the Tesla Coil?

Not in any way except in the minds of fanatics.

>

>Are there any books discussing the applications of the coil?

Present day applications? Not that Im aware of.

Im sure there are some psuedo science books extolling the virtues

of Tesla coils that arent worth the paper they are printed on.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: Fwd: Applications of the Coil?

Date: Tue,  7 Apr 1998 12:17:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 07 Apr 1998 07:17:00 -0500

Subject: Re: Re: Fwd: Applications of the Coil?

Organization: MailExcite  (http://www.mailexcite.com:80)

Talk about far out extremist. A few nights ago, I believe it was on the discovery

channel, there was a guy who claimed that his coil was able to boost the power of

his mind. With the coil in operation he is supposed to be able to levetate objects

and bend spoons. Now thats the strangest thing I've heard of to date.

Paul M

---

Einigkeit und Freiheit

>From: Mike Hammer <mhammer@misslink.net>

>Date: Mon, 06 Apr 1998 19:38:56 -0500

>Subject: Re: Fwd: Applications of the Coil?

>

>At 05:22 PM 4/6/98 -0700, you wrote:

>>

>>From: Trevor Riehl <triehl@compusmart.ab.ca>

>>Date: Sun, 05 Apr 1998 12:25:09 -0600

>>Subject: Applications of the Coil?

>>

>>Originally, what was the coil created for?

>

>Tesla first started to use his coils to generate

>high frequency high voltage AC. He had started to build dynamos

>with many poles to generate high frequencies but mechanical

>limitation kept him from exploring even higher frequencies.

>

>Since this was before the invention of the vacuum tube triode

>and its use as an RF oscillator the Tesla coil was put into that

>use.

>

>It eventually became the basis of all early radio transmitters.

>Marconi's famous trans-Atlantic radio equipment was nothing

>but a copy of an earlier Tesla design.

>

>At its most basic level a Tesla coil is just a RF oscillator.

>

>

>>

>>Are there any modern day applications?  Future applications?

>

>Not many. Some uses exist for the detection of leaks in

>neon sign tubes and generating high voltages for insulation testing.

>

>>

>>To what degree is the HAARP project related to the Tesla Coil?

>

>Not in any way except in the minds of fanatics.

>

>>

>>Are there any books discussing the applications of the coil?

>

>Present day applications? Not that Im aware of.

>Im sure there are some psuedo science books extolling the virtues

>of Tesla coils that arent worth the paper they are printed on.

>

>TTYL!

>

>

>Mike Hammer

>mhammer@misslink.net

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: High Voltage Humor

Date: Wed,  8 Apr 1998 00:24:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred C. Erpel (Home)" <aerpel@pil.net>

Date: Tue, 7 Apr 1998 20:24:39 -0400

Subject: High Voltage Humor

This is a multi-part message in MIME format.

All,

Get some neat ideas on how to use the high voltage you generate to do

routine cooking around home.  My favorite is the Jacob's ladderkabob. And

don't miss the Q & A section, it's a hoot.

Regards,

Alfred Erpel

http://www.foundrygroup.com/cbrush/cook/index.html

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: series gap

Date: Wed,  8 Apr 1998 02:44:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 07 Apr 1998 19:44:14 -0700

Subject: Re: series gap

I here a lot of talk about series gaps, how does it work and how can I build one?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary Design

Date: Thu,  9 Apr 1998 21:37:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Thu, 09 Apr 1998 16:37:28 -0500

Subject: Primary Design

Organization: MailExcite  (http://www.mailexcite.com:80)

So far Bill (The arcstarter) and I have been very successful with our 4" coil.

We are the process of building a 7". Actual diameter is 6.87". On the 4" coil

we used a flat wound spiral this gave us excellent results. We plan to use the

same design.

On the 4" we used .25" refrigerator tubing. For the 7" I was thinking about using

.375" tubing. Is there any significant adavantage to this ?

Paul M

---

Einigkeit und Freiheit

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: series gap

Cc:

Bcc:

X-Attachments:

At 02:44 AM 4/8/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Tue, 07 Apr 1998 19:44:14 -0700

>Subject: Re: series gap

>

>I here a lot of talk about series gaps, how does it work and how can I build one?

>

A series gap is pretty much like it sounds. A number of gaps

connected in series. There are many ways to implement one but

a common and popular design that works very well with neons

is the so called Richard Quick style cylinder gap.

This gap design was popularized (not invented) by Richard

Quick so ended up bearing his name.

Its made with sections of hard copper tubing mounted radially inside

a piece of PVC pipe. The PVC pipe serves as a housing and

serves to channel the airflow.

A series gap like this has much better efficiency that a single

gap as the arc is broken up into several smaller arcs so the

heat is spread out over a larger area. This gives the gap the ability

to extinguish the arc (quench) better thus improving efficiency.

Check out Dave Bronakowski's archive site for the file sparkgap.zip.

The URL is:http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Primary Design

Date: Thu,  9 Apr 1998 15:14:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 09 Apr 1998 10:14:00 -0500

Subject: Re: Primary Design

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Thu, 09 Apr 1998 16:37:28 -0500

> Subject: Primary Design

> Organization: MailExcite  (http://www.mailexcite.com:80)

>

> So far Bill (The arcstarter) and I have been very successful with our 4" coil.

> We are the process of building a 7". Actual diameter is 6.87". On the 4" coil we

> used  a flat wound spiral this gave us excellent results. We plan to use the same

> design.

> On the 4" we used .25" refrigerator tubing. For the 7" I was thinking about using

> .375" tubing. Is there any significant adavantage to this ?

>

> Paul M

> ---

> Einigkeit und Freiheit

>

Paul,

Using the larger diameter tubing will help reduce losses and corona, and

will let you push more power more efficiently through the primary. If

you're planning on running over 3 kW, I'd recommend going with the

larger OD tubing...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: vacuum spark gap

Date: Fri, 10 Apr 1998 03:24:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Schulhauser <genesch@azstarnet.com>

Date: Thu, 09 Apr 1998 20:24:40 -0700

Subject: vacuum spark gap

I understand that it is desirable to have a

rapid-quenching spark gap. Would placing

the gap in a vacuum serve this purpose?

Gene Schulhauser

Tucson, AZ

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Thu,  9 Apr 1998 23:23:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 09 Apr 1998 18:23:57 -0500

Subject: Re: vacuum spark gap

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Gene Schulhauser <genesch@azstarnet.com>

> Date: Thu, 09 Apr 1998 20:24:40 -0700

> Subject: vacuum spark gap

>

> I understand that it is desirable to have a

> rapid-quenching spark gap. Would placing

> the gap in a vacuum serve this purpose?

>

> Gene Schulhauser

> Tucson, AZ

>

Gene,

Excellent Question!

In theory - YES! If you could create an extremely good vacuum, there

would be little remaining gas to ionize. However, even a vacuum will

"break down" under enough voltage stress, permitting current to flow

through a conducting path made up of vaporized electrode ions and

electrons. Once the energy dissipation in the gap declines to a low

enough level, the metal vapor redeposits on the side walls, and the gap

rapidly recovers.

Vacuum switches have become very popular in high power switching (such

as in power company HV circuit breakers and contactors) because of their

excellent recovery characteristics. However, they're really not meant

for the repetitive switching seen in Tesla Coils. Under repetitive

switching, the main problems are erosion of the electrode material, and

that the redeposited metal will eventually short-circuit the switcth.

>From a more practical standpoint for coilers, a pressurized spark-gap

using certain gases (pure hydrogen or sulfer hexafluoride for example)

is the another "next best" alternative...

Safe coiling to you, Gene!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: REVISED AWG Wire Table for Coilers

Date: Fri, 10 Apr 1998 00:58:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 9 Apr 1998 20:58:59 -0400

Subject: REVISED AWG Wire Table for Coilers

Fellow Coilers,

A few people e-mailed me to say that I was a bit conservative on the amp

ratings for the AWG wire table I e-mailed to the TESLA and TESLA-2 lists.

I realize that we Coilers love to push things to the limit. In going back over

my AWG sources I did find one source that went as high as 23.679 amps for

AWG #10. Someone mentioned that UL codes allowed 23 amps for AWG #12. Such a

large value must be assuming the wire is enclosed in a metal conduit which

is acting as a heat sink. I have re-written my program to reflect a Max

recommended continuous current of 23.679 amps for a #10 AWG bare copper wire

cooled by ambient air.

I am sorry if sending this second revised AWG table means a few more seconds

of download time for the List members. Dump the earlier table and file this

revised table away for future reference.

Hope this helps.

Fr. Tom McGahee

***************

AWG Wire Table for BARE COPPER Wire Compiled by a program written by Fr. Tom Mc

Gahee

Compiled by Fr. Tom McGahee tom_mcgahee@sigmais.com

Permission granted to copy freely so long as credit line above is included

AWG = American Wire Gauge size

Dia-mils = Diameter in mils (1 mil = .001 inch)

TPI = Turns Per Inch (Ignoring thickness of unknown insulation)

Dia-mm = Diameter in millimeters (For comparison with non-USA coilers)

Circ-mils = Circular Mils. (circular mils = diameter in mils squared)

Ohms/Kft = Ohms Per 1,000 Feet

Ft/Ohm = Feet Per Ohm

Ft/Lb = Feet Per Pound

Ohms/Lb = Ohms Per Pound

Lb/Kft = Pounds Per 1,000 Feet

NormAmps = Normal Average Amp Capacity based on 500 circular mils per Amp

MaxAmps = Maximum recommended Average Amp Capacity in Open Air based on 438.489

circular mils per Amp

Actual Amp capacity of a wire depends on form factor and method of cooling!

MaxAmps assumes free flow of air around wire. Do NOT exceed this maximum without cooling!

Wire wrapped in a coil or without any form of cooling may over-heat at MaxAmps!

Many factors govern the ACTUAL Max Amps you can pass through a wire continuously.

Be careful!

AWG = American Wire Gauge size from 0000 to 40

Dia-mils = Diameter in mils (1 mil = .001 inch)

TPI = Turns Per Inch [Note that this is for BARE WIRE. Insulation thickness varies]

Dia-mm = Diameter in millimeters. This was included to help when dealing with metric Coilers.

Circ-mils = Cross sectional Area in Circular Mils. ( circular mils = diameter in mils squared )

Ohms/Kft = Ohms Per 1,000 ft.

Ft/Ohm = Number of feet required for 1 Ohm of resistance

Ft/Lb = Feet Per Pound

Ohms/Lb = Ohms Per Pound

Lb/Kft = Pounds Per 1,000 feet

*AMPS = Conservative Amp Rating based on 750 circulare mils per Amp

MaxAmps = Maximum allowable current based on 500 circular mils per Amp. Do NOT exceed this rating.

AWG       Dia-mils   TPI        Dia-mm    Circ-mils  Ohms/Kft   Ft/Ohm    Ft/Lb     Ohms/Lb   Lb/Kft    *Amps     MaxAmps

0000       459.99     2.1740    11.684     211592     0.0490       20402       1.5613    0.0001    640.48     282.12    423.18

000         409.63     2.4412    10.405     167800     0.0618        16180      1.9688    0.0001    507.93     223.73    335.60

00           364.79     2.7413    9.2657     133072      0.0779       12831      2.4826    0.0002    402.80     177.43    266.14

0            324.85     3.0783    8.2513     105531      0.0983       10175      3.1305    0.0003    319.44     140.71     211.06

1            289.29     3.4567    7.3480     83690        0.1239       8069.5     3.9475    0.0005    253.33     111.59     167.38

2            257.62     3.8817    6.5436     66369        0.1563       6399.4     4.9777    0.0008    200.90     88.492     132.74

3            229.42     4.3588    5.8272     52633        0.1970       5075.0     6.2767    0.0012    159.32     70.177     105.27

4            204.30     4.8947    5.1893     41740        0.2485       4024.7     7.9148    0.0020    126.35     55.653     83.480

5            181.94     5.4964    4.6212     33101        0.3133       3191.7     9.9804    0.0031    100.20     44.135     66.203

6            162.02     6.1721    4.1153     26251        0.3951       2531.1    12.585     0.0050    79.460     35.001     52.501

7            144.28     6.9308    3.6648     20818        0.4982       2007.3    15.869     0.0079    63.014     27.757     41.635

8            128.49     7.7828    3.2636     16509        0.6282       1591.8     20.011    0.0126    49.973     22.012     33.018

9            114.42     8.7396    2.9063     13092        0.7921       1262.4     25.233    0.0200    39.630     17.456     26.185

AWG      Dia-mils   TPI         Dia-mm    Circ-mils  Ohms/Kft   Ft/Ohm    Ft/Lb     Ohms/Lb    Lb/Kft    *Amps    MaxAmps

10         101.90     9.8140     2.5881      10383       0.9989        1001.1    31.819    0.0318      31.428    13.844    20.765

11         90.741    11.020      2.3048      8233.9      1.2596        793.93    40.122    0.0505      24.924    10.978    16.468

12         80.807    12.375      2.0525      6529.8      1.5883        629.61    50.593    0.0804      19.765    8.7064    13.060

13         71.961    13.896      1.8278      5178.3      2.0028        499.31    63.797    0.1278      15.675    6.9045    10.357

14         64.083    15.605      1.6277      4106.6      2.5255        395.97    80.447    0.2031      12.431    5.4755    8.2132

15         57.067    17.523      1.4495      3256.7      3.1845        314.02    101.44    0.3230       9.8579   4.3423    6.5134

16         50.820    19.677      1.2908      2582.7      4.0156        249.03   127.91     0.5136       7.8177   3.4436    5.1654

17         45.257    22.096      1.1495      2048.2      5.0636        197.49   161.30     0.8167       6.1997   2.7309    4.0963

18         40.302    24.813      1.0237      1624.3      6.3851        156.62   203.39     1.2986       4.9166   2.1657    3.2485

19         35.890    27.863      0.9116      1288.1      8.0514        124.20   256.47     2.0648       3.8991   1.7175    2.5762

AWG    Dia-mils  TPI          Dia-mm    Circ-mils  Ohms/Kft    Ft/Ohm    Ft/Lb     Ohms/Lb     Lb/Kft     *Amps    MaxAmps

20         31.961    31.288     0.8118      1021.5       10.153        98.496    323.41    3.2832        3.0921    1.3620    2.0430

21         28.462    35.134     0.7229      810.10       12.802        78.111    407.81    5.2205        2.4521    1.0801    1.6202

22         25.346    39.453     0.6438      642.44       16.143        61.945    514.23    8.3009        1.9446    0.8566    1.2849

23         22.572    44.304     0.5733      509.48       20.356        49.125    648.44    13.199        1.5422    0.6793    1.0190

24         20.101    49.750     0.5106      404.03       25.669        38.958    817.66    20.987        1.2230    0.5387    0.8081

25         17.900    55.866     0.4547      320.41       32.368        30.895    1031.1    33.371        0.9699    0.4272    0.6408

26         15.940    62.733     0.4049      254.10       40.815        24.501    1300.1    53.061        0.7692    0.3388    0.5082

27         14.195    70.445     0.3606      201.51       51.467        19.430    1639.4    84.371        0.6100    0.2687    0.4030

28         12.641    79.105     0.3211      159.80       64.898        15.409    2067.3    134.15        0.4837    0.2131    0.3196

29         11.257    88.830     0.2859      126.73       81.835        12.220    2606.8    213.31        0.3836    0.1690    0.2535

AWG     Dia-mils  TPI       Dia-mm    Circ-mils   Ohms/Kft     Ft/Ohm    Ft/Lb     Ohms/Lb      Lb/Kft    *Amps     MaxAmps

30         10.025    99.750     0.2546      100.50       103.19        9.6906    3287.1    339.18        0.3042    0.1340    0.2010

31         8.9276    112.01     0.2268      79.702       130.12        7.6850    4145.0    539.32        0.2413    0.1063    0.1594

32         7.9503    125.78     0.2019      63.207       164.08        6.0945    5226.7    857.55        0.1913    0.0843    0.1264

33         7.0799    141.24     0.1798      50.125       206.90        4.8332    6590.8    1363.6        0.1517    0.0668    0.1003

34         6.3048    158.61     0.1601      39.751       260.90        3.8329    8310.8    2168.1        0.1203    0.0530    0.0795

35         5.6146    178.11     0.1426      31.524       328.99        3.0396    10480     3447.5        0.0954    0.0420    0.0630

36         5.0000    200.00     0.1270      25.000       414.85        2.4105    13215     5481.7        0.0757    0.0333    0.0500

37         4.4526    224.59     0.1131      19.826       523.11        1.9116    16663     8716.2        0.0600    0.0264    0.0397

38         3.9652    252.20     0.1007      15.723       659.63        1.5160    21012     13859         0.0476    0.0210    0.0314

39         3.5311    283.20     0.0897      12.469       831.78        1.2022    26496     22037         0.0377    0.0166    0.0249

40         3.1445    318.01     0.0799       9.8880      1048.9        0.9534    33410     35040         0.0299    0.0132    0.0198

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary Design

Cc:

Bcc:

X-Attachments:

At 09:37 PM 4/9/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Thu, 09 Apr 1998 16:37:28 -0500

>Subject: Primary Design

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>So far Bill (The arcstarter) and I have been very successful with our 4" coil.

>We are the process of building a 7". Actual diameter is 6.87". On the 4" coil

>we used a flat wound spiral this gave us excellent results.

>We plan to use the same design.

>On the 4" we used .25" refrigerator tubing. For the 7" I was thinking about using

>.375" tubing. Is there any significant adavantage to this ?

>

>Paul M

>---

Paul,

Considering the power level you are running at the use of 3/8 copper

instead of 1/4 might be a good idea. Whether or not you would see any

visible difference remains to be seen.

Both 1/4 and 3/8 inch copper are excellent low loss conductors.

I would use the 3/8 just to be sure.

Since your new secondary is going to be quite a bit lower

in frequency make sure you have plenty of primary turns.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Fri, 10 Apr 1998 16:38:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 10 Apr 1998 09:38:14 -0700

Subject: Re: vacuum spark gap

Organization: Self

Tesla-2 wrote:

>

> From: Gene Schulhauser <genesch@azstarnet.com>

> Date: Thu, 09 Apr 1998 20:24:40 -0700

> Subject: vacuum spark gap

>

> I understand that it is desirable to have a

> rapid-quenching spark gap. Would placing

> the gap in a vacuum serve this purpose?

>

> Gene Schulhauser

> Tucson, AZ

>

Gene,

This is off the top of my head, and I may be wrong, but:

1) wouldn't that be an effective Geissler Tube? QUITE conductive.

2) Air is a dielectric, as opposed to a Vacuum, which is QUITE

conductive, and doesn't really ever effectively stop conducting.

3) The point is to 'quench' conductivity, ie blow the conductive,

ionized air away, to allow the caps to recharge.

One cycle of the 3-5 kHz: ...Caps charge, gaps breakdown, caps dump

energy to primary, gaps become non-conductive (quench)..

4) 'Tuning' a gap in a vacuum would be a bear!

Now I have a couple of questions:

1) Can (if) caps loose energy back thru the conducting gaps; what would

2 diodes between the pri gaps and caps do for efficiency, given that

potential loss?

2) What effect does 'quench' rate have on gap frequency? Would air flow

directed between the series gap electrodes & normal to the discharge (as

opposed to parallel to it), be any advantage?

3) Given that a cool gap element would quench more easily than a hot

one, I'm thinking of a gap design where the gap elements would be cooled

internally with a flow of a fluid dielectric (distilled water or cap

oil); where the gap area would be 'ajacent sphericals' as opposed to

'linearly tangent tubes', which would give 'point' discharge gaps, which

could be more effectively 'blown'.

I toss these out, in hopes of stimulating further conjecture and perhaps

informed response, realizing that R&D, while anathema to production, is

the heart and soul of product improvement...

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum spark gap

Cc:

Bcc:

X-Attachments:

At 03:24 AM 4/10/98 GMT, you wrote:

>From: Gene Schulhauser <genesch@azstarnet.com>

>Date: Thu, 09 Apr 1998 20:24:40 -0700

>Subject: vacuum spark gap

>

>I understand that it is desirable to have a

>rapid-quenching spark gap. Would placing

>the gap in a vacuum serve this purpose?

>

>Gene Schulhauser

>Tucson, AZ

>

Gene,

Perhaps you have seen reference to a "vacuum" gap

and you thought it involves a gap in an evacuated

vessel. Not so. The vacuum gap is called that because

the airflow is supplied by a vacuum cleaner motor.

In order to achieve good quenching the hot ions that

are clogging up the gap from the break down of the gap

need to be removed. These hot ions are ionized air and bits

of ionized material ablated from the electrodes. These

ions are conductive so tend to keep the gap "lit" longer

than desirable.

To remove these ions the easiest method is forced air.

Simply blowing some air through the gap with a fan can help.

Also using a number of narrow gaps in series rather

one single wide one helps by distributing the heat

across several gaps. Less heat means fewer ions.

Higher speed air helps but the best method has been

to suck high speed air through the teeth of the gap

with a strong vacuum cleaner. In fact in the vacuum

gap I built the air speed is too high and this tends

to over quench and reduce performance. I get best output

running my vacuum motor at about 60 percent. Others using

vacuum gaps report similar results.

A number of these vacuum gaps (mine included) are pretty

much standard RQ type cylinder gaps with slots cut into the PVC

housing to admit airflow between the electrodes.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum spark gap

Cc:

Bcc:

X-Attachments:

At 04:38 PM 4/10/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 10 Apr 1998 09:38:14 -0700

>Subject: Re: vacuum spark gap

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: Gene Schulhauser <genesch@azstarnet.com>

>> Date: Thu, 09 Apr 1998 20:24:40 -0700

>> Subject: vacuum spark gap

>>

>> I understand that it is desirable to have a

>> rapid-quenching spark gap. Would placing

>> the gap in a vacuum serve this purpose?

>>

>> Gene Schulhauser

>> Tucson, AZ

>>

>Gene,

>This is off the top of my head, and I may be wrong, but:

>1) wouldn't that be an effective Geissler Tube? QUITE conductive.

I may be wrong but I believe Geissler tubes as well as other

devices of this nature (neon tubes, plasma globes) have the

vacuum doped with ionizable noble gases. (neon, helium, argon, etc)

One consequence of metal electrodes, high voltage, and a vacuum

is the creation of X-rays. Not something to be taken lightly.

>2) Air is a dielectric, as opposed to a Vacuum, which is QUITE

>conductive, and doesn't really ever effectively stop conducting.

Vacuum is actually a dielectric as is air. Both have a certain

breakdown voltage. Vacuum is actually a quite good dielectric

as in the quite expensive Jennings vacuum caps.

>3) The point is to 'quench' conductivity, ie blow the conductive,

>ionized air away, to allow the caps to recharge.

>

>  One cycle of the 3-5 kHz: ...Caps charge, gaps breakdown, caps dump

>energy to primary, gaps become non-conductive (quench)..

>

>4) 'Tuning' a gap in a vacuum would be a bear!

By tuning I assume you mean varying the gap distance.

This is not really necessary and it usually leads to

blown caps and/or transformers as many just cant resist

opening the gap just a little more to get just a little

more output. A fixed cylinder type gap is preferred as the

adjustment is simply in increments of the number of electrodes

and if built correctly will never be "too wide".

>

>Now I have a couple of questions:

>

>1) Can (if) caps loose energy back thru the conducting gaps; what would

>2 diodes between the pri gaps and caps do for efficiency, given that

>potential loss?

Not much. Remember we are dealing with AC here. The caps

go through a complete polarity reversal twice each cycle.

>

>2) What effect does 'quench' rate have on gap frequency?

By gap frequency do you mean pulse rate? In static gaps

thats pretty much set by the AC line frequency at 120 PPS.

Twice each cycle. It is possible if using a rather small

cap to get more than one gap firing per haf cycle.

> Would air flow directed between the series gap electrodes & normal to the

>discharge (as opposed to parallel to it), be any advantage?

Not sure I quite understand what you are getting at.

Airflow serves two purposes. Cooling the electrodes

and removal of hot ions to improve quenching.

Best queching occurs when the airflow is directed

directly between the electrodes so it can stretch and cool the arc.

>

>3) Given that a cool gap element would quench more easily than a hot

>one, I'm thinking of a gap design where the gap elements would be cooled

>internally with a flow of a fluid dielectric (distilled water or cap

>oil); where the gap area would be 'adjacent sphericals' as opposed to

>'linearly tangent tubes', which would give 'point' discharge gaps, which

>could be more effectively 'blown'.

Thats not really a new idea but it doesnt really help that

much. This will surely help keep the electrodes cool but it

does nothing for the removal of hot ions to aid quenching.

Airflow accomplishes both.

>

>I toss these out, in hopes of stimulating further conjecture and perhaps

>informed response, realizing that R&D, while anathema to production, is

>the heart and soul of product improvement...

>

>Gene Waldo

>

Keep tossing Gene.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sat, 11 Apr 1998 02:37:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 10 Apr 1998 21:37:29 -0500

Subject: Re: vacuum spark gap

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Scott MacGregor <smacgregor@aasp.net>

> Date: Fri, 10 Apr 1998 23:43:30 -0700

> Subject: Re: vacuum spark gap

> Organization: EES,Co.

<SNIP>

>

> Wacky thought! What if a fluid solvent was to wash away the deposits?

> --

>

> Regards,

> Scott MacGregor EES,Co.

Scott,

Well, not TOO wacky - this principle is used in large oil-insulated

circuit breakers and switchgear - the oil rapidly cools the arc and

electrodes, helping to quench the power arc. Unfortunately, the high

temperature arc chemically breaks down some of the oil during this

process. This results in the formation of microscopic conductive carbon

particles which stay suspended in the oil, reducing its dielectric

strength, so it's not really very practical for repetitive (Tesla Coil)

operation. Other insulating fluids and high pressure gases (N2, SF6,

air) have been used as well, but thus far for Tesla Coils the best

solution seems to be using multiple air-insulated gaps and rapid air

flow (static/vacuum gaps as Mike has described) or moving/fixed

electrodes in a rotary gap.

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Binary Xmissions??

Date: Sat, 11 Apr 1998 02:45:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 10 Apr 1998 19:45:08 -0700

Subject: Binary Xmissions??

Organization: Self

Mike & All,

Nothing can be done about the List's intolerance of binary files?

I am working on xmitting hand sketches (hate ASCII jobs), but will have

to be binary, I suppose...

As for ASCII's, the "Dwg."'shift' can alleviated some times by slight

widening of the window, I have found.

Mike, If this works out, I'll send you our ckt, as there are some mods I

would appreciate comment on...  Also I'm not clear on the Question: pri

cap in series or across pri coil?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sat, 11 Apr 1998 06:21:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sat, 11 Apr 1998 02:21:51 -0400 (EDT)

Subject: Re: vacuum spark gap

On Sat, 11 Apr 1998, Tesla-2 wrote:

>

> Wacky thought! What if a fluid solvent was to wash away the deposits?

>

Yeah, but the metal is sputtering from one conductor to the other - you'd

have the conductor eroding away on you.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Binary Xmissions??

Date: Sat, 11 Apr 1998 06:23:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Fri, 10 Apr 1998 23:23:12 PDT

Subject: Re: Binary Xmissions??

It was written:

>I am working on xmitting hand sketches (hate ASCII jobs), but will have

>to be binary, I suppose...

Why not get a webpage (possibly for free via geocities etc) and place

the artwork there for all to see (or not-see)?

For example - the envelope detection circuit for tuning coils:

http://www.geocities.com/CapeCanaveral/Hangar/6160/envelope.gif

(It's rough but I was asked to provide it.)

Just a thought...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sat, 11 Apr 1998 06:43:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Fri, 10 Apr 1998 23:43:30 -0700

Subject: Re: vacuum spark gap

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Thu, 09 Apr 1998 18:23:57 -0500

> Subject: Re: vacuum spark gap

> Organization: Stoneridge Engineering

>

> Tesla-2 wrote:

> >

> > From: Gene Schulhauser <genesch@azstarnet.com>

> > Date: Thu, 09 Apr 1998 20:24:40 -0700

> > Subject: vacuum spark gap

> >

> > I understand that it is desirable to have a

> > rapid-quenching spark gap. Would placing

> > the gap in a vacuum serve this purpose?

> >

> > Gene Schulhauser

> > Tucson, AZ

> >

>

> Gene,

>

> Excellent Question!

>

> In theory - YES! If you could create an extremely good vacuum, there

> would be little remaining gas to ionize. However, even a vacuum will

> "break down" under enough voltage stress, permitting current to flow

> through a conducting path made up of vaporized electrode ions and

> electrons. Once the energy dissipation in the gap declines to a low

> enough level, the metal vapor redeposits on the side walls, and the gap

> rapidly recovers.

>

> Vacuum switches have become very popular in high power switching (such

> as in power company HV circuit breakers and contactors) because of their

> excellent recovery characteristics. However, they're really not meant

> for the repetitive switching seen in Tesla Coils. Under repetitive

> switching, the main problems are erosion of the electrode material, and

> that the redeposited metal will eventually short-circuit the switcth.

> >From a more practical standpoint for coilers, a pressurized spark-gap

> using certain gases (pure hydrogen or sulfer hexafluoride for example)

> is the another "next best" alternative...

>

> Safe coiling to you, Gene!

>

> -- Bert --

>

Wacky thought! What if a fluid solvent was to wash away the deposits?

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: HV-wire

Date: Sat, 11 Apr 1998 07:01:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 11 Apr 1998 00:01:21 -0700

Subject: HV-wire

Organization: EES,Co.

I am wondering if this "cable" could be of interest to Tesla coilers?

Huge amounts of expanded PE insulation, around very large copper clad

aluminum center conductors? If this is of interest:

http://www.expage.com/page/bigwire

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sat, 11 Apr 1998 09:35:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fxphoto@centuryinter.net (fxphoto)

Date: Sat, 11 Apr 1998 05:35:23 -0400 (EDT)

Subject: Re: vacuum spark gap

>From: Gene Waldo <gwaldo@1stnetusa.com>

>

>3) Given that a cool gap element would quench more easily than a hot

>one, I'm thinking of a gap design where the gap elements would be cooled

>internally with a flow of a fluid dielectric (distilled water or cap

>oil); where the gap area would be 'ajacent sphericals' as opposed to

>'linearly tangent tubes', which would give 'point' discharge gaps, which

>could be more effectively 'blown'.

>

>I toss these out, in hopes of stimulating further conjecture and perhaps

>informed response, realizing that R&D, while anathema to production, is

>the heart and soul of product improvement...

>

>Gene Waldo

Gene,

What a simple design! I like it. For lack of an informed response from me

here is more conjecture..( well I am still learning ).

Start with a 3/4" thick 30" long plywood board about 3" wide and epoxy a

30" length of 2" dia copper tube to one side of it. The center of the tube

should be 1/2" in from the edge of the board. Mark the tube in 1"

increments. Drill a 1/4" hole vertically through the tube and wood through

the first two marks. Do not drill the third mark, but skip it and drill the

fourth and fifth. Skip the sixth and continue in this way for the length of

the tube. ( drill two, skip one) Put 1/4" bolts through the holes to bolt

the tube to the wood. Using a fine bladed saw, cut through the tube at the

skipped marks. The width of the cut will be the width of your gap. Attach

your vacuum cleaner to one end of the tube ( a high speed blower might even

work well ) and an air volume control on the other.

The end result should be ten tubes 3" long, mounted end to end on a base board.

The addition of a central 1 1/2" dia tube ( non conductive and non flamable)

through the center of the 2" tubes would improve the velocity of the air and

therefore the venturi effect , pulling the arc into the fast moving air and

shearing it off.

The problems that I see, are

1) maintaining an even gap width and cleaning the burrs from cutting the gap.

2) The length, I believe would cause extra power loss through resistance.

( I think the better quenching might override this though)

3) what material to use for the central tube? Fiberglass made with epoxy

resin maybe.

Additional conjecture:

Has anyone explored Geiger-Meuller tubes used in Geiger counters?

They operate at 1KV potential and a very low amperage (probably less than a

microamp). I don't remember the ionizing gas (maybe nitrogen or argon) but

it is mixed with a halogen of either chlorine or bromine in order to quench

the ionization.

Bill Turbett

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Binary Xmissions??

Date: Sat, 11 Apr 1998 18:34:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sat, 11 Apr 1998 11:34:17 -0700

Subject: Re: Binary Xmissions??

Organization: Self

Tesla-2 wrote:

>

> From: "Bill the arcstarter" <arcstarter@hotmail.com>

> Date: Fri, 10 Apr 1998 23:23:12 PDT

> Subject: Re: Binary Xmissions??

>

> It was written:

> >I am working on xmitting hand sketches (hate ASCII jobs), but will have

> >to be binary, I suppose...

>

> Why not get a webpage (possibly for free via geocities etc) and place

> the artwork there for all to see (or not-see)?

>

> For example - the envelope detection circuit for tuning coils:

> http://www.geocities.com/CapeCanaveral/Hangar/6160/envelope.gif

> (It's rough but I was asked to provide it.)

>

> Just a thought...

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

Bill,

This too, will probably come....  For the time being, I am somewhat

daunted by such additional new prospects, and have, at the moment,

little concept of the intracies of such a setup process.  I was told by

my server that one was available to me, but need to better understand

the "to-do's" and "not-to-do's" of web pages; I have seen some that

'worked' and some that I thot clumsy...

Do I need that CD (paraphrasing): "10,000 Buttons for Webpage Builders"?

How do I devise correct organization & strategies? Or is it really much

simpler?

For the time being, Mike Hammer has offered to host my excreta on his

site.

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large Co-ax

Date: Sun, 12 Apr 1998 02:42:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sat, 11 Apr 1998 19:42:28 -0700

Subject: Re: Large Co-ax

Organization: EES,Co.

If anyone has an interest in large diameter co-ax "hard" cable, I've

posted information at http://www.expage.com/page/bigwire. This cable has PE

outer jacket, solid aluminum outer conductor, expanded PE dielectric,

and a copper plated aluminum center conductor. It can be stripped of the

outer shield, leaving a large conductor covered by a very thick layer of

PE. Might have uses for coilers.

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Binary Xmissions??

Date: Sun, 12 Apr 1998 04:10:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sun, 12 Apr 1998 00:10:47 -0400 (EDT)

Subject: Re: Binary Xmissions??

On Sat, 11 Apr 1998, Tesla-2 wrote:

> Do I need that CD (paraphrasing): "10,000 Buttons for Webpage Builders"?

>

I wouldn't buy it, if you're decent enough with HTML you won't need it.

> How do I devise correct organization & strategies? Or is it really much

> simpler?

>

HTML is pretty simple.  Contact me off-list and I'll give you more

detailed info on the ins and outs.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 07:10:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Sun, 12 Apr 1998 00:10:21 -0700 (PDT)

Subject: Re: vacuum spark gap

It was written:

> Vacuum switches have become very popular in high power switching (such

> as in power company HV circuit breakers and contactors) because of their

> excellent recovery characteristics. However, they're really not meant

> for the repetitive switching seen in Tesla Coils....

> From a more practical standpoint for coilers, a pressurized spark-gap

> using certain gases (pure hydrogen or sulfer hexafluoride for example)

> is the another "next best" alternative...

> -- Bert --

Bert -

Correct me if I'm wrong, but isn't hydrogen explosively flammable?

Wouldn't the spark from the gap ignite it with some rather unpleasant results?

(Re: Hindenburg disaster.) Just wondering...

-- MadManDan --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 12:00:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 12 Apr 1998 08:00:14 EDT

Subject: Re: vacuum spark gap

MadmanDan,

Hydrogen is extremely flammable if oxygen is present.  I presume the setup

Bert was referring to is designed to exclude air/oxygen which would be okay.

I am not sure I would risk that myself, but it is possible to have such a

system.  I would try helium before that since it is inert.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 12:54:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 12 Apr 1998 07:54:46 -0500

Subject: Re: vacuum spark gap

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Sun, 12 Apr 1998 00:10:21 -0700 (PDT)

> Subject: Re: vacuum spark gap

>

>  It was written:

>

> > Vacuum switches have become very popular in high power switching (such

> > as in power company HV circuit breakers and contactors) because of their

> > excellent recovery characteristics. However, they're really not meant

> > for the repetitive switching seen in Tesla Coils....

> > From a more practical standpoint for coilers, a pressurized spark-gap

> > using certain gases (pure hydrogen or sulfer hexafluoride for example)

> > is the another "next best" alternative...

>

> > -- Bert --

>

>  Bert -

>         Correct me if I'm wrong, but isn't hydrogen explosively flammable?

> Wouldn't the spark from the gap ignite it with some rather unpleasant results

?

> (Re: Hindenburg disaster.) Just wondering...

>                                                 -- MadManDan --

>

MadManDan,

Hydrogen is explosive only when mixed with an oxidizing gas, such as

oxygen. A sparkgap that's surrounded by pure hydrogen actually quenches

(recovers its insulating properties) considerably faster than one

surrounded by air, and if properly sealed poses no great explosive

danger.

The quenching and cooling properties of hydrogen has been used to make

high speed sparkgaps (called Quenchotrons) which were capable of hitting

up to 100,000 breaks per second at up to 10,000 Amps/pulse! These have

been used to generate stroboscopic (freeze-motion) x-rays.

A Quenchotron consisted of a series of copper or copper-tungsten disks,

each separated by a mica separator to create a gap of only 0.004". These

operated in a pure hydrogen atmosphere. Each gap broke down at around

400 volts, and you merely connected enough in series to get to the

operating voltage you desired.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 13:24:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sun, 12 Apr 1998 09:24:25 -0400 (EDT)

Subject: Re: vacuum spark gap

On Sun, 12 Apr 1998, Tesla-2 wrote:

>         Correct me if I'm wrong, but isn't hydrogen explosively flammable?

> Wouldn't the spark from the gap ignite it with some rather unpleasant results

?

> (Re: Hindenburg disaster.) Just wondering...

>                                                 -- MadManDan --

Not if you backfill with hydrogen - no air.  Without air, you couldn't

hardly light a match in there.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 15:45:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Sun, 12 Apr 1998 08:45:20 -0700

Subject: Re: vacuum spark gap

Organization: Self

Tesla-2 wrote:

>

> From: Dan Murphy <danmurphy@1stnetusa.com>

> Date: Sun, 12 Apr 1998 00:10:21 -0700 (PDT)

> Subject: Re: vacuum spark gap

>

>  It was written:

> > Vacuum switches have become very popular in high power switching

> >(such as in power company HV circuit breakers and contactors) because

> >of their excellent recovery characteristics. certain gases (pure

> >hydrogen or sulfur hexafluoride for example)

> >- Bert --

>

>  Bert -

>  Correct me if I'm wrong, but isn't hydrogen explosively flammable?

> Wouldn't the spark from the gap ignite it with some rather unpleasant

> results?

> (Re: Hindenburg disaster.) Just wondering...

>                                                 -- MadManDan --

>

Dan,

HAH!! YES! You are a WRONGO-ARISSIUM-ARRU-AREENIONI!   Well, partly...

Hydrogen does combine readilly with Oxygen, producing copious quantities

of steam, given very slight excuse.  However, Hydrogen (H2) in an

uncontaminated (sans O2) form will generally just sit there and take

lots of abuse.

Which brings the interesting question: since: H2+O2-> H2O+O, and since

Ozone (O3) is such a super oxidixer, what would H2+O3 -> H2O+O2 give us?

Hotter(?) Steam that supports combustion?  Is O3 stable enough to last

for this mixture? or would the reaction be spontaneous? Sounds like

maybe a neat rocket fuel?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 16:10:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Sun, 12 Apr 1998 09:10:28 -0700 (PDT)

Subject: Re: vacuum spark gap

>>         Correct me if I'm wrong, but isn't hydrogen explosively flammable?

>> Wouldn't the spark from the gap ignite it with some rather unpleasant results?

>> (Re: Hindenburg disaster.) Just wondering...

>>                                                 -- MadManDan --

>

>MadManDan,

>

>Hydrogen is explosive only when mixed with an oxidizing gas, such as

>oxygen. A sparkgap that's surrounded by pure hydrogen actually quenches

>(recovers its insulating properties) considerably faster than one

>surrounded by air, and if properly sealed poses no great explosive

>danger.

>

>-- Bert --

Gentlemen:

I stand corrected. (I figured there must be a catch somewhere.)

-- MadManDan --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum spark gap

Cc:

Bcc:

X-Attachments:

At 08:31 PM 4/12/98 GMT, you wrote:

>From: Scott MacGregor <smacgregor@aasp.net>

>Date: Sun, 12 Apr 1998 13:31:33 -0700

>Subject: Re: was vacuum spark gap

>Organization: EES,Co.

>

>Tesla-2 wrote:

>>

>> From: Gene Waldo <gwaldo@1stnetusa.com>

>> Date: Sun, 12 Apr 1998 08:45:20 -0700

>> Subject: Re: vacuum spark gap

>> Organization: Self

>>

>> Hydrogen does combine readilly with Oxygen, producing copious quantities

>> of steam, given very slight excuse.  However, Hydrogen (H2) in an

>> uncontaminated (sans O2) form will generally just sit there and take

>> lots of abuse.

>> Which brings the interesting question: since: H2+O2-> H2O+O, and since

>> Ozone (O3) is such a super oxidixer, what would H2+O3 -> H2O+O2 give us?

>> Hotter(?) Steam that supports combustion?  Is O3 stable enough to last

>> for this mixture? or would the reaction be spontaneous? Sounds like

>> maybe a neat rocket fuel?

>>

>>Gene

>

>Gene,

>

>It's been a few years since my last Chemistry classes, but if I remember

>correctly, the formula is 2H2+O2+(energy)->2H2O+heat. No oxygen in the

>product. The reverse is, 2H2O+(energy)->2H2+O2. A ratio of 2 hydrogen to

>each oxygen.

>

>With ozone it could be: 3H2+2O3+(energy)->3H20+O3! Beyond me!

We've strayed a little far from the topic with this thread.

Lets end it here.

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 19:15:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Sun, 12 Apr 1998 13:15:08 -0600

Subject: Re: vacuum spark gap

Gene wrote:

>Hydrogen does combine readilly with Oxygen, producing copious quantities

>of steam, given very slight excuse.  However, Hydrogen (H2) in an

>uncontaminated (sans O2) form will generally just sit there and take

>lots of abuse.

>Which brings the interesting question: since: H2+O2-> H2O+O, and since

>Ozone (O3) is such a super oxidixer, what would H2+O3 -> H2O+O2 give us?

>Hotter(?) Steam that supports combustion?  Is O3 stable enough to last

>for this mixture? or would the reaction be spontaneous? Sounds like

>maybe a neat rocket fuel?

Yup. It makes a great rocket fuel. Sort of. Gaseous O3 isn't dense enough,

so usually liquid ozone is used (LoZ). The problem is that liquid ozone is

real touchy and can spontaneously explode. Another great oxidizer is O3Fl3.

Much more bang for your buck. However, given the restrictions of LoZ,

combined with the even more disgusting characteristics of liquid fluorine,

this is only a near-theoretical oxidizer limited to the lab.

Anyway, back to the subject. I haven't completed my first Tesla coil yet,

so I wouldn't know. How much ozone to they produce? I wonder if you could

trap the O3 and then pipe it into the spark gap, just for pyrotechnic

effect? :-) For air blast gaps, would a pressurized gas work better? Coming

out of a tank it would be much cooler than simply pressurized air. Has

anybody tried this?

Daniel Lockhart

p.s. I started winding my RF chokes last night on the new 2-inch diameter

ferrite donuts with awg 28 wire. Not fun. Is there an easy way to wind

them!? I thought my secondary coil was boring...

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Binary Xmissions??

Cc:

Bcc:

X-Attachments:

At 02:45 AM 4/11/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 10 Apr 1998 19:45:08 -0700

>Subject: Binary Xmissions??

>Organization: Self

>

>Mike & All,

>Nothing can be done about the List's intolerance of binary files?

The list server will handle binaries but insists on encoding

them base64 (MIME). It also insists on breaking files larger

than 32k into 32k blocks. What this means is you have to reassemble

the blocks and decode the base64 encoding. Not real difficult

to do but a pain. Its simpler to avoid binaries altogether

and that avoids the problem of subscribers who do not want or

have email accounts that cannot handle binaries.

>

>I am working on xmitting hand sketches (hate ASCII jobs), but will have

>to be binary, I suppose...

>As for ASCII's, the "Dwg."'shift' can aleviated some times by slight

>widening of the window, I have found.

Ascii diagrams can get clobbered by various mail programs

that insist on reformating the text to suit them.

Microsoft programs are terrible this way as well as

their insistance on HTML as the default file type for email.

>

>Mike, If this works out, I'll send you our ckt, as there are some mods I

>would appreciate comment on...

Send away. I am working on updates on my web page that include the

pics you already sent. If you have other drawings or pics you

want including send them ASAP.

>Also I'm not clear on the Question: pri cap in series or across pri coil?

This question seems to come up again and again because their are many

old coil books and schematics that show the main cap connected directly

across the transformer. This arrangement will work but subjects

the transformer to high levels of RF energy. If you consider that

the capacitor is part of the oscillator tank circuit and anything

in parallel with it is going to see the same voltages you will

begin to see the problem.

By placing the gap directly across the transformer instead

the transformer is somewhat protected from the RF voltage

by means of the energized gap. While the gap is firing

and while the tank circuit is oscillating the gap appears

as a short circuit across the transformer secondary winding.

There is no performance difference between the 2 arrangements.

The shunt gap is preferred soley on the basis of transformer

protection. And lets face it, our poor over worked neons need

all the help they can get.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: HV-wire

Cc:

Bcc:

X-Attachments:

At 07:01 AM 4/11/98 GMT, you wrote:

>From: Scott MacGregor <smacgregor@aasp.net>

>Date: Sat, 11 Apr 1998 00:01:21 -0700

>Subject: HV-wire

>Organization: EES,Co.

>

>I am wondering if this "cable" could be of interest to Tesla coilers?

>Hugh amounts of expanded PE insulation, around very large copper clad

>aluminum center conductors? If this is of interest:

>http://www.expage.com/page/bigwire

>--

>

>Regards,

>Scott MacGregor EES,Co.

Scott,

I dont think this will make acceptable HV wire due to

the foam polyethylene dielectric. Foam dielectric cables

are great for RF service where the air in the foam reduces

losses. But this same air will seriously degrade voltage

handling capability.

Check the voltage specs for foam RG8 cables vs solid polyethylene

RG213 cables. Foam RG8 has a voltage rating of 1500 volts RMS.

RG213 rating is 5kv RMS.

This cable seems to me to be very similar to that used by cable TV.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 20:31:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Sun, 12 Apr 1998 13:31:33 -0700

Subject: Re: vacuum spark gap

Organization: EES,Co.

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Sun, 12 Apr 1998 08:45:20 -0700

> Subject: Re: vacuum spark gap

> Organization: Self

>

> Hydrogen does combine readilly with Oxygen, producing copious quantities

> of steam, given very slight excuse.  However, Hydrogen (H2) in an

> uncontaminated (sans O2) form will generally just sit there and take

> lots of abuse.

> Which brings the interesting question: since: H2+O2-> H2O+O, and since

> Ozone (O3) is such a super oxidixer, what would H2+O3 -> H2O+O2 give us?

> Hotter(?) Steam that supports combustion?  Is O3 stable enough to last

> for this mixture? or would the reaction be spontaneous? Sounds like

> maybe a neat rocket fuel?

>

>Gene

Gene,

It's been a few years since my last Chemistry classes, but if I remember

correctly, the formula is 2H2+O2+(energy)->2H2O+heat. No oxygen in the

product. The reverse is, 2H2O+(energy)->2H2+O2. A ratio of 2 hydrogen to

each oxygen.

With ozone it could be: 3H2+2O3+(energy)->3H20+O3! Beyond me!

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum spark gap

Date: Sun, 12 Apr 1998 23:19:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fxphoto@centuryinter.net (fxphoto)

Date: Sun, 12 Apr 1998 19:19:34 -0400 (EDT)

Subject: Re: vacuum spark gap

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Sun, 12 Apr 1998 00:10:21 -0700 (PDT)

>Subject: Re: vacuum spark gap

> Bert -

>        Correct me if I'm wrong, but isn't hydrogen explosively flammable?

>Wouldn't the spark from the gap ignite it with some rather unpleasant results?

>(Re: Hindenburg disaster.) Just wondering...

>                                                -- MadManDan --

>

Dan,

Bert is 100% right. I know from personal experience that they use a hydrogen

atmosphere to surround large generators as it is an excellent dielectric.

Hydrogen by itself is not explosively flammable, however if you add about

5% oxygen...Watch out!

Bill Turbett

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Dan's RF chokes-was vacuum gaps

Cc:

Bcc:

X-Attachments:

At 07:15 PM 4/12/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Sun, 12 Apr 1998 13:15:08 -0600

>Subject: Re: vacuum spark gap

>

>

>Anyway, back to the subject. I haven't completed my first Tesla coil yet,

>so I wouldn't know. How much ozone to they produce? I wonder if you could

>trap the O3 and then pipe it into the spark gap, just for pyrotechnic

>effect? :-)

The spark gap is already a major producer of ozone.

Ozone can cause serious respiratory problems so

provide adequate ventilation.

>For air blast gaps, would a pressurized gas work better? Coming

>out of a tank it would be much cooler than simply pressurized air. Has

>anybody tried this?

Probably wouldnt make a great deal of difference. My short experience

with air blast gaps was not worthwhile. They are quite loud

and didnt really give me any better results than a good cylinder gap.

I've had the best luck so far with a modified cylinder gap

using vacuum cleaner for airflow.

>

>Daniel Lockhart

>

>p.s. I started winding my RF chokes last night on the new 2-inch diameter

>ferrite donuts with awg 28 wire. Not fun. Is there an easy way to wind

>them!? I thought my secondary coil was boring...

Toroids are a pain to wind especially if you have many turns.

The best way is to use a winding shuttle. Cut a strip of wood

a few inches long with a V notch in each end. Wind sufficient

wire on the wood strip to wind the toroid. Wind the wire

lengthwise on the strip laying the wire in the V notches.

Use the strip to pass the wire through the center of the toroid

as you wind.

Where did you get the toroids? New from Amidon or surplus?

Large ferrites in surplus have been a bit rare lately.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Dan's RF chokes-was vacuum gaps

Date: Mon, 13 Apr 1998 03:42:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Sun, 12 Apr 1998 23:42:37 -0400 (EDT)

Subject: Re: Dan's RF chokes-was vacuum gaps

On Mon, 13 Apr 1998, Tesla-2 wrote:

> The best way is to use a winding shuttle.

>

If you're really set against winding them manually - design and build a

toroid winding machine.  Should be easy enough to make IT pass the shuttle

around.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Quenchotrons!

Date: Mon, 13 Apr 1998 17:48:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Mon, 13 Apr 1998 10:48:00 PDT

Subject: Quenchotrons!

Bert Hickman wrote:

>The quenching and cooling properties of hydrogen has been used to make

>high speed sparkgaps (called Quenchotrons) which were capable of hitting

>up to 100,000 breaks per second at up to 10,000 Amps/pulse! These have

>been used to generate stroboscopic (freeze-motion) x-rays.

>

>A Quenchotron consisted of a series of copper or copper-tungsten disks,

>each separated by a mica separator to create a gap of only 0.004".

>These operated in a pure hydrogen atmosphere. Each gap broke down at around

>400 volts, and you merely connected enough in series to get to the

>operating voltage you desired.

Often Hydrogen is the "running" environment of choice for large rotating

machines such as motor-generator sets, power dynamos, etc.  I was at a

hydro-plant once where their equipment ran under hydrogen.

Supposedly hydrogen offers a few benefits:

*Lower windage losses (less 'viscous' than air)

*Better heat transfer capacity than air

Perhaps it also acts as a dielectric or insulator to some extent - just

a guess...

They ran the hydrogen at either above or below atmospheric pressure.

Something about if it did leak - the results would be safer one way vs

the other, but I can't recall exactly which way it worked.

Can anyone provide me with additional references about H-quenched gaps

like the Quenchotron Bert described?  I'd thought about building

something like this once... Any chance argon would work?  (I have argon,

but no hydrogen)

Mike Hammer once told me of the mica-copper quenched gaps used in the

early days of radio.  These things ran in air, at least until the arcing

burned up all the O2 and N2 trapped inside the gaps.  My tesla site has

a couple of links along these lines.

Did these quenchotrons use hydrogen at atmospheric pressure and temp, or

at some other pressure??

More about this later...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Binary Xmissions??

Date: Tue, 14 Apr 1998 13:30:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 14 Apr 1998 18:30:37 +0500

Subject: Re: Binary Xmissions??

On Sun, 12 Apr 1998 14:20:38 -0500 In Tesla-2

Mike Hammer <mhammer@misslink.net> wrote;

> The list server will handle binaries but insists on encoding

> them base64 (MIME). It also insists on breaking files larger

> than 32k into 32k blocks. What this means is you have to reassemble

> the blocks and decode the base64 encoding. Not real difficult

> to do but a pain. Its simpler to avoid binaries altogether

> and that avoids the problem of subscribers who do not want or

> have email accounts that cannot handle binaries.

I'm rather amazed! Most listserve programs have no problem dealing with UUEncoded

or XXEncoded binary files. The 32K blocks is unbelievable it's the first I've heard

that's actualy worse than JUNO's 64K block limitation!

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Large vs small primary L

Date: Tue, 14 Apr 1998 13:30:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 14 Apr 1998 18:30:39 +0500

Subject: Re: Large vs small primary L

On Thu, 02 Apr 1998 19:42:23 -0600 In Tesla-2

Mike Hammer <mhammer@misslink.net> wrote;

> I also agree with the points you make here and thats exactly

> how the math works out. But, as Im sure you'll agree, Tesla

> coils often defy mathematical attempts to predict their behaviour.

More like NEVER agree with the mathematical models! Seriously there are many

variables that are not even being considered yet.

> While in theory what you outlined above is true in practice

> I dont think its valid. In my opinion and in my experience

> high L low C coils perform better than high C low L coils.

> I believe the increase in I2R losses and gap losses in low

> L coils more than make up for the theoretical increase.

>

> Perhaps with a very low loss system with a truly exceptional

> gap system this might apply but how many of us can say our coils

> fall into that category?

>

> A good example is John Freau's coil. He has had 42 inches

> of output from his coil with a single 12kv/30ma neon.

> There is really nothing remarkable about John's coil

> EXCEPT for a 34 turn primary coil.

>

> My own coil has delivered more output for less input

> than most others with similar coils. I get 54 inches

> of output with just over 1200 VA. I have a 17 turn

> primary and the more turns I have tapped in the better

> it works.

I think it depends on how the coil is designed from the start. Dr. Tesla's

coils mostly had only one or two turns on their primarys and were rather good

performers. I have had great success with primaries of less than 5 turns, but

those coils also tended to approach a length/diameter ratio of 1 to 2 for the

secondary also like most of Dr. Tesla's.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Quenchotrons!

Date: Tue, 14 Apr 1998 13:45:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Tue, 14 Apr 1998 06:45:16 -0700

Subject: Re: Quenchotrons!

Organization: EES,Co.

Tesla-2 wrote:

>

> From: "Bill the arcstarter" <arcstarter@hotmail.com>

> Date: Mon, 13 Apr 1998 10:48:00 PDT

> Subject: Quenchotrons!

>

>

> They ran the hydrogen at either above or below atmospheric pressure.

> Something about if it did leak - the results would be safer one way vs

> the other, but I can't recall exactly which way it worked.

I would assume that they would run the units with a slight positive

pressure of hydrogen. (maybe 1 lb/sq in) Any migration of H2 could be

carried away into the atmosphere by circulating fans.

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Report on My First Coil

Date: Tue, 14 Apr 1998 18:33:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Tue, 14 Apr 1998 12:33:46 -0600

Subject: Report on My First Coil

Hey all,

I think I've came to a good place where I know what everything is going to

be like on my first coil. I thought I'd through out the numbers and see if

anybody had any comments.

T1:             9 kV, 30+ ma rebuilt neon sign transformer      Done

T1 safety

components:    One safety spark gap                            Not made

Two 2-inch diameter ferrite toroid chokes       Done

Spark Gap:      RQ-type with copper pipe; 6 gaps 0.028" apart   Not made

Capacitor:      Saltwater/bottle type; 0.0088 microfarads       Not made

Primary:        20 turns of 0.25" copper refrigeration tubing

0.25" apart at a 30 degree angle        In-the-process

Secondary:      A little over 1000 feet of awg 28 wire

on a 3.5" O.D. pvc pipe                 Done

Discharge

terminal:      One beautiful 10.5" O.D. 4.25" thick toroid     Done

Enclosure:      One 18" x 18" x 16" gloss black wooden box      Done

Future upgrades:

(a). Add one more 9 kV, 30 ma transformer to bring it up to 9 kv, 60 ma. I

don't know exactly how or where this would help, but I wouldn't think it

would hurt.

(b). Replace the saltwater caps with a rolled poly. I can't to this until I

add the second neon transformer, as this will affect the capacitor size. I

don't want to add the second transformer now, because I want to start small

and figure this whole thing out first.

Any comments, criticism or suggestions are very welcomed!

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitor Equation Coefficients!?

Date: Tue, 14 Apr 1998 18:36:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Daniel Lockhart <molly@cmn.net>

Date: Tue, 14 Apr 1998 12:36:58 -0600

Subject: Capacitor Equation Coefficients!?

Hey all,

For my tesla coil I'm making a temporary saltwater/bottle cap. I've come

across what seems like (to me) conflicting information and equations.

I have three sourses:

a). The "math.txt" from the Tesla Coil Archive

(http://www.velocity.net/~djb/files.shtml),

b). The java page at http://www.ticnet.com/jgore/tesla/javates.htm#Index

and

c). The e-mail exchanges between Bert Hickman and Gene Waldo on Tesla-2.

The equation for computing the capacitance of Leyden-type caps on the file

"math.txt" is:

2

C = .0884 k ( pi  r   + 2  pi  r  l )

------------------------------

1,000,000 t

C = Capacitance in microfarads

k = dielectric strength

r = jar radius in centimeters

l = height of the jar portion used (in centimeters)

t = thickness of the jar wall in centimeters

pi = 3.141592654

2

r   = r x r  (radius squared)

This gives a capacitance value of 0.00038741 microfarads for a jar with a

1.5 inch radius, dielectric strength ("k") of 6, thickness of 0.1 inches,

and water-touching height of 7 inches.

Plugging in the same numbers in the java page, I get 0.00041657 microfarads.

Using Bert's equation (as below), and converting to picofarads to

microfarads, I get: 0.00098516 microfarads. A difference of 2.36 times!

Which equation should I go with?

Back a few weeks with Gene and Bert:

> Gene,

> There's a problem in the formula you're using....

>

>      Dielectric Constant (k):        7 (Approx.)

>      Jar Diameter (D)                3.6   Inches

>      Jar Height Used  (H)            7.25  Inches

>      Thickness of Jar (T)            0.1   Inches

>      Number of caps in parallel (N)  14

>      Area (A) = Pi*D*H + Pi*(D/2)^2 = 92.2 Square Inches

>

>    Plugging the above numbers into the capacitance equation:

>      C = 0.2248*k*A/T  (per bottle) = 1450 pF

>      Ctot = 14*C = 0.023 uF

>

> Hope this helps!

>

> --Bert --

> Bert,

> That helps a lot!  I KNOW, however that someone published the eq w/the

> denominator as:  T*1,000,000! I couldn't have made that up!  Could I?

The equation Gene used was in microfarads, hence the 1 000 000 in the

denominator. Thus, the only difference between Bert's equation and the

equation from "math.txt" is the coefficient in the numerator. One

coefficient is 0.0884 whilst the other is 0.2248. I suspect the coefficient

for the java equation was the only thing that was different with it too.

What coefficient should I go with? Do different coefficients reflect

different amounts of optimism or capacitor making ability? :-) My needed

capacitance is 0.0088 microfarads, so this means I need anywhere from 23 to

9 bottles! This hardly seems precise... :-)

I certainly hope Bert's equation is closer, since I don't really look

forward to putting together 23 bottles for this small Tesla coil I'm

making! Maybe I should just take the average of all three equations...  :-)

Thanks for any help,

Daniel Lockhart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Equation Coefficients!?

Date: Tue, 14 Apr 1998 21:05:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Tue, 14 Apr 1998 17:05:25 -0400

Subject: Re: Capacitor Equation Coefficients!?

At 06:36 PM 4/14/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Tue, 14 Apr 1998 12:36:58 -0600

>Subject: Capacitor Equation Coefficients!?

>

>Hey all,

>

>For my tesla coil I'm making a temporary saltwater/bottle cap. I've come

>across what seems like (to me) conflicting information and equations.

>

>I have three sourses:

>

>a). The "math.txt" from the Tesla Coil Archive

>(http://www.velocity.net/~djb/files.shtml),

>

>b). The java page at http://www.ticnet.com/jgore/tesla/javates.htm#Index

>

>        and

>

>c). The e-mail exchanges between Bert Hickman and Gene Waldo on Tesla-2.

>

>The equation for computing the capacitance of Leyden-type caps on the file

>"math.txt" is:

>

>                        2

>    C = .0884 k ( pi  r   + 2  pi  r  l )

>         ------------------------------

>                 1,000,000 t

>

>    C = Capacitance in microfarads

>    k = dielectric strength

>    r = jar radius in centimeters

>    l = height of the jar portion used (in centimeters)

>    t = thickness of the jar wall in centimeters

>   pi = 3.141592654

>

>      2

>    r   = r x r  (radius squared)

>

>This gives a capacitance value of 0.00038741 microfarads for a jar with a

>1.5 inch radius, dielectric strength ("k") of 6, thickness of 0.1 inches,

>and water-touching height of 7 inches.

>

>Plugging in the same numbers in the java page, I get 0.00041657 microfarads.

>

>Using Bert's equation (as below), and converting to picofarads to

>microfarads, I get: 0.00098516 microfarads. A difference of 2.36 times!

>

>Which equation should I go with?

>

>Back a few weeks with Gene and Bert:

>

>> Gene,

>> There's a problem in the formula you're using....

>>

>>      Dielectric Constant (k):        7 (Approx.)

>>      Jar Diameter (D)                3.6   Inches

>>      Jar Height Used  (H)            7.25  Inches

>>      Thickness of Jar (T)            0.1   Inches

>>      Number of caps in parallel (N)  14

>>      Area (A) = Pi*D*H + Pi*(D/2)^2 = 92.2 Square Inches

>>

>>    Plugging the above numbers into the capacitance equation:

>>      C = 0.2248*k*A/T  (per bottle) = 1450 pF

>>      Ctot = 14*C = 0.023 uF

>>

>> Hope this helps!

>>

>> --Bert --

>

>> Bert,

>> That helps a lot!  I KNOW, however that someone published the eq w/the

>> denominator as:  T*1,000,000! I couldn't have made that up!  Could I?

>

>The equation Gene used was in microfarads, hence the 1 000 000 in the

>denominator. Thus, the only difference between Bert's equation and the

>equation from "math.txt" is the coefficient in the numerator. One

>coefficient is 0.0884 whilst the other is 0.2248. I suspect the coefficient

>for the java equation was the only thing that was different with it too.

>

>What coefficient should I go with? Do different coefficients reflect

>different amounts of optimism or capacitor making ability? :-) My needed

>capacitance is 0.0088 microfarads, so this means I need anywhere from 23 to

>9 bottles! This hardly seems precise... :-)

>

>I certainly hope Bert's equation is closer, since I don't really look

>forward to putting together 23 bottles for this small Tesla coil I'm

>making! Maybe I should just take the average of all three equations...  :-)

>

>Thanks for any help,

>

>Daniel Lockhart

The equation from the math.txt file is for METRIC units and not inches. You

didn't convert the inches into centimeters when you used that formula. That

is why your answer is so different from the other one. Hope this helps.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Report on My First Coil

Cc:

Bcc:

X-Attachments:

At 06:33 PM 4/14/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Tue, 14 Apr 1998 12:33:46 -0600

>Subject: Report on My First Coil

>

>Hey all,

>

>I think I've came to a good place where I know what everything is going to

>be like on my first coil. I thought I'd through out the numbers and see if

>anybody had any comments.

>

>T1:             9 kV, 30+ ma rebuilt neon sign transformer      Done

>T1 safety

> components:    One safety spark gap                            Not made

>                Two 2-inch diameter ferrite toroid chokes       Done

>Spark Gap:      RQ-type with copper pipe; 6 gaps 0.028" apart   Not made

>Capacitor:      Saltwater/bottle type; 0.0088 microfarads       Not made

>Primary:        20 turns of 0.25" copper refrigeration tubing

>                        0.25" apart at a 30 degree angle        In-the-process

>Secondary:      A little over 1000 feet of awg 28 wire

>                        on a 3.5" O.D. pvc pipe                 Done

>Discharge

> terminal:      One beautiful 10.5" O.D. 4.25" thick toroid     Done

>Enclosure:      One 18" x 18" x 16" gloss black wooden box      Done

>

>

Sounds good to me Daniel.

>Future upgrades:

>

>(a). Add one more 9 kV, 30 ma transformer to bring it up to 9 kv, 60 ma. I

>don't know exactly how or where this would help, but I wouldn't think it

>would hurt.

It will double your power input. 540 vs 270 VA. This will not

double your output however. It will add a few extra inches

of spark with no other changes required. However to make the most

of the added current capability you will need to increase cap size.

If you balance your increased cap size with an increase in toroid size

along with the increase in power you will get the best output.

>

>(b). Replace the saltwater caps with a rolled poly. I can't to this until I

>add the second neon transformer, as this will affect the capacitor size. I

>don't want to add the second transformer now, because I want to start small

>and figure this whole thing out first.

Thats a good plan.

One thing I would recommend is to make your primary roomy

enough to handle a 4.5 inch diameter secondary. That way

you can build a larger secondary without having to make

another primary.

The coupling may end up a little low on the 3 inch secondary

but low power coils often work best with rather loose coupling

anyway.

Keep us posted of your progress.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitor Equation Coefficients!?

Cc:

Bcc:

X-Attachments:

At 06:36 PM 4/14/98 GMT, you wrote:

>From: Daniel Lockhart <molly@cmn.net>

>Date: Tue, 14 Apr 1998 12:36:58 -0600

>Subject: Capacitor Equation Coefficients!?

>

>Hey all,

>

>For my tesla coil I'm making a temporary saltwater/bottle cap. I've come

>across what seems like (to me) conflicting information and equations.

>

<BIG SNIP>

Daniel,

When I used SW bottle caps on my first coil I could never

get the formulas to correlate well with reality. The dielectric

constant of the glass can vary so widely that its almost impossible

to predict the capacitance by formula.

I would suggest purchasing a meter to measure capacitance.

It will be an invaluable tool.

Using 12 oz brown beer bottles (long necks are best) I got an

average of 725 pf per bottle. 12 of these will get you .0087 mfd.

I would start with far less. Start with 6 or maybe 8 bottles and add

one at a time retuning for best output. Doing this you may find

that .0088 mfd is NOT the best value for your system.

With Tesla coils some experimentation is always called for.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RF & My new camera?

Date: Wed, 15 Apr 1998 00:49:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 14 Apr 1998 17:49:06 -0700

Subject: RF & My new camera?

Organization: Self

All,

I just dropped big bucks on a digital camera to shoot pics for my

friends on the net! In reality Bert pushed me into it.  Couldn't find

film for my old, Old OLD Polaroid! Promised him some pics of a rot SG I

had come across...  Anyway, if and when we* get this TESLA popping and

fuming, how close can I safely get w/o blowing the chips out of it?

I gather from all the pics on this T-2 list, that there seems little

danger....   But, y'alls don' know MY luck!! Any precautions?

*WE are Dan Murphy and I.  You can see some of our lashup taking shape

at Mike Hammer's diggs, in a section of his page called:

< http://www.misslink.net/mhammer/waldo.htm >

Dan and I are newbies and will welcome critique and questions.  Laughing

and pointing are verboten!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Equation Coefficients!?

Date: Wed, 15 Apr 1998 02:29:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 14 Apr 1998 21:29:22 -0500

Subject: Re: Capacitor Equation Coefficients!?

Organization: Stoneridge Engineering

Daniel,

The only real diffrence between the two forms is that the dimensions of

one on the math page are for capacitor plate dimensions in centimeters,

and the formula I showed are dimensioned in inches. The ratio of

.2248/0.0884 is the same as the ratio of cm/inch ~ 2.54. Pick whichever

you are more comfortable with...

-- Bert --

Tesla-2 wrote:

>

> From: Daniel Lockhart <molly@cmn.net>

> Date: Tue, 14 Apr 1998 12:36:58 -0600

> Subject: Capacitor Equation Coefficients!?

>

> Hey all,

>

> For my tesla coil I'm making a temporary saltwater/bottle cap. I've come

> across what seems like (to me) conflicting information and equations.

>

> I have three sourses:

>

> a). The "math.txt" from the Tesla Coil Archive

> (http://www.velocity.net/~djb/files.shtml),

>

> b). The java page at http://www.ticnet.com/jgore/tesla/javates.htm#Index

>

>         and

>

> c). The e-mail exchanges between Bert Hickman and Gene Waldo on Tesla-2.

>

> The equation for computing the capacitance of Leyden-type caps on the file

> "math.txt" is:

>

>                         2

>     C = .0884 k ( pi  r   + 2  pi  r  l )

>          ------------------------------

>                  1,000,000 t

>

>     C = Capacitance in microfarads

>     k = dielectric strength

>     r = jar radius in centimeters

>     l = height of the jar portion used (in centimeters)

>     t = thickness of the jar wall in centimeters

>    pi = 3.141592654

>

>       2

>     r   = r x r  (radius squared)

>

> This gives a capacitance value of 0.00038741 microfarads for a jar with a

> 1.5 inch radius, dielectric strength ("k") of 6, thickness of 0.1 inches,

> and water-touching height of 7 inches.

>

> Plugging in the same numbers in the java page, I get 0.00041657 microfarads.

>

> Using Bert's equation (as below), and converting to picofarads to

> microfarads, I get: 0.00098516 microfarads. A difference of 2.36 times!

>

> Which equation should I go with?

>

> Back a few weeks with Gene and Bert:

>

> > Gene,

> > There's a problem in the formula you're using....

> >

> >      Dielectric Constant (k):        7 (Approx.)

> >      Jar Diameter (D)                3.6   Inches

> >      Jar Height Used  (H)            7.25  Inches

> >      Thickness of Jar (T)            0.1   Inches

> >      Number of caps in parallel (N)  14

> >      Area (A) = Pi*D*H + Pi*(D/2)^2 = 92.2 Square Inches

> >

> >    Plugging the above numbers into the capacitance equation:

> >      C = 0.2248*k*A/T  (per bottle) = 1450 pF

> >      Ctot = 14*C = 0.023 uF

> >

> > Hope this helps!

> >

> > --Bert --

>

<SNIP>

> The equation Gene used was in microfarads, hence the 1 000 000 in the

> denominator. Thus, the only difference between Bert's equation and the

> equation from "math.txt" is the coefficient in the numerator. One

> coefficient is 0.0884 whilst the other is 0.2248. I suspect the coefficient

> for the java equation was the only thing that was different with it too.

>

> What coefficient should I go with? Do different coefficients reflect

> different amounts of optimism or capacitor making ability? :-) My needed

> capacitance is 0.0088 microfarads, so this means I need anywhere from 23 to

> 9 bottles! This hardly seems precise... :-)

>

> I certainly hope Bert's equation is closer, since I don't really look

> forward to putting together 23 bottles for this small Tesla coil I'm

> making! Maybe I should just take the average of all three equations...  :-)

>

> Thanks for any help,

>

> Daniel Lockhart

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Binary Xmissions??

Date: Wed, 15 Apr 1998 03:29:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>> The list server will handle binaries but insists on encoding

T>> them base64 (MIME). It also insists on breaking files larger

T>> than 32k into 32k blocks. What this means is you have to reassemble

T>I'm rather amazed! Most listserve programs have no problem dealing with

T>UUEncod

T>ed

Consider that the list server is running on my BBS computer system on

software written almost 10 years ago, with no upgrades presently

available, and none planned by the authors.  :(   Buying the latest &

(not so) greatest version of the BBS software just to be able to run

new listserver would cost me close to $1,200.00.

My ISP's system calls mine every hour and does a mail transfer. My

computer processes the incoming internet mail, and then calls back to my

ISP with the outbound list mail. It's reasonably fast, it keeps the list

secure 'cause it's in my house, and on my machine, and it's cheap,

costing me about $10.00 a month for the ISP service. If I put it

on my ISP's machine, it would triple the cost.

Ralph

---

Does your cat come when called...or is it a real cat?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: terminal

Date: Wed, 15 Apr 1998 03:44:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "krumins" <krumins@bvsd.k12.co.us>

Date: Tue, 14 Apr 1998 21:44:24 -0600

Subject: terminal

anyone know at what type of store i could purchase a metal ball for the

terminal on my tesla coil? thanks, Aaron Krumins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Waldo-Murphy-report/hammer pg.

Date: Wed, 15 Apr 1998 07:21:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 15 Apr 1998 00:21:04 -0700

Subject: Waldo-Murphy-report/hammer pg.

Organization: Self

Mike,

At this stage I don't think captions would be too meaningfull..

My concerns are:  1)eliminating ckt 'weirdness'.... before that gets

posted.

2) Did (you get) I send the 'bottom view' of pic#4 ? Could a simple

contact (perhaps even w/a small wad of steel wool (many vs one 'point

contact')between?)) to the 2ndry top be effective? Or is bolting the

order of the day?  My implementation of the 'bottom' approach was to

facilitate (develop) a "mix & match" approach to terminal caps... along

with a rather gross 'primary (tap)tune' & 'rocking-in' & 'peaking out'

to 'the nearest RCH' (big items for me) with a variable capacitator:

either NaCl bathed bottles or poly-plate caps w/mech variable capacitor

in oil.  IT IS ABOUT "TWEAK ON THE FLY'!  ( I think..   ~~<;( )  I'm

asking...

The rest of you:

Specific question: Is there apt to be any viability to, or interest in,

such an approach? Who (you ham'ers, out there), can turn us on to a BIG

OLD BRASS tuning capacitor?

General question: What correlation do we find between calc values and

empirical values for our caps? Is it that I'm going to have to pop for a

C-meter?  You guys with 54", where are the real lip-locks on the RCH's?

If, like me, you don't quite get the drift, check out:

< http://www.misslink.net/mhammer/waldo.htm >  to watch the

'Waldo-Murphy Progress Report'; see us strangle in divergent views,

mixed metaphores, odd bits of wire and general newbieness.  Nikola did

it all by himself... ..we can't?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 08:01:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Wed, 15 Apr 1998 04:01:51 -0400 (EDT)

Subject: Re: RF & My new camera?

On Wed, 15 Apr 1998, Tesla-2 wrote:

>  Anyway, if and when we* get this TESLA popping and

> fuming, how close can I safely get w/o blowing the chips out of it?

>

The average mechanical camera handles high-voltage really well,

semiconductors and HV rarely get along.  I dunno, I'd rather buy a

disposable Kodak camera than risk taking a digital camera near the

TC...(What do they go for these days? $300?)  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: terminal

Date: Wed, 15 Apr 1998 08:04:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Wed, 15 Apr 1998 04:04:11 -0400 (EDT)

Subject: Re: terminal

On Wed, 15 Apr 1998, Tesla-2 wrote:

>

> anyone know at what type of store i could purchase a metal ball for the

> terminal on my tesla coil? thanks, Aaron Krumins

>

A toroid works much better - there are a couple of ways someone on the

list can enlighten you on.  How big is your coil?  BTW, if you get your

hands on some ball bearings and decide to use them, remember to anneal the

suckers first, they're hard as rock and'll go through drillbits like beaf

at a weight-watchers convention...  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 12:44:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fxphoto@centuryinter.net (fxphoto)

Date: Wed, 15 Apr 1998 08:44:38 -0400 (EDT)

Subject: Re: RF & My new camera?

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 14 Apr 1998 17:49:06 -0700

>Subject: RF & My new camera?

>Organization: Self

>

>All,

>I just dropped big bucks on a digital camera to shoot pics for my

>friends on the net! Anyway, if and when we* get this TESLA popping and

>fuming, how close can I safely get w/o blowing the chips out of it?

Gene,

I would trust your digital camera taking all sorts of pictures of the parts,

design and everything about the coil until you start it poppin  n' fumin.

Then go out and get a cheap disposable camera for ten or fifteen dollars.

(the kind you send the whole camera back to develop the film.) You will get

a much higher resolution picture from film, This type of camera takes great

pictures,

and bottom line if you happen to forget and leave it setting next to the

coil when you fire it up, you are only out ten or fifteen dollars.

DO NOT try to protect the camera with a Faraday cage surrounding it as this

would put a ground in the photographers hands and I would not even want to

hear about the results!

IMHO    Bill Turbett

P.S. if you do decide to use a throw away camera and you need a time

exposure, let me know . I am sure we can rearrange the shutter mechanism if

need be.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor info

Date: Wed, 15 Apr 1998 12:59:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: yubba <yubba@clara.net>

Date: Wed, 15 Apr 98 12:59:42 GMT

Subject: capacitor info

Organization: self

Hi fellow coilers

My plate cap is coming along nicely. The whole floor is covered with bits

of litho plates and poly dust covers.,and i now know the formula for the plate

area in my sleep. I am going for 20 mil of poly and 6 sets of caps in series

which should give me a good margin for voltage breakdown. I have tapped into a

endless supply of litho plates at scrap price direct from a metal refinery,so if

any UK coilers want to try making a flat plate cap using them let me know off

list. I have tracked down a Uk suppier of ALPOLIC and they have sent me some

samples.The same people also do a mirror material which is thinner, 2 mm which

looks easier to cut. I am not sure about the dielectric though. It is black in

colour and might contain carbon. The other problem is the price, 300 for a 8x4

sheet. thats three hundred pounds!. It might be worth trying if got from a

building site but I agree with Fr tom that it is a lot of work. The address if

anyone is interested is

DERALAM LAMINATES LIMITED

unit 462 464 Walton Summit Centre

Bamber Bridge

PRESTON

LANCS

tel 01772 315888

They are the sole suppliers in the UK and will send samples if asked and will

also deliver for an extra 20

Hope this is usefull to someone.

ps found a good site for pulse cap specs with a program to work out parameters.

it's called pcccad from Evox Rifa AB Kalmar Sweden.

{gleaned from electronics and wireless world may 1998.}

safe coiling to yer

bob lightning without rain

bob@clara.net

a tidy workbench is the sign of a sick mind

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 13:50:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Wed, 15 Apr 1998 13:50:15 GMT

Subject: Re: RF & My new camera?

Organization: White Wing Logic

On Wed, 15 Apr 1998 00:49:06 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 14 Apr 1998 17:49:06 -0700

>Subject: RF & My new camera?

>Organization: Self

>

>All,=20

>I just dropped big bucks on a digital camera to shoot pics for my

>friends on the net! In reality Bert pushed me into it.  Couldn't find

>film for my old, Old OLD Polaroid! Promised him some pics of a rot SG I

>had come across...  Anyway, if and when we* get this TESLA popping and

>fuming, how close can I safely get w/o blowing the chips out of it?  =20

>

>I gather from all the pics on this T-2 list, that there seems little

>danger....   But, y'alls don' know MY luck!! Any precautions?=20

>

>*WE are Dan Murphy and I.  You can see some of our lashup taking shape

>at Mike Hammer's diggs, in a section of his page called:

>   < http://www.misslink.net/mhammer/waldo.htm >

I wouldn't be too worried about damage as long as you stay away from

the range of corona streamers - don't put the camera closer to the

coil than you'd put yourself! You will probably see noise on the LCD

(if it has one) or final pictures before any risk of damage. Always

run on batteries, so there's no trailing wire to act as an antenna.

I've had little success getting good pics with a digital cam (Kodak

DC210), as there's a lot less flexibility with exposure time and

aperture, and also sensitivity isn't as good as fast film (the manual

quotes about 100ASA equivalent).

The impulsive and high-intensity nature of TC discharges can cofuse

the pants off of a digicam's auto exposure system, and you can also

get problems when the CCD saturates, smearing,  and loss of colour.

Film has  a much wider dynamic range, so it is easier to get good

results.

Film also has much bettter resolution, which is good for picking out

the individual streamers .

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: terminal

Date: Wed, 15 Apr 1998 14:22:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Wed, 15 Apr 1998 09:22:58 -0500

Subject: RE: terminal

Hi Aaron,

McMaster-Carr    http://www.mcmaster.com  has hollow plastic floats you

could cover with foil for a top terminal.  They are made from

polypropylene and come in 5, 6 and 8 in. dia.   They also have steel,

stainless steel and copper spherical floats in many sizes.

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Wednesday, April 15, 1998 8:48 AM

To:     Minnich, Roy

Subject:        terminal

From: "krumins" <krumins@bvsd.k12.co.us>

Date: Tue, 14 Apr 1998 21:44:24 -0600

Subject: terminal

anyone know at what type of store i could purchase a metal ball for the

terminal on my tesla coil? thanks, Aaron Krumins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF & My new camera?

Cc:

Bcc:

X-Attachments:

At 12:49 AM 4/15/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 14 Apr 1998 17:49:06 -0700

>Subject: RF & My new camera?

>Organization: Self

>

>All,

>I just dropped big bucks on a digital camera to shoot pics for my

>friends on the net! In reality Bert pushed me into it.  Couldn't find

>film for my old, Old OLD Polaroid! Promised him some pics of a rot SG I

>had come across...  Anyway, if and when we* get this TESLA popping and

>fuming, how close can I safely get w/o blowing the chips out of it?

Gene as I have said many times before modern microelectronics

and Tesla coils do not mix well together. No way would I bring

an expensive digital camera anywhere near an operating coil.

This goes for modern fancy SLRs too. With all the electronics in them

you risk real damage. Get an old completely mechanical SLR.

My trustly old Olympus OM-1 just loves sparks and taking

pictures of stars at 15 degrees below zero. Try that with

a modern wonder camera.

Another good mechanical SLR commonly available used is the Pentax K-1000.

>*WE are Dan Murphy and I.  You can see some of our lashup taking shape

>at Mike Hammer's diggs, in a section of his page called:

>   < http://www.misslink.net/mhammer/waldo.htm >

>

>Dan and I are newbies and will welcome critique and questions.  Laughing

>and pointing are verboten!

Aww. That takes all the fun out of it. :)

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: terminal

Cc:

Bcc:

X-Attachments:

At 03:44 AM 4/15/98 GMT, you wrote:

>From: "krumins" <krumins@bvsd.k12.co.us>

>Date: Tue, 14 Apr 1998 21:44:24 -0600

>Subject: terminal

>

>anyone know at what type of store i could purchase a metal ball for the

>terminal on my tesla coil? thanks, Aaron Krumins

>

Metal spheres arent too common. Ive used styrofoam balls

from the craft store covered with aluminum foil. If you

get the foil on rather smoothly a 6 inch diameter one works well.

Look for an old aluminum world globe at a flea market or garage sale.

Ive heard some people use a metal ball from a flag pole. They are only

about 4 inches in diameter though.

Mcmaster Carr sells some copper and stainless steel spheres but

the prices are out of my league.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 16:08:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 15 Apr 1998 09:08:56 -0700

Subject: Re: RF & My new camera?

Organization: Self

Tesla-2 wrote:

>

> From: Paul Anderson <madhobby@geeky1.ebtech.net>

> Date: Wed, 15 Apr 1998 04:01:51 -0400 (EDT)

> Subject: Re: RF & My new camera?

>

> On Wed, 15 Apr 1998, Tesla-2 wrote:

>

> >  Anyway, if and when we* get this TESLA popping and

> > fuming, how close can I safely get w/o blowing the chips out of it?

> >

> The average mechanical camera handles high-voltage really well,

> semiconductors and HV rarely get along. ..(What do they go for these days? $300?)

> TTYL!

>

> Paul Anderson

> madhobby@geeky1.ebtech.net

Paul, et al,

So... are the neat discharge pics we're seeing scanned-analog shots or

are we able to go direct digital .jpg in any cases? Horror stories,

anyone?  Faraday cages for cameras?  Where is the digital back for my

Linhoff?

Paul, $300 scoots is starters- SVGA cuts in at about $900+

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re[2]: Binary Xmissions??

Date: Wed, 15 Apr 1998 16:42:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 15 Apr 1998 12:42:58 -0400

Subject: Re[2]: Binary Xmissions??

--IMA.Boundary.7898562980

Content-Type: text/plain; charset=US-ASCII

Content-Transfer-Encoding: 7bit

Content-Description: cc:Mail note part

>I have majordomo on my system, and UUCP access.  I'll put the list on my

>box for free, software handles attachments, digests, archiving, etc. with

>no trouble whatsoever.  You could admin remotely and moderation is a peice

>of cake.  Let me know if you're interested.  TTYL!

A digest would be neat. I'm set up as digest for the rest of my lists

too.

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 17:48:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Wed, 15 Apr 1998 10:48:00 -0700 (PDT)

Subject: Re: RF & My new camera?

At 02:55 PM 4/15/98 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Wed, 15 Apr 1998 09:55:27 -0500

>Subject: Re: RF & My new camera?

>

>Gene as I have said many times before modern microelectronics

>and Tesla coils do not mix well together. No way would I bring

>an expensive digital camera anywhere near an operating coil.

>

>This goes for modern fancy SLRs too. With all the electronics in them

>you risk real damage. Get an old completely mechanical SLR.

>My trusty old Olympus OM-1 just loves sparks...

Right on, Mike - And fortunately, I too am a big fan of the OM-1, so

when the time comes, mine'll be there on the tripod, to record the big

moment... I'll dig out my little 1/2 page "ScanMan" to digitize the images.

MadManDan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RF & My new camera

Date: Wed, 15 Apr 1998 21:03:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 15 Apr 1998 14:03:59 -0700

Subject: RF & My new camera

Organization: Self

Mike & Dan,

No way was I about to expose those electronics to that environment, in

any case...  My Nikormat FT will stand up, I'm sure...

Now we learn that Dan is hiding a scanner! How old is it? Color? If not,

what would happen with the Polaroid 4x5 N/P film (type 55, was it)? Just

scan the neg and computer-prosess to neg (ie Pos) gif or jpg it ???

Even at 640x480, this digital is no line copy system, as you both saw!

What's the res of the scanner 64x48?  If you can scan color, we are in!

just get some 4x5 polacolor film; got the camera!

Gene

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Modulation

Date: Wed, 15 Apr 1998 21:18:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: John Ervin Jr <jwejr@aye.net>

Date: Wed, 15 Apr 1998 17:18:02 -0400

Subject: Modulation

I have been watching the posting to the Tesla group for about 2 months now.

I saw one of the messages make a reference to 60Hz being modulated or

superimposed on the final frequency of the Tesla coils, and that you would

not want to get hit, or be very near any of the arcs produced.

My question is this:  If the line voltage is rectified to a straight 180v

DC then modulated to another frequency on the primary side of a neon xfmr,

say 1kHz, would this work for a Tesla coil setup?  Has anyone in this group

ever tried this?

John Ervin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Wed, 15 Apr 1998 22:15:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Paul Anderson <madhobby@geeky1.ebtech.net>

Date: Wed, 15 Apr 1998 18:15:16 -0400 (EDT)

Subject: Re: RF & My new camera?

On Wed, 15 Apr 1998, Tesla-2 wrote:

>

> Paul, et al,

> So... are the neat discharge pics we're seeing scanned-analog shots or

> are we able to go direct digital .jpg in any cases?

>

Most of them are scanned in - or framegrabbed from a video.  I'd use a

disposable kodak camera, then take the negatives to a kodak dealer and get

it put on PhotoCD, which you can mess with using Photoshop on your

computer.  It's a _LOT_ safer.

TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Thu, 16 Apr 1998 00:47:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Wed, 15 Apr 1998 17:47:23 -0700

Subject: Re: RF & My new camera?

Organization: Self

Tesla-2 wrote:

>

> From: Paul Anderson <madhobby@geeky1.ebtech.net>

> Date: Wed, 15 Apr 1998 18:15:16 -0400 (EDT)

> Subject: Re: RF & My new camera?

>

> On Wed, 15 Apr 1998, Tesla-2 wrote:

> > Paul, et al,

> > So... are the neat discharge pics we're seeing scanned-analog shots

> > are we able to go direct digital .jpg in any cases?

> Most of them are scanned in - or framegrabbed from a video.  I'd use a

> disposable kodak camera, then take the negatives to a kodak dealer and

> it put on PhotoCD, which you can mess with using Photoshop on your

> computer.  It's a _LOT_ safer.

> TTYL!

> Paul Anderson

> madhobby@geeky1.ebtech.net

Paul,

And so what, prithe, is so different about an $800 vidicam vs an $800

digicam? I DO NOT dispute anything said, but remain confused, and

certain that I'll keep my vid stuff as far away as the other! My klunky

old Linhoff should do fine w/4x5 polacolor packs? N' Dan dusts off his

scanner....

Gene Waldo

PS Do you guys with the fancy terminal sign-offs hand-do those each

time, or are there like macros...? whatever they are?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Measuring capacitance

Date: Thu, 16 Apr 1998 00:57:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ESchulz531 <ESchulz531@aol.com>

Date: Wed, 15 Apr 1998 20:57:14 EDT

Subject: Measuring capacitance

Daniel and all,

You can figure out the capacitance of a non polarized capacitor with a

volt meter and an ampere meter.  By connecting it to an alternating voltage

source, such as a wall socket.

C = i / (2 * Pi * f * V)

i is current in amps

f is frequency in Hz,

V is volts

C is capacitance in farads, multiply by a million for micro farads.

With a 0.01 uf capacitor, 120 volts, and 60 Hz you should get about 0.45 ma

Erik

ESchulz531@aol.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: spark pictures (was RF and my new camera)

Cc:

Bcc:

X-Attachments:

At 05:48 PM 4/15/98 GMT, you wrote:

>From: Dan Murphy <danmurphy@1stnetusa.com>

>Date: Wed, 15 Apr 1998 10:48:00 -0700 (PDT)

>Subject: Re: RF & My new camera?

>

>At 02:55 PM 4/15/98 GMT, you wrote:

>>From: Mike Hammer <mhammer@misslink.net>

>>Date: Wed, 15 Apr 1998 09:55:27 -0500

>>Subject: Re: RF & My new camera?

>>

>>Gene as I have said many times before modern microelectronics

>>and Tesla coils do not mix well together. No way would I bring

>>an expensive digital camera anywhere near an operating coil.

>>

>>This goes for modern fancy SLRs too. With all the electronics in them

>>you risk real damage. Get an old completely mechanical SLR.

>>My trusty old Olympus OM-1 just loves sparks...

>

>Right on, Mike - And fortunately, I too am a big fan of the OM-1, so

>when the time comes, mine'll be there on the tripod, to record the big

>moment... I'll dig out my little 1/2 page "ScanMan" to digitize the images.

>

>                                                MadManDan

Great to meet a fellow OM-1 user. Surely it must be the greatest camera

for the money ever produced. And the Olympus Zuiko lenses are really

terrific and available quite cheaply used. Ive had mine well

over 20 years and it still works great despite being used in sub zero

temperatures and getting hit with an errant 40 inch + streamer.

For Dan and Gene and others wanting to take pictures of

your operating coil here are a few hints.

Use a good quality fast color or BW film. Since very few people

do BW these days I wont go into details about it. If someone

wants to converse about BW film off list feel free.

I used Kodak PPF which is the professional version of

the consumer Kodak Gold 400. Either is a good film

since it maintains a proper color balance at low light levels.

The biggest mistake beginners make is to shoot in total or

nearly total darkness with too long an exposure.. The streamers

are then very bright possibly even over exposed against a black

background. You see very little of the coil itself and without

a background cant tell how long the streamers are.

I kill the room lights but light up the wall behind the coil

with a dim light in a reflector. Set the camera up on a tripod

(a tripod is a must) and if it has a light meter set exposure

and F stop to properly record the dimly lit wall. Focus on

the coil but meter for the background. This will record the

background well and the streamers will take of themselves.

The streamers will show bright against the dimmer background.

The best exposures are from 1/4 to 1/2 second. If its shorter

than this you may have a hard time recording a long streamer.

If its much longer than this the individual streamers get blotted

out.

My best results have been 1/2 second exposures at F 4 with

ASA 400 film processed normally.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor info

Date: Thu, 16 Apr 1998 02:39:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: yubba <yubba@clara.net>

Date: Thu, 16 Apr 98 02:39:43 GMT

Subject: capacitor info

Organization: self

Hi fellow coilers

My plate cap is coming along nicely. The whole floor is covered with bits

of litho plates and poly dust covers.,and i now know the formula for the plate

area in my sleep. I am going for 20 mil of poly and 6 sets of caps in series

which should give me a good margin for voltage breakdown. I have tapped into a

endless supply of litho plates at scrap price direct from a metal refinery,so if

any UK coilers want to try making a flat plate cap using them let me know off

list. I have tracked down a Uk suppier of ALPOLIC and they have sent me some

samples.The same people also do a mirror material which is thinner, 2 mm which

looks easier to cut. I am not sure about the dielectric though. It is black in

colour and might contain carbon. The other problem is the price, 300 for a 8x4

sheet. thats three hundred pounds!. It might be worth trying if got from a

building site but I agree with Fr tom that it is a lot of work. The address if

anyone is interested is

DERALAM LAMINATES LIMITED

unit 462 464 Walton Summit Centre

Bamber Bridge

PRESTON

LANCS

tel 01772 315888

They are the sole suppliers in the UK and will send samples if asked and will

also deliver for an extra 20

Hope this is usefull to someone.

ps found a good site for pulse cap specs with a program to work out parameters.

it's called pcccad from Evox Rifa AB Kalmar Sweden.

{gleaned from electronics and wireless world may 1998.}

safe coiling to yer

bob lightning without rain

bob@clara.net

a tidy workbench is the sign of a sick mind

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Dan's RF chokes-was vacuum gaps

Date: Thu, 16 Apr 1998 05:34:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Thu, 16 Apr 1998 15:34:07 +1000

Subject: Re: Dan's RF chokes-was vacuum gaps

>> The best way is to use a winding shuttle.

>>

>If you're really set against winding them manually - design and build a

>toroid winding machine.  Should be easy enough to make IT pass the shuttle

>around.  TTYL!

I was going to make toroid chokes but the thought of winding a toroid put me

off straight away.  Do you have any more information on homemade toroid

winding machines/devices.

From Scott Hayward    smash@eisa.net.au

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: vacuum cleaner blowing

Date: Thu, 16 Apr 1998 05:43:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Thu, 16 Apr 1998 15:43:39 +1000

Subject: vacuum cleaner blowing

To the list,

Can you use a vacuum cleaner in reverse to provide the air for a cylinder

gap.  I have wanted to build one of these but I can't get my hands on

anything that has a high air output.

From Scott Hayward    smash@eisa.net.au

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: spark pictures (was RF an

Date: Thu, 16 Apr 1998 05:54:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>The biggest mistake beginners make is to shoot in total or

T>nearly total darkness with too long an exposure.. The streamers

T>are then very bright possibly even over exposed against a black

T>background. You see very little of the coil itself and without

T>a background cant tell how long the streamers are.

<snip>

If you have seen the picture on my home page of the ITS coil with me

standing beside it, that one was taken with ASA 400 film, exposure of 5

seconds @ f8.0 with the camera on a tripod in total darkness. I reset

the shutter and tripped the 20 watt-second flash from 20 feet away to

illuminate the room and coil.

Ralph

---

Dreams move on if you wait too long.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF & My new camera?

Date: Thu, 16 Apr 1998 09:29:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Thu, 16 Apr 1998 05:29:55 EDT

Subject: Re: RF & My new camera?

Gene, Dan,

I won't recommend taking your expensive new camera and experimenting with

it, but a friend of mine came over and took a roll a film of my coil with one

of those fancy SLR cameras.  I was worried about his camera to say the least,

but he seemed eager to try it in spite of my mentioning my concern.  The

camera survived without any trouble.  The pictures, however, did not come out

as good as ones taken with an Olympus OM-1.

Mike Hollingsworth.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Modulation

Date: Thu, 16 Apr 1998 13:55:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Scott MacGregor <smacgregor@aasp.net>

Date: Thu, 16 Apr 1998 06:55:39 -0700

Subject: Re: Modulation

Organization: EES,Co.

Tesla-2 wrote:

>

> From: John Ervin Jr <jwejr@aye.net>

> Date: Wed, 15 Apr 1998 17:18:02 -0400

> Subject: Modulation

>

> I have been watching the posting to the Tesla group for about 2 months now.

>  I saw one of the messages make a reference to 60Hz being modulated or

> superimposed on the final frequency of the Tesla coils, and that you would

> not want to get hit, or be very near any of the arcs produced.

>

> My question is this:  If the line voltage is rectified to a straight 180v

> DC then modulated to another frequency on the primary side of a neon xfmr,

> say 1kHz, would this work for a Tesla coil setup?  Has anyone in this group

> ever tried this?

>

> John Ervin

You would need a completely redesigned x-former to operate at 1kHz. Core

material, etc. However, if you rectified the OUTPUT, then mod to 1kHz,

who knows. All sorts of recalculations for the TC winding specs, etc.

I'm not sure what the performance would be like.

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Modulation

Date: Thu, 16 Apr 1998 17:37:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Thu, 16 Apr 1998 12:37:23 -0500

Subject: RE: Modulation

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Thursday, April 16, 1998 12:21 PM

To:     Minnich, Roy

Subject:        Modulation

From: John Ervin Jr <jwejr@aye.net>

Date: Wed, 15 Apr 1998 17:18:02 -0400

Subject: Modulation

I have been watching the posting to the Tesla group for about 2 months now.

I saw one of the messages make a reference to 60Hz being modualted or

superimposed on the final frequency of the Tesla coils, and that you would

not want to get hit, or be very near any of the arcs produced.

My question is this:  If the line voltage is rectified to a straight 180v

DC then modulated to another frequency on the primary side of a neon xfmr,

say 1kHz, would this work for a Tesla coil setup?  Has anyone in this group

ever tried this?

John Ervin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: list digest (was Binary Xmissions)

Cc:

Bcc:

X-Attachments:

At 04:42 PM 4/15/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Wed, 15 Apr 1998 12:42:58 -0400

>Subject: Re[2]: Binary Xmissions??

>

>

>  A digest would be neat. I'm set up as digest for the rest

>  of my lists too.

>

Tom I agree a digest would be nice as I take the digest form

of several other lists I'm on. But thats not going to be an

option with this list for a while. Some time in the future maybe.

A digest of sorts is available in the list archives.

These archives are not the raw output of the list.

They are highly edited to remove excessive quotes,

extraneuous message headers and off topic posts.

I try to tidy up the formating to make it easier

to read and even correct spelling errors that I find.

I even remove the extra bulk that cc:mail attaches. :)

The archives will come in bimonthly installments

and I always post to the list when the next archive

is prepared. Im almost finished with the Jan/Feb archive.

It should have been done long ago but time has been at a

premium lately.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: 12 Kv/ 30 ma coil

Date: Thu, 16 Apr 1998 22:44:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE D <gordrew@home.com>

Date: Thu, 16 Apr 1998 15:44:03 -0700

Subject: 12 Kv/ 30 ma coil

I have noticed there are many ways to "build" a 12Kv/30 ma coil. I was

cureous as to what some of your success/failors with a coil on a 12kv/30ma

base.

thanks,

MIKE D

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: 12 Kv/ 30 ma coil

Date: Thu, 16 Apr 1998 23:54:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Michael Baumann" <baumann@proton.llumc.edu>

Date: Thu, 16 Apr 1998 16:54:57 -0700

Subject: RE: 12 Kv/ 30 ma coil

Well, I have had pretty good luck with that value. I finally got some pics,

and when I finally get my page set up at geocities, I'll post them there.

Specs:

12KV 30ma neon as power

3.5"x10" winding of 24awg wire (about 500 turns) for secondary

Gap is multi-segment cylinder

12"x3" toroid

Cap is 0.0072uf poly plate.

Primary is 4.5" ID, 30deg saucer. 1/4" tubing, tapped at around 8 turns.

Best performance to date: 14"

consistantly hits 12"

> -----Original Message-----

> From: Tesla-2 [mailto:tesla-2@emachine.com]

> Sent: Thursday, April 16, 1998 3:44 PM

> To: baumann@proton.llumc.edu

> Subject: 12 Kv/ 30 ma coil

>

>

> From: MIKE D <gordrew@home.com>

> Date: Thu, 16 Apr 1998 15:44:03 -0700

> Subject: 12 Kv/ 30 ma coil

>

>

>         I have noticed there are many ways to "build" a 12Kv/30

> ma coil. I was curious as to what some of your success/failors with

> a coil on a 12kv/30ma

> base.

>

> thanks,

> MIKE D

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Hv cable

Date: Fri, 17 Apr 1998 12:31:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 17 Apr 1998 07:31:59 -0500

Subject: Hv cable

Organization: MailExcite  (http://www.mailexcite.com:80)

Last night I was using X-ray cable for my coil setup. I power my coil with a 14.4

Kv pole pig. I hope to use the cable to power my coil outside the house. X-ray cable

is shielded with a grounded brade. This keeps down the EMF and acts as a safety in

the event the cable is punctured. It also has capacitance ! 50' of cable seems to

have a lot of capacitance. Last night during a dry run (cables not connected to load

or tank circuit) I heard a loud pop at the other end of the cable. The shielding

was stripped back about 2" from the conductor and yet due to resonates is was arcing

around the cable to the shielding (a full 2"). I guess the moral of this story is

if you use X-ray cable be careful. Also I have noticed a residual charge sometimes

remains in the cable. This can surly bite you.

Paul M.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Some pics of my coil

Date: Fri, 17 Apr 1998 15:24:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Michael Baumann" <baumann@proton.llumc.edu>

Date: Fri, 17 Apr 1998 08:24:42 -0700

Subject: Some pics of my coil

I finally got around to snapping some shots of my "baby" coil. This is the one

I use for demos at schools, and to convince neon shop owners to give up those

dead transformers :)

http://www.geocities.com/CapeCanaveral/Launchpad/3574/

Michael Baumann

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Hv cable

Cc:

Bcc:

X-Attachments:

At 12:31 PM 4/17/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 17 Apr 1998 07:31:59 -0500

>Subject: Hv cable

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>Last night I was using X-ray cable for my coil setup. I power my coil with a 14.4

>Kv pole pig. I hope to use the cable to power my coil outside the house.

>X-ray cable is shielded with a grounded braid. This keeps down the EMF and acts

>as a safety in the event the cable is punctured. It also has capacitance!

>50' of cable seems to have a lot of capacitance. Last night during a dry run

>(cables not connected to load or tank circuit) I heard a loud pop at the other

>end of the cable. The shielding was stripped back about 2" from the conductor and

>yet due to resonates is was arcing around the cable to the shielding (a full 2").

> I guess the moral of this story is if you use X-ray cable be careful.

> Also I have noticed a residual charge sometimes remains in the cable.

> This can surely bite you.

>

>Paul M.

Paul,

This is not really suprising. Most cable of this type has

a considerable amount of capacitance. After all what is a capacitor?

2 conductors separated by an insulator. Even zip cord has capacitance.

The capacitance of RG213 type coaxial cable is 30.8 pf per foot.

Ive made parallel resonant circuits to be used as antenna traps

using lengths of coaxial cable as a HV capacitor.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Some pics of my coil

Cc:

Bcc:

X-Attachments:

At 03:24 PM 4/17/98 GMT, you wrote:

>From: "Michael Baumann" <baumann@proton.llumc.edu>

>Date: Fri, 17 Apr 1998 08:24:42 -0700

>Subject: Some pics of my coil

>

>I finally got around to snapping some shots of my "baby" coil.

>This is the one I use for demos at schools, and to convince neon

>shop owners to give up those dead transformers :)

Thats a good idea. Ive done the same thing. Its amazing

how cooperative some neon shops can get once they see

what you are up to.

>

>http://www.geocities.com/CapeCanaveral/Launchpad/3574/

Nice looking coil Michael and it seems to be working well.

16 inches on a 3 incher from a single 12kv/30 is not bad at

all. The best I did with a single 12kv/30 is 22 inches on a

4 inch diameter secondary.

I like those caps you built. They look great and I bet they perform

just as great.

I think you can afford a little more gap. You are using only

6 gaps at .03 for a total of .180 inch. Thats a little conservative

for 12kv. Ive run .180 of gap at 9kv with good results. Try adding

another electrode to your gap to bring it up to .210 inch. That

should increase your primary energy level a bit.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: henry

Date: Fri, 17 Apr 1998 19:20:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Reinier Heeres <rwh@worldonline.nl>

Date: Fri, 17 Apr 1998 21:20:43 +0200

Subject: henry

Hi!

Could anybody explain the unit "henry" (what it means and what it is used for).

It's in math.txt (with Tesla equations), but I do not understand what it

means...

Reinier

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: henry

Date: Fri, 17 Apr 1998 18:46:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tim Osmulski <goatburn@connect.net>

Date: Fri, 17 Apr 1998 18:46:40 +0000

Subject: Re: henry

> Reinier Heeres <rwh@worldonline.nl>   wrote:

> Could anybody explain the unit "henry" (what it means and what it is

> used for).

>

A "henry" is a measure of inductance.  It is used to describe the "capacity"

of an inductor in terms of energy storage.  For example, we often speak of

30 micro-Farad capacitors and 1.5 milli-Farad capacitors.  This is how we

"measure" them.  In the same way, we can speak of 14 micro-Henry inductors

and 7.17 milli-Henry inductors.

The more loops of wire an inductor has, the more inductance it has.  The

shape also matters; flat wide inductors generally have more inductance than

tall skinny ones.  (Right?)

Tim Osmulski

goatburn@connect.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Pri coil config..?

Date: Fri, 17 Apr 1998 19:50:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 17 Apr 1998 12:50:17 -0700

Subject: Pri coil config..?

Organization: Self

All,

The business of Primary tuning and strike rails leads me offer some

thoughts and questions.

Given that (I'm assuming) plex is xparent to the field effects from the

pri, whynot invert the (flat pancake)pri (wound on plex sheet) so that

the coil is under the plex base and mounted 6-8" above any lower

structures? this should(?)may:  1)lengthen the air gap path and thus

more adequately isolate the pri from term cap; and 2)allow for more

klutzy pri tapping connection* mechanics, shielded and beneath the 2ndry

base; 3) perhaps lower the need for the rail?

* what is preferred? just an alligator/battery clip? Or something more

refined?

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: henry

Date: Fri, 17 Apr 1998 21:34:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

Date: Fri, 17 Apr 1998 15:34:37 -0600 (MDT)

Subject: Re: henry

On Fri, 17 Apr 1998, Tesla-2 wrote:

> From: Reinier Heeres <rwh@worldonline.nl>

> Date: Fri, 17 Apr 1998 21:20:43 +0200

> Subject: henry

>

> Hi!

>

> Could anybody explain the unit "henry" (what it means and what it is used for).

> It's in math.txt (with Tesla equations), but I do not understand what it

> means...

A Henry is a unit of inductance.  It also is the name of an emminently

incompetent proprietor of a hobby shop in Worcester, MA.  Has a tendency

towards flatulence and sniffing model glue.

>

> Reinier

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: henry

Date: Fri, 17 Apr 1998 22:24:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 17 Apr 1998 15:24:50 -0700

Subject: Re: henry

Organization: Self

Tesla-2 wrote:

>

> From: Reinier Heeres <rwh@worldonline.nl>

> Date: Fri, 17 Apr 1998 21:20:43 +0200

> Subject: henry

>

> Hi!

>

> Could anybody explain the unit "henry" (what it means and what it is

> used for).

>

Reinier,

'Henry' is the unit of inductance; Mathematical symbol: L.

It is used to determine from coil design, the effects of interacting

coils. No. of turns, dia. & lenght "aspect ratio" and such. This leds to

'inductive reactance', which is measured in Ohms, usually. Usually only

has applications in HF systems.

Above several MHz the inductance of relatively short lengts of wire

becomes important.

Example: A straight wire 4" long & .025" dia has an inductance of 0.2

uH, and has an inductive reactance of ~100 ohms, at 80 MHz.

Look up Resonant Circuits; R-L-C coupling; where we find Time Constants

(L/R) or (RxC), "rise and decay times" as a new factor, to describe a

circuit's full Voltage or Current.

If that doesn't help much, don't feel bad...

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: henry

Cc:

Bcc:

X-Attachments:

At 07:20 PM 4/17/98 GMT, you wrote:

>From: Reinier Heeres <rwh@worldonline.nl>

>Date: Fri, 17 Apr 1998 21:20:43 +0200

>Subject: henry

>

>Hi!

>

>Could anybody explain the unit "henry" (what it means and

>what it is used for).

>It's in math.txt (with Tesla equations), but I do not understand what it

>means...

>

>Reinier

The Henry is the basic unit of inductance. Its named after the

American physicist Joseph Henry.

Inductors oppose a change in current flow. The more rapid

the change the more opposition the inductor shows.

Inductors are most often used in Tesla coils in conjunction

with capacitors to form resonant circuits.

I would recommend getting and reading a book on basic electronics.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: henry

Date: Fri, 17 Apr 1998 23:50:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ElctrcEngr <ElctrcEngr@aol.com>

Date: Fri, 17 Apr 1998 19:50:11 EDT

Subject: Re: henry

###################

** You Wrote **

Hi!

Could anybody explain the unit "henry" (what it means and what it is used for).

It's in math.txt (with Tesla equations), but I do not understand what it

means...

Reinier

######################

A henry is a unit of measure like a farad for capacitors, or ohm for

resistors.  A henry applies to inductors (aka coils).  One henry =

1T*m*m/A....  This is one tesla meter squared per ampere, but none of this is

really all that important.  Just remember that an inductor is a bunch of coils

of wire.  The more times the wire is wrapped, the higher the inductance, also

whatever is in the middle of the coils affects the inductance (measured in

henrys).

The unit 'henry' is used in transformers, as in tesla coils, because both

sides of the transformer are inductors.  There is quite a lot to say about

transformers that I don't want to bore you about, but there is plenty of

information and formulas on the internet about them.  But that is essentially

what a henry is and why it is used in tesla coils.

Brent

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum cleaner blowing

Cc:

Bcc:

X-Attachments:

At 05:43 AM 4/16/98 GMT, you wrote:

>From: "Scott Hayward" <smash@eisa.net.au>

>Date: Thu, 16 Apr 1998 15:43:39 +1000

>Subject: vacuum cleaner blowing

>

>

>To the list,

>Can you use a vacuum cleaner in reverse to provide the air for a cylinder

>gap.  I have wanted to build one of these but I can't get my hands on

>anything that has a high air output.

>

>From Scott Hayward    smash@eisa.net.au

>

Scott,

Yes you can. In fact using a vacuum cleaner motor to suck

air through the gap results in better performance than blowing

air through it.

Be aware that a heavy duty cannister type vacuum cleaner motor

may actually provide too much airflow. Best bet is to use

a variac to control the motor speed. Another method would

be to provide some type of variable venting to reduce airflow.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Dan's RF chokes-was vacuum gaps

Date: Sat, 18 Apr 1998 04:40:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 17 Apr 1998 21:40:28 -0700

Subject: Re: Dan's RF chokes-was vacuum gaps

Organization: Self

Tesla-2 wrote:

>

> From: "Scott Hayward" <smash@eisa.net.au>

> Date: Thu, 16 Apr 1998 15:34:07 +1000

> Subject: Re: Dan's RF chokes-was vacuum gaps

>

> >> The best way is to use a winding shuttle. I was going to make toroid

> >chokes but the thought of winding a toroid put me off straight away.

> >From Scott Hayward    smash@eisa.net.au

Scott,

I'm not sure of the specific (labor vs efficiency) advantages of toroids

over cyl chokes---

You can see ours (24 ga, on 2" PVC (Dan wound 'em, so yammer at him)) at

http://www.misslink.net/mhammer/waldo.htm

Gene waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Sat, 18 Apr 1998 04:55:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Fri, 17 Apr 1998 21:55:07 -0700

Subject: Re: vacuum cleaner blowing

Organization: Self

Tesla-2 wrote:

>

> From: "Scott Hayward" <smash@eisa.net.au>

> Date: Thu, 16 Apr 1998 15:43:39 +1000

> Subject: vacuum cleaner blowing

>

> To the list,

> Can you use a vacuum cleaner in reverse to provide the air for a

> cylinder gap.

>

> >From Scott Hayward    smash@eisa.net.au

Scott,

My sense is that the ideal ser gap tubes mtd in a large cyl would be

best, if slots were cut parallel to gaps in the cyl; cap one end and

apply vacuum to the other end; inducted air would impinge directly at

each gap.

I'm guessing, having not built this type yet. Discussion?

You also loose the visual excitement of an operating series gap! So go

for plex, over PVC! Should make a great night light!

Gene Waldo

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Dan's RF chokes-was vacuum gaps

Cc:

Bcc:

X-Attachments:

At 04:40 AM 4/18/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 17 Apr 1998 21:40:28 -0700

>Subject: Re: Dan's RF chokes-was vacuum gaps

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: "Scott Hayward" <smash@eisa.net.au>

>> Date: Thu, 16 Apr 1998 15:34:07 +1000

>> Subject: Re: Dan's RF chokes-was vacuum gaps

>>

>> >> The best way is to use a winding shuttle. I was going to make toroid

> >chokes but the thought of winding a toroid put me off straight away.

>

>> >From Scott Hayward    smash@eisa.net.au

>

>Scott,

>I'm not sure of the specific (labor vs efficiency) advantages of toroids

>over cyl chokes---

>You can see ours (24 ga, on 2" PVC (Dan wound 'em, so yammer at him)) at

>http://www.misslink.net/mhammer/waldo.htm

>

>Gene waldo

>

Gene and Scott,

Toroidal cores are not difficult to wind. Just a little

time consuming. Ive wound many of them over the years.

From tiny beads to giant 5 inch diameter ones.

They offer so many advantages in many circuits that you should

consider them. The greatest advantage is their self shielding

property. The magnetic flux is contained entirely within the core.

This isnt a great concern in Tesla coil use but is of tremendous

value in radio circuits.

They also give a much higher inductance than air core chokes

usually with fewer turns. For this reason they often do not

require the damping resistors needed with air core chokes

with many turns of wire.

Air core chokes work fine with Tesla coils but should be wound

with smaller guage wire (AWG 26-28) to give greater inductance

and lower Q and really should be used with series damping resistors

to further lower the Q.

Remember for Tesla coil RF chokes we DO NOT want an efficient

low loss high Q choke. We want it lossy and low Q.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Pri coil config..?

Date: Sat, 18 Apr 1998 15:41:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Rick L. Kirby" <rkirby@earthlink.net>

Date: Sat, 18 Apr 1998 08:41:20 -0700

Subject: Re: Pri coil config..?

Tesla-2 wrote:

>

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Fri, 17 Apr 1998 12:50:17 -0700

> Subject: Pri coil config..?

> Organization: Self

>

> All,

> The business of Primary tuning and strike rails leads me offer some

> thoughts and questions.

>

> Given that (I'm assuming) plex is xparent to the field effects from the

> pri, whynot invert the (flat pancake)pri (wound on plex sheet) so that

> the coil is under the plex base and mounted 6-8" above any lower

> structures? this should(?)may:  1)lengthen the air gap path and thus

> more adequately isolate the pri from term cap; and 2)allow for more

> klutzy pri tapping connection* mechanics, shielded and beneath the 2ndry

> base; 3) perhaps lower the need for the rail?

>

> * what is preferred? just an alligator/battery clip? Or something more

> refined?

>

>  Gene Waldo

>

Gene

That is the configuration I used on my last coil. My whole pri circuit

is in a lexan box the pri coil is mounted on the under side

of the lid and makes tapping any point on the pri very easy. I use a

wide face 1/2" brass battery style clip that I formed to fit my 1/4"

copper tubing. I then mounted my strike rail on the top side.

TTYL :^)

Rick L. Kirby

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: "control box"

Date: Sat, 18 Apr 1998 18:13:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE D <gordrew@home.com>

Date: Sat, 18 Apr 1998 11:13:33 -0700

Subject: "control box"

I remember reading briefly about the control box used to turn on and off

the tesla coil. Are there any sources for plans for such a box?

--Mike Drew

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: "control box"

Cc:

Bcc:

X-Attachments:

At 06:13 PM 4/18/98 GMT, you wrote:

>From: MIKE D <gordrew@home.com>

>Date: Sat, 18 Apr 1998 11:13:33 -0700

>Subject: "control box"

>

>

>I remember reading briefly about the control box used to turn on and offf

>the tesla coil. Are there any sources for plans for such a box?

>

>--Mike Drew

>

Mike,

None that im aware of. Like Tesla coils themselves there as many

designs as there are coilers. Basically what I consider to be

important in a control box is your variac and proper fusing.

This is also a good place for your AC line filter to keep RF

hash off the AC mains. Place the filter between the variac and

your transformer.

Provide a heavy duty on/off switch. Remember to use a switch

with a rating at least that of your variac. Since most variacs

will easily handle more than their rating some extra margin in the

switch would be a good idea.

I like to use a keyswitch. By keeping the key with you at all

times you can be sure no one will inadvertantly turn on the power.

Provide a panel lamp so you KNOW when the power is on.

If you want to really go first class some metering of the AC output

would be great. Voltage and current panel meters are available

for a modest sum.

A small AC muffin fan to circulate air would be a good idea

if you are going to exceed the rating on your variac at all.

Hope this helps some.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 12 Kv/ 30 ma coil

Cc:

Bcc:

X-Attachments:

At 10:44 PM 4/16/98 GMT, you wrote:

>From: MIKE D <gordrew@home.com>

>Date: Thu, 16 Apr 1998 15:44:03 -0700

>Subject: 12 Kv/ 30 ma coil

>

>

>  I have noticed there are many ways to "build" a 12Kv/30 ma coil.

>I was curious as to what some of your success/failures are with a

>coil on a 12kv/30ma base.

>

>thanks,

>MIKE D

>

>

Mike,

A 12kv/30ma neon is a good choice for a small coil and

can be the basis for a larger coil.

To get the most out of your 12kv/30ma neon your main

cap size should be around .0066 mfd. However most neons

will put out somewhat more than their name plate suggests.

I had good luck using a .0077 plate cap with a single 12kv/30ma.

My best results at that power level were 22 inches on my

4.5 X 22 secondary and 6 inch diameter spherical terminal.

Im sure I could surpass that record now.

My suggestion is to build a good poly plate cap to match the neon

and wind a 4.5 X 20 inch secondary with AWG 24. Wind a primary

using 1/4 inch copper tubing with at least 12 turns. Make this

a shallow saucer shape with about a 20 degree slope.

Build a good RQ type cylinder gap with 9 electrodes of 3/4 inch

copper pipe cut 3 inches long. Space each electrode .025 inches

for a total of .200 inches of gap. Get a strong muffin fan for airflow.

Start with a spherical terminal about 5 or 6 inches in diameter

then go to a 15 X 3 or 16 X 4 dryer duct toroid.

Dont forget the RF chokes and safety gap.

Im sure a coil of this type should be capable of 24 inch streamers

with your single 12kv/30.

Let us know how you are progressing.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Sun, 19 Apr 1998 00:48:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 19 Apr 1998 10:48:41 +1000

Subject: Re: vacuum cleaner blowing

>> To the list,

>> Can you use a vacuum cleaner in reverse to provide the air for a

>> cylinder gap.

>>

>> >From Scott Hayward    smash@eisa.net.au

>

>Scott,

>My sense is that the ideal ser gap tubes mtd in a large cyl would be

>best, if slots were cut paralell to gaps in the cyl; cap one end and

>apply vacuum to the other end; inducted air would impinge directly at

>each gap.

>I'm guessing, having not built this type yet. Discussion?

>

>You also loose the visual excitement of an operating series gap! So go

>for plex, over PVC! Should make a great night light!

>

>Gene Waldo

I think I can try to imagine what you are talking about and it sounds like a

good idea.  A bit complicated maybe.  I agree that plexiglass is better for

the visual effect.  I have one gap on my crappy TC and even it looks kind of

impressive.

From Scott Hayward   smash@eisa.net.au

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Good Toroid among other things

Date: Sun, 19 Apr 1998 01:07:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Sun, 19 Apr 1998 11:07:38 +1000

Subject: Good Toroid among other things

To anyone on the list,

First of all thanks to the people who answered my last questions.

I live in Australia where most things are in metric so I have some

measurement questions.  I can only find one place close by that sells

enammelled copper wire in reletively small quantities.  They sell .6mm

enammeled copper wire.  Is this suitable for a 4" x 26" secondary coil.  I

can also get .63mm wire.(These measurements are the diameter of the copper

only).  The insulation is a double layer of polyester.

The next question is, how do you get such a neat looking toroid.  I got a

hold of some of that plastic pipe that you can bend and I joined it with

tape, but that wouldn't hold.  Could you use a rubber car tyre tube.  It is

a perfect toroid shape.  But then how do you get the aluminium to go on

nicely.

How do you test if a neon is putting out 12000V or 6000V if the transformer

is rated at 12000V.  I have a digital multimeter and a CRO.  That's about it.

Does anyone know where you can get copper strap to join the ground terminals

together with and last but not least, does wood conduct at high voltages or

not because I could make a fairly good support stand for my primary out of

wood but some people have warned me off it.

Thank you for listening to me blabber on.

From Scott Hayward    smash@eisa.net.au

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Good Toroid among other things

Date: Sun, 19 Apr 1998 05:29:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Sun, 19 Apr 1998 01:29:19 EDT

Subject: Re: Good Toroid among other things

Scott,

0.6 mm wire is just about halfway between 23 and 22 gauge wire.  The 0.63mm

is just slightly smaller than 22 g. Either will do nicely for a secondary.

I use an aluminum dryer duct as a toroid.  I don't know how available it is

there, but you should look at hardware stores.

If your neon is rated at 12kV, and sparks can be drawn to the case from

both leads you should be getting the 12kV on the nameplate. I used  long

plastic rods affixed to the leads with elelctrical tape to check a neon that

was suspected of being blown.

Hope this helps.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Good Toroid among other things

Date: Sun, 19 Apr 1998 15:39:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Sun, 19 Apr 1998 08:39:51 PDT

Subject: Re: Good Toroid among other things

It was written:

>The next question is, how do you get such a neat looking toroid.  I got a

>hold of some of that plastic pipe that you can bend and I joined it with

>tape, but that wouldn't hold.  Could you use a rubber car tyre tube. It is

>a perfect toroid shape.  But then how do you get the aluminium to go on

>nicely.

Most people over here use flexible corrugated aluminum ductwork,

typically used for venting your clothes dryer to the outside world.

This is available in several sizes starting at about 3.5 inches across.

Another source is available at an auto parts store.  This is aluminum

ductwork used for connecting the hot air intake on a car's air intake to

the "hot box", which is a thin metal shield bolted to the exhaust

manifold, etc.  Anyway - go to a car place and browse!

It sounds as if you are trying to wrap aluminum foil over an otherwise

nonconductive toriodal form.  If you can find this flex-duct, this would

be easier!  Mike Hammer supports the bottom a his toroids using a

pie-pan, which (I found) you can get for nearly $0 at a second hand

store!

(half the fun of coiling is the scavenging...)

We recently tried using a 18-inch aluminum sphere as a top terminal. We

suffered severe arcing from this sphere to the secondary and primary.

Part of the benefit of the toroidal shape is that the arcs are less

likely to form near the union of the top terminal and the secondary!

>

>How do you test if a neon is putting out 12000V or 6000V if the

transformer

>is rated at 12000V.  I have a digital multimeter and a CRO.  That's

about

>it.

Well - here is how I did this.  I fed the secondary of the neon with my

line power (120v) and measured the voltage at the neon's primary.  For

example - my 120v to 12000v neon (not sure what your line voltage is)

has a conversion ratio of 100:1.  So applying 120 to the 12000 side

(secondary) of the neon should produce something close to 120/100 = 1.2

volts AC.

>

>Does anyone know where you can get copper strap to join the ground

terminals

>together with

If you need copper - try a building supply house - look for copper roof

flashing!  But it won't be free.

>and last but not least, does wood conduct at high voltages or

>not because I could make a fairly good support stand for my primary out

of

>wood but some people have warned me off it.

Wook conducts high voltages quite nicely, if not at first, then soon

enough...  Untreated wood is probably a bad idea as it will certainly

fail eventually (if not immediately).  Once a carbon track forms, it's

all over :(

I sealed the end of my secondary using two disks of plywood which I

boiled in paraffin for about an hour (per Mike's instructions).  This

seems to have worked well as no streamers have penetrated the inside of

the secondary (yet...)

More later!

-Bill the arcstarter

Watch me scavenge at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/boots.gif

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum cleaner blowing

Cc:

Bcc:

X-Attachments:

At 04:55 AM 4/18/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Fri, 17 Apr 1998 21:55:07 -0700

>Subject: Re: vacuum cleaner blowing

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: "Scott Hayward" <smash@eisa.net.au>

>> Date: Thu, 16 Apr 1998 15:43:39 +1000

>> Subject: vacuum cleaner blowing

>>

>> To the list,

>> Can you use a vacuum cleaner in reverse to provide the air for a

>> cylinder gap.

>>

>> >From Scott Hayward    smash@eisa.net.au

>

>Scott,

>My sense is that the ideal ser gap tubes mtd in a large cyl would be

>best, if slots were cut paralell to gaps in the cyl; cap one end and

>apply vacuum to the other end; inducted air would impinge directly at

>each gap.

>I'm guessing, having not built this type yet. Discussion?

>

>You also loose the visual excitement of an operating series gap! So go

>for plex, over PVC! Should make a great night light!

>

>Gene Waldo

>

Scott, Gene and others.

A vacuum cleaner is the best source of airflow for

a static gap. Sucking the air through the teeth of

the gap provides better performance than blowing air

through the gap.

Gene what you describe sounds exactly like my vacuum gap design.

It is a more or less standard cylinder gap with the ends of

the PVC pipe housing sealed. One end has a PVC nipple attached

for attachment of the vacuum cleaner hose. The other end is sealed.

Airflow is admitted into the gap by 1/8 inch wide slots cut into

the PVC pipe housing. These slots run parallel to the electrodes

and directly in line with each gap area. The airflow goes through

these slots directly into the arc channel.

This gap has worked better than anything Ive used so far.

It provides optimum quenching and handles all the power

I have available.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: breaking in caps

Date: Mon, 20 Apr 1998 03:09:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: yubba <yubba@clara.net>

Date: Mon, 20 Apr 98 03:09:14 GMT

Subject: breaking in caps

Organization: self

hi all

I am almost ready to start thinking about putting all my bits together.

The primary is finished ,so is the spark gap. The caps should be ready in a few

days, havent wound the secondary yet . I was going to do that while waiting for

the caps to settle in. My question is about breaking in the caps.  Is it ok to

run the system without the secondary? . what I was planning to do was run the

system up slowly to help break in the caps. I know shorted turns are  a definite

no no but what about  no load conditions? Also what about  RFI from this set up.

Do I need to do it away from any electronics/ computers. Also I have found a

cake tin which would make a great toroid. The only problem is it's only 9"

diameter by 4" deep. would this do as a first top load to tune my coil?  coil

details secondary  4" x 24" 800 turns 22AWG :  9Kv 50mA neon : 0.0177 uF cap

:10 turns 3/8" primary : RQ spark gap. I havent come across this question in the

archives so hope it 's not to obvious. Many thanks

bob golding  lightning without rain

bob yubba@clara.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: breaking in caps

Date: Mon, 20 Apr 1998 04:23:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Sun, 19 Apr 1998 21:23:21 -0700 (PDT)

Subject: Re: breaking in caps

On Mon, 20 Apr 1998, Tesla-2 wrote:

> the caps to settle in. My question is about breaking in the caps.  Is it ok to

> run the system without the secondary? . what I was planning to do was run the

> system up slowly to help break in the caps. I know shorted turns are  a definite

> no no but what about  no load conditions? Also what about  RFI from this set up

>

Bad. Very Bad. A Thing To Be Avoided. There is nowhere for the energy

to go.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: breaking in caps

Cc:

Bcc:

X-Attachments:

At 03:09 AM 4/20/98 GMT, you wrote:

>From: yubba <yubba@clara.net>

>Date: Mon, 20 Apr 98 03:09:14 GMT

>Subject: breaking in caps

>Organization: self

>

>hi all

>     I am almost ready to start thinking about putting all my bits together.

>The primary is finished ,so is the spark gap. The caps should be ready in a few

>days, havent wound the secondary yet . I was going to do that while waiting for

>the caps to settle in. My question is about breaking in the caps.  Is it ok to

>run the system without the secondary?

No. You do not want to do this. Without the secondary there is nowhere

for the RF energy to go. It will tend to backup into your supply

system and possibly damage your transformer.

>what I was planning to do was run the system up slowly to help break in the caps.

>I know shorted turns are a definite no no but what about  no load conditions?

The best way to break in your new cap is to run it on your complete coil.

Keep primary circuit voltage low by running with a very narrow gap.

The pulsing action of the Tesla tank circuit will actually vibrate

the plates of the cap quite vigorously. This will help shake air bubbles loose.

Open the gap in stages. Allow plenty of run time at each level before

opening the gap further. Maximum gap for 9 kv should be around .180 inch.

>Also what about RFI from this set up?

RFI is usually not a big problem except right in the vicinity of the coil.

If you have RF chokes to keep RF out of your transformer and a decent AC

line filter to keep RF off the AC mains that will help. A good RF ground

and short tank circuit wiring also will help.

>Do I need to do it away from any electronics/ computers.

Just to be safe I would. I always unplug my computer before coiling.

That includes the modem line.

> Also I have found a cake tin which would make a great toroid.

> The only problem is it's only 9" diameter by 4" deep.

> would this do as a first top load to tune my coil?

Sure. Ive used all kinds of various metal objects as toploads.

If your cake tin has a sharp edge the streamers will all come from that edge.

A smooth shape will build up more voltage before breaking down and deliver

longer streamers.

>coil details secondary  4" x 24" 800 turns 22AWG :  9Kv 50mA neon : 0.0177 uF cap

>:10 turns 3/8" primary : RQ spark gap. I havent come across this question in the

>archives so hope it 's not to obvious. Many thanks

I assume from the cap size you meant 9kv/60ma neon.

Sounds like a good system to me. Should be able to get 30 inches

from it without much problem. That will probably require a sizable

toroid but you can work your way up to it. I found a 16 X 4 toroid

worked well on my coil of this size.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: breaking in caps

Date: Mon, 20 Apr 1998 11:40:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 20 Apr 1998 06:40:21 -0500

Subject: Re: breaking in caps

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: yubba <yubba@clara.net>

> Date: Mon, 20 Apr 98 03:09:14 GMT

> Subject: breaking in caps

> Organization: self

>

> hi all

>       I am almost ready to start thinking about putting all my bits together.

> The primary is finished ,so is the spark gap. The caps should be ready in a few

> days, havent wound the secondary yet . I was going to do that while waiting for

> the caps to settle in. My question is about breaking in the caps.  Is it ok to

> run the system without the secondary? . what I was planning to do was run the

> system up slowly to help break in the caps. I know shorted turns are  a definite

> no no but what about  no load conditions? Also what about  RFI from this set up

> .

> Do I need to do it away from any electronics/ computers. Also I have found a

> cake tin which would make a great toroid. The only problem is it's only 9"

> diameter by 4" deep. would this do as a first top load to tune my coil?  coil

> details secondary  4" x 24" 800 turns 22AWG :  9Kv 50mA neon : 0.0177 uF cap

> :10 turns 3/8" primary : RQ spark gap. I havent come across this question in the

> archives so hope it 's not to obvious. Many thanks

>

> bob golding  lightning without rain

>

> bob yubba@clara.net

>

Bob,

Not a good thing to do, since it will cause your spark gap to absorb

most of the energy in your system. It may also cause your system to

radiate more RFI. Always use a secondary, and make sure you're getting

top "breakout"  to load down your system.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Resonat rise

Date: Mon, 20 Apr 1998 12:29:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 20 Apr 1998 07:29:33 -0500

Subject: Resonant rise

Organization: MailExcite  (http://www.mailexcite.com:80)

Mike,

Is resonate rise always the case with transformers and capacitors in an AC circuit?

I have heard that a 15 kv circuit can often rise to 50 kv. Does this phenomen always

occur when the circuit is open or under load. Does it matter ? Is the rise primarily

based on the square root of 2. Will I always get 1.4142 X the output voltage of the

transformer.

Thanks,

Paul (A mad Scientist)

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Hamfests and chokes

Date: Mon, 20 Apr 1998 18:00:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Mon, 20 Apr 1998 13:00:04 -0500

Subject: Hamfests and chokes

Hello all,

I went to my first Hamfest yesterday and made a killing!  I got a 10 Amp

variac for $15 and 7.5 Amp for $5.  I found a dealer of magnet wire that

sells wire by the pound and has just about any size you want.

I got two ferrite rings that are about 2.5 in. o.d. to use to make

chokes.  How many turns/feet would I need on each ring using #26.  I'm

planning a TC using a 12kv 30ma neon.

Thanks,

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Resonant rise

Cc: fleischwulf@mailexcite.com

Bcc:

X-Attachments:

At 12:29 PM 4/20/98 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Mon, 20 Apr 1998 07:29:33 -0500

>Subject: Resonat rise

>Organization: MailExcite  (http://www.mailexcite.com:80)

>

>Mike,

> Is resonate rise always the case with transformers and capacitors in an AC

>circuit? I have heard that a 15 kv circuit can often rise to 50 kv. Does this

>phenomen always occur when the circuit is open or under load. Does it matter?

Paul,

Resonant rise refers to the condition with the secondary impedance

of your transformer is equal to the impedance of your main cap at 60 HZ.

In other words your transformer secondary inductance and main capacitance

form a resonant circuit at 60 HZ.

The inductance in this case can be as much as several hundred henries

in neon sign transformers. 60 hz resonance serves to defeat the internal

shunting of the neon allowing you to draw more power.

Its not without risk. The voltage can rise very quickly under

these conditions. If you have your gap set to a safe width then

the voltage will be limited to the breakdown voltage of the gap.

However a misfire due to heavily oxidized electrodes can lead to

very high voltages and a blown cap.

Large transformers dont need 60 hz matching with their caps

as there is plenty of power available.

>Is the rise primarily based on the square root of 2.

>Will I always get 1.4142 X the output voltage of the transformer?

No. Resonant rise depends mainly on the Q of the resonant circuit.

A neon secondary is pretty low Q. Maybe on the range of 3 or 4.

If you place a cap across a 15kv neon the cap will charge to the peak

value of the AC. That is 1.414 X RMS. So your cap could see 21210 volts.

Again you need to have your gap set to fire at well below this voltage.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Resonant rise

Date: Mon, 20 Apr 1998 20:55:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 20 Apr 1998 16:55:21 -0400

Subject: Re: Resonant rise

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Resonat rise

> Date: Monday, April 20, 1998 8:29 AM

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 20 Apr 1998 07:29:33 -0500

> Subject: Resonat rise

> Organization: MailExcite  (http://www.mailexcite.com:80)

>

> Mike,

>  Is resonate rise always the case with transformers and capacitors in an AC circuit?

I'm not Mike, but I'll try to answer some of your questions anyhow...

Resonant rise will occur when the transformer and cap are matched

at the frequency of the mains (usually 50 or 60 Hz).

> I have heard that a 15 kv circuit can often rise to 50 kv. Does this phenomena

> always occur when the circuit is open or under load. Does it matter ?

GREATEST resonant rise is achieved when there is minimal external

loading. Loading will remove the energy that is being stored via

the resonant action. But, hey, when the main spark gap fires,

what do you expect but that the stored energy now gets dumped?

Resonant rise is particularly useful when the transformer voltage is

rather low to start with. You have to be careful, though, because

if left unhampered, the resonant rise can easily destroy almost

any transformer, including a pole pig. But that is what the main spark

gap is for: to fire when the voltage reaches a certain pre-determined

value.

> Is the rise primarily based on the square root of 2?

> Will I always get 1.4142 X the output voltage of the

> transformer.

Nope, the Q of the circuit will determine the ultimate voltage achieved. It is

not all that hard to achieve Q values that will allow a 15KV transformer to

reach 50KV or MORE. The problem is that such large voltages will

destroy the transformer's insulation. So we USE resonant rise, but try to

LIMIT the maximum value by adjusting the spark gap.

Please note that MAXIMUM resonant rise is only achieved when the circuit

is allowed to build up to this level over SEVERAL CYCLES. We coilers want

to remove the stored energy BEFORE that happens.

>

> Thanks,

>

> Paul (A mad Scientist)

>

>

> ---

> Einigkeit und Freiheit

>

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Henry defined

Date: Mon, 20 Apr 1998 21:46:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Mon, 20 Apr 1998 14:46:55 -0700

Subject: Henry defined

Hello All:

It seemed that none of the responses to the 'Henry' question gave an

actual definition of the unit. A Henry is the unit of inductance, that,

in a closed circuit, produces an emf of 1 volt when the current varies

uniformly at 1 ampere/second. Hope this helps.

Eric

edavidson@icva.gov

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secondary Winding

Date: Mon, 20 Apr 1998 21:48:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Basura, Brian" <brian.basura@unistudios.com>

Date: Mon, 20 Apr 1998 14:48:07 -0700

Subject: Secondary Winding

Hello,

I'm finishing winding my 6 1/4" secondary with 22AWG magnet wire.  I was

wondering how people terminated the top wire and connected it to their

toroid. I'm planning on soldering the bottom of the winding to a small

copper plate, epoxy that to the form, then bolt the ground wire to the

plate.  I'm not sure what to do at the top.

Thanks,

Brian D

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Hamfests and chokes

Cc:

Bcc:

X-Attachments:

At 06:00 PM 4/20/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Mon, 20 Apr 1998 13:00:04 -0500

>Subject: Hamfests and chokes

>

>Hello all,

>

>I went to my first Hamfest yesterday and made a killing!  I got a 10 Amp

>variac for $15 and 7.5 Amp for $5.  I found a dealer of magnet wire that

>sells wire by the pound and has just about any size you want.

Good deal Roy. Dont you just love those hamfests? Which hamfest was it?

Maybe the Dupage County hamfest?

If you or anyone has plans to attend the hamfest in Princeton, IL

this coming June let me know and we can meet. Get there early before

I get the good stuff!

The wire dealer was probably Joe Cassata at the Coax Connection

wasnt it? Ive posted Joes address and phone number several times here

on the list. Joe sells wire at fair prices and sells to lots of coilers.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

>

>I got two ferrite rings that are about 2.5 in. o.d. to use to make

>chokes.  How many turns/feet would I need on each ring using #26.

>I'm planning a TC using a 12kv 30ma neon.

I know the cores you got. Ive bought several of them myself.

Try to get 125 turns of AWG 26 on those cores. Leave

at least a 1/2 inch space between start and finish to prevent

arcing. Before winding cover the cores with at least 2 layers

of good quality vinyl tape. The cores are slightly conductive

and will arc if you dont insulate them first.

>Thanks,

>

>Roy Minnich

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: rf chokes

Date: Mon, 20 Apr 1998 23:51:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: yubba <yubba@clara.net>

Date: Mon, 20 Apr 98 23:51:54 GMT

Subject: rf chokes

Organization: self

hi all

Thanks for the feedback on  the dangers of running without the secondary . I'm

glad I asked first!! . I now have another question  this time about chokes. I

went to the local emporium today and picked up lots of " interesting " items

including 4 ferrite cores. They are 2" x 21/2" and about 1/2" thick with a 1"

x11/2" window.  They have a 1/8" gap on one side and are made from 2 c shaped

pieces epoxyed together.  I have on clues to the composition of the ferrite

material.  Wouldthey be any good as rf filter chokes?.  As i have 4 of them I

could glue 2 of them together if needed.  If they are suitable could someone

tell me the wire gauge needed and number of turns, or even hit me with the math

s

needed to work it out myself. Many thanks.

bob golding  lighning without rain

bob yubba@clara.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Resonant rise

Date: Tue, 21 Apr 1998 00:22:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 20 Apr 1998 20:22:35 EDT

Subject: Re: Resonat rise

Fleischwulf,

I am sure your letter was intended for MHammer, but I thought I'd offer my

two cents worth.    Resonant rise occurs when a reflected wave occurs with

constructive interference.  This theoretically happens when a resonant

oscillation is reflected on a quarter wave or odd multiple of the quarter wave

coil.  Now, we all know that this is a situation that arises on the perfect

air core coil with no self capacitance, and the perfect reflection is just

theoretical.  On the other hand, a resonant rise can occur to some degree if

the reflection is just slightly out of phase with the propagating wave such

that the amplitudes are still additive(except where the wave function would be

near zero).   Then the phase shift would be cancelling for a brief time.  This

is more likely the situation that occurs in reality.  In other words, the

addition of the amplitudes would likely not be double the propagation wave's

amplitude, but would result in some value less than that.   I would guess that

the reason that a 15kV tranny would be able to generate a 50kV resonant rise

is due to the addition of the primary frequency plus the associated odd

harmonics whose amplitudes would be additive as well(minus the destructive

interference from the even harmonics).   These resonant waveforms and

reflections at some point could add to the peak of the 60Hz wave of the tranny

and cause the brief high voltage spikes that may indeed reach 50kV at some

point.

Is that about two cents yet?

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Tue, 21 Apr 1998 02:43:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Scott Hayward" <smash@eisa.net.au>

Date: Tue, 21 Apr 1998 12:43:58 +1000

Subject: Re: vacuum cleaner blowing

Mike wrote:

>Scott, Gene and others.

>

>A vacuum cleaner is the best source of airflow for

>a static gap. Sucking the air through the teeth of

>the gap provides better performance than blowing air

>through the gap.

>

>Gene what you describe sounds exactly like my vacuum gap design.

>It is a more or less standard cylinder gap with the ends of

>the PVC pipe housing sealed. One end has a PVC nipple attached

>for attachment of the vacuum cleaner hose. The other end is sealed.

>

>Airflow is admitted into the gap by 1/8 inch wide slots cut into

>the PVC pipe housing. These slots run parallel to the electrodes

>and directly in line with each gap area. The airflow goes through

>these slots directly into the arc channel.

>

>This gap has worked better than anything Ive used so far.

>It provides optimum quenching and handles all the power

>I have available.

But how do you cut the slots in the PVC pipe.

By the way.  I just bought myself 1000 Metres of 0.6mm enammelled copper

wire.  It cost me AUS $42   (ouch )

From Scott Hayward   smash@eisa.net.au

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: breaking in caps

Date: Tue, 21 Apr 1998 04:38:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Mon, 20 Apr 1998 21:38:51 -0700

Subject: Re: breaking in caps

Here's an idea, use two sided tape and stick your cap on top of a clothes dryer,

turn it on, the vibrations from th dryer should help release some bubbles.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Tue, 21 Apr 1998 06:45:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dan Murphy <danmurphy@1stnetusa.com>

Date: Mon, 20 Apr 1998 23:45:15 -0700 (PDT)

Subject: Re: vacuum cleaner blowing

At 02:43 AM 4/21/98 GMT, you wrote:

>From: "Scott Hayward" <smash@eisa.net.au>

>Mike wrote:

>

>>Airflow is admitted into the gap by 1/8 inch wide slots cut into

>>the PVC pipe housing. These slots run parallel to the electrodes...

>>This gap has worked better than anything Ive used so far.

>>It provides optimum quenching and handles all the power I have available.

>

>But how do you cut the slots in the PVC pipe?

>

>From Scott Hayward   smash@eisa.net.au

Scott -

I'm sure there are a variety of ways to cut the slots, PVC being

an easily workable material. If it were me, I'd probably use my router with

an 1/8" straight bit, or possible a Dremel-type tool. Most anything with an

1/8" cutting edge would do it -  I might even try the table saw, my blades

have a 1/8" kerf, but I would have to devise some kind of jig to hold the

round pipe securely as I dropped it down onto the blade. I wouldn't attempt

to hold it barehanded! Good way to lose fingers... I've seen it happen. :-(

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- Talk is cheap... the supply far exceeds the demand! ---

<danmurphy@1stnetusa.com>

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Tue, 21 Apr 1998 07:15:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 21 Apr 1998 00:15:16 -0700

Subject: Re: vacuum cleaner blowing

Organization: Self

Tesla-2 wrote:

>

> From: "Scott Hayward" <smash@eisa.net.au>

> Date: Tue, 21 Apr 1998 12:43:58 +1000

> Subject: Re: vacuum cleaner blowing

>

> Mike wrote:

>

> >Scott, Gene and others.

> >

> >A vacuum cleaner is the best source of airflow for

> >a static gap. Sucking the air through the teeth of

> >the gap provides better performance than blowing air

> >through the gap.

> >

> >Gene what you describe sounds exactly like my vacuum gap design.

> >It is a more or less standard cylinder gap with the ends of

> >the PVC pipe housing sealed. One end has a PVC nipple attached

> >for attachment of the vacuum cleaner hose. The other end is sealed.

> >

> >Airflow is admitted into the gap by 1/8 inch wide slots cut into

> >the PVC pipe housing. These slots run parallel to the electrodes

> >and directly in line with each gap area. The airflow goes through

> >these slots directly into the arc channel.

> >

> >This gap has worked better than anything Ive used so far.

> >It provides optimum quenching and handles all the power

> >I have available.

>

> But how do you cut the slots in the PVC pipe.

>

Scott,

Us'ns up here on the right side of the planet call that "creative field

expedience".  Gonna involve some jig'n'fixturing, no doubt; unless you

know of a milling machine with an indexing head.  I'd get a couple of

practice lengths of gap tube and play with pushing it down on a table

saw blade, set at low cut depth, between my Cu tube mtg holes...

Mike's done it so he probably has the right answer!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

PS: damn! my e-mail only lets me have 4 lines for my sign-off!!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coilers in Tennessee

Date: Tue, 21 Apr 1998 07:50:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kent Rankin" <kentrankin@theatreorgans.com>

Date: Tue, 21 Apr 1998 03:50:09 -0400

Subject: Coilers in Tennessee

I was wondering if anyone on the list knows of any coilers or coiling

resources in the state of Tennessee. Preferably in Knoxville, TN.

Thanks,

Kent Rankin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: vacuum cleaner blowing

Date: Tue, 21 Apr 1998 09:37:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: wwl@netcomuk.co.uk (Mike Harrison)

Date: Tue, 21 Apr 1998 09:37:34 GMT

Subject: Re: vacuum cleaner blowing

Organization: White Wing Logic

>

>But how do you cut the slots in the PVC pipe.

Just drill a line of holes - for this application you don't really

need to bother cutting/filing them out into a slot.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Hamfests and chokes

Date: Tue, 21 Apr 1998 12:38:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Wikkerink <bert@mitra.com>

Date: Tue, 21 Apr 1998 08:38:04 -0400

Subject: Re: Hamfests and chokes

Organization: Mitra Imaging Inc.

Hi all,

Speaking of hamfests, I am planning on going to my first at the end of May in

Rochester, NY. From what everyone else says about them, I should be in for a

good time.

Bert Wikkerink

To reply to me personally, remove the antispam 'Z' from my email address.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: breaking in caps

Date: Tue, 21 Apr 1998 14:35:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Tue, 21 Apr 1998 09:35:14 -0500

Subject: Re: breaking in caps

OK...the dryer is a good idea.  I personally used my stereo speaker...set it on

top and let the music roll.  Seemed to work well.

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 04/21 12:38 AM >>>

From: mike nicholas <705728@ican.net>

Date: Mon, 20 Apr 1998 21:38:51 -0700

Subject: Re: breaking in caps

Here's an idea, use two sided tape and stick your cap on top of a clothes dryer,

turn it on, the vibrations from th dryer should help release some bubbles.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum cleaner blowing

Cc:

Bcc:

X-Attachments:

At 07:15 AM 4/21/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 21 Apr 1998 00:15:16 -0700

>Subject: Re: vacuum cleaner blowing

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: "Scott Hayward" <smash@eisa.net.au>

>> Date: Tue, 21 Apr 1998 12:43:58 +1000

>> Subject: Re: vacuum cleaner blowing

>>

>> Mike wrote:

>>

>> >Scott, Gene and others.

>> >

>> >A vacuum cleaner is the best source of airflow for

>> >a static gap. Sucking the air through the teeth of

>> >the gap provides better performance than blowing air

>> >through the gap.

>> >

>> >Gene what you describe sounds exactly like my vacuum gap design.

>> >It is a more or less standard cylinder gap with the ends of

>> >the PVC pipe housing sealed. One end has a PVC nipple attached

>> >for attachment of the vacuum cleaner hose. The other end is sealed.

>> >

>> >Airflow is admitted into the gap by 1/8 inch wide slots cut into

>> >the PVC pipe housing. These slots run parallel to the electrodes

>> >and directly in line with each gap area. The airflow goes through

>> >these slots directly into the arc channel.

>> >

>> >This gap has worked better than anything Ive used so far.

>> >It provides optimum quenching and handles all the power

>> >I have available.

>>

>> But how do you cut the slots in the PVC pipe.

>>

>Scott,

>Us'ns up here on the right side of the planet call that "creative field

>expedience".  Gonna involve some jig'n'fixturing, no doubt; unless you

>know of a milling machine with an indexing head.  I'd get a couple of

>practice lengths of gap tube and play with pushing it down on a table

>saw blade, set at low cut depth, between my Cu tube mtg holes...

>Mike's done it so he probably has the right answer!

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene, Scott and others.

To cut the slots I tried various approaches including the table

saw approach. While that worked I got the best results from

using an 1/8 inch cutter tool meant for use in a Dremel MotoTool.

I chucked the tool in my drill press and made a jig to hold

the PVC. All I had to do then was lower the bit into the PVC

with the drill press and then feed the PVC into the cutter.

You need to run the speed fairly high and feed the material

slowly to prevent melting the PVC.

Like Gene said I took some scraps of PVC and practiced a bit

until I got it right.

Lacking a drill press and the cutter bit I suppose you

could create the slots by drilling a series of holes with

a hand drill and then cut open the slot with a razor saw

or a length of hacksaw blade. Then smooth it up with a file.

Seems like lots of work though.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coilers in Tennessee

Cc:

Bcc:

X-Attachments:

At 07:50 AM 4/21/98 GMT, you wrote:

>From: "Kent Rankin" <kentrankin@theatreorgans.com>

>Date: Tue, 21 Apr 1998 03:50:09 -0400

>Subject: Coilers in Tennessee

>

>  I was wondering if anyone on the list knows of any coilers or coiling

>resources in the state of Tennessee. Preferably in Knoxville, TN.

>

>           Thanks,

>           Kent Rankin

One experienced coiler I know of in Tennesse is my good friend

Gary Weaver. Gary is in Murfreesboro. Gary is not on this list however.

Check out his web page at: http://home.earthlink.net/~gweaver/

His email address is: gweaver@earthlink.net

Tell him I sent you. Wont get you anything in return but sarcasm

but tell him anyway.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: testing neons

Cc:

Bcc:

X-Attachments:

At 03:39 PM 4/19/98 GMT, you wrote:

>From: "Bill the arcstarter" <arcstarter@hotmail.com>

>Date: Sun, 19 Apr 1998 08:39:51 PDT

>Subject: Re: Good Toroid among other things

>

>>

>>How do you test if a neon is putting out 12000V or 6000V if the

>>transformer is rated at 12000V?  I have a digital multimeter and

>>a CRO. That's about it.

>

>Well - here is how I did this.  I fed the secondary of the neon with my

>line power (120v) and measured the voltage at the neon's primary.  For

>example - my 120v to 12000v neon (not sure what your line voltage is)

>has a conversion ratio of 100:1.  So applying 120 to the 12000 side

>(secondary) of the neon should produce something close to 120/100 = 1.2

>volts AC.

>

Scott and Bill,

I have used the method Bill talks about here of putting 120 Volts

into the secondary and reading the resulting voltage on the primary.

My results with this method were mixed at best. It seems that this

doesn not provide enough energy to energize the core and can

give erroneous readings.

A much better method is to inject a low AC voltage into the primary

and then read the resulting secondary voltages. This gives much better

repeatable results. I use 4 volts from my variac and then read the

voltage on the secondary. With 4 volts on the primary you can expect

to see 400 volts between the outputs of a 12kv neon. Or 200 volts

from each side to ground. Using this method you can tell if one side

of your neon has a problem. The voltages should be nearly identical.

Also this is a good method of picking neons to be used in parallel.

Try to parallel neons closely matched in voltage output.

If you dont have a variac to deliver the low AC voltage get a inexpensive

6.3 VAC step down transformer. With 6.3 volts in you can expect 315

volts AC from each output to the case ground.

Careful. This is still enough voltage to give you a nasty shock.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: breaking in caps

Date: Tue, 21 Apr 1998 17:59:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Tue, 21 Apr 1998 10:59:07 -0700

Subject: Re: breaking in caps

Organization: Self

Tesla-2 wrote:

>

> From: Kevin Batchelor <kbatchelor@arslimited.com>

> Date: Tue, 21 Apr 1998 09:35:14 -0500

> Subject: Re: breaking in caps

>

> OK...the dryer is a good idea.  I personally used my stereo speaker..

>

> Kevin

>

> >>> Tesla-2 <tesla-2@emachine.com> 04/21 12:38 AM >>>

> From: mike nicholas <705728@ican.net>

> Date: Mon, 20 Apr 1998 21:38:51 -0700

> Subject: Re: breaking in caps

>

My notion would utilize 'clench-relax' effect: apply like a 6vdc lantern

battery to the cap and using a rotary or some cycling sw type thing to

charge and discharge the cap: 1 cycle: 'chg +/- |disch| chg -/+| disch'.

It may not even have to be that complex, but this should a apply direct

physical 'pumping' action to the plates. Run this at a moderate rate for

a day or two... commentary?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Hamfests and chokes

Date: Tue, 21 Apr 1998 20:00:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Tue, 21 Apr 1998 15:00:20 -0500

Subject: RE: Hamfests and chokes

Hi Mike and all,

The Hamfest was at the Hawthorne Race Course in Chicago.  I found it at

the URL you posted a while ago.  Yep pretty neat place to find stuff for

cheap.  I figure it would have cost me $200 for the same stuff out of a

catalog and I got it for 1/10 that.  I saw a big cast iron transformer

rated 15kv  500mA!!!!!  Unfortunately it was priced at $99.00    Wonder

how he came up with that price?  Why not $98.00 or $100???? ;-)

Thanks for the info. in winding the ferrite rings.  I think I remember a

power resistor was to be wired in series with each choke too.  What

value and wattage would these resistors be??  Should I get the ceramic

wire wound type or the kind that has the slick looking anodized aluminum

heat sink around the resistor.  I was thinking that the aluminum heat

sink might allow arcs to jump across.  Yes/no????

The ferrite rings have a snug-fitting plastic case.  I think I should

leave it on and wind the wire right over it to make the chokes.  Yes/no?

Yep, it was Joe Cassatta the wire man I spoke with.  Nice guy.

For other northern Illinois coilers:

I picked up some fliers and there is a Hamfest at the Sandwich

Fairgrounds in Sandwich, IL on May 3,.  There is another Hamfest at the

Dupage County Fairgrounds in Wheaton on June 14, and another at the

Waubonsee Community College in Sugar Grove on July 19,.

Happy coiling,

Roy Minnich

----------

From:   Tesla-2[SMTP:tesla-2@emachine.com]

Sent:   Tuesday, April 21, 1998 1:38 PM

To:     Minnich, Roy

Subject:        Re: Hamfests and chokes

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 20 Apr 1998 17:13:23 -0500

Subject: Re: Hamfests and chokes

At 06:00 PM 4/20/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Mon, 20 Apr 1998 13:00:04 -0500

>Subject: Hamfests and chokes

>

>Hello all,

>

>I went to my first Hamfest yesterday and made a killing!  I got a 10 Amp

>variac for $15 and 7.5 Amp for $5.  I found a dealer of magnet wire that

>sells wire by the pound and has just about any size you want.

Good deal Roy. Dont you just love those hamfests? Which hamfest was it?

Maybe the Dupage County hamfest?

If you or anyone has plans to attend the hamfest in Princeton, IL

this coming June let me know and we can meet. Get there early before

I get the good stuff!

The wire dealer was probably Joe Cassata at the Coax Connection

wasnt it? Ive posted Joes address and phone number several times here

on the list. Joe sells wire at fair prices and sells to lots of coilers.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

>

>I got two ferrite rings that are about 2.5 in. o.d. to use to make

>chokes.  How many turns/feet would I need on each ring using #26. =20

>I'm planning a TC using a 12kv 30ma neon.

I know the cores you got. Ive bought several of them myself.

Try to get 125 turns of AWG 26 on those cores. Leave

at least a 1/2 inch space between start and finish to prevent

arcing. Before winding cover the cores with at least 2 layers

of good quality vinyl tape. The cores are slightly conductive

and will arc if you dont insulate them first.

>Thanks,

>

>Roy Minnich 20

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Hamfests and chokes

Cc:

Bcc:

X-Attachments:

At 08:00 PM 4/21/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Tue, 21 Apr 1998 15:00:20 -0500

>Subject: RE: Hamfests and chokes

>

>Hi Mike and all,

>

>The Hamfest was at the Hawthorne Race Course in Chicago.  I found it at

>the URL you posted a while ago.  Yep pretty neat place to find stuff for

>cheap.  I figure it would have cost me $200 for the same stuff out of a

>catalog and I got it for 1/10 that.  I saw a big cast iron transformer

>rated 15kv  500mA!!!!!  Unfortunately it was priced at $99.00    Wonder

>how he came up with that price?  Why not $98.00 or $100???? ;-)

Thats sounds like it might have been what is known as a bombarder.

Its a heavy duty transformer neon shops use to purge contaminants

from their tubes. It usually includes an external inductive ballast

for current limiting. These are just about the ultimate in neon supplies

and cost a small fortune new.

>

>Thanks for the info. in winding the ferrite rings.  I think I remember a

>power resistor was to be wired in series with each choke too.

The series damping resistors are normally only required with

air core chokes with many turns of wire. With your ferrite

toroids they probably wont be needed. I never needed them with mine.

>

>The ferrite rings have a snug-fitting plastic case.  I think I should

>leave it on and wind the wire right over it to make the chokes.  Yes/no?

I wouldnt. Tape up the toroid and wind your wire directly on it.

>

>Yep, it was Joe Cassatta the wire man I spoke with.  Nice guy.

Yes he is and he just loves coilers. Probably because we buy pounds

of wire at a time. He can get you almost anything wire wise.

>

>

>For other northern Illinois coilers:

>I picked up some fliers and there is a Hamfest at the Sandwich

>Fairgrounds in Sandwich, IL on May 3,.

The Sandwich fest was always a favorite of mine. It a small

hamfest but I always seemed to find nice goodies there.

>There is another Hamfest at the Dupage County Fairgrounds in

>Wheaton on June 14,

I dont remember this one. Seems to me thats about the same time

as the 6 meter club hamfest that was held at the Santa Fe Raceway.

Perhaps its the same fest just not at the race track anymore?

>Waubonsee Community College in Sugar Grove on July 19,.

This is a good one and I actually made it back to the Chicago

area for this one last year. Only problem is its held out on the

asphalt parking lot at the college and last year it was sunny and

95 degrees.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Secondary Winding

Date: Tue, 21 Apr 1998 22:51:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Steve Falco <sfalco@worldnet.att.net>

Date: Tue, 21 Apr 1998 18:51:46 -0400

Subject: Re: Secondary Winding

Tesla-2 wrote:

>

> From: "Basura, Brian" <brian.basura@unistudios.com>

> Date: Mon, 20 Apr 1998 14:48:07 -0700

> Subject: Secondary Winding

>

> Hello,

>

> I'm finishing winding my 6 1/4" secondary with 22AWG magnet wire.  I was

> wondering how people terminated the top wire and connected it to their

> toroid. I'm planning on soldering the bottom of the winding to a small

> copper plate, epoxy that to the form, then bolt the ground wire to the

> plate.  I'm not sure what to do at the top.

>

> Thanks,

> Brian D

My toroid is plastic garden drain pipe, covered with aluminum metal

adhesive tape.  I just stick the wire from the top of the coil into the

aluminum tape.  Given the high voltage available, and the relatively

small capacitance of the toroid (20-30pF), it is hard to make a bad

connection!  A very high powered coil might need something fancier, but

at the 1.5 KW level I was running, it worked fine.

Steve Falco

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Hamfests and chokes

Date: Tue, 21 Apr 1998 23:58:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Date: Tue, 21 Apr 1998 18:58:03 -0500

Subject: RE: Hamfests and chokes

Hi Mike and all,

I saw a guy at the Hamfest that had some toroid coils that were close to

4 in. in dia.  Must have had a mile of # 38 wire wrapped on them.  Would

a bigger ferrite core like this be better for our TC applications.  Of

course I'd have to cut the #38 wire off and wrap some #26 on them and

that would be a pain but....

Roy Minnich

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum cleaner blowing

Cc:

Bcc:

X-Attachments:

At 07:15 AM 4/21/98 GMT, you wrote:

>From: Gene Waldo <gwaldo@1stnetusa.com>

>Date: Tue, 21 Apr 1998 00:15:16 -0700

>Subject: Re: vacuum cleaner blowing

>Organization: Self

>

>Tesla-2 wrote:

>>

>> From: "Scott Hayward" <smash@eisa.net.au>

>> Date: Tue, 21 Apr 1998 12:43:58 +1000

>> Subject: Re: vacuum cleaner blowing

>>

>> Mike wrote:

>>

>> >Scott, Gene and others.

>> >

>> >A vacuum cleaner is the best source of airflow for

>> >a static gap. Sucking the air through the teeth of

>> >the gap provides better performance than blowing air

>> >through the gap.

>> >

>> >Gene what you describe sounds exactly like my vacuum gap design.

>> >It is a more or less standard cylinder gap with the ends of

>> >the PVC pipe housing sealed. One end has a PVC nipple attached

>> >for attachment of the vacuum cleaner hose. The other end is sealed.

>> >

>> >Airflow is admitted into the gap by 1/8 inch wide slots cut into

>> >the PVC pipe housing. These slots run parallel to the electrodes

>> >and directly in line with each gap area. The airflow goes through

>> >these slots directly into the arc channel.

>> >

>> >This gap has worked better than anything Ive used so far.

>> >It provides optimum quenching and handles all the power

>> >I have available.

>>

>> But how do you cut the slots in the PVC pipe.

>>

>Scott,

>Us'ns up here on the right side of the planet call that "creative field

>expedience".  Gonna involve some jig'n'fixturing, no doubt; unless you

>know of a milling machine with an indexing head.  I'd get a couple of

>practice lengths of gap tube and play with pushing it down on a table

>saw blade, set at low cut depth, between my Cu tube mtg holes...

>Mike's done it so he probably has the right answer!

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene, Scott and others.

To cut the slots I tried various approaches including the table

saw approach. While that worked I got the best results from

using an 1/8 inch cutter tool meant for use in a Dremel MotoTool.

I chucked the tool in my drill press and made a jig to hold

the PVC. All I had to do then was lower the bit into the PVC

with the drill press and then feed the PVC into the cutter.

You need to run the speed fairly high and feed the material

slowly to prevent melting the PVC.

Like Gene said I took some scraps of PVC and practiced a bit

until I got it right.

Lacking a drill press and the cutter bit I suppose you

could create the slots by drilling a series of holes with

a hand drill and then cut open the slot with a razor saw

or a length of hacksaw blade. Then smooth it up with a file.

Seems like lots of work though.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: theory

Date: Wed, 22 Apr 1998 01:01:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE D <gordrew@home.com>

Date: Tue, 21 Apr 1998 18:01:35 -0700

Subject: theory

I am trying write a spreadsheet that can run through many values for the

height and raduis of a secondary coil. As well as differnt kinds of

primarys and differnt terminals.  My overall goal is to be able to predict

the best coil based on a 12Kv/30ma transformer and to gain more knowlage

about the differnt ratios in general.

But my one question is, what kind of values would i be loking for in the

secondary, primary, and terminal to see that it is at peek performance.

--Mike Drew

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Hamfests and chokes

Cc:

Bcc:

X-Attachments:

At 11:58 PM 4/21/98 GMT, you wrote:

>From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

>Date: Tue, 21 Apr 1998 18:58:03 -0500

>Subject: RE: Hamfests and chokes

>

>Hi Mike and all,

>

>I saw a guy at the Hamfest that had some toroid coils that were close to =

>4 in. in dia.  Must have had a mile of # 38 wire wrapped on them.  Would =

>a bigger ferrite core like this be better for our TC applications.  Of =

>course I'd have to cut the #38 wire off and wrap some #26 on them and =

>that would be a pain but....

>

>Roy Minnich

Those would be great for some high power coils

but the 2.5 inch diameter ones you have should be fine

up to 1500 watts or so. Maybe more. Ive run mine close

to 1500 watts with no problems.

I probably would have grabbed a couple for future use

but Im a pack rat when it comes to hamfests. I buy

all kinds of things that I just know I will need "someday".

Thats why my basement , my garage, my dad's garage, and a big

storage locker are all packed full of stuff.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:  chokes

Date: Wed, 22 Apr 1998 01:13:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Tue, 21 Apr 1998 18:13:45 -0700

Subject: Re:  chokes

I get a lot of magnets out of magnetrons, from microwave ovens.  They are

very powerfull, non metallic, they mesure 2 5/8" diam. X 3/8" thick window is

7/8",. Q: Will these make good chokes?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Resonat rise

Date: Wed, 22 Apr 1998 02:50:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Tue, 21 Apr 1998 19:50:09 PDT

Subject: Re: Resonat rise

Mike Hollingsworth, coilers,

It was written:

>   I am sure your letter was intended for MHammer, but I thought I'd offer my

>two cents worth.    Resonant rise occurs when a reflected wave occurs with

>constructive interference.  This theoretically happens when a resonant

>oscillation is reflected on a quarter wave or odd multiple of the quarter wave

>coil.  Now, we all know that this is a situation that arises on the perfect

>air core coil with no self capacitance, and the perfect reflection is just

>theoretical.  On the other hand, a resonant rise can occur to some degree if

>the reflection is just slightly out of phase with the propagating wave such

>that the amplitudes are still additive(except where the wave function would be

>near zero).   Then the phase shift would be cancelling for a brief time.  This

>is more likely the situation that occurs in reality.  In other words, the

>addition of the amplitudes would likely not be double the propagation wave's

>amplitude, but would result in some value less than that.   I would guess that

>the reason that a 15kV tranny would be able to generate a 50kV resonant rise

>is due to the addition of the primary frequency plus the associated odd

>harmonics whose amplitudes would be additive as well(minus the destructive

>interference from the even harmonics).   These resonant waveforms and

>reflections at some point could add to the peak of the 60Hz wave of the tranny

>and cause the brief high voltage spikes that may indeed reach 50kV at some

>point.

>   Is that about two cents yet?

>Mike Hollingsworth

Yes, this is the most general case.  However most tesla systems are more

closely (or at least adequately) modeled as a collection of "lumped"

components.  The spatial transmission line approach is generally not

needed for most tesla work.  (correct me if I am wrong here...)

I think Paul's initial question was one stemming from his observation

that the voltage coming out of his neon's secondary will increase if a

smallish (0.01 uf etc) cap is attached across it's output.

It must be noted that this condition will, under no-load conditions,

without core saturation or current limiting (ie - current drawn out of

the neon is less than its rated value) produce a sine-wave of increased

amplitude (higher voltage) than that produced without the output

capacitor.

In this case what we really have is the "Capacitive reactance" of the

capacitor cancels out some of the "Inductive reactance" of the current

limited neon secondary.

You really need to understand the vector aspects of this - but you'll

have to trust me that the capacitor can (and does) cancel out some of

the inductance (which was designed into the neon specifically to limit

the output current).  This causes the observed rise of the output

voltage.

The 'ideal' capacitor size which coilers compute via C=I/(2*PI*F*V) is

the size of the capacitor which will theoretically create the highest

level of resonant rise, for a transformer of voltage V, current I, and

frequency F.  This does not *quite* happen in real life due to various

nonlinearities, particularly the magnetic shunting of the neon's core.

The characterization of the transformer's impedance being Vmax/Imax is

seldom (if ever) valid for any particular point in time...

(Perhaps someone thought this was interesting - or am I just boring the

list?)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Mother's favorite recipe for homemade rolled capacitors...

Date: Wed, 22 Apr 1998 03:08:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Tue, 21 Apr 1998 20:08:10 PDT

Subject: Mother's favorite recipe for homemade rolled capacitors...

Hello coilers!

I was thinking about trying to build some rolled poly/PVC caps for a

"small" coiling project.  A friend (who is not on the list) just plopped

down $300 for a large qty of 2 mil copper foil.  I want to instruct him

on how to use it most efficiently before he cuts it all up into disks to

fit into a 5-gallon bucket, or something equally garish...

I've not tried building a roll cap yet - all my caps up until now have

been poly plate units using only one thick (0.060 inch) slab of poly

between conductors.

Can anyone toss out a few URL's describing common construction

techniques/recipies for rolled caps?  I want to check my calculations a

bit.

How do I search/grep or otherwise access the tesla-2 message

archives???

I poked aroung the web-ring for rolled instructions but came up dry.

This has been discussed/beaten to death on the list - but I can't find

it when I need it!

Thanks!

-Bill the arcstarter

Just a little click and you'll be smiling...

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: [Fwd: Mother's favorite recipe for homemade rolled capacitors...]

Date: Wed, 22 Apr 1998 09:10:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Abdullah M Fazaa <amfazaa@expec.com>

Date: Wed, 22 Apr 1998 12:10:09 +0300

Subject: [Fwd: Mother's favorite recipe for homemade rolled capacitors...]

hello Bill....

>  Can anyone toss out a few URL's describing common construction

> techniques/recipies for rolled caps?  I want to check my calculations a

> bit.

>

look at these picture in this sight , it gives you a general

construction of how the HV capacitors are arranged and assembled

together for tesla coils purposes.... you have different types of

capacitors .

http://home.earthlink.net/~gweaver/cap.htm

I 've seen more than one topic for rolled capacitors construction , but

I have to search for them again and provide you the whole thing you

asked .... Sorry ...I have no time today. But the above documented site

for the original auther should satisfy your need.

Abdo little-engineer

amfazaa@expec.com

From: tesla-2@emachine.com (Tesla-2)

Subject: Mother's favorite recipe for homemade rolled capacitors...

Date: Wed, 22 Apr 1998 03:08:10 GMT

Message-Id: <98042123155845698@emachine.com>

Organization: The Energy Machine Information System 504-733-3462

To: amfazaa@expec.com

From: "Bill the arcstarter" <arcstarter@hotmail.com>

Date: Tue, 21 Apr 1998 20:08:10 PDT

Subject: Mother's favorite recipe for homemade rolled capacitors...

Hello coilers!

I was thinking about trying to build some rolled poly/PVC caps for a

"small" coiling project.  A friend (who is not on the list) just plopped

down $300 for a large qty of 2 mil copper foil.  I want to instruct him

on how to use it most efficiently before he cuts it all up into disks to

fit into a 5-gallon bucket, or something equally garish...

I've not tried building a roll cap yet - all my caps up until now have

been poly plate units using only one thick (0.060 inch) slab of poly

between conductors.

Can anyone toss out a few URL's describing common construction

techniques/recipies for rolled caps?  I want to check my calculations a

bit.

How do I search/grep or otherwise access the tesla-2 message

archives???

I poked aroung the web-ring for rolled instructions but came up dry.

This has been discussed/beaten to death on the list - but I can't find

it when I need it!

Thanks!

-Bill the arcstarter

Just a little click and you'll be smiling...

http://www.geocities.com/CapeCanaveral/Hangar/6160

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Rochester Hamfest

Date: Wed, 22 Apr 1998 13:11:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Wikkerink <bert@mitra.com>

Date: Wed, 22 Apr 1998 09:11:32 -0400

Subject: Rochester Hamfest

Organization: Mitra Imaging Inc.

Apologies are in order. I had inserted a "Z" in my email address to

prevent spamming. I neglected to remove it when I posted to the list.

Here is the body of my message again.

Bert Wikkerink

Is anyone planning to go to the hamfest in Rochester, New York ?. I plan

on going and would like to meet anyone from the list who might be going

as well. The URL for the 'fest is:

http://www.rochesterhamfest.org/rochester.asp

Let me know.

Bert Wikkerink

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: chokes

Cc:

Bcc:

X-Attachments:

At 07:03 PM 4/22/98 GMT, you wrote:

>From: mike nicholas <705728@ican.net>

>Date: Wed, 22 Apr 1998 12:03:15 -0700

>Subject: Re: chokes

>

>

>

>mike nicholas wrote:

>

>>    I get a lot of magnets out of magnetrons, from microwave ovens.  They are

>> very powerfull, non metallic, they mesure 2 5/8" diam. X 3/8" thick window is

>> 7/8",. Q: Will these make good chokes?

>

No.  The permeability of the material would be quite low and would tend to saturate easily. Definitely not good for RF use. The fact that itsa magnet is probably irrelevant.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: chokes

Date: Wed, 22 Apr 1998 19:03:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: mike nicholas <705728@ican.net>

Date: Wed, 22 Apr 1998 12:03:15 -0700

Subject: Re: chokes

mike nicholas wrote:

>    I get a lot of magnets out of magnetrons, from microwave ovens.  They are

> very powerfull, non metallic, they mesure 2 5/8" diam. X 3/8" thick window is

> 7/8",. Q: Will these make good chokes?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Sci EXPO & HV demo

Date: Thu, 23 Apr 1998 07:30:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Gene Waldo <gwaldo@1stnetusa.com>

Date: Thu, 23 Apr 1998 00:30:42 -0700

Subject: Sci EXPO & HV demo

Organization: Self

Mike Hammer,

As threatened, Dan & I made it to the EXPO at a local college. Lotsa

neat lectures: JPL On Jupiter & moons flyby; "Lucy's" bag man (he 1st

bagged her bones) and that sort of stuff- piece d'resistance was the HV

display-after dark: lots of disembodied energy! Fortunately we made

instant rapport with crew, and had free access to shoot equip during

setup. 3 or 4 neat coilers in audience...  Biggest power disch was

~4-5'.

Two coils on stage: 'conventional (rot sp gap)' & 'tube'. One guy sits

in 'electric bird bath' pond, and sqirts juice from more prominent

appendages! Lit flour tube from most prominent orifice...n'at kinda

stuff.

After processing, I may have up to 9 xmissable jpg pics....  what can

you stand?

-----------

Bert, FYI: was able to show your pics of rot gap to bystander: said unit

was out of radar system.

===================

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

===================

PS: Kent Rankin, and CAP builders:

Don't miss: < http://home.earthlink.net/~gweaver/cap.htm > !!

Mike: and how come Gary isn't on T-2?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: flat plate design quiestion

Date: Thu, 23 Apr 1998 13:51:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com

Date: Thu, 23 Apr 1998 09:51:17 -0400

Subject: flat plate design quiestion

Mike and all,

I have FINALLY finished cutting 980 squares of poly and over 220

aluminum plates for my 9 caps.  I am now in the process of stadcking

them.  I found it was much easier to make a 3 sided box out of cardboard

cut to the exact size of the poly to lay the sheets of poly and aluminum

in to keep them from sliding around while stacking.

Anyway, my questions is how do you clamp them together?  I have

heard of using zip ties but I am sure that would cause them to warp.  Do

you usually just wedge them in a container and fill them with oil?

Thanks again for all the help

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: bombarder

Cc:

Bcc:

X-Attachments:

At 01:51 AM 3/22/98 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Sat, 21 Mar 1998 20:51:06 -0500

>Subject: bombarder

>

>Hi all

>

>anyone know where I can get one of those bombarders that Mike described..

>or something similar.

They are pretty uncommon Chris. You would only find

one at a neon shop that does its own tubes and those

are getting pretty rare these days. The bombarder is

very expensive. Over $1000. As a result Ive heard that some

neon shops buy a pole pig to use as a bombarder.

I know a guy that got a bombarder cheap when a neon shop

in his town went out of business.

>I would have paid  the 100$ for one without a second thought.

You and me both.

>

>I would like to find the guy who had that one.

>

>anyone with any info please contact me off list at  cwatkins@seidata.com

>thanks

>Chris Watkins

>

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: What type of wire?

Date: Thu, 23 Apr 1998 19:26:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <Tom.Stock@gsbsc.gensig.com>

Date: Thu, 23 Apr 1998 15:26:49 -0400

Subject: What type of wire?

Ok I'm almost done with my coil.

Specs so far:

22" x 4.5" Secondary, #24 wire (done and mounted)

14 turns, .25" Flat Primary (done and mounted)

6 Saltwater Caps (6 more too for 2 neons) (done)

RQ Spark Gap (3/4" Pipe, 10 pcs) (working on it now)

Torid chokes (searching for toroids)

Safety gap (working on it now)

Input: 16kv/20ma Neon (and another one for once I get it going)

What type of wiring can I use just to get sparking? I have 12 guage

solid copper house wire, but this stuff is only rated to 400V.

Can I use it until I can afford to get something better or is this

dangerous?

Thanks

-tom

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate design question

Cc:

Bcc:

X-Attachments:

At 01:51 PM 4/23/98 GMT, you wrote:

>From: rpittman@juno.com

>Date: Thu, 23 Apr 1998 09:51:17 -0400

>Subject: flat plate design question

>

>Mike and all,

>        I have FINALLY finished cutting 980 squares of poly and over 220

>aluminum plates for my 9 caps.  I am now in the process of stacking

>them.  I found it was much easier to make a 3 sided box out of cardboard

>cut to the exact size of the poly to lay the sheets of poly and aluminum

>in to keep them from sliding around while stacking.

One thing I did when stacking up my caps was to "glue" together

the sheets of poly with a small amount of mineral oil.

On this last cap I was using ten 6 mil sheets for each

dielectric layer. So I stuck together 10 squares with mineral

oil. The oil kept the poly sheets from sliding all over the place

and I was able to treat the stack of ten sheets as a single unit.

Unfortunately that cap failed as it was a single cap. It had 60 mils

of poly and failed rather spectacularly after a time at 12kv.

I learned my lesson. I will never use a single cap above 9kv

again. Two is series minimum for 12kv and 3 in series for 15kv.

>  Anyway, my questions is how do you clamp them together?  I have

>heard of using zip ties but I am sure that would cause them to warp.  Do

>you usually just wedge them in a container and fill them with oil?

>

Sandwich the caps between sheets of plexiglass or Lexan

and just wedge them into the container. Its helpful if its

a little loose to begin with to let the air out and then

put tighter wedges in later.

>Thanks again for all the help

>

>Adios

>Ronnie

Good luck with your cap Ronnie. With 9 in series it should

be indestructible.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla-2 list archive

Cc:

Bcc:

X-Attachments:

All,

I have finally managed to complete the 10th archive

of the Tesla-2 list. This latest archive covers

posts from January and February 1998.

I know its a bit late but Ive been terribly busy. (read lazy)

For those that asked these archive files are highly edited.

I remove all the extraneous message headers, frivolous posts

and over long signatures. I also try to correct formating

problems and spelling errors where I find them. I try to

make it easier and more enjoyable to read.

The archives may be found on Dave Bronakowski's

well organized archive site at:

http://www.velocity.net/~djb/tesla.shtml

Or on the Funet.fi ftp site in Finland maintained

by Kristian Ukkonnen at:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

Many thanks to Dave and Kristian for making the space available

to us all.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: flat plate design quiestion

Date: Thu, 23 Apr 1998 20:36:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Michael Baumann" <baumann@proton.llumc.edu>

Date: Thu, 23 Apr 1998 13:36:22 -0700

Subject: RE: flat plate design quiestion

Well.. it depends on how you intend to sandwitch the plates. Personally, I used

3/8" Lexan. I compressed with a vise, then drilled and tapped #6 holes in

the edges I then cut a 1/4" wide strip across to hold the sections together.

Be aware that poly expands as it absorbs oil, so allow some edge space.

Check out http://www.geocities.com/CapeCanaveral/Lauchpad/3574

I have some pics of my caps on the baby coil page.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical

Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

> -----Original Message-----

> From: Tesla-2 [mailto:tesla-2@emachine.com]

> Sent: Thursday, April 23, 1998 6:51 AM

> To: baumann@proton.llumc.edu

> Subject: flat plate design quiestion

>

>

> From: rpittman@juno.com

> Date: Thu, 23 Apr 1998 09:51:17 -0400

> Subject: flat plate design quiestion

>

> Mike and all,

>         I have FINALLY finished cutting 980 squares of poly and over 220

> aluminum plates for my 9 caps.  I am now in the process of stadcking

> them.  I found it was much easier to make a 3 sided box out of cardboard

> cut to the exact size of the poly to lay the sheets of poly and aluminum

> in to keep them from sliding around while stacking.

>         Anyway, my questions is how do you clamp them together?  I have

> heard of using zip ties but I am sure that would cause them to warp.  Do

> you usually just wedge them in a container and fill them with oil?

>

> Thanks again for all the help

>

> Adios

> Ronnie

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: What type of wire?

Cc:

Bcc:

X-Attachments:

At 07:26 PM 4/23/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Thu, 23 Apr 1998 15:26:49 -0400

>Subject: What type of wire?

>

>

>     Ok I'm almost done with my coil.

>

>

>     Specs so far:

>

>     22" x 4.5" Secondary, #24 wire (done and mounted)

>     14 turns, .25" Flat Primary (done and mounted)

>     6 Saltwater Caps (6 more too for 2 neons) (done)

>

>     RQ Spark Gap (3/4" Pipe, 10 pcs) (working on it now)

>     Toroid chokes (searching for toroids)

>     Safety gap (working on it now)

>

>     Input: 16kv/20ma Neon (and another one for once I get it going)

>

>     What type of wiring can I use just to get sparking? I have 12 guage

>     solid copper house wire, but this stuff is only rated to 400V.

>

>     Can I use it until I can afford to get something better or is this

>     dangerous?

>

>     Thanks

>     -tom

Tom,

Your coil sounds great and should be a good performer.

Start thinking about a poly cap to replace those bottles.

Once you get some sparks you will want longer ones for sure!

For the wiring from 120V AC to your neon(s) use wire adequate

for the expected current flow. This will be several amps

and the current will add up quick when you start paralleling

neons. Make sure this is properly fused. Dont run your coil

without a fuse!

From the neons to the safety gap, RF chokes and main gap

you can use almost anything. Small guage stranded wire is OK

but remember that the insulation is rather poor on this

type of wire. Not a problem if you have adequate airspace

around the wire to prevent arcing. Ive used PVC insulated

AWG 18 stranded wire for this without any problems.

If arcing is going to be a problem then use some heavily insulated

wire like neon sign wire. I can get scraps of this wire free

from my local neon shop. (I spend more time in their dumpster

than they do)

You can also use COPPER automotive ignition wire. Make sure its copper

wire not the more common resistance wire.

From the main gap to the capacitor and primary coil think

big. Heavy solid copper wire is best. You need something flexible

for the primary tap so use a heavy stranded wire but keep it short.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: bombarder

Cc:

Bcc:

X-Attachments:

At 01:51 AM 3/22/98 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Sat, 21 Mar 1998 20:51:06 -0500

>Subject: bombarder

>

>Hi all

>

>anyone know where I can get one of those bombarders that Mike described..

>or something similar.

They are pretty uncommon Chris. You would only find

one at a neon shop that does its own tubes and those

are getting pretty rare these days. The bombarder is

very expensive. Over $1000. As a result Ive heard that some

neon shops buy a pole pig to use as a bombarder.

I know a guy that got a bombarder cheap when a neon shop

in his town went out of business.

>I would have paid  the 100$ for one without a second thought.

You and me both.

>

>I would like to find the guy who had that one.

>

>anyone with any info please contact me off list at  cwatkins@seidata.com

>thanks

>Chris Watkins

>

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Acrylic and Polyethelene

Cc:

Bcc:

X-Attachments:

At 04:31 AM 4/24/98 GMT, you wrote:

>From: "Scott Hayward" <smash@eisa.net.au>

>Date: Fri, 24 Apr 1998 14:31:03 +1000

>Subject: Acrylic and Polyethelene

>

>To anyone on the list,

>>To anyone on the list,

>>Is Acrylic the same as Perspex (spelling?)  as I know it is the same as

>>Plexiglass.  And if you use an Acrylic coil form do you have to seal it?

I think acrylic is one of those generic names that could

apply to several different plastics.

Plexiglass (Perspex) aka Lucite is polymethyl methacrylate

or PMMA. These would make good secondary forms but have

a tendency to be brittle. No sealing necessary.

>>

>>Thanks for your feedback on the last couple of questions.  It has been most

>>helpful.

>

>I forgot to add.  I can get polyethelene for the coil form and would I have

>to seal it before winding the wire on.  If I do I'll probably just go for

>the PVC.

Polyethylene is the ultimate low loss coil form. Go for it.

No sealing necessary.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: What type of wire?

Cc:

Bcc:

X-Attachments:

At 12:18 PM 4/24/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Fri, 24 Apr 1998 08:18:09 -0400

>Subject: Re: What type of wire?

>

>

>     >From the main gap to the capacitor and primary coil think

>     >big. Heavy solid copper wire is best. You need something flexible for

>     >the primary tap so use a heavy stranded wire but keep it short.

>

>     What would you consider heavy in solid copper wire?

>     What size stranded wire should I use?

>

>     Is AWG 12 solid copper heavy enough?

>

>     Thanks!

>     -tom

>

>     This weekend!! I hope!!

Awg 12 will be fine to start. It will work OK at low

power levels. You can use what you have for now to get your coil

operating and sparking. Then later when you start wanting

to optimize for better efficiency replace the wiring

with something heavier.

For my tank circuit wiring I use solid AWG 6 copper.

For the flexible primary tap lead I use a 14 inch length

of AWG 8 stranded wire.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Question about primary loss

Cc:

Bcc:

X-Attachments:

At 12:35 PM 4/24/98 GMT, you wrote:

>From: <Tom.Stock@gsbsc.gensig.com>

>Date: Fri, 24 Apr 1998 08:35:25 -0400

>Subject: Question about primary loss

>

>

>

>     Ok so it just occured to me that I may have made a mistake when I

>     built my primary. I attached the primary to a 1" thick piece of

>     particle board (because I had it). The rails that hold the primary in

>     shape are polyethylene, and hold the coil about 1" off the particle

>     board table (flat spiral).

>

>     Knowing how much moisture gets absorbed by particle board, I think I

>     may have just reduced the output of my (unfinished) coil.

>

>     Tell me it aint so....

>

>     Well, I'm going to run it anyway. It's just one more thing to look

>     forward to...I'll build a poly cap, replace the table, and BIGGER

>     HOTTER SPARKS will be my reward.

>

>     I'll just let it sit in the back yard in the sun for an hour or so and

>     let the neighbors get a good look at it....while it looses some

>     moisture.

>

>     I just HAVE to see some sparks from this thing soon!!!!!

>

>     -tom

>

>

Tom,

I dont think its anything to worry about. If you want

you can slap a coat of varnish on it. I just dont think

its going to be a problem.

The coil form for my primary is built from wood. The primary

is left over from an earlier coil and I built the form

from wood because thats all I had available at the time.

This primary has worked very well for me despite being

built on wood. Thats why its on its 3rd coil.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tar in NST's

Cc:

Bcc:

X-Attachments:

At 03:58 PM 4/24/98 +1000, you wrote:

>Why is tar in the NST's in the first place and if I took it out and made a

>plastic container for it and immersed it in mineral oil would that be better

>for it's lifespan and current adjustment etc.

>

>From Scott Hayward    smash@eisa.net.au

>

Scott,

Tar is a low cost potting compound that works fairly

well for the intended use for a NST. Thats lighting

a neon tube. A transformer may last for years in that

kind of duty. Neons are made to be cheap.

Removing the tar and replacing the insulation with

something better will definitely give you a better

transformer. But do not attempt to unpot a working

neon. The chances of damaging a winding are pretty good.

Even with experience it only takes one slip to gouge

out a hole in one of the delicate windings. No loss

if the transformer was dead to begin with.

You dont have to put them in oil. You can insulate

them with some heavy poly. I cut some heavy HDPE

from a plastic bottle and use it to insulate some

of my rebuilds. You can then fire them dry. Saves

the mess of all that oil.

Use your good neons as is and unpot them after they fail.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 24 Apr 1998 09:27:07 -0400

Subject: Re: flat plate design quiestion

X-Mailer: IPAD 2.0

From: rpittman@juno.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>I read something at the archive that said closed cell foam wedged

>between the plate stacks and box walls will keep it all compressed

>tightly together without deforming the stack.  The closed cell foam will

>not bleed air in to any dielectric oil you might fill the container

>with.  Also orienting the plates vertically lets any air trapped in the

>oil escape much more easily.

>

Thanks alot guys for all the help.

Kevin you talked about using nylon bolts to compress them, did you

mean across the stack and then use the zip ties to pull the bolts together?

Roy do you know were you can find this type of foam easily and cheaply?

Thanks again

Adios

Ronnie

Comments: Authenticated sender is <alfred.skrocki@cybernetworking.com>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 25 Apr 1998 15:34:46 +0500

Subject: Re: Tar in NST's

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 24 Apr 1998 15:58:46 +1000 In Tesla-2

Scott Hayward <smash@eisa.net.au> wrote;

> Why is tar in the NST's in the first place and if I took it out and made a

> plastic container for it and immersed it in mineral oil would that be better

> for it's lifespan and current adjustment etc.

Hi Scott, the reason "tar" is used in neon sign transformers is that they

have to be able to work in some rather harsh environments like outside in

freezing temperatures and also sitting aand backing in the hot sun all day!

This is just too much to ask of mineral oil. Yes you are right and many of

us unpot our neon sign transformers and submerge them in mineral oil, this

greatly extends the life of the transformer.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Comments: Authenticated sender is <alfred.skrocki@cybernetworking.com>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 25 Apr 1998 15:34:47 +0500

Subject: Re: Tar in NST's

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 24 Apr 1998 22:45:55 -0400 In Tesla-2

Josh Bedell <moleking@concentric.net> wrote;

> ...However, having destroyed several neons myself while attempting to unpot

> them, I reccomend that if the trannie works, leave well enough alone.

> Especially if it's the only one you've got.

Josh, how are you unpotting your transformers to destroy them? The standard

procedure is to freeze the transformer then crack out the pitch by hitting

it with a hammer in a few choice places. I prefer to open the top and bake

the little buggers in a 350 degree F. oven and pour out most of the pitch,

then soak out that remaining in several scrubs with a tooth brush and

solvent.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Date: Sat, 25 Apr 1998 14:50:33 -0700 (PDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: GOOD GRADE OF LDPE

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello All -

I just got off the phone with Ron, a tech rep for Carlisle

Plastics. They manufacture the construction grade poly sheeting which is

stocked at my local Home Depot. I called to ask him if their product in the

10 foot by 25 foot roll was, in fact, polyethylene, since the label just

read "Plastic Sheeting." (Their 100 foot rolls are labeled "Clear

Polyethylene.") You may find what he told me of interest. He said that the

10'X 25' roll was "consumer grade," rather than "construction grade," and

that consumer grade was a higher grade, with more "virgin" material, as

opposed to more recycled mat'l in construction grade. This was good news to

me, as I really didn't want to pop for 100 ft of mat'l when the 25 foot roll

will be plenty of mat'l for my flat plate cap. So - I get the high grade

stuff, AND spend less money! Good way to start the day.  TTYL

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- Talk is cheap... the supply far exceeds the demand! ---

<danmurphy@1stnetusa.com>

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

X-Mailer: IPAD 2.0

Date: Sat, 25 Apr 1998 21:03:49 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: testing neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi ! I'm just getting started with my Telsa coil. I got a free

12K 30 ma trans from a sign shop and bought another for. $30.00.

The free one draws a 1" spark to the case on one end, but nothing

on the other. Both ends read 8.5K -+ 500 ohms. I don't understand.

The $30.00 trans draws 1" sparks from both ends. $30.00 well spent.

Anyway when I and if I get both going how should I connect them and

what will be the advantage ? Or should I just start with one ?

I bought a 4.5 OD PVC pipe. What is the normal wire gauge for that

Diameter ?

I also bought 10 copper pipe fittings for the spark gap. They are 3/4"

ID pipe sweat fittings 1.5" long. Will theses work for a spark gap?

No cutting .29 each.

Thanks to all

Jim

X-Mailer: IPAD 2.0

Date: Sat, 25 Apr 1998 20:29:42 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: testing neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:03 PM 4/25/98 -0400, you wrote:

>Hi ! I'm just getting started with my Telsa coil. I got a free

>12K 30 ma trans from a sign shop and bought another for. $30.00.

Thats good. $30 for a 30ma neon isnt a great deal but its

a typical price. The free ones are always the best even if you

have to rebuild them.

>The free one draws a 1" spark to the case on one end, but nothing

>on the other. Both ends read 8.5K -+ 500 ohms. I don't understand.

Well what you have is the classic failure mode of neons. The side

that no longer sparks is bad. What has happened is that some of

the tar potting compound has carbonized and provided a discharge

path for that secondary shorting it out.

The good news is that you can unpot this transformer and restore

it to operation. Check the list archives for instructions or download

the neons.txt file from Dave Bronakowski's archive site.

The URL is: http://www.velocity.net/~djb/tesla.shtml

>

>The $30.00 trans draws 1" sparks from both ends. $30.00 well spent.

>

>Anyway when I and if I get both going how should I connect them and

>what will be the advantage ? Or should I just start with one ?

Start with one. Size your capacitor to match just one. You will

want around .0066 mfd. Any value from .005-.0075 will probably

work fine.

When you get a second 12kv/30ma transformer simply parallel them.

Check for proper phasing. One way you will get 60 ma the other

way you will get nearly zero.

Then build a second identical cap to the first then parallel

them to increase your capacitance to make most effective use

of your higher power.

>

>I bought a 4.5 OD PVC pipe. What is the normal wire gauge for that

>Diameter ?

AWG 24 is preferred although AWG 22 or 26 could be used.

Go for around 20 inches of AWG 24.

>

>I also bought 10 copper pipe fittings for the spark gap. They are 3/4"

>ID pipe sweat fittings 1.5" long. Will theses work for a spark gap?

Yep should work fine. Somewhat longer electrodes would handle

more power by sinking more heat but what you have should be fine

below 1KVA. Provide plenty of cooling airflow.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 25 Apr 1998 23:48:48 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: testing neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, Do you ever sleep?

Thanks

Jim

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 00:23:44 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: testing neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:48 PM 4/25/98 -0400, you wrote:

>Mike, Do you ever sleep?

>

>Thanks

>

>Jim

Well Ive tried but just cant seem to get

the hang of it. But I keep trying.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 01:33:39 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: GOOD GRADE OF LDPE

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:50 PM 4/25/98 -0700, you wrote:

>Hello All -

>          I just got off the phone with Ron, a tech rep for Carlisle

>Plastics. They manufacture the construction grade poly sheeting which is

>stocked at my local Home Depot. I called to ask him if their product in the

>10 foot by 25 foot roll was, in fact, polyethylene, since the label just

>read "Plastic Sheeting." (Their 100 foot rolls are labeled "Clear

>Polyethylene.") You may find what he told me of interest. He said that the

>10'X 25' roll was "consumer grade," rather than "construction grade," and

>that consumer grade was a higher grade, with more "virgin" material, as

>opposed to more recycled mat'l in construction grade. This was good news to

>me, as I really didn't want to pop for 100 ft of mat'l when the 25 foot roll

>will be plenty of mat'l for my flat plate cap. So - I get the high grade

>stuff, AND spend less money! Good way to start the day.  TTYL

>

Dan,

Ive used both grades with equal success in my caps.

The key is multiple thin sheets for each layer

and series connected caps.

I got a big 2500 sq foot roll of 6 mil construction

grade pretty cheap ($20). Ive used it to make several

quite successful caps. Both flat plate and rolled.

One thing you have to do is inspect the sheets for

serious flaws and imbedded grit.

TTYL!

Mike Hammer

mhammer@misslink.net

Comments: Authenticated sender is <alfred.skrocki@cybernetworking.com>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 26 Apr 1998 17:00:08 +0500

Subject: Re: Acrylic and Polyethelene

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 24 Apr 1998 04:31:03 GMT In tesla-2

Scott Hayward <smash@eisa.net.au> wrote;

> >To anyone on the list,

> >Is Acrylic the same as Perspex (spelling?)  as I know it is the same as

> >Plexiglass.

Acrylic, Plexiglas, Lucite, and Perspex are all the same. The chemical name

for them is polymethyl methacrylate.

> > And if you use an Acrylic coil form do you have to seal it?

Since polymethyl methacrylate is not particularly hygroscopic it isn't

necessary to seal it.

> I forgot to add.  I can get polyethelene for the coil form and would I have

> to seal it before winding the wire on.  If I do I'll probably just go for

> the PVC.

I personaly would opt for the polyethylene since it has a much lower power

factor than polymethyl methacrylate, which means it will waste less energy

heating up the plastic form thus giving you more energy to make bigger

sparks with. {8-)

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: "Steve Falco" <sfalco@worldnet.att.net>,

"Tesla-2" <tesla-2@emachine.com>

Subject: Measuring Capacitance: Methods

Date: Sun, 26 Apr 1998 12:23:58 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

*********************************************************

FAQ regarding METHODS AND MEANS FOR MEASURING CAPACITANCE

*********************************************************

File this away for future reference.

This FAQ has been prepared by Fr. Tom McGahee

tom_mcgahee@sigmais.com

for use by the Tesla List and the Tesla-2 List. Copy freely.

From time to time List members ask questions about how to measure

capacitance. This FAQ attempts to show several methods. The info is

drawn from a number of sources, including past posts by Steve Falco

sfalco@worldnet.att.net and myself.

The easiest method is to buy a meter that measures capacitance. You

can sometimes find a Digital Multi Meter (DMM) that includes the ability

to measure capacitance. Make sure that it covers the range that you are

interested in. Most coilers would want a meter that can measure from a

few picofarads to at least 1 microfarad.

An LCR meter is a meter specifically designed to measure Inductance,

Capacitance, and Resistance. In general these meters cover a greater range

than the DMMs and also have a higher accuracy. They also generally cost

more.

Some companies sell meters that ONLY measure capacitance. An LCR meter

is generally more useful to a coiler.

You can often find such meters at hamfests and swap meets, and at very

reasonable prices. When I go looking for such meters I always carry

an assortment of batteries and capacitors of known value so that I can

test the meters before buying. BTW, a working LCR meter and/or DMM is

a good thing to bring with you to hamfests to test components before

you buy them!

Companies like Radio Shack and B&K Precision sell reasonably priced

meters that will measure capacitances in the ranges of interest

to us coilers.

For those who do not own a capacitance meter there are several

methods available that will allow you to measure capacitance. Some of

these methods are those used in professional capacitance meters, and

others are simply el-cheapo techniques that a coiler on a limited

budget can use to quickly find the approximate value of their

homemade capacitor.

1) You can measure the RC time constant.

T=R*C

Where: T is the time in seconds required to charge to 63%

of the battery voltage, and

R is resistance in ohms, and

C is capacitance in farads.

Note that if you measure resistance in megohms, then the capacitance

will come out in microfarads.

Re-arrange the formula so that

C=T/R

charge your cap through a resistor, and note how long it takes the

voltage to build from 0 volts to 63% of the battery voltage.

Use the formula C=T/R to determine the capacitance.

The circuit might look like this:

----------- Switch ------ Resistor --------------------

|                                        |            |

Battery                                 capacitor      voltmeter

|                                        |            |

-------------------------------------------------------

Note that the meter used should have as high an impedance as possible

to avoid loading down the circuit. In most cases you would probably be

using a 10 Megohm DMM. You can increase the effective input impedance of

the meter by using an operational amplifier connected as a Buffer

in front of the meter. You can use an LF356 for this purpose.

Start with the switch open, and the voltmeter showing 0 volts (short out

the cap for a few seconds to discharge it.  Then close the switch, start

your stopwatch, and wait till the voltmeter hits battery voltage x 0.63.

Sadly, this only works well for large capacitors like electrolytics,

because you can do the timing with a watch.  But for small capacitors

such as are used in tesla coils, you need to use a storage oscilloscope

or electronic timer/counter to measure the time 'cause it will be in

milliseconds or even microseconds.

And don't think you can get around this by making the resistor large,

like maybe a few hundred megohms, because then the leakage in your

voltmeter or o'scope will bias the readings.  (The resistor must be no

more than say 10% of the impedence of your test equipment.)

2) You can improve on method (1) by charging the capacitor with a

constant current. In this case

C=I*T

Where: C is in farads

I is in amps, and

T is time in seconds to charge to 1 volt.

From a practical point of view we normally want to measure microfarads,

so we would use microamps.

This FAQ will not give details of the constant current source and

means for measuring time, but the author (Fr. Tom McGahee) published

an article in Popular Electronics, October 1976, that gives full

construction details and theory of operation of a unit suitable for

use by coilers. This unit used an analog meter for the output, but

a digital meter can be substituted for greater accuracy. Since the

article was published, much better operational amplifiers have become

readily available. I recommend substuting the 741 op amp with

an LF356 or other pin-compatible device.

A method for measuring capacitance digitally using constant current

charging was published by the author (Fr. Tom Mcgahee) in the January 23,

1975 issue of Electronics magazine, page 103. Basically a window

comparator is used to produce a pulse whose width is proportional

to capacitance. This pulse is then used to gate an oscillator's signal

into a counter. The counter will display the result digitally. I have

attained useful accuracies of more than .01% with this method.

A further method that is based on constant current charging is to create

a triangle wave generator using an operational amplifier connected as

an integrator and driving an RS Flip Flop triggered by two different

voltage levels from the output of the integrator. If the output of the

Flip Flop in turn controls the input current(s) to the integrator, then the

PERIOD of the output of the Flip Flop will be *directly* proportional

to the capacitance. The author has used the above technique to design

automatic digital capacitance meters that provide updated readings

hundreds of times per second, with an accuracy of better than .01%.

3) You can also build a simple 555 timer astable oscillator.

You can find such circuits in many hobby electronics books. You

measure the frequency of oscillation with your home-made capacitor,

then substitute known capacitors in the circuit until you get about

the same frequency.  Then your capacitor and the known capacitor are

roughly the same value.  This can be pretty accurate, and works

for the values used in Tesla coils, but you need a means to

measure or compare the frequencies. The resistor in series with the

capacitor can be made variable, and then you can calibrate the

potentiometer position in terms of capacitance. That is better

than having to match the capacitor exactly, but you do have

to go through a calibration procedure. Potentiometer position

versus cap value for a given frequency may be slightly non-linear.

You can also measure the frequency of the known and unknown

capacitance and determine capacitance based on the fact that the

lower the capacitance, the higher the resulting frequency. I

find it easier to measure the ratio of the PERIODs, as the period

is directly proportional to the capacitance. This method is accurate

to about 1%.

4) The "Guide to Electronic Measurements and Laboratory Practice"

(Stanley Wolf, Prentice-Hall, 1973) says the most accurate way is with a

bridge circuit (until the advent of digital measurement techniques):

--------------------------------

|                  |             |

|                 R1             R2

|                  |             |

|                  |             |

AC generator            |--- meter ---|

|                  |             |

|                  |             |

|                 R3            R?

|                  |             |

|                 C3            C?

|                  |             |

--------------------------------

The equations are not too bad:

R? = (R2 x R3) / R1

C? = C3 x (R1 / R2)

where R1, R2, R3, C3 are permanent parts of the bridge meter, and R? and

C? are your unknown capacitor.  (That R? is confusing, but it represents

the loss or leakage in your capacitor.  If you have a good capacitor, it

will be pretty close to 0.)

Anyway, we make R1 and R3 adjustable potentiometers, and play around

with them until the meter says we have balanced the bridge.  Then we

disconnect the potentiometers and measure the values they were set to

with our ohm-meter, do a little math, and we know our capacitor value.

For best results try to get a C3 with a value very near what you THINK the

value of <C?> is going to be. To further improve the accuracy of the

circuit, you can choose R1 and R2 to be resistors whose resistance is close

to the capacitive reactance of C3. These improvements affect the practical

*resolution* of the answer for <C?>. You will get a greater final accuracy,

because the ranges have been matched.

Also, you can leave out <R3> and <R?> in most cases. Use of R3 can cause

the experimenter to miscalculate <C?> when using just a meter. (meter

should be high impedance or it will only be about 10% accurate). If you

really want to measure the dissipation/leakage, then you need a measurement

of the relative PHASE difference between the two legs of the bridge. With a

differential oscilloscope you adjust R1 and R3 until the scope shows only a

dot in the xy mode. At that point you have all the data to calculate both

capacitance and leakage. (This assumes that the reference capacitor has

little or no leakage of its OWN, of course!)

It is much less hassle to just leave <R3> out and ignore the leakage value,

as it is not all that important to us coilers.

5) Cheapest way: you can make a voltage divider:

|------------------------------

|                              |

|                            resistor

|                              |

120 volt AC (wall plug)                   |

|                              |

|                            capacitor

|                              |

|------------------------------

Measure the voltage across the resistor and the capacitor.  Don't get

zapped - you are playing with line voltages here and there is not much

between you and eternity.

C = (Vr / R) / (2 x pi x f x Vc)

where Vr is the voltage across the resistor, R is the resistor in ohms,

pi is 3.141, f is 60 Hertz (or 50 in some countries - you know who you

are), and Vc is the voltage across the capacitor.

Please note that you have to actually measure Vr and Vc. You can

*not* assume that Vr+Vc=120 since there is a current phase shift

through the capacitor.

By using the existing 120 VAC line voltage we let the power company

supply us with a rock-solid 60Hz low impedance voltage source. For

free!

So there you are: measure two voltages, do a little math, and get a

result accurate to maybe 10%.  You need a fairly high impedence meter to

read the voltages. I recommend a DMM with a 10 Megohm impedance

if at all possible. Lower impedance meters will introduce HUGE

errors when the capacitance is small.

Pick a resistor such that Vr is somewhere around 50 volts for reasonable

accuracy.  You don't want Vr to be 1 volt or 119 volts - get it somewhere in the

middle by chosing a suitable value for R.

When done making the measurements, remember to always DISCHARGE the

capacitor, as it MAY have a large voltage stored in it!

6) There is one more way that uses even simpler math. It uses two capacitors

instead of a capacitor and a resistor.

|------------------------------

|                              |

|                            Known Capacitor

120 volt AC (wall plug)                   |

|                              |

|                            Unknown Capacitor

|                              |

|------------------------------

This method makes use of the fact that the smaller the capacitor value, the

greater the proportion of total voltage a given capacitor will drop across

itself.

It helps if you choose a known value that is close to the one you want to

measure!

CK=Known Capacitor  CU=Unknown Capacitor  EK= Voltage across Known

Capacitor

EU=Voltage across Unknown Capacitor

Formula is based on the fact that EK*CK=EU*CU  Rearranging it in terms of

CU we get:

CU=(EK*CK)/EU

Always use a high impedance meter, such as a digital unit or a tube VTVM to

keep errors low. As in method (5), measure each voltage with the meter.

Do not assume that the total will be 120 volts. Among other things, the

actual voltage deliverd at your wallplug may be anything from 105 to 125

volts. Or even 210 to 240 for some of our European coilers.

When done making the measurements, remember to always DISCHARGE *each* of

the capacitors separately (NOT in series), to ensure that each gets fully

discharged. If you remove the caps when the voltage is at its peak, you

can have some nasty voltages present!

Several of the techniques outlined above involve using the voltage

from your wall outlet. PLEASE BE CAREFUL!

If substituting a signal generator for your voltage source, be aware that

the impedance of the signal generator may introduce significant errors.

7) I can think of at least one more method, which is based on the fact that

the larger a capacitor is, the greater the amount of current it will

conduct *at a given frequency*. This method assumes that the applied

frequency is a SINE WAVE, well below the frequency limit of your digital

meter, and that your digital meter has an AC current range. Some don't!

For a *given* sine wave frequency  C1/I1=C2/I2 so we re-arrange and get:

C1=C2/(I1*I2)

The frequency chosen is arbitrary, but I like something between 1KHz and

about 1/2 the frequency limit of my meter. The higher the frequency you

use, the smaller the capacitance you can accurately measure.

Again, try to use as a reference a value that is close to what you are

measuring. That keeps the resolution of the answer higher. Assume you want

to measure a capacitor that you think is pretty close to .01 mfd.  First

connect a *sine wave* generator in series with the known capacitor and an AC

current meter. Adjust the output of the generator to about ten volts, and

then put the meter on the scale that gives you the most resolution of the

current value. You may have to play with the frequency to find one that

works well, but stay below the frequency limit of your meter! Then, leaving

ALL things exactly the same, exchange capacitors and use the reference

capacitor. Record the new reading and apply the math to find the unknown

capacitance. Note that if your signal generator has a large impedance, then

you will have a large error unless the two capacitors are close in value...

then the error is effectively cancelled.

I don't recommend D'arsonval meters because they are not linear with

frequency. A digital meter works best. The value of the internal current

shunt will add less than .1% error to your final calculated value, provided

you made your measurement with the scale that yielded the largest number of

significant digits on the readout. In other words 1.234 is better than

01.23  or 001.2

With careful attention to detail you can get a value within 5% or better.

Hope this helps.

Fr. Tom McGahee

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 11:30:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: new list software

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

As you may noticed the Tesla-2 list is running under new software.

As Ralph mentioned in his post this was necessary as the old list

running on his home machine was overloading the mail system at

his ISP. The new listserv is running on a server located at his

ISP so is now online 24/7. Posts will be sent out to the list

much faster now and this should take care of the dropped message

problem.

The change should be mostly transparent to the subscribers. The only

thing you probably have noticed is the sender on all the posts is

the original sender rather than Tesla-2. I would prefer it the old

way but unfortunately thats not an option.

If you have a mail program with filtering capability like Eudora

its a simple matter to filter the incoming mail into its own mailbox.

The procedure for unsubscribing from the list has changed so everyone

should make note of it for future reference. The new software allows

you to temporarily suspend your subscription for vacations and similar

circumstances.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 26 Apr 1998 11:00:16 -0700 (PDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: Arc Stabilizer available

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey Everyone -

I saw this at a local pawn shop yesterday: Sears Hi Frequency

Arc Stabilizer, prim volts: 115 VAC, Sec. volts: 3500.  He wants $150 for

it. I have no use for it myself, plus don't have the $$. If anybody wants

it, let me know.

(E-mail direct to me)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- Talk is cheap... the supply far exceeds the demand! ---

<danmurphy@1stnetusa.com>

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Date: Sun, 26 Apr 1998 11:08:05 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: mhammer@misslink.net, danmurphy@1stnetusa.com

Subject: Latest CKT/TEXT:W/M Project

Sender: tesl-err@emachine.com

Mike & Dan,

You'll note the (maybe) addition Volt & Current meters to the lashup;

how handy will these be? located & spec'd OK? My sense is that the I

meter may be of least value as it should only read some low responding

average value, depending upon the particular momentary activity at the

main (G3) gap?

Mike, Does the new T-2 List configuration allow us any more leeway for

the xmission of binary files? Or would that just result in more general

system clutter?

---------------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 14:29:19 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Wintesla

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, When I run the Wintesls program, I see 2 frequencies

for the secondary on the right side of the chart. Which one

do I use to try and match the primary.

Jim

Date: Sun, 26 Apr 1998 13:51:20 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Neon Transformers & HV Wire

Sender: tesl-err@emachine.com

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your ZIP CODE to

determine shipping cost.

Thanks,

Tedd

"CONVENTIONAL" neon transformers, not old, not rusty, good

working condition.  They are not labeled, the ratings are from my

testing/measuring.  Good for Tesla coil, Jacob's ladder, HV cap charging

(with proper rectification), "hi-potting", experiments, etc.

$18 used  15kv30ma  in=porc ins  out=porc insul (1 loose HV insul)

$18 used  12kv30ma  in=porc ins   out=porc insul

$16 used  9kv60ma  in=porc ins   out=porc insul (no lid)

$10 used  7.5kv60ma  in=porc ins  out=porc ins (miss 1 HV ins, other chipped)

$10 used  6kv60ma  in=porc ins   out=porc insul

$11 used  7.5kv30ma  in=AC cord  out=2 wires, w/switch

$9  used  6kv30ma  in=porc ins   out=porc insul

$7  used  3?kv30ma  in=porc ins   out=porc insul

NEW Neon transformer HV wire:

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $.50

WOOD CHOKE FORMS, might be good for winding chokes to protect neon

transformer on small Tesla coil.  Approximate dimensions in inches:

2.25 length, 1.25 diameter, 4 notches (each .3 wide, .25 deep), .4

diameter hole thru length of form.  6 available @ $1.50 each.

Date: Sun, 26 Apr 1998 14:20:07 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnetos

Sender: tesl-err@emachine.com

Has anyone experimented with magneto generated Hv pulses? I think that a

magneto generated pulse (rotating magnetic field and stationary step-up

x-former) could provide the energy necessary to power a Tesla coil

without the attendant mechanical difficulties of a rotary spark gap.

Frequency would be dependent on rpm of drive motor. E and I would be

dependent on x-former design and magnetic field strength.

Does anyone know the voltage/current relationship of a simple lawnmower

magneto output? Could this design form the basis for a Tesla coil?

Just a thought.......

--

Regards,

Scott MacGregor EES,Co.

-

Comments: Authenticated sender is <alfred.skrocki@cybernetworking.com>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 27 Apr 1998 00:01:41 +0500

Subject: Re: Magnetos

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sun, 26 Apr 1998 14:20:07 -0700 In Tesla-2

Scott MacGregor <smacgregor@aasp.net> wrote;

> Has anyone experimented with magneto generated Hv pulses? I think that a

> magneto generated pulse (rotating magnetic field and stationary step-up

> x-former) could provide the energy necessary to power a Tesla coil

> without the attendant mechanical difficulties of a rotary spark gap.

Yes Scott, you could use a magneto to provide the HV to power a small Tesla

coil, BUT you will still need a spark gap!

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Date: Sun, 26 Apr 1998 13:28:22 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Measuring Capacitance Methods - Fr. T. MaGahee

Sender: tesl-err@emachine.com

All,

Just printed Fr. McGahee's excellent article, which leads me to some

questions; minor points of bugment:

1) Is 'x mfd = x ufd" in our common parlance? Could this be a point of

confusion?  How should (do) we express '.00x f' vs. '.000 00x f' ?

2) How sinusoidal is the usual Tesla tank output, given the usual mains ac input?

When we speak of "(AC)volts", in (especially) measurement modes (seeming

not to realise that measurement instruments may vary from user to user),

we may incur significant discrepancies which may be as great as 41% more

or 39% less to the listener than to the speaker.

The common instruments used are:

CRO's (vPk, vPk-Pk)

DVM's (vPk)

True RMS meters (vRMS)

Analog (D'Arsenvall) (vAvg)

The folowing relationships are offered to newbies, as a caution, with

the proviso that they are only valid for sinusoidal measurements; wave

form distortions knock them into a cocked hat!

RMS = .707 x Pk

Avg = .637 x Pk

Pk  = 1.414 x RMS

RMS = 1.11 x Avg

Pk  = 1.57 x Avg

Avg = .9 x RMS

If distortions enter the picture, then our frame of reference changes

significantly: FFT measurement means are required and V^2/Hz is our

unit.

3) Given that full Tesla outputs generally would rather jump past any

instrumentation (probably frying them in the process), do very low ac

input levels allow 'peaking' of a system, which would translate to full

power operation?

---------------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 18:59:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wintesla

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:29 PM 4/26/98 -0400, you wrote:

>Hi all, When I run the Wintesla program, I see 2 frequencies

>for the secondary on the right side of the chart. Which one

>do I use to try and match the primary.

>

>Jim

Jim,

The top frequency is the unloaded (bare) frequency

of your secondary. The 2nd frequency is the expected

resonant frequency with a discharge terminal. Remember

that adding a toroid or other form of discharge terminal

will lower your resonant frequency by adding capacitance

to the coil.

You should use the lower frequency for matching to your primary

however its best to have a couple extra turns in the primary

because Tesla coils dont always (more like never) turn out like

the math predicts.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 21:37:05 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Wintesla

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:59 PM 4/26/98 -0500, you wrote:

>At 02:29 PM 4/26/98 -0400, you wrote:

>

Mike,

Here is the design Wintesla calls for, do they look OK ?

12K 30ma Trans

Primary =  .250 tube .250 spacing 18 turns, 20 degrees  211.91 Hz.

Cap .005uf     Start diam. 5.5 inches

total diam 23"

Secondary 4.50 Diameter 24 Gauge wire Length 24 inches res freq 211.91

H/D aspect 5.33

Jim

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: chrisw@kellnet.com (Chris Wendorff)

Subject: Variac

Date: Sun, 26 Apr 98 22:22:25 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone have a variac for sale or know where I could find a good deal on

one.

Thanks,

Chris Wendorff

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Binary files

Date: Mon, 27 Apr 1998 03:18:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

GW>Mike, Does the new T-2 List configuration allow us any more leeway for

GW>the xmission of binary files? Or would that just result in more general

GW>system clutter?

Hi!

I'm checking on that now...

Ralph

---

Evolution stops when stupidity is no longer fatal.

X-Mailer: IPAD 2.0

Date: Sun, 26 Apr 1998 20:52:53 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: MIKE D <gordrew@home.com>

Subject: Mineral Spirits?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Is mineral sprits the same as mineral oil?

--Mike Drew

Date: Sun, 26 Apr 1998 22:56:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Mineral Spirits?

Sender: tesl-err@emachine.com

MIKE D wrote:

>

>

>

> Is mineral sprits the same as mineral oil?

>

> --Mike Drew

Mike,

NO! Mineral oil is much "thicker" and is not flammable unless heated to

several hundred degrees. Don't use mineral spirits in your caps - while

it might have adequate electrical strength, it is much too flammable to

use!

-- Bert --

Date: Sun, 26 Apr 1998 21:19:16 -0700 (PDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: Re: Mineral Spirits?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:52 PM 4/26/98 -0700, you wrote:

>

>Is mineral sprits the same as mineral oil? --Mike Drew

No, Mike - Mineral spirits are what you use to clean your paint brushes

with.  It's a volatile solvent, and I wouldn't recommend it for your

capacitor, unless you get a license from the BATF for manufacturing a

class 3 explosive device!

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- Talk is cheap... the supply far exceeds the demand! ---

<danmurphy@1stnetusa.com>

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

X-Mailer: IPAD 2.0

Date: Mon, 27 Apr 1998 00:02:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wintesla

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:37 PM 4/26/98 -0400, you wrote:

>At 06:59 PM 4/26/98 -0500, you wrote:

>>At 02:29 PM 4/26/98 -0400, you wrote:

>>Mike,

>Here is the design Wintesla calls for, do they look OK ?

>

>12K 30ma Trans

>Primary =  .250 tube .250 spacing 18 turns, 20 degrees  211.91 Hz.

>Cap .005uf     Start diam. 5.5 inches

>total diam 23"

>

>Secondary 4.50 Diameter 24 Gauge wire Length 24 inches res freq 211.91

>H/D aspect 5.33

>

>Jim

>

Jim,

It will probably work OK but if I was building it I

would make some changes. A .0066 mfd cap will be a

better match for your 12kv/30ma neon than will .005 mfd.

This will also lessen the number of required primary

turns.

I would also shorten the secondary winding some.

With 24 inches of AWG 24 you are looking at over 1100 turns.

Best to keep that under 1000 turns. My own 4.5 inch secondary

is wound with 21 inches of AWG 24. This is right at 975 turns.

This will raise your resonant frequency somewhat also reducing

the number of required primary turns.

Just my .02

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 26 Apr 1998 22:07:38 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Mineral Spirits?

Sender: tesl-err@emachine.com

MIKE D wrote:

>

>

>

> Is mineral sprits the same as mineral oil?

>

> --Mike Drew

Mike,

Mineral oil is what you chuff down to relieve constipation and to soothe

the ravages of your unpotted NST, or making caps.

Mineral spirits are generally solvents of the kerosene order, or paint

thinner.

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm

Sender: tesl-err@emachine.com

Date: Mon, 27 Apr 1998 10:50:52 +0300

From: Abdullah M Fazaa <amfazaa@expec.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: mhammer@misslink.net

Subject: getting shocks from disconnected secondary coil

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi mike & all ......

When I 've disconnected all the Tesla components and the electricity

was also  disconnected , still I'm getting unpleasant stored

electrostatic shocks from the secondary coil even if I left the

disconnected parts of TC for two - three days. That happens whe when I'm

moving my hand slowly from top of the coil to the bottom and when I

reach the place in which the sparks comes out from the tunrs of the

secondary coil -during my TC test before disconnecting the TC parts- I

always get that stored electrostatic shock .......could anyone tell me

What is the meaning of this ..???

By the way , those in-between turns sparks of my secondary coil occur

when my coil was over coupling  , but when I rised the secondary coil

3-5" upward more and re-tune my primary , those in-between sparks

dissappeared.

Now , I'm always wearing gloves for hand protection .

thanks..

Please E-mail me on < amfazaa@expec.com >

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting shocks from disconnected secondary coil

Date: Mon, 27 Apr 1998 07:49:25 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Abdullah M Fazaa <amfazaa@expec.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Cc: mhammer@misslink.net

> Subject: getting shocks from disconnected secondary coil

> Date: Monday, April 27, 1998 3:50 AM

>

> hi mike & all ......

>

>  When I 've disconnected all the Tesla components and the electricity

> was also  disconnected , still I'm getting unpleasant stored

> electrostatic shocks from the secondary coil even if I left the

> disconnected parts of TC for two - three days. That happens whe when I'm

> moving my hand slowly from top of the coil to the bottom and when I

> reach the place in which the sparks comes out from the tunrs of the

> secondary coil -during my TC test before disconnecting the TC parts- I

> always get that stored electrostatic shock .......could anyone tell me

> What is the meaning of this ..???

>

>

> By the way , those in-between turns sparks of my secondary coil occur

> when my coil was over coupling  , but when I rised the secondary coil

> 3-5" upward more and re-tune my primary , those in-between sparks

> dissappeared.

>

> Now , I'm always wearing gloves for hand protection .

>

>

>  thanks..

>

>  Please E-mail me on < amfazaa@expec.com >

Abdullah,

The secondary and other elements are exhibiting a stored dielectric

charge. It is generally harmless, although it can be somewhat suprising

the first time you encounter it. This charge is enhanced in secondaries

that have been coated or sealed, as the coating acts as a dielectric

and stores charge.

What I do is to place my hand on the top of the coil and slowly run

my other hand up and down the coil until I am sure that I have removed

the stored charge.

Hope this helps.

Fr. Tom McGahee

Date: Mon, 27 Apr 1998 06:54:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting shocks from disconnected secondary coil

Sender: tesl-err@emachine.com

Abdullah M Fazaa wrote:

>

> hi mike & all ......

>

>  When I 've disconnected all the Tesla components and the electricity

> was also  disconnected , still I'm getting unpleasant stored

> electrostatic shocks from the secondary coil even if I left the

> disconnected parts of TC for two - three days. That happens whe when I'm

> moving my hand slowly from top of the coil to the bottom and when I

> reach the place in which the sparks comes out from the tunrs of the

> secondary coil -during my TC test before disconnecting the TC parts- I

> always get that stored electrostatic shock .......could anyone tell me

> What is the meaning of this ..???

>

> By the way , those in-between turns sparks of my secondary coil occur

> when my coil was over coupling  , but when I rised the secondary coil

> 3-5" upward more and re-tune my primary , those in-between sparks

> dissappeared.

>

> Now , I'm always wearing gloves for hand protection .

>

>  thanks..

>

>  Please E-mail me on < amfazaa@expec.com >

Abdullah,

Almost every coiler who's played with a disruptive coil runs into this

sooner or later. This will occur especially with coils having a

relatively thick coating of insulation over the secondary wire, such as

multiple coatings of polyurethane or epoxy.

The outer surface of this insulating layer becomes highly charged during

normal operation of the coil. Because the insulation is typically very

good, these charges become "trapped" within the insulating layer. If you

bring a conductive object (like the back of your hand) near, the trapped

charge can then dissipate to you, charging up your body if you're

wearing rubber-soled shoes. You then get "zapped" if you touch the

toroid, since it's grounded. You can also feel the electrostatic field

attracting the hairs on the back of your hand as you apprach the top of

the winding. The coil also acts like a big dust magnet (sort of like an

electrostatic precipitator). After a while the top of the coil will

become coated with a very fine layer of attracted dust from the air.

BTW, the trapped charges can be a problem when you go to move the coil -

simply picking up the secondary coil can transfer substantial charge to

you - if you then accidently touch the toroid to your forehead.... ZAP!

It's been known to happen to many a coiler (your's truly included! :^)).

-- Bert --

From: Hollmike <Hollmike@aol.com>

Date: Mon, 27 Apr 1998 08:10:11 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Mineral Spirits?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Mineral spirits is a solvent for removing varnish, laquer, etc.  It is not

the same as mineral oil.

Mike Hollingsworth

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 27 Apr 1998 08:51:49 -0400

Subject: First Spark!!

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok I've now become an official coiler! Over the weekend, I built my

spark gap, and wired up my coil. I built a polyethylene box for the

bottle caps, and hammered some pipes in the back yard.

I took alot of shortcuts (ie: no chokes, scrap wire, no fan on RQ

spark gap, etc) but ended up with a constant 12" of spark with a

single 15KV neon!

This thing is a blast. I didn't expect it to be so loud even though

everyone else says this after first spark too.

I had alot of fun laying a flourescent light on a metal shelf 5 feet

away and watching it light.

If anyone wants a couple of pictures, let me know, I scanned some last

night.

-tom

Date: Mon, 27 Apr 1998 12:54:57 +0000

From: JamesMereness <mereness@dti.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Source for pre-made cap

Sender: tesl-err@emachine.com

I have an opportunity to feature my coil in a show this weekend, but

first need to upgrade my 10KV 0.02uf to at least 15KV so I can use my

new 9000V 120mA NST.  Not enough time to experiment with home-builts;

does anyone know of a source that would sell me a 0.02uf 15KV (or

higher)?

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Measuring Capacitance Methods - Fr. T. MaGahee

Date: Mon, 27 Apr 1998 10:31:40 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Comments interspersed within original text.

----------

> From: Gene Waldo <gwaldo@1stnetusa.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Measuring Capacitance Methods - Fr. T. MaGahee

> Date: Sunday, April 26, 1998 4:28 PM

>

> All,

> Just printed Fr. McGahee's excellent article, which  leads me to some

> questions; minor points of bugment:

>

> 1) Is 'x mfd = x ufd" in our common parlance? Could this be a point of

> confusion?  How should (do) we express '.00x f' vs. '.000 00x f' ?

MF Mf MFD Mfd mfd ufd are all used *in practice* to denote microfarad. I agree

that it can at times be confusing. I have never seen 1,000 ufd referred

to as a millifarad *in practice*, though technically that is what it is.

pf pfd uuf uufd all refer to a picofarad, sometimes also called a "puff".

>

> 2) How sinusoidal is the usual Tesla tank output, given the usual mains

> ac input?

The *output* is quite sinusoidal. See Bert Poole's web site for Malcolm

Watt's waveforms. The waveform is decaying, of course, not steady, so

any attempt to measure it using an ac voltmeter is doomed to failure.

It is best observed on a scope.

> When we speak of "(AC)volts", in (especially) measurement modes (seeming

> not to realise that measurement instruments may vary from user to user),

> we may incur significant discrepancies which may be as great as 41% more

> or 39% less to the listener than to the speaker.

>

> The common instruments used are:

>

> CRO's (vPk, vPk-Pk)

> DVM's (vPk)

> True RMS meters (vRMS)

> Analog (D'Arsenvall) (vAvg)

>

> The folowing relationships are offered to newbies, as a caution, with

> the proviso that they are only valid for sinusoidal measurements; wave

> form distortions knock them into a cocked hat!

>

> RMS = .707 x Pk

> Avg = .637 x Pk

> Pk  = 1.414 x RMS

> RMS = 1.11 x Avg

> Pk  = 1.57 x Avg

> Avg = .9 x RMS

>

Yes, and many DVMs/DMMs are actually average-reading measuring devices

adjusted to give the "correct" answers for a pure sine wave at 1Khz.

Put in a square wave or any non-sinusoidal waveform, and you get out

wildly erroneous readings on the AC scales.

> If distortions enter the picture, then our frame of reference changes

> significantly: FFT measurement means are required and V^2/Hz is our

> unit.

>

> 3) Given that full Tesla outputs generally would rather jump past any

> instrumentation (probably frying them in the process), do very low ac

> input levels allow 'peaking' of a system, which would translate to full

> power operation?

TUNING at low power is a very useful technique. Note that you may still

require tweaking at full power since the ions hanging around the toroid

will add to the effective load capacitance, lowering the operating

frequency. The higher the Q of the circuit the more exacting the tuning

has to be to get *maximum* arc.

> ---------------------------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

> ---------------------------------------------------------------

>    What's good for the planet is more OZONE! See the machinery

>     forming up!   http://www.misslink.net/mhammer/waldo.htm

Hope this helps.

Fr. Tom McGahee

X-Mailer: IPAD 2.0

Date: Mon, 27 Apr 1998 09:45:20 -0500

From: Kevin Batchelor <kbatchelor@arslimited.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: flat plate design question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Kevin you talked about using nylon bolts to compress them,  did

>you mean across the stack and then use the zip ties to pull the

>bolts together?

>Thanks again

>Adios

>Ronnie

Ronnie:

Not sure how you constructed your caps.  In my case I used regular steel bolts at

the top of my caps as connectors from each plate.  Once my cap had settled into the

oil for about a month most of the air had come out, however since my bolts were only

at the top the cap fanned out going down.  My solution was to connect a few zip ties together around the caps and pull them down tight.  This reduced the spacing between plates and increased my performance.

Kevin

X-Mailer: IPAD 2.0

Date: Mon, 27 Apr 1998 14:01:14 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Spark!!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:51 AM 4/27/98 -0400, you wrote:

>

>

>     Ok I've now become and official coiler! Over the weekend, I built my

>     spark gap, and wired up my coil. I built a polyethylene box for the

>     bottle caps, and hammered some pipes in the back yard.

Great news Tom. Always good to hear of a new coil coming to life.

>

>     I took alot of shortcuts (ie: no chokes, scrap wire, no fan on RQ

>     spark gap, etc) but ended up with a constant 12" of spark with a

>     single 15KV neon!

>

Ok now that the initial fun is over its time to get to

work and start optimizing things. Get a safety gap across

that neon. Shorten up that tank circuit wiring and replace

with heavier conductors. Get a strong fan on that gap.

You should be able to get twice that amount of spark from your

single 15kv/30ma neon.

>     This thing is a blast. I didn't expect it to be so loud even though

>     everyone else says this after first spark too.

Invest in some hearing protection. The more spark you get

the louder it will be.

>

>     I had alot of fun laying a flourescent light on a metal shelf 5 feet

>     away and watching it light.

Try also placing an incandescent bulb in the arc channel. If you can get

the arc to travel through the bulb the filament will light. Also

large clear decorator bulbs make dandy ready made plasma globes.

Also try placing an incandescent bulb in series with your ground

lead. The bulb will light quite brightly on the ground current.

this illustrates why a good RF ground is so important.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.152.188]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Arc stabilizer available

Date: Mon, 27 Apr 1998 12:11:13 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

It was written:

>I saw this at a local pawn shop yesterday: Sears Hi Frequency

>Arc Stabilizer, prim volts: 115 VAC, Sec. volts: 3500.  He wants $150

>for it. I have no use for it myself, plus don't have the $$. If

>anybody wants it, let me know.

(sorry for posting this to both lists but I thought people might be as

curious about this device as I once was - and the original message was

also cross-posted...)

I "discovered" modern tesla coiling via some research into just this

sort of device.  Basically what you will find inside is nearly an entire

tesla primary circuit. Neon, cap, gap, and output coil ("primary").

This box still sells for something like $750 new out of the Sears

Industrial catalog.  Not a bad markup or something your average coiler

could scrounge for about $50. :)

This device is designed to superimpose a High Frequency/High Voltage

(generated by the gap/cap/primary) onto the welding current, supplied by

the welder you'd attach the box to.  Weldors (folk who weld) like this

since it eliminates having to manually "strike" the arc - the HF/HV does

that for you (by ionizing the air) once the welding rod approaches the

work.  To some extent, using this device will allow you to do a limited

amount of TIG welding using a stick welder for a power supply.  Of

course you still need an inert gas supply, etc for the TIG.

I finally did discover what type of ferrite cores to use in my homemade

coupling transformer.  The core is important not due to it's effective

permeability (the core raises the primary inductance by a factor of only

about 2).  However, the core greatly increases the coupling between the

two coils by (?) causing more of the flux to link the output coil.  In a

situation like this (I think) you want as high a coupling coefficient if

possible.  Turns ratio is 1:1 and there is no use of any resonant rise

phenomenon since output capacitance is variable and unknown.

My TIG arcstarter runs on the live half of a half-dead 12000v 30 ma

neon, using a 4-gap @ 0.028 RQ with fan, and only about 1500 pf of

capacitance (doorknobs).  The output will pierce vinyl electrical tape,

or jump over 1/2 inch under argon, which is more than sufficient for my

purposes.

If anyone has any questions feel free to contact me.  One of these days

I'll get it all on the page, but don't look there just yet!

-Bill the arcstarter - experienced amateur weldor

"I'll try to weld anything!"

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Mon, 27 Apr 1998 20:04:04 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Wintesla

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:02 AM 4/27/98 -0500, you wrote:

>At 09:37 PM 4/26/98 -0400, you wrote:

>>At 06:59 PM 4/26/98 -0500, you wrote:

>>>At 02:29 PM 4/26/98 -0400, you wrote:

>>>Mike,

>>Here is the design Wintesla calls for, do they look OK ?

>>

>>12K 30ma Trans

>>Primary =  .250 tube .250 spacing 18 turns, 20 degrees  211.91 Hz.

>>Cap .005uf     Start diam. 5.5 inches

>>total diam 23"

>>

>>Secondary 4.50 Diameter 24 Gauge wire Length 24 inches res freq 211.91

>>H/D aspect 5.33

>>

>>Jim

>>

>

>Jim,

>It will probably work OK but if I was building it I

>would make some changes. A .0066 mfd cap will be a

>better match for your 12kv/30ma neon than will .005 mfd.

>This will also lessen the number of required primary

>turns.

>

>I would also shorten the secondary winding some.

>With 24 inches of AWG 24 you are looking at over 1100 turns.

>Best to keep that under 1000 turns. My own 4.5 inch secondary

>is wound with 21 inches of AWG 24. This is right at 975 turns.

>

>This will raise your resonant frequency somewhat also reducing

>the number of required primary turns.

>

>Just my .02

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

Mike, one more question if you have the time.

If I make a tube poly Cap. .0066 do you know the length and width

if the alum. flashing and what size pvc should be used.

I found dementions for a .01, .005, .0025, and .0014.

Thanks

Jim

From: DavidH8083 <DavidH8083@aol.com>

Date: Mon, 27 Apr 1998 20:11:27 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: corr@enid.com, chip@pupman.com, mhammer@misslink.net

Subject: Buyers Warning!

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

It is with no small portion of dismay that I post the follwing warning

to the subscribers of both Tesla lists.  I have to report that in spite of the

fact that by far the vast majority of the list members are extremely generous

and faultlessly honest, I have been stung in bad deal.

A young coiler, Shaun O'Brien of Enid, Oklahoma, offered some parts for

sale over his Tesla web site.  A deal was struck via E-mail and my check sent

and cashed.  At that point Mr. O'Brien ceased all communication, refusing to

reply to my inquiries about forthcoming shipment or even to respond to the

copy of the cancelled check that I sent him to jog his memory.  All efforts to

embarrass him into fulfilling his end of the bargain have failed and I must

chalk up this deal as a loss.  The amount in question is small (less than

$100) and Mr. O'Brien knows that I have no intention of journeying from NYC to

Enid to press my claim in small claims court, but I feel that I would be

remiss if I did not at least report this incident to the other members of

these lists, so that they may avoid similar unfortunate circumstances.

My apologies for taking up list space with such a petty matter, but

I felt that this warning should be posted so that others may avoid the risk.

David Hutchison

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  T2 ADMIN - List server "features"

Date: Tue, 28 Apr 1998 06:06:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

J>Ralph ---

J>Now I'm receiving ANOTHER bounce-back post when I make a mailing to the List.

J>Plus I'm still getting the <jd2@flash.net> bounce-back.....

<super snip>

Sigh..  another interesting "feature" of the new list server rears

its ugly head. It seems that this version of list server software

manages to spoof some of the mail servers out there so that it makes

them think the proper address to send error messages to is the original

poster to the list, rather than the list server, as it should be.

The result of this "feature" is that if you post a message, and there

are any delivery errors with your message, YOU get ALL the error

messages sent back to you!  Just delete them for the time being and

forget about them.  This problem is being worked on. (Actually, this

problem would not exist if the list server could show the list name as

the sender - in that case, all the error messages would go back to the

list server.)

By the way, it is a good idea to place the letters "T2" (for Tesla-2)

in the subject line of your posts so that we can sort the Tesla-2 list

messages more easily.  We're also trying to see if it will be possible

for the list server to place the list name in the senders name position

instead of the message poster.  That would fix the problems noted above.

Ralph

---

Fatal System Error - Please insert a quarter to fix it.

Date: Mon, 27 Apr 1998 23:47:54 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Buyers Warning!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Its good to know, but I generally consider it a bad idea to send money to someone

that I dont know, there is no way of telling how honest a person is by words on

email.

DavidH8083 wrote:

>        It is with no small portion of dismay that I post the following warning

> to the subscribers of both Tesla lists.  I have to report that in spite of the

> fact that by far the vast majority of the list members are extremely generous

> and faultlessly honest, I have been stung in bad deal.

>

>        A young coiler, Shaun O'Brien of Enid, Oklahoma, offered some parts for

> sale over his Tesla web site.  A deal was struck via E-mail and my check sent

> and cashed.  At that point Mr. O'Brien ceased all communication, refusing to

> reply to my inquiries about forthcoming shipment or even to respond to the

> copy of the cancelled check that I sent him to jog his memory.  All efforts to

> embarrass him into fulfilling his end of the bargain have failed and I must

> chalk up this deal as a loss.  The amount in question is small (less than

> $100) and Mr. O'Brien knows that I have no intention of journeying from NYC to

> Enid to press my claim in small claims court, but I feel that I would be

> remiss if I did not at least report this incident to the other members of

> these lists, so that they may avoid similar unfortunate circumstances.

>

>           My apologies for taking up list space with such a petty matter, but

> I felt that this warning should be posted so that others may avoid the risk.

>

> David Hutchison

> davidh8083@aol.com

Date: Tue, 28 Apr 1998 00:20:25 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Buyers Warning!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

mike nicholas wrote:

> Its good to know, but I generally consider it a bad idea to send money to someone

> that I dont know, there is no way of telling how honest a person is by words on

> email. If you want revenge email the people of his town that are online(easy to

> find out)and send them email to what he has done, or if you want to be really dirty

> get a hotmail adress in his name and send some nasty messages to people in his

> town, revenge can be a little childish, but it does send a strong message!

>

> DavidH8083 wrote:

>

> >        It is with no small portion of dismay that I post the follwing warning

> > to the subscribers of both Tesla lists.  I have to report that in spite of the

> > fact that by far the vast majority of the list members are extremely generous

> > and faultlessly honest, I have been stung in bad deal.

> >

> >        A young coiler, Shaun O'Brien of Enid, Oklahoma, offered some parts for

> > sale over his Tesla web site.  A deal was struck via E-mail and my check sent

> > and cashed.  At that point Mr. O'Brien ceased all communication, refusing to

> > reply to my inquiries about forthcoming shipment or even to respond to the

> > copy of the cancelled check that I sent him to jog his memory.  All efforts to

> > embarrass him into fulfilling his end of the bargain have failed and I must

> > chalk up this deal as a loss.  The amount in question is small (less than

> > $100) and Mr. O'Brien knows that I have no intention of journeying from NYC to

> > Enid to press my claim in small claims court, but I feel that I would be

> > remiss if I did not at least report this incident to the other members of

> > these lists, so that they may avoid similar unfortunate circumstances.

> >

> >           My apologies for taking up list space with such a petty matter, but

> > I felt that this warning should be posted so that others may avoid the risk.

> >

> > David Hutchison

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: yubba <yubba@clara.net>

Subject: secondary coil form

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Date: Wed, 29 Apr 98 00:33:03 GMT

Sender: tesl-err@emachine.com

Hi all

I have almost finished my coil. caps done, primary wound, RQ gap finished.

I was about to start sealing my 4" pvc pipe ready for winding when I spotted

some 7" polyethylene gas pipe at my college. I asked the workmen and they said

ok take it. What are you going to use it for? making lightning, OH that's ok

then. Must keep up the mad scientist image. I digress. My question is should I

use this as a secondary coil form instead of the 4" pvc I was planning to use?

Apart from adding more turns to my primary and having a bigger top load is there

anything else I need to change?

I will be using a 9kV 50 mA neon, 0.0177uF caps, 7 section 1"1/2 dia 2" long RQ

gap 22AWG wire. 3/8 dia primary. 240 volt 50 Hz supply. Any comments welcome.

bob golding lightning without rain

yubba@clara.net

X-Mailer: IPAD 2.0

Date: Tue, 28 Apr 1998 18:46:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: secondary coil form

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:43 PM 4/28/98 -0700, you wrote:

>Hi all

>	I have almost finished my coil. caps done, primary wound, RQ gap finished.

>I was about to start sealing my 4" pvc pipe ready for winding when I

>spotted some 7" polyethylene gas pipe at my college. I asked the workmen

>and they said ok take it. What are you going to use it for? making

>lightning, OH that's ok then. Must keep up the mad scientist image. I

>digress. My question is should I use this as a secondary coil form instead

>of the 4" pvc I was planning to use? Apart from adding more turns to my

>primary and having a bigger top load is there anything else I need to change?

>I will be using a 9kV 50 mA neon, 0.0177uF caps, 7 section 1"1/2 dia 2"

>long RQ gap 22AWG wire. 3/8 dia primary. 240 volt 50 Hz supply. Any

>comments welcome.

>

Bob,

Go ahead and build the 4 incher as you had planned. Save that

polyethylene pipe for a second coil. A 7 inch secondary will

be more difficult to get running and will require more power

and space. You primary will have to be enlarged. You will need

to increase the inside diameter to prevent overcoupling and you

will need quite a few more turns to account for the lower

resonant frequency.

Dont sell a 4 incher short. Mine puts out 54 inch streamers

and I know of another 4 inch coil putting out 5 foot streamers.

As far as sealing the PVC goes what you want to do is bake the pipe

at a low temperature (130 F) for about 30-45 minutes to drive out

the moisture. Then seal it with polyurethane varnish while still

warm. The varnish will be drawn into the pores of the PVC as it cools.

Put two coats on the outside of the form and one coat inside.

One its dry wind on your winding then place about 6 fairly heavy

coats over the wire. You want the coating thick enough that you

can no longer feel the wire. The surface should be perfectly smooth.

While you can build a 4 incher with AWG 22 the optimum wire size

is AWG 24. Ive built one of each - 4.5 X 22 with AWG 22

4.5 X 21 with AWG 24. I get better performance from the secondary

with AWG 24.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 28 Apr 1998 20:47:32 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Moleking <moleking@concentric.net>

Subject: Looking for RQ Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Folks,

I'm getting ready to bump my power up from 15kv 30ma to 90 ma via my

recently acquired 15kv 60 ma neon.  Since I'll be upgrading other

components of my coil in the process, I figure it's time to look into a

Richard Quick cylindrical gap.  I'm currently using an L-bracket style

series gap with 4 adjustable gaps.  I've been getting decent results, but

as those bolts heat up, output plummets.

The thing is, I don't really feel like building a new gap...I've barely got

enough time these days to do ANYTHING w/ the coil, and it seems like a RQ

gap would take a while.

Anyone interested in selling one?  Please respond off-list.  Thanks

Josh Bedell

From: Hollmike <Hollmike@aol.com>

Date: Wed, 29 Apr 1998 00:52:39 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: secondary coil form

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bob,

I would wind both coils if I were you.  The 9kV, 50ma neon is good for the

4 inch coil, but might be a little too small for the 7 inch.   I use two 15kV,

30 ma neons for a 5.5 inch secondary and now want more power just for it.

Mike Hollingsworth

Sender: tesl-err@emachine.com

Date: Wed, 29 Apr 1998 07:59:20 +0300

From: Abdullah M Fazaa <amfazaa@expec.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: amfazaa@expec.com

Subject: spark into beam

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI ALL

Have anybody of you thought to convert that huge energy -in which it

comes out as a spark form from the Tesla secondary coil- into a straight

, directable , concentrated beam..... ???

THINK ABOUT IT FOR A WHILE .......

NIKOLA TESLA IS A MAN OUT OF HIS TIME

What is a variac .... and what is its function.. and what does it do to

Tesla coil ...??

abdo the little engineer

< amfazaa@expec.com >

Date: Wed, 29 Apr 1998 01:35:18 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

cc: amfazaa@expec.com

Subject: Re: spark into beam

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 29 Apr 1998, Abdullah M Fazaa wrote:

> HI ALL

>

> Have anybody of you thought to convert that huge energy -in which it

> comes out as a spark form from the Tesla secondary coil- into a straight

> , directable , concentrated beam..... ???

>

He he...  What do you think a spark is?

>

> NIKOLA TESLA IS A MAN OUT OF HIS TIME

>

Ugh, great, just what the world needs...  Another tesla worshipper.

Wireless power transmission was, and still is a BAD IDEA.  It's just too

inefficient.  Tesla couldn't hold on to a dollar if was sewed to his

forehead, either.  I'm tired of people that idolize Nikola Tesla as some

sort of god, glossing over his failings.  He was a brilliant scientist,

true - BUT - wireless power transmission is a fallacy.  The idea never has

worked well, even in the 60s experimentation was done with microwave power

transmission, which was MORE efficient than tesla's design ever COULD have

been, the power loss was still unacceptable.  Now I suppose you're going

to start discussing the suppression of free energy devices by a

close-minded scientific community?

>

> What is a variac .... and what is its function.. and what does it do to

> Tesla coil ...??

>

A variac is a variable transformer, it allows the input voltage into the

neon sign transformer to be varied, thus varying the output of the coil.

TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

X-Mailer: IPAD 2.0

Date: Tue, 28 Apr 1998 23:43:50 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: secondary coil form

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all

I have almost finished my coil. caps done, primary wound, RQ gap finished.

I was about to start sealing my 4" pvc pipe ready for winding when I

spotted some 7" polyethylene gas pipe at my college. I asked the workmen

and they said ok take it. What are you going to use it for? making

lightning, OH that's ok then. Must keep up the mad scientist image. I

digress. My question is should I use this as a secondary coil form instead

of the 4" pvc I was planning to use? Apart from adding more turns to my

primary and having a bigger top load is there anything else I need to change?

I will be using a 9kV 50 mA neon, 0.0177uF caps, 7 section 1"1/2 dia 2"

long RQ gap 22AWG wire. 3/8 dia primary. 240 volt 50 Hz supply. Any

comments welcome.

Date: Wed, 29 Apr 1998 01:06:45 -0600 (MDT)

From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: spark into beam

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 29 Apr 1998, Paul Anderson wrote:

> On Wed, 29 Apr 1998, Abdullah M Fazaa wrote:

> Ugh, great, just what the world needs...  Another tesla worshipper.

> Wireless power transmission was, and still is a BAD IDEA.  It's just too

> inefficient.  Tesla couldn't hold on to a dollar if was sewed to his

It's much less a matter of efficiency, and much more

a matter of economics.

As far as Telsa's personal money problems, well, after

he went cold-turkey on gambling, his penchant for destroying

electric plants and ordering expensive custom transformers from

New York to C-springs probably had some bearing on the matter.

I think that the janitor of the C-springs lab even

tried to take electronic parts in lieu of deliquent back salary

owed from telsa, and the overall bills were > 200,000 bucks.

> forehead, either.  I'm tired of people that idolize Nikola Tesla as some

> sort of god, glossing over his failings.  He was a brilliant scientist,

> true - BUT - wireless power transmission is a fallacy.  The idea never has

> worked well, even in the 60s experimentation was done with microwave power

> transmission, which was MORE efficient than tesla's design ever COULD have

> been, the power loss was still unacceptable.  Now I suppose you're going

> to start discussing the suppression of free energy devices by a

> close-minded scientific community?

>

> >

> > What is a variac .... and what is its function.. and what does it do to

> > Tesla coil ...??

> >

> A variac is a variable transformer, it allows the input voltage into the

> neon sign transformer to be varied, thus varying the output of the coil.

> TTYL!

>

>

> ---

> Paul Anderson

> madhobby@geeky1.ebtech.net

From: <Tom.Stock@gsbsc.gensig.com>

Date: Wed, 29 Apr 1998 08:22:19 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: spark into beam

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> NIKOLA TESLA IS A MAN OUT OF HIS TIME

>

>Ugh, great, just what the world needs...  Another tesla worshipper.

Yeah, actually, the way I sort of feel about Tesla. is that he just liked

playing with his coils because of the large "wow" factor.  I think he

wasted most of his invention potential just making bigger version of the

same fairly useless lightning and noise generator, hoping something

significant would happen. He should have just looked for a market for

ozone <grin>!

Now I keep seeing things claiming tesla responsible for a 3 mile wide

crater somewhere...using some sort of radio wave interference. I think

this is wishfull thinking.

-tom

X-Mailer: IPAD 2.0 X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 ADMIN - List server

Date: Wed, 29 Apr 1998 08:11:56 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Ralph.

Sorry for wasting your time...

Just a quick note to let you know that some AOL people on tesla-2 have

apparently blocked access from HOTMAIL (where my email comes from).

So - these three guys don't get my posts to tesla-2!

Other AOL users can receive my mail w/o problems.

I know that the problem is being worked on.  Just thought I'd confirm

this particular manifestation of the issue!

Thanks! (response is optional)

Bill (the arcstarter)

>

>J>Ralph ---

>

>J>Now I'm receiving ANOTHER bounce-back post when I make a mailing to the List.

>

>J>Plus I'm still getting the <jd2@flash.net> bounce-back.....

>

><super snip>

>

>   Sigh..  another interesting "feature" of the new list server rears

>its ugly head. It seems that this version  of list server software

>manages to spoof some of the mail servers out there so that it makes

>them think the proper address to send error messages to is the original

>poster to the list, rather than the list server, as it should be.

>

>   The result of this "feature" is that if you post a message, and there

>are any delivery errors with your message, YOU get ALL the error

>messages sent back to you!  Just delete them for the time being and

>forget about them.  This problem is being worked on. (Actually, this

>problem would not exist if the list server could show the list name as

>the sender - in that case, all the error messages would go back to the

>list server.)

>

>   By the way, it is a good idea to place the letters "T2" (for Tesla-2)

>in the subject line of your posts so that we can sort the Tesla-2 list

>messages more easily.  We're also trying to see if it will be possible

>for the list server to place the list name in the senders name position

>instead of the message poster.  That would fix the problems noted above.

>

>   Ralph

>---

>  Fatal System Error - Please insert a quarter to fix it.

>

Full-Name: Bert Wikkerink

Date: Wed, 29 Apr 1998 12:10:32 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 ADMIN - List server

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

I am also filtering out mail from hotmail.com, except in a few instances,

yours being one of them. I basically filter out all messages from

hotmail.com, but allow anything from the Tesla-2 list. I have a really great

perl script running on our server at the place where I work. Anyone on the

list who wants a copy of this script can find it at

http://www.nags.org/index.html

Bert

to reply to me personally, please remove the 'Z' from my reply to address.

Bill the arcstarter wrote:

> Hi Ralph.

>  Sorry for wasting your time...

>

>  Just a quick note to let you know that some AOL people on tesla-2 have

> apparently blocked access from HOTMAIL (where my email comes from).

>

> So - these three guys don't get my posts to tesla-2!

>

> Other AOL users can receive my mail w/o problems.

>

> I know that the problem is being worked on.  Just thought I'd confirm

> this particular manifestation of the issue!

>

> Thanks! (response is optional)

>

> Bill (the arcstarter)

>

Date: Wed, 29 Apr 1998 11:40:23 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 ground rods

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Whats better two 6' ground rods or one 12' ground rod?

X-Mailer: IPAD 2.0

Date: Wed, 29 Apr 1998 11:23:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2 ground rods

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:40 AM 4/29/98 -0700, you wrote:

>

>

>Whats better two 6' ground rods or one 12' ground rod?

>

Mike,

Two 6 footers would be better. Three 4 footers would be

better still. A lot depends on your soil conditions.

In general you cant overdo your RF ground. More is always

better.

I have extensive ground systems beneath my amateur radio

antennas. Ive buried literally miles of wire in the ground

in order to achieve the lowest possible ground impedance.

Tesla coils dont need that extensive a ground system but

it must be capable of handling rather high currents with

minimal loss. The more power you run the better your ground

needs to be.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 29 Apr 1998 16:46:49 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: spark into beam

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 29 Apr 1998 Tom.Stock@gsbsc.gensig.com wrote:

> He should have just looked for a market for

> ozone <grin>!

>

He probably could have actually made some decent money charging people to

see his coils - I know a 6 foot arc sure draws people's attention these days.

> Now I keep seeing things claiming tesla responsible for a 3 mile wide

> crater somewhere...using some sort of radio wave interference. I think

> this is wishfull thinking.

>

More like distilled BS.  For a laugh, check out http://www.svpvril.com -

this is the home of the Keelyanites.  John Keely was a twit and a

swindler, and not a very good one at it, either.  How Nikola Tesla got

grouped in with the free energy freaks, I'll never know.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Wed, 29 Apr 1998 19:42:28 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 ADMIN - List server

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 29 Apr 1998, Bert Wikkerink wrote:

>

> I am also filtering out mail from hotmail.com, except in a few instances,

> yours being one of them.

>

Why?  Hotmail is a good service, and they are taking legal action against

spammers that forge hotmail addresses in their headers.  The best thing

you can do is forward the whole message to abuse@hotmail.com and request

that the person's service be revoked.  Hotmail is very active against

spammers.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: T2 ADMIN - List server

Date: Thu, 30 Apr 1998 03:38:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Just a quick note to let you know that some AOL people on tesla-2 have

>apparently blocked access from HOTMAIL (where my email comes from).

>So - these three guys don't get my posts to tesla-2!

>Other AOL users can receive my mail w/o problems.

>I know that the problem is being worked on.  Just thought I'd confirm

>this particular manifestation of the issue!

Yes, this was traced to a few sites blocking email coming from sites

which were NOT really Hotmail or AOL servers, but reported that the

message they were sending was coming from a subscriber located at

Hotmail or AOL.

IOW, a subscriber at AOL or Hotmail posts to the list.  The listserver

sends the post through the server at SSTAR.COM, which, correctly puts in

the list traffic that the original message was posted from a subscriber

located at Hotmail or AOL, but that the list message is being sent from

SSTAR.COM.

However, due to getting lots of spam messages, the administrator of

the receiving system has set his system to block any email sent from

AOL or Hotmail addresses, if the message did not come directly from the

Hotmail or AOL servers.  As a result, ANY message posted to ANY mailing

list not on the AOL or Hotmail servers, and which does not put a list

name in the FROM field will be bounced.

The solution is to contact the administrator of the system refusing

the mail and request that email from SSTAR.COM be allowed through.  That

fixes the problem.  John Souvestre, administrator of SSTAR.COM, the ISP

for the list, has already contacted Hotmail and AOL, so those sites

should be accepting mail from the list already or very soon.

Ralph

---

Fine: a tax for doing wrong..Tax: a fine for doing right

Date: Wed, 29 Apr 1998 22:14:00 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re:t2 rf ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I took your advise Mike, I sunk 3- 6' copper rods in the ground instead of

two, I will connect them with 3/8" refrigeration tubing to the base of my coil,

My specs. up to date:

NST: 15Kv 60ma

gap: air blast but will upgrade to rq gap later

safety gap: none

chokes: none

secondary:10 3/4" 680 turns #20 wire

primary: 20 turns 3/8" soft copper ref. tubing

toroid: 21" sewer pipe with foil tape

cap: 2-0.01 mfd crazy carpet caps in series for now

ground: 3-6' 1/2" copper in ground

expected spark length: unknown

A few questions first:

How long of streamers should I expect?

Can I use a toroidal magnatron magnet as a choke?

How much rf interferance will this thing put out?

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 01:08:47 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:t2 rf ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:14 PM 4/29/98 -0700, you wrote:

>   I took your advise Mike, I sunk 3- 6' copper rods in the ground instead of

>two, I will connect them with 3/8" refrigeration tubing to the base of my

coil,

Sounds like a great RF ground to me.

>

>My specs. up to date:

>

>NST: 15Kv 60ma

>gap: air blast but will upgrade to rq gap later

>safety gap: none

>chokes: none

Dont neglect the chokes and especially the safety gap.

You can destroy your neon in seconds. It needs all the

protection you can give it.

>secondary:10 3/4" 680 turns #20 wire

>primary: 20 turns 3/8" soft copper ref. tubing

>toroid: 21" sewer pipe with foil tape

>cap: 2-0.01 mfd crazy carpet caps in series for now

Two .01 mfd caps in series equals .005 mfd. Thats

a little small for a 15kv/60ma neon. You need a pair

of .02 mfd caps in series to equal .01 mfd.

>ground: 3-6' 1/2" copper in ground

>expected spark length: unknown

>

>

>A few questions first:

>

>How long of streamers should I expect?

Hard to say. The power input and cap is a little on the low side

for a 10 inch coil. What you have would be better suited for

a 4 or 6 inch diameter secondary.

15kv/60ma is 900VA and at that power level I was able to get

40 inch streamers from my 4.5 X 21 inch secondary.

>Can I use a toroidal magnatron magnet as a choke?

No. The material isnt suitable.

>How much rf interferance will this thing put out?

Could be a great deal but over a rather short range.

The RF chokes will help by keeping the RF in the tank circuit.

Long leads in the tank circuit act like antennas and tend to radiate

signals. Keep those leads short. An AC line filter on the AC input

to your neon will help keep RF out of the AC mains.

TTYL!

Mike Hammer

mhammer@misslink.net

To:	"Tesla-2" <tesla-2@emachine.com>

Date:	Thu, 30 Apr 1998 04:54:09 -0700

From:	"Leben Lang und Frei" <fleischwulf@mailexcite.com>

X-Sent-Mail: off

X-Mailer: IPAD 2.0

Subject: Increased power & voltage 4.5" coil

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Date: Thu, 30 Apr 1998 06:54:58 -0500

From: fleischwulf@mailexcite.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Coilers,

I have a question regarding power input and increased voltage. So far I have been

able to reach a total spark length of 60"(most of the time it runs 52" to 54"). This

was accomplished using a pole pig at 14.4 Kv (total power input 3 kw). I now can

reach a voltage of 19.2kv. My caps will handle 25kv safetly. I am wondering what

to expect under the following conditions:

Input voltage: 19,200

power input: 4.5 to 6 kw.

capacitance: .026 uf.

Gap air quenched: .4 - .45

Paul M.

---

Einigkeit und Freiheit

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 11:26:06 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Increased power & voltage 4.5" coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:54 AM 4/30/98 -0700, you wrote:

>Coilers,

>  I have a question regarding power input and increased voltage.

>So far I have been able to reach a total spark length of 60"

>(most of the time it runs 52" to 54"). This was accomplished using

>a pole pig at 14.4 Kv (total power input 3 kw). I now can reach a

>voltage of 19.2kv. My caps will handle 25kv safely. I am wondering what

>to expect under the following conditions:

>

>Input voltage: 19,200

>power input: 4.5 to 6 kw.

>capacitance: .026 uf.

>Gap air quenched: .4 - .45

>

>Paul M.

Paul,

I believe you are at the point where even substantial increases

in power level will yield little improvement in spark output.

I am in the same situation. I get 54 inches of spark with my 4

inch coil run 12kv/100ma. If I add another 12kv/30ma in parallel

with this I get no apparent increase in spark length. If I add

a 12kv/60ma in parallel I get maybe another inch.

I just think we are both at a point where more power just

isnt going to help matters. We have to face the fact that

our 4 inch secondaries are pretty much maxed out.

The only way I see for the extra power to be of any benefit is if

you also substantially increase your toroid size.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 20:54:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: t2 rf ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:24 PM 4/30/98 -0700, you wrote:

>

>

>Mike Hammer wrote:

>

>> At 10:14 PM 4/29/98 -0700, you wrote:

>> >I took your advise Mike, I sunk 3- 6' copper rods in the ground instead of

>> >two, I will connect them with 3/8" refrigeration tubing to the base of my

>> coil,

>>

>> Sounds like a great RF ground to me.

>>

>> >

>> >My specs. up to date:

>> >

>> >NST: 15Kv 60ma

>> >gap: air blast but will upgrade to rq gap later

>> >safety gap: none

>> >chokes: none

>>

>> Dont neglect the chokes and especially the safety gap.

>> You can destroy your neon in seconds. It needs all the

>> protection you can give it.

>>

>

>What specs should I have for my safety gap?

The safety need not be anything elaborate. I use a couple

bolts. You can place a grounded center post between the

gap electrodes if you want. This want the arc overs go

directly to ground. Space the electrodes far enough apart

so its just beyond breakdown using just your neon alone.

This will be slightly wider than your main gap.

Gene Waldo made a pretty good and functional safety gap.

A picture of it can be found on my web page.

The URL is: http://www.misslink.net/mhammer/waldo.htm

>What guage wire should I use for my chokes?

I recommend rather small guage wire. AWG 26 or 28.

350-400 turns of this on 2 inch PVC will give plenty

of inductance.

>If I use air core chokes what size resistor should I use to eliminate self

>resonance?

This is not at all critical. Anything from 300-1500 ohms.

The power rating should be high enough so they dont overheat.

25 watts is usually more than sufficient.

<SNIP>

>

>Whats a good cheap source of choke cores, can I use AM antenna cores?

If you mean ferrite rods for loopstick antennas the answer is yes.

Make sure they are big enough. You will want about 100 turns

of AWG 26 or 28. Small ferrite cores may tend to saturate.

Last year at the Peoria, Il hamfest I saw a guy with

a box full of 3/4 inch diameter ferrite rods 6 inches

long for $1 each. And like an idiot I never bought any.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.149.163]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: How I set my safety gaps

Date: Thu, 30 Apr 1998 19:22:41 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I've not seen any instructions on how to set your safety gaps - but for

what it's worth - here is how I do it:

You need:

*Safety gap

*Neon/pig/MOT or whatever

*Variac

*Nothing else

My safety gap is really nasty looking - made out of a couple of steel

hardware store "L" brackets with a nut brazed onto one with an eye-screw

through the nut - slovenly glued onto an old plastic IC carrier tube!

(Dan's/Gene's stuff on mhammer's page is really slick - much better than

anything I'm likely to produce!)

Anyway - the point is that the gap is adjustable (and lockable) via the

threads on the eyescrew.

I wire the gap across the transformer, and adjust it so it will *just*

fire once the variac is adjusted for nominal input voltage.  I then lock

it down and thats where I leave it!  I figure this way - if the voltage

is even barely higher then the neon usually puts out - it'll fire the

gap.  Better safe than sorry.

I think that using the variac (as opposed to just flipping a power

switch) during this process is important, as it eliminates any turn-on

surges which might lead you to a wrong setting...

I also run with series R and chokes when I'm actually powering something

(like an arc stabilizer, induction heater, or (on rare occasions) a

tesla coil...)

One neat thing with this setup - you can really see the resonant rise

effect when you place a cap across the output of a neon.  Example -

placing a 500pf doorknob across my neon caused the safety gap to fire

when I was feeding the primary only 70 VAC instead of 120 VAC, etc...

Comments welcome from everyone except coilers blocking HOTMAIL :)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Thu, 30 Apr 1998 20:41:33 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: How I set my safety gaps

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

>  I've not seen any instructions on how to set your safety gaps - but for

> what it's worth - here is how I do it:

>

> You need:

> *Safety gap

> *Neon/pig/MOT or whatever

> *Variac

> *Nothing else

>

> My safety gap is really nasty looking - made out of a couple of steel

> hardware store "L" brackets with a nut brazed onto one with an eye-screw

> through the nut - slovenly glued onto an old plastic IC carrier tube!

> (Dan's/Gene's stuff on mhammer's page is really slick -

>

> Example - placing a 500pf doorknob across my neon caused the safety gap to fire...

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

Bill,

What the hell is a doorknob?  I've seen this term bandied about, and

figured, sit back, Waldo, it'll come to you; well, it hasn't!

THANKS for the strokes! ~~<:))))  New stuff on our (Mike's) page...

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 23:02:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: T2 what is a doorknob?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

>> -Bill the arcstarter

>> Starting arcs in Cinci, OH

>> http://www.geocities.com/CapeCanaveral/Hangar/6160

>One neat thing with this setup - you can really see the resonant rise

>effect when you place a cap across the output of a neon.  Example -

>placing a 500pf doorknob across my neon caused the safety gap to fire

>when I was feeding the primary only 70 VAC instead of 120 VAC, etc...

>

>

>Bill,

>What the hell is a doorknob?  I've seen this term bandied about, and

>figured, sit back, Waldo, it'll come to you; well, it hasn't!

>

>THANKS for the strokes! ~~<:))))  New stuff on our (Mike's) page...

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

A doorknob is a capacitor shaped like... well... a doorknob.

They are usually ceramic dielectric and are not very well

suited as Tesla coil tank caps. Some coilers use them as

bypass caps on their neons to bypass the RF to ground.

I do not recommend such procedures for the exact reason Bill

stated above. They can cause unwanted resonances within the neon

and lead to a quick death for your neon.

If anyone insists on using bypass caps make them small.

On the order of 150 pf. Using larger ones will sometimes

lead to problems.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 21:18:51 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: MIKE D <gordrew@home.com>

Subject: Variac

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Is it absolutly nessasary to have a variac to run a 12kv/30ma transformer?

--Mike Drew

Date: Thu, 30 Apr 1998 21:24:47 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: t2 rf ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

> At 10:14 PM 4/29/98 -0700, you wrote:

> >   I took your advise Mike, I sunk 3- 6' copper rods in the ground instead of

> >two, I will connect them with 3/8" refrigeration tubing to the base of my

> coil,

>

> Sounds like a great RF ground to me.

>

> >

> >My specs. up to date:

> >

> >NST: 15Kv 60ma

> >gap: air blast but will upgrade to rq gap later

> >safety gap: none

> >chokes: none

>

> Dont neglect the chokes and especially the safety gap.

> You can destroy your neon in seconds. It needs all the

> protection you can give it.

>

What specks should I have for my safety gap?What guage wire should I use for my

chokes?

If I use air core chokes what size resistor should I use to eliminate self

resonance?

> >secondary:10 3/4" 680 turns #20 wire

> >primary: 20 turns 3/8" soft copper ref. tubing

> >toroid: 21" sewer pipe with foil tape

> >cap: 2-0.01 mfd crazy carpet caps in series for now

>

> Two .01 mfd caps in series equals .005 mfd. Thats

> a little small for a 15kv/60ma neon. You need a pair

> of .02 mfd caps in series to equal .01 mfd.

>

I have two more 0.01mfd available

> >ground: 3-6' 1/2" copper in ground

> >expected spark length: unknown

> >

> >

> >A few questions first:

> >

> >How long of streamers should I expect?

>

> Hard to say. The power input and cap is a little on the low side

> for a 10 inch coil. What you have would be better suited for

> a 4 or 6 inch diameter secondary.

> I do have a 5" secondary wound, I guess it would be a good time to test it

> 15kv/60ma is 900VA and at that power level I was able to get

> 40 inch streamers from my 4.5 X 21 inch secondary.

>

> >Can I use a toroidal magnatron magnet as a choke?

>

> No. The material isnt suitable.

>

> >How much rf interferance will this thing put out?

>

> Could be a great deal but over a rather short range.

> The RF chokes will help by keeping the RF in the tank circuit.

> Long leads in the tank circuit act like antennas and tend to radiate

> signals. Keep those leads short. An AC line filter on the AC input

> to your neon will help keep RF out of the AC mains.

>

Whats a good cheap source of choke cores, can I use AM antena cores?

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 30 Apr 1998 23:26:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Variac

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:18 PM 4/30/98 -0700, you wrote:

>

>

>Is it absolutly nessasary to have a variac to run a 12kv/30ma transformer?

>

>--Mike Drew

Nope. Not at all. However once you start

paralleling neons for more power you will

want to acquire a variac to bring the power

level up slowly. Saves wear and tear on your

caps and transformers.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 30 Apr 1998 22:04:43 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: danmurphy@1stnetusa.com

Subject: T2- Spec the best?

Sender: tesl-err@emachine.com

All, Mike, et al:

Seems one of my taberna amigos is a lifelong commercial electrician; his

son is in NEON, in Chicago or some other place.. we chat....  I mention

transformers and like that there... he says, "What do you need?"

Guess what.... I tell him!  Then he asks really some trouble-making

questions: (kinda: how big?) We were talking NPS, and then I drifted

into 'PolePigs', and then it really fell apart! He wanted specs! Like

I'm supposed to know?!

As for NPS: I guessed at 15kV, 120 ma; ??? what's good? it'll cost me

shipping.

As for PolePigs, ... 'IF' we ever get to that point (120vac avail.

only??!), what do we want? His buddies in the poleyards need to know!

I'm two or three scotches into this and this guy is banging on my

shoulder, telling me how dumb I am!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery

forming up!   http://www.misslink.net/mhammer/waldo.htm