From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF chokes

Date: Sun,  2 Nov 1997 03:25:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sat, 1 Nov 1997 21:25:35 -0600

Subject: Re: RF chokes

>The best approach is to wind some chokes on ferrite cores

>of some kind. Ferrite toroids are good as well as ferrite rods.

Ferrite toroids?  Like those found in small appliance step-down xfrmrs?  And

if it's a closed toroid, isn't it rather difficult to wind so many turns?

How about just a chunk of half inch steel rod?  Would it perform similarly

to a "ferrite" core?

And by "resistors wired in series". . . do you mean one on each end of the

winding with one terminal hooked to the winding and the other to the

input/output?

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF chokes

Cc:

Bcc:

X-Attachments:

At 03:25 AM 11/2/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sat, 1 Nov 1997 21:25:35 -0600

>Subject: Re: RF chokes

>

>

>>The best approach is to wind some chokes on ferrite cores

>>of some kind. Ferrite toroids are good as well as ferrite rods.

>

>Ferrite toroids?  Like those found in small appliance step-down xfrmrs?

Those would be rather small. The ones I used were 2 1/2 inches

in diameter. The smaller ones wont hold enough turns to make

any significant amount of inductance.

>And if it's a closed toroid, isn't it rather difficult to wind so many turns?

Cut a stick a few inches long. Like a popsicle stick.

Cut a notch in each end and wind the length of wire you need

for your choke lengthwise on this stick. As you wind the wire

on the toroid pass this stick shuttle through the center of

the toroid.

>How about just a chunk of half inch steel rod?  Would it perform similarly

>to a "ferrite" core?

No. It would have low inductance compared to ferrites and

would saturate very easily. Very poor choice.

>

>And by "resistors wired in series". . . do you mean one on each end of the

>winding with one terminal hooked to the winding and the other to the

>input/output?

>

You need one resistor connected in series with each choke.

The purpose of the resistor is to kill the Q of the choke

thus preventing any oscillations from occurring.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Changes in resonate frequency

Cc:

Bcc:

X-Attachments:

At 01:38 PM 10/31/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 31 Oct 1997 08:38:08 EST

>Subject: Changes in resonate frequency

>

>

>     How do I compensate for changes in resonate frequency due to the

>     addition of my toroid ? I plan to make the toroid from dryer duct.

>     Assuming the overall diameter of the toroid is 12" and the diameter of

>     the dryer duct is 4" , how will this effect my resonate frequency ?

It will lower your resonant frequency significantly.

My present 4 inch secondary resonates at 395 KHZ unloaded (bare)

and goes down to 254 KHZ with a 19 X 6 toroid. These are measured

resonant frequencies.

The larger the toroid the more the frequency will drop.

>     Can I simply change the tap on my primary ?

Yep. Thats why I keep telling everyone to have plenty of primary

turns. You will need these extra turns to bring larger toroids

to resonance.

>

>     My secondary is wound using 22 AWG on a 4.5" dia. form for a length of

>     23" Input voltage is 15 kv at a max current of 180 ma. Primary cap is

>     .02 mf (two of these in series) I will initially begin using 60ma and

>     a variac. Power up is planned for 11/15/97. Wish me luck.

LUCK!

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark Gap Construction

Date: Tue,  4 Nov 1997 23:26:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Tue, 4 Nov 1997 17:26:51 -0600

Subject: Spark Gap Construction

Howdy All,

I need to make a new spark gap and Mike recommended that I go with the

design described in the file for building a cylinder gap that's at the ftp

site.  So I d/l the file, but there are a few questions I have regarding

construction and voltage handling.

1.  How many gaps should I use to quench a 15kv, 30ma coil?

2.  Can I use the basic design described (6" pvc, 1-1/2" copper pipe) with

4" pvc and 3/4" pipe?

3.  When wiring the gap every other electrode is connected together, then

one set is hooked to each of the two HV outputs, one of which goes to the

cap (then the primary) and the other directly to the primary, right?  (where

the safety and spark gaps are wired parallel to the xfrmr, and the cap is

wired in series, on one HV leg, to the primary)

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark Gap Construction

Cc:

Bcc:

X-Attachments:

At 11:26 PM 11/4/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Tue, 4 Nov 1997 17:26:51 -0600

>Subject: Spark Gap Construction

>

>Howdy All,

>

>I need to make a new spark gap and Mike recommended that I go with the

>design described in the file for building a cylinder gap that's at the ftp

>site.  So I d/l the file, but there are a few questions I have regarding

>construction and voltage handling.

>

>1.  How many gaps should I use to quench a 15kv, 30ma coil?

No rules set in concrete here. Generally speaking the more

total gap distance there is the higher the cap voltage will

rise and the more output you can expect. Unfortunately every increase

in gap spacing increases the stress and strain on your transformer

and caps.

What appears to work well is 8 gaps (9 electrodes) spaced no more

than .03 inches apart for 12kv. This amounts to a maximum .240 inch

of gap. For 15kv use 10 gaps (11 electrodes) spaced a maximum of

.03 inches would be appropriate. These are fairly safe values

that shouldnt over stress components too much.

My own gap unit has 9 gaps (10 electrodes) spaced .025 inch apart

for a maximum of .225 inch of gap for 12kv neon use. This has allowed

for over 50 inches of spark with my 4 inch coil.

>

>2.  Can I use the basic design described (6" pvc, 1-1/2" copper pipe) with

>4" pvc and 3/4" pipe?

Yes. This is exactly what I have for my gap. However you can only

place a maximum of 10 pieces of 3/4 inch pipe inside 4 inch PVC.

This will give you 9 series gaps. Spaced at .03 inch thats a maximum

of .270 inches. That should be plenty. You will need to place a

plexiglass or Lexan strip in between the first and last electrodes

to prevent the arc from cheating and bypassing the other electrodes.

>

>3.  When wiring the gap every other electrode is connected together, then

>one set is hooked to each of the two HV outputs, one of which goes to the

>cap (then the primary) and the other directly to the primary, right?  (where

>the safety and spark gaps are wired parallel to the xfrmr, and the cap is

>wired in series, on one HV leg, to the primary)

>

You dont need to connect any of the electrodes together. If you want

to use all the gaps just connect to the first and last electrodes

and the arc will jump from electrode to electrode around the ring.

The amount of gap in the circuit depends on the number of electrodes

in between what you use for the first and last. You can select

any number of series gaps from 1 up to the maximum you have in the unit.

The gap distance is varied in steps as well as the number of series gaps.

Its harder to explain than it really is. The text file accompanying

the the cylinder gap design isnt really very clear.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Safety Gaps

Date: Wed,  5 Nov 1997 06:25:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Wed, 5 Nov 1997 00:25:56 -0600

Subject: Safety Gaps

Another construction question, this time the safety gap:

My SG is made of two L-brackets with 3" brass #10 screws capped with a

stainless cap nut (couldn't find them in brass) and the center electrode is

also a brass screw with two brass nuts tightened together at the contact

point (at the same level as the two cap nuts).  From everything I've read it

sounds like this should be sufficient, but one thing bothers me. . .I copied

some parts and supplies "where to get it" info off someone's web site and

one of the items listed was welding electrodes which were supposed to be

good for making safety gaps.  My concern is, am I going to burn up my

current SG quick enough that I run the risk of leaving my xfrmr unprotected

at some point during operation because I have a fried SG that wasn't up to

the task?

Also, any good starting points for distance between the electrodes?  I

understand to adjust it so that it fires intermittently, but with my coil

not yet throwing much of a spark, and never having seen any arcing in the

SG, I'd like to know how far is too far, I guess?  Or should I just set it

real close then back the electrodes out till it stops firing continually?

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety Gaps

Cc:

Bcc:

X-Attachments:

At 06:25 AM 11/5/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Wed, 5 Nov 1997 00:25:56 -0600

>Subject: Safety Gaps

>

>

>Another construction question, this time the safety gap:

>

>My SG is made of two L-brackets with 3" brass #10 screws capped with a

>stainless cap nut (couldn't find them in brass) and the center electrode is

>also a brass screw with two brass nuts tightened together at the contact

>point (at the same level as the two cap nuts).  From everything I've read it

>sounds like this should be sufficient, but one thing bothers me. . .I copied

>some parts and supplies "where to get it" info off someone's web site and

>one of the items listed was welding electrodes which were supposed to be

>good for making safety gaps.  My concern is, am I going to burn up my

>current SG quick enough that I run the risk of leaving my xfrmr unprotected

>at some point during operation because I have a fried SG that wasn't up to

>the task?

This sounds just fine. The safety gap wont be handling a lot of power

only the hopefully occasional kickback. My safety is nothing more

than a couple 1/4 20 bolts. Its seen lots of action and the electrode

faces look pretty scarred up. But I have never cleaned it.

Yours will work just fine.

>

>Also, any good starting points for distance between the electrodes?  I

>understand to adjust it so that it fires intermittently, but with my coil

>not yet throwing much of a spark, and never having seen any arcing in the

>SG, I'd like to know how far is too far, I guess?  Or should I just set it

>real close then back the electrodes out till it stops firing continually?

>

Im running mine at about 3/8 inch. If Im not getting any activity

on it I close it up a bit. If its firing constantly I back it off

a bit. I would think anything over 1/2 inch wont be very "safe".

At first you wont be producing much RF energy. So your

transformer isnt in very much danger. But if you are way out

of tune kickbacks will be more frequent.

Start with a value somewhat above your main gap spacing.

Adjust according to the amount of activity.

Once your coil is running smoothly you should have very few

kickbacks.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Poly capacitors

Date: Thu,  6 Nov 1997 14:09:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 06 Nov 1997 09:09:09 EST

Subject: Poly capacitors

Now that I am finally in to the swing of things and a veteran

cap builder, I'm taking on a special project. Mission Impossible,

build a reliable 15 kv poly cap. If I use a single sheet of .060"

as a base and add sheets of 4 mil to build up the thickness I think

I should be able to make it work. Can anyone suggest as a minimum

thickness.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly capacitors

Cc:

Bcc:

X-Attachments:

At 02:09 PM 11/6/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 06 Nov 1997 09:09:09 EST

>Subject: Poly capacitors

>

>

>

>       Now that I am finally in to the swing of things and a veteran

>     cap builder, I'm taking on a special project. Mission Impossible,

>     build a reliable 15 kv poly cap. If I use a single sheet of .060"

>     as a base and add sheets of 4 mil to build up the thickness I think

>     I should be able to make it work. Can anyone suggest as a minimum

>     thickness.

>

>

Do you mean to build a single cap to handle 15kv rather than

2 or more in series?

If thats your aim forget it. Just not possible to build a single

cap that will be bullet proof at 15 kv. No matter what the poly

thickness corona will begin to form at around 12kv. (even in oil)

This will serve to breakdown the poly dielectric over time and lead

to failure. Just cant be helped.

My single .01 mfd plate cap with 60 mils (10 X 6 mil) of poly just recently

failed. There was a 1/2 inch diameter hole blown through one of the

60 mil poly layers. The plates of aluminum flashing on either side

of the site had dents in them. They looked like someone had attacked

them with a ball pean hammer. This was while running only 12kv.

At the time of the failure this cap was in parallel with my older

plate cap. This older cap had only 48 mils of poly. It sustained

no damage and continues to function well. Whats the difference?

The older cap has 6 caps in series each with 8 mils of poly.

6 X 8 mils = 48 mils. Even if the cap would see 18kv spikes

thats only 3kv per cap. No chance for any corona to occur.

This has reaffirmed my belief in series caps. Right now Im using

a pair of .03 mfd 24 mil caps in series for 48 mils at .015 mfd.

They are working quite well at 12kv.

Ive begun work on a new plate cap and this will be two caps in series

with each cap being .02 mfd with 5 X 6 mil poly. Im also going

back to foil as plate material. Flashing doesn't have enough

advantage to justify the extra cost.

If you are determined to try any way I would double up your poly

for 120 mils thickness. Provide a 2 inch wide clearance all around

your plates. Use real transformer oil not mineral oil. Build it

in oil to eliminate air bubbles. And be prepared for it to fail

eventually any way.

Adding 4 mil sheets to a 60 mil base would probably not buy you

much extra voltage margin.

I would use that 4 mil poly to make 4 series caps with 16 mils each.

The 4 in series would equal 64 mils and handle 15 kv easily.

I have known coilers that have lost single rolled caps using 120 mils

of poly with 14400 pole pigs.

Happy capping

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Mineral Oil - Capacitors

Date: Thu,  6 Nov 1997 19:03:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 06 Nov 1997 14:03:14 EST

Subject: Mineral Oil - Capacitors

Does anyone know the difference between light and Heavy mineral oil ?

I got a pretty good deal on light oil by the gallon but I haven't

purchased any as of yet. Which is better light or heavy. I have

also heard of people using motor oil, has anyone tried this.

Perhaps the synthetic oil would work well, any comments ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Poly capacitors

Date: Thu,  6 Nov 1997 20:36:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: ms291 <ms291@mindspring.com>

Date: Thu, 06 Nov 1997 15:36:28 -0500

Subject: Re: Poly capacitors

Hi,

I have been lurking on this list for a couple of weeks, so I figure I

better ask some kind of question.. :)

A friend and I are endeavoring to build our first coil. So far we have

wound a couple of secondary coils, and some RFCs to protect the

used-but-low-miles 15kV 60ma neon that I probably paid too much for...

(but it did make for an excellent Jacob's ladder during halloween)

Anyhow.. we have decided to try constructing a few PVC rolled caps. We

have access to a vacuum pump and a chamber, so that will help. Trying to

find some oil wholesale place that will sell us a few gallons of

transformer oil...

We have purchased several lengths of 30 mil thick HDPE for the cap. We

were going to try 2 or 3 layers of HDPE using some aluminum flashing for

plates.  All rolled inside either 4 or 6" PVC.

My question is this: I have seen many references to LDPE, but none to

HDPE. Is it okay to use the HDPE? I wondered if there is something about

HDPE (dielectric, RF dissipation..) that would rule it out as the cap

dielectric?

A second question: I have repeatedly seen treatises on the advantage of

series caps over a single thick dielectric cap. What I was wondering was

if anyone had tried making two caps in series inside a single PVC pipe

section? (And whether or not it survived)

I also managed to get a neon sign which is over the piano right now (it

was free and it works... unfortunately.. it read "Dannon Frozen Yogurt"

:) I have been eyeing the 9 kV 20 ma transformer as the 'first try'

neon.. don't want to blow the 15/60 right of the bat.. guess it would

also be easier on the caps...

Jim Horan

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Mineral Oil - Capacitors

Cc:

Bcc:

X-Attachments:

At 07:03 PM 11/6/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 06 Nov 1997 14:03:14 EST

>Subject: Mineral Oil - Capacitors

>

>

>     Does anyone know the difference between light and Heavy mineral oil ?

"Mineral oil" is a rather general term given to a whole class

of light oils. Nearly any of them would be suitable.

Lower viscosity (light) oils are preferable as they allow trapped air

bubbles to more easily escape.

In fact many grades of transformer oil are very thin. Almost

water like.

>     I got a pretty good deal on light oil by the gallon but I haven't

>     purchased any as of yet. Which is better light or heavy. I have

>     also heard of people using motor oil, has anyone tried this.

>     Perhaps the synthetic oil would work well, any comments ?

>

Get the light oil. Avoid any kind of motor oil. Petroleum or

synthetic. Todays oils abound with anti-wear, anti foam and

all other sorts of additives. None of these will do much

for the electrical characteristics.

Some silicone based oils such as refrigeration oil are suitable.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly capacitors

Cc:

Bcc:

X-Attachments:

At 08:36 PM 11/6/97 GMT, you wrote:

>From: ms291 <ms291@mindspring.com>

>Date: Thu, 06 Nov 1997 15:36:28 -0500

>Subject: Re: Poly capacitors

>

>Hi,

>

>I have been lurking on this list for a couple of weeks, so I figure I

>better ask some kind of question.. :)

Welcome aboard Jim. My comments are interspersed below...

>A friend and I are endeavoring to build our first coil. So far we have

>wound a couple of secondary coils, and some RFCs to protect the

>used-but-low-miles 15kV 60ma neon that I probably paid too much for...

>(but it did make for an excellent Jacob's ladder during halloween)

>

>Anyhow.. we have decided to try constructing a few PVC rolled caps. We

>have access to a vacuum pump and a chamber, so that will help. Trying to

>find some oil wholesale place that will sell us a few gallons of

>transformer oil...

Dont neglect asking your local power company. This doesn't always

work (didnt for me) but some people have been able to get a couple

gallons of transformer oil free or quite cheaply this way.

Never hurts to try.

>

>We have purchased several lengths of 30 mil thick HDPE for the cap. We

>were going to try 2 or 3 layers of HDPE using some aluminum flashing for

>plates.  All rolled inside either 4 or 6" PVC.

>

>My question is this: I have seen many references to LDPE, but none to

>HDPE. Is it okay to use the HDPE? I wondered if there is something about

>HDPE (dielectric, RF dissipation..) that would rule it out as the cap

>dielectric?

High density polyethyle is fine from a dielectric standpoint.

Its ratings are nearly identical to the low density variety.

However HDPE will be very difficult to roll into a cap.

It is not soft and supple like LDPE. Best bet would be to

use it for a plate type cap or get some LDPE for a rolled up cap.

>

>A second question: I have repeatedly seen treatises on the advantage of

>series caps over a single thick dielectric cap. What I was wondering was

>if anyone had tried making two caps in series inside a single PVC pipe

>section? (And whether or not it survived)

As a matter of fact someone has. Fr. Tom Mcgahee posted a rather

lengthy description of a multi section rolled cap design he

was working on. It is a sound idea. Rather that repost the whole thing

here again you can download the file from list member Dave Bronakowsi's

FTP site. The address is:

ftp://ftp.velocity.net/usr/djb/tesla/texts/

and the file name is multiroll.txt

>I also managed to get a neon sign which is over the piano right now (it

>was free and it works... unfortunately.. it read "Dannon Frozen Yogurt"

>:) I have been eyeing the 9 kV 20 ma transformer as the 'first try'

>neon.. don't want to blow the 15/60 right of the bat.. guess it would

>also be easier on the caps...

>

Yes that is a good idea. I often use a 9kv neon for tuning

purposes or whenever my coil has been changed enough to be unsure

of the tuning. You never want to blast full power into a mistuned

coil. Doing so will only damage components.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark-gap/cap configuration

Date: Fri,  7 Nov 1997 14:46:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Fri, 07 Nov 1997 08:46:18 -0600

Subject: Spark-gap/cap configuration

Hi All!

I have a quick question. I know this has been asked to death but I'm

still a little confused. Which is the more optimal placement for the gap

and the cap. It seems to be a toss up in my mind. There may be a slight

advantage to having the gap across the transformer for a bit of

over-voltage protection. It seems there would only be a difference at

high frequencies, since the primary has almost no impedance at 60 Hz.

They are basically in parallel. Am I missing something? Any comments

would be appreciated! Thanks.

Eric Davidson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:Spark-gap/cap configuration

Cc:

Bcc:

X-Attachments:

At 02:46 PM 11/7/97 GMT, you wrote:

>From: Eric Davidson <edavidson@icva.gov>

>Date: Fri, 07 Nov 1997 08:46:18 -0600

>Subject: Spark-gap/cap configuration

>

>Hi All!

>

>I have a quick question. I know this has been asked to death but I'm

>still a little confused. Which is the more optimal placement for the gap

>and the cap. It seems to be a toss up in my mind. There may be a slight

>advantage to having the gap across the transformer for a bit of

>over-voltage protection. It seems there would only be a difference at

>high frequencies, since the primary has almost no impedance at 60 Hz.

>They are basically in parallel. Am I missing something? Any comments

>would be appreciated! Thanks.

>

>Eric Davidson

>

Eric,

The ONLY reason for placing the gap across the transformer

is for protection against RF. Performance wise there is NO

difference between the two configurations.

With the main cap placed in parallel across the transformer

any RF appearing on the cap will also appear on the transformer

secondary since they are in parallel. Not a good condition.

This RF will erode the transformer insulation until a breakdown

occurs.

By placing the gap in parallel across the transformer its secondary

winding is somewhat protected from the high levels of RF energy

by the energized gap. The energized gap represents a short circuit.

This is not foolproof and some RF can and will sneak by the

main gap. Thats why RF chokes and a safety gap are still

important.

None of these 3 protective measures can accomplish much on

their own. But when all used together they provide a good

level of protection for the neon transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Mineral Oil - Capacit

Date: Fri,  7 Nov 1997 18:06:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Fri, 7 Nov 1997 12:06:20 -0600

Subject: Re: Mineral Oil - Capacit

>"Mineral oil" is a rather general term given to a whole class

>of light oils. Nearly any of them would be suitable.

>Lower viscosity (light) oils are preferable as they allow trapped air

>bubbles to more easily escape.

Does an air bubble equal failure?  When I took apart a cap (to use the

plates building up two others) I did find a couple of dry spots the the oil

never got to.  Since it didn't blow-up you might be thinking I've already

answered my own question, but you would be assuming that the coil I had the

cap hooked to actually worked! :)

CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Car teslas

Date: Fri,  7 Nov 1997 18:28:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Fri, 07 Nov 1997 12:28:16 -0600

Subject: Car teslas

Organization: MailExcite  (http://www.mailexcite.com)

Normally, I wouldn't ask a question this simple, but it has been bothering me very

much, and some of you just have a nack for giving the answer in clear and easy-

to-understand terms!

Well, basically, I'm trying to figure out how to run a tesla coil from the 12vdc

current in my car. I have the HV power supply and coil parts, but the problem is,

the DC volage from the car will not run my HV power supply. 12vAC current will

run it, but not 12vDC.

Is there a schematic that would allow me to run this transformer off of my 12vDC

car battery? Or perhaps is there a simple device, such as a capacitor, that could

be placed so that it would allow the DC current to be interrupted and cause my

transformer would fire?

Any comments would be much appreciated!

--Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Crash!

Date: Fri,  7 Nov 1997 19:26:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Robert Deppen <thorn@bslnet.com>

Date: Fri, 07 Nov 1997 11:26:37 -0800

Subject: Crash!

Hello all,

I have recently experienced a complete crash of my computer.  I have

lost all of the data that I have saved.  Can someone please repost the

schematic layout of a twin gap circuit, as well as the explanation of

the gap that uses the multiple pipes?

I also need the formula for determining the capacitance in accordance

with a 12/30 franceformer.

Thanks!

Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Mineral Oil - Capacit

Cc:

Bcc:

X-Attachments:

At 06:06 PM 11/7/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Fri, 7 Nov 1997 12:06:20 -0600

>Subject: Re: Mineral Oil - Capacit

>

>

>>"Mineral oil" is a rather general term given to a whole class

>>of light oils. Nearly any of them would be suitable.

>>Lower viscosity (light) oils are preferable as they allow trapped air

>>bubbles to more easily escape.

>

>Does an air bubble equal failure?  When I took apart a cap (to use the

>plates building up two others) I did find a couple of dry spots the the oil

>never got to.  Since it didn't blow-up you might be thinking I've already

>anserwed my own question, but you would be assuming that that coil I had the

>cap hooked to actually worked! :)

>

An air bubble provides a place for corona to form.

The whole point of putting a cap in oil is to prevent

the formation of corona. Corona will form much easier at a much lower

voltage in air than in oil. When corona forms it can deteriorate

the dielectric and promote an early failure.

By running caps in series so that the maximum voltage each

individual cap sees is low corona is much less of a problem.

This is also why a single cap will often fail at 12kv or above.

At these voltages corona will sometimes form even in oil.

One way to prvent dry spots is to assemble the cap under oil.

Or simply coat each poly sheet with oil during assembly. Work

the air bubbles out toward the edges with the fingers. Messy

work but it does the job.

Given sufficient time vertical plate caps will bubble out

all the trapped air. Just dont put full power to them for

a while and ionize those trapped air bubbles.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Crash!

Cc:

Bcc:

X-Attachments:

At 07:26 PM 11/7/97 GMT, you wrote:

>From: Robert Deppen <thorn@bslnet.com>

>Date: Fri, 07 Nov 1997 11:26:37 -0800

>Subject: Crash!

>

>Hello all,

>

>

>   I have recently experienced a complete crash of my computer.  I have

>lost all of the data that I have saved.

Sorry to hear that Robert. I suffered a major crash a while

back and it wasnt at all fun. Fortunately I was able to recover

95 percent of my data.

> Can someone please repost the schematic layout of a twin gap circuit,

Ok. I hope this is the schematic you were wanting. This has been posted

several times so its probably the one you meant. If not let me know.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

> as well as the explanation of the gap that uses the multiple pipes?

This is the cylinder gap that was popularized by Richard Quick.

It is therefore somewhat wrongly referred to as a RQ gap.

It contains electrodes cut from sections of 3/4 or 1 inch copper

pipe. The electrodes can be from 2 - 6 inches in length. The larger

electrodes sink more heat so will handle higher power levels.

The electrodes are mounted inside a section of PVC pipe. The are mounted

in radial fashion around the inside wall of the PVC pipe.

The mounting bolts protrude to the outside of the PVC housing

and provide connection points to the electrodes.

You select the amount of gap you need by moving your connection

point to the various electrodes. The number of gaps and therefore

the spacing depends on the number of electrodes between the electrodes

you made your connections to. The arc will jump from electrode to

electrode around the ring.

Download the file for this gap unit from Dave Bronakowski's FTP site.

The address is:  ftp://ftp.velocity.net/usr/djb/tesla/texts/

The file name is sparkgap.zip

>I also need the formula for determining the capacitance in accordance

>with a 12/30 franceformer.

OK. First you need to determine the impedance of your transformer.

1. Determine your neon sign transformer (or other transformer's)

Impedence:

E

Z  =  ---

I

Z = Impedence

E = volts

I = current in Amps

For your 12kv/30ma neon this is   12kv/.03 = 400k ohms

2. Next you need to find the capacitor value that has this same imedance

at the 60 (or 50) HZ line frequency.

1

C =   -------------------

2 x pi x Z x .00006

C = capacitance in microfarads needed for primary capacitor.

Z = Impedence from equation one (Transformer Impedence)

pi = 3.14

Note: The .00006 is the 60 Hz AC

In our above example:

C= 1/ 6.28*400000*.00006

C= 1/150.72

C= .0066 mfd

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Car teslas

Date: Sat,  8 Nov 1997 02:53:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Fri, 07 Nov 1997 18:53:17 -0800

Subject: Re: Car teslas

Tesla-2 wrote:

>

> From: tesla.coil@mailexcite.com

> Date: Fri, 07 Nov 1997 12:28:16 -0600

> Subject: Car teslas

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Normally, I wouldn't ask a question this simple, but it has been bothering me

> very much, and some of you just have a nack for giving the answer in clear and easy-

> to-understand terms!

>

> Well, basically, I'm trying to figure out how to run a tesla coil from the 12 vdc

> current in my car. I have the HV power supply and coil parts, but the problem is,

> the DC volage from the car will not run my HV power supply. 12vAC current will

> run it, but not 12vDC.

>

> Is there a schematic that would allow me to run this transformer off of my 12vDC

> car battery? Or perhaps is there a simple device, such as a capacitor, that could

> be placed so that it would allow the DC current to be interrupted and cause my

> transformer would fire? Any comments would be much appreciated!

>

> --Jay

>

I am sure there are better answers fortcoming, but a circuit like

a "vibrator" in tube-type car radios would pulse the d.c.

Think of a 12VDC relay, wired such that the relay contacts break the

circuit to the relay coil as soon as the relay begins to energize.

Bzzzzzzzzzzzzzzzz.......pulsating d.c.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Car teslas

Cc:

Bcc:

X-Attachments:

At 06:28 PM 11/7/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Fri, 07 Nov 1997 12:28:16 -0600

>Subject: Car teslas

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Normally, I wouldn't ask a question this simple, but it has been bothering me very

>much, and some of you just have a nack for giving the answer in clear and easy-

>to-understand terms!

>

>Well, basically, I'm trying to figure out how to run a tesla coil from the 12vdc

>current in my car. I have the HV power supply and coil parts, but the problem is,

>the DC voltage from the car will not run my HV power supply. 12vAC current will run

>it, but not 12vDC.

>

>Is there a schematic that would allow me to run this transformer off of my 12vDC

>car battery? Or perhaps is there a simple device, such as a capacitor, that could

>be placed so that it would allow the DC current to be interrupted and cause my

>transformer would fire? Any comments would be much appreciated!

>

>--Jay

Jay,

What kind of HV supply is this?

To get 12VAC from 12vdc one approach would be an inverter supplying

120 VAC and using a 12.6 filament transformer running from that.

Not sure how the inverter would react to the greating varying

load presented by a Tesla coil though.

Not sure what else to recommend without knowing more about the

HV supply you are using.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Plasma globe pressures

Date: Sat,  8 Nov 1997 13:19:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Sat, 8 Nov 1997 13:19:13 -0000

Subject: Plasma globe pressures

Hi Guys,

I'm playing with a flyback transformer to make some plasma in a glass

vessel. I've got a vacuum pump, but I don't know what pressure I need to

create lively plasma. Will I get any results at about 50 to 100mm Hg ? My

flyback produces about 30 KV and works at 22 Khz..producing a 1 Inch spark.

I don't think my pump will reach 1 to 2 mm Hg (It's difficult to achieve

on the kitchen sink!) and wonder if a high frequency tesla coil will

produce good results at the pressures I beleive my pump can manage? Any

suggestions will be most grateful.....

Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plasma globe pressure

Date: Sat,  8 Nov 1997 14:53:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Sat, 8 Nov 1997 15:53:34 +0100 (CET)

Subject: Re: Plasma globe pressures

> vessel. I've got a vacuum pump, but I don't know what pressure I need to

> create lively plasma. Will I get any results at about 50 to 100mm Hg ? My

> flyback produces about 30 KV and works at 22 Khz..producing a 1 Inch spark.

Yes (although rather poor). Have a look at my homepage, "Things to do:

simple Plasma Globe".

---------------------------------------------------------------------------

Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de

The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/

info, plans, photos of home-built HV-sources, sparks, HV-experiments

---------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plasma globe pressures

Cc:

Bcc:

X-Attachments:

At 01:19 PM 11/8/97 GMT, you wrote:

>From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

>Date: Sat, 8 Nov 1997 13:19:13 -0000

>Subject: Plasma globe pressures

>

>Hi Guys,

>        I'm playing with a flyback transormer to make some plasma in a glass

>vessel. I've got a vacuum pump, but I don't know what pressure I need to

>create lively plasma. Will I get any results at about 50 to 100mm Hg ? My

>flyback produces about 30 KV and works at 22 Khz..producing a 1 Inch spark.

>

> I don't think my pump will reach 1 to 2 mm Hg (It's difficult to acheive

>on the kitchen sink!) and wonder if a high frequency tesla coil will

>produce good results at the pressures I believe my pump can manage? Any

>suggestions will be most grateful.....

>

>        Regards,

>        Stuart

>

Stuart,

It takes a pretty good vacuum to create plasma in air. A Tesla coil

may help somewhat due to its higher voltage and frequency but

the determining factor is really how good a vacuum you can produce.

If your pump wont pull enough to create good plasma try introducing

some other gases. Helium will produce plasma at nearly 1 ATM.

Argon will also have much the same effect. These gases are pretty

easy to come by. Check with a local welding supplier. Neon can be

found at a local neon sign shop.

Other nobles gases like krypton,and Xenon would be interesting to experiment

with but arent as easy to come by.

A interesting web page with some info is:

http://www-astro.physics.uiowa.edu/~rnw/globe.html

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Introducing gasses into g

Date: Sat,  8 Nov 1997 23:43:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Sat, 8 Nov 1997 23:43:02 -0000

Subject: Introducing gasses into globes

Hi again,

If I get my hands on some neon or helium for a higher pressure plasma

globe, how do I introduce the gas into the globe?

Do I:

1) Make two connections into the globe putting the eg. neon in one

connection and wait for it to permeate the entire globe until nothing but

neon comes out of the other connection,

2) Or do I create as high a vacuum as I can in the globe then introduce

a little bit of the eg. neon gas,

3) Or do I do as in 1) above then re-pump the globe down as low as I can?

Any answers are most appreciated.

Regards,

Stuart,

P.S How do you EASILY check for leaks in a vacuum system without an

expensive 'ultrasonic' noise detector?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Mineral Oil - Capacit

Date: Sun,  9 Nov 1997 00:48:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 08 Nov 1997 16:48:42 -0800

Subject: Re: Mineral Oil - Capacit

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Chris Federico <cfed@comsource.net>

> Date: Fri, 7 Nov 1997 12:06:20 -0600

> Subject: Re: Mineral Oil - Capacit

>

> >"Mineral oil" is a rather general term given to a whole class

> >of light oils. Nearly any of them would be suitable.

> >Lower viscosity (light) oils are preferable as they allow trapped air

> >bubbles to more easily escape.

>

> Does an air bubble equal failure?  When I took apart a cap (to use the

> plates building up two others) I did find a couple of dry spots the the oil

> never got to.  Since it didn't blow-up you might be thinking I've already

> anserwed my own question, but you would be assuming that that coil I had the

> cap hooked to actually worked! :)

>

> CF

>

Chris,

It depends upon the thickness of the dielectric, the size of the air

bubble, the voltage you're running across the capacitor, and the

individual and total runtime durations.

In general, any entrapped air which is subjected to enough voltage

stress to cause ionization will, eventually, cause failure of the

capacitor's dielectric material. Ozone, nitrogen oxides, ultraviolet

light, microscopic partial discharges (i.e., tiny sparks), and localized

thermal heating create a dielectric-killing combination. While it's not

possible to remove all entrapped air short of using a vacuum pump, you

can design the cap so that during normal use (during a gradual break-in

period) air can work its way out from the electrically stressed areas.

Stacking the plates so that they are vertical. Using an oil-wicking

material like kraft paper in addition to the LDPE also helps. Finally,

connecting identical capacitor sections in series to reduce the

dielectric stress is another trick the pros use.

Safe cappin' to you, Chris!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Microwave parts

Date: Sun,  9 Nov 1997 03:02:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sat, 8 Nov 1997 21:02:29 -0600

Subject: Microwave parts

I remember reading something about scavenging parts out of a microwave for use

in a TC.  Was it just the xfrmr, or something else?  The one at work

crapped-out and as far as I can tell it wasn't the xfrmr that went bad.

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Holy Cow!!

Date: Sun,  9 Nov 1997 18:10:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sun, 9 Nov 1997 12:10:51 -0600

Subject: Holy Cow!!

Mike,

You shoulda warned me!!  I had no idea what to actually expect when this

thing finally decided to try to resonate.  I also had no idea how loud and

stinky the safety gap was when it actually worked!

Scared the livin' hell outta me!! :)

That all being said, I seem to have a problem.  Or three.

First, what can I do to seal a spot about half way up the secondary where a

spark broke out (man, this thing was throwin' sparks everywhere, around the

sec, to the primary, up to the terminal, quite a show if I had left it on

for more than a second).  Please don't tell me it's ruined.

I had to widen the safety gap to more than 1/2" total to get it not to fire

the instant I turned the coil on. I remember your warning about not going

over 1/2" (but after rereading that post I'm not sure if you meant 1/2" per

side, or total?)

How harmful is the ozone created by the safety gap?  Should I worry about

venting it somewhere or if I'm creating enough to smell that quickly is

something else wrong?

Should I be trippin' a GFI (either 10 or 15 amp, I think)?  even with

nothing else operating on that circuit?

I borrowed a meter from work and the caps measured at .006 mfd.  Too much?

The program I was using said 56 plates should only be .005.

I did get one spark off the top of the disharge sphere, about 6 inches

straight up, but couldn't tell if it came from the break-out arcing to the

sphere, or the proper path.

Thanks . . . Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Dangerous?

Date: Sun,  9 Nov 1997 20:26:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Sun, 9 Nov 1997 21:26:39 +0100

Subject: Dangerous?

Hi all!

This regards the schematic of the tesla coil. On all the schematics Ive

seen has the secondary and the primary been connected to each other.

Until I read in a text Ive believed that it should be so. But when I read a

mail about setting up a tesla coil, I read this: "...In fact if you're running

your coil with the bottom of your secondary tied to to the bottom of the

primary coil like is shown in some older drawings that is very dangerous.

This places 60hz primary energy directly on the secondary. Never tie your

secondary and primary together."

I don't know who wrote that mail but I think i found it in the tesla-2 listarchive.

Is this true? Or have I misunderstood what the text meant? Those drawings I've seen

had all their primary and secondary tied together. So, why is this dangerous?

Thanks

Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introducing gasses into g

Cc:

Bcc:

X-Attachments:

At 11:43 PM 11/8/97 GMT, you wrote:

>From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

>Date: Sat, 8 Nov 1997 23:43:02 -0000

>Subject: Introducing gasses into globes

>

>Hi again,

> If I get my hands on some neon or helium for a higher pressure plasma

>globe, how do I introduce the gas into the globe?

>

>        Do I:

>        1) Make two connections into the globe putting the eg. neon in one

>           connection and wait for it to permeat the entire globe until

>            nothing but neon comes out of the other connection.

>

>        2) Or do I create as high a vacuum as I can in the globe then introduce

>           a little bit of the eg. neon gas,

>        3) Or do I do as in 1) above then re-pump the globe down as low as I can?

>

>        Any answers are most appreciated.

>

Stuart,

First let me qualify my answer to your questions by stating my experience

with plasma globes has be confined to a few crude experiments. So this

is largely guessing on my part.

I would think that all 3 of the approaches you outlined are viable

methods. Each would have different end results. Each method would

end up with a different mix of neon and air. Which one will give

the best plasma you will have to determine experimentally.

From what I have read it seems that your method number two is the

most common approach.

First thing to do would be to see if your vacuum system can draw enough

vacuum to create plasma in air. This will give you a chance to check for leaks

and to get a feel for your equipment before using up any of your other

gases.

Keep us posted on your results.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Holy Cow!!

Cc:

Bcc:

X-Attachments:

At 06:10 PM 11/9/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sun, 9 Nov 1997 12:10:51 -0600

>Subject: Holy Cow!!

>

>Mike,

>

>You shoulda warned me!!  I had no idea what to actually expect when this

>thing finally decided to try to resonate.  I also had no idea how loud and

>stinky the safety gap was when it actually worked!

>Scared the livin' hell outta me!! :)

Congratulations Chris. Yes it does get quite loud. I wear hearing

protection all the time whenever I run my coil.

>

>That all being said, I seem to have a problem.  Or three.

>

>First, what can I do to seal a spot about half way up the secondary where a

>spark broke out (man, this thing was throwin' sparks everywhere, around the

>sec, to the primary, up to the terminal, quite a show if I had left it on

>for more than a second).  Please don't tell me it's ruined.

Its probably OK. If you had sparks breakout in the middle of the secondary

thats probably a sign you are way overcoupled. Try raising up the

secondary about an inch farther away from the primary to reduce

coupling. Keep raising the secondary until the unwanted arcing

quits. Patch the spot on the secondary with a glob of epoxy.

Using a larger output terminal or toroid will help some too

by loading down the secondary some. Since you are close to resonance

now you can take advantage of the larger terminal to get longer sparks.

One other possibility is that your primary circuit is tuned to

twice the secondary frequency so the resonant point is appearing

halfway up the secondary rather than at the top. I dont think

that is the case here though.

>

>I had to widen the safety gap to more than 1/2" total to get it not to fire

>the instant I turned the coil on. I remember your warning about not going

>over 1/2" (but after rereading that post I'm not sure if you meant 1/2" per

>side, or total?)

You maybe running a little too wide on your main gap. Close

down the main gap a little and things should settle down a bit.

If you are using a safety with a grounded center post then its

1/2 inch per side.

>

>How harmful is the ozone created by the safety gap?  Should I worry about

>venting it somewhere or if I'm creating enough to smell that quickly is

>something else wrong?

Ozone is rather nasty and can cause all kinds of respiratory

problems. Ventilation is a must. Even worse are the various

oxides of nitrogen that are also formed.

Vent it or learn to like it. :)

>

>Should I be trippin' a GFI (either 10 or 15 amp, I think)?  even with

>nothing else operating on that circuit?

Dont run coils on GFI circuits. The pulsing very non-sinusoidal

current draw will drive GFI breakers nuts.

>

>I borrowed a meter from work and the caps measured at .006 mfd.  Too much?

>The program I was using said 56 plates should only be .005.

Not uncommon at all for caps to measure somewhat differently

from calculations. .006 is probably just fine on your 15kv/30 neon.

I wouldnt worry about it as most neons will put out a little more than

their nameplate rating.

>I did get one spark off the top of the disharge sphere, about 6 inches

>straight up, but couldn't tell if it came from the break-out arcing to the

>sphere, or the proper path.

>

>Thanks . . . Chris

>

Keep at it and keep us posted on your results.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Dangerous?

Cc:

Bcc:

X-Attachments:

At 08:26 PM 11/9/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Sun, 9 Nov 1997 21:26:39 +0100

>Subject: Dangerous?

>

>

>

>Hi all!

>

>This regards the schematic of the tesla coil. On all the schematics I've

>seen has the secondary and the primary been connected to each other. Until I

>read in a text I've believed that it should be so. But when I read a mail

>about setting up a tesla coil, I read this: "...In fact if you're running

>your coil with the bottom of your secondary tied to to the bottom of the

>primary coil like is shown in some older drawings that is very dangerous.

>This places 60hz primary energy directly on the secondary. Never tie your

>secondary and primary together."

>I don't know who wrote that mail but I >think I found it in the tesla-2

>list archive. Is this true? Or have I misunderstood what the text meant?

>Those drawings I've seen had all their primary and secondary tied together.

>So, why is this dangerous?

>Thanks

>

>Christopher Eriksson

>

Well Chris you understood the text correctly. You DO NOT want

your primary and secondary coils tied together.

This is dangerous for the reason you quoted above. It places

possibly lethal 60 hz energy on your secondary.

From a performance stand point a good low loss ground connection

tied to the base of the secondary is very important for efficient

operation.

A Tesla secondary needs to work against ground. If you deny it a

ground connection it will still try to work against the wiring

in your system as ground. This will often work at low power levels.

But this type of artificial ground will easily be overwhelmed.

At best this will severely limit performance. At worst

it could lead to unwanted arcing and destruction of components.

Safe and well grounded coiling!

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Microwave parts

Cc:

Bcc:

X-Attachments:

At 03:02 AM 11/9/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sat, 8 Nov 1997 21:02:29 -0600

>Subject: Microwave parts

>

>

>I remember reading something about scavenging parts out of a microwave for use

> in a TC.  Was it just the xfrmr, or something else?  The one at work

>crapped-out and as far as I can tell it wasn't the xfrmr that went bad.

>

Microwave ovens can yield some interesting and useful

parts but not a lot of them are really suited for coiling.

The typical microwave oven transformer runs around 2200 volts

at 300-500 ma. Thats too low a voltage and way too much current

for use on a coil. You can wire up a pair of transformers with

the secondary windings in series and the primary coils in parallel.

With proper phasing you can get around 4500 volts from the pair.

While still low this can be used on a coil but it wont be easy.

You will need very large caps and a heavy duty gap to handle

the amount of current. Some have run 3 or 4 in series with some

success to get higher voltages but this will exceed the insulation

rating of the outside transformers and they will likely breakdown.

The HV diode and filter cap from the oven would be useful to

make a HV DC supply but the cap is not useful for use on a coil.

Its meant as a power supply filter cap.

The magnetron from the oven can be dismantled to yield some

interesting magnets. Various other parts like fans, switches,

connectors and wiring are useful. And dont forget that nice heavy

duty AC line cord. Other peoples junk is my favorite source of parts.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: First Coil Confusion

Date: Mon, 10 Nov 1997 01:11:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alex Parker" <axparker@javanet.com>

Date: Sun, 9 Nov 1997 17:11:56 -0800

Subject: First Coil Confusion

Hi all,

Excuse me while I raise a question which has probably been asked to death.

Despite the enormous amounts of literature, schematics, and books I've read

on Tesla coils, I have yet to find one that gives detailed instructions on

how to wire the whole apparatus together. I have recently completed the

construction of all my coil parts, and I attempted to wire them together in

what I thought was the correct way, according to the schematics that I

have. When I turned on the juice, the spark gap immediately started firing,

stinking up my basement and waking up the neighbors. I let the thing run

for a full 5 seconds, but no spark issued forth from the terminal, or any

part of the coil other than the gap. Being a beginner to the HV field, and

the electronics field in general, I haven't got the slightest clue what

might be going wrong. Here are my questions:

My plans say that the capacitor should be connected in parallel across the

HV legs of the transformer. Does this mean that I should have 2 leads going

into the capacitor, and two leads going out; or a 2 lead cap with each one

of it's lead's across one of the HV legs? Also, I am in the dark about how

the secondary coil is connected to the rest of the circuit, or if it is

connected to the rest of the circuit at all. If anyone could take the time

to explain the wiring to me, or knows where I can find some DETAILED

instructions, it would be greatly appreciated.

Thanks for listening.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Coil Confusion

Cc:

Bcc:

X-Attachments:

At 01:11 AM 11/10/97 GMT, you wrote:

>From: "Alex Parker" <axparker@javanet.com>

>Date: Sun, 9 Nov 1997 17:11:56 -0800

>Subject: First Coil Confusion

>

>Hi all,

>

> Excuse me while I raise a question which has probably been asked to death.

>Despite the enormous amounts of literature, schematics, and books I've read

>on Tesla coils, I have yet to find one that gives detailed instructions on

>how to wire the whole apparatus together. I have recently completed the

>construction of all my coil parts, and I attempted to wire them together in

>what I thought was the correct way, according to the schematics that I

>have. When I turned on the juice, the spark gap immediately started firing,

>stinking up my basement and waking up the neighbors. I let the thing run

>for a full 5 seconds, but no spark issued forth from the terminal, or any

>part of the coil other than the gap. Being a beginner to the HV field, and

>the electronics field in general, I haven't got the slightest clue what

>might be going wrong.

I would say you probably have it wired correctly since the gap is firing.

Your problem is that your primary circuit and secondary coil are not

tuned to the same frequency. If they are more than a little off of being

on the same frequency you will get no output.

Try tapping various turns on your primary coil. Bring a small

flourescent light bulb near the secondary. It will light up

even if your coil is very far off tune. Use the light as a tuning

aid.

Also try bringing a grounded wire near your secondary terminal.

Often this will draw out a spark.

Try all tap positions on your primary coil and note the ouput level.

Do you get the most output with all the primary turns in?

If so that means your secondary coil is resonating at a lower

frequency than your primary circuit can tune.

The cure is to add more primary turns or increase the main cap

size. Both will allow the primary circuit to tune to a lower frequency.

Or you can raise the secondary frequency some by using a smaller

toroid or discharge terminal. Beginners often use way too large

of a toroid at first.

If you get best output with very few primary turns then your

secondary frequency is too high for your primary. Best cure here

is to increase toroid size to lower the secondary resonant frequency.

Or decrease the size of the main capacitor to raise the primary frequency.

Experimentation is called for to find the best combination for

YOUR coil. Each coil is a little different and has characteristics

of its own.

Here are my questions:

>

>My plans say that the capacitor should be connected in parallel across the

>HV legs of the transformer. Does this mean that I should have 2 leads going

>into the capacitor, and two leads going out; or a 2 lead cap with each one

>of it's lead's across one of the HV legs?

You should NOT have your cap in parallel across the transformer. I know

lots of books will show the cap across the transformer but it is not

the best method. Place your spark gap in parallel across the tranformer.

>Also, I am in the dark about how the secondary coil is connected to the

>rest of the circuit, or if it is connected to the rest of the circuit at all.

The secondary is NOT physically connected to any part of the circuit.

The secondary is placed within the magnetic field of the primary

and its base is connected directly to ground. It is not connected

to the primary in any way.

Im going to post my often used ascii schematic here for you to ponder

and think upon.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap with grounded center post

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

Should be pretty self explanatory but if something isn't

clear all you have to do is ask.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: First Coil Confusion

Date: Mon, 10 Nov 1997 15:27:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Mon, 10 Nov 1997 10:27:32 -0500

Subject: Re: First Coil Confusion

Alex:

Mike listed some great suggestions for improving your coil.  I've got one more...either operate your coil when the neighbors aren't around...or find another

location to coil.  Coils are not quiet.  I often wear ear protection when running

mine or offer it to guests.  You will need time to tune your coil.  It may

take an evening or two of trying various things to get it tuned.  This is part

of the fun.

I coil in my garage to keep the noise level down.  Also remember that as a coiler

you need to coil responsibly.  This includes not only audible sound but also

electrical interference.  Both of these items can seriously annoy your neighbors.

If possible, operate someplace with some shielding...I'm fortunate that the walls

in my garage are covered with insulation and foil.  This acts to suppress the sound

and electrical interference.  Also, ground your coil separate from any house ground.  This will keep your electrical items "safe" and help the operation of your coil.

Good luck and keep us posted.  It's always exciting to know a new coil is online!

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 11/09 8:11 PM >>>

From: "Alex Parker" <axparker@javanet.com>

Date: Sun, 9 Nov 1997 17:11:56 -0800

Subject: First Coil Confusion

Hi all,

Excuse me while I raise a question which has probably been asked to death.

Despite the enormous amounts of literature, schematics, and books I've read

on Tesla coils, I have yet to find one that gives detailed instructions on

how to wire the whole apparatus together. I have recently completed the

construction of all my coil parts, and I attempted to wire them together in

what I thought was the correct way, according to the schematics that I

have. When I turned on the juice, the spark gap immediately started firing,

stinking up my basement and waking up the neighbors. I let the thing run

for a full 5 seconds, but no spark issued forth from the terminal, or any

part of the coil other than the gap. Being a beginner to the HV field, and

the electronics field in general, I haven't got the slightest clue what

might be going wrong. Here are my questions:

My plans say that the capacitor should be connected in parallel across the

HV legs of the transformer. Does this mean that I should have 2 leads going

into the capacitor, and two leads going out; or a 2 lead cap with each one

of it's lead's across one of the HV legs? Also, I am in the dark about how

the secondary coil is connected to the rest of the circuit, or if it is

connected to the rest of the circuit at all. If anyone could take the time

to explain the wiring to me, or knows where I can find some DETAILED

instructions, it would be greatly appreciated.

Thanks for listening.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Plasma, IS IT FLAMABLE?

Date: Mon, 10 Nov 1997 19:30:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: esmit@ilink.nis.za

Date: Mon, 10 Nov 1997 21:30:22 +0200

Subject: Plasma, IS IT FLAMABLE?

Hi,

Long time no write.

I have a question.

I want to build a "plasma gate" a theory i have about a kind of

dimensional/black hole that can be created through plasma. I just want to

know if i use say neon/Helium or whatever, is it flammable. Will it burst

into flames if i put my TC into the gas.

I really want to experiment and test my theory with plasma. I want to put

the gas(helium) into a long glass tube and are afraid that the gas might

explode and glass will hit me.

How does a person get so a great vaccuum in the glass tube?

say if i use helium, and put it at 1 mm Hg(whatever that is) and make plasma

what is its effect against a tube filled with helium at .001 mm Hg pressure?

Do i need a strong magnet to affect the plasma?

any comments?

Thanks

Richard Smit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tuning trouble

Date: Tue, 11 Nov 1997 00:09:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Mon, 10 Nov 1997 18:09:26 -0600

Subject: Tuning trouble

OK.  So I fixed the holes in the sec' insulation, tapped the primary at

11-2/3 turns, fired it up, and blamm, out through the sec insulation again

(allthough I couldn't see the burned spot at first, I thought maybe I had

bought a brand of conductive polyurethane :)

Then I raise the sec' 4", and add a grounded strike ring, blamm, same thing

happens (except now the breakout hits the strike ring and doesn't set the

safety gap a sparkin' like it did before).

So (thinking I'm still overcoupled) I try tapping only 5 turns, a point well

below the base of the sec', and same thing.  Then I notice the burned

through spots, and figure the adjustments made "post-burn" don't mean a

thing cuz the current never reached the discharge terminal, right?

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla-2 archives

Cc:

Bcc:

X-Attachments:

To all,

I have placed an 8th archive of the Tesla-2 list online.

The archive file name is tes2arc8.zip and contains the

posts for September and October.

The archives are available at:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

They are also available on Dave Bronakowski's ftp site:

ftp://ftp.velocity.net/usr/djb/tesla/tesla2/

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Plasma globes and pressur

Date: Tue, 11 Nov 1997 08:47:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Tue, 11 Nov 1997 08:47:26 -0000

Subject: Plasma globes and pressure

Hi Guys,

I've just put together a vacuum pump, a pickle jar!, and a flyback

transformer in the hope of producing plasma. My pump would only go down to

50mm Hg unfortunately but if I squinted like mad in total darkness I could

see thin streamers traverse the length of the jar! I didn't expect miracles

due to the poor pump I have but any result is a step in the right

direction. I guess that if I could reach the 1mm target pressure my pickle

jar would light up like a Christmas tree, right??

Anyway, I found a good article somewhere among Bill Beaty's web page on

plasma at room (1 At) pressure using Argon and Helium. I intend to follow

the advice and fork out for some Argon of reasonable purity - does anyone

know if 98% pure Argon will work at 1 Bar as good as 99.98 % pure Argon??

Also, will the range of pressure from 50mm Hg to 760mm Hg vary the type of

discharge attained? The article explained Helium would leak through just

about anything given enough time which makes sense, but since Argon is

heavier than air I expect whatever glass jar I use for display purposes

will not leak the Argon, thus retaining display properties. Is this a

correct assumption?? I'll find most of the answers experimentally but wish

to avoid time consuming and costly mistakes if at all possible.

My next project will be to construct a Tesla coil to power the Argon globe

(should everything work O.K) but only need a small one about 3 to 4 inches

high and preferably transistorised. I appreciate Mike Hammers advice about

unreliable transistor drivers for Tesla coils, but alas I only need watts

of power not megawatts!!! If anyone has a good design for one with

construction details I'll be truely grateful.

Regards,

Stuart.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla-2 archives

Date: Tue, 11 Nov 1997 10:40:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kent Vander Velden" <graphix@iastate.edu>

Date: Tue, 11 Nov 1997 04:40:01 -0600

Subject: Re: Tesla-2 archives

>The archives are available at:

>ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

>

>They are also available on Dave Bronakowski's ftp site:

>ftp://ftp.velocity.net/usr/djb/tesla/tesla2/

There is also a mirror of ftp://nic.funet.fi/pub/sci/electrical/tesla/

local to US at ftp://toybox.cc.iastate.edu/pub/tesla/funet/

Changes at funet are reflected on toybox at ~12am CST.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Pole pig

Date: Tue, 11 Nov 1997 15:04:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 11 Nov 1997 10:04:10 EST

Subject: Pole pig

---------------------------- Forwarded with Changes ---------------------------

From: Paul S. Marshall at EES01_TECHSVC

Date: 11/11/97 9:40AM

To: Addressing Open at SWIFTMAIL

Receipt Requested

Subject: Pole pig

-------------------------------------------------------------------------------

Fellow coilers,

I finally got a pole pig. Its rated at 25 kilowatts. The primary is

13,200 v. The secondary is 120 v. I'm using a 2 kilowatt load in

series with the primary to limit current. I'm probably getting a

maximum of 140 ma. for a total of 1800 watts. (Nice arc). I also

bought a 240v Variac 2 kilowatts. To date we have only discussed

the upper end of cap design. You know what's the largest cap we can

charge ? Well my question is what is the minimum capacitance I need to

drive my coil ? I eventually intend to raise the power output to

5 or 6 kw. My coil is almost identical to yours. The one thing that I

like about having the pole pig is the ability to run at variable power

simply by changing series loads.

1.) Will I still need a safty gap ?

2.) Can I use a rotary gap, or will a static gap still be the way

to go?

PS: I got 15 gallons of free transformer oil, with my purchase.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Cap

Date: Tue, 11 Nov 1997 15:23:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Tue, 11 Nov 1997 16:23:40 +0100

Subject: Cap

Hi!

I need some help, I'm about finishing my first tesla coil. But i got the cap

left. So could you give me some dimensions of the cap and how many layers?

I'm using polyethylene sheet used for coversheet in building and stuff

and some aluminium foil (baking type, a little bit thicker than ordinary if

you know what I mean) As you may have understood, i want to build a plate cap.

I don't know what it's called in the U.S. But can anyone help me with some

dimensions of a cap. I think i need around 0.02uF(?).

Coil specs

primary: 10 turns with 6mm copper tubing. flat type winding (know i need

a saucer shaped but i use this for a while, and I need more turns too)

Tranny: 2 parallel 15kV 20mA oilburner transformer.

Secondary: height 53.3cm about 975 turns with 24AWG copper wire.

A 4.33"(110mm) PVC tubing which has been dried and varnished. A 8mm

plexiglass disc as used for capping the top. But now it's the capacitor

left. So if you would like to help me with this i would be very glad...

Any answers appreciated.

Thanks,

Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plasma, IS IT FLAMABLE?

Cc:

Bcc:

X-Attachments:

At 07:30 PM 11/10/97 GMT, you wrote:

>From: esmit@ilink.nis.za

>Date: Mon, 10 Nov 1997 21:30:22 +0200

>Subject: Plasma, IS IT FLAMABLE?

>

>Hi,

>

>Long time no write.

>

>I have a question.

>I want to build a "plasma gate" a theory i have about a kind of

>dimensional/black hole that can be created through plasma. I just want to

>know if i use say neon/Helium or whatever, is it flammable. Will it burst

>into flames if i put my TC into the gas.

>

>I really want to experiment and test my theory with plasma. I want to put

>the gas(helium) into a long glass tube and are afraid that the gas might

>explode and glass will hit me.

>

>How does a person get so a great vaccuum in the glass tube?

>say if i use helium, and put it at 1 mm Hg(whatever that is) and make plasma

>what is its effect against a tube filled with helium at .001 mm Hg pressure?

>Do i need a strong magnet to affect the plasma?

>

Richard,

Plasma globes are filled mainly with inert gases. Either rarified

air or some mixture of noble gases like helium, neon and argon.

These gases are chemically inert. They will not combine with

anything and will not explode.

However when pulling a vacuum on a glass vessel there is always

the risk of implosion due to the vacuum. Use only heavy walled

vessels and place some sort of protective screen between you

and the glass vessel.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning trouble

Cc:

Bcc:

X-Attachments:

At 12:09 AM 11/11/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Mon, 10 Nov 1997 18:09:26 -0600

>Subject: Tuning trouble

>

>

>OK.  So I fixed the holes in the sec' insulation, tapped the primary at

>11-2/3 turns, fired it up, and blamm, out through the sec insulation again

>(allthough I couldn't see the burned spot at first, I thought maybe I had

>bought a brand of conductive polyurethane :)

>

> Then I raise the sec' 4", and add a grounded strike ring, blamm, same thing

>happens (except now the breakout hits the strike ring and doesn't set the

>safety gap a sparkin' like it did before).

>

>So (thinking I'm still overcoupled) I try tapping only 5 turns, a point well

>below the base of the sec', and same thing.  Then I notice the burned

>through spots, and figure the adjustments made "post-burn" don't mean a

>thing cuz the current never reached the discharge terminal, right?

>

>

>Chris

Chris,

Your results seem rather odd. Are you saying that you are still

getting breakouts from the center of the secondary winding

despite raising the secondary 4 inches away from the primary

and varying the primary tap position from 5 to 11+ turns?

Either you were so grossly overcoupled to begin with that

raising the secondary 4 inches was insufficient or you

are not changing the tuning by varying the tap position.

That could be due to a wiring error.

You should be able to throw the coil far enough out of tune

that no sparks at all will form.

Check your wiring against the schematic diagram again carefully.

Patch the damage to your secondary and raise the secondary up

a couple more inches. Keep experimenting with it.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Pole pig

Cc:

Bcc:

X-Attachments:

At 03:04 PM 11/11/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 11 Nov 1997 10:04:10 EST

>Subject: Pole pig

>

>

>     Fellow coilers,

>       I finally got a pole pig. Its rated at 25 kilowatts. The primary is

>     13,200 v. The secondary is 120 v. I'm using a 2 kilowatt load in

>     series with the primary to limit current. I'm probably getting a

>     maximum of 140 ma. for a total of 1800 watts. (Nice arc).

Using resistive ballast you are most likely not getting 13200 volts

either as you are not getting 120 volts to the pig.

What you need is an inductive ballast like a arc welder with just

enough resistive ballast to smooth out the inductive surges.

>     I also bought a 240v Variac 2 kilowatts.

Thats a pretty small variac. Only a little over 8 amps at 240V.

Probably handle 10 or 12 OK.

>

>     To date we have only discussed the upper end of cap design.

>     You know what's the largest cap we can charge ?

>     Well my question is what is the minimum capacitance I need to

>     drive my coil ? I eventually intend to raise the power output to

>     5 or 6 kw. My coil is almost identical to yours. The one thing that I

>     like about having the pole pig is the ability to run at variable power

>     simply by changing series loads.

The minimum required capacitance is what will bring your coil to resonance

with most of your primary turns tapped in. You want to use as large a cap

as you can to get the biggest bang. With a pig you dont need to worry

about impedance matching as you have plenty of power to charge whatever

size cap you select.

>

>     1.) Will I still need a safty gap ?

Yes. Pigs are very rugged but not indestructible. A properly

set safety is also a good indicator of when things are not working

right. A constantly firing safety is usually an indication of

a problem like too wide a main gap or poor tuning.

>     2.) Can I use a rotary gap, or will a static gap still be the way

>         to go?

>

A good static gap will work on a pig. Especially a vacuum quenched

design. However as you start getting the power level up above 2 KVA

a rotary begins to show some advantages.

>     PS: I got 15 gallons of free transformer oil, with my purchase.

15 extra gallons? Or you mean the oil in the pig?

If you got 15 extra gallons thats just great.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap

Cc:

Bcc:

X-Attachments:

At 03:23 PM 11/11/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Tue, 11 Nov 1997 16:23:40 +0100

>Subject: Cap

>

>

>

>

>

>Hi!

>

>I need some help, I'm about finishing my first tesla coil. But i got the cap

>left. So could you give me some dimensions of the cap and how many layers?

>

>I'm using polyethylene sheet used for coversheet in building and stuff

>and some aluminium foil (baking type, a little bit thicker than ordinary if

>you know what I mean)

What is the thickness of your polyethylene sheeting?

Thats important information I need to know before laying

out a possible cap design.

>As you may have understood, I want to build a plate cap.

>

>I don't know what it's called in the U.S. But can anyone

>help me with some dimensions of a cap. I think i need around 0.02uF(?).

No. .02 mfd is way too much for your two 15kv/20ma OBITs.

The proper matching cap size would be .0035 mfd for 1 transformer

and up to .007 mfd for the two in parallel.

The method for determining proper cap size was posted recently.

I would plan for using the two units in parallel and build the larger

cap. With only 10 primary turns and a small cap you may not be able

to tune to a very low frequency. Better start with a small terminal

on your secondary.

Think seriously about adding at least 4 more turns to your primary.

<snip specs>

For 15kv you want to have a minimum of 60 mils of poly

split between a minimum of 2 caps in series. Each cap will need

to have twice the needed capacitance value.

To get .007 mfd you will need two .014 mfd caps in series.

Give me the thickness of your polyethylene and I will recommend

a more specific cap design.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Hey! You experienced coil

Date: Wed, 12 Nov 1997 06:39:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Wed, 12 Nov 1997 00:39:58 -0600

Subject: Hey! You experienced coilers out there. . .

What gives?!?  First, no one mentioned the demonic barking noises that a

coil can generate, and now I find out that, although harmless, you do feel

a jolt from the sparks of a TC (OK, maybe it was just a tingle, but it did

startle me, especially so when the current jumps from my innocently

outstretched hand to the other that's holding the connection of the two

power cords I was using to turn the coil on and off, and then into that

connection.  And if feeling an electrical connection of any sort between

your hand and a source of 120vac doesn't get your attention, I don't know

what will).  So what is it with you guys, do you get a kick out of us

newbies scaring the hell out of ourselves as we discover the fun and

excitment of coilin'??  :)

But seriously, I do have a success to report.  I had ten inches of spark

arcing from the terminal to ground.  I know that aint much, but considering

it's the first I've seen over 3/16th of an inch, it's pretty exciting for me.

However, all is not well in mudville just yet.

If I tap more than ten turns I still get mid-sec' break-out.  Any thoughts

on why, or is this just normal over-coupling?  And will a larger discharge

terminal help crank up the power without break-out (I'm gonna build a 4x??"

toroid, if it will help).

And, I'm getting a pretty significant corona off of the top sec' turn,

especially where the winding stops and turns at a 90 degree angle to connect

to the discharge terminal.  Anyway to fix that?

Also, is it normal for a corona to form between the grounded target and the

discharge terminal before the spark jumps in toto?  The corona would form,

then fingers of sparks would start to dance off both the terminal and the

ground, within the corona, before the spark jumped (as distance between the

two decreased).  And is it OK to use the same ground source for the target

as the system?  I was thinking that there may be some detriment to

performance if there was such a close completion of the circuit.

And finally, getting back to my very first question, Yeah, I guess I'll get

a kick out of hearing the next guy complain about not being warned, since it

already happened to me :)

Thanks All . . . Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Hey! You experienced

Date: Wed, 12 Nov 1997 13:54:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Wed, 12 Nov 1997 08:54:12 -0500

Subject: Re: Hey! You experienced coil

Chris:

I've been coiling for almost a year now and I've never been shocked.  Perhaps

you need to look at your safety measures and improve them.  I would be leary of a

strike from my coil it could be a life changing experience.

Needless to say...I'm told that you ain't an experienced coiler until you get

bit once.

I'm not sure what the specs are on your coil, but you should use a different

ground for your target then the system ground.  I also have corona form between

discharge terminal and target before a strike...it's a nice experience.

Keep experimenting with your coil...try different discharge terminals of varying sizes.  You may also want to add some primary turns...I originally had 13 turns

but had to add two more to initially tune my system.  I realize some of the

frustration you're having...but realize that this is all part of the fun!

Good Luck,

Kevin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Hey! You experienced coil

Cc:

Bcc:

X-Attachments:

At 06:39 AM 11/12/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Wed, 12 Nov 1997 00:39:58 -0600

>Subject: Hey! You experienced coilers out there. . .

>

>

>What gives?!?  First, no one mentioned the demonic barking noises that a

>coil can generate, and now I find out that, allthough harmless, you do feel

>a jolt from the sparks of a TC (OK, maybe it was just a tingle, but it did

>startle me, especially so when the current jumps from my innocently

>outstretched hand to the other that's holding the connection of the two

>power cords I was using to turn the coil on and off, and then into that

>connection.  And if feeling an electrical connection of any sort between

>your hand and a source of 120vac doesn't get your attention, I don't know

>what will).  So what is it with you guys, do you get a kick out of us

>newbies scarring the hell out of ourselves as we discover the fun and

>excitment of coilin'??  :)

Well Chris getting a good jolt from your coil is almost a right

of passage. I have posted on here before many times that taking

streamer hits to the body is not a good idea. Its not really

dangerous with small coils but with larger coils putting out

intense streamers it can be dangerous.

While operating my coil several months ago I reached in to

pick up something and a 40 + inch streamer came out and tapped

me on the back of the hand. It hurt like hell and I still have

a scar from the RF burn.

Also note that the voltage and energy in the primary circuit

of a coil are lethal. Dont ever put your self in a position

where you could come in contact with the live primary circuit.

No second chances here.

>

>But seriously, I do have a success to report.  I had ten inches of spark

>arcing from the terminal to ground.  I know that aint much, but considering

>it's the first I've seen over 3/16th of an inch, it's pretty exciting for me.

Good to hear you are getting some success. Just takes a little

experimentation.

>

>However, all is not well in mudville just yet.

>

>If I tap more than ten turns I still get mid-sec' break-out.  Any thoughts

>on why, or is this just normal over-coupling?  And will a larger discharge

>terminal help crank up the power without break-out (I'm gonna build a 4x??"

>toroid, if it will help).

As you tap in more primary turns your coupling will increase some.

And mild overcoupling may not be visible at lower power levels.

So as you increase the power be getting closer to resonance

its not unusual to have to decrease coupling.

A larger terminal will help by loading down the secondary.

The larger terminal will require you to tap in even more primary

turns.

Whenever I have had any large scale primary/secondary

breakdowns I put a dab of epoxy over the pit and when thats

dry slap another coat of varnish on the secondary.

My secondary probably has about 10 coats on it now.

That nice thick coating enhances the insulation.

Using 4 inch duct around a couple pie pans results in a

good 16 X 4 toroid. This is a good size for a 4 inch

secondary.

>

>And, I'm getting a pretty significant corona off of the top sec' turn,

>especially where the winding stops and turns at a 90 degree angle to connect

>to the discharge terminal.  Anyway to fix that?

Yep.

Get your terminal or toroid down as close to the top of the

secondary winding as possible. Certainly no more than a couple

inches away. If you have to lop off some of the top of the

secondary form then by all means do so.

Changing to a larger toroid will also help.

>

>Also, is it normal for a corona to form between the grounded target and the

>discharge terminal before the spark jumps in toto?  The corona would form,

>then fingers of sparks would start to dance off both the terminal and the

>ground, within the corona, before the spark jumped (as distance between the

>two decreased).

Yep. Thats the way it works.

> And is it OK to use the same ground source for the target

>as the system?  I was thinking that there may be some detriment to

>performance if there was such a close completion of the circuit.

Yes thats fine. No problem at all in doing that.

>

>And finally, getting back to my very first question, Yeah, I guess I'll get

>a kick out of hearing the next guy complain about not being warned, since it

>already happened to me :)

>

>Thanks All . . . Chris

Well Im sure its been a well learned lesson.

Im sure you have developed the proper respect for the voltages present.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Flyback Problems?

Date: Wed, 12 Nov 1997 21:36:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Wed, 12 Nov 1997 21:36:26 -0000

Subject: Flyback Problems?

Eureka!!!!,

I've introduced pure Argon into my plasma globe jar and guess what?, I've

succeeded in producing plasma! There's only one slight snag... I can only

achieve the plasma if I arc a grounded piece of wire close to the high

voltage electrode that's powering the globe. When I simply touch the glass

at the other end of the jar with a grounded wire I see no plasma...WHY????

Is it because my flyback isn't switching properly until it drives a load

ie. arc? Someone please help me find the solution before I give up and buy

one!!

Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Vacuum pump

Date: Wed, 12 Nov 1997 22:33:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Wed, 12 Nov 1997 17:33:50 -0500 (EST)

Subject: Vacuum pump

Hi,

I have this automobile electric air pump. You know the kind. It just plugs

into your cars cigarette lighter and you flip the switch and you can pump

air into your cars tire, etc. I took off the back cover of it to see the

inside, and all it is is what looks to be a positive displacement pump. It's

an electric motor connected to a wheel which rotates and drives a piston

into a chamber to pump air, and I am assuming there is a check valve in

there so the air only goes one way.

My question is if you built a housing for the pump so its intake was no

longer the atmosphere, but a container such as a plasma globe or a capacitor

you wanted to pump down, could you use this device to do it? Any idea how

much vacuum you could pull if you could? I bought this on sale at K-mart for

less than $10 awhile ago so it might be a cheap alternative if it works.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Plasma Globes

Date: Thu, 13 Nov 1997 01:18:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Thu, 13 Nov 1997 01:18:12 -0000

Subject: Plasma Globes

Hi All,

Just a quick report on my findings while playing with my plasma globe

setup. I discovered a very small leak in my vacuum pump fittings which

effectively halved the altimate vacuum acheivable. I've now got at best

25mm Hg. Although this is much higher than all the articles I've read on

producing plasma, I still acheived a reasonably faithful reproduction of a

store bought model. Just to get things a bit brighter I back filled my

globe to 1 Bar with Argon then repumped down to 25mm. The effect was to at

least quadruple the light intensity of the plasma.

One problem I did have was the connection of the globe to the flyback.

I discovered that you must position the H.T output as close as physically

possible to the globe with no long leads in between. Any corona arcing to

ground effectively bypassed the globe thus producing zero plasma. Now that

I know how to build these things I'll get a decent globe (not the pickle

jar I've used so far!) and construct a central terminal like a store bought

globe. I need to redesign the severly overheated 3055 flyback circuit with

FETs and utilise a more sophisticated control of the high voltage output by

monitoring the primary coil current, and also adding frequency control. If

anyone's interested in a copy of my proposed design, E-Mail me......

Forgive any spelling mistakes since it's 12 A.M in England now and I'm

whacked! I've spent the last 5 hours playing with the gear above. Looks

like I'll have to make it up to the missus!!!!

Regards,

Stuart.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: coupling height ?

Date: Thu, 13 Nov 1997 04:27:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Wed, 12 Nov 1997 23:27:12 -0500

Subject:  coupling height ?

ok i finally finished my primary, and prepared a winding setup for the secondary.

my question is what would be a good starting height for the secondary in

relation to the primary.

I have made the secondary height adjustable via scissors jack underneath.

it has a range of 9 inches travel.

Iam trying to decide a minimum and max distance from the primary inside turn

to the bottom of the secondary winding so I can start winding in the correct place.

my specs

secondary 10.75" dia 34" planned length  18ga wire approx 800 turns

primary    21 turns .368dia tubing   30 deg inverted spiral

inside rad 7.1"c/l of tube  outside rad 20.74 c/l

probably  a little extra but I can always remove some should it be

necessary.

I know its overkill but the plexiglass forms for the primary were cnc

milled to provide exact spacing

and  angle but most of all  because it looks very neat.

Being a cnc programmer I have free use of cnc equipment.

also- anyone care to estimate how much capacitance i should start with?

I figured the extra primary turns should require less cap's

thanks in advance for any help.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Vacuum pump

Date: Thu, 13 Nov 1997 13:02:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

Date: Thu, 13 Nov 1997 08:02:08 -0500

Subject: Re: Vacuum pump

I purchased a refurbished refrigerator compressor from the owner of a

refrigerator repair company for $20.00. (Actually two at $20.00 ea.,

one being a big Copeland industrial compressor from a Fedders

commercial fridge.)  I hooked the small one up to a capacitor and was

able to pull 26.5" Hg according to the cheapo gauge I put inline.

Sadly, a combination of overworking the compressor and letting castor

oil from the cap leak into the compressor has damaged it somewhat.

Now it needs to be warmed-up to start cold and only pulls 25" Hg,

which is still enough to make the oil in the cap. bubble quite a bit.

I've learned that a blower aimed at the compressor is essential to

keep its temperature down.

I hear that the best refrigerator pumps for compressor work are rotary

compressor.  I've never played with one of those.

26.5" Hg is still considered a "coarse" vacuum, and I doubt the pump you

describe will be able to pull that much mercury.  However, a vacuum gauge

can be had for only $9.00 from your friendly neighborhood Grainger depot.

Allegedly, they sell only to businesses, but I've never had a problem

buying stuff for myself.

Just saunter in as if you own the place and tell them what you want.  If

they ask you were you work, tell them, but add you want to put it on your

credit card.

Aric

______________________________ Reply Separator ________________________________

_

Subject: Vacuum pump

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    11/12/97 10:33 PM

My question is if you built a housing for the pump so its intake was no

longer the atmosphere, but a container such as a plasma globe or a capacitor

you wanted to pump down, could you use this device to do it? Any idea how

much vacuum you could pull if you could? I bought this on sale at K-mart for

less than $10 awhile ago so it might be a cheap alternative if it works.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coupling height ?

Cc:

Bcc:

X-Attachments:

At 04:27 AM 11/13/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Wed, 12 Nov 1997 23:27:12 -0500

>Subject:  coupling height ?

>

>ok i finally finished my primary, and prepared a winding setup

>for the secondary. my question is what would be a good starting

>height for the secondary in relation to the primary.

With a 30 degree conical primary start with the bottom turn

of the secondary approximately in the middle of the primary.

This will probably overcouple some so if you get any pri/sec

arcing raise the secondary up a half inch at a time till the arcing stops.

>I have made the secondary height adjustable via scissors jack underneath.

>it has a range of 9 inches travel.

A metal jack? Might not be a good idea to have that amount of metal

in the primary field.

>I am trying to decide a minimum and max distance from the primary

>inside turn to the bottom of the secondary winding so I can start

>winding in the correct place.

This is something very hard to predict beforehand. Its one of

those parameters that has to be determined experimentally.

>

>my specs

>secondary 10.75" dia 34" planned length  18ga wire approx 800 turns

>

>primary    21 turns .368dia tubing   30 deg inverted spiral      inside

>radi 7.1"c/l of tube  outside radi 20.74 c/l

>

>probably  a little extra but I can always remove some should it be

>necessary.

>

>I know its overkill but the plexiglass forms for the primary were cnc

>milled to provide exact spacing

>and  angle but most of all  because it looks very neat.

>Being a cnc programmer I have free use of cnc equipment.

>

>also- anyone care to estimate how much capacitance i should start with?

>I figured the extra primary turns should require less cap's

The capacitance is largely determined by your power transformer.

You want to use as large a cap as your transformer can charge.

But you also dont want so large a cap that you are only using

3 or 4 primary turns.

Let me know what size of transformer you are planning to use on this

coil.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plasma Globes

Cc:

Bcc:

X-Attachments:

At 01:18 AM 11/13/97 GMT, you wrote:

>From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

>Date: Thu, 13 Nov 1997 01:18:12 -0000

>Subject: Plasma Globes

>

>Hi All,

>        Just a quick report on my findings while playing with my plasma globe

>setup. I discovered a very small leak in my vacuum pump fittings which

>effectively halved the altimate vacuum acheivable. I've now got at best

>25mm Hg. Although this is much higher than all the articles I've read on

>producing plasma, I still acheived a reasonably faithful reproduction of a

>store bought model. Just to get things a bit brighter I back filled my

>globe to 1 Bar with Argon then repumped down to 25mm. The effect was to at

>least quadruple the light intensity of the plasma.

<SNIP>

Stuart,

Thanks for the report. So argon does seem to produce

decent plasma at rather moderate vacuum. Have you tried any

helium yet?

I plan on doing some plasma globe experiments this winter

as soon as I clear out the 1000 projects Ive already started.

Keep up the good work.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plasma Globes

Date: Fri, 14 Nov 1997 16:04:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Fri, 14 Nov 1997 10:04:59 -0600

Subject: Re: Plasma Globes

> Stuart,

>

> Thanks for the report. So argon does seem to produce

> decent plasma at rather moderate vacuum. Have you tried any

> helium yet?

>

> I plan on doing some plasma globe experiments this winter

> as soon as I clear out the 1000 projects Ive already started.

>

> Keep up the good work.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike, Stuart et al:

Been kinda following the whole plasma globe thing and thought I'd add my

2 cents. In general, the inert gases increase in 'resistance' as the

atomic number decreases. Helium has a much higher resistance than neon

which is why it is almost never used in luminous tubes ('neon' signs).

For the hobbyist, argon is the most economical choice.  You can buy it

from any sign supply house. It actually comes as a mixture of argon and

neon called 1050 (80% argon/20% neon). You can also buy lecture bottles

of argon, but they contain many liters of gas and you only need a

milliliter or so for the average plasma globe. Argon has a much lower

'resistance' and allows more latitude with regard to pressure and

voltage and the globe will run cooler. If you have the $$$ you could try

krypton for nicer color.  Xenon is pretty boring color wise and quite

expensive. Also, to the persons pulling a vacuum on a pickle jar: don't

do it! No commercial food jars, especially the larger ones, can safely

contain a vacuum, they are very thin and the slightest flaw can cause

them to fail. A 250 ml (no larger) round bottom flask works nice and the

ground glass fitting makes the evacuation and introduction of the gas

much easier. I would recommend the Kontes Chemistry and Life Sciences

Products catalog as a great source of really cool glassware. It's worth

a look. They have a lot of high vacuum glassware, fittings, valves etc.

If you have skill with a torch you can make almost anything.

Eric Davidson

edavidson@icva.gov

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plasma Globes

Cc:

Bcc:

X-Attachments:

At 04:04 PM 11/14/97 GMT, you wrote:

>From: Eric Davidson <edavidson@icva.gov>

>Date: Fri, 14 Nov 1997 10:04:59 -0600

>Subject: Re: Plasma Globes

>

>Mike, Stuart et al:

>

>Been kinda following the whole plasma globe thing and thought I'd add my

>2 cents. In general, the inert gasses increase in 'resistance' as the

>atomic number decreases. Helium has a much higher resistance than neon

>which is why it is almost never used in luminous tubes ('neon' signs).

>For the hobbyist, argon is the most economical choice.  You can buy it

>from any sign supply house.

My experiments along that line were done using some argon from a

local welding shop. Seemed to work well.

>You can buy it from any sign supply house. It actually comes as a

>mixture of argon and neon called 1050 (80% argon/20% neon).

Now this I didnt know. This sounds most interesting. What color

plasma does this combination yield if you know?

>Also, to the persons pulling a vacuum on a pickle jar: don't

>do it! No commercial food jars, especially the larger ones, can safely

>contain a vacuum, they are very thin and the slightest flaw can cause

>them to fail.

Yes this is good advice. My experiments were with a Ball

quart jar meant for home canning. Its much heavier than

your typical grocery store jar.

An implosion of a glass vessel would not be any fun at all.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: TC's & Door Openers

Date: Sat, 15 Nov 1997 02:18:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Fri, 14 Nov 1997 20:18:29 -0600

Subject: TC's & Door Openers

Has anyone heard of the RF output of a TC scrambling, say, the reciever on a

garage door opener?  I increased the size of the antenna on my opener some

time ago and never had any trouble with it.  And it worked fine right up

until the second time I succesfully ran my TC.  Coincidince?  (I re-tuned

the opener to recieve the signal from the remote and it works fine)

No biggee, just thought it worth mentioning.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Glass Globes

Date: Sat, 15 Nov 1997 14:15:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Sat, 15 Nov 1997 14:15:18 -0000

Subject: Glass Globes

Hi All,

Does anyone know where to buy decent glass globes for plasma studies?

I can't seem to find any lighting stores fixtures that are suitable.....

Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC's & Door Openers

Cc:

Bcc:

X-Attachments:

At 02:18 AM 11/15/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Fri, 14 Nov 1997 20:18:29 -0600

>Subject: TC's & Door Openers

>

>

>Has anyone heard of the RF output of a TC scrambling, say, the reciever on a

>garage door opener?  I increased the size of the antenna on my opener some

>time ago and never had any trouble with it.  And it worked fine right up

>until the second time I succesfully ran my TC.  Coincidince?  (I re-tuned

>the opener to recieve the signal from the remote and it works fine)

>

>No biggee, just thought it worth mentioning.

>

Chris,

I have heard numerous reports of people destroying their

garage door openers with their Tesla coils.

At the very least remove the antenna and unplug the opener from

the power before running your coil. You may consider constructing

a metal shield to go around your opener when running your coil.

Modern electronics and Tesla coils do not mix very well.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Glass Globes

Cc:

Bcc:

X-Attachments:

At 02:15 PM 11/15/97 GMT, you wrote:

>From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

>Date: Sat, 15 Nov 1997 14:15:18 -0000

>Subject: Glass Globes

>

>Hi All,

>        Does anyone know where to buy decent glass globes for plasma studies?

> I can't seem to find any lighting stores fixtures that are suitable.....

>        Regards,

>        Stuart

Stuart,

I cant help you with sources in the UK but a local

home improvement store here (Lowes) has some nice 8 inch

diameter globes that Ive been thinking about.

These seem to be of fairly heavy construction and should hold

a vacuum OK.

This is a project I want to work on this winter and your success

has encouraged me.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Light Dimmer

Date: Sat, 15 Nov 1997 20:58:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Mike Wilson <nwilson@skyenet.net>

Date: Sat, 15 Nov 1997 15:58:59 -0500

Subject: Light Dimmer

Hello,

I was wondering if anybody has try to use a light dimmer to control

a neons voltage?

Thanks

Mike Wilson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Light Dimmer

Cc:

Bcc:

X-Attachments:

At 08:58 PM 11/15/97 GMT, you wrote:

>From: Mike Wilson <nwilson@skyenet.net>

>Date: Sat, 15 Nov 1997 15:58:59 -0500

>Subject: Light Dimmer

>

>Hello,

>    I was wondering if anybody has try to use a light dimmer to control

>a neons voltage?

>

>Thanks

>Mike Wilson

>

Mike,

Light dimmers are typically triac controlled devices and not

suitable for inductive loads like a transformer. It will probably

be destroyed almost instantly.

It works by cutting off power at different points along the 60 cycle

sine wave feeding the light. It doesnt actually vary the voltage

at all. There is just no substitute for a variac to control your

transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: help with cap.

Date: Sun, 16 Nov 1997 15:18:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Sun, 16 Nov 1997 16:18:25 +0100

Subject: help with cap.

Hi,

I've sent a mail to this list regarding if you could help

me with making a capacitor. But I forgot to give you

the specs of the poly sheeting. It's 0.2mm thick,

about 39.37 mils. I cant reply the old mail because

I've formatted my hd. But I think you said that I would need

about 0.035uF. So, what dimensions and how many layers

should I do? I'm using baking type aluminium foil.

Thanks,

Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Light Dimmer

Date: Sun, 16 Nov 1997 20:51:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 16 Nov 1997 15:51:57 -0500

Subject: Re: Light Dimmer

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Light Dimmer

> Date: Saturday, November 15, 1997 6:28 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 15 Nov 1997 17:28:12 -0600

> Subject: Re: Light Dimmer

>

> At 08:58 PM 11/15/97 GMT, you wrote:

> >From: Mike Wilson <nwilson@skyenet.net>

> >Date: Sat, 15 Nov 1997 15:58:59 -0500

> >Subject: Light Dimmer

> >

> >Hello,

> >    I was wondering if anybody has try to use a light dimmer to control

> >a neons voltage?

> >

> >Thanks

> >Mike Wilson

> >

>

> Mike,

>

> Light dimmers are typically triac controlled devices and not

> suitable for inductive loads like a transformer. It will probably

> be destroyed almost instantly.

>

> It works by cutting off power at different points along the 60 cycle

> sine wave feeding the light. It doesnt actually vary the voltage

> at all. There is just no substitute for a variac to control your

> transformer.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike & Mike,

Mike Hammer is right about the results, but just a hair off on the

reason why.

Actually a Triac works by turning ON at some point AFTER the zero

crossing point along an AC waveform. It then STAYS on until the

next zero crossing. If we could turn such a device OFF whenever

we wanted, we would be very happy. Unfortunately, we can only

control the turn ON point.

The poor transformer does NOT see a nice clean sine wave, but a

waveform with a fast rising edge more like the leading edge of a

square wave. Coils do not like fast rising waves! They respond

by further distorting the waveform and dumping out of phase

energy into the circuit mix. The triac now sees a spike of

several hundred volts, and is often destroyed by it.

The absolute worst case is when the triac is gated on at the instant

the AC is at its peak. This corresponds to the 90 and 270

degree points. Peak currents and peak voltages can now far exceed

the triac's ratings.

Even if the triac managed to survive, the poor transformer

would be going wild with the strange waveforms, and there

may even be damage to other electrical items connected up

on the same electrical wiring. Like your living room stereo

or TV. The damage you can do is amazing!!

A variac, on the other hand, produces clean sine waves

of varying amplitude. Transformers thrive on such stuff.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: help with cap.

Cc:

Bcc:

X-Attachments:

At 03:18 PM 11/16/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Sun, 16 Nov 1997 16:18:25 +0100

>Subject: help with cap.

>

> Hi,

>Ive sent a mail to this list regarding if you could help

>me with making a capacitor. But I forgot to give you

>the specs of the poly sheeting. Its 0.2mm thick,

>about 39.37 mils. I cant reply the old mail because

>Ive formatted my hd. But i think you said that i would need

>about 0.035uF. So, what dimensions and how many layers

>should I do? I'm using baking type aluminium foil.

>Thanks,

>Christopher Eriksson

>

Chris,

I calculate that .2 mm poly sheeting would be just under 8 mils.

7.9 mils to be exact. 39.37 mils would be 1 mm.

So I am proceeding on the assumption you have 8 mil poly.

This is a good size for caps.

In a previous post you said you had two 15kv/20ma OBITs.

(oil burner ignition transformer)

The proper cap for these would be .0035 mfd for one or .007

mfd if you ran both transformers in parallel.

I recommend running the transformers in parallel

for 15kv/40ma and building a .007 mfd cap to match.

For 15kv a minimum of 2 caps in series is required

and a minimum of 30 mils of poly in each.

So assuming we need .007 mfd and two series caps each one

needs to be .014 mfd.

So lets try this design out for a flat plate cap.

Using 18  8 X 8 inch plates (actual overlapping area)

and 4 layers of 8 mil poly for each dielectric layer yields

about .015 mfd.

Construct two of these caps and put them in series for .0075 mfd.

This gives you the equivalent of 64 mils of poly dielectric.

This should be adequate for 15kv provided you dont try to

open your gap too wide.

A somewhat more robust design would be three .021 mfd caps

in series.

For this cap use 3 layers of 8 mil poly for each dielectric layer.

Using 8 X 8 plates you would need 20 plates for .022 mfd.

Build three of these caps and wire them in series and this will

make a very robust .0073 mfd cap. This gives you the equivalent

of 72 mils of poly dielectric.

If you are going to the effort the build a cap I would

go with the second design. Its more sure to survive and

give long service.

If you make it in a vertical plate configuration it wont need

to be vacuum pumped. The air will rise out slowly on its

own given sufficient time. Run the cap with very narrow

gap at first to break it in slowly.

Also a plate cap is expandable. If later you get a larger

transformer and want a little more capacitance you can just

add more plates to your existing cap.

These are just a couple possible designs. Lots of different

configurations are possible.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: high voltage input

Date: Mon, 17 Nov 1997 02:58:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Sun, 16 Nov 1997 20:58:51 -0600

Subject: high voltage input

Organization: MailExcite  (http://www.mailexcite.com)

Hey eveybody! I was browsing on the net and I came across some guy

( Tedd Payne tpayne@netnitco.net) who is selling a 75kvAC power supply.

Has anyone bought anything from this guy?

He sells alot of stuff like this. Also, do I want to use 75kv as my primary input

for my TC? I've never even heard of a regular transformer putting out 75kv - is

this possible, and is so, should I use it in my TC? Any info is appreciated!

--Jay

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: TC's & Door Openers

Date: Mon, 17 Nov 1997 03:55:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kevin A. Zachary" <kzachary@kih.net>

Date: Sun, 16 Nov 1997 22:55:59 -0500

Subject: Re: TC's & Door Openers

> Modern electronics and Tesla coils do not mix very well.

What is the 'recommended' distance to keep most electronic devices away

from your Tesla Coil?

I have noticed some of the Tesla coil pictures where arcs are striking

garage door openers, the electric cord that supplies power to the coil,

etc... This brings about a question or a comment that I heard several years

ago... Is it possible for a Tesla Coil to somehow shoot the high voltage it

produces back into the house and eventually back to  the nearest

substation?

KevinZ

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: high voltage input

Cc:

Bcc:

X-Attachments:

At 02:58 AM 11/17/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Sun, 16 Nov 1997 20:58:51 -0600

>Subject: high voltage input

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Hey eveybody! I was browsing on the net and I came across some guy

>( Tedd Payne tpayne@netnitco.net ) who is selling a 75kvAC power supply.

> Has anyone bought anything from this guy? He sells alot of stuff like this.

No Ive never bought anything from Ted. He has been around for quite

a while and Ive never heard anything bad about him.

>Also, do I want to use 75kv as my primary input for my TC?

NO YOU DO NOT!

>I've never even heard of a regular transformer putting out 75kv - is

>this possible, and is so, should I use it in my TC?

>Any info is appreciated!

>

This is most likely a transformer from a X-ray machine. These

transformers often come as high as 125kv. Maybe even more.

Voltages above 15kv require extraordinary measures just to keep

the voltage where its supposed to be. Once you start getting above

20kv it becomes exceedingly difficult to prevent arcing.

And you dont need such high voltages. Its the power delivered

to the secondary thats important not just voltage. And consider

the difficulty of building a cap that can withstand that kind of

voltage.

Add to that the fact that such transformers are exceedingly

dangerous. Normal insulations just wont do.

On my coil I use only 12kv neons. I have never run short of power.

I am getting in excess of 50 inches of ouput from 12kv/100ma worth

of neons.

Stick with 9-15kv.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Ballast for pole pigs/pot

Date: Mon, 17 Nov 1997 09:25:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: esmit@ilink.nis.za

Date: Mon, 17 Nov 1997 11:25:44 +0200

Subject: Ballast for pole pigs/potential transformers.

don't speak to technically, i am new to this stuff, PLEASE?

Hi,

I have read that when you buy a pole pig or potential transformer and want

to run it on a Tesla coil you should use inductive ballasting or resistance

ballasting.

Will you answer some questions for me?

* How do you know if a transformer isn't current limited?

* What does an inductive ballast and resistive ballast do?

* What is an inductive ballast(i know it is an arc welder, but is that the

machine that you use to meld iron to each other?)

* I probably wont use my fathers arc welder (if it's what i think it is)

because it might be actually blow up or something, what else can i do?

* What is the difference between a pole pig and a potential transformer?

Thanks

Richard

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: high voltage input

Date: Mon, 17 Nov 1997 11:27:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Shane Randle" <randlec@scnc.addison.k12.mi.us>

Date: Mon, 17 Nov 1997 06:27:49 -0500

Subject: Re: high voltage input

>From: tesla.coil@mailexcite.com

>Date: Sun, 16 Nov 1997 20:58:51 -0600

>Subject: high voltage input

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Hey eveybody! I was browsing on the net and I came across some guy

>( Tedd Payne tpayne@netnitco.net) who is selling a 75kvAC power supply.

>Has anyone bought anything fromthis guy?

>He sells alot of stuff like this. Also, do I want to use 75kv as my primary

>input for my TC? I've never even heard of a regular transformer putting out

>75kv - is this possible, and is so, should I use it in my TC?

>Any info is appreciated!

>

>--Jay

>

Jay,

That does sound very interesting, but it would be way too much voltage for

use with a Tesla Coil. When you get into voltages that high, it is nearly

impossible to insulate your circuit enough. If you could insulate that much

voltage, it might make a heck of a Jacob's Ladder, but that is still

probably way too much voltage.

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: New Capacitors

Date: Mon, 17 Nov 1997 14:51:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 17 Nov 1997 09:51:09 EST

Subject: New Capacitors

I have finally finished work on two new plate caps. Each cap

consists of 48 plates each having an area of 60 square inches.

The dielectric is polyethylene sheet 12" x 8" x .06". The cores

are held together with wire ties and are immersed in transformer

oil. The calculated capacitance is .0258 uf each. These caps are

being used in series with a 15 kv 120 ma neon sign transformer.

The arc from the capacitors are almost white and extremely loud.

When these caps are in series the total capacitance drops to .0129

uf. I would like to add a few commercial caps to the circuit, I have

10) 2 kv .5 uf ceramic caps that I could also place in a separate

series circuit, parallel to my plate caps. Am I correct in my

calculations .5uf / 10 = .05uf + .0129uf = .0629uf.

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Plasma Globes

Date: Mon, 17 Nov 1997 15:18:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Mon, 17 Nov 1997 09:18:08 -0600

Subject: Re: Plasma Globes

Mike:

I'm not sure how the 1050 looks in a plasma globe but in a 'neon' sign

it looks just like pure argon. In signs, the neon is in there to

increase the operating temperature of the tube (because of its higher

'resistance') to facilitate the vaporization of the mercury, which is

added when using phosphor coated tubes. There is another mixture called

1051, which is used for cold weather outdoor tubes, it is 55%/45%

argon/neon or vice versa. It's been a while since I've used it and I

can't remember the exact mix. It is used in phosphor coated tubes also.

I've never used it without mercury so I can't vouch for its color by

itself. The argon from welding shops is probably at least 99% pure, and

it does work fine. One thing to note is that nitrogen/oxygen (air)

produces a similar blue/violet/indigo color very similar to argon, so

air impurities would be of little problem. The same is not the case for

neon. Any leftover gasses will cause the tube to look purple/blue until

they are scavenged by the electrodes. Proper bombarding is a must for a

clean, long living tube.

One last thing on pulling a vacuum on questionable glassware. The rule

we used in grad school was NEVER pull a vacuum on any flask bigger than

1 liter. Those globes you get at home centers for ceiling lights or

whatever may look thick but are they free from small scratches and other

flaws which could cause them to fail? Are the properly annealed to

eliminate stresses which form when all glasses are heated and molded?

I've been involved in implosions and had to remove porcupine-like shards

from my hands and arms its not fun. Please be careful!

I'm getting closer to finishing my coil. I got the secondary wound and

coated with varnish last weekend. Thanks again for all your advice! Take

care!

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Capacitors

Cc:

Bcc:

X-Attachments:

At 02:51 PM 11/17/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 17 Nov 1997 09:51:09 EST

>Subject: New Capacitors

>

>

>     I have finally finished work on two new plate caps. Each cap

>     consists of 48 plates each having a area of 60 square inches.

>     The dielectric is polyethylene sheet 12" x 8" x .06". The cores

>     are held together with wire ties and are emmersed in transformer

>     oil. The calculated capacitance is .0258 uf each. These caps are

>     being used in series with a 15 kv 120 ma neon sign transformer.

Paul,

Sounds like a good cap. It might be a bit large for a single

15kv/60ma neon but that depends on the neon. I find that most neons

will handle a somewhat larger cap than the calculations indicate.

With the 15kv/120ma it wont be a problem. Should work fine.

>

>     The arc from the capacitors are almost white and extremely loud.

>     when these caps are in series the total capacitance drops to .0129

>     uf. I would like to ad a few commercial caps to the circuit, I have

>     10) 2 kv .5 uf ceramic caps that I could also place in a separate

>     series circuit, parallel to my plate caps. Am I correct in my

>     calculations .5uf / 10 = .05uf + .0129uf = .0629uf.

>

Your calculations are correct by why place lossy

ceramic caps in parallel with your nice new efficient

poly cap? Ceramic caps will work for short periods

as TC caps but have a tendency to overheat and self destruct.

Plus I would bet the 2kv rating on those caps

is a DC rating. If so then the AC rating is no more

than 1 kv each and most likely somewhat less than that.

If you need more capacitance why not just build another

poly unit to place in parallel with the first. You can

never have enough quality capacitance on hand.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: TC questions?

Date: Mon, 17 Nov 1997 20:08:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kevin A. Zachary" <kzachary@kih.net>

Date: Mon, 17 Nov 1997 15:08:32 -0500

Subject: TC questions?

Hello -all,

Before I begin I should state that I have been out of the

'electric/electronics' field for quite some time and I am very rusty on

terms, conditions, etc.  I know, shame on me, but I still remember to "stay

far away before you plug it in".

A couple of questions for you...

What is the difference between having a vertical, 30 degree, x-degree, or

flat primary?

What exactly is a Faraday cage?

What steps have you (the coilers out there) taken to prevent your coil

from arcing onto the main power cord to your project, hence, back into your

household wiring?

After 'typing' to Steve Cole, he mentioned the possibility of RFI

passing back thru the household wiring.  What steps can you take to prevent

that?  I have lots of electronic/computer equipment that would not enjoy

that.

What advantages does a rotary spark gap have over just a plain one?  If

the rotary gap is more advantagous, how do you build one?

I have noticed that some secondary coils are grounded while others are

tied into the primary coil... What difference might that have on the coil?

Your fellow future coiler to be, that is, if I don't get fried,

KevinZ

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC's & Door Openers

Cc:

Bcc:

X-Attachments:

At 03:55 AM 11/17/97 GMT, you wrote:

>From: "Kevin A. Zachary" <kzachary@kih.net>

>Date: Sun, 16 Nov 1997 22:55:59 -0500

>Subject: Re: TC's & Door Openers

>

>> Modern electronics and Tesla coils do not mix very well.

>

>What is the 'recommended' distance to keep most electronic devices away

>from your Tesla Coil?

The farther the better. I always unplug my computer whenever I plan

to run the coil. This includes the phone line to the modem.

The worlds best (and cheapest) surge protector is pulling the plug

out of the wall.

>

>I have noticed some of the Tesla coil pictures where arcs are striking

>garage door openers, the electric cord that supplies power to the coil,

>etc... This brings about a question or a comment that I heard several years

>ago... Is it possible for a Tesla Coil to somehow shoot the high voltage it

>produces back into the house and eventually back to  the nearest

>substation?

No. That is not likely.

In a properly designed coil you have protective measures

to prevent such an occurrance. This is meant to protect

your transformer from harmful RF voltages.

You should have RF chokes and a safety gap. The chokes

knock down the level of RF spikes and the safety gap

limits any overvoltages by breaking down.

If the RF energy made it past these measures it would have

a hard time getting past the inductance of the transformer windings.

This is a good reason to attach a good RF ground to your

neon transformer case. A breakdown here would go to ground

instead of into the AC circuit.

As far as streamers hitting AC wiring and getting into the system

that is possible. But the RF energy in those streamers is desperately

trying to get to ground. It wont go far before finding ground.

Its a good idea to shield any wiring that could be hit with grounded

metal shields.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Physics lab

Date: Mon, 17 Nov 1997 22:22:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jaime L Lubelczyk <jaimely@student.umass.edu>

Date: Mon, 17 Nov 1997 17:22:52 -0500 (EST)

Subject: Physics lab

hi,

i need to design a physics lab, and i want to do something with

Tesla coils... anyone have any ideas, i'd be grateful to hear them.

thanks

jaime

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 1 Bar Argon Plasma

Date: Mon, 17 Nov 1997 23:06:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Mon, 17 Nov 1997 23:06:35 -0000

Subject: 1 Bar Argon Plasma

Hi All,

I've been playing around with low pressure Argon to obtain plasma.

I've just run out of Argon.....damn!...... and haven't tried discharges

thru pure Argon at 1 Bar (room pressure!). Has anyone else tried this and

do you get a capacitive effect like a normal store bought plasma globe??

Does it matter if the electrode in the center is enclosed or not?

All answers appreciated,

Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ballast for pole pigs/pot

Cc:

Bcc:

X-Attachments:

At 09:25 AM 11/17/97 GMT, you wrote:

>From: esmit@ilink.nis.za

>Date: Mon, 17 Nov 1997 11:25:44 +0200

>Subject: Ballast for pole pigs/potential transformers.

>

>don't speak to technically, i am new to this stuff, PLEASE?

Ok Richard Ill give it a try.

>

>Hi,

>

>I have read that when you buy a pole pig or potential transformer and want

>to run it on a Tesla coil you should use inductive ballasting or resistance

>ballasting.

>

>Will you answer some questions for me?

> * How do you know if a transformer isn't current limited?

Nearly all transformers are NOT current limited.

The only ones that are current limited include neon sign transformers,

oil burner ignition transformers (OBIT) and SOME microwave

oven transformers.

Almost anything else is sure to require external

current limiting. This includes power line distribution

transformers (pole pig) and potential transformers.

> * What does an inductive ballast and resistive ballast do?

They serve to limit the flow of current in the transformer

primary circuit to safe and manageable levels.

Without current limiting these transformers will try

to draw enormous amounts of current if they are short

circuited. (as in a spark gap)

> * What is an inductive ballast(i know it is an arc welder, but is that the

>machine that you use to meld iron to each other?)

Inductive ballast is a large inductor (coil) with an iron

core used to limit current flow. Inductance offers resistance (impedance)

to the flow of AC current. By varying the amount of inductance

the amount of current limiting is varied.

You can limit the current with just resistance but the amount

of resistance required will dissipate a considerable amount

of energy. This will limit the voltage available to the transformer

and limit the voltage output.

An inductor will limit the current with much less loss thus allowing

higher voltages while still limiting current.

An AC electric arc welder makes a good inductive ballast

as it usually has a variable power control which varies

the inductance. Its used with the low voltage side (welding clamps)

shorted together.

Some additional resistance is added to the circuit to smooth

out surges caused by the large inductance.

> * I probably wont use my fathers arc welder(if it's what i think it is)

>because it might be actually blow up or something, what else can i do?

If you acquire a pole pig you MUST provide some means of controlling it.

You MUST have a large variac to contol voltage input to the pig

and you MUST have either resistive or inductive ballasting or both.

> * What is the difference between a pole pig and a potential transformer?

>

A pig is a large robust transformer encased in a steel tank

filled with oil. A potential transformer is much smaller

and is dry. No steel tank and no oil. Potential transformers

are used by the utility companies to drop a few KVA of power

down to 220 volts to run some fans or other equipment at a substation.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Power Consumption

Date: Tue, 18 Nov 1997 14:09:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 18 Nov 1997 09:09:09 EST

Subject: Power Consumption

How do I calculate the power consumed by my capacitors when the

circuit is at rest (power on, no load) ? I have two poly caps in

series with a 15 kv 120 ma transformer. Each cap consists of 48

plates with a plate area of 60 square inches and a 60 mil dielectric.

t

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC questions?

Cc:

Bcc:

X-Attachments:

At 08:08 PM 11/17/97 GMT, you wrote:

>From: "Kevin A. Zachary" <kzachary@kih.net>

>Date: Mon, 17 Nov 1997 15:08:32 -0500

>Subject: TC questions?

>

>Hello -all,

>

>  Before I begin I should state that I have been out of the

>'electric/electronics' field for quite some time and I am very rusty on

>terms, conditions, etc.  I know, shame on me, but I still remember to "stay

>far away before you plug it in".

>

>  A couple of questions for you...

>

>  What is the difference between having a vertical, 30 degree, x-degree, or

>flat primary?

Really the only difference is the degree of coupling that each

primary shape provides.

A vertical (solenoid) typr primary provides the highest degreee of

coupling. In fact it often overcouples greatly.

The flat spiral provides the lowest degree of coupling. And it

may not provide enough coupling at power levels below 1000 watts.

The sloping saucer shape primary provides a degree of coupling

between these two extremes. The 30 degree slope is not any kind

of magic value. Any degree of slope can be used. The flatter (less slope)

used the less coupling is provided.

Any of these 3 primary shapes can be made to perform. Each has characteristics

that make it better for certain applications.

A solenoid is best for small coils at lower power levels that often

need all the coupling they can get.

The saucer shape is best for medium power level coils.

The flat spiral is best suited for high power coils where

overcoupling is likely to be a problem.

>

>   What exactly is a Faraday cage?

A Faraday cage is a grounded metal room. It is used to either

keep RF energy inside from leaking out or to keep sensitive

equipment inside from being affected by RF fields from outside.

I used to work inside a metal room when I repaired radio

equipment in Chicago. You could go inside and seal up the room

and align receivers without outside interference. This was a large

(and expensive) Faraday cage.

>

>   What steps have you (the coilers out there) taken to prevent your coil

>from arcing onto the main power cord to your project, hence, back into your

>household wiring?

Well the best way to prevent that problem is a little care

in the placement of the wiring. As long as its outside the

strike zone then there should not be a problem.

If you have AC wiring inside the strike zone of your coil

it should be protected with grounded metal shields.

>

>   After 'typing' to Steve Cole, he mentioned the possibility of RFI

>passing back thru the household wiring.  What steps can you take to prevent

>that?  I have lots of electronic/computer equipment that would not enjoy

>that.

First line of defense is RF chokes. These attenuate the level

of RF pulses coming back toward your supply transformer.

Next is your safety gap. This should have a grounded center post.

Any high RF voltages will then flash over safely to ground.

Your neon transformer should have its case and therfore its core

connected to RF ground. This is in case a failure occurs. If there

is an arc over inside the transfromer it will likely arc to the

core and safely to ground instead of arcing to the primary.

These measures pretty much take care of any high level RF problems.

But some low level RF hash can still ride up the AC line.

To cure this a commercial AC line filter is used.

I use 2 of these purchased as surplus. One is connected

between my variac and neons. The second is placed on the incoming

AC line into my control box.

>

>   What advantages does a rotary spark gap have over just a plain one?  If

>the rotary gap is more advantageous, how do you build one?

A rotary has no advantage in small to medium power coils. A well made

multiple electrode static gap will perform as well with a lot less

complexity and less strain on components.

In fact a rotary should NOT be used on a neon powered coil at all.

The higher pulse rate and varying breakdown voltage place too great

a strain on neon transformers. Neons perform better with a well made

static gap like the so called RQ type cylinder gap.

On a higher power coil with a large robust transformer a rotary

can be used to advantage by running at higher pulse rates.

>

>   I have noticed that some secondary coils are grounded while others are

>tied into the primary coil... What difference might that have on the coil?

>

You do NOT want your secondary tied to your primary in any way.

The base of the secondary goes directly to RF ground.

Connecting the bottoms of the primary and secondary together is

often shown in old diagrams and by workers that dont know any better.

Better and safer performance is had by running the base of the

secondary to RF ground.

If the primary and secondary are tied together this places lethal

60hz high voltage on your secondary coil. Not a safe condition.

And for efficient operation the current at the base of the

secondary needs to run to ground. Without a good ground

connection you will severely limit performance.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Physics lab

Cc:

Bcc:

X-Attachments:

At 10:22 PM 11/17/97 GMT, you wrote:

>From: Jaime L Lubelczyk <jaimely@student.umass.edu>

>Date: Mon, 17 Nov 1997 17:22:52 -0500 (EST)

>Subject: Physics lab

>

>        hi,

>        i need to design a physics lab, and i want to do something with

>Tesla coils... anyone have any ideas, i'd be grateful to hear them.

>

>thanks

>jaime

>

Jaime,

Im not really sure how to answer your question.

Your inquiry is a little too general. Give us a little

more idea about what you want to do.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Ballast for pole pigs

Date: Tue, 18 Nov 1997 21:25:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Tue, 18 Nov 1997 15:25:58 -0600

Subject: Re: Ballast for pole pigs

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 17 Nov 1997 17:23:42 -0600

> Subject: Re: Ballast for pole pigs/pot

>  * What is the difference between a pole pig and a potential transformer?

>

>

> A pig is a large robust transformer encased in a steel tank

> filled with oil. A potential transformer is much smaller

> and is dry. No steel tank and no oil. Potential transformers

> are used by the utility companies to drop a few KVA of power

> down to 220 volts to run some fans or other equipment at a substation.

Mike and Richard:

A potential transformer is actually quite different from a 'pig'. PTs

are usually found in substations and are used to reduce the line

potential to a safely measurable level. Usually around 120 volts. They

are rated by the ratio of HV to LV and not kVA. They are not used to

deliver power to a load. I doubt they could supply a few kVA, at least

not for long. I have seen several Tesla coil Web sites that use PTs for

their coils, so they must provide some power. My guess is that their

power capability would be similar to a NST. The allure of PTs is the

high voltage you can get on the 'secondary' (HV side), 115kV is common.

The cooling fans on large transformers are often powered by a 'pig' hung

on the side of the transformer. In the case of larger, higher voltage

transformers, say 345kV/69kV, there will sometimes be a tertiary winding

on the transformer to supply 14.4-12.47kV for a 'pig' to provide 120/240

to run the cooling fans etc. There are also current transformers, which

are like a high voltage version of a clamp-on ammeter. There are also

reactor transformers....

Eric

edavidson@icva.gov

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitors & waveforms

Date: Tue, 18 Nov 1997 22:12:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Tue, 18 Nov 1997 17:12:09 -0500

Subject: capacitors & waveforms

Hi.

can someone explain how a capacitor raises the voltage when applied to ac.

does it hold the wave at peak longer or raise the peak?

One other thought is that it takes longer to bring the wave back to 0

if this is so then it must surely change the frequency.

If anyone has a drawing of the waveform after capacitance is applied

i would like to see it.

I believe i understand the basic construction of poly caps except for the

connections.

in a plate cap if the plates are standing upright where do the wires connect?

also same for a rolled poly cap?

should I use glue between layers on a plate cap? & what kind?

thanks for any help

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: pole pigs

Date: Tue, 18 Nov 1997 22:20:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Tue, 18 Nov 1997 17:20:46 -0500

Subject: pole pigs

I am wondering what is a average price for a pole pig.

the ammunitions proving ground nearby has been shut down

and some ex employees have told me all the pole pigs have been

taken down and will most likely be sold at the DRMO auction

in indianapolis.

I heard some were never used and some old ones had pcb oil

in them.

just wondering?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Power Consumption

Cc:

Bcc:

X-Attachments:

At 02:09 PM 11/18/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 18 Nov 1997 09:09:09 EST

>Subject: Power Consumption

>

>

>     How do I calculate the power consumed by my capacitors when the

>     circuit is at rest (power on, no load) ?

Well any power is going to be reactive in nature and not really

consumed. If you just want to know what the current draw would be

find the reactance of the caps and apply ohms law.

>     I have two poly caps in

>     series with a 15 kv 120 ma transformer. Each cap consists of 48

>     plates with a plate area of 60 square inches and a 60 mil dielectric.

>

Paul this isnt a nice thing to be doing to your transformer.

A sure way to damage it. Those caps are going to charge up to

1.414 times the plate rating of the neon. In this case 21.21 KV.

This is way over the rating of the transformer and not a nice

thing to subject your caps to either.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: pole pigs

Cc:

Bcc:

X-Attachments:

At 10:20 PM 11/18/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Tue, 18 Nov 1997 17:20:46 -0500

>Subject: pole pigs

>

>I am wondering what is a average price for a pole pig.

Price varies depending on capacity.

The most common choice for coiling is a 14.4kv 10kva unit.

Prices range from $200-300. This is for a refurbished unit.

>the ammunitions proving ground nearby has been shut down

>and some ex employs have told me all the pole pigs have been

>taken down and will most likely be sold at  the DRMO auction

> in indianapolis.

>I heard some were never used and some old ones had pcb oil

>in them.

>

Do NOT take a pig that contains PCB oil. You would only be getting

yourself a problem.

While at first glance a pig seems an ideal solution it really

isnt. They are rather inexpensive but the REQUIRED control

circuitry is not.

You will need a large 30 amp 240V variac to control the input voltage

and you need inductive plus resistive ballasting for current

limiting. The required control circuitry can easily cost 2 to 3 times

what you spent on the transformer.

If you require power levels above 2KVA then a pig is the only

way to go. But for power levels up to 2KVA neons are the

simple solution.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitors & waveforms

Cc:

Bcc:

X-Attachments:

At 10:12 PM 11/18/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Tue, 18 Nov 1997 17:12:09 -0500

>Subject: capacitors & waveforms

>

>

>Hi.

>can someone explain how a capacitor raises the voltage when applied to ac.

>does it hold the wave at peak longer or raise the peak?

One way is that the cap will charge to 1.414 times the RMS value

of the AC. In other words the cap will charge to the peak value of the AC.

The voltage will track the AC waveform.

One other possibility is 60 hz resonance between the cap

and the inductance of the transformer winding feeding.

This can cause quite high voltages due to the resonant condition.

>One other thought is that it takes longer to bring the wave back to 0

>if this is so then it must surely change the frequency.

>

>If anyone has a drawing of the waveform after capacitance is applied

>i would like to see it.

The waveform is not altered. However due to the capacitance

the voltage and current are no longer in step with each other.

>

>I believe I understand the basic construction of poly caps except for the

>connections.

>

>In a plate cap if the plates are standing upright where do the wires

>connect?

I made one of my plate caps with square sheets of aluminum foil

with an extended tab. The tabs of all like plates are then bunched

together and fastened with a small nut and bolt.

Connections to the cap are made to the bunched up tabs.

>also same for a rolled poly cap?

Lots of ways here but my favorite is to wrap the inner foil

several times tightly around a piece of copper pipe. Then

tape down the joint. The copper pipe provides a low loss

connection to the inner foil. Solder a heavy piece of copper wire

to the pipe to bring to the outside.

The cap is then rolled up using the copper pipe as a center.

A piece of aluminum strap is rolled up on the end

of the outer foil to make the other connection.

>

>should I use glue between layers on a plate cap? & what kind?

>

No. Dont use any glue. Just stack up your plates and poly sheets.

If the poly slide around on you a lot put a puddle of oil in the

center of each sheet. The oil film will keep them from sliding

around so much.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ballast for pole pigs

Cc:

Bcc:

X-Attachments:

At 09:25 PM 11/18/97 GMT, you wrote:

>From: Eric Davidson <edavidson@icva.gov>

>Date: Tue, 18 Nov 1997 15:25:58 -0600

>Subject: Re: Ballast for pole pigs

>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Mon, 17 Nov 1997 17:23:42 -0600

>> Subject: Re: Ballast for pole pigs/pot

>

>>  * What is the difference between a pole pig and a potential transformer?

>>

>>

>> A pig is a large robust transformer encased in a steel tank

>> filled with oil. A potential transformer is much smaller

>> and is dry. No steel tank and no oil. Potential transformers

>> are used by the utility companies to drop a few KVA of power

>> down to 220 volts to run some fans or other equipment at a substation.

>

>Mike and Richard:

>

>A potential transformer is actually quite different from a 'pig'. PTs

>are usually found in substations and are used to reduce the line

>potential to a safely measurable level. Usually around 120 volts. They

>are rated by the ratio of HV to LV and not kVA. They are not used to

>deliver power to a load. I doubt they could supply a few kVA, at least

>not for long. I have seen several Tesla coil Web sites that use PTs for

>their coils, so they must provide some power. My guess is that their

>power capability would be similar to a NST. The allure of PTs is the

>high voltage you can get on the 'secondary' (HV side), 115kV is common.

>The cooling fans on large transformers are often powered by a 'pig' hung

>on the side of the transformer. In the case of larger, higher voltage

>transformers, say 345kV/69kV, there will sometimes be a tertiary winding

>on the transformer to supply 14.4-12.47kV for a 'pig' to provide 120/240

>to run the cooling fans etc. There are also current transformers, which

>are like a high voltage version of a clamp-on ammeter. There are also

>reactor transformers....

>

>Eric

Eric,

I dont really disagree with your comments. You probably

know more about it than I do. But PTs are much more capable

than NSTs.(neon sign transformer)

The only PTs Ive seen are a pair owned by a semi local coiler.

The name plate on one of them says 1.5 KVA. And I have seen him

run this transformer at 3KVA. Didnt even get warm. The other

information on the plate was some ratio but I dont remember now

what it was.

This transformer has a 14.4KV primary and a 240 VAC secondary.

They require external current limiting like a pole pig would.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: higher voltage

Date: Wed, 19 Nov 1997 16:16:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Wed, 19 Nov 1997 10:16:58 -0600

Subject: higher voltage

Organization: MailExcite  (http://www.mailexcite.com)

In one of the recent replies, I heard about someone charging their capacitors to

about 20kv when their power supply was rated at about 15kv. Does this mean that the

actual output voltage from the capacitors will be higher than the input voltage?

If so, what determines the output voltage?

--Jay

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: mini-coil

Date: Wed, 19 Nov 1997 17:20:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Wed, 19 Nov 1997 11:20:53 -0600

Subject: mini-coil

What is the smallest feasible size for a TC?  I was thinking that I could

make a small portable model with microwave xfrmr.  Or would the high current

rating make that impractical?  (I understand that most devices only draw the

current they need, but I have no idea what sort of current a TC would try to

gobble up).

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Power Consumption

Date: Wed, 19 Nov 1997 17:55:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 19 Nov 1997 12:55:43 -0500

Subject: Re: Power Consumption

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Power Consumption

> Date: Tuesday, November 18, 1997 9:32 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Tue, 18 Nov 1997 20:32:35 -0600

> Subject: Re: Power Consumption

>

> At 02:09 PM 11/18/97 GMT, you wrote:

> >From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

> >Date: 18 Nov 1997 09:09:09 EST

> >Subject: Power Consumption

> >

> >

> >     How do I calculate the power consumed by my capacitors when the

> >     circuit is at rest (power on, no load) ?

>

> Well any power is going to be reactive in nature and not really

> consumed. If you just want to know what the current draw would be

> find the reactance of the caps and aply ohms law.

>

>

> >     I have two poly caps in

> >     series with a 15 kv 120 ma transformer. Each cap consists of 48

> >     plates with a plate area of 60 square inches and a 60 mil dielectric.

> >

>

>  Paul this isnt a nice thing to be doing to your transformer.

> A sure way to damage it. Those caps are going to charge up to

> 1.414 times the plate rating of the neon. In this case 21.21 KV.

> This is way over the rating of the transformer and not a nice

> thing to subject your caps to either.

>

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike & Paul,

It is even worse than you think!!!

When you connect just the cap up to the transformer you are just

BEGGING for a disaster! Mike mentioned 1.414 times the RMS value

of the transformer. Yeah, but that totally ignores RESONANCE

phenomenon. If the inductive reactance of the transformer is

equal to the capacitive reactance, then the voltage across BOTH

transformer and cap can rise to MANY times the peak voltage.

I have gotten over 50KV from a 12KV neon before it died. This

is an experiment you do NOT want to do on your transformer!!!

Always, always, always, always, always use safety gaps!!!!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: higher voltage

Cc:

Bcc:

X-Attachments:

At 04:16 PM 11/19/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Wed, 19 Nov 1997 10:16:58 -0600

>Subject: higher voltage

>Organization: MailExcite  (http://www.mailexcite.com)

>

>In one of the recent replies, I heard about someone charging their

>capacitors to about 20kv when their power supply was rated at about 15kv.

>Does this mean that the actual output voltage from the capacitors will be

>higher than the input voltage?

>If so, what determines the output voltage?

>

>--Jay

Jay,

The nameplate rating on a 15kv neon is 15kv RMS. Using the

transformer to charge a cap will allow the cap to charge to

peak value of the AC waveform. In this case thats  1.414 X 15kv = 21.21KV.

This is not necessarily a good thing. This voltage is over

the rating for the neon and will serve to break down the

insulation over time. Also it places more strain on your

caps.

The primary voltage in a properly constructed Tesla coil

is mostly determined by the spacing on the main spark gap.

Once the voltage reaches a certain level the gap breaks down

and discharges the energy stored in the cap. Increasing the

gap spacing increases the voltage the cap is allowed to charge to.

Too much gap spacing will allow for too high a voltage and lead

to cap and/or transformer failure.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: mini-coil

Cc:

Bcc:

X-Attachments:

At 05:20 PM 11/19/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Wed, 19 Nov 1997 11:20:53 -0600

>Subject: mini-coil

>

>

>

>What is the smallest feasible size for a TC?  I was thinking that I could

>make a small portable model with microwave xfrmr.  Or would the high current

>rating make that impractical?  (I understand that most devices only draw the

>current they need, but I have no idea what sort of current a TC would try to

>gobble up).

>

>Thanks. . .CF

>

Chris,

Unfortunately microwave oven transformers are not really

suitable for coils. The voltage is way too low (2200 volts)

and the current is too high.

I made a mini coil last winter. I had 660 turns of AWG 32

on a piece of 2 inch PVC. The winding length was 6 inches.

Running with about .007 mfd of capacitance and a 9kv/30am neon

I got 14 inches of spark. This doesnt sound like much but remember

the secondary was only 6 inches long. I had planned on running it

on a 12kv/30ma neon but this small coil wouldnt handle that much power.

If I was building it again I would make a somewhat taller secondary

wound with AWG 28. A 10kv/23 ma OBIT (oil burner ignition transformer)

makes a fine power supply for a small coil. These are often easier to

find than neons. Check heating contractors and furnace service

companies. OBITs have no real resale value so you can often get old ones

for nothing.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: very small coils

Date: Wed, 19 Nov 1997 19:29:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Wed, 19 Nov 1997 13:29:36 -0600

Subject: very small coils

Organization: MailExcite  (http://www.mailexcite.com)

I'm trying to make an ultrasmall TC. I heard that someone built a coil powered by

a 9v battery. Is this really possible? Also, I tried to wind a secondary about 2cm

x 8cm with what I believe to be 40 guage wire. Needless to say, it was quite a

challenge, and the first time I sent current though it the wire ripped into shreds!

Has anyone out there built an ultrasmall coil? I'd love to hear about it and how

you built it!

--Jay

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: very small coils

Cc:

Bcc:

X-Attachments:

At 07:29 PM 11/19/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Wed, 19 Nov 1997 13:29:36 -0600

>Subject: very small coils

>Organization: MailExcite  (http://www.mailexcite.com)

>

>I'm trying to make an ultrasmall TC. I heard that someone built a coil

>powered by a 9v battery. Is this really possible?

No not really. Coil performance is dependant on power.

Not much power available from a 9V battery.

> Also, I tied to wind a secondary about 2cm x 8cm with what

>I believe to be 40 guage wire. Needless to say, it was quite a

>challenge, and the first time I sent current though it the wire

>ripped into shreds!

I bet. AWG 40 is some mighty small wire.

>

>Has anyone out there built an ultrasmall coil?

>I'd love to hear about it and how you built it!

>

>--Jay

Since several people showed some interest in the mini coil

I mentioned in an earlier post I dug up the original

write up I made on this coil. This was previously posted

on this list on February 17th of this year.

<begin include>

Some of you may be interested in the small coil

I built this winter. My big coil is out in my unheated

garage having outgrown the basement. The cold

weather we had really put a damper on my enthusiasm.

So to satisfy my craving for ozone I built a small

coil out of materials I had on hand.

Secondary. 660 turns # 32 on 2 inch PVC. OD 2 3/8 in.

Winding length 6 inches. Measured resonant frequency

920 KHZ

Primary    9 1/2 turns # 6 solid copper in saucer shape

Cap        using my old salt water caps from my first coil

Capacitance approx .006 mfd

Gap        5  3/4 inch copper pipes mounted side by side on

plexiglass plate. Spacing .03 inch. Total gap distance .120 inch

Power      single 9kv/30ma neon

Terminal   copper toilet tank float. Resonant frequency with

terminal in place 650 KHZ

Resonant frequency with 5 inch diameter foil covered

styrofoam ball 630 KHZ

I kind of used what I had around for this project.

AWG 32 wire is pretty small. But I had a spool of it lying around.

The gap was made from some small pieces of 3/4 inch copper

pipe I had laying around.

It took some fiddling to find a decent tune. It would figure

I would make the primary too small. Every primary I've made

I ended up adding to.

Tapped all the way out on the last turn got me 8 inch arcs and

lots of corona. And I mean lots. I may make a small toroid

to help suppress the corona.

8 inch arcs don't sound like a big deal but remember the winding

on this secondary is only 6 inches long.

I added another two turns to the primary and used a 5 inch sphere

terminal. This worked better and I was able to get 12 inch sparks.

A bit of experimenting with the coupling and tuning got the output

to 13 inches. I was planning on going to a 12kv/30ma neon but

I think I have this little coil maxed out. It had 13 inch output

but the coil was breaking down all over. Some arcs from the middle

of the coil were 5 or 6 inches long.

I backed down the gap a bit and reduced the coupling by raising

up the secondary a bit. It now happily delivers 12 inch sparks.

This is with a single 9kv/30ma neon and a 6 inch long secondary.

I'm not recommending this as a project for anyone. This coil

while fun has a couple of serious shortcomings.

First is the use of # 32 wire in the secondary. This severely

limits the amount of power it can handle. But I used it because

it was on hand.

And a proper size plastic film cap would be much better than the SW

caps. But again I already had them so put them to use.

This would of been a better coil with a longer secondary wound with

heavier wire. Something like 750 turns of #28. This would of been about

10 inches long and had a similar resonant frequency.

<end include>

If I was building that coil today I would go for a slightly

taller secondary wound with heavier wire. I would use a better

cap more closely matched to the neon. A decent toroid to push

the resonant frequency down enough to allow for 10-12 primary turns.

Still it was a fun coil and I made it with what I had laying around.

Comments or questions always welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: 2 neons in parallel

Date: Wed, 19 Nov 1997 23:14:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Mike Wilson <nwilson@skyenet.net>

Date: Wed, 19 Nov 1997 18:14:58 -0500

Subject: 2 neons in parallel

Hi All,

I am going to hook up two neons in parallel,  I know how to phase

then together but  was wondering if I should run a safety gap and chokes

on each one or will one work ok.

Thanks

Mike Wilson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 2 neons in parallel

Cc:

Bcc:

X-Attachments:

At 11:14 PM 11/19/97 GMT, you wrote:

>From: Mike Wilson <nwilson@skyenet.net>

>Date: Wed, 19 Nov 1997 18:14:58 -0500

>Subject: 2 neons in parallel

>

>Hi All,

>    I am going to hook up two neons in parallel,  I know  how to phase

>then together but  was wondering if I should run a safety gap and chokes

>on each one or will one work ok.

>

>Thanks

>Mike Wilson

>

Mike,

I've run my paralleled neons through a single safety

gap and single set of RF chokes with little problem.

However I do recommend that you give each transformer its own

individual safety gap. Mount it right on top the case and

make it a a permanent part of the transformer. Thats the safest

thing to do. A single set of RF chokes will be fine.

A rebuilding job Im doing on a large 15kv/60ma neon

is going to incorporate its own built in safety gap.

I would also like to remind everyone that paralleled

neons must have the same voltage rating. You CANNOT

parallel a 12kv and a 15kv neon. However neons with the same

voltage rating but different current ratings can be paralleled.

You CAN parallel a 12kv/60ma and a 12kv/30ma for instance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Physics lab

Date: Thu, 20 Nov 1997 13:33:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jaime L Lubelczyk <jaimely@student.umass.edu>

Date: Thu, 20 Nov 1997 08:33:55 -0500 (EST)

Subject: Re: Physics lab

> Im not really sure how to answer your question.

> Your inquiry is a little too general. Give us a little

> more idea about what you want to do.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

I didn't mean to be vauge, it's just a matter of I really don't

know what the options are. The lab is about electromagnetism, and the

professor has just told us to design a lab to study some principle of e&m.

I don't know much about Tesla cols, so I don't know what they can do.  So

I guess I'm open to any ideas at all about possible labs.

jaime

pax amor et lepos in iocando

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: large cap

Date: Fri, 21 Nov 1997 17:56:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Fri, 21 Nov 1997 18:56:45 +0100

Subject: large cap

I was just wondering what will happen if you got a huge cap, well..large

anyway, and some kind of switch. You fully charge the cap and then just

turn the switch on so it will be a huge discharge and would it then be

longer sparks? I saw the "Can crusher" page and they must have a pretty

large cap?

But, how will the Tesla coil behave with such a large discharge? would it be

one huge discharge with a lotta sparks?

This might be a little dumb question.

//Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: newbie question on spark

Date: Sat, 22 Nov 1997 00:37:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Fri, 21 Nov 1997 19:37:52 -0500 (EST)

Subject: newbie question on spark gaps

First of all thanks for all the help & advice so far. My two year old son

doesn't leave me too much spare time to work on my coil set up but I've

managed to finish my spark gap (not much progress I admit but two year olds

come first). It's a multiple gap that will house up to six gaps made from

3/8" brass bolts, housed in a plexiglass box with a muffin fan at one end.

I've mounted a sheet of 1/4" heat absorbing glass above the bolts to keep the

top of the box from melting from the heat. It seems to work just fine and I

hope the housing adds an added safety factor but I have a few questions:

What is the proper wiring for a multiple gap - series or parallel?

What is a good gap distance? When I open them up to 3/8" or so the arcs are

very violent and the heat builds up even with the fan on.

Are multiple gaps a waste of time? I'm running a 15K 30 ma Franceformer Neon

Transformer.

Thanks again for all the help.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark length = ? volts

Date: Sat, 22 Nov 1997 02:52:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Mike Wilson <nwilson@skyenet.net>

Date: Fri, 21 Nov 1997 21:52:17 -0500

Subject: Spark length = ? volts

Hi,

I got my first coil fired today and with a little tweaking I got

well over 2 foot sparks. The good news is there was no smoke, shorts or

fire. Everything ran fine and now I think its time to upgrade my caps

from saltwater to ? (I haven't decided yet). My question is can someone

tell me about how many volts to the length of spark is. I know it wont

be very accurate, just a ballpark figure will do like 1 foot = ? volts.

Thanks

Mike Wilson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: newbie question on sp

Date: Sat, 22 Nov 1997 05:20:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 21 Nov 1997 21:20:57 -0800

Subject: Re: newbie question on spark

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Fri, 21 Nov 1997 19:37:52 -0500 (EST)

> Subject: newbie question on spark gaps

>

> First of all thanks for all the help & advice so far. My two year old son

> doesn't leave me too much spare time to work on my coil set up but I've

> managed to finish my spark gap (not much progress I admit but two year olds

> come first). It's a multiple gap that will house up to six gaps made from

> 3/8" brass bolts, housed in a plexiglass box with a muffin fan at one end.

> I've mounted a sheet of 1/4" heat absorbing glass above the bolts to keep the

> top of the box from melting from the heat. It seems to work just fine and I

> hope the housing adds an added safety factor but I have a few questions:

> What is the proper wiring for a multiple gap - series or parallel?

> What is a good gap distance? When I open them up to 3/8" or so the arcs are

> very violent and the heat builds up even with the fan on.

> Are multiple gaps a waste of time? I'm running a 15K 30 ma Franceformer Neon

> Transformer.

> Thanks again for all the help.

>

Hi!

The gaps should always be connected in series. This "spreads out" the

arc into pieces, sharing the thermal heating over all of the electrodes,

making the breakdown voltage more consistent, and helping to extinguish

(quench) the arc. Increasing the air velocity by using a vacuum cleaner

motor to suck air into the box will also help quench the arc and cool

the elctrodes. If you set each gap top be in the 0.030 - 0.060" range,

you'll end up with a total of about 0.2 - 0.4" which should work just

fine with your system.

Safe gappin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: large cap

Cc:

Bcc:

X-Attachments:

At 05:56 PM 11/21/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Fri, 21 Nov 1997 18:56:45 +0100

>Subject: large cap

>

> I was just wondering what will happen if you got a huge cap, well..large

>anyway, and some kind of switch. You fully charge the cap and then just

>turns the switch on so it will be a huge discharge and would it then be

>longer sparks? I saw the "Can crusher" page and they must have a pretty

>large cap?

>But, how will the tesla coil behave with such a large discharge? would it be

>one huge discharge with a lotta sparks?

>This might be a little dumb question.

> //Christopher Eriksson

>

>

Chris,

What you are talking about here is running your coil in

what is called single-shot mode. This has been done before.

You can charge your cap from a DC supply and then discharge

in one shot.

However do realize that the cap is still part of the primary

side resonant circuit. So you cant make that cap bigger without

affecting the primary resonant frequency.

It the resonant circuits that make a Tesla coil work.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Physics lab

Cc:

Bcc:

X-Attachments:

At 01:33 PM 11/20/97 GMT, you wrote:

>From: Jaime L Lubelczyk <jaimely@student.umass.edu>

>Date: Thu, 20 Nov 1997 08:33:55 -0500 (EST)

>Subject: Re: Physics lab

>

>> Im not really sure how to answer your question.

>> Your inquiry is a little too general. Give us a little

>> more idea about what you want to do.

>>

>> TTYL!

>> Mike Hammer

>> mhammer@misslink.net

>

>        i didn't mean to be vauge, it's just a matter of i really don't

>know what the options are.  the lab is about electromagnetism, and the

>professor has just told us to design a lab to study some principle of e&m.

>i don't know much about tesla cols, so i don't know what they can do.  so

>i guess i'm open to any ideas at all about possible labs.

>

>jaime

>

Jaime,

Naturally you can expect me or anyone here to recommend building

a Tesla coil for your project. Tesla coils illustrate many principles

besides electromagnetism. You have resonant circuits, pulse driven

oscillators, high voltage RF energy and electrostatics all involved.

The best way to find out what a coil can do is to build and

operate one. That will give you experience you wont ever

get from a book.

Start with a small coil and optimize it for best output.

Once you get it going you will be suprised at the number

of experiments and demonstrations that will come to mind.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark length = ? volts

Cc:

Bcc:

X-Attachments:

At 02:52 AM 11/22/97 GMT, you wrote:

>From: Mike Wilson <nwilson@skyenet.net>

>Date: Fri, 21 Nov 1997 21:52:17 -0500

>Subject: Spark length = ? volts

>

>Hi,

>    I got my first coil fired today and with a little tweaking I got

>well over 2 foot sparks.

New coil and 2 foot sparks? Well done.

> The good news is there was no smoke, shorts or fire.

Well as the old radio man saying goes " Tune for minimum smoke".

> Everything ran fine and now I think its time to upgrade my caps

>from saltwater to ? (i haven't decided yet).

Definitely polyethylene or polypropylene dielectric.

Anything else isnt worth the effort.

Check out the FTP sites for files on building plate

and rolled caps.

Check out http://www.velocity.net/~djb/files.shtml

for a well designed page by list member Dave Bronakowski with many

coiling files.

Plate type caps give the best pulse performance

and when oriented vertically dont require a vacuum pump to remove

air bubbles.

Rolled caps are somewhat easier and quicker to build

and an extra advantage is using sections of PVC pipe

for a tank and housing. Rolled caps give good results.

However they almost require vacuum pumping to remove trapped air.

> My question is can someone

>tell me about how many volts to the length of spark is. I know it wont

>be very accurate, just a ballpark figure will do like 1 foot = ? volts.

This is an often asked question with no really good answer.

Extremely difficult to tell just how much voltage is present

in any given spark. Just far too many variables involved.

Suffice it to say that typical Tesla coils run from

100-500kv with sparks lengths from 1 to 4 feet.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: very small coils

Date: Sat, 22 Nov 1997 23:43:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Sat, 22 Nov 1997 23:43:20

Subject: Re: Re: very small coils

I have made a ultra small coil with a 1"1/16" X 2"7/8" secondary made

from #30 wire. A primary with 6 turns of #24 wire .25" apart, 2"1/8"

diameter. I plan on using one of my 12Kv 30mA or mabey my 9Kv 30mA. I will

use my bottle saltwater caps with a total of .0052uF(about 1300pF a

peice). When I did the calculations it figured up I should use a toroid

that's 2" X 10". I think this is pretty large but it might be right? All I

have made so far is the primary and the secondary. I used a percription

bottle to wind the secondary on because it's what I had laying a round.

Any suggestion on making this small coil? Later

--The Choadester.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RFI, etc.

Date: Sat, 22 Nov 1997 23:52:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kevin A. Zachary" <kzachary@kih.net>

Date: Sat, 22 Nov 1997 18:52:17 -0500

Subject: RFI, etc.

Perhaps I will be able to answer questions someday...  but for now.. I must

ask...

What is the typical voltage found at the primary if charged with a 15Kv

neon and a suitable flat glass capacitor?

I have been told to 'ground' my secondary coil, what are some good ways of

doing that?

I have been told not to ground my secondary to the primary, even if old

schematics show that.  What are the advantages/disadvantages of this or

ground?

Do AC line filters connected to both sides of a neon help dampen RFI?

How far away from a Tesla Coil can RFI be a problem?  Does RFI travel by

air or by wire?

How does one determine the resonant frequency of their primary and

secondary coils?

What type of camera settings and film are good for taking pictures of a

coil in action?  I have a Canon EOS Rebel with all kinds of settings for

aperature, shutter speed, etc.

Does anyone have an old issue of Electronics Now, probably around 1980 or

1981 that contains a Tesla Coil project calling for 30Kv neon, plate glass

capacitor, static spark gap, #10 primary, 3" PVC secondary (or something

real close to that)?

I'll stop now.. :-)   I have learned a great deal by being a member of this

group... I just wish we had access to the internet (like we do today) back

in the early 80s when I was a mad scientist... lucky to be alive now.

KevinZ

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: mw transformer and sealer

Date: Sun, 23 Nov 1997 00:52:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sat, 22 Nov 1997 18:52:35 -0600

Subject: mw transformer and sealers

Anyone know an easy way to figure out the voltage output of a microwave

transformer?  I was thinking that I'd hook-up a 9v battery to the primary and

check the voltage on the secondary then do the math to get the 120vac input

= Xvac output.  Will this work?  (DC vs AC inputs really shouldn't make a

difference, should it?)

And, has anyone used the Plasti-Dip spray for sealing a secondary?   They

call it "rubber", it insulates tool handles (at least the "dip" stuff does)

and it comes in a clear spray.  Sounds good to me.  Any contrary opinions?

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla coil web ring

Cc:

Bcc:

X-Attachments:

Coiler Jerry Gore has started a Tesla coil web page ring.

This is a network of sorts of web pages all

having Tesla related material. You can navigate from

site to site by following the web ring links.

If anyone wants to check this out they can access the ring

from any member site. I happen to believe that my web page

is an excellent place to start. :)

My URL is : http://www.misslink.net/mhammer/

and you will find the web ring link at the bottom

of my main page. There are currently 34 web sites on the ring.

If any of you have Tesla related web sites and want to join

the web ring go to this URL on Jerry Gore's web site:

http://rampages.onramp.net/~jgore/tesla/ring/join.htm

If you have questions or encounter problems you may email me direct.

Also let me know of your pages so I may add you to my link

page. Im looking for reciprocal links from other coiling pages.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: mw transformer and sealer

Cc:

Bcc:

X-Attachments:

At 12:52 AM 11/23/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sat, 22 Nov 1997 18:52:35 -0600

>Subject: mw transformer and sealers

>

>

>Anyone know an easy way to figure out the voltage output of a microwave

>transformer?  I was thinking that I'd hook-up a 9v battery to the primary and

>check the voltage on the secondary then do the math to get the 120vac input

>= Xvac output.  Will this work?  (DC vs AC inputs really shouldn't make a

>difference, should it?)

Chris,

A transformer is an AC only device. DC wont do anything except

warm the wires up.

What I do to check my transformers and neons is input a small

voltage like 5 VAC from my variac and measure the output.

For instance on the neon I just rebuilt an input of

5 volts AC gives me 531 volts output. This results in a turns

ratio of 106 to 1. So with 120 volts input this particular

transformer puts out 12720 volts.

I use this method to match up transformers for parallel

operation. I put the neons closest together in voltage

together for parallel operation.

If you dont have a variac you can use a 6.3 volt filament

transformer to step down the line voltage for the test.

Whatever method you use remember to keep the input voltage low enough

to keep the expected output within range of your meter.

>

>

>And, has anyone used the Plasti-Dip spray for sealing a secondary?   They

>call it "rubber", it insulates tool handles (at least the "dip" stuff does)

>and it comes in a clear spray.  Sounds good to me.  Any contrary opinions?

>

>Thanks. . .CF

>

Yes I tried some of this one time long ago and

the results were disappointing. It doesnt seem to be

as good an insulating material as you would think.

Perhaps it was the fault of the coloring agent. The stuff I

used was red.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RFI, etc.

Cc:

Bcc:

X-Attachments:

At 11:52 PM 11/22/97 GMT, you wrote:

>From: "Kevin A. Zachary" <kzachary@kih.net>

>Date: Sat, 22 Nov 1997 18:52:17 -0500

>Subject: RFI, etc.

>

>Perhaps I will be able to answer questions someday...  but for now.. I must

>ask...

Well Kevin you got a whole bunch of questions here.

Lets see what we can make of them.

>

>What is the typical voltage found at the primary if charged with a 15Kv

>neon and a suitable flat glass capacitor?

The primary circuit voltage is mostly determined by

the spark gap width. A wide gap will permit the cap to

charge to a higher voltage before the gap breaks down.

With a 15kv neon you dont want that gap breakdown voltage

to be any higher than 15kv if you expect that neon to survive

for long.

A flat plate glass capacitor is not in any way "suitable".

It is heavy, bulky, fragile and gives really lousy performance

due to the very high RF dissipation factor of glass.

Substitute a stack of polyethylene sheets for each glass plate

and put the whole thing in transformer oil. Not much more effort

required than building the glass cap but it will work far better.

>

>I have been told to 'ground' my secondary coil, what are some good ways of

>doing that?

I epoxy a copper or brass plate to the outside of my secondary form

just below the winding. The bottom lead of the secondary gets soldered

to this plate. I then solder a 24 inch of AWG 8 wire to this and terminate

it with a large ring terminal. My ground lead is also AWG 8 and runs

to a pair of 4 foot ground rods hammered into the ground. The free end

of the ground lead also terminates in a large ring terminal.

I connect the two ring terminals together with a large bolt when

I want to run the coil.

>

>I have been told not to ground my secondary to the primary, even if old

>schematics show that.  What are the advantages/disadvantages of this or

>ground?

There are NO advantages to doing this and many disadvantages.

First is a question of safety. Tying the primary and secondary

together places lethal 60 hz energy on your secondary.

Secondly a Tesla secondary NEEDS a ground connection. If you

dont supply one it will try to push against the other circuit

wiring as a counterpoise.(artificial ground)

The problem with this is a severe drop in performance.

The heavy RF current at the base of the coil has no where to go.

>

>Do AC line filters connected to both sides of a neon help dampen RFI?

You want an AC line filter on the 120 VAC side of your neon.

You do not want one on the HV side. On the HV side you will

have RF chokes (one in each HV lead) and a safety gap right

at the transformer.

>

>How far away from a Tesla Coil can RFI be a problem?

A Tesla coil properly tuned for spark ouput is a very poor

RF radiator. The direct RF radiation rarely goes more than a

few hundred feet.

> Does RFI travel by air or by wire?

Both. Like I said above direct radiation is seldom

a problem. But RF hash or spikes conducted into the AC

wiring of your home can be a problem.

But by using RF chokes in the HV leads and an AC line

filter on the 120V side this can be practically eliminated.

Its best to run your coil on a separate circuit. Certainly

not on a circuit containing any sensitive electronics like

computers, TVs or VCRs.

>

>How does one determine the resonant frequency of their primary and

>secondary coils?

By applying the resonant frequency formula

1

----------

2pi*SQR(LC)

You find the secondary inductance by using Wheeler's formula.

Wheeler's formula for inductance of solenoids

L(uH) = (r^2) * (N^2) / (9*r + 10*h)

where:

r = coil RADIUS in inches

N = number of turns

h = coil height in inches

Next you determine the secondary winding self capacitance using

Medhurst's formula.

Medhurst's formula for self capacitance

C  = K x D

where:  C = capacitance in picofarads

K = constant which depends on the ratio of the coil height to diameter

x = means multiply K times D

D = solenoidal coil diameter in centimeters

H = coil height in centimeters

H/D       K

5.0     0.81

4.5     0.77

4.0     0.72

3.5     0.67

3.0     0.61

2.5     0.56

2.0     0.50

1.5     0.47

1.0     0.46

Now that you have the inductance and self C then apply the resonant

frequency formula.

1

Fo =   ----------

2pi*SQR(LC)

>

>What type of camera settings and film are good for taking pictures of a

>coil in action?  I have a Canon EOS Rebel with all kinds of settings for

>aperature, shutter speed, etc.

Better off forgetting that. That fancy microelectronic camera

of yours will go belly up anywhere near an operating coil.

You want a totally mechanical camera.

I use my trusty totally mechanical Olympus OM-1 for this and it just

loves sparks.

The mistake many make when taking coil pictures is shooting

in almost total darkness and too long an exposure. The individual

streamers can no longer be seen and you cant judge the size

of the sparks without a background.

So dim the room lighting and light the wall up dimly behind the coil.

Focus on the coil and meter for a good exposure of the lit

background. Let the sparks take care of themselves.

Ive used ASA 400 film and shutter speeds of 1/4-1/2 second

at f5.6 or f8. If you want to see how this looks check out

the pics on my web site. http://www.misslink.net/mhammer/

>

>Does anyone have an old issue of Electronics Now, probably around 1980 or

>1981 that contains a Tesla Coil project calling for 30Kv neon, plate glass

>capacitor, static spark gap, #10 primary, 3" PVC secondary (or something

>real close to that)?

Yea I got it in storage somewhere. Forget it. Poor design

and truly awful performance.

>

>I'll stop now.. :-)   I have learned a great deal by being a member of this

>group... I just wish we had access to the internet (like we do today) back

>in the early 80s when I was a mad scientist... lucky to be alive now.

>

Hope I got all your questions answered.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: TC & Tuning basics

Date: Sun, 23 Nov 1997 15:41:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sun, 23 Nov 1997 09:41:44 -0600

Subject: TC & Tuning basics

How about an explanation of the basics of tuning a TC?

I know that the number of turns tapped into on the primary effects primary

field strength, and the field inducts the current in the secondary. . . .

But what about total spark gap distance?  From reading Mike's post about RFI

it sounds like gap length effects the amount of charge stored and released

by the cap.  True?  And, if so, does reducing SG distance reduce cap

charging and thus field strength of the primary?  And, if so, could I reduce

SG distance and then increase coupling (more primary turns, or lower the

secondary into the primary field) and get different, hopefully better, results?

And, does the current need to necessarily induced at, or below (if

over-coupling is a problem with a solenoid primary), the lowest turn on the

secondary?  Or if the base of the primary is above the bottom turn of the

secondary am I simply "reducing" the length of my secondary?

Also, how about a some talk about how the whole of a TC's resonant circuit

actually works together as it relates to resonance.  I guess I don't "get"

how there is a resonance set up between the primary HV circuit and the

secondary.  Or is the only resonance of components, that really matters, in

the primary circuit, and the resulting inductance provided to the secondary

just a unique way to use that resonance and not part of the resonant circuit

at all?

*And, back to caps, I'm guessing the cap gets charged 60 times a second and

releases the stored voltage 60 times a second, right?  So while the cap's

charging, what's happening to the primary winding?  Is it dropping to zero,

or close, or is it still getting some juice and the subsequent pulse from

the cap is the main source of amplified voltage/resonance?

Finally, and just out of curiosity, how long does your average mineral oil

immersed polyprop plate cap actually hold a charge after the power's turned

off?  I almost always discharge mine before fiddlin' whith the primary

circuit, but have never even seen the hint of a spark.

Sorry for the long post, hope the questions aren't too stupid (I'm not sure

I even know enough to ask intelligent questions).

Thanks . . . Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Fw: 1 Bar Argon Plasma

Date: Sun, 23 Nov 1997 17:26:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Sun, 23 Nov 1997 17:26:29 -0000

Subject: Fw: 1 Bar Argon Plasma

----------

> From: Stuart Marsh <stuart@anker-auto.netkonect.co.uk>

> To: tesla-2 <tesla-2@emachine.com>

> Subject: 1 Bar Argon Plasma

> Date: 17 November 1997 23:06

>

> Hi All,

>      I've been playing around with low pressure Argon to obtain plasma.

> I've just run out of Argon.....damn!...... and haven't tried discharges

> thru pure Argon at 1 Bar (room pressure!). Has anyone else tried this and

> do you get a capacitive effect like a normal store bought plasma globe??

> Does it matter if the electrode in the center is enclosed or not?

>                              All answers appreciated,

>                              Regards,

>                              Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: mw transformer and se

Date: Sun, 23 Nov 1997 17:30:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 23 Nov 1997 12:30:41 -0500

Subject: Re: mw transformer and sealer

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: mw transformer and sealer

> Date: Saturday, November 22, 1997 7:52 PM

>

> From: Chris Federico <cfed@comsource.net>

> Date: Sat, 22 Nov 1997 18:52:35 -0600

> Subject: mw transformer and sealers

>

>

> Anyone know an easy way to figure out the voltage output of a microwave

> transformer?  I was thiknin that I'd hook-up a 9v battery to the primary and

> check the voltage on the secondary then do the math to get the 120vac input

> = Xvac output.  Will this work?  (DC vs AC inputs really shouldn't make a

> difference, should it?)

>

>

> And, has anyone used the Plasti-Dip spray for sealing a secondary?   They

> call it "rubber", it insulates tool handles (at least the "dip" stuff does)

> and it comes in a clear spray.  Sounds good to me.  Any contrary opinions?

>

> Thanks. . .CF

>

Chris,

You *cannot* determine the voltage by applying a DC at the primary

(or secondary, for that matter). Transformers must operate with a

changing input signal. This is usually a sine wave, which is the

shape of AC from your wall socket.

Here is the simplest way to measure the value of your microwave

transformer. Since most people do not have meters that can take the

approximately 2,500 volts AC that appears on the secondary of a

neon transformer, it is actually best to do things somewhat

BACKWARDS :)

Apply 120 VAC to the SECONDARY and measure the voltage at the

primary. (The voltage applied may be slightly off from 120 VAC...

record the actual applied voltage wherever I say 120VAC)

Put the larger voltage over the smaller voltage and multiply by

the wall socket voltage to get the HV output.

For example, let's say you put 120 VAC into the secondary and

got 6 volts out.

HV= (120*120)/6

so HV=2,400 volts

One note of caution: The secondary has one side connected to the

transformer case in most microwave transformers. Thus while you

are performing this test you may possibly have 120VAC at the case.

So please make sure the transformer case is resting on an

insulating surface, and do not touch it while making the

measurements.

This method of measuring can also be applied to neons, with the

same note of caution. The really good news is that this method

is very accurate and allows you to use regular low voltage

meters. On neons you can measure each output independantly,

by using the case center tap as one connection,or you can attach

the 120VAC to both HV leads. But be aware that the case center

tap in EITHER case may have a voltage on it with respect to

AC ground, so please be careful.

When using a meter to measure an unknown voltage, ALWAYS start

out by assuming the voltage will be much higher than what you

expect. For example, you may guess that you are going to get

6 volts out, but START OUT on at least the 100 VAC scale. Once

you have the approximate value discerned, then turn things off,

adjust the meter to the appropriate scale (say 10 VAC), and

then make a final measurement after turning things back on.

Another note of caution. When measuring voltage from a

transformer with many windings, please put a piece of electrical

tape on the unused windings. Otherwise a few of them may

inadvertantly touch one another, *YOU*, or something or

someone else. Always exercise prudent caution when working

with live electrical circuits. Personally, I do not even change

meter settings unless the circuit is off or I have removed the

test lead. It's a pain, but you and your meter will both

live longer.

The Plasti-Dip is not designed as a High Voltage insulator,

but it is still somewhat useful. It is useful to cover exposed 120VAC

wiring, such as the exposed 120VAC terminals on a Neon.

I have found Red Devil Clear Caulking Adhesive to be a good and

rather inexpensive all-purpose HV material for use in Tesla Coiling.

You can use it to hold things together and also insulate things.

It dries reasonably fast and has a hard rubbery consistency

once fully set. This is one of the items that I use ALL the time

for a thousand and one different things! Get a caulking gun so

you can apply it.

Hope this helps,

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Answer: TC & Tuning basic

Date: Sun, 23 Nov 1997 21:40:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 23 Nov 1997 16:40:32 -0500

Subject: Answer: TC & Tuning basics

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: TC & Tuning basics

> Date: Sunday, November 23, 1997 10:41 AM

>

> From: Chris Federico <cfed@comsource.net>

> Date: Sun, 23 Nov 1997 09:41:44 -0600

> Subject: TC & Tuning basics

>

>

> How about an explanation of the basics of tuning a TC?

>

> I know that the number of turns tapped into on the primary effects primary

> field strength, and the field inducts the current in the secondary. . . .

>

> But what about total spark gap distance?  From reading Mike's post about RFI

> it sounds like gap length effects the amount of charge stored and released

> by the cap*.  True?  And, if so, does reducing SG distance reduce cap

> charging and thus field strength of the primary?  And, if so, could I reduce

> SG distance and then increase coupling (more primary turns, or lower the

> secondary into the primary field) and get different, hopefully better,

results?

>

Spark gap distance directly affects the maximum voltage to which the

capacitor will charge. Q=C*V  That is, total charge in Coulombs is a

function of the product of voltage and capacitance. Charge has to do

with the total number of electrons being "stored" in the capacitor.

Opening a spark gap wider increases the voltage to which the cap can

charge...  and also the voltage that will appear across the OTHER TC

components. Interestingly enough, the transformer and cap can "resonate"

such that a 15KV neon and a cap can resonate (at 60Hz) to produce a

voltage that exceeds the transformer rating by a factor of three or more

quite readily. Such as in 50KV for a 15KV transformer. If you open

the main spark gap too wide, poof goes a cap or transformer. That is why

intelligent coilers always employ safety gaps. Yeah, well, Ok, there are a few

people that are generally intelligent who err in this matter and pride

themselves on not having lost a cap or tranny. So far. But eventually physics will

prevail. Remember, once a cap or tranny is damaged, it goes downhill from

there. Exercise prudence and equip your TC components with safety gaps.

They may seem superfluous to you... at least until you lose your first

expensive TC component to resonant rise or kickback.

It is useful to use a segmented or multi-gap main spark gap. This dissipates

the heat over several sections, and allows more power to be processed without

overheating. The RQ (Richard Quick) spark gaps pioneered by Richard Hull are

good examples of segmented spark gaps.

You asked about the relationship between Spark Gap distance and coupling.

Apples and oranges, I am afraid. Increasing gap distance normally increases

max cap voltage (assuming the gap actually ever fires!). Coupling has to do

with how the primary energy is transferred to the secondary (and vice versa!)

Generally speaking, as you increase coupling you increase the number of

flux lines that cut through both primary and secondary. For example, two coils

separated by ten feet would not have very high coupling. Two bifilar wound

coils might have a coupling approaching 1. But always less than 1.

For classical Tesla coils the coupling normally ranges from .2 to about .3

In Magnifier coils the primary/secondary coupling is much higher, normally

around .6

Coupling is not so much a matter of the number of turns, but the geometry

of the two in relation to one another. As two separate coils are brought

closer together, the coupling will increase. Larger diameter coils couple

more readily than small diameter coils.

Raising and lowering your secondary changes the coupling. In general, for

the classical base-grounded Tesla coil, the coupling is adjusted by raising

the secondary above the lowest turn on the primary. This reduces coupling.

If you LOWER the secondary below the primary you complicate matters as far

as the coupling goes. Yes, you can run a Tesla coil with a solenoid secondary

that is raised above the base of the primary. But that does not mean that

that is optimal. There are MANY ways that you can make a Tesla coil that

works. The key is to find the ways that make it work WELL. Or even better,

BEST!

> And, does the current need to necessarily induced at, or below (if

> over-coupling is a problem with a solenoid primary), the lowest turn on the

> secondary?  Or if the base of the primary is above the bottom turn of the

> secondary am I simply "reducing" the length of my secondary?

>

You obviously cannot induce any useful currents "below the lowest turn

on the secondary", simply because if it is, indeed, the LOWEST turn,

then there are no more turns below it. And you cannot induce a current

into "thin air". But I think I know what you are trying to get at, even if

the verbiage is somewhat muddled. You want to know what happens if you

drop the secondary below the primary. And that is something very useful to

know.

Some people find that their coil works best if they drop their secondary

below the primary. Well, yes, it may in fact seem that way, but if they

took the time to try RAISING their secondaries and packing their primary

tighter, they would find that they could accomplish the same thing. Of

course, if a peson has already build a loose solonoid primary and is very fond

of it, then he may be loath to actually try anything else. For over 20 years

I used solenoid primaries. And tall, thin secondaries with really fine wire.

But today I use wider secondaries with a max H/D ratio of about 4 to 1, and

mostly flat spiral secondaries. And LARGE toploads.

Now don't get me wrong. Solenoid primaries are great. For magnifiers. But

they tend to over-couple on classical Tesla coils. I do use them on multi-layer

Oudin coils, but these suckers run at much lower frequencies. Everything

has its proper place. "A" is not always better than "B". But there are

definitely places where "A" is better than "B". And places where "B" beats

the socks off of "A". Hey, do what *I* do: experiment. and then make up

your mind based on the results. But always keep an open mind!!

By the way, the chief joy in building a Tesla coil is not necessarily in

running it. Many of us get great pleasure from the building. That explains

why so many coilers takes such great delight in making their creations

beautiful to behold.

> Also, how about a some talk about how the whole of a TC's resonant circuit

> actually works together as it relates to resonance.  I guess I don't "get"

> how there is a resonance set up between the primary HV circuit and the

> secondary.  Or is the only resonance of components, that really matters, in

> the primary circuit, and the resulting inductance provided to the secondary

> just a unique way to use that resonance and not part of the resonant circuit

> at all?

>

OK, this is a very important and FUNDAMENTAL point: A Tesla coil is not

"A TUNED CIRCUIT". It is TWO TUNED CIRCUITS. The Tesla coil only really

"works" as a true Tesla coil when the resonant frequency of the primary

circuit (Primary and Main Cap) are exactly tuned to the (current) resonant

frequency of the entire secondary circuit (secondary, inter-turn capacitance,

self-capacitance due to size and shape [Medhurst], and glorious Top Load

capacitance).  Hmmm. You are wondering why I said (current) resonant frequency.

The Tesla coil's resonance is affected by things like the presence of your

body in the same room. When the Tesla coil is operating it has an ion

cloud around the secondary and the topload that CHANGE its actual operating

frequency from that which you have so meticulously calculated using

resonance formulae and Wheelers's and Medhurst's formulae. Fact is, until

your coil is actually functioning, you don't know the EXACT operating

frequency. That is why we have tappable primaries! After all our careful

measuring and calculating, we still have to slide the old tap around to

get "best spark".

> *And, back to caps, I'm guessing the cap gets charged 60 times a second and

> releases the stored voltage 60 times a second, right?  So while the cap's

> charging, what's happening to the primary winding?  Is it dropping to zero,

> or close, or is it still getting some juice and the subsequent pulse from

> the cap is the main source of amplified voltage/resonance?

>

Close. It charges 120 times a second. It charges on both the positive and

negative halves of the 60Hz wave. Some people use synchronous spark gaps

to ensure that they have 120 Breaks Per Second (BPS).

While the cap is charging, there is a low amount of charging current going

through the primary. In a typical coil with a 12KV 30 ma Neon, it will be

about 30 ma (or more if using resonant charging). This charging current

is next to nothing and can safely be ignored as far as secondary effects

are concerned. The "Magic" happens when the cap charges to the breakdown

voltage of the main spark gap. Then the energy stored in the cap is released

in microseconds, and hundreds to thousands of amps happily surge into the

primary. Yep, that massive piece of copper tubing wound in the shape of a coil

will actually get the FULL 12KV*1.414 across it. At that instant it may have

a DC resistance of a fraction of an ohm, but an AC resistance that is much

larger.

The HEAVY current now flowing causes a massive magnetic flux that extends

from the primary and envelops at least a portion of the secondary. If the

secondary is tuned to the SAME resonant frequency as the primary, it will respond

by creating a wave that reaches a maximum voltage node right at the top

of the coil. (If not, it develops the HV node somewhere else, and you

experience nasty HV breakout along your secondary).

The base of the Tesla coil has massive BASE CURRENTS. Thus the need for

a really GOOD RF ground system! You can think of the topload spark as

jumping off of the coil. It needs a good solid ground to push against!

If a spark breaks out from the topload, then the energy from the primary is

dissipated, and the spark goes out on the "first notch". If the spark has not

yet broken out, then the secondary energy "returns" to the primary. This

returned

energy may keep the poor main spark gap firing... in which case you do NOT

have first notch quenching, and energy in the system is "wasted" (as far as we

spark-loving Tesla coilers are concerned). It is hard to quench on the first

notch.

Rotary spark gaps help, as does a blast of air, as in a vacuum gap, or forced

air gap. Special rotary spark gap designs have been made which effectively

increase the speed with which the spark is "pulled". This helps.

A vacuum RQ gap is a tremendous improvement over two carriage bolts. Do not

neglect the design of the spark gap. Bad design here will limit maximum

energy transfer drastically!

> Finally, and just out of curiosity, how long does your average mineral oil

> immersed polyprop plate cap actually hold a charge after the power's turned

> off?  I almost always discharge mine before fiddlin' whith the primary

> circuit, but have never even seen the hint of a spark.

>

Depends on what you mean by "hold a charge". If you charge it up to say 15KV,

it will discharge fairly rapidly down to a few KV due to corona losses. Ah, but

then it has much less leakage, and can hold a rather nasty charge for extended

periods of time. How extended? Depends on your pain threshold. I have had

polypropylenes hold a charge of several KV for hours. I have measured several

hundreds of volts of charge for weeks. Also, be aware of the fact that after a

capacitor is discharged, it can spontaneously regain a charge of several

hundred volts with ease.

When storing HV caps, short out the terminals with a bare wire.

Now, normally the main cap gets discharged via the primary and/or transformer

(depending on how you wired it). However, as a general safety precaution,

ALWAYS discharge the main cap before inserting your fragile human body into

the circuit. Just this past week on Chip's Tesla list someone posted about

how they had tried moving the primary tap WHILE THE COIL WAS OPERATING. After

the experiment the person in question touched the main cap and got a generous

jolt of electricity. Thank God he is still alive! ALWAYS DISCHARGE CAPS!

> Sorry for the long post, hope the questions aren't too stupid (I'm not sure

> I even know enough to ask intelligent questions).

>

> Thanks . . . Chris

>

Well, Chris, I hope I answered at least some of your questions. Good Questions!

That is the first sign of a really intelligent person.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: spark gaps

Date: Mon, 24 Nov 1997 05:18:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Sun, 23 Nov 1997 23:18:27 -0600

Subject: spark gaps

Organization: MailExcite  (http://www.mailexcite.com)

Hey everybody! Well, I have a few observations and questions and I'm wondering if

anyone has any comments or thoughts about them.

To power my coil, I use a flyback transformer from an old TV. Is this a good idea,

and does anyone know what voltage these things generally run at?

Also, I've noticed that the longer the spark gap, the longer the spark on my secondary.

Then I hear about spark gaps that fire 50 times a second. Well, when I built one

that did fire about that fast, the performance dropped. I noticed as I let the gap

fire rapidly, the performance also dropped radically.  What's going on?

One more thing. My secondary coil is 3"x9" and it is normally powered by an 8kv xformer.

After reading some articles on the net, I heard that a torrid was optional, so I

left this part out. Well, I'm hoping some of you experienced guys can spot my problem

and help me out! (I'm a newbie, can you tell? :) ) Here's a few Q's:

1: How do you determine capacitance without an expensive meter?

2: Is a toroid necessary? If so, how close does it need to be to the top of the    secondary i.e. can the end of the secondary wire run a few inches to the toroid?

3: I've noticed that the more primary windings that I have, the better the output.

I have a coil with about 25 primary windings, and even still, the more I wind on

this one, the better (I've still only got about 3 inch sparks!). Well, I've never

even heard of any more than 12 windings, so something must be terribly wrong!!

Any common problems that might be causing this?

4: How exactly do you 'tap' a primary?

These 3 inch sparks are driving me crazy! I crave foot long ones! :) Any comments

are much appreciated!!

--Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: How does one make high Q

Date: Mon, 24 Nov 1997 08:19:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: pacster@ibm.net

Date: Mon, 24 Nov 1997 08:19:37 GMT

Subject: How does one make high Q caps?

Hello !

Finally!, my exams are over for the year, so now I can get coiling again (I

definitely prefer coiling to exams). I was reading some posts on capacitors

and it was said that for a 15KV cap. the polyethelene should be around

90mils thick. Coming from a 'S.I. units' country I'm a little confused, is

that thickness 24.5/1000 x 90 = 2.28 mm? If so then its no wonder that my

caps cook after only one minute of coil operating, I was using 0.45 mm of

poly between plates, oops!

What is the recommended thickness for 7.5, 9, 10, 12 and 15 KV poly capacitors?

I am hoping to build high Q caps from now on and after a fair bit of reading

apparently these can improve the capacitor Q:

1. Clean all sides of the poly with meth. spirits.

2. Round all foil edges.

3. Keep the plates as tight as possible .

4. Add lay sheets of foil between the layers of poly.

5. Add sheets of oil soaked Craft paper next to each live foil plate.

6. Provide the biggest terminal ends for each plate.

7. Put the caps in series.

8. Submerge the cap in light mineral oil.

9. Use equal numbers of + and - plates.

10. When using caps in series they should be the same individual part capacitances.

11. Vacuum pressurising the caps.

12. Keep all dust and dirt out of the cap.

What is the most useful in improving the Q and what has little effect on the

Q, is there anything else that improves the Q? Normally I roll my caps, but

I think that if I should be using a thickness 2.3mm of poly this could be a

bit of a task. But perhaps I could make ten 10nF caps and parallel them up

to 100nF and then series four sets of them down to 25nF?

I would also like to take a look at my coil on the scope when it is running.

To do this can I just connect a piece of wire to the 10x probe and dangle

the wire in the air about 10 to 15 feet away from my coil (do I need to

connect the scope earth to anything ?), and then fire up my coil, with

hopefully my new high Q caps (so that I can watch it for more than a minute

at a time). Is this O.K. or is there a safer and better method?

Thanks all for your time everyone!

Mark

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Answer: TC & Tuning b

Date: Mon, 24 Nov 1997 16:28:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Mon, 24 Nov 1997 11:28:42 -0500 (EST)

Subject: Re: Answer: TC & Tuning basic

At 09:40 PM 11/23/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Sun, 23 Nov 1997 16:40:32 -0500

>Subject: Answer: TC & Tuning basics

>

>

>

>----------

>> From: Tesla-2 <tesla-2@emachine.com>

>> To: tom_mcgahee@sigmais.com

>> Subject: TC & Tuning basics

>> Date: Sunday, November 23, 1997 10:41 AM

>>

>> From: Chris Federico <cfed@comsource.net>

>> Date: Sun, 23 Nov 1997 09:41:44 -0600

>> Subject: TC & Tuning basics

>>

>>

>> How about an explanation of the basics of tuning a TC?

>>

>> I know that the number of turns tapped into on the primary effects primary

>> field strength, and the field inducts the current in the secondary. . . .

>>

>> But what about total spark gap distance?  From reading Mike's post about RFI

>> it sounds like gap length effects the amount of charge stored and released

>> by the cap*.  True?  And, if so, does reducing SG distance reduce cap

>> charging and thus field strength of the primary?  And, if so, could I reduce

>> SG distance and then increase coupling (more primary turns, or lower the

>> secondary into the primary field) and get different, hopefully better, results?

>>

>

>Spark gap distance directly affects the maximum voltage to which the

>capacitor will charge. Q=C*V  That is, total charge in Coulombs is a

>function of the product of voltage and capacitance. Charge has to do

>with the total number of electrons being "stored" in the capacitor.

<BIG SNIP>

>Fr. Tom McGahee

Capacitors confuse me! I thought the total number of electrons within a

capacitor does not change. For example, when we "charge" a capacitor, we are

not storing any additional electrons in it, we are just removing electrons

from one plate and placing them on the opposite plate. One plate has more

electrons now, and the other plate has less electrons, creating a charge

imbalance.

So, if we are not adding any new charge, charging a capacitor must not mean

storing charge, nor storing electrons, but storing energy. But how is energy

stored??? I think that the potential difference does work by causing the

separation of charge, and the higher the voltage, the more charges are

separated, and this separation of charge is energy or the ability to do

work, but I am still unclear about it.

If voltage causes charge separation, surely there must be some limiting

voltage at which all the charges have separated, assuming we have an

indestructible capacitor? So if we increase the voltage now on the

capacitor, does that mean it will no longer store energy?

I remember a post where someone said that energy is stored in the dielectric

and not on the plates, and conductors are incapable of confining charge, but

can only transport it. And that they could carefully disassemble a charged

capacitor, mail the dielectric to someone else who then assembles a

capacitor using that same dielectric and their own plates, and amazingly

enough they have a charged capacitor. Or, if you short out a capacitor, let

it sit for awhile, it will still develop some potential across its terminals

due to "dielectric memory".

So if this is true, does this mean that the electrons which are charged

particles somehow leave the plate and go into the dielectric? If not, then

the plates must still have a charge imbalance. So how is energy stored then

if the dielectric is air or a vacuum? And how does the dielectric material

effect the capacitance value? Now, don't you see why capacitors confuse me?

I am still amazed at this point that I do not have a good grasp on how

capacitors store energy and yet they are such a vital part of tesla coils.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC & Tuning basics

Cc:

Bcc:

X-Attachments:

At 03:41 PM 11/23/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sun, 23 Nov 1997 09:41:44 -0600

>Subject: TC & Tuning basics

>

Chris,

Since Fr. Tom did such a good job on answering your questions

I wont repeat what he went over. I would like to comment

some on this one question.

>

>How about an explanation of the basics of tuning a TC?

>

I posted a short discourse on coil tuning on this list

a few months ago. With all the new subscribers it would

be of benefit to repost it in response to your question.

<begin include>

Several people have asked me via email and on the

IRC Tesla channel about how to go about tuning

a coil and the steps to take.

So I thought I would share some info on the subject with the list.

Once a Tesla coil is set up and ready to fire, certain steps

should be taken before power is applied. Don't expect to throw

full power into a new coil or a newly rearranged system. If you do

you may damage components. The coil must be brought into tune

and up to full power gradually.

The use of a variac is highly recommended for controlling

the input voltage to whatever step up transformer you

may be using. (neon, obit) Never throw full power to a coil

until you are sure the system is in proper tune.

Set the primary circuit up first. Double check all of your

connections. Run your grounds and double check those. Never take

any connection for granted. Set the secondary into the

primary and attach the RF ground. Use a small discharge terminal

at first. A small spherical terminal works well at this point.

A smaller terminal will break down and produce spark

at a lower power level than a larger terminal. This allows

you to search for the proper tune with less stress placed on

components. If you dont have a smaller terminal and only have

a large toroid place a point discharger on the toroid. Sparks

will easily break out from such a point. Often times a well placed

thumbtack will work well.

CLOSE YOUR SPARK GAPS DOWN!

I can't overemphasize this point. Don't run a lot of gap.

This will limit the peak voltages in the primary to low

levels while things are out of tune.

As things are brought closer to resonance the gap can be opened

a little at a time to increase the primary energy level.

Apply a bit of power until the spark gap fires. If no spark

breakout is seen from the discharge terminal don't worry. Use a

flourescent bulb as an indicator. As the coil gets closer to resonance

the farther away and brighter the bulb will glow. Turn off the coil

and move the tap connection on the primary coil a half turn and

check the bulb brightness. Then move it a full turn in the

opposite direction and note the brightness of the bulb again.

This way you can determine the direction the tap needs to be

moved to locate the proper tune.

Open the main and safety gaps up in steps. Apply the same voltage

input (low power) and check for discharge. Once spark has begun

to break out of the discharge terminal use it as a visual

indicator of tune.

If you find the best tune point is with very few primary

turns then your primary cap is too large. Adding a larger toroid

to your secondary will lower its resonant frequency and allow

you to tap in more primary turns also.

If you get tuned all the way out to your last primary turn

and you still havnt found the resonant point then your primary

cap may be too small or your toroid too large.

This often happens with beginner coils wound with far too

many turns of small guage wire. The large number of secondary turns

causes the secondary resonant frequency to be very low and beyond the range

of your primary circuit. Cutting down the secondary to no more than 900-1000

turns will help here.

A system in proper tune should not break down the safety gap often. The

safety gap once set properly should not be too active. Once the coil

is in good tune the safety gap electrodes may be opened to the point

where it only fires occasionally.

If your safety gap is firing continuously then you may be out of tune

or you may be running too wide a main gap. Try closing down your main gap

slightly and retuning for best output.

Hope this is helpful.

As always comments, criticism and large sums of money welcome.

TTYL!

<end include>

The only thing I would like to add to that is you should

always work towards having as many primary turns tapped in

as possible. Higher primary inductances improve gap operation

and more fully envelope the secondary with its magnetic field.

One thing you can do to allow for more primary turns include

adding a larger toroid to the secondary. This will give several

benefits. The lower resonant frequency will allow for more primary

turns and the larger capacitance represented by the toroid will

give longer hotter sparks. Plus lower frequencies give longer sparks

due to the lowering of several loss factors like corona loss and

loss due to skin effect.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: mw transformer and se

Cc:

Bcc:

X-Attachments:

At 05:30 PM 11/23/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Sun, 23 Nov 1997 12:30:41 -0500

>Subject: Re: mw transformer and sealer

>

>

>

>----------

>> From: Tesla-2 <tesla-2@emachine.com>

>> To: tom_mcgahee@sigmais.com

>> Subject: mw transformer and sealer

>> Date: Saturday, November 22, 1997 7:52 PM

>>

>> From: Chris Federico <cfed@comsource.net>

>> Date: Sat, 22 Nov 1997 18:52:35 -0600

>> Subject: mw transformer and sealers

>>

>>

>> Anyone know an easy way to figure out the voltage output of a microwave

>> transformer?  I was thiknin that I'd hook-up a 9v battery to the primary and

>> check the voltage on the secondary then do the math to get the 120vac input

>> = Xvac output.  Will this work?  (DC vs AC inputs really shouldn't make a

>> difference, should it?)

>>

>Chris,

>You *cannot* determine the voltage by applying a DC at the primary

>(or secondary, for that matter). Transformers must operate with a

>changing input signal. This is usually a sine wave, which is the

>shape of AC from your wall socket.

>

>Here is the simplest way to measure the value of your microwave

>transformer. Since most people do not have meters that can take the

>approximately 2,500 volts AC that appears on the secondary of a

>neon transformer, it is actually best to do things somewhat

>BACKWARDS :)

>

>Apply 120 VAC to the SECONDARY and measure the voltage at the

>primary. (The voltage applied may be slightly off from 120 VAC...

>record the actual applied voltage wherever I say 120VAC)

>

>Put the larger voltage over the smaller voltage and multiply by

>the wall socket voltage to get the HV output.

>

>For example, let's say you put 120 VAC into the secondary and

>got 6 volts out.

>

>HV= (120*120)/6

>so HV=2,400 volts

>

>One note of caution: The secondary has one side connected to the

>transformer case in most microwave transformers. Thus while you

>are performing this test you may possibly have 120VAC at the case.

>So please make sure the transformer case is resting on an

>insulating surface, and do not touch it while making the

>measurements.

>

>This method of measuring can also be applied to neons, with the

>same note of caution. The really good news is that this method

>is very accurate and allows you to use regular low voltage

>meters. On neons you can measure each output independantly,

>by using the case center tap as one connection,or you can attach

>the 120VAC to both HV leads. But be aware that the case center

>tap in EITHER case may have a voltage on it with respect to

>AC ground, so please be careful.

>

<snip>

Fr. Tom and Chris,

I have used the 120VAC line voltage applied to the secondary

of neon transformers method for determining voltages with

mixed results.

It does not seem to give consistant results and often gives

readings that cant possibly be right. I attribute this

to insufficient excitation of the transformer core.

Perhaps it would work better on a microwave oven transformer

but doesnt seem to be a reliable method for neons.

I have had much better repeatable results by simply putting a low

voltage on the neon primary and measuring the resulting secondary

voltage.

For instance with a 12kv neon you might expect to see about 400 volts

output with 4 volts input. A 12kv neon has a nominal 100:1 turns

ratio.

Ive used this method to tag all my neons and can pair

up units closely matched in voltage.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Sundry Questions:

Date: Mon, 24 Nov 1997 17:50:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 24 Nov 1997 09:50:14 -0800 (PST)

Subject: Sundry Questions:

Hello all, time to ask for help from the list, Mr. Hammer has been

much help up to know (thanks :))

I have been running my 3 1/2 x 10" coil using salt water caps for a

few weeks.. and learned a lot. Like why RFC and Bypass caps with

saftey gaps are needed. Scratch one 9KV neon. Live and learn.

Good side effect is I am now running on a salvaged 12KV 30Ma :)

I have switched from a single gap, to an RQ style gap (but I run it

on a flat peice of Polycarb), with this, I am getting 12" sparks.

On to the questions:

WRT Chokes:

I had wound a pair on some 1 1/2" toroids, but could not keep

them from self-arcing, so I am switching to a 2"x6"x28G air core

choke, Measured inductance is ~3.75mH, and I'll be using

1K 60W Resistors in series with these. Any problem with this?

[note that the resistors are wirewound, could this cause me trouble?]

Capacitors:

I have given serious thought to upgrading to rolled Poly caps, or even

flatplate. So I called my local plastics house, and found that they

want to sell me a roll of LDPE 4'x85' 60mil for $238. Needless to say,

the wife will frown heavily on that cost. [nevermind the fact that at

8' strips this will make 15 caps.] Am I getting taken for a ride? My

other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant of 3.6.

This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

well, but I don't have the price handy.],

The question is: does anyone know what the RF losses for kapton are?

would it work for a tesla coil? [ Note that we use it in our

RF Amp for our syncrotron, but we need the standoff more than the

RF loss protection].

If I use the Kapton, I will probably go for flat-plate, as even

at 2mil, this stuff is farily rigid. How do you go about pressing

the plates togther? rigid plastic held with nylon screws?

My "Next" Coil:

I was considering a 6" x 24" x 22G secondary, but have gotten

suggestions that I consider a 4.5 x 21 x 24G instead. I was going to

use PVC as the form. All bets may be off tho. What do you all think

of PolyCarb as a coil form? I may fall into a bunch of "scrap" 0.25"

thick PolyCarb tubing, various sizes, from 3" out to 8" ID.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Sundry Questions:

Cc:

Bcc:

X-Attachments:

At 05:50 PM 11/24/97 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Mon, 24 Nov 1997 09:50:14 -0800 (PST)

>Subject: Sundry Questions:

>

>Hello all, time to ask for help from the list, Mr. Hammer has been

>much help up to know (thanks :))

>

>I have been running my 3 1/2 x 10" coil using salt water caps for a

>few weeks.. and learned a lot. Like why RFC and Bypass caps with

>saftey gaps are needed. Scratch one 9KV neon. Live and learn.

>Good side effect is I am now running on a salvaged 12KV 30Ma :)

>I have switched from a single gap, to an RQ style gap (but I run it

>on a flat peice of Polycarb), with this, I am getting 12" sparks.

>

>On to the questions:

>WRT Chokes:

>I had wound a pair on some 1 1/2" toroids, but could not keep

>them from self-arcing, so I am switching to a 2"x6"x28G air core

>choke, Measured inductance is ~3.75mH, and I'll be using

>1K 60W Resistors in series with these. Any problem with this?

>[note that the resistors are wirewound, could this cause me trouble?]

This sounds just fine to me. Note that wirewound resistors have inductance

and inductance is just what you need. Wirewounds are just fine in

this application. Dont forget the safety gap.

>

>Capacitors:

>I have given serious thought to upgrading to rolled Poly caps, or even

>flatplate. So I called my local plastics house, and found that they

>want to sell me a roll of LDPE 4'x85' 60mil for $238. Needless to say,

>the wife will frown heavily on that cost. [nevermind the fact that at

>8' strips this will make 15 caps.]

60 mil thicknesses of poly are not cheap and really not the

best choice for robust caps. Multiple layers of thinner material

will have a better voltage rating than a single thick sheet.

10 layers of 6 mil poly will have a much greater voltage capability

than a single 60 mil sheet. Plus the fact that 6 mil poly is available

inexpensively at lumber yards and home improvement centers.

Always use multiple caps in series for voltages over 10kv.

Single caps are prone to failure at 12-15kv no matter what their

dielectric thickness. For the utmost in reliability I recommend

2 series caps for 12kv and 3 in series for 15kv.

You want the individual caps to have at least 24 mils of dielectric.

This gives you the equivalent of 48 mils for 12kv and 72 for 15kv

provided run run 2 in series for 12kv and 3 in series for 15kv.

> My other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant

>of 3.6. This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

>well, but I don't have the price handy.

>The question is: does anyone know what the RF losses for kapton are?

>would it work for a tesla coil? [ Note that we use it in our

>RF Amp for our syncrotron, but we need the standoff more than the

>RF loss protection].

Kapton is the trade name for polyimide. It has good voltage characteristics

and a good dielectric constant but, (isnt there always a but?)

its RF dissipation factor leaves a little to be desired.

LDPE has a dissipation factor of .08 at 1 mhz. This very low

figure is why its preferred for caps.

Polyimide (PI) has a dissipation factor of 5 at 1 mhz.

This means it has a dissipation factor over 60 times greater

than LDPE. This puts in in a similar category as mylar with

a 1mhz dissipation factor of 20.

My advice is to forget the Kapton and go for polyethylene

or polypropylene.

>If I use the Kapton, I will probably go for flat-plate, as even

>at 2mil, this stuff is farily rigid. How do you go about pressing

>the plates togther? rigid plastic held with nylon screws?

I built my flat plates caps into a rectangular tank and wedged

spacers between the caps and tank walls to hold everything tight.

>

>

>

>My "Next" Coil:

>I was considering a 6" x 24" x 22G secondary, but have gotten

>suggestions that I consider a 4.5 x 21 x 24G instead. I was going to

>use PVC as the form. All bets may be off tho. What do you all think

>of PolyCarb as a coil form? I may fall into a bunch of "scrap" 0.25"

>thick PolyCarb tubing, various sizes, from 3" out to 8" ID.

>

Polycarbonate plastic also known as Lexan would make an excellent

form material having much lower loss than commonly used PVC

pipe. If you get the Lexan cheap or free by all means use it.

If you have to buy stick with properly prepared PVC. The difference

wont be equal to the extra cost.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gaps

Cc:

Bcc:

X-Attachments:

At 05:18 AM 11/24/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Sun, 23 Nov 1997 23:18:27 -0600

>Subject: spark gaps

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Hey everybody! Well, I have a few observations and questions and I'm wondering if

>anyone has any comments or thoughts about them.

>

>To power my coil, I use a flyback transformer from an old TV. Is this a good idea,

>and does anyone know what voltage these things generally run at?

Anywhere from 15-30KV and current levels around 1-2ma. The best ones

are from old BW televisions. More modern ones run at lower voltage

into a voltage tripler. Flybacks are only suitable for plasma globes

and small low power Tesla coils.

>

>Also, I've noticed that the longer the spark gap, the longer the spark on my secondary.

>Then I hear about spark gaps that fire 50 times a second. Well, when I built one

>that did fire about that fast, the performance dropped. I noticed as I let the

>gap fire rapidly, the performance also dropped radically.  What's going on?

The idea behind the higher firing rate for rotary gaps is to

recharge and discharge the main cap many times per 1/2 AC cycle.

I would say your flyback supply is not supply enough energy for

that to be possible. You are probably only partially charging

your cap at each gap presentation.

You would get better performance from a well designed cylinder type

static gap.

>

>One more thing. My secondary coil is 3"x9" and it is normally powered by an 8kv

>xformer. After reading some articles on the net, I heard that a toroid was optional,

>so I left this part out. Well, I'm hoping some of you experienced guys can spot my

>problem and help me out! (I'm a newbie, can you tell? :) ) Here's a few Q's:

>

>1: How do you determine capacitance without an expensive meter?

Well if you mean how to determine tha capacitance of a homemade

capacitor then the best way is to calculate it by formula.

This gives fairly accurate results.

Capacitance (in microfarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value derived from this

formula must be multiplied by two, or, the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

Polyethylene has a dielectric constant of between 2.0 and 2.3.

A nominal value of 2.0 works well on home constructed caps.

>

>2: Is a toroid necessary? If so, how close does it need to be to the top of the

> secondary i.e. can the end of the secondary wire run a few inches to the toroid?

No a toroid is not necessary but it sure helps to give you long sparks

and suppress corona near the top of the secondary. For the toroid to successfully

suppress corona it must be down close to the top of the secondary winding.

It can be located a little above the secondary winding but certainly not

any more than 2 or 3 inches.

Other materials can be pressed into service as top loads. Ive often used

a copper toilet tank float. Works well. Foil covered styrofoam balls

work well. Ive used various types of metal cans with varying success.

>

>3: I've noticed that the more primary windings that I have, the better the output.

>I have a coil with about 25 primary windings, and even still, the more I wind on

>this one, the better (I've still only got about 3 inch sparks!). Well, I've never

>even heard of any more than 12 windings, so something must be terribly wrong!!

My own coil has 17 primary turns and works best with as many primary

turns as I can get.

>Any common problems that might be causing this?

Well I would say that if as you add more primary turns one at a time that you

keep getting better output and you get best output with all the primary turns

in that you are not quite reaching resonance. Your secondary coil is resonating

at a lower frequency than your primary can tune down to.

The cure is to lower the frequency your primary will tune to or raise the

frequency of the secondary. Since you already have 20 turns in your primary

I would add some additional primary capacitance. If it would be difficult

to increase the size of the main cap then you can raise the frequency of your

secondary some by removing a few turns. Beginners often fall prey to the

"more turns are better theory" and wind secondary coils with 1000s of turns.

You really only need 700-1000 turns. If you have more than 1000 turns pull

some off and that will raise the secondary frequency.

>

>4: How exactly do you 'tap' a primary?

I use a large copper alligator clip with extra copper pieces

soldered to the jaws to increase surface area.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Questions:

Date: Mon, 24 Nov 1997 23:20:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 24 Nov 1997 18:20:36 -0500

Subject: Re: Sundry Questions:

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Sundry Questions:

> Date: Monday, November 24, 1997 12:50 PM

>

> From: baumann@proton.llumc.edu

> Date: Mon, 24 Nov 1997 09:50:14 -0800 (PST)

> Subject: Sundry Questions:

>

> Hello all, time to ask for help from the list, Mr. Hammer has been

> much help up to know (thanks :))

>

> I have been running my 3 1/2 x 10" coil using salt water caps for a

> few weeks.. and learned a lot. Like why RFC and Bypass caps with

> saftey gaps are needed. Scratch one 9KV neon. Live and learn.

> Good side effect is I am now running on a salvaged 12KV 30Ma :)

> I have switched from a single gap, to an RQ style gap (but I run it

> on a flat peice of Polycarb), with this, I am getting 12" sparks.

>

> On to the questions:

> WRT Chokes:

> I had wound a pair on some 1 1/2" toroids, but could not keep

> them from self-arcing, so I am switching to a 2"x6"x28G air core

> choke, Measured inductance is ~3.75mH, and I'll be using

> 1K 60W Resistors in series with these. Any problem with this?

> [note that the resistors are wirewound, could this cause me trouble?]

>

Measure their inductance. If the inductance is too high, then the

voltage drop across them will be greater than you think, at least

for RF stuff. What you want to guard against is internal arcing

between windings. Many wire wound power resistors have the windings

separated far enough that you would have no trouble. But you may

only be able to tell by trying them out. If they arc internally, take

'em out.

> Capacitors:

> I have given serious thought to upgrading to rolled Poly caps, or even

> flatplate. So I called my local plastics house, and found that they

> want to sell me a roll of LDPE 4'x85' 60mil for $238. Needless to say,

> the wife will frown heavily on that cost. [nevermind the fact that at

> 8' strips this will make 15 caps.] Am I getting taken for a ride?

The price is actually quite good... comes to about $22 for a 4'x8'

section... but the bad news is you have to buy the whole thing. Or

do you? Did you specifically ask if you could buy 4'x8' sections?

That is the size most commonly seen by coilers.

> My  other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant of 3.6.

> This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

> well, but I don't have the price handy.]

> The question is: does anyone know what the RF losses for kapton are?

> would it work for a tesla coil? [ Note that we use it in our

> RF Amp for our syncrotron, but we need the standoff more than the

> RF loss protection].

> If I use the Kapton, I will probably go for flat-plate, as even

> at 2mil, this stuff is farily rigid. How do you go about pressing

> the plates togther? rigid plastic held with nylon screws?

>

Don't know much about Kapton yet. The voltage and dielectric specs are

great, however. If you go ahead with this, keep us informed. What is your

source?

You can make really nice compression plates from 1/4" thick fiberglass

material, like that used as the base for PC boards. Not cheap and hard

to fabricate, though. Chews up drill bits!

I have had reasonable luck with fiberboard backed up with aluminum

angle bar. The angle bar keeps the fiberboard flat and provides the

strength you need while keeping weight low. 1/4" fiberboard and either

"L" or "T" shaped aluminum angle bracket stock. I use two to four

such angle sections and regular metal bolts. Note that I keep the bolts

an extra 1/2" away from the edge of the poly. Improvise! I once used

a sort of composite pressboard 3/4" thick. Use what you can get.

>

>

> My "Next" Coil:

> I was considering a 6" x 24" x 22G secondary, but have gotten

> suggestions that I consider a 4.5 x 21 x 24G instead. I was going to

> use PVC as the form. All bets may be off tho. What do you all think

> of PolyCarb as a coil form? I may fall into a bunch of "scrap" 0.25"

> thick PolyCarb tubing, various sizes, from 3" out to 8" ID.

>

Great stuff for forms. Just be aware that certain things like alchohol

can cause the surface to craze. Before using anything on the form, always

test with a piece of scrap polycarbonate to see what the effects are.

>

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Sealing Plasma Globes

Date: Tue, 25 Nov 1997 01:01:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Tue, 25 Nov 1997 01:01:17 -0000

Subject: Sealing Plasma Globes

Hi All!,

After much deliberation I've gone the whole hog and actually got a glass

globe blown for me including the center electrode. Now the only problem is

- HOW THE HECK DO I SEAL THE DAMN THING??? AAARRGGGH!! - .Please excuse the

outburst but I've tried every conceivable adhesive to seal my new globe

including glass reinforced plastic and epoxy but nothing will contain the

plasma pressure longer than several minutes. I can't afford to kit up on

glass blowing gear to do the job properly so if anyone has any sensible

ideas I'd love to know about 'em!

Best Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

Date: Mon, 24 Nov 1997 21:51:00 -0500 (EST)

X-Sender: djb@velocity.net

From: Dave <djb@velocity.net>

Subject: Capacitor Analogies

>From: tesla-2@emachine.com (Tesla-2)

>Subject: Capacitor Analogies

>Date: Mon, 24 Nov 1997 22:53:13 GMT

>Organization: The Energy Machine Information System 504-733-3462

>To: djb@velocity.net

>Content-Length: 11428

>

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Mon, 24 Nov 1997 17:53:13 -0500

>Subject: Capacitor Analogies

>

>

>

>----------

>> From: Tesla-2 <tesla-2@emachine.com>

>> To: tom_mcgahee@sigmais.com

>> Subject: Re: Answer: TC & Tuning b

>> Date: Monday, November 24, 1997 11:28 AM

>>

>> From: Dave <djb@velocity.net>

>> Date: Mon, 24 Nov 1997 11:28:42 -0500 (EST)

>> Subject: Re: Answer: TC & Tuning basic

>>

>> At 09:40 PM 11/23/97 GMT, you wrote:

>> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>> >Date: Sun, 23 Nov 1997 16:40:32 -0500

>> >Subject: Answer: TC & Tuning basics

>> >

>> >

>> >----------

>> >> From: Tesla-2 <tesla-2@emachine.com>

>> >> To: tom_mcgahee@sigmais.com

>> >> Subject: TC & Tuning basics

>> >> Date: Sunday, November 23, 1997 10:41 AM

>> >>

>> >> From: Chris Federico <cfed@comsource.net>

>> >> Date: Sun, 23 Nov 1997 09:41:44 -0600

>> >> Subject: TC & Tuning basics

>> >>

>> >>

>> >> How about an explanation of the basics of tuning a TC?

>> >>

>> >> I know that the number of turns tapped into on the primary effects primary

>> >> field strength, and the field inducts the current in the secondary. . . .

>> >>

>> >> But what about total spark gap distance?  From reading Mike's post about RFI

>> >> it sounds like gap length effects the amount of charge stored and released

>> >> by the cap*.  True?  And, if so, does reducing SG distance reduce cap

>> >> charging and thus field strength of the primary?  And, if so, could I reduce

>> >> SG distance and then increase coupling (more primary turns, or lower the

>> >> secondary into the primary field) and get different, hopefully better, results?

>> >>

>> >

>> >Spark gap distance directly affects the maximum voltage to which the

>> >capacitor will charge. Q=C*V  That is, total charge in Coulombs is a

>> >function of the product of voltage and capacitance. Charge has to do

>> >with the total number of electrons being "stored" in the capacitor.

>>

>> <BIG SNIP>

>>

>> >Fr. Tom McGahee

>>

>> Capacitors confuse me!

>

>They confuse a lot of people.

>

>> I thought the total number of electrons within a

>> capacitor does not change. For example, when we "charge" a capacitor, we are

>> not storing any additional electrons in it, we are just removing electrons

>> from one plate and placing them on the opposite plate. One plate has more

>> electrons now, and the other plate has less electrons, creating a charge

>> imbalance.

>

>Yep.

>

>>

>> So, if we are not adding any new charge, charging a capacitor must not mean

>> storing charge, nor storing electrons, but storing energy. But how is energy

>> stored??? I think that the potential difference does work by causing the

>> separation of charge, and the higher the voltage, the more charges are

>> separated, and this separation of charge is energy or the ability to do

>> work, but I am still unclear about it.

>>

>

>OK, so we give the Analogy for the day: Imagine a rubber band. Stretch the

>rubber band. Does it weigh more? No. Does it have any more electrons? No.

>Is there something different about the rubber band in its stretched and

>unstretched modes? You betcha!

>

>Capacitors store (let's call it) energy. In the dielectric. Not unlike

>stretched rubber bands. The stretching in this case has to do with the

>effect of applying an electrical potential. Like charges repel and unlike

>charges attract, and all that stuff. Force. The very Stuff of physics!

>Doesn't matter if the force is from a spring or a rubber band or even due

>to Mr. Gravity. Force acts on things and wants to make 'em MOVE. In the

>case of Mr. Capacitor the "stuff" that moves is electrons. The number of

>electrons is called the Charge. The Force or Pressure is called Voltage.

>The MOVEMENT or FLOW of electrons is called Current.

>

>There are a number of analogies that we can use to help us get a mental

>image of what is going on. Some analogies are better than others. The

>best analogies are those that most closely mimic as many aspects as

>possible.

>

>First an analogy that only mimics a few aspects: Consider a capacitor

>to be like a scuba tank. The bigger the scuba tank the more air it can hold

>at a given pressure. For a given size scuba tank the amount of air stored

>is a function of the size of the tank and the applied pressure. If you

>apply too much pressure the tank can explode. The scuba tank can store

>pressurized air for very long periods of time. If there is a leak, the

>scuba tank will slowly lose pressure and thus actual air as well.

>

>Not a really bad analogy in many ways. But you say, "a full scuba tank

>weighs more than an empty scuba tank." At this point the analogy fails.

>Miserably. But it is STILL a useful analogy as long as you understand

>its limitations.

>

>A better analogy: Imagine a cylinder which is divided into two sections

>by a nice piece of stretchy baloon material right in the middle. Each section

>has a nozzle attached. If you push air INTO the left hand nozzle, the

>pressure will stretch the baloon material and it will move the baloon material

>over to the right a bit. This will result in air EXITING from the right

>nozzle. The two cylinders TOGETHER will still have the same total volume.

>But there is a pressure difference. The baloon material is responsible for

>this state of affairs. Let's call the balloon material Mr. Dielectric. Now,

>as you push more and more air into the left chamber, the balloon material will

>distort more and more to the right. Obviously the SIZE of the chambers

>influences how much of this distortion you can get away with. The distortion

>is caused by the applied pressure differential. If the air was not free to

>come out of the right chamber, then the balloon material would not be free

>to move into the right chamber. OK, now imagine a sturdier, heftier balloon

>material. It takes more pressure to get it to move, but when it does move it

>pushes back with a vengeance. [Different dielectric materials influence

>the total capacitance in much the same way]. Increase the pressure and the

>balloon distortion increases. Up to the point where the poor balloon ruptures.

>[This is similar to the voltage rating on a cap.] The air you pumped IN

>wants to go OUT of the same nozzle it entered. And it WILL if you let it.

>If you cap it off, however, the air inside is trapped. If you let the air

>go OUT the left side, air must also ENTER the right side. The air activity

>at the right nozzle is a mirror image of the air activity at the left nozzle.

>You can only pump so much air into the left chamber before the balloon material

>will burst. [Capacitor destruction. Permanent in this case.] As you put

>more air IN, it becomes harder to get additional air in. When the balloon

>material

>is distended, it wants to go back to its rest position. If you let the air

>freely flow, it can return to this position quite rapidly. Limited by the

>opening size [equivalent to internal resistance.] If you apply a pressure at

>the left chamber air will be forced out of the right chamber. If you move air

>in and out of the left chamber, the same quantity of air will move through

>the right chamber, but opposite in direction (in/out). However, if you are

>looking at the direction not as in/out but as from left to right and from

>right to left, you would say that when air is moving left to right in one

>chamber, it is moving left to right in the other chamber also. Just like a

>real capacitor, AC in gives you AC out. Or to put it another way, AC currents

>readily flow "through" the assembly, even though there is a physical

>barrier (dielectric) that prevents "actual" current flow.

>

>All analogies limp, but this one happens to handle a lot of the capacitor

>characteristics quite well.

>

>

>> If voltage causes charge separation, surely there must be some limiting

>> voltage at which all the charges have separated, assuming we have an

>> indestructible capacitor? So if we increase the voltage now on the

>> capacitor, does that mean it will no longer store energy?

>>

>

>Greater voltages tend to make the charges separate further, because there

>is more pressure. However, for a given dielectric and a given thickness

>of the same, there is a breakdown point where the material is physically

>damaged, so to speak. This corresponds to the volts per mil rating.

>And at high frequencies the dielectric is also going to begin to show

>heating effects. This relates to power factor.

>

>> I remember a post where someone said that energy is stored in the dielectric

>> and not on the plates, and conductors are incapable of confining charge, but

>> can only transport it. And that they could carefully disassemble a charged

>> capacitor, mail the dielectric to someone else who then assembles a

>> capacitor using that same dielectric and their own plates, and amazingly

>> enough they have a charged capacitor. Or, if you short out a capacitor, let

>> it sit for awhile, it will still develop some potential across its terminals

>> due to "dielectric memory".

>>

>

>All true. Unfortunately the double cylinder/balloon model fails to show

>those effects. That is because in a "real" dielectric the atoms themselves

>form both the cylinders AND the balloon part. The atoms can push and

>pull against one another in a way that the model cannot show. The capacitor

>plates are just the connections to the dielectric. All the real action

>takes place in the dielectric. The plates serve to distribute the charge and

>to conduct the current (which is just a moving charge, after all). Because

>the dielectric is an INSULATOR, the conducting plates are needed to have the

>thing ACT like a "capacitor", but you can remove the plates and still

>have the stored charge. A cool experiment, and easy to perform.

>

>> So if this is true, does this mean that the electrons which are charged

>> particles somehow leave the plate and go into the dielectric? If not, then

>> the plates must still have a charge imbalance. So how is energy stored then

>> if the dielectric is air or a vacuum?

>

>A vacuum is nothing. Once you introduce the electrons, it ain't nothin'

>no more :)  A vacuum doesn't store electrons. Put an electron into a vacuum

>and where is it supposed to go? Vacuums have a very high breakdown voltage.

>Which is another way of saying that it takes a lot of pressure to get an

>electron to move from here to there in a vacuum. No CARRIERS available.

>Yeah, a lot of it looks paradoxical at first, but it all makes sense when

>you look at it right.

>

>The good news is you don't have to understand every single aspect of

>capacitors to be able to use them effectively. It's like driving a car.

>You don't have to know how a carburetor works to be able to use it. Just

>make sure you got one, and push in on the gas!

>

>

>> And how does the dielectric material

>> effect the capacitance value? Now, don't you see why capacitors confuse me?

>>

>> I am still amazed at this point that I do not have a good grasp on how

>> capacitors store energy and yet they are such a vital part of tesla coils.

>>

>> Dave

>>

>Sit back, relax and enjoy the arcs and sparks. Soak up the wonder of it all.

>Learn all you can, but realize that your knowledge will always be incomplete.

>And that's perfectly OK. Enjoy the journey, and don't worry about whether

>you are there yet. You're on the way, and that's all that matters!

>

>Hope this helps.

>Fr. Tom McGahee

>

>

From: tesla-2@emachine.com (Tesla-2)

Subject: What would life be withou

Date: Tue, 25 Nov 1997 03:25:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kevin A. Zachary" <kzachary@kih.net>

Date: Mon, 24 Nov 1997 22:25:44 -0500

Subject: What would life be without questions?

Does anyone have any plans for a small to medium sized TC that they would

send me... I have a few parts on hand, like a 3"x24"x24G secondary and a

plate glass cap (but I'd like to try a poly stacked or rolled cap - need

specs though).

What exactly is a safety gap?  Where is it placed in the TC circuit?

Anyone have some close-up pictures of a safety gap, spark gaps, AC chokes,

filters, etc. so that I can get an idea of what to try to build or what to

look for at an electronics shop?

With all of the discussion of discharging caps, inefficient glass caps,

etc.  Its a thousand wonders how I am here today...  I got my plans from an

electronics magazine back in 1980 or so... it called for a 30KV

transformer, the closest thing I could find was a 15KV, hence I divided

everything in the circuit by two and tweaked... don't ask me how I got

8-12" sparks from a single wire at the top...

Someone help me with this fact or myth... If you hold a flourescent tube

over your TC and allow the spark to hit the tube... Will you get

electrocuted if you touch the opposite metal end of the tube?  I have felt

a tingling sensation while holding the glass near the end...

I'm still gathering information... and searching for a mechanical camera!

:-)

KevinZ

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Questions:

Date: Tue, 25 Nov 1997 04:23:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 24 Nov 1997 20:23:24 -0800

Subject: Re: Sundry Questions:

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: baumann@proton.llumc.edu

> Date: Mon, 24 Nov 1997 09:50:14 -0800 (PST)

> Subject: Sundry Questions:

>

> Hello all, time to ask for help from the list, Mr. Hammer has been

> much help up to know (thanks :))

>

> I have been running my 3 1/2 x 10" coil using salt water caps for a

> few weeks.. and learned a lot. Like why RFC and Bypass caps with

> saftey gaps are needed. Scratch one 9KV neon. Live and learn.

> Good side effect is I am now running on a salvaged 12KV 30Ma :)

> I have switched from a single gap, to an RQ style gap (but I run it

> on a flat peice of Polycarb), with this, I am getting 12" sparks.

Congratulations! You're sparklength is longer than your coil length.

That's not too bad for a small coil using saltwater caps!

>

> On to the questions:

> WRT Chokes:

> I had wound a pair on some 1 1/2" toroids, but could not keep

> them from self-arcing, so I am switching to a 2"x6"x28G air core

> choke, Measured inductance is ~3.75mH, and I'll be using

> 1K 60W Resistors in series with these. Any problem with this?

> [note that the resistors are wirewound, could this cause me trouble?]

Looks good! Ferrites are somewhat conductive and the high voltages that

develops across the choke terminals is very difficult to insulate from

the semiconducting core. 3.75 mH air wound chokes should work very well

and the 1K 60 watt wirewound resistors should also be perfectly fine.

>

> Capacitors:

> I have given serious thought to upgrading to rolled Poly caps, or even

> flatplate. So I called my local plastics house, and found that they

> want to sell me a roll of LDPE 4'x85' 60mil for $238. Needless to say,

> the wife will frown heavily on that cost. [nevermind the fact that at

> 8' strips this will make 15 caps.] Am I getting taken for a ride?

That comes to about $22.40 per equivalent 4' x 8' sheet, which is in the

ballpark... although I've had quotes for 4' x 8' sheets for around

$16/sheet in the Chicago area. However, if you're not planning on

building LOTS of caps, this is much more than you'll need, and the price

is not all that more attractive than for single sheets. One possible

benefit is that, being rolled, it may have less scratches and other

handling induced damage.

> My other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant of 3.6.

> This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

> well, but I don't have the price handy.],

> The question is: does anyone know what the RF losses for kapton are?

> would it work for a tesla coil? [ Note that we use it in our

> RF Amp for our syncrotron, but we need the standoff more than the

> RF loss protection].

> If I use the Kapton, I will probably go for flat-plate, as even

> at 2mil, this stuff is farily rigid. How do you go about pressing

> the plates togther? rigid plastic held with nylon screws?

>

Kapton (polyimide or PI) is an excellent, high temperature dielectric.

However, it's also fairly lossy stuff, coming with a loss tangent of 2 x

10-3 at 60 Hz and 5 x 10-3 at 1 MHz.  This is about 10-50 times lossier

than LDPE, and is in the same league as Mylar. Go with LDPE for your

Tesla caps - it's cheaper AND better performing.

> My "Next" Coil:

> I was considering a 6" x 24" x 22G secondary, but have gotten

> suggestions that I consider a 4.5 x 21 x 24G instead. I was going to

> use PVC as the form. All bets may be off tho. What do you all think

> of PolyCarb as a coil form? I may fall into a bunch of "scrap" 0.25"

> thick PolyCarb tubing, various sizes, from 3" out to 8" ID.

>

<SNIP>

Go with larger coilform sizes if you're thinking about increasing power

levels in the near future. You may also want to increase the coil length

a bit more to get more clearance to the primary. PVC, acrylic (PMMA), or

polycarbonate will all work very well. If you use PVC, dry it first, and

seal it with polyurethane. The other materials require no pretreatment.

Safe cappin' and coilin' to you, Michael!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Questions:

Date: Tue, 25 Nov 1997 05:33:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Mon, 24 Nov 1997 21:33:18 -0800 (PST)

Subject: Re: Sundry Questions:

On Mon, 24 Nov 1997, Tesla-2 wrote:

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > I am switching to a 2"x6"x28G air core

> > choke, Measured inductance is ~3.75mH, and I'll be using

> > 1K 60W Resistors in series with these. Any problem with this?

> > [note that the resistors are wirewound, could this cause me trouble?]

> >

>

> Measure their inductance. If the inductance is too high, then the

> voltage drop across them will be greater than you think, at least

> for RF stuff.

This would be a good thing, yes? Since I want them to ruin the Q?

I admit that I am a DC type of guy, this RF stuff is somewhat

pure magic at times :) [And I did, but was not sure if I could

trust what the LCR had to say.. turns out they are 2.5mH, giving

me a total choke of 6.25mH. This left me with *very* thin blue

sparks across the safety gap, about one per 15 seconds of operation.

> What you want to guard against is internal arcing

> between windings. Many wire wound poewer resistors have the windings

> separated far enough that you would have no trouble. But you may

> only be able to tell by trying them out.

No arcs that I could see. Of course these are coated in some pretty heavy

epoxy or similar material.

> The price is actually quite good... comes to about $22 for a 4'x8'

> section... but the bad news is you have to buy the whole thing. Or

> do you? Did you specifically ask if you could buy 4'x8' sections?

> That is the size most commonly seen by coilers.

Yep, Can't. Only decent plastics supplier in the area, so they pretty

much call the shots. Surprised to find that the price is good - the

place is known for gougeing.

>

>

> > My other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant of 3.6.

> > This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

> > well, but I don't have the price handy.]

Slight Correction. The guy that quoted me this price goofed. Should have

been 18" x 25' for $35, still pretty good compared to the poly.

>

> Don't know much about Kapton yet. The voltage and dielectric specs are

> great, however. If you go ahead with this, keep us informed. What is your

> source?

Will do. I am getting some scraps for testing in the next few days.

There are advantages in working for a company that makes particle

accelerators.

I don't have the name of the supplier handy, but if this works out,

I will definitely let everyone know.

I talked with our RF gurus a little more today, and found out

some more particulars: They are using a 5 mil single thickness

to hold off 13.5 KV at 450Mhz, operating in a pulse mode.

To quote one of the guys: 200Khz? That's practically DC!

Oh.. and no oil bath. This is to air, but with 2.5" buffer zones.

>

> You can make really nice compression plates from 1/4" thick fiberglass

> material, like that used as the base for PC boards. Not cheap and hard

> to fabricate, though. Chews up drill bits!

G10? Yeah, brutal stuff on tooling, we have to machine it sometimes

as it does not outgas much. How about 3/8" PolyCarb?  I just got

a bunch of 8"x10" scrap pieces, pretty rigid stuff.

> Hope this helps.

> Fr. Tom McGahee

Quite a bit, thank you. I do hope you will continue to be on this list.

I did see your post about funding - and I hope your order can understand

how this helps many.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Questions:

Date: Tue, 25 Nov 1997 13:36:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 25 Nov 1997 08:36:55 -0500

Subject: Re: Sundry Questions:

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Sundry Questions:

> Date: Tuesday, November 25, 1997 12:33 AM

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Mon, 24 Nov 1997 21:33:18 -0800 (PST)

> Subject: Re: Sundry Questions:

>

> On Mon, 24 Nov 1997, Tesla-2 wrote:

>

> > From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > > I am switching to a 2"x6"x28G air core

> > > choke, Measured inductance is ~3.75mH, and I'll be using

> > > 1K 60W Resistors in series with these. Any problem with this?

> > > [note that the resistors are wirewound, could this cause me trouble?]

> > >

> >

> > Measure their inductance. If the inductance is too high, then the

> > voltage drop across them will be greater than you think, at least

> > for RF stuff.

> This would be a good thing, yes?

Yes.

> Since I want them to ruin the Q?

Definitely.

> I admit that I am a DC type of guy, this RF stuff is somewhat

> pure magic at times :) [And I did, but was not sure if I could

> trust what the LCR had to say.. turns out they are 2.5mH, giving

> me a total choke of 6.25mH. This left me with *very* thin blue

> sparks across the safety gap, about one per 15 seconds of operation.

Looks great!

> > What you want to guard against is internal arcing

> > between windings. Many wire wound poewer resistors have the windings

> > separated far enough that you would have no trouble. But you may

> > only be able to tell by trying them out.

> No arcs that I could see. Of course these are coated in some pretty heavy

> epoxy or similar material.

>

> > The price is actually quite good... comes to about $22 for a 4'x8'

> > section... but the bad news is you have to buy the whole thing. Or

> > do you? Did you specifically ask if you could buy 4'x8' sections?

> > That is the size most commonly seen by coilers.

> Yep, Can't. Only decent plastics supplier in the area, so they pretty

> much call the shots. Surprised to find that the price is good - the

> place is known for gougeing.

>

Sometimes you can get a 4'x8' sheet for as low as $16. I mean for 60 mil.

What we coilers need to do is to identify what kind of END USERS are

buying this stuff in such huge quantities. Sometimes you can get an

end user to sell you a (to them) small section at their cost plus a few bucks.

I buy all my PC board stock from a company that actually makes PC

boards. A couple of times a year I buy a 4'x8' sheet of single

sided G10 at their cost, AND the owner is nice enough to let me

scavenge all the loose ends I find in his waste barrel.

> >

> >

> > > My other option is Kapton, rated at 2.5Kv/Mil (AC), Dielectric constant of 3.6.

> > > This I can get in 18"x50'x2mil for $35, [and 15 mil is available as

> > > well, but I don't have the price handy.]

>

> Slight Correction. The guy that quoted me this price goofed. Should have

> been 18" x 25' for $35, still pretty good compared to the poly.

>

> >

> > Don't know much about Kapton yet. The voltage and dielectric specs are

> > great, however. If you go ahead with this, keep us informed. What is your

> > source?

> Will do. I am getting some scraps for testing in the next few days.

> There are advantages in working for a company that makes particle

> accelerators.

> I don't have the name of the supplier handy, but if this works out,

> I will definitely let everyone know.

> I talked with our RF gurus a little more today, and found out

> some more particulars: They are using a 5 mil single thickness

> to hold off 13.5 KV at 450Mhz, operating in a pulse mode.

> To quote one of the guys: 200Khz? That's practically DC!

> Oh.. and no oil bath. This is to air, but with 2.5" buffer zones.

>

Hmmmm. It looks like your employer is not running this stuff in any kind

of sustained RF environment. A pulse here and there is one thing,

but sustained RF application normally translates into HEATING of the

dielectric. That is why poly is so darned attractive. Dielectric

constant is only 2.2, but it has a very low dissipation factor.

I recommend sticking with poly, but don't be afraid to experiment with

the Kapton. It OBVIOUSLY will have a number of uses for us coilers,

even if not for caps.

Try building a multiple layer dielectric two plate cap. Place it in

parallel with your main TC cap. Let 'er rip for a minute. Power down

and then check the thing for heat. Just remember that in a larger cap

the heat will build up more because of the multiple layers that will

trap the heat. But we only know for sure by actually experimenting.

Never be afraid to try out an idea. And share the results!

> >

> > You can make really nice compression plates from 1/4" thick fiberglass

> > material, like that used as the base for PC boards. Not cheap and hard

> > to fabricate, though. Chews up drill bits!

> G10? Yeah, brutal stuff on tooling, we have to machine it sometimes

> as it does not outgas much. How about 3/8" PolyCarb?  I just got

> a bunch of 8"x10" scrap pieces, pretty rigid stuff.

>

Try it. Use large washers or the like to avoid hole fracturing.

You will probably find that it will bend a bit, but some compression

is better than none at all.

Note that assembly is best done under oil. You want to first get oil

between all surfaces. Then the compression can expel air and oil

with no problem. Trapped air will kill the cap eventually.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitor Analogies

Date: Tue, 25 Nov 1997 13:48:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Tue, 25 Nov 1997 08:48:05 -0500 (EST)

Subject: Re: Capacitor Analogies

At 10:53 PM 11/24/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Mon, 24 Nov 1997 17:53:13 -0500

>Subject: Capacitor Analogies

>> Capacitors confuse me!

>

>They confuse a lot of people.

>

>> I thought the total number of electrons within a

>> capacitor does not change. For example, when we "charge" a capacitor, we are

>> not storing any additional electrons in it, we are just removing electrons

>> from one plate and placing them on the opposite plate. One plate has more

>> electrons now, and the other plate has less electrons, creating a charge

>> imbalance.

>

>Yep.

>

>>

>> So, if we are not adding any new charge, charging a capacitor must not mean

>> storing charge, nor storing electrons, but storing energy. But how is energy

>> stored??? I think that the potential difference does work by causing the

>> separation of charge, and the higher the voltage, the more charges are

>> separated, and this separation of charge is energy or the ability to do

>> work, but I am still unclear about it.

>>

>

>OK, so we give the Analogy for the day: Imagine a rubber band. Stretch the

>rubber band. Does it weigh more? No. Does it have any more electrons? No.

>Is there something different about the rubber band in its stretched and

>unstretched modes? You betcha!

>

>Capacitors store (let's call it) energy. In the dielectric. Not unlike

>stretched rubber bands. The stretching in this case has to do with the

>effect of applying an electrical potential. Like charges repel and unlike

>charges attract, and all that stuff. Force. The very Stuff of physics!

>Doesn't matter if the force is from a spring or a rubber band or even due

>to Mr. Gravity. Force acts on things and wants to make 'em MOVE. In the

>case of Mr. Capacitor the "stuff" that moves is electrons. The number of

>electrons is called the Charge. The Force or Pressure is called Voltage.

>The MOVEMENT or FLOW of electrons is called Current.

>

>There are a number of analogies that we can use to help us get a mental

>image of what is going on. Some analogies are better than others. The

>best analogies are those that most closely mimic as many aspects as

>possible.

>

>First an analogy that only mimics a few aspects: Consider a capacitor

>to be like a scuba tank. The bigger the scuba tank the more air it can hold

>at a given pressure. For a given size scuba tank the amount of air stored

>is a function of the size of the tank and the applied pressure. If you

>apply too much pressure the tank can explode. The scuba tank can store

>pressurized air for very long periods of time. If there is a leak, the

>scuba tank will slowly lose pressure and thus actual air as well.

>

>Not a really bad analogy in many ways. But you say, "a full scuba tank

>weighs more than an empty scuba tank." At this point the analogy fails.

>Miserably. But it is STILL a useful analogy as long as you understand

>its limitations.

>

>A better analogy: Imagine a cylinder which is divided into two sections

>by a nice piece of stretchy baloon material right in the middle. Each section

>has a nozzle attached. If you push air INTO the left hand nozzle, the

>pressure will stretch the baloon material and it will move the baloon material

>over to the right a bit. This will result in air EXITING from the right

>nozzle. The two cylinders TOGETHER will still have the same total volume.

>But there is a pressure difference. The baloon material is responsible for

>this state of affairs. Let's call the balloon material Mr. Dielectric. Now,

>as you push more and more air into the left chamber, the balloon material will

>distort more and more to the right. Obviously the SIZE of the chambers

>influences how much of this distortion you can get away with. The distortion

>is caused by the applied pressure differential. If the air was not free to

>come out of the right chamber, then the balloon material would not be free

>to move into the right chamber. OK, now imagine a sturdier, heftier balloon

>material. It takes more pressure to get it to move, but when it does move it

>pushes back with a vengeance. [Different dielectric materials influence

>the total capacitance in much the same way]. Increase the pressure and the

>balloon distortion increases. Up to the point where the poor balloon ruptures.

>[This is similar to the voltage rating on a cap.] The air you pumped IN

>wants to go OUT of the same nozzle it entered. And it WILL if you let it.

>If you cap it off, however, the air inside is trapped. If you let the air

>go OUT the left side, air must also ENTER the right side. The air activity

>at the right nozzle is a mirror image of the air activity at the left nozzle.

>You can only pump so much air into the left chamber before the balloon material

>will burst. [Capacitor destruction. Permanent in this case.] As you put

>more air IN, it becomes harder to get additional air in. When the balloon material

>is distended, it wants to go back to its rest position. If you let the air

>freely flow, it can return to this position quite rapidly. Limited by the

>opening size [equivalent to internal resistance.] If you apply a pressure at

>the left chamber air will be forced out of the right chamber. If you move air

>in and out of the left chamber, the same quantity of air will move through

>the right chamber, but opposite in direction (in/out). However, if you are

>looking at the direction not as in/out but as from left to right and from

>right to left, you would say that when air is moving left to right in one

>chamber, it is moving left to right in the other chamber also. Just like a

>real capacitor, AC in gives you AC out. Or to put it another way, AC currents

>readily flow "through" the assembly, even though there is a physical

>barrier (dielectric) that prevents "actual" current flow.

>

>All analogies limp, but this one happens to handle a lot of the capacitor

>characteristics quite well.

>

>

>> If voltage causes charge separation, surely there must be some limiting

>> voltage at which all the charges have separated, assuming we have an

>> indestructible capacitor? So if we increase the voltage now on the

>> capacitor, does that mean it will no longer store energy?

>>

>

>Greater voltages tend to make the charges separate further, because there

>is more pressure. However, for a given dielectric and a given thickness

>of the same, there is a breakdown point where the material is physically

>damaged, so to speak. This corresponds to the volts per mil rating.

>And at high frequencies the dielectric is also going to begin to show

>heating effects. This relates to power factor.

>

>> I remember a post where someone said that energy is stored in the dielectric

>> and not on the plates, and conductors are incapable of confining charge, but

>> can only transport it. And that they could carefully disassemble a charged

>> capacitor, mail the dielectric to someone else who then assembles a

>> capacitor using that same dielectric and their own plates, and amazingly

>> enough they have a charged capacitor. Or, if you short out a capacitor, let

>> it sit for awhile, it will still develop some potential across its terminals

>> due to "dielectric memory".

>>

>

>All true. Unfortunately the double cylinder/balloon model fails to show

>those effects. That is because in a "real" dielectric the atoms themselves

>form both the cylinders AND the balloon part. The atoms can push and

>pull against one another in a way that the model cannot show. The capacitor

>plates are just the connections to the dielectric. All the real action

>takes place in the dielectric. The plates serve to distribute the charge and

>to conduct the current (which is just a moving charge, after all). Because

>the dielectric is an INSULATOR, the conducting plates are needed to have the

>thing ACT like a "capacitor", but you can remove the plates and still

>have the stored charge. A cool experiment, and easy to perform.

>

>> So if this is true, does this mean that the electrons which are charged

>> particles somehow leave the plate and go into the dielectric? If not, then

>> the plates must still have a charge imbalance. So how is energy stored then

>> if the dielectric is air or a vacuum?

>

>A vacuum is nothing. Once you introduce the electrons, it ain't nothin'

>no more :)  A vacuum doesn't store electrons. Put an electron into a vacuum

>and where is it supposed to go? Vacuums have a very high breakdown voltage.

>Which is another way of saying that it takes a lot of pressure to get an

>electron to move from here to there in a vacuum. No CARRIERS available.

>Yeah, a lot of it looks paradoxical at first, but it all makes sense when

>you look at it right.

>

>The good news is you don't have to understand every single aspect of

>capacitors to be able to use them effectively. It's like driving a car.

>You don't have to know how a carburetor works to be able to use it. Just

>make sure you got one, and push in on the gas!

>

>

>> And how does the dielectric material

>> effect the capacitance value? Now, don't you see why capacitors confuse me?

>>

>> I am still amazed at this point that I do not have a good grasp on how

>> capacitors store energy and yet they are such a vital part of tesla coils.

>>

>> Dave

>>

>Sit back, relax and enjoy the arcs and sparks. Soak up the wonder of it all.

>Learn all you can, but realize that your knowledge will always be incomplete.

>And that's perfectly OK. Enjoy the journey, and don't worry about whether

>you are there yet. You're on the way, and that's all that matters!

>

>Hope this helps.

>Fr. Tom McGahee

Thanks Fr. McGahee for the insights. Those were very good analogies and

helped, but I still don't fully understand what is physically happening

inside a capacitor. Maybe it's not all that really important to know as far

as building a Tesla coil, or maybe it's beyond the scope of this list, but I

always seem to feel a lil uneasy when I don't fully understand something.

I really thought I had a good grasp on how they worked til I read that the

energy is stored in the dielectric and that conductors are incapable of

confining charge. I seem to get lost when thinking that charge separation

creates an E-field which them somehow stores energy in a dielectric. I also

don't fully comprehend how a capacitor "reactance" changes as a function of

the applied frequency. Well, think I'll read through my physics books now.

Hopefully I can gain some more insights soon. Thanks again for your help.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Measuring Transformers

Date: Tue, 25 Nov 1997 14:42:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 25 Nov 1997 09:42:15 -0500

Subject: Measuring Transformers

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: mw transformer and se

> Date: Monday, November 24, 1997 12:49 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 24 Nov 1997 11:49:01 -0600

> Subject: Re: mw transformer and se

>

> At 05:30 PM 11/23/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Sun, 23 Nov 1997 12:30:41 -0500

> >Subject: Re: mw transformer and sealer

> >

> >

> >

> >----------

> >> From: Tesla-2 <tesla-2@emachine.com>

> >> To: tom_mcgahee@sigmais.com

> >> Subject: mw transformer and sealer

> >> Date: Saturday, November 22, 1997 7:52 PM

> >>

> >> From: Chris Federico <cfed@comsource.net>

> >> Date: Sat, 22 Nov 1997 18:52:35 -0600

> >> Subject: mw transformer and sealers

> >>

> >>

> >> Anyone know an easy way to figure out the voltage output of a microwave

> >> transformer?  I was thiknin that I'd hook-up a 9v battery to the primary and

> >> check the voltage on the secondary then do the math to get the 120vac input

> >> = Xvac output.  Will this work?  (DC vs AC inputs really shouldn't make a

> >> difference, should it?)

> >>

>

>

> >Chris,

> >You *cannot* determine the voltage by applying a DC at the primary

> >(or secondary, for that matter). Transformers must operate with a

> >changing input signal. This is usually a sine wave, which is the

> >shape of AC from your wall socket.

> >

> >Here is the simplest way to measure the value of your microwave

> >transformer. Since most people do not have meters that can take the

> >approximately 2,500 volts AC that appears on the secondary of a

> >neon transformer, it is actually best to do things somewhat

> >BACKWARDS :)

> >

> >Apply 120 VAC to the SECONDARY and measure the voltage at the

> >primary. (The voltage applied may be slightly off from 120 VAC...

> >record the actual applied voltage wherever I say 120VAC)

> >

> >Put the larger voltage over the smaller voltage and multiply by

> >the wall socket voltage to get the HV output.

> >

> >For example, let's say you put 120 VAC into the secondary and

> >got 6 volts out.

> >

> >HV= (120*120)/6

> >so HV=2,400 volts

> >

> >One note of caution: The secondary has one side connected to the

> >transformer case in most microwave transformers. Thus while you

> >are performing this test you may possibly have 120VAC at the case.

> >So please make sure the transformer case is resting on an

> >insulating surface, and do not touch it while making the

> >measurements.

> >

> >This method of measuring can also be applied to neons, with the

> >same note of caution. The really good news is that this method

> >is very accurate and allows you to use regular low voltage

> >meters. On neons you can measure each output independantly,

> >by using the case center tap as one connection,or you can attach

> >the 120VAC to both HV leads. But be aware that the case center

> >tap in EITHER case may have a voltage on it with respect to

> >AC ground, so please be careful.

> >

>

> <snip>

>

> Fr. Tom and Chris,

>

> I have used the 120VAC line voltage applied to the secondary

> of neon transformers method for determining voltages with

> mixed results.

>

> It does not seem to give consistant results and often gives

> readings that cant possibly be right. I attribute this

> to insufficient excitation of the transformer core.

>

> Perhaps it would work better on a microwave oven transformer

> but doesnt seem to be a reliable method for neons.

>

> I have had much better repeatable results by simply putting a low

> voltage on the neon primary and measuring the resulting secondary

> voltage.

>

> For instance with a 12kv neon you might expect to see about 400 volts

> output with 4 volts input. A 12kv neon has a nominal 100:1 turns

> ratio.

>

> Ive used this method to tag all my neons and can pair

> up units closely matched in voltage.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike always loves it when he gets to correct me :)

What I said is correct, but I failed to mention some

basic facts of life. So here they are:

Transformers operate based on a volts per turn basis. If the

applied volts per turn goes down too low, then the core is

not adequately excited, as Mike said, and most of the input

is wasted in eddy currents and such. In other words, you

gotta have a certain minimum volts per turn or your errors

become huge in relation to your answer.

Anything less than 3% of normal operation is going to give

you large errors. That means that the voltage measured at

the primary will be fairly accurate as long as it is at least

4 volts. (Conversely, putting in anything less than 4 volts

to the primary will give you an erroneous output as well. The

error factors involved are applicable to both measuring methods)

The method works quite well with microwave transformers, because

they are normally 2,400 to 2,700 volts. 120 volts is about

5% of that, and so errors are tolerable as there is sufficient

core excitation.

You can help things along a *bit* by only applying voltage to

ONE HV bushing and the case ground. On a typical 12KV neon

which produces 6KV per side you would now have a 50 to 1

ratio to the primary. 120 V at the 6KV side would give you

about 2.4 volts at the primary. Error here will be high, but

this method will at least tell you if you have a 9 12 or 15KV

neon. It will also help in identifying damaged secondary

windings, if the damage is an open or short. It will not

usually identify carbon tracking problems, as these show up

only when the secondary has High Voltage actually present.

Another technique you can resort to  which is *very* accurate

is to use another neon! Let's say you have a known good

9KV neon. Connect ONE HV bushing from the 9KV to one of the

unknown transformer's HV bushings. Connect the cases together.

Apply 120 to the primary of the 9KV neon. Read the voltage at the

primary of the unknown transformer. 240 means the transformer

is 4.5KV. 120V means the transformer is also a 9KV unit. 90 v

would mean you had a 12KV unit. 72 volts would mean 15KV.

With this method, error will be too small to measure, because

you are using a good volts per turn ratio.

Note that it is best to use a neon that has a small KV rating

as the first neon so that the second neon's breakdown voltage

is never exceeded. This makes the relative voltage ratings of

the transformers reciprocally related to the voltages measured

at the respective primaries. But I believe in safety first.

It is good to know several different techniques. Each has its

place, depending upon what sort of equipment you have on hand.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Polycarb

Date: Tue, 25 Nov 1997 16:14:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Tue, 25 Nov 1997 10:14:32 -0600

Subject: Re: Sundry Polycarb

> How about 3/8" PolyCarb?  I just got a bunch of 8"x10" scrap pieces,

pretty rigid stuff.

Just a note of caution in case you haven't worked with polycarb much:  Be

careful when using power tools to work it.  The stuff is so resilient that

it tends to heat and deform (which can cause it to bind while being cut, and

go zzingin' across the room) because it  isn't as brittle as acrylic and

doesn't "chip-out" well.  The zzingin' thing happened to me and the force to

my fingers was so extreem they were numb for several minutes (which I could

live with since I had to check to make sure they were all there, it happened

so fast).  I was using a power miter, but the same thing could have happened

on a radial arm saw, or table saw.  I won't work it with anything except a

bandsaw (or hand tools) now.

Good Luck. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sundry Resistors

Date: Tue, 25 Nov 1997 16:14:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Tue, 25 Nov 1997 10:14:33 -0600

Subject: Re: Sundry Resistors

Any suggestions on a good source for high kv rated resistors?  (I looked at,

dare I say it, Radio Shack, and the best they have is 500v)

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: TC & Tuning basics

Date: Tue, 25 Nov 1997 16:14:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Tue, 25 Nov 1997 10:14:35 -0600

Subject: Re: TC & Tuning basics

>One thing you can do to allow for more primary turns include

>adding a larger toroid to the secondary.

Thanks for the re-post Mike.

Two questions:

1.  If my safety gap only fires when a breakout arcs to the primary,

is my safety gap too wide?

2.  What do you, and the other list members, think of the sewer pipe, alum tape

wrapped toroid design?

Thanks . . CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Sealing Plasma Globes

Date: Tue, 25 Nov 1997 16:30:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Tue, 25 Nov 1997 10:30:28 -0600

Subject: Re: Sealing Plasma Globes

> From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

> Date: Tue, 25 Nov 1997 01:01:17 -0000

> Subject: Sealing Plasma Globes

>

> Hi All!,

>         After much deliberation I've gone the whole hog and actually got a glass

> globe blown for me including the center electrode. Now the only problem is

> - HOW THE HECK DO I SEAL THE DAMN THING??? AAARRGGGH!! - .Please excuse the

> outburst but I've tried every conceivable adhesive to seal my new globe

> including glass reinforced plastic and epoxy but nothing will contain the

> plasma pressure longer than several minutes. I can't afford to kit up on

> glass blowing gear to do the job properly so if anyone has any sensible

> ideas I'd love to know about 'em!

>                 Best Regards,

>                 Stuart

Stuart:

Forget about trying to seal glass with any kind of glue, epoxy or any

other sticky goop. It will only bring frustration and heartache. Though

these compounds seem as if they would work, they do not form a good

enough seal to be a long term solution. It is even difficult to get a

good seal when wires are fused to the glass. Platinum is used alot in

this application because its coefficient of expansion is similar to that

of glass. Remember, the tiniest leak can contaminate the gas in the

globe and foul the color and over time the pressure will increase until

the globe will no longer function properly. The good news is that you

don't need a lot of glassblowing gear to seal a globe. A small hand-held

torch will do the job just fine. I'm assuming you have some sort of

tubulation to use for pumping and filling the globe. Simply pump down

the globe as far as you can and fill with the desired gas. If you have a

large quantity of gas, say a lecture bottle or cylinder, you can back

fill a few times with the desired gas to make up for a lower quality

vacuum pump (most of us hobbyists cant afford a pump with < 1 micron

blank off). After the the last pump down, fill to say 0.5mm lower than

the desired pressure. Hold the torch close to the tubulation near the

globe and heat GENTLY. The vacuum will draw the glass together and

create a perfect seal. Just be careful not to overheat or the vacuum

will pull through and the game will be over. It's really not that hard,

just heat slowly and you wont have a problem.

Eric

<edavidson@icva.gov>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: TC & Tuning basics

Cc:

Bcc:

X-Attachments:

At 04:14 PM 11/25/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Tue, 25 Nov 1997 10:14:35 -0600

>Subject: Re: TC & Tuning basics

>

>>One thing you can do to allow for more primary turns include

>>adding a larger toroid to the secondary.

>

>Thanks for the re-post Mike.

>

>Two questions:

>

>1.  If my safety gap only fires when a breakout arcs to the primary,

>    is my safety gap too wide?

Yes it could be. Or your coil may be running very smoothly

with very little RF kickback. That does happen.

I would narrow your safety a bit until it begins to fire

occasionally. I always feel better when I know the safety is

working.

>

>2  What do you, and the other list members, think of the sewer pipe, alum tape

> wrapped toroid design?

>

Its just fine. As long as you can get the aluminum tape on smooth

it will work great. I just prefer the flexible aluminum duct

since you dont have to cover it with foil or tape except at the

joint.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Sundry Resistors

Cc:

Bcc:

X-Attachments:

At 04:14 PM 11/25/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Tue, 25 Nov 1997 10:14:33 -0600

>Subject: Re: Sundry Resistors

>

>Any suggestions on a good source for high kv rated resistors?  (I looked at,

>dare I say it, Radio Shack, and the best they have is 500v)

>

>Thanks

>

One good source for all kinds of useful surplus electronics

is MECI. They have a web site. The URL is:

http://www.meci.com/

Definitely get a copy of their catalog.

I usually buy things like that at local hamfest. At the last

hamfest I bought 20  310 ohm 50 watt resistors for 10 cents each.

Such a deal!

If you want to check on possible hamfests in your area check out

the home page of the ARRL.

http://www.arrl.org

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: More Capacitor Analogies

Date: Tue, 25 Nov 1997 19:26:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 25 Nov 1997 14:26:16 -0500

Subject: More Capacitor Analogies

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Capacitor Analogies

> Date: Tuesday, November 25, 1997 8:48 AM

>

> From: Dave <djb@velocity.net>

> Date: Tue, 25 Nov 1997 08:48:05 -0500 (EST)

> Subject: Re: Capacitor Analogies

>

> At 10:53 PM 11/24/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Mon, 24 Nov 1997 17:53:13 -0500

> >Subject: Capacitor Analogies

>

> >> Capacitors confuse me!

> >

> >They confuse a lot of people.

> >

SUPER-SNIP

> >>

> >Sit back, relax and enjoy the arcs and sparks. Soak up the wonder of it all.

> >Learn all you can, but realize that your knowledge will always be incomplete.

> >And that's perfectly OK. Enjoy the journey, and don't worry about whether

> >you are there yet. You're on the way, and that's all that matters!

> >

> >Hope this helps.

> >Fr. Tom McGahee

>

>

> Thanks Fr. McGahee for the insights. Those were very good analogies and

> helped, but I still don't fully understand what is physically happening

> inside a capacitor. Maybe it's not all that really important to know as far

> as building a Tesla coil, or maybe it's beyond the scope of this list, but I

> always seem to feel a lil uneasy when I don't fully understand something.

>

> I really thought I had a good grasp on how they worked til I read that the

> energy is stored in the dielectric and that conductors are incapable of

> confining charge. I seem to get lost when thinking that charge separation

> creates an E-field which them somehow stores energy in a dielectric.

In the analogy presented, this is equivalent to the stretching of the

balloon material. The e-fields are lines of force, are they not? In the

balloon material this would correspond to the molecules holding on to

one another under forced tension. The direction of stress sets up a

distortion in the material. The result is that the material wants to

move back to its original position. The path it will follow in doing

this corresponds to the lines of force.

Pulling out of the analogy and dropping into full reality mode, do you

see how the e-fields are related to the charge tensions within the

dielectric? They are not things in and of themselves, but the direct

result of these tensions. The forces causing the tensions are directly

related to electrical charges. The direction along which these forces

align themselves gives the e-fields their shapes.

Between two parallel metal plates these forces express themselves in a

direction perpendicular to the plates. At the EDGES of the plates they

cease being parallel to one another and begin to curve. The shape of the

e-field here indicates simply that a charged particle would travel in a

curve at these points.

> I also

> don't fully comprehend how a capacitor "reactance" changes as a function of

> the applied frequency.

Analogy time again. Moving charges around in the dielectric is somewhat

analogous to inertia. So let's do a simple inertia experiment:

Seat yourself in a chair and raise your feet up off the floor by a couple

of inches. Keep your feet raised for the duration of the experiment. This

is so that you won't be coupling your body to the floor and destroying

some of the effects we are trying to produce.

OK, now stick both arms out from your sides and extend them OUTWARD as

far as you can. One one each side, of course. Ball up your fists like

you were going to hit somebody. SLOWLY move both arms up and down  for a

total distance of about one foot. Now keep moving both arms up and down

for a distance of about one foot, but now INCREASE the speed at which

you are doing this. Keep increasing until you can't go any faster and still

move them the *full* distance.

OK, now what you should have observed is that as you increased the

frequency (hint, hint) of your wild arm flapping, your body began to

react to all this by bouncing up and down violently. You were doing

work. And a fair amount of it, I might add. See how frequency enters in?

Moving charges around is like moving anything else around. The faster

you try to do it, the harder it is to do it. When you flapped your arms, the

forces involved gravity and basic inertia. In the dielectric you are

working against the tendency of the atoms to stay lined up pretty much

the way they were. Sort of like working against a whole bunch of things

interconnected by lots of rubber bands in all directions. Now grab one

of these things and start shaking it back and forth. As in AC. As one

piece moves, the others also move (but less). Release it and it will

attempt to return to its original state. In the process it will go into

a slight oscillation. If you have really distorted the local rubber

bands then it may not return all the way immediately. Over time it will

drift. Just like in a capacitor dielectric.

Remember that analogies are designed to help us understand one or more

*aspects* of something else. As such they are always incomplete and even

erroneous on some details. But they also help us to *visualize* and

comprehend things that would otherwise just boggle the mind.

> Well, think I'll read through my physics books now.

> Hopefully I can gain some more insights soon. Thanks again for your help.

>

> Dave

>

Hope this helps a bit more.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Surplus stuff

Date: Tue, 25 Nov 1997 19:44:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Tue, 25 Nov 1997 11:44:35 -0800 (PST)

Subject: Re: Surplus stuff

On 25 Nov, Mike Hammer wrote:

>

> I usually buy things like that at local hamfest. At the last

> hamfest I bought 20  310 ohm 50 watts resistors for 10 cents each.

> Such a deal!

Related kinda: I have a shop nearby that is like a permanant

hamfest, [Nearby, as in up the street !]

I noticed that they had a 3 phase 0-220V Variac, I think each

phase was rated at 8.9A. I can get more details if anyone needs

such a beast.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: More Capacitor Analogies

Date: Tue, 25 Nov 1997 21:20:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 25 Nov 1997 16:20:39 -0500

Subject: More Capacitor Analogies

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Capacitor Analogies

> Date: Tuesday, November 25, 1997 8:48 AM

>

> From: Dave <djb@velocity.net>

> Date: Tue, 25 Nov 1997 08:48:05 -0500 (EST)

> Subject: Re: Capacitor Analogies

>

> At 10:53 PM 11/24/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Mon, 24 Nov 1997 17:53:13 -0500

> >Subject: Capacitor Analogies

>

> >> Capacitors confuse me!

> >

> >They confuse a lot of people.

> >

SUPER-SNIP

> >>

> >Sit back, relax and enjoy the arcs and sparks. Soak up the wonder of it all.

> >Learn all you can, but realize that your knowledge will always be incomplete.

> >And that's perfectly OK. Enjoy the journey, and don't worry about whether

> >you are there yet. You're on the way, and that's all that matters!

> >

> >Hope this helps.

> >Fr. Tom McGahee

>

>

> Thanks Fr. McGahee for the insights. Those were very good analogies and

> helped, but I still don't fully understand what is physically happening

> inside a capacitor. Maybe it's not all that really important to know as far

> as building a Tesla coil, or maybe it's beyond the scope of this list, but I

> always seem to feel a lil uneasy when I don't fully understand something.

>

> I really thought I had a good grasp on how they worked til I read that the

> energy is stored in the dielectric and that conductors are incapable of

> confining charge. I seem to get lost when thinking that charge separation

> creates an E-field which them somehow stores energy in a dielectric.

In the analogy presented, this is equivalent to the stretching of the

balloon material. The e-fields are lines of force, are they not? In the

balloon material this would correspond to the molecules holding on to

one another under forced tension. The direction of stress sets up a

distortion in the material. The result is that the material wants to

move back to its original position. The path it will follow in doing

this corresponds to the lines of force.

Pulling out of the analogy and dropping into full reality mode, do you

see how the e-fields are related to the charge tensions within the

dielectric? They are not things in and of themselves, but the direct

result of these tensions. The forces causing the tensions are directly

related to electrical charges. The direction along which these forces

align themselves gives the e-fields their shapes.

Between two parallel metal plates these forces express themselves in a

direction perpendicular to the plates. At the EDGES of the plates they

cease being parallel to one another and begin to curve. The shape of the

e-field here indicates simply that a charged particle would travel in a

curve at these points.

> I also don't fully comprehend how a capacitor "reactance" changes as a function

> of the applied frequency.

Analogy time again. Moving charges around in the dielectric is somewhat

analogous to inertia. So let's do a simple inertia experiment:

Seat yourself in a chair and raise your feet up off the floor by a couple

of inches. Keep your feet raised for the duration of the experiment. This

is so that you won't be coupling your body to the floor and destroying

some of the effects we are trying to produce.

OK, now stick both arms out from your sides and extend them OUTWARD as

far as you can. One one each side, of course. Ball up your fists like

you were going to hit somebody. SLOWLY move both arms up and down  for a

total distance of about one foot. Now keep moving both arms up and down

for a distance of about one foot, but now INCREASE the speed at which

you are doing this. Keep increasing until you can't go any faster and still

move them the *full* distance.

OK, now what you should have observed is that as you increased the

frequency (hint, hint) of your wild arm flapping, your body began to

react to all this by bouncing up and down violently. You were doing

work. And a fair amount of it, I might add. See how frequency enters in?

Moving charges around is like moving anything else around. The faster

you try to do it, the harder it is to do it. When you flapped your arms, the

forces involved gravity and basic inertia. In the dielectric you are

working against the tendency of the atoms to stay lined up pretty much

the way they were. Sort of like working against a whole bunch of things

interconnected by lots of rubber bands in all directions. Now grab one

of these things and start shaking it back and forth. As in AC. As one

piece moves, the others also move (but less). Release it and it will

attempt to return to its original state. In the process it will go into

a slight oscillation. If you have really distorted the local rubber

bands then it may not return all the way immediately. Over time it will

drift. Just like in a capacitor dielectric.

Remember that analogies are designed to help us understand one or more

*aspects* of something else. As such they are always incomplete and even

erroneous on some details. But they also help us to *visualize* and

comprehend things that would otherwise just boggle the mind.

> Well, think I'll read through my physics books now.

> Hopefully I can gain some more insights soon. Thanks again for your help.

>

> Dave

>

Hope this helps a bit more.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Mechanical Cameras

Date: Wed, 26 Nov 1997 03:07:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>I'm still gathering information... and searching for a mechanical camera!

T>:-)

T>KevinZ

Kevin,

If you want to locate a mechanical camera, try stopping at a few pawn

shops.  They usually have scads of them.  Another possibility is most

any second hand store - they usually have lots of the 126 and 35mm

variety on the shelves for a couple of bucks. Another but probably more

expensive option is one of the older camera shops. They usually have

some trade in's available.

Ralph

---

If you believe in telekinesis, raise my hand.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: How does one make high Q

Date: Wed, 26 Nov 1997 10:29:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: pacster@ibm.net

Date: Wed, 26 Nov 1997 10:29:37 GMT

Subject: How does one make high Q caps?

Hello !

Finally!, my exams are over for the year, so now I can get coiling again (I

definitely prefer coiling to exams). I was reading some posts on capacitors

and it was said that for a 15KV cap. the polyethelene should be around

90mils thick. Coming from a 'S.I. units' country I'm a little confused, is

that thickness 24.5/1000 x 90 = 2.28 mm? If so then its no wonder that my

caps cook after only one minute of coil operating, I was using 0.45 mm of

poly between plates, oops!

What is the recommended thickness for 7.5, 9, 10, 12 and 15 KV poly capacitors?

I am hoping to build high Q caps from now on and after a fair bit of reading

apparently these can improve the capacitor Q:

1. Clean all sides of the poly with meth. spirits.

2. Round all foil edges.

3. Keep the plates as tight as possible .

4. Add lay sheets of foil between the layers of poly.

5. Add sheets of oil soaked Craft paper next to each live foil plate.

6. Provide the biggest terminal ends for each plate.

7. Put the caps in series.

8. Submerge the cap in light mineral oil.

9. Use equal numbers of + and - plates.

10. When using caps in series they should be the same individual part

capacitances.

11. Vacuum pressurising the caps.

12. Keep all dust and dirt out of the cap.

What is the most useful in improving the Q and what has little affect on the

Q, is there anything else that improves the Q? Normally I roll my caps, but

I think that if I should be using a thickness 2.3mm of poly this could be a

bit of a task. But perhaps I could make ten 10nF caps and parallel them up

to 100nF and then series four sets of them down to 25nF?

I would also like to take a look at my coil on the scope when it is running.

To do this can I just connect a piece of wire to the 10x probe and dangle

the wire in the air about 10 to 15 feet away from my coil (do I need to

connect the scope earth to anything ?), and then fire up my coil, with

hopefully my new high Q caps (so that I can watch it for more than a minute

at a time). Is this O.K. or is there a safer and better method?

Thanks all for your time everyone!

Mark

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: How does one make hig

Date: Wed, 26 Nov 1997 15:40:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 26 Nov 1997 07:40:38 -0800

Subject: Re: How does one make high Q

Organization: Stoneridge Engineering

Hi Mark!

Welcome to coiling! Your questions and some answers are below.

Tesla-2 wrote:

>

> From: pacster@ibm.net

> Date: Wed, 26 Nov 1997 10:29:37 GMT

> Subject: How does one make high Q caps?

>

> Hello !

>

<SNIP>

> What is the recommended thickness for 7.5, 9, 10, 12 and 15 KV poly capacitor

s?

>

I'm the conservative sort, so I build my caps with significant margin.

For "bulletproof" Tesla Coil operation, I'd recommend the following

thicknesses. If you build the caps with multiple sheets of thinner

dielectric material, you could go thinner, but I wouldn't recommend less

than 75% of the recommended thickness. BTW, if you're "sizing" your caps

to your transformers, use the "bulletproof" thickness to insure they're

robust enough to handle to added stress coming from power-mains (50 or

60 Hz) resonant rise.

Bulletproof         Bulletproof

RMS transformer   ~Total Dielectric   ~Total Dielectric    75%

Source Voltage:    Thickness (mils)   Thickness (mm)    Thickness

7.5 KV               45                 1.10           0.83

9.0 KV               54                 1.32           0.99

12.0 KV               75                 1.84           1.38

15.0 KV               90                 2.21           1.65

20.0 KV              120                 2.94           2.21

> I am hoping to build high Q caps from now on and after a fair bit of reading

> apparently these can improve the capacitor Q:

>

<list SNIPP'ed>

> What is the most useful in improving the Q and what has little affect on the

> Q, is there anything else that improves the Q?

High Q requires using an efficient low leakage, low loss dielectric

(like LDPE), low inductance and low resistance interconnections between

plates, and removing all entrapped air (another source of losses as well

as being a cap killer). Suitably-constructed flat plate OR rolled caps

fit this description very nicely!

Normally I roll my caps, but

> I think that if I should be using a thickness 2.3mm of poly this could be a

> bit of a task. But perhaps I could make ten 10nF caps and parallel them up

> to 100nF and then series four sets of them down to 25nF?

As long as you don't introduce excessive interconnection resistance and

inductance, this should work.

>

> I would also like to take a look at my coil on the scope when it is running.

> To do this can I just connect a piece of wire to the 10x probe and dangle

> the wire in the air about 10 to 15 feet away from my coil (do I need to

> connect the scope earth to anything ?), and then fire up my coil, with

> hopefully my new high Q caps (so that I can watch it for more than a minute

> at a time). Is this O.K. or is there a safer and better method?

This works very well! You may want to plug the scope into a different

branch circuit to minimize conducted RFI pickup.

>

> Thanks all for your time everyone!

>

> Mark

>

<SNIP>

Safe studyin; and coilin' to you, Mark!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitor Quality Factors

Date: Wed, 26 Nov 1997 19:23:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 26 Nov 1997 14:23:34 -0500

Subject: Capacitor Quality Factors

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: How does one make high Q

> Date: Wednesday, November 26, 1997 5:29 AM

>

> From: pacster@ibm.net

> Date: Wed, 26 Nov 1997 10:29:37 GMT

> Subject: How does one make high Q caps?

>

> Hello !

>

> Finally!, my exams are over for the year, so now I can get coiling again (I

> definitely prefer coiling to exams). I was reading some posts on capacitors

> and it was said that for a 15KV cap. the polyethelene should be around

> 90mils thick. Coming from a 'S.I. units' country I'm a little confused, is

> that thickness 24.5/1000 x 90 = 2.28 mm? If so then its no wonder that my

> caps cook after only one minute of coil operating, I was using 0.45 mm of

> poly between plates, oops!

>

> What is the recommended thickness for 7.5, 9, 10, 12 and 15 KV poly capacitors?

>

> I am hoping to build high Q caps from now on and after a fair bit of reading

> apparently these can improve the capacitor Q:

>

> 1. Clean all sides of the poly with meth. spirits.

Keeps the crud out.

> 2. Round all foil edges.

This cuts down on corona.

> 3. Keep the plates as tight as possible .

Increases capacitance and decreases chance of bubbles.

> 4. Add lay sheets of foil between the layers of poly.

This enhances charge distribution across dielectric layers.

> 5. Add sheets of oil soaked Craft paper next to each live foil plate.

This cuts down on air bubbles.

I add the kraft paper between all surfaces. The idea is to reduce

air bubbles, and these can form anywhere. It may be overkill,

but I like the idea of oil being wicked up between ALL surfaces.

> 6. Provide the biggest terminal ends for each plate.

Maximizes current transfer to the outside world.

By the way, I personally do not like the use of "tabs" on the

capacitor plates. I prefer to use the entire edge of the plate

as my connection. This requires that the plates extend out

opposite ends and uses a bit more aluminum foil (whoopie-doo!),

but it maximizes current path and makes the cap plates easier

to build. (Making tabs is a pain in the you-know-what).

> 7. Put the caps in series.

Above 10KV you really can't make a single cap that will stand up

to the corona. Over time the dielectric fails, especially at the

ends. Making many caps and putting them in series makes it

so each cap has a low enough voltage that corona is not a major problem.

> 8. Submerge the cap in light mineral oil.

The light mineral oil penetrates more readily between layers than the

heavy mineral oils. The oil helps in the exclusion of air, which

in the presence of HV RF results in spot heating and nasty corona.

> 9. Use equal numbers of + and - plates.

Otherwise one set of plates is being wasted :(

> 10. When using caps in series they should be the same individual part

> capacitances.

Very important! The capacitors in series will share voltage in

inverse proportion to their relative capacitance. In simpler terms,

the smallest capacitor will get the biggest voltage. Zap!

> 11. Vacuum pressurising the caps.

Not necessary with most flat cap designs. In fact, a dangerous thing

to attempt if your case has flat sides! The rolled cap designs in

cylindrical PVC pipe take the vacuum well, however.

> 12. Keep all dust and dirt out of the cap.

Goes right back to #1. Don't let crud get inside EVER! It becomes

a site for corona generation and mucks up the oil.

>

> What is the most useful in improving the Q and what has little affect on the

> Q, is there anything else that improves the Q?

OK, my own personal thoughts on factors affecting the Quality of the capacitor.

In groups, with like items together.

Relative scale of 1-5, where higher is better:

First the things that directly affect capacitance :)

(5) Size of plates: larger is better

(5) Number of plates: more is better

(5) Distance between plates: shorter distance is better

(5) Dielectric material: higher dielectric constant is better

(5) Dissipation factor of dielectric: lower is better

(5) Breakdown voltage of dielectric: higher is better

Things that make it work better and live longer

(5) Use multiple thin dielectric sheets between plates

(2) Use inactive floating plates between multiple dielectric sheets

(3) Low inductance: flat caps are better than rolled caps here

(4) Low resistance internal connections: larger surface connections are better

(3) Low resistance external connectors: larger is better

(5) Exclusion of air by using oil (reduction of corona)

(5) Exclusion of air by using vacuum or compression

(4) Exclusion of air by allowing it to escape (vertical flat plates best)

(3) Exclusion of air by using Kraft paper around active plates

(2) Exclusion of air by using Kraft paper around inactive floating plates

(5) Reduction of corona by using multiple series-connected capacitors

(5) Reduction of corona by allowing adequate extra poly around end overlap

(5) Reduction of corona by allowing adequate extra poly around top/bottom

(2) Reduction of corona by using rounded edges on plates

(3) Reduction of corona by using adequate container insulation

(3) Reduction of corona by excluding small foreign matter

(2) Reduction of corona by excluding large foreign matter

(1) Obsessive-compulsive cleanliness of all surfaces and materials

Other factors

(3) Size (smaller is better)

(3) Weight (lighter is better)

(2) Portability (the more portable the better. No messy oil spills)

(1) Aesthetics (the way *You* think it looks)

(0) Aesthetics (the way *Other* coilers think it looks)

(-5) Aesthetics (the way *NON* coilers think it looks)

> Normally I roll my caps, but

> I think that if I should be using a thickness 2.3mm of poly this could be a

> bit of a task. But perhaps I could make ten 10nF caps and parallel them up

> to 100nF and then series four sets of them down to 25nF?

>

> I would also like to take a look at my coil on the scope when it is running.

> To do this can I just connect a piece of wire to the 10x probe and dangle

> the wire in the air about 10 to 15 feet away from my coil

Yes.

> (do I need to

> connect the scope earth to anything ?),

Yes. Earth. Mr. Ground. The Ultimate Reference point.

> and then fire up my coil, with

> hopefully my new high Q caps (so that I can watch it for more than a minute

> at a time). Is this O.K. or is there a safer and better method?

>

Safest method: Don't build a coil. Watch TV instead.

Better method: Fire it up and enjoy the sparks. Tweak for maximum Fun Factor.

> Thanks all for your time everyone!

>

> Mark

>

Mark,

I hope the above comments are helpful.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Mechanical Cameras

Cc:

Bcc:

X-Attachments:

At 03:07 AM 11/26/97 GMT, you wrote:

>

>T>I'm still gathering information... and searching for a mechanical camera!

>T>:-)

>

>T>KevinZ

>

>Kevin,

>

>   If you want to locate a mechanical camera, try stopping at a few pawn

>shops.  They usually have scads of them.  Another possibility is most

>any second hand store - they usually have lots of the 126 and 35mm

>variety on the shelves for a couple of bucks. Another but probably more

>expensive option is one of the older camera shops. They usually have

>some trade in's available.

>

>   Ralph

>

In my totally biased and completely unfair opinion the greatest

camera ever produced (affordable that is) is the Olympus OM-1.

This is a completely mechanical camera with a internal light meter.

Mine is well over 20 years old and works great despite spending

nights outdoors covered with dew or frost taking astronomical

pictures and having taken at least one direct hit from a 40 inch

streamer. Try that with your fancy do everything microelectronic

wonder.

These cameras are available used at quite reasonable prices

and the Olympus Zuiko lenses are some of the best.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitors

Date: Thu, 27 Nov 1997 03:46:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Mike Wilson <nwilson@skyenet.net>

Date: Wed, 26 Nov 1997 22:46:33 -0500

Subject: capacitors

Hello All,

I just finished up my new cap. It has vertically stacked plates with 50

10" x 9" plates space at .090 mil (15 - .006 mil LDPE), a little over 4

1/2" thick including the 1/4" plexi to hold it together. I know I should

of made 2 or 3 little ones but what the heck you only live once and

building parts for my coil is what I like doing best. If I did the math

right it should be about .022uf but it is about .025uf with a meter. I

was wondering if it will go down after I break it in to get the air out?

To tell you the truth I thought it would be lower than .022uf because it

is home built.

I let it sit for a couple of hours and then hooked it up. I ran it at

30volts AC in on a 15k neon with .050 spark gap for a couple of minutes.

Right away I could tell that this cap is better than the saltwater caps

I have been using, the spark gap sounded a lot meaner. How long should I

wait before I open this puppy up? Its hard waiting...real hard! I guess

I am going to run it a couple more times tonight then let sit until

tomorrow. Maybe open the gap some more tomorrow.

Mike Wilson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: current limiting

Date: Thu, 27 Nov 1997 05:34:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 27 Nov 1997 00:34:04 -0500

Subject: current limiting

I have a lincoln arc welder that i was hoping to use as a current limiter.

but after some thought i have no idea how to go about it.

can someone please explain how to do this.

I was planning on dissasembling the welder for parts, but upon opening the

cover it does not appear to have many heavy duty componets for tesla service.

mostly a couple of circuit boards /capacitor and a transformer..

I had hoped to use the amperage dial as a current limiter/rehostat but it

is obviously way to small.

this is a small portable unit with a handle on top to carry.

any help appreciated.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 3 phase variac

Date: Thu, 27 Nov 1997 05:49:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 27 Nov 1997 00:49:45 -0500

Subject: 3 phase variac

to Michael Baumann

I actually need a 120v variac rated for 20-30A .

I considered using a 220+ model and wiring the 2+ legs in parallel

from the 120 leg.

I would be interested to know the price on the one you listed.

if it was a good enuf deal i would pay to ship it here.(Hanover IN).

if it is still available please send me some more info.

thanks Chris Watkins

cwatkins@seidata.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitors

Cc:

Bcc:

X-Attachments:

At 03:46 AM 11/27/97 GMT, you wrote:

>From: Mike Wilson <nwilson@skyenet.net>

>Date: Wed, 26 Nov 1997 22:46:33 -0500

>Subject: capacitors

>

>Hello All,

>  I just finished up my new cap. It has vertically stacked plates with 50

>10" x 9" plates space at .090 mil (15 - .006 mil LDPE), a little over 4

>1/2" thick including the 1/4" plexi to hold it together. I know I should

>of made 2 or 3 little ones but what the heck you only live once and

>building parts for my coil is what I like doing best.

I did this same experiment not too long ago. I built

a single .01 mfd cap with 60 mils of poly for use

with 12kv neons. It gave great performance but eventually

failed. This only served to confirm my conviction that series

caps are the only way to go.

My .0077 mfd plate cap consisting of 6 caps each with 8 mils

of poly in series has survived plenty of abuse and is over

two years old.

>If I did the math

>right it should be about .022uf but it is about .025uf with a meter. I

>was wondering if it will go down after I break it in to get the air out?

>To tell you the truth I thought it would be lower than .022uf because it

>is home built.

Actually the capacitance will likely go up some more due to

the fact that the oil has a higher dielectric constant than

the air its displacing. Its not unusual for caps to measure

differently than the calculations indicate. The great thing about

plate caps is you can easily add or subtract a couple plates

to get the value you want.

> I let it sit for a couple of hours and then  hook it up. I ran it at

>30volts AC in on a 15k neon with .050 spark gap for a couple of minutes.

>Right away I could tell that this cap is better than the saltwater caps

>I have been using, the spark gap sounded a lot meaner. How long should I

>wait before I open this puppy up? Its hard waiting...real hard! I guess

>I am going to run it a couple more times tonight then let sit until

>tomorrow. Maybe open the gap some more tomorrow.

>

>Mike Wilson

>

Take it easy on the new cap. Its still full of air bubbles

that will easily ionize with 15kv. This is one reason single

caps are not recommended. The only thing to do is let the air

bubble out. Run it on your coil with a very narrow gap

and low power setting. Even though you may not be getting

much output the pulsing action of the tank circuit will

vibrate the plates and shake out the air.

Keep up the low power break in for as long as you can.

I wouldnt want to apply full power to it for a few weeks.

Bring the power level up gradually.

Good luck with your new cap.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: current limiting

Date: Thu, 27 Nov 1997 10:00:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 27 Nov 1997 02:00:04 -0800

Subject: Re: current limiting

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Thu, 27 Nov 1997 00:34:04 -0500

> Subject: current limiting

>

> I have a lincoln arc welder that i was hoping to use as a current limiter.

> but after some thought i have no idea how to go about it.

> can someone please explain how to do this.

>

> I was planning on dissasembling the welder for parts, but upon opening the

> cover it does not appear to have many heavy duty componets for tesla service.

> mostly a couple of circuit boards /capacitor and a transformer..

> I had hoped to use the amperage dial as a current limiter/rehostat but it

> is  obviously way to small.

>

> this is a small portable unit with a handle on top to carry.

> any help appreciated.

>

Chris,

No need to take it apart! Simply take your existing welding cables and

short the far ends together. Connect the input side of the welder in

series with your Tesla Coil variac. That's it!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Gotta brag somewhere :)

Date: Fri, 28 Nov 1997 03:49:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Thu, 27 Nov 1997 19:49:15 -0800 (PST)

Subject: Gotta brag somewhere :)

With the advice of Fr Tom, and Mike Hammer, I have managed to coax

14" sparks from my little coil. This is linear distance from the

breakout to a grounded strike wire. Dunno what the curve actually was.

Specs:

Input power: 12KV Neon, 30ma for 360 W

Primary: 9 Turns 1/4" copper tubing. 1/4" space, tapped at 8.7 turns,

the first turn is 0.5" from the secondary. All tank circut

wiring is either 12G House Wiring, or 14G HV Wiring.

Secondary: 3.5"x10"x24g magnet wire, First turn is 1.2 inches above the

primary.

Cap: eight saltwater bottle caps, for a measured .0055uf

Sparc Gap: Linear version of an RQ type gap. 7 1" dia x2" long

hard copper tubes, gapped at 0.032 for a total gap

of 0.19" [ Does not get warm at all during runs, even

without a fan!]

Toroid: 12.5" Major Axis, with 3" minor, constructed from

Invereted pie tins, with 3" aluminum dryer tubing.

Power Protection: Safety Gap + 0.7 nf Caps to RF ground on each leg +

2"x6"x28g Aircore Choke (3.7 mh) + 1K 60W Resistor (that has

an additional 2.5 mh inductance)  [ One question: this

resistor belongs between the Xfrmr output and the choke, right?]

Right now,  I am building a set of plate caps to replace the bottle caps,

and also will be experimenting with a new rolled cap design - after I

test it, I'll let you all know how it goes.

[other question: Just how 'slippery' is Diala-AX? in other words,

how hard is it to seal something after punching a hole in it? I

picked up 5 gallons of the stuff from a wholesaler so I figure

I'll sink my caps in it.]

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Gotta brag somewhere

Date: Fri, 28 Nov 1997 14:00:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 28 Nov 1997 09:00:33 -0500

Subject: Re: Gotta brag somewhere :)

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Gotta brag somewhere :)

> Date: Thursday, November 27, 1997 10:49 PM

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Thu, 27 Nov 1997 19:49:15 -0800 (PST)

> Subject: Gotta brag somewhere :)

>

>

> With the advice of Fr Tom, and Mike Hammer, I have managed to coax

> 14" sparks from my little coil. This is linear distance from the

> breakout to a grounded strike wire. Dunno what the curve actually was.

>

> Specs:

> Input power: 12KV Neon, 30ma for 360 W

> Primary: 9 Turns 1/4" copper tubing. 1/4" space, tapped at 8.7 turns,

>          the first turn is 0.5" from the secondary. All tank circut

>          wiring is either 12G House Wiring, or 14G HV Wiring.

> Secondary: 3.5"x10"x24g magnet wire, First turn is 1.2 inches above the

>            primary.

> Cap: eight saltwater bottle caps, for a measured .0055uf

> Sparc Gap: Linear version of an RQ type gap. 7 1" dia x2" long

>            hard copper tubes, gapped at 0.032 for a total gap

>            of 0.19" [ Does not get warm at all during runs, even

>            without a fan!]

> Toroid: 12.5" Major Axis, with 3" minor, constructed from

>         Invereted pie tins, with 3" aluminum dryer tubing.

> Power Protection: Safety Gap + 0.7 nf Caps to RF ground on each leg +

>         2"x6"x28g Aircore Choke (3.7 mh) + 1K 60W Resistor (that has

>         an additional 2.5 mh inductance)  [ One question: this

>         resistor belongs between the Xfrmr output and the choke, right?]

>

>

>

> Right now,  I am building a set of plate caps to replace the bottle caps,

> and also will be experimenting with a new rolled cap design - after I

> test it, I'll let you all know how it goes.

>

> [other question: Just how 'slippery' is Diala-AX? in other words,

> how hard is it to seal something after punching a hole in it? I

> picked up 5 gallons of the stuff from a wholesaler so I figure

> I'll sink my caps in it.]

>

>

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

Michael,

Congratulations! Glad our input helped.

Your next step up will be to get a wider coil form. Look for something

like 4 1/2" or 6"  make the H/D ratio 4/1 or slightly less. Then

parallel another 12KV 30 ma neon. Increase cap size to at least .007 ufd

(depends on operating frequuency chosen). This should easily double

the spark size to 28". You may have to play with the toroid and

primary tap. Build a 14 turn primary, even though initial tap point

may be between 6 and 8 turns. That will allow you to experiment with

larger caps and toploads.

Welcome to the wonderful world of coiling!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: current limiting

Date: Fri, 28 Nov 1997 14:46:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 28 Nov 1997 09:46:37 -0500

Subject: Re: current limiting

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: current limiting

> Date: Thursday, November 27, 1997 12:34 AM

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Thu, 27 Nov 1997 00:34:04 -0500

> Subject: current limiting

>

> I have a lincoln arc welder that i was hoping to use as a current limiter.

> but after some thought i have no idea how to go about it.

> can someone please explain how to do this.

>

> I was planning on dissasembling the welder for parts, but upon opening the

> cover it does not appear to have many heavy duty componets for tesla service.

> mostly a couple of circuit boards /capacitor and a transformer..

> I had hoped to use the amperage dial as a current limiter/rehostat but it

> is  obviously way to small.

>

> this is a small portable unit with a handle on top to carry.

> any help appreciated.

>

>

Chris,

Short out the welding electrodes. Connect the INPUT wires in SERIES

with your Transformer's primary. TADAAA! Instant ballast. You use the

dial thing to adjust the amount of primary current desired.

It is really quite simple.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Toroid construction

Date: Fri, 28 Nov 1997 15:17:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Fri, 28 Nov 1997 09:17:11 -0600

Subject: Re: Toroid construction

> I just prefer the flexible aluminum duct

>since you dont have to cover it with foil or tape except at the

>joint.

Hmmm.  I almost bought a section of that.  But it looked as if there were

too many small radius corners for it to work well. (each flex section

interlock)  That's not a problem?  Do you still use foil covered styrofoam

for the center?  And does the center have to be mounted in the center

(horizontally) of the toroid?  (I was considering mounting it an inch or so

lower to raise the toroid above the last sec winding)

BTA. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Toroid construction

Cc:

Bcc:

X-Attachments:

At 03:17 PM 11/28/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Fri, 28 Nov 1997 09:17:11 -0600

>Subject: Re: Toroid construction

>

>> I just prefer the flexible aluminum duct

>>since you dont have to cover it with foil or tape except at the

>>joint.

>

>Hmmm.  I almost bought a section of that.  But it looked as if there were

>too many small radius corners for it to work well. (each flex section

>interlock)  That's not a problem?  Do you still use foil covered styrofoam

>for the center?  And does the center have to be mounted in the center

>(horizontally) of the toroid?  (I was considering mounting it an inch or so

>lower to raise the toroid above the last sec winding)

>

>BTA. . .CF

>

>

>

No the ridges in the duct cause no problems as they are

smooth and continuous. You just need to smooth some aluminum

tape over the joint where the ends meet .

I use a pair of back to back aluminum pie pans bolted together

inside the ring of duct material. As you draw the bolt down

and pull the pans tighter together they grip the duct and make

it a solid unit.

For a larger 24 X 6 toroid that was too large for pie pans

I cut a disk from thin plywood and covered it with foil using

contact cement. I then mounted this disk into the middle of the

ring using some RTV silicone adhesive.

I would want the bottom part of the toroid even with the

top secondary turn or slightly above. Changing the toroid

height will change the tuning slightly so be sure to retune

for best output. The best toroid position has to be determined

experimentally.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Small coil

Date: Fri, 28 Nov 1997 21:21:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Fri, 28 Nov 1997 22:21:31 +0100

Subject: Small coil

I'm making a small coil now and wonder if these specs sounds ok. Secondary:

2" PVC tube wound with 29AWG copper wire, length of winding: 8" capped at

top with 8mm plexiglass disc. Primary: 14 turns of 3/16" copper tubing.

saucer shape with 20 degrees alignment. transformer: one 20mA 15kV oil

burner transformer. Both the secondary and primary windings are easy to

change so any tips? longer/shorter secondary/primary? I've also done an

toroid of 3" dryer duct. can you recommend a suitible capacitance for this

one? about 0.01uF?

Thanks

Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small coil

Cc:

Bcc:

X-Attachments:

At 09:21 PM 11/28/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Fri, 28 Nov 1997 22:21:31 +0100

>Subject: Small coil

>

>I'm making a small coil now and wonder if these specs sounds ok.

>

> Secondary: 2" PVC tube wound with 29AWG copper wire,

> length of winding: 8" capped at top with 8mm plexiglass disc.

> Primary: 14 turns of 3/16" copper tubing. saucer shape with 20 degrees

> alignment.

> transformer: one 20mA 15kV oil burner transformer. Both the secondary and

> primary windings are easy to change so any tips? longer/shorter

> secondary/primary? I've also done an toroid of 3" dryer duct.

> can you recommend a suitable capacitance for this one? about 0.01uF?

>Thanks

>Christopher Eriksson

>

>

Chris,

The biggest cap your 15kv/20ma OBIT can charge comfortably is

.0035 mfd. The rest of your coil sounds just fine.

With a good efficient cap and a decent spark gap I would bet it

will perform just fine. Let us know how it turns out.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Plasma Globe

Date: Sat, 29 Nov 1997 02:18:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Stuart Marsh" <stuart@anker-auto.netkonect.co.uk>

Date: Sat, 29 Nov 1997 02:18:57 -0000

Subject: Plasma Globe

Hi All,

I've discovered at last why my plasma globe would only work if I placed

the side of the flyback transformer directly next to the globe. The

secondardary coil had burnt out and the arc was having to jump the internal

gap! I've now replaced the secondary with another flyback and everything

works brilliantly!!

I think I've overcooked things slightly since the glass globe (it's fully

annealed and designed for the job unlike the pickle jar I originally used!)

gets quite warm and I can draw 10mm sparks from the glass with my fingers.

Needless to say the RF heats up the skin pretty quickly!

Does anyone know how to lower the power of the classic 3055 transistor

circuit without affecting frequency of operation, or dropping Watts across

a resistor?

Regards,

Stuart

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Small coil

Date: Sat, 29 Nov 1997 03:37:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 28 Nov 1997 19:37:40 -0800

Subject: Re: Small coil

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

> Date: Fri, 28 Nov 1997 22:21:31 +0100

> Subject: Small coil

>

> I'm making a small coil now and wonder if these specs sounds ok. Secondary:

> 2" PVC tube wound with 29AWG copper wire, length of winding: 8" capped at

> top with 8mm plexiglass disc. Primary: 14 turns of 3/16" copper tubing.

> saucer shape with 20 degrees alignment. transformer: one 20mA 15kV oil

> burner transformer. Both the secondary and primary windings are easy to

> change so any tips? longer/shorter secondary/primary? I've also done an

> toroid of 3" dryer duct. can you recommend a suitible capacitance for this

> one? about 0.01uF?

> Thanks

> Christopher Eriksson

>

Christopher,

Sounds like a very nice small coil. Smaller diameter coils can use a bit

more length to get their inductance up, and operating frequency down a

bit. I'd suggest increasing the coilform length a bit - say to 12" for

an aspect ratio of about 6. Assuming your toroid is about 10-12" in

diameter by 3" thick, this will bring your operating frequency to around

500 - 550 kHz. Your 15 KV 20 MA transformer will let you drive over 4200

pF (assuming 50 Hz power) or 3500 pF if your mains power is 60 Hz. 0.01

will be too big for your 20 mil transformer to charge properly. Assuming

you go with about a 3000-4000 pF tank cap, you should be in about the

proper tuning range for the size primary somewhere between turn 10 and

12. (BTW, I assumed you had 1" clearance all around between the coil form

and the innermost primary turn).

Safe coilin' to you!

-- Bert H --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Small coil

Cc:

Bcc:

X-Attachments:

At 09:21 PM 11/28/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Fri, 28 Nov 1997 22:21:31 +0100

>Subject: Small coil

>

>I'm making a small coil now and wonder if these specs sounds ok.

>

> Secondary: 2" PVC tube wound with 29AWG copper wire,

> length of winding: 8" capped at top with 8mm plexiglass disc.

> Primary: 14 turns of 3/16" copper tubing. saucer shape with 20 degrees

> alignment.

> transformer: one 20mA 15kV oil burner transformer. Both the secondary and

> primary windings are easy to change so any tips? longer/shorter

> secondary/primary? I've also done an toroid of 3" dryer duct.

> can you recommend a suitable capacitance for this one? about 0.01uF?

>Thanks

>Christopher Eriksson

>

>

>Chris,

>The biggest cap your 15kv/20ma OBIT can charge comfortably is

>.0035 mfd. The rest of your coil sounds just fine.

>With a good efficient cap and a decent spark gap I would bet it

>will perform just fine. Let us know how it turns out.

>TTYL!

>Mike Hammer

>mhammer@misslink.net

oops!

Chris,

As Bert pointed out to you in his reply the PROPER size cap

for your 15kv/20ma OBIT on 50 HZ is .0042 mfd. I just didnt

think about the different line frequency you have in Sweden.

Sorry about that.

One other note. Just because this is a low power coil

dont neglect the RF chokes and safety gap. You want

to keep that OBIT alive. Give it all the protection

you can.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: current lags voltage

Date: Sun, 30 Nov 1997 03:41:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sat, 29 Nov 1997 22:41:04 -0500

Subject: current lags voltage

I have 4 microwave transformers in series

output 7750v .5A  4kva input 110v

what size power factor correction cap should i use on this setup?

Or should I use one at all.

I have no scope or meter to test such functions

thanks again;

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: specs & questions

Date: Sun, 30 Nov 1997 14:15:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Sun, 30 Nov 1997 09:15:40 -0500

Subject: specs & questions

I have finally gotten some time to do a few things

here is what I have so far

secondary 10.76 dia 800 turns 18ga wire 34.6 length

I have no toroid yet -intend on a 6"x36" for starters.

larger later as i intend to make this a transmitter in future.

primary 21 turns .368 dia tubing -inverted 30 deg spiral

inside turn at 7.1" radi@ tube center

outside 20.74" radi@ tube center

tube center to center dist. .75"as measured on 30 deg slope.

transformer's

4 microwave trannies in series

approx 70.5/1 step up ratio yeilding 7750v at 110v input

I estimate these at 500mA+ &4000VA  total,as they are fairly large as

microwaves go.

these trannies seem to be very reliable in my rigged up tests with 12 "red

wolf" beer b capacitors and some large gaps.

(even at 220v input 1 leg although I fried a  13a variac)

I have seperated the secondary's from the cores,and all are in mineral oil.

"each one resembles a shrunken pole pig"

(thanks to list member Jim Buck for the helpful info)

I should soon have the ability to raise the input voltage

somewhat,possibly 220v max (1 leg only) before my variac.

I have a 5kva 480v 1ph in - 220v out transformer

that I plan to run in reverse and use 1 leg. (any comments?)

now for the questions...

Am I correct in my assumption that more primary voltage will need less cap's

although a "tougher" cap will be required?

I anticipate needing .1uF to .15uF to acheive resonance and have room

for a larger terminal.

does this sound in the ballpark?

after considering capacitor construction I know why I should have started

with a smaller coil for my first attempt. Oh well too late now!

assuming I ever get this thing tuned correctly - with poly caps what

discharge length should I expect .

anyone tried using EDM grade graphite for a gap    I have a bunch

and it seems it should be the near perfect material espec the high

grades (copper impregnated).

working on scanning some pictures if I can get the company

scanner working.

thanks to everyone who put this list up!

I'm 31 now and have wanted to build a tesla coil since I was 14.

Until now I could not find enuf info to get started !

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: specs & questions

Date: Sun, 30 Nov 1997 17:32:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jimbuck1@aol.com

Date: Sun, 30 Nov 1997 12:32:23 -0500 (EST)

Subject: Re: specs & questions

In a message dated 97-11-30 10:07:55 EST, you write:

> Subj:  specs & questions

>  Date:        97-11-30 10:07:55 EST

>  From:        tesla-2@emachine.com (Tesla-2)

>  To:  jimbuck1@aol.com

>

>  From: "Chris Watkins" <cwatkins@seidata.com>

>  Date: Sun, 30 Nov 1997 09:15:40 -0500

>  Subject: specs & questions

>

>  I have finally gotten some time to do a few things

>  here is what I have so far

>

>  secondary 10.76dia 800 turns 18ga wire 34.6 length

I recently built a secondary almost exactly like that. It works almost to well.

With 3 microwave transformers running about 1500 watts I get 5-6' streamers.

With more power the streamers don't have enough room indoors.

>  I have no torrid yet -intend on a 6"x36" for starters.

>  larger later as i intend to make this a transmitter in future.

>

>  primary 21 turns .368 dia tubing -inverted 30 deg spiral

>  inside turn at 7.1" radi@ tube center

>  outside 20.74" radi@ tube center

>  tube center to center dist. .75"as measured on 30 deg slope.

I only have 9 turns on my primary but if you don't have enough capacitance in

your primary tank & you use a large toroid you may need 21.

>

>  transformer's

>  4 microwave trannies in series

>  approx 70.5/1 step up ratio yeilding 7750v at 110v input

>  I estimate these at 500mA+ &4000VA  total,as they are fairly large as

>  microwaves go.

>

>  these trannies seem to be very reliable in my rigged up tests with 12 "red

>  wolf" beer b capacitors and some large gaps.

>  (even at 220v input 1 leg although I fried a  13a variac)

>

>  I have seperated the secondary's from the cores,and all are in mineral oil.

>   "each one resembles a shrunken pole pig"

>  (thanks to list member Jim Buck for the helpful info)

>

>  I should soon have the ability to raise the input voltage

>  somewhat,possibly220v

>  max (1leg only) before my variac. I have a 5kva 480v 1ph in - 220v out

>  transformer

>  that I plan to run in reverse and use 1 leg. (any comments?)

>

>  now for the questions...

>

>  Am I correct in my assumption that more primary voltage will need less cap's

>  although a "tougher" cap will be required?

>

>  I anticipate needing .1uF to .15uF to acheive resonance and have room

>  for a larger terminal.

>  does this sound in the ballpark?

With my 10" coil running 6000 volts/250 ma. the performance of the coil peaks

with .11uf .  When I ran the coil at 6000 volts/500 ma I used  about .17 uf,

which is all the good caps I have.  Its probally not quite enough capacitance

but the coil does put out some nice 2" thick streamers that strike everything

in the room when it was configured that way.

>

>  after considering capacitor construction I know why I should have started

>  with a smaller coil for my first attempt. Oh well too late now!

I know what you mean.  I still need to build more caps to run higher power

levels.

>

>  assuming I ever get this thing tuned correctly - with poly caps what

>  discharge

>  length should I expect .

I'm not sure, if everythings built right at 4kva it should be really huge.

I recently put an animated gif of my 10" TC on my web page.  The new gif, the

middle animation, is with the coil using all rolled poly caps around .11 uf &

6000 volts/250 ma from the transformers.  Look at http:/members.aol.com/jimbuck1

Also if your interested I have some more pictures of the coil running 3kw;

the camera starts recording static at this power level & they are not real

great but they do show what you can expect at 3kw.  Let me know if you want

to see them & I can e-mail you a few.

>

>  anyone tried using EDM grade graphite for a gap    I have a bunch

>  and it seems it should be the near perfect material espec the high

>  grades (copper impregnated).

>

>  working on scanning some pictures if I can get the company

>  scanner working.

>

>  thanks to everyone who put this list up!

>  I'm 31 now and have wanted to build a tesla coil since I was 14.

>  Until now I could not find enuf info to get started !

>

>

>

Jim Buck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: specs & questions

Cc:

Bcc:

X-Attachments:

At 02:15 PM 11/30/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Sun, 30 Nov 1997 09:15:40 -0500

>Subject: specs & questions

>

>I have finally gotten some time to do a few things

>here is what I have so far

>

<SNIP>

>

>I should soon have the ability to raise the input voltage

>somewhat,possibly 220v max (1leg only) before my variac.

>I have a 5kva 480v 1ph in - 220v out transformer

>that I plan to run in reverse and use 1 leg. (any comments?)

For your 4 series connected transformers I would series connect

the primaries of each leg and the parallel connect the two legs.

The feed the whole thing off of 220VAC. Get a large 220V variac.

Be careful. What you have here is quite lethal. This amount

of power can REMOVE body parts.

>

>now for the questions...

>

>Am I correct in my assumption that more primary voltage will need less

>cap's although a "tougher" cap will be required?

Higher voltage transformers have a higher impedance so

match with smaller higher impedance caps.

>

>I anticipate needing .1uF to .15uF to acheive resonance and have room

>for a larger terminal. does this sound in the ballpark?

Yep. Lots of caps to build.

>

>after considering capacitor construction I know why I should have started

>with a smaller coil for my first attempt. Oh well too late now!

Yes its a very ambitious coil with lots of difficulties

to overcome. You would have been better off starting with

a basic neon powered coil for the experience.

Your large current capability and large caps will demand a good

gap system. Build it heavy duty to sink lots of heat and use

lots of forced air. A rotary gap with some heavy duty static

gaps in series can be used to good advantage here.

>

>assuming I ever get this thing tuned correctly - with poly caps what

>discharge length should I expect .

A lot depends on your gap but I would say 4 to 6 feet.

>

>anyone tried using EDM grade graphite for a gap    I have a bunch

>and it seems it should be the near perfect material espec the high

>grades (copper impregnated).

Do not use grahite or carbon for gap electrodes. Think about how

a carbon arc lamp works. One you strike an arc with carbon electrodes

it does not want to go out. Just the opposite of what you need.

>

>working on scanning some pictures if I can get the company

>scanner working.

>

Do not attempt to post them to the list. This list server doesnt

appreciate binaries very well.

You can send them through if they are UUENCODED and less than 32k.

If you want I can display your pics for a time on my web site.

Contact me direct if you want to do this.

TTYL!

Mike Hammer

mhammer@misslink.net

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Microwave Transformers

Date: Mon,  1 Dec 1997 00:40:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MATTHEW A PRICE <ch038map@mode.lanl.k12.nm.us>

Date: Sun, 30 Nov 1997 17:40:04 -0700 (MST)

Subject: Microwave Transformers

Hey all-

I'm new to this list, and to coils, so this may seem like a dumb

question. I recently obtained a microwave transformer, and was kind of

confused to see it had 5 wires connected to it. I was expecting 4 (2 in, 2

out). All I have worked with to this point are ignition coils. Can someone

please tell me how to hook this up? Also, would this be a good enough high

voltage source for a really small coil (1.5"*6")? Thanks for your help.

Matthew

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave Transformer

Date: Sun, 30 Nov 1997 21:28:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: <choad69@usa.net>

Date: Sun, 30 Nov 1997 21:28:00

Subject: Re: Microwave Transformers

Matt,

Ok lets start with the wires. There are really 6 (including the iron

core). The 2 wires comming off a smaller coil with heavy guage wire.

That's the primary. The iron core and a wire comming off the larger thin

wire coil is your secondary about 2Kv -4Kv 300mA-500maA. the other 2 wires

are the low voltage secondary. Don't use these. It's about 3 -5 volts

lots of current. Microwave transformers arn't really good for Tesla Coils

as they are to low voltage and way to much current. You can always put

one together with it and see for your self. You'll need large value cap

for this also.

C ya l8r

Shaun O'Brien

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: current lags voltage

Cc:

Bcc:

X-Attachments:

At 03:41 AM 11/30/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Sat, 29 Nov 1997 22:41:04 -0500

>Subject: current lags voltage

>

>I have 4 microwave transformers in series

>output 7750v .5A  4kva input 110v

>what size power factor correction cap should i use on this setup?

>Or should I use one at all?

Microwave oven transformers are notoriously inefficient.

More so then even neons. But power factor correction really

doesnt give you anything back. The only reason you would

want to bother is if the current draw is exceeding the capabilites

on your AC circuit and/or variac. If your variac can

handle the load then dont bother.

It would probably take quite a bit of capacitance to fully

correct your bank. But all you need is enough capacitance

to make things manageable. I would start with 100 mfd and

try various values looking for the lowest current draw.

Use ONLY AC rated oil filled caps. Since you are planning

on some 220 volt operation use ony 440 volts AC rated caps

or better.

I dug down (deep) into my magic bag of formulas and turned up

this formula for getting a ballpark estimate of the required

PFC cap.

Cpfc = (0.5*Vout*Iout*1e6)/(2*Pi*F*Vin^2)  in uF

where Vout = faceplate HV output voltage of the neons

Iout = combined faceplate current of all neons

Pi = 3.14159

F = Line Frequency (50 or 60 Hz)

Vin = Mains voltage (typically 120 or 240V)

>I have no scope or meter to test such functions

Get a meter. You NEED one.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: current lags voltage

Date: Mon,  1 Dec 1997 07:36:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>>I have no scope or meter to test such functions

T>Get a meter. You NEED one.

Having a meter and other test equipment really helps keep the magic

smoke inside the transformers. =8+)

Ralph

---

Knock hard here #### to alter display colors.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Small coil

Date: Mon,  1 Dec 1997 21:29:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Mon, 1 Dec 1997 22:29:34 +0100

Subject: Small coil

Small coil

I've just calculated a cap for my mini coil. This is actually the first

time I've calculated a cap, so sounds this ok for about 4200pF wich you

recommended me. 0.2mm Polyethene sheeting and 10 layers of baking type

aluminium foil. I plan to use 3 layers of poly between the aluminium

plates. The dimension of the plates is about 7.1" * 7.1" wich should give

me about 4200pF(?). And just one thing more, can I mold (I don't know the

right word for this) in my cap in epoxy? Or do i have to put it in oil?

The problem might be the air, but it gets more.."dry" that way.

Thanks for all your help so far.

//Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Shell Diala-EX for sale

Date: Mon,  1 Dec 1997 23:59:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 01 Dec 1997 17:59:22 -0600

Subject: Shell Diala-EX for sale

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Greetings!

I have come into the posession of about 20 gallons

of used Shell Diala-EX dielectric oil.  As a service

to members of this list, I will sell this oil basically

at the cost of shipping.

This oil was removed from medical X-ray equipment

and is clean and otherwise uncontaminated, etc.

I have already constructed four poly plate caps using

this oil with no problems!

(Mike Hammer has already received some, and gave me

permission to post this to the list!)

Anyone interested should contact me directly at:

arcstarter@mailexcite.com

Thanks!

If this appears doublespaced then I apologize.  This

is apparently a problem with mailexcite. :(

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: re: Small Xformer salvage

Date: Tue,  2 Dec 1997 00:24:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

Date: Mon, 1 Dec 1997 17:24:10 -0700 (MST)

Subject: re: Small Xformer salvaged, Want Coil.

Hello,

I have obtained a small transformer that was  for a home bar

neon sign.  I am not sure of the power/voltage output.  Could

someone help me construct a modest coil build around this xformer?

I am thinking of a demonstration unit for the holidays.  Mike

Hammer has been of wonderful assistance in the past, perhaps you

could drop me a line, Mike (and anyone else who would care to) at

jpbazemo@ouray.cudenver.edu.  Thank you very much.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Shell Diala-EX for sale

Cc:

Bcc:

X-Attachments:

At 11:59 PM 12/1/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Mon, 01 Dec 1997 17:59:22 -0600

>Subject: Shell Diala-EX for sale

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> Greetings!

> I have come into the posession of about 20 gallons

>of used Shell Diala-EX dielectric oil.  As a service

>to members of this list, I will sell this oil basically

>at the cost of shipping.

>This oil was removed from medical X-ray equipment

>and is clean and otherwise uncontaminated, etc.

>I have already constructed four poly plate caps using

>this oil with no problems!

>(Mike Hammer has already received some, and gave me

>permission to post this to the list!)

>

>Anyone interested should contact me directly at:

>arcstarter@mailexcite.com

>Thanks!

>If this appears doublespaced then I apologize.  This

>is apparently a problem with mailexcite. :(

>-Bill

>

>

>

Bill,

Well it didnt come through doubled spaced. Thats really

strange. Oh well. At least you can post to the list now.

As Bill mentioned I did receive some of the oil from

him and it is an excellent grade of real transformer oil.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Microwave Transformers

Cc:

Bcc:

X-Attachments:

At 12:40 AM 12/1/97 GMT, you wrote:

>From: MATTHEW A PRICE <ch038map@mode.lanl.k12.nm.us>

>Date: Sun, 30 Nov 1997 17:40:04 -0700 (MST)

>Subject: Microwave Transformers

>

>

>Hey all-

>  I'm new to this list, and to coils, so this may seem like a dumb

>question. I recently obtained a microwave transformer, and was kind of

>confused to see it had 5 wires connected to it. I was expecting 4 (2 in, 2

>out). All I have worked with to this point are ignition coils. Can someone

>please tell me how to hook this up? Also, would this be a good enough high

>voltage source for a really small coil (1.5"*6")? Thanks for your help.

>                Matthew

Matt,

The 5 wires on your transformer are most likely 2 for the primary,

2 for the magnetron filament, and a single lead for the HV output.

The other end of the HV winding is tied to the core.

The primary should be easy to identify being composed of heavy

wire. The HV output lead tied to the core is usually pretty

obvious. The filament winding isnt of much use and the leads

can be clipped off.

Unfortunately a microwave oven transformer isnt very useful

for a Tesla coil. The voltage is typically around 2200 volts

which is far two low for spark gap operation.

Some individuals have had success running 2 or 3 oven transformers

in series to raise the voltage but this is tricky to accomplish.

The best power source for a small coil would be a 7500 volt 30ma

neon sign transformer. I have one of these and it works quite well

on small coils. A somewhat more common 9kv/30ma neon would work

just as well.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla coil for sale?

Date: Tue,  2 Dec 1997 15:57:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 02 Dec 1997 09:57:40 -0600

Subject: Tesla coil for sale?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Some guy has a tesla coil for sale.  Check out:

http://crosley.simplenet.com/swapshop/tc.html

Does this belong to anyone on the list??

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla coil for sale?

Date: Tue,  2 Dec 1997 17:14:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Tue, 02 Dec 1997 12:14:42 -0500

Subject: Re: Tesla coil for sale?

Not a bad looking coil.  However, I think buying one is a poor choice.  The fun

is in building it and maintaining it.  You want to put your blood, sweat, and

money into your creation.  It's a whole experience which is denied by purchasin

g one.  I'll bet that it doesn't come with much of a warranty!

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 12/02 12:04 PM >>>

From: arcstarter@mailexcite.com

Date: Tue, 02 Dec 1997 09:57:40 -0600

Subject: Tesla coil for sale?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Some guy has a tesla coil for sale.  Check out:

http://crosley.simplenet.com/swapshop/tc.html

Does this belong to anyone on the list??

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Paraffin secondary coatin

Date: Tue,  2 Dec 1997 19:44:31 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 02 Dec 1997 13:44:31 -0600

Subject: Paraffin secondary coatings?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I am in the process of finishing up my secondary.

It is wound on a 4-inch diameter PVC core, about

24 inches long.  I used three or four coats of

polyurethane sealant varnish to glue the wires down

to the core.

My question is - should I put on an additional coat

of paraffin to prevent breakouts?  I've sealed off the

ends of the PVC using wooden disks boiled in paraffin

and I was thinking that the wax would coat the secondary windings

rather well if needed...

(I bring the subject of breakouts up, since back in

the summer I witnessed a fellow coiler catch his

secondary on fire due to breakouts from the wire

to his wooden supports!!  This coil was featured

on the front cover of Lindsay's book catalog back

in 1995 or thereabouts.)

Another possibly obvious question - how do I keep this form from falling over

during operation?  How do I mount the base of the PVC in a secure manner??

Using the obvious things like nails and screws seems like a bad idea due

to the voltages, etc.

With a bit of luck we might be able to fire this beast

up in the next weekend or so!

Thanks!

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla coil safety

Cc:

Bcc:

X-Attachments:

Hello all

Safety in use of our Tesla coils is a subject

that hasnt been talked about much on this list.

With the large number of new subscribers I thought

this a good time to explore this topic.

The dangers involved with operating a Tesla fall

into several categories. Lets explore each of these

categories in more detail.

1. Electrocution

This is the number one danger. The voltages we are dealing

with are deadly. The neon sign transformers have voltages

from 6000 - 15000 volts and this voltage alone can kill you.

Its made even more dangerous in the primary circuit of a

Tesla coil due to the storage of energy in the main caps.

The secondary coil output voltage is much less dangerous

although it can be deadly under the right circumstances.

Its is primarily RF energy and tends to flow along the

outside of conductors like the human body. But the output

often contains a considerable 60 hz component and its this

60hz energy that is deadly.

Plus in our effort to increase the stength and length of the

output streamers from our coils we are pumping more current

into those streamers. The use of large toroids and high power

make the streamers just that much more dangerous.

Often some coils have a tendency to arc down into their

primary coils. This arc connects the secondary directly

to the deadly 60hz energy in the primary coil via the ionized

air path. If you are attached to another streamer at the same

time you are effectively connected to the primary. This could

be a deadly situation.

Here are a few dos and donts.

Do built a control system for your coil. House your

variac into this controller and provide proper fusing

and power switches. Provide a power on indicator so

you KNOW when power is applied. A keyswitch on/off

switch will also keep someone from accidently turning on the

power while your are making adjustments. Make it a habit

of keeping the key WITH YOU.

Do not make any kind of adjustments with the power on.

It's easy to forget sometimes. Make it a habit of turning

the power OFF. Just turning down the variac to zero is

not good enough. Some variacs will put out 1-2 volts at

their zero setting. 2 volts into a 15kv neon transformer

could result in 250 volts. Enough for a nasty shock.

Never assume anything is OFF. Check for yourself.

Do use a little care in your construction. Jerry rigged

setups are OK for a short test but something a little more

permanent should be a priority. Having high voltage leads

coming loose under power can be quite dangerous.

Do discharge any high voltage capacitors before working

on them. In most cases the main cap in a Tesla coil is discharged

as soon as the power is removed the path being through the transformer

winding. If your neon secondary should fail and open up during

a run a lethal charge could remain in your caps. If you are going

to do high voltage work utilizing capacitors make and USE a

shorting wand with a 100K or more power resistor to safely discharge

your caps.

2. Ozone

Ozone is classified as a health hazard at quite

low concentrations. If you can smell it the concentration

is already well above safe limits. The smell is a slightly

sweet bleach type odor. You may also notice some stronger

biting odors. These are caused by various oxides of nitrogen.

These are even more noxious than ozone. These gases are produced

in great quantity by our spark gap and secondary output. They

are a fact of life with high voltage and should be dealt with.

Provide some form of airflow to the outside if you coil indoors.

Open a window for fresh air. If all else fails limit run times

and remove yourself to fresher air.

3. Fire or explosion

The high power streamers from our coils can set objects on fire.

Be aware of what objects are getting struck by the output streamers.

Remove any flammables from the vicinity of the coil.

Make sure your AC wiring is adequate for the expected load and

properly fused. Overloaded circuits can overheat.

Keep a fire extinguisher in the coiling room at all times.

Make sure you get the type that is safe for use on electrical fires.

The main tank circuit capacitors are storing terrific amounts

of energy. If the cap fails internally that energy could cause

the capacitor to explode. I have seen SW bottle caps blow up

and heard of several commercial pulse caps exploding in Tesla use.

One of my poly and oil rolled caps developed an internal problem

and began to overheat. I had drilled a small vent hole in the

top cover. After one rather long run enough pressure had built

up that oil was squirting out the vent hole. Without the vent

it would have probably exploded. That cap went into the junk pile.

Do not use metal cased caps like typically found in microwave

ovens. These are meant as power supply filter caps and are not

suitable for Tesla use. They use mylar dielectric and will

overheat and likely explode.

Vent your homemade caps and place others in a strong container

to contain the blast if a cap should explode.

4. Noise

Tesla coils are noisy devices. Small coils are just loud.

Larger coils are absolutely deafening. Its a good idea

to wear some sort of hearing protection while running

your coil indoors. I wear a pair of ear muffs meant for

use on a pistol range.

Im sure there are other hazards to avoid when building

and using Tesla coils. Use your head and if you dont have

any idea on how to do things safely then get assistance.

More safety information can be had by downloading the

Tesla coil safety FAQ from Dave Bronakowski's archive site.

The file name is safety.txt and the URL is:

http://www.velocity.net/~djb/files.shtml

Live long and prosper!

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Paraffin secondary coatin

Cc:

Bcc:

X-Attachments:

At 07:44 PM 12/2/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Tue, 02 Dec 1997 13:44:31 -0600

>Subject: Paraffin secondary coatings?

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I am in the process of finishing up my secondary.

>It is wound on a 4-inch diameter PVC core, about

>24 inches long.  I used three or four coats of

>polyurethane sealant varnish to glue the wires down

>to the core.

>

>My question is - should I put on an additional coat

>of paraffin to prevent breakouts?  I've sealed off the

>ends of the PVC using wooden disks boiled in paraffin

>and I was thinking that the wax would coat the secondary

>windings rather well if needed...

No. I wouldnt bother. If you want more insulation slap

on a couple more coats of varnish. The nice glossy

polyurethane looks cool. Paraffin looks yucky.

As long as the coating is heavy enough so you can no longer

feel the individual wires it will be fine. It should feel

perfectly smooth.

<SNIP>

>Another possibly obvious question - how do I keep this form from

>falling over during operation?  How do I mount the base of the

>PVC in a secure manner??  Using the obvious things like nails

>and screws seems like a bad idea due to the voltages, etc.

I dont seal the bottom of the form. I cut a disk of 3/4 inch

wood and mount this in the center of a larger disk using only

glue. (You're right. No nails or screws allowed)

This smaller disk is a snug fit in the end of the

secondary form with the larger disk forming a foot.

I know one other coiler who telescoped a smaller piece of

plastic pipe inside the secondary form. The fit was tight enough

that the secondary stayed where it was put and made it easy to adjust

the height for coupling changes. Sounds like a good idea to me.

>

>With a bit of luck we might be able to fire this beast

>up in the next weekend or so!

>

>Thanks!

>

>-Bill

Great. keep us posted.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: re: Small Xformer salvage

Cc:

Bcc:

X-Attachments:

At 12:24 AM 12/2/97 GMT, you wrote:

>From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

>Date: Mon, 1 Dec 1997 17:24:10 -0700 (MST)

>Subject: re: Small Xformer salvaged, Want Coil.

>

>

>

>Hello,

>

>   I have obtained a small transformer that was  for a home bar

>neon sign.  I am not sure of the power/voltage output.  Could

>someone help me construct a modest coil build around this xformer?

Hello Jon. Havnt talked to you in a while. Glad to

see your still kicking.

Is there a plate on your neon? If not you need to

determine its ratings so you know what size if cap

to use.

If you have a meter you can inject 5 or 6 volts AC into

the primary of the neon amd read the secondary voltage.

Measure between one of the HV outputs and the case.

With 6 volts input a 9kv neon will put out around 225 volts.

A 12kv one will read around 300 volts and a 15kv one about 375.

If you have a meter that reads AC milliamps you can connect it

directly across the HV outputs and feed 120 volts into the primary.

The meter will send the neon into limiting and you will read

the short circuit current of the neon.

>

>I am thinking of a demonstration unit for the holidays.  Mike

>Hammer has been of wonderful assistance in the past, perhaps you

>could drop me a line, Mike (and anyone else who would care to) at

>jpbazemo@ouray.cudenver.edu.  Thank you very much.

>

>

Will do.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Safety gaps vs. Spark gap

Date: Wed,  3 Dec 1997 05:08:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kevin A. Zachary" <kzachary@kih.net>

Date: Wed, 3 Dec 1997 00:08:08 -0500

Subject: Safety gaps vs. Spark gaps, etc.

Hello -all,

What is the difference between a safety gap and a spark gap?  What

exactly is a safety gap and where is it placed within the primary circuit

(a good-looking schematic will help me alot!)?

The old saying "a picture is worth a thousand words" works well for me...

If anyone has some close-up pictures of their coil, parts, etc. that I can

view, please let me know your URL or send them directly to me via e-mail...

I know that our listserver doesn't like pictures (according to Mike).

There has been a lot of discussion concerning spark gaps in a series,

rotory gaps, maybe even parallel?... which method is best for a 12-24"

spark generating coil?

Finally, I have decided that my old flat glass capacitor might have been

OK years ago, but now has been replaced by either flat or rolled poly caps.

Just exactly what is poly?  polyethene?  Is this stuff anything like

plexiglass?  I'm not exactly sure what I should be looking for...

My next primary will probably be copper tubing instead of #10 household

wire, any suggestions for the tubing diameter?  1/4", 3/8", ... ?  Is there

a problem with arcing between the primary turns since it is not insulated

(if that makes a difference in the HV land)?

That's enough for now.. :-)   Many thanks to all for your info!

KevinZ

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: re: Small Xformer salvage

Date: Wed,  3 Dec 1997 08:38:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>If you have a meter that reads AC milliamps you can connect it

T>directly across the HV outputs and feed 120 volts into the primary.

T>The meter will send the neon into limiting and you will read

T>the short circuit current of the neon.

If you don't have an AC milliameter, you can take a 100 ohm, 5 watt

resistor and connect it between the secondary terminals.  Read the

voltage across the resistor with an AC voltmeter when the transformer is

energized.  You will read 1 volt AC across the resistor for every 10 MA

of current from the transformer.

WARNING!!  Make all the meter connections to the transformer BEFORE

you turn on the power!  Turn the power off BEFORE you try to adjust or

disconnect anything.

Ralph

---

Little girls, like butterflies, need no excuse. -Heinlein

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla coil safety

Date: Wed,  3 Dec 1997 14:06:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Wed, 03 Dec 1997 09:06:29 -0500

Subject: Re: Tesla coil safety

Mike:

Thanks for sharing the reminders on coil safety, but you left one out.

5.)  Wives don't particularily find tesla coils that interesting.  Don't waste

your time discussing the finer points of coiling with them...they usually just

don't get it.  Above all you may have to decide which is better to have the coil or your wife.

Safe Coiling,

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 12/02 6:56 PM >>>

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 02 Dec 1997 16:37:20 -0600

Subject: Tesla coil safety

Hello all

Safety in use of our Tesla coils is a subject

that hasnt been talked about much on this list.

With the large number of new subscribers I thought

this a good time to explore this topic.

<SNIP>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Saftey Gapping, both xfrm

Date: Wed,  3 Dec 1997 18:01:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Wed, 3 Dec 1997 10:01:40 -0800 (PST)

Subject: Safety Gapping, both xfrmr and cap?

I just read an interesting little document from Bert Pool, wherein he

discussing placing a spark-gap across the capacitor. What is the

collective opinion on this practice?

I would think that the gap, if it fires, is gonna be a rather loud

*BANG* as it discharges the cap.

Related question: Currently my saftey gap consists of a pair of

coat hanger wires, with a 1/4" radius curve at the gap point. There

is a centerwire between them, grounded to the case. I am checking into

changing to spherical terminals, specifically, bearings soldered into

copper pipe.

Comments?

Progress note: My 0.004 cap is nearing completion, finally. It is

3 0.012 caps, constructed of 22 5x8" plates, with 5 sheets of 6mil

poly between each sheet. The stacks are then bound in 3/8" Polycarb

and will be placed in a 8x9x12 3/8" polycarb box, and then covered in

oil. Should look nice :). Because of the ready supply of scrap

polycarb, the total cost of this beast, excluding my time, is about

$25, including poly, aluminum foil, and 3/4 gallon Diala-AX.

Who says a home-built cap has to be expensive. I intend to build

a couple of 0.003 caps to work in parallel with this [ smaller plates

due to differing size scrap]

Since this was no small amount of work, I am very curious about the

effectivness of a safety-gap to protect the cap

Thanks

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety gaps vs. Spark gap

Cc:

Bcc:

X-Attachments:

At 05:08 AM 12/3/97 GMT, you wrote:

>From: "Kevin A. Zachary" <kzachary@kih.net>

>Date: Wed, 3 Dec 1997 00:08:08 -0500

>Subject: Safety gaps vs. Spark gaps, etc.

>

>Hello -all,

>

>  What is the difference between a safety gap and a spark gap?  What

>exactly is a safety gap and where is it placed within the primary circuit

>(a good-looking schematic will help me alot!)?

The spark gap is the brains of a Tesla coil. Its function is

like a high power high speed switch. It allows the main cap

to charge up and then to dump that charge into the primary coil

then quickly switch back so the cap can begin charging once again.

A safety gap is a single static gap placed across the HV outputs

of your supply transformer or neon. Its purpose is to breakdown

at voltages that exceed the transformer rating. By breaking down

and conducting it limits the voltage to a value safe for the transformer.

The safety gap can have a grounded center post to conduct excess

voltages safely to ground.

Here is my almost famous ascii coil schematic.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

>

>  The old saying "a picture is worth a thousand words" works well for me...

>If anyone has some close-up pictures of their coil, parts, etc. that I can

>view, please let me know your URL or send them directly to me via e-mail...

>I know that our listserver doesn't like pictures (according to Mike).

You can start with my web page at:

http://www.misslink.net/mhammer/

Then at the bottom of the page follow the links to the

Tesla coil web ring. This is almost 40 linked web pages all with

Tesla or Tesla coil info.

>

>  There has been a lot of discussion concerning spark gaps in a series,

>rotary gaps, maybe even parallel?... which method is best for a 12-24"

>spark generating coil?

It depends mostly on your power level. The best gap for neon

transformer powered coils is the cylinder gap popularized

by Richard Quick. Download the file sparkgap.zip from

Dave Bronakowski's archive site at:

http://www.velocity.net/~djb/files.shtml

Do not use a rotary gap with neon powered coils. They offer no

real advantage and can even damage a neon transformer.

>

>  Finally, I have decided that my old flat glass capacitor might have been

>OK years ago, but now has been replaced by either flat or rolled poly caps.

> Just exactly what is poly?  polyethene?  Is this stuff anything like

>plexiglass?  I'm not exactly sure what I should be looking for...

Plexiglass is PMMA - polymethyl methacrylate.

What you want is polyethylene. Polyethylene is available

as high density or low density material. HDPE or LDPE.

HDPE is rather stiff and should only be used for flat plate

caps. LDPE is soft and supple and can be used for either

flat plates or rolled up caps.

LDPE sheeting in rolls is sold at many lumber yards and home improvement

centers. Its comes in many different thicknesses. Get the thickest

you can find. Thats usually going to be 6 mils. Thicker poly can be

ordered from plastics dealers. Consult the yellow pages.

>

>  My next primary will probably be copper tubing instead of #10 household

>wire, any suggestions for the tubing diameter?  1/4", 3/8", ... ?  Is there

>a problem with arcing between the primary turns since it is not insulated

>(if that makes a difference in the HV land)?

1/4 inch soft copper tubing will make an excellent primary coil

and will handle a considerable amount of power. Several KVA.

The space between turns need only be from 1/4 inch to 3/8 inch.

There is not a problem of arcing between turns as the voltage difference

between adjacent turns isnt very great.

>

>  That's enough for now.. :-)   Many thanks to all for your info!

>

>KevinZ

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark gap/safety gap

Date: Wed,  3 Dec 1997 21:08:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Wed, 3 Dec 1997 22:08:37 +0100

Subject: Spark gap/safety gap

I just read the mail "safety gaps vs spark" and read that the spacing

between the primary turns just need to be from 1/4 to 3/8". On my mini coil

there is about 8mm's spacing between the turns, will this work? Or is the

spacing that important?

my coil spec's

Secondary: 2" PVC tube, wounded with 12" of 29AWG copper wire

Primary 12 turns of 3/16 soft copper tubing in 22 degrees alignment.

Transformer: 15kV 20mA OBT

Capacitor: 4200pF poly plate cap.

Spark gap: dont know yet.

Toroid made of 3" dryer duct.

But now to my main question, I know i need some safety gap, but I dont know

how to make one? Can I just use 2 bolts (them with rounded heads, dont know

what they are called in the US), and how close together? And what type of

Spark gap would you recommend for this coil? I've been thinking of those

static gaps with copper tubes coupled in series, but how are the tubes

connected to each other, or are they?

And distance between them? This is a lot of questions but I hope you can

answer some of them.

//Christopher Eriksson

Sweden

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark gap/safety gap

Cc:

Bcc:

X-Attachments:

At 09:08 PM 12/3/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Wed, 3 Dec 1997 22:08:37 +0100

>Subject: Spark gap/safety gap

>

>I just read the mail "safety gaps vs spark" and read that the spacing

>between the primary turns just need to be from 1/4 to 3/8". On my mini

>coil there is about 8mms spacing between the turns, will this work?

>Or is the spacing that important?

Spacing isnt really that critical. 8mm will be just fine.

Tighter spacing produces somewhat more coupling so can be

useful in flat primaries that often dont couple enough.

Other than that the spacing only affects the inductance of the

coil with tighter spacing resulting in somewhat more inductance.

>my coil specs

>Secondary: 2" PVC tube, wounded with 12" of 29AWG copper wire

>Primary 12 turns of 3/16 soft copper tubing in 22 degrees alignment.

>Transformer: 15kV 20mA OBT

>Capacitor: 4200pF poly plate cap.

>Spark gap: dont know yet.

>Toroid made of 3" dryer duct.

>But now to my main question, I know i need some safety gap, but I dont

>know how to make one? Can I just use 2 bolts (them with rounded heads,

>dont know what they are called in the US), and how close together?

A safety gap isnt critical in design at all. You can make it from

just about anything. I use 2 bolts like you suggested with their

rounded heads facing each other. One is made adjustable. The setting

depends on the voltage of your transformer. With 15kv I would try for

around .375 inch or 9.5 mm. Adjust up or down slightly according

to the level of activity.

>And what type of Spark gap would you recommend for this coil?

>Ive been thinking of those static gaps with copper tubes coupled

>in series, but how are the tubes connected to each other, or are they?

The cylinder type gap is one of the best for neons (or for your OBIT).

The individual electrodes are not connected together. You make your

connection to the two end electrodes and the spark jumps from

electrode to electrode.

>And distance between them?

The distance should be about .025-.03 inches ( .635 - .76 mm) between each

gap. You should have 10 of these gaps (11 electrodes) for best

results with 15kv. This gives a maximum gap distance of .250 - .300

inches. You dont want to use all the gap at first. You can select

fewer gaps to start. This will keep the power level low until you get in tune.

>

>//Christopher Eriksson

>Sweden

>

>

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Saftey Gapping, both xfrm

Cc:

Bcc:

X-Attachments:

At 06:01 PM 12/3/97 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Wed, 3 Dec 1997 10:01:40 -0800 (PST)

>Subject: Saftey Gapping, both xfrmr and cap?

>

>I just read an interesting little document from Bert Pool, wherein he

>discussing placing a spark-gap across the capacitor. What is the

>collective opinion on this practice?

Its a good idea although I have never done it myself. And I have

had several cap failures too. Make it an integral part of

the cap. Permanently mounted. The gap should be rather wide

say around 1/2 inch. This is a gap you dont really want to see

firing. If its firing then that probably indicates your main

gap is way too wide.

>

>I would think that the gap, if it fires, is gonna be a rather loud

>*BANG* as it discharges the cap.

Yes it will tend to bark rather loudly. It will definitely

alert you that something is amiss.

>

>Related question: Currently my safety gap consists of a pair of

>coat hanger wires, with a 1/4" radius curve at the gap point. There

>is a centerwire between them, grounded to the case. I am checking into

>changing to spherical terminals, specifically, bearings soldered into

>copper pipe.

>

>Comments?

Probably overkill. The safety gap doesnt have to be anything fancy.

I just use a couple bolts for mine.

>

>Progress note: My 0.004 cap is nearing completion, finally. It is

>3 0.012 caps, constructed of 22 5x8" plates, with 5 sheets of 6mil

>poly between each sheet. The stacks are then bound in 3/8" Polycarb

>and will be placed in a 8x9x12 3/8" polycarb box, and then covered in

>oil. Should look nice :). Because of the ready supply of scrap

>polycarb, the total cost of this beast, excluding my time, is about

>$25, including poly, aluminum foil, and 3/4 gallon Diala-AX.

>Who says a home-built cap has to be expensive. I intend to build

>a couple of 0.003 caps to work in parallel with this [ smaller plates

>due to differing size scrap]

>

>Since this was no small amount of work, I am very curious about the

>effectiveness of a safety-gap to protect the cap.

Sounds like a great cap. With 3 units in series and 30 mils

of poly in each cap it should be nearly bullet proof.

Adding a safety gap to it would just be some extra insurance.

>Thanks

>

>Michael Baumann

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: dialectic constant of gla

Cc:

Bcc:

X-Attachments:

At 02:16 AM 12/5/97 GMT, you wrote:

>From: Dsurfr <Dsurfr@aol.com>

>Date: Thu, 4 Dec 1997 21:16:47 EST

>Subject: dialectic constant of glass

>Organization: AOL (http://www.aol.com)

>

>Sorry to keep posting such mundane questions but you keep giving me correct

>answers I can't find anywhere else. I'm trying to make a cap rated .0053uf

>using salt water/beer bottles. My problem in my calculation of how many

>bottles I will need is a confusion on my part as to the dialectic constant I

>should use for "beer bottle" glass. I have found postings ranging from 6 to 8.

>Any help will be much appreciated.

>Jim

Jim unfortunately I cant be any more specific than a constant of 6-8

for glass. There are so many types of glass and the composition of

glass used in bottles varies widely. The harder borosilicate glasses

like Pyrex have higher dielectric constants while the softer glasses

have lower constants. Best bet is to use a nominal value of 7.0

Your capacitance does not have to be exactly .0053 mfd. Considerable

tolerance is permitted.

I got approximately 700 pf per 12oz beer bottle. So 7 or 8

of these in parallel would get you in the ball park.

The nice thing about bottle caps is its easy to add or

subtract some to find the best combination.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Microwave doubler?

Date: Thu,  4 Dec 1997 16:40:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 04 Dec 1997 10:40:03 -0600

Subject: Microwave doubler?

Organization: MailExcite  (http://www.mailexcite.com)

Perhaps it'd be possible to use a microwave oven

transformer to power a doubler/tripler/multiplier

circuit to boost the voltage.  This circuit

uses diodes and capacitors in such a way that

it produces a DC output which is a multiplied

version of an AC (line frequency) input.

Current levels are usually low, but then again, we

have loads of amps to start with.  The caps used

in the doubler need not be pulse rated, you could

probably use 60Hz rated motor starting caps.

The output cap would be a pulse rated tesla coil type capacitor,

which would connect to an otherwise conventional setup.

This is possibly just a wild idea!  The surplus place

down the road has lots of microwave oven

transformers, HV diodes and the like.  It'd be nice

if this stuff could be used for coiling!

-Bill

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: I fired it up, sort of.

Date: Thu,  4 Dec 1997 18:42:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 04 Dec 1997 12:42:57 -0600

Subject: I fired it up, sort of.

Organization: MailExcite  (http://www.mailexcite.com)

OK, I couldn't wait any longer, so last night I tried

firing up my coil for the first time!  Here is what I had

going:

4 inch diameter PVC, 28 inches long, 26 g wire

"Toroid" emulator - an old 2 lb coffee can

No RF ground to speak of (!)

Secondary consisting of six turns of stranded

(insulated)

14 gauge wire haphazardly wrapped around the

base of the coil. (diameter of turns = 6 inches)

Air/poly cap using only 0.006 inch between foils.

(severely over-rated)  About 0.02 uF calculated.

RQ-style quenched gap w/o a fan

RF-blocking chokes with ferrite cores

Safety gap

No variac, just an on/off switch.

Small gauge stranded hookup wire using alligator

clips in the primary circuit. (!)

Power supply: One of:

7Kv at 5 mA ionization transformer

Chrysler ignition coil w/ solid state driver

Both power sources were (alternately) connected

to a 12 KV-rated diode bridge, which then fed the

capacitor through the blocking choke, etc.  This

allows me to avoid trying to match the cap value

to the impedence of the transformer, etc

.......................................................................

So - did it work?  Well, sort of.  I did get a bunch of

small (2 inches max) streamers, and lots of corona.

Max discharge length to a grounded strike wire was

<2 inches.

No attempt was made at tuning.

My air/poly cap decided it'd had enough, and blew

out after about 10 minutes.  I'm surprised it lasted that

long since I could see blue corona coming from within

the poly.  A capacitor demolition derby!

The good oil/poly caps were over at a friend's place,

so unfortunately I couldn't try those.

Obviously lots of room for improvement!  Comments?

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

------

From: tesla-2@emachine.com (Tesla-2)

Subject: Styrene as a coil form?

Date: Fri,  5 Dec 1997 17:32:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 5 Dec 1997 09:32:44 -0800 (PST)

Subject: Styrene as a coil form?

I have found a 40" section of 6" Styrene pipe, and am considering using

this as a coil form, instead of PVC.

It is the stuff known as 'drain pipe'.

It would sure beat having to buy pipe if it works, as the shortest

length of 6" PVC I can buy is 20' out here.

Oh, and for that monster coil I can dream about.. would a 3:1 H:D ratio

be ok for an 18" diameter coil?   [ I have a shot at some 18" PVC in

the next few months. but it will only be about 50" long]

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Styrene as a coil form?

Cc:

Bcc:

X-Attachments:

At 05:32 PM 12/5/97 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Fri, 5 Dec 1997 09:32:44 -0800 (PST)

>Subject: Styrene as a coil form?

>

>

>I have found a 40" section of 6" Styrene pipe, and am considering using

>this as a coil form, instead of PVC.

>It is the stuff known as 'drain pipe'.

>It would sure beat having to buy pipe if it works, as the shortest

>length of 6" PVC I can buy is 20' out here.

Depends on what you mean by styrene.

Polystyrene (PS) is an excellent material and has a

low loss factor. PS sheeting will also make a fine

stacked plate capacitor. It has a little more loss

than polyethylene but would still be excellent.

It would be great for a secondary form.

Acrylonitrile-butadiene-styrene (ABS) has a much higher dissipation

factor and is in the same league as mylar. It would not be

useful for capacitors but I would guess it would be OK

as a secondary form material having lower loss than PVC.

>

>

>Oh, and for that monster coil I can dream about.. would a 3:1 H:D ratio

>be ok for an 18" diameter coil?   [ I have a shot at some 18" PVC in

>the next few months. but it will only be about 50" long]

>

Geez. That would be a monster. 3 to 1 is fine for

that size of coil. You will want to use fairly heavy wire.

A 48 inch winding of AWG 14 would be about 720 turns with

a resonant frequency of around 100 KHZ unloaded.

This will require some really hefty caps and large primary for that

low a resonant frequency and lots of power.

A very serious (and expensive) coil.

But if you're gonna dream, dream BIG right?

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: H /d ratio's

Date: Fri,  5 Dec 1997 18:09:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 05 Dec 1997 12:09:16 -0600

Subject: H /d ratio's

Organization: MailExcite  (http://www.mailexcite.com)

I was wondering, what is the proper height to width

ratio for a 6" dia. coil. What is the proper wire gage

to use and winding length.

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: H /d ratio's

Cc:

Bcc:

X-Attachments:

At 06:09 PM 12/5/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 05 Dec 1997 12:09:16 -0600

>Subject: H /d ratio's

>Organization: MailExcite  (http://www.mailexcite.com)

>

>

>

>I was wondering, what is the proper height to width

>ratio for a 6" dia. coil. What is the proper wire gage

>to use and winding length.

>

>Paul S. Marshall

Paul,

Using 6 inch PVC with a nominal OD of 6.5 inches and a

4 to 1 H/D ratio gives us a 26 inch winding length.

Wind this with AWG 22 for around 960 turns and an

unloaded resonant frequency of 260 KHZ.

I know of several coils with these exact dimensions

and they are potent performers. This will also be my

next coil.

This is an excellent size coil being small enough

to fire with neons and big enough to take several

KVA from a pole pig power supply.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety Gap

Cc:

Bcc:

X-Attachments:

At 08:28 PM 12/5/97 GMT, you wrote:

>From: Jester511 <Jester511@aol.com>

>Date: Fri, 5 Dec 1997 15:28:22 EST

>Subject: Safety Gap

>Organization: AOL (http://www.aol.com)

>

>Hey,

>

>What is the purpose of a safety gap.Any information you can give me

>about it would be very helpful.

>

>

>Thanks,

>

>Calvin

>

Calvin,

A safety gap is used as a means of protection for

our supply transformer. The most common supply transformer

is a neon sign transformer. Neons are great. They are cheap

and readily available. But they are a bit fragile. They have

very fine wire in their secondary windings and are insulated

with tar. This makes them easily damaged.

The whole purpose of the spark gap, primary coil

and main capacitor in a Tesla coil is to process

the 60 hz energy from the supply transformer and

convert it to RF energy to be fed to the secondary.

But that RF energy will quickly cause problems if it

kicks back toward toward your neon.

So we use RF chokes on the HV supply lines coming from

the neon and place a safety gap directly across the HV outputs.

This safety gap will break down and arc over if the voltage

gets too high thus limiting the voltage to a safe value.

It can be made very simply. Nothing fancy is needed.

Just a couple round head bolts positioned head to head and

spaced about 3/8 apart is a good start.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: SG Airlow

Date: Thu,  4 Dec 1997 23:54:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@dynasty.net>

Date: Thu, 04 Dec 1997 23:54:56

Subject: SG Airlow

How critical is the airspeed velocity of an unladen swallow. . .uh, sorry,

wrong question

OK, here goes,  How critical is the velocity of the air passing through the

SG?  I understand about removing the ionized air in order to keep

resistance relatively constant, but will any sort of an increase in

performance be gained from increasing the velocity of the air from my

muffin fan?

Reason I ask is, I was considering using something like a model rocket nose

cone placed in the center of the assembly to act as a venturi to increase

airspeed.  But would it be worth the effort?

Thanks   CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Used neons

Date: Fri,  5 Dec 1997 00:19:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@dynasty.net>

Date: Fri, 05 Dec 1997 00:19:02

Subject: Used neons

Some time ago someone on the list mentioned that their supply of used neons

had dried up.  There is a shop near me that has close to twenty of them

collecting dust.  They want $25 apiece.  Voltage ratings from 4-15Kv, and I

didn't see any ma ratings above 30 (but I didn't look real closely).  If

anyone is interested drop me a line.

Or I'd be more than willing to work out a trade for an OBIT (can't find any

in this area).

Thanks,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: dielectric constant of glass

Date: Fri,  5 Dec 1997 02:16:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr <Dsurfr@aol.com>

Date: Thu, 4 Dec 1997 21:16:47 EST

Subject: dielectric constant of glass

Organization: AOL (http://www.aol.com)

Sorry to keep posting such mundane questions but you keep giving me correct

answers I can't find anywhere else. I'm trying to make a cap rated .0053uf

using salt water/beer bottles. My problem in my calculation of how many

bottles I will need is a confusion on my part as to the dialectic constant I

should use for "beer bottle" glass. I have found postings ranging from 6 to 8.

Any help will be much appreciated.

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: dialectic constant of

Date: Fri,  5 Dec 1997 15:30:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 5 Dec 1997 07:30:51 -0800 (PST)

Subject: Re: dialectic constant of gla

On  5 Dec, Tesla-2 wrote:

>

> Sorry to keep posting such mundane questions but you keep giving me correct

> answers I can't find anywhere else. I'm trying to make a cap rated .0053uf

> using salt water/beer bottles. My problem in my calculation of how many

> bottles I will need is a confusion on my part as to the dialectic constant I

> should use for "beer bottle" glass. I have found postings ranging from 6 to 8

.

> Any help will be much appreciated.

> Jim

Based on very recent experience [ I am just now switching from SW to

poly caps, a bit more work than I originally thought, but fun], you

will find considerable variation even with the same kind of bottle.

I used 12oz brown beer bottles, what are known in homebrewing as

'shorties', Very cylindrical, but not as tall as a longneck. Easy

for me, I have about 8 cases of these bottles :).

They were wrapped in HD foil, and then taped. The ratings varied between

.64 and .77 nf per bottle. I just kept adding bottles till I ran

out of stoppers, was running a total of 8, that gave me about .0057.

I probably should have added one more, but decided to build my poly cap

instead.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: SG Airlow

Cc:

Bcc:

X-Attachments:

At 11:54 PM 12/4/97 GMT, you wrote:

>From: Chris Federico <cfed@dynasty.net>

>Date: Thu, 04 Dec 1997 23:54:56

>Subject: SG Airlow

>

>

>How critical is the airspeed velocity of an unladen swallow. . .uh, sorry,

>wrong question

Have to look that one up.

>

>OK, here goes,  How critical is the velocity of the air passing through the

>SG?  I understand about removing the ionized air in order to keep

>resistance relatively constant, but will any sort of an increase in

>performance be gained from increasing the velocity of the air from my

>muffin fan?

>

>Reason I ask is, I was considering using something like a model rocket nose

>cone placed in the center of the assembly to act as a venturi to increase

>airspeed.  But would it be worth the effort?

>

>Thanks   CF

>

Yes I believe it would be worthwhile. The airflow serves two purposes.

First is provides cooling air to remove excess heat.

The more power you run the more heat generated and the more

important good airflow becomes.

Second the airflow removes hot ions from between the electrodes

and can actually serve to stretch and cool the arc. This improves

the ability of the gap to quench quickly.

Placing your nose cone in the center of the gap assembly will

force the airflow over the electrodes where it will do the most good.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark Gap

Date: Sat,  6 Dec 1997 05:51:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Fireworkar <Fireworkar@aol.com>

Date: Sat, 6 Dec 1997 00:51:32 EST

Subject: Spark Gap

Organization: AOL (http://www.aol.com)

I am using a 15kv Neon S Transformer. I want a good Spark Gap but dont have an

air compressor to make a static Gap. My question is could I use a fan or some

thing cheap, or is there a better Idea for a srark Gap?

Also Where can I find the door Knob Caps to protect the Neon S transformer(

what is there rating and how many do I need). One last thing can someone

Direct me to a good place to find directions to make the RF Chokes to protect

my Transformer.

Thank You!! sorry about the spelling.  Any and all help will be appriciated

sorry if the Questions are dumb this is new stuff to me. Thanks again

You can E-mail me at (Fireworkar@aol.com)

Erick Allen

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark Gap

Cc:

Bcc:

X-Attachments:

At 05:51 AM 12/6/97 GMT, you wrote:

>From: Fireworkar <Fireworkar@aol.com>

>Date: Sat, 6 Dec 1997 00:51:32 EST

>Subject: Spark Gap

>Organization: AOL (http://www.aol.com)

>

>I am using a 15kv Neon S Transformer. I want a good Spark Gap but dont have an

>air compressor to make a static Gap. My question is could I use a fan or some

>thing cheap, or is there a better idea for a spark Gap?

Dont need compressed air. Especially for a neon transformer. You want

to build a cylinder gap. This has electrodes cut from hard copper

pipe mounted radially inside a piece of PVC pipe. A muffin fan is mounted

on top to provide cooling air.

Check Dave Bronakowski's archive site for the file sparkgap.zip.

The URL is: http://www.velocity.net/~djb/tesla.shtml

>

>Also Where can I find the door Knob Caps to protect the Neon transformer

>What is there rating and how many do I need?

You do not need any bypass caps on your neons. I dont use any.

My first and only attempt to place bypass caps on a neon resulted

in a smoked transformer. I believe that bypasses can sometimes

set up unwanted resonances in some transformers.

If you use some RF chokes and a safety gap you dont

need bypass caps.

>One last thing can someone direct me to a good place to find directions

>to make the RF Chokes to protect my Transformer.

Really not that difficult as the value is not at all critical.

Anything from 2-10 mh works fine. I use 120 turns of AWG 26

wound on large 2.5 inch diameter ferrite toroids purchased

surplus. This gives about 2.25 mh.

If you cant find toroids ferrite rods work as well.

Failing that you can wind air core chokes on 2 inch PVC

pipe. A 6 inch long winding of AWG 26 is about 350 turns

and results in about 2.5 mh of inductance.

The use of some series resistance is recommended with air

core chokes to dampen any oscillations that might occur.

Use anything from 300-1500 ohms and 10 or more watts

depending on your power level.

Place one resistor and one choke in series in each

HV leg. These go in the HV lines in between the safety gap

and main gap.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Safety Gap

Date: Fri,  5 Dec 1997 20:28:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jester511 <Jester511@aol.com>

Date: Fri, 5 Dec 1997 15:28:22 EST

Subject: Safety Gap

Organization: AOL (http://www.aol.com)

Hey,

What is the purpose of a safety gap. Any information you can give me

about it would be very helpful.

Thanks,

Calvin

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety Gap

Date: Fri,  5 Dec 1997 22:15:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Fri, 5 Dec 1997 14:15:35 -0800 (PST)

Subject: Re: Safety Gap

On  5 Dec, Tesla-2 wrote:

> Hey,

>

>  What is the purpose of a safety gap. Any information you can give me

> about it would be very helpful.

>

It provides safety. Sorry, couldn't pass that up.

Truth be known, it extends the serviceable life of your neon transformer

by shorting out any voltage that would exceed the capability of your

windings. There is some nasty kickback from your primary at times.

You need one. Trust me. Learn from my foolishness. I have a nice little

9KV boat anchor from running without one. The transformer lasted all

of about 3 minutes of use.

Place it directly across the output terminals of your neon, setting

it so that is just barely does not fire when your neon is disconnected

from your coil.

If you do not have a variac, you need to be a little more careful in

setting the gap, as I have learned it is possible to think you have set

it "just so" but not. When you switch on the neon, there is a

possiblity of a transient that will fire your SG, that is greater than

the running voltage. If the gap fires on powerup, see if you can blow

it out with a blast of air. if the spark goes out, and does not refire,

you can close the gap a bit more.

I have my gap set on the 'hairy edge' of firing, which I am told is a

bit conservitive.

Your gap *SHOULD* fire every few seconds during operation  - if it

doesn't, close it up a little. Seeing that little spark now and again

is a reassurance that the device will still be with you in the future.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Top load shapes

Date: Sat,  6 Dec 1997 06:37:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@dynasty.net>

Date: Sat, 6 Dec 1997 00:37:30 cst

Subject: Top load shapes

Organization:

I'd like to hear an opinion or two on an idea I have for a rather unusual top

load design.

Several 12" aluminum plates, spaced 1" apart all connected.  This isn't a very

good description . . . anybody familiar with

the tape reals for a 16 channel sound recorder?  Three trapezoidal holes in each plate and a 1" plastic spacer at the core.

As I see it, if it would make a "good" top load someone would've come up with the idea before now, but what I want to

know is what are the chances of it actually working (I'm going for visual impact here, more so than great performance)?

Any opinions?  Or should I just try it and see what happens?

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Spark Gap

Date: Sun,  7 Dec 1997 03:55:35 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Fireworkar <Fireworkar@aol.com>

Date: Sat, 6 Dec 1997 22:55:35 EST

Subject: Re: Spark Gap

Organization: AOL (http://www.aol.com)

Thanks for the Help that is exactly what I wanted to know.  It was a huge help

thanks again!!!!

Erick Allen

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Top load shapes

Cc:

Bcc:

X-Attachments:

At 06:37 AM 12/6/97 GMT, you wrote:

>From: Chris Federico <cfed@dynasty.net>

>Date: Sat, 6 Dec 1997 00:37:30 cst

>Subject: Top load shapes

>Organization:

>

>

>I'd like to here an opinion or two on an idea I have for a rather unusuall top

>load design.

>

>Several 12" aluminum plates, spaced 1" apart all connected.  This isn't a very

>good description . . . anybody familiar with the tape reals for a 16 channel >sound recorder?  Three trapezoidal holes in each plate and a 1" plastic spacer >at the core.

>As I see it, if it would make a "good" top load someone would've come up with >the idea before now, but what I want to know is what are the chances of it >(I'm going for visual impact here, more so than great performance)?

>

>Any opinions?  Or should I just try it and see what happens?

>

Chris,

I would just try it and see what happens. Any mass

of metal can be used as a top load. I've used lots of various

pots, pans, and metal containers of all sorts. Some give

quite interesting results.

However, if the goal is long sparks you need something

smooth with no edges or abrupt changes in radius. Those

sharp corners will promote spark break out at quite low

voltages. You will end up with many smaller streamers rather

than a single large one.

The ultimate terminal shape for voltage buildup is the sphere.

However the toroid is easier to build and has the added benefit

of suppressing corona.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Poly surprise

Date: Mon,  8 Dec 1997 18:58:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 8 Dec 1997 10:58:42 -0800 (PST)

Subject: Poly surprise

This is a heads up, for all those that are building poly caps. I got

a nasty little surprise this weekend. I had built a flat-plate cap,

0.006 mf, made from *lots* of sheets of 6 mil poly. 430+ to be more

precise. Because I wanted to ensure that the plates were as tight as

possible, I made tie-straps across the tops and sides of the plate.

Each individual cap was approximately 10x7x1.5".

[the plates are between slabs of 3/8" polycarb, you use what you have.]

I filled it with oil, and and put it away for a few days. when I pulled

it out, all of my tie-straps had been broken off.

Why? Because the poly apparenly absorbed some of the oil, and expanded.

Less than 5%, but for 10", that amounts to 1/2" exapansion, more than

enough to be a problem. I've reworked the tiedowns, but thought I would

warn others, as I have not seen this mentioned in any of the

documentaion for capacitor constrution.  I was using Diala-AX for the

oil.

I hope to fire this at full power by the weekend [currently firing

at about 40%, but not seeing the bubbles I was told to expect, could

I have all the air out already? I did fill it very slowly, so that

the oil was always rising via capillary action faster than the oil

was coming in [ I used a siphon through IV line, it took 7 hours to

siphon the 1 gallon, I did not plan it that way.. it just worked out

that way] no other way to fill it without a vacuum pump.

-

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly surprise

Cc:

Bcc:

X-Attachments:

At 06:58 PM 12/8/97 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Mon, 8 Dec 1997 10:58:42 -0800 (PST)

>Subject: Poly surprise

>

>This is a heads up, for all those that are building poly caps. I got

>a nasty little surprise this weekend. I had built a flat-plate cap,

>0.006 mf, made from *lots* of sheets of 6 mil poly. 430+ to be more

>precise. Because I wanted to ensure that the plates were as tight as

>possible, I made tie-straps across the tops and sides of the plate.

>Each individual cap was approximately 10x7x1.5".

>[the plates are between slabs of 3/8" polycarb, you use what you have.]

>I filled it with oil, and and put it away for a few days. when I pulled

>it out, all of my tie-straps had been broken off.

>Why? Because the poly apparently absorbed some of the oil, and expanded.

>Less than 5%, but for 10", that amounts to 1/2" exapansion, more than

>enough to be a problem. I've reworked the tiedowns, but thought I would

>warn others, as I have not seen this mentioned in any of the

>documentaion for capacitor constrution.  I was using Diala-AX for the

>oil.

Not really a suprise there Mike. Polyethylene does indeed swell

slightly in oil. It really isnt much of a problem. Also some of your

increase may be due to the addition of an oil film to each

surface. Each individual film is quite thin but when taken all

together add up.

I built one of my early plate caps using large rubber bands to hold

the individual caps sandwiched between sheets of plexiglass.

The mineral oil ate up the rubber bands and they all broke.

Also note that hot glue doesnt work in mineral oil. It doesnt

dissolve or anything like that but it tends to debond from

whatever its stuck to.

>

>I hope to fire this at full power by the weekend [currently firing

>at about 40%, but not seeing the bubbles I was told to expect, could

>I have all the air out already? I did fill it very slowly, so that

>the oil was always rising via capillary action faster than the oil

>was coming in [ I used a siphon through IV line, it took 7 hours to

>siphon the 1 gallon, I did not plan it that way.. it just worked out

>that way] no other way to fill it without a vacuum pump.

>

Thats a good method. Ive done the same thing on the theory

that the slowly rising oil can more easily push the air out

than having the air bubbling up through the oil.

Also the use of real transformer oil rather than mineral oil

will allow the bubbles to escape easier.

Increase your power level slowly. You will find a point

where some bubbles begin to appear. It wont be unusual

to have it bubble at full power after every run for a while.

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: How does one make high Q

Cc:

Bcc:

X-Attachments:

At 08:19 AM 11/24/97 GMT, you wrote:

>From: pacster@ibm.net

>Date: Mon, 24 Nov 1997 08:19:37 GMT

>Subject: How does one make high Q caps?

>

>Hello !

>

>Finally!, my exams are over for the year, so now I can get coiling again (I

>definitely prefer coiling to exams). I was reading some posts on capacitors

>and it was said that for a 15KV cap. the polyethelene should be around

>90mils thick. Coming from a 'S.I. units' country I'm a little confused, is

>that thickness 24.5/1000 x 90 = 2.28 mm? If so then its no wonder that my

>caps cook after only one minute of coil operating, I was using 0.45 mm of

>poly between plates, oops!

Yep. Except the conversion constant is 25.4, not 24.5.

0.45 mm is 17.7 mils. 1 mil = .001 inch.

>

>What is the recommended thickness for 7.5, 9, 10, 12 and 15 KV poly capacitors?

This is a matter of some speculation and disagreement.

You have two trains of thought here.

One is to use much more dielectric than necessary to build

in an extra safety margin.

Thats why you often see values of 90 or more mils recommended

for caps. And I seen lots of 90 mil single caps fail at 15kv.

The other is to use series connected caps and take advantage

of the higher PER MIL rating of thinner dielectrics.

My feeling is that its OK to use single caps up to 9kv.

Over 9kv you MUST use series connected caps to distribute

the strain. This is the way to go. By keeping the voltage

each individual cap sees low you can use thinner cheaper

poly and have a more robust long lived cap.

Ive found that 24 mils of poly work very well at 9kv.

For 12kv use I recommend 48 mils of total poly thickness

achieved through the use of at least two series caps.

My present rolled caps are each .03mfd with 24 mils (4 X 6 mils)

of poly each. I run them in series for .015 mfd 48 mils.

That setup runs quite happily at 12kv.

My .008 mfd plate cap has 6 series connected .048 mfd caps

each with 8 mils (2 X 4 mils) of poly. This cap has seen lots of use

and abuse for almost 2 years.

For 15kv I recommend a bare minimum of 60 mils divided

in at least 3 series caps. Thats 20 mils each.

3 series connected 24 mil caps would not be overkill.

>

>I am hoping to build high Q caps from now on and after a fair bit of reading

>apparently these can improve the capacitor Q:

>

>1. Clean all sides of the poly with meth. spirits.

Yes. This cleans the dirt and grit and other contaminates

off your plates and poly. This doesnt improve the Q really

but insures you dont have a focal point for a breakdown.

>2. Round all foil edges.

Reduces the likelihood of corona. Corona is the death

of a cap. Thats why single caps often fail at 15kv. That high

voltage can cause corona even in oil.

>3. Keep the plates as tight as possible .

Provides the highest capacitance.

>4. Add lay sheets of foil between the layers of poly.

This can be done and will improve performance slightly but I dont

believe the benefit is worth the added complexity.

>5. Add sheets of oil soaked kraft paper next to each live foil plate.

This will actually reduce capacitance and increase loss slightly

but its a great method of reducing the problem with trapped air.

If you are using large plates or are building a rolled cap

without access to a vacuum pump this works wonders.

If you have a vacuum pump or are building a plate cap with 8 X 8

plates or less then dont bother with this step.

>6. Provide the biggest terminal ends for each plate.

Very important. This lowers the equivalent series resistance

and inductance of the cap and provides better pulse capability.

>7. Put the caps in series.

The only way to go for survivable HV caps.

>8. Submerge the cap in light mineral oil.

All HV caps need to be in oil to improve voltage rating, displace

air and suppress corona. Real transformer grade oil is better

than mineral oil due to lower moisture content and lower viscosity.

>9. Use equal numbers of + and - plates.

Goes without saying.

>10. When using caps in series they should be the same individual part

>capacitances.

The voltages wont divide equally unless the capacitance of each

individual cap is equal to the others.

>11. Vacuum pressurising the caps.

Gets the air out.

>12. Keep all dust and dirt out of the cap.

>

>What is the most useful in improving the Q and what has little affect on the

>Q, is there anything else that improves the Q? Normally I roll my caps, but

>I think that if I should be using a thickness 2.3mm of poly this could be a

>bit of a task. But perhaps I could make ten 10nF caps and parallel them up

>to 100nF and then series four sets of them down to 25nF?

Plate type caps have much lower inductance than rolled types

therefore better pulse characteristics.

25 nf is a lot of capacitance and much more than required

for most neon power coils. I use only .015 mfd with my 12kv/100ma

power supply and get in excess of 4 feet of spark.

Here is a chart showing proper matching caps for various neon sizes.

Neon	            VA     Impedance    Matching Capacitor

7500v @  30MA	225	 250K		 .0106uf

9KV   @  30MA	270 	 300K	       .0088uF

9KV   @  60MA	540	 150K	       .0177uF

9KV   @  90MA     810    100K        .0265uf

12KV  @  30MA	360	 400K	       .0066uF

12KV  @  60MA	720	 200K	       .0133uF

12KV  @  90MA	1080	 133.33K  	 .0199uF

12KV  @ 120MA     1440   100K        .0265uf

15KV  @  30MA	450	 500K	       .0053uF

15KV  @  60MA	900	 250K	       .0106uF

15KV  @  90MA	1350	 166.66K 	 .0159uF

15KV  @ 120MA     1800   125K        .0212uf

Note that these are not necessarily the best cap sizes. Often

a somewhat smaller cap will provide better operation by

allowing for a larger primary inductance.

And different neons sometimes seem to like a different amount

of capacitance. A little experimentation is in order.

>

>I would also like to take a look at my coil on the scope when it is running.

>To do this can I just connect a piece of wire to the 10x probe and dangle

>the wire in the air about 10 to 15 feet away from my coil (do I need to

>connect the scope earth to anything ?), and then fire up my coil, with

>hopefully my new high Q caps (so that I can watch it for more than a minute

>at a time). Is this O.K. or is there a safer and better method?

Thats the way to do it. Quite instructive and eye opening.

With this you can tell just how much coupling you have

and how well your gap is performing.

Be forewarned not to use a modern solid state scope

for this. Modern electronics and Tesla coils do not

mix well. Get an old tube type scope. It doesnt need

to be anything fancy. Any of the old Heath or Eico scopes

would be fine as well as any of those wonderful old Tektronix

scopes. I purchased a non working tube type Tek at a recent

hamfest for $25. A couple HV diodes and a couple leaky caps

later I had a perfect scope to use around the coil.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Power vs tunning

Date: Wed, 10 Dec 1997 18:08:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 10 Dec 1997 12:08:58 -0600

Subject: Power vs tunning

Organization: MailExcite  (http://www.mailexcite.com)

I am ready to power up my coil and I have a few

questions reguarding power levels, capacitance

and tunning. My inital power level will be 500 w

or less. I have 4 .026 mf caps that I can configure to accomidate power levels

up

to 3kw. I will also use a

variac to ease my way into each power setting. My

question is:

1.) Will a change in coupling occurr as the power /

capacitance increases ?

2.) Will the Rf of the primary change as power /

Capacitance increases ?

3.) Should the primary taps change with increased

power ?

4.) I will begin testing this weekend using a flat

wound spiral made from 1/4" refrigerator tubing,

will the coupling change with the addition of

power.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: How do I tune it?

Cc:

Bcc:

X-Attachments:

At 09:05 PM 12/10/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Wed, 10 Dec 1997 15:05:18 -0600

>Subject: How do I tune it?

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I am about to fire up the coil for the first time.

>

> Any advice on how to tune it?  What signs am

>I looking for?

Double and triple check your wiring. Once you are sure

things are OK set your primary tap at around the halfway

point. Apply power and crank up the variac.

Once the gap breaks down and is firing smoothly

stop turning up the variac. Bring a small flourescent

tube near the secondary terminal. If it lights up

you are producing RF. You will be able to get this even if

way out of tune.

Limit run times to 30 seconds or less at first until you are

sure things are running OK. Check for any signs of overheating.

On the next run bring a grounded wire near the secondary terminal

and see if you can draw off a spark. Note its length.

Kill the power and move the primary tap up two full turns.(more turns)

Fire it up to the same power level and try to draw the spark

again. Notice if you are getting more or less spark.

If you cant get any spark then tune using the flourescent tube.

But your coil has to be pretty far out of whack for that to happen.

If you are getting more spark then keep moving the tap one turn

at a time looking for the peak. As you get closer to resonance you will

notice the sound and character of the coil changing.

If you get all the way tapped out on the primary and still

havnt reached the peak then your secondary resonant frequency

is a bit too low for your chosen primary components.

You can add a couple more primary turns or a little more

primary cap.

If you find that the best tune point occurs with very few primary

turns thats usually an indication of using too large a primary

capacitor. Adding a larger toroid to the secondary can help

a little here.

>

> For HF blocking chokes, I have about 33 turns

>of #22 wire on toroid cores, about 2 inch o.d.

>and 2  inches tall.  Do you suppose this is enough?

I would go for more turns of smaller wire. You dont need

AWG 22. You only need to pass a couple hundred ma at most.

The secondary windings in a 60ma neon are typically AWG 32.

I wound my toroids with 120 turns of AWG 28.

You dont want a nice low loss high Q choke. You want

it lossy and inefficent and low Q.

You want to get at least 2 mh of inductance to be effective.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Power vs tunning

Cc:

Bcc:

X-Attachments:

At 06:08 PM 12/10/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Wed, 10 Dec 1997 12:08:58 -0600

>Subject: Power vs tunning

>Organization: MailExcite  (http://www.mailexcite.com)

>

>

>I am ready to power up my coil and I have a few

>questions reguarding power levels, capacitance

>and tuning. My inital power level will be 500 w or less.

>I have 4 .026 mf caps that I can configure to accomodate

>power levels up to 3kw. I will also use a variac to ease my

>way into each power setting. My question is:

>1.) Will a change in coupling occur as the power /

>    capacitance increases ?

The coupling wont change but an overcoupling condition that isnt

visible at low power can begin to show up as the power level goes up.

This manifests itself as corona between the pri/sec and actual

arcing in worse cases. Simply raise up the secondary a bit away

from the primary to reduce coupling if arcing occurs.

>2.) Will the Rf of the primary change as power /

>    Capacitance increases ?

Not sure I understand your question.

Will the RF level change? Certainly. As you apply more power

you want it to become RF energy.

Will the RF frequency change?

Not as a result of power increases but it will certainly change

as a result of changing the primary capacitance.

This also limits the amount of capacitance you can use. You have to

keep the primary circuit tuned to the same frequency as the secondary.

You just cant put on a large capacitor and still expect to tune to

the same frequency.

>3.) Should the primary taps change with increased

>    power ?

Once you find the proper tune it wont change very much.

The actual operating frequency will usually drop a bit under

power as the cloud of ionized air forms around the toroid.

This ion cloud adds capacitance to the system dropping the

secondary resonant frequency slightly.

>4.) I will begin testing this weekend using a flat

>    wound spiral made from 1/4" refrigerator tubing,

>    will the coupling change with the addition of

>    power.

Yes slightly as I indicated above. You most likely

wont have any over coupling problems with your flat spiral

primary. You are more likely to find it doesnt couple

well enough for maximum output. But it will work fine

to get you up and running. Then you can see just what

size and shape of primary you need.

Good luck.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Variac's

Date: Wed, 10 Dec 1997 03:59:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Wed, 10 Dec 1997 03:59:36 +0000

Subject: Variac's

Is a Variable transformer the same thing as a variac?  I just got the

latest MCM sales flyer and they have such a beast, 0-132VAC @ 2.5A,

for $54.95.  God price for new with a 1 year warranty?

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Variac's

Cc:

Bcc:

X-Attachments:

At 03:59 AM 12/10/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Wed, 10 Dec 1997 03:59:36 +0000

>Subject: Variac's

>

>Is a Variable transformer the same thing as a variac?  I just got the

>latest MCM sales flyer and they have such a beast, 0-132VAC @ 2.5A,

>for $54.95.  God price for new with a 1 year warranty?

>

>Thanks. . .CF

>

A 2.5 A variac is really pretty small. While it can probably

handle 4 or maybe 5 amps intermittently thats still pretty

small.

A single 15kv/30ma neon will draw over 5 amps.

You want to get a 10 amp variac at the very least.

Some show up in various coilers hands from time to time.

Ask around. Hamfests are a good source for variacs at good prices.

Failing that try one of the surplus houses. C and H usually

has variacs.

C and H Sales

2176 E. Colorado Blvd

Pasadena, Ca 91107

1-800-325-9465

email c&h@thegrid.net

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: trouble with e-mail

Date: Wed, 10 Dec 1997 18:09:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 10 Dec 1997 12:09:56 -0600

Subject: trouble with e-mail

Organization: MailExcite  (http://www.mailexcite.com)

I seem to be having trouble with my e-mail, please

forgive me if I have sent any duplicated messages

to Tesla2

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: How do I tune it?

Date: Wed, 10 Dec 1997 21:05:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 10 Dec 1997 15:05:18 -0600

Subject: How do I tune it?

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I am about to fire up the coil for the first time.

Any advice on how to tune it?  What signs am

I looking for?

For HF blocking chokes, I have about 33 turns

of #22 wire on toroid cores, about 2 inch o.d.

and 2  inches tall.  Do you suppose this is enough?

Thanks!

-Bill

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: capacitor box

Date: Thu, 11 Dec 1997 22:50:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 11 Dec 1997 17:50:49 -0500

Subject: capacitor box

what do some of you use as a container for a stack cap?

I am planning to build a fairly large cap

my poly sheets will be 12 x 20" stacked to 12 -15"

I considered making a plexiglass box -but what to bond it with?

if i dont find something better I may use a large cooler.

It already has handles and is non conductive but it looks funky.

I'm aiming for 2 caps in series in the same box for

a total of .15uF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitor box

Cc:

Bcc:

X-Attachments:

At 10:50 PM 12/11/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Thu, 11 Dec 1997 17:50:49 -0500

>Subject: capacitor box

>

>what do some of you use as a container for a stack cap?

Ive used several things with differing amounts of success.

I put an early plate cap in a 1.25 gallon rubbermaid container.

This wasnt too bad but it wasnt square so the plates had to be

made to fit the container.

I later moved an expanded form of this same cap to

a plywood tank with a plexglass inner liner. The plexiglass

was cemented to the interior with RTV silicone and a solid

bead of RTV was run along all seams. A plexiglass cover

is attached with small screws. This has worked out very

well and has not leaked. The advantage here is you can

make your tank just the size you want.

My latest tanks are also of plywood but Ive substituted

2 coats of polyester resin for the plexglass liner. This

is considerably less expensive and less work. This has

worked out well and have had no leaks thus far.

>I am planning to build a fairly large cap

>my poly sheets will be 12 x 20" stacked to 12 -15"

>I considered making a plexiglass box -but what to bond it with?

A plexiglass box will have to be fairly thick to

be strong enough on its own. I used inexpensive 3/32

material inside my plywood tanks.

Plexiglass can be bonded with methylene chloride type

solvents or plain epoxy does well.  Plexiglass can tend

to be brittle so you may want to try Lexan instead.

>if i dont find something better I may use a large cooler.

>It already has handles and is non conductive but it looks funky.

>I'm aiming for 2 caps in series in the same box for

>a total of .15uF

I hope you mean .015 mfd and not .15 mfd.

.15 mfd is a very large capacitor and much larger than

you need for a Tesla coil.

What size transformer do you have? That pretty much determines

the size of cap you need.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Variac's

Date: Thu, 11 Dec 1997 14:43:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Thu, 11 Dec 1997 08:43:40 -0600

Subject: Re: Variac's

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Wed, 10 Dec 1997 03:59:36 +0000

> Subject: Variac's

>

> Is a Variable transformer the same thing as a variac?  I just got the

> latest MCM sales flyer and they have such a beast, 0-132VAC @ 2.5A,

> for $54.95.  God price for new with a 1 year warranty?

>

> Thanks. . .CF

Chris:

Almost the same. A Variac is a variable AUTO transformer. My guess is,

that whoever is talking about a variable transformer really means auto

transformer. Hope this helps.

Eric

<edavidson@icva.gov>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Bypass caps and Neons *NEEDED!*

Cc:

Bcc:

X-Attachments:

At 03:35 AM 12/12/97 GMT, you wrote:

>From: Michael Baumann <baumann@proton.llumc.edu>

>Date: Thu, 11 Dec 1997 19:35:08 -0800 (PST)

>Subject: Bypass caps and Neons *NEEDED!*

>

>Well.. I just got the answer to my bypass question tonight.

>I was doing 90% power tests of my new cap tonight, and decided

>to see what would happen if I did not have bypass caps.

>Results:

>1)Safety Gap firing almost constantly.

>2) A couple of really scary arcs from my choke to my resistor.

>

>The Cap is .0061 mf, with a 12KV 30ma transformer.

>

>I decided to put the caps back in, and the whole setup settled

>right down.

>

>I also need to open the safety gap across the cap, but am not exactly sure

>how I should set that.

>--

Sounds to me that maybe your chokes arent doing their job well

if you still have that much RF at your neon.

I get very litle difference with or without bypass caps so

I choose to go without.

In order to prevent any problems with bypass caps use a small value.

500 pf is way too big. Somewhere around 100 pf would be fine.

If bypasses are working for you then by all means use them.

Im all for anything that works.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitor box+transfo

Cc:

Bcc:

X-Attachments:

At 03:13 AM 12/12/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Thu, 11 Dec 1997 22:13:08 -0500

>Subject: Re: capacitor box+transformers

>

>

>

>----------

>>

>> At 10:50 PM 12/11/97 GMT, you wrote:

>> >From: "Chris Watkins" <cwatkins@seidata.com>

>> >Date: Thu, 11 Dec 1997 17:50:49 -0500

>> >Subject: capacitor box

>> >I'm aiming for 2 caps in series in the same box for

>> >a total of .15uF

>>

>> I hope you mean .015 mfd and not .15 mfd.

>> .15 mfd is a very large capacitor and much larger than

>> you need for a Tesla coil.

>>

>> What size transformer do you have? That pretty much determines

>> the size of cap you need.

>>

>> TTYL!

>>

>> Mike Hammer

>

>my transformer's are 4 microwave units wired with the secondaries

>in series.  the output is 7750v at estimated .5a+ with 120v input per unit.

>the secondary lead's have been removed from the cores and all are in oil.

>this is to power a 10.76" secondary w 800t 18ga wire.

Ah. Yes I remember that now. You will need that much capacitance

for your MW trannies.

>

>I actually calculated that I could use more cap's but want to start with

>.15uF. If I am wrong please let me know . I would hate to do all that work if

>unnecessary.

>

>0.15 mfd may not be the best choice. Depends a great deal

on the actual impedance of the transformers. Experimentation

is called for.

>I am still unsure if I can input  220 v to these correctly

>if so I could use less cap and less current draw per line.

Wire the transformers in pairs. Each pair should have its

primary windings in series and run from 220V. Then wire the

two pairs in parallel. Watch for proper phasing.

>

>is the primary tied to the core on microwave xformers?

No.

>

>I have input 220 into them but it seems to be a tremendous  load on

>my variac with only a 6 pack capacitor

>(granted my variac is too small, but a larger one is on its way!)

>

MW oven transformers are hugely inefficient. You are probably

draw some really high current. A bit of inductive ballast

would help by limiting the current.

>I am in process of a multiple RQ gap to handle it.

>comments appreciated.

>

MW oven transformers are going to be difficult.

Think heavy duty when building your gap. You want large

electrodes and LOTS of airflow.

I would think in terms of 1 inch copper pipe and 4 inch

long electrodes at a minimum.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 25 AWG

Cc:

Bcc:

X-Attachments:

At 03:20 PM 12/12/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Fri, 12 Dec 1997 09:20:13 -0600

>Subject: 25 AWG

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>  I have been given several pounds of 25 AWG magnet

>wire. I know that this wire is pretty light duty. I

>am thinking of winding a 4" diameter coil using it.

>Does anyone have a feel for what the winding length should be ?

>

>Thanks,

>

>Paul S. Marshall

>

Paul,

For a 4 incher Ive always recommend from 20-22 inches depending

on whether you used AWG 22 or AWG 24. The preferred wire for

a 4 incher is 24.

With AWG 25 you will be getting around 52 turns per inch

so an 18 inch winding will yield about 935 turns and a resonant

frequency of around 390 KHZ unloaded.

If you want a lower frequency coil then go with a 20 inch winding

of approximately 1036 turns and an unload resonant frequency of

362 KHZ.

I wouldnt go any farther than 20 inches. Normally anything more

than 1000 turns is just wasted wire.

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor box

Date: Thu, 11 Dec 1997 23:59:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Thu, 11 Dec 1997 18:59:15 -0500

Subject: Re: capacitor box

Cap box.  I too have always found this somewhat of a challenge.  However it can

be overcome.  I would say go for the cooler...those handles will come in handy

once it's full of oil.

My cap box is a large plastic container acquired from Wal-Mart.  It is quite la

rge but houses all three of my caps which are 12"x14" each.  Since I had so muc

h dead space to fill in and not enough oil, I have just put bricks in to fill t

he space.  This however has made my cap quite heavy.

Just remember it don't have to look pretty to work!  I would however recommend

a more manageable size of cap than something 20" long.

Good Luck,

Kevin Batchelor

KBATCHELOR@ARSLIMITED.COM

>>> Tesla-2 <tesla-2@emachine.com> 12/11 5:50 PM >>>

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 11 Dec 1997 17:50:49 -0500

Subject: capacitor box

what do some of you use as a container for a stack cap?

I am planning to build a fairly large cap

my poly sheets will be 12 x 20" stacked to 12 -15"

I considered making a plexiglass box -but what to bond it with?

if i dont find something better I may use a large cooler.

It already has handles and is non conductive but it looks funky.

I'm aiming for 2 caps in series in the same box for

a total of .15uF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor box+transfo

Date: Fri, 12 Dec 1997 03:13:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 11 Dec 1997 22:13:08 -0500

Subject: Re: capacitor box+transformers

----------

>

> At 10:50 PM 12/11/97 GMT, you wrote:

> >From: "Chris Watkins" <cwatkins@seidata.com>

> >Date: Thu, 11 Dec 1997 17:50:49 -0500

> >Subject: capacitor box

> >I'm aiming for 2 caps in series in the same box for

> >a total of .15uF

>

> I hope you mean .015 mfd and not .15 mfd.

> .15 mfd is a very large capacitor and much larger than

> you need for a Tesla coil.

>

> What size transformer do you have? That pretty much determines

> the size of cap you need.

>

> TTYL!

>

> Mike Hammer

my transformer's are 4 microwave units wired with the secondaries

in series.  the output is 7750v at estimated .5a+  with 120v input per unit.

the secondary lead's have been removed from the cores and all are in oil.

this is to power a 10.76" secondary w 800t 18ga wire.

I actually calculated that I could use more cap's but want to start with .15uF.

If I am wrong please let me know . I would hate to do all that work if

unnecessary.

I am still unsure if I can input  220 v to these correctly

if so I could use less cap and less current draw per line.

is the primary tied to the core on microwave xformers?

I have input 220 into them but it seems to be a tremendous  load on

my variac with only a 6 pack capacitor

(granted my variac is too small, but a larger one is on its way!)

doing so showed no ill effects to the xformers, but the gap was flaming madly!

I am in process of a multiple RQ gap to handle it.

comments appreciated.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Bypass caps and Neons *NE

Date: Fri, 12 Dec 1997 03:35:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Thu, 11 Dec 1997 19:35:08 -0800 (PST)

Subject: Bypass caps and Neons *NEEDED!*

Well.. I just got the answer to my bypass question tonight.

I was doing 90% power tests of my new cap tonight, and decided

to see what would happen if I did not have bypass caps.

Results:

1)Safety Gap firing almost constantly.

2) A couple of really scary arcs from my choke to my resistor.

The Cap is .0061 mf, with a 12KV 30ma transformer.

I decided to put the caps back in, and the whole setup settled

right down.

I also need to open the safety gap across the cap, but am not exactly sure

how I should set that.

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor box

Date: Fri, 12 Dec 1997 13:29:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 12 Dec 1997 07:29:38 -0600

Subject: Re: capacitor box

Organization: MailExcite  (http://www.mailexcite.com)

Chris,

I am using a pole pig to power my coil at 13,200v

my initial power settings will be between 1 and 3 kw.

My total capacitance will probably not exceed .05 mf.

What kind of a transformer are you using ? My caps

consist of 48 8 x 12 inch sheets immeresed in

transformer oil. For a container I used a ruddermaid

resealable box 6" deep 13 x 9. The poly is .060" thick.

Each cap is approximately .026 mf. 4 caps in each

parallel circuit and 2 parallel circuits are in series.

These caps are very efficient,and should be capable of handling anything I can

throw at them. Of course this system is configuration dependent.

Paul Marshall

---

Einigkeit und Freiheit

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Thu, 11 Dec 1997 17:50:49 -0500

>Subject: capacitor box

>

>what do some of you use as a container for a stack cap?

>I am planning to build a fairly large cap

>my poly sheets will be 12 x 20" stacked to 12 -15"

>I considered making a plexiglass box -but what to bond it with?

>if i dont find something better I may use a large cooler.

>It already has handles and is non conductive but it looks funky.

>I'm aiming for 2 caps in series in the same box for

>a total of .15uF

>

>

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 25 AWG

Date: Fri, 12 Dec 1997 15:20:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 12 Dec 1997 09:20:13 -0600

Subject: 25 AWG

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have been given several pounds of 25 AWG magnet

wire. I know that this wire is pretty light duty. I

am thinking of winding a 4" diameter coil using it.

Does anyone have a feel for what the winding length should be ?

Thanks,

Paul S. Marshall

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave Powered TC

Date: Fri, 12 Dec 1997 15:55:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 12 Dec 1997 10:55:47 -0500

Subject: Re: Microwave Powered TC

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: capacitor box+transfo

> Date: Thursday, December 11, 1997 10:13 PM

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Thu, 11 Dec 1997 22:13:08 -0500

> Subject: Re: capacitor box+transformers

>

>

>

> ----------

> >

> > At 10:50 PM 12/11/97 GMT, you wrote:

> > >From: "Chris Watkins" <cwatkins@seidata.com>

> > >Date: Thu, 11 Dec 1997 17:50:49 -0500

> > >Subject: capacitor box

> > >I'm aiming for 2 caps in series in the same box for

> > >a total of .15uF

> >

> > I hope you mean .015 mfd and not .15 mfd.

> > .15 mfd is a very large capacitor and much larger than

> > you need for a Tesla coil.

> >

> > What size transformer do you have? That pretty much determines

> > the size of cap you need.

> >

> > TTYL!

> >

> > Mike Hammer

>

> my transformer's are 4 microwave units wired with the secondaries

> in series.  the output is 7750v at estimated .5a+   with 120v input per

> unit.

> the secondary lead's have been removed from the cores and all are in oil.

> this is to power a 10.76" secondary w 800t 18ga wire.

>

> I actually calculated that I could use more cap's but want to start with

> .15uF.

> If I am wrong please let me know . I would hate to do all that work if

> unnecessary.

>

> I am still unsure if I can input  220 v to these correctly

> if so I could use less cap and less current draw per line.

>

Putting in 220 is somewhat iffy. The cores are already undersized

if you know what I mean, and I experienced saturation when I tried

the same experiment about a year ago. The primary started to heat up

excessively.... but these effects will vary depending on the

particular transformer you are using. An oil immersed unit is

more robust, as the oil helps distribute the heat by convection.

However, personally I would not push the microwave trannies

by overvolting the secondaries that much. When the things are

actually driving a Tesla coil there will be some additional stresses

to deal with as it is. Give your units a fighting chance to survive!

> is the primary tied to the core on microwave xformers?

>

Not in any of the ones I have worked with.

> I have input 220 into them but it seems to be a tremendous  load on

> my variac with only a 6 pack capacitor

> (granted my variac is too small, but a larger one is on its way!)

>

Cores are really undersized, and the current waveforms on these

trannies is NOT sinusoidal *as-is*. Don't push them too far or

they will self-destruct, oil or not.

> doing so showed no ill effects to the xformers, but the gap was flaming

> madly!

> I am in process of a multiple RQ gap to handle it.

> comments appreciated.

>

The gap must be capable of very high current levels, especially

with the size cap you are using. Make the copper tubes at least

4 inches long. This increased surface are will help prevent

excessive heat buildup. Use a vacuum cleaner motor instead of a

muffin fan, and if I were you I would build a full-blown vacuum

gap. With the large caps you need all the quenching help you can get.

Place the tubing sections parallel and close together, and use

many sections. That will help you a lot, as the voltage is somewhat

small, and you will be relying on resonant rise to get the voltage

up to firing potential.

Another plus would be a variable speed rotary gap. That would allow

you to use smaller caps and still process more energy. The voltage

at the gap will be somewhat smallish, however, so the gap

distance must be set small.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First Coil

Cc:

Bcc:

X-Attachments:

At 08:59 PM 2/28/97 GMT, you wrote:

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Fri, 28 Feb 1997 14:59:54 -0600

>Subject: First Coil

>

>Hello everyone, my name is Mike White, I am new on the list.

Welcome aboard Mike.

>I recently started aquiring components for my first coil.

>I currently have 4-15kv 30 ma and a 7.5kv 30 ma and many half working

>or dead transformers. My current plan is to build a coil that I can

>learn with, but is somewhat upgradable. I would like to start with one transformer

>and work my way up to 3 or so. If possible I would like to use the same coil or

>most of it anyway. I know that I will have to build more caps. If anyone has any

>suggestions about the secondary size to use I would appreciate the advice.

Well Mike I think a secondary wound on 4 inch PVC pipe is just about

the perfect size. It can operate well at low powers from a single neon

and can handle 1500 watts or more with good design. My own 4 incher

delivers 50 inch sparks at around 1200 watts and yet I have run it

quite easily with a 7500V/30 ma neon.

Use AWG 24 for your secondary and a winding length of 20-21 inches.

>Another question I have is can I use the primary winding of a 15kv 30

>ma transformer as a choke or does it need more resistance I estimate the

>number of wraps as about 300. If so does it need a core. I have several

>laying around from unpotted neons.

If you mean as a RF choke then I would say no. It will probably

have more inductance than needed and it will have considerable

impedance at 60 hz. What you want in a RF choke is virtually

no impedance at 60 hz but a high impedance to the RF being

generated in the primary circuit.

Wind 100 or more turns of small guage wire (AWG 26 or so) on

a large ferrite toroid or rod.

Or wind 350 turns of AWG 26 on 2 inch PVC pipe for an air core choke.

This will result in about 2.5 mh of inductance.

Any value from 2-10 mh will work fine.

> One of the difficulties that I know I will have is I live in a

>apartment. With a good ground to a water pipe, Could I operate a coil at

>low power levels 20% or so, without causing my neighbors too much

>discomfort. I only want to get it set up correctly here and then I will

>take it somewhere else to let it rip. I dont know if they have a

>computer or not, but mine is only one wall and 20ft away. If anyone can

>help me with some advice I would be greatful.

The only way to know for sure is to give it a try. I would attempt

to provide my own ground rather than trying a water pipe. Very often

water pipes are isolated from ground by plastic plumbing or the water

meter itself.

Best insurance for your computer is to unplug it whenever you run

your coil. This should include the telephone line to your modem.

Tesla coils and modern electronics do not mix well.

TTYL!

Mike Hammer

mhammer@misslink

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: capacitor box

Date: Sat, 13 Dec 1997 20:13:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Steve Falco <sfalco@worldnet.att.net>

Date: Sat, 13 Dec 1997 15:13:10 -0500

Subject: Re: capacitor box

>

> My latest tanks are also of plywood but Ive substituted

> 2 coats of polyester resin for the plexglass liner. This

> is considerably less expensive and less work. This has

> worked out well and have had no leaks thus far.

When you say polyester resin, what exactly do you mean?

I will be building a cap box soon too, and I'd like to

use something known to work.  A brand name would be

helpful.  (Don't know if you mean a poly varnish or

something more like the Behr Build-50, or something

else entirely.)

Steve Falco

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: capacitor box

Cc:

Bcc:

X-Attachments:

At 08:13 PM 12/13/97 GMT, you wrote:

>From: Steve Falco <sfalco@worldnet.att.net>

>Date: Sat, 13 Dec 1997 15:13:10 -0500

>Subject: Re: capacitor box

>

>>

>> My latest tanks are also of plywood but Ive substituted

>> 2 coats of polyester resin for the plexglass liner. This

>> is considerably less expensive and less work. This has

>> worked out well and have had no leaks thus far.

>

>When you say polyester resin, what exactly do you mean?

>I will be building a cap box soon too, and I'd like to

>use something known to work.  A brand name would be

>helpful.  (Don't know if you mean a poly varnish or

>something more like the Behr Build-50, or something

>else entirely.)

>

>        Steve Falco

>

Steve,

I used polyester resin like you would use for

making fiberglass. This is the resin that uses MEKP

as the hardening agent.

Its quite inexpensive by the quart. Much less so than

epoxy resins like Behr Build-50.

As far as a brand name the stuff I used was called

Mr. Fix Fiberglass Resin. It was purchased for

$3 per quart from a local discount store.

By the way, polyester resin like this is NOT suitable

for coating a secondary coil. Far too brittle and tends

to crack and come loose probably due to differences in thermal

expansion.

It has worked out well for the tank of my rebuilt neon transfomer.

I hope to have some scans of this on my web site soon.

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Coil

Cc:

Bcc:

X-Attachments:

At 02:14 PM 12/15/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>.com>

>Date: Mon, 15 Dec 1997 08:14:32 -0600

>Subject: New Coil

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

>  I had some real success this weekend. Let me give you

>the vitals.:

>

>4" dia. Coil wound with 24 gage wire, winding length 22.5"

>

>Richard Quick style gap, muffin fan quenched.

>Primary: Flat wound 1/4" copper tube 12 turns.

>Three caps, two in parallel the third in series with

>the other two. Total capacatance .015uf. Bill Pollack

>came over and pitched in. Bill worked as the tuning

>genius while I handled the power supply. We started

>out with the neon runnung at 50 % power 7.5 kv at

>120 ma. we used a basket ball as a top terminal. when

>tuned we were getting 25" sparks. We then switched

>over to our pole pig. 13.2 kv and ran at the low

>current setting (1500 w). This gave us 29 - 30 "

>sparks. Late yesterday I went to HQ and purchased some

>4" dryer duct. From this I made a toroid. (classic-

>2 pie pans and dryer duct) From this configuration

>I'm getting 35" sparks. Some of the arcs actually hit

>the basement floor. I think I still have a way to go

>before it is optmized (tuned). I have one question

Paul and Bill,

Nice going guys. Great results from a new coil.

>

>What should I try next ?

>

>New primary

Depends on the amount of coupling you are getting with

your present primary. Operate in a very dark room and

look for corona between primary and secondary.

If you have corona then you are getting plenty of coupling.

Raise the secondary slightly until corona just stops.

If you see no corona increase coupling by moving secondary

closer to primary.

The setting of maximum coupling with a flat spiral primary

is when the bottom turn of the secondary is even with the plane

of the primary turns.

If you are already at maximum coupling and feel you need more

then the only remedy is a new primary. A saucer shaped one with

a 15-30 degree slope would be best.

>more capacitance

Yes but only if you dont lose too many primary turns.

With all the power you have and a static gap you need

a high primary inductance to insure good gap operation.

Gap losses are probably your greatest source of loss at this

point.

Add more primary capacitance but balance that with additional

secondary capacitance in the form of a larger toroid so you

dont lose primary inductance.

With the power you are generating you will really benefit

from a toroid with a larger cross section. This will allow

higher voltages to build up before spark break out.

Go for 6 inch duct and work it into about a 20 inch diameter

ring. You can probably still use the pie pans from the smaller

toroid.

>better tuning

Always try to refine your tuning. Getting the spark you are getting

you must be very close if not right on the money.

As Im sure you can tell more power is not the answer at this point.

If you doubled your input power you may only get a couple more

inches of spark. You have plenty of power already. What you need

is to utilize what you have.

Make sure your tank circuit wiring is short and direct.

Use heavy leads as the RF currents are extremely high.

Check your primary coil tap for signs of heating.

At these power levels a quality RF ground is very

important. Make that ground lead as short and straight as

possible. Make it from quite heavy conductors.

And the most important thing to do is to get pictures!

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 35 inch sparks

Cc:

Bcc:

X-Attachments:

At 09:01 PM 12/15/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Mon, 15 Dec 1997 15:01:56 -0600

>Subject: 35 inch sparks

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Coilers,

> I misinterpreted what Paul told me about his tests,

>he experienced arcs from the primary to the

>secondary when he RAISED the primary up closer

>to the secondary.

>

>This seems to make sense, as the coupling would be

>increased.  I haven't seen this particular experiment

>yet.

Yes this does make more sense and is a good indication

that you are getting adequate coupling with your flat

spiral primary. If this is the case then there is no

reason to change primary shapes. Nothing to be gained.

Adjust coupling as tight as possible without having heavy

corona or primary secondary arcing. Having just a faint

trace of light corona is just fine.

>

>Last I heard - Paul was going out to do a bit of holiday

>shopping - he's buying himself some 6 inch dryer duct,

>among other things!  Personally I think he might need

>a bigger house to contain this coil before too long.

>

Nah. Just ask Jim Buck who regularly fires his 10 inch

3 KW coil in his kitchen and bounces streamers off the walls

and ceiling.

>Thanks to all the list members who have helped us

>out with this coil.  Particularly to Mike Hammer - who

>basically fed us his recipe for success!

Well its not really my recipe. I only copied and used

what others before me had done.

>

>I might wind up another secondary - this time maybe

>a bit shorter - and leave all other things the same.  I

>still have more than enough of the exact same sort

>of wire (24 g).

The important thing is that the primary circuit resonate

at the same frequency as the secondary. All other considerations

are of less importance.

>

>-Bill the arcstarter

>

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ay Carumba! 35 inch arcs!

Cc:

Bcc:

X-Attachments:

At 04:48 PM 12/15/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Mon, 15 Dec 1997 10:48:14 -0600

>Subject: Ay Carumba! 35 inch arcs!

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Just thought I'd add my comments on top of Paul's:

>

>>Coilers,

>>  I had some real success this weekend. Let me give you

>>the vitals.:

>>

>>4" dia. Coil wound with 24 gage wire, winding length

>>22.5"

>

>Our winding length might be closer to 26 inches,

>as I purposly over-wound, thinking we pull some off,

>which we never did.  Perhaps 20-21 inches would

>be better?

Removing turns will raise the secondary resonant frequency.

You will have to tap in less primary turns to match.

I wouldnt.

The most important consideration is that the primary and secondary

resonate at the same frequency. All other considerations

are less important.

If you find your frequency becoming to low to tune with the

addition of a larger toroid then yes remove some of those

extra turns.

>

>>

>>Richard Quick style gap, muffin fan qunched.

>

>We ended up using all 10 gaps, at 0.028 inche each.

>I suspect that the resonance between the neon and

>the cap allowed up to reach this voltage as the

>neon by itself would not fire that many gaps.

>

>This might be dangerous for the neon??

.280 inch total gap is not too bad for 15kv.

You wouldnt want anymore thats for certain.

>

>We did have a safety gap and series inductance

>which did fire on occasion.

Narrow that safety gap so its firing more than

just on occasion.

>

>>Primary: Flat wound 1/4" copper tube 12 turns.

>>Three caps, two in parallel the third in series with

>>the other two. Total capacatance .015uf.

>

>These are our 0.060 poly and foil caps, immersed in

>the surplus diala-ex I acquired from work!

>

>> Bill Pollack

>>came over and pitched in. Bill worked as the tunning

>>genius while I handled the power supply. We started

>>out with the neon running at 50 % power 7.5 kv at

>>120 ma.

>

>We first used an old pot lid, about 10 inchs diameter

>for a top terminal.  Set primary for 13 turns, and

>0.026 uf cap.  Only 1 inch discharge to a grounded wire.

>

>Reduced primary turns inward, eventually stopping

>at 3 turns.  Were getting some corona, and perhaps

>8 inche arcs to a grounded wire.

>

>We then put another cap in series to cut down the

>total capacitance, and set the turns back to about

>10 turns.  This enabled up to pull perhaps 20

>inches of arc from the pot lid to a grounded wire.

>

Here is a good illustration of what Ive been trying to say.

A large cap will deliver more energy per bang but its no

good unless it resonates with the primary at the secondary

resonant frequency.

<SNIP>

>>

>>What should I try next ?

>>

>>New primary

>>more capacitance

>>better tuning

>

>Personally I think we can get better results if we

>'harden' some of our primary circuit wiring.  We are

>presently using stranded 14-g lamp cord and

>alligator clips, which I suspect eats up some power.

>Using solid 12 or 10 gauge seems appropriate.

Yes absolutely. And dont forget that ground lead.

The RF currents in the tank circuit can be very high.

Certainly tens of amperes and maybe more.

I2R losses can mount up.

<SNIP>

>

>-Bill Pollack - the arcstarter.

>

Well done guys. Looks like you will be surpassing me in no time.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: New Coil

Date: Mon, 15 Dec 1997 14:14:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 15 Dec 1997 08:14:32 -0600

Subject: New Coil

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I had some real success this weekend. Let me give you

the vitals.:

4" dia. Coil wound with 24 gage wire, winding length

22.5"

Richard Quick style gap, muffin fan qunched.

Primary: Flat wound 1/4" copper tube 12 turns.

Three caps, two in parallel the third in series with

the other two. Total capacatance .015uf. Bill Pollack

came over and pitched in. Bill worked as the tunning

genius while I handled the power supply. We started

out with the neon runnung at 50 % power 7.5 kv at

120 ma. we used a basket ball as a top terminal. when

tuned we were getting 25" sparks. We then switched

over to our pole pig. 13.2 kv and ran at the low

current setting (1500 w). This gave us 29 - 30 "

sparks. Late yesterday I went to HQ and purchased some

4" dryer duct. From this I made a toroid. (classic-

2 pie pans and dryer duct) From this configuration

I'm getting 35" sparks. Some of the arcs actually hit

the basement floor. I think I still have a way to go

before it is optmized (tuned). I have one question

What should I try next ?

New primary

more capacitance

better tuning

suggestions appriciated

Thanks,

Paul & Bill

-Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

From: tesla-2@emachine.com (Tesla-2)

Subject: tuning??

Date: Tue, 16 Dec 1997 05:33:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: MIKE drew <gordrew@home.com>

Date: Mon, 15 Dec 1997 21:33:58 -0800

Subject: tuning??

I have heard this mentioned several time but cannot get a clear definition

on what it is, and how to do it.

Any help would be much apreciated.

--MIKE

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Ay Carumba! 35 inch arcs!

Date: Mon, 15 Dec 1997 16:48:14 GMT

Organization: The Energy Machine Information System 504-733-3462

mhammer@misslink.net

From: arcstarter@mailexcite.com

Date: Mon, 15 Dec 1997 10:48:14 -0600

Subject: Ay Carumba! 35 inch arcs!

Organization: MailExcite  (http://www.mailexcite.com)

Just thought I'd add my comments on top of Paul's:

>Coilers,

>  I had some real success this weekend. Let me give you

>the vitals.:

>

>4" dia. Coil wound with 24 gage wire, winding length

>22.5"

Our winding length might be closer to 26 inches,

as I purposly over-wound, thinking we pull some off,

which we never did.  Perhaps 20-21 inches would

be better?

>

>Richard Quick style gap, muffin fan qunched.

We ended up using all 10 gaps, at 0.028 inche each.

I suspect that the resonance between the neon and

the cap allowed up to reach this voltage as the

neon by itself would not fire that many gaps.

This might be dangerous for the neon??

We did have a safety gap and series inductance

which did fire on occasion.

>Primary: Flat wound 1/4" copper tube 12 turns.

>Three caps, two in parallel the third in series with

>the other two. Total capacatance .015uf.

These are our 0.060 poly and foil caps, immersed in

the surplus diala-ex I acquired from work!

> Bill Pollack

>came over and pitched in. Bill worked as the tunning

>genius while I handled the power supply. We started

>out with the neon running at 50 % power 7.5 kv at

>120 ma.

We first used an old pot lid, about 10 inchs diameter

for a top terminal.  Set primary for 13 turns, and

0.026 uf cap.  Only 1 inch discharge to a grounded wire.

Reduced primary turns inward, eventually stopping

at 3 turns.  Were getting some corona, and perhaps

8 inche arcs to a grounded wire.

We then put another cap in series to cut down the

total capacitance, and set the turns back to about

10 turns.  This enabled up to pull perhaps 20

inches of arc from the pot lid to a grounded wire.

Next:

>we used a basket ball as a top terminal. when

>tuned we were getting 25" sparks. We then ...

The basketball was covered in foil.  We had

to tap in more primary turns due to the increased

capacitance (?) of the top terminal!

>over to our pole pig. 13.2 kv and ran at the low

>current setting (1500 w). This gave us 29 - 30 "

>sparks. Late yesterday I went to HQ and purchased some

>4" dryer duct. From this I made a toroid. (classic-

>2 pie pans and dryer duct) From this configuration

>I'm getting 35" sparks. Some of the arcs actually hit

>the basement floor. I think I still have a way to go

>before it is optmized (tuned). I have one question

>

>What should I try next ?

>

>New primary

>more capacitance

>better tuning

Personally I think we can get better results if we

'harden' some of our primary circuit wiring.  We are

presently using stranded 14-g lamp cord and

alligator clips, which I suspect eats up some power.

Using solid 12 or 10 gauge seems appropriate.

Paul tells me he did try raising the secondary up,

away from the primary.  This caused lots of arcing

bewteen the secondary windings, etc.

I'm not sure why raising the secondary caused this

arcing, as I would have thought this would reduce

coupling and therefore power transfer.  Any comments??

All in all we had a bast getting this thing going and

we are far from done fooling with it!!

-Bill Pollack - the arcstarter.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: tuning??

Cc:

Bcc:

X-Attachments:

At 05:33 AM 12/16/97 GMT, you wrote:

>From: MIKE drew <gordrew@home.com>

>Date: Mon, 15 Dec 1997 21:33:58 -0800

>Subject: tuning??

>

>

>I have heard this mentioned several time but cannot get a clear definition

>on what it is, and how to do it.

>

>Any help would be much apreciated.

>

>--MIKE

>

Mike,

A Tesla coil is composed of two resonant circuits.

The secondary inductance and stray capacitance plus

any top load capacitance (toroid) form one resonant circuit.

The primary coil along with the main capacitor form the other

resonant circuit.

Both of the circuits MUST resonate at the SAME frequency for

a Tesla coil to operate. Its the resonant circuits that are

resposible for the spectacular voltage rise.

Tuning is the selection of component values to adjust

these resonant frequencies. This is most often accomplished

by varying the inductance of the primary coil with a movable

tap. The secondary resonant frequency can be adjusted by

changing the size of the toroid.

When tuning a Tesla coil we use the spark output

as a visible indicator. You select different taps

on the primary coil looking for the greatest spark output.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: 35 inch sparks

Date: Mon, 15 Dec 1997 21:01:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Mon, 15 Dec 1997 15:01:56 -0600

Subject: 35 inch sparks

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I misinterpreted what Paul told me about his tests,

he experienced arcs from the primary to the

secondary when he RAISED the primary up closer

to the secondary.

This seems to make sense, as the coupling would be

increased.  I haven't seen this particular experiment

yet.

Last I heard - Paul was going out to do a bit of holiday

shopping - he's buying himself some 6 inch dryer duct,

among other things!  Personally I think he might need

a bigger house to contain this coil before too long.

Thanks to all the list members who have helped us

out with this coil.  Particularly to Mike Hammer - who

basically fed us his recipe for success!

I might wind up another secondary - this time maybe

a bit shorter - and leave all other things the same.  I

still have more than enough of the exact same sort

of wire (24 g).

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New record

Cc:

Bcc:

X-Attachments:

At 02:46 PM 12/16/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>.com>

>Date: Tue, 16 Dec 1997 08:46:14 -0600

>Subject: New record

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Fellow coilers,

>

>Last night I replaced my top terminal which was made

>from 4" to a new terminal made from 6" duct. The

>results were dramatic. Streamers struck everything.

>they hit the floor, the workbench, the primary, even

>a transformer across the room. Some of the strikes

>measure over 40". It was spectactular ! I think Bill

>is going to try and take some pictures.

Sounds great. I definitely want to see the pictures.

If you are getting strikes down into the primary make sure

you have a grounded strike ring. This is an open loop placed

above and slightly outside the last primary turn. This is

used to provide a place for the strikes to go safely to ground

instead of into your primary circuit.

Make sure this is an open loop. A closed loop represents a

shorted turn and can really suck up the energy.

>

>Note:The phone upstairs in the bedroom is electronic,

>it is on the same circuit with the coil. Whenever I

>run the coil the phone upstairs rings and the digital

>clock resets its self. I think I'll re-wire the tank

>circuit tonight. I bought some 10 guage wire last night.

You have RF chokes on the HV supply lines but what about

a line filter on the AC side? I use a commercial AC line filter

in between my transformers and variac.

Suitable filters can be purchased very cheaply from surplus

places like MECI. I got mine for $1 at a hamfest.

This will help a great deal in keeping RF out of the AC mains.

Running the coil on its own separate AC circuit is advisable.

>

>Note: The safety gap on the neon fires regularly, the

>safety gap on the pig never fires. Any Ideas ? I've

>got it set pretty close, .312"- .375". It did fire once

>when I used the 6" diameter duct. It was a very thin

>blue arc.

Lets see. Your neon is 15kv and the pig runs 13.2 KV

doesnt it? That could be part of it. Plus you are most likely

seeing some 60 hz resonance effects with the neon that you

wont see with the pig. Both of those could mean higher

primary circuit voltages than you think.

Dont remove the safety when using the pig. Even though it

doesnt fire often it is there in case of a problem. Your

pig isnt likely to get damaged but it can happen. They

are not indestructible.

>

>Paul Marshall

>---

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New record

Cc:

Bcc:

X-Attachments:

At 06:01 PM 12/16/97 GMT, you wrote:

>From: "Alfred C. Erpel" <aerpel@Op.Net>

>Date: Tue, 16 Dec 1997 13:01:07 -0500

>Subject: Re: New record

>

>

<SNIP>

>Paul,

>

>    I'll venture my opinion.

> so far in all my reading on Tesla coils, I've yet to see someone say

>"... and when I took my large toroid off and put the smaller one on, the

>coils performance got much better." But I've seen lots of people see

>improvement with a larger toroid/top load.  I believe this is because, in

>general, the natural frequency of the secondary coil system as determined

>by 1 / 2 pi sqrt(LsCs) on most coils turns out to be much higher than the

>1/4 wave frequency as determined by the number of feet of coil on the

>secondary.

>So when you increase the capacitance of the secondary coil system, you

>may have brought its resonating frequency closer to its 1/4 wave frequency

>and this must be important.

>    If anyone knows better, please correct me.

>

>

>Tsch=FCs,

>Alfred

>

Alfred,

You are partially correct. The natural resonant frequency of the

secondary is determined by 1 / 2 pi sqrt(LsCs) as you said.

And the addition of a top load does decrease that resonant

frequency because of the additional capacitance.

But the 1/4 wave frequency of the length of wire is irrelevant.

That relationship is no longer valid once the wire is wound into

a coil. Trying to get a coil to resonate at the 1/4 wave frequency

of its stretched out wire is a waste of time.

The increased performance from the larger toroid is due to

3 main reasons.

First the increased cross section (from 4 to 6 inches in Pauls case)

allows a higher voltage to build up before a spark breaks out.

Higher voltage means longer sparks.

Second the increased capacitance of the toroid acts like any

good capacitor should. By storing charge. Charge up any cap

and discharge it quickly. What do you get? A brighter stronger

spark with more current in it. This increases spark length by

increasing streamer current and more strongly ionizing the spark

channel.

Third the lower resonant frequency due to the increased capacitance

allows for longer sparks by reducing losses.

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: New record

Date: Tue, 16 Dec 1997 14:46:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

.com>

Date: Tue, 16 Dec 1997 08:46:14 -0600

Subject: New record

Organization: MailExcite  (http://www.mailexcite.com)

Fellow coilers,

Last night I replaced my top terminal which was made

from 4" to a new terminal made from 6" duct. The

results were dramatic. Streamers srtuck everything.

they hit the floor, the workbench, the primary, even

a transformer across the room. Some of the strikes

measure over 40". It was spectactular ! I think Bill

is going to try and take some pictures.

Note:The phone upstairs in the bedroom is electronic,

it is on the same circuit with the coil. Whenever I

run the coil the phone upstairs rings and the digital

clock resets its self. I think I'll re-wire the tank

circuit tonight. I bought some 10 gage wire last night.

Note: The safety gap on the neon fires regularly, the

safety gap on the pig never fires. Any Ideas ? I've

got it set pretty close, .312"- .375". It did fire once

when I used the 6" diameter duct. It was a very thin

blue arc.

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New record

Date: Wed, 17 Dec 1997 14:41:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred C. Erpel" <aerpel@Op.Net>

Date: Wed, 17 Dec 1997 09:41:51 -0500

Subject: Re: New record

>But the 1/4 wave frequency of the length of wire is irrelevant.

>That relationship is no longer valid once the wire is wound into

>a coil. Trying to get a coil to resonate at the 1/4 wave frequency

>of its stretched out wire is a waste of time.

Mike,

If the 1/4 wave frequency is irrelevant, why are we supposed to consider

it when designing a coil?

Regards,

Alfred

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New record

Date: Tue, 16 Dec 1997 18:01:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred C. Erpel" <aerpel@Op.Net>

Date: Tue, 16 Dec 1997 13:01:07 -0500

Subject: Re: New record

>Fellow coilers,

>Last night I replaced my top terminal which was made

>from 4" to a new terminal made from 6" duct. The

>results were dramatic. Streamers srtuck everything.

>they hit the floor, the workbench, the primary, even

>a transformer across the room. Some of the strikes

>measure over 40". It was spectactular ! I think Bill

>is going to try and take some pictures.

>Note:The phone upstairs in the bedroom is electronic,

>it is on the same circuit with the coil. Whenever I

>run the coil the phone upstairs rings and the digital

>clock resets its self. I think I'll re-wire the tank

>circuit tonight. I bought some 10 gage wire last night.

>Note: The safety gap on the neon fires regularly, the

>safety gap on the pig never fires. Any Ideas ? I've

>got it set pretty close, .312"- .375". It did fire once

>when I used the 6" diameter duct. It was a very thin

>blue arc.

>Paul Marshall

>---

>Einigkeit und Freiheit

Paul,

I'll venture my opinion.

So far in all my reading on tesla coils, I've yet to see someone say

"... and when I took my large toroid off and put the smaller one on, the

coils performance got much better." But I've seen lots of people see

improvement with a larger toroid/top load.  I believe this is because, in

general, the natural frequency of the secondary coil system as determined by

1 / 2 pi sqrt(LsCs) on most coils turns out to be much higher than the 1/4

wave frequency as determined by the number of feet of coil on the secondary.

So when you increase the capacitance of the secondary coil system, you may

have brought its resonating frequency closer to its 1/4 wave frequency and

this must be important.

If anyone knows better, please correct me.

Tsch=FCs,

Alfred

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New record

Cc:

Bcc:

X-Attachments:

At 02:41 PM 12/17/97 GMT, you wrote:

>From: "Alfred C. Erpel" <aerpel@Op.Net>

>Date: Wed, 17 Dec 1997 09:41:51 -0500

>Subject: Re: New record

>

>

>>But the 1/4 wave frequency of the length of wire is irrelevant.

>>That relationship is no longer valid once the wire is wound into

>>a coil. Trying to get a coil to resonate at the 1/4 wave frequency

>>of its stretched out wire is a waste of time.

>

>

>Mike,

>

> If the 1/4 wave frequency is irrelevant, why are we supposed to consider

>it when designing a coil?

>

>Regards,

>Alfred

Alfred,

Actually you dont need to consider it at all. The 1/4 frequency

of the stretched out wire bears no relationship to the

actual resonant frequency of that same wire wound into a

secondary coil.

In fact it would be very difficult to get a secondary coil

to resonate at that 1/4 wave frequency. It would take a very

large toroid to do so and would offer no particular advantage.

Other than the advantages the toroid itself would bring.

The resonant frequency of a secondary is determined by the

inductance and self capacitance of the windings. The

inductance is determined by the number of turns and the

dimensions of the form. The self capacitance solely by the

dimensions of the form.

I have no idea what the length of wire I used in my secondaries

is nor do I care. What I do care about is the inductance and

resonant frequency.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Strike Ring

Date: Tue, 16 Dec 1997 23:19:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Tue, 16 Dec 1997 23:19:36 +0000

Subject: Re: Strike Ring

> Make sure this is an open loop. A closed loop represents a

> shorted turn and can really suck up the energy.

An open loop?  Not a grounded complete circle?  I don't get it, a

ground is a ground is a ground, ain't it?  A strike hits the loop and

goes to ground.  What difference would it make if half the electrons

go one way and the other half go the other way, as long as they all

go to ground?  (the "half. . .and. . .half" is based on my rather

loose, and completely improper,  application of the Hiesenberg

Uncertainty principle :)

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Performance "4.5 x 26" coil 24 AWG

Cc:

Bcc:

X-Attachments:

At 07:23 PM 12/17/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Wed, 17 Dec 1997 13:23:46 -0600

>Subject: Performance "4.5 x 26" coil 24 AWG

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Fellow coilers,

> I have just finished work on my first true Tesla coil.

>This was a joint effort between myself and Bill Pollack

>a fellow Engineer and TESLA-2 subscriber. My coil is

>wound on 4" PVC. It is wound using 24 AWG for a total

>winding length of 26" The primary is a flat spiral

>it was wound using 1/4" copper tubing the turns are

>spaced about .25". The total number of turns is just

>12. The power supply is 13.2 kv distribution

>Transformer, which is current limited to about 2200w.

>The capacitors are .026uf, two of which are in parallel

>and a third in series. I'm using a Richard Quick style

>gap (10).025" in series. My top load is roughly 20"

>in diameter and is made from 6" dia. dryer duct. I'm

>getting routine srteamers of 35" lengths and an

>occasional 40" strike. My question is what should I

>expect from this coil ? Is the preformance I'm getting

>about right or is it a dog ? I don't have any previous

>experence reguarding coils so I just don't know.

>Your comments are much appriciated. Thanks for all of

>the help, I know this will be an on going project.

>

>Your fellow coiler,

>

>Paul S. Marshall

>

Paul,

It sounds like you and Bill are doing great. I believe

your 4 incher is capable of more. My own 4 inch secondary

gives me 48-50 inches of spark with slightly more than

1200 watts input.

What can you do to improve output? Some things you've already

done like adding a larger toroid and improving the tank circuit

wiring. Try to identify areas of loss. Look for and eliminate

any traces of corona. Corona represents power loss.

The biggest source of loss is always going to be the spark gap.

Anything you do here to improve efficiency will help.

Your RQ type gap is probably not quenching the power from your

pig as well as it could. Opening your gap a little more may

help boost your primary energy level a little. If you

are using 10 gaps at .025 inch thats a 1/4 inch total. You

can use a little more than that especially with the pig.

With the power you are running you need to work hard to remove

excess heat from the gap. Replace your muffin fan with an

attachment for your shop vac. Suck air through the gap instead

of blowing air through it. This will improve the quenching.

If your gap is made from 3/4 inch copper pipe consider building

a larger unit using 1 inch diameter pipe. Make these 4 or 5 inches

long.

If you are going to change over completely to pig operation

consider building a rotary gap. Use the rotary in series with

your existing static gap. Make the static gap do the work

and let the rotary do the switching.

Try various levels of coupling. Some times just moving the

secondary a 1/2 inch can make a difference.

If I understand what you are doing with your cap you are running

.052 mfd in series with .026 mfd. This results in about .017 mfd

of capacitance. I hope you realize that the voltage will not

divide equally due to the unequal capacitances in your series string.

I would add a 4th .026 mfd cap in parallel with the single cap

to boost your capacitance to .026 mfd and make your cap string

more symmetrical. This will also allow you to utilize more of

your transformers power. Balance this capacitance increase with

another larger yet toroid. Consider going for 24 X 6.

I figured I was probably doing as well as or better than

anyone else with a 4 inch coil. However I would be willing

to bet my 52 inch record is going to be in jeopardy soon.

Take pictures.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 35 inch arcs

Cc:

Bcc:

X-Attachments:

At 09:23 PM 12/17/97 GMT, you wrote:

>From: baumann@proton.llumc.edu

>Date: Wed, 17 Dec 1997 13:23:36 -0800 (PST)

>Subject: Re: 35 inch arcs

>

>On 17 Dec, Tesla-2 wrote:

>> From: arcstarter@mailexcite.com

>> Date: Wed, 17 Dec 1997 14:37:38 -0600

>> Subject: 35 inch arcs

>> Organization: MailExcite  (http://www.mailexcite.com)

>>

>> Coilers,

>>  I was wondering if our coil is being hindered by the

>> fact that the primary is resting about 1/4 inch off the

>> concrete floor, basically the ground plane.

>>

>>  I'm sure that any metal flakes, rebar etc inside the

>> floor is probably sucking up some power.

>> Anyone keen on a bit of induction heating??? :)

>>

>>  Would we benefit from raising the whole thing up a

>> foot or two??

>>

>> Thanks!

>>

>> -Bill the arcstarter

>>

>>

>>

>Yes, yes, and again yes..

>Your primary is going to couple with the ground anyway, even without the

>rebar etc... I have been told that 12" is a minimum lift.. I use

>about 13" on my little 3.5x10 coil, and I get 14" streamers with only

>12KV 30mA neon as the power supply.

>

>--

>--

>Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

>San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

I agree. Any pieces of metal can soak up energy from the

primary magnetic field. Most of these losses will be pretty small

unless they happen to be in the form of a closed ring.

I would move your coil up and give it some clearance.

12 inches minimum like Michael suggested seems reasonable to me.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Strike Ring

Date: Wed, 17 Dec 1997 12:59:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 17 Dec 1997 07:59:22 -0500

Subject: Re: Strike Ring

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Strike Ring

> Date: Tuesday, December 16, 1997 6:19 PM

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Tue, 16 Dec 1997 23:19:36 +0000

> Subject: Re: Strike Ring

>

>

> > Make sure this is an open loop. A closed loop represents a

> > shorted turn and can really suck up the energy.

>

> An open loop?  Not a grounded complete circle?  I don't get it, a

> ground is a ground is a ground, ain't it?  A strike hits the loop and

> goes to ground.  What difference would it make if half the electrons

> go one way and the other half go the other way, as long as they all

> go to ground?  (the "half. . .and. . .half" is based on my rather

> loose, and completely improper,  application of the Hiesenberg

> Uncertainty principle :)

>

> Chris

>

Chris,

The strike rail, if a complete CLOSED LOOP, is ALSO a one-turn

short-circuit as far as the PRIMARY is concerned. This will

suck away energy that should be being delivered to the "real"

secondary... the one with the toroid on top :)

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Performance "4.5 x 26" coil 24 AWG

Date: Wed, 17 Dec 1997 19:23:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 17 Dec 1997 13:23:46 -0600

Subject: Performance "4.5 x 26" coil 24 AWG

Organization: MailExcite  (http://www.mailexcite.com)

Fellow coilers,

I have just finished work on my first true Tesla coil.

This was a joint effort between myself and Bill Pollack

a fellow Engineer and TESLA-2 subscriber. My coil is

wound on 4" PVC. It is wound using 24 AWG for a total

winding length of 26" The primary is a flat spiral

it was wound using 1/4" copper tubing the turns are

spaced about .25". The total number of turns is just

12. The power supply is 13.2 kv distribution

Transformer, which is current limited to about 2200w.

The capacitors are .026uf, two of which are in parallel

and a third in series. I'm using a Richard Quick style

gap (10).025" in series. My top load is roughly 20"

in diameter and is made from 6" dia. dryer duct. I'm

getting routine srteamers of 35" lengths and an

occasional 40" strike. My question is what should I

expect from this coil ? Is the preformance I'm getting

about right or is it a dog ? I don't have any previous

experence reguarding coils so I just don't know.

Your comments are much appriciated. Thanks for all of

the help, I know this will be an on going project.

Your fellow coiler,

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Performance "4.5 x 26" coil 24 AWG

Date: Wed, 17 Dec 1997 20:08:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Wed, 17 Dec 1997 12:08:14 -0800 (PST)

Subject: Re: Performance "4.5 x 26" coil 24 AWG

Paul:

If you don't mind my asking, what did you pay for your pig? I have a

line on a potential transformer, 1.5KVA, 12KV out for just under $300,

so I'd like to see how that compares. BTW: Your pig needs 220, or 110

for input?

Thanks

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 35 inch arcs

Date: Wed, 17 Dec 1997 20:37:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Wed, 17 Dec 1997 14:37:38 -0600

Subject: 35 inch arcs

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I was wondering if our coil is being hindered by the

fact that the primary is resting about 1/4 inch off the

concrete floor, basically the ground plane.

I'm sure that any metal flakes, rebar etc inside the

floor is probably sucking up some power.

Anyone keen on a bit of induction heating??? :)

Would we benefit from raising the whole thing up a

foot or two??

Thanks!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: 35 inch arcs

Date: Wed, 17 Dec 1997 21:23:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Wed, 17 Dec 1997 13:23:36 -0800 (PST)

Subject: Re: 35 inch arcs

On 17 Dec, Tesla-2 wrote:

> From: arcstarter@mailexcite.com

> Date: Wed, 17 Dec 1997 14:37:38 -0600

> Subject: 35 inch arcs

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>  I was wondering if our coil is being hindered by the

> fact that the primary is resting about 1/4 inch off the

> concrete floor, basically the ground plane.

>

>  I'm sure that any metal flakes, rebar etc inside the

> floor is probably sucking up some power.

> Anyone keen on a bit of induction heating??? :)

>

>  Would we benefit from raising the whole thing up a

> foot or two??

>

> Thanks!

>

> -Bill the arcstarter

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Yes, yes, and again yes..

Your primary is going to couple with the ground anyway, even without the

rebar etc... I have been told that 12" is a minimum lift.. I use

about 13" on my little 3.5x10 coil, and I get 14" streamers with only

12KV 30mA neon as the power supply.

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: 35 inch arcs

Date: Thu, 18 Dec 1997 03:10:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 17 Dec 1997 19:10:24 -0800

Subject: Re: 35 inch arcs

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: arcstarter@mailexcite.com

> Date: Wed, 17 Dec 1997 14:37:38 -0600

> Subject: 35 inch arcs

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>  I was wondering if our coil is being hindered by the

> fact that the primary is resting about 1/4 inch off the

> concrete floor, basically the ground plane.

>

>  I'm sure that any metal flakes, rebar etc inside the

> floor is probably sucking up some power.

> Anyone keen on a bit of induction heating??? :)

>

>  Would we benefit from raising the whole thing up a

> foot or two??

>

> Thanks!

>

> -Bill the arcstarter

>

Bill,

YEP!! After you raise the coil up from the floor, start out at lower

power levels since you WILL be coupling more energy into the secondary

and less into the rebar...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Our setup (35 inch arc)

Date: Thu, 18 Dec 1997 16:20:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Thu, 18 Dec 1997 10:20:00 -0600

Subject: Our setup (35 inch arc)

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

Several people have asked for more details on our

coil.  Here is the scoop:

We use either a 7500 V at 125 mA neon, or a pole

pig.  The primary of the pig is presently configured

for 125V input, and we have an old microwave

transformer in series with the pig primary for

current limiting.

Up in Dayton, OH is a 5-story building.  The 2nd floor

of this building is the home of MECI electronics,

the famous mail order company.  The 1st and 3rd

floors are part of Mendelson's electronic salvage.

They had about 8 pigs of various sizes, all

non-PCB.

The pig is a 25KVA unit, cost $250, and supplies 13.8

KV.  It weighs 485 lbs and is about 3 ft tall.

Mendeson's used their fork truck to put the thing

into the back of my suburban, and it was about

all three people could do to get the thing out,

once we got down to Paul's place.

The primary of the pig can be configured for 240

operation.  We haven't tried this yet.

We have NO variac but limit power by various series loads consisting of tungsten

lamps, space heaters, and shorted-out microwave transformers.

Our RQ gap is probably underdesigned as I only

used 3/4 inch copper pipe, 2 inches long.  I will

probably have to build a better one soon as the

output arcs grow much shorter as the thing heats

up.  An old microwave squirrel-cage fan cools the

gaps.

We have four identical poly-plate caps.  Each is

about 0.026 uf, and consists of 48 plates

of 0.060 poly, 8 by 12 inches, with heavy duty

kitchen foil conductors.  The whole pack is

oriented horizontally in a rubbermaid box, plastic

zip-ties hold the plates together, and the

box is filled with diala-EX oil.

Tuning is of course highly dependent on the particular

top terminal we use, and is adjusted by tapping the

primary and/or using various combos of series/parallel

caps.

More later!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Couple of questions

Cc:

Bcc:

X-Attachments:

At 06:02 AM 3/7/97 GMT, you wrote:

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Fri, 7 Mar 1997 00:02:19 -0600

>Subject: Couple of questions

>

>  I have begun to make some progress on my coil this week dispite finals.

> I just wanted to ask a few questions and tell what I have been up to in

> case I have errored.

>

> I completed half of my first cap it is .01uF @ 7500 V and consists of

>15-60 sqi of foil with 10-4 mil poly plates constructed horizontally in

>baby oil. I left about 1.5 inches on each of the sides. I tested it at

>7500v a few minutes after construction and it seems to work fine. I will

>put 2 in series for my coil.

>From what I understand poly and oil have nearly the same dialectric

>coff, is it possible to build an foil poly cap without oil? I guess I

>don`t understand how failure occurs, Through the poly or around it.

Yes polyethylene and mineral oil both have dielectric constants

of around 2.2. And yes it is possible to build a poly cap without

oil but it would not be advisable. The purpose of the oil is to exclude

air which has a lower dielectric constant of 1 and to prevent corona.

Corona will quickly deteriorate polyethylene and promote a failure.

The most common failure mode for poly caps is dielectric fatigue.

This often happens in single caps used at 12kv or more. The thickness

of the dielectric really doesnt matter. Failure will occur eventually.

Failure can also occur with an arc forming along the surface of the

poly and traveling toward the edge and then down on the adjacent plate.

This is a common failure in plate type caps. Having adequate margins

and excluding air through the use of oil are effective remedies.

Every type of cap failure can be reduced or eliminated by simply

running the caps at a lower voltage. Use a series string of caps

to keep the voltage each individual cap sees low.

Every one of my cap failures have been single caps. None of my series

connected caps have ever failed.

>

>  I began to wind my secondary, 24 guage on 4" pcv, But ran out of wire at

>the half way point. Is it ok to solder it and put a little extra varnish

>on it? I would hate to rewind it.

Yes you can do this. Try to make the splice as smooth and as flat

as possible. Take a file and smooth over the connection. I would add

a glob of epoxy over the connection for extra insurance.

>

> I finished my toroid, not very pretty 4" drain pipe coated with paper

>mache and glued foil to that. I cannot seem to get all the sharp edges

>down. How big of an impact will that have? I guess I could find some

>drier duct someplace.

Sharp edges will encourage premature breakouts at a lower voltage.

Try to smooth it down as well as you can. Try running over the foil

with the back of a spoon as a burnishing tool.

>

> If anyone sees something I am doing wrong, I would appreciate it.

>Thanks,

>Mike White

>

Looks good to me Mike.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Couple of questions

Date: Fri, 19 Dec 1997 13:20:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Fri, 19 Dec 1997 07:20:27 -0600

Subject: Re: Couple of questions

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

The only question that I feel qualified to answer is

the one you have asked regarding the failure mode of

capacitors. There are two predominate failures. One is

dielectric penetration which occurrs for one of two

reasons:

1)Severe over-voltage, due o resonate rise, or a failure

to design to the proper dielectric thickness.

2)Trapped air between the plates which allows localized

over heating to occur. This causes a breakdown in

dielectric strength and subsequent failure.

The other failure mode is flash-over. This occures when

you run the capacitor dry or when trapped air is

present at the plate boundries. This is the main reason

for using oil. I have built several high quality poly

caps and I can tell you it is worth the time it takes

to build them right.

As for the toroid, I'm sure that when they are smooth

they will function much better. I do not however

think that I would worry too much about a few rough spots.

Paul S. Marshall

---

Einigkeit und Freiheit

>From: michael white <mjwhite@agen.okstate.edu>

>Date: Fri, 7 Mar 1997 00:02:19 -0600

>Subject: Couple of questions

>

> I have begun to make some progress on my coil this week despite finals.

>

>I just wanted to ask a few questions and tell what I have been up to

>in case I have errored.

>

>  I completed half of my first cap it is .01uF @ 7500 V and consists of

>15-60sqi of foil with 10-4 mil poly plates constructed horizontaly in

>baby oil. I left about 1.5 inches on each of the sides. I tested it at

>7500v a few minutes after construction and it seems to work fine. I will

>put 2 in series for my coil.

>From what I understand poly and oil have nearly the same dialectric

>coff, is it possible to build an foil poly cap without oil? I guess I

>don`t understand how failure occurs, Through the poly or around it.

>

>   I began to wind my secondary,24gage on 4" pcv, But ran out of wire at

>the half way point. Is it ok to solder it and put a little extra varnish

>on it? I would hate to rewind it.

>

>   I finished my toroid, not very pretty 4" drain pipe coated with paper

>mache and glued foil to that. I cannot seem to get all the sharp edges

>down. How big of an impact will that have? I guess I could find some

>drier duct someplace.

>

> If anyone sees something I am doing wrong, I would appreciate it.

>Thanks,

>Mike White

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: little xformer

Date: Sat, 20 Dec 1997 02:29:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

Date: Fri, 19 Dec 1997 19:29:32 -0700 (MST)

Subject: Re: little xformer

I found the specs on the little neon xformer:

120volts ac primary

4000 volts, 30mA secondary

I want to make a small coil built around this, any pointers?

thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave Oven Transformers

Date: Sat, 20 Dec 1997 16:34:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Grayson Dietrich <greystone21@rocketmail.com>

Date: Sat, 20 Dec 1997 08:34:50 -0800 (PST)

Subject: Re: Microwave Oven Transformers

Matthew, a transformer from a micromave oven will be

MUCH more than adequate for a coil of the dimensions

that you described. I myself recently procured(from a

friend, for free) a 2000volt 500watt transformer of

the same variety. The voltage is rather low, but once

an arc is started(at about 2mm) I can pull a 5-6 inch

arc of of it. You said that your's had five leads? 2

are input, 2 are a low-voltage output for the

magnetron filament. One other, and the metal core of

the transformer itself are the high-voltage output.

If youy want to run a T.C. with this you will have to

limit the current for its primary, easily done with a

lightbulb of the same wattage rating as the

transformer. Another, and perhaps better, way would

be to use some sort of triode.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: wiring/variac

Date: Sat, 20 Dec 1997 21:33:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 20 Dec 1997 21:33:04 +0000

Subject: wiring/variac

Well, I have my 10 amp variac and now I have some questions about

where to put it in the system.

1.  Do I need to keep the ac line filter between the variac and the

tranny?

2.  ac motors, such as my SG fan, will brown/burn-out at less than

nominal operating voltage, correct?  (if so I need to re-wire it's ac

hook-up to the other side of the variac).

3.  Does the variac heat-up much while in use?  The one I have didn't

have a case, so I made one out of a serve-n-save container.  Don't

want to find out in mid-run that it won't take the heat.

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wiring/variac

Cc:

Bcc:

X-Attachments:

At 09:33 PM 12/20/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sat, 20 Dec 1997 21:33:04 +0000

>Subject: wiring/variac

>

>

>Well, I have my 10 amp variac and now I have some questions about

>where to put it in the system.

Great. You will find that having a variac provides you with

so much control over your system that you will wonder how you

ever got along without it.

Plus most variacs can be wired for 0-140 VAC output with

120 VAC input. This extra voltage can give you an extra boost

of power from your neons. This extra voltage does not seem

to hurt the neons in any way. Just take into account the extra

voltage when designing caps for your system.

>

>1.  Do I need to keep the ac line filter between the variac and the

>tranny?

Yes. You want to keep RF out of your variac. I have my variac

housed in a control box and have an AC line filter between

the variac and neons and a second filter on the main AC input

to the control box. This might be overkill but doesnt hurt.

>

>2.  ac motors, such as my SG fan, will brown/burn-out at less than

>nominal operating voltage, correct?  (if so I need to re-wire it's ac

>hook-up to the other side of the variac).

Yes. You do not want to run other devices through your variac.

Reserve its capacity for your neons.

>

>3.  Does the variac heat-up much while in use?  The one I have didn't

>have a case, so I made one out of a serve-n-save container.  Don't

>want to find out in mid-run that it won't take the heat.

>

>Thanks. . .CF

>

Variacs are rated for continous duty operation. As such they

can supply more current than their rating for short periods.

Doing so will generate more heat in the windings. You do not

want your variac completely enclosed so the heat could build up.

I mounted a small ac muffin fan in my control box to move around

a little air to make sure the variac receives proper cooling.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wiring/variac

Date: Sun, 21 Dec 1997 00:45:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 21 Dec 1997 00:45:27 +0000

Subject: Re: wiring/variac

> Make sure you have the wall power going into the "LOAD" terminals, and

> your experiment running off the "LINE" terminals.  Yes this is reverse of

> normal operation, since usually line filters are trying to keep RFI out

> of the equipment, and are designed so.

Uh, my variac doesn't have "LOAD" and "LINE" terminals labeled as

such.  What it did have was a hand-written sticker stating which of

the five posts were which.  So, I should run it backwards??  I

checked the output, and it runs from 0-150vac, seemed to work OK (and

the little "dot" at ~80% was right on at 124vac).

And one other thing about the wiring.  Since there was no ground

marked, I just ran the ground from the input to the output, not

connected to the variac at all.  Is this OK?

CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: microwave oven transformers (MOTs)

Date: Sun, 21 Dec 1997 03:09:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Sat, 20 Dec 1997 21:09:56 -0600

Subject: microwave oven transformers (MOTs)

Organization: MailExcite  (http://www.mailexcite.com)

About those microwave transformers...

I have three transformers, which appear to contain

magnetic shunts, so you would think they are current

limited.  However, if you short the secondary,

(or, more to the point, if your static gap fires :) )

then the primary draws, in my case, over 30 amps!!  My

secondary current was measured using a clamp-on

ammeter at over one amp!

I have had large difficulties quenching the 'power

arcs' which these uwave xfmrs are capable of

producing.  Static gap design seems to be further

complicated by the relatively low (~2000) voltage of

these units, which just plain won't jump as much as

the smallest neons allow.

I was able to partially beat this by using two

identical xfmrs, with the primaries in series running

on 110VAC.  The phasing was such that I could tie both

cases together and to powerline ground.  Output was

from both high voltage output terminals.

This configuration gives me a more managable short

circuit current, and even seems to give a slight boost

to the open circuit voltage for some reason.

I am considering using some sort of synchronous rotary

gap for use with uwave xfmrs.  Does this seem

appropriate??

If anyone has any sucess stories about using MOTs in

tesla coils I'd like to hear about it!

-Bill the arcstarter

>Matthew, a transformer from a micromave oven will be

>MUCH more than adequate for a coil of the dimensions

>that you described. I myself recently procured(from a

>friend, for free) a 2000volt 500watt transformer of

>the same variety. The voltage is rather low, but once

>an arc is started(at about 2mm) I can pull a 5-6 inch

>arc of of it. You said that your's had five leads? 2

>are input, 2 are a low-voltage output for the

>magnetron filament. One other, and the metal core of

>the transformer itself are the high-voltage output.

>If youy want to run a T.C. with this you will have to

>limit the current for its primary, easily done with a

>lightbulb of the same wattage rating as the

>transformer. Another, and perhaps better, way would

>be to use some sort of triode.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tuning

Date: Sun, 21 Dec 1997 03:41:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 21 Dec 1997 03:41:39 +0000

Subject: Tuning

Now that I've got the variac controlling my coil I can at least see

the symptoms of my out of tune system.  Here's where I'm at:

All runs at approx 70% power, 15kv 30ma tranny, 13"

primary (solenoid), 21x6.25 sec, 8" alum sphere topload.

1.  tapped 11 turns, bottom sec winding aligned with bottom pri

winding:  heavy corona between mid-point on sec and strike ring, as

well as at the top turn of the sec.

2.  Raised the sec 2":  less corona, same places.

3.  Tapped 9-10 turns, respectively:  corona gone from strike-ring to

sec, still visible at top of sec, and sparks drawn off were 3 and 4

inches (at 9 and 10 turns tapped)

4.  Raised sec 4":  tapped 10 turns, some corona at top of sec, max

4" spark drawn off.

So, my first thought is to make a 4x24" toroid for my top load, 1. to

lower the freq of the sec (and hopefully get closer to tune), and 2.

to try to eliminate the corona.  Is this the next best step, or

should I try something else first?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: microwave oven transformers (MOTs)

Date: Sun, 21 Dec 1997 04:51:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Sat, 20 Dec 97 23:51:42 -0500

Subject: Re: microwave oven transformers (MOTs)

>About those microwave transformers...

>I have three transformers, which appear to contain

>magnetic shunts, so you would think they are current

>limited.  However, if you short the secondary,

>(or, more to the point, if your static gap fires :) )

>then the primary draws, in my case, over 30 amps!!  My

>secondary current was measured using a clamp-on

>ammeter at over one amp!

This is normal.  The shunts are there for current limiting, but unlike

neon transformers, microwave units do not operate continuously with a

very low impedance load.  The shunting does limit the current, but

typically to the 300ma-1amp range as you have seen first hand.  Microwave

oven transformers from home units normally output 1800-2500V RMS at

0.5-2.0kVA, at least the ones I've owned.

>I was able to partially beat this by using two

>identical xfmrs, with the primaries in series running

>on 110VAC.

My suggestion is to put the primaries in anti-parallel, ground the cases

together to your midpoint RF ground, and run the HV outputs to your tank

circuit.  Also, ignore the shunts and pretend these guys are non-limited

like potential transformers.  Use some inductive ballasting.  With my

microwave supply, I ballasted a pair of them with the secondary coil of a

transformer I pulled from a battery charger.  The transformer was

originally 120VAC to 12V @ 30A.  This was about the right inductance to

get things started, and I increased power by removing the "I" portion of

the ballast transformer's core, then my 15A breaker tripped.  The ballast

did not even get warm, and it cured the problem of hot power arcs in the

gap.

>I am considering using some sort of synchronous rotary

>gap for use with uwave xfmrs.  Does this seem

>appropriate??

A sync rotary is probably not necessary.  A non-sync rotary should be

fine, since you will not be using enough tank capacitance to be anywhere

near the resonance point between your transformer and tank cap.

Microwave trannies are pretty robust, just make sure you have the proper

safety gap distance.  Since you will be running the transformers at twice

their rated output voltage, you might want to think about potting them in

something, like mineral oil, vasoline or paraffin to help insulate them.

-Adam

Adam Smith

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning

Cc:

Bcc:

X-Attachments:

At 03:41 AM 12/21/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sun, 21 Dec 1997 03:41:39 +0000

>Subject: Tuning

>

>

>Now that I've got the variac controlling my coil I can at least see

>the symptoms of my out of tune system.  Here's where I'm at:

>

>All runs at approx 70% power, 15kv 30ma tranny, 13"

>primary (solenoid), 21x6.25 sec, 8" alum sphere topload.

>

>1.  tapped 11 turns, bottom sec winding aligned with bottom pri

>winding:  heavy corona between mid-point on sec and strike ring, as

>well as at the top turn of the sec.

Heavy corona indicates over coupling. The corona at top is due to

inadequate supression from spherical terminal. Note that corona

represents power loss.

>2.  Raised the sec 2":  less corona, same places.

Good move. The lower coupling reduces the corona.

>

>3.  Tapped 9-10 turns, respectively:  corona gone from strike-ring to

>sec, still visible at top of sec, and sparks drawn off were 3 and 4

>inches (at 9 and 10 turns tapped)

>

>4.  Raised sec 4":  tapped 10 turns, some corona at top of sec, max

>4" spark drawn off.

Sounds like this is about where you want your coupling

set for now. Some fine adjustment can come later.

To reduce corona at top of secondary you can switch to

a toroid terminal or lower your sphere closer to the

top of the secondary winding. If you have several inches

of spacing here try to reduce it. The closer the terminal

is to the top secondary turns the better the corona suppression

will be.

>

>So, my first thought is to make a 4x24" toroid for my top load,

>1. to lower the freq of the sec (and hopefully get closer to tune),

>and 2.to try to eliminate the corona.  Is this the next best step, or

>should I try something else first?

Im afraid that adding a toroid will make your tuning problems worse.

It appears you are getting more output as you tap in more primary turns.

3 inches at turn 9 and 4 inches at turn 10.

This indicate that your secondary resonant frequency is lower

than what your primary circuit will tune. So you need to either

lower your primary resonant frequency or raise your secondary

frequency. The easiest thing to do is lower the primary frequency.

You can do this two ways. Add more main capacitance or add more primary

turns. Since you have a 15kv/30ma neon your primary cap can only

be as large as .0053 mfd. If you are lower than this then add more

capacitance. If you already have this amount of capacitance then

you need to add more primary turns. If you presently have 10 turns

I would add at least four more. Even more than this might be desirable

to tune a larger toroid later on.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wiring/variac

Cc:

Bcc:

X-Attachments:

At 12:45 AM 12/21/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sun, 21 Dec 1997 00:45:27 +0000

>Subject: Re: wiring/variac

>

>

>> Make sure you have the wall power going into the "LOAD" terminals, and

>> your experiment running off the "LINE" terminals.  Yes this is reverse of

>> normal operation, since usually line filters are trying to keep RFI out

>> of the equipment, and are designed so.

>

>Uh, my variac doesn't have "LOAD" and "LINE" terminals labeled as

>such.  What it did have was a hand-written sticker stating which of

>the five posts were which.  So, I should run it backwards??  I

>checked the output, and it runs from 0-150vac, seemed to work OK (and

>the little "dot" at ~80% was right on at 124vac).

I believe what Adam was referring to was the load and line inputs

on an AC line filter. You use such a filter installed backwards

with the LOAD terminals running to your variac and the LINE terminals

go to your neons.

I prefer to have my filter between the variac and neons

to keep RF out of the variac.

>

>And one other thing about the wiring.  Since there was no ground

>marked, I just ran the ground from the input to the output, not

>connected to the variac at all.  Is this OK?

>

Actually you do not want your 60 hz AC ground connected to

your neons at all. You want the neon cases connected to your

RF ground system. This is in case of an arc inside your neon

from the secondary to the core. You want that energy to flow

directly to ground not flowing around your house on the 60 hz

ground. Keep your 60 hz and RF grounds seperate.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wiring/variac

Date: Sun, 21 Dec 1997 23:31:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sun, 21 Dec 1997 23:31:06 +0000

Subject: Re: wiring/variac

> Actually you do not want your 60 hz AC ground connected to

> your neons at all. You want the neon cases connected to your

> RF ground system. This is in case of an arc inside your neon

> from the secondary to the core. You want that energy to flow

> directly to ground not flowing around your house on the 60 hz

> ground. Keep your 60 hz and RF grounds seperate.

It makes so much sense I feel stupid not thinkin' of that myself.

But it again begs the question, is it OK to run this ground to the

same ground that the sec's connected to?

Thanks Mike,  CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wiring/variac

Date: Mon, 22 Dec 1997 01:10:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Sun, 21 Dec 97 20:10:41 -0500

Subject: Re: wiring/variac

>Well, I have my 10 amp variac and now I have some questions about

>where to put it in the system.

>1.  Do I need to keep the ac line filter between the variac and the

>tranny?

If you only have one, put it between the wall outlet and the variac.

Make sure you have the wall power going into the "LOAD" terminals, and

your experiment running off the "LINE" terminals.  Yes this is reverse of

normal operation, since usually line filters are trying to keep RFI out

of the equipment, and are designed so.  Some are bidirectional, but very

few.  Also, ground the RFI case and your variac ground (housing) to your

low voltage ground, not the RF ground.

>2.  ac motors, such as my SG fan, will brown/burn-out at less than

>nominal operating voltage, correct?  (if so I need to re-wire it's ac

>hook-up to the other side of the variac).

Most AC motors have no problem with using a variac for speed control, as

long as you are giving the motor enough voltage to spin it, and it's

corresponding shaft mounted fan.  Now with more than the nominal voltage,

you can certainly have problems :-)  I use a 1kVA variac for my vacuum

cleaner SG motor.

>3.  Does the variac heat-up much while in use?  The one I have didn't

>have a case, so I made one out of a serve-n-save container.  Don't

>want to find out in mid-run that it won't take the heat.

At it's rated power throughput (e.g. 1.5kVA) it will not run hot.  Your

container should be fine.

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tuning, top loads & spark length

Date: Tue, 23 Dec 1997 13:23:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 23 Dec 1997 07:23:58 -0600

Subject: Tuning, top loads & spark length

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

I have made some interesting observations over the

last couple of days and I wanted to get your opinion.

1). Increased power seems to increase the brightness

of the streamers but does little to increase the

length. I'm speaking of current 120 ma. to 240 ma.

13,200 v.

2). I cleaned my gaps yesterday and apparently

widened them slightly. This seemed to change the

tunning as well. Now it seems that the 4"x 20"

top terminal and the 6" x 24" can share the same

tap on the primary and preform well. As a matter

of fact, it seems that the streamer length is about

the same. Can this be ?

3). My cp is about .017 mf. What are you running ? At

the moment I seem to be maxed out at 45". Please

tell me about your top load and tank circuit ?

Thanks Again,

Paul Marshall

Merry Christmas & happy New Year

--

Einigkeit und Freiheit

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Weather / Performance

Date: Mon, 22 Dec 1997 15:01:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 22 Dec 1997 09:01:59 -0600

Subject: Weather / Performance

Organization: MailExcite  (http://www.mailexcite.com)

I have two questions:

What affect does weather have on coil performance ?

For instance would you expect to have longer streamers

on a cold damp day or a hot and dry day ?

Is there any way to approximate the output voltage ?

If we look at streamer length, humidity, frequency,

and temperature, would it be possible to make an

educated guess ?

Thanks again,

A Merry Christmas & a Happy New year.

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wiring/variac

Cc:

Bcc:

X-Attachments:

At 11:31 PM 12/21/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sun, 21 Dec 1997 23:31:06 +0000

>Subject: Re: wiring/variac

>

>

>> Actually you do not want your 60 hz AC ground connected to

>> your neons at all. You want the neon cases connected to your

>> RF ground system. This is in case of an arc inside your neon

>> from the secondary to the core. You want that energy to flow

>> directly to ground not flowing around your house on the 60 hz

>> ground. Keep your 60 hz and RF grounds seperate.

>

>It makes so much sense I feel stupid not thinkin' of that myself.

>But it again begs the question, is it OK to run this ground to the

>same ground that the sec's connected to?

>

>Thanks Mike,  CF

Yes. When we speak of RF ground this is the same ground that the

base of the secondary is tied to.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning, top loads & spark length

Cc:

Bcc:

X-Attachments:

At 01:23 PM 12/23/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Tue, 23 Dec 1997 07:23:58 -0600

>Subject: Tuning, top loads & spark length

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Mike,

> I have made some interesting observations over the

>last couple of days and I wanted to get your opinion.

>

>1). Increased power seems to increase the brightness

>    of the streamers but does little to increase the

>    length. I'm speaking of current 120 ma. to 240 ma.

>    13,200 v.

Not really unusual. Spark length vs power input is very non-linear.

You may reach a point where doubling the input power adds only

a couple of inches of spark.

Larger diameter toroids help by increasing streamer current and

a large cross section toroid helps by allowing the voltage to

rise to a higher value before breakout occurs.

I would also bet that as you get the input power higher your gap

is not quenching as well as at lower powers.

>2). I cleaned my gaps yesterday and apparently

>    widened them slightly. This seemed to change the

>    tunning as well. Now it seems that the 4"x 20"

>    top terminal and the 6" x 24" can share the same

>    tap on the primary and preform well. As a matter

>    of fact, it seems that the streamer length is about

>    the same. Can this be ?

No. The gap will have very little effect on the resonant

frequency. And your two toroids will add a different

amount of capacitance to your secondary thus providing

different resonant frequencies.

You probably arent able to tell the difference in

tuning by judging spark length at high power. Try

tuning at a lower power level. I would go ahead and

tap one extra primary turn for the 24 X 6 toroid.

Sometimes its hard to tell the difference with a

one turn change in the primary. Try changing two turns

at a time and the change will be more easily visible.

>3). My cp is about .017 mf. What are you running ? At

>    the moment I seem to be maxed out at 45". Please

>    tell me about your top load and tank circuit ?

I am presently running two .03 mfd rolled caps in series

for .015 mfd. These caps have 24 mils (4 X 6 mils) of

poly in each. In series these work just fine at 12kv.

Power is my recently rebuilt neon putting out 12720/105ma.

This transformer doesnt seem to like as much capacitance

as my parallel neon bank did even though the power level

is nearly the same.

I plan a new .02 mfd plate cap for this transformer.

Toroid is 19 X 6 inches. Primary is tapered solenoid with

17 turns. Tapped at turn 15 for this setup. Best sparks

with this setup have been 50 inches.

I have had 52 inch sparks from this coil using a parallel

bank of neons and .0227 mfd of capacitance.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: little xformer

Cc:

Bcc:

X-Attachments:

At 02:29 AM 12/20/97 GMT, you wrote:

>From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

>Date: Fri, 19 Dec 1997 19:29:32 -0700 (MST)

>Subject: Re: little xformer

>

>

>

>I found the specs on the little neon xformer:

>

>        120volts ac primary

>

>        4000 volts, 30mA secondary

>

>

>I want to make a small coil built around this, any pointers?

>

Jon,

This neon is a little low in voltage for good gap operation

but since you have it go ahead and see what you can do with it.

Because of the low voltage and resulting low impedance

of this transformer it will take a fairly large cap

for best performance. It will take about .02 mfd.

The good news is you can take advantage of the low voltage

and use thinner polyethylene dielectric. 18 mils (3 X 6 mils)

should be more than adequate for this.

For a plate cap using 6 X 6 inch plates (actual overlapping area)

and 18 mils of poly only 24 plates would be required. The low

voltage makes a single cap like this possible.

For a rolled cap try plates 6 inches wide and 72 inches long.

With three 6 mil poly strips cut 9 inches wide and at least

76 inches long for each layer.

Cut a 12 inch wide roll of heavy duty aluminum foil in half

for this.

Your gap for this will need to be close spaced due to the low

voltage. I would try 3 series gaps spaced at .02 inch to start.

Good luck.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Weather / Performance

Date: Mon, 22 Dec 1997 20:08:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 22 Dec 1997 12:08:25 -0800

Subject: Re: Weather / Performance

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 22 Dec 1997 09:01:59 -0600

> Subject: Weather / Performance

> Organization: MailExcite  (http://www.mailexcite.com)

>

> I have two questions:

>

> What effect does weather have on coil preformance ?

> For instance would you expect to have longer streamers

> on a cold damp day or a hot and dry day ?

>

> Is there any way to approximate the output voltage ?

> If we look at streamer length, humidity, frequency,

> and temperature, would it be possible to make an

> educated guess ?

>

> Thanks again,

> A Merry Christmas & a Happy Newyear.

>

> Paul S. Marshall

> ---

> Einigkeit und Freiheit

>

Paul,

Generally, I've found that dry conditions seem to be better than damp.

Spark breakdown "works" at a somewhat lower voltage in dry air. Also,

leakage currents in the coil form tend to be greater in damp weather. I

find that my coil performs best in the wintertime when the air's dry and

cold.

Safe coilin' to you, and Merry Christmas, Paul!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Weather / Performance

Date: Mon, 22 Dec 1997 20:36:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 22 Dec 1997 12:36:19 -0800

Subject: Re: Weather / Performance

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Mon, 22 Dec 1997 09:01:59 -0600

> Subject: Weather / Performance

> Organization: MailExcite  (http://www.mailexcite.com)

>

> I have two questions:

>

> What effect does weather have on coil preformance ?

> For instance would you expect to have longer streamers

> on a cold damp day or a hot and dry day ?

>

> Is there any way to approximate the output voltage ?

> If we look at streamer length, humidity, frequency,

> and temperature, would it be possible to make an

> educated guess ?

>

> Thanks again,

> A Merry Christmas & a Happy Newyear.

>

> Paul S. Marshall

> ---

> Einigkeit und Freiheit

>

Paul,

Generally, I've found that dry conditions seem to be better than damp.

Spark breakdown "works" at a somewhat lower voltage in dry air. Also,

leakage currents in the coil form tend to be greater in damp weather. I

find that my coil performs best in the wintertime when the air's dry and

cold.

The second question is MUCH more difficult to answer. The SIMPLE answer

is there's really no easy way the average experimenter can accurately

relate spark length to voltage since power level, operating frequency,

break-rate, toroid size, and a whole host of other factors significantly

impact this. A "ballpark estimate can be made if you can make the coil

do "single shot" gap firings, and you measure the distance that a single

spark will jump between the toroid and a grounded rod. You can estimate

the voltage at about 20 kV per inch. However, this approach WON'T work

for a rapidly firing coil, since subsequent sparks have a easier time

following the trails blazed by their predecessors. This makes streamer

length so much longer in an optimally-running coil.

An upper limit for the output voltage can be made if you can estimate

your total secondary capacitance (Cs), primary tank capacitance (Cp),

and the primary gap breakdown voltage (Vg, and invoke Conservation of

Energy.

Vout (max) = Vgap* SQRT(Cp/Cs)

For example, If Vgap = 15,000 volts, Cp = .01uF and Cs = 25pf, then the

best case output voltage would be:

Vout (max) = 15000 * SQRT(1e-8/25e-12) = 15000*SQRT(400) = 300 kV

However, because of various losses in your system, the actual peak

voltage will more likely be 65 - 80% of this value.

Safe coilin' to you, and Merry Christmas, Paul!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla organizations?

Date: Tue, 23 Dec 1997 15:12:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 23 Dec 1997 09:12:29 -0600

Subject: Tesla organizations?

Organization: MailExcite  (http://www.mailexcite.com)

Hello.

How can I find out more about any of the tesla

organizations such as the TCBA or TCBOR?

I've heard that some of these groups have publications, videos, and the like

available for purchase.

Thanks!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla organizations?

Cc:

Bcc:

X-Attachments:

At 03:12 PM 12/23/97 GMT, you wrote:

>From: arcstarter@mailexcite.com

>Date: Tue, 23 Dec 1997 09:12:29 -0600

>Subject: Tesla organizations?

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Hello.

> How can I find out more about any of the tesla

>organizations such as the TCBA or TCBOR?

>

> I've heard that some of these groups have publications, videos,

> and the like available for purchase.

>

>Thanks!

>

>-Bill the arcstarter

>

Bill,

TCBA

Tesla Coil Builders Association.

Mr. Harry Goldman

c/o Tesla Coil Builders Association

3 Amy Lane

Queensbury, NY 12804-9432

Harry publishes a quarterly newsletter that contains

some interesting articles. It a bit expensive at $25

for a quarterly so I let my subscription lapse.

TCBOR

Tesla Coil Builders of Richmond

This is the group headed up by Richard Hull in

Richmond, Va. They have available an extensive

list of videos. They have some quite excellent instructional

tapes on building various coil components and a huge list

of video reports of their coil experiments. I have seen a few

of the tapes and wish I could view them all but unfortunately

cant afford to do so.

Try this URL for the TCBOR:

http://cator.hsc.edu/~kmd/tcbor/

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: cap voltages

Date: Tue, 23 Dec 1997 03:48:29 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Tue, 23 Dec 1997 03:48:29 +0000

Subject: cap voltages

True or False.

The voltage handling capacity of a set of caps wired in series is the

sum of each individual caps voltage rating.  (10, 1.5kv caps = 15kv

handling capacity).

And the cap/volt rating should be at least 1.414x the transformer's

secondary output.

Thanks All, Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla organizations?

Date: Tue, 23 Dec 1997 15:43:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

Date: Tue, 23 Dec 1997 20:43:02 +0500

Subject: Re: Tesla organizations?

On Tue, 23 Dec 1997 09:12:29 -0600 Bill

<arcstarter@mailexcite.com> wrote;

> Hello.

> How can I find out more about any of the tesla

> organizations such as the TCBA or TCBOR?

>

>  I've heard that some of these groups have publications, videos, and the like

>available for purchase.

Hi Bill, T.C.B.A. is the Tesla Coil Builder's Association. TCBA NEWS is the

official publication for the TESLA COIL BUILDER'S ASSOCIATION, an

educational organization devoted to research and dissemination of

information on the theory, construction, and operation of the high-voltage

high-frequency transformer known as the Tesla Coil.

TCBA NEWS is published 4 times a year by Harry Goldman, 3 Amy Lane,

Queensbury, NY  12804.  The telephone number is (518) 792-1003.  The

annual subscription rates were as follows (they may have changed).

UNITED STATES ------- $30             EUROPE ------------- $ *

CANADA -------------- $               AUSTRALIA ---------- $ *

MEXICO -------------- $               ASIA --------------- $ *

JR-SR HIGH SCHOOL (USA ONLY) $12**

* Air Delivery

** Students of junior and senior high school levels must have

their student status substantiated by their school principal

or science teacher.  Official school stationery should be used.

TCBA welcomes typewritten manuscripts pertaining to Tesla coils as well as

information regarding its inventor, Nikola Tesla.  Manuscripts on other

high-voltage devices and techniques are also accepted.  The author of an

accepted article is provided with two copies of the newsletters in which

the article appears.  If an author is a currently paid-up member of TCBA,

his or her subscription will be extended by one issue.

TCBA will also extend a subscription by one issue for every new member a

subscriber brings into the organization.  By signing four new members, a

subscriber will be given a 1-year extension to TCBA NEWS.

All backissues of the newsletter are available to subscribers.  Those

interested in obtaining back issues need only to contact the publisher at

the address:

T.C.B.A., Tesla Coil Builders Association

3 Amy Lane

Queensbury, NY  12804

518-792-1003

--------------------------------------------------------------------------

T.C.B.O.R. is the Tesla Coil Builders of Richmond. They have a WEB site and

it's URL is;

http://cator.hsc.edu/~kmd/tcbor/

P.S. My ASCII signature may be a bit messed up I'm working on cleaning it

up. <Grin>

Sincerely

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

Visit my Do-It-Yourself Aquarium WEB page at;

http://www.geocities.com/CapeCanaveral/4606

.ooo0   0ooo.

-----(   )---(   )-----

\ (     ) /

\_)   (_/

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Air Core RF Chokes

Date: Tue, 23 Dec 1997 16:09:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 23 Dec 1997 10:09:18 -0600

Subject: Air Core RF Chokes

Organization: MailExcite  (http://www.mailexcite.com)

Last night I added some air core chokes to my tesla

coil circuit. The RF chokes were air core wound on 1"

PVC. The wire I used was #18 AWG stranded and

insulated. The first choke had about 100 turns of wire

on it. While winding the second one I ran out of wire

after about 70 turns. I placed the RF chokes on the

Between the caps and the transformer. When I powered

up, the gap fired normally but there was no output from

the top terminal! The air core choke with the lesser

number of turns became so hot that the rubber

insulation on the choke almosted melted. I tried to

tune the primary circuit to no avail. Everything else

seemed normal.

Can someone give me some ideas. I'm at a loss here.

When I removed the hot choke from the circuit, the coil

ran fine. The one choke (100 turns) is still in the

the circuit.

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: CAPACITOR

Date: Tue, 23 Dec 1997 16:35:43 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Joseph" <joseph@michianatoday.com>

Date: Tue, 23 Dec 1997 11:35:43 -0500

Subject: CAPACITOR

Would anyone have any ideas on who made this cap so I can try to trace it

down? Looking for size and if its an oil one? Listed just like below, send

company a request but so far no answer.

Thanks

Joe

15KVDC, 0.05ufd, Del Elec, thats how its listed in catalog.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: A few disjoint thoughts

Date: Tue, 23 Dec 1997 20:39:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: arcstarter@mailexcite.com

Date: Tue, 23 Dec 1997 14:39:04 -0600

Subject: A few disjoint thoughts

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I've been talking to Paul Marshall (aka fleischwulf)

about his air choke problem.  I can't see what is

wrong with the way he did this.  Any help would

be appreciated.  Thanks!

Some thoughts about coils and the like:

1) Seems like fine tuning of the primary resonant

frequency could be done by slightly twisting

or looping the cables which attach to the primary.

Looping one of these cables will add inductance and

therefore reduce the oscillation freq of the primary.

This might be useful for making slight, fine adjustments

although I haven't tried it.

Of course you are also wasting a few amp-turns

which could have been used to power the

secondary...

2) A 1991 Popular electronics article on building a

solid state tesla coil seems to actually be a magnifier.

This circuit used a flyback approack (iron core) to

produce a couple thousand volts at an adjustable

frequency (controlled by a 555 timer or the like...).

This voltage was then piped out of the control box

and routed to what amounts to a 5 gallon bucket with

1000 turns of wire on it.

Isn't this basically a maggy??

No - I'm not building one :)

3)   Paul and I elevated our coil up off the concrete

floor by about a foot or so.  We noticed absolutely no

change in performance.  This is odd since many people

on the list thought we would see great things by

doing this!  Any thoughts on why not??

4) I'm going to find one of those small twisted metal

wire and tinsel christmas trees, and put it on the top

terminal.  We'll make a flaming corona monster

out of it, and sing:

"Oh, tesla tree,

oh tesla tree,

How lovely are your branches..."

More later.  Merry Christmas!

-Bill the arcstarter

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: CAPACITOR

Cc:

Bcc:

X-Attachments:

At 04:35 PM 12/23/97 GMT, you wrote:

>From: "Joseph" <joseph@michianatoday.com>

>Date: Tue, 23 Dec 1997 11:35:43 -0500

>Subject: CAPACITOR

>

>Would anyone have any ideas on who made this cap so I can try to trace it

>down? Looking for size and if its an oil one? Listed just like below, send

>company a request but so far no answer.

>

>Thanks

>

>Joe

>

>15KVDC, 0.05ufd, Del Elec, thats how its listed in catalog.

>

No I dont have any info on this but just want to point out

a couple of things.

First of all a 15KVDC cap is only good for 7500 volts AC

and probably somewhat less in Tesla coil service.

Second .05 mfd is quite a bit of capacitance. It would take

a very large transformer to charge this big a cap.

You can address both issues by putting several of these caps

in series. 5 of them in series will have a pretty good voltage

rating and offer .01 mfd of capacitance which would be a good

match for a 15kv/60ma neon.

But how much do these cost? Probably quite a bit.

And what kind of dielctric are they?

Most high voltage caps are not suitable for pulse duty

in a Tesla coil. Ceramic caps are usable but prone to overheating.

Mylar caps are no good at all usually meant for use

as HV DC power supply filters.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Air Core RF Chokes

Cc:

Bcc:

X-Attachments:

At 04:09 PM 12/23/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Tue, 23 Dec 1997 10:09:18 -0600

>Subject: Air Core RF Chokes

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Last night I added some air core chokes to my tesla

>coil circuit. The RF chokes were air core wound on 1"

>PVC. The wire I used was #18 AWG stranded and

>insulated. The first choke had about 100 turns of wire

>on it. While winding the second one I ran out of wire

>after about 70 turns. I placed the RF chokes on the

>Between the caps and the transformer. When I powered

>up, the gap fired normally but there was no output from

>the top terminal! The air core choke with the lesser

>number of turns became so hot that the rubber

>insulation on the choke almosted melted. I tried to

>tune the primary circuit to no avail. Everything else

>seemed normal.

>

>Can someone give me some ideas. I'm at a loss here.

>When I removed the hot choke from the circuit, the coil

>ran fine. The one choke (100 turns) is still in the

>the circuit.

>

>Paul Marshall

Paul,

Not sure what your problem was but I will speculate a little.

First the inductance of your chokes were probably quite low.

Probably too low to be of any benefit. You didnt need to

use AWG 18 wire for them. You need them to carry milliamps

only. Not Amps. Plus the diameter is too small to get much

inductance. I would recommend some chokes using AWG 22 or 24 and

2 inch PVC.

Secondly having the chokes with different values disturbs

me. Could setup an imbalance in your system.

As some others pointed out you might have hit an unfortunate

resonance with your chokes. Thats seems the most likely answer.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: A few disjoint thoughts

Date: Wed, 24 Dec 1997 03:12:00 GMT

Organization: The Energy Machine Information System 504-733-3462

T>Coilers,

T> I've been talking to Paul Marshall (aka fleischwulf)

T>about his air choke problem.  I can't see what is

T>wrong with the way he did this.  Any help would

T>be appreciated.  Thanks!

If the choke happens to be series resonant with the main capacitor at

the frequency of the primary circuit, it will cause very large RF

currents to flow through the choke, instead of the primary coil.  this

will heat it up severely.

I have melted 1" PVC tube with less than 100 watts of power at 27 MHz

accidentally while trying to make an RF choke.  Removing turns of wire

will raise the resonant frequency, but that will cause the choke to  let

the RF energy pass through the choke instead of blocking it.  Adding

more turns is the thing to do in this case.  When the resonant frequency

of the choke and the main capacitor is lower than the primary resonance,

the choke will act as a choke and block RF from passing though it.

<snip>

T>4) I'm going to find one of those small twisted metal

T>wire and tinsel christmas trees, and put it on the top

T>terminal.  We'll make a flaming corona monster

T>out of it, and sing:

T> "Oh, tesla tree,

T>   oh tesla tree,

T>   How lovely are your branches..."

ROFLMAO!!!  I can just see it.....  <G>

Ralph

---

Props keep pilots cool...when they stop, pilots sweat!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: A few disjoint thoughts

Date: Wed, 24 Dec 1997 05:30:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Tue, 23 Dec 1997 21:30:48 -0800 (PST)

Subject: Re: A few disjoint thoughts

On Tue, 23 Dec 1997, Tesla-2 wrote:

>

> Coilers,

>  I've been talking to Paul Marshall (aka fleischwulf)

> about his air choke problem.  I can't see what is

> wrong with the way he did this.  Any help would

> be appreciated.  Thanks!

Just a thought: Could his 70 turn choke have been resonant

with some other component in the system? Thus the high

current in that choke and not the other 100 turn unit.

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Curious about Coil form

Date: Wed, 24 Dec 1997 05:46:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Michael Baumann <baumann@proton.llumc.edu>

Date: Tue, 23 Dec 1997 21:46:57 -0800 (PST)

Subject: Curious about Coil form

I think I may have either hit the jackpot, or am begging

trouble. A buddy that owns a landscaping business told me he

had a 30" chunk of 6" ABS drainpipe. Great says I, thanks so much :)

Well, I got the pipe yesterday, and lo and behold, he is wrong, it is

not ABS, but instead HDPE! Can you say low-loss coil form? I knew you

could!

Or is it? Problem is that it is black inside, with a 3-4 mil white

coating. I did the following tests, to see if the black was carbon

based (I think not) but would love to hear some second opinions, even if I

ignore them :)

Test 1: Ohmmeter leads dug into the edge, approx 2mm apart - meter

reads open circut.

Test 2: 15KV probes on inside - no corona around the tips until they

were forming a corona between two high points in air, not to the

plastic

Test 3: wall thickness is 0.150", with sharp point directly opposite

heavy corona, but no arc.

Think I am safe to make this into a form???

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Curious about Coil form

Date: Wed, 24 Dec 1997 15:06:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 24 Dec 1997 07:06:15 -0800

Subject: Re: Curious about Coil form

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Michael Baumann <baumann@proton.llumc.edu>

> Date: Tue, 23 Dec 1997 21:46:57 -0800 (PST)

> Subject: Curious about Coil form

>

> I think I may have either hit the jackpot, or am begging

> trouble. A buddy that owns a landscaping business told me he

> had a 30" chunk of 6" ABS drainpipe. Great says I, thanks so much :)

>

> Well, I got the pipe yesterday, and lo and behold, he is wrong, it is

> not ABS, but instead HDPE! Can you say low-loss coil form? I knew you

> could!

>

> Or is it? Problem is that it is black inside, with a 3-4 mil white

> coating. I did the following tests, to see if the black was carbon

> based (I think not) but would love to hear some second opinions, even if I

> ignore them :)

>

> Test 1: Ohmmeter leads dug into the edge, approx 2mm apart - meter

>         reads open circut.

>

> Test 2: 15KV probes on inside - no corona around the tips until they

>         were forming a corona between two high points in air, not to the

>         plastic

> Test 3: wall thickness is 0.150", with sharp point directly opposite

>         heavy corona, but no arc.

>

> Think I am safe to make this into a form???

>

> --

> Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

> San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

>

>

Michael,

Excellent experimental method! While test 1 would likely be inconclusive

for high voltatge application, tests 2 and 3 seem to indicate that,

whatever the coloring agent, it doesn't seem to seriously affect HDPE's

insulating capabilities. Go for it!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: cap voltages

Date: Wed, 24 Dec 1997 15:20:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 24 Dec 1997 07:20:12 -0800

Subject: Re: cap voltages

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Federico" <cfed@dynasty.net>

> Date: Tue, 23 Dec 1997 03:48:29 +0000

> Subject: cap voltages

>

> True or False.

>

> The voltage handling capacity of a set of caps wired in series is the

> sum of each individual caps voltage rating.  (10, 1.5kv caps = 15kv

> handling capacity).

>

> And the cap/volt rating should be at least 1.414x the transformer's

> secondary output.

>

> Thanks All, Chris

>

Chris,

Ignoring leakage currents, capacitors in series will equally share the

total voltage ONLY if they are identically sized. The best bet is to use

identical capacitors so that their other characteristics are also

matched (leakage current, equivalent series resistance, dielectric loss,

temperature coefficient, ...). If you use different valued caps, the one

having the smallest capacitance will see the greatest voltage across it.

Regarding voltage rating, the multiplier should be at least 3-4X the

transformer's RMS output rating. The greater the multiplier, the more

safety margin you'll have. A capacitor subjected to Tesla Coil service

really gets "hammered" (NPI Mike!)! A typical starting point is to take

the peak-to-peak voltage (2.82X RMS), and increase it a bit more. For 15

KV neon service this means 45 - 60 KV. Ideally, the capacitor should be

rated (and spec'ed for) AC service.

Hope this helps!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: *TC Survey* - Progress Update

Date: Thu, 25 Dec 1997 07:09:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Thu, 25 Dec 97 02:09:41 -0500

Subject: *TC Survey* - Progress Update

I am hard at work on formatting the survey data as I write this.  As a

sign of good faith, I am giving you the address so you can check out the

progress of the survey results page.  Right now, I am building the table

of contents, which has quite a bit of useful data all by itself.

At the address below, you will find the data table which will serve as

the table of contents to the complete survey results.  I have completed

the table for the 0-3kVA setups, with the results from my surveys only.

Next, I will be adding the 3kVA+ systems and an extra table of data from

previous surveys and listserv postings.  No proofing/checking has been

done, so there may be errors.

Go here:    http://www.tiac.net/users/absmith/tesla/tcresults.html

I want feedback, guys!!!! It's not too late for additions and

revolutionary ideas.  I want this survey to serve the Tesla Coiling

community as best it can.

Thanks,

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary frequency/coupling

Date: Sat, 27 Dec 1997 05:19:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 27 Dec 1997 05:19:02 +0000

Subject: primary frequency/coupling

I think I know the answer to this question, but I've thought that in

the past, and found out otherwise.  So, does the shape of the primary

have any effect on it's frequency?  Or, are there any other physical

characteristics that effect freq.?  (I was thinking of re-doing my

primary, to help solve my overcoupling problem, and was hoping there

would be some side benefit to doing so.)

Thanks. . .CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitors

Date: Sat, 27 Dec 1997 05:19:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 27 Dec 1997 05:19:02 +0000

Subject: Re: Capacitors

>      Also, are these wire ties

>      wrapped around the stacks?  do you have a problem with the centers

>      bulging?

I used plastic zip straps to hold my caps together, and noticed that

they bulged in the middle.  So what I did was cut 3/8" strips of

acrylic, slid them under the straps, then rotated them up on edge,

and forced them out toward the edges of the caps (as far as they

would go with the finger pressure I could exert).  Then I made

spacers, also out of acrylic, to hold them in place.  Works pretty

well.

Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: variac/cap charge

Date: Sat, 27 Dec 1997 19:18:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 27 Dec 1997 19:18:17 +0000

Subject: variac/cap charge

Something I noticed the first time I ran my TC using the variac.

When I was trying to discharge my caps I kept getting small arcs.

This was with the variac turned down to zero, but still pluged in.

After I unplugged it I only got one last arc from the caps when

discharged.

Is this sort of voltage leakage to be expected?

CF

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary frequency/coupling

Date: Sat, 27 Dec 1997 19:18:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 27 Dec 1997 19:18:17 +0000

Subject: Re: primary frequency/coupling

From the responses that Mike H. and Adam posted I think I would lower

the frequency of the primary if I raised its inductance (and the

frequency of my primary seems to be too high to properly resonate

with my sec').  I would do the math to test my assumption, but I

don't know how to determine the mh of a primary coil.

And since I can't, here are the specs:  13"d, 15 turns, 1/4"

copper-1/4" spaced primary.  .06 mfd cap.  The sec' is 6.25"x21,

24 gauge wire, 8"alum. sphere topload.

As to the over-coupling issue. . .I understand about raising the

sec', but from a purely esthetic viewpoint I'd prefer not to have to

raise it 4" to eliminate the problem (which is what it takes, 4").

Thanks,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hand in pocket

Date: Sat, 27 Dec 1997 19:18:17 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Sat, 27 Dec 1997 19:18:17 +0000

Subject: Re: hand in pocket

> I am looking for the statement that Tesla made about working with one hand in

> your pocket when around high voltage.

Although I can't help with your search, I thought I might relate the

response I got from the manager of the neon shop when I asked him his

opinion about such a practice.  He said, "It doesn't make a lick of

difference whether the voltage goes across your heart or not.  If

enough juice hits you it will cook your muscles, and later cause

gangrene that can kill you."  He also stated that he had seen it

happen where a worker took a hit, got up, said he was just a little

"achey" then ended up dead several days later.

FWIW,  Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

From: tesla-2@emachine.com (Tesla-2)

Subject: New Tranny questions

Date: Sun, 28 Dec 1997 17:48:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com (Ronnie R Pittman)

Date: Sun, 28 Dec 1997 12:48:18 -0500

Subject: New Tranny questions

Hey Mike and all,

Well I am finally coming out of my tesla dormancy :).  For christmas this

year,  santa claus (A.K.A. my wife)  brought me a used 15 kv 30 ma

transformer.  Needless to say I was thrilled.  The next day I went back

and talked to them for a while and bought another transformer for another

25 dollar.  These were both the old jefferson variety and both working.

They also gave me a transco 15kv 30 ma ttransformer for free b/c it had

one of the hv insulators broken off.  I tested them all by drawing arcs

from the case and all 3 had both sides working.  I finally know where I

can get pretty much as many as I want (only problem is that it is down in

miami near my parents house which is 4.5 hours away.  (Not too bad a

price to pay)

Anyway with these new acquisitions brings a whole slew of

questions.  Here goes.

1.  What would be the best thing to use on the one with the broken

insulator to kee[ it from shorting to the case?  I thought about

silicone, epoxy, or hot glue but was not sure which would be best and

easiest to use.

2.  If I want to connect them in parrelel to get 15kv and 90 ma do I just

connect allthe hv leads in parralel or is there something else I need to

know?

3. Now I am really needing to build those plate caps.  Mike I know you

sent me some dimensions before but that was when I was planning to use my

old 6 kv tranny.  What would you suggest to handle up to 15 kv and maybe

120 mz if I ever get another tranny.?

4.  I am also wondering if I should rebuild my primary.  I have 50 ' of

3/8 " copper tubing that I have not used.  I was planning on making the

30 degree rise variety.  Currently I have on the coil a helical primary

made out of 10 ga solid copper wire. Any suggestions?

5.  Out of curiosity, if use the rf chokes on the ferite cores, and a

safety gap that fires occasionally, is it likely that I will be able to

keep the trannys  working for a pretty good while?  Is there anything

else I can do to try to keep them alive?

6.  I have the  RQ spark gap with the 2" electrodes in a 6" pipe.  Do you

recomend building another one of these to maybe spread out the spark gap

and increase quenching?

Well I guess that is all I have time for right now.  Once again

any and all help is extremely appreciated.

Adios.

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Tranny questions

Cc:

Bcc:

X-Attachments:

At 05:48 PM 12/28/97 GMT, you wrote:

>From: rpittman@juno.com (Ronnie R Pittman)

>Date: Sun, 28 Dec 1997 12:48:18 -0500

>Subject: New Tranny questions

>

<SNIP>

> Anyway with these new acquisitions brings a whole slew of

>questions.  Here goes.

>

>1.  What would be the best thing to use on the one with the broken

>insulator to keep it from shorting to the case?  I thought about

>silicone, epoxy, or hot glue but was not sure which would be best and

>easiest to use.

I would try to build up a new insulator from plexiglass or lexan.

Failing that the next best thing would be silicone RTV.

>

>2.  If I want to connect them in parrelel to get 15kv and 90 ma do I just

>connect all the hv leads in parallel or is there something else I need to

>know?

Connect the HV leads in parallel. Also connect the primaries in parallel

observing the correct phasing. If you get a heavy spark when bringing

the HV leads together in parallel then switch around one of the sets

of primary leads. When phased properly you should get little or no

spark when bringing the HV leads together.

>

>3. Now I am really needing to build those plate caps.  Mike I know you

>sent me some dimensions before but that was when I was planning to use my

>old 6 kv tranny.  What would you suggest to handle up to 15 kv and maybe

>120 mz if I ever get another tranny.?

15kv is really rough on caps and the only sure way to

build a reliable cap for that voltage is to run at least 2 caps

in series. I recommend 2 series caps for 12kv and 3 series caps

for 15kv. These can be individual units or housed in the same

container. I would use 24 mils of dielectric (4 X 6 mils) for

each dielectric layer and 3 caps in series giving the equivalent

of 72 mils. This is plenty when using 3 series caps.

You want about .0053 mfd of capacitance for each 15kv/30ma neon

you use. Thats means each cap in the series string should have

about .0159 mfd of capacitance.

I would build a flat plate cap with three .0159 mfd sections in series.

Put all three sections in the same tank. Use this with one of

your 15kv/30ma neons. Build a second identical cap and use it in

parallel with the first to give you .0106 to make best use

of 15kv/60ma.

>

>4.  I am also wondering if I should rebuild my primary.  I have 50 ' of

>3/8 " copper tubing that I have not used.  I was planning on making the

>30 degree rise variety.  Currently I have on the coil a helical primary

>made out of 10 ga solid copper wire. Any suggestions?

Your AWG 10 primary is more than adequate for a single 15kv/30

and probably OK for 15kv/60. 3/8 inch copper tubing is overkill.

1/4 inch tubing is good up to several thousand watts.

I would use your current primary as is. Put your effort into

good quality caps and a good gap system.

>

>5.  Out of curiosity, if you use the rf chokes on the ferrite cores, and a

>safety gap that fires occasionally, is it likely that I will be able to

>keep the trannys working for a pretty good while?

Yes. That should keep the RF to a manageable level and the safety gap

will prevent overvoltage conditions.

>  Is there anything else I can do to try to keep them alive?

Yes. Make sure your circuit has the spark gap across the transformer.

You do NOT want the cap across the transformer.

Keep run times short. 5 minutes may seem like a short time to you

but its an eternity to a neon in a Tesla coil. Keep runs to

1 or 2 minutes maximum or so with similar off periods for cooling off.

Watch the maximum gap. Over wide gaps are probably the number one cause

of neon failures. I feel the maximum gap for 15kv should be about .300

inch and .250 for 12kv.

>

>6.  I have the  RQ spark gap with the 2" electrodes in a 6" pipe.  Do you

>recomend building another one of these to maybe spread out the spark gap

>and increase quenching?

Depends on the number and spacing of the electrodes. 10 gaps

spaced about .03 inch is the maximum for 15kv.

Use your existing gap and build another later if necessary.

TTYL!

Mike Hammer

mhammer@misslink

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: plans&cap

Cc:

Bcc:

X-Attachments:

At 06:16 PM 12/29/97 GMT, you wrote:

>From: torghn@juno.com (Jared Neilsen P Neilsen)

>Date: Mon, 29 Dec 1997 11:16:00 -0700

>Subject: plans&cap

>

>I am very new to the tesla coil hobby and new to this list, this will be

>my first post.  because I am new to this I really need some help,  first

>off there are a ton of generic plans out there for building tesla coils,

>what I am interested is a very specific plan.  So if any one you have or

>know where I can get plans to a tesla coil that are a little more

>specific than the normal stuff out ther pleas let me know.

Jared,

Most of the "plans" you see are generic because a Tesla coil

is pretty much a generic device. Specific plans arent really

practical as you will want to build your coil around parts you

have or can get easily.

The first step for you is to find a suitable transformer.

This is most often a neon sign transformer. You want neons

with voltage ratings of 9, 12, or 15kv and current ratings

of 30 or 60 ma. The bigger transformers produce higher power

levels. Since this is a first coil for you I recommend getting

a 9kv/30ma neon transformer. But you may have to take what you

can get so if a 12kv/30ma neon comes your way then youll want

to build your coil around that.

Most of the rest of the coil will be determined by your transformer

so you need to get that first.

The method I recommend is to develop your own "plans" around

the materials you have available. Tell us what size transformer

you have or are able to find and then we'll work out some

guidelines for a suitable coil.

>

>There has also been a lot of talk about capacitors lately on this list.

>Some people have recommended some capacitors that they say work great,

>where can I get the plans for this capacitor?  Any and all help would be

>greatly appreciated.

>

Take a look at Dave Bronakowsi's Tesla coil archive site.

There you will find files on building many parts of a coil.

The address is: http://www.velocity.net/~djb/tesla.shtml

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New Tranny questions

Date: Sun, 28 Dec 1997 22:29:24 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Sun, 28 Dec 97 17:29:24 -0500

Subject: Re: New Tranny questions

Good grab! Here are a few suggestions

>  Anyway with these new acquisitions brings a whole slew of

>questions.  Here goes.

>1.  What would be the best thing to use on the one with the broken

>insulator to keep it from shorting to the case?  I thought about

>silicone, epoxy, or hot glue but was not sure which would be best and

>easiest to use.

Red Devil Clear acrylic caulk works fine, and silicone will probably also

work OK.  If you can, try to find some plexiglas tubing or similar item

to replace the bushing with.  Then caulk it to death.

>2.  If I want to connect them in parrelel to get 15kv and 90 ma do I just

>connect allthe hv leads in parralel or is there something else I need to

>know?

You need to get them in phase, and you need to check that they are all

putting out the correct nameplate voltage.  If there are internal shorts

in the secondary, it may not be obvious from looking at the arc, but it

will unbalance the transformers when paralleled.

To get the transformers  in phase, connect two of them to the mains, then

connect together one HV terminal from each tranny, and see if you can

draw an arc between the remaining two HV terminals.  If you can, they are

out of phase, so switch the primary connections on one of them, or

physically reorient them.  Repeat with the third transformer.

>3. Now I am really needing to build those plate caps.  Mike I know you

>sent me some dimensions before but that was when I was planning to use my

>old 6 kv tranny.  What would you suggest to handle up to 15 kv and maybe

>120 mz if I ever get another tranny.?

I would not recommend building plate caps.  I've done it and quite

frankly, it sucked.  I ended up with an extrememly robust and low

inductance cap, but it's huge, heavy and difficult to get leak-proof.

I would recommend you build 4 identical 0.01 uF rolled caps, each using a

total of 60 mil LDPE dielectric.  I have built two of these, and they

each handle 10kVAC tesla survice, no problem.  You would wire them in

series-parallel for use with your 15kV units.   The dimensions for caps

in 4" PVC drain pipe (what I used) are:  4 plys 30mil LDPE per cap, each

12" x 72".  10" wide aluminum flashing 72" long, two of these per cap.

This rolls up to fit in a 4" ID drain pipe and requires about 40 oz. of

mineral oil to fill.  If you buy all the parts for 4 caps, you will get a

very low cost per capacitor.

My costs (in Boston) would be about $100 for material for all four caps.

If you want the cost breakdown, email me.

>4.  I am also wondering if I should rebuild my primary.  I have 50 ' of

>3/8 " copper tubing that I have not used.  I was planning on making the

>30 degree rise variety.  Currently I have on the coil a helical primary

>made out of 10 ga solid copper wire. Any suggestions?

The tubing should give you a noticeable bit of improvement.  10 ga wire

is puny for this kind of application.

>5.  Out of curiosity, if use the rf chokes on the ferite cores, and a

>safety gap that fires occasionally, is it likely that I will be able to

>keep the trannys  working for a pretty good while?  Is there anything

>else I can do to try to keep them alive?

You can try to close your main gap a bit.  If this does not stop the

safety gap firing, your chokes are probably ringing.  Add some resistance

in series with the chokes.  About 250-1000 ohms @ 100W should do the

trick.  The resistance is just to kill the Q of the choke, so it's value

is not critical.  Use only as much as is needed.

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitors

Date: Mon, 29 Dec 1997 10:14:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Harvey" <jeff@techgallery.com>

Date: Mon, 29 Dec 1997 10:14:44 +0000

Subject: Re: Capacitors

Organization: Life & Energy Systems

>   I have built 4) 48 plate capacitors. Each cap gives

> me about .026 mf. The caps are in parallel and seperated into 2 units the units

> are in series. Each cap cost about $25.00. This circuit handles 15 Kv

> easly. I found four rubber maid containers which fit the caps very nicely.

> The cores are held together with wire ties and submerged in transformer oil.

> Be careful they will hold a charge long after they have been disconnected.

> They can discharge enough energy to kill you. Believe me they are extremely

> efficient low loss caps. My caps have never leaked, they did vent a little bit > when I fired them up the first time. I used 60 mil poly and I can't say enough > about their  performance.

> They are great.

> ---

> Einigkeit und Freiheit

>

> Can you give any construction info on these caps? I would sure appreciate any

> more information. Thanks Jeff

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitors

Date: Mon, 29 Dec 1997 13:24:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 29 Dec 1997 07:24:40 -0600

Subject: Capacitors

Organization: MailExcite  (http://www.mailexcite.com)

Adam,

I have built 4) 48 plate capacitors. Each cap gives

me about .026 mf. The caps are in parallel and seperated

into 2 units. the units are in series. Each

cap cost about $25.00. This circuit handles 15 Kv

easly. I found four rubber maid containers which fit

the caps very nicely. The cores are held together with

wire ties and submerged in transformer oil. Be careful

they will hold a charge long after they have been

disconnected. They can discharge enough energy to kill

you. Believe me they are extreamly efficiant low loss

caps. My caps have never leaked, they did vent a little

bit when I fired them up the first time. I used 60 mil

poly and I can't say enough about their preformance.

They are great.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: (No Subject)

Date: Mon, 29 Dec 1997 13:58:59 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 29 Dec 1997 07:58:59 -0600

Subject: (No Subject)

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I finally got some 48" streamers. My coil is powered

via a 25 kva pole pig which is current limited by

inductive ballasting to give me about 3.5 kw. My run

times have been limited to approximatly 30 seconds.

The streamers are very hot. One run arced over to a

utility hook in the basement. Shortly I smell burning

wood. The hook was 5' above floor and some how the

strike made it to ground ! There were charred two by

fours just behind the hook. I thing I had better build

a farraday cage ! I would hate to tell someone how I

burned the house down !

Thanks to everyone who helped and for all of the

great data I obtained from this listing !

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Preformance improvment

Date: Mon, 29 Dec 1997 14:14:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 29 Dec 1997 08:14:14 -0600

Subject: Preformance improvment

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

One of the greatest improvement made to my coil

was ducting air into the spark gap. This reduced to

power drain to the circuit and gave me longer

streamers. The sreamers went from 45" to a woping 48".

Safe coiling for ever !

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitors

Date: Mon, 29 Dec 1997 14:56:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

Date: Mon, 29 Dec 1997 09:56:00 -0500

Subject: Re: Capacitors

Do you still have the P/N for those Rubbermaid containers.  I've idly

perused the selection at several variety and food stores, and haven't

found anything I felt would be suitable.  Also, are these wire ties

wrapped around the stacks?  do you have a problem with the centers

bulging?

Aric

______________________________ Reply Separator ________________________________

Subject: Capacitors

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    12/29/97 1:24 PM

From: fleischwulf@mailexcite.com

Date: Mon, 29 Dec 1997 07:24:40 -0600

Subject: Capacitors

Organization: MailExcite  (http://www.mailexcite.com)

Adam,

I have built 4) 48 plate capacitors. Each cap gives

me about .026 mf. The caps are in parallel and seperated

into 2 units. the units are in series. Each

cap cost about $25.00. This circuit handles 15 Kv easly.

I found four rubber maid containers which fit the caps

very nicely. The cores are held together with wire ties

and submerged in transformer oil. Be careful they will

hold a charge long after they have been disconnected.

They can discharge enough energy to kill you. Believe me

they are extreamly efficiant low loss caps. My caps have

never leaked, they did vent a little bit when I fired

them up the first time. I used 60 mil poly and I can't

say enough about their preformance. They are great.

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re: Capacitors

Date: Mon, 29 Dec 1997 16:14:48 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Mon, 29 Dec 1997 10:14:48 -0600

Subject: Re: Re: Capacitors

Organization: MailExcite  (http://www.mailexcite.com)

Reguarding your post on caps,

The rubbermaid containers are approximatly 14 x 10 x 6

They are sold at K-mart in a package of two. They cost $8.49 and seal very well.

The stack or core is flat and fits the container perfectly. The wire ties are

tightened by hand and do not cause any bending. The plate size is 12 x 8. This will give you 48 plates when cut from a 4' x 8' sheet.

The cost per 4' x 8' sheet is $17.00. Use heavy duty aluminum foil.

This works very nicely. Leave 6" or so for tabs. When the stack is

complete pull the tabs together and connect them with

a 1/4 - 20 bolt and washer. I found the aluminum

flashing to be too stiff for this application.

Paul Marshall

---

Einigkeit und Freiheit

>From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

>Date: Mon, 29 Dec 1997 09:56:00 -0500

>Subject: Re: Capacitors

>

>     Do you still have the P/N for those Rubbermaid containers.  I've idly

>     perused the selection at several variety and food stores, and haven't

>     found anything I felt would be suitable.  Also, are these wire ties

>     wrapped around the stacks?  do you have a problem with the centers

>     bulging?

>

>     Aric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: plans&cap

Date: Mon, 29 Dec 1997 18:16:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: torghn@juno.com (Jared Neilsen P Neilsen)

Date: Mon, 29 Dec 1997 11:16:00 -0700

Subject: plans&cap

I am very new to the tesla coil hobby and new to this list, this will be

my first post.  because I am new to this I really need some help,  first

off there are a ton of generic plans out there for building tesla coils,

what I am interested is a very specific plan.  So if any one you have or

know where I can get plans to a tesla coil that are a little more

specific than the normal stuff out ther pleas let me know.

There has also been a lot of talk about capacitors lately on this list.

Some people have recommended some capacitors that they say work great,

where can I get the plans for this capacitor?  Any and all help would be

greatly appreciated.

Jared Neilsen

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:coil size vs power

Cc:

Bcc:

X-Attachments:

At 02:47 PM 12/30/97 GMT, you wrote:

>From: fleischwulf@mailexcite.com

>Date: Tue, 30 Dec 1997 08:47:51 -0600

>Subject: (No Subject)

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Mike,

>

> Bill Pollack and I have achieved a max streamer length of 48" thanks to

> you and the people on Tesla-2. My question is: Is there a limit to the

> possible streamer length produced by a 4" diameter coil. I think there

> must be and I think you have achieved it.

> What is the limiting factor in coil design to be specifc as it relates

> to coil diameter ?

>

Yes there is a limit to the amount of power that a specific size secondary

can handle. It really depends on how much power your secondary can absorb

before breaking down. The turn to turn voltage can get high enough to break

down the insulation and streamers will tend to want to break out of the

secondary winding itself. I have had these problems several times.

Increasing toroid size and reducing the coupling slightly both help.

My coil is at the point that it needs the loading from the streamers.

I use a foil bump on my toroid to promote breakout in one direction.

If I remove this bump so as to delay breakout I get secondary breakdowns

all over the place. Once streamers form things settle down.

I am pretty much at the limit for my coil.

Note that I am at this limit running considerably less power than you are.

My transfomer runs 12720 V and 105 ma with .0156 mfd of capacitance.

This results in 48 inch streamers with occasional hits in excess of 50 inches.

What you probably need more than anything else is a larger toroid.

>

>

>Second question: When Bill wound the coil, he allowed extra windings. The >winding length is approximately 26 inches. Will the extra windings effect coil >performance in a negative way ?

No. The extra turns give your secondary a lower resonant frequency allowing

you to use more primary capacitance and inductance. In fact a large toroid

to decrease the frequency further will improve performance in several ways.

First a lower frequency means lower losses.

The larger toroid will allow for higher voltages before breakout.

And the lower frequency will allow for more primary inductance which

will improve gap operation.

>

>

>

>Bill and I are planing a second coil. We are hoping to build a 6" diameter >coil. We will probably increase the capacitance to .036 uf and keep the

>power level to 3.5 Kw. Provided that we do everything well and we are able

>to operate at the same level of efficiency, what should we expect as far as >streamer length ?

>

Starting at about 4 foot of spark up to maybe 6 feet.

Before moving up to a larger coil I would experiment with your 4 incher

some. Try to get more spark with less power. My results show it can be

done. I would try more toroid and more primary inductance. My coil works

better the more primary turns I can get tapped in.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Tranny questions

Cc:

Bcc:

X-Attachments:

At 04:06 PM 12/30/97 GMT, you wrote:

>From: rpittman@juno.com (Ronnie R Pittman)

>Date: Tue, 30 Dec 1997 11:06:41 -0500

>Subject: Re: New Tranny questions

>

>

>>Good grab! Here are a few suggestions

>

>        Thanks!

>

>

>>I would not recommend building plate caps.  I've done it and quite

>>frankly, it sucked.  I ended up with an extrememly robust and low

>>inductance cap, but it's huge, heavy and difficult to get leak-proof.

Most people with big unwieldly plate caps have built them into some type

of pre-existing container. While this works this often leads to the cap

being larger than necessary and requiring much more oil than necessary.

Ive built my own tanks for my flat plate caps and had good results.

I built my first tanks from plywood with a plexiglass insert. RTV silicone

was used to attach the plexiglass and a bead of RTV was run along all seams

to seal them. A plexiglass cover is attached with screws to the top of

the tank.

Later tanks disposed of the plexiglass entirely. I coated the interior of

the plywood tanks with two coats of polyester resin. This was quicker and

easier and less costly.

The advantage of building your own tank is that it can be sized perfectly

for the aplication and minimize the oil requirement.

>>

>>I would recommend you build 4 identical 0.01 uF rolled caps, each using a

>>total of 60 mil LDPE dielectric.  I have built two of these, and they

>>each handle 10kVAC tesla survice, no problem.  You would wire them in

>>series-parallel for use with your 15kV units.   The dimensions for caps

>>in 4" PVC drain pipe (what I used) are:  4 plys 30mil LDPE per cap, each

>>12" x 72".  10" wide aluminum flashing 72" long, two of these per cap.

>>

>>This rolls up to fit in a 4" ID drain pipe and requires about 40 oz. of

>>mineral oil to fill.  If you buy all the parts for 4 caps, you will get a

>>very low cost per capacitor.

>>My costs (in Boston) would be about $100 for material for all four caps.

>>If you want the cost breakdown, email me.

>

>      I still have not heard the end to this debate nor know enough

>about either to decide.  I like the apparent sturdiness of the rolled

>type,  it does not seem like the plates will slide around or anything.

>It would also seem like it is easier to build instead of having to try to

>keep all the plates stacked together.  The down side seems to be the cost

>and the vacuuming that has to be done.  From what I understand the most

>expensive part is the aluminum flashing.  ( I bought a 10' x 100' roll of

>6 mil plastic yesterday which I hope will be plenty) .  Would it be

>possible and feasible to make a rolled cap out of H.D. aluminum foil and

>put that in the drain pipe?  If so how would you do it?

Yes. I've built both flat plate caps and rolled caps using both foil

and flashing. I much prefer foil. Its much less costly and far easier

to roll. You have to be a bit careful as its easy to crinkle it.

Get the Reynolds Heavy Duty foil. Avoid store brands as they are often

already wrinkled on the roll.

I made my rolled caps by winding the cap around a length of 3/4 inch

copper tubing as a center hub. First solder a length of heavy AWG 8

wire around one end of the copper pipe. Then wrap the inner foil tightly

around the copper pipe several times and tape down solidly. The copper

pipe makes an excellent low inductance connection to the inner foil.

The length of copper wire soldered to the pipe is the connection to

the outside.

Wind up the cap tightly as you can. To make the connection to the outer

foil I take a length of foil and fold it over and over several times

to make a thick strap about 1 inch wide. I then fold this strap up

in the end of the outer foil several times tightly and then tape

this down to the outside of the cap. The strap then runs along the

side of the cap to the top for the other connection to the outside.

This construction method I got from Fr. Tom McGahee.

Check out the file by Fr. Tom on Dave Bronakowski's archive site.

The file describes a method for producing series connected roll

caps on the same roll but the construction method is useful for

single caps as well. The file name is multirol.txt and the URL

for dave's site is: http://www.velocity.net/~djb/tesla.shtml

Rolled caps like this are quick and relatively easy to build. But they

really need to be vacuum pumped to remove trapped air.

Flat plate caps with the plates oriented vertically will allow the

air to escape and will provide the best pulse performance.

My rolled caps work quite well but are definitely inferior

to my plate caps. The plate caps require much more time and effort

to build but I believe they are worth the effort.

The plates can be held together firmly by wedging inserts between the

tank walls and the caps. And the biggest advantage to plate caps

is that you can easily add more plates later to increase the capacitance.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: plans&cap

Date: Mon, 29 Dec 1997 19:41:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: baumann@proton.llumc.edu

Date: Mon, 29 Dec 1997 11:41:36 -0800 (PST)

Subject: Re: plans&cap

On 29 Dec, Tesla-2 wrote:

> So if any one you have or

> know where I can get plans to a tesla coil that are a little more

> specific than the normal stuff out ther pleas let me know.

>

> There has also been a lot of talk about capacitors lately on this list.

> Some people have recommended some capacitors that they say work great,

> where can I get the plans for this capacitor?  Any and all help would be

> greatly appreciated.

>

> Jared Neilsen

Jared: hit this URL, it has a number of very helpful sections for

the beginner, as well as a link into the tesla ring:

http://www.eskimo.com/~billb/tesla/tesla.html

--

--

Michael Baumann  Optivus Technology Inc.|Loma Linda University Medical Center

San Bernardino, California. (909)799-8308 |Internet: baumann@llumc.edu

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: variac/cap charge

Cc:

Bcc:

X-Attachments:

At 07:18 PM 12/27/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Sat, 27 Dec 1997 19:18:17 +0000

>Subject: variac/cap charge

>

>Something I noticed the first time I ran my TC using the variac.

>When I was trying to discharge my caps I kept getting small arcs.

>This was with the variac turned down to zero, but still pluged in.

>After I unplugged it I only got one last arc from the caps when

>discharged.

>

>Is this sort of voltage leakage to be expected?

>

>CF

>

Glad you discovered this little gotcha in this way.

Many variacs do not go completely to zero volts when

set at minimum. They can put out 1 or even 2 volts.

Even 1 volt on the primary of a 15kv neon can result

in 125 volts output. And your cap can charge to 1.414 times

this value or 176 volts. Enough for a nasty shock.

Put a heavy duty power switch in the primary of your variac

with a power ON indicator. Get in the habit of checking this

indicator and turn the power OFF before working on your coil.

Live long and prosper.

TTYL!

Mike Hammer

mhammer@misslink

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: transformer

Cc:

Bcc:

X-Attachments:

At 07:10 PM 12/30/97 GMT, you wrote:

>From: torghn@juno.com (Jared Neilsen P Neilsen)

>Date: Tue, 30 Dec 1997 12:10:23 -0700

>Subject: transformer

>

>Thanks for the info on the plans for a testacy (?) coil,

(hope you mean Tesla coil)

> now I'm left with finding a transformer. Do any of you know of a good

> place to get them?

> I don't even know where to begin.

>

The best place to begin is with your local yellow pages.

Look under signs and mark down the addresses of all the sign

companies in your area. Make special note of any that advertise

service and neon signs.

Do not call them on the phone. That makes it too easy for

them to just say no to get you off the phone. Go in person.

Ask for used or dead neon sign transformers. Take whatever you can get.

Many times dead transformers will turn out to be just fine.

Dead neons should be free or at most a couple bucks apiece.

Used known good transformers range from $10-30 for 30ma units

and $20 on up for 60ma ones. Do not buy new transformers. Too

expensive.

If you dont have any worthwhile sign shops in your area travel

to a nearby larger town. Ive hit every sign shop within a 75 mile

radius of me.

If you completely wash out on a neon search then try furnace

and heating contractors for oil burner ignition transformers. (OBIT)

OBITs have very little resale value so often go to the scrap heap.

You may have to crawl over a pile of junk to remove a dirt grimy

transformer from an old furnace but remember this is fun.

Junk piles and trash dumpsters are a prime source of coil parts.

OBITs generally are not as robust as neons not meant for continous

duty operation but work rather well for a small coil. Typical

ratings are 6kv/20ma and 10kv/23ma

Keep your eyes open. Ive seen neons at flea markets, junk stores

and hamfests.

good hunting.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: plans&cap

Date: Tue, 30 Dec 1997 00:35:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Mon, 29 Dec 1997 19:35:03 EST

Subject: Re: plans&cap

Organization: AOL (http://www.aol.com)

Jared,

As stated by Mike Hammer,  you are better advised to design your own coil

rather than use a specific one.   If you need to better understand theory

involved, a pretty good reference for this(including design of coils"made

easy"), you may wish to obtain the "TESLA COIL CONSRUCTION GUIDE" .  You can

get this by writing to :

John H. Couture

10823 New Salem Point

San Diego, CA 92126

This manual is a decent reference.  I have compared it to many and found

that most are similar in content.  It allows designs to be made without

needing to know math to much extent.  I verified all the equations used and

found them to be correct.  It also gives some good safety advise and some

theory that is not difficult for the beginner to understand. This manual also

has several plans for coils as you had asked for in your letter.  The plans

range from a little 70kV  coil that uses a car coil as the power source (about

50VA) to a 1500kV coil that uses the pole type transformer (5kVA).  It is a

straightforward manual for the beginner.  This manual costs about $20(at least

it did about three years ago).

Good Luck and LET THE SPARKS FLY!!!

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: choke parts

Cc:

Bcc:

X-Attachments:

At 04:11 PM 12/30/97 GMT, you wrote:

>From: "Chris Federico" <cfed@dynasty.net>

>Date: Tue, 30 Dec 1997 16:11:41 +0000

>Subject: choke parts

>

>2 questions about parts for RF chokes:

>

>1.  Will wiring 2-150 ohm/50w resistors in series give me appropriate

>values for use with my chokes?

>

>2.  Is a ferrite toroid with the dimensions 1.16"o.d., 9/16"i.d. and

>19/32" length be efective as cores?  Or, since they are smaller than

>the 2" recommended, could I double them up to increase the damping?

>

>Thanks. . .Chris

Chris,

You probably wont need the series resistance with your

toroidal chokes. They wont usually ring up like an air

core choke with many more turns of wire. I do not use

any series resistance with my toroidal chokes.

Your toroid cores are a bit small. If you have 4 of them

I would suggest putting two in series in each HV leg.

Try to get as many turns as you can on each core.

Insulate the cores with a couple layers of good vinyl

tape and wind as much wire as you can remembering to

leave enough gap between the first and last turns so it doesnt

arc across them. If you can manage about 50 turns on each

core and use two in series on each side I bet they will work fine.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary frequency/coupling

Date: Tue, 30 Dec 1997 05:59:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Tue, 30 Dec 97 00:59:42 -0500

Subject: Re: primary frequency/coupling

>I think I know the answer to this question, but I've thought that in

>the past, and found out otherwise.  So, does the shape of the primary

>have any effect on it's frequency?  Or, are there any other physical

>characteristics that effect freq.?

Chris, the shape of the primary affects its self inductance.  The

frequency at which your tank system resonates depends on the capacitor

value as well, the exact relationship being  F = 1 / (2*pi*sqrt(L*C))

Where F is frequency in Hertz, L is the primary inductance in Henrys

(typically under 10^-4), and C is your tank capacitance in Farads.

Notice that the units for the formula are whole Farads and Henry's, not

microhenrys and microfarads.

The "tighter" or "closer" you wind your primary, the more self inductance

it will have.  What  are the dimensions of your primary now?  What shape?

Most coils that are severely overcoupled have secondary-hugging

solenoidal primaries, which is not ideal for a two coil tuned system.

>(I was thinking of re-doing my

>primary, to help solve my overcoupling problem, and was hoping there

>would be some side benefit to doing so.)

Changing shape of the primary will affect coupling, as the shape and

strength of the primary's magnetic field will change.  The strength of

the field within the secondary coil can also be altered by moving it

towards or away from the primary coil.  As you move the coil away (raise

it up) the coupling will decrease.  By raising the secondary coil you can

loosen the coupling without changing the inductance of your primary,

making it easy to find the sweet spot without retuning each time.

If you are going from a solenoid primary to a flat spiral or inverted

conical coil, you will see a decrease in primary inductance for the same

amount of wire/tubing.  More turns will be required.  If you can give us

some specs for you coil, we can do some of the math for you and give you

some advice on what would make sense.  Primary, secondary and toroid

dimensions and spatial relationships, as well as tank capacitance and

supply transformer specs should do the trick.

Check out my Tesla Coil Survey results page too!  You can enter your coil

data there if you wish.

www.tiac.com/users/absmith/tesla/tcresults.html

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: hand in pocket

Date: Tue, 30 Dec 1997 07:13:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: clara02@sprynet.com

Date: Mon, 29 Dec 1997 23:13:47 -0800

Subject: hand in pocket

I am looking for the statement that Tesla made about working with one hand in

your pocket when around high voltage. In R. A. Fords, Homemade Lightning, on

page 51, he states, " A good practice, recommended by Nikola Tesla, is to use

only one hand and keep the other in your pocket." I would like to be able to

quote Tesla on this, but I can't find the statement in any of my Tesla books.

Can anyone help me out by supplying the Tesla quote, where I can find the quote

or what book it is in or anything else that might help me in my search.

Thank you

W. Encil Scott

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: hand in pocket

Date: Tue, 30 Dec 1997 12:38:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Tue, 30 Dec 1997 07:38:52 EST

Subject: Re: hand in pocket

Organization: AOL (http://www.aol.com)

Encil,

I have heard that quote before and if I can find the source, I will relay

it to you.  However, the idea behind this safety rule is that if you happen to

become part of an energized circuit, the current will flow somewhere through

your body besides going across you chest - stopping you heart.  I personally

will NEVER allow my body to become part of an energized circuit!!!!  Your

primary capacitor is likely to be able to store energy in excess of 1 joule.

This will be an extremely painful shock whether it kills you or not.  A better

rule to quote is " Always disconnect your power source and discharge the

capacitor(s) before touching any part of  a high voltage circuit".  If you

have capaciitors is series, you should also dicharge each capacitor rather

that just  bleeding the bank from the "ends" as each intermediate cap may

still hold a charge.

Mike Holligsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary frequency/coupling

Date: Tue, 30 Dec 1997 13:05:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Tue, 30 Dec 1997 08:05:23 EST

Subject: Re: primary frequency/coupling

Organization: AOL (http://www.aol.com)

Chris,

What Adam Smith said was a little ambiguous to me.  The self inductance of

your primary coil will be constant once is is wound.  The shape of it will

affect the mutual inductance between the primary and secondary.  This mutual

inductance adds to the self inductance in the circuit, thereby altering the

frequency.  Being that the secondary inductance is on the order of a thousand

times greater than the primary inductance, the mutual inductance affects the

primary circuit much more than the secondary.   Coupling that is too tight

will result in a lower Q value of the overall system lowering the output of

the secondary in spite of the fact that this tight coupling would tend to

transfer more energy between the primary and secondary.  In general, the

larger the system, the looser the coupling is required for optimum output.

Mike Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Over coupling

Date: Tue, 30 Dec 1997 14:30:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 30 Dec 1997 08:30:37 -0600

Subject: Over coupling

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

I noticed your post on power loss due to over coupling. I am currently

powering my 4" diameter coil using a pole pig. The pig is inductively

ballasted to limit the current.  The power supplied to the coil is

approximately 3.5 kw. I'm using a flat wound spiral made from 1/4" copper

tubing. The first winding of the primary is approximately .5" from the secondary.

It is roughly aligned with the first winding of the secondarycoil.

I'm running 13.2 - 14.4 kvac. My output streamers are 44" to 48 inches long.

My capacitors are delivering .017 uf. Should I expect more ? The streamers are

bright white 6" to 8 inches from the toroid. I'm using 4" diameter dryer duct. The overall diameter is about 20". I found your post to be very interesting.

Thanks,

Have a happy holiday !

Paul S. Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: (No Subject)

Date: Tue, 30 Dec 1997 14:47:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 30 Dec 1997 08:47:51 -0600

Subject: (No Subject)

Organization: MailExcite  (http://www.mailexcite.com)

Mike,

Bill pollack and I have achieved a max streamer length of 48" thanks to you

and the people on Tesla-2. My question is: Is there a limit to the possible streamer length produced by a 4" diameter coil. I think there must be and I

think you have achieved it. What is the limiting factor in coil design to be specifc as it relates to coil diameter ?

Second question:  When Bill wound the coil, he allowed extra windings.

The winding length is approximately 26 inches. Will the extra windings

effect coil performance in a negative way ?

Bill and I are planing a second coil. We are hoping to build a 6" diameter

coil.

We will probably increase the capacitance to .036 uf and keep the power level

to 3.5 Kw. Provided that we do everything well and we are able to operate at

the same level of efficiency, what should we expect as far as streamer length ?

Thanks again,

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New Tranny questions

Date: Tue, 30 Dec 1997 15:57:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com (Ronnie R Pittman)

Date: Tue, 30 Dec 1997 10:57:26 -0500

Subject: Re: New Tranny questions

<snip>

>15kv is really rough on caps and the only sure way to

>build a reliable cap for that voltage is to run at least 2 caps

>in series. I recommend 2 series caps for 12kv and 3 series caps

>for 15kv. These can be individual units or housed in the same

>container. I would use 24 mils of dielectric (4 X 6 mils) for

>each dielectric layer and 3 caps in series giving the equivalent

>of 72 mils. This is plenty when using 3 series caps.

>

>You want about .0053 mfd of capacitance for each 15kv/30ma neon

>you use. Thats means each cap in the series string should have

>about .0159 mfd of capacitance.

>

>I would build a flat plate cap with three .0159 mfd sections in

>series.

>Put all three sections in the same tank. Use this with one of

>your 15kv/30ma neons. Build a second identical cap and use it in

>parallel with the first to give you .0106 to make best use

>of 15kv/60ma.

About how many plates would that  be and what would be the area

of each plate.  I guess what you really are after is area correct?  Is

there a formula to calculate the capacitance with area of the plate as a

function?

>Your AWG 10 primary is more than adequate for a single 15kv/30

>and probably OK for 15kv/60. 3/8 inch copper tubing is overkill.

>1/4 inch tubing is good up to several thousand watts.

Seeing as I already have the 3/8" tubing, is there any

disadvantage to using it ?

>I would use your current primary as is. Put your effort into

>good quality caps and a good gap system.

I was wondering if I should put an old canister vacuum motor on

top of the gap instead of the muffin fan to improve air flow out of the

spark gap.  Any ideas?

>Yes. That should keep the RF to a manageable level and the safety gap

>will prevent overvoltage conditions.

I bought some rod shaped ferrite cores at skycraft yesterday that

are about 3/8 " in diameter and about 1" long for $ .5  each.  Would

these work for choke cores and if so how many winds and of what diameter

wire?

I also looked at variacs there and found I could get 0-140 v ac

starting at $25 for a 3 amp and up to $1500 for a huge one.  How many

amps would I need to run the 3 neons in parallel?  I think that I

calculated about 4.1 amps /neon which would mean I would need one to

handle 12 amps.  Am I off in my calculations?  I think I saw some 10 amp

units for about $100

>

>>  Is there anything else I can do to try to keep them alive?

>

>Yes. Make sure your circuit has the spark gap across the transformer.

>You do NOT want the cap across the transformer.

>

>Keep run times short. 5 minutes may seem like a short time to you

>but its an eternity to a neon in a Tesla coil. Keep runs to

>1 or 2 minutes maximum or so with similar off periods for cooling off.

Wow!!! 1-2 minutes.  I usually do not run mine for over about 25

seconds.  And I thought that was pushing it.

Thanks again for all the great help and your valuable time

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New Tranny questions

Date: Tue, 30 Dec 1997 16:06:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: rpittman@juno.com (Ronnie R Pittman)

Date: Tue, 30 Dec 1997 11:06:41 -0500

Subject: Re: New Tranny questions

>Good grab! Here are a few suggestions

Thanks!

>I would not recommend building plate caps.  I've done it and quite

>frankly, it sucked.  I ended up with an extrememly robust and low

>inductance cap, but it's huge, heavy and difficult to get leak-proof.

>

>I would recommend you build 4 identical 0.01 uF rolled caps, each

>using a total of 60 mil LDPE dielectric.  I have built two of these,

>and they each handle 10kVAC tesla survice, no problem.  You would wire

>them in series-parallel for use with your 15kV units.   The dimensions for

>caps in 4" PVC drain pipe (what I used) are:  4 plys 30mil LDPE per cap,

>each 12" x 72".  10" wide aluminum flashing 72" long, two of these per cap.

>

>This rolls up to fit in a 4" ID drain pipe and requires about 40 oz.

>of mineral oil to fill.  If you buy all the parts for 4 caps, you will

>get a very low cost per capacitor.

>My costs (in Boston) would be about $100 for material for all four caps.

>If you want the cost breakdown, email me.

I still have not heard the end to this debate nor know enough

about either to decide.  I like the apparent sturdiness of the rolled

type,  it does not seem like the plates will slide around or anything.

It would also seem like it is easier to build instead of having to try to

keep all the plates stacked together.  The down side seems to be the cost

and the vacuuming that has to be done.  From what I understand the most

expensive part is the aluminum flashing.  ( I boughr a 10' x 100' roll of

6 mil plastic yesterday which I hope will be plenty) .  Would it be

possible and feasible to make a rolled cap out of H.D. aluminum foil and

put that in the drain pipe?  If so how would you do it?

>Add some resistance in series with the chokes.  About 250-1000 ohms @ 100W >should do the trick.  The resistance is just to kill the Q of the choke, so

>it's value is not critical.  Use only as much as is needed.

I will have to wait to try this seeing as the caps are not built yet.

Thanks for the great suggestions.  I really appreciate it.

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: choke parts

Date: Tue, 30 Dec 1997 16:11:41 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Federico" <cfed@dynasty.net>

Date: Tue, 30 Dec 1997 16:11:41 +0000

Subject: choke parts

2 questions about parts for RF chokes:

1.  Will wiring 2-150 ohm/50w resistors in series give me appropriate

values for use with my chokes?

2.  Is a ferrite toroid with the dimensions 1.16"o.d., 9/16"i.d. and

19/32" length be efective as cores?  Or, since they are smaller than

the 2" recommended, could I double them up to increase the damping?

Thanks. . .Chris

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: 240 volt  9 amp Variac  For sale

Date: Tue, 30 Dec 1997 17:47:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Tue, 30 Dec 1997 11:47:05 -0600

Subject: 240 volt  9 amp Variac  For sale

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have a 240 v. 2200 w. variac for sale. It is in great shape. I'm asking

$115.00 which includes shipping. I will ship C.O.D. If anyone is interested please E-Mail me at:

Fleishwulf@mailexcite.com

The variac is in great shape. My power needs have just exceeded the 2200

w. limit. I would consider a trade just let me know what you have available.

Thanks again,

Paul S. Marshall

--

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: choke parts

Cc:

Bcc:

X-Attachments:

At 04:42 AM 12/31/97 GMT, you wrote:

>From: Adam <absmith@tiac.net>

>Date: Tue, 30 Dec 97 23:42:19 -0500

>Subject: Re: choke parts

>

>

>>Chris,

>>You probably wont need the series resistance with your

>>toroidal chokes. They wont usually ring up like an air

>>core choke with many more turns of wire. I do not use

>>any series resistance with my toroidal chokes.

>

>I don't think you can really generalize it that much.  Whether or not

>your chokes will cause problems depends on the Q of the choke itself

>(dependant on wire gauge, self-inductance and the core material...), and

>where your chokes and bypass caps resonance point is relative to your TC

>tank frequency.  Realize that the choke and the bypass cap form a

>two-pole lowpass LCR filter which will very likely be resonant if not

>specifically designed otherwise.  If this filter's resonance point

>(cutoff point) is near or above your operating frequency, you will

>develop *huge* voltages across the bypass caps, capable of serious neon

>damage.

>

Adam,

I dont disagree at all with what you said however my experience

has shown the ferrite chokes with low turns count dont have

a tendancy to ring like high turns count air core chokes will.

Ive built chokes on several types of surplus ferrite cores

with good success and scoping them has not shown any serious

ringing problems.

I also do not recommend the use of bypass caps. These can often introduce

unwanted and unpredictable resonances. IMO they just arent necessary.

I once fried a perfectly good Jefferson 12kv/30ma neon in seconds after

installing bypass caps.

Also I dont believe that completely eliminating safety gap firing

is a good thing. If your safety isnt firing then its probably spaced

too wide. Its not there just for RF kickbacks. It also protects

against 60 hz overvoltage conditions. I narrow my safety until

it does fire frequently. That way I know its limiting the voltage

to a safe value.

TTYL!

Mike Hammer

mhammer@misslink

From: tesla-2@emachine.com (Tesla-2)

Subject: transformer

Date: Tue, 30 Dec 1997 19:10:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: torghn@juno.com (Jared Neilsen P Neilsen)

Date: Tue, 30 Dec 1997 12:10:23 -0700

Subject: transformer

Thanks for the info on the plans for a testacy coil, now I'm left with

finding a transformer.  Do any of you know of a good place to get them?

I don't even know where to begin.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: 240 volt  9 amp Variac  For sale

Date: Tue, 30 Dec 1997 21:40:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: crowekri <crowekri@scott.net>

Date: Tue, 30 Dec 1997 13:40:40 -0800

Subject: Re: 240 volt  9 amp Variac  For sale

Organization: home

I'd love to buy it from you if it is still availiable.

thanks,

Shane Crowson

17245 Co. Rd. 51

Jemison, Al 35085

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Tue, 30 Dec 1997 11:47:05 -0600

> Subject: 240 volt  9 amp Variac  For sale

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

> I have a 240 v. 2200 w. variac for sale. It is in great shape. I'm asking

>$115.00 which includes shipping. I will ship C.O.D. If anyone is interested >please E-Mail me at:

>

> Fleishwulf@mailexcite.com

>

> The variac is in great shape. My power needs have just exceeded the 2200

>w. limit. I would consider a trade just let me know what you have available.

>

> Thanks again,

>

> Paul S. Marshall

> --

> Einigkeit und Freiheit

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Neon Transformers and HV Wire for Sale

Date: Wed, 31 Dec 1997 02:14:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Tedd Payne <tpayne@netnitco.net>

Date: Tue, 30 Dec 1997 20:14:00 -0600

Subject: Neon Transformers and HV Wire for Sale

"Transformer Seekers",

I've seen a couple of posts looking for neon transformers, so here are

some I have for sale, and also some HV wire.  If interested, please

contact me DIRECTLY at:

tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your zip code (to

determine shipping cost).

Thanks,

Tedd

"CONVENTIONAL" neon transformers in clean (not old, not rusty), good

working condition.  They are not labeled, the ratings are from my

testing/measuring.

Shipping is extra.

$6  used  3kv20ma     in = 2 wires      out = 2 wires, small, not potted

$5  NEW   5kv30ma?   input=2 wires     out =2 sockets (soc damaged)

$5  used  6kv15ma     in = porc ins     output=porc insul  no lid

$6  used  6kv30ma     in = 2 wires     out =2 sockets

$9  used  6kv60ma     in = porc ins     output=porc insul

$5  NEW   7.5kv30ma?  input=3 wires    out =2 sockets (soc damaged)

$8  used  7.5kv30ma?  input=2 wires     out =2 wires  switch

$10 used  9kv30ma     in = porc ins     output=porc insul  no lid

$12 used  9kv30ma     in = porc ins     output=porc insul

$14 used  12kv25-30ma  in = porc ins     output=porc insul  no lid

$17 used  12kv30ma   in = porc ins     output=porc insul

$17 used  15kv30ma    in = 2 wires     output= 2 wires, w/switch

$18 used  15kv30ma    in = AC cord     output= 6" leads, w/switch

$12 used  15kv30ma  in = porc ins  out=porc insul, no lid, missing 1

insul

NEW Neon transformer HV wire, flexible gray, rated 15KV, $1.00 per foot.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: choke parts

Date: Wed, 31 Dec 1997 04:42:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Tue, 30 Dec 97 23:42:19 -0500

Subject: Re: choke parts

>Chris,

>You probably wont need the series resistance with your

>toroidal chokes. They wont usually ring up like an air

>core choke with many more turns of wire. I do not use

>any series resistance with my toroidal chokes.

I don't think you can really generalize it that much.  Whether or not

your chokes will cause problems depends on the Q of the choke itself

(dependant on wire gauge, self-inductance and the core material...), and

where your chokes and bypass caps resonance point is relative to your TC

tank frequency.  Realize that the choke and the bypass cap form a

two-pole lowpass LCR filter which will very likely be resonant if not

specifically designed otherwise.  If this filter's resonance point

(cutoff point) is near or above your operating frequency, you will

develop *huge* voltages across the bypass caps, capable of serious neon

damage.

For your chokes to be effective, the 1/(2pi*sqrt(LC)) point for your

chokes and bypass caps must be well below your tank resonance frequency.

Otherwise, the chokes are either useless or harmful, leaning towards the

latter.

I took the 2mH ferrite chokes from my 400kHz system and popped them into

my 160kHz system and they arced over and failed instantly.   With this

new system I did not achieve adequate choking until I went to 4" OD iron

powder chokes (material 26 perm=75).  150 turns of PVC wire on these guys

made decent 6.2mH chokes.  In combo with my 500pF bypass caps, the cutoff

point is about 90kHz, far enough below 160kHz to work.  I also found that

a bit of series resistance helped, and completely eliminated safety gap

firings.  375 Ohms at 100W is what I am using with each choke.

I would recommend 3" or bigger material 26 or material 52 iron powder

cores to anybody building chokes for coils.  They are cheap and easy to

get from http://www.bytemark.com/amidon/

>Your toroid cores are a bit small. If you have 4 of them

>I would suggest putting two in series in each HV leg.

Yes.  Ferrites, esp. the higher permeability mixes, saturate with very

little ampere-turns compared to iron powder.  You are putting substantial

60Hz current through these chokes, and the more core material you use the

better.  Any idea what type of ferrite you have?

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla Coil Survey Results

Date: Wed, 31 Dec 1997 05:04:02 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Wed, 31 Dec 97 00:04:02 -0500

Subject: Tesla Coil Survey Results

I took a survey of coil data from coilers in the www.pupman.com group,

and the survey results are now online.  This data is especially valuable

to beginners.

http://www.tiac.com/users/absmith/tesla/tcresults.html

There is a lot of data here from existing coil systems, and even the

table of contents is educational.  I have not had a chance yet to parse

the survey entries into nice readable text and tables, so the full

systems specs sheets may seem a bit cryptic, especially to the newbies.

A peek at the survey forms themselves should be able to clear up any

abiguity about units.

If you want to, there are links on this page to the survey pages where

you can enter your own coil data.  I will then manually enter the data I

receive into the table.

Have fun!

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: High quality non messy poly plate caps

Date: Wed, 31 Dec 1997 10:17:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Jeff Harvey" <jeff@techgallery.com>

Date: Wed, 31 Dec 1997 10:17:55 +0000

Subject: Re: High quality non messy poly plate caps

Organization: Life & Energy Systems

>

>

> Coilers,

>    I have seen alot of traffic in the past few days

> reguarding cap's. Having been a Manufacturing Engineer

> for almost 2o years in the aerospace field, I have had

> to find cheaper better ways to make things. I do not

> wish to argue the merits of plate caps vs rolled caps.

>    For those who are interested I will provide

> instructions for making inexpensive poly plate type

> caps. Using my method you can make a cap in about 2 1/2

> hours (.026 uf). These caps look very prefessional.

> They are extreamly efficant and almost bullet proof !

>    In a series circuit they are capable of handling

> surges of 22 - 25 kv. At 15 kv they are almost

> indistructable if made correctly. They do require a

> break in period to allow unwanted air to escape. I

> do not wish to beat the cap thing to death and will

> not post any more information unless requested by

> individuals.

>

> Thanks,

>

> Paul Marshall

>

> I would love to see the instructions for building your plate cap!

> Thanks Jeff

>

> -----

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary frequency/coupling

Date: Wed, 31 Dec 1997 13:07:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Hollmike <Hollmike@aol.com>

Date: Wed, 31 Dec 1997 08:07:20 EST

Subject: Re: primary frequency/coupling

Organization: AOL (http://www.aol.com)

Chris,

I did some quick calculations of your design and found that you may not be

suffering from overcoupling as thought.  It appears to me that you may be

trying to operate your system with the secondary being a harmonic of the

primary frequency.  My calculations show that tuning of your primary should be

at about 6.5 turns(5.5 if the capacitance of the sphere is negleted). Perhaps

others can verify this for you.  I don't know what type of transformer you are

powering you system with (so I cannot comment on the primary cap/transformer

impedance match), but for your secondary design it seems that your cap only

needs to be in the neighborhood of .02uF.  This would allow for more turns on

your primary to achieve resonance.My calculations for the inductances are

using the Wheeler equation:

L = R^2*N^2 / ((9*R + 10*B)1000)  mH

where R = radius in inches, B = coil height in inches, N = number of turns

( "^" denotes squared and " * " multiply)

I hope this has helped.

MIke Hollingsworth

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: High quality non messy poly plate caps

Date: Wed, 31 Dec 1997 14:16:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: fleischwulf@mailexcite.com

Date: Wed, 31 Dec 1997 08:16:28 -0600

Subject: High quality non messy poly plate caps

Organization: MailExcite  (http://www.mailexcite.com)

Coilers,

I have seen alot of traffic in the past few days

reguarding cap's. Having been a Manufacturing Engineer

for almost 2o years in the aerospace field, I have had

to find cheaper better ways to make things. I do not

wish to argue the merits of plate caps vs rolled caps.

For those who are interested I will provide

instructions for making inexpensive poly plate type

caps. Using my method you can make a cap in about 2 1/2

hours (.026 uf). These caps look very prefessional.

They are extreamly efficant and almost bullet proof !

In a series circuit they are capable of handling

surges of 22 - 25 kv. At 15 kv they are almost

indistructable if made correctly. They do require a

break in period to allow unwanted air to escape. I

do not wish to beat the cap thing to death and will

not post any more information unless requested by

individuals.

Thanks,

Paul Marshall

---

Einigkeit und Freiheit

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: High quality non messy poly plate caps

Date: Wed, 31 Dec 1997 18:58:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: crowekri <crowekri@scott.net>

Date: Wed, 31 Dec 1997 10:58:56 -0800

Subject: Re: High quality non messy poly plate caps

Organization: home

Tesla-2 wrote:

>

> From: fleischwulf@mailexcite.com

> Date: Wed, 31 Dec 1997 08:16:28 -0600

> Subject: High quality non messy poly plate caps

> Organization: MailExcite  (http://www.mailexcite.com)

>

> Coilers,

>   <snipped>

>    For those who are interested I will provide

> instructions for making inexpensive poly plate type

> caps.

I would be interested in seeing these instructions

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: choke parts

Date: Wed, 31 Dec 1997 22:54:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Adam <absmith@tiac.net>

Date: Wed, 31 Dec 97 17:54:52 -0500

Subject: Re: choke parts

>I dont disagree at all with what you said however my experience

>has shown the ferrite chokes with low turns count dont have

>a tendancy to ring like high turns count air core chokes will.

You must have some reasonably high permeability material.  The higher the

permeability, the lower the saturation threshold.  These will work since

you are at low power, but like my ferrite toroids, they will not carry

over to a bigger project.

It may seem like keep repeating that point, but it's for a good reason.

After seeing my survey results, it is clear that most coilers using

toroid chokes have them WAY underized.  A 3" ferrite 'roid, regardless of

the material, is only good in about a 1kVA setup, or less.  Most coilers

also have too little inductance for the choke to actually work properly.

My chokes are 6mH and weigh 9 pounds each, and I know they are doing

their job.  I run up to 200mA through them without problems.

>I also do not recommend the use of bypass caps. These can often introduce

>unwanted and unpredictable resonances.

Unwanted, yes.  Unpredictable, no.  Bypass caps are only troublesome when

the chokes are not doing their job.  If the chokes work, the bypass caps

are a very good idea.  If the chokes are too small or two low in

inductance, only then will the bypass caps turn against you.  ..and with

a vengence!  You can't afford to be at all arbitrary about your bypass

cap and choke values.

>I once fried a perfectly good Jefferson 12kv/30ma neon in seconds after

>installing bypass caps.

Not surprising.  They were probably not matched well to the chokes.  On

thing for sure, if you don't have ANY chokes or series resistance, then

you must also not have bypass caps.  If you have bypass caps and no

chokes or R's, when the main gap stops firing, your bypass caps suddenly

become part of a larger multi-MHz tank circuit (with the main cap and

primary) and ring up to very huge voltages, right across the neon.

>Also I dont believe that completely eliminating safety gap firing

>is a good thing. If your safety isnt firing then its probably spaced

>too wide.

Yes, widing the safety gap will stop it from firing, this is true :-)

That's not what I did however.  I have the main gap at 0.150" and the

safety at 0.176".  The safety gap fired when I had feeble ferrite chokes,

now it does not.  This means that the choke-bpc LPF is no longer ringing

up huge voltages across the bypass caps, which is what was most likely

firing the gap.

>I narrow my safety until

>it does fire frequently. That way I know its limiting the voltage

>to a safe value.

That's certainly safe, but I feel much better when I have the system

tweeked to the point where I can set my safety gap to just a hair more

than the main gap and never see it fire.  Again, this means GOOD chokes.

I always ramp up the voltage to the coil with a variac also.

-Adam

--------------------------------------------------------------------------

Adam Smith

absmith@tiac.net

Epoch, Inc. Digital Music Project

www.tiac.net/users/absmith/                 MP3 Demo Tracks Now Available!

--------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4