ES1:  This is a partial picture of my control "board".

Lower left is two variacs which I run in series to control the speed of the

rotary gap motor.  One is set to limit the maximum rpm and the other is used

to bring the speed up slowly.  Above these are the 60 cycle supple lines

going out to the coil.  One is for the rotary gap motor, the other is for the

fan motors on the rotary and the static gap used in series.

The heavy powerstat variac is in the center bottom - used to contro the

voltage to the pole pig.  Above that is the ammeter and voltmeter to monitor

the pole pig primary.  Above that you can see the primary feed lines going to

the pole pig.

To the right, are the 60 cycle line filters and some home made toroid filters

used in series with the 60 cylce ground connection.

Upper right is the plug for the welder (used to limit pole pig primary

current) and the plug for the resistive bank (oven elements) used in parallel

with the welder primary also to control the pole pig primary current.