> Richard Quick writes

For those who are interested, I have some specific information on

the coil pictured in the BIGCOIL.GIF. This particular coil is

unmistakable and I immediately recognized it.

This coil was constructed by Greg Leyh in California. In the

early 1990's this was the largest Tesla coil in the U.S., and

possibly the world. For years this coil was stored at the old

Phoenix Iron Works in San Francisco, and this particular photo

was taken sometime after April 10, 1993, when the new toriod was

installed.

The secondary coil is wound on an open wooden form. The coil form

construction is that of a polygon, not a true circle. The secon-

dary is four feet in diameter, twelve feet high, and space wound

with No. 8 AWG insulated stranded wire.

The toriod discharge terminal is 65 inches in diameter by 24

inches thick.

The rotary spark gap is spun by a one-horsepower motor with a top

speed of 5000 rpm. The rotor electrodes are 1/2-inch copper slugs

cut from switch station buss bars and riveted to a one foot

diameter steel disk. The rotor electrodes move past two 1/2-inch

thick fixed electrodes of tungsten.

The primary coil made of 5 inch wide by 3/8 inch thick electro-

lytic copper bar wound into a coil of 2-1/2 turns. This system

has a total of fourteen commercial pulse discharge capacitors

which are wired in parallel with the same copper bar as used in

the primary.

This coil is capable of strikes 20 feet in length. I believe that

the rating given (1.5 mV) for this coil is in error as someone

placed a decimal that does not belong here. This is a 15 mV coil.