HARTWCK.GIF and HARTWCK2.GIF (1024x768 and 800x600)

This scan shows another "bargin basement" coil built by Dave

Hartwick. The secondary coil is wound with enamel covered magnet

wire on a six inch diameter thin wall PVC plastic drain pipe.

The plastic pipe was sealed with several coats of polyurethane

before the coil was wound, and the coil was coated again with

several coats of sealer over the windings to reduce corona loss.

The discharge terminal is an inverted stainless steel mixing

bowl. The discharger is taped to the top of the secondary with

several turns of black electrical friction tape. This taping

also helps to prevent corona loss and premature breakout from

the tightly rounded lip of the bowl.

The primary coil is about six turns of three-wire electrical

extension cord that has all three conductors fused at the ends.

The six turn primary is a "flat pancake" (Archimedes spiral)

wound on a coil form constructed of plexiglas strips mounted in

a simple hardwood frame. Pegs cut from wooden dowels are plugged

into holes drilled in the plexiglas strips to act as spacers be-

tween primary turns.

The spark gap is a cylinder static gap constructed from 14, two

inch long sections of 1-inch diameter hard copper water pipe

mounted inside of a short length of standard six-inch PVC drain

pipe. Air flow through the pipe is provided by a small muffin fan.

Only seven gaps are used, with the spacing between electrodes set

at about .030 inches. This cylinder gap is mounted horizontally

instead of the usual vertical construction. A plastic drinking

cup and some foam insulation have been crammed into the spark

gap to choke the airflow through the center of the unit and

increase the speed and amount of airflow between the electrodes.

The capacitor used in this coil project is a small homemade

plastic/foil capacitor constructed using a small plate of plexi-

glas. The designer/builder stated to me that he was not happy

with the capacitor performance, having seen flashovers and

heavy corona losses from the uninsulated plates. The actual

capacitor can be seen directly behind the base of the secondary.

It is a long foil covered rectangle.

The power supply pictured here is a single 12 kV 30 mA neon sign

transformer.

One little note that I pointed out the the builder: the primary

coil form is sitting on top of a formica table. The section of

the table top visible in the photo (and the scan) is edged with

a strip of heavy aluminum. I suspected that this strip goes

completely around the table top and forms a single shorted turn

"square" coil! I picked up the phone an called. Being familiar

with this type of table construction, I asked if there was a

tiny gap where the strip of aluminum edging (the single square

turn) met. I waited a moment as the builder crawled under the

table to discover the an arc had been forming at this location

and the the pressboard tabletop had been carbonized at this

location!

This coil was designed, built, and operated by: Dave Hartwick

191 Highland Rd., Chambersburg, PA  17201  USA