SPARKGP1 GIF and SPARKGP2 GIF  (1024x768 and 800x600)

These are two scans of a spark gap system that I designed and

built for operation at power levels between 1 - 5 kVA. This

particular gap system has been very successful and has seen

many hours of run time. The quench time is low enough that

small Tesla Magnifiers can be fired without problem despite

the very close coupling factor between the primary and secondary

coils in Magnifier systems.

The small rotary gap in the center has eight tungsten electrodes

mounted on a steel ring. The ring is bolted to a multi-layer

marine grade fiberglas board that insulates the conductive ring

from the motor shaft. The motor is a Bodine 1/10 horse-power

fixed speed that rotates at 3450 rpm. The two stationary elect-

rodes are also tungsten, mounted on high temperature porcelain

insulators with heavy brass caps. The motor and stationary

electrodes are mounted to a custom made base of butchers block.

This particular rotary gap was designed and built by Gary Legel

of California, and I was lucky enough obtain it when he was

scaling back on his coiling equipment. It had obviously never

used when I unpacked it!

In series with the rotary gap, one on either side, are two

custom cylinder static gaps. These two cylinder static gaps

do most of the quenching, leaving the rotary gap to simply

commutate the arc.

This photo is taken from above. The gap system is sitting on the

gap shelf on my rolling Tesla coil workstation. Barely visible

below the gaps are the sides of three capacitors stored in the

workstation.

System was designed, built, and operated by Richard T. Quick II

10028 Manchester Rd., Suite 253, Glendale, MO  63122  USA; with

the exception of the custom rotary gap which was designed and

constructed by Gary Legel, 1306 Sheppard Dr., Fullerton, CA

92631  USA