From rwstephens@ptbo.igs.net Wed Apr 24 19:55:58 1996

ltc01rws.jpg

This is my largest TC to date. This unit stands 9 feet tall to top of

toroid discharge terminal as shown here. I  built it in December'94 -Jan'95.

It is shown here in test setup on a cold Sunday in January '95 after

performing beautifully in its first and only powered test to date the

evening before. This coil threw an 18 foot long streamer, recorded on

videotape (see ltc04rws.jpg), with just 10 kVA of input power.  The

mains service available during this test was only 40 amps@230 volts.

The secondary is wound with 4200 feet of #12 THHN stranded copper wire,

close wound to a length of 67.5 inches on an open concept form. Operating

frequency with large 67 inch x 15 inch aluminum torroid is 54KHz. Primary

is 12 turns of 3/4 inch aluminum CATV distribution coax.  System resonated

at 9 turns with two parallel Condenser Products Co., 0.025 uF,  60KVDC pulse

discharge caps (polypropelene & silicon oil) shown under grounded strike

shield..  Rotary break is behind the caps. The 10KVA, 16000 volt pole pig

was left inside the lab and the HV was brought out to the coil on a length

of RG-214 coax.

ltc02rws.jpg

Large 67 inch by 15 inch topload toroid with extended coil protection

ring.  Shown upside down.

ltc03rws.jpg

Here I am trying to estimate the length of some streamers that hit

the ground early during the test with a tape measure.  Coil stands 9

feet tall to top of toroid discharge terminal.

ltc04rws.jpg

An 18 foot streamer to air at just 10 kVA input power.  This is the

longest streamer I have created with a Tesla coil to date.  This is

more remarklable when you factor in the cold , dry dense air at minus

20 Centigrade, January 1995.  Coil stands 9 feet tall to top of

toroid discharge terminal.

ltc05rws.jpg

A very high energy 11 foot streamer strikes the ground. Coil stands 9 feet

tall to top of toroid discharge terminal.

ltc06rws.jpg

Convoluted streamer upwards to air. Discharge toroid is 67 inches

across.