This is the high speed air blown, 1725 RPM rotary spark gap I

developed for my Medium Tesla Coil as seen in the mtc__rws.jpg

pictures posted here.  The 14, 1/8 th inch diameter thoriated

tungsten electrodes are replaceable, being held in place by 8-32 hex

drive setscrews  in 3/8ths inch, fine thread bolts with a precision hole coaxially

down the center of each to hold the tungsten rod.  The 3/8" bolts are epoxied

into the 1/4 inch thick Glastic disk.  Glastic is a pink colored

glass fiber and epoxy type panelboard with superior insulation and

carbon tracking resistance used by the electrical industry.  It not

only beats phenolic in strength, but especially in carbon tracking resistance by

about a bizillion times! This tungsten is available at any good welding supplier

as replacement tips for tig welders.  The break rate is approximately

400 PPS with a dwell time of roughly 200 microseconds as shown here  The 6

inch white PVC cylinder contains a 6.5 amp Lamb vacuum cleaner blower.

This blower allows continuous extended operation at a tested 5 kVA, and

probably twice that figure without trouble.