IRSIM(1)

IRSIM Users's Manual

NAME

irsim - An event-driven logic-level simulator for MOS circuits

SYNOPSIS

irsim [-s] prm file sim file ... [+hist file] [-cmd file ...]

-  -------- --------       ---------    --------

DESCRIPTION

IRSIM is an event-driven logic-level simulator for MOS (both N and P)

transistor circuits.  Two simulation models are available:

switch

Each transistor is modeled as a voltage-controlled switch.  Useful

for initializing or determining the functionality of the network.

linear

Each transistor is modeled as a resistor in series with a voltage-

controlled switch; each node has a capacitance.  Node values and

transition times are computed from the resulting RC network, using

Chorng-Yeoung Chu's model.  Chris Terman's original model is not

supported any more.

If the -s switch is specified, 2 or more transistors of the same type

connected in series, with no other connections to their common

source/drain will be stacked into a compound transistor with multiple

-------

gates.

The prm file is the electrical parameters file that configure the dev-

-

ices to be simulated.  It defines the capacitance of the various layers,

transistor resistances, threshold voltages, etc... (see presim(1)).

If prm file does not specify an absolute path then IRSIM will search for

--------

the prm file as follows (in that order):

--------

1) ./<prm file> (in the current directory).

--------

2) ${CAD_ROOT}/irsim/prm/<prm file>

--------

3) ${CAD_ROOT}/irsim/prm/<prm file>.prm

--------

The default search directory (/usr/local/lib) can be overriden by setting

the environment variable CAD_ROOT to the appropriate directory prior to run-

ning IRSIM (i.e. setenv CAD_ROOT /usr/beta/mycad).

IRSIM first processes the files named on the command line, then (assum-

ing the exit command has not been processed) accepts commands from the

user, executing each command before reading the next.

File names NOT beginning with a '-' are assumed to be sim files (see

sim(5)), note that this version does not require to run the sim files

through presim.  These files are read and added to the network database.

There is only a single name space for nodes, so references to node "A"

in different network files all refer to the same node.  While this

feature allows one to modularize a large circuit into several network

files, care must be taken to ensure that no unwanted node merges happen

1

IRSIM(1)

IRSIM Users's Manual

due to an unfortunate clash in names.

File names prefaced with a '-' are assumed to be command files: text

files which contain command lines to be processed in the normal fashion.

These files are processed line by line; when an end-of-file is encoun-

tered, processing continues with the next file. After all the command

files have been processed, and if an "exit" command has not terminated

the simulation run, IRSIM will accept further commands from the user,

prompting for each one like so:

irsim>

The hist file is the name of a file created with the dumph command (see

-                                            -----

below).  If it is present, IRSIM will initilize the network to the state

saved in that file.  This file is different from the ones created with

the ">" command since it saves the state of every node for all times,

including any pending events.

This version supports changes to the network through the update command.

Also, the capability to incrementally re-simulate the network up to the

current time is provided by the isim command.

COMMAND SUMMARY

@ filename            take commands from command file

--------

? wnode...            print info about node's source/drain connections

-----

! wnode...            print info about node's gate connections

-----

< filename            restore network state from file

--------

> filename            write current network state to file

--------

<< filename           same as "<" but restores inputs too

--------

| comment...          comment line

-------

activity from [to]    graph circuit activity in time interval

----  --

ana wnode...          display nodes in analyzer window

-----

analyzer wnode...     display nodes in analyzer window

-----

assert wnode [m] val  assert that wnode equals value

-----  -  ---              -----        -----

assertWhen nodeT valT node val

assert when a condition is met

back [time]           move back to time

----                         ----

c [n]                 simulate for n clock cycles (default:1)

-                               -

changes from [to]     print nodes that changed in time interval

----  --

clock [node [val]]    define value sequence for clock node

----  ---

clear                 clear analyzer window (remove signals)

d [wnode]...          print display list or specified node(s)

-----

debug [debug level...]

-----------

set debug level (default: off)

decay [n]             set charge decay time (0 => no decay)

-

display [arg]...      control what gets displayed when

---

dumph filename...     write net history to file

--------

hist [on|off]         turn history on or off

exit [status]         return to system

------

flush [time]          flush out history up to time (default: now)

----                                   ----

2

IRSIM(1)

IRSIM Users's Manual

h wnode...            make node logic high (1) input

-----

has coords            print YES if transistor coordinates are available

-

inputs                print current list of input nodes

ires [n]              set incremental resolution to n ns

-                                             -

isim [filename]       incrementally resimulate changes form filename

--------                                              --------

l wnode...            make node logic low (0) input

-----

logfile [filename]    start/stop log file

--------

model [name]          set simulation model to name

----                                   ----

p                     step clock one simulation step (phase)

path wnode...         display critical path for last transition of a

-----

node

powlogfile [filename] start/stop power logfile

--------

powtrace -[node]...   start/stop power tracing  of specified

----

node(s)/vector(s)

powstep               toggle the display of power estimate for each

timestep

print comment...      print specified text

-------

printp                print a list of all pending events

printx                print all undefined (X) nodes

q                     terminate input from current stream

R [n]                 simulate for n cycles (default:longest sequence)

-                               -

readh filename        read history from filename

--------                          --------

report[level]         set/reset reporting of decay events

-----

s [n]                 simulate for n ns. (default: stepsize)

-                               -

stepsize [n]          set simulation step size to n ns.

-                                       -

set vector value      assign value to vector

------ -----             -----    ------

setlog[file|off]      log net changes to file (off -> no log)

---- ---                                ---

setpath               set search path for cmd files

stats                 print event statistics

sumcap                print out the sum of the capacitance of all nodes

t [-]wnode...         start/stop tracing of specified nodes

-----

tcap                  print list of shorted transistors

time [command]        print resource utilization summary

-------

until wnode [mask] value count

delayed assert based on the clock count.

u wnode...            make node undefined (X) input

-----

unitdelay [n]         force transitions to take n ns. (0 disables)

-                                    -

update filename       read net changes from file

--------

V [node [value...]]   define sequence of inputs for a node

----  -----

vector label node...  define bit vector

----- ----

vsupply voltage       set supply voltage for calculating power (default

-------

5V)

w [-]wnode...         add/delete nodes from display list

-----

wnet [filename]       write network to file

--------

x wnode...            remove node from input lists

-----

Xdisplay [host:n]     set/show X display (for analyzer)

---- -

COMMAND DESCRIPTIONS

Commands have the following simple syntax:

3

IRSIM(1)

IRSIM Users's Manual

cmd arg1 arg2 ... argn <newline>

---- ----     ----

where cmd specifies the command to be performed and the argi are argu-

----

ments to that command.  The arguments are separated by spaces (or tabs)

and the command is terminated by a <newline>.

If cmd is not one of the built-in commands documented below, IRSIM

appends ".cmd" to the command name and tries to open that file as a com-

mand file (see "@" command).  Thus the command "foo" has the same effect

as "@ foo.cmd".

Notation:

...  indicates zero or more repetitions

[ ]  enclosed arguments are optional

node name of node or vector in network

wnode

name of node or vector in network, can include '*' wildcard which

matches any sequence of zero or more characters.  The pair of char-

acters '{' and '}' denote iteration over the limits enclosed by it,

for example: name{1:10} will expand into name1, name2 ... name10. A

-----  -----     ------

3rd optional argument sets the stride, for example: name{1:10:2}

will expand into name1, name3, ... name7, name9.

-----  -----      -----  -----

| comment...

Lines beginning with vertical bar are treated as comments and

ignored -- useful for comments or temporarily disabling certain

commands in a command file.

Most commands take one or more node names as arguments.  Whenever a node

name is acceptible in a command line, one can also use the name of a bit

vector.  In this case, the command will be applied to each node of the

vector (the "t" and "d" treat vectors specially, see below).

vector label node...

----- ----

Define a bit vector named "label" which includes the specified

nodes.  If you redefine a bit vector, any special attributes of the

old vector (e.g., being on the display or trace list) are lost.

Wild cards are not accepted in the list of node names since you

would have no control over the order in which matching nodes would

appear in the vector.

The simulator performs most commands silently.  To find out what's hap-

pened you can use one of the following commands to examine the state of

the network and/or the simulator.

set vector value

------ -----

Assign value to vector. For example, the following sequence of com-

-----    ------

mands:

4

IRSIM(1)

IRSIM Users's Manual

vector BUS bit.1 bit.2 bit.3

set BUS 01x

The first command will define BUS to be a vector composed of nodes

---

bit.1, bit.2, and bit.3. The second command will assign the follow-

--- -  --- -      --- -

ing values:

bit.1 = 0

bit.2 = 1

bit.3 = X

Value can be any sequence of [0,1,h,H,l,L,x,X], and must be of the

same length as the bit vector itself.

d [wnode]...

-----

Display.  Without arguments displays the values all nodes and bit

vectors currently on the display list (see w command).  With argu-

ments, only displays the nodes or bit vectors specified.  See also

the "display" command if you wish to have the display list printed

out automatically at the end of certain simulation commands.

w [-]wnode...

-----

Watch/unwatch one or more nodes.  Whenever a "d" command is given,

each watched node will displayed like so:

node1=0 node2=X ...

To remove a node from the watched list, preface its name with a '-

'.  If wnode is the name of a bit vector, the values of the nodes

-----

which make up the vector will be displayed as follows:

label=010100

where the first 0 is the value of first node in the list, the first

1 the value of the second node, etc.

assert wnode [mask] value

-----  ----  -----

Assert that the boolean value of the node or vector wnode is value.

-----    -----

If the comparison fails, an error message is printed.  If mask is

----

given then only those bits corresponding to zero bits in mask take

----

part in the comparison, any character other than 0 will skip that

bit.  The format of the error message is the following:

(tty, 3): assertion failed on 'name' 10X10 (1010X)

Where name is the name of the vector, followed by the actual value

----

and the expected value enclosed in parenthesis.  If a mask is

----

specified, then bits that were not compared are printed as '-'.

until wnode [mask] value count

Acts just like the assert command except it requires an additional

argument <count> which is the max number of clock cycles to run.

5

IRSIM(1)

IRSIM Users's Manual

Instead of just testing the current state, like assert, until tests

for true and if false it runs clock cycles until condition becomes

true or count runs out.

ana wnode...

-----

This is a shorthand for the analyzer command (described below).

analyzer wnode...

-----

Add the specified node(s)/vector(s) to the analyzer display list

(see irsim-analyzer(3) for a detailed explanation).  If the

analyzer window does not exist, it will be created.  If no argu-

ments are given and the analyzer window already exists, nothing

happens.

Xdisplay [host:display]

---- -------

You must be able to connect to an X-server to start the analyzer.

If you haven't set up the DISPLAY environment variable properly,

the analyzer command may fail. If this is the case you can use the

Xdisplay command to set it from within the simulator.  With no

arguments, the name of the current X-server will be printed.

clear

Removes all nodes and vectors from the analyzer window.  This com-

mand is most useful in command scripts for switching between dif-

ferent signals being displayed on the analyzer.

"?" and "!" allow the user to go both backwards and forwards through the

network.  This is a useful debugging aid.

? wnode...

-----

Prints a synopsis of the named nodes including their current values

and the state of all transistors that affect the value of these

nodes.  This is the most common way of wandering through the net-

work in search of what went wrong.

The output from the command ? out looks like

---

out=0 (vl=0.3 vh=0.8) (0.100 pf) is computed from:

n-channel phi2=0 out=0 in=0 [1.0e+04, 1.3e+04, 8.7e+03]

pulled down by (a=1 b=1)  [1.0e+04, 1.3e+04, 8.8e+03]

pulled up [4.0e+04, 7.4e+04, 4.0e+04]

The first line gives the node's name and current value, its low and

high logic thresholds, user-specifed low-to-high and high-to-low

propagation delays if present, and its capacitance if nonzero.

Succeeding lines list the transistor whose sources or drains con-

nect to this node: the transistor type ("pulled down" is an n-

channel transistor connected to gnd, "pulled up" is a depletion

pullup or p-channel transistor connected to vdd), the values of the

gate, source, and drain nodes, and the modeling resistances.  Sim-

ple chains of transistors with the same implant type are collapsed

by the -s option into a single transistor with a "compound" gate;

-

compound gates appear as a parenthesized list of nodes (e.g., the

pulldown shown above).  The three resistance values -- static,

6

IRSIM(1)

IRSIM Users's Manual

dynamic high, dynamic low -- are given in Kilo-ohms.

Finally, any pending events for a node are listed after the electr-

ical information.

! wnode...

-----

For each node in the argument list, print a list of transistors

controlled by that node.

tcap

Prints a list of all transistors with their source/drain shorted

together or whose source/drain are connected to the power supplies.

These transistors will have no effect on the simulation other than

their gate capacitance load.  Although transistors connected across

the power supplies are real design errors, the simulator does not

complain about them.

Any node can be made an input -- the simulator will not change an input

node's value until it is released.  Usually on specific nodes -- inputs

to the circuit -- are manipulated using the commands below, but you can

fool with a subcircuit by forcing values on internal nodes just as

easily.

h wnode...

-----

Force each node on the argument list to be a high (1) input.  Over-

rides previous input commands if necessary.

l wnode...

-----

Like "h" except forces nodes to be a low (0) input.

u wnode...

-----

Like "h" except forces nodes to be a undefined (X) input.

x wnode...

-----

Removes nodes from whatever input list they happen to be on.  The

next simulation step will determine the correct node value from the

surrounding circuit.  This is the default state of most nodes.

Note that this does not force nodes to have an "X" value -- it sim-

ply removes them from the input lists.

inputs

prints the high, low, and undefined input lists.

It is possible to define a sequence of values for a node, and then cycle

the circuit as many times as necessary to input each value and simulate

the network.  A similar mechanism is used to define the sequence of

values each clock node goes through during a single cycle.

Each value is a list of characters (with no intervening blanks) chosen

from the following:

7

IRSIM(1)

IRSIM Users's Manual

1, h, H     logic high (1)

0, l, L     logic low (0)

u, U        undefined (X)

x, X        remove node from input lists

Presumably the length of the character list is the same as the size of

the node/vector to which it will be assigned.  Blanks (spaces and tabs)

are used to separate values in a sequence.  The sequence is used one

value at a time, left to right.  If more values are needed than supplied

by the sequence, IRSIM just restarts the sequence again.

V [node [value...]]

----  -----

Define a vector of inputs for a node.  After each cycle of an "R"

command, the node is set to the next value specified in the

sequence.

With no arguments, clears all input sequences (does not affect

clock sequences however).  With one argument, "node", clears any

input sequences for that node/vector.

clock [node [value...]]

----  -----

Define a phase of the clock.  Each cycle, each node specified by a

clock command must run through its respective values.  For example,

clock phi1 1 0 0 0

clock phi2 0 0 1 0

defines a simple 4-phase clock using nodes phi1 and phi2. Alterna-

----     ----

tively one could have issued the following commands:

vector clk phi1 phi2

clock clk 10 00 01 00

With no arguments, clears all clock sequences.  With one argument,

"node", clears any clock sequences for that node/vector.

After input values have been established, their effect can be propagated

through the network with the following commands.  The basic simulated

time unit is 0.1ns; all event times are quantized into basic time units.

A simulation step continues until stepsize ns. have elapsed, and any

--------

events scheduled for that interval are processed.  It is possible to

build circuits which oscillate -- if the period of oscillation is zero,

the simulation command will not return.  If this seems to be the case,

you can hit <ctrl-C> to return to the command interpreter.  Note that if

you do this while input is being taken from a file, the simulator will

bring you to the top level interpreter, aborting all pending input from

any command files.

When using the linear model (see the "model" command) transition times

are estimated using an RC time constant calculated from the surrounding

circuit.  When using the switch model, transitions are scheduled with

unit delay.  These calculations can be overridden for a node by setting

its tplh and tphl parameters which will then be used to determine the

8

IRSIM(1)

IRSIM Users's Manual

time for a transition.

s [n]

-

Simulation step.  Propogates new values for the inputs through the

network, returns when n (default: stepsize) ns. have passed.  If n

-           --------                       -

is specified, it will temporarily override the stepsize value.

--------

Unlike previous versions, this value is NOT remembered as the

default value for the stepsize parameter.  If the display mode is

--------

"automatic", the current display list is printed out on the comple-

tion of this command (see "display" command).

c [n]

-

Cycle n times (default: 1) through the clock, as defined by the

-

"clock" command.  Each phase of the clock lasts stepsize ns.  If

--------

the display mode is "automatic", the current display list is

---------

printed out on the completion of this command (see "display" com-

mand).

p    Step the clock through one phase (or simulation step).  For exam-

ple, if the clock is defined as above

clock phi1   1 0 0 0

clock phi2   0 0 1 0

then "p" will set phi1 to 1 and phi2 to 0, and then propagate the

effects for one simulation step.  The next time "p" is issued, phi1

and phi2 will both be set to 0, and the effects propagated, and so

on.  If the "c" command is issued after "p" has been used, the

effect will be to step through the next 4 phases from where the "p"

command left off.

R [n]

-

Run the simulator through n cycles (see the "c" command).  If n is

-                                   -

not present make the run as long as the longest sequence.  If

display mode is automatic (see "display" command) the display is

printed at the end of each cycle.  Each "R" command starts over at

the beginning of the sequence defined for each node.

back time

----

Move back to the specified time.  This command restores circuit

state as of time, effectively undoing any changes in between.  Note

----

that you can not move past any previously flushed out history (see

flush command below) as the history mechanism is used to restore

the network state.  This command can be useful to undo a mistake in

the input vectors or to re-simulate the circuit with a different

debug level.

path wnode...

-----

display critical path(s) for last transition of the specified

node(s).  The critical path transistions are reported using the

following format:

9

IRSIM(1)

IRSIM Users's Manual

node -> value @ time (delta)

----    -----   ----  -----

where node is the name of the node, value is the value to which the

----                          -----

node transitioned, time is the time at which the transistion

----

occurred, and delta is the delay through the node since the last

-----

transition.  For example:

critical path for last transition of Hit v1:

-

phi1-> 1 @ 2900.0ns , node was an input

PC driver-> 0 @ 2900.4ns    (0.4ns)

-

PC b q1-> 1 @ 2904.0ns    (3.6ns)

- -

tagDone b v1-> 0 @ 2912.8ns    (8.8ns)

- -

tagDone1 v1-> 1 @ 2915.3ns    (2.5ns)

-

tagDone1 b v1-> 0 @ 2916.0ns    (0.7ns)

- -

tagDone v1-> 1 @ 2918.4ns    (2.4ns)

-

tagCmp b v1-> 0 @ 2922.1ns    (3.7ns)

- -

tagCmp v1-> 1 @ 2923.0ns    (0.9ns)

-

Vbit b v1-> 0 @ 2923.2ns    (0.2ns)

- -

Hit v1-> 1 @ 2923.5ns    (0.3ns)

-

activity from time [to time]

---------  -------

print histogram showing amount of circuit activity in the specified

time inteval.  Actually only shows number of nodes which had their

most recent transition in the interval.

changes from time [to time]

---------  -------

print list of nodes which last changed value in the specified time

interval.

printp

print list of all pending events sorted in time.  The node associ-

ated with each event and the scheduled time is printed.

printx

print a list of all nodes with undefined (X) values.

Using the trace command, it is possible to get more detail about what's

happening to a particular node.  Much of what is said below is described

in much more detail in "Logic-level Simulation for VLSI Circuits" by

Chris Terman, available from Kluwer Academic Press.  When a node is

traced, the simulator reports each change in the node's value:

[event #100] node out.1: 0 -> 1 @ 407.6ns

The event index is incremented for each event that is processed.  The

transition is reported as

old value -> new value @ report time

--- -----    --- -----   ------ ----

Note that since the time the event is processed may differ from the

event's report time, the report time for successive events may not be

strictly increasing.

10

IRSIM(1)

IRSIM Users's Manual

Depending on the debug level (see the "debug" command) each calculation

of a traced node's value is reported:

[event #99] node clk: 0 -> 1 @ 400.2ns

final value( Load )  V=[0.00, 0.04]  => 0

-

..compute tau( Load )

-

{Rmin=2.2K  Rdom=2.2K  Rmax=2.2K}  {Ca=0.06  Cd=0.17}

tauA=0.1  tauD=0.4 ns

[event #99: clk->1] transition for Load: 1 -> 0 (tau=0.5ns,

delay=0.6ns)

In this example, a calculation for node Load is reported.  The calcula-

----

tion was caused by event 99 in which node clk went to 1.  When using the

linear model (as in this example) the report shows

current value -> final value

------- -----    ----- -----

The second line displays information regarding the final value (or dc)

analysis for node "Load"; the minimun and maximum voltages as well as

the final logical value (0 in this case).

The next three lines display timing analysis information used to esti-

mate the delays.  The meaning of the variables displayed can be found

Chu's thesis: "Improved Models for Switch-Level Simulation".

When the final value is reported as "D", the node is not connected to an

----- -----

input and may be scheduled to decay from its current value to X at some

later time (see the "decay" command).

-------

"tau" is the calculated transition time constant, "delta" is when any

consequences of the event will be computed; the difference in the two

times is how IRSIM accounts for the shape of the transition waveform on

subsequent stages (see reference given above for more details).  The

middle lines of the report indicate the Thevenin and capacitance parame-

ters of the surrounding networks, i.e., the parameters on which the

transition calculations are based.

debug [ev dc tau taup tw spk][off][all]

-- -- --- ---- -- ---  ---  ---

Set debugging level.  Useful for debugging simulator and/or circuit

at various levels of the computation.  The meaning of the various

debug levels is as follows:

ev      display event enqueueing and dequeueing.

dc      display dc calculation information.

tau     display time constant (timing) calculation.

taup    display second time constant (timing) calculation.

tw      display network parameters for each stage of the tree walk,

this applies to dc, tau, and taup.  This level of debugging

detail is usually needed only when debugging the simulator.

11

IRSIM(1)

IRSIM Users's Manual

spk     displays spike analysis information.

all     This is a shorthand for specifying all of the above.

off     This turns off all debugging information.

If a debug switch is on then during a simulation step, each time a

watched node is encounted in some event, that fact is indicated to

the user along with some event info.  If a node keeps appearing in

this prinout, chances are that its value is oscillating.  Vice

versa, if your circuit never settles (ie., it oscillates) , you can

use the "debug" and "t" commands to find the node(s) that are caus-

ing the problem.

Without any arguments, the debug command prints the current debug

level.

t [-]wnode...

-----

set trace flag for node.  Enables the various printouts described

above.  Prefacing the node name with '-' clear its trace flag.  If

"wnode" is the name of a vector, whenever any node of that vector

changes value, the current time and the values of all traced vec-

tors is printed.  This feature is useful for watching the relative

arrival times of values at nodes in an output vector.

System interface commands:

> filename

--------

Write current state of each node into specified file.  Useful for

making a breakpoint in your simulation run.  Only stores values so

isn't really useful to "dump" a run for later use, i.e., the

current input lists, pending events, etc. are NOT saved in the

state file.

< filename

--------

Read from specified file, reinitializing the value of each node as

directed.  Note that network must already exist and be identical to

the network used to create the dump file with the ">" command.

These state saving commands are really provided so that complicated

initializing sequences need only be simulated once.

<< filename

--------

Same as "<" command, except that this command will restore the

input status of the nodes as well.  It does not, however, restore

-----

pending events.

dumph [filename]

--------

Write the history of the simulation to the specified file, that is;

all transistions since time = 0.  The resulting file is a machine-

independent binary file, and contains all the required information

to continue simulation at the time the dump takes place.  If the

filename isn't specified, it will be constructed by taking the name

of the sim file (from the command line) and appending ".hist" to

-

12

IRSIM(1)

IRSIM Users's Manual

it.

readh filename

--------

Read the specified history-dump file into the current network.

This command will restore the state of the circuit to that of the

dump file, overwriting the current state.

flush [time]

----

If memory consumption due to history maintanance becomes prohibi-

tive, this command can be used to free the memory consumed by the

history up to the time specified.  With no arguments, all history

up to the current point in the simulation is freed.  Flushing out

the history may invalidate an incremental simulation and the por-

tions flushed will no longer appear in the analyzer window.

setpath [path...]

----

Set the search-path for command files.  Path should be a sequence

----

of directories to be searched for ".cmd" files, "." meaning the

current directory.  For eaxmple:

setpath . /usr/me/rsim/cmds /cad/lib/cmds

With no arguments, it will print the current search-path.  Ini-

tially this is just ".".

print text...

----

Simply prints the text on the user's console.  Useful for keeping

user posted of progress through a long command file.

logfile [filename]

--------

Create a logfile with the specified name, closing current log file

if any; if no argument, just close current logfile.  All output

which appears on user's console will also be placed in the logfile.

Output to the logfile is cleverly formatted so that logfiles them-

selves can serve as command files.

setlog [filename | off]

--------   ---

Record all net changes, as well as resulting error messages, to the

specified file (see "update" command).  Net changes are always

appended to the log-file, preceding each sequence of changes by the

current date.  If the argument is off then net-changes will not be

---

logged.  With no arguments, the name of the current log-file is

printed.

The default is to always record net changes; if no filename is

specified (using the "setlog" command) the default filename

irsim changes.log will be used.  The log-files are formatted so

------------- ---

that log-files may themselves be used as net-change files.

wnet [filename]

--------

Write the current network to the specified file.  If the filename

isn't specified, it will be constructed by taking the name of the

sim file (from the command line) and appending ".inet" to it.  The

-

13

IRSIM(1)

IRSIM Users's Manual

resulting file can be used in a future simulation run, as if it

were a sim file.  The file produced is a machine independent binary

file, which is typically about 1/3 the size of the sim file and

about 8 times faster to load.

time [command]

-------

With no argument, a summary of time used by the simulator is

printed.  If arguments are given the specified command is timed and

a time summary is printed when the command completes.  The format

of the time summary is Uu Ss E P% M, where:

-  -  - -  -

U => User time in seconds

-

S => System time in seconds

-

E => Elapsed time, minutes:seconds

-

P => Percentage of CPU time (((U + S)/E) * 100)

-

M => Median text, data, and stack size use

-

q

Terminate current input stream.  If this is typed at top level, the

simulator will exit back to the system; otherwise, input reverts to

the previous input stream.

exit [n]

-

Exit to system, n is the reported status (default: 0).

-

Simulator parameters are set with the following commands.  With no argu-

ments, each of the commands simply prints the current value of the

parameter.

decay [n]

-

Set decay parameter to n ns. (default: 0).  If non-zero, it tells

-

the number of ns. it takes for charge on a node to decay to X.  A

value of 0 implies no decay at all.  You cannot specify this param-

eters separately for each node, but this turns out not to be a

problem.  See "report" command.

display [-][cmdfile][automatic]

-------  ---------

set/reset the display modes, which are

cmdfile     commands executed from command files are displayed to

user before executing.  The default is cmdfile = OFF.

-------   ---

automatic   print out current display list (see "d" command) after

completion of "s" or "c" command.  The default is

automatic = ON.

---------   --

Prefacing the previous commands with a "-" turns off that display

option.

model [name]

----

Set simulation model to one of the following:

switch

14

IRSIM(1)

IRSIM Users's Manual

Model transistors as voltage controlled switches.  This model

uses interval logic levels, without accounting for transistor

resistances, so circuits with fighting transistors may not be

accuratelly modelled.  Delays may not reflect the true speed of

----

the circuit as well.

linear

Model transistors as a resistor in series with a voltage con-

trolled switch.  This model uses a single-time-constant computed

from the resulting RC network and uses a two-time-constant model

to analyze charge sharing and spikes.

The default is the linear model.  You can change the simulation

model at any time -- even with events pending -- as only new calcu-

lations are affected.  Without arguments, this command prints the

current model name.

report [level]

-----

When level is nonzero, report all nodes which are set to X because

of charge decay, regardless on whether they are being traced.  Set-

ting level to zero disables reporting, but not the decay itself

(see "decay" command).

stepsize [n]

-

Specify duration of simulation step or clock phase. n is specified

- -- ---------

in ns. (nanoseconds).  Floating point numbers with up to 1 digit

past the decimal point are allowed.  Further decimals are trucated

(i.e. 10.299 == 10.2).

unitdelay [n]

-

When nonzero, force all transitions to take n ns.  Setting the

-

parameter to zero disables this feature.  The resolution is the

same as for the "stepsize" command.

stats

Print event statitistics, as follows:

changes = 26077

punts (cns) = 208 (34)

punts = 0.79%, cons punted = 16.35%

-

nevents = 28012; evaluations = 27972

Where changes is the total number of transistions recorded, punts

-------                                               -----

is the number of punted events, (cns) is the number of consecutive

---

punted events (a punted event that punted another event).  The

penultimate line shows the percentage of punted events with respect

to the total number of events, and the percentage of consecutive

punted events with respect to the number of punted events.  The

last line shows the total number of events (nevents) and the number

of net evaluations.

Incremental simulation commands:

15

IRSIM(1)

IRSIM Users's Manual

Irsim supports incremental changes to the network and resimulation of

the resulting network.  This is done incrementally so that only the

nodes affected by the changes, either directly or indirectly, are re-

evaluated.

update filename

--------

Read net-change tokens from the specified file.  The following

net-change commands are available:

add     type gate source drain length width [area]

delete  type gate source drain length width [area]

move    type gate source drain length width [area] g s d

cap     node value

N       node metal-area poly-area diff-area diff-perim

M       node M2A M2P MA MP PA PP DA DP PDA PDP

thresh  node low high

Delay   node tplh tphl

For a detailed dscription of this file see netchange(5).  Note that

this is an experimental interface and is likely to change in the

future.

Note that this command doesn't resimulate the circuit so that it

may leave the network in an inconsistent state.  Usually this com-

mand will be followed by an isim command (see below), if that is

not the case then it's up to the user to initilize the state of the

circuit.  This command exists only for historical reasons and will

probably disappear in the future.  It's use is discouraged.

isim [filename]

--------

Read net-change tokens from the specified file (see netchange(5))

and incrementally resimulate the circuit up to the current simula-

tion time (not supported yet).

ires n

-

The incremental algorithm keeps track of nodes deviating from their

past behavior as recorded in the network history.  During resimula-

tion, a node is considered to deviate from its history if it's new

state is found to be different within n ns of its previous state.

-

This command allows for changing the incremental resolution.  With

no arguments, it will print the current resolution.  The default

resolution is 0 ns.

powlogfile [filename]

--------

Opens filename for writting nodal transition reports. The format of

--------

the report is the same you get when you trace a node normaly.  With

no arguments powlogfile just closes the opened logfile and prints

out a power dissipation summary. Nodal transitions in inputs are

not included in the transition count.

powtrace [[-]node...]

----

The syntax of this command is the same as the normal t (trace)

16

IRSIM(1)

IRSIM Users's Manual

command.  If you want to trace and report power dissipation for all

the nodes just use powtrace *. Use powtrace -node if you want to

exclude some nodes.

powstep

Toggles whether dynamic power estimation is displayed after each

timestep. The ynamic power displayed will only be for the nodes

that have been selected using the powtrace command.

vsupply voltage

-------

Sets the V variable for use in the P=CV^2/(2t) expression where C

is capacitance switched, and t is the timestep.  The default value

for vsupply is 5.0 Volts.

sumcap

Gives a sum of all nodal capcitances, not just those selected with

the powtrace command.

SEE ALSO

presim(1) (now obsolete)

rsim(1)

irsim-analyzer(3)

sim(5)

netchange(5)

17